3GPP TSG-RAN WG1 Meeting #124	R1-260XXXX

Gothenburg, Sweden, Feb 9th – 13th, 2026

Agenda Item:	10.4

Source:	Moderators (Ericsson, MediaTek)

Title:	Summary #4 of Energy Efficiency Study

Document for:	Discussion, Decision

Introduction

This summary synthesizes observations and proposals from contributions submitted to 3GPP TSG RAN WG1 #124, for EE for 6G Radio (6GR) in AI 10.4 and as described in the SID. The contributions address remaining issues on BS power consumption models, UE power consumption models, and necessary evaluation assumptions.

Please note that this summary has partially been produced with the help of AI. Companies are therefore encouraged to verify that their views are correctly represented.

Summary of Agreements in RAN1 #124

  - Proposals for Online Discussion

BS modeling

TBD

UE modeling

Proposal 4.1.2-1b:

For CA reception based on one RF receive chain, power scaling factor is same as that of bandwidth scaling with the same total bandwidth.

- Same PDCCH BD number is assumed. FFS: Additional scaling w.r.t. increased PDCCH BD number.

- Scaling on steady power consumption after carrier activation, no implication on carrier activation/deactivation delay.

Proposal 4.1.1-3a:

Update PDCCH+PDSCH bandwidth scaling table in the 6GR UE power model with the following values

- Subject to adaptation delay = Tmin, no larger than NR BWP switch delay (Type 2)

    - SMaxBW can be zero, or the other option: Z1 and Z2 for MaxBW = 200% and 400%, respectively. Company to report which option assumed in their evaluation.

| ΓB | ΓTput=1 | ΓTput=1/2 | ΓTput=1/4 |
| --- | --- | --- | --- |
| 5% | 0.30 + SMaxBW | - | - |
| 20% | 0.40 + SMaxBW | - | - |
| 100% | 1.00 + SMaxBW | - | - |
| 200% | 2.00 + SMaxBW | 1.45 + SMaxBW | - |
| 400% | 4.60 | 3.20 | 2.40 |

- If adaptation delay 5ms≤T≤[10 ms] is allowed, the following scaling factors for ΓB = 5% and 20% are applied:

| ΓB | Scaling factor when adaptation delay T>Tmin is assumed |
| --- | --- |
| 5% | 0.20 |
| 20% | 0.25 |

Proposal 4.1.1-4a:

Update PDCCH-only bandwidth scaling table in the 6GR UE power model with the following values:

- Subject to adaptation delay = Tmin, no larger than NR BWP switch delay (Type 2)

- SMaxBW can be zero, or the other option: V1 and V2 for MaxBW = 200% and 400%, respectively. Company to report which option assumed in their evaluation.

- Other DL signal/channel processing: refer to the first scaling column (ΓTput=1).

| ΓB | ΓTput=1 | ΓTput<=1/2 |
| --- | --- | --- |
| 5% | 0.65 + SMaxBW | - |
| 20% | 0.70 + SMaxBW | - |
| 100% | 1.00 + SMaxBW | - |
| 200% | 1.60 + SMaxBW | 1.30 + SMaxBW |
| 400% | 3.20 | 2.40 |

- If adaptation delay 5ms≤T≤[10 ms] is allowed, the following scaling factors for ΓB = 5% and 20% are applied:

| ΓB | Scaling factor when adaptation delay T>Tmin is assumed |
| --- | --- |
| 5% | 0.35 |
| 20% | 0.40 |

Proposal 4.1.4-3a:

Adopt power scaling formula for PDCCH candidate/BD reduction:

P(α) = max(P_microsleep, α × P_t + (1 − α) × 0.85 × P_t)

where α = configured #BD candidates / maximum #BD candidates.

Proposal 4.1.5-1a:

Include the scaling rules for micro sleep and light sleep in the 6GR UE power model:

- Subject to adaptation delay = Tmin, no larger than NR BWP switch delay (Type 2)

- SMaxBW can be zero, or the other option: U1 and U2 for MaxBW = 200% and 400%, respectively. Company to report which option assumed in their evaluation.

| ΓB | ΓTput=1 |
| --- | --- |
| <=20% | 0.70 + SMaxBW |
| 100% | 1.00 + SMaxBW |
| >=200% | 2.40 + SMaxBW |

- If adaptation delay 5ms≤T≤[10 ms] is allowed, the following scaling factors for ΓB = 5% and 20% are applied:

| ΓB | Scaling factor when adaptation delay T>Tmin is assumed |
| --- | --- |
| <= 20% | 0.40 |

Proposal 4.2.2-2a:

- For EE Processing 2-RX in FR1, adopt the following values:

| Sleep State | 2-RX | 1-RX (agreed; reference only) |
| --- | --- | --- |
| Micro sleep (X1) | 14 | 10 |
| Light sleep (X2) | 17 | 12 |
| Deep/Ultra-deep (X3) | 21 | 15 |
| Additional energy overhead | 21 | 15 |

- FFS: Values for 10MHz BW

Moderator note: Please check Boundary_condition_check.xlsx for the above values

Proposal 4.2.6-2a:

Adopt the following UL scaling for 6GR UE power consumption model:

|  | Scaling | Comment |
| --- | --- | --- |
| Short PUCCH | Short PUCCH power = 0.3 x uplink power (Assume short PUCCH of 1-symbol) | Applicable for FR1 (including around 7GHz) and FR2 (including 24.25 GHz – 52.6 GHz). |
| SRS | SRS power = 0.3 x uplink power (Assume SRS of 1-symbol) | Applicable for FR1 (including around 7GHz) and FR2 (including 24.25 GHz – 52.6 GHz). |

- FFS: Scaling for different symbol numbers

Proposal 4.1.3-2a:

Include the following UL antenna scaling factors (2Tx relative to 1Tx):

| TX Power | Scaling Factor (2Tx relative to 1Tx) |
| --- | --- |
| 0 dBm | [1.6] |
| 23 dBm | [1.4] |
| 26 dBm | [1.7] |

Note: UE reference configuration is 1TX chain

| Company | 2Tx @0dBm | 2Tx @23dBm | 2Tx @26dBm (Z9) | Notes |
| --- | --- | --- | --- | --- |
| Huawei | 2 | 1.6 | 1.6 | No scalilng or TX number for 23/26dBm, since PA is the dominate part |
| MediaTek | 2 | 2 | 2 |  |
| Qualcomm | 1.4 | 1.2 | 1.6 |  |
| Spreadtrum | 1.4 | 1.2 |  | Reuse scaling rule in TR 38.840 |
| Xiaomi | 1.4 | 1.2 |  |  |
| Avg(all) | 1.64 | 1.44 | 1.733 |  |

Proposal 4.1.2-2a:

UL CA scaling for the 6GR UE power consumption model:

At TX power ≤ 10 dBm: same as DL CA scaling (i.e., follows Proposal 4.1.2-1b for single-RF-chain).

| TX Power | 2CC | 4CC |
| --- | --- | --- |
| ≤ 10 dBm | Same as DL CA | Same as DL CA |
| > 10 dBm | [1.2] | [1.5] |

FFS: Multi-RF-chain UL CA scaling.

Proposal 4.1.4-2 (FFS):

FFS: PDCCH symbol duration scaling for the 6GR UE power model:

| PDCCH Duration | Scaling Factor |
| --- | --- |
| 1-symbol | 0.9 |
| 2-symbol (ref) | 1.0 |
| 3-symbol | 1.1 |

Proposal 4.2.3-1 (FFS):

FFS: PDSCH-only and BB-only active states for evaluation purposes in the 6GR UE power model.

Proposal 4.2.4-1 (FFS):

FFS: Ultra deep sleep state for IoT UEs in the 6GR UE power model:

Transition energy: [15000]

Transition time: [800 ms]

Made Agreements

Agreement

For NES evaluation purposes and relative comparison of different candidate energy saving schemes for 6GR, define the following baseline network configurations

- SSB with 20 ms periodicity, at least for single cell

- SIB1, if available, company to report assumed periodicity from {20 ms, 160 ms}

- RO, if available, with 10/20 ms periodicity

Furthermore, to assist comparisons

- Companies to report sa, sp and sf values for BS processing of the above signal(s)/channel(s)

- Companies to report the average network load in %

- Companies can evaluate and report other configuration(s) with justification

Note: The corresponding evaluation is not intended for energy efficiency comparison with 5G/NR.

Agreement

The following transition times are adopted for BS models CAT1 and CAT2, if supported, for BS reference configuration Set 4:

| Power state | BS Category 1, Set 4 | BS Category 2, Set 4 |
| --- | --- | --- |
| Deep sleep | 50 ms | 10 s |
| Light sleep | 6 ms | 640 ms |

FFS: whether delta value is needed due to larger number of TXRU

Agreement

The following relative transition energies are adopted for BS models CAT1 and CAT2, if supported, for BS reference configuration Set 4:

| Power state | BS Category 1, Set 4 | BS Category 2, Set 4 |
| --- | --- | --- |
| Deep sleep | 1075 | 31500 |
| Light sleep | 163 | 1344 |

FFS: whether delta value is needed due to larger number of TXRU

Agreement

IF a BS model CAT 2.1 (2-plus) is introduced, it has the following transition energy characteristics for Set 1-3. FFS: Set 4:

| Transition Time (Deep Sleep) | Transition Time (Light Sleep) | Additional Transition Energy (Deep Sleep) | Additional Transition Energy (Light Sleep) |
| --- | --- | --- | --- |
| 2 s | 100 ms | 3400 | 170 |

Agreement

Include the following UL long PUCCH/PUSCH/PRACH power values in the UE power model:

- Note: UE reference configuration is 1TX chain

| TX Power | Relative Power |
| --- | --- |
| 0 dBm | 250 (agreed; reference only) |
| 10 dBm (X4) | 340 |
| 15 dBm (X5) | 410 |
| 20 dBm (X6) | 560 |
| 23 dBm | 700 (agreed; reference only) |
| 26 dBm (X7) | 1100 |

Agreement

Adopt the following cross-slot scheduling scaling for evaluation for 6GR UE power consumption model:

|  | Scaling | Comment |
| --- | --- | --- |
| PDCCH-only | Power of cross-slot scheduling is 0.7x same-slot scheduling | Applicable for FR1 (including around 7GHz) and FR2 (including 24.25 GHz – 52.6 GHz). |

Agreement

Include the following DL antenna scaling factors in 6GR UE power consumption model:

- Note: Applicable for FR1 (including around 7GHz) and FR2 (including 24.25 GHz – 52.6 GHz), where, for FR2, number of DL antenna assumed is up to [4]

- FFS: Scaling for 8Rx

| UE Rx Antenna Configuration | Scaling |
| --- | --- |
| 6Rx | 1.4 × 4Rx |
| 4Rx | 1.0 |
| 2Rx | 0.7 × 4Rx |
| 1Rx | 0.7 × 2Rx |

Base Station Modeling

Baselines for Assessment

Baseline evaluation configurations

Summary of companies’ views

To assess the relative gains of new schemes, companies proposed standard "always-on" periodicities for baseline evaluations.

There is a broad consensus for a 20 ms baseline for the Primary Cell (PCell). NTT DOCOMO suggests a baseline of 160 ms for Secondary Cells (SCells) or capacity carriers.

Views are split. OPPO, CATT, vivo, and CMCC support a 20 ms baseline to reflect typical 5G deployments. Conversely, Tejas, Samsung, and Jio propose 160 ms (the standard NR maximum) to avoid an unrealistically "worst-case" baseline.

Most companies support a 10 ms or 20 ms baseline.

Xiaomi and Samsung propose using Search Space #0 (SS#0) for paging in the baseline to avoid extra power modeling. Tejas suggests treating paging as traffic-dependent rather than hard-fixed.

FL comments and proposals

This topic was only briefly discussed in RAN1 #123 and it is FL’s view that RAN1 was close to an agreement. For that reason, FL continues where RAN1 left off last time as a first attempt of a proposal:

FL Proposal 3.1.1.1

For NES evaluation purposes and relative comparison of different candidate energy saving schemes for 6GR, define the following baseline network configurations

- SSB with 20 ms periodicity, for Pcell

- SIB1, if available, company to report assumed periodicity from {20 ms, 160 ms}

- RO, if available, with 10/20 ms periodicity

Furthermore, to assist comparisons

- Companies to report sa, sp and sf values for BS processing of the above signal(s)/channel(s)

- Companies to report the average network load in %

- Companies can evaluate and report other configuration(s) with justification

Note: The corresponding evaluation is not intended for energy efficiency comparison with 5G/NR.

FL Proposal 3.1.1.1b

For NES evaluation purposes and relative comparison of different candidate energy saving schemes for 6GR, define the following baseline network configurations

- SSB with 20 ms periodicity, at least for Pcell

- SIB1, if available, company to report assumed periodicity from {20 ms and 160 ms}

- RO, if available, with 10/20 ms periodicity

Furthermore, to assist comparisons

- Companies to report sa, sp and sf values for BS processing of the above signal(s)/channel(s)

- Companies to report the average network load in %

- Companies can evaluate and report other configuration(s) with justification

Note: The corresponding evaluation is not intended for energy efficiency comparison with 5G/NR.

Companies’ comments

Companies’ comments on the above proposal are appreciated.

| Company | Comment |
| --- | --- |
| CMCC | We agree with companies that SIB1 periodicity can be up to 160ms according to TS 38.331, but such configuration may not exist in real deployment since larger SIB1 periodicity may result in failure on cell selection procedure in real test. Therefore, we suggest to consider a more realistic configuration in real deployment to reflect the power consumption in real NR network. For PRACH configuration, we are ok with both values since both configurations are used in some area for real deployment. |
| LG Electronics | Support |
| OPPO | We support FL proposal |
| ZTE, Sanechips | ‘for Pcell’ could be misleading. It could be removed and keep it open. The meaning of “average” network load is confusing, and the “average” can be deleted. |
| Ericsson | Ok |
| Spreadtrum | Support |
| CEWiT | Support |
| DCM | We are fine with the proposal in general. |
| Xiaomi | Paging is also an important common signal in the commercial network and needs to be taken into account. However, for the sake of simplification, it can be assumed that the SS for paging is SS#0. We recommend adding a note to clarify this. Since we need to study scenarios such as OD-SSB and SIB1, and to facilitate subsequent evaluation and statistics, we suggest considering the periodicity of SSB and SIB1 for Scell / NES cell. This can be included either in this proposal or in a separate new proposal. |
| Nokia | OK |
| Qualcomm | Generally ok with the proposal. |
| Samsung | Generally OK |
| Huawei, HiSilicon | We don’t see the need to agree on a baseline given the parallel discussion in initial access that are closely related to the SS/SSB/SIB/RACH design. However, if really something can be agreed based on majority interest, we suggest the following: - For the design of common signals/channels, we would like to see the comparison from not only NES perspective but also other aspects e.g. coverage. Suggest to add one bullet at the moment since details will/can be added once clear from IA session: - Coverage impact as well as enhancement techniques should also be taken into account for relative comparison of different candidate energy saving schemes. Details to be added according to the progress from initial access. - Agree with CMCC that although 160 ms SIB1 periodicity is supported but rarely is a typical deployment choice. Fine to choose from e.g. 20/40ms. - Suggest to add Paging channel with details to be FFS. |
| InterDigital | OK |
| Apple | OK |
| CATT | We have the same comment on SIB1 periodicity as CMCC. And the update as follows: FL Proposal 3.1.1.1 For NES evaluation purposes and relative comparison of different candidate energy saving schemes for 6GR, define the following baseline network configurations - SSB with 20 ms periodicity, for Pcell - SIB1, if available, company to report assumed periodicity from {20 ms, 160 ms} - RO, if available, with 10/20 ms periodicity |
| vivo | Agree that 160ms SIB1 should be removed. |
| Huawei, HiSilicon | We propose to defer the discussion of this proposal which is not essential. We do not see the need of the proposal, if the baseline does not consider coverage aspects, thus it can be discussed once initial access has more progress. |

Network load levels

Summary of companies’ views

The contributions present two main perspectives regarding network load levels for 6G energy efficiency evaluations: reusing the legacy 5G definitions or refining them based on real-world traffic data to better capture low-load scenarios.

Proposals to Reuse Legacy 5G Load Levels

Several contributors advocate for maintaining the load level definitions established in TR 38.864 to ensure consistency and comparability between 5G and 6G results.

Spreadtrum, CATT, Xiaomi, ZTE, and vivo propose reusing the 5G load levels as a starting point. These levels are defined based on Resource Utilization (RU) ratio (L) as: Empty (L=0%), Low (0 < L ≤ 15%), Light (15 < L ≤ 30%), and Medium (30 < L ≤ 50%).

Spreadtrum notes that there is no strong motivation to redefine load levels as they represent cell resource utilization rather than time distribution. vivo argues that using the same definitions prevents drawing different conclusions for the same technique across 5G and 6G. Jio proposes using these legacy levels for baseline evaluations, specifically specifying that cell-common signaling (SSB, SIB1, PRACH) should be evaluated at an empty load (L=0).

Proposals to Refine Load Levels

Some contributors argue that existing 5G definitions are skewed and do not accurately reflect operational realities, where cells often operate at very low loads.

Ericsson, Huawei, and Tejas observe that the majority of real-world traffic is concentrated below 15% load. Ericsson’s data across 100+ networks shows that 61.2% of sampled periods operate below 15% load, and 40.8% operate below 5%. To avoid "over-averaging" and masking the benefits of energy-saving techniques that activate during low traffic, contributors propose finer granularity in the low-load regime.

Huawei and Ericsson propose: Empty (L=0%), Low (0 < L ≤ 5%), Light (5 < L ≤ 15%), Medium (15 < L ≤ 35%), and High (L > 35%) based on traffic loads in actual networks, see Figure 1.

Figure 1: Traffic load distributions in actual networks (Huawei, Ericsson).

Tejas proposes: Empty (L=0%), Low (0 < L ≤ 10%), Light (10 < L ≤ 20%), and Medium (20 < L ≤ 40%).

Ericsson points out that time-domain energy savings (e.g., deep sleep) are most effective at these lower load levels, which are not adequately represented by the legacy 15% "Low load" threshold.

Other Observations and Methodologies

ZTE notes that load level (L%) is defined as the percentage of resources used for UE-specific PDSCH/PUSCH. ZTE also highlights that traffic load is a statistical parameter measured on the cell side, not from the UE's perspective.

Lenovo suggests that 3GPP could additionally consider frequency range-specific daily average metrics for the amount of time a network spends in certain load levels, as lower frequencies often carry higher loads while higher bands (e.g., FR3) may experience bursty traffic.

OPPO and NTT DOCOMO are open to studying redefined load levels but note that since current field data is based on 5G, its ability to predict 6G service characteristics requires further study.

FL comments and proposals

A majority of companies want to leave the load levels unchanged. On the other hand, RAN1 must recognize that real world data from two major network vendors fairly consistently indicates that present load levels are a bit high.

FL believes load levels for NES assessment should evenly cover various network operating points to ensure a comprehensive overview. At the same time, some companies question the relevancy of the presented data by Ericsson and Huawei. FL (and Tejas) proposes a middle way:

FL Proposal 3.1.2.1

The following load levels (L) are used for NES evaluations in 6GR:

Empty	L=0%

Low		0 < L ≤ 10%

Light		10 < L ≤ 20%

Medium	20 < L ≤ 40%

FL Proposal 3.1.2.1b

The following load levels (L) are used for NES evaluations in 6GR:

Empty	L=0%

Low		0% < L ≤ 10%

Light		10% < L ≤ 20%

Medium	20% < L ≤ 40%

High		40% < L

Companies’ comments

Companies’ comments on the above proposal are appreciated.

| Company | Comment |
| --- | --- |
| CMCC | Fine with the proposal. |
| LG Electronics | No strong view on the criteria for determining load level. However, we believe HIGH load level will be defined, i.e., L > 40 % as HIGH in this proposal. |
| OPPO | We understand that the collected real field data reflect the NW load situation in 5G. But we would like to know if we are sure that this distribution also can represent 6G. If we are uncertain, we should stay with legacy NW load. |
| ZTE, Sanechips | Open to consider. |
| Ericsson | Low 0 < L ≤ 5% and Light 5 < L ≤ 15% fits better with the data that has been provided by companies. But this is a step in the right direction and the important thing is that companies report the load in percent, see FL proposal 3.1.1.1 |
| CEWiT | Support |
| DCM | We support this proposal. We are also OK to make the low rate lower than 10% (e.g., 5%). |
| Nokia | OK |
| Samsung | OK |
| Huawei, HiSilicon | Can accept. |
| InterDigital | Ok |
| Apple | We thank the major NW vendors for providing real field traffic. However, we do not think the change of cell load definition is needed. The real field traffic mainly falls in the low load traffic may be due to many reasons, and may present different characteristics if we look into more details, e.g. only look at dense urban case, or only look at rural case, etc. According to FL Proposal 3.1.1.1, companies can already report the exact cell load obtained in the evaluation, it is already possible to look into details, therefore we don not see any need to update the cell range definition. |
| CATT | OK with the proposal. |
| vivo | We suggest not to change the load definition due to the following reasons: 1) non-alignment between 5G and 6G load definition that may result different observations for the same technique; 2) aligning certain RU points that needs to be evaluated is enough. As one way forward, certain RU points for evaluation can be defined, e.g., 0%, 10%, 20%, 30%, 40%, 50% can be assumed but not mentioning low/ligh/medium load. |

BS power consumption model updates

The following is in FL’s view the flow for the discussion on BS power model updates based on urgency and logical relations:

- Updates to Set 4, since Set 4 is incomplete without them.

- Updates, if any, to CAT2 BS model, since that is needed to be able to discuss further steps.

- Down-selection of BS power models, if any

- Further enhancements to the chosen BS power models, based on selected model, read LPR. For example, in FL’s view, the usefulness of LPR is quite limited if combined with CAT2 BS model due to CAT2’s long transition times.

To succeed with all this, companies and delegates need to be willing to compromise and show good faith.

Updates to Set 4

Summary of companies’ views

The contributions provide detailed observations and proposals for BS configuration Set 4, which is specifically designed for operation in the 7 GHz spectrum (cmWave/FR3). The discussions focus on finalizing its power consumption parameters and ensuring evaluation comparability.

A baseline configuration for Set 4 has already been established in previous meetings (RAN1#123) with the following properties and relative power levels:

|  | Set 4 around 7** GHz |
| --- | --- |
| Duplex | TDD |
| System BW | 400 MHz |
| SCS | 30 kHz |
| Number of TRP | 1 |
| Total number of DL TX Rus | 256 |
| Total DL power level | 56 dBm |
| Total number of UL Rx Rus | 256 |
| **Pending agreement in 11.2 whether to evaluate 15 GHz. |  |

For evaluation purposes, relative power values (normalized to the Deep Sleep state) have been defined for CAT1 and CAT2. It is observed that Set 4 relative power values are significantly higher than those for Sets 1–3 because Set 4 features much larger bandwidth and higher antenna count.

| Power state | Set 4 |  |
| --- | --- | --- |
|  | CAT 1 BS | CAT 2 BS |
| Deep sleep | 1 | 1 |
| Light sleep | 31 | 3.1 |
| Micro sleep | 85 | 7.3 |
| Active DL | 530 | 67 |
| Active UL | 175 | 10 |

Transition time and energy

As transition energies for Set 4 were not previously finalized, several companies have submitted specific proposals. Many contributors (Nokia, Huawei, ZTE, CMCC) propose a new Category 2.1 that optimizes transition times for 6G-era hardware. This allows a BS to transition to Light Sleep in 100ms and Deep Sleep in 1s or 2s.

Xiaomi proposes Set 4 transition energy of 17,000 for Deep Sleep and 1,344 for Light Sleep (for Cat 2) whereas Apple suggests calculating transition energy using the formula:

ETransit=PMicro-PDeep×TTransit2

Ericsson notes that because Set 4 relative power levels (P2 and P3) are higher than other sets, the transition energies should be increased accordingly.

Companies’ preferences regarding transition times and transition energies for CAT1and CAT2, respectively are provided below where green highlighting indicates (close to) consensus in FL’s understanding:

Table 1: Companies' proposed transition times and transition energies for CAT1 BS model and BS reference configuration Set 4.

| Company | Transition Time (Deep Sleep) | Transition Time (Light Sleep) | Transition Energy (Deep Sleep) | Transition Energy (Light Sleep) |
| --- | --- | --- | --- | --- |
| Xiaomi | 50 ms | 6 ms | 1075 | 165 |
| Apple | 50 ms | 6 ms | 2100 | 162 |
| Ericsson | 50 ms | 6 ms | 1000 | 160 |
| Samsung | — | — | 1075 | 165 |
| Average | 50 ms | 6 ms | 1312 | 163 |
| Median | 50 ms | 6 ms | 1075 | 163 |

Table 2: Companies' proposed transition times and transition energies for CAT2 BS model and BS reference configuration Set 4.

| Company | Transition Time (Deep Sleep) | Transition Time (Light Sleep) | Transition Energy (Deep Sleep) | Transition Energy (Light Sleep) |
| --- | --- | --- | --- | --- |
| Xiaomi | 10 s | 640 ms | 17000 | 1344 |
| Apple | 10 s | 640 ms | 31500 | 1344 |
| Ericsson | 10 s | 640 ms | 17000 | 1340 |
| Samsung | 10 s | 640 ms | 17000 | 1344 |
| Average | 10 s | 640 ms | 20625 | 1343 |
| Median | 10 s | 640 ms | 17000 | 1344 |

Scaling and Practical Deployments

Ericsson observes that the 400 MHz / 256 RU configuration represents a "maximal" case that may be rare in early 6G deployments. Both Ericsson and Jio propose that scaling rules should be used to evaluate more common configurations, such as 100 MHz bandwidth and 128 DL/UL RUs.

Tejas argues that 15 GHz operation should not reuse Set 4 parameters and instead requires its own dedicated "Set 5" configuration due to distinct RF/deployment assumptions.

FL’s comments and proposals

There are multiple proposals for updates and RAN1 will not be able to cover them all. FL focus of the proposals that have the largest impact on evaluations and prioritized spectrum.

There is a strong consensus among companies for Set 4 transition times per the above. FL proposes:

FL Proposal 3.2.1.1

The following transition times are adopted for BS models CAT1 and CAT2 for BS reference configuration Set 4:

| Power state | BS Category 1, Set 4 | BS Category 2, Set 4 |
| --- | --- | --- |
| Deep sleep | 50 ms | 10 s |
| Light sleep | 6 ms | 640 ms |

FL Proposal 3.2.1.1b

The following transition times are adopted for BS models CAT1 and CAT2, if supported, for BS reference configuration Set 4:

| Power state | BS Category 1, Set 4 | BS Category 2, Set 4 |
| --- | --- | --- |
| Deep sleep | 50 ms | 10 s |
| Light sleep | 6 ms | 640 ms |

Moreover, there is an almost equally strong consensus among companies for Set 4 transition energies:

FL Proposal 3.2.1.2

The following relative transition energies are adopted for BS models CAT1 and CAT2 for BS reference configuration Set 4:

| Power state | BS Category 1, Set 4 | BS Category 2, Set 4 |
| --- | --- | --- |
| Deep sleep | 1075 | 17000 |
| Light sleep | 163 | 1344 |

FL Proposal 3.2.1.2b

The following relative transition energies are adopted for BS models CAT1 and CAT2, if supported, for BS reference configuration Set 4:

| Power state | BS Category 1, Set 4 | BS Category 2, Set 4 |
| --- | --- | --- |
| Deep sleep | 1075 | 31500 |
| Light sleep | 163 | 1344 |

Since BS reference configuration Set 4 was not a typical deployment, FL also propose to introduce a Set 4 Typical to more accurately reflect a typical deployment in this spectrum.

FL Proposal 3.2.1.3

A typical BS reference configuration for Set 4 is introduced, including the following configuration parameters:

|  | Set 4 Typical around 7 GHz |
| --- | --- |
| Duplex | TDD |
| System BW | 100 MHz |
| SCS | 30 kHz |
| Number of TRP | 1 |
| Total number of DL TX RUs | 128 |
| Total DL power level | [53] dBm |
| Total number of UL Rx RUs | 128 |

Companies’ comments

Companies’ comments on the above proposal are appreciated.

| Company | Comment |
| --- | --- |
| CMCC | For Set 4+Cat 1, we are generally fine. For Set 4+Cat 2, since there is no product for commercial use in current stage, we may first discuss Proposal 3.2.2.1 (whether there can be potential enhancement on the new BS for 7GHz scenario), and then look back to this proposal. |
| LG Electronics | We don’t see the necessity of FL Proposal 3.2.1.3 in addition to the following agreement. Agreement Add the following as one of reference configurations for BS power consumption Set 4 around 7** GHz. Duplex; TDD. System BW; 400 MHz. SCS; 30 kHz. Number of TRP; 1. Total number of DL TX Rus; 256. Total DL power level; [62] dBm. Total number of UL Rx Rus; 256. **Pending agreement in 11.2 whether to evaluate 15 GHz.. Agreement Total DL power level for Set 4 is 56 dBm. |
| OPPO | We support the three FL proposals |
| ZTE, Sanechips | Proposal 3.2.1.1 and proposal 3.2.1.2 can be considered if cat 1 and cat 2 are supported to be used in 6GR. So, the proposal can be updated as follows. FL Proposal 3.2.1.1 The following transition times are adopted for BS models CAT1 and CAT2 if supported for BS reference configuration Set 4: FL Proposal 3.2.1.2 The following relative transition energies are adopted for BS models CAT1 and CAT2 if supported for BS reference configuration Set 4: For proposal 3.2.1.3, we already have a reference configuration for Set 4, it is not necessary to introduce another typical reference configuration for the same set. |
| Ericsson | Ok |
| Spreadtrum | We agree with proposal 3.2.1.1, reuse TR 38.864 is ok. |
| DCM | We support this proposal. Seeing the reference config. of FR2, 100Mhz BW is a reference, which is a typical value as one CC. We should follow the same principle for FR3. |
| Xiaomi | Ok |
| Nokia | For FL Proposal 3.2.1.1, it is OK for us to consider the hardware reusing of Cat-1 and Cat-2 for Set4 For FL Proposal 3.2.1.2, it is unclear for us why only the Light sleep is updated for Set4. Isn’t the value for Deep sleep should be updated as well, i.e. with the value of 31500? For FL Proposal 3.2.1.3, isn’t sufficient for the Set4 configuration agreement we had from Nov meeting, even it is not perfect, but we could somehow live with the agreement from Nov meeting. |
| Qualcomm | We are ok with proposals 3.2.1.1 and 3.2.1.2, but not 3.2.1.3. RAN1 already agreed on a reference configuration for Set 4. Companies can use that configuration along with the scaling rules to present results for other configurations. |
| Samsung | Regarding FL Proposal 3.2.1.3, we share the view that the current reference configuration (400 MHz / 256 RUs) represents a maximal case and may not reflect typical deployments. However, we note that the power values for a typical deployment scenario can already be derived from the agreed reference configuration using the existing scaling rules. Therefore, it is unclear to us why an additional "Set 4 Typical" reference configuration needs to be introduced separately. We are concerned that this may cause confusion, e.g., whether further scaling is to be applied on top of the typical configuration. We suggest that clarification is needed on the intended use of Set 4 Typical and its relationship with the existing scaling rules. |
| Huawei, HiSilicon | Motivation for the 3rd needs further discussion. We assume the scaling method can be used to achieve the same effect. |
| InterDigital | Ok with the first 2 proposals related to Set 4. For 3.2.1.3, intention to introduce additional evaluation assumption should be clarified. |
| Apple | We support FL Proposal 3.2.1.1 and 3.2.1.3 For Proposal 3.2.1.2, the transition energy for deep sleep state is the same as legacy Set 1,2,and 3, however, for Set 4, the power for micro sleep state and light sleep state are higher than Set 1,2,and 3, so we think higher values of transition energy should be supported for Set 4. Below we provide our proposed values: Power state; BS Category 1, Set 4; BS Category 2, Set 4. Deep sleep; 1075 1300; 17000 30000. Light sleep; 163; 1344. |
| CATT | We are ok for the Proposal 3.2.1.3. About the transmission time and transition energy for set 4, due to lack of deployment study, we suggest to referrer to 5G NR model as starting point. And if necessity, we can update the model until set4 deployment is studied. |
| vivo | We don’t see the need of re-opening the discussion for Proposal 3.2.1.3. |
| Huawei, HiSilicon | We think more discussion may be needed due to newly identified issues, including - Use cases (e.g., UE-initiated on-demand SIB1) - Specification impact - Restrictions in operation (signals and channels) - Performance limitations - Applicability of Set1 ref config to above 7GHz - As one TRP, it should be possible to use the legacy implementation defined in 5G with up to 64 TRxU - Feasibility of Cat 1 for Set 4 ref config with more than 64 TRxU - What is the assumption of the implemented modules that drives more than 64 TRxUs and how same transition operation is maintained in that case |

Updates to legacy sets

Summary of companies’ views

Companies generally support reusing and updating the legacy NR BS reference configurations while introducing new sets to address specific 6G frequency bands and operational modes.

There is broad consensus to adopt the three reference radio configuration sets defined in TR 38.864 as the structural starting point for 6G evaluations.

Updates to Frequency Ranges

Contributors have proposed clarifying or expanding frequency definitions to avoid overlap with the new 7 GHz Set 4:

vivo observes that current definitions of Set 1 (FR1 up to 7.125 GHz) overlap with the 6G Set 4 (~7 GHz). To prevent inconsistent power values for the same spectrum, they propose clarifying that Set 1 corresponds strictly to sub-6 GHz.

Tejas recommends defining a separate "Set 5 around 15 GHz" rather than reusing Set 4 parameters. They argue that 15 GHz operation involves distinct deployment and RF assumptions—such as different array sizes and transmit power levels—that materially affect power scaling and sleep opportunities.

Vodafone, Telecom Italia, and Deutsche Telekom propose a dedicated "Set 5 SBFD" to model semi-static Sub-band Full Duplex operation with 100 MHz bandwidth.

Go-to-Sleep and Wake-Up Transition Time Relations

Companies have proposed refined modelling of the transition between active and sleep states to reflect physical hardware constraints and 2030-era advancements:

Ericsson observes that in practice, "ramping down" to a sleep state is significantly faster than "ramping up" to an active state, as the latter requires re-calibrating oscillators and re-initiating interfaces. They propose that the total transition time be split into 20% for ramp-down and 80% for ramp-up.

ZTE and Nokia suggest that a unified set of transition times and energies should be applied across Sets 1, 2, 3, and 4 to reflect consistent hardware capabilities across all frequency ranges.

InterDigital observes that legacy transition energies were uniform across Sets 1, 2, and 3. They propose that if transition values are updated for the 7 GHz Set 4, they must also be updated for Set 3 to maintain consistency in modelling FR2/FR3 hardware.

FL’s comments and proposals

It has previously been discussed whether an explicit agreement is needed for existing Sets 1-3, and the consensus was that it is not. Hence, precious time will not be used for it. In FL’s view, vivo has a point in differentiating between legacy Set 1 and new Set 4 to pre-empt future discussions and disagreements. Possibly, such an agreement should also include Set 2. For that reason, FL propose:

FL Proposal 3.2.1.3

Set 1 [and Set 2] are used for sub-6 GHz spectrum of FR1 in 6GR SI.

FL apologizes for erroneous numbering in this section and provides corrections below:

FL Proposal 3.2.2.1

Set 1 is limited to sub-6 GHz spectrum of FR1 in 6GR SI.

Moreover, Ericsson touches upon a valid point in dividing transition times between going to sleep and waking up. It is worth noting that at present there is no agreement on the split although many companies assume a 50/50 split. There are valid arguments about why it should be something else.

FL Proposal 3.2.1.5

Transition times are divided into 20 % ramp-down time and 80 % ramp-up time.

Companies’ comments

Companies’ comments on the above proposal are appreciated.

| Company | Comment |
| --- | --- |
| CMCC | Okay but from our understanding this may not impact evaluation methodology and just for clarification? |
| OPPO | We share the same view with CMCC |
| ZTE, Sanechips | The motivation for this two proposals should be clarified. We don’t see the necessity to discuss these proposals with high priority. |
| Ericsson | Support. Ramp-up requires much more careful coordination and timing between circuits and interfaces |
| Spreadtrum | For proposal 3.2.1.5, in our view, both ramp-down time and ramp-down time should be considered together in evaluation. We don’t see the necessity to divide the total transition time into two parts. |
| CEWiT | We share the same view with Spreadtrum |
| DCM | For proposal 3.2.1.3, we support this proposal. For proposal 3.2.1.5, we would like to understand how this impacts the outcome of the SLS simulation result. |
| Nokia | For FL Proposal 3.2.1.5, we see it can make sense in practice, but we don’t see why it needs to be captured with an agreement. What matters is the two ways (ramping up and down) |
| Qualcomm | While we see the point that split might not be 50-50, we are not clear on why there is a need to update. This is similar to the UE power model. |
| Samsung | We agree that in practice, ramp-up takes longer than ramp-down due to hardware re-initialization procedures. However, we note that in TR 38.864, the transition energy E and the total transition time T are defined as aggregate values that already encompass both the ramp-down and ramp-up phases together. Since both phases are inherently captured within the existing definitions of T and E, we do not see the need to separately specify the ramp-down/ramp-up ratio. Introducing an explicit split may add unnecessary complexity to the evaluation framework without clear benefit, as the overall transition time and energy remain the same regardless of the internal split. |
| Huawei, HiSilicon | This should be up to companies report as highly related to implementation. In R18, it is assumed 50%:50% when calculating the final transition time. |
| Apple | For Proposal 3.2.1.3, we see the intention and are OK to do the clarification. We do not see the need to discuss Proposal 3.2.1.5. |
| CATT | We are OK for the FL proposal 3.2.1.3. About the FL Proposal 3.2.1.5, we think it is unnecessary to model the transition time as two parts due to lack of the application case of model. |
| vivo | For Proposal 3.2.1.3, we support it as proponent. We do not see the need to discuss Proposal 3.2.1.5. |
| Huawei, HiSilicon | For proposal 3.2.2.1, we asked this question last meeting but there were misleading comments that FR1 defined in 5G does not include around 7GHz. So it overlaps as now clarified, then we need to discuss whether we can reuse the same set1 for some cases. |

Updates to CAT2 BS power model

Summary of companies’ views

The contributions focus on updating the Cat 2 BS power model to better reflect the hardware advancements anticipated for the 6G timeframe, as the legacy 5G NR parameters are viewed as too conservative.

General Observations on Cat 2 Limitations

Contributors observe that the 5G Cat 2 model represents conservative BS capabilities from the 5G-Advanced era, which includes relatively long transition times (e.g., 10 seconds for deep sleep).

Tejas and Apple note that these prolonged transition times make it difficult for cells with active user traffic to take advantage of deeper sleep states, as user traffic often arrives in intervals (~200ms) much shorter than the transition requirements.

ZTE highlights that while Cat 1 is often too optimistic, the legacy Cat 2 is pessimistic for a 10-year 6G horizon, necessitating a more accurate unified model.

NTT DOCOMO argues that keeping the legacy Cat 2 parameters (based on Rel-15) would provide misleading insights when evaluating 6G energy-efficiency techniques.

Nokia and Qualcomm suggest that while Cat 1 targets future hardware, Cat 2/2.1 is useful for evaluating initial 6G deployments that may reuse existing hardware or early 6G-era components.

Proposals for CAT 2.1 (CAT 2-plus)

The majority of contributions propose the creation of a new category, referred to as Cat 2.1 or Cat 2-plus, to model enhanced 6G hardware.

There is a strong consensus among proponents to significantly reduce transition latencies compared to the legacy model:

- Light Sleep: Proposals generally converge on 100 ms (proposed by Nokia, Huawei, CMCC, ZTE, Samsung, LG, and NTT DOCOMO).

- Deep Sleep: Proposals vary, including 1 second (ZTE), 2 seconds (Huawei, CMCC, Samsung, LG, and NTT DOCOMO), and 5 seconds (Nokia).

- Additional Transition Energy: Contributors propose new values for transition energy, typically scaled alongside the time reductions.

- Relative Power Values: Nokia proposes improving relative power values for each state by 10–20% over legacy Cat 2 to reflect enhanced component efficiency in basebands, PAs, and antennas.

The following table summarizes the parameter updates proposed by various companies for the CAT 2-plus or CAT 2.1 BS power models for BS reference configuration Sets 1-3. These updates are intended to reflect anticipated hardware advances for 6G, specifically targeting more efficient transitions and improved relative power values compared to the legacy 5G Category 2 model defined in TR 38.864.

Table 3: Companies' proposed transition times and transition energies for CAT 2.1 (2-plus) BS model for BS reference configuration Sets 1-3.

| Company | Transition Time (Deep Sleep) | Transition Time (Light Sleep) | Additional Transition Energy (Deep Sleep) | Additional Transition Energy (Light Sleep) | Relative Power Updates |
| --- | --- | --- | --- | --- | --- |
| Nokia | 5 s | 100 ms | 8500 | 170 | 10–20% improvement across all states |
| Huawei | 2 s | 100 ms | — | — | — |
| CMCC | 2 s | 100 ms | 3400 | 170 | — |
| ZTE | 1 s | 100 ms | 1700 | 170 | — |
| Samsung | 2 s | 100 ms | 3400 | 170 | — |
| LG | 2 s | 100 ms | 3400 | 170 | — |
| NTT DOCOMO | 2 s | 100 ms | 3400 | 170 | — |
| Qualcomm | — | 100 ms | — | — | — |
| CATT |  | Reduced | Reduced | — | — |
| Average | 2.3 s | 100 ms | 3967 | 170 |  |
| Median | 2 s | 100 ms | 3400 | 170 |  |

Arguments against updating CAT 2 BS model

Several companies argue against replacing or modifying the original Cat 2 base station model, emphasizing its importance as a realistic and grounded baseline for 6G evaluations.

Apple, Xiaomi, and Qualcomm observe that the legacy Cat 2 model realistically represents the hardware capabilities of base stations currently deployed in the field. AT&T adds that these 5G-era deployments are expected to partly persist into the early phases of 6G implementation, making the preservation of the original model critical for inclusive network planning.

Xiaomi points out that many operators may choose to support 6G through software upgrades to existing 5G hardware. Maintaining the original Cat 2 transition times (e.g., 10 seconds for deep sleep) is necessary to accurately assess the performance of 6G features on this actual, non-upgraded hardware.

Apple and InterDigital argue that Cat 2 serves as a vital conservative or "pessimistic" baseline. They suggest that relying solely on optimized models like Cat 1 or the proposed Cat 2.1 risks distorting the outcomes of energy efficiency studies by assuming hardware efficiencies that may not be achievable in a base station's high-processing environment.

InterDigital notes that the original Cat 2 model allows for an accurate evaluation of energy saving gains (ESG) of new 6G techniques over existing 5G techniques. They argue that using a futuristic model as a baseline can lead to an inaccurate understanding of relative benefits and potentially incorrect design directions for 6G.

Qualcomm and AT&T suggest that instead of replacing Cat 2, the 6G power model should include all three categories (Cat 1, Cat 2, and Cat 2.1). This approach ensures the study is inclusive of the full diversity of products and deployment timelines expected for the 2030-era networks.

FL’s comments and proposals

The polarized situation from RAN1 #123 seemingly remains. There is a fairly strong consensus among companies supporting updates of CAT 2, what those updates should include. On the other hand, there is a strong opposition to making any updates at all. To facilitate further discussions on what models to use for 6GR SI, FL proposes that RAN1 first agrees on what a hypothetical CAT 2.1 (2-plus) BS model would comprise and leave the remaining discussion until later.

FL Proposal 3.2.3.1

IF a BS model CAT 2.1 (2-plus) is introduced, it has the following transition energy characteristics:

| Transition Time (Deep Sleep) | Transition Time (Light Sleep) | Additional Transition Energy (Deep Sleep) | Additional Transition Energy (Light Sleep) |
| --- | --- | --- | --- |
| 2 s | 100 ms | 3400 | 170 |

FL Proposal 3.2.3.1b

IF a BS model CAT 2.1 (2-plus) is introduced, it has the following transition energy characteristics for Set 1-3. FFS: Set 4:

| Transition Time (Deep Sleep) | Transition Time (Light Sleep) | Additional Transition Energy (Deep Sleep) | Additional Transition Energy (Light Sleep) |
| --- | --- | --- | --- |
| 2 s | 100 ms | 3400 | 170 |

Companies’ comments

Companies’ comments on the above proposal are appreciated.

| Company | Comment |
| --- | --- |
| CMCC | Support the proposal. The values should be set according to the product plan on time point of the commercial use for 6GR (e.g. in the year of 2030). Thus, the value should be future-proofed considering the evolution on both HW and SW, and potential increase on network energy consumption for new 7GHz scenario. |
| LG Electronics | Support |
| OPPO | We support this proposal |
| ZTE, Sanechip | Support |
| Ericsson | Transition time (light and deep sleep) – ok Transition energy light sleep – ok Transition energy deep sleep: 3400 is very much. 1700 is better. |
| CEWiT | Share views with Ericsson, 3400 is very much for transition energy of deep sleep |
| DCM | We support this proposal. |
| Xiaomi1 | No strong views for the values of the new Cat. However, from evaluation point of view, 5G cats 1 represent advanced implementation, while cat 2 represents legacy. We think these two categories could be sufficient. |
| Nokia | OK |
| Samsung | Regarding the broader discussion on which BS power models to use for 6GR SI, our preference is to retain both CAT 1 and CAT 2 as the primary evaluation models. Evaluating with both categories provides a comprehensive view of energy saving gains across different deployment timelines. However, if RAN1 concludes that only a single BS power model category should be selected for evaluation purposes, we would support the introduction of CAT 2.1 as a reasonable compromise that balances between the optimistic assumptions of CAT 1 and the conservative legacy CAT 2 parameters. |
| Huawei, HiSilicon | Fine |
| Apple | For evaluating NES gain, we think Cat 1 and Cat 2 are already enough, whereas Cat 1 provides an upper bound and Cat 2 represents the case where reusing 5G BS hardware. Introducing additional categories would make discussion more complicated. |
| CATT | Support |
| Huawei, HiSilicon | We think more discussion may be needed due to newly identified issues, including - Use cases (e.g., UE-initiated on-demand SIB1) - Specification impact - Restrictions in operation (signals and channels) - Performance limitations - Applicability of Set1 ref config to above 7GHz - As one TRP, it should be possible to use the legacy implementation defined in 5G with up to 64 TRxU - Feasibility of Cat 1 for Set 4 ref config with more than 64 TRxU - What is the assumption of the implemented modules that drives more than 64 TRxUs and how same transition operation is maintained in that case |

Selection of BS models to use in 6GR SI

Summary of companies’ views

The contributions for the 6G Radio (6GR) Study Item (SI) reflect a broad range of views on which Base Station (BS) power models to utilize, primarily debating the use of legacy 5G categories (Cat 1 and 2) and a newly proposed, hardware-optimized category (Cat 2.1).

General Observations on Power Model Categories

Cat 1: Contributors observe that Cat 1 represents an aggressive hardware capability with very short transition times (e.g., 6ms light sleep, 50ms deep sleep). While some view it as a necessary target to reflect long-term 6G trends, others argue it is overly optimistic or "UE-like" and may lead to misleading design decisions.

Cat 2: This model reflects hardware assumptions from 2023 with conservative transition times (e.g., 640ms light sleep, 10s deep sleep). It is viewed as a realistic baseline for initial deployments, though many note its long latencies make deeper sleep states impractical under typical user traffic arrivals (~200ms).

Cat 2.1 (2-plus): A significant number of companies propose this category to reflect anticipated hardware evolution by 2030. It bridges the gap by significantly reducing transition latencies of the Cat 2 model (e.g., to 100ms for light sleep and 1–5s for deep sleep).

Proposals on BS Model Selection

There is general consensus that the 5G BS power consumption model framework (TR 38.864) should be the starting point for 6G, with adjustments made for 6G numerology and frequency ranges like 7 GHz.

Ericsson, Huawei, LG, NTT DOCOMO, AT&T, and CEWiT support using both Cat 1 and Cat 2.1 (or modified Cat 2) to assess energy efficiency across both near-term and long-term 6G horizons. Qualcomm further proposes adopting all three (Cat 1, Cat 2, and Cat 2.1) to avoid a deadlock in selection.

Ericsson, Samsung and Huawei argues Cat 1 should be the primary baseline to provide incentives for vendors to achieve higher efficiency whereas Nokia and InterDigital prefer Cat 2 (or Cat 2-plus) as the primary baseline to ensure results are grounded in achievable reality.

ZTE advocates for a unified model (Cat 2.1) that accurately represents 6G system characteristics.

Spreadtrum suggests using only one category—whichever is selected—to simplify evaluation comparison.

Observations on Transition Times and Absolute Power

Tejas and Apple observe that BS energy saving is highly sensitive to transition times; if transition latencies exceed traffic inter-arrival times, sleep benefits are negated.

Nokia emphasize that to meaningfully compare 6G NES gains against 5G, all models must assume a common absolute power level for the deep sleep state.

FL’s comments and proposals

Companies agree that the BS models in TS 38.864 serve as the starting point for 6GR Bs models. Which of those models, including updates of them, should be used is a different matter. In FLs understanding, RAN1 has the following options:

- Agreeing on which (one or more) BS models to use

- Not agreeing on any BS down-selection, in which case all existing models may be used, i.e., both CAT 1 and CAT 2, while omitting CAT 2.1.

In FL’s view, the agreement with the least resistance is the following, provided transition characteristics of BS model CAT 2.1 has been agreed:

FL Proposal 3.2.4.1

BS model CAT 2.1 is introduced. All BS models CAT 1, CAT 2 and CAT 2.1 may be used for evaluations in 6GR SI.

Companies’ comments

Companies’ comments on the above proposal are appreciated.

| Company | Comment |
| --- | --- |
| CMCC | Support the proposal, but further discuss may needed on whether Set 4 can belong to Cat 2 only, Cat 2.1 only or Cat 2+Cat 2.1. From our understanding, it may only belong to Cat 2.1. |
| LG Electronics | Our preference is to set Cat 1 and Cat 2.1 as baseline. However, if company views cannot be merged, we can accept FL Proposal 3.2.4.1. |
| OPPO | Support this proposal |
| ZTE, Sanechips | CAT 2.1 represents realistic 6G deployments which should be the baseline. For CAT1, 6G may evolve toward to this. But for CAT2, it represents 5G. |
| Ericsson | Among the major Infra vendors, E///, Hua and Samsung all support CAT 1, hence it should be supported. Ok to evaluate with more than one model. CAT 2.1 should replace CAT 2. |
| Spreadtrum | Considering the workload, we think it would be better to make downselection. Moreover, if three models are used and the evaluation results vary significantly, how to proceed with the discussion will also be a issue. |
| CEWiT | Support this proposal in general, with preference to set Cat 1 and Cat 2.1 as baseline. |
| DCM | We are generally fine with the proposal, but may be prioritization for the use of the evaluation can be determined such that at least Cat 2.1 and Cat 1 should be prioritize for evaluation for 6GR since network energy saving techniques should be valid for the 6GR devices. Companies can confirm the NES for NR devices, but it should be optional considering that in the future, NR devices will be replaced. |
| Xiaomi | OK with comments. We prefer to keep a smaller number of categories, the two categories introduced in NR could be sufficient. However, considering the discussion in the last meeting for this issue and for progress, we can also accept the introduction of all three categories. |
| Nokia | Support |
| Qualcomm | Support proposal 3.2.4.1 |
| Samsung | See comment in section 3.2.3.3 |
| Huawei, HiSilicon | Ok- can be combined with P3.2.2.1 for efficient discussion. |
| InterDigital | Share similar view with Xiaomi to consider minimal BS CATs for evaluations. We also do not think CAT 2.1 to replace CAT 2, since CAT 2 provides a good baseline for legacy BS. |
| Apple | Similar comment as for FL Proposal 3.2.2.1. |
| CATT | OK with the proposal. |
| Huawei, HiSilicon | We think more discussion may be needed due to newly identified issues, including - Use cases (e.g., UE-initiated on-demand SIB1) - Specification impact - Restrictions in operation (signals and channels) - Performance limitations - Applicability of Set1 ref config to above 7GHz - As one TRP, it should be possible to use the legacy implementation defined in 5G with up to 64 TRxU - Feasibility of Cat 1 for Set 4 ref config with more than 64 TRxU - What is the assumption of the implemented modules that drives more than 64 TRxUs and how same transition operation is maintained in that case |

Low-power radio / Active EE mode

Summary of companies’ views

The contributions present a variety of perspectives on the implementation of a Low-Power Radio (LPR) or Energy Efficient (EE) mode for BSs, ranging from enthusiastic support for new scaling rules to skepticism regarding hardware feasibility and performance trade-offs.

Observations regarding BS Low-Power Radio / EE Mode

The BS LP/EE mode is designed to support essential but lightweight functions—such as transmitting synchronization signals or monitoring for Uplink Wake-up Signals (UL-WUS) and PRACH—while allowing most baseband components and the main radio to remain in a deep sleep state. Some alternative implementation assumptions for the LPR are presented in Figure 2, courtesy of Apple.

Figure 2: Assumptions for low power mode operation (Apple).

Proponents such as Huawei, Spreadtrum, CATT, and CEWiT argue that a Base Station (BS) low-power radio (LPR) or active energy-efficient (EE) mode is a vital enabler for Network Energy Saving (NES), particularly under low-traffic conditions. This mode allows the BS to handle essential but lightweight functions—such as monitoring Uplink Wake-up Signals (UL-WUS) or transmitting synchronization sequences—while keeping the primary baseband and most transceiver units (TRxUs) in a deep sleep state. Huawei observes that monitoring UL-WUS in LP-mode can provide a 16% NES gain compared to waking the main radio, as it permits the BS to reserve only a small fraction (e.g., 25%) of hardware for active response while the remaining 75% stays powered down. Supporters like InterDigital, Qualcomm, and Ofinno suggest this architecture effectively balances network energy savings with user Quality of Service (QoS) by maintaining synchronization and responsiveness without the massive power overhead of full main-radio operation.

Conversely, companies like ZTE, Ericsson, and NTT DOCOMO raise significant technical and economic objections, arguing that the introduction of such a mode is not well-motivated. ZTE and Ericsson note that implementing LPR via separate hardware would increase CAPEX, size, and weight, whereas using shared modules limits energy savings because critical components like clock synthesizers and oscillators must remain active. Furthermore, reducing active TRxUs or bandwidth to save power degrades cell coverage, increases false detection rates, and reduces synchronization accuracy. Ericsson also highlights that the wake-up latency of the main radio—which could take up to a second for Category 2 base stations—introduces unacceptable access delays and necessitates complex random-access procedures with multiple RACH configurations. NTT DOCOMO observes that the number of downlink transmissions is the primary driver of energy consumption regardless of the radio type, and that LPR schemes requiring a specific modular hardware architecture fall largely outside the standardizing scope of 3GPP.

Proposals for BS LP/EE Mode

Several companies (Huawei, CATT, Spreadtrum, LG, OPPO, InterDigital, Ofinno) support an optional scaling rule for modeling the BS in LP/EE mode where static power consumption, Pstatic, is scaled based on the fraction of active TRxUs (sLP):

Pstatic DL/UL=sLP∙P3+1-sLP∙Pi

In this formula, P3 is the micro-sleep power and Pi is the power of a deeper sleep state (light or deep).

CEWiT proposes a simpler scaling rule for the active-state power in LP mode: Pstatic=K∙P3, where K<1.

For transitions to and from LP mode, contributors propose applying a transition energy value of 1-sLP∙E, where E is the transition energy of the corresponding sleep state.

New Power States

Qualcomm proposes adding two explicit states: "Active DL using EE Modem Mode" and "Active UL using EE Modem Mode" (assigned a new power value P6 for simple wake-up signals).

vivo proposes a new state specifically for UL WUS monitoring, with a power value between micro-sleep and active UL.

Functional Limitations

CATT proposes that LP mode functionalities be restricted to predetermined deterministic sequences and low-complexity algorithms.

InterDigital and Ofinno advocate for the study of low-power receivers and transmitters on the network side to enable coordinated simultaneous wake-ups of both the network and device.

FL’s comments and proposals

LPR is a topic that polarizes companies. In FL’s view, the agreement that was discussed during RAN1 #123 had strong support among LPR proponents. This is also reflected in the coherent proposals provided by supporting companies. Hence, discussions may continue using that as a starting point:

FL Proposal 3.2.5.1

An optional scaling rule for modeling the BS in a low power or energy efficient mode scales static power consumption, Pstatic, based on the fraction of active TRxUs (sLP),

Pstatic DL/UL=sLP∙P3+1-sLP∙Pi,

where sLP=12,14, and i=1,2. Transitions to and from the LP/EE mode, apply a transition energy value of

1-sLP∙E

where E is the transition energy of the corresponding sleep state.

FL Proposal 3.2.5.2

Further study a low power mode with respect to

- Use cases (e.g., UE-initiated on-demand SIB1)

- Compatible BS power model categories

- Specification impact

- Restrictions in operation (signals and channels)

- Performance limitations

- Other aspects are not precluded

Companies’ comments

Companies’ comments on the above proposal are appreciated.

| Company | Comment |
| --- | --- |
| LG Electronics | Support |
| OPPO | Support this proposal |
| ZTE, Sanechips | - No any impacts on initial access For the DL signal transmission, especially PSS/SSS, we do not think this LPR is needed, since it would significantly impact the coverage, which should be discussed under initial access agenda. If the coverage of PSS/SSS is degraded, the cell coverage range is smaller, how the UE camp on this cell and discover this cell is unclear. If the coverage of PSS/SSS is not degraded, the feasibility and necessity should be discussed firstly. For the UL sequence receiving, if there is any new UL signal, e.g., UL WUS, is designed, and the BS could process it in a relaxing mode, it could be possible to further reduce the power. If the BS is going to receive the existing UL signal, e.g., PRACH, there is no any motivation to support this LPR, since still the coverage for PRACH receiving would be degraded. Therefore, in our understanding, if the LPR has any impacts on the initial access related channels, it should be left to other agenda discussion and further study the feasibility and necessity. We can not just define a scaling model and ignore its negative impacts, e.g., coverage. - Application scenario should be clear For any transmission with coding scheme, including PBCH, PDSCH, PDCCH, we do not think scaling on static part make sense. The reason is the coding scheme would cause the baseband processing in AAU/RRU keep active, it is not possible the static part could be scaled down. If the LPR is used for receiving new designed UL WUS with low power, by sacrificing more UE’s transmission, it could be more make sense. - No necessity to introduce LPR. All the description is just a scaling rule, not related to LPR. There is no need to imply introducing a new mode for gNB to implement. - the scaling factor is determined by TRxUs should be clarified. If remember correctly, 1/4 TRxUs is assumed for sLP=12,14. Additionally, 1/4 TRxUs would correspond to 1/4 dynamic power, it would be impossible to have 1/4 static power scaling, since anyway there would be some components, e.g., the processing chip, is shared. Therefore, if 1/4 TRxUs is assumed, scaling factor for static part should be 1/2 at least and if 1/2 TRxUs is assumed, scaling factor for static part should be 3/4 at least. |
| Ericsson | The scaling is far too aggressive: - With BS ref configuration set 4 and operation with 64 antennas  P_static = 0.25*85+0.75*1 = 22 according to the proposal. - With BS ref configuration set 1 and operation with 64 antennas  P_static = 55 It is unreasonable to have much lower P_static with BS reference configuration set 4 than BS reference configuration set 1. The proposal needs to include how to guarantee coverage is the same, particularly in uplink. |
| CEWiT | Support the proposal |
| DCM | We do not support this proposal. As we discussed in the last meeting, this technique is very much into one of NW implementations. We should first focus on achieving NES techniques with the typical NW implementation. However, if this technique does not imply any technique including LPR, we are fine to include just an optional equation (not specify that the equation is regarding LPR). |
| Nokia | First, we want to understand which LP model in Figure-2 relates to this modeling. Practically, the LP model in the left of Figure-2 can be quite different from the LP mode in the right of Figure-2. We don’t think the two LP models in Figure-2 can be modelled with the same. We don’t think the LP model module separated from MR is practically feasible. |
| Qualcomm | In our view, antenna adaptation/TxRU is one method of adding an energy-efficient mode to the base-station power model. Another method to put portions of the base station into sleep while keeping others active to save power. How this would be implemented is up to the basestation implementation, as in the EE processing mode for the UE. To support this other implementation, the transition energy and time need to be decoupled from the number of active TxRUs. FL Proposal 3.2.5.1-Qualcomm An optional scaling rule for modeling the BS in a low power or energy efficient mode scales static power consumption, Pstatic, based on the fraction of active TRxUs (sLP), Pstatic DL/UL=sLP∙P3+1-sLP∙Pi, where sLP=12, 14, and i= 1, 2. Transitions to and from the LP/EE mode, including when sLP={1}, apply a transition energy value of ET<E and transition time T where (1-sLP)∙E where E is the transition energy of the corresponding sleep state. FFS what signals can be transmitted/received in low-power/energy-efficient mode. |
| Samsung | We see the LP/EE mode concept as largely implementation-dependent, and it remains unclear what additional spec support from RAN1 is needed to enable this mode. Before deciding whether to introduce a dedicated power model, we believe the following should be clarified first: - What specific channels and/or signals (e.g., SSB, PRACH, UL-WUS) are expected to be transmitted or monitored in LP/EE mode, and with what functionality limitations; - What normative specification changes, if any, are required to support this mode beyond implementation-specific optimizations. Without such clarification, we are not convinced that formalizing the LP/EE mode scaling rule in the evaluation framework provides meaningful added value at this stage. |
| Huawei, HiSilicon | Support. Thanks for the questions/comments from below companies. @ZTE, Ericsson, DCM We need to discuss Power model and Techniques/limitations separately. We do not specify power model, neither for Cat 1 and Cat 2. It is just a model based on implementation, and used for evaluation. We do not determine the power model according to application scenarios, neither for Cat 1 and Cat 2. However the impact due to certain implementation certainly should be taken into account, which we agree. We then clarify the potential spec impact as below. - For initial access, currently we see one use case of on-demand SIB1 with UE triggered UL WUS. However, UL WUS can also work without LP mode. We show benefits of larger NES benefits with LP-mode. In this sense, the spec impact of LP-mode used for on-demand SIB1 can be the same/similar as MR with UL WUS. - The coverage impact should also be considered. Therefore, currently we assume simple/limited signal can be used for LP mode. Since sequence detection is much simpler than payload detection and decoding, the coverage impact could be mitigated. Furthermore, we think the techniques for MR coverage enhancements should be reused as much as possible, for example, if repetition is assumed for relevant signal (e.g. PRACH) in MR, the difference would be repetition number for LP mode. @ZTE, Ok to not define a mode. Ok to discuss other values for 1/4 TRX used case. See modifications below. @Ericsson This is mostly because of implementation change. Even without LP mode, - 85 for BS ref configuration set 4 and operation with 64 antennas - 55 for BS ref configuration set 1 and operation with 64 antennas is assumed. But ok to discuss other values. @DCM The potential spec impact is clarified above, which in our view is quite margin compared to MR without LP-mode. One Note is explicitly added below. @Nokia Our implementation is the left figure in Figure 2. Although we believe we should decouple the BS power model from techniques, from our side, we are fine with the following modifications An optional scaling rule for modeling the BS in a with low power or energy efficient mode scales static power consumption, Pstatic, based on the fraction of active TRxUs (sLP), Pstatic DL/UL=sLP∙P3+1-sLP∙Pi, Where sLP=14, and i= 1, 2. Transitions to and from the LP/EE mode, apply a transition energy value of (1-sLP)∙E where E is the transition energy of the corresponding sleep state. Other values for sLP and E are up to company report with justification, e.g. sLP=12 with 25% active TRxUs. Note: Strive for a common design framework for operation with and without the above scaling method, including coverage impact. FFS what signals can be transmitted/received in low-power/energy-efficient case. |
| Apple | Generally OK with the model itself. However as mentioned by Ericsson, we need to understand how to guarantee the coverage of the signals transmitted/receive by this LP mode. |
| vivo | As some companies indicate, what kinds of signals and channels can be transmitted/received in this mode should be incorporated in this mode. Regarding the impact, same coverage should be maintained. |
| Huawei, HiSilicon | @Ericsson - Regarding the comparison between With BS ref configuration set 4 and operation with 64 antennas  P_static = 0.25*85+0.75*1 = 22 according to the proposal. - With BS ref configuration set 1 and operation with 64 antennas  P_static = 55 We want to further note that the normalized deep sleep power value of 1 is different between two sets, so these two cases are not comparable. @ZTE As per offline discussion with ZTE, we can take a compromised S_LP for 25% active TRxU case. However, given the newly added proposal from moderator and its such uncertainty, we put bracket with the original value. @Moderator As we commented earlier, spec impact/limitations/etc should not be a pre-requisite for a BS power modeling (we did not discuss Cat1 and Cat2 as such). For discussion purpose, we also need some details of the model/scaling for discussing the potential limitation or restrictions. Based on the above, we suggest the following: An optional scaling rule for modeling the BS in a with low power or energy efficient mode scales static power consumption, Pstatic, based on the fraction of active TRxUs (sLP), Pstatic DL/UL=sLP∙P3+1-sLP∙Pi, Where sLP=[14] assuming 14 of the TRxUs are active, and i= 1, 2. Transitions to and from the LP/EE mode, apply a transition energy value of (1-sLP)∙E where E is the transition energy of the corresponding sleep state. Other values for sLP and corresponding E are up to company report with justification, e.g. sLP=12 with 25% active TRxUs. Further study the above with respect to - Use cases (e.g., UE-initiated on-demand SIB1) - Compatible BS power model categories - Specification impact - Restrictions in operation (signals and channels) - Performance limitations - Other aspects are not precluded |

BS power model for SBFD

Summary of companies’ views

The contributions indicate that Sub-band Full Duplex (SBFD) is recognized as a key 6G technology for enhancing coverage and reducing latency, though companies emphasize that it requires specific power modeling to evaluate its impact on Network Energy Saving (NES) accurately.

Accounting Rules for Simultaneous DL/UL

A central point of discussion is how to account for power during symbols where transmission and reception occur simultaneously:

Jio and vivo propose that for energy accounting during simultaneous DL and UL symbols in SBFD, the Uplink (UL) reception power may be neglected, following the established handling for Frequency Division Duplex (FDD) systems.

ZTE offers an alternative (Alt 1) where active SBFD operation is simply approximated as active Downlink (DL) power consumption, given that DL power typically dwarfs UL reception power.

ZTE (Alt 2) and Qualcomm suggest a more granular approach where total SBFD power is the sum of static power, active dynamic DL power, and active dynamic UL power. Qualcomm specifically proposes a formula where dynamic components are weighted by the fraction of active transceiver units (sa) and the occupied bandwidth (sf) assigned to the DL and UL sub-bands.

Proposed Reference Configurations

ZTE suggests that reference configurations for SBFD can reuse Set 1 (100 MHz TDD) from the legacy 5G BS model.

Vodafone, Telecom Italia, and Deutsche Telekom propose defining an entirely new reference configuration, "Set 5 SBFD," to explicitly study semi-static SBFD operation with a 100 MHz system bandwidth and 30 kHz subcarrier spacing.

Qualcomm advocates leveraging the Rel-18 advanced duplexing study (TR 38.858) as the starting point for modeling antenna arrays and deployment scenarios for 6G SBFD energy efficiency.

Hardware and Implementation Observations

Vodafone observes that SBFD requires modified RF chain components, such as improved linearization and the addition of Digital Pre-Distortion (DPD) units per antenna branch to mitigate self-interference. These enhancements, along with additional feedback chains, are expected to change the base station's power profile compared to legacy TDD systems.

Vodafone and its partners argue that existing power consumption formulas must be updated to include accurate Power Amplifier (PA) components specifically for SBFD implementations. They further contend that no NES techniques should be defined for SBFD base stations until these relevant power model updates are agreed upon to ensure operators understand the OPEX implications.

Jio includes SBFD handling as a required item in its proposed mandatory reporting template, requiring companies to explicitly state if UL RX power is being neglected during simultaneous symbols to ensure cross-company comparability.

FL’s comments and proposals

In FL’s understanding, no SBFD networks have been deployed, and no one knows for certain the power costs to it. Still, there is a value in specifying a model for it for future work. That model should consider two things:

- The reduced number of antennas used for both DL Tx and UL Rx, and

- The increased Rx complexity and linearity SBFD inevitably implies from increased interference mitigation requirements.

Currently, companies only address the first bullet. FL’s attempt to proposal is then:

FL Proposal 3.2.6.1

SBFD is modeled according to

PSBFD=Pstatic+κPdynamicULsa=12,sfUL+PdynamicDLsa=12,sfDL

where κ>1 , exact value FFS, is a penalizing scaling factor and sfUL and sfDL are the fractions of UL and DL bandwidths, respectively.

Companies’ comments

Companies’ comments on the above proposal are appreciated.

| Company | Comment |
| --- | --- |
| LG Electronics | OK with the direction. However, why is the value of κ larger than 1? |
| OPPO | We are fine with this direction. But we wonder if the Sa value should also be FFS? |
| ZTE, Sanechips | Generally, the BS power consumption caused by the DL transmission is higher than that of UL reception. And, the difference of the DL power consumption and the UL power consumption is reflected in the dynamic part of DL power consumption and UL power consumption. So, the factor κ may not be necessary. In addition, the proportion of antenna reduction can be FFS. The proposal can be modified as: FL Proposal 3.2.6.1 SBFD is modeled according to PSBFD=Pstatic+κPdynamicULsa=12,sfUL+PdynamicDLsa=12,sfDL PSBFD=Pstatic+PdynamicULsa,sfUL+PdynamicDLsa,sfDL where κ>1 , exact value FFS, is a penalizing scaling factor Sa is the fractions of the TRxU and sfUL and sfDL are the fractions of UL and DL bandwidths, respectively. |
| Ericsson | We are fine with the proposal, but it is nothing that we should prioritize. |
| Spreadtrum | In our view, DL power consumption is more larger than UL, so we prefer that active SBFD operation is simply approximated as active DL. |
| DCM | If this study is needed to be pursued, we should discuss whether/how the model is formulated in a meaningful manner. For the prioritization, we share the same view as Ericsson. |
| Nokia | We prefer to delay the discussions on BS power model for SBFD. As commented by operators, SBFD requires a complete redesign of the BS RF including additional RF and digital components/circuitry to improve the receiver linearity and mitigate the self-interference. Roughly speaking, we estimate that a SBFD BS would consume significantly more power than the sum of the DL+UL components of a regular TDD base station. Therefore, as starting point, the existing power models do not seem applicable for SBFD BSs, so we do not support the current proposal. Additional aspects need to be considered due to the significant RF redesign compared to traditional TDD BSs, e.g. as studied in TR 38.858, Section 9 and 10. |
| Qualcomm | We don't think the scaling factor (k) is needed. 1) depending on Tx power level and isolation level, a passive solution is possible that depends on panel separation. 2) even for higher Tx power, the extra power for digital interference cancellation (if needed) is small compared to overall BB processing. |
| Samsung | We understand the motivation to model SBFD power consumption for future evaluations. However, we note that UL reception power is typically much smaller compared to DL transmission power and can be considered negligible in practical scenarios, as also observed by several other companies. Therefore, approximating SBFD power consumption as active DL power (i.e., neglecting the UL component) would be a simpler and sufficiently accurate approach without the need to introduce an additional scaling factor κ\kappa |
| Huawei, HiSilicon | We are open to study. At least modification is needed for the following aspects. - Our understanding is that the processing for UL and DL would be different among TRxU chains, impacting the static power as well. A factor ‘A’ (range within [0, 1]) can be introduced for that purpose. - The differentiation of Sa for DL and UL can be flexible – e.g. a factor ‘B’ for UL and (‘A-B’) for DL as the ratio of TRxU used for UL/DL respectively, instead of fixed 1/2. In legacy, the UL part is not scaled with BW. If the reason is to consider UL BW impact, we think such need should also apply to the general mode/non-SBFD since larger BW/impact is expected compared to the BW used for SBFD. |
| Apple | We are open to discuss about the model. However, one fundamental question is that is there any SBFD specific NES schemes whose performance that may not be covered by legacy models? Or do we need to evaluate the relevant schemes under the SBFD model assumption? For the equation, we are wondering whether this is considering that sa part of the antennas are used for UL while 1-sa part of the antennas are used for DL?If so, this needs to be reflected in the model. Also for the SBFD operation, the interference suppression at NW side may increase the power consumption, we are wondering how we can reflect such factor in the model. |
| vivo | We prefer a simple modeling for SBFD symbols, e.g., neglecting UL reception part. |

Remaining Evaluation Assumptions for UE Power Consumption

  - Power Scaling Rules

For parameters where companies propose different numerical values reflecting different implementation approaches, Avg(All) values are generally recommended for discussion and convergence. Since 3GPP does not mandate specific implementations, the power model should be implementation-agnostic. Avg(All) captures the diversity of valid industry approaches across different architectures, process nodes, and design choices. Please check Initial parameter check draft folder for more specific averaged values from all companies’ inputs.

    - Bandwidth Scaling

      - Detailed Company Contributions (Collapsed; Please Expand to View)

PDCCH+PDSCH Slot Scaling

| Source | TDoc ID | Content Summary |
| --- | --- | --- |
| Nokia | R1-2600030 | Proposes omitting ΓB=5% due to limited delta to 20% case. Suggests two adaptation delay values only if justified. For T: 20%→0.47, 100%→{1, 0.76}, 200%→{2.02, 1.31}, 400%→{4.88, 2.82, 1.79}. T configurable in range {≥1ms, ≤5ms}. |
| Spreadtrum/UNISOC | R1-2600110 | Remove 5% and 400% in UE power model. Remove ΓTput=1/2 for 100MHz in PDCCH+PDSCH state. |
| Huawei/HiSilicon | R1-2600142 | T vs Tmin have different scaling factors. S_MaxBW=0/0.15/0.52 for MaxBW∈{100,200,400}MHz. For T: 5%→X01, 20%→0.23, 100%→{1, 0.79}, 200%→{2.85, 2}, 400%→{7.45, 4.9, 3.2}. For Tmin: add S_MaxBW to values. Static power doesn't change with ΓTput; dynamic power does. |
| OPPO | R1-2600197 | Linear scaling below 100MHz, linear with higher ratio above 100MHz. For T: 5%→0.28, 20%→0.4, 100%→1, 200%→{1.7, 1.6}, 400%→{3.6, 3.4, 3.2}. For Tmin: add S_MaxBW. S_MaxBW=0/0.3/0.5 for MaxBW. |
| Xiaomi | R1-2600427 | For T: 5%→0.35, 20%→0.5, 100%→1, 200%→1.7, 400%→3.4 (ΓTput=1 only). For Tmin: add SMaxBW. References RedCap model for 20% case (0.4-0.5 range). Above 100MHz uses CA scaling (2CC=1.7x, 4CC=3.4x). Static power >50% for Tmin. Remove ΓTput=1/2 and 1/4. |
| ZTE/Sanechips | R1-2600456 | For T: 5%→0.15, 20%→0.25, 100%→1, 200%→{1.6, 1.12}, 400%→{3.55, 2.485, 1.74}. For Tmin: add SMaxBW. Uses formula P(ΓB)=Pmicro-sleep+(Pt-Pmicro-sleep)*ΓB. For different ΓTput, applies 0.7 scaling factor (Y=0.7*X, Z=0.7*Y). |
| vivo | R1-2600502 | For T: 5%,20%→0.56, 100%→1, 200%→{1.7, 1.02}, 400%→{3.4, 1.7, 1.02}. For Tmin: add delta+SMaxBW. Based on NR100MHz eMBB to NR20MHz RedCap (0.56 for PDCCH+PDSCH). CA scaling for >100MHz (1.7, 3.4). T=5-10ms, Tmin=1-2ms. For ΓTput reduction: 0.6 (200→100MHz), 0.5 (400→200MHz), 0.3 (400→100MHz) for max schedulable BW reduction; 0.7 for rank reduction. |
| Samsung | R1-2600754 | For T and Tmin (identical): 5%→0.23, 20%→0.31, 100%→1, 200%→{2.71, 1}, 400%→{7.05, 2.71, 1}. Power scales ~quadratically for larger BW reflecting bandwidth-dependent and voltage-dependent consumption across multiple operating points. S_MaxBW=0. |
| Apple | R1-2600826 | Single column (Tmin only, based on DCI BWP switching Type 2). 5%→0.4, 20%→0.5, 100%→{1, 0.8}, 200%→{1.7, 1.4}, 400%→{3, 2.5, 2.2} assuming 2x200MHz implementation. S_MaxBW=0. T and Tmin differentiation not necessary if Tmin sufficiently large. |
| Ericsson | R1-2600960 | Only Tmin=1ms considered. 5%→0.2, 20%→0.4, 100%→1, 200%→{1.8, 1.7}, 400%→{3.4, 3.2, 3}. Impact of ΓTput scaling is small (RF power not scaled). Long adaptation delay (>1ms) should not be considered—focus on fast BW adaptation due to 5G BWP switching delay drawback. |
| Lenovo | R1-2601106 | Extend NR power model to 5-20MHz regime. X01=0.29, X02=0.4 as starting points. Uses NR model: 0.4+0.6*(X-20)/80. At 5MHz gives 0.29. Static power nearly BW-independent for small BW; lower bound is micro-sleep. |
| Google | R1-2601205 | Support Option 1 (S_MaxBW>0) as baseline to realistically model static power overhead of wideband-capable UEs in narrow bandwidths. S_MaxBW=0 unrealistic—assumes perfect gear-shifting (400MHz device = 20MHz device at 20MHz operation). Non-zero penalty needed to avoid overestimating EE. |
| MediaTek | R1-2601240 | Power scaling follows (f/f₀)×(V/V₀)² relationship. For T=5ms: 5%→0.35, 20%→0.4, 100%→{1, 0.95}, 200%→{1.7, 1.6}, 400%→same as Tmin. For Tmin=2ms: add S_MaxBW, 200%→{2, 1.9}, 400%→{5.6, 5.3, 5.0}. S_MaxBW=0.05 (200MHz), 0.2 (400MHz). Example: 200MHz with T→1.7=(1.2)×(1.2)²; with Tmin→2.0=(1.4)×(1.2)². 400MHz→5.6=(2.2)×(1.6)². |
| Qualcomm | R1-2601271 | For ΓB>20MHz: T and Tmin identical. For ΓB≤20MHz: can differ (X01=0.6*X11, X02=0.6*X12). For T: 5%→0.6*0.32, 20%→0.6*0.40, 100%→1, 200%→{2.3, 1.4}, 400%→{5.2, 3.0, 2.2}. For Tmin: 5%→0.32+SMaxBW, 20%→0.40+SMaxBW. Scaling equation for X≥100MHz: X^1.4/100^1.4×0.5 + X^0.9/100^0.9×0.5 (0.9 for RF sub-linear, 1.4 for BB, α=0.5 weighting). For ΓB≤100MHz, reuse NR 38.840 linear scaling. SMaxBW=0. |

PDCCH-only Slot Scaling

| Source | TDoc ID | Content Summary |
| --- | --- | --- |
| Nokia | R1-2600030 | For T (Ncntrl=2): 20%→0.29, 100%→{0.36, 0.33}, 200%→{0.48, 0.38}, 400%→{0.8, 0.51, 0.41}. Values relative to Pref=300 units. |
| Spreadtrum/UNISOC | R1-2600110 | Remove ΓTput in PDCCH-only state. |
| Huawei/HiSilicon | R1-2600142 | PDCCH-only less sensitive to BW change than PDCCH+PDSCH. For T: 5%→X01, 20%→0.455, 100%→{1, 1}, 200%→{2, 2}, 400%→{4.225, 4.225, 4.225}. For Tmin: add S_MaxBW. S_MaxBW=0/0.45/1.55 for MaxBW∈{100,200,400}MHz. ΓTput has no impact (same values for 1 and 1/2). |
| OPPO | R1-2600197 | For T: 5%→0.35, 20%→0.5, 100%→1, 200%→{1.5, 1.4}, 400%→{3, 2.8, 2.4}. For Tmin: add S_MaxBW. |
| Xiaomi | R1-2600427 | For T: 5%→0.5, 20%→0.6, 100%→1, 200%→1.7, 400%→3.4 (ΓTput=1 only). For Tmin: add SMaxBW. Same CA-based scaling as PDCCH+PDSCH for >100MHz. References RedCap: 0.5 for 20% case. Remove ΓTput entirely. |
| ZTE/Sanechips | R1-2600456 | For T: 5%→0.45, 20%→0.5, 100%→1, 200%→{1.45, 1.015}, 400%→{2.65, 1.855, 1.3}. For Tmin: add SMaxBW. Uses 0.7 scaling for different ΓTput. |
| vivo | R1-2600502 | For T: 5%,20%→0.7, 100%→1, 200%→{1.7, 1.02}, 400%→{3.4, 1.7, 1.02}. For Tmin: add delta+SMaxBW. From NR100MHz eMBB to NR20MHz RedCap: scaling=0.7 for PDCCH-only (50*1.4=70 from 100). |
| Samsung | R1-2600754 | UE operates at single lowest clock/voltage point, resulting in linear power scaling with BW. For T and Tmin (identical): 5%→0.05, 20%→0.2, 100%→1, 200%→{2, 1}, 400%→{4, 2, 1}. Linear scaling reflects single operating point. S_MaxBW=0. |
| Apple | R1-2600826 | Single column (Tmin). 5%,20%→0.85, 100%→1, 200%→{1.2, 1.2}, 400%→{1.6, 1.6, 1.6} assuming 2x200MHz. PDCCH-only more RF-dominated than BB vs PDCCH+PDSCH, leading to larger scaling factor. No ΓTput impact expected or needed. S_MaxBW=0. |
| Ericsson | R1-2600960 | Only Tmin=1ms. Assumes PDCCH candidates/BDs linearly scaled with BW. 5%→0.2, 20%→0.4, 100%→1, 200%→1.8, 400%→3.4. Note: scaling can differ if CCE/BD limits considered (less BW-dependent). ΓTput not modeled. |
| Lenovo | R1-2601106 | Define CORESET/search space configuration for BW scaling. Reuse NR model (TR 38.840) for PDCCH candidate reduction: P(α)=max(Micro-sleep, α*Pt+(1-α)*0.7Pt). BW scaling depends on CORESET configuration: scenario 1 (scale #candidates with BW) vs scenario 2 (increase CORESET duration for similar #candidates at 5MHz). Use linear model S=0.4+0.6*(X-20)/80 or modified version. |
| Qualcomm | R1-2601271 | For T: 5%→0.6*0.32, 20%→0.6*0.60, 100%→1, 200%→{1.9, 1.4}, 400%→{3.7, 2.6, 2.1}. For Tmin: 5%→0.32+SMaxBW, 20%→0.60+SMaxBW. PDCCH scaling from 38.840 revisited for ΓB=20% (NR would give 40 < microsleep 45). Less aggressive scaling (0.6) fixes this. SMaxBW=0. |

Other DL Processing / Micro Sleep Scaling

| Source | TDoc ID | Content Summary |
| --- | --- | --- |
| Nokia | R1-2600030 | For T (Ncntrl=0): 20%→0.15, 100%→{0.19, 0.16}, 200%→{0.26, 0.19}, 400%→{0.48, 0.26, 0.19}. |
| Spreadtrum/UNISOC | R1-2600110 | Simple scaling rule for micro-sleep. Reuse legacy 45 power units for 100MHz, redefine X units for 200MHz. |
| OPPO | R1-2600197 | Other DL signal/channel processing: For T: 5%→0.35, 20%→0.5, 100%→1, 200%→1.5, 400%→3. For Tmin: add S_MaxBW. |
| CATT | R1-2600294 | Scaling rule for micro sleep should be considered. Since micro-sleep requires UE ready for any non-sleep state in next slot, it's related to BW adaptation scaling rules. For 200MHz vs 100MHz, less components can be turned off (P#1>P#2). Exact values need discussion. |
| Xiaomi | R1-2600427 | Other DL: same as PDCCH-only (5%→0.5, 20%→0.6, 100%→1, 200%→1.7, 400%→3.4). Micro-sleep: scale by 0.4 (≤20MHz), 1 (20-160MHz), 1.7 (≥160MHz). Rationale: less components can be turned off for 200MHz than 100MHz during micro-sleep. Match NR RedCap model for ≤20MHz. |
| ZTE/Sanechips | R1-2600456 | Reuse scaling factor for PDCCH+PDSCH or PDCCH-only for other DL signal/channel processing. Since power similar to PDCCH-only or PDCCH+PDSCH states, reuse for simplicity. |
| vivo | R1-2600502 | Other DL: For T: 5%,20%→0.7, 100%→1, 200%→1.7, 400%→3.4. For Tmin: add delta+SMaxBW. Micro-sleep: same scaling as PDCCH-only for simplicity. T=5-10ms, Tmin=1-2ms. |
| Samsung | R1-2600754 | Micro-sleep: follow PDCCH-only scaling table without additional factors. Since zero transition time, UE can't adjust clock/voltage. BW dependency inherited from PDCCH HW configuration. Additional scaling factors (Z3, Z4) add complexity without significant accuracy improvement. Simple, tractable model. |
| Apple | R1-2600826 | Other DL (Rx open full slot, should >PDCCH-only): Tmin only. 5%,20%→0.85, 100%→1, 200%→1.2, 400%→1.8 assuming 2x200MHz. |
| Ericsson | R1-2600960 | Other DL signals (e.g., sync signal): power depends on design; BW scaling may not apply for fixed BW transmissions. |
| Google | R1-2601205 | Adopt scaling rule for Micro-sleep as function of both gap duration and target active BW, reflecting standby power for wideband readiness. Current model assumes fixed 45. Short gap→light micro-sleep (can't turn off LO/flush pipeline). Long gap→deeper micro-sleep. 400MHz readiness requires more standby current than 20MHz. |
| MediaTek | R1-2601240 | Other signal processing: For T=5ms: 5%→0.35, 20%→0.4, 100%→1, 200%→1.7, 400%→same as Tmin. For Tmin=2ms: add S_MaxBW, 200%→2, 400%→5.6. S_MaxBW=0.05 (200MHz), 0.2 (400MHz). |
| Qualcomm | R1-2601271 | Other DL: reuse PDCCH-only scaling for ΓTput=1. Micro-sleep: For T: 5%,20%→0.6, 100%→1, 200%→{1.7, 1.4}, 400%→{3.0, 2.4, 2.0}. For Tmin: 5%,20%→1, 100%→1. Zero transition time→insufficient to change clock/voltage. Scale both RF and BB components with sub-linear exponent for >100MHz. |

Adaptation Delay

| Source | TDoc ID | Content Summary |
| --- | --- | --- |
| Nokia | R1-2600030 | Consider further need of two adaptation delay values; only support two if sufficient justification exists. T configurable in range {≥1ms, ≤5ms}. T sufficient to enable UE confirmation in UL. Omit adaptation interruption for TDD; for FDD assume 1 slot switching interruption. |
| vivo | R1-2600502 | T similar to Fast SCell Activation Delay (TS 38.133 §8.3.16): 5-10ms. Tmin similar to BWP switching delay, minus parameter loading: 1-2ms. For max configured BW evaluation, scaling factor should converge to one set regardless of adaptation delay. |
| Samsung | R1-2600754 | Identical scaling factors for Tmin and T provide conservative and broadly applicable baseline. Power primarily determined by target BW configuration rather than adaptation delay duration. Implementation data shows S_MaxBW=0. |
| Apple | R1-2600826 | Differentiation between T and Tmin may not be necessary. If Tmin sufficiently large (e.g., DCI-based BWP switching Type 2), difference can be minimal. Propose merge into single column based on Tmin assumption. Tmin: use DCI BWP switching Type 2 as starting point, adjustable for actual 6GR procedure. |
| Ericsson | R1-2600960 | Long BW adaptation delay (>1ms) should not be considered for 6G evaluations. One main drawback of 5G BWP was switching delay. Focus in 6G should be on fast BW adaptation. Only Tmin=1ms considered. |
| Lenovo | R1-2601106 | Xij=X(i+1)j corresponding to different adaptation delays. S_MaxBW is additional static power for tight adaptation (additional circuitry remains active). Tightening delay reduces micro-sleep opportunities; additional always-on power upper-bounded by PPDCCH-only - Pmicro-sleep = 55 units. |
| MediaTek | R1-2601240 | Power scaling follows (f/f₀)×(V/V₀)² relationship. Longer adaptation time (5ms)→gradual clock/voltage adjustment, better efficiency. Shorter time (2ms)→aggressive overclocking/voltage boosting, higher power. Examples: 200MHz with T→1.7, with Tmin→2.0; 400MHz→5.6 requiring significant overclock/voltage increase. |
| Qualcomm | R1-2601271 | For ΓB>20MHz: T and Tmin identical. For ΓB≤20MHz: can differ. Threshold at ΓB=20% models switching between reduced-capability and eMBB modems. Higher threshold assumes too much HW overhead/duplication. For ΓB=100%,200%: same scaling for T and Tmin (like ΓB=400%). SMaxBW=0 at smaller active BWs. |

S_MaxBW (Static Power Overhead)

| Source | TDoc ID | Content Summary |
| --- | --- | --- |
| Huawei/HiSilicon | R1-2600142 | One max BW corresponds to one fixed additional relative power value (static power increasing is same). Due to different basis, S_MaxBW values differ for different channels. PDCCH: S_MaxBW=0/0.45/1.55 for MaxBW∈{100,200,400}MHz. PDCCH+PDSCH: S_MaxBW=0/0.15/0.52 for MaxBW∈{100,200,400}MHz. |
| OPPO | R1-2600197 | Option 1: S_MaxBW=0/0.3/0.5 for MaxBW. |
| Xiaomi | R1-2600427 | Discuss and determine S_MaxBW value for Option 1. Option 1 (SMaxBW=0/Z1/Z2 for MaxBW∈{100,200,400}MHz) assumes standby power varies under different max BWs. Non-zero value needed, but specific values require further discussion. |
| vivo | R1-2600502 | Two interpretations: a) max BW supported by UE; b) max BW configured to UE (≤max supported). Consider b) reflects SMaxBW motivation. When UE works at max configured BW (400/200/100MHz), scaling factor should converge to one set regardless of adaptation delay. FFS values of delta, SMaxBW. |
| Samsung | R1-2600754 | Implementation data shows identical values for both adaptation delays (S_MaxBW=0). Design approach where power primarily determined by target BW configuration rather than adaptation delay duration. |
| Apple | R1-2600826 | Adopt S_MaxBW=0 (Option 2). Same reason as T/Tmin differentiation not being necessary. |
| Lenovo | R1-2601106 | S_MaxBW≤0.18 based on gap between PDCCH-only and micro-sleep states. S_MaxBW is additional static power for tight adaptation requirements. Additional always-on power upper-bounded by PPDCCH-only - Pmicro-sleep = 55 units; therefore S_MaxBW = [ΔPstatic,max]/Prelaxed,ref ≤ 55/300 ≈ 0.18. |
| Google | R1-2601205 | Support Option 1 (S_MaxBW>0) as baseline to realistically model static power overhead. S_MaxBW=0 assumes perfect gear-shifting where 400MHz device = 20MHz device at 20MHz operation—physically unrealistic for early 6G. Wideband UEs have static leakage/overhead from unused but powered-on RF/analog chains that can't be fully gated off instantaneously. Non-zero penalty needed to avoid overestimating EE of wideband devices in low-load conditions. |
| MediaTek | R1-2601240 | S_MaxBW=0.05 for MaxBW=200MHz, S_MaxBW=0.2 for MaxBW=400MHz. Integrated into scaling tables for both PDCCH+PDSCH and other signal processing. |
| Qualcomm | R1-2601271 | SMaxBW=0 view. No fundamental justification for treating ΓB=200% or 100% differently from ΓB=400% (which assumes unified scaling for both delays). SMaxBW zero at remaining smaller active BW values. |

      - Company Views Summary

| Company | PDCCH+PDSCH 200% | PDCCH+PDSCH 400% | PDCCH-only 200% | PDCCH-only 400% | S_MaxBW (200/400) | TDoc |
| --- | --- | --- | --- | --- | --- | --- |
| Apple | 1.7 | 3.0 | 1.2 | 1.6 | 0 / 0 | R1-2600826 |
| Ericsson | 1.8 | 3.4 | 1.8 | 3.4 | FFS | R1-2600960 |
| Huawei | 2.7 | 7.45 | 1.55 | 4.23 | 0.15 / 0.52 | R1-2600142 |
| MediaTek | 2.0 | 5.6 | 2.0 | 5.6 | 0.05 / 0.2 | R1-2601240 |
| Nokia | 2.02 | 4.88 | 0.48 | 0.80 | N/A | R1-2600030 |
| OPPO | 1.7 | 3.6 | 1.5 | 3.0 | 0.3 / 0.5 | R1-2600197 |
| Qualcomm | 2.3 | 5.2 | 1.9 | 3.7 | 0 / 0 | R1-2601271 |
| Samsung | 2.32 | 5.66 | 1.7 | 3.1 | 0 / 0 | R1-2600754 |
| vivo | 1.7 | 3.4 | 1.7 | 3.4 | FFS | R1-2600502 |
| Xiaomi | 1.7 | 3.4 | 1.7 | 3.4 | FFS | R1-2600427 |
| ZTE | 1.85 | 3.55 | 1.55 | 2.65 | FFS | R1-2600456 |
| Lenovo | - | - | - | - | FFS | R1-2601106 |
| Google | - | - | - | - | >0 | R1-2601205 |
| Avg(All) | 1.98 | 4.57 | 1.55 | 3.15 | 0.10 / 0.24 | - |

Summary

Consensus exists on including all five DL bandwidth points {5%, 20%, 100%, 200%, 400%} for both PDCCH+PDSCH and PDCCH-only scaling, with 13 of 15 companies providing values. At 200% (200 MHz), ΓTput=1, there is moderate clustering around 1.7-2.0 for PDCCH+PDSCH (CA-based approach from Xiaomi, vivo, OPPO, Apple, Ericsson), with higher values from Samsung (2.32), Qualcomm (2.3), Huawei (2.7), MediaTek (2.0) reflecting different implementation characteristics. At 400% (400 MHz), different implementation approaches yield CA-based scaling (~3.4) and steeper non-linear scaling (~5.2-7.45), both reflecting valid architectural choices. The Avg(All) of 4.57 captures this implementation diversity.

On adaptation delay, 5 companies (Ericsson, Apple, Samsung, Nokia, Qualcomm partial) support a single-column Tmin approach (1-2 ms), while 7 companies provide separate T vs Tmin columns. On S_MaxBW static power overhead, 7 companies (Google, Huawei, MediaTek, OPPO, Xiaomi, vivo, Lenovo) support S_MaxBW > 0 and 5 companies (Samsung, Apple, Qualcomm, Ericsson, Nokia) support S_MaxBW = 0, representing different implementation approaches to maximum bandwidth power modeling.

For PDCCH-only, strong support exists for removing ΓTput variations (Spreadtrum, Xiaomi, Apple, Ericsson), with Huawei showing identical values for ΓTput=1 and 1/2, confirming no throughput impact. Below 100 MHz, linear extension of the NR model (TR 38.840) is supported by Lenovo, Qualcomm, and OPPO.

From the averaged values of companies’ input values (please check Initial parameter check draft folder), the difference between T and Tmin is small, it is therefore suggested to consider adding (small) S_MaxBW while keeping only T_min (<= NR BWP switch delay). Please refer to the following proposals for detailed values.

      - Proposals

Proposal 4.1.1-3: (1st Round; High) PDCCH+PDSCH Bandwidth Scaling Table

Update PDCCH+PDSCH bandwidth scaling table in the 6GR UE power model with the following values

- Subject to adaptation delay = Tmin, no larger than NR BWP switch delay (Type 2)

- Additional SMaxBW = 0.05 and 0.10 is added to rows with ΓB = 5%, 20%, 100% and 200% for MaxBW = 200% and 400%, respectively

| ΓB | ΓTput=1 | ΓTput=1/2 | ΓTput=1/4 |
| --- | --- | --- | --- |
| 5% | [0.35] | - | - |
| 20% | [0.40] | - | - |
| 100% | 1.00 | - | - |
| 200% | [2.00] | [1.45] | - |
| 400% | [4.60] | [3.20] | [2.40] |

| Company Name | Comments/Views |
| --- | --- |
| Huawei, HiSilicon | We cannot accept the power model with only a single adaptation delay Tmin. First, based on the excel file, 20MHz with T_Tput=1, the average values for {T, Tmin with 200M} are {0.39, 0.484}, the difference is ~0.2, which is much larger than 0.05. The difference actually comes from the fact that based on some implementation assumption, scaling factor of Tmin is larger than that of T. Second, different companies may have different assumption on the T/Tmin values. For example, most companies think Tmin=1~2ms, while some company consider Tmin=3ms. With different understanding, the scaling can also be different. To our understanding, different views are based on different implementation assumptions. For example, if the power consumption is adapted by voltage/clock adjusting, then the adaptation can be faster, and there is no differentiation with T and Tmin. Maybe also no impact by maxBW. (See the views from 5 companies.) However, if the power consumption is adapted by circuit components on/off, different static power can be achieved by T and Tmin. (See the views from 7 companies.) To fairly reflect the two possibility options, and to move forward, we suggest maybe we can consider two options for the BW scaling model: - Option #1: different power values for T and Tmin, and non-zero value of S_maxBW - Option #2: a single power value for Tmin, and zero value of S_maxBW |
| OPPO | We are either OK to have 2 sets of values for T or Tmin, as long as we can commonly agree. However, if there can not be consensus, only one of them can eb picked. If the Tmin taken, then we suggest to increase the SMaxBW to much larger number to address HW’s concern. Our, suggested value would be 0.2 and 0.4 respectively. |
| ZTE, Sanechips | - For PDCCH only, we also support to remove the throughput impact and could be discussed in 4.1.1-4. - For different adaptation delay, there could be some power consumption impacts, which is up to implementation. But in our understanding, the static part should not be scaled and only dynamic part could be scaled. Therefore, the scale factor should not be quite small. - For throughput, it only has impacts on baseband processing. If baseband processing occupies 50% power consumption, then 1/5 throughput(20%) brings 80% baseband processing reduction at maximum. The corresponding scaling factor is 0.6. Considering the static part should not scaled, the scaling factor should be larger. |
| Ericsson | - Agree to only consider adaptation delay = Tmin. This is beneficial for more uniform evaluations and observations. - In our view, the scaling for 400% is over-estimated. A value ≤4 is more reasonable. - Regarding 5%: we think the value should be smaller (e.g., <0.3). We noticed that a couple of companies report exactly the same value for 5% and 20%. While we understand that the rate of power saving decreases for smaller bandwidths, still there must be some gains due to reduced BB. components (e.g., reduced FFT, data processing, etc). Also, using the scaling rule from TR 38.840 (X MHz = 0.4 + 0.6 * (X - 20) / 80) which is valid for 10 MHz case, the scaling for 10 MHz is 0.32. Therefore, for 5 MHz it should be less than 0.3. |
| Spreadtrum | Firstly, since the difference of scaling factors between 5MHz and 20MHz is only 0.05, we think it is unnecessary to consider these two bandwidths simultaneously. In the evaluation, there may be little difference between 5MHz and 20MHz. 5MHz should be removed. Secondly, we think clarification is needed for that whether proposal 4.1.1-3 is only for adaptation delay = Tmin or any value of adaptation delay. |
| Xiaomi | Since this proposal corresponds to Tmin​, i.e., a very short switching time, based on previous discussions, the UE may not be able to reduce static power consumption, and only dynamic power consumption can be reduced. Therefore, for ΓB = 20% and 5%, the energy consumption reduction may not achieve the values listed in the table. Take the case where 100 MHz is reduced to 20 MHz as an example. RedCap provides a good reference: when RedCap reduces the bandwidth from 100 MHz to 20 MHz and simultaneously reduces the antenna from 4Rx to 2Rx, the scaling factor for PDCCH+PDSCH is 0.4. RedCap can be regarded as a scaling that reduces both static and dynamic power consumption. Here only the bandwidth is reduced within a short switching time —why can a scaling factor of 0.4 be achieved? |
| Nokia.1 | The power values are aggressively scaled. If 400% to ΓTput = ¼, is larger than 200MHz ΓTput = 1. |
| Qualcomm | OK with the suggested proposal from the moderator. As mentioned in our contribution, our view is that SmaxBW=0. Our preference is to simplify as such. |
| Samsung | We support the single-column Tmin approach for adaptation delay. Based on our implementation understanding, power consumption adaptation can be achieved through voltage/clock adjustment, where the adaptation is fast enough that there is less meaningful differentiation between T and Tmin​. |
| Nokia2 | Also a note that based on our evaluation, we did not see major difference when going below 20% e.g. to 5% for 100MHz device. Thus we suggested to omit the ΓB=5%. For the adaptation delay, we suppose that ~2ms is OK for the UE power saving study. However but note that the robust adaptation e.g. for BW, should take priority in actual 6GR design. |
| CATT | We are OK with the proposal. In 6GR, faster bandwidth adaptation than 5G should be considered as a baseline. Thus, smaller bandwidth adaptation delay and S_maxBW=0 should be considered for scaling factors. From CATT’s perspective, the scaling rules on bandwidth adaptation is expiated as simpler as possible. |

Moderator

This proposal has the highest divergence. Three intertwined debates: (1) T vs Tmin adaptation delay, (2) S_MaxBW value, (3) absolute scaling at 400%.

Suggestion: keep T > Tmin only for ΓB = 5% and 20% (useful for low-capability UEs), with the main table using Tmin. This addresses Huawei's core concern (T column preserved where it matters) while simplifying the main table per Samsung/Ericsson/Qualcomm preference.

For S_MaxBW, the 7-vs-5 split favors non-zero. A compromise of small S_MaxBW values (0.05/0.10) is proposed as two options in the table, letting companies evaluate both.

Additionally, extend ΓTput down to 1/4 for PDCCH+PDSCH. Keep 5% and 20% as separate rows (companies can evaluate significance).

Proposal 4.1.1-3a: (2nd Round; High) PDCCH+PDSCH Bandwidth Scaling Table

Update PDCCH+PDSCH bandwidth scaling table in the 6GR UE power model with the following values

- Subject to adaptation delay = Tmin, no larger than NR BWP switch delay (Type 2)

  - Additional SMaxBW can be zero, or the other option: 0.15 and 0.35 is added to rows with ΓB = 5%, 20%, 100% and 200% for MaxBW = 200% and 400%, respectively

| ΓB | ΓTput=1 | ΓTput=1/2 | ΓTput=1/4 |
| --- | --- | --- | --- |
| 5% | [0.30] | - | - |
| 20% | [0.40] | - | - |
| 100% | 1.00 | - | - |
| 200% | [2.00] | [1.45] | - |
| 400% | [4.60] | [3.20] | [2.40] |

- Additional scaling included for ΓB = 5% and 20%, and 5ms≤T≤10 ms:

| ΓB | Scaling factor when adaptation delay T>Tmin is assumed |
| --- | --- |
| 5% | 0.20 |
| 20% | 0.25 |

| Company Name | Comments/Views |
| --- | --- |
| Huawei, HiSilicon | We are fine with the framework to move on. For the initial values, they are reasonable from our side. |
| Qualcomm | Structure looks OK. Could you please replace the values with variable names as discussed in the offline session? |

Proposal 4.1.1-4: (1st Round; High) PDCCH-only Bandwidth Scaling Table

Update PDCCH-only bandwidth scaling table in the 6GR UE power model with the following values

- Subject to adaptation delay = Tmin, no larger than NR BWP switch delay (Type 2)

- Additional SMaxBW = 0.05 and 0.15 is added to rows with ΓB = 5%, 20%, 100% and 200% for MaxBW = 200% and 400%, respectively

| ΓB | ΓTput=1 | ΓTput=1/2 |
| --- | --- | --- |
| 5% | [0.45] | - |
| 20% | [0.50] | - |
| 100% | 1.00 | - |
| 200% | [1.55] | [1.30] |
| 400% | [3.20] | [2.40] |

| Company Name | Comments/Views |
| --- | --- |
| Huawei, HiSilicon | Similar comments as proposal 4.1.1-3 |
| OPPO | Seems the 5% is larger, Suggest to change to 0.40. |
| ZTE, Sanechips | In our understanding, the static part should not be scaled and only dynamic part could be scaled. Therefore, the scale factor for 5% and 20% BW should be larger, for example, >0.53 for 5% BW, and >0.6 for 20% BW. |
| Spreadtrum | Similar comments in proposal 4.1.1-3. In addition, we think there is no relationship between PDCCH-only state and PDSCH throughput. |
| Xiaomi | Similar comments as the previous one. When RedCap reduces the bandwidth from 100 MHz to 20 MHz and simultaneously reduces the antenna from 4Rx to 2Rx, the scaling factor for PDCCH-only is 0.5. Here only the bandwidth is reduced within a short switching time -why can a scaling factor of 0.5 be achieved? |
| Qualcomm | OK with the proposal |
| Nokia2 | For the PDCCH only case, we assume PDCCH+PDSCH as the reference point (300units) not PDCCH only (100units). I’ve updated the excel, (3x). The average values are slightly changed. Also for PDCCH-only case we suggested to omit the ΓB=5%. |
| CATT | See our comments in proposal 4.1.1-3. |

Moderator

The reference-point confusion (Nokia2) is important to clarify: the scaling factors in the table are relative to PDCCH-only at 100 MHz (= 100 units), not PDCCH+PDSCH (300 units).

Suggestion: PDCCH-only includes ΓTput = {1, 1/2} only (no 1/4). Spreadtrum's concern about no throughput relationship is partially addressed — ΓTput for PDCCH-only reflects that PDSCH throughput configuration impacts PDCCH configuration/monitoring.

OPPO's suggestion (5% = 0.40) is close to Avg(All) = 0.46; the current value reflects data from multiple companies. Keep Avg(All).

Proposal 4.1.1-4a: (2nd Round; High) PDCCH-only Bandwidth Scaling Table

Update PDCCH-only bandwidth scaling table in the 6GR UE power model with the following values

- Subject to adaptation delay = Tmin, no larger than NR BWP switch delay (Type 2)

  - Additional SMaxBW can be Zero, or the other option: 0.4 and 1.0 is added to rows with ΓB = 5%, 20%, 100% and 200% for MaxBW = 200% and 400%, respectively

| ΓB | ΓTput=1 | ΓTput=1/2 |
| --- | --- | --- |
| 5% | [0.50] | - |
| 20% | [0.60] | - |
| 100% | 1.00 | - |
| 200% | [1.60] | [1.30] |
| 400% | [3.20] | [2.40] |

- Additional scaling is included for ΓB = 5% and 20%, and 5ms≤T≤10 ms:

| ΓB | Scaling factor when adaptation delay T>Tmin is assumed |
| --- | --- |
| 5% | 0.35 |
| 20% | 0.40 |

For other DL signal/channel processing, refer to the first scaling column of the above tables

| Company Name | Comments/Views |
| --- | --- |
| Huawei, HiSilicon | We are fine with the framework to move on. For the initial values, they are reasonable from our side. |
| Qualcomm | Structure looks OK. Could you please replace the values with variable names as discussed in the offline session? Per the offline discussion, could you please replace “ΓTput=1/2” with “ΓTput <= 1/2”? This is so that it is clear what value to use when ΓTput=1/4. |

Proposal 4.1.1-4: (1st Round; High) Bandwidth Scaling Table for Other DL Signal/Channel Processing

Update bandwidth scaling table for Other DL Signal/Channel Processing in the 6GR UE power model with the following values

- Subject to adaptation delay = Tmin, no larger than NR BWP switch delay (Type 2)

| ΓB | ΓTput=1 |
| --- | --- |
| 5% | [0.50] |
| 20% | [0.60] |
| 100% | 1.00 |
| 200% | [1.70] |
| 400% | [3.50] |

| Company Name | Comments/Views |
| --- | --- |
| Huawei, HiSilicon | Similar comments as proposal 4.1.1-3 |
| ZTE, Sanechips | For other DL signal/channel processing, since the corresponding power consumption is the same as that in the PDCCH-only state or similar to that in the PDCCH+PDSCH state, the scaling factors for the PDCCH+PDSCH or PDCCH-only states can be reused for simplicity. |
| Spreadtrum | Similar comments in proposal 4.1.1-3 |
| Qualcomm | Prefer to reuse the PDCCH scaling with gamma TPUT =1 |
| Samsung | Same preference with QC |

Moderator

Merged to Proposal 4.1.1-4a

    - CA Scaling

      - Detailed Company Contributions (Collapsed; Please Expand to View)

DL Carrier Aggregation

| Source | TDoc ID | Content Summary |
| --- | --- | --- |
| CATT | R1-2600294 | Proposes to DEPRIORITIZE CA and antenna scaling discussion until AI-10.1 determines feature scope (4CC UL support, 6Rx/8Rx/4Tx support). |
| Xiaomi | R1-2600427 | Proposes 2CC=Z5 of 1CC; 4CC=Z6 of 1CC (worst case configuration). Activation/deactivation follows RAN4 spec; transition energy FFS. Applicable FR1/FR2. |
| MediaTek | R1-2601240 | Proposes 2CC=Z5=1.7 of 1CC; 4CC=Z6=3.4 of 1CC (worst case). Observes separate RF/BB engines result in lower per-MHz power vs wideband single carrier. |
| Qualcomm | R1-2601271 | Proposes β-factor approach: Contiguous intra-band β1=1.10 (2CC), 1.20 (4CC); Inter-band/non-contiguous β2=1.15 (2CC), 1.25 (4CC) applied to single-CC BW scaling. |

UL Carrier Aggregation

| Source | TDoc ID | Content Summary |
| --- | --- | --- |
| Xiaomi | R1-2600427 | Proposes at ≤10dBm: same as DL; at >10dBm: 2CC=1.2x of 1CC, 4CC=Z7 of 1CC. Limits scaling up to 4CC. Applicable FR1/FR2. |
| MediaTek | R1-2601240 | Proposes at ≤10dBm: same as DL; at >10dBm: 2CC=1.2x, 4CC=Z7=1.5 of 1CC. Observes RF dominates at low power, but baseband significant at high power with >2CC. |

      - Company Views Summary

| Company | DL 2CC | DL 4CC | UL 2CC (>10dBm) | UL 4CC (>10dBm) | Notes | TDoc |
| --- | --- | --- | --- | --- | --- | --- |
| MediaTek | 1.7 | 3.4 | 1.2 | 1.5 | Worst-case CA config; separate RF/BB engines | R1-2601240 |
| Qualcomm (contig.) | 1.10 | 1.20 | - | - | Beta-factor on single-CC BW scaling | R1-2601271 |
| Qualcomm (non-contig.) | 1.15 | 1.25 | - | - | Inter-band / non-contiguous | R1-2601271 |
| Xiaomi | FFS | FFS | 1.2 | FFS | Up to 4CC; at <=10dBm same as DL | R1-2600427 |
| CATT | - | - | - | - | Deprioritize until AI-10.1 | R1-2600294 |
| Avg(All) | 1.32 | 1.95 | 1.2 | 1.5 |  | - |

Summary

Two DL CA scaling approaches are proposed, reflecting different implementation architectures. MediaTek proposes absolute scaling (2CC=1.7, 4CC=3.4) assuming worst-case configuration with separate RF/BB engines, showing exactly linear scaling per CC count. Qualcomm proposes a relative beta-factor approach applied on top of single-CC BW scaling, distinguishing contiguous intra-band (beta=1.10/1.20) from inter-band (beta=1.15/1.25), yielding sub-linear scaling with diminishing per-CC overhead. The Avg(All) inclusive of both methodologies is 2CC=1.32, 4CC=1.95. Xiaomi supports the concept with FFS placeholder values. CATT proposes deprioritizing CA scaling discussion pending AI-10.1 feature scope decisions.

For UL CA, strong convergence exists: MediaTek and Xiaomi agree that at <=10 dBm, UL CA scaling is the same as DL CA. At >10 dBm, both support 2CC=1.2x of 1CC. For 4CC at >10 dBm, only MediaTek provides an explicit value of 1.5, noting RF dominance at high power reduces per-CC overhead relative to DL.

      - Proposals

Proposal 4.1.2-1: (1st Round; High) DL Carrier Aggregation Scaling

Include the following DL carrier aggregation scaling factors in the UE power model (relative to 1CC power at same bandwidth):

| Config | Scaling Factor | Notes |
| --- | --- | --- |
| 1CC | 1.0 | Reference |
| 2CC | [1.4] | Common for all CA configs |
| 4CC | [2.0] | Common for all CA configs |

Moderator Note: Different implementation approaches (absolute scaling from MediaTek and beta-factor from Qualcomm) yield different numerical values. Avg(All) provides an implementation-agnostic value consistent with 3GPP's principle of not mandating specific implementations. Activation/deactivation delay follows RAN4 specification. Transition energy: FFS.

| Company Name | Comments/Views |
| --- | --- |
| Huawei, HiSilicon | In our view, these scaling factors are too small. We may wait for the outcome of BW scaling, then we can discuss the CA scaling. If finally two options of BW scaling are agreed, then for CA scaling there can also be two options: - For option #1, use the same value as the BW scaling case with adaptation delay T. - For option #2, additional scaling factor > 1, compared with the same total BW of single cell |
| OPPO | Generally OK. |
| ZTE, Sanechips | Intra-band CA and inter-band CA should be different. For intra-band CA, it could be like this. But for inter-band CA, it should be accumulated based on each CC. Additionally, other number of CCs also should be defined. |
| Spreadtrum | We prefer to reuse 5G scaling factor, e.g., 1.7x and 3.4x for 2CC and 4CC respectively. |
| Nokia.1 | 400MHz single CC power scaling is 4.6. However, if we use 4CC the scaling is only 2. |
| Qualcomm | Generally, we agree with the direction of deriving the CA scalings from by accounting for additional scaling factor >1 compared with the same total BW of single cell. However, the proposal is not clear in whether the scaling is with respect to the aggregated bandwidth or an individual CC. In our view, it should be the aggregate bandwidth as in Huawei’s Option 2. For example, for 2 CCs each of 100 MHz, the relative power for CA 2x100 MHz is beta * (relative power of a single CC with total BW of 200 MHz). If this is the intention of the proposal, the scaling factors in the table are too large since they would be applied on top of the BW scaling factors. Differentiation is needed in our view for inter- vs intra-band cases. |
| Samsung | We share the concern that the proposed CA scaling factors (1.4 for 2CC, 2.0 for 4CC) are too small. |
| CATT | For 2CC and 2Tx, the power consumption model in 5G NR can be used for 6GR. However, for larger CA (4CC) and larger antenna numbers (e.g. 6Rx, 8Rx and 4Tx), whether they are supported will be determined by other agenda e.g. AI-10.1. Thus, the discussion on scaling for CA and larger antenna numbers can be deprioritized. |

Moderator

CA scaling is a multiplicative factor on topic of the 1CC power consumption (which can be scaled according to bandwidth scaling). From companies’ inputs quoted below, 2x factor for 2CC and 4.2x factor for 4 CC is suggested. It is noticed that CA architecture may exploit more parallel processing blocks instead of voltage boost that causes second order increment in power consumption (P∝V2), which provides some reason why averaged CA power consumption can be smaller than bandwidth scaling.

| Company | 2CC (Z5) | 4CC (Z6) |
| --- | --- | --- |
| Huawei | 2 | 4.225 |
| MediaTek | 1.7 | 3.4 |
| Qualcomm (contig.) | 2.2 | 4.8 |
| Qualcomm (non-contig.) | 2.3 | 5 |
| Spreadtrum | 1.7 | 3.4 |
| Avg(all) | 1.98 | 4.165 |

Proposal 4.1.2-1a: (2nd Round; High) DL Carrier Aggregation Scaling

Include the following DL carrier aggregation scaling factors (w.r.t. 1CC power at same bandwidth) in the UE power model:

| Config | Scaling Factor | Notes |
| --- | --- | --- |
| 1CC | 1.0 | Reference |
| 2CC | [2.0] | Common for all CA configs |
| 4CC | [4.2] | Common for all CA configs |

| Company Name | Comments/Views |
| --- | --- |
| Huawei, HiSilicon | We are generally OK with this direction, but we also have several suggestions: - The collected values on the table above is for PDCCH-only, the PDCCH+PDSCH part is missing. - Since for single carrier case we already have separate table for PDCCH+PDSCH and PDCCH-only, it is better to also consider separate table for different channels for CA case. The logic from FL is fine for us, we can have a relatively smaller scaling factor for CA, compared with single carrier case with the same total BW. - Maybe we need to clarify the use case for CA. Is every carrier assumed to be only 100M, or larger carrier BW can also be applied with the same scaling factor? |
| Qualcomm | We do not agree with the proposal. We also disagree with the statement that CA can exploit parallelism not available to the single CC case. The parallel baseband hardware could be used in both multi-CC and single-CC cases. On the other hand, there is additional overhead in the multi-CC case that does not exist in the single CC case. Also that assumption does not compare two receivers with the same area. We think it would be clearer to scale the multi-CC case from the single CC case with the total aggregated bandwidth. E.g. 2x100MHz from the 200MHz case; the 2x200MHz from 400MHz case; and 4x100MHz from the 400MHz case. We also think there should be distinction between the cases when a single RF chain is used vs. when multiple RF chains are used. The power of the second will be higher than the first. |

Proposal 4.1.2-2: (1st Round; High) UL Carrier Aggregation Scaling

Include the following UL carrier aggregation scaling factors:

- Scaling definition up to 4CC. Applicable for FR1 and FR2.

| TX Power | 2CC | 4CC |
| --- | --- | --- |
| ≤10 dBm | Same as DL | Same as DL |
| >10 dBm | [1.2] | [1.5] |

| Company Name | Comments/Views |
| --- | --- |
| Huawei, HiSilicon | This direction is fine. Detailed values can be discussed later after DL CA. |
| OPPO | For UL, I have clarification. Is the TX power means total power of all UL carriers and the scaling will apply to the power units of UL transmission? Then, that is OK. |
| ZTE, Sanechips | Open to discuss, and also should be based on intra-band and inter-band CA case. |
| Qualcomm | We agree with Huawei that this is better discussed later, particularly after the UL Tx power discussion |

Moderator

Given the update of DL CA scaling factor, the scaling number is relatively constant. Moderator suggests companies’ further consideration.

Proposal 4.1.2-2a: (2nd Round; High) UL Carrier Aggregation Scaling

Include the following UL carrier aggregation scaling factors:

- Scaling definition up to 4CC. Applicable for FR1 and FR2.

| TX Power | 2CC | 4CC |
| --- | --- | --- |
| ≤10 dBm | Same as DL (2.0) | Same as DL (4.2) |
| >10 dBm | [1.2] | [1.5] |

| Company Name | Comments/Views |
| --- | --- |
| Qualcomm | OK with the structure, but please replace the values with variable names. |

    - Antenna Scaling

      - Detailed Company Contributions (Collapsed; Please Expand to View)

DL Antenna Scaling

| Source | TDoc ID | Content Summary |
| --- | --- | --- |
| CMCC | R1-2600387 | Proposes TR 38.840 methodology as baseline with FFS for updates. Considers impact of DL/UL antenna number and processing complexity in PDCCH. |
| Xiaomi | R1-2600427 | Proposes 6Rx power as Z8 times 4Rx for FR1; 2Rx is 0.7x 4Rx for FR1/FR2; 1Rx is 0.7x 2Rx for FR2. Assumes same antenna elements per Rx chain. |
| MediaTek | R1-2601240 | Proposes detailed scaling: 6Rx=Z8=1.43x 4Rx; 8Rx=2.0x 4Rx for FR1; 2Rx=0.7x 4Rx; 1Rx=0.7x 2Rx. Observes 2-codeword processing increases complexity faster for 6Rx transition. |
| Qualcomm | R1-2601271 | Proposes 6Rx (4 layers)=1.4x 4Rx for FR1; 2Rx=0.7x 4Rx for FR1; 1Rx=0.7x 2Rx for FR2. Notes scaling beyond 4Rx needed for 6GR. |

UL Antenna Scaling

| Source | TDoc ID | Content Summary |
| --- | --- | --- |
| Xiaomi | R1-2600427 | Proposes power-dependent scaling: 2Tx is 1.4x 1Tx at 0dBm; 1.2x at 23dBm; Z9 at 26dBm for FR1. Notes 4Tx not considered for FR2. |
| MediaTek | R1-2601240 | Proposes constant 2.0x scaling at all power levels (0dBm, 23dBm, Z9=2.0 at 26dBm). Observes separate TX chains double power consumption for 1Tx→2Tx transition. |
| Qualcomm | R1-2601271 | Proposes power-dependent scaling: 2Tx is 1.4x at 0dBm; 1.2x at 23dBm; 1.6x at 26dBm. Emphasizes need for 26dBm scaling inclusion. |

      - Company Views Summary

DL Antenna Scaling

| Company | 6Rx | 8Rx | 2Rx (rel. 4Rx) | 1Rx FR2 (rel. 2Rx) | TDoc |
| --- | --- | --- | --- | --- | --- |
| MediaTek | 1.43 | 2.0 | 0.7 | 0.7 | R1-2601240 |
| Qualcomm | 1.4 | - | 0.7 | 0.7 | R1-2601271 |
| Xiaomi | - | - | 0.7 | 0.7 | R1-2600427 |
| Samsung | - | - | 0.7 | - | R1-2600754 |
| Ericsson | - | - | 0.7 | - | R1-2600960 |
| CATT | - | - | 0.7 | - | R1-2600294 |
| Lenovo | - | - | 0.7 | - | R1-2601106 |
| vivo | - | - | 0.7 | - | R1-2600502 |
| Avg(All) | 1.415 | 2.0 | 0.7 | 0.7 | - |

UL Antenna Scaling (2Tx relative to 1Tx)

| Company | 0 dBm | 23 dBm | 26 dBm | Architecture | TDoc |
| --- | --- | --- | --- | --- | --- |
| MediaTek | 2.0 | 2.0 | 2.0 | Separate TX chains (constant) | R1-2601240 |
| Qualcomm | 1.4 | 1.2 | 1.6 | Shared/switched chains (power-dep.) | R1-2601271 |
| Xiaomi | 1.4 | 1.2 | - | Shared/switched chains (power-dep.) | R1-2600427 |
| Avg(All) | 1.6 | 1.47 | 1.8 |  | - |

Summary

DL antenna scaling shows universal consensus on lower-order configurations: all 8 contributing companies agree that 2Rx = 0.7 x 4Rx for FR1/FR2, and 1Rx = 0.7 x 2Rx for FR2 (MediaTek, Qualcomm, Xiaomi, Samsung, Ericsson, CATT, Lenovo, vivo). For 6Rx, MediaTek (1.43) and Qualcomm (1.4) show tight convergence (2% difference), attributed to 2-codeword processing overhead. For 8Rx, only MediaTek proposes 2.0x (exactly linear scaling), pending AI-10.1 feature scope.

UL antenna scaling reflects different architectural approaches. Qualcomm and Xiaomi propose power-dependent scaling (1.4x at 0 dBm, 1.2x at 23 dBm) reflecting shared/switched chains where RF PA dominance increases at higher power. MediaTek proposes constant 2.0x at all power levels, arguing modern implementations use separate TX chains that double power consumption. The Avg(All) inclusive values are: 0 dBm=1.6, 23 dBm=1.47, 26 dBm=1.8.

      - Proposals

Proposal 4.1.3-1: (1st Round; High) DL Antenna Scaling Factors

Include the following DL antenna scaling factors in the UE power model:

| Config | Scaling Factor | Reference |
| --- | --- | --- |
| 4Rx (FR1) | 1.0 | Baseline |
| 2Rx (FR1/FR2) | 0.7 × 4Rx | TR 38.840 |
| 1Rx (FR2) | 0.7 × 2Rx | TR 38.840 |
| 6Rx (FR1) | [1.4] × 4Rx | Avg(All) |
| 8Rx (FR1) | [2.0] × 4Rx | Adoption conditional on AI-10.1 |

| Company Name | Comments/Views |
| --- | --- |
| Huawei, HiSilicon | For 6RX, the value is OK. For 8RX, there is not enough input, we need to further check the value. |
| OPPO | OK |
| ZTE, Sanechips | We think the power model in TR.38.840 can be viewed as starting point. However, we are open to discuss it. |
| Spreadtrum | We share the same view with Huawei. |
| Xiaomi | OK |
| Nokia.1 | 4Rx 400MHz vs 8Rx 200MHz there is a discrepancy in the power values if we consider the above two configurations. |
| Qualcomm | For 6RX, the value is reasonable. For 8Rx, we suggest [1.7]. |
| CATT | We are OK with the value for 2RX. However, the motivation for 6RX and 8RX in EE processing is not clear for us. |

Moderator

Close to consensus with some minor update to 8RX value.

Proposal 4.1.3-1a: (2nd Round; High; Stable) DL Antenna Scaling Factors

Include the following DL antenna scaling factors in the UE power model:

| Config | Scaling Factor |
| --- | --- |
| 4Rx (FR1) | 1.0 |
| 2Rx (FR1/FR2) | 0.7 × 4Rx |
| 1Rx (FR2) | 0.7 × 2Rx |
| 6Rx (FR1) | [1.4] × 4Rx |
| 8Rx (FR1) | [1.85] × 4Rx |

| Company Name | Comments/Views |
| --- | --- |
| Qualcomm | Structure looks OK. Could you please replace values with variables? |

Proposal 4.1.3-2: (1st Round; High) UL Antenna Scaling (2Tx vs 1Tx)

Include the following UL antenna scaling factors (2Tx relative to 1Tx):

| TX Power | Scaling Factor |
| --- | --- |
| 0 dBm | [1.5] |
| 23 dBm | [1.5] |
| 26 dBm | [1.8] |

| Company Name | Comments/Views |
| --- | --- |
| Huawei, HiSilicon | UL power is mainly dominated by the PA power, especially for high transmit power case. So at least for 23dBm and 26dBm, the scaling factor should be 1. For 0dBm, we may further discuss the value, but should be smaller than 1.5. |
| OPPO | We agree with Huawei, the scaling should be much smaller. |
| ZTE, Sanechips | We think the power model in TR.38.840 can be reused as starting point. |
| Qualcomm | OK with the approach also taking into account the updated values from the excel sheet. |

Moderator

Given companies’ additional inputs, moderator suggests to consider the updated average values.

| Company | 2Tx @0dBm | 2Tx @23dBm | 2Tx @26dBm (Z9) | Notes |
| --- | --- | --- | --- | --- |
| Huawei |  | 1 | 1 | No scalilng or TX number for 23/26dBm, since PA is the dominate part |
| MediaTek | 2 | 2 | 2 |  |
| Qualcomm | 1.4 | 1.2 | 1.6 |  |
| Spreadtrum | 1.4 | 1.2 |  | Reuse scaling rule in TR 38.840 |
| Xiaomi | 1.4 | 1.2 |  |  |
| Avg(all) | 1.55 | 1.32 | 1.533 |  |

Proposal 4.1.3-2a: (2nd Round; High) UL Antenna Scaling (2Tx vs 1Tx)

Include the following UL antenna scaling factors (2Tx relative to 1Tx):

| TX Power | Scaling Factor |
| --- | --- |
| 0 dBm | [1.5] |
| 23 dBm | [1.3] |
| 26 dBm | [1.5] |

| Company Name | Comments/Views |
| --- | --- |
| Huawei, HiSilicon | After some further internal check, we changed our mind. Though the power consumption of UL is dominated by the PA part (especially for high transit power), the PA efficiency difference should be considered. For example, to transmit 26dBm in total, the PA efficiency of 1TX (26dBm) is higher than that of 2TX (23dBm*2). This impact should not be ignored. A typical PA efficiency curve is shown above, based on which, the scaling factors are {2, 1.6, 1.6} for {0dBm, 23dBm, 26dBm}. Please the FL to take into account. Sorry again for this mistake I made before. |

    - PDCCH Related Scaling

      - Detailed Company Contributions (Collapsed; Please Expand to View)

PDCCH Processing Scaling

| Source | TDoc ID | Content Summary |
| --- | --- | --- |
| Xiaomi | R1-2600427 | Proposes cross-slot scheduling scaling factor of 0.7x same-slot scheduling (applicable for FR1 and FR2), reusing TR 38.840 scaling rules |
| Qualcomm | R1-2601271 | **Proposal 9:** Cross-slot PDCCH power = 0.7x same-slot (FR1/FR2); **Proposal 10:** PDCCH symbol duration scaling: 0.9x for 1 symbol, 1.1x for 3 symbols; **Proposal 11:** Candidate reduction scaling P(α) = α·Pt + (1-α)·0.95Pt (updated from NR); **Proposal 12:** Include effects of both increasing and decreasing PDCCH blind decodes/CCEs on UE power |
| ZTE/Sanechips | R1-2600456 | **Proposal 18:** Reduced scheduled duty cycle should exclude PDCCH monitoring occasion reduction (avoid double counting); **Proposal 19:** Include processing relaxing technique in reduced scheduled duty cycle; **Proposal 21:** PDCCH candidate scaling P(d) = {Pmicro-sleep, c=0; c×Pt +0.7×(1-c)×Pt, c>0} where c = C_use/C_ref; **Proposal 22:** Multi-state slot power P = Σ(i=1 to 14) P(i) |
| Lenovo | R1-2601106 | **Proposal 6:** Define CORESET/search space configuration for BW scaling scenarios of PDCCH-only state; **Proposal 7:** For same BW, reuse NR model P(α) = max(Micro-sleep, α·Pt + (1-α)·0.7Pt); discusses two BW scaling scenarios (scaled candidates vs. increased CORESET duration) |
| NTT DOCOMO | R1-2601179 | **Observation 2:** TR 38.840 power model inadequate for evaluating variety of PDCCH monitoring configurations (CORESET size, CCEs/BDs); **Proposal 9:** Study extending NR UE power consumption scaling w.r.t. PDCCH monitoring configuration; highlights need for multi-carrier PDCCH enhancement evaluation |
| CAICT | R1-2601323 | **Proposal 3:** Specify two-stage PDCCH with low-power pre-indication before full control decode; OFDM-based WUS design can be considered; discusses LP-WUS as potential C-DRX replacement for maximum energy saving |

      - Company Views Summary

| Company | Cross-slot Factor | Symbol Count (1/2/3-sym) | BD/CCE Coeff. (k) | TDoc |
| --- | --- | --- | --- | --- |
| NTT DOCOMO | 0.8 | - | - | R1-2601179 |
| Qualcomm | 0.7 | 0.9 / 1.0 / 1.1 | 0.95 | R1-2601271 |
| Xiaomi | 0.7 | - | - | R1-2600427 |
| ZTE | - | - | 0.7 | R1-2600456 |
| Lenovo | - | - | 0.7 | R1-2601106 |
| Avg(All) | 0.73 | 0.9 / 1.0 / 1.1 | 0.78 | - |

Summary

For cross-slot scheduling, Qualcomm and Xiaomi propose 0.7x of same-slot scheduling power (reusing TR 38.840), while NTT DOCOMO proposes 0.8x. The Avg(All) is 0.73. For PDCCH symbol duration scaling, only Qualcomm proposes differentiation: 0.9x for 1-symbol, 1.0x for 2-symbol (reference), 1.1x for 3-symbol PDCCH, reflecting microsleep duration opportunities.

For PDCCH candidate/BD reduction, three companies propose scaling formulas with coefficient k: ZTE (k=0.7) and Lenovo (k=0.7) maintain the NR legacy value from TR 38.840, while Qualcomm proposes k=0.95, arguing the NR model overestimates BD reduction impact. The Avg(All) k=0.78. The general formula is P(a) = a x Pt + (1-a) x k x Pt, where a = C_use/C_ref. Lenovo additionally proposes a floor function: P(a) = max(P_microsleep, a x Pt + (1-a) x k x Pt).

Proposal 4.1.4-1: (1st Round; High) Cross-Slot Scheduling Scaling

For cross-slot scheduling, PDCCH power = [0.75] × same-slot PDCCH power (Avg(All)). Applicable for FR1 and FR2..

| Company Name | Comments/Views |
| --- | --- |
| Huawei, HiSilicon | OK. But to clarify, the ‘cross-slot scheduling’ here means UE knows it will not be scheduled with K0=0 (e.g., by configuring all K0>0, or indicated with K0min>0 in R16 manner) |
| ZTE, Sanechips | OK |
| Spreadtrum | We prefer to reuse 5G scaling factor, e.g., 0.7. |
| Xiaomi | Suggest to reuse NR value, i.e., 0.7x. The reasons for increasing this energy consumption scaling value need to be clarified and achieve consensus. Otherwise, the 5G values should be reused |
| Qualcomm | We prefer the 0.7 scaling factor which is the same as NR and seems to also be reasonable from an implementation perspective. |
| Samsung | Generally OK |

Moderator

Suggest to reuse NR 0.7 scaling

Proposal 4.1.4-1a: (2nd Round; High; Stable) Cross-Slot Scheduling Scaling

For cross-slot scheduling, PDCCH power = 0.7 × same-slot PDCCH power. Applicable for FR1 and FR2..

| Company Name | Comments/Views |
| --- | --- |
| Huawei, HiSilicon | OK |
| Qualcomm | OK with proposal |

Proposal 4.1.4-2: (1st Round  2nd Round; Medium) PDCCH Symbol Duration Scaling

Include PDCCH symbol duration scaling:

| PDCCH Symbols | Scaling Factor |
| --- | --- |
| 1 symbol | [0.9] |
| 2 symbols | 1.0 (reference) |
| 3 symbols | [1.1] |

| Company Name | Comments/Views |
| --- | --- |
| Huawei, HiSilicon | We don’t see the need to define this model. |
| ZTE, Sanechips | If we define this for PDCCH, also there is a need for all the other channels. Otherwise, all the channels are not scaled within one slot. |
| Spreadtrum | We share the same view with Huawei. |
| Qualcomm | We support the proposal and think it is important to capture, especially since the PDCCH control region will be discussed including the duration. The number of PDCCH symbols directly impacts microsleep duration in a slot and this was assumed in the 38.840 power model. The more PDCCH symbols, the less microsleep state and the higher the slot power. |
| Samsung | Same view with Huawei |
| Nokia2 | We are not sure if this model is needed. |

Proposal 4.1.4-3: (1st Round; High) BD/CCE Reduction Scaling

Adopt power scaling formula for PDCCH candidate/BD reduction:

P(α) = max(P_microsleep, α × P_t + (1 − α) × [0.75] × P_t)

where α = configured #BD candidates / maximum #BD condidates.

| Company Name | Comments/Views |
| --- | --- |
| Huawei, HiSilicon | We share the similar view as QC that the BD/CCE impact is not too much. We also think 0.95 should be a better value mapping to current UE implementation. |
| CMCC | Fine for the structure, and okay to further discuss the specific value in the bracket. |
| ZTE, Sanechips | Ok |
| Ericsson | We do not see any need to update the existing scaling factor from TR 38.840. Note that the existing scaling was also used in Rel-17 for RedCap evaluations. |
| Spreadtrum | We prefer to reuse 5G scaling factor, e.g., 0.7. |
| CEWiT | Fine with the proposal, and okay to further discuss the value in the bracket. |
| DCM | We are generally fine with the proposal and open to discussing the specific value in brancket. |
| Xiaomi | OK. We understand this update is necessary, and the power for PDCCH-only shall not be lower than that of micro-sleep state. |
| Samsung | We also prefer to reuse legacy scaling factor. |
| Nokia2 | While the 6GR PDCCH design is open the BD adaptation does not provide too high benefit, thus tend to agree with QCM and Huawei. |

Moderator

Consider a compromised value between 0.7 and 0.95  [0.8]

Proposal 4.1.4-3a: (2nd Round; High) BD/CCE Reduction Scaling

Adopt power scaling formula for PDCCH candidate/BD reduction:

P(α) = max(P_microsleep, α × P_t + (1 − α) × [0.8] × P_t)

where α = configured #BD candidates / maximum #BD candidates.

| Company Name | Comments/Views |
| --- | --- |
| Qualcomm | We prefer to use a higher value than 0.8. BD/CCE impact on power is not much. We also think 0.95 should be a better value mapping to current UE implementation. We can compromise to 0.9 |

    - Sleep Related Scaling

      - Detailed Company Contributions (Collapsed; Please Expand to View)

Other DL Processing / Micro Sleep Scaling

| Source | TDoc ID | Content Summary |
| --- | --- | --- |
| Nokia | R1-2600030 | For T (Ncntrl=0): 20%→0.15, 100%→{0.19, 0.16}, 200%→{0.26, 0.19}, 400%→{0.48, 0.26, 0.19}. |
| Spreadtrum/UNISOC | R1-2600110 | Simple scaling rule for micro-sleep. Reuse legacy 45 power units for 100MHz, redefine X units for 200MHz. |
| OPPO | R1-2600197 | Other DL signal/channel processing: For T: 5%→0.35, 20%→0.5, 100%→1, 200%→1.5, 400%→3. For Tmin: add S_MaxBW. |
| CATT | R1-2600294 | Scaling rule for micro sleep should be considered. Since micro-sleep requires UE ready for any non-sleep state in next slot, it's related to BW adaptation scaling rules. For 200MHz vs 100MHz, less components can be turned off (P#1>P#2). Exact values need discussion. |
| Xiaomi | R1-2600427 | Other DL: same as PDCCH-only (5%→0.5, 20%→0.6, 100%→1, 200%→1.7, 400%→3.4). Micro-sleep: scale by 0.4 (≤20MHz), 1 (20-160MHz), 1.7 (≥160MHz). Rationale: less components can be turned off for 200MHz than 100MHz during micro-sleep. Match NR RedCap model for ≤20MHz. |
| ZTE/Sanechips | R1-2600456 | Reuse scaling factor for PDCCH+PDSCH or PDCCH-only for other DL signal/channel processing. Since power similar to PDCCH-only or PDCCH+PDSCH states, reuse for simplicity. |
| vivo | R1-2600502 | Other DL: For T: 5%,20%→0.7, 100%→1, 200%→1.7, 400%→3.4. For Tmin: add delta+SMaxBW. Micro-sleep: same scaling as PDCCH-only for simplicity. T=5-10ms, Tmin=1-2ms. |
| Samsung | R1-2600754 | Micro-sleep: follow PDCCH-only scaling table without additional factors. Since zero transition time, UE can't adjust clock/voltage. BW dependency inherited from PDCCH HW configuration. Additional scaling factors (Z3, Z4) add complexity without significant accuracy improvement. Simple, tractable model. |
| Apple | R1-2600826 | Other DL (Rx open full slot, should >PDCCH-only): Tmin only. 5%,20%→0.85, 100%→1, 200%→1.2, 400%→1.8 assuming 2x200MHz. |
| Ericsson | R1-2600960 | Other DL signals (e.g., sync signal): power depends on design; BW scaling may not apply for fixed BW transmissions. |
| Google | R1-2601205 | Adopt scaling rule for Micro-sleep as function of both gap duration and target active BW, reflecting standby power for wideband readiness. Current model assumes fixed 45. Short gap→light micro-sleep (can't turn off LO/flush pipeline). Long gap→deeper micro-sleep. 400MHz readiness requires more standby current than 20MHz. |
| MediaTek | R1-2601240 | Other signal processing: For T=5ms: 5%→0.35, 20%→0.4, 100%→1, 200%→1.7, 400%→same as Tmin. For Tmin=2ms: add S_MaxBW, 200%→2, 400%→5.6. S_MaxBW=0.05 (200MHz), 0.2 (400MHz). |
| Qualcomm | R1-2601271 | Other DL: reuse PDCCH-only scaling for ΓTput=1. Micro-sleep: For T: 5%,20%→0.6, 100%→1, 200%→{1.7, 1.4}, 400%→{3.0, 2.4, 2.0}. For Tmin: 5%,20%→1, 100%→1. Zero transition time→insufficient to change clock/voltage. Scale both RF and BB components with sub-linear exponent for >100MHz. |

      - Company Views Summary

| Company | Microsleep BW Scaling Approach | <=20 MHz | 20-160 MHz | >=160 MHz | TDoc |
| --- | --- | --- | --- | --- | --- |
| Xiaomi | BW-dependent thresholds | 0.4 | 1.0 | 1.7 | R1-2600427 |
| Samsung | Follow PDCCH-only table | - | - | - | R1-2600754 |
| vivo | Follow PDCCH-only table | - | - | - | R1-2600502 |
| Google | f(gap duration, target BW) - FFS | - | - | - | R1-2601205 |
| Spreadtrum | Reuse 45 units @100MHz, redefine for 200MHz | - | - | - | R1-2600110 |
| Qualcomm | Separate table with sub-linear >100MHz | 0.6 | 1.0 | 1.7 | R1-2601271 |
| Avg(All) |  | 0.5 | 1.0 | 1.7 | - |

Summary

The microsleep scaling debate centers on whether microsleep should follow the PDCCH-only scaling table or have separate BW-dependent values. Samsung, vivo, ZTE, and Qualcomm (for other DL) support reusing the PDCCH-only table for simplicity, with Samsung providing technical justification: "zero transition time means UE cannot adjust clock/voltage; BW dependency inherited from PDCCH HW configuration." Xiaomi proposes BW-threshold approach (0.4 for <=20 MHz, 1.0 for 20-160 MHz, 1.7 for >=160 MHz). Google proposes the most sophisticated model as a function of both gap duration and target active BW, noting 400 MHz readiness requires more standby current than 20 MHz.

      - Proposals

Proposal 4.1.5-1: (1st Round; High) Microsleep Scaling Approach

Adopt PDCCH-only scaling table for microsleep BW dependency.

| Company Name | Comments/Views |
| --- | --- |
| Huawei, HiSilicon | We support to define the scaling rule for micro-sleep. Depending on the outcome of PDCCH-only scaling with BW, we can further discuss this part. From our view, different scaling factor can be defined with different BW adaptation delay T and Tmin. In addition, light sleep may also be scaled. The same scaling factor can be used for both micro-sleep and light-sleep. |
| OPPO | OK |
| ZTE, Sanechips | Micro sleep power is not related to bandwidth for one UE. For different UE types with different bandwidth, it could be further discussed. |
| Spreadtrum | We think PDCCH-only scaling table is too complexity and there is no relationship between PDSCH throughput and micro sleep. We prefer a simple scaling rule for micro sleep, e.g., 2 power consumption value for 100MHz and 200MHz respectively. |
| Qualcomm | OK with methodology |
| CATT | OK |

Moderator

Suggestion (joint consideration with bandwidth scaling): (1) ΓTput = 1 only for other DL signals/channels; (2) Simplify the table by merging 5% & 20% → ≤ 20% and 200% & 400% → ≥ 200%; (3) Keep T > Tmin sub-table for ≤ 20% only.

Proposal 4.1.5-1a: (2nd Round; High)

- Include scaling table for micro sleep BW dependency.

| ΓB | Scaling factor |
| --- | --- |
| <= 20% | [0.55] |
| 100% | 1.00 |
| >= 200% | [2.40] |

- Additional scaling is included for ΓB≤ 20% and 5 ms≤T≤10 ms

| ΓB | Scaling factor when adaptation delay T>Tmin is assumed |
| --- | --- |
| <=20% | 0.4 |

- DL antenna scaling is also applicable to micro-sleep power

- FFS: The above scaling rules also applicable to light sleep

| Company Name | Comments/Views |
| --- | --- |
| Huawei, HiSilicon | We are OK with this direction. But it seems the S_maxBW is missing in this case. To align the logic with PDCCH-only and PDCCH+PDSCH, it is better to also give two options on the S_maxBW for micro-sleep BW scaling. |
| Qualcomm | OK, values can be further discussed |

  - Power States and Relative Power Values

For parameters where companies propose different numerical values reflecting different implementation approaches, Avg(All) values are recommended for discussion and convergence. Since 3GPP does not mandate specific implementations, the power model should be implementation-agnostic. Avg(All) captures the diversity of valid industry approaches across different architectures, process nodes, and design choices.

    - UL Power Values

      - Detailed Company Contributions (Collapsed; Please Expand to View)

UL Transmission Power Levels

| Source | TDoc ID | Content Summary |
| --- | --- | --- |
| Huawei/HiSilicon | R1-2600142 | **Proposal 7:** Suggested values X4=420, X5=490, X6=620, X7=920. Based on PA efficiency model where power = Poutput/η + PBB. BB:RF split is 2:3 at 0dBm (100:150 units). Uses PA-centric approach with fixed baseband overhead. |
| OPPO | R1-2600197 | **Proposal 9:** TX power values X4=275, X5=330, X6=475, X7=1150. Equal scaling approach: distributes 450-unit gap (from 250@0dBm to 700@23dBm) proportionally across power levels. |
| ZTE/Sanechips | R1-2600456 | **Proposal 16:** X4=22.5, X5=71.15, X6=225, X7=900. Linear interpolation in linear scale based on reference points 0dBm (250 units) and 23dBm (700 units). |
| Samsung | R1-2600754 | **Proposal 6:** X4=450 (10dBm), X5=560 (15dBm), X6=650 (20dBm), X7=1020 (26dBm). Acknowledges non-linear scaling due to PA efficiency variation, output power back-off, and RF-chain overhead. |
| Apple | R1-2600826 | **Proposal 8:** Power values 300 (10dBm), 380 (15dBm), 480 (20dBm), 1150 (26dBm). Presented in table format with anchor points 250@0dBm and 700@23dBm. |
| Ericsson | R1-2600960 | **Observation 11 & Proposal 15:** X4=325 (10dBm, 10% PA eff.), X5=380 (15dBm, 20% PA eff.), X6=540 (20dBm, 30% PA eff.), X7=980 (26dBm, 50% PA eff.). Linear interpolation/extrapolation with explicit PA efficiency modeling (5% at 0dBm to 50% at 26dBm). **Proposal 12:** UE power model should include 26dBm for 7GHz coverage requirements per SID objective. |
| Lenovo | R1-2601106 | **Proposal 2:** X4=280, X5=350, X6=500, X7=1100. PA-efficiency-based interpolation with η=10% (0dBm), 35% (23dBm), 38% (26dBm). Assumes η scales linearly in dB as function of Tx power. Static/BB components don't scale with Tx power. |
| MediaTek | R1-2601240 | **Proposal 9 & Observation 11:** X4=260, X5=250, X6=650, X7=1650. Accounts for voltage gear quantization (finite voltage levels). Power proportional to square of voltage; highest TX power shows significantly higher consumption. Typical PA efficiency 30-40% in FR1. |
| Qualcomm | R1-2601271 | **Proposal 17:** X4=496 (10dBm), X5=513 (15dBm), X6=644 (20dBm), X7=770 (26dBm). Presented in table format including FR1 and FR2 columns. |

      - Company Views Summary

| Company | X4 (10 dBm) | X5 (15 dBm) | X6 (20 dBm) | X7 (26 dBm) | Methodology | TDoc |
| --- | --- | --- | --- | --- | --- | --- |
| Apple | 300 | 380 | 480 | 1150 | Empirical | R1-2600826 |
| Ericsson | 325 | 380 | 540 | 980 | PA efficiency model | R1-2600960 |
| Huawei | 420 | 490 | 620 | 920 | PA efficiency model | R1-2600142 |
| Lenovo | 280 | 350 | 500 | 1100 | PA efficiency interp. | R1-2601106 |
| MediaTek | 260 | 350 | 650 | 1650 | Voltage gear quantiz. | R1-2601240 |
| OPPO | 275 | 330 | 475 | 1150 | Equal scaling | R1-2600197 |
| Qualcomm | 496 | 513 | 644 | 770 | Empirical | R1-2601271 |
| Samsung | 450 | 560 | 650 | 1020 | PA efficiency + NL | R1-2600754 |
| ZTE | 22.5 | 71 | 225 | 900 | Linear interp. (linear) | R1-2600456 |
| Avg(All) | 314 | 380 | 532 | 1071 |  | - |

Summary

Nine companies submitted UL transmission power values for X4-X7, building on agreed anchor points of 250 (0 dBm) and 700 (23 dBm). Different implementation approaches yield a range of values, particularly for X4 (10 dBm) ranging from 22.5 (ZTE) to 496 (Qualcomm). Five companies (Huawei, Ericsson, Lenovo, MediaTek, Samsung) explicitly use PA efficiency-based modeling (P = P_output/eta + P_BB), recognizing non-linear scaling. ZTE's values reflect a linear-domain interpolation methodology. MediaTek's X7=1650 reflects voltage gear quantization characteristics at high TX power. All nine companies include X7 values for 26 dBm to support 7 GHz coverage per SID objectives.

It is recommend Avg(All) values for UL power levels, which are inclusive of all methodologies (PA efficiency-based, measurement-based, linear interpolation). This approach captures implementation diversity without mandating a specific methodology, consistent with 3GPP principles.

      - Proposals

Proposal 4.2.1-1: (1st Round; High) UL Power Values

Include the following UL long PUCCH/PUSCH/PRACH power values in the UE power model:

| TX Power | Relative Power |
| --- | --- |
| 0 dBm | 250 (agreed) |
| 10 dBm (X4) | [310] |
| 15 dBm (X5) | [380] |
| 20 dBm (X6) | [530] |
| 23 dBm | 700 (agreed) |
| 26 dBm (X7) | [1070] |

| Company Name | Comments/Views |
| --- | --- |
| Huawei, HiSilicon | We are OK to use the average value from companies. But ZTE’s input seems to be strange. Their values for 10/15/20dBm are even smaller than 0dBm. A correction from ZTE is needed first. |
| OPPO | We are fine to accept the proposal. |
| ZTE, Sanechips | Sorry for the confusion provided in the Tdoc for these values. The values should be updated by adding 250, which corresponds to the value at 0 dBm. The correct values should be 272.5@10dBm, 321@15dBm, 475@20dBm, 1150@26dBm. |
| Ericsson | Fine with the proposal. |
| DCM | We are fine with the proposal. |
| Qualcomm | OK to use the average, but it should be based on the excel sheet updated numbers collected by the FL. |
| Samsung | OK |
| CATT | OK |

Moderator

There is good consensus here, and moderator apply the latest average value based on company’s updates.

Proposal 4.2.1-1a: (2nd Round; High; Stable) UL Power Values

Include the following UL long PUCCH/PUSCH/PRACH power values in the UE power model:

| TX Power | Relative Power |
| --- | --- |
| 0 dBm | 250 (agreed) |
| 10 dBm (X4) | [310] |
| 15 dBm (X5) | [380] |
| 20 dBm (X6) | [530] |
| 23 dBm | 700 (agreed) |
| 26 dBm (X7) | [1080] |

| Company Name | Comments/Views |
| --- | --- |
| Huawei, HiSilicon | OK |
| Qualcomm | Ok with the proposal |

    - EE Processing Power Values

      - Detailed Company Contributions (Collapsed; Please Expand to View)

EE Processing State

| Source | TDoc ID | Content Summary |
| --- | --- | --- |
| Nokia | R1-2600030 | **Proposes 2RX power values:** Micro: 14 (10×1.4), Light: 17 (12×1.4), Deep/Ultra-deep: 21 (15×1.4). Also proposes 4RX values: Micro: 27 (10×2.7), Light: 32 (12×2.7), Deep/Ultra-deep: 40 (15×2.7). **Observation 7:** WUS may benefit from SSS reception for CFO correction and timing sync. **Proposal 15:** Consider CFO inaccuracy of ~1ppm with SSS availability. **Observation 8:** Number of RX chains depends on underlying sleep state; restricted to 1RX for ultra-deep/deep sleep. **Proposal 16:** Minimum RX chains for 6G WUS depends on RRC state and frequency range. |
| Spreadtrum/UNISOC | R1-2600110 | **Proposal 5:** For 2RX, power values are 14, 17, and 21 for micro, light, and other sleep. **Proposal 6:** Fix one slot reception time for evaluation; detailed design for WI phase. |
| Huawei/HiSilicon | R1-2600142 | **Proposal 6:** Suggests different values based on BW and RX: For 5MHz 2RX: {13, 16, 20} for {micro, light, deep}. For 10MHz 1RX: {11, 12, 16} for {micro, light, deep}. |
| OPPO | R1-2600197 | **Proposal 9:** For 2RX, factors are X1=14, X2=17, X3=21. Also provides UL power values: X4=275, X5=330, X6=475, X7=1150. |
| CATT | R1-2600294 | **Proposal 13:** Power values are around 14, 17, and 21 for 2-RX during micro, light, and deep/ultra-deep sleep. Based on antenna scaling factor 0.7 from 4-Rx to 2-Rx (implying 1.4x from 1-Rx to 2-Rx), calculated as 10/0.7, 12/0.7, 15/0.7. |
| CMCC | R1-2600387 | **Agrees with baseline EE Processing characteristics** (5MHz BW, 1-RX/2-RX, residue CFO up to 5ppm, residue timing offset up to 2us, NF=6GR UE NF, reception time up to one slot, additional energy overhead 15, ramp time 2ms). FFS on 2-RX power values X1, X2, X3. |
| Xiaomi | R1-2600427 | **Proposal 13:** Adopt linear scaling for different reception time within a slot. **Proposal 14:** For 2Rx, adopt X1=13 (micro), X2=15 (light), X3=18 (other). Rationale: Use antenna scaling factor <0.7 considering EE processing has lower complexity than MR. **Proposal 15:** Determine number of sync signals for sync/re-sync with candidate values 5 and 10. |
| ZTE/Sanechips | R1-2600456 | **Proposal 12:** Update Noise Figure assumption to NF = 6GR UE NR + 2dB. Rationale: Current NF same as baseline cannot explain power reduction from 100 (SSB measurement) to 10-15 (EE processing). **Proposal 13:** No scaling for different reception time within one slot (for simplification). **Proposal 14:** 2Rx power = 1Rx power × 1.5 (as starting point). **Proposal 15:** If RRM measurements in EE processing, use scaling factors from TR 38.840: sync 4 cells P×1.2, sync 8 cells P×1.5, async 4 cells P×1.4, async 8 cells P×1.7. |
| vivo | R1-2600502 | **Proposal 2:** For 2-Rx, proposes X1=12 (micro), X2=14.4 (light), X3=18 (other). Rationale: Scaling factor 1.2 (smaller than 1.4) because BB processing for DL WUS is simpler than non-EE processing. **Proposal 3:** For 10MHz BW, same values X1=12, X2=14.4, X3=18. Rationale: 10MHz needed for large coverage; scaling factor 1.2 from 5MHz to 10MHz. |
| Ofinno | R1-2600600 | **Agrees with baseline EE Processing.** **Observation 3:** UE consumes different power for different DL WUS symbol lengths. **Proposal 1:** Relative power for EE processing (1-RX) should be {8, 10, 12} when DL WUS length is {2, 4, 8} symbols. **Proposal 2:** Discuss whether [2]ms ramp-down after EE processing is in addition to ramp-up time when DL WUS indicates wake up. |
| Samsung | R1-2600754 | **Agrees with baseline EE Processing.** **Observation 8:** 1-RX to 2-RX scaling factor of 0.7 from TR 38.875 provides reasonable basis. **Proposal 4:** For 2-RX, adopt X1=15, X2=18, X3=22. **Proposal 5:** For reception time <1 slot, calculate relative power as: N/14 × P1 + P2, where N=number of OFDM symbols, P1=EE processing power per slot, P2=sleep state power per slot. |
| Ericsson | R1-2600960 | **Agrees with baseline EE Processing.** **Proposal 13:** For 2-Rx in FR1, based on TR 38.840 antenna scaling (2-Rx = 1.4× 1-Rx): X1=14, X2=17, X3=21. **Proposal 14:** For FR2, proposes values for 1Rx and 2Rx: D1=30 (micro, 1Rx), D2=42 (micro, 2Rx), D3=36 (light, 1Rx), D4=50 (light, 2Rx), D5=45 (other, 1Rx), D6=63 (other, 2Rx). Based on PDCCH-only 175 units (2Rx, 100MHz) scaled to 20MHz. |
| Lenovo | R1-2601106 | **Proposal 1:** Adopt X1=15, X2=18, X3=22 for 2-RX in FR1. Based on antenna-scaling 0.7 for 4RX→2RX (FR1) and 2RX→1RX (FR2), implying 1/0.7≈1.43 from 1RX to 2RX. |
| Google | R1-2601205 | **Observation 2:** EE processing power is correlated with configured monitoring bandwidth of Wake-Up Signal. **Proposal 2:** Relative power for EE processing should apply bandwidth scaling factor proportional to active RF bandwidth required for WUS monitoring. Rationale: Power depends on active RF bandwidth; fixed value underestimates wideband WUS or overestimates narrowband. |
| MediaTek | R1-2601240 | **Proposal 8:** For EE Processing, proposes 2RX = 2× 1RX: Micro (10→20), Light (12→24), Other (15→30). **Observation 10:** EE Processing with 1Rx is optimized low-power path; 2Rx requires double hardware (not flexible switching), so double power consumption. |
| Qualcomm | R1-2601271 | **Agrees with baseline EE Processing.** **Proposal 15:** For 2-RX, X1=12.5, X2=15, X3=17. **Proposal 16:** Relative power for WUS with different reception time can be scaled from agreed values. Example: 7-symbol WUS → X1=5, X2=6, X3=7.5 (with remaining symbols at sleep power). |

      - Company Views Summary

| Company | X1 (Micro sleep) | X2 (Light sleep) | X3 (Other sleep) | Scaling Factor | TDoc |
| --- | --- | --- | --- | --- | --- |
| CATT | 14 | 17 | 21 | 1.4x | R1-2600294 |
| Ericsson | 14 | 17 | 21 | 1.4x | R1-2600960 |
| Huawei | 13 | 16 | 20 | ~1.3x | R1-2600142 |
| Lenovo | 15 | 18 | 22 | 1.43x | R1-2601106 |
| MediaTek | 20 | 24 | 30 | 2.0x | R1-2601240 |
| Nokia | 14 | 17 | 21 | 1.4x | R1-2600030 |
| OPPO | 14 | 17 | 21 | 1.4x | R1-2600197 |
| Qualcomm | 12.5 | 15 | 17 | ~1.25x | R1-2601271 |
| Samsung | 15 | 18 | 22 | 1.5x | R1-2600754 |
| Spreadtrum | 14 | 17 | 21 | 1.4x | R1-2600110 |
| vivo | 12 | 14.4 | 18 | 1.2x | R1-2600502 |
| Xiaomi | 13 | 15 | 18 | <1.4x | R1-2600427 |
| ZTE | 15 | 18 | 22.5 | 1.5x | R1-2600456 |
| Avg(All) | 14.27 | 17.18 | 21.12 |  | - |

Summary

Universal consensus (16 companies) exists on 1-RX baseline power values: 10 (micro sleep), 12 (light sleep), 15 (deep/ultra-deep sleep), with configuration parameters including 5 MHz BW, 5 ppm CFO, 2 us timing offset, and 2 ms ramp time.

For 2-RX power values, 13 companies provide proposals with 7 distinct scaling approaches reflecting different implementation characteristics. One approach (1.4x yielding 14/17/21) is proposed by Nokia, Spreadtrum, OPPO, CATT, Ericsson based on TR 38.840 antenna scaling. Samsung and Lenovo propose 1.43-1.5x (15/18/22). Lower values from Xiaomi (13/15/18) and vivo (12/14.4/18) argue EE processing has lower complexity. MediaTek proposes 2.0x (20/24/30), reflecting a separate-chain architecture. The Avg(All) values (X1=14.27, X2=17.18, X3=21.12) capture the full range of implementation approaches without favoring any specific architecture.

      - Proposals

Proposal 4.2.2-2: (1st Round; High) EE Processing 2-RX Values

For EE Processing 2-RX in FR1, adopt the following values:

| Sleep State | 2-RX Power |
| --- | --- |
| Micro sleep (X1) | [14] |
| Light sleep (X2) | [17] |
| Deep/Ultra-deep (X3) | [21] |

Moderator note: The Avg(All) values are inclusive of all 13 company proposals spanning scaling factors from 1.2x to 2.0x.

| Company Name | Comments/Views |
| --- | --- |
| Huawei, HiSilicon | We are OK. Besides, additional scaling on EE processing with 10MHz&1RX should also be defined, since increasing BW is an efficient way to enhance the coverage performance. |
| OPPO | Agree. |
| ZTE, Sanechips | The scaling factor between 2Rx and 1Rx is OK for us. But we would question the feasibility of the relative power for 1Rx. The relative powers of EE processing in FR1 are 10/12/15 with the same Noise Figure (NF) as 6GR UE NR. However, the relative power of SSB measurement is 100 with same NF assumption, which is much larger than the power consumption of EE processing. In generally, the BB part consumes 30~40% power of the receiver power. Assuming the noise figure is the same and the RF front-end is shared, the reduced power mainly comes from BB processing relaxing. Even the entire BB power consumption (up to ~40%) is eliminated, it is not feasible to reduce the relative power from 100 down to 10~15. Thus, it is unclear whether EE processing can realistically achieve the assumed relative power value, and this should be further discussed. We think the noise figure assumption of EE processing, or the power values of EE processing for 1Rx should be updated. |
| Ericsson | Fine with the proposal. Note that scaling is also needed for FR2 (we provided estimates in our Tdoc R1-2600960). |
| Spreadtrum | Support |
| DCM | We are fine with the proposal. |
| Xiaomi | The antenna scaling methodology (0.7x) specified in TR 38.840 may be used to derive power values for 2 Rx. However, we note that EE processing entails lower complexity than MR, so a value slightly smaller than that obtained by dividing by 0.7 can be considered. |
| Nokia.1 | Agree with FL proposal. |
| Qualcomm | Support Huawei, HiSilicon to include the 10 MHz. There are multiple proposals in the DL WUS design agenda item to study BW > 5 MHz. |
| Samsung | Fine |
| CATT | OK with the values in this proposal. |

Moderator

Strong convergence on [14, 17, 21]. The 1.4x scaling factor from TR 38.840 has the widest support. Include FFS for 10MHz to address Huawei and Qualcomm suggestions.

Proposal 4.2.2-2a: (2nd Round; High; Stable) EE Processing 2-RX Values

- For EE Processing 2-RX in FR1, adopt the following values:

| Sleep State | 2-RX Power |
| --- | --- |
| Micro sleep (X1) | [14] |
| Light sleep (X2) | [17] |
| Deep/Ultra-deep (X3) | [21] |

- FFS: Power values for 10MHz BW

| Company Name | Comments/Views |
| --- | --- |
| Huawei, HiSilicon | OK with the values of 2RX. We also strongly support the FFS on 10MHz, to leave the door open for different methods to enhance the coverage performance. It is better to align some value for 10MHz case in this meeting. If not, we can also accept to discuss it in the next meeting. |
| Qualcomm | OK with the proposal. We also share similar views with Huawei, we should have a value for the 10 MHz case |

    - New Active Power State(s)

      - Detailed Company Contributions (Collapsed; Please Expand to View)

Active State Definitions

| Source | TDoc ID | Content Summary |
| --- | --- | --- |
| Xiaomi | R1-2600427 | **Proposal 12:** No additional power states needed. Rationale: All UE operation scenarios already covered (data channels, control channels, reference signals, low-power receiver processing). Individual signal variants can use scaling methods. |
| ZTE/Sanechips | R1-2600456 | **Proposal 11:** PDSCH-only power state from TR 38.840 should be introduced in 6G UE power models. Rationale: Reducing PDCCH monitoring occasions is crucial for power saving and results in PDSCH-only state. |
| Ofinno | R1-2600600 | **Agrees with baseline active states:** PDCCH-only (FR1: 100, FR2: 175), SSB or CSI-RS proc. (FR1: 100, FR2: 175), PDCCH + PDSCH (FR1: 300, FR2: 350). |
| Samsung | R1-2600754 | **Agrees with baseline active states:** PDCCH-only (FR1: 100, FR2: 175), SSB or CSI-RS proc. (FR1: 100, FR2: 175), PDCCH + PDSCH (FR1: 300, FR2: 350). |
| Apple | R1-2600826 | **Agrees with baseline active states:** PDCCH-only (FR1: 100, FR2: 175), SSB or CSI-RS proc. (FR1: 100, FR2: 175), PDCCH + PDSCH (FR1: 300, FR2: 350). |
| LG Electronics | R1-2600553 | **Proposal #3:** Introduce baseband-only PDSCH processing power state for slots where PDSCH is not scheduled (RF in micro sleep), to evaluate relaxed PDSCH baseband processing. Proposes new state with relative power X8 (FR1) and X9 (FR2). |
| Qualcomm | R1-2601271 | **Agrees with baseline active states.** **Proposal 14:** Specify BB Processing only state for processing PDSCH or PDCCH+PDSCH: For ΓB=200MHz, ΓTput=1/2: Relative Power 190. For ΓB=400MHz, ΓTput={1/2, 1/4}: Relative Power 460, 220. Rationale: With reduced scheduled duty cycle and N1 timeline relaxation, UE can keep BB at lower power; RF in microsleep while BB processes previously scheduled PDSCH. |

      - Company Views Summary

| Company | Position on New States | Proposed Values | TDoc |
| --- | --- | --- | --- |
| Xiaomi | No additional states needed | All scenarios covered by existing states + scaling | R1-2600427 |
| ZTE | Add PDSCH-only state (from TR 38.840) | Reducing PDCCH MOs creates PDSCH-only slots | R1-2600456 |
| LG Electronics | Add BB-only PDSCH processing state | X8 (FR1), X9 (FR2), RF in microsleep | R1-2600553 |
| Qualcomm | Add BB Processing only state | 190 (200MHz, 1/2); 460/220 (400MHz, 1/2, 1/4) | R1-2601271 |
| Ofinno | Agree with baseline | - | R1-2600600 |
| Samsung | Agree with baseline | - | R1-2600754 |
| Apple | Agree with baseline | - | R1-2600826 |

Summary

Strong consensus (6 companies: Ofinno, Samsung, Apple, ZTE, LG, Qualcomm) confirms the three baseline active states from RAN1 #123: PDCCH-only (FR1: 100, FR2: 175), SSB/CSI-RS (FR1: 100, FR2: 175), and PDCCH+PDSCH (FR1: 300, FR2: 350).

Regarding additional states, three positions emerge: (a) Xiaomi argues no additional states are needed since all scenarios are already covered by scaling methods; (b) ZTE proposes reintroducing the PDSCH-only state from TR 38.840 for evaluating reduced PDCCH monitoring; (c) LG Electronics and Qualcomm propose new BB-only processing states where RF is in microsleep while BB processes previously scheduled PDSCH, with Qualcomm providing specific power values (190 for 200 MHz, ΓTput=1/2).

      - Proposals

Proposal 4.2.3-1: (1st Round  2nd round; High) Additional Active States

Introduce additional active states: (a) PDSCH-only state (ZTE, from TR 38.840) for reduced PDCCH monitoring evaluation; (b) BB-only PDSCH processing state (LG Electronics, Qualcomm) with RF in microsleep. Companies to indicate preference and provide power values if supportive.

| Company Name | Comments/Views |
| --- | --- |
| Huawei, HiSilicon | We don’t see the strong need for the additional updates. Supporting companies may further explain the motivations. |
| LG Electronics | From our understanding, if PDSCH processing relaxation is applied, UE needs to process/decode a PDSCH even for a slot where the PDSCH is not scheduled. According to current UE power modeling, UE power consumption for PDSCH is defined only during a slot where PDSCH is scheduled. In that sense, we think it is necessary to define BB-only PDSCH processing state and corresponding scaling factor. |
| ZTE, Sanechips | We believe that a PDSCH-only state is needed. PDCCH reduction methods have been discussed in the EE techniques discussion, and such reductions may result in a PDSCH-only state, which therefore should be studied. For BB-only PDSCH processing state, we think it has already be used in Bandwidth scaling factor(ΓTput), the motivation is unclear. |
| Spreadtrum | For PDSCH-only state, it is included in Proposal 4.1.1-4. For BB-only PDSCH processing state, in our view, DL bandwidth adaptation rule in section 4.1.1 already reflect the PDSCH processing relaxation. To simplify evaluation, it is unnecessary to introduce BB-only PDSCH processing state. |
| Qualcomm | In RAN1#123 agreement for UE power model there were 3 techniques that companies can report to achieve the reduction in maximum schedulable PDSCH throughput: Reduced scheduled max BW Reduced scheduled max rank Reduced scheduled duty cycle BB processing PDSCH power state is required to evaluate the reduced scheduled duty cycle. This state assumes the UE is capable of processing a PDSCH at a relaxed processing timeline comparing to a default timeline (e.g. equivalent of N1 timeline). To answer Huawei/HiSilicon question about the motivation, specifying such a state allows companies to evaluate the EE gains for both the UE and the NW from all 3 techniques. As a design, it is noted that the intention is not to downselect or agree on any of the 3 options but rather give the NW the flexibility to choose any of the three via the scheduling DCI when the UE is scheduled with limited throughput. Propose to include BB processing state as suggested in R1-2601271 |
| CEWiT | Support |
| DCM | We support introducing PDSCH-only state for reduced PDCCH monitoring evaluation, reusing the values from TR38.840, i.e., 280 for FR1 and 325 for FR2. |
| Samsung | Same view with Huawei |
| CATT | Similar view with Huawei, we We don’t see the strong need to introduce additional power state. |

    - Ultra Deep Sleep State

      - Detailed Company Contributions (Collapsed; Please Expand to View)

Sleep State Definitions and Parameters

| Source | TDoc ID | Content Summary |
| --- | --- | --- |
| Spreadtrum/UNISOC | R1-2600110 | **Proposal 7:** Clarifies that Ultra Deep Sleep (UDS) is only for IoT devices, not for eMBB devices. |
| OPPO | R1-2600197 | **Proposal 7:** For 6GR eMBB UE, proposes introducing new power state between Deep-sleep and Ultra Deep-sleep with: power value 0.1, transition energy 10000, total transition time 100ms/150ms. For Ultra deep sleep: total transition time 1600ms, transition energy 40000. |
| CMCC | R1-2600387 | **Agrees with baseline sleep states** (Ultra Deep Sleep: 0.05+Y*, Deep Sleep: 1+Y*, Light Sleep: 20, Micro sleep: 45). **Proposal 2:** Proposes alternative transition values for Ultra deep sleep from TR 38.869: (15000, 400ms) and/or (40000, 800ms) instead of (40000, 1600ms). Rationale: 1600ms is too large for MBB UE (larger than typical I-DRX cycle of 1.28s). |
| Xiaomi | R1-2600427 | **Agrees with baseline sleep states** (same table as CMCC with Ultra Deep Sleep: 0.05+Y*, Deep Sleep: 1+Y*, Light Sleep: 20, Micro sleep: 45). **Agrees with baseline transition parameters** including Ultra deep sleep (40000, 1600ms). |
| ZTE/Sanechips | R1-2600456 | **Agrees with baseline sleep states** (same as CMCC/Xiaomi). **Proposal 17:** Proposes updating Ultra deep sleep transition parameters to (25000, 600ms) based on advanced manufacturing processes in 6GR. Rationale: In 5G NR Alt 1: (15000, 400ms), Alt 2: (40000, 800ms); 6GR should have further reduced latency/power. |
| vivo | R1-2600502 | **Agrees with baseline sleep states.** **Proposal 4:** For Ultra deep sleep, proposes adopting (15000, 800ms) specifically for IoT UE. Rationale: IoT UE has less complexity than eMBB UE, should use baseline values from TR38.869. |
| Ofinno | R1-2600600 | **Agrees with baseline sleep states** including Ultra Deep Sleep (0.05+Y*, not intended for connected mode), Deep Sleep (1+Y*), Light Sleep (20), Micro sleep (45). **Agrees with baseline transition parameters** including Ultra deep sleep (40000, 1600ms). |
| Apple | R1-2600826 | **Agrees with baseline sleep states** (Ultra Deep Sleep: 0.05+Y*, Deep Sleep: 1+Y*, Light Sleep: 20, Micro sleep: 45). **Agrees with baseline transition parameters** including Ultra deep sleep (40000, 1600ms). |
| Google | R1-2601205 | **Observation 1:** Ultra Deep Sleep with high transition energy and 1600ms latency is impractical for handheld devices with frequent background traffic; energy penalty of transitions outweighs static power savings. **Proposal 1:** For 6G handheld/eMBB scenarios, Ultra Deep Sleep should be optional or disabled by default, with Deep Sleep (~20ms transition) as primary baseline for IDLE/INACTIVE mode. Detailed rationale provided on background application traffic patterns. |

      - Company Views Summary

| Company | UDS Transition Energy | UDS Total Trans. Time | Notes | TDoc |
| --- | --- | --- | --- | --- |
| Xiaomi | 40000 | 1600 ms | Baseline (RAN1 #123) | R1-2600427 |
| Ofinno | 40000 | 1600 ms | Baseline | R1-2600600 |
| Apple | 40000 | 1600 ms | Baseline | R1-2600826 |
| CMCC | 15000/40000 | 400/800 ms | TR 38.869 alternatives; 1600ms too large | R1-2600387 |
| ZTE | 25000 | 600 ms | 6GR optimized (advanced mfg) | R1-2600456 |
| vivo | 15000 | 800 ms | IoT UE specific | R1-2600502 |
| OPPO | 10000 | 100/150 ms | New state between DS and UDS | R1-2600197 |
| Spreadtrum | - | - | UDS for IoT only, not eMBB | R1-2600110 |
| Google | Optional/disabled | - | Impractical for handheld eMBB | R1-2601205 |

Summary

Strong consensus exists on sleep state power values (Ultra Deep Sleep: 0.05+Y*, Deep Sleep: 1+Y*, Light Sleep: 20, Micro sleep: 45) and basic transition parameters for Deep/Light/Micro sleep states (9 companies agree).

Different implementation approaches yield 5 distinct positions on Ultra Deep Sleep transition parameters: (1) Baseline (40000, 1600 ms) supported by Xiaomi, Ofinno, Apple; (2) TR 38.869 alternatives (15000, 400 ms) or (40000, 800 ms) from CMCC, noting 1600 ms exceeds typical I-DRX cycle of 1.28 s; (3) 6GR optimized (25000, 600 ms) from ZTE based on advanced manufacturing; (4) IoT-specific (15000, 800 ms) from vivo; (5) Optional/disabled for eMBB per Google, arguing UDS is impractical for handheld devices with frequent background traffic. Spreadtrum clarifies UDS is for IoT only.

      - Proposals

Proposal 4.2.4-1: (1st Round  2nd Round; High) IoT UE Ultra Deep Sleep Parameters

For IoT UE, adopt UDS transition parameters: transition energy = 15000, total transition time = 800 ms.

| Company Name | Comments/Views |
| --- | --- |
| Huawei, HiSilicon | For IoT UEs, since its capability is low, we are not sure whether they can perform faster transition than eMBB UEs. Supporting companies may further clarify. |
| CMCC | Fine with the proposal. |
| OPPO | Agreed. |
| ZTE, Sanechips | Our proposed value is kind of compromise between eMBB UE and IoT UE. The first issue is we define a compromised value for both UE types or define them separately. If we define it separately, for IoT UE and normal UE, we think the transition time should be smaller than 5G NR. 800ms for IoT UE is too large. 600ms would be OK with 400ms for ramp up and 200ms for ramp down. |
| Ericsson | We support this proposal. This option was present in TR 38.869, it is important to have the model for IoT devices as well. |
| DCM | We are fine with the proposal. |
| Qualcomm | In our view, we do not need to provide separate transition values for different device types. We do propose to update the bracketed agreed values to be the faster ones from NR: 20000 energy units and 800ms total transition time |
| Samsung | Same view with Huawei |
| CATT | We do not have strong view on transition energy/time. But the transition time for 6GR should be no larger than 5G NR for Ultra Deep Sleep with considering the UE in 6GR may have higher capability than 5G NR. |

    - IOT-Specific Power Values

      - Detailed Company Contributions (Collapsed; Please Expand to View)

Sleep State Definitions and Parameters

| Source | TDoc ID | Content Summary |
| --- | --- | --- |
| Spreadtrum/UNISOC | R1-2600110 | **Proposal 7:** Clarifies that Ultra Deep Sleep (UDS) is only for IoT devices, not for eMBB devices. |
| OPPO | R1-2600197 | **Proposal 7:** For 6GR eMBB UE, proposes introducing new power state between Deep-sleep and Ultra Deep-sleep with: power value 0.1, transition energy 10000, total transition time 100ms/150ms. For Ultra deep sleep: total transition time 1600ms, transition energy 40000. |
| CMCC | R1-2600387 | **Agrees with baseline sleep states** (Ultra Deep Sleep: 0.05+Y*, Deep Sleep: 1+Y*, Light Sleep: 20, Micro sleep: 45). **Proposal 2:** Proposes alternative transition values for Ultra deep sleep from TR 38.869: (15000, 400ms) and/or (40000, 800ms) instead of (40000, 1600ms). Rationale: 1600ms is too large for MBB UE (larger than typical I-DRX cycle of 1.28s). |
| Xiaomi | R1-2600427 | **Agrees with baseline sleep states** (same table as CMCC with Ultra Deep Sleep: 0.05+Y*, Deep Sleep: 1+Y*, Light Sleep: 20, Micro sleep: 45). **Agrees with baseline transition parameters** including Ultra deep sleep (40000, 1600ms). |
| ZTE/Sanechips | R1-2600456 | **Agrees with baseline sleep states** (same as CMCC/Xiaomi). **Proposal 17:** Proposes updating Ultra deep sleep transition parameters to (25000, 600ms) based on advanced manufacturing processes in 6GR. Rationale: In 5G NR Alt 1: (15000, 400ms), Alt 2: (40000, 800ms); 6GR should have further reduced latency/power. |
| vivo | R1-2600502 | **Agrees with baseline sleep states.** **Proposal 4:** For Ultra deep sleep, proposes adopting (15000, 800ms) specifically for IoT UE. Rationale: IoT UE has less complexity than eMBB UE, should use baseline values from TR38.869. |
| Ofinno | R1-2600600 | **Agrees with baseline sleep states** including Ultra Deep Sleep (0.05+Y*, not intended for connected mode), Deep Sleep (1+Y*), Light Sleep (20), Micro sleep (45). **Agrees with baseline transition parameters** including Ultra deep sleep (40000, 1600ms). |
| Apple | R1-2600826 | **Agrees with baseline sleep states** (Ultra Deep Sleep: 0.05+Y*, Deep Sleep: 1+Y*, Light Sleep: 20, Micro sleep: 45). **Agrees with baseline transition parameters** including Ultra deep sleep (40000, 1600ms). |
| Google | R1-2601205 | **Observation 1:** Ultra Deep Sleep with high transition energy and 1600ms latency is impractical for handheld devices with frequent background traffic; energy penalty of transitions outweighs static power savings. **Proposal 1:** For 6G handheld/eMBB scenarios, Ultra Deep Sleep should be optional or disabled by default, with Deep Sleep (~20ms transition) as primary baseline for IDLE/INACTIVE mode. Detailed rationale provided on background application traffic patterns. |

DL Power Values and Scaling Baseline

| Source | TDoc ID | Content Summary |
| --- | --- | --- |
| NTT DOCOMO | R1-2601179 | **Observation 2 / Table 2:** Comparison of eMBB UE (TR 38.840) vs RedCap UE (TR 38.875) power consumption shows non-linear scaling between UE types. For 2Rx, 100MHz: Deep Sleep (1 vs 0.8), Light Sleep (20 vs 18), Micro Sleep (45 vs 31), PDCCH-only (70 vs 50/40), PDCCH+PDSCH (210 vs 120), SSB/CSI-RS (70 vs 50). Demonstrates that power scaling from eMBB to RedCap is not proportional. |

      - Company Views Summary

| Company | Position | Key Data/Values | TDoc |
| --- | --- | --- | --- |
| Spreadtrum | UDS is IoT-only, not for eMBB | Separate from connected mode | R1-2600110 |
| vivo | IoT UDS: (15000, 800 ms) | Less complexity than eMBB; TR 38.869 Alt 1 | R1-2600502 |
| NTT DOCOMO | Non-linear eMBB vs RedCap scaling | RedCap/eMBB: DS 0.8/1, LS 18/20, MS 31/45, PDCCH 50/70 | R1-2601179 |

Summary

Spreadtrum and vivo advocate for IoT-specific Ultra Deep Sleep parameters, with vivo proposing (15000, 800 ms) transition parameters based on TR 38.869 Alt 1, arguing IoT UE has less complexity than eMBB. NTT DOCOMO provides important context through RedCap comparison data showing non-linear scaling between eMBB and RedCap UE types: for 2Rx/100 MHz, RedCap power ranges from 57% (PDCCH+PDSCH: 120/210) to 90% (Deep Sleep: 0.8/1) of eMBB power, demonstrating that scaling is state-dependent and not proportional.

      - Proposals

Proposal 4.2.5-2: (1st Round  2nd Round; High) Low-Tier UE Power Model Consideration

Whether to develop a separate low-tier / IoT UE power model with non-linear scaling adjustments, or use parametric scaling from the eMBB baseline.

| Company Name | Comments/Views |
| --- | --- |
| Huawei, HiSilicon | To our understanding, low tier UEs can have a lower static power due to its smaller BW. It is with the same logic as the BW scaling of T, where only the components supporting a smaller BW are ON. Therefore, we suggest to use the scaling factors for BW with T to derive the power consumption of low-tier UE. |
| ZTE, Sanechips | Without clear definition on low- tier UE, e.g., bandwidth, we may not need to define corresponding values. |
| Ericsson | No need to develop a separate model for low-tier devices. The scaling rules can be used accordingly. |
| Spreadtrum | In our contribution, we want to clarify the UDS state is not for eMBB device, eMBB device can enter the deep sleep state at most. We don’t mean that we need two set models. For the purpose of simplification, we think only one set power model is enough. |
| DCM | We are fine with the proposal and are open to discussing whether to develop a separate power model, or whether reusing the eMBB model with updated scaling factors w.r.t. BW and number of antennas would be sufficient to reflect the UE power consumption of lowtier/IoT UEs. |
| Xiaomi | Both approaches are theoretically feasible. We have a slight preference for the first one. The first approach may require agreement on the model, which will involve additional standardization discussions, but can avoid misalignment among power values across different power states in later phases. The second approach may introduce certain potential issues, including challenges in determining power values when capabilities are reduced in multiple dimensions (e.g., values falling below sleep states, which have been encountered in RedCap.) |
| Qualcomm | Suggest to reuse the eMBB power model we agree upon by changing the reference configuration. This is consequence benefit from having to specify the active BW in %. |
| Samsung | Same view with QC |

    - Others

      - Detailed Company Contributions (Collapsed; Please Expand to View)

SSB/CSI-RS and Synchronization

| Source | TDoc ID | Content Summary |
| --- | --- | --- |
| Xiaomi | R1-2600427 | **SSB scaling:** One SSB power = 0.75 of two SSB power (75 power units); **CSI-RS:** FFS for scaling w.r.t. number of symbols for CSI-RS |
| Ericsson | R1-2600960 | **Observation 13:** For DL signals like sync signal, power consumption depends on design and bandwidth scaling may not be applicable for fixed bandwidth transmissions |

UL Channel Scaling

| Source | TDoc ID | Content Summary |
| --- | --- | --- |
| Xiaomi | R1-2600427 | **Short PUCCH:** Power = 0.3x uplink power (reference: 1-symbol PUCCH, FR1/FR2); **SRS:** Power = 0.3x uplink power (FR1/FR2); **PDSCH-only slot:** 280 power units for FR1, 325 for FR2 (assumes same PDSCH symbols as PDCCH+PDSCH case) |
| Spreadtrum/UNISOC | R1-2600110 | **Proposal 12:** Not support baseband-only PDSCH processing state and related scaling rule (note: appears misplaced in original extraction) |

      - Company Views Summary

| Company | Topic | Position / Values | TDoc |
| --- | --- | --- | --- |
| Xiaomi | SSB scaling | 1 SSB = 0.75x of 2 SSB power (75 units) | R1-2600427 |
| Ericsson | SSB/sync signal | BW scaling may not apply to fixed-BW transmissions | R1-2600960 |
| Xiaomi | Short PUCCH | 0.3x UL power (1-symbol ref., FR1/FR2) | R1-2600427 |
| Xiaomi | SRS | 0.3x UL power (FR1/FR2) | R1-2600427 |
| Xiaomi | PDSCH-only slot | FR1: 280, FR2: 325 (same PDSCH symbols as PDCCH+PDSCH) | R1-2600427 |

Summary

These topics receive minimal attention with primarily Xiaomi providing NR baseline values from TR 38.840. For SSB scaling, Xiaomi proposes 1 SSB = 0.75x of 2 SSB power, while Ericsson cautions that BW scaling may not apply to fixed-bandwidth transmissions like SSB. For UL channels, Xiaomi proposes Short PUCCH = 0.3x and SRS = 0.3x of UL reference power, and PDSCH-only slot at FR1: 280 / FR2: 325 power units. These are direct TR 38.840 reuse without modification, and the low participation suggests general acceptance of NR adequacy for these parameters.

      - Proposals

Proposal 4.2.6-1: (1st Round  2nd Round; High) SSB and Reference Signal Scaling

Adopt NR baseline for SSB: 1 SSB = 0.75x of 2 SSB power.

| Company Name | Comments/Views |
| --- | --- |
| Huawei, HiSilicon | OK |
| OPPO | We don’t think SSB scaling can be explicitly given in 6G. |
| ZTE, Sanechips | OK |
| Spreadtrum | Support |
| Xiaomi | OK |
| Samsung | OK |

Proposal 4.2.6-2: (1st Round  2nd Round; High) UL Channel and PDSCH-only Scaling

Adopt the following for 6GR UE power consumption model (same as TR 38.840 counterparts):

- Short PUCCH = 0.3x UL power (1-symbol reference); SRS = 0.3x UL power

- PDSCH-only slot: FR1 = 280, FR2 = 325 power units.

- Applicable FR1 and FR2.

| Company Name | Comments/Views |
| --- | --- |
| Huawei, HiSilicon | OK |
| ZTE, Sanechips | OK |
| Spreadtrum | Support |
| Xiaomi | OK |
| Qualcomm | OK |
| Samsung | OK |

  - Others

      - Detailed Company Contributions (Collapsed; Please Expand to View)

PDCCH Processing Scaling

| Source | TDoc ID | Content Summary |
| --- | --- | --- |
| Xiaomi | R1-2600427 | Proposes cross-slot scheduling scaling factor of 0.7x same-slot scheduling (applicable for FR1 and FR2), reusing TR 38.840 scaling rules |
| Qualcomm | R1-2601271 | **Proposal 9:** Cross-slot PDCCH power = 0.7x same-slot (FR1/FR2); **Proposal 10:** PDCCH symbol duration scaling: 0.9x for 1 symbol, 1.1x for 3 symbols; **Proposal 11:** Candidate reduction scaling P(α) = α·Pt + (1-α)·0.95Pt (updated from NR); **Proposal 12:** Include effects of both increasing and decreasing PDCCH blind decodes/CCEs on UE power |
| ZTE/Sanechips | R1-2600456 | **Proposal 18:** Reduced scheduled duty cycle should exclude PDCCH monitoring occasion reduction (avoid double counting); **Proposal 19:** Include processing relaxing technique in reduced scheduled duty cycle; **Proposal 21:** PDCCH candidate scaling P(d) = {Pmicro-sleep, c=0; c×Pt +0.7×(1-c)×Pt, c>0} where c = C_use/C_ref; **Proposal 22:** Multi-state slot power P = Σ(i=1 to 14) P(i) |
| Lenovo | R1-2601106 | **Proposal 6:** Define CORESET/search space configuration for BW scaling scenarios of PDCCH-only state; **Proposal 7:** For same BW, reuse NR model P(α) = max(Micro-sleep, α·Pt + (1-α)·0.7Pt); discusses two BW scaling scenarios (scaled candidates vs. increased CORESET duration) |
| NTT DOCOMO | R1-2601179 | **Observation 2:** TR 38.840 power model inadequate for evaluating variety of PDCCH monitoring configurations (CORESET size, CCEs/BDs); **Proposal 9:** Study extending NR UE power consumption scaling w.r.t. PDCCH monitoring configuration; highlights need for multi-carrier PDCCH enhancement evaluation |
| CAICT | R1-2601323 | **Proposal 3:** Specify two-stage PDCCH with low-power pre-indication before full control decode; OFDM-based WUS design can be considered; discusses LP-WUS as potential C-DRX replacement for maximum energy saving |

      - Company Views Summary

| Company | Topic | Position | TDoc |
| --- | --- | --- | --- |
| ZTE | Reduced scheduled duty cycle definition | Exclude PDCCH MO reduction; include processing relaxation | R1-2600456 |
| Lenovo | CORESET/search space configuration | Define reference config for BW scaling; scale #candidates or CORESET duration | R1-2601106 |
| ZTE | Multi-state slot power | P_slot = Sum(i=1..14) P(i) for symbol-level granularity | R1-2600456 |
| CAICT | Two-stage PDCCH / LP-WUS | Low-power pre-indication before full DCI decode | R1-2601323 |
| Multiple | BW switching transition energy | FFS - critical for fair BW adaptation evaluation | Multiple |

Summary

Several topics require attention but do not fit into existing subsections. ZTE proposes clarifying the scope of "reduced scheduled duty cycle" to exclude PDCCH monitoring occasion reduction (avoiding double-counting) while including processing relaxation techniques. ZTE also proposes symbol-level power granularity (P_slot = Sum P(i) for i=1..14) for accurate mixed-state modeling. Lenovo raises the need to define CORESET and search space reference configurations as a prerequisite for PDCCH scaling. CAICT proposes a two-stage PDCCH architecture with LP-WUS pre-indication, which is a feature-level enhancement beyond pure power modeling. BW switching transition energy modeling is noted by multiple companies as FFS but critical for fair evaluation of BW adaptation schemes.

      - Proposals

Proposal 4.3-1: (1st Round  2nd Round; Medium) What additional topics to further discussed for UE power consumption model?

Moderator note: Please provide your views on additional power state(s), relative power value(s), power scaling rule(s) need to be discussed

| Company Name | Comments/Views |
| --- | --- |
| CMCC | We suggest to further discuss the specific value on group paging rate since it is TBD in previous agreement and such value is important for Idle mode-related UE EE evaluation. As we discussed in our tdoc, during Rel-16 study phase in UE power saving and Rel-18 study phase in LP-WUS, companies considered several optional values as baseline, i.e., 10%, 1%, 0.1%, 0.01% and 0.001%. On the basis of this, the values can be directly reused during 6GR UE EE evaluation. |
| ZTE, Sanechips | We think the power of one slot with multiple power states(for example, S slot includes DL and UL power state.) should be discussed. When multiple power states in one slot, the power of the slot can be calculated according to: P=i=114Pi P(i) is the power of ith symbol, and can be calculated according to the P/14, where P is the value of a power state in one slot. |
| Ericsson | EE processing in FR2 |

Contacts

Below is a contact list for companies’ delegates following the energy efficiency topic in the 6G Radio SI:

| Company | Delegates | Email address(es) |
| --- | --- | --- |
| Apple | Sigen Ye; Dan Wu; Hong He; Seunghee Han | sigen_ye@apple.com; dan_wu4@apple.com; hhe5@apple.com; seunghee.han@apple.com; |
| ASUSTeK | Eddie Lin | eddie1_lin@asus.com |
| AT&T | Ahmed Hindy, Ralf Bendlin | Ahmed.hindy@att.com; rb691m@att.com; |
| CATT | Shupeng Li; Miaomiao Liu | lsp@catt.cn; liumiaomiao@catt.cn; |
| CEWiT | Deepak Agarwal; Deepak PM | deepak@cewit.org.in; deepakpm@cewit.org.in; |
| CMCC | Xiaodong Shen; Minghan Jiao | shenxiaodong@chinamobile.com; jiaominghan@chinamobile.com; |
| DCM | Takashi Ikeuchi; Taichi Shichijo; Naoya Shibaike; Shinya Kumagai | takashi.ikeuchi.gs@nttdocomo.com; taichi.shichijou.ma@nttdocomo.com; naoya.shibaike.eg@nttdocomo.com; shinya.kumagai.yw@nttdocomo.com; |
| Ericsson | Magnus Åström; Gustav Lindmark; Mohammad Mozaffari; Yanpeng Yang | magnus.astrom@ericsson.com; gustav.lindmark@ericsson.com; mohammad.mozaffari@ericsson.com; yanpeng.yang@ericsson.com; |
| ETRI | Sunghyun Moon; Junghoon Lee | sh.moon@etri.re.kr; jh.lee@etri.re.kr; |
| Fainity | ChieMing | chieming@fainnov.com; |
| Fainity | YenHua Li | yenhua@fainnov.com; |
| Fraunhofer | Geordie George; Gustavo Costa; Nazanin Vatanian; Elke Roth-Mandutz | geordie.george@iis.fraunhofer.de; gustavo.wagner.oliveira.da.costa@iis.fraunhofer.de; nazanin.vatanian@iis.fraunhofer.de; elke.roth-mandutz@iis.fraunhofer.de; |
| Fujitsu | Lei Zhang | zhanglei@fujitsu.com; |
| Futurewei | George Calcev; Hussain Elkotby | gcalcev@futurewei.com; helkotby@futurewei.com; |
| Google | Alex Liou | alexliou@google.com; |
| Huawei, HiSilicon | Yi Wang; Yifan Xue; Xiaolei Tie; Yan Cheng; Matthew Webb | wangyi6@huawei.com; xueyifan1@huawei.com; tiexiaolei@hisilicon.com; chengyan.cheng@huawei.com; matthew.webb@huawei.com; |
| IIT Kanpur | Dheeraj Naidu Amudala; Jyotirmay Saini | dheeraja@iitk.ac.in; jsaini@iitk.ac.in; |
| Lenovo | Karthikeyan Ganesan; Ali Ramadan Ali Yuantao Zhang | kganesan@lenovo.com; aali@lenovo.com; zhangyt18@lenovo.com |
| LG Electronics | Seonwook Kim; Suckchel Yang; Sechang Myung; Youngdae Lee | Sseonwook.kim@lgepartner.com; suckchel.yang@lge.com; sechang.myung@lge.com; youngdae.lee@lge.com; |
| MediaTek | Chiou-Wei Tsai; Weide Wu | cw.tsai@mediatek.com; weide.wu@mediatek.com; |
| NEC | Pravjyot Singh Deogun, Liu Yun | pravjyot.deogun@emea.nec.com; |
| Nokia | Naizheng Zheng; David Bhatoolaul; Cássio Ribeiro, Ganesh Venkatraman, Jorma Kaikkonen | naizheng.zheng@nokia-sbell.com; david.bhatoolaul@nokia.com; cassio.ribeiro@nokia.com; ganesh.venkatraman@nokia.com; jorma.kaikkonen@nokia.com; |
| Ofinno | Ryan Keating; Yunjung Yi | rkeating@ofinno.com; yyi@ofinno.com; |
| OPPO | Zhisong Zuo; Hao Lin; Zhenshan Zhao | zuozhisong@oppo.com; lin.hao@oppo.com; zhaozhenshan@oppo.com; |
| Orange | Raphael Visoz | raphael.visoz@orange.com; |
| Panasonic | Hongchao Li; Suzuki Hidetoshi; Iwata Ayako; Kuruvatti, Nandish; Horiike, Naoto | Hongchao.Li@eu.panasonic.com; suzuki.hidetoshi@jp.panasonic.com; iwata.ayako@jp.panasonic.com; Nandish.Kuruvatti@eu.panasonic.com; Naoto.Horiike@eu.panasonic.com; |
| Qualcomm | Gabi Sarkis; Hung Ly; Diana Maamari | gsarkis@qti.qualcomm.com; hdly@qti.qualcomm.com; dmaamari@qti.qualcomm.com; |
| Samsung | Jaewon Lee, Youngbum Kim; Hongbo Si; Emad Farag; Qi Xiong, Sa Zhang | jcookiet.lee@samsung.com; youngbum.kim@samsung.com; hongbo.si@samsung.com; e.farag@samsung.com; q1005.xiong@samsung.com; sa.zhang@samsung.com |
| Sharp | Hiroki Takahashi; Juan Liu; Emily Lai | takahashi.hiroki@mail.sharp; juan.liu@cn.sharp-world.com; emily.ch.lai@sharp-world.com.tw; |
| Spreadtrum | Yu Ding; Huan Zhou; Zhenzhu Lei; Hualei Wang; Lei Gu | Yu.Ding@unisoc.com; Huan.Zhou@unisoc.com; Reven.Lei@unisoc.com; Hualei.Wang@unisoc.com; Lei.gu@unisoc.com; |
| TCL | Rongling Jian; Wenwen Huang; Yuanqing Yang | rongling.jian@tcl.com; wenwen5.huang@tcl.com; yuanqing4.yang@tcl.com; |
| vivo | Xin Qu; Lihui Wang; Gen Li; Huan Wang; Xueming Pan | quxin@vivo.com; wanglihui@vivo.com; reagan.li@vivo.com; wanghuan@vivo.com; panxueming@vivo.com; |
| Xiaomi | Lei Wang; Sicong Zhao | wanglei25@xiaomi.com; zhaosicong@xiaomi.com; |
| ZTE, Sanechips | Youjun Hu; Xuan Ma; Mengzhu Chen | hu.youjun1@zte.com.cn; ma.xuan1@zte.com.cn; chen.mengzhu@zte.com.cn; |
| Sony | Yuu Ichikawa; Naoki Kusashima | Yuu.Ichikawa@sony.com; Naoki.Kusashima@sony.com; |

All email addresses (separated by “;”)

magnus.astrom@ericsson.com; gustav.lindmark@ericsson.com; mohammad.mozaffari@ericsson.com; yanpeng.yang@ericsson.com; alexliou@google.com; rongling.jian@tcl.com; wenwen5.huang@tcl.com; yuanqing4.yang@tcl.com; Hongchao.Li@eu.panasonic.com; suzuki.hidetoshi@jp.panasonic.com; iwata.ayako@jp.panasonic.com; Nandish.Kuruvatti@eu.panasonic.com; Naoto.Horiike@eu.panasonic.com; gsarkis@qti.qualcomm.com; hdly@qti.qualcomm.com; dmaamari@qti.qualcomm.com; zhanglei@fujitsu.com; chieming@fainnov.com; naizheng.zheng@nokia-sbell.com; david.bhatoolaul@nokia.com; cassio.ribeiro@nokia.com; takahashi.hiroki@mail.sharp; juan.liu@cn.sharp-world.com; emily.ch.lai@sharp-world.com.tw; wangyi6@huawei.com; xueyifan1@huawei.com; tiexiaolei@hisilicon.com; chengyan.cheng@huawei.com; matthew.webb@huawei.com; shenxiaodong@chinamobile.com; jiaominghan@chinamobile.com; lsp@catt.cn; liumiaomiao@catt.cn; sh.moon@etri.re.kr; jh.lee@etri.re.kr; pravjyot.deogun@emea.nec.com; wanglei25@xiaomi.com; zhaosicong@xiaomi.com; quxin@vivo.com; wanglihui@vivo.com; reagan.li@vivo.com; wanghuan@vivo.com; panxueming@vivo.com; hu.youjun1@zte.com.cn; ma.xuan1@zte.com.cn; chen.mengzhu@zte.com.cn; jcookiet.lee@samsung.com;  youngbum.kim@samsung.com; hongbo.si@samsung.com; e.farag@samsung.com; q1005.xiong@samsung.com; dheeraja@iitk.ac.in; sigen_ye@apple.com; dan_wu4@apple.com; hhe5@apple.com; seunghee.han@apple.com; kganesan@lenovo.com; aali@lenovo.com; geordie.george@iis.fraunhofer.de; gustavo.wagner.oliveira.da.costa@iis.fraunhofer.de; nazanin.vatanian@iis.fraunhofer.de; elke.roth-mandutz@iis.fraunhofer.de; gcalcev@futurewei.com; helkotby@futurewei.com; takashi.ikeuchi.gs@nttdocomo.com; taichi.shichijou.ma@nttdocomo.com; naoya.shibaike.eg@nttdocomo.com; shinya.kumagai.yw@nttdocomo.com; deepak@cewit.org.in; deepakpm@cewit.org.in; rkeating@ofinno.com; yyi@ofinno.com; cw.tsai@mediatek.com; weide.wu@mediatek.com; Yu.Ding@unisoc.com; Huan.Zhou@unisoc.com; Reven.Lei@unisoc.com; Hualei.Wang@unisoc.com; Lei.gu@unisoc.com; Sseonwook.kim@lgepartner.com; suckchel.yang@lge.com; sechang.myung@lge.com; youngdae.lee@lge.com; zuozhisong@oppo.com; lin.hao@oppo.com; zhaozhenshan@oppo.com; jsaini@iitk.ac.in; raphael.visoz@orange.com; Ahmed.hindy@att.com; rb691m@att.com; Yuu.Ichikawa@sony.com; Naoki.Kusashima@sony.com; eddie1_lin@asus.com; zhangyt18@lenovo.com; sa.zhang@samsung.com; ganesh.venkatraman@nokia.com; jorma.kaikkonen@nokia.com;

References

Summary of agreements in RAN1 #123

EE metrics and evaluation methodology

Agreement

For evaluation purposes and relative comparison over different candidate energy saving schemes for 6GR, define the following baseline power saving configurations for UE for evaluation purpose for FR1 (including around 7GHz):

- 5G NR I-DRX (1.28s cycle) for idle mode

  - Group paging rate (for a PO): TBD

- 5G NR C-DRX settings of (cycle, on-duration timer, inactivity timer) are assumed for the following 6GR traffic models for connected mode:

  - VoIP: (40 ms, 8 ms, 10 ms)

  - FTP3: (160 ms, 8 ms, 100 ms)

  - Instant message: (320 ms, 8 ms, 100 ms)

  - XR: (16 ms, 10 ms, 4 ms)

- Companies can evaluate and report other traffic(s) and/or configuration(s) with justification

Note: The corresponding evaluation is not intended for energy efficiency comparison with 5G/NR.

BS power consumption model

Agreement

Add the following as one of reference configurations for BS power consumption

|  | Set 4 around 7** GHz |
| --- | --- |
| Duplex | TDD |
| System BW | 400 MHz |
| SCS | 30 kHz |
| Number of TRP | 1 |
| Total number of DL TX Rus | 256 |
| Total DL power level | [62] dBm |
| Total number of UL Rx Rus | 256 |
| **Pending agreement in 11.2 whether to evaluate 15 GHz. |  |

Agreement

Total DL power level for Set 4 is 56 dBm.

The following relative power levels applies to Set 4 for a CAT 1 BS and a CAT 2 BS:

| Power state | Set 4 |  |
| --- | --- | --- |
|  | CAT 1 BS | CAT 2 BS |
| Deep sleep | 1 | 1 |
| Light sleep | 31 | 3.1 |
| Micro sleep | 85 | 7.3 |
| Active DL | 530 | 67 |
| Active UL | 175 | 10 |

UE power consumption model

Agreement

Include the following non-sleep states as 6G UE power consumption model.

- Other power state(s) is not precluded

- FFS: Configuration and relative power value(s) for EE processing in FR2 (including 24.25 GHz – 52.6 GHz)

- Note: Pending agreement in 11.2 whether to evaluate 15 GHz

| Power State | Characteristics | Relative Power in FR1 (including around 7GHz) | Relative Power in FR2 (including 24.25 GHz – 52.6 GHz) |
| --- | --- | --- | --- |
| EE Processing | Processing DL WUS of OFDM-based sequence(s) for at least wake-up indication and, if applicable, other 6GR signal(s) of OFDM-based sequence(s) for synchronization and/or measurement, in an energy efficient manner, based on the following configuration: FR1 (including around 7GHz) - 5 MHz BW, 1-RX / 2-RX for reception; FFS: Power values for 2-RX, i.e., X1, X2, X3 - Residue CFO up to [5] ppm and residue timing offset up to [2] us - Noise Figure (NF) = 6GR UE NF - Reception time up to one slot - Note: Whether/how to scale power values for different reception time setting within one slot to be further discussed/decided FR2 (including 24.25 GHz – 52.6 GHz) - FFS Note: No implication on which configuration(s) to be supported by 6GR EE-processing can only be performed during a sleep state with additional relative power value added w.r.t. the sleep state and without triggering UE transition out of the sleep state. Additional energy overhead [15], in unit of (relative power x ms), is included for each time entering or leaving EE processing during ultra-deep/deep sleep. Ramp-up or ramp-down time for EE processing is [2] ms. | 10 / X1 (during micro sleep) 12 / X2 (during light sleep) 15 / X3 (during other sleep) | FFS |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 | 175 |
| SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell. TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. | 100 | 175 |
| PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. | 300 | 350 |
| UL | Long PUCCH or PUSCH or PRACH FFS: Power values for more Tx power levels, i.e., X4, X5, X6, X7 | 250 (0 dBm) X4 (10 dBm) X5 (15 dBm) X6 (20 dBm) 700 (23 dBm) X7 (26 dBm) | 350 (FFS Tx power level) |

Agreement

Include the following sleep states for 6G UE power consumption model.

Note: Ultra Deep Sleep is not intended for connected mode

| Power State | Characteristics | Relative Power |
| --- | --- | --- |
| Ultra Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing or frequency may not always be maintained. | 0.05 + Y* |
| Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not always be maintained. | 1 + Y* (Optional: 1) |
| Light Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. | 20 |
| Micro sleep | Immediate transition is assumed for power saving study purpose from or to a non-sleep state | 45 |
| * Y value equals to 0.1, if EE processing is assumed for the evaluation; zero, otherwise. |  |  |

| Sleep type | Additional transition energy: (Relative power x ms) | Total transition time** |
| --- | --- | --- |
| Ultra deep sleep | [40000] | [1600] ms*** |
| Deep sleep | 450 | 20 ms*** |
| Light sleep | 100 | 6 ms |
| Micro sleep | 0 | 0 ms* |
| * Immediate transition is assumed for power saving study purpose from or to a non-sleep state ** Ramp-up time is no less than half of the total transition time *** Time for sync/re-sync is not included |  |  |

Agreement

For the UE power model, assume:

- ΓB: The ratio of active bandwidth to the bandwidth of reference configuration (100MHz)

- ΓTput: The ratio of the maximum schedulable PDSCH throughput w.r.t. that of current active bandwidth, ΓTput∈{1,1/2,1/4}

  - FFS: How the maximum schedulable PDSCH throughput ratio ΓTput is ensured by NW

  - Companies to report the assumed technique to achieve the reduction in the maximum schedulable PDSCH throughput. Current techniques include at least: reduced scheduled maximum bandwidth, reduced scheduled maximum rank, and reduced scheduled duty cycle.

  - No antenna adaptation is assumed simultaneously

- For evaluation purpose,

  - Company to report Tmin value for the evaluation

  - T>Tmin is another relaxed adaptation delay. Company to report T value for the evaluation.

  - The adaptation delay is applied when adapting ΓB and/or ΓTput

- Companies to report SMaxBW from one of the two options:

  - Option 1: SMaxBW=0/Z1/Z2 for Maximum BW (MaxBW) assumed for the evaluation ∈{100MHz,200MHz,400MHz}

  - Option 2: SMaxBW=0

- Note: No implication on which configuration(s) and what adaptation(s) to be supported by 6GR

Include the following scaling for DL bandwidth adaptation for 6GR UE power consumption model:

- For PDCCH+PDSCH:

| ΓB | ΓTput | Scaling factor when adaptation delay T is assumed | Scaling factor when adaptation delay Tmin is assumed |
| --- | --- | --- | --- |
| 5% | 1 | X01 | X11 +SMaxBW |
| 20% | 1 | X02 | X12 +SMaxBW |
| 100% | {1, 1/2} | 1 | 1+SMaxBW |
| 200% | {1, 1/2} | {X03, Y03} | {X13, Y13} + SMaxBW |
| 400% | {1, 1/2, 1/4} | {X04, Y04, Z04} |  |

- For PDCCH-only:

| ΓB | ΓTput | Scaling factor when adaptation delay T is assumed | Scaling factor when adaptation delay Tmin is assumed |
| --- | --- | --- | --- |
| 5% | 1 | X21 | X31 +SMaxBW |
| 20% | 1 | X22 | X32 +SMaxBW |
| 100% | 1 | 1 | 1+SMaxBW |
| 200% | {1, 1/2} | {X23, Y23} | {X33, Y33} + SMaxBW |
| 400% | {1, 1/2, 1/4} | {X24, Y24, Z24} |  |

- Other DL signal/channel processing:

| ΓB | Scaling factor when adaptation delay T is assumed | Scaling factor when adaptation delay Tmin is assumed |
| --- | --- | --- |
| 5% | X41 | X51 +SMaxBW |
| 20% | X42 | X52 +SMaxBW |
| 100% | 1 | 1+SMaxBW |
| 200% | X43 | X53 + SMaxBW |
| 400% | X44 |  |

- Not applicable to sleep states and EE processing state

- Further consolidate the table for cases of a same scaling factor

- FFS: Scaling factor for other value(s) of ΓB<100%

For UL bandwidth adaptation,

- No scaling for FR1 and around 7GHz

- In case scaling is needed for FR2 (including 24.25 GHz – 52.6 GHz), companies can report the assumed scaling factor

Note: The term “bandwidth” used here is reusing the definition as 5GNR, which will be updated according to 6GR discussion

- Note: All columns and rows of the above tables will be further checked, and corresponding values will be checked and confirmed/extended.