3GPP TSG RAN WG1#124			    R1-260xxxx

Gothenburg, SE, February 9th – 13th, 2026

Agenda item:	10.5.2.4

Source: 	Moderators (ZTE, Apple)

Title: 	Moderator summary #1 on beam management for DL and UL

Document for:	Discussion and Decision

- Introduction

This summary summarizes observations and proposals from contributions submitted to 3GPP TSG RAN WG1#124, for beam management for downlink and uplink on 6G Radio (6GR) in A.I. 10.5.2.4. The contributions elaborate lessons learnt from 5G beam management, identify the new 6GR scenarios and requirements, and propose the corresponding 6G Day-1 features. Generally speaking, there is a common consensus of having a unified 6GR beam management framework to accommodate various deployment scenario(s) (sTRP, DPS/repetition, mTRP (CJT/NCJT), cell-free, etc.), different frequency range(s) (sub-6GHz, around 7GHz, around 15GHz, FR2, etc), DL/UL transmission schemes, classical and advanced beam measurement (e.g., AI/ML, compressed sensing, etc.), and NW/UE-initiated report/switching procedure.

- Plan

Per Mr. Chair’s guidance, for 6GR-beam management for downlink and uplink, at least the following aspects should be considered for the subsequent discussion.

#### Beam management for downlink and uplink Note 1: Including proposals for beam management related aspects, such as beam indication, beam report and beam recovery, etc.

Based on the contributions from companies [1]-[39], the followings are provided in this document:

- Summary of companies’ views on each of open issues raised by interested companies, where the open issues are categorized as follow:

  - Issue 1 – Lessons learnt from 5G, 6GR-BM scenario(s), requirement(s) and scopes

  - Issue 2 – Evaluation methodology (EVM) for beam management

  - Issue 3 – NW initiated beam management (e.g., beam measurement, report and indication)

  - Issue 4 – UE initiated beam management (e.g., beam failure recovery, UE-initiated/event-driven beam reporting, etc.)

- Observations and recommended proposals are based on the summary of companies’ views

- Contact Person

For potential offline discussion, companies/delegates are encouraged to enter the contact information in the table below:

Table 0 Contact Information

| Company | Point(s) of contact | Email address(es) |
| --- | --- | --- |

- Lessons learnt from 5G, 6GR-BM scenario(s), requirement(s) and scopes

This section synthesizes observations and proposals from contributions, regarding on beam management for DL and UL, for lessons learnt from 5G, deployment/targeted scenario(s) and the corresponding scope(s).

  - NW initiated procedure

    - Lessons learnt from 5G-NR

Moderator thanks companies’ valuable inputs on lessons learnt from 5G-NR beam management procedure (NW-initiated). Then, the following observations are proposed for companies’ check and comment:

Observation 4.1.1:

On NW-initiated beam management for DL and UL, regarding the lessons learnt from 5G-NR, the following aspects are identified from RAN1 perspective:

|  | Lessons | Description | Reported companies |
| --- | --- | --- | --- |
| 1 | Definition of beam management | Definition of beam management [12] According to the TR 38.802, beam management in 5G NR was defined as follows: - Beam management: a set of L1/L2 procedures to acquire and maintain a set of TRP(s) and/or UE beams that can be used for DL and UL transmission/reception, which include at least following aspects: - Beam determination: for TRP(s) or UE to select of its own Tx/Rx beam(s). - Beam measurement: for TRP(s) or UE to measure characteristics of received beamformed signals - Beam reporting: for UE to report information of beamformed signal(s) based on beam measurement - Beam sweeping: operation of covering a spatial area, with beams transmitted and/or received during a time interval in a predetermined way. | xiaomi, China Telecom, CMCC, |
| 2.1 | QCL-related aspect – Definition & source QCL-RS | QCL definition - [11] … Quasi-Co-Location (QCL) is a fundamental physical layer concept in 5G NR. It is defined as follows: Two antenna ports are said to be quasi co-located if properties of the channel over which a symbol on one antenna port is conveyed can be inferred from the channel over which a symbol on the other antenna port is conveyed. Source QCL-RS (e.g., SSB, P-TRS) - [1] … in 5G NR, periodic TRS is UE-specific and serves as a primary QCL source reference signal for downlink reception, supporting channel estimation for CSI-RS, PDCCH DM-RS, and PDSCH DM-RS. In contrast, uplink reception of PUCCH and PUSCH relies directly on DM-RS. The use of UE-specific periodic TRS introduces significant system overhead and contributes to increased network energy consumption. For 6G, it is therefore desirable to revisit the reliance on periodic TRS and explore mechanisms to reduce or eliminate its transmission, such as using aperiodic TRS transmitted in vicinity of data transmissions (PDSCH) or relying more on DMRS for downlink reception. - [8] … According to commercial practices, the TRS related QCL chains don't help much in improving system performance, but takes up valuable time frequency resources and increases the workload of base station. - [11] … mandates the SSB as the primary or sole source for QCL relationships, which becomes unsuitable in high-mobility scenarios (e.g., high-speed train) or for flexible access based on CSI-RS. For instance, in high-speed train scenarios using Single Frequency Network (SFN) transmission or in multi-TRP/cell free cooperative scenarios, the SSB may fail to provide the precise QCL information required for a specific TRP. This rigidity becomes a bottleneck for rapid beam switching and dynamic TRP cooperation. - [29] … periodic TRS is used as QCL source RS for many other signals including CSI-RS for CSI acquisition, PDSCH DMRS, and PDCCH DMRS. Periodic TRS is an always-on signal that contributes to high reference signal overhead as well as UE and network energy consumption, which will increase in denser network with more TRPs and short inter-site distances - [37] ... With increasing mobility at higher carrier frequencies, fewer number of TRS resources for tracking by the UE, the faster rate of change of the strongest beams serving the UE and higher latency of update of QCL references for TRS via RRC act as a bottleneck for accurate and reliable channel parameter estimations required for PDCCH and PDSCH demodulation. | Nokia, ZTE, CMCC, Ericsson, China Telecom, Fraunhofer IIS, Fraunhofer HHI, CEWiT |
| 2.2 | QCL-related aspect – unified TCI framework | Unified TCI framework (R17/18) vs Separate TCI/spatial-relation (R15/16) - [1] … Fragmented development of overlapping beam measurement and reporting features across working groups leads to divergent designs, increased system complexity, and longer development cycles. - [5] … In 5G BM framework, a lot of separate features were supported, leading to big redundancy in the spec., e.g., redundant configuration, measurement and reporting. - [8] … 5G has standardized a multitude of redundant features that turned out to be either never commercialized or proved to be highly impractical in real-world deployments. A prime example is the overly complicated QCL-chain design. The complexity of this design made it difficult to implement and manage efficiently. Secondly, although the Rel-17 unified TCI framework was a step forward in standardizing beam indication… - [10] … In 6GR, DL/UL beam switching within a same TRP/cell or among different TRPs/cells is based on TCI state indication/update - Rel‑17/18 unified TCI together with ICBM as starting point - At least for TRPs/cells under the same BBU - Support more than one indicated TCI states for DL/UL - [11] … In 6GR, a unified TCI framework for mTRP should be studied and the unified TCI framework in 5G NR can be seen as a starting point. - [22] … Unified TCI framework in NR helped to facilitate joint DL/UL beam indication, reduce signaling overhead and associated ambiguities, but still built on top of legacy framework and having potentially scope for further simplification - [23] … The unified TCI framework provides a scalable basis for beam management, including support for multi-TRP operation, with a clear beam application timeline and consistent beam usage across downlink and uplink channels. - [36] … As one key component for multi-beam operation, unified TCI state has been introduced in R17 to switch the common beam associated with multiple channels/signals via a single beam switch command, instead of multiple individual commands. This saves both beam indication overhead and latency, compared with R15 TCI state framework. Therefore, the beam management based on unified TCI state framework should be considered as baseline in 6G, across frequency ranges. | Apple, Futurewei, Huawei, ZTE, Samsung, MediaTek, Ericsson, Qualcomm, Nokia, CMCC, NTT DOCOMO, TCL, CATT, Ofinno, Google, NEC, China Telecom, IDC, Fujitsu, Sharp, ETRI, Transsion, Sony, AT&T, Fraunhofer IIS, Fraunhofer HHI, CEWiT, xiaomi, |
| 3 | Beam sweeping, and tracking | Beam sweeping pattern (intra vs inter-symbol) - [6] … One issue with the CSI-RS for beam management in 5G/NR is that the CSI-RS is designed for CSI measurement, i.e., the UE can measure the full channel state information from one CSI-RS resource. However, the beam measurement does not need the UE to measure the full channel state information. In contrast, the RSRP-like measurement is sufficient. Actually, using full channel state information for beam management would be non-efficient way. Furthermore, one key feature for beam measurement is to support beam sweeping efficiently so that the system can complete the transmission of all candidate Tx beams with the least resource and shortest time latency. - [12] … Study per-port L1-RSRP report for fast beam sweeping based on a N ports CSI-RS resource with one-to-one mapping between port and Tx beam. - [21] … The per-symbol beam sweeping approach treats the OFDM symbol as an indivisible unit for beamforming. However, analog beamforming can be applied independently to different subcarrier groups within a symbol if the beamforming weights are set per subband rather than wideband. This enables intra-symbol beam sweeping where multiple beam directions are transmitted simultaneously in different frequency regions of the same symbol. [22] Beam sweeping pattern (intra vs inter-symbol) Early beam report/refinement starting from random access - [36] … As reflected in coverage study in 5G, DL/UL messages in initial access are typically bottleneck due to suboptimal used beams. However, the beam refinement can only happen after RRC setup complete. Therefore, it would be beneficial to consider early beam refinement to improve the coverage of initial access messages - [8] … For 6GR beam management during initial access, early sync/CSI for multi-TRP should be considered to enable a smooth transition from wide to narrow beams. This can be achieved by Msg1/Msg3 to indicate the beam measurement results for CJT and Msg4/late message to trigger the early CSI for mTRP. To be more specific, the UE can first report beam ID, TRP ID, or RSRP information to the base station via Msg1 (random access preamble) or Msg3 (connection request message) during initial access or using reciprocity/beam correspondence, - [12] ... To reduce beam determination latency, early beam reporting during initial access should be considered. We propose reusing the framework of early CSI triggering with potential enhancements if necessary. It means that after Msg 1 transmission, the UE shall continue measuring the SSB/CSI-RS or the UE shall start measuring triggered by Msg 4. Beam reporting can be realized via Msg 3, or a PUSCH indicated in Msg 4. To avoid increasing UCI payload size, beam reports should minimally include RS ID(s)… [36] Early beam report in Msg3 based on SSB | OPPO, Apple, ZTE, Qualcomm, xiaomi, |
| 4 | Report quantity or metrics | Lack of interference information report - [5] …, beam reporting focused on channel beam reporting and neglected interference beam reporting which is essential for beam based scheduling. Although L1-SINR reporting was supported, the strong restriction on IMR configuration, i.e., each CMR is associated with a particular IMR, making the evaluation of interference unpractical. … the configuration of CMR and IMR is too restrictive: each CMR is associated with a particular IMR. It is not possible for UE to identify the weak interference beams or strong interference beams among all the other beams. Second, interference is reflected by the reported L1-SINR of CMR and no explicit interference beam information is reported. With the L1-SINR of a CMR based on the pre-configured IMR, BS cannot know which beam will cause weak or strong interference to the reported beam and thus cannot determine which beam should or should not be scheduled together with the reported beam. - [18] ..., later 3GPP Rel-16, L1-SINR-based beam measurement and reporting were introduced to facilitate interference-aware beam selection. The gNB can configure UE to measure and report L1-SINR based on SSB/CSI-RS resources. The following resource settings for L1-SINR measurement have been supported | Huawei/HiSi’, ZTE, China Telecom, |
| 5 | Beam activation | Beam activation MAC-CE(s) for different channels/RSs/time-domain behaviors - [12] … Another feature supported in 5G NR to reduce the signalling overhead is that TCI states on all CCs in one CC list can be activated by a single MAC CE. - [17] … a unified framework should be strived for both TRP activation/indication (e.g. for CJT based multi-TRP targeting FR1) and beam activation/indication (e.g. targeting FR2), such as a set of TRPs (CSI-RS resources) can be activated and then TRP(s) can be further indicated for CJT CSI report or CJT operation, … - [18] … the signalling framework of ‘RRC+MAC-CE+DCI’ is identified widely used for well-balanced signaling overhead, indication/activation/deactivation latency and UE complexity of QCL-tracking. - [22] … TCI state activation determines which beams from the configured pool are available for dynamic indication. In NR, the Unified TCI States Activation/Deactivation MAC CE can activate up to 8 TCI states for subsequent DCI-based selection. The MAC CE format differs depending on whether the RRC has configured joint mode (single TCI for DL+UL) or separate mode (independent DL and UL TCI states) … Study to define a single MAC CE for TCI activation from the common TCI list per serving cell that is applicable to all the downlink and uplink channels/signals as the baseline [10] 5G NR beam switching based on TCI state activation and indication Long latency of TCI state activation latency - [36] … for a single known DL applicable TCI state (corresponding SSB is L1 reported within 1.28s before the TCI state selection command), the activation latency includes time to the 1st trans of SSB plus 2ms processing time (up to ~20 ms). The activation latency increase is more pronounced for a single unknown DL applicable TCI state (not L1 reported/measured in last 1.28s and, | ZTE, Apple, Qualcomm, xiaomi, NEC, China Telecomm, OPPO, IDC, MediaTek, |
| 6 | Beam association or indication | Beam association/QCLed - [8] … 5G often assumes a hierarchical scanning method of SSB wide beam scanning and CSI-RS narrow beam refinement, but the beam width characteristics of RS are transparent to the UE. However, it may have different implementation in the field depending on scenarios. 6G needs to reconsider the definitions of wide beams, narrow beams or the same beam (even if they are configured as QCL type D relationships) and their impact on system performance. This includes whether to explicitly or implicitly indicate wide beams/narrow beams to the UE to optimize the efficiency of channel estimation. Default QCL assumption - [6] …, In 5G/NR, numerous UE behaviors of default TCI state or QCL (i.e., default Tx beam) were specified. The first default TCI state behavior was specified for dynamic PDSCH in release 15. … the UE has to start to buffer the PDSCH with assuming a ‘default beam’, which is the motivation for the specification of default TCI state in release 15. Following the same technical reason, default TCI state was specified for PDSCH in various scenarios including mTRP system and for aperiodic CSI-RS resource reception. - [19] …, if the scheduling or triggering offset for receiving a PDSCH or CSI-RS is smaller than a corresponding threshold, the UE needs to use default beam(s) to buffer the corresponding PDSCH or CSI-RS reception(s). Such default beam operation was greatly simplified in Rel-17 under the unified TCI framework due to the common beam design principle, but it got convoluted again in Rel-18 for multi-TRP operation Beam indication cost - [22] … This mismatch means the TCI field carries redundant information in most DCIs. For a UE scheduled every 1 ms with beam coherence of 100 ms, approximately 99% of TCI field transmissions indicate no change from the previous beam. The 3-bit overhead per DCI accumulates to meaningful DCI size increase, particularly for multi-TRP scenarios where TCI fields are needed per TRP… Beam updates would occur via MAC CE when the network determines a beam change is needed, which aligns with the actual beam change frequency. - [29] … one of the narrowband SSB will be used as the source RS for the multi-port CSI-RS. In such scenarios, the network may need to frequently switch the TCI state when the UE moves across the cell, which leads to high signalling overhead. - [37] … In 5G NR, the PDSCH-scheduling DCI formats 1_1, 1_2 and 1_3 may be used for the indication of TCI-states for both DL and UL channels and RSs in the unified TCI framework. This may also extend for multi-TRP scenarios when up to two joint TCI-states or up to two DL and UL TCI-states may be mapped to and indicated by a codepoint of the TCI indication field of the DCI format… RAN1 shall at least study the decoupling of the TCI-states indication for DL and UL in different DCI formats. Analog beam mis-alignment/mis-match due to pre-determined candidate beam(s) - [5] … transmission is based on the pre-defined beam pattern in 5G BM framework. In high frequency, like FR2-1, the beam is very narrow and can only cover a very small range of angle, leading to performance loss when the channel has a path with a relatively large angle spread or has multiple strong paths. In addition, even for a channel with pure LOS path (single path with small angle spread), when the path does not well match the center of a beam, e.g., the path is located among multiple adjacent beams, the BF gain loss can be very big (e.g., up to 8 dB). - [36] … the current analog beam selection is among a set of pre-determined candidate beams in a fixed codebook. The selected candidate beam may not well match the channel. According to initial results, this may result in substantial spectral efficiency degradation, compared with better optimized analog beam weights Others - Lack of QCL indication among SSBs from different CC [36] - Lack of dynamical TCI update (also other parameter, e.g., time-domain offset) for P-CSI-RS [8], [36] - Limitation on maximum number of activated/indicated TCI state(s) [33] | Samsung, ZTE, OPPO, Huawei, Qualcomm, Apple, Fraunhofer IIS, Fraunhofer HHI, xiaomi, |
| 7 | UL beam management | UL beam measurement - [6] … SRS is used for UL beam measurement. Similar to DL beam measurement, SRS resource are design for UL channel state information measurement and some SRS resources are used for UL beam measurement. Such SRS resources are configured with usage of beam management. Each SRS resource corresponds to one UE transmit beams. The specification does not support configuring beam sweeping behavior one the SRS resources used for beam measurement. - [10] …6GR should prioritize DL-based UL beam management as the baseline, treating UL-based methods as supplementary tools for specific use cases, e.g., asymmetric DL/UL multi-TRP operation… - [12] … For UE support beam correspondence without UL beam sweeping, DL beam sweeping is sufficient for UL beam management. It means that the best DL Rx beam can be the best UL Tx beam at UE side. In this case, additional UL beam sweeping is unnecessary… - [18] … The similar procedures are defined for UL beam sweeping, i.e., U-1, U-2, U-3. U-1 is used for coarse beam sweeping and selection of UE Tx beams/TRP Rx beam(s) based on TRP measurement on different UE Tx beams. Based on U-1 procedure, U-2 and U-3 are used for fine beam sweeping for TRP Rx beams and UE Tx beams respectively. The TRP measurement can be based on SRS resource or DL beam correspondence. - [25] … clearer support of UL BM is needed considering target scenarios such as UL only TRP scenario described in Section 2.2. For example, CSI-RS like configuration (e.g., repetition=on/off) can be adopted to SRS for BM. - [33] …To ensure stable high-frequency utilization in 6GR, UL beam management will become as important as DL beam management. Particularly, considering that UEs with multiple panels will become practical in 6GR, enhancement of UL beam management for finding optimal UL beam pairs across multiple panels should be studied | Qualcomm, OPPO, MediaTek, NTT DOCOMO, ZTE, China Telecom, LGE, Sharp, Panasonic, xiaomi, |

Table 4.1.1 Company input for Observation 4.1.1

| Company | Input |
| --- | --- |
| Mod V00 | - Please input your further views/inputs on observation 4.1.1, i.e., lessons learnt from 5G-NR, if any important information is missing. |

    - Targeted scenarios and requirements of 6GR beam management

Moderator thanks companies’ valuable inputs on targeted scenarios of 6GR beam management (NW-initiated). Then, the following observations is proposed for companies’ check and comment:

Observation 4.1.2:

On NW-initiated beam management for DL and UL, regarding 6GR target scenarios and requirements of DL and UL beam management, the following aspects are identified from RAN1 perspective:

|  | Scenario(s) | Description | Reported companies |
| --- | --- | --- | --- |
| 1 | 6GR target scenario(s) – multi-TRP/cell-free operation | Cell free/CJT-scenario for 6G beam management - [11] … The evolution toward 6G is expected to leverage advanced multi-antenna architectures to meet unprecedented demands for capacity, reliability, and seamless coverage. In order to reduce the service interruptions caused by handovers in the procedure of mobility, a deployment scheme called “cell free” is proposed by companies in 6GR study, where a cell free area (CFA) consists of multiple cells/TRPs, … From deployment perspective, 12~20 cells/TRPs within a CFA may connect to one BBU, which can be regarded as ideal backhaul (i.e., ideal delay/frequency/phase synchronization) [11] Example for cell-free area - [19] …A geographical area served by a network is partitioned into cells. Cells can be mapped to or associated with RUs, DUs or CUs. A UE can connect to the network through one or more RUs associated with a serving cell. Or depending on propagation channel conditions, a UE can connect through RUs not associated with the serving cell. As a UE moves within the geographical area covered by the network, the cell or cells through which the UE connects to the network changes, and hence the RU(s) through which the UE connects to the network change. A UE can communication through multiple TRPs (i.e., a coordination set) using CJT … on L1 seamless mobility in 6GR, support CJT operation across different cells such that a coordination set can comprise TRPs from different cells … support to represent a coordination set by a set of TCI states, and the UE to perform measurement reporting based on the RSs specific to the TCI states hence the corresponding coordination set. … support to use L1 control DCI format to switch between coordination sets hence TCI states that correspond to the coordination sets. [19] Inter-cell beam management/mobility for CJT mTRP sTRP/mTRP and intra-/inter-cell beam management starting from T0/Day1 - [1] … the need for a future-proof beam reporting and beam management framework in 6G that avoids fragmented designs for single-TRP and multi-TRP operation, and that consistently supports both intra-cell and inter-cell scenarios from the first release. … Support beam management for intra- and inter-cell operation in 6G using a unified TCI state framework. … Support a beam reporting and beam management framework in 6G that avoids fragmented designs for single-TRP and multi-TRP operations, for both intra- and inter-cell scenarios within a unified TCI state framework. - [5] … sTRP based beam reporting is a basic feature applied for general cases. mTRP based beam reporting is the enabler of mTRP based transmission. Without mTRP based beam reporting, beam based mTRP transmission is not functional. For 6G, we believe both sTRP based and mTRP based beam reporting should be supported. - [8] … support cell-free and multi-TRP mode, the concept of cell-cluster should be introduced and it consists of multiple TRPs, e.g., up to [12~20] as shown in Figure 2-1. The beam or TRP switching within or across cell clusters should be addressed which may impact the beam management procedures. Particularly, in order to avoid unnecessary inter-TRP/cell-cluster switching latency, TRPs within a cell-cluster should be from intra-DU only, or both intra-DU and inter-DU. Using TRPs from the same DU makes coordination easier and cuts switching time. Using TRPs from different DUs can expand coverage … [8] Intra/inter-cell cluster beam management - [10] …6GR beam management should prioritize a unified, RRC-free intra-cluster mobility model with only essential L1 adjustments and native multi-TRP operation, so that a UE experiences seamless connectivity as it moves among coordinated cells/TRPs under the same BBU. - [13] …, intra-cell beam switch, inter-cell beam switch and cell switch will happen as UE moves around. In NR, intra-cell and inter-cell beam switch can be triggered by TCI indication signaling after L1 beam measurement and report. … To simplify signaling design and facilitate seamless mobility, a unified signaling design for LTM and intra-cell/inter-cell beam switch should be studied in 6G. - [18] … We support single-TRP, DPS and/or multi-TRP transmission for beam management in 6G Day-1. In current NR specifications, the number of TRPs is restricted considering overhead and complexity during multi-TRP transmission. To meet 6G requirement of better capacity and seamless handover, more TRPs enabling simultaneous transmission are needed, e.g., 4 TRPs. - [33] … regarding multi-TRP operation in NR, single-DCI multi-TRP and multi-DCI multi-TRP were supported. Single-DCI multi-TRP was specified for ideal backhaul scenario, and multi-DCI multi-TRP was specified for ideal/non-ideal backhaul scenario. In most scenarios where benefits from multi-TRP operation can be gained, it is generally reasonable to assume an ideal backhaul, so we think it is best to prioritize ideal backhaul scenario. [10] Example for cell/TRP cluster Multi-TRP operation: Dynamic TRP on/off - [15] … Instead, dynamic on/off mechanism may allow low switching time as well as wake-up time. One candidate solution we can consider is to indicate dormancy for each TRP. Moreover, energy efficiency is another key aspect to consider for 6GR. Network should be able to enable or disable additional TRPs/multi-TRP operations. 6GR should minimize always-on signals for multi-TRPs operation. For example, beam management may be dynamically turned on / off or adapted depending on a UE/network power state. When UE is in a power saving state (or operate in WUS monitoring phase), beam management should be minimized (e.g., a minimum set of beam detection RSs is measured). - [36] … mechanism to dynamically indicate the TRP on/off status and solutions to modify original configuration based on this indication would be desirable to minimize overhead and ensure robust performance. In 5G, switching between sTRP and mTRP operation has been discussed based on mTRP unified TCI state framework. Therefore, the dynamic TRP on/off can be further studied with mTRP unified TCI state framework as starting point | ZTE, Huawei, Ericsson, MediaTek, Qualcomm, NTT DOCOMO, OPPO, vivo, Kyocera, Spreadtrum, Ofinno, China Telecom, Lenovo, Sharp, ETRI, Rakuten, xiaomi |
| 2 | 6GR target scenario(s) – Hybrid/analog beam-forming, e.g., for new spectrum | Hybrid/analog beam-forming in NW, e.g., around 4/7GHz, around 30GHz, etc - [4] … For ultra large antenna arrays, the number of antenna ports is increased to provide rich multipath and spatial degrees of freedom, and spatial multiplexing through finer beamforming can further enhance the throughput of single user and multi-user MIMO. In general, full digital beamforming can lead to high hardware overhead and seems to be limited in FR1 deployment. The hybrid beamforming technique in NR can be also the fundamental beamforming architecture for ultra large antenna arrays. In other words, 6G shall support digital, analog and hybrid beamforming architectures; - [7] … The 6GR introduces the 7-24 GHz frequency band. Although this band experiences less attenuation than millimeter waves, it suffers greater attenuation compared to lower frequency bands. The deployment objective for the 7-24 GHz band is to utilize base stations designed for lower frequency bands while achieving the same coverage as the lower frequencies. This requires the 7-24 GHz band to also employ a hybrid architecture of analog antennas and digital links to form narrower beams. - [9] … For the frequency range around 7GHz, although the propagation loss at around 7GHz is less severe than that in FR2, the hybrid beamforming architecture would be applied similar to those of FR2. The advantage of hybrid beamforming lies in its ability to significantly reduce system cost and complexity while providing performance close to that of fully digital beamforming. Given that beam management is needed in hybrid beamforming architecture, beam management should be supported for the around 7GHz band. - [31] … physical dimension of the panel to grow from ~0.6m to 1.5m or 2.0m in Upper Mid-Band (~7GHz). When this happens the Rayleigh distance, which define the near-field boundary and is proportional to the square of the physical length/width of the antenna array, jumps from around 6 meters at 3.5GHz to around 100 meters at 7GHz (even bigger distance for coherent Widely Spaced Multi-Panel (WSMP) architecture). This means that a considerable number of served UEs will be found in the near field … the impact of analog phase shifters in hybrid beamforming architectures on the TCI framework for BM in upper-mid frequency bands from 7.125 GHz to 24.25 GHz | ZTE, Ericsson, Huawei/HiSi’, Samsung, Spreadtrum, TCL, CATT, LGE, Panasonic, |
| 3 | Target NW scenario(s) – high-speed train (HST) | High-speed-training scenario - [10] … The High-Speed Train (HST) scenario represents a critical and defining use case for 6GR BM/mobility, pushing the limits of any network architecture. The extreme velocity of HST renders traditional, reactive RRC-based handovers entirely inadequate, as the latency of the procedure far exceeds the UE's brief dwell time within a single cell's coverage. This necessitates a fundamental shift towards a proactive, RRC-free mobility framework. The predictable trajectory of a train is a key asset that should be leveraged, allowing the network to pre-configure a UE or train-mounted relay with a sequence of L1 profiles for all cells/TRPs along a track segment. - [18] …, one potential use case of multi-TRP transmission is high-speed train (HST) scenario. It experiences severe Doppler frequency shift variations along with highly concentrated traffic demand within a specified time interval. Although Rel-15/16 NR supports FR2 deployments via multi-beam operation, signalling latency and overhead especially for beam indication are high for common beam operation. It hampers deployments in high-speed scenarios. [10] Example for cell/TRP cluster operation under HST | MediaTek, ZTE, CMCC, BJTU, China Telecom, |
| 4 | Target NW scenario(s) – DL/UL TRP decoupling | DL/UL-TRP decoupling, e.g., as a motivation for UL beam management enh. - [23] … a decoupled DL/UL-TRP deployment, where a DL-TRP and a UL-TRP are geographically separated and connected via backhaul. As a result, the traditional assumption of DL/UL beam correspondence or reciprocity may no longer hold. This decoupled multi-TRP deployment can enable several attractive use cases. For example, it can support network energy saving by selectively activating or deactivating UL or DL functionalities depending on traffic conditions. - [25] … To address the coverage issue, both UE-side solutions (e.g., UL beamforming, repetition, power boosting, etc.) and NW-side solutions have been considered in releases in 3GPP. Among the NW-side solutions, many operators have been proposed to support UL only TRP where small and light TRPs having only UL functionality are deployed to enhance UL coverage. In this regard, some enhancements have been introduced in Rel-19/20 NR MIMO such as 2TA and UL PC enhancement. From BM perspective, separate UL BM from DL BM is quite important for this scenario so that UL beam is targeted to a UL only TRP and DL beam is targeted to a TRP with DL transmission. - [8] … how UE does beam search e.g. hierarchical beam search and whether the network can save some overhead of sending the same beam on different types of RS. Similarly, for uplink, more information on beam relationship can help the network to do fast beam search especially for the case where uplink/downlink beam correspondence does not hold (e.g. UL-only TRP). [25] DL/UL TRP decoupling in UL-TRP/HetNet | IDC, ZTE, NTT DOCOMO, LGE, |
| 5 | Device type(s) | New realistic irregular handheld UT model introduced by R19 channel model - [8] … considering the newly introduced UT model in 38.901 and the significant differences in the measured RSRP at different receiving antenna ports, 6G needs to consider the adaptability of different UE models, including the UT model. For example, through measurement and reporting from different UE antenna ports, dynamic indication, switching, and on/off operations of UE antenna ports or panels can be achieved. [8] R19 introduced realistic irregular handheld UT model (left), and CDF of RSRP difference(s) between UE antenna ports of legacy UE vs new UT (right) Multi-UE panel operation - [36] … In 5G, …when UE rotates/moves with asymmetric panels, it can update the UL panel port number for the UL beam indicated by the reported DL RS. In addition, this also allows UE to dynamically turn on/off ports for energy efficient operation. After receiving the report, gNB will schedule SRS with the reported port number for UL sounding to determine TPMI … In 6G, similar concept can be extended to DL. To support asymmetric panels with different port numbers and to facilitate DL spatial domain adaptation based on DL traffic demand, UE can further report max DL rank for each reported DL RS in the beam report. - [9] … Dynamic UE panel on/off -> UE capability updates of UL port or DL rank - [18] … For 6G multi-panel UE reporting, the panel-specific report of UE-panel states, e.g. inactive, active for DL /UL measurement, active for DL reception only, active for UL transmission, or other combinations of UE-panel states can be studied by considering the larger number of UE panels simultaneous transmission for 6G high-end UEs. The linking or association of UE panels with CSI-RS/SSB resources or resource sets, SRS resource sets, and/or PUCCH resource groups, can be further discussed - [33] …regarding the uplink, enhancements for multi-panel UEs (MPUE) would be required to find good UL beam pairs based on local blockage conditions especially for FR2, thereby improving UL coverage and reliability...Study UL beam management enhancement for UE with multiple panels to find good UL beam pairs. Single beam operation in FR2 - [16] … Another potential enhancement is the QCL-TypeD indication. The FR2 UE may use a single-beam operation to communicate with the network, e.g., for UE power saving. Then there is no need to indicate the QCL-TypeD for such UE. | Qualcomm, Google, China Telecom, ZTE, CATT, NTT DOCOMO, |
| 6 | Frequency range | Being applicable for all frequency ranges - [1] … beam management for all frequency ranges in 6G and study frequency range dependent beam refinement procedures, including procedures similar to P1, P2, and P3 in NR. - [2] … Support of upper midband (UMB) - [31] … For 6G, it has been discussed to consider upper-mid frequency bands from 7.125 GHz to 24.25 GHz as a part of 6G spectrum. For upper-mid frequency bands from 7.125 GHz to 24.25 GHz, because of shorter wavelength, antennas can be smaller which allows the base station to pack hundreds of antenna elements into the same physical space used by a 3.5 GHz antenna. | Nokia, Futurewei, Panasonic, |
| 7 | AI/ML for 6GR beam management | AI/ML based beam measurement and report (7 sub-cases) - Sub-case A: Inter-Cell/M-TRP DL Tx beam prediction and management; - Sub-Case B: Cross frequency DL Tx beam /beam pair prediction; - Sub-Case C: Tx-Rx beam pair prediction; - Sub-Case D: Beam prediction for initial access; - Sub-Case E: DL Tx beam prediction for spatial and/or temporal domain with additional local UE information; - Sub-Case F: reinforcement learning-based approach beam selection Besides, some other advanced mechanisms, e.g., compressed sensing [6,8], Low-interference/weak-DL-Tx beam reporting [5] [8] [16] and Online Sensing-Aided Geometric Prediction [34], are proposed. | All COMPANIES |
| 8 | Requirements on unifying framework of beam management and mobility | Unified design for beam measurement and mobility - [5] … considering the redundancy as given above, we believe it is essential to support a unified beam reporting framework for MIMO and mobility. For more detailed discussions regarding unified BM for MIMO and mobility, please refer to our companion papers - [13] … To avoid the above issues in NR and strive for a simple system design in 6G, a unified and simplified beam measurement and report configuration framework for different scenarios should be studied. As beam measurement on serving cell and report can be achieved based on either measurement and report configuration framework for ICBM or for LTM, and both of them also support inter-cell scenarios, they could be considered. - [19] … to reduce overhead/latency and achieve energy saving gains during mobility procedures, we propose to introduce layer-1 (L1) mobility, in which measurement/reporting procedures, beam indication and cell switching signaling can all be based on L1 signaling or more specifically DCI based indications. For instance, at least for some deployment scenarios, the cell switching command (CSC) MAC CE can be replaced by DCI based indications providing “seamless” cell switching experience to the UE, which can be a prominent candidate technology for 6GR… - [25] … In 6GR, L1 based beam management operations are supported for DL Tx/Rx beam selection and UL Tx/Rx beam selection purposes. Consider how to support beam-level mobility, e.g., in L1 with a common signaling framework for other BM operations or in L2 with a separate signaling framework as in NR LTM. - [29] … RAN1 defines measurements to support mobility procedures and … makes no distinction between measurements used for beam-level and cell-level mobility. - [31] … in 6GR, to achieve seamless connectivity and free the UE from the constraints of a traditional cell boundaries, as well as reduce latency and signaling overhead, it is valuable to study whether a unified framework could support both BM and mobility via L1/L2/L3 procedures in 6GR. The unified framework should be considered under different deployment scenarios, e.g., intra-cell/inter-cell BM, single-DCI-based multi-TRP, multi-DCI-based multi-TRP, single cell with multiple carriers, etc. The unified framework should also consider FR2 RF propagation aspects such as extremely narrow pencil beam and analogue beam sweeping. | Huawei/HiSi’, vivo, Samsung, Ericsson, Spreadtrum, LGE, ZTE, Panasonic, Sony, CEWiT, |
| 9 | Other requirements | Requirement for beam training - [29] … Future deployments at higher bands will have a large number of beams, e.g. 1000+ beams, and it is important for NW to quickly find the best beam to serve the UE. One solution is to use the Rel-19 beam prediction feature to enable the UE to predict the best beam and report such predictions to the NW. Delivery of other key transmission parameter(s) - [10] … To support DL/UL beam switching among different TRPs/cells, 6GR should study the essential L1 parameters other than QCL assumptions (e.g., PCI, UL power-control references/parameters, timing advance, cell-specific DL/UL control channel resources) that can be updated based on or along with TCI state indication/update w/o RRC reconfiguration TN vs NTN - [27] … SI includes harmonization of TN and NTN, beam management procedures may need to account for NTN‑specific conditions. A study for an early identification of common elements of beam management between TN and NTN will be crucial. - [28] … Strive to design unified beam management and bandwidth operation for both TN and NTN. Beam management and bandwidth operation, which are required for either of NTN scenario or coverage limited TN scenario, can be considered as a part of the unified design. On top of the unified design framework, it is important to study NTN-specific features for beam management and bandwidth operation as well, to minimize UE/gNB/satellite complexities and implementation burdens, in Rel-20 6GR SI. | Ericsson, MediaTek, Sharp, ETRI, |

Table 4.1.2 Company input for Observation 4.1.2

| Company | Input |
| --- | --- |
| Mod V00 | - Please input your further views/inputs on observation 4.1.2, if any important information is missing. |

    - Scope of NW-initiated beam management

Moderator thanks companies’ valuable inputs on scopes/objectives of 6GR beam management (NW-initiated). Based on the inputs on lessons learnt and targeted scenarios/requirements, the following proposal is proposed for companies’ check and comment:

Proposal 4.1.3:

On NW-initiated beam management for DL and UL of 6GR, at least of following aspects should be included:

- QCL-related aspects, e.g., definition of QCL/beam, QCL type/chain

- Beam measurement/report/indication within a same TRP in a cell;

- Beam measurement/report/indication among different TRPs, i.e., multi-TRP operation, in a cell;

- Beam measurement/report/indication across different cells, i.e., inter-cell

Note: Strive to have a unified framework for beam management for single/multi-TRP, intra/inter-cell, and lower-layer mobility.

Table 4.1.3 Company input for Proposal 4.1.3

| Company | Input |
| --- | --- |
| Mod V00 | - Please input your views on proposals 4.1.3, i.e., scopes/objectives of 6GR beam management (NW-initiated). |

  - UE initiated procedure

    - Overview of UE-Initial Beam Management Procedure

Network-initiated reporting typically relies on periodic or aperiodic triggers, which can introduce reporting latency and unnecessary signaling overhead. In contrast, user UE-initiated reporting is event-driven, enabling the UE to report changes only when necessary—for example, when a superior beam pair is identified or when the current beam degrades, based on measurements or even AI-based prediction without explicit measurement. This proactive reporting approach reduces downtime associated with waiting for network-side measurement cycles, thereby helping to prevent radio link failures and maintain higher data rates.

As summarized in Table 4-1, companies were unanimous in proposing studies or reaching conclusions that support UE-initiated beam reporting for 6GR in contributions, citing key advantages over NW-initiated reporting, including lower overhead and reduced reporting latency.

Table 4-1: On UE-Initial Beam Management in 6GR

|  | Proposals | Supported companies (#) |
| --- | --- | --- |
| 1 | Study or support UE-Initiated beam management procedure for 6GR | - Ericsson, Qualcomm, Huawei, Samsung, Nokia, ZTE, MediaTek, CMCC, NTT DoCoMo, Apple, CATT, Xiaomi, vivo, AT&T, OPPO, Ofinno, NEC, Lenovo, Interdigital, LGe, Panasonic, Sony, Fraunhofer IIS, Spreadtrum, TCL, China Telecom, ETRI, CEWiT, Transssion Holding (29) |

UE-initiated/event-driven beam reporting was introduced in Release 19. There are several lessons raised in company contributions identified learned from 5G NR and limitations, including functional overlap between UEIBR Event 1 and the Release 15 beam failure recovery (BFR) procedure, as well as a limited use case restricted to sTRP. These aspects should be considered and mitigated in the design of UE-initiated beam management for 6GR. Table 4-2 presents the lessons learned that are identified in company contributions.

Table 4-2: Lessons learned from legacy system

|  | Legacy Problems raised by companies | Reported companies |
| --- | --- | --- |
| 1 | Overlapping BFR and UE-initiated beam reporting functions cause spec redundancy and inefficiency. | Huawei, ZTE, MTK, Samsung, vivo, CATT, Xiaomi, OPPO, Lenovo, Sony |
| 2 | LTM and UEIBR require different report containers, one is MAC-CE and the other is UCI, complicating the specification and implementation | Nokia, NTT DoCoMo, vivo, Samsung, vivo |
| 3 | 5G UE-initiated beam reporting supports sTRP only, not mTRP | Qualcomm, CMCC, Xiaomi, NEC, Lenovo |
| 4. | TCI state activation latency degrades throughput and lacks optimization for UE-initiated beam reporting. | MTK, Samsung, Spreadtrum |
| 5 | In NR, the UE-initiated CSI reporting was limited to beam information | Ofinno |
| 6 | Periodic/'always-on' RS for beam failure recovery creates excessive network overhead and high energy demands in 5G NR | Interdigital |

| Companies are welcome to share their views if anything raised in contribution is missed in Table 4-2. |  |
| --- | --- |
| Company | Comments |

    - Target scenarios of 6GR UE-Initiated Beam Management Procedure

Deployment scenarios fundamentally shape how a feature is defined. Without a clear scope of use cases, there is a risk that UE-initiated beam reporting in 6GR looks good on paper but breaks—or wastes resources—when deployed. Table 4-3 summarizes the deployment scenarios and use cases UEIBR discussed by several companies for 6GR system.

Table 4-3: Deployment scenario and use cases in 6GR for UE-Initiated beam reporting

| Index | Scenario(s) | Reported companies |
| --- | --- | --- |
| 1 | Single TRP and multiple TRP use cases | Qualcomm, Huawei, ZTE, CMCC, Xiaomi, NEC, Lenovo, Interdigital, Spectrum, China Telecom |
| 2 | Intra-cell and Inter-cell use case | Huawei, ZTE, NTT DoCoMo, Xiaomi, lenovo, Panasonic, Spectrum, China Telecom |
| 3 | Decoupled DL/UL-TRP deployement | Interdigital |

| Companies are welcome to share their views, if any scenario and use cases proposed in contribution is missed in Table 4-3. |  |
| --- | --- |
| Company | Comments |

    - Scope of UE-initiated beam management in 6GR

In addition, several companies discussed the study scope of UE-initiated beam reporting for 6GR, as summarized in Table 4-4. This includes event definitions, the components of UE-initiated beam reporting (e.g., resource configuration, report format and content, and containers), as well as mechanisms to reduce beam application time.

Table 4-4: Study scope of 6GR UEIBR proposed by companies

|  | Study Scope of UEIBR Operation |  | Supported companies |
| --- | --- | --- | --- |
| 1 | - Event definition and corresponding procedure - For each event, - Measurement resource configuration - report format/content - report container. - Necessity of network’s response to the UEIBR including TCI state update. |  | - ZTE, Samsung, OPPO, NTT DoCoMo, NEC, Interdigital |
| 2 | Integrate Beam Failure Recovery into the UE-initiated beam management |  | - Huawei, ZTE, MTK, Samsung, vivo, CATT, Xiaomi, OPPO, Lenovo, Sony |
| 3 | Mechanism to reduce beam application time for UEIBR | - Study beam application time reduction | - Samsung |
|  |  | - Study UE-based TCI state activation and indication | - Qualcomm |
|  |  | - Study UE autonomous TCI state/beam switch for FR1 and FR3 | - Qualcomm, Sony |
|  |  | - Study and specify UE declared readiness for downlink reception (optionally uplink transmission) on candidate TCI state(s), | - MTK |
| 4 | Unified framework should be studied for MIMO and Mobility |  | - Nokia, NTT DoCoMo, vivo, Samsung, vivo |
| 5 | Extension of UE-initiated reporting to additional CSI contents beyond beam information. |  | - Ofinno |

| FL Proposal 4-1 Study of UE-initiated/event-driven beam reporting (UEIBR) mechanisms for 6GR, covering at least the following aspects: - Targeting for sTRP and mTRP deployment scenarios with intra-cell, inter-cell, and Cell-free use cases. - Key study areas include: - Target use case for each event - Event definition, triggering conditions and related procedures. - Measurement resource configuration - Report format and content - Report container, with the objective of a unified report container for both UEIBR and mobility use cases. - The necessity and design of network’s response to the UEIBR including TCI state update. - Mechanism to reduce beam application time for UEIBR. |  |
| --- | --- |
| Company | Comments (Please provide modified propoal if it is acceptable in general but something needs to be revised) |

- Evaluation methodology (EVM) for beam management

This section synthesizes observations and proposals on evaluation methodology (EVM) for evaluating/studying beam measurement, report and indication procedure among different companies, including various deployment scenario(s) (single TRP, multi-TRP/cell-free), different frequency range(s) (around 4GHz/7GHz, around 15GHz, around 30GHz), classical and advanced beam measurement (e.g., AI/ML, compressed sensing, etc.), and NW/UE-initiated report/switching procedure.

Per companies’ in-depth input, candidate templates for system and link level simulations can be found as follows:

- As in the first step, we stabilize the first column on which ‘transmission parameters’, i.e., ‘attributes’, should be provided in the respective table(s);

- After that, we can try to stabilize the contents of each rows in the table per companies’ input accordingly (e.g., in RAN1#124-bis).

Table 5.1 Template for link-level evaluation of 6GR beam management

| Parameter | Value | Note (examples for interpretation only) |
| --- | --- | --- |
| Carrier Frequency |  | e.g., 4GHz, 7GHz, 30GHz |
| Duplex / Waveform |  | e.g., TDD, CP-OFDM |
| Subcarrier Spacing |  | e.g., For 4GHz/7GHz: 30KHz; For 30GHz:120KHz |
| Simulation Bandwidth |  | e.g., 8 RBs for data allocation; Note: First 2 OFDM symbols for PDCCH, and following 12 OFDM symbols for data channel |
| Channel Model |  | e.g., CDL-A /B/C model |
| Delay Spread |  | e.g., 100ns |
| gNB Antenna Config |  | e.g., per outcome from A.I. 11.1 |
| Multi-TRP operation |  | e.g., the number of TRP(s) = {1, 2, 4} - CDL channel model is generated per TRP independently; - Backhaul: ideal or non-ideal; - sync: ideal or non-ideal; |
| TXRU mapping to antenna elements |  | e.g., A single TXRU is mapped per panel per subarray per polarization |
| Beam-forming scheme |  | e.g., Companies explain the details, e.g., DFT-based TXRU mapping weights and/or digital beam-forming |
| Procedure of beam sweeping |  | e.g., Companies explain the details, e.g., DFT-based |
| Criteria for beam selection |  | e.g., To maximize RSRP/SINR |
| UE Antenna Config |  | e.g., per outcome from A.I. 11.1 |
| BS array orientation |  | e.g., azimuth 0°, mechanic downtilt: 90° in GCS. |
| UE array orientation |  | e.g., ΩUT,a uniformly distributed on [0, 360] degree, ΩUT,b = 0°, ΩUT,g = 0° |
| UE antenna element radiation pattern |  | For 4 GHz/7GHz: - Option-1: Omni-directional with 0dBi gain; - Option-2: According to Table 7.3-2 in TR 38.901 (radiation power pattern for handheld UT) For 30 GHz: See Table A.2.1-8 in TR 38.802 |
| UE Velocity |  | e.g., 30 km/h, 3km/h. |
| MIMO Scheme |  | e.g., SU-MIMO as baseline |
| Receiver Type |  | e.g., MMSE-IRC as baseline; other advanced receiver is not precluded |
| Channel Estimation |  | e.g., Idle or realistic |
| MCS |  | e.g., NR MCS |
| Performance Metric |  | e.g., Top-1/K beam prediction accuracy, BLER or Spectrum Efficiency (SE) |

Table 5.2 Companies’ input on link-level evaluation of 6GR beam management

| Company | Input |
| --- | --- |
| Mod V00 | Please input your views on template for link-level evaluation of 6GR beam management (e.g., for evaluating QCL-related aspects, beam measurement and reporting (AI/non-AI), etc.) - Note: in this round, we only focus on the first column of first column on which ‘transmission parameters or attributes’ should be provided in the corresponding table. |

Table 5.3 Template for system-level evaluation of 6GR beam management

| Parameter | Value | Note (examples for interpretation only) |
| --- | --- | --- |
| Scenario (Carrier frequency) |  | e.g., Indoor hotspot, Urban macro, Dense urban (one-layer, and two-layer) |
| Mode |  | DL/UL SU-MIMO/MU-MIMO |
| System BW |  | e.g., 4GHz: 20MHz (DL+UL) 7GHz: 100 MHz (DL+UL); 30GHz: 100 MHz |
| Subcarrier spacing for data |  | e.g., 4GHz/7GHz: 30KHz; 30GHz: 120 KHz |
| Channel model |  | e.g., TR 38.901 with spatial consistency |
| BS Antenna Configuration |  | e.g., per outcome from A.I. 11.1 |
| TXRU mapping to antenna elements |  | e.g., A single TXRU is mapped per panel per subarray per polarization |
| Beam-forming scheme |  | e.g., Companies explain the details, e.g., DFT-based TXRU mapping weights and/or digital beam-forming |
| Multi-TRP operation |  | e.g., maximum number of coordinated TRP(s) = {1, 2, 4} - Backhaul: ideal or non-ideal; - sync: ideal or non-ideal; |
| Criteria for selection for serving TRP(s) |  | e.g., Companies explain the details |
| Criteria for beam selection for serving TRP(s) |  | e.g., Companies explain the details |
| UE Antenna Configuration |  | e.g., per outcome from A.I. 11.1 |
| Inter-panel calibration for UE |  | e.g., Ideal, non-ideal (optional) – Explain any errors |
| Beam correspondence |  | e.g., Ideal, non/partial beam correspondence – explain details. |
| Link adaptation |  | e.g., based on CSI-RS |
| UE receiver type |  | e.g., MMSE-IRC as baseline; other advanced receiver is not precluded |
| BS Tx Power |  | e.g., per outcome from A.I. 11.1 |
| Maximum UE Tx Power |  | e.g., per outcome from A.I. 11.1 |
| Inter site distance |  | e.g., per outcome from A.I. 11.1 |
| UE Speed and UE distribution |  | e.g., per outcome from A.I. 11.1 |
| UE rotation |  | e.g., No UE rotation, or UE rotation – explain details |
| BS Antenna height |  | e.g., macro: 25m, micro: 10m, indoor: 3m |
| UE Antenna height |  | e.g., 1.5 m |
| Car penetration Loss |  | e.g., 38.901, sec 7.4.3.2: μ = 9 dB, σp = 5 dB |
| UE Velocity |  | e.g., 30 km/h, 3km/h. |
| UE mobility feature (optional) |  | e.g., Option 3: Random direction straight-line trajectories (from 38.843). - Initial UE location, moving direction and speed: UE is randomly dropped in a cell, and an initial moving direction is randomly selected, with a fixed speed. - The initial UE location should be randomly drop within the following blue area: where d1 is the minimum distance that UE should be away from the BS. - Each sector is a cell and that the cell association is geometry based. - During the simulation, inter-cell handover or switching should be disabled. |
| Scheduling algorithm |  | e.g., PF scheduler |
| MCS |  | e.g., Use NR MCS |
| Traffic Model |  | e.g., FTP model 1/3 with packet size 0.5 Mbytes (other value is not precluded) Other traffic models including full buffer are not precluded |
| CSI Feedback |  | e.g., with a periodicity of 20ms |
| Resource Utilization |  | e.g., 30%, 50% |
| Metric |  | e.g., Top-1/K prediction accuracy, average UPT, 5%-ile UPT Companies can report other observations, e.g., outage, latency, SINR, RSRP |

Table 5.4 Template for system-level evaluation of 6GR beam management

| Company | Input |
| --- | --- |
| Mod V00 | Please input your views on template for system-level evaluation of 6GR beam management - Note: in this round, we only focus on the first column of first column on which ‘transmission parameters or attributes’ should be provided in the corresponding table. |

- NW initiated beam management

  - QCL-related aspects

FL note 1: Including QCL-related aspect, e.g., definition of QCL/beam, QCL type and chain, etc.

Table 6.1.1 Summary for QCL-related aspects

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 6.1.1 | Definition of QCL/beam | FL note: Per companies input, the following QCL definition from NR is highlighted, and then the details on source QCL-RS, QCL assumptions for CJT, and wide/narrow beam(s) can be further studied. - [5G-NR QCL definition]: Two antenna ports are said to be quasi co-located if properties of the channel over which a symbol on one antenna port is conveyed can be inferred from the channel over which a symbol on the other antenna port is conveyed. Proposed 6.1.1: Study QCL definition for DL and UL beam management, and corresponding QCL relationship/chain between source QCL-RS(s) and target channel(s)/RS(s). - Study whether/how to introduce other RS (e.g., DMRS) as a source RS of QCL state, - Study whether/how to introduce more than one QCL assumptions for a RS (e.g., CSI-RS or DMRS) for accommodating multi-TRP-CJT operation. - Study whether/how to distinguish among the cases of the same beam relationship and wide-narrow beam relationship. Supported by: Nokia, ZTE, CMCC, Ericsson, China Telecom, Fraunhofer IIS, Fraunhofer HHI, CEWiT |

Table 6.1.2 Companies’ input on QCL-related aspects

| Company | Input |
| --- | --- |
| Mod V00 | - Please input your views on proposals 6.1.1, i.e., QCL-related aspects. |

  - DL beam measurement and report (NW initiated)

FL note 1: Including beam measurement and report, e.g., beam sweeping procedure (P1/2/3), framework for targeted TRP operation mode (sTRP or mTRP, DL or UL), types of reference signal and corresponding time-domain behavior, and report metrics (e.g., L1-RSRP, L1-SINR), and report format, etc.

FL note 2: Including advanced beam measurement and report, e.g., AI/ML-based (spatial or temporal, NW/UE-side model, sTRP or mTRP), compressed sensing, etc.

FL note 3: Including framework of signaling beam measurement and report, e.g., a framework of resource set, resource setting, report setting, etc, which may be coordinated with AI 10.5.3.1.

Table 6.2.1 Summary for DL beam measurement and report (NW initiated)

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 6.2.1 | Beam measurement and report | FL note: Per companies input, the following is to support the basic framework/description of beam measurement and report for sTRP and mTRP, and then to clarify the related aspects for further study. Proposed 6.2.1: Support DL beam measurement and report procedure on DL RS(s) for both single-TRP and multi-TRP (including intra/inter-cell) scenario(s). - DL RS at least comprises sync signal or CSI-RS. - The following aspects can be further studied: - Basic procedure of beam sweeping, e.g., P1/2/3, inter/intra-symbol beam sweeping, - Measurement metrics, e.g., RSRP or SINR based - Time-domain behavior for both beam measurement and report, e.g., aperiodic, semi-persistent, periodic - Report format with/without the assumption of simultaneous reception, e.g., group and non-group based beam report - Early beam report/refinement during initial access Supported by: ZTE, Huawei, Ericsson, MediaTek, Qualcomm, NTT DOCOMO, OPPO, vivo, Kyocera, Spreadtrum, Ofinno, China Telecom, Lenovo, Sharp, ETRI, Rakuten, AT&T, Fraunhofer IIS, Fraunhofer HHI, CEWiT |
| 6.2.2 | Advanced beam measurement and report | FL note: Per companies input, re-organize the advanced beam measurement and report (including AI and non-AI). Proposed 6.2.2: Regarding DL beam measurement and report (NW initiated), the following aspects (relevant to AI/ML or non-AI/advanced schemes (e.g., compressed sensing)) can be further studied: - Spatial domain DL Tx beam prediction and management - FFS: Applicable for sTRP (e.g., BM-case1 as in Rel-19) and/or inter-Cell/M-TRP (e.g., Sub-case A in Table E1 for BM) - FFS: additional local UE information (e.g., Sub-case E in Table E1 for BM) - Temporal DL Tx beam prediction and management - FFS: Applicable for sTRP (e.g., BM-case2 as in Rel-19) and/or inter-Cell/M-TRP (e.g., Sub-case A in Table E1 for BM) - FFS: additional local UE information (e.g., Sub-case E in Table E1 for BM) - Cross-frequency DL Tx beam/beam pair prediction (e.g., Sub-case B in Table E1 for BM) - Dynamically computed analog beam alignment, e.g., accurate angular estimation [5], [6], [19], [36] - Low-interference/weak-DL-Tx beam reporting to provide explicit interference information among beams for beam-based scheduling [5] [8] [16] (e.g., Sub-Case G in Table E1 for BM) - Others are not precluded, e.g., Tx-Rx beam pair prediction (e.g., Sub-case C in Table E1 for BM) Supported by: Ericsson, Qualcomm, Nokia, Apple, Futurewei, Huawei, ZTE, Samsung, NEC, China Telecom, IDC, Fujitsu, Sharp, ETRI, Transsion, MediaTek, CMCC, NTT DOCOMO, TCL, CATT, Ofinno, Google, Sony, AT&T, Fraunhofer IIS, Fraunhofer HHI, CEWiT |
| 6.2.3 | Framework of signaling beam measurement and report | FL note: Per companies input, the framework of signaling beam measurement and report should be studied for supporting sTRP/mTRP (intra/inter-cell) beam measurement. BTW, as critical requirements on general framework of RS measurement and CSI report, this discussion and subsequent outcome may be relevant to the final decision in A.I. 10.5.3.1. Proposed 6.2.3: Regarding DL beam measurement and report (NW initiated), further study framework of signaling beam measurement and report. Supported by: Futurewei, Huawei, ZTE, Samsung, MediaTek, Ericsson, Qualcomm, Nokia, CMCC, NTT DOCOMO, CATT, Ofinno, Google, NEC, China Telecom, IDC, Fujitsu, Sharp, ETRI, AT&T, |

Table 6.2.2 Companies’ input on DL beam measurement and report (NW initiated)

| Company | Input |
| --- | --- |
| Mod V00 | - Please input your views on proposals of 6.2.1~6.2.3. |

  - UL beam measurement and related procedure (NW initiated)

FL note 1: Including UL beam measurement, e.g., usage of UL beam measurement (non-beam correspondence, asymmetric), types of reference signal and time-domain behavior, UL-beam sweeping procedure (U1/2/3), framework of signalling measurement, and principle for enabling UL multi-panel, if needed.

FL note 2: Including others on UL beam management, e.g., advanced UL-beam measurement, if any, e.g., AI/ML-based (spatial or temporal, NW/UE-side model), compressed sensing and others, etc.

Table 6.3.1 Summary for UL beam measurement and related procedure (NW initiated)

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 6.3.1 | UL beam measurement | FL note: First of all, for 6GR, we need to identify the usages/scenarios of introducing or enhancing UL beam measurement, while considering that, with beam correspondence, DL beam measurement and joint DL and UL TCI indication can be considered as a baseline/back-up. Proposed 6.3.1: Study UL beam sweeping and measurement at least for the following usages: - UL metric aware beam selection, e.g., due to MPE - Non/partial-beam correspondence - Asymmetric DL sTRP and UL mTRP - … Supported by: Qualcomm, OPPO, MediaTek, NTT DOCOMO, ZTE, China Telecom, LGE, Sharp, Panasonic, IDC, |

Table 6.3.2 Companies’ input on UL beam measurement and related procedure (NW initiated)

| Company | Input |
| --- | --- |
| Mod V00 | - Please input your views on proposals of 6.3.1 |

  - Beam indication (NW initiated)

FL note 1: Including definition of unified-TCI (i.e., joint, or separate DL and UL), and any other parameter(s) carried in TCI state besides for QCL, e.g., UL power control or TAG ID, etc.

FL note 2: Including beam application procedure, e.g., target channel(s)/RS(s), beam application timing, independent/extra-signaling for other cases (e.g., common PDCCH, CSI-RS, SRS), Cross-CC beam indication procedure, etc.

Table 6.4.1 Summary for beam indication (NW initiated)

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 6.4.1 | Basic TCI indication | Proposed 6.4.1: Regarding NW initiated beam indication, support unified transmission configuration indicator (TCI) indication framework for both single-TRP and multi-TRP operations - Common TCI state(s) can be applied to PDSCH, PUSCH, CSI-RS (optional), SRS (optional), PDCCH (optional) and PUCCH (optional); - One TCI state comprises at least one resource QCL-RS and QCL type(s). - FFS: Separate TCI indication/activation/configuration procedure for channel/RSs which do NOT follow the unified TCI indication. - Note: TCI/QCL state is for the purpose of discussion. Supported by: Apple, Futurewei, Huawei, ZTE, Samsung, MediaTek, Ericsson, Qualcomm, Nokia, CMCC, NTT DOCOMO, TCL, CATT, Ofinno, Google, NEC, China Telecom, IDC, Fujitsu, Sharp, ETRI, Transsion, Sony, AT&T, Fraunhofer IIS, Fraunhofer HHI, CEWiT |
| 6.4.2 | Advanced feature | Proposed 6.4.2: Regarding NW initiated beam indication, further study the following aspects - Temporal domain beam prediction-based beam switching/indication - Other transmission parameter(s) for DL and UL provided by the unified TCI indication, besides for QCL/spatial-filtering assumption. - Configuration framework and applicable range of TCI state(s), e.g., BWP-specific, cell-specific, etc. Supported by: ZTE, Apple, Qualcomm, xiaomi, NEC, China Telecomm, OPPO, IDC, |

Table 6.4.2 Companies’ input on beam indication (NW initiated)

| Company | Input |
| --- | --- |
| Mod V00 | - Please input your views on proposals of 6.4.1~6.4.2 |

- UE initiated beam management

[Reserved]

- Proposals for Online Discussion

[Reserved]

# References

| 1 | R1-2600038 | On beam management for downlink and uplink in 6GR | Nokia |
| --- | --- | --- | --- |
| 2 | R1-2600056 | Discussion on beam management for downlink and uplink for 6GR air interface | FUTUREWEI |
| 3 | R1-2600093 | Beam Management for Multi-TRP transmission in 6GR | Kyocera Corporation |
| 4 | R1-2600118 | Discussion on beam management for downlink and uplink for 6GR | Spreadtrum, UNISOC |
| 5 | R1-2600150 | Beam management for downlink and uplink | Huawei, HiSilicon |
| 6 | R1-2600193 | Discussions on 6GR multi-beam operation | OPPO |
| 7 | R1-2600221 | Beam management for downlink and uplink | TCL |
| 8 | R1-2600230 | Discussion on beam management for downlink and uplink | ZTE Corporation, Sanechips |
| 9 | R1-2600305 | Discussion on beam management for 6GR | CATT |
| 10 | R1-2600347 | Beam management and mobility | MediaTek Inc. |
| 11 | R1-2600395 | Discussion on beam management for downlink and uplink transmission | CMCC |
| 12 | R1-2600435 | Discussion on beam management for downlink and uplink | Xiaomi |
| 13 | R1-2600510 | Discussion on beam management for 6GR downlink and uplink | vivo |
| 14 | R1-2600568 | Discussion on AI-Driven Beam Management Methods | BJTU |
| 15 | R1-2600606 | Initial views on Beam Management | Ofinno |
| 16 | R1-2600631 | Beam Management for Downlink and Uplink | Google |
| 17 | R1-2600656 | Beam management for downlink and uplink | NEC |
| 18 | R1-2600699 | Discussion on beam management for downlink and uplink | China Telecom |
| 19 | R1-2600762 | Views on beam management for downlink and uplink in 6GR | Samsung |
| 20 | R1-2600781 | Discussion on beam management for downlink and uplink in 6GR | Lenovo |
| 21 | R1-2600796 | Discussion on beam management for 6GR downlink and uplink | BeammWave AB |
| 22 | R1-2600834 | On beam management aspects for downlink and uplink | Apple |
| 23 | R1-2600852 | Beam management and TCI framework for 6GR MIMO operation | InterDigital, Inc. |
| 24 | R1-2600874 | Discussion on beam management in 6GR | Fujitsu |
| 25 | R1-2600889 | Discussion on beam management for downlink and uplink | LG Electronics |
| 26 | R1-2600912 | Beam management for DL and UL | Panasonic |
| 27 | R1-2600922 | Beam management for downlink and uplink | Sharp |
| 28 | R1-2601009 | Discussion on beam management for 6GR | ETRI |
| 29 | R1-2601027 | Discussion on beam management for downlink and uplink | Ericsson |
| 30 | R1-2601036 | Discussion on beam management for downlink and uplink | Transsion Holdings |
| 31 | R1-2601069 | Discussion on Beam Management Procedures for 6GR | Panasonic |
| 32 | R1-2601135 | Discussion on beam management for downlink and uplink | Sony |
| 33 | R1-2601188 | Discussion on Beam management for downlink and uplink | NTT DOCOMO, INC. |
| 34 | R1-2601214 | Discussion on beam management for downlink and uplink in 6GR | Pengcheng Laboratory |
| 35 | R1-2601224 | Discussion on 6GR Beam Management Framework | AT&T |
| 36 | R1-2601279 | Beam management for downlink and uplink | Qualcomm Incorporated |
| 37 | R1-2601325 | On beam management for downlink and uplink in 6GR | Fraunhofer IIS, Fraunhofer HHI |
| 38 | R1-2601340 | Discussion on Beam Management of Downlink and Uplink | Rakuten Mobile, Inc |
| 39 | R1-2601398 | Discussion on Beam management for 6GR | CEWiT |