3GPP TSG RAN WG1 #124	R1-2601465

Gothenburg, SE, Feb. 9th ~ 13th, 2026

Agenda Item:	10.5.3.3

Title:	FL summary #2 on other aspects related to CSI

Source:	Moderator (Lenovo)

Document for:	Discussion and Decision

Introduction

According to 6GR SID, we have the following objective

- Physical Layer structure for 6GR,

…

  - MIMO operation [RAN1, RAN4]

…

The following email thread is assigned for the discussion

10.5.3.3	Other aspects

Note 1: Including proposals for CSI acquisition and report jointly considering both downlink and uplink, other reference signal(s) design and transmission, e.g., for tracking, etc.

[124-R20-6GR-Other Aspects related to CSI] Email discussion on Rel-20 6GR-Other Aspects related to CSI – Bingchao (Lenovo)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

In this contribution, we summarize the contributions submitted to agenda 10.5.3.3 on other aspects related to CSI in this meeting.

- Work plan

Based on contributions submitted in this agenda item and according to chairman’s guidance, the following aspects of CSI will be handled in this agenda in this meeting

- Aspect#1: Reference signal for time and frequency (T/F) tracking

  - Potential scenarios should be considered

  - Aspects need to be considered for the RS for tracking

  - Evaluation methodology

- Aspect#2: Joint DL and UL based DL CSI acquisition

  - Potential use cases to be evaluated

  - Potential schemes to be evaluated

  - Evaluation methodology

This summary will be used for the 1st online discussion

- Contact Person

For potential offline discussions, companies/delegates are encouraged to provide the contact information in the following table:

Table 1 Contact Information

| Company | Point(s) of contact | Email address(es) |
| --- | --- | --- |
| OPPO | Wendong Liu, Wenhong Chen | liuwendong1@oppo.com chenwenhong@oppo.com |
| MediaTek | Darcy | darcy.tsai@mediatek.com |
| Nokia | Filippo | filippo.tosato@nokia.com |
| Nokia | Mihai Enescu | Mihai.enescu@nokia.com |
| vivo | Hao Wu | hao.wu@vivo.com |
| Samsung | Ameha | amehat.abebe@samsung.com |
| Apple | Huaning, Ankit, Yuan | Huaning.niu@apple.com a.bhamri@apple.com |
| Interdigital | Afshin Haghighat | Afshin.haghighat@interdigital.com |
| NEC | Yukai Gao Peng Guan | gao_yukai@nec.cn guan_peng@nec.cn |
| CMCC | Yuhua Cao | caoyuhua@chinamobile.com |
| Ericsson | Siva Muruganathan Xinlin Zhang | siva.muruganathan@ericsson.com xinlin.zhang@ericsson.com |
| Ofinno | Jaehoon Chung | jchung@ofinno.com |
| ETRI | Woncheol Cho Wooram Shin | woncheol@etri.re.kr w.shin@etri.re.kr |
| Spreadtrum | Dawei Ma Shijia Shao | Dawei.ma@unisoc.com Shijia.Shao@unisoc.com |
| ZTE | Ling Yang Hanchao Liu | yang.ling17@zte.com.cn liu.hanchao@zte.com.cn |
| Futurewei | Weimin Xiao | weimin.xiao@futurewei.com |
| Futurewei | Zhigang Rong | zrong@futurewei.com |
| Futurewei | Baoling Sheen | bsheen@futurewei.com |
| Futurewei | Jialing Liu | Jialing.liu@futurewei.com |
| Sony | Jose Flordelis | jose.flordelis@sony.com |
| Sony | Naoki Kusashima | naoki.kusashima@sony.com |
| AT&T | Ahmed Hindy | ahmed.hindy@att.com |
| Huawei | Yubo Yang | yangyubo1@huawei.com |
| Xiaomi | Guozeng Zheng | zhengguozeng@xiaomi.com |
| Xiaomi | Qin Mu | muqin@xiaomi.com |

Online/Offline proposals

For Wednesday online

Proposal 3.3: Adopt the following simulation assumptions for tracking RS evaluation

Table 2: LLS assumptions for T/F tracking

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD and TDD, CP-OFDM |
| Carrier Frequency/SCS | 700MHz/15kHz, 4GHz/30kHz, 7GHz/30kHz, 30GHz/120kHz |
| Number of TRPs | NTRP=1, 2, 4 Companies should report the transmission assumptions for the RS transmission for tracking in case of more than 1 TRP. |
| Bandwidth | 5MHz, 20MHz, 100MHz |
| MIMO scheme | SU-MIMO, Rank 1,2,3,4 |
| Modulation | QPSK, 16QAM, 64QAM, 256QAM and 1024QAM |
| Precoding granularity | PRG = {TBD, wideband} |
| Channel Model | CDL and TDL channel in TR38.901 For CDL-C for 4, 7 and 30 GHz - with combination of ASA and ASD scaling values in sec. 7.7.5.1 in TR 38.901, for above 6 GHz cases - ZSA = 5°, ZSD = 1° The CDL table is translated so that the strongest cluster’s AoD and AoA occur at a random angle for both the antenna panels of TRP and UE in the local coordinate. The value of the random angle is selected to be uniformly distributed from +30 to -30 degree. The random value is chosen independently for both AoD and AoA. In mTRP cases, the channel is generated per TRP. |
| Delay spread | 30ns, 100ns, 300ns, 1000ns |
| UE speed | 3km/h, 10km/h, 120km/h, 350km/h 500km/h |
| Initial time offset (TO) | 1/X CP, X= 8 Other values can be reported by companies. |
| CFO | Initial acquisition - TRP: uniform distribution +/- 0.05 ppm - UE: uniform distribution +/- 5, 10, 20ppm (each company to choose one) Non-initial acquisition - Per TRP: uniform distribution +/- 0.05 ppm - UE: uniform distribution +/- 0.1 ppm Note 1: Those parameters are used for simulation assumptions for synchronization signals/channels in NR (TR38.802). Note 2: Other values can be reported by companies. |
| Drift rate | 0.2 ppm/sec Other values can be reported by companies. |
| BS antenna configuration | Around 700MHz carrier frequency - 4TXRUs/32AEs: (M,N,P,Mg,Ng; Mp, Np)= (8, 2, 2, 1, 1, 1, 2), (dH,dV) = (0.5, 0.5)λ Around 4GHz carrier frequency - 32TXRUs/128AEs: (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1,2,8). (dH, dV) = (0.5, 0.5)λ Around 7GHz carrier frequency - 128 TXRUs/728AEs: (M, N, P, Mg, Ng, Mp, Np) = (24,16,2, 1, 1, 4,16). (dH,dV) = (0.5,0.8)λ Around 30GHz carrier frequency - 8TXRUs / 512AEs: (M, N, P, Mg, Ng, Mp, Np) = (8, 8, 2, 2, 2; 1, 1), (dH,dV) = (0.5, 0.5)λ |
| UE antenna configuration | Follow the agreement in 10.1. |
| Channel estimation | Realistic channel estimation. Companies can report the CE methods. |
| Performance metric | Estimation error (e.g., CFO/Dopler rmse), BLER, Throughput |

FL proposal 6.0a: For joint DL and UL based DL CSI acquisition, reuse the evaluation methodology agreed in 10.5.3.1 and in 10.5.3.2.

- Note: Specific aspects on joint operation can be reported by companies.

FL proposal 3.1: Study and evaluate the following aspects on RS for time/frequency tracking to guarantee the performance

- Frequency domain factors

  - Bandwidth

  - Frequency domain density

- Time domain factors

  - Time domain behavior: e.g., Periodic, Aperiodic, On-demand

  - Time domain density: e.g., Periodicity, number of symbols and inter-symbol interval

Note: 5G NR TRS is used as the baseline for study.

For Friday online

Reference signal for time and frequency tracking

Issue#1: Scenarios for time/frequency tracking RS

Contributions proposal

| Nokia | Proposal 1: In the study for TRS like signal in 6GR, use cases and corresponding requirements should be clarified. |
| --- | --- |
| FUTUREWEI | Proposal 5: Support early/on-demand tracking acquisition in 6G: - As a mandatory feature for fast SCell/Secondary Component Carrier (SCC) activation - As an optional feature for early CSI acquisition before CONNECTED. Proposal 4: Study to introduce multi-port TRS as a QCL source for DL transmissions using massive MIMO antenna arrays, to reduce the mismatch of the precoding of TRS and that of DMRS/CSI-RS, improving the channel estimation performance. |
| Spreadtrum | Proposal 4: Study whether NR requirements for time/frequency tracking is sufficient for RAN1 to continue future work for 6GR. Proposal 3: For fine time/frequency tracking, consider to reuse CSI-RS design with dedicated configuration. |
| CATT | Proposal 1: Study a dedicated RS (e.g., TRS) for time and frequency tracking for 6GR. |
| CMCC | Proposal 1: Study the m-TRP early TRS triggering during initial access. Proposal 3: In 6G, the TRS configuration may be decoupled from the CSI reporting framework. It is proposed to study a "measurement-only" resource management design where TRS can be activated strictly as a physical layer synchronization signal, independent of a CSI-ReportConfig. Proposal 2: Study a simplified reference signal design for early access in 6G, where a single triggered RS burst can be utilized for both time-frequency tracking and early CSI acquisition, thereby reducing reference signal overhead in multi-TRP/CFA(cell-free area) scenarios. Proposal 4: Study a hybrid configuration framework for TRS in 6G. Specifically, in addition to legacy RRC-based configuration, 6G may introduce SIB-based configuration to enable common TRS availability for UEs in idle/inactive states and to facilitate the configuration of early TRS resources prior to dedicated signaling. |
| vivo | Proposal 1: Study more flexible TRS pattern in time domain in 6GR, including the gap between two TRS symbols and the number of occupied slots. (FR3) |
| Ericsson | Proposal 1: 6G should define a more flexible UE fine synchronization framework that does not always rely on periodic TRS transmissions for all deployments and circumstances. |
| Google | Proposal 1: Support the TRS in 6G based on the 5G TRS structure for connected mode UE. Proposal 2: Support the TRS for idle mode UE, where the TRSs can be one-to-one associated with SSBs. Proposal 3: Support the receive the PDCCH/PDSCH for SIB based on the TRS in idle mode, i.e., TRS assisted demodulation. Proposal 5: Study the RLM adaptation based on the TDCP measured from TRS. |
| Lenovo | The study of CSI-RS design should consider both communication and sensing. |
| Rakuten | Proposal 2: Study enhancements for 6GR multi-TRP coordination, including efficient and scalable frequency synchronization for CJT, e.g., inter-TRP reference signal designs; leveraging external aids for residual Doppler compensation. |
| Qualcomm | Proposal 1: Any study on overhead reduction for TRS should take UE operation and user experience consideration. Proposal 2: 6G should aim to simplify ecosystem support for positioning and sensing, prioritizing “minimal additional effort from the infrastructure side” and “minimal additional air interface overhead”. Proposal 3: In the study of 6G reference signals for tracking, also consider the goal to enable unified and integrated Sensing, Positioning, and Communication. |
| NEC | Proposal 2: Support to reuse NR TRS (both periodic and aperiodic) as starting point, and 1 slot TRS pattern can be supported from 6G Day1. Proposal 3: Study to support aperiodic TRS for CJT calibration, TDCP or other channel property information. |
| OPPO | - Support periodic TRS in 6G day 1. - Further study whether to support aperiodic TRS - TRS as specific CSI-RS as the starting point - Consider possible usages during design of TRS, e.g. XDCP and CJT calibration measurement |
| MediaTek | Proposal 1: 6GR should support a dedicated sync/reference signal for DL synchronization in connected-mode, which allows on-demand and flexible transmission across the time, frequency, and spatial domains. Proposal 2: To maximize efficiency and reduce overhead, the connected-mode SS/RS should be non-SSB-based and a versatile signal that unifies support for DL synchronization, mobility/beam management, and channel property reporting (TDCP/FDCP/SDCP). |
| Apple | Proposal 5: For 6GR, maintain NR TR TRS framework for fine frequency/time tracking for connected mode: - Further study the periodicity range, TRS pattern considering high speed scenario, high MCS, etc. |
| vivo | Proposal 2: Study jointly using SSB and TRS for T/F tracking for TRS overhead reduction in 6GR. Proposal 3: For UEs with advanced/AI receiver, study whether it is necessary to have TRS as the QCL source for PDSCH reception. - Strive to have no extra enhancement on DMRS pattern if TRS is not used as QCL source |
| Qualcomm | Proposal 4: RAN1 to study how to improve UL performance in high mobility scenarios. |

Observation and summary

Companies generally support studying the reference signals for fine time/frequency tracking for UEs at least in connected mode. Most companies proposed to introduce 5G TRS like signals for such purpose. However, ZTE[9], vivo[13] and Ericsson[14] pointed that in some cases, DMRS signals may be enough. Companies including Nokia[2], FUTUREWEI [3], CMCC[11] and Google[15] proposed to also support TRS for idle mode UE or UE in initial access phase for potential early CSI acquisition. Please note that TRS for idle mode UE in initial access is being specified for 5GA UE in R20. FUTUREWEI [3] also proposed to support TRS for fast SCell/Secondary Component Carrier (SCC) activation, which is also being specified in R20 for 5GA UE.

In summary, most companies think that fine time/frequency tracking is critical issue for the DL channel/signal, e.g., PDSCH DMRS and CSI-RS reception. Because of the narrow bandwidth, SSB based T/F tracking may not satisfy the requirement [19]. Regarding the RS used for fine T/F synchronization, dedicated RS, e.g., TRS, or other RS, e.g., DMRS, can be considered [9]. On the other hand, if the UE has the capability with AI based receive, vivo’s [13] simulation results prove that it’s possible to obtain the T/F tracking without TRS.

Regarding the use case, both UE in connected mode and idle model are mentioned. Companies are encouraged to consider whether one or both should be supported in day 1.

FL Proposals (Round 1)

FL proposal 3.1a: Study the following options for fine time/frequency tracking(at least to provide QCL source information for average delay, delay spread, Doppler shift, and Doppler spread)

- Option 1: Dedicated reference signal for T/F tracking, e.g., TRS

- Option 2: Other reference signals

  - DMRS

  - On demand SS/RS

  - SSB

Other options are not precluded.

FL proposal 3.1b: Study the RS for finer time/frequency tracking for the following use cases:

- UE in connected mode

- UE in idle mode, e.g., for early CSI acquisition

| Company | Comment |
| --- | --- |
| FL | Please share your views on FL proposals 3.1a and 3.1b. |
| OPPO | For proposal 3.1a, we are fine to study time/frequency tracking in 6G day 1. For option 1, it is not very clear to us, since TRS in 5G NR is also a kind of CSI-RS. The intention of option 1 is TRS in 6G is a dedicated reference signal for T/F tracking instead of a specific CSI-RS, or just follow the manner in 5G NR? For option 2, some companies propose that other reference signals (E.g., DMRS or SSB) can be used for T/F tracking. We are fine to consider it as supplementary to TRS. However, if they are used for tracking independently, the tracking performance and UE complexity should be carefully studied. Mod: Even TRS is configured by a specific CSI-RS, it is still actually a dedicated RS, which cannot be used for beam management and CSI acquisition in 5G NR. For the study for 6GR, the intention of option 1 is to have a dedicated reference for T/F tracking, how to configure it, e.g., by a specific CSI-RS, should be discussed in normative phase. For proposal 3.1b, UE in connected mode should be studied firstly. Whether/how to support T/F tracking for UE in idle mode can be discussed later. |
| MediaTek | FL proposal 3.1a: Our interpretation to Option 1 in this proposal is, following 5G NR where there is a dedicated signal for tracking, i.e., TRS. Option 2 is we can use other RS for tracking purpose. In our view, we are fine with both, if any signal can be provided with sufficient density in time/frequency domain, sufficient bandwidth, and sufficient number of transmission occasions before PDSCH reception. A dedicated signal like NR TRS is a straightforward design. However, reusing other signals may be beneficial for overhead reduction. For option 2, we’d like add “OD-SS/RS” as the candidate. Mod: Added as an option. FL proposal 3.1b: We share similar view as OPPO that we should focus on connected mode. For idle mode or even initial access, it is more proper to discuss in initial access agenda item (i.e., study of OD-SS/RS). FL proposal 3.1a: Consider the following options for time/frequency tracking in connected mode - Option 1: Dedicated reference signal for T/F tracking, e.g., TRS - Option 2: Reference signals for other purposes, e.g., DMRS, on-demand SS/RS, or SSB Other options are not precluded. |
| Nokia | Proposal 3.1a Support this study. To clarify the difference between Option 1 and Option 2, we suggest this rewording FL proposal 3.1a: Consider the following options for time/frequency tracking - Option 1: Dedicated reference signal for T/F tracking, e.g., TRS CSI-RS for tracking - Option 2: Other reference signals for other purposes, e.g., DMRS or SSB Mod: Captured in updated version. Proposal 3.1.b Similar views as OPPO and MTK, we can focus on connected mode |
| vivo | Proposal 3.1a The categorization of options is not quite clear in our view. In 5G, SSB is also a signal which can be used for sync/tracking, given it is a signal which can be used as TCI source. However, DMRS cannot be used as TCI source, neither be used without any TCI source (SSB or TRS). Hence it is better to list DMRS as a sole option, while SSB can be categorized into same option as TRS. FL proposal 3.1a: Consider the following options for time/frequency tracking - Option 1: Dedicated reference signal for T/F tracking, e.g., TRS/SSB - Option 2: Reference signals for other purposes, e.g., DMRS or SSB Or we can simply list TRS, SSB and DMRS into separate 3 options. Mod: Thanks for the suggestion, the intention of 3.1a is for fine time/frequency tracking. The proposal is reformulated by considering your suggestion. |
| Qualcomm | FL proposal 3.1b: Generally OK with the 2nd proposal and we think that time/frequency tracking should be considered for both connected and idle UEs with a unified design and not do it separately as it was done in 5G. We have a good 5G baseline that includes enhancements across multiple releases. FL proposal 3.1a: We don’t think that the current proposal builds upon potential pain points that were identified about 5G TRS, and doesn’t do justice to the strong support of 5G TRS shown in the papers, nor its deployment success and its usefulness. Before agreeing to study new options, it should be evident what is considered as a starting point, and what potential problems we see with it so that we have more efficient discussions. Consider 5G TRS as the starting point for time/frequency tracking in 6GR, and identify potential enhancements/modifications to be studied. Mod: My intention of this proposal is to list the potential RS for T/F tracking. Regarding whether 5G TRS can be taken as the start point can be further discussed (yes, lots of companies proposed similar proposal). At least 5G NR TRS can be used as the baseline for performance comparison. |
| Samsung | Fine with the proposal. To us, FL’s classification is clear. We echo with other companies that we may first focus on connected mode. |
| Xiaomi | For proposal 3, our understanding on Option 1 is the CSI-RS for tracking in NR, which should be the starting point. The necessity of enhancements should be justified. For proposal 4, it’s better to focus on connected mode first. Idle mode may need to be discussed in initial access. |
| Fujitsu | Support FL proposal 3.1a. For the FL proposal 3.1b, we also think it can discuss connected mode firstly. |
| Apple | Regarding Proposal 3.1a, we believe Option 1 (Dedicated RS) must be the baseline because Option 2 (SSB/DMRS) has fundamental technical deficiencies that make it unsuitable for 6G tracking requirements. Specifically, SSB-based tracking relies on PBCH DMRS which is confined to intra-slot measurements, physically preventing the inter-slot processing necessary for accurate Doppler estimation, while DMRS is dynamically scheduled and lacks the persistent density required to maintain synchronization during sparse traffic. These physical limitations inevitably lead to a performance wall; indeed, our evaluation confirms that SSB-only tracking causes severe Doppler estimation errors and a significant throughput degradation at periodicities >40ms. Therefore, removing dedicated TRS would effectively cap 6G performance at low MCS, which is unacceptable Also, we are open to studying RS for idle mode (Proposal 3.1b) to assist initial access. |
| InterDigital | Support both in principle, however for 1b, we should start with the connected mode first. |
| LG | Support the proposal with focusing on connected mode, first. |
| NEC | Generally fine with the proposals. Regarding Proposal 3.1a, the RS are listed as options, is that intended for down-selection? Seems DMRS can be as complement for tracking but not sole. Maybe we can just list the aspects for study. |
| CMCC | On Proposal 3.1a, we support Option 1. We emphasize that the 6G TRS design should be decoupled from the CSI reporting framework to enable a lean, measurement-only L1 signal. Additionally, a unified RS design serving both tracking and Early CSI acquisition should be explored to minimize overhead. On Proposal 3.1b, we support the identified use cases. To effectively support the "Idle mode" and "Early CSI" use cases, we propose studying a hybrid configuration framework (i.e., combining SIB-based and legacy RRC-based configurations). For the "Idle mode/Early CSI" case, we stress the importance of SIB-based configuration and the support for m-TRP/cell-free area scenarios to facilitate early CJT operations. FL proposal 3.1b: Study the RS for finer time/frequency tracking for the following use cases: - UE in connected mode - UE in idle mode, e.g., for early CSI acquisition - UEs in connected and idle mode (hybrid configuration framework) Mod: Per Chairman’s guidance, how to provide the configuration should be discussed in normative phase. We can first focus on the issues and requirement for different use case. |
| Ericsson | We agree that Periodic TRS has several drawbacks with periodic TRS including large overhead and unnecessary power consumption for NW. Comment for FL proposal 3.1a: We are supportive of the proposal. It should be emphasized that the two options are not mutually exclusive. For instance, a dedicated DMRS pattern could be used as a reference signal for T/F tracking. So, we suggest to add this as a note in FL proposal 3.1a. Mod: Does it belong to the bullet on DMRS? Comment for FL proposal 3.1b: We are supportive of the proposal. |
| Google | FL proposal 3.1a: In our view, the SSB bandwidth is small, which is not sufficient for time offset tracking. We can study DMRS based approach, but we think SSB should be removed. FL proposal 3.1b: Support |
| Ofinno | Fine with the proposals considering connected mode first |
| ETRI | Support |
| TCL | For proposal 3.1a, we believe the TRS should be used for T/F tracking as a starting point, and then further consideration can be given to using another RS, such as DMRS or SSB. For proposal 3.1b, we support studying T/F tracking for the connected state first. |
| Spreadtrum | FL proposal 3.1a: Support the proposal to list two possible ways to perform time/frequency tracking. For option 2, we are not sure whether we can make an early decision since the design of DMRS and SSB are not clear yet. One way is to focus on the required RE pattern for time/frequency tracking, and then whether DMRS or SSB can be configured as the required RE pattern can be further discussed. FL proposal 3.1b: We prefer to focus on RRC CONNECTED mode in this agenda. |
| Futurewei | The requirement for TRS should be clarified and aligned across all companies. Therefore, we suggest the following clarification of the first proposal: Suggested FL proposal 3.1a: Consider the following options for time/frequency tracking (at least to provide QCL source information for average delay, delay spread, Doppler shift, and Doppler spread) Mod: Captured. |
| ZTE | For FL proposal 3.1a, we have the following comments: #1: At this stage, we think that the term “study” is more suitable than “consider”. #2: To avoid any necessary ambiguity, we tend to reflect “fine” T/F sync and tracking in the main sentence. #3: For candidate options, we think that it would be better to discuss which reference signal can be used to achieve “fine” T/F sync and tracking, e.g., TRS, enhanced DMRS w/o PDSCH, rather than following the classification of dedicated RS or RS for other purposes. Besides, according to the discussion of initial access agenda item, SSB is considered for coarse T/F sync. So we don’t think that SSB is needed especially we have clarified the intention of this proposal in #2. With above considerations, we propose the following update for FL reference. FL proposal 3.1a: Consider Study the following reference signals options for fine time/frequency sync and tracking - Option 1: TRSDedicated reference signal for T/F tracking, e.g., TRS - Option 2: Reference signals for other purposes, e.g., Enhanced DMRS or SSB - Option 3: Enhanced DMRS with PDSCH. Other options are not precluded. Mod: Considering that we don’t have a basic DMRS yet, thus ‘enhanced’ may not be suitable at this stage. Further, you may need to clarify the difference between option 2 and option 3. For FL proposal 3.1b, we agree with the proposal with the following minor change. FL proposal 3.1b: Study the RS for finer time/frequency sync and tracking for the following use cases: - UE in connected mode - UE in idle mode, e.g., for early CSI acquisition Mod: Could you clarify what the difference between sync and tracking? |
| Ericsson | Comment for FL proposal 3.1a: We are supportive of the proposal. It should be emphasized that the two options are not mutually exclusive. For instance, a dedicated DMRS pattern could be used as a reference signal for T/F tracking. So, we suggest to add this as a note in FL proposal 3.1a. Mod: Does it belong to the bullet on DMRS? Ericsson Yes, this belongs to the bullet on DMRS. Perhaps you can formulate the note under DMRS as FFS: whether the DMRS is a dedicated DMRS pattern used for fine T/F tracking On the main bullet, we agree with ZTE that study is appropriate since we now include multiple reference signals. We suggest the following changes: FL proposal 3.1a: Consider Study the following options for fine time/frequency tracking(and at least to provide QCL source information for average delay, delay spread, Doppler shift, and Doppler spread) |

FL Proposals (Round 2)

FL proposal 3.1: Study and evaluate the following aspects on RS for time/frequency tracking to guarantee the performance

- Frequency domain factors

  - Bandwidth

  - Frequency domain density

- Time domain factors

  - Periodic

  - Aperiodic

  - On-demand

Note: 5G NR TRS is used as the baseline for study.

| Company | Comment |
| --- | --- |
| FL | Based on the online discussion, a new proposal is provided. We only focus on the required frequency and time domain resources for tracking RS. Please share your comments. |
| Qualcomm | Indeed this proposal is a better starting point compared to the previous 3.1 proposal. However, it still doesn’t give justice to the previous study during 5G timeframe, the results that were submitted then, to the deployment success of TRS, its usefulness, and doesn’t build on top of what are the potential enhancements. “Redesign” just for the sake of “redesigning” should be avoided, and we should try to really agree on how to improve 6G according to the SID. For example, I want to remind people on the following bullets that were agreed back in 5G time (88bis): - The RS design for time and frequency tracking should study - Tracking range for the frequency error - E.g. in LTE, CRS: +/-2KHz, DMRS: +/-1KHz, CSI-RS: +/-100Hz - Resolution for the timing error - E.g. in LTE, PSS/SSS: 1us, 10MHz BW CRS: 0.1us - Transmission timing e.g. burst period and off duration - Transmission bandwidth, e.g. wideband, partial band - The support of different sets of QCL parameters - The support of Delay spread estimation and Doppler spread estimation Our suggestion remains to emphasize that 5G TRS is the starting point, and study what aspects companies think can be improved, and not redesign/restudy something unless there is a clear need/aspect; then we should focus on identifying these aspects. Therefore, we propose to combine proposal 3.1 with proposal 3.2, with an example shown below. Using 5G TRS as a starting point, study and evaluate potential enhancements at least related to the following aspects for the RS for time/frequency tracking: to guarantee the performance - Frequency domain RS pattern - Time domain RS pattern - Tracking performance - Resource efficiency - Energy saving for NW - UE-side complexity - Support of multi-TRP deployment - Support of multi-carrier operation - Support of multiple use-cases - Possibility to transmit PDSCH data with RS for tracking - Mismatch on the beams between the Tracking RS and associated channel/signal, e.g., PDSCH DMRS and CSI-RS for QCL parameter acquisition |
| Ericsson | We don’t think the intention is to agree any schemes at this point. So, we don’t think “Using 5G TRS as a starting point” as suggested by Qualcomm. It would be ok to add a note that the 5G TRS can be used as baseline for study. Also, when we say 5G TRS are we referring to 5G periodic TRS or 5G aperiodic TRS or both? |
| MediaTek | Supportive to clarify 5G “periodic” CSI-RS for tracking should be used as the baseline |
| CATT | Support. |

FL Proposals (Round 3)

FL proposal 3.1: Study and evaluate the following aspects on RS for fine time/frequency tracking

- Frequency domain factors

  - Bandwidth

  - Frequency domain density

- Time domain factors

  - Time domain behavior: e.g., periodic, aperiodic, on-demand

  - Time domain density: e.g., periodicity, number of symbols, interval between symbols

Note: 5G NR TRS is used as the baseline for study.

FL proposal 3.1-v2: Study and evaluate at least the following aspects on RS for fine time/frequency tracking

- Frequency domain factors

  - Bandwidth

  - Frequency domain density

- Time domain factors

  - Time domain behavior: e.g., periodic, semi-persistent, aperiodic, on-demand

  - Time domain density: e.g., periodicity, number of slots, number of symbols and interval between symbols in a slot, etc.

- Spatial domain factors

  - Spatial domain precoder applied to RS for fine time/frequency tracking

Note: 5G NR TRS is used as the baseline for study. evaluation.

Note: Both connected and idle mode can be studied.

| Company | Comment |
| --- | --- |
| FL | Please check the updated proposal. |
| CMCC | Regarding FL Proposal 3.1, we support the identified factors, especially a specific focus on the "Time domain factors" to guarantee robustness in challenging environments. Specifically, the study should evaluate whether the symbol/slot duration and density in current TRS patterns are sufficient to combat High-Doppler effects in High-Speed scenarios. |
| Docomo | We agree with CMCC that symbol gap of the tracking RS should be also considered. In Rel.16 RAN4 HST, the max Doppler shift for bi-directional SFN in HST is 870Hz because of 4-symbol gap of a TRS. This has a problem that some band could not support 500km/h. On the other hand, LTE CRS symbol gap was 3-symbol and the max Doppler shift for bi-directional SFN in HST is LTE was 972Hz, which can support more bands for 500kh/h. Considering HST scenarios in 6G, smaller symbol gap than NR TRS should be considered. |
| Qualcomm | As it was pointed out in our previous comment, we do think that 3.1.5 and 3.2.5 are really the same topic. What aspects should be considered for the RS for tracking design? One aspect is the Pattern itself, and the other are all the additional aspects that are being identified in 3.2.5. Solely looking at the pattern itself is not as useful nor insightful. If indeed we want to split them up, our suggestion is to make it clear that this proposal is only about the “RS pattern”: Study and evaluate the following aspects on the RS Time and frequency pattern for fine time/frequency tracking - Frequency domain factors - Bandwidth - Frequency domain density - Time domain factors - Time domain behavior: e.g., periodic, semi-persistent, aperiodic, on-demand - Time domain density: e.g., periodicity, number of symbols, interval between symbols Note: 5G NR TRS is used as the baseline for study. Our view however remains that having a proposal that includes the “pattern” aspects together with those in 3.2.5 is a better approach. Mod: Semi-persistent is captured. |
| CATT | Since 5G NR TRS is used as the baseline for study, we propose to study burst-based structure adopted in NR TRS design as one of the time domain factors: FL proposal 3.1: Study and evaluate the following aspects on RS for fine time/frequency tracking - Frequency domain factors - Bandwidth - Frequency domain density - Time domain factors - Time domain behavior: e.g., periodic, aperiodic, on-demand - Time domain density: e.g., periodicity, number of symbols, interval between symbols - Burst-based structure: e.g., the number of bursts Note: 5G NR TRS is used as the baseline for study. Mod: I think the burst structure is included in the time density part. |
| Xiaomi | The following parameters discussed in 5G can be a starting point. 6GR should study the necessity on the enhancement. - X: the length of TRS burst in terms of number of slots - Y: the TRS burst periodicity in terms of number of slots - Sf: TRS subcarrier spacing - St: TRS symbol spacing within a slot - N: Number of OFDM symbols per TRS within a slot - B: TRS bandwidth in terms of number of RBs |
| Nokia | We agree in principle. However we have a few comments: We think we do not need the Note that the baseline is NR TRS. A burst structure suggested by CATT may be acceptable, but we have already time domain behavior and some e.g. mentions, so this can be understood there! |
| Mod | Thanks for all your discussion. Based on your input and offline discussion. Please check the updated version. |

Issue#3: Aspects should be considered for 6GR TRS design

Contributions proposal

| Nokia | Proposal 3: If TRS usage(s) is kept in 6GR, study the need to enable more flexible TRS adaptation for the purpose of overhead reduction /energy savings, without compromising performance. |
| --- | --- |
| Huawei | Proposal 3: 6GR TRS for finer time/frequency synchronization shall be studied, considering NR TRS as a starting point and minimizing 6G TRS overhead as much as possible. |
| Ericsson | Proposal 2: Study means for performing fine UE synchronization via reference signals used for demodulation (DMRS) in 6G. |
| NEC | Proposal 2: Support to reuse NR TRS (both periodic and aperiodic) as starting point, and 1 slot TRS pattern can be supported from 6G Day1. Proposal 3: Study to support aperiodic TRS for CJT calibration, TDCP or other channel property information. |
| Apple | Proposal 5: For 6GR, maintain NR TR TRS framework for fine frequency/time tracking for connected mode: - Further study the periodicity range, TRS pattern considering high speed scenario, high MCS, etc. |
| Fujitsu | Proposal 5: In 6GR, more flexible design for TRS can be considered and studied. |
| LGE | Proposal 3: Study TRS usages in addition to the original purpose of time and frequency tracking and consider NR TRS design as a baseline for further discussion. Proposal 4: TRS is supported by aggregating multiple NZP CSI-RS resources as in NR. |
| TCL | Proposal 1 ：For TRS configuration, 6G should consider further studying flexible time domain behavior, such as trigger methods, periodic adaptation, and other related aspects. |
| CATT | Proposal 2: The following TRS pattern can be considered by 6GR: a burst-based structure with small time and frequency spacing of RS within each burst. |
| OPPO | - Support periodic TRS in 6G day 1. - Further study whether to support aperiodic TRS - TRS as specific CSI-RS as the starting point - Consider possible usages during design of TRS, e.g. XDCP and CJT calibration measurement |
| ZTE | Proposal 4: The design of RS for tracking in 6GR should consider the following aspects: - RS overhead reduction - NW energy consumption reduction - UE energy consumption and complexity reduction - Support multi-TRP scenario (e.g., enable UE transparent multi-TRP operation) Proposal 5: Regarding RS for tracking, study the following candidates for fine time and frequency tracking in 6GR to reduce RS overhead and NW energy consumption caused by configuring dedicated periodic TRS as in 5G-NR. - Scheme-1: DMRS based tracking, e.g., PDSCH DMRS, or DMRS only - Scheme-2: AP-TRS based tracking (standalone) |
| Google | Proposal 4: Support the time-domain and spatial-domain adaptation for TRS with regard to network energy saving. |
| Samsung | Proposal 1: For 6GR, study an appropriate TRS design considering the following aspects. - To adjust TRS overhead, strive to reduce/adapt TRS time/frequency resource allocation - To achieve more NES gain, strive to minimize “always-on” periodic signal and alternate with other signal(s) that UEs should receive/observe/measure. - Study whether/how to minimize per-TRP tracking - Study whether/how to modify QCL source/target relationship providing/achieving channel parameters considering TRS design |
| Lenovo | Proposal 11: Take 5G NR TRS as the baseline for 6GR TRS evaluation. Proposal 12: Link level simulation can be used for the 6GR TRS evaluation and at least the following scenarios should be considered: - New frequency band: 700MHz and FR3 frequency band (e.g. 7GHz) - Large number of TXRUs (e.g. more than 128) with beamforming - Higher speed scenarios. |
| MediaTek | Proposal 1: 6GR should support a dedicated sync/reference signal for DL synchronization in connected-mode, which allows on-demand and flexible transmission across the time, frequency, and spatial domains. Proposal 2: To maximize efficiency and reduce overhead, the connected-mode SS/RS should be non-SSB-based and a versatile signal that unifies support for DL synchronization, mobility/beam management, and channel property reporting (TDCP/FDCP/SDCP). |
| Ofinna | Proposal #2: For 6GR, study the necessity and feasibility of a low-overhead tracking RS as ‘glitch RS’ to be inserted near slot boundaries to enable phase/amplitude estimation and facilitate joint channel estimation across slots. |
| vivo | Proposal 2: Study jointly using SSB and TRS for T/F tracking for TRS overhead reduction in 6GR. Proposal 3: For UEs with advanced/AI receiver, study whether it is necessary to have TRS as the QCL source for PDSCH reception. - Strive to have no extra enhancement on DMRS pattern if TRS is not used as QCL source |
| CMCC | Proposal 1: Study the m-TRP early TRS triggering during initial access. Proposal 2: Study a simplified reference signal design for early access in 6G, where a single triggered RS burst can be utilized for both time-frequency tracking and early CSI acquisition, thereby reducing reference signal overhead in multi-TRP/CFA(cell-free area) scenarios. |

Observation and summary

Regarding the reference signaling design for tracking, companies identified the following issues based on the analysis on 5G NR TRS framework and by considering the 6GR use cases:

- Periodic TRS with fixed frequency density causes larger overhead[2], [6], [9] [13], [17], [23] [20]

- Always-on TRS causes high power consumption for NW and UE [2], [9], [15], [17]

- TRS may not be needed for fine time/frequency tracking for some cases, e.g., coarse time and frequency tracking based on SSB would be enough, or fine T/F tracking can be obtained by DMRS [9],[14]

- Periodic TRS transmission configured via RRC lacks flexibility [2], [8], [21]

Considering that TRS is also used for CJT calibration for multi-TRP deployment and TDCP reporting, OPPO[7], NEC[16], MediaTek[20] and Rakuten[27] proposed the study of TRS should also consider the other purpose including CJT calibration and channel property reporting, e.g., TDCP, in addition to time/frequency synchronization.

In 5G NR, TRS is also used for the QCL parameters including Doppler spread, Doppler shift, average delay and delay spread acquisition for the DMRS and CSI-RS reception. Thus, lots of companies including Nokia[2], FUTUREWEI[3], OPPO[7], Samsung[17], proposed that the TRS design should consider the impact on the QCL relation. Nokia [2], FUTUREWEI [3] and ZTE[9] pointed out that TRS in 5G NR is actually cell-specific transmitted by using a wide beam, which is usually different from the beam used for the DMRS and CSI-RS for CSI acquisition. Thus, the observed QCL parameters may also be different. That would lead to channel estimation performance degradation, SINR lost and lower system performance.

Nokia [2], FUTUREWEI[3] pointed that the TRS in 5G NR is also used to provide the QCL parameter including Doppler spread, Doppler shift, delay spread and average delay estimates of the radio channel so that the UE would be able to set and update parameters for the channel estimation filters for the purpose of reception of downlink CSI-RS, PDCCH DMRS and PDSCH DMRS.

ZTE[9], vivo[13] and Ericsson[14] demonstrated that DMRS based tracking is possible for some cases, e.g., for the UE with an AI receiver for channel estimation and demodulation.

Apple’s [19] simulation shows that TRS based tracking provided significant performance gain over SSB based tracking.

Qualcomm[26] and Lenovo[18] proposed that the TRS design should also consider that it may be used for sensing and positioning.

Some companies have proposed possible solutions to address certain issues. For example, FUTUREWEI [3] proposed to support multi-port TRS

Taking the above issues into account, we provide the potential FL proposals for discussion in the following section.

FL proposals (Round 1)

FL proposal 3.2a: Study the following aspects for the RS design for fine time/frequency tracking

- Tracking performance

- Lower overhead/ Resource efficiency

- Energy saving for NW

- UE-side complexity

- Flexible transmission, e.g., flexible density in frequency and time domain

- Support of multi-TRP deployment

- Support of multi-carrier operation

- Support of multiple use-cases

- Possibility to transmit PDSCH data with RS for tracking (e.g., when DMRS is used as RS for tracking).

- Mismatch on the beam between the TRS and associated channel/signal, e.g., DMRS and CSI-RS for QCL parameter acquisition

FL proposal 3.2b: The study of RS for tracking needs to consider it can be used for the following purpose

- QCL parameters acquisition

- [CJT calibration]

- [Channel property reporting, e.g., TDCP]

- Positioning & sensing

- [Mobility]

- [Early CSI acquisition]

- [Beam management]

- Other purpose is not precluded.

| Company | Comment |
| --- | --- |
| FL | Please share your views on FL proposals 3.2a and 3.2b. |
| OPPO | For proposal 3.2a, we are generally fine for these aspects. To us, the UE-side complexity to support fine T/F tracking should also be considered. Mod: Captured. |
| MediaTek | FL proposal 3.2a - Flexible transmission in time and frequency domains are the most important design aspect to guarantee the tracking performance, including density in frequency and time domains, bandwidth, and periodicity (or number of transmission burst) - Mod: Please check whether the performance/overhead in the first bullets capture your proposal. - Support of multi-carrier operation is also important, as some carrier may not even contain SSB in 6G. - Mod: Captured - “Mismatch on the beam between the TRS and associated channel/signal” means the Tx beam of TRP and the Tx beam of the target channel/signal may not the same? This bullet needs more clarification. - Mod: For example, a wide beam is used for TRS transmission, while a narrow beam is used for the PDSCH/DMRS transmission with is associated with the TRS. FL proposal 3.2b - QCL parameters acquisition should not be the additional purpose of supporting tracking RS. It is the main purpose of tracking RS. In 5G NR, when PDCCH/PDSCH is QCLed with TRS in TypeA, it means UE will perform time/frequency tracking based on the associated TRS. - We’d like to add “beam measurement” as one candidate. - Mod: Captured. |
| Qualcomm | For Proposal 3.2a, we have the following comments: - we need to add in the list: - fine timing/frequency tracking performance and enhanced spectral efficiency - UE-side complexity - Support of multiple use-cases - We should recall the 6GR SI guideline: “Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs” - With regards to “lower overhead”, we think it should be rephrased, to just “Resource efficiency ”; otherwise it should be clear “lower overhead compared to what and under which scenario” - Unclear why “flexible transmission” is emphasized; we should rather emphasize the 6GR SI guideline to “minimize the adoption of multiple options for the same functionality, avoid excessive configurations” For Proposal 3.2b, we think we should list/agree what are the potential purposes of this RS for tracking, and clearly “QCL parameters acquisition and tracking” is the main purpose. It should not be considered an “additional purpose”. We support to add positioning & sensing, mobility, beam management, in the list Mod: Captured. |
| Samsung | Fine with the proposal. For the first proposal, support for mTRP is already part of the second proposal, no need to repeat it. Beam mismatch and other issues can be generalized as ‘tracking performance”. With this, we propose the following simplification: FL proposal 3.2a: Considering the following aspects for the RS design for tracking - Lower overhead - Energy saving for NW - Tracking performance - Flexible transmission, e.g., flexible density in frequency and time domain - Support of multi-TRP deployment - Mismatch on the beam between the TRS and associated channel/signal, e.g., DMRS and CSI-RS for QCL parameter acquisition Mod: My intention of this proposal is to list all the aspects that are identified as a pain points of the TRS design in 5G at this early stage. |
| Fujitsu | Support FL proposal 3.2a and FL proposal 3.2b in principle. We share the similar view with Samsung. The MTRP in FL proposal 3.2a includes the CJT calibration in FL proposal 3.2b. It is preferred to further clarify. Mod: MTRP in 3.2a is more related with the TRS overhead, but MTRP in 3.2b is more related with the CJT calibration. |
| Apple | For Proposal 3.2a, we suggest prioritizing ‘Tracking Performance' (e.g., Doppler/Delay estimation accuracy, CFO/TO RMSE) and robustness to high mobility/high MCS as the primary design aspects rather than lower overhead or energy saving. Overhead reduction at the expense of tracking accuracy will simply degrade demodulation performance for high-order modulation (e.g., 256QAM), ultimately wasting more energy through retransmissions. We support Proposal 3.2b, as accurate QCL parameter acquisition is a critical functionality that must be preserved. Mod: Captured. |
| InterDigital | Support in principle |
| LG | Support the proposal. |
| NEC | Support. |
| CMCC | On FL Proposal 3.2a, we generally support the identified aspects, particularly the emphasis on multi-TRP deployment and overhead reduction. In this context, we would like to clarify that our corresponding views are reflected in our Proposal 1 and Proposal 2, which were not included in the summary table and are hereby supplemented： Proposal 1: Study the m-TRP early TRS triggering during initial access. Proposal 2: Study a simplified reference signal design for early access in 6G, where a single triggered RS burst can be utilized for both time-frequency tracking and early CSI acquisition, thereby reducing reference signal overhead in multi-TRP/CFA(cell-free area) scenarios. Specifically, to support multi-TRP, we propose studying m-TRP early TRS triggering during initial access to enable macro-diversity and CJT in cell-free areas; to achieve lower overhead, we propose studying a simplified reference signal design for early access where a single triggered RS burst can be utilized for both time-frequency tracking and early CSI acquisition. Consequently, regarding FL Proposal 3.2b, we suggest adding "Early CSI acquisition" to the list of purposes to align with the goal of reducing pilot overhead in dense multi-TRP scenarios. FL proposal 3.2b: The study of RS for tracking needs to consider it can also be used for the following purpose - QCL parameters acquisition - CJT calibration - Channel property reporting, e.g., TDCP - Other purpose is not precluded. - Early CSI acquisition Mod: Captured. |
| Ericsson | Comment on FL proposal 3.2a In case DMRS is used as RS for tracking, it can be used to transmit PDSCH data as well. So, we suggest to add one more criterion: - Possibility to transmit PDSCH data with RS for tracking (e.g., when DMRS is used as RS for tracking). Mod: Captured. Comment on FL proposal 3.2b The issue of UE assisted reporting for CJT and channel property reporting is being discussed in agenda 10.5.3.1. So, it is better to wait for some more progress in that agenda before discussing what reference signal should be used for measurement of those cases. |
| Google | Support both proposals. But we think the RS for tracking should be called as tracking reference signal to be aligned the term in chair’s note. |
| ETRI | Support |
| TCL | For proposal 3.2a, we are fine with these aspects. The multi-carrier scenario may also need to be considered, e.g. SCMC. |
| Spreadtrum | FL proposal 3.2a: Support to study the aspects during the TRS design. FL proposal 3.2b: We don’t think this proposal is needed. For TRS design, we just focus on how to design the TRS to achieve T/F tracking performance. Whether TRS can be reused for other purpose shall be discussed in the other agenda, e.g. DL CSI. |
| Futurewei | Support FL proposal 3.2a. @MediaTek: “Mismatch on the beam between the TRS and associated channel/signal, e.g., DMRS and CSI-RS for QCL parameter acquisition” means the TRS beam is generally different from DMRS beam or CSI-RS beam, which leads to reduced channel estimation performance. That’s why we proposed multi-port TRS to resolve this mismatch issue. Agree with Samsung and Apple that tracking performance is very critical. Agree with CMCC on adding early CSI acquisition to FL Proposal 3.2b, and further clarify for connected or before connected: FL proposal 3.2b: The study of RS for tracking needs to consider it can also be used for the following purpose - QCL parameters acquisition - CJT calibration - Tracking performance - Channel property reporting, e.g., TDCP - Other purpose is not precluded. - Early CSI acquisition, e.g., during SCell/second component carrier activation, or before UE enters CONNECTED. Mod: Captured. |
| ZTE | For FL proposal 3.2a, we generally support the proposal with the following minor changes. FL proposal 3.2a: Considering the following aspects for the RS design for fine time/frequency sync and tracking - Lower RS overhead - Energy saving for NW/UE - Flexible configuration transmission, e.g., flexible density in frequency and time domain - Support of multi-TRP deployment - Multi-user multiplexing - Mismatch on the beam between the TRS and associated channel/signal, e.g., DMRS and CSI-RS for QCL parameter acquisition Mod: Captured. For FL proposal 3.2b, we think that it is premature to discuss whether RS for fine time/frequency sync and tracking will be used for CJT calibration and/or channel property reporting e.g., TDCP. Because the relevant conclusions have not been reached in 6GR in CSI agenda item. Thus, we have the following update for the proposal. FL proposal 3.2b: At least The study of the RS for fine time/frequency sync and tracking needs to consider it can also be used for the following purpose QCL parameters acquisition. - CJT calibration - Channel property reporting, e.g., TDCP - Other purpose is not precluded. |
| Ericsson | Comment on FL proposal 3.2b We reiterate our comment that the issues of UE assisted reporting for CJT, channel property reporting, early CSI acquisition are being discussed in agenda 10.5.3.1. So, it is better to wait for some more progress in that agenda before discussing what reference signal should be used for measurement of those cases. Also, Beam management and mobility will be discussed in separate agendas as well. So, suggest to remove these as well for now. |

FL proposals (Round 2)

FL proposal 3.2a-v2: Study the following aspects for the RS design for fine time/frequency tracking

- Tracking performance

- Resource efficiency

- Energy saving for NW

- UE-side complexity

- Support of multi-TRP deployment

- Support of multi-carrier operation

- Support of multiple use-cases

- Possibility to transmit PDSCH data with RS for tracking

- Mismatch on the beams between the Tracking RS and associated channel/signal, e.g., PDSCH DMRS and CSI-RS for QCL parameter acquisition

(Will be discussed in next meeting) FL proposal 3.2b-v2: The study of RS for tracking needs to consider it may also be used for other purposes

- QCL parameters acquisition

- Positioning & sensing

- CJT calibration

- Channel property reporting, e.g., TDCP

- Early CSI acquisition

- [Mobility]

- [Beam management]

- Other purpose is not precluded.

| Company | Comment |
| --- | --- |
| FL | Please share your comments on the updated version. |
| Qualcomm | Regarding FL proposal 3.2b-v2: We support including the crossed-out use cases as part of the study. In our view, unified solutions will ultimately provide the most efficient approach for both the network and the UE, and therefore should be thoroughly examined before introducing redundant functionality. This is a key aspect of the SID, and we should avoid defining multiple solutions for the same purpose unless there is a good justification. Many of the items listed above relate to long-term channel statistics acquisition and tracking (e.g., channel property reporting, positioning, sensing, beam management, and mobility). While we acknowledge that QCL parameter acquisition remains the primary purpose of RS for tracking, we also believe these functions are closely linked to the other use cases and purposes identified in the list. Having it as part of the study will enable all companies, and delegates across subagendas, to consider if there are opportunities to more efficiently use the network resources. With regards to the “QCL parameter acquisition” bullet, I would change it as: - QCL parameters (average delay, delay spread, Doppler shift, and Doppler spread) acquisition & tracking |
| OPPO | We prefer to preserve the deleted blue items, at least for [CJD calibration] and [Channel property reporting] to see whether RS for tracking can be directly extended to handle these issues. In this stage, it is better to keep open for further study. |
| Ericsson | Regarding the deleted blue bullets, the intention is to avoid duplicated discussion in other agenda items. As we pointed out before, the crossed out items are still being discussed in other agendas. Whether these will be introduced or not is still under study. So, we can revisit the question of whether RS for tracking can be used for these items after the other agenda items have made some progress. |
| MediaTek | Fine with FL proposal 3.2a-v2. However, we don’t know why QCL parameters acquisition is put in FL proposal 3.2b-v2 instead of FL proposal 3.2a-v2. As mentioned by QC, it is the primary purpose of RS for tracking. |
| CATT | Proposal 3.2a-v2: Support in principle. The following aspects can also be considered for the RS design for fine time/frequency tracking: - Support of both TN and NTN scenario • Support of different frequency bands Proposal 3.2b-v2: CJTC calibration shall be kept and can be studied at this stage. |

FL proposals (Round 3)

FL proposal 3.2a-v2: Considering the following aspects for the RS design for fine time/frequency tracking

- Tracking performance

- Resource efficiency

- Energy saving for NW

- UE-side complexity

- Support of multi-TRP deployment

- Support of multi-carrier operation

- Support of multiple use-cases

- Possibility to transmit PDSCH data with RS for tracking

- Mismatch on the beams between the Tracking RS and associated channel/signal, e.g., PDSCH DMRS and CSI-RS for QCL parameter acquisition

FL proposal 3.2a-v3: Considering the following aspects for the tracking RS design for fine time/frequency tracking

- Tracking performance

- Overhead

- Energy saving for NW

- UE-side complexity

- Support of multi-TRP deployment

- Support of multi-carrier operation

- Support of TN and NTN

- Determination of QCL property(s)

- Possibility to transmit PDSCH data with RS for tracking RS

- Mismatch on the beams/precoders between the Tracking RS and associated channel/signal, e.g., PDSCH DMRS and CSI-RS for QCL parameter acquisition

- Support of multi-user multiplexing

(will be discussed in next meeting) FL proposal 3.2b-v2: The study of RS for tracking needs to consider it may also be used for other purposes

- QCL parameters, e.g., (e.g. average delay, delay spread, Doppler spread, Doppler shift, time synchronization tracking, CFO tracking) acquisition

- Positioning & sensing

- CJT calibration

- Channel property reporting, e.g., TDCP

- Early CSI acquisition

- Mobility

- [Beam management]

- Other purpose is not precluded.

| Company | Comment |
| --- | --- |
| FL | Please check the updated proposal. |
| CMCC | Regarding FL Proposal 3.2a-v2, we support the aspect of "Support of multi-TRP deployment". In the context of 6G, we emphasize that this study should explicitly cover cell-free scenarios, where high-density TRP coordination introduces unique challenges for tracking signal design and interference management. FL proposal 3.2a-v2: Study the following aspects for the RS design for fine time/frequency tracking - Tracking performance - Resource efficiency - Energy saving for NW - UE-side complexity - Support of multi-TRP deployment - Support of cell-free scenarios - Support of multi-carrier operation - Support of multiple use-cases - Possibility to transmit PDSCH data with RS for tracking - Mismatch on the beams between the Tracking RS and associated channel/signal, e.g., PDSCH DMRS and CSI-RS for QCL parameter acquisition Mod: Most companies think that cell-free is not clear and it can be captured by multi-TRP scenario |
| Docomo | For Proposal 3.2a-v2, we suggest to add the following. - Symbol gap to determine the max Doppler shift In Rel.16 RAN4 HST, the max Doppler shift for bi-directional SFN in HST is 870Hz because of 4-symbol gap of a TRS. This has a problem that some band could not support 500km/h. On the other hand, LTE CRS symbol gap was 3-symbol and the max Doppler shift for bi-directional SFN in HST is LTE was 972Hz, which can support more bands for 500kh/h. Considering HST scenarios in 6G, smaller symbol gap than NR TRS should be considered. Mod: I think it part of the design of TRS as in proposal 3.1.5. |
| Qualcomm | As we pointed out in 3.1.5, we prefer to combine these 2 proposals from our side. We need to study the Time/Freq aspects of a RS for tracking considering all the factors shown in 3.2.5. Mod: I will try to propose both for the online discussion. |
| CATT | Proposal 3.2a-v2: We propose to consider both TN and NTN scenarios, and frequency bands in tracking RS design. In NTN scenarios, LEO satellites feature a relatively high moving speed, which results in large and rapidly varying Doppler frequency offsets and timing drift. Therefore, time-frequency tracking for LEO/VLEO imposes more stringent requirements on the time-domain density and periodicity of TRS. Similar to NR, tracking RS designs in different frequency bands are different. Therefore, the following modification is proposed. FL proposal 3.2a-v2: Considering the following aspects for the RS design for fine time/frequency tracking - Tracking performance - Resource efficiency - Energy saving for NW - UE-side complexity - Support of multi-TRP deployment - Support of multi-carrier operation - Support of multiple use-cases (e.g., TN, NTN, different frequency bands) - Possibility to transmit PDSCH data with RS for tracking Mismatch on the beams between the Tracking RS and associated channel/signal, e.g., PDSCH DMRS and CSI-RS for QCL parameter acquisition Mod: TN and NTN scenario is captured. |
| Qualcomm2 | After further check, we also support both proposals from CMCC (cell-free scenarios) AND “considering both TN and NTN scenarios”. Especially for the 2nd, please note that GNSS-less NTN scenarios, the worst casne max doppler drift can be 0.27 ppm/s at 600km (38.821), and it could be even worst for lower trajectories (e..g 300km) In the EVM we agreed of a drift of 0.2 ppm/s cosndering TN scenarios only when this discussion happened. We need to add in this proposal the aspect of TN/NTN consideration so that there is an opportunity to discuss this aspect. However, it should be outside the bullet of “multiple use-cases” and be a separate bullet. Mod: TN and NTN scenario is captured. |
| Nokia | FL proposal 3.2a-v2: - Possibility to transmit PDSCH data with/without RS for tracking - Mismatch on the beams between the Tracking RS and associated channel/signal, e.g., PDSCH DMRS and CSI-RS for QCL parameter acquisition We think we should look into PDSCH reception without TRS assistance also to identify better the exact utilization, like dependencies on PDSCH MCS. Mod: It seems this is not the intention for this bullet. Its more related with the multiplexing of data and Tracking RS. W.r.t the QCL, we should first identify what are the QCL parameters and then see how TRS potentially fits a QCL chain. FL proposal 3.2b-v2: As there are not yet agreements related to QCL, we would prefer to change QCL parameters acquisition to really say which parameters are intended, e.g.: average delay, delay spread, Doppler spread, Doppler shift, time synchronization tracking, CFO tracking Mod: Captured. |

(Closed)Evaluation methodology (EVM)

In this meeting, ZTE[9], vivo[13], Ericsson[14] and Apple[19] provide primary simulation result on the T/F tracking by using LLS. CATT[10] and Samsung[17] also provided EVM on the evaluation for the T/F tracking. Different companies provide simulation with different evaluation assumptions. Basically, LLS should at least be used for the evaluation and a common EVM is expected for UE fine synchronization evaluation. And we plan to discuss the simulation assumptions beginning from the following table.

Table 2: LLS assumptions for T/F tracking

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD and TDD, CP-OFDM |
| Carrier Frequency/SCS | 700MHz/15kHz, 4GHz/30kHz, 7GHz/30kHz, 30GHz/120kHz |
| Number of TRPs | 1, 2, 4 Companies should report the transmission assumptions for the RS transmission for tracking in case of more than 1 TRP. |
| Bandwidth | 5MHz, 20MHz, 100MHz |
| MIMO scheme | SU-MIMO, Rank 1,2,3,4 |
| Modulation | QPSK, 16QAM, 64QAM, 256QAM and 1024QAM |
| Precoding granularity | PRG = {TBD, wideband} |
| Channel Model | CDL and TDL channel in TR38.901 For CDL-C for 4, 7 and 30 GHz - with combination of ASA and ASD scaling values in sec. 7.7.5.1 in TR 38.901, for above 6 GHz cases - ZSA = 5°, ZSD = 1° The CDL table is translated so that the strongest cluster’s AoD and AoA occur at a random angle for both the antenna panels of TRP and UE in the local coordinate. The value of the random angle is selected to be uniformly distributed from +30 to -30 degree. The random value is chosen independently for both AoD and AoA. In mTRP cases, the channel is generated per TRP. |
| Delay spread | 30ns, 100ns, 300ns, 1000ns |
| UE speed | 3km/h, 10km/h, 120km/h, 350km/h 500km/h |
| Initial time offset (TO) | 1/X CP, X= 8 Other values can be reported by companies. |
| Initial CFO | Initial acquisition - TRP: uniform distribution +/- 0.05 ppm - UE: uniform distribution +/- 5, 10, 20ppm (each company to choose one) Non-initial acquisition - Per TRP: uniform distribution +/- 0.05 ppm - UE: uniform distribution +/- 0.1 ppm Note 1: Those parameters are used for simulation assumptions for synchronization signals/channels in NR (TR38.802). Note 2: Other values can be reported by companies. |
| Drift rate | 0.2 ppm/sec Other values can be reported by companies. |
| BS antenna configuration | Around 700MHz carrier frequency - 4TXRUs/32AEs: (M,N,P,Mg,Ng; Mp, Np)= (8, 2, 2, 1, 1, 1, 2), (dH,dV) = (0.5, 0.5)λ Around 4GHz carrier frequency - 32TXRUs/128AEs: (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1,2,8). (dH, dV) = (0.5, 0.5)λ Around 7GHz carrier frequency - 128 TXRUs/728AEs: (M, N, P, Mg, Ng, Mp, Np) = (24,16,2, 1, 1, 4,16). (dH,dV) = (0.5,0.8)λ Around 30GHz carrier frequency - 8TXRUs / 512AEs: (M, N, P, Mg, Ng, Mp, Np) = (8, 8, 2, 2, 2; 1, 1), (dH,dV) = (0.5, 0.5)λ |
| UE antenna configuration | Follow the agreement in 10.1. |
| Channel estimation | Realistic channel estimation Companies can report the CE methods |
| Performance metric | Estimation error (e.g., CFO/Dopler rmse) , BLER, Throughput |

| Company | Comment |
| --- | --- |
| FL | Please share your views on the simulation assumptions. |
| OPPO | Generally Ok to this EVM with some minor comments: - For carrier frequency, 4GHz can be used instead of 3.5GHz following the discussion in 10.1. - For SCS, for TDD, only 30KHz was agreed, and 15kHz should be deleted. - For modulation order, it seems not necessary to restrict it as QPSK and 16QAM. - For BS antenna config, it should be aligned with output of 10.1 EVM. - For UE antenna config, it should be (1,2,2,1,1,1,2) |
| MediaTek | - We also prefer to delete 15Khz - We also prefer no restriction on modulation order. It’s important to see how DL synchronization impact to different modulation orders. - For BS/UE antenna config, prefer to align with 10.1 EVM. |
| Qualcomm | - OK to focus on 30 KHz SCS - Modulation orders should include high modulation: 64QAM, 256QAM, 1024QAM, and SU-MIMO Rank 2,4 should be included. Perfomance should be guaranteed across all the SINR. - Larger BW should be evaluated, e.g., as large as 100 MHz - For BS antenna config, it should be aligned with output of 10.1 EVM. - How come and we are not also evaluating higher speed scenarios? With regards to the last 2 rows, we should discuss what needs to be evaluated. By having it fully up to companies, each one will evaluate something different and we open up the discussion unnecessarily. Similar studies were also done in 5G, and at this point, we should try to identify the main pain points of 5G design and try to see what enhancements are needed. |
| Apple | To ensure the simulation results meaningfully differentiate the options, we propose stressing the tracking loop with more realistic 6G conditions: 1) Higher Velocities (e.g., 120 km/h) must be added, as 3 km/h is insufficient to highlight the breakdown of SSB/DMRS tracking; 2) Higher Modulation (e.g., 256QAM/1024QAM) should be included, as lower orders like QPSK/16QAM hide the impact of residual tracking errors. |
| Qualcomm2 | We would like to add the following clarifications and comments: - In addition to the large-bandwidth scenario mentioned above in our first response, it would be beneficial to also include a small-bandwidth case—such as 5 MHz. - It is important to align on whether the evaluation will consider back-to-back PDSCH transmissions or cold-start scenarios (e.g., a single PDSCH or a single PDSCH followed by a retransmission). - With regards to high speed, we suggest to add 120kmh (and change 10 kmh to 30 kmh), and delay spread as large as 1000ns. Note that these scenarios are also included in the LLS assumptions of 10.5.3.1 - For carrier frequency, 4GHz can be used instead of 3.5GHz - Add also 700 MHz FDD with 15 KHz SCS. Assumption on PRG needs to be clarified. 5G spec supports both narrow PRG and wideband PRG. Both such scenarios are important: PRG = {4, wideband} |
| CMCC | Regarding the above evaluation methodology, while we support the baseline, we suggest extending the evaluation scope to include Multi-TRP scenarios to align with cell-free deployments and verify cooperative tracking gains. Furthermore, given the critical role of TRS in Doppler estimation, we recommend adding high-mobility cases (e.g., >100 km/h) as the current low-velocity assumptions are insufficient to fully evaluate tracking robustness. |
| Ericsson | Several more parameters need to be added in the above table. The most important ones are the UE frequency offset and UE time offset. Other parameters can be seen in Table 1 of our contribution. |
| Futurewei | To study the mismatch between TRS and CSI-RS/DMRS, the BS beamforming is important, especially for around 7 GHz. For BS antenna configuration at around 7 GHz, large array size, such as 2048 AE, should be evaluated to ensure coverage matching 4 GHz. To reduce energy consumption and cost, 128 TXRUs or even lower should be evaluated. So the following or the like should be considered: 128 TXRUs, 2048 AEs, (M, N, P, Mg, Ng, Mp, Np) = (32, 32, 2, 1, 1; 8, 8), (dH, dV) = (0.5, 0.5) |
| ZTE | For the simulation assumption provided by FL, we have the following comments: #1: In order to support Multi-TRP scenario, we also need to add parameters related to multi-TRP, e.g., the the number of TRP(s) = {1, 2, 4}; CDL channel model is generated per TRP independently; Backhaul: ideal or non-ideal; sync: ideal or non-ideal; TXRU mapping to antenna elements; beam-forming scheme; #2: for carrier frequency, we prefer to consider 4/7/30GHz; #3: for subcarrier spacing, 4/7GHz: 30kHz, 30GHz: 120kHz; #4: for bandwidth, the current 20MHz might be a bit too small. #5: UE velocity: 60km/h, 120km/h can be considered for supporting mid-high speed scenario. |
| Mod | Please check the updated table. |
| Mod | Please check the updated table, especially on the highlighted parts. |
| Ericsson | On channel model, agree with the FL’s update as this model was used in TR 38.802 Table A.1.5-1 during NR study. With this update, we can remove ‘The channel parameters should be aligned with the evaluation in NR’ can be removed. On CFO, ok with the update from FL as this is what was used in TR 38.802 Table A.1.5-1 during NR study. Regarding Frequency drift, the following reference mentions “the frequency drift rate (ppm/sec) in worst case should consider 0.16 ppm/sec.” Hence, the used value for evaluations in this study shall not be worse than that. https://www.3gpp.org/ftp/tsg_ran/wg1_rl1/TSGR1_AH/NR_AH_1706/Docs/R1-1710832.zip For initial timing offset, we can use the timing error limits defined in Table 7.1.2-1 of 3GPP TS 38.133. For 30 KHz, Te = 0.26 micro seconds according to this table. And CP/8 = 0.29 microseconds which is higher than the Te limit. CP/2 is way off. Hence, we should remove CP/2. Since we use different subcarrier spacings in this evaluation, we can say the initial timing offset is less than Te limit as defined in Table 7.1.2-1 of 3GPP TS 38.133. The values can be reported by companies. Regarding BS antenna configuration, since these are link level evaluations, we can remove ‘for indoor’ and ‘for outdoor’. We need a configuration for 700 MHz, and we suggest the following: Configuration for around 700MHz: 4TXRU 32AEs (M, N, P, Mg, Ng; Mp, Np) = (8, 2, 2, 1, 1, 1, 2). (dH, dV) = (0.5, 0.5)λ For around 4GHz, we suggest to use the following for 32 TXRUs instead of what is captured currently: 32TXRU 128 AEs (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1,2,8). (dH, dV) = (0.5, 0.8)λ For around 7GHz, we suggest the following instead of 32 TXRUs. 128TXRU 768AEs (M, N, P, Mg, Ng, Mp, Np) = (24,16,2, 1, 1, 4,16). (dH,dV) = (0.5,0.8)λ Regarding UE antenna configuration, there seems to be a typo. (1, 2, 2, 1, 1, 1, 2). But for this row, we can simply follow what is agreed in Agenda 10.1 instead of making a new agreement. |

Reference signal for phase tracking

Per chairman’s guidance, PT-RS related discussion will be handled in 10.5.2.2 and 10.5.2.3 for DL and UL, respectively.

RS for other purpose

| Ofinno | Proposal #3: Investigate the potential specification impacts on multi-slot-centric design (including TBS determination, UCI multiplexing rules, and definitions on CSI reference resources) to accommodate flexible transmission and new RS type. |
| --- | --- |

| Company | Comment |
| --- | --- |
| FL | Please share your views, if any. |

CSI acquisition by jointly DL and UL

  - Evaluation methodology (EVM)

To demonstrate the performance gain of joint DL and UL based DL CSI acquisition, companies are encouraged to provide simulation results. Different simulation assumptions are used for different companies in this meeting. Considering that this topic is related with the CSI report and SRS, and to avoid duplicated discussion, I tend to propose to reuse the simulation assumptions for CSI reporting for this topic. While companies can provide detailed SRS related parameters in the individual contributions.

On the other hand, non-ideal aspects may need to be considered. Samsung [17] provided a modelling method on the SRS power imbalance as Hsrs=ΑHDL, where HDL is the true DL channel matrix and Α=diag1,α2…αNTX is a diagonal matrix with diagonal coefficient αi representing the power ratio of the i-th antenna with respect to the antenna that has highest power. Noise modelling may also need to be considered, as used by Apple in [19], for both DL and UL by considering the Tx power for CSI-RS and SRS transmission.

Another thing is that both AI and non-AI schemes are proposed by companies for this topic, for a fair comparation, a same EVM is preferred for both schemes. In addition to the regular MIMO KPI including throughput and spectral efficiency, intermediate KPI, e.g., SGCS, which is used for AI based CSI report in 5G NR should also be used.

Regarding to the AI/ML based schemes, the model generalization and scalability performance will influence the practical system performance, thus the following aspects are proposed to be considered for the AI/ML related use case evaluation:

- Deployment scenarios with different channel assumptions,

  - Various deployment scenarios (e.g., UMa, Umi, …)

  - Various channel delay spreads, e.g., 30ns, 100ns, 300ns, etc.

  - Various UE speeds, e.g., 3km/h, 30km/h, 120km/h, etc.

- Antenna configurations,

  - Various # of transmitting antenna element number and/or port number

- Various frequency domain resource (e.g., bandwidth, SRS density),

  - E.g., various bandwidth for SRS and CSI

  - E.g., various frequency densities for SRS and/or CSI

- Various time domain factors

  - Various SRS periodicities

  - Various CSI reporting periodicites

FL proposals (Round 1)

FL proposal 6.0a: For joint DL and UL based DL CSI acquisition, reuse the evaluation methodology agreed in 10.5.3.1 and in 10.5.3.2.

- Note: Specific aspects on joint operation can be reported by companies.

FL proposal 6.0b: For joint DL and UL based DL CSI acquisition evaluation,

- considering the SRS power imbalance

  - e.g., by using the model Hsrs=ΑHDL, where HDL is the true DL channel matrix and Α=diag1,α2…αNTX is a diagonal matrix with diagonal coefficient αi representing the power ratio of the i-th antenna with respect to the antenna that has highest power in dB scale.

- considering the noising and/or interference modelling of the CSI-RS and/or SRS reception.

- considering the amplitude/phase mismatch between Tx and Rx

FL proposal 6.0c: Intermediate KPI, e.g., SGCS, can be additional used for performance comparison in addition to the system level performance

| Company | Comment |
| --- | --- |
| FL | Please share your views on FL proposal 6.0a, 6.0b and 6.0c. |
| OPPO | Ok for proposal 6.0a and proposal 6.0c. For proposal 6.0b, whether/how to model the SRS power imbalance can be further discussed. We can wait for more inputs from companies. |
| Qualcomm | 6G is expected to provide enhanced separate DL- and UL-based CSI acquisition design. While these designs are not available yet, we do not see an urgent need for “joint DL- and UL-based CSI acquisition”. But we are open to studying the candidate schemes under rigorous link- and system-level simulation. For Proposal 6.0a, - OK to use EVM for DL CSI reporting as a starting point - Depending on the detailed candidate solutions, UE-side and gNB side RF impairments may need to be modeled, e.g., if the solution assumes UE-side Tx/Rx reciprocity, the amplitude/phase mismatch between Tx and Rx needs to be properly modeled. - Link-level simulation is a must. For Proposal 6.0b, - The noise modeling is only applied to system-level simulation; realistic channel estimation based on SRS and CSI-RS shall be performed in link-level simulation. - The noise modeling needs to be verified by link-level simulations. For Proposal 6.0c, - SGCS cannot reflect the impact of inter-layer interference. SGCS cannot be the only/final KPI for performance comparison. |
| Samsung | Support. We suggest the above formulation with specific distribution for {α2…αNTX} in DB scale. |
| Huawei, HiSilicon | Proposal 6.0b, for the first bullet, whether/how to model the SRS power imbalance can wait for decision of 10.5.3.2. For the second bullet, we think interference should also be considered in the simulation. |
| Fujitsu | Support |
| Apple | For SRS power imbalance, we believe it was mentioned as not-RAN1 led use case (RAN4 led). |
| InterDigital | Support @Apple: The RAN4-led is supposed to look at AI-based solutions. |
| Ericsson | On Proposal 6.0a: We assume the intention is to align the EVM with DL/UL CSI EVMs. In that case, it may be good to consider both the EVM in 10.5.3.1 and 10.5.3.2 as a starting point. On Proposal 6.0b: Modeling of SRS power imbalance is also related to 10.5.3.2, whatever we agree here should be aligned with 10.5.3.2. In our view, it might be sufficient to discuss it in 10.5.3.2 and adopt the agreement from there. On Proposal 6.0c: In our understanding, this proposal is for general evaluations assumptions that apply to joint UL/DL CSI acquisition. The use of intermediate KPI is more related to AI/ML based use cases. Hence, it may be sufficient to discuss 6.0c in corresponding AI/ML use cases. |
| Google | OK with proposal 6a/6b. But for proposal 6c, we are not sure whether we still need the intermediate KPI |
| ETRI | Proposal 6.0b: Similar view as HW and Ericsson |
| ZTE | For proposal 6.0b, whether model of SRS power imbalance used for AI based scheme, or both of AI and non-AI based ones, need to be clarified. |
| Futurewei | Ok with the proposals. For the SRS antenna imbalance modeling, suggest to keep it high level. |
| Mod | Please check the updated proposal to capture companies suggestion. |

FL proposals (Round 3)

FL proposal 6.0a-v3: For joint DL and UL based DL CSI acquisition, the EVMs agreed in 10.5.3.1 and in 10.5.3.2 are taken as the start point.

FL proposal 6.0a-v3: For joint DL and UL based DL CSI acquisition, reuse the evaluation methodology for LLS and SLS agreed in DL CSI / UL CSI agendas.

·         Note: Specific necessary aspects on joint operation will be discussed.

(Will be discussed in next meeting) FL proposal 6.0b-v3: For joint DL and UL based DL CSI acquisition evaluation, the following aspects can be considered

- noising modelling

- interference modelling

- amplitude/phase mismatch between Tx and Rx

- SRS power imbalance

| Company | Comment |
| --- | --- |
| FL | Please check the updated proposal. |
| Docomo | Fine with the proposal. |
| CATT | Support. |
| ZTE | For proposal 6.0a-v3, MTRP scenario needs to be considered during evaluation. Because one intention of joint DL and UL based DL CSI acquisition is to suppress interference for SRS measurement. and the multi-TRP scenario is a typical interference scenario for SRS measurement. Since many candidate schemes has been provided by many companies for joint DL and UL based DL CSI acquisition, the baseline should be aligned firstly for evaluation to enable better comparison. In summary, I suggest aligned baseline can be need for potential scheme. FL proposal 6.0a: For joint DL and UL based DL CSI acquisition, reuse the evaluation methodology used for CSI reporting in 10.5.3.1 and the evaluation assumption for SRS in 10.5.3.2 for both LLS and SLS. - Considering MTRP scenario for evaluation - Considering aligned baseline for potential scheme to enable better comparison For proposal 6.0b-v3, regarding evaluation, consider: - Association between Rx port for DL CSI and Tx port for SRS; - timeline of DL CSI and SRS measurement; Hence FL proposal 6.0b-v3: For joint DL and UL based DL CSI acquisition evaluation, the following aspects can be considered - noising modelling - interference modelling - amplitude/phase mismatch between Tx and Rx - SRS power imbalance - Port mapping between Rx and Tx; - Timeline of DL CSI and UL CSI; Mod: Considering that the impairment aspect may depend on the joint schemes, the details will be discussed in next meeting. |
| Qualcomm | Clarify this proposal is for both LLS and SLS. Clarify whether “amplitude/phase mismatch between Tx and Rx” is for UE or gNB or both. Mod: Captured. |

CSI acquisition for TDD

Contributions proposal:

| OPPO | Proposal 1: Support CSI report without PMI for reciprocity based transmission in 6GR day 1. |
| --- | --- |
| ZTE | Proposal 1: Study joint CSI-RS and SRS based CSI acquisition in 6GR to achieve an optimal trade-off between CSI acquisition accuracy and report overhead. |
| CMCC | Proposal 5: In 6GR, a joint CSI reporting mechanism based on SRS and CSI-RS to enhance performance in TDD scenarios could be studied. |
| Google | Proposal 6: For joint UL/DL CSI acquisition, study one SRS configured with multiple usages based on the following options: - SRS configured for codebook and antenna switching - SRS configured for non-codebook and antenna switching |
| NEC | Proposal 1: Study the feasibility of jointly downlink and uplink CSI acquisition. |
| Lenovo | Proposal 8: Study enhanced CSI acquisition and report mechanism for joint downlink and uplink based on reciprocity property. |
| Apple | Proposal 1: For CSI feedback and SRS fusion, reuse the MIMO evaluation methodology as a starting point. Use the per-RB SGCS and/or per-RBG SGCS, instead of per-sub band SGCS as the intermediate KPI. Proposal 2: For CSI feedback and SRS fusion in TDD system, study fusion of different CSI feedback candidate with SRS selection: - CSI feedback is channel with matching antenna elements as SRS sounded antennas - CSI feedback is channel from different antenna elements compared to SRS sounded antennas. - CSI feedback is the precoding matrix. - CSI feedback is the transformed matrix using UE side precoding vector U matrix. Proposal 3: For CSI feedback and SRS fusion in TDD system, SRS and CSI feedback fusion study should assume un-calibrated UE Tx/Rx chain. Proposal 4: Study CSI feedback and SRS fusion in FDD system. |
| Ofinno | Proposal 1: Study a unified framework for joint DL and UL CSI acquisition to compensate for RF hardware implementation issue and reduce RS overhead. |
| Qualcomm | Proposal 5: Prioritize independent DL-based and UL-based CSI acquisition over joint DL-UL CSI acquisition. |
| Pengcheng Lab | Proposal 1: Study a unified, parameterized and scalable CSI codebook framework for 6G joint DL/UL CSI acquisition. The framework should support seamless operation between reciprocity-based and feedback-based modes within a single codebook family. |

Observation and summary:

SRS based DL CSI acquisition is an important feature for TDD deployment under the assumption of DL-UL channel reciprocity. Some companies pointed out that the accuracy of DL CSI acquired solely based on SRS may not meet the requirements of 6GR. Reasons include: 1) Due to the limited UL transmit power, the reception SINR of SRS is generally low, resulting in insufficient channel estimation accuracy; 2) In order to increase the transmit power of SRS, particularly when the sounding bandwidth is large, frequency hopping is typically employed, and the longer frequency hopping period also leads to the issue of CSI aging. Given that, joint DL and UL based DL CSI acquisition should be studied at least for TDD system. However, Qualcomm [26] proposed that Joint DL-UL CSI acquisition can be a network implementation choice and transparent to UE.

FL proposals (Round 1)

FL proposal 6.1: Study joint DL and UL based CSI acquisition in TDD system.

| Company | Comment |
| --- | --- |
| FL | Please share your views on FL proposal 6.1. |
| OPPO | Support. |
| MediaTek | Support |
| Qualcomm | UE transparent schemes shall be the baseline. UE-side and gNB-side RF impairment must be properly modelled. Link-level simulation is a must. |
| Samsung | Support |
| Xiaomi | Support proposal 6.1. Cat.1 is a general direction and Cat.2 and Cat.3 are the solutions need to be discussed. We should identify the solutions and evaluation assumptions to proceed. |
| Fujitsu | Support |
| Apple | Support |
| InterDigital | Not sure what this exactly mean. |
| LG | Support |
| NEC | Support |
| CMCC | We support FL Proposal 6.1. Given the inherent coverage and capacity limitations of SRS in TDD scenarios, relying solely on reciprocity is insufficient for robust CSI acquisition. Therefore, we emphasize that the study should focus on a joint reporting mechanism that explicitly combines SRS (for reciprocity) and CSI-RS (for residual error compensation) to optimize the trade-off between overhead and accuracy, rather than treating them as independent or purely network-implementation features. |
| Ericsson | We are open to study. Regarding the scope, we think that CSI acquisition jointly using a) channel estimation based on SRS and b) interference based on IPN feedback measured on DL RS, is also in the scope and shall be studied. As we evaluated in our DL CSI paper (R1-2601038), using IPN feedback for SRS-based DL can provide decent system level throughput gain. We are fine to either discuss it in Cat.1 or list it as a separate category. |
| Google | OK |
| Ofinno | Fine to study |
| ETRI | Support |
| TCL | Support |
| Spreadtrum | We don’t have strong view on this proposal. It is important to clarify the concept of ‘joint DL and UL based DL CSI acquisition’ and make sure companies have the same understanding. |
| ZTE | Candidate schemes should be provided by companies FL proposal 6.1: Study joint DL and UL based CSI acquisition in TDD system. - Candidate schemes provided by companies |
| Futurewei | Support |
| Sony | We recommend changing “DL CSI acquisition” to “CSI acquisition,” as the proposal targets TDD systems. Furthermore, we believe that using long-term channel information feedback can help combine DL and UL channel observations. (We provide more details in our contribution R1-2601137.) Thus, it should be within the scope of this proposal. As long-term channel information is not mentioned in this section, it would be good add a note: FL proposal 6.1: Study joint DL and UL-based DL CSI acquisition in a TDD system. Note: Long-term channel information, e.g., a covariance matrix reported by the UE, can be used to assist. Mod: ‘DL’ is removed. The long term information is listed in proposal 6.3.3 |
| CATT | Support. |

Cat.1: Additional channel information feedback

Contributions proposal:

| Huawei | Proposal 1: 6GR should study and consider leveraging long-term channel information to facilitate sparse SRS design and high-precision uplink-based CSI acquisition. Proposal 2: 6GR MIMO should study the utilization of long-term channel information to facilitate SRS based CSI acquisition, especially for multi-TRP cooperation scenarios. |
| --- | --- |
| ZTE | Proposal 2: Support compressed covariance report for DL CSI acquisition in 6GR, e.g., SRS based CSI acquisition. |
| CMCC | Proposal 5: In 6GR, a joint CSI reporting mechanism based on SRS and CSI-RS to enhance performance in TDD scenarios could be studied. Proposal 6: In 6GR, DL-assisted SRS transmission, utilizing downlink channel characteristics to assist SRS transmission for coverage enhancement and capacity optimization can be studied. |
| DOCOMO | Proposal 2.1: Study the measurement and reporting of the long-term channel covariance matrix based on DL RS to support inter-cell UL interference suppression. |

Observation and summary

At least four companies’ study shown that long-term channel information, e.g., the channel covariance matrix, can be used for inter-TRP and inter-cell interference suppression for SRS based channel estimation to increase the corresponding CSI accuracy. This enhancement is beneficial for improving both the capacity and coverage of SRS, since it helps improve the accuracy of sparse SRS-based channel estimation. ZTE [9] provided preliminary simulation results to demonstrate that covariance-assisted SRS-based CSI acquisition provides superior CSI estimation accuracy compared to SRS-only based CSI acquisition as follows

Given that, companies are encouraged to study long-term channel information, e.g., DL channel covariance, feedback to increase the SRS based CSI estimation accuracy for TDD system.

FL proposals (Round 1)

FL proposal 6.2: Study the long-term channel information assisted SRS-based and CSI-RS based CSI acquisition for TDD system. The long-term channel information can be

- multipath components (MPC) related information, e.g., Power Angular Spectrum (PAS)PAS or Power Delay Profile (PDP)

- Covariance matrix

- Interference plus noise (IPN) information

Other options are not precluded

| Company | Comment |
| --- | --- |
| FL | Please share your views on FL proposal 6.2. |
| OPPO | It is not clear what is ‘long-term channel information’, and what is the difference between long-term and short-term information. In this stage, it is better to collect more results for joint DL and UL, then we can discuss which way is better for further study. |
| MediaTek | It should be one use case of joint DL and UL based DL CSI acquisition in TDD system. Do we need a separate proposal? |
| Qualcomm | The categorization is confusing. “Cat 1” clearly has a larger scope which includes those covered by “Cat 2” and “Cat 3”. We do not need a dedicated proposal to study a specific scheme at this stage. |
| Samsung | We suggest to modify “long term channel information” to “second order channel statistics”. This is not necessarily mean long-term time domain averaging. FL proposal 6.2: Study the long-term channel information second-order channel statistics assisted SRS-based CSI acquisition for TDD system. |
| Huawei, HiSilicon | Support the proposal, it is beneficial using long-term channel information to assist SRS reception for interference rejection and/or spatial/frequency-domain channel interpolation. To clarify the long-term channel information, the proposal can be modified as below: FL proposal 6.2: Study the long-term channel information (e.g., MPC related information (PAS, PDP), channel covariance matrix) assisted SRS-based CSI acquisition for TDD system. |
| Xiaomi | The long-term channel information assisted SRS-based DL CSI acquisition is one approach of joint DL and UL based DL CSI acquisition, which can be studied as an option together with other options, e.g., AI based solution under the topic of joint DL and UL based DL CSI acquisition. Better to identify the boundaries/definitions between options. |
| InterDigital | Not sure what this leads to. If the intention is to look into using channel covariance matrix, then it would be UE transparent method and no need to discuss it here. |
| NEC | Find with the study. |
| CMCC | Regarding FL Proposal 6.2, we support the general direction of leveraging channel information to assist SRS-based CSI acquisition. However, we would like to clarify that in our contribution, the primary mechanism we propose to utilize long-term channel information for enhancing TDD performance corresponds to our Proposal 5: Proposal 5: In 6GR, a joint CSI reporting mechanism based on SRS and CSI-RS to enhance performance in TDD scenarios could be studied. which advocates for a joint CSI reporting mechanism based on SRS and CSI-RS, rather than solely Proposal 6 as listed in the summary. Furthermore, regarding Proposal 6, we would like reiterate that DL-assisted SRS transmission, which utilizes downlink channel characteristics to assist SRS transmission, is a highly valuable technique for coverage enhancement and capacity optimization and should be considered an integral part of the study. FL proposal 6.2: Study the long-term channel information assisted SRS-based CSI acquisition and DL assisted SRS enhancement for TDD system. Mod: Please check whether the updated version includes your point. |
| Ericsson | We are open to study. The scope can be clarified, e.g., is this limited to channel covariance matrix? |
| Google | We suggest removing the “long-term”. The network can also use the reported CSI + layer energy to get the channel covariance matrix. |
| ETRI | We are fine with Samsung's updated proposal. Second-order channel statistics may include spatial, frequency, and/or time-domain covariance/correlation. |
| TCL | Fine to study |
| Spreadtrum | We are not sure the exact definition of ‘long-term channel information’. It is better to list some candidate for further study. |
| ZTE | Support in general, long-term channel information can be further clarified,such as covariance reporting from UE. |
| Futurewei | Support and fine with the further clarifications of the long-term channel informaiton. |
| Mod | Please check the updated version by list multiple candidates on the long term channel information. To make the logic for clause 6.2, 6.3 and 6.4 more clear, I take long-term channel information feedback(6.3) and AI based scheme(6.4) as Cat.1 and Cat.2 of CSI acquisition for TDD(6.2). |
| Sony | Generally fine with the proposal. In R1-2601137, we describe the use of long-term channel information reported by the UE to assist CSI acquisition based on UL SRS and DL CSI-RS. We would prefer to study this in 6.2 (see our comment there), but it can also be studied in this section, in which case the following update would be needed: FL proposal 6.2: Study the long-term channel information assisted SRS-based and CSI-RS-based CSI acquisition for TDD system. The long-term channel information can be - multipath components(MPC) related information, e.g., Power Angular Spectrum (PAS)PAS or Power Delay Profile (PDP) - Covariance matrix - Interference plus noise (IPN) information Other options are not precluded Mod: Captured. |

FL proposals (Round 2)

(Will be discussed in next meeting) FL proposal 6.2-v2: Study the additional channel information assisted SRS-based and CSI-RS based CSI acquisition for TDD system. The additional channel information can be

- Multipath components (MPC) related information, e.g., Power Angular Spectrum (PAS)PAS or Power Delay Profile (PDP)

- Covariance matrix

- Interference plus noise (IPN) information

Other options are not precluded

| Company | Comment |
| --- | --- |
| FL | In this version, I change the wording ‘long-term’ channel information to ‘additional’ channel information. Please share you comment, if any. |
| CATT | Support in principle. For the third bullet, a general description (e.g., interference information) is preferred. |
| Qualcomm | Need clarification on the definition of “power angular spectrum (PAS)” |
| ZTE | For proposal 6.2-v2, regarding second bullet, covariance matrix is a bit general, I think we should clarify which covariance matrix should be considered, e.g., channel covariance matrix, or interference covariance matrix, or both. FL proposal 6.2: Study the long-term channel information assisted SRS-based and CSI-RS based CSI acquisition for TDD system. The long-term channel information can be - multipath components (MPC) related information, e.g., Power Angular Spectrum (PAS)PAS or Power Delay Profile (PDP) - Covariance matrix, e.g., channel covariance matrix from CSI-RS measurement - Interference plus noise (IPN) information Other options are not precluded |

Cat.2: AI based joint DL and UL CSI

Contributions proposal

| OPPO | Proposal 2: Support to study fusion of downlink CSI feedback and SRS measurement in 6GR. Two-sided model |
| --- | --- |
| ZTE | Proposal 3: Study AI/ML based CSI acquisition via CSI-RS and SRS in 6GR, e.g., NW-sided model-based method, and two-sided model-based method. - To identify the performance and complexity by comprehensive simulation evaluation. |
| Samsung | For 6GR, after sufficient progress in the study for DL/UL CSI acquisition, further study AI-based CSI acquisition and report considering both downlink, e.g., CSI-RS, and uplink reference, e.g., SRS/DMRS, signals. Consider both NW-side and two-sided model-based approach. |
| Apple | Proposal 1: For CSI feedback and SRS fusion, reuse the MIMO evaluation methodology as a starting point. Use the per-RB SGCS and/or per-RBG SGCS, instead of per-sub band SGCS as the intermediate KPI. |
| LGE | Proposal 1: Study reciprocity-based CSI acquisition including CSI compression extension with SRS, i.e., AIML sub-case D. |
| Ofinno | Proposal #1: Study a unified framework for joint DL and UL CSI acquisition to compensate for RF hardware implementation issue and reduce RS overhead. AI based solution is taken as a example. |

Observation and summary

At least 6 companies proposed to study the AI based joint DL and UL CSI acquisition to increase the CSI accuracy as well as reduce the RS overhead. OPPO[7], ZTE[9] Samsung[17] and Apple[19] provided preliminary simulation results which demonstrated that fused CSI with SRS based CSI reconstruction can provided observed performance gain over both SRS only and CSI only based scheme in terms of SGCS.

The following two use cases are considered by those companies.

Sub-case D1: CSI reconstruction with CSI feedback with SRS based on two-sided model

For this sub-case, the two-sided model based CSI compression, which is being specified in R20 5GA, is used for CSI report. At the NW side, compressed CSI or recovered CSI and CSI calculated based on SRS are jointly used to recover the full channel matrix information[7][9][17][19]. An example framework is illustrated as follows [9]:

The related simulation results are listed in the following table

Table 3 Preliminary results on fused CSI and SRS with two sided-model

| Companies | Simulation results and observations |
| --- | --- |
| OPPO | - SGCS comparisons SGCS; Layer 1; Layer 2; Layer 3; Layer 4. Benchmark 1; 0.813; 0.689; 0.636; 0.592. Benchmark 2; 0.792; 0.687; 0.645; 0.612. Option 1; 0.915; 0.844; 0.794; 0.752. |
| Samsung | Figure 2. SGCS gain of SRS-assisted explicit CSI reconstruction Observation 2: Two-sided model for SRS-assisted explicit channel reconstruction from spatially sub-sampled CSI-RS measurement achieves significant SGCS gain over SRS-based DL-CSI acquisition for large channel dimensions. |
| Apple | Table 2: Per RB/RBG SGCS comparison without noise. Per RB SGCS; Per RBG SGCS; Per subband SGCS. e-type 2 config 3; 0.6661; 0.7358; 0.8631. Pure ML Precoder Compression Model; 0.6380; 0.7027; 0.8121. SRS sounding with 16 hops; 0.3738; 0.4027; 0.4549. SRS sounding with 4 hops; 0.6158; 0.6501; 0.7078. Fusion of ML model with SRS sounding with 16 hops; 0.6624; 0.7193; 0.8119. Fusion of ML model with SRS sounding with 4 hops; 0.7205; 0.7535; 0.8102. Table 3: Per RB/RBG SGCS comparison with noise impact. Per RB SGCS; Per RBG SGCS; Per subband SGCS. e-type 2 config 3; 0.6003; 0.7167; 0.8354. Pure ML Precoder Compression Model; 0.6349; 0.6990; 0.8064. SRS sounding with 16 hops; 0.3403; 0.3912; 0.4510. SRS sounding with 4 hops; 0.4557; 0.5799; 0.6809. Fusion of ML model with SRS sounding with 16 hops; 0.6587; 0.7148; 0.8082. Fusion of ML model with SRS sounding with 4 hops; 0.7015; 0.7343; 0.7958. As shown in Table 2 and Table 3, the per-RB and per-RBG SGCS, used as intermediate metrics, enable a granular evaluation of how effectively the fused estimate tracks the true channel eigenvectors across the frequency domain, capturing both fine-grained multipath components and the trade-off between rapid temporal variations and noise. |

Sub-case D2: CSI reconstruction with CSI feedback with SRS based on NW-sided model

For this used case, NW-sided model is employed. The model input includes CSI obtained by SRS and the CSI reported by the UE. The reported CSI can be CSI obtained by a CSI codebook, e.g., compressed explicit channel matrix[7], eType 2 CSI codebook in NR [9] or CSI obtained by a linear projection [17][19]. An example framework is illustrated as follows [9]:

Table 4 Preliminary results on fused CSI and SRS with NW sided-model

| Companies | Simulation results and observation |
| --- | --- |
| OPPO | - SGCS comparisons SGCS; Layer 1; Layer 2; Layer 3; Layer 4. Benchmark 1; 0.813; 0.689; 0.636; 0.592. Benchmark 2; 0.792; 0.687; 0.645; 0.612. Option 2; 0.894; 0.804; 0.705; 0.633. |
| ZTE | Table-4 SGCS performance for different cases Cases\Layer; 1; 2; 3; 4. Case 1: Pure SRS -measurements based CSI acquisition; 0.79 (5%); 0.70 (7%); 0.65 (0%); 0.64 (-4.6%). Case 2: Traditional eType II Codebook based CSI acquisition; 0.73 (13%); 0.55 (36%); 0.33 (96%); 0.26 (134%). Case 3: SRS -extrapolation based CSI acquisition; 0.57 (45%); 0.46 (63%); 0.46 (41%); 0.41 (48%). Case 4 (proposed scheme): One-sided-model based CSI acquisition via CSI-RS and SRS; 0.83; 0.75; 0.65; 0.61. Observation 3: Regarding one-sided-model based CSI acquisition via CSI-RS and SRS, we have the following observation(s): - For Layer 1, one-sided-model based CSI acquisition achieves SGCS performance gains of 5%, 13%, and 45%, compared to pure SRS measurements, traditional eType II Codebook, and SRS extrapolation, respectively. - For Layer 2, one-sided-model based CSI acquisition achieves SGCS performance gains of 7%, 36%, and 63%, compared to pure SRS measurements, traditional eType II Codebook, and SRS extrapolation, respectively. - For Layer 3, one-sided-model based CSI acquisition achieves SGCS performance gains of 0%, 96%, and 41%, compared to pure SRS measurements, traditional eType II Codebook, and SRS extrapolation, respectively. - For Layer 4, one-sided-model based CSI acquisition achieves SGCS performance gains of -4.6%, 134%, and 48%, compared to pure SRS measurements, traditional eType II Codebook, and SRS extrapolation, respectively. |

In summary, based on the provided simulation result, performance gain can be observed for both use cases. We proposed to study both sub-use cases. Interested companies are encouraged to provide further simulation results.

FL proposals (Round 1)

FL proposal 6.3a:

- Study and evaluate the following sub-cases for the AI based CSI reconstruction with CSI feedback with SRS:

- [Sub-case D1: CSI reconstruction with compressed CSI feedback with SRS based on two-sided model

  - Option 1: The model input is the compressed precoding channel matrix and SRS measurement

  - Option 2: The model input is the compressed raw channel matrix and SRS measurement]

- Sub-case D2: CSI reconstruction with CSI feedback with SRS based on NW-sided model

  - Option 1: The model input is the precoding matrix, e.g., 5G NR eType II CSI feedback, and SRS measurement

  - Option 2: The model input is the projected/transformed channel matrix and SRS measurement

Note: This proposal is only used for evaluation purposes.

FL proposal6.3b: For the evaluation of AI-based joint DL and UL CSI acquisition, the following is considered:

- KPIs

  - Intermediate KPI: SGCS

  - Model complexity at UE side (for two-sided model)

  - Model complexity at NW side (for both single-sided and two-sided model)

- Benchmark

  - NR SSCC approach with 2-sided model

  - NR eType II CSI CB

  - SRS only based channel matrix

| Company | Comment |
| --- | --- |
| FL | Please share your views on FL proposal 6.3a and 6.3b. |
| OPPO | Support to further study. |
| MediaTek | It is too early to agree to this study, as the 6G DL CSI feedback framework and UL SRS is yet to be discussed. It should be deprioritized until there is more clarity on the DL and UL CSI. |
| Qualcomm | The categorization is confusing. “Cat 1” clearly has a larger scope which includes those covered by “Cat 2” and “Cat 3”. We do not need a dedicated proposal to study a specific scheme at this stage. Mod: Here new Cat.1 and Cat.2 are only used to list the examples provided by companies’ contributions on how to make joint DL and UL based CSI acquisition. |
| Samsung | Support |
| Huawei, HiSilicon | Disagree to study this sub-use case at 6G. 1) Sub-use case D1 is subject to two-sided model, where inter-vendor collaboration is the necessary part. From the study of the past two releases at 5G, it needs significant efforts at spec level and/or product development level, which causes huge challenges to the wide deployment of two-sided model at 6G day one. Therefore, we would prefer to focus on one-sided model cases at 6G R20 study. 2) Regarding the Target CSI type, if the type is precoding matrix, the fusion with SRS is subject to NW side implementation, and there is no difference from the 5G two-sided model for sub-case D1, or non-AI based CSI feedback for sub-case D2. If the type is explicit CSI such as channel matrix, it is subject to the generic explicit CSI feedback which is discussed at Agenda 10.5.3.1, and there is no need for duplicated study at 10.5.3.3. 3) The study on the fusion case is already discussed at 5G R20. If justified, it can be directly specified at 5G, and there is no need for duplicated study at 6G. Mod: The proposal in this meeting is only used to list the schemes provided by companies. We can prioritize D2 and it is only used for evaluation in study phase. Whether it has spec impact can be further studied. |
| Xiaomi | Open to study the AI based joint DL and UL CSI acquisition, the detailed evaluation can be postponed until some progress on DL and UL CSI as MediaTek commented. |
| Apple | Open to study |
| InterDigital | Do not support, we have a same view as MediaTek |
| LG | Open to study |
| Ericsson | Similar view as MediaTek, it is premature to study this when codebook and SRS are still to be studied. Another aspect is that inter-vendor training collaboration issue is known to be challenging and has not been addressed properly in NR, thus D2 can be prioritized over D1. Mod: The proposal in this meeting is only used to list the schemes provided by companies. We can prioritize D2 and it is only used for evaluation in study phase. Whether it has spec impact can be further studied. |
| Google | We also think it is premature to study this. The baseline performance is unclear. |
| Ofinno | Open to study |
| ETRI | Open to study |
| TCL | Fine to study |
| Spreadtrum | We believe that the study of this use case is still premature. For two-side model, it is currently in the WI stage in 5G R20, and many inter-vendor issues have not yet been resolved. Therefore, in the first stage of 6G, two-side model should not be considered; instead, one-side model with an existing standardized foundation should be given priority. Moreover, for non-AI CSIRS and SRS, the details have not been determined yet. Directly considering the integration of these two would bring about numerous uncertainties. |
| ZTE | Support in general, for proposal 6.3a, spec impact need to be considered. |
| Futurewei | We also believe it is premature and may not be needed to discuss this use case or Cat as it is being discussed in Rel-20 5GNR CSI compression use case. At least we can wait till the progress in 5GNR is stable. |

FL proposals (Round 2)

(Will be discussed in next meeting) FL proposal 6.3a-v2:

- Study and evaluate the following sub-cases for the AI based CSI reconstruction with CSI feedback with SRS:

- Sub-case D2: CSI reconstruction with CSI feedback with SRS based on NW-sided model

  - Option 1: The model input is the precoding matrix, e.g., 5G NR eType II CSI feedback, and SRS measurement

  - Option 2: The model input is the projected/transformed channel matrix and SRS measurement

Note: This proposal is only used for evaluation purposes.

| Company | Comment |
| --- | --- |
| FL | For AI based scheme, can we prioritize sub-case D2 and this is only used for evaluation purposes? |
| OPPO | Not agree. For the evaluation for fusion of DL CSI feedback and SRS, we should keep open and evaluate both of 2-sided model and NW-sided model, so that we can clearly find that how much performance loss of NW-side model may have compared to 2-sided model. As far as we know, this sub-use case is not well studied in 5G NR, where only few companies provide limited results. Hence in 6G, for joint DL and UL CSI, this sub-use case should be included for careful study. |
| CATT | We think this sub-case should be deprioritized. The baseline for the NW-sided model is unclear. |
| ZTE | For AI-based method, key point is to identify the performance and complexity. FL proposal 6.3a-v2: - Study and evaluate the following sub-cases for the AI based CSI reconstruction with CSI feedback with SRS: - Sub-case D2: CSI reconstruction with CSI feedback with SRS based on NW-sided model - Option 1: The model input is the precoding matrix, e.g., 5G NR eType II CSI feedback, and SRS measurement - Option 2: The model input is the projected/transformed channel matrix and SRS measurement Note: This proposal is only used for evaluation purposes. Note: Identify the performance and complexity |

Cat.4: CSI framework for 6GR MIMO

Contributions proposal

| FUTUREWEI | - Proposal 1: For 6G MIMO/RS/CSI: MIMO/RS/CSI evolution is still critical for 6G! Adopt 5G NR MIMO/RS/CSI framework as a starting point for 6G MIMO/RS/CSI development Study the following general areas for 6G MIMO/RS/CSI enhancements: Support of upper midband (UMB) Fast beam acquisition for FR2 General Performance (SE and UPT) enhancements. Observation 1: AI/ML should be considered as part of the MIMO framework from day-1 since AI/ML-assisted operation may impact fundamental air-interface tradeoffs (e.g., overhead vs. performance and robustness). Proposal 2: RAN1 to discuss high-level MIMO framework principles that enable integration of AI/ML-based functionality from day-1 of 6G, which also facilitates evaluation for AI/ML use cases. - Multiple framework options may be considered in the 6G study, considering various flexibilities as reference viewpoints Proposal 3: Identify a few AI/ML use cases to facilitate the study and assessment of AI/ML-ready MIMO framework design. |
| --- | --- |
| InterDigital | Proposal 1: Study the following aspects for decision as whether an AI/ML-based solution should be supported for CSI processing in 6GR. - Model Life Cycle Management (LCM) Overhead - Computational and Hardware Complexity, i.e., processing and memory requirement - Energy efficiency, e.g., measuring power consumption vs. spectral gain - Interoperability and Testing, e.g., the number of the tests, data sets, etc. Proposal 2: Support identification of relevant use cases prior to discussing evaluation assumptions and metrics. Proposal 3: Support use of state-of-the-art non-AI/ML based solutions, as the benchmark for evaluation of AI/ML use cases, rather than NR CSI-RS design which is inherited from LTE. |
| Lenovo | Proposal 1: Consider carrying all CSI reporting type by PUSCH. Proposal 2: Carefully study the container for the CSI report, e.g., UCI vs MAC CE. Proposal 3: Carefully study the use cases for periodic, semi-persistent and aperiodic CSI report for NW initiated CSI reporting. Proposal 4: Take the 5G NR CSI report setting and CSI resource setting framework as the baseline for the CSI acquisition and report for 6GR MIMO. Proposal 5: Study early CSI reporting feature for various use cases in the 6GR CSI acquisition and reporting procedure. Proposal 6: Study UE initiated CSI report for CSI acquisition. Proposal 7: Carefully study the CSI process capability for 6GR device. |
| Pengcheng Lab. | Proposal 1: Study a unified, parameterized and scalable CSI codebook framework for 6G joint DL/UL CSI acquisition. The framework should support seamless operation between reciprocity-based and feedback-based modes within a single codebook family. Proposal 2: Study CSI-RS design mechanisms exploiting channel sparsity and dynamic adaptation, including: - Sparse CSI-RS patterns in frequency/time/spatial domains. - Dynamic adaptation of CSI-RS port number and density based on channel conditions and traffic needs. - Mechanisms for reduced measurement burden, such as implicit CSI derivation from other DL signals (e.g., TRS, DMRS) or UL reciprocity information. |
| Rakuten | Proposal 1: Adopt a holistic and integrated architectural and operational framework for multi-TRP coordination and gNB full-duplex operation, focusing on efficient and scalable synchronization, calibration, and self-interference channel estimation procedures. Proposal 4: Study reference signal design for gNB full-duplex self-interference acquisition, at least for separated transmit-receive antenna sets, including: dedicated RS designs for self-interference; dynamic RS for self-interference configuration mechanisms; AI/ML-assisted self-interference channel estimation techniques. |

Observation and summary

Some companies provide views on the CSI or MIMO framework for 6GR design. Considering that related issue depends on the progress of other agenda items, and more discussions may be taken in 10.5.3.1. Thus, the related discussion under this agenda is pended.

FL proposal

TBD

Agreements

RAN1#124

Agreement

Adopt the following simulation assumptions for tracking RS evaluation

Table 1: LLS assumptions for T/F tracking

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD and TDD, CP-OFDM |
| Carrier Frequency/SCS | 700MHz/15kHz, 4GHz/30kHz, 7GHz/30kHz, 30GHz/120kHz |
| Number of TRPs | NTRP=1, 2, 4 Companies should report the transmission assumptions for the RS transmission for tracking in case of more than 1 TRP. |
| Bandwidth for PDSCH | 5MHz, 20MHz, 100MHz, bandwidth for tracking RS can be reported by companies |
| MIMO scheme | SU-MIMO, Rank 1,2,3,4 |
| Modulation | QPSK, 16QAM, 64QAM, 256QAM and 1024QAM |
| Precoding granularity | PRG = wideband, other value(TBD) |
| Channel Model | CDL channel in TR38.901 For CDL-C for 4, 7 and 30 GHz - with combination of ASA and ASD scaling values in sec. 7.7.5.1 in TR 38.901, for above 6 GHz cases - ZSA = 5°, ZSD = 1° The CDL table is translated so that the strongest cluster’s AoD and AoA occur at a random angle for both the antenna panels of TRP and UE in the local coordinate. The value of the random angle is selected to be uniformly distributed from +30 to -30 degree. The random value is chosen independently for both AoD and AoA. In mTRP cases, the channel is generated per TRP. |
| Delay spread | 30ns, 100ns, 300ns, 1000ns |
| UE speed | 3km/h, 10km/h, 120km/h, 350km/h 500km/h |
| Initial time offset (TO) | 1/X CP, X= 8 Other values can be reported by companies. |
| CFO | Initial acquisition - TRP: uniform distribution +/- 0.05 ppm - UE: uniform distribution +/- 5, 10, 20ppm (each company to choose one) Non-initial acquisition - Per TRP: uniform distribution +/- 0.05 ppm - UE: uniform distribution [+/- 0.1] ppm Note 1: Those parameters are used for simulation assumptions for synchronization signals/channels in NR (TR38.802). Note 2: Other values can be reported by companies. |
| Drift rate | 0.2 ppm/sec Other values can be reported by companies. |
| BS antenna configuration | Around 700MHz carrier frequency - 4TXRUs/32AEs: (M,N,P,Mg,Ng; Mp, Np)= (8, 2, 2, 1, 1, 1, 2), (dH,dV) = (0.5, 0.5)λ Around 4GHz carrier frequency - 32TXRUs/128AEs: (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1,2,8). (dH, dV) = (0.5, 0.5)λ Around 7GHz carrier frequency - 128 TXRUs/728AEs: (M, N, P, Mg, Ng, Mp, Np) = (24,16,2, 1, 1, 4,16). (dH,dV) = (0.5,0.8)λ Around 30GHz carrier frequency - 8TXRUs / 512AEs: (M, N, P, Mg, Ng, Mp, Np) = (8, 8, 2, 2, 2; 1, 1), (dH,dV) = (0.5, 0.5)λ |
| UE antenna configuration | Follow the agreement in 10.1. |
| Channel estimation | Realistic channel estimation. Companies can report the CE methods. |
| Performance metric | Estimation error (e.g., CFO/Dopler rmse), BLER, Throughput |

References

- RP-253876, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone

- R1-2600041	Other aspects of 6GR physical layer operation	Nokia

- R1-2600059	Other Aspects for CSI acquisition	FUTUREWEI

- R1-2600121	Discussion on other aspects of CSI acquisition and report for 6GR	Spreadtrum, UNISOC

- R1-2600135	6GR CSI: Considerations for Evaluation of AI/ML-based Solution	InterDigital, Inc.

- R1-2600153	Other aspects for MIMO operation	Huawei, HiSilicon

- R1-2600196	Other aspects for CSI acquisition	OPPO

- R1-2600224	Other aspects for MIMO operation	TCL

- R1-2600233	Discussion on other aspects of CSI acquisition and report	ZTE Corporation, Sanechips

- R1-2600308	Discussion on other aspects for CSI acquisition and report	CATT

- R1-2600398	Discussion on other aspects of CSI acquisition and report	CMCC

- R1-2600438	Discussion on other aspects	Xiaomi

- R1-2600513	Discussion on other aspects of CSI acquisition for 6GR	vivo

- R1-2600578	Other aspects	Ericsson España S.A.

- R1-2600634	Other Aspects of CSI Acquisition	Google

- R1-2600674	Discussion on other aspects for CSI and reference signals	NEC

- R1-2600765	Discussion on other aspects on CSI acquisition and report	Samsung

- R1-2600783	Discussion on miscellaneous aspects on CSI acquisition and report	Lenovo

- R1-2600837	On other aspects of CSI acquisition	Apple

- R1-2600860	On time/frequency tracking RS in connected mode	MediaTek Inc.

- R1-2600877	Discussion on 6GR PTRS and TRS design	Fujitsu

- R1-2600892	Discussion on TRS, PTRS and reciprocity based CSI	LG Electronics

- R1-2601097	Discussion on other aspects of CSI acquisition and report	Ofinno

- R1-2601191	Discussion on Other aspects of CSI acquisition and report	NTT DOCOMO, INC.

- R1-2601201	Discussion on other aspects of CSI acquisition and report	Pengcheng Laboratory

- R1-2601282	Other aspects of CSI acquisition and reporting	Qualcomm Incorporated

- R1-2601344	Discussion on Other Aspects of CSI Acquisition	Rakuten Mobile, Inc