3GPP TSG RAN WG1 #124                               		                                       R1-2601447

Gothenburg, SE, Feb. 9th – 13st, 2026

Source:	Moderator (OPPO)

Title:	FL Summary #1 for 10.5.5

Agenda Item:	10.5.5

Document for:	Discussion and Decision

# Introduction

This document summarizes issues proposed in company contributions of AI 10.5.5

# Contact Person

Please enter your contact information in the table below for potential offline discussion.

| Company | Contact person | Email address(es) |
| --- | --- | --- |
| TCL | SHAHID JAN | Shahid.jan@tcl.com |

# Issues for Discussions

The issues proposed in tdocs are categorized into three subsections: interference management (including CLI, RIM and interference for MRSS), UE reporting (including SR/BSR, reporting for EE and some extra uplink report) and physical procedures (so far include power control, UL Tx switching and interference classification. etc).

## Interference Management

| # | Issue |
| --- | --- |
| 1.1 | UE-to-UE cross-link interference: Many companies (Nokia, Spreadtrum, Huawei/HiSilicon, OPPO, Ericsson, CMCC, vivo, InterDigital, TCL, Ofinno, Google, NEC, Samsung, ETRI, DOCOMO, Qualcomm) proposed to support/study mechanisms for handling UE-to-UE CLI and proposals can be summarized as: - Introduce UE-to-UE CLI measurement and reporting mechanism in 6GR. UE-to-UE CLI measurement and reporting (SRS-RSRP and RSSI) in 5G can be the baseline/starting point. - Identify 6G deployment scenarios where the cross-link interference has impact. - Support L1, L3 or both L1/L3 reporting, unify/combine L1 and L3 measurement and reporting. - Consider both RSRP and RSSI measurement. - Study the report contents: RSRP, RSSI, information to indicate whether each CLI measurement exceed a threshold. - Study and evaluate the benefit of CSI/CQI type CLI measurement and reporting. Study beam-aware CLI handling. - Study reporting mechanism: network-controlled periodic/semi-persistent/aperiodic report, UE-initiated/Event-triggered CLI reporting. - Consider various inter-UE interference types: including both inter-subband and co-channel interference, intra-cell and inter-cell. Consider various deployment scenarios: flexible duplexing, dynamic TDD, SBFD, multi-TRP. - Study additional scheme for handling UE-to-UE CLI: e.g, power control - Consider dynamic and short-term interference characteristics. - Information exchange between gNBs to assist handling inter-cell UE-to-UE CLI, for example resource for measuring CLI, TDD or SBFD configurations. - Study the beam failure due to CLI in SBFD. - Study the interference measurement and reporting enhancement for supporting cooperative sensing in multi-node environment. - Study UE-based interference measurement without explicit interference resource configuration. - Study the SRS transmission dedicated for CLI measurement. - Study group-common DCI mechanism to trigger CLI measurement/reporting and SRS transmission for a group of UEs. Mod: Based on the proposals in contribution, I would suggest to have a comprehensive study on UE-to-UE cross link interference, as in the following proposal 1.1: Proposal 1.1 Study the mechanisms to handle UE-to-UE cross-link interference for 6GR: - Study and identify the applicable scenarios of UE-to-UE cross-link interference; - The following deployment scenarios can be considered: semi-static TDD, dynamic TDD, SBFD, MRSS, multi-TRP, interference caused by sensing to communication. - Consider inter-band, intra-band and adjacent carrier interference. - Study the characteristics of UE-to-UE cross-link interference, for example what is the potential interference signal level, what is the potential impact on system operation, the time-domain characteristics (e.g., dynamic or semi-static, bursty) and frequency-domain characteristics (e.g., wideband or narrowband, semi-static or dynamic frequency domain location); - Study the mechanisms and techniques for mitigating UE-to-UE cross-link interference; - For each candidate mechanism, evaluate and analyze the performance benefit, impact to the system and complexity at BS/UE. - Study the candidate uplink resource or signal for measuring UE-to-UE cross-link interference: - For example: PUSCH, PUCCH, SRS, UL-CSI-RS, PRACH or other signals/channels. - For the UE-to-UE cross-link interference measurement: - Study and identify the candidate measurement metrics; - Study the requirement for measurement, for example the accuracy requirement, whether it is narrow band or wideband measurement, how often the measurement is needed; - Study the reporting mechanism for UE-to-UE cross-link interference measurement: - Study the reporting requirement, for example reporting frequency, whether it is periodic reporting or one-shot reporting. - Study the reporting contents, potential payload size range and how to quantize the measurement results. - Study the reporting mechanisms: - L1 reporting or upper layer reporting, or support both in a unified framework - Network-controlled reporting and/or UE-initiated/event-triggered reporting - Periodic, semi-persistent or aperiodic reporting. - The uplink channel(s) or signal(s) to carry the UE-to-UE cross-link interference measurement. |
| 1.2 | Inter-BS cross-link interference: Companies (Nokia, Spreadtrum, Huawei/HiSilicon, OPPO, ZTE, CATT, CMCC, vivo, InterDigital, TCL, Ofinno, NEC, Samsung, ETRI, DOCOMO, Qualcomm) proposed to support/study mechanisms to handle the BS-to-BS (inte-BS) cross-link interference and the proposals can be summarized as: - Support measurement and reporting mechanism for identify gNB-to-gNB CLI. - The inter-gNB measurement and information exchange (measurement result, configuration of SBFD, etc) and UL resource muting can be starting point. - Further enhancement on UL resource muting and CSI-RS/SSB based inter-gNB CLI measurement. - Study new procedure for mitigate the gNB-to-gNB CLI, for example DL power adjustment. - Consider CLI coordination between TRPs with TRP on/off adaptation. - Proactive coordination for inter-BS CLI. - Inter-band interference shall be considered for inter-BS CLI - Study the benefits/limitations of rel-19 UL resource muting: it has a few restrictions: only PUSCH scheduled by 0_1/0_2/0_3, only comb-2 in frequency domain, up to 2 symbols, semi-static on/off, limitation w.r.t PTRS and DFT-s-OFDM, etc. - Study UL muting with muting pattern aligned with BS-to-BS measurement RS. - For BS interference mitigation, consider at least information exchange of 6G RS for channel/interference measurements, strongest beams and exchange of 6G frame structure time and frequency configurations for 6G. Mod: Based on the proposals in contribution, I would suggest to have a comprehensive study on BS-to-BS cross link interference, as in the following proposal 1.2: Proposal 1.2: Study the mechanisms to handle BS-to-BS cross-link interference for 6GR: - Study and identify the applicable scenarios of BS-to-BS cross-link interference; - The following deployment scenarios can be considered: semi-static TDD, dynamic TDD, SBFD, MRSS, multi-TRP, interference caused by sensing to communication. - Consider inter-band, intra-band and adjacent carrier interference. - Study the characteristics of BS-to-BS cross-link interference, for example what is the potential interference signal level, what is the potential impact to the system operation, the time-domain characteristics (e.g., dynamic or semi-static, bursty) and frequency-domain characteristics (e.g., wideband or narrowband, semi-static or dynamic frequency domain location); - Study the mechanisms and techniques that can mitigate or avoid BS-to-BS cross-link interference: - For example, DL power reduction, beam-aware scheduling, information exchange between BSs on channel/interference measurement, strongest beams, frame structure, proactive coordination between BSs. - For each considered mechanism, evaluate the performance benefit, impact to the system operation, and complexity at BS/UE. - Study the mechanisms for measuring and identifying the BS-to-BS cross-link interference: - Study the candidate resource for measuring BS-to-BS cross-link interference; - Study and identify the measurement metric(s); - Study the requirement on BS-to-BS cross-link interference measurement, for example measurement accuracy, measurement frequency, measurement bandwidth requirement; |
| 1.3 | Remote Interference Management(RIM) Companies (Nokia, Spreadtrum, Huawei/HiSilicon, OPPO, Ericsson, ZTE, CMCC, Ofinno) proposed to support/study remote interference management for 6GR: - 6GR supports RS, detection and mitigation for remote interference. - RIM RS and procedure shall be considered for 6GR - Strive to reduce the configurability of RIM RS. - Reuse both RIM-RS type 1 (sent by the victim BS) and type 2 (sent by the aggressor BS) specified in 5G/NR for 6GR. - Reuse the RIM wireless and backhaul framework for coordination communication. However, some company proposed to prioritize RIM framework 1 but down prioritize the solutions requiring backhaul - RIM framework in 5G/NR is the starting point for study RIM for 6GR. - The study of RIM shall consider coexist of different TDD slot configurations, the impact on RIM-RS reception and measurement. - Proactive RIM triggering Proposal 1.3 Study the mechanism for handling remote interference in 6GR: - Study the applicable scenarios for remote interference between remote cells due to atmospheric ducting; - Study the impact of remote interference to the system, including the impact to the uplink reception and the impact to the downlink reception. - Study the characteristics of the remote interference, for example, potential interference signal level, dynamic or semi-static in time domain and frequency domain, bandwidth size etc.; - Study the candidate mechanisms for measuring/detecting/identifying remote interference, including. - The resource used to detect and measure the remote interference; - Measurement metrics - Requirement on measurement. - Study the mechanisms to mitigate remote interference. - For each candidate mechanism, evaluate the benefits and impact to the system. - The study shall consider the coexistence of different TDD slot configurations of different BSs in the same carrier. |
| 1.4 | Interference handling in MRSS Companies (Nokia, Spreadtrum, Ofinno, Google, DOCOMO) discussed the interference management for MRSS: - Spreadtrum proposed to study the following aspects: resource allocation coordination between 6G-5G, aligned numerology and waveform, and aligned UL/DL direction. - Ofinno proposed to study how to handle CLI across NR and 6GR - Google proposed to support configuring 6G UE to measure 5G-SSB for L1-RSRP report for 5G/6G interference. - However, DOCOMO suggested that CLI handling study for MRSS is not needed because coexistence of 6G SBFD and half-duplex TDD in 5G is not realistic. Proposal 1.4: Study whether/how to handle the interference from 5G to 6G system in MRSS of 6GR: - Study whether it is needed to handle the interference in MRSS of 6GR. - Study the characteristic of interference from 5G system to 6G system in MRSS, for example, whether it is BS-to-BS cross-link interference and/or UE-to-UE cross-link interference; - Study the mechanisms to handle the cross-link interference from 5G to 6G system: - For example: resource allocation coordination between 6G and 5G, to align numerology, waveform and UL/DL direction between 5G and 6G, 6G UE measures 5G-SSB for interference measurement. - For each candidate mechanism, evaluate the performance benefit, impact to the system operation and complexity at BS/UE. |

| Company | Comments |
| --- | --- |
| Mod00 | Please share your views on the above Issues 1.1~1.4 |
| TCL | We support all proposals from 1.1 to 1.4. |

## UE UL reporting

| # | Issue |
| --- | --- |
| 2.1 | UE UL reporting: Companies (Spreadtrum, Huawei/HiSilicon, OPPO, CATT, CMCC, Xiaomi, vivo, Sharp) proposed to study the transmission and SR and BSR for 6GR: - The SR and BSR mechanism for requesting uplink resource scheduling specified in 5G should be supported in 6GR. - Study the solution that can reduce the latency caused by SR/BSR for critical-latency service, e.g., immersive communication. - Study mechanisms to support scheduling request during cell DTX/DRX for critical-latency data. - Study early BSR for latency reduction, for example early BSR can be transmitted through L1 BSR transmitted in PUCCH. - Study transmitting contention based BSR in shared CG-PUSCH. Companies provided proposals for various uplink reporting: - CMCC proposed to design a general event-trigger/UE-initiated uplink transmission mechanism to serve multiple purposes: SR, UE-initiated beam reporting, NACK-only reporting, rank/channel long-term statistical, UCI format shall contain different types of UCI payloads. - TCI suggested to study UE-triggered beam reporting, - Sharp proposed to study UCI types including LLR and UEIRI - CATT proposed to study UE reporting of interference-related condition to provide complementary interference status beyond CSI and HARQ. - Google proposed to study UE grouping information report to facilitate the UE-group based beam management and mobility. The proposal is to allow UE to report the UE reporting information, e.g., reporting the UEs that share common mobility property. - InterDigital proposed to study the benefit of L1 timing/frequency synchronization measurement and reporting for L1-triggered mobility, DL/UL carrier decoupling and mTRP operation, and also proposed to study PDSCH-based reporting (for example delta MCS). - Ofinno proposed to study physical report/indication for NW and UE energy efficiency, the information can include service/KPI information, packet arrival/data availability, traffic information, device battery/power-saving state, etc - MediaTek also provided proposals for NW and UE energy efficiency: (1) NW-to-UE indication mechanisms that can benefit UE's energy efficiency, including but not limited to data availability, traffic pattern, and NW load status indications (2) UE energy efficiency feedback. - Samsung proposed that UAI framework shall consider the NW’s decision complexity and whether UE-specific NAI is needed or not. Mod: Based on the proposals in contributions, I would suggest we shall first have a comprehensive study on UL reporting, including what kind of UL reporting we need, what is use case and benefit to system of each reporting, the reporting contents/potential payload size, what is the report requirement for each type of UL reporting: reporting frequency and reporting mechanism, etc. Proposal 2.1: Study the uplink reporting other than CSI/HARQ for 6GR: - Study the following candidate uplink reporting: - UE reporting to request uplink resource scheduling (for example SR and BSR); - NACK-only reporting; - UE reporting/indication for power saving/energy efficiency; - UE reporting for antenna coherence status; - UE reporting of UE grouping information; - UE reporting L1 timing/frequency synchronization measurement for mobility, DL/UL carrier decoupling and mTRP operation; - UE reporting interference-related condition to provide complementary interference status; - PDSCH-based feedback (e.g., deltaMCS) to support link adaptation. - Other candidates are not precluded. - For each candidate uplink reporting, study the following aspects: - The use case and benefit to the system operation; - The reporting content; - Potential payload size range; - The requirement for the reporting, for example whether it needs periodic report or one-time report, what is the reporting frequency, etc. - The reporting mechanism: periodic, semi-persistent, aperiodic or UE-initiated; - Which UL channel or UL signal to carry the reporting; - Whether/how to introduce UCI type for this UE reporting; - Study whether/how to design a unified event-triggered uplink transmission mechanism for multiple different UL reportings. Study NW-to-UE indication mechanism that can benefit UE’s energy efficiency - Study the indication information, for example, data availability, traffic pattern, NW load status, service requirement, KPI information. - Study the indication mechanisms and study whether UE-specific UAI or cell-specific is needed. |

| Company | Comments |
| --- | --- |
| Mod00 | Please share your views on the above Issues 2.1 |
| TCL | We generally support the proposal. However, in the UL reporting section, the proposals from our TDoc are not reflected. Our TDoc discussed that UL reporting may include sensing reports. In UE-based sensing scenarios, when a UE performs sensing operations, the UE may need to report sensing results to the base station, which may rely on PUSCH transmission or use UCI to convey the sensing report. Moreover, in event-driven or UE-triggered beam management, the UE may also need to report beam measurements to the base station. Therefore, it is recommended to include the following in Proposal 2.1. Proposal 2.1: Study the uplink reporting other than CSI/HARQ for 6GR: - Study the following candidate uplink reporting: - UE reporting to request uplink resource scheduling (for example SR and BSR); - NACK-only reporting; - UE reporting/indication for power saving/energy efficiency; - UE reporting for antenna coherence status; - UE reporting of UE grouping information; - UE reporting L1 timing/frequency synchronization measurement for mobility, DL/UL carrier decoupling and mTRP operation; - UE reporting interference-related condition to provide complementary interference status; - PDSCH-based feedback (e.g., deltaMCS) to support link adaptation. - UE reporting for sensing results - UE reporting for beam measurement or UE-trigger beam reporting - Other candidates are not precluded. - For each candidate uplink reporting, study the following aspects: - The use case and benefit to the system operation; - The reporting content; - Potential payload size range; - The requirement for the reporting, for example whether it needs periodic report or one-time report, what is the reporting frequency, etc. - The reporting mechanism: periodic, semi-persistent, aperiodic or UE-initiated; - Which UL channel or UL signal to carry the reporting; - Whether/how to introduce UCI type for this UE reporting; - Study whether/how to design a unified event-triggered uplink transmission mechanism for multiple different UL reportings. Study NW-to-UE indication mechanism that can benefit UE’s energy efficiency - Study the indication information, for example, data availability, traffic pattern, NW load status, service requirement, KPI information. Study the indication mechanisms and study whether UE-specific UAI or cell-specific is needed. |

## Other Procedures

| # | Issue |
| --- | --- |
| 3.1 | Physical Procedures: Companies proposed to study or support some physical procedures for 6GR: - Nokia proposed to study cell DTX/DRX for energy efficiency. - Google proposed to support PHR report during initial access to facilitate the scheduling of uplink signals during initial access. - Apple suggested to study UL Tx switching based on a semi-static pattern for 6GR. The argument is dynamic Tx switching has a few issues: UE side must extend uplink processing timeline and it may cause ambiguity between UE and BS. - Ericsson proposed to study physical layer techniques including BS and UE measurement and procedures to support interference identification and classification in 6GR, and also proposed to support allocating UEs with different bandwidth capabilities in different frequency regions in one carrier during initial access. Proposal 3.1: Study the following physical procedures for 6GR: - Study the support of cell DTX/DRX in RAN1 including linked adaption of other energy efficiency schemes and transmissions; - Study the power control and PHR report during initial access to facilitate the scheduling of uplink channels/signals during initial access; - Study to support UL Tx switching based on semi-static pattern for 6GR; - Study physical techniques including BS and UE measurement and procedures to support interference identification and classification in 6GR; - Study the procedure to support allocating UEs with different bandwidth capabilities to different frequency regions the carrier of initial access. |

| Company | Comments |
| --- | --- |
| Mod00 | Please share your views on the above Issues 3.1 |
| TCL | Support the proposal |

# Proposals for Online Discussion

…

# Contributions in 10.5.5

- R1-2600044	On other physical layer signals, channels and procedures in 6GR	Nokia

- R1-2600124	Discussion on other physical layer signals, channels and procedure for 6GR	Spreadtrum, UNISOC

- R1-2600156	Other physical layer signals and procedures	Huawei, HiSilicon

- R1-2600208	Discussion on other physical layer signals, channels and procedure	OPPO

- R1-2600213	Other physical layer signals, channels and procedure	Ericsson

- R1-2600271	Discussion on 6G other physical layer signals, channels and procedure	ZTE Corporation, Sanechips

- R1-2600311	Discussion on interference measurements and UE reporting for PHY-layer information beyond CSI and HARQ	CATT

- R1-2600401	Discussion on interference measurements, report and management	CMCC

- R1-2600441	Considerations on 6GR SR and BSR transmission schemes	Xiaomi

- R1-2600516	Discussion on other physical layer signals, channels and procedure for 6GR	vivo

- R1-2600525	Other physical layer signals, channels and procedure	InterDigital, Inc.

- R1-2600541	Other physical layer signals, channels and procedure	TCL

- R1-2600608	Views on Other physical layer signals, channels and procedure	Ofinno

- R1-2600636	Other Physical Layer Signals and Procedure	Google

- R1-2600662	Discussion on Other physical layer signals, channels and procedure	NEC

- R1-2600768	Discussion on other physical layer signals, channels and procedure	Samsung

- R1-2600840	On other physical layer signals, channels and procedure	Apple

- R1-2600927	Discussions on other physical layer signals, channels and procedure	Sharp

- R1-2601014	Discussion on other physical-layer signal and procedure	ETRI

- R1-2601194	Discussion on other physical layer signals, channels and procedures	NTT DOCOMO, INC

- R1-2601227	Views on 6GR AI/ML use case prioritization	AT&T

- R1-2601241	On UE feedback for joint energy efficiency optimization	MediaTek Inc.

- R1-2601284	Other physical layer signals, channels and procedure	Qualcomm Incorporated