3GPP TSG RAN WG1 #124                                                                                           R1-260xxxx

Gothenburg, SE, Feb. 9th ~ 13th, 2026

Source:	Moderator (vivo)

Title:	Summary #1 on 6GR DL WUS operation in RRC states

Agenda Item:	10.6.1.2

Document for:	Discussion and Decision

- Introduction

This contribution summarizes the discussions on 6G DL WUS operation in RRC states in RAN1# 124.

- Proposals for online sessions

  - Proposals for Tuesday online sessions

[FL1]Proposal 4r2: Study 6GR DL WUS waking up UE for paging [PDCCH monitoring] at least in RRC idle state, for at least:

- UE energy saving gain

- Impact to latency, if applicable

- Network overhead

- Network energy consumption

- Other impacts, if any

- Other RRC state is not precluded and it is up to RAN2 discussion

- FFS other functionalities

[FL1]Proposal 6r1:  Study 6GR DL WUS procedure for at least in RRC idle state, for at least:

- WUS monitoring periodicity

- WUS monitoring occasions

- WUS indication granularity

- UE behaviors after waking-up, including direct [RACH transmission], PO determination, and synchronization, if applicable

- Multi-beam operation including at least QCL assumption

- [Wake up delay]

- Other impacts, if any

- Other RRC state is not precluded and it is up to RAN2 discussion

[FL1]Proposal 8r1: Study 6GR DL WUS triggering PDCCH monitoring in RRC connected state, for at least:

- UE energy saving gain

- Impact to UPT, if applicable

- Impact to latency, if applicable

- Impact to QoS/delay budget satisfaction rate, if applicable

- Network overhead

- Network energy consumption

- Othe impacts, if any

[FL1]Proposal 9: for RRC connected state, study DL WUS triggering PDCCH monitoring with and without C-DRX.

  - Proposals for

- 6GR DL WUS operation: general

RRC states for study

Almost all companies discuss that NR WUS is beneficial for both RRC idle and connected states for power saving, and it is worthy to study 6GR DL WUS for both RRC idle and connected states.

- [FL1]Proposal 1: Study 6GR DL WUS procedure for both RRC idle and RRC connected states.

| Company | Y/N | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Y |  |
| Nokia | Y |  |
| Panasonic | Y | We agree to study for both RRC states, but whether to minimize the differences in behavior between different RRC states should also be clarified. |
| LGE | Y | Support the proposal |
| Xiaomi | Y |  |
| FW | Y |  |
| InterDigital | Y |  |
| Spreadtrum | Y |  |
| WILUS | Y |  |
| TCL | Y |  |
| Sharp | Y |  |
| Ofinno | Y | Suggest to agree to study the procedure for at least RRC idle and RRC connected. |
| vivo | Y |  |
| Lenovo | Y | How about RRC inactive state if supported? |
| DOCOMO | Y |  |
| Samsung | Y |  |
| CATT | Y |  |
| Qualcomm | Y |  |
| Ericsson | Y | It should also be supported for other RRC states (e.g., Inactive) depening on the definition. Can add “at least” or FFS: other RRC states pending definition. |
| Google | Y |  |
| mod |  | Based on the inputs, update the proposal as suggested by companies: - [FL1]Proposal 1r1: Study 6GR DL WUS procedure for both at least RRC idle and RRC connected states. |
| ZTE, Sanechips | Y | OK with mod’s update |
| Apple | Y | Just wonder, the reason for not including “inactive” is the inactive state may not exist in 6G? |
| Huawei, HiSilicon | Y | OK with the updated version, whether to support inactive state can be up to RAN2. |
| MTK1 | Y | We support this proposal as it aligns with our view that 6GR DL WUS is beneficial for both RRC idle and connected states for power saving |

WUS operation for different device types

[vivo][CMCC][TCL][Lenovo][Sony] [NICT] discuss that NR WUS is applicable to use cases such as IoT, wearables and eMBB. In 6GR, both eMBB and IoTdevices desire high UE energy efficiency and thus, it is worthy to study 6GR DL WUS for both of them. To streamlinenetwork operations and specification effort, a unified DL WUS procedure design is recommended for both eMBB andIoT use cases.

- [FL1]Proposal 2: Study an unified 6GR DL WUS procedure for both eMBB and IoT use cases.

| Company | Y/N | Comments |
| --- | --- | --- |
| Huawei, HiSilicon |  | It is not clear what a ‘unified procedure’ means for eMBB and IoT use cases. More clarification is needed. Additionaly, 6G IoT use cases and their requirements are not clear at this stage of 6G study. So we do not think this proposal is not needed at this stage. |
| Nokia |  | We would like add a note like the one below, to avoid the scenario where we compromise eMBB WUS design to meet IoT limitations, in the same way OOK compromised LP-WUS design in release-19. Note: IoT design choices should not compromise the performance of eMBB WUS. |
| Panasonic | Y | We support this direction. |
| LGE | N | We are fine with studying WUS in case of eMBB. However, we are not sure IoT can follow the same procedure with eMBB, since 6GR IoT requrirement has not been specified yet. It is premature to conclude that both eMBB and IoT share the same procedure. Therefore, we suggest to study eMBB first. |
| Xiaomi | Y |  |
| InterDigital |  | Agree with Huawei. |
| Spreadtrum | Y |  |
| WILUS | Y |  |
| TCL |  | We agree 6GR DL-WUS is applicable to all devices, but if unified 6GR DL WUS procedure for both eMBB and IoT use cases is not still clear for us. Also, it is unclear for IoT capability, e.g., if Redcap could be as IoT device. Thus, we suggest following, - [FL1]Proposal 2: Study an unified 6GR DL WUS procedure for both eMBB and IoT all diverse devices use cases. |
| OPPO | Y | Single produre works for both eMBB and IoT devices should be persued as basic solution. |
| Sharp |  | If the IoT use cases inclue A-IoT in NR, it can have different procedures if WUS is supported for the case |
| Ofinno |  | Agree with Huawei. It is not clear yet what differences would be there for DL WUS (where LP-WUS is already assumed to be a relatively narrowband signal in 5G) |
| vivo | Y | Similar as NR WUS operation, the target use cases should include both eMBB and IoT UEs. |
| Lenovo | Y | At least applicable to EE processing state with 5 MHz BW. |
| DOCOMO |  | It is better to clarify the difference of eMBB and IoT on DL WUS operation. |
| Samsung | Y |  |
| CATT | Y |  |
| Qualcomm |  | We agree with the intention of having a single design and procedure if possible. |
| Google | Y | We generally support a unified design to minimize fragmentation. However, we share the concern that design choices for IoT constraints should not compromise the performance (e.g., robustness, capacity) of eMBB WUS. The study should prioritize eMBB requirements first |
| moderator |  | The intention of this proposal is to strive for a common procedure, e.g., for waking up, measurement, etc. for eMBB UE and IoT UE. Regarding the performance or requirements for signal design, it is a separate discussion which will be in section .1. The proposal is updated to reflect the comments. - [FL1]Proposal 2r1: Study an unified 6GR DL WUS procedure for both eMBB and IoT use cases. - Note: IoT should not compromise the performance of 6GR DL WUS for eMBB. |
| ZTE, Sanechips |  | Due to the different service characteristics of IoT and eMBB, whether the procedures can be fully unified requires further study and evaluation. For example, if different use case has different wake-up delay capability and the gap between WUS and PDCCH is different, whether it is kind of unified design? Even we could prioritize the eMBB performance but still difficult to aligh what kind of design is unified design. |
| Apple |  | “Unified” is not clear to us either. For example, R19 LP-WUS support for iDRX and eDRX, are they considered as unified or not? |
| Huawei, HiSilicon |  | Our concern on ‘unfinied’ is not resolved yet, more clarifications on which part is unified is still needed. |
| MTK1 |  | We support this proposal at least for connected mode to streamline network operations and specification effort through a unified DL WUS procedure design. For idle model, given eMBB and IoT may assume different MR sleep states (Deep Sleep vs. Ultra-Deep Sleep), it is better to capture the following FFS: Proposal 2: Study an unified 6GR DL WUS procedure for both eMBB and IoT use cases. - FFS impact of assumed MR sleep state(s) for idle |

Sync proceduce for 6GR DL WUS

[Qualcomm][vivo][Huawei][CMCC][Samsung][Docomo][CATT][Spreadtrum][Futurewei][sony][OPPO] discuss that considering  6GR DL WUS is based on OFDM sequence, and thus it is possible to perform synchronization based on 6GR synchronization signal, i.e., PSS/SSS by EE processing, and thus, dedicated reference signal for EE processing measurement may not be amsungy to reduce system overhead, it is suggested to strive for unified synchronization signal for EE processing and non-EE processing.

Further, [Huawei] discusses that LR may only have 1RX or 2RX ports, while the MR may be with 4RX ports depending on the discussion. With different number of RX ports, the synchronization accuracy of DL SS for LR shall be worse than that for MR. Therefore, the synchronization accuracy difference by detecting SSB for MR and LR should be carefully considered.

- (closed and will be handled in 10.6.1.1)[FL1]Proposal 3: 6GR DL WUS study assumes synchronization is based on 6GR synchronization signals (PSS/SSS), and study at least the following:

- Achievable synchronization accuracy for WUS reception

| Company | Y/N | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Y |  |
| Nokia | Y |  |
| Panasonic |  | We support this direction but we should wait the progress 6G synchronization signal discussion in 10.5.1.1 and WUS design in 10.6.1. |
| LGE | Y | Support the proposal |
| Xiaomi |  | If PSS/SSS is adopted as the synchronization signal for DL WUS, once the signal generation of PSS/SSS is different from that of DL WUS, it may lead to an increase in the complexity of LR. For instance, the time-domain detection sequence of DL WUS is required, but the frequency-domain detection sequence of PSS/SSS is needed. Therefore, it is necessary to study other synchronization signals at least in the SI stage, such as LP-SS like signal. |
| FW |  | We think this more appropriately discussed in AI 10.6.1.1 |
| Spreadtrum | Y |  |
| WILUS | Y |  |
| TCL |  | It is too early to decide whether 6GR PSS/SSS could be used for sync assumption. We still now do not know what the 6GR sync signal structure is, and thus, we suggest to postpone this proposal, or correction as following, - [FL1]Proposal 3: 6GR DL WUS study assumes synchronization is based on 6GR synchronization signals (PSS/SSS) or other methods, and study at least the following: - Achievable synchronization accuracy for WUS reception |
| OPPO |  | We need to consider different options for the synchronization. In our study, we see 3 options: - 1) The sync part of WUS - 2)The Low Power sync signal specific for WUS - 3)The SSB for initial access After discussion and evaluation we can then conclude. |
| Ofinno | Y | - Suggest minor change: - [FL1]Proposal 3: 6GR DL WUS study assumes synchronization is based on 6GR synchronization signals (e.g., PSS/SSS), and study at least the following: - Achievable synchronization accuracy for WUS reception |
| vivo | Y |  |
| Lenovo | Y | SSB can be detected over a larger window to compensate for smaller # of RX. |
| DOCOMO | Y |  |
| Samsung |  | We support the intention of this proposal. However, considering that it has also been discussed in 10.6.1.1, it’s better to avoid duplicated discussion in 10.6.1.2. For this proposal, it should be better to add “at least based on 6GR synchronization signals (PSS/SSS)”in the main bullet because the resource for DL WUS is not decided yet, and we are not sure whether only PSS/SSS is sufficient to satisfied the synchronization accuracy for WUS reception. |
| CATT | Y |  |
| Qualcomm | Y |  |
| Ericsson | Y | Support minor change from Ofinno |
| Google |  | While reusing PSS/SSS reduces network overhead, we are concerned about the complexity and power consumption of detecting 6GR PSS/SSS using EE processing. |
| Mod |  | As pointed out by Panasonic and FW, companies provide inputs for sync in both .1 and .2, after discussing this with Magnus, we have consensus that this issue would be handled in .1. |
| MTK1 | N | Based on the assumption of EE processing, the design should target ±5 ppm residual CFO tolerance without accurate frequency tracking to maximize EE processing benefits. In this regard, we suggest the following revision to the proposal: [FL1]Proposal 3 (with modifcation): 6GR DL WUS study assumes synchronization error up to the values : - Residue CFO up to [5] ppm and residue timing offset up to [2] us |

- 6GR DL WUS operation in RRC idle

  - Applied scenarios/use cases and identified issues

    - Use case #1: Power saving for paging

[Nokia] [Huawei] [Qualcomm] [Ericsson] [MTK] [Apple] [vivo] [amsung][ZTE][DOCOMO][Futurewei][Spreadtrum][LGE][CATT][CMCC][OPPO][TCL][Sharp][NEC]

[InterDigital][ETRI][Google][Ofinno][Sony][WILUS][Lenovo] [NICT]propose to study power saving for paging based on 6GR DL WUS.

Specifically, companies propose the following paging related use cases:

- DL WUS triggering paging PDCCH monitoring: [Nokia] [vivo] [Huawei][Qualcomm] [Ericsson] [Apple][amsung][ZTE][DOCOMO][Futurewei][Spreadtrum][LGE][CATT][CMCC][OPPO][TCL][Sharp][NEC][InterDigital][ETRI][Google][Ofinno][Sony][WILUS][Lenovo] [MTK]

- DL WUS indicating paging information directly:[Huawei][ZTE][Sharp]

- SI updated and ETWS/CMAS indication by DL WUS: [Nokia][Spreadtrum][CATT][LGE][WILUS]

For DL WUS triggering PDCCH monitoring in RRC idle state, DL WUS can be used to reduce the unnecessary wake-up overhead as illustrated by below figure from [Qualcomm], an UE monitors WUS to check whether to wake up for paging PDCCH monitoring, if yes, the UE wakes up from ultra deep sleep or deep sleep, otherwise, the UE continues keeping in ultra deep sleep or deep sleep for power saving purpose.

[Huawei] provides evaluation results on energy saving gain and latency impact of 6G DL WUS in RRC idle state as shown below, indicating that 6G DL WUS can provide 64.3% energy saving gain compared with R15 paging mechanism and if it is based on per UE indication, the paging latency is not increased but  the RACH latency is slightly increased. [Nokia][Ericsson][xiaomi][InterDigital] propose to study attainable UE power saving gain in IDLE/Inactive. Besides, based on the agreed evalution assumptions as listed below for energy saving schemes for 6GR, FL suggests the following proposal:

RAN1 #122bis Agreement

At least the following NR metrics,

- Network energy saving gain relative to baseline for BS

- UE energy saving gain relative to baseline for UE

- Impact to UPT (User-Perceived Throughput), if applicable,

as well as the metrics

- Impact to latency, if applicable

- Impact to QoS/delay budget satisfaction rate, if applicable

are used for 6G energy efficiency evaluation.

RAN1 #123 Agreement

For evaluation purposes and relative comparison over different candidate energy saving schemes for 6GR, define the following baseline power saving configurations for UE for evaluation purpose for FR1 (including around 7GHz):

- 5G NR I-DRX (1.28s cycle) for idle mode

  - Group paging rate (for a PO): TBD

- 5G NR C-DRX settings of (cycle, on-duration timer, inactivity timer) are assumed for the following 6GR traffic models for connected mode:

  - VoIP: (40 ms, 8 ms, 10 ms)

  - FTP3: (160 ms, 8 ms, 100 ms)

  - Instant message : (320 ms, 8 ms, 100 ms)

  - XR: (16 ms, 10 ms, 4 ms)

- Companies can evaluate and report other traffic(s) and/or configuration(s) with justification

Note: The corresponding evaluation is not intended for energy efficiency comparison with 5G/NR.

- [FL1]Proposal 4: For 6GR DL WUS triggering UE for below paging related use cases in RRC idle state,

- Triggering paging PDCCH monitoring

- WUS indicating paging information

- SI updated and ETWS/CMAS indication

study UE performance and network impact, regarding at least the following:

- UE energy saving gain

- Impact to latency, if applicable

- Network overhead

- Network energy consumption

- Other impacts, if any

| Company | Y/N | Comments |
| --- | --- | --- |
| Huawei, HiSilicon |  | For the first part, we understanding it is discussing the functionality of DL WUS. If so, there still other potential functionalities, like PO adaptation (see Huawei_v1). But we also notice that in Section 4.2.3 new functionalities are also discussed, so maybe this proposal is focusing on the wakeup functionality? If so, we would like to focus on the first bullets (see Huawei_v2), since short message is not ‘paging related’. - [FL1]Proposal 4-Huawei_v1: For 6GR DL WUS triggering UE for below paging related use cases in RRC idle state, - Triggering paging PDCCH monitoring - WUS indicating paging information - SI updated and ETWS/CMAS indication - PO adaptation study UE performance and network impact, regarding at least the following: - UE energy saving gain - Impact to latency, if applicable - Network overhead - Network energy consumption - Other impacts, if any - [FL1]Proposal 4-Huawei_v2: For 6GR DL WUS triggering UE for below paging related use cases in RRC idle state, - Triggering paging PDCCH monitoring - WUS indicating paging information - SI updated and ETWS/CMAS indication study UE performance and network impact, regarding at least the following: - UE energy saving gain - Impact to latency, if applicable - Network overhead - Network energy consumption - Other impacts, if any |
| Nokia |  | Would like to clarify the: (1) scope of “WUS indicating paging information” - is the intention here to cover UE-specific “direct to RACH” style WUS? I.e. are we considering only UE ID or also other possible information (2) should RAN1 make some assumptions of the PO to MO configuration for this study, or is it meant to cover all possible configurations, including, dynamic, fixed, etc? |
| Panasonic | Y | We support this direction. |
| LGE | Y | We are generally fine with the proposal. However, we are a little bit confused what it means for WUS indicating paging information. In our understanding, paging information may include the paging occasion as well as paging adaptation. However, paging adaptation is marked as new functionality in 4.2.3. Could the FL clarify wheather paging information includes paging adaptation? |
| Xiaomi |  | We are generally fine with the proposal. However, the 3rd use case belongs to “other function”. When it comes to other functions, it is necessary to ensure that the payload size meets the requirement for wake-up before discussing whether to support the other function. Therefore, we suggest deleting it here. - [FL1]Proposal 4: For 6GR DL WUS triggering UE for below paging related use cases in RRC idle state, - Triggering paging PDCCH monitoring - WUS indicating paging information - SI updated and ETWS/CMAS indication study UE performance and network impact, regarding at least the following: - UE energy saving gain - Impact to latency, if applicable - Network overhead - Network energy consumption - Other impacts, if any |
| FW |  | Our understanding is that the intention to evaluate UE-specific and UE-group specific wake-up indication. If so, we prefer to use this language instead, and may something like the following may be considered - [FL1]Proposal 4-FW: For 6GR DL WUS triggering UE for below paging related use cases in RRC idle state, considering both UE-group specific and UE-specific wake-up indication, - Triggering paging PDCCH monitoring - WUS indicating paging information - SI updated and ETWS/CMAS indication study UE performance and network impact, regarding at least the following: - UE energy saving gain - Impact to latency, if applicable - Network overhead - Network energy consumption - Other impacts, if any |
| InterDigital |  | We support FW’s version. |
| ETRI | Y | We are generally fine with the direction. |
| Spreadtrum |  | We think available indication of sync signals (e.g., TRS) should be studied. - [FL1]Proposal 4: For 6GR DL WUS triggering UE for below paging related use cases in RRC idle state, - Triggering paging PDCCH monitoring - WUS indicating paging information - SI updated and ETWS/CMAS indication - Available indication of sync signals study UE performance and network impact, regarding at least the following: - UE energy saving gain - Impact to latency, if applicable - Network overhead - Network energy consumption - Other impacts, if any |
| WILUS | Y | We support the proposal |
| TCL | Y | Agree with this proposal. |
| OPPO | Y | We are generally OK with the direction. |
| Sharp |  | Should we say we study 6GR DL WUS triggering UE for use cases in the main bullet? At least for sub bullet 3, it needs more discussion. |
| Ofinno |  | Suggest the following update at this stage of the discussion: - [FL1]Proposal 4: For 6GR DL WUS triggering UE for below paging related use cases in RRC idle state, - Triggering paging PDCCH monitoring - WUS indicating paging information - Other information - SI updated and ETWS/CMAS indication study UE performance and network impact, regarding at least the following: - UE energy saving gain - Impact to latency, if applicable - Network overhead - Network energy consumption - Other impacts, if any |
| vivo | Y | In our understanding this proposal is to study all the functionalities currently covered by paging can be indicated by WUS. |
| Lenovo | Y | In addition, we can study if paging occasions can be monitored and received in the EE state. |
| DOCOMO | Y | For “WUS indicating paging information”, we would like to clarify whether the paing PDCCH/PDSCH is still necessary or not? |
| Samsung |  | Based on our understanding, it is better to clarify the basic functionality as “Triggering paging PDCCH monitoring” and further study the feasibility of the other two functionalities. Regarding WUS indicating paging information, since the information bits of DL WUS are limited, we are not sure whether paging information can be carried by DL WUS. And since the relability of DL WUS is lower than paging PDCCH, it is better to use DL WUS to trigger paging monitoring other than replace it. |
| Qualcomm | N | We prefer to start with the basic functionality required and then consider additional cases. For example, it is not clear that there will be PO adaptation and that should wait for a decision from he initial access agenda. - [FL1]Proposal 4-Qualcomm: For 6GR DL WUS triggering UE for below paging related use cases in RRC idle state, - Triggering paging PDCCH monitoring - WUS indicating paging information - SI updated and ETWS/CMAS indication study UE performance and network impact, regarding at least the following: - UE energy saving gain - Impact to latency, if applicable - Network overhead - Network energy consumption - Other impacts, if any |
| Ericsson |  | Support QC proposal that start from paging PDCCH monitoring. |
| Google | Y | We support studying multiple use cases, especially ‘SI update indication’ which offers significant idle mode power savings. However, we caution that ‘WUS indicating paging information directly’ should not significantly increase the WUS payload size or detection complexity, as this could compromise the coverage and energy efficiency of the LP receiver. In addition, the last third bullet is uncleart to us. What is “Network overhead”? Resource overhead or signlaing overhead? |
| Mod |  | Based on inputs, it seems companies prefer to focus on well known/basic use case/functionality of DL WUS in idle- waking up UE for paging, the proposal is updated as below: - [FL1]Proposal 4r1: For Study 6GR DL WUS triggering waking up UE for paging below paging related use cases in RRC idle state, - Triggering paging PDCCH monitoring - WUS indicating paging information - SI updated and ETWS/CMAS indication study UE performance and network impact, regarding at least the following: - UE energy saving gain - Impact to latency, if applicable - Network overhead - Network energy consumption - Other impacts, if any @Huawei, Nokia, FW, Xiaomi, Qualcomm, the proposal is updated to focus on waking up functionality first, and functionalities other than waking up will be discussed in section 4.2.3 @ google network overhead mainly refers to resource overhead of WUS |
| ZTE, Sanechips |  | Based on the updated version, we feel it is too early to say the DL WUS is for paging. Also it could be used for RACH or data transmission. We should study all the potential function firstly. Additionally, not only the idle state, inactive state also should be considered. Currently, we could keep it open like ‘at least in idle state’. |
| Apple |  | OK with the updated proposal (P4r1) |
| Huawei, HiSilicon | Y | We are OK with the update. To rsesovle ZTE’s concern, a sub-bullet like ‘FFS functionalities other than wakeup’ can be added (though I know this is under discussion in other section). |
| MTK1 |  | We suggest to first examine baseline paging indication functionality. The 2nd and 3rd use cases can be pursue further after baseline functionality can be fulfilled. [FL1]Proposal 4 (with modification): For 6GR DL WUS triggering UE for at least the following paging related use case in RRC idle state, - Triggering paging PDCCH monitoring - FFS: WUS indicating paging information, SI updated and ETWS/CMAS indication study UE performance and network impact, regarding at least the following: • UE energy saving gain • Impact to latency, if applicable • Network overhead • Network energy consumption • Other impacts, if any |
| Mod |  | Slightly updated based on newly received comments and make a clean version for easier discussion - [FL1]Proposal 4r2: Study 6GR DL WUS waking up UE for paging PDCCH monitoring at least in RRC idle state, regarding at least the following: - UE energy saving gain - Impact to latency, if applicable - Network overhead - Network energy consumption - Other impacts, if any - FFS other functionalities - Other RRC state is not precluded and it is up to RAN2 discussion |
| Mod2 |  | Based on the online, updated as below: [FL2]Proposal 4: Study 6GR DL WUS waking up UE for paging purpose at least in RRC idle state, for at least: - UE energy saving gain - Impact to latency, if applicable - Network overhead - Network energy consumption - Other impacts, if any - Other RRC state if supported |
| Huawei & HiSilcion |  | Based on the discussion yesterday we support removing “paging purpose”. At this stage we are open to study all the possibilities of UL WUS. |
| vivo | Y | We support this proposal to keep paging in the main bullet. We need to have some focus for DL WUS usage for idle mode. |

In addition, [Nokia] [Qualcomm][vivo][Huawei][Ericsson][ZTE][Samsung][LGE][MTK][Google]  identify the following issues for study:

- To reduce false alarm rate caused by WUS indication granularity  [Nokia] [Qualcomm][vivo][Huawei] [Ericsson][ZTE][Google]

- To reduce latency for DL WUS triggered paging reception caused by synchronization after waking up   [Nokia] [Qualcomm][vivo]

- To improve NES gain by considering PO adaptation [Nokia][Huawei][Samsung][LGE][MTK]

    - Use case #2: Power saving for RRM measurement

[Qualcomm][vivo][Huawei][Nokia][CATT][xiaomi][Samsung][Ericsson][DOCOMO][OPPO][WILUS][InterDigital][NEC][Lenovo] discuss RRM measurement including both serving cell and neighboring cell performed by EE processing can reduce the overall wakeup duration of UE, which brings considerable power saving gain.

[vivo] provides evaluation results, indicating that for the case with serving cell RRM measurement performed by EE processing and neighboring cell measurement by non-EE processing with relaxation facor of 4~16, 20.82%~26.1% power saving gain can be observed if neighboring cell measument with intra-frequency measurement only is assumed and  18.53%~25.36% power saving gain can be observed if neighboring cell measument with both intra-and inter-frequency measurement is assumed; for the case with both serving cell and neighboring cell RRM measurement offloaded from non-EE processing to EE processing, 28.06% power saving gain can be observed if neighboring cell measument with intra-frequency measurement only is assumed  43% power saving gain can be observed if neighboring cell measument with both intra-and interfrequency measurement is assumed.

- (merged to proposal 7)[FL1]Proposal 5: Study power saving gain for RRM measurement including both serving cell and neighboring cell in RRC idle performed by EE processing.

| Company | Y/N | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Y |  |
| Nokia |  | Should this proposal be merged with proposal 7? |
| LGE | Y | We are generally fine with the proposal. It seems that Proposal 5 and 7 target for studying performance of RRM measurement in RRC idle performed by EE processing. One suggestion is to merge Proposal 5 and 7, since they are discussing similar topics. |
| Xiaomi | Y |  |
| FW | Y |  |
| InterDigital | Y |  |
| Spreadtrum | Y |  |
| WILUS | Y |  |
| TCL | Y |  |
| OPPO | Y | Support. |
| Ofinno | Y |  |
| vivo | Y |  |
| Lenovo | Y | We already support RRM measurement for the serving cell in 5G Rel19 and question remains is the coverage, whether there is any entry and exit condition and its impact on the power savings. The study should include the feasibility to do neighbour cell measurement, complexity and its limitation. |
| DOCOMO |  | Although the proposal may be reasonable, we still think that before above proposal, it is better to discuss/decide the RS for RRM measumrenet and coverage of such RS for measurement, e.g., Proposal 7 in section 4.2.2. |
| Samsung | Y | This issue is also discussed in 10.6.1.1. Duplicated discussion should be avoided. |
| CATT | Y |  |
| Qualcomm | Y |  |
| Ericsson | Y |  |
| Google | Y | Offloading RRM measurements (both serving and neighbor cells) to EE processing is critical for 6G battery life. The study must address the trade-off between power consumption and measurement accuracy/neighbor cell identification robustness to ensure mobility performance is not degraded. |
| Mod |  | It seems no concern from companies to study PSG of RRM measurement, and as suggested by companies, this proposal is merged to proposal 7. |
| MTK1 | N | We have strong technical concerns regarding neighbor cell measurement in EE processing. It adds significant complexity (e.g., SIC) which conflicts with the low-power assumption. We suggest to study for the serving cell first. [FL1]Proposal 5 (with modification): Study power saving gain for RRM measurement for serving cell and neighboring cell in RRC idle performed by EE processing. - FFS: neighboring cell |

In addition, for RRM measurement based on EE processing,  the following issues are identified by [vivo][Huawei][NEC]:

- Neighboring cell identification [vivo][Huawei][NEC]

- Interference handling [Huawei]

  - Functionalities/procedures by 6GR DL WUS operation

    - 6GR DL WUS monitoring procedure

[Nokia][vivo][Huawei][Qualcomm][Ericsson][Samsung][ZTE][Apple][CATT][OPPO][Spreadtrum][TCL][Futurewei][Google][Sony][Sharp] propose to study DL WUS procedure by considering the following aspects:

- WUS monitoring periodicity [Nokia][vivo]

- WUS monitoring occasions [vivo] [Qualcomm][ZTE][Spreadtrum][OPPO][Sony]

- WUS indication granularity [Nokia][vivo][Huawei] [Ericsson][Samsung][ZTE][OPPO][Spreadtrum][Google][Futurewei][Sharp]

- Dynamic PO [Nokia][Huawei][MTK][Sharp]

- UE behaviors after waking-up[vivo][Huawei][ZTE]

- Additional synchronization/reference signals prior paging occasion(s) to reduce latency for DL WUS triggered paging reception [Nokia][vivo][Apple][TCL][Spreadtrum][CATT]

[HW][ZTE] discuss the UE behavior upon DL WUS detection: if DL WUS is designed to indicate per group/subgroup information, the functionality of DL WUS is to wake up MR to perform paging procedure;while if DL WUS is designed to indicate UE dedicated information, the functionality of DL WUS may be to wake up MR to perform RACH procedure

[E///][vivo][Spreadtrum][Google] propose to study 6GR DLWUS moniroing procedure by taking NR WUS monitoring procedure as a starting point/baseline.

- [FL1]Proposal 6: For RRC idle state, study DL WUS procedure regarding at least the following aspects:

- WUS monitoring periodicity

- WUS monitoring occasions

- WUS indication granularity

- UE behaviors after waking-up, including direct RACH transmission, PO determination, and synchronization, if applicable

| Company | Y/N | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Y |  |
| Nokia |  | Like noted in our paper, the number and need of MOs as well the indication granularity would depend on the 6GR WUS physical design. From e.g. power saving perspective it is of course relevant to study the impact of these assumptions. The last bullet seems to be more related to proposal 4, thus suggest to remove the bullet |
| LGE | Y | We are generally fine with the proposal. |
| Xiaomi | Y |  |
| FW | Y |  |
| Spreadtrum |  | We think the follow aspects on DL WUS procedure also should be studied. - Wake-up delay - QCL for multi-beam operation Therefore, we suggest to modified the proposal as follow: - [FL1]Proposal 6: For RRC idle state, study DL WUS procedure regarding at least the following aspects: - WUS monitoring periodicity - WUS monitoring occasions - WUS indication granularity - Wake-up delay - QCL for multi-beam operation - UE behaviors after waking-up, including direct RACH transmission, PO determination, and synchronization, if applicable |
| WILUS | Y | We support the proposal |
| TCL | Y | We agree with this proposal. For last sub-bullet, we think DL-WUS could be used for reminding UE to receive 6GR SSB or other sync signal for UE sync, which is beneficial for UE to reduce the waiting time for SSB reception. |
| OPPO | Y |  |
| Ofinno |  | In our view WUS indication granularity may depend on what information is agreed to be indicatd by DL WUS. We suggest to either delay the discussion on this aspect until more progress is made or change the bullet to say “paging monitoring indication granularity”. |
| Vivo | Y |  |
| Lenovo | Y | The need for any entry and exit condition if any |
| DOCOMO |  | For the detailed configuration of DL WUS in IDLE, it is a bit early for discussion. For the behaviors after waking-up, more clarification on these procedure are needed. |
| Samsung | Y |  |
| CATT |  | - WUS indication granularity |
| Qualcomm | Y | We support the proposal, but it need to also include the gap between WUS monitoring occasion and the PO - [FL1]Proposal 6-Qualcomm: For RRC idle state, study DL WUS procedure regarding at least the following aspects: - WUS monitoring periodicity - WUS monitoring occasions - WUS indication granularity - UE behaviors after waking-up, including direct RACH transmission, PO determination, and synchronization, if applicable - Time between the WUS monitoring occasion and the associated PO. |
| Ericsson |  | Include minimum gap and offset configuration as discussed in Rel-19. |
| Google | Y | Spreadtrum’s update looks good to us. Studying ‘Wake-up delay’ is beneficial of enovo power saving with latency. |
| ZTE, Sanechips |  | In NR, multi-beam operations are supported for LP-WUS and LP-SS for idle/inactive state, where the group WUS are associated with SSB. In 6G, the beam relationship of LP-WUS should also be designed. In NR, UE can report the LP-WUS wake-up delay of capability. In 6G, even LPR may not be required, the wake up delay still needs to be considered. As discussed above, the beam, and wake up delay of WUS should also be considered. In addition, WUS triggers UE specific PDCCH monitoring，UE specific PDSCH receiving should also be added in UE behaviors after waking-up. And following modification is proposed. - [FL1]Proposal 6: For RRC idle/inactive state, study DL WUS procedure regarding at least the following aspects: - WUS monitoring periodicity - WUS monitoring occasions - WUS indication granularity - Beam of WUS - Wake up delay of WUS - UE behaviors after waking-up, including direct RACH transmission, PO determination, UE specific PDCCH monitoring，UE specific PDSCH receiving, and synchronization, if applicable |
| Apple |  | Generally fine with the proposal. Can include multi-beam operation and wake-up delay. |
| MTK1 |  | Regarding "WUS indication granularity", we emphasize the necessity of subgrouping to minimize false alarms. Regarding UE behaviors after wake-up, we suggest to study baseline operations first. [FL1]Proposal 6 (with modification): For RRC idle state, study DL WUS procedure regarding at least the following aspects: - WUS monitoring periodicity - WUS monitoring occasions - WUS indication granularity, including UE subgrouping - UE behaviors after waking-up, including direct RACH transmission, at least PO determination, and synchronization, if applicable |
| Mod |  | Slightly updated based on newly received comments and make a clean version for easier discussion: - [FL1]Proposal 6r1: Study DL WUS procedure for at least RRC idle state, regarding at least the following aspects: - WUS monitoring periodicity - WUS monitoring occasions - WUS indication granularity - UE behaviors after waking-up, including direct RACH transmission, PO determination, and synchronization, if applicable - Multi-beam operation including at least [ QCL assumption ] - [Wake up delay] - Other RRC state is not precluded and it is up to RAN2 discussion |
| Mod2 |  | [FL2]Proposal 6: Study 6GR DL WUS procedure for at least in RRC idle state, for at least: - WUS monitoring periodicity - WUS monitoring occasions - WUS indication granularity - UE behaviors after waking-up, including direct [RACH transmission], PO determination, and synchronization, if applicable - Multi-beam operation including at least QCL assumption - [Wake up delay] - Other impacts, if any - Other RRC state is not precluded and it is up to RAN2 discussion |
| Huawei & HiSilcon |  | We support deleting every thing after “UE behaviors after waking-up”. At this stage we are open to explose all the posibilites of using UL WUS. |
| vivo | Y | Considering [FL2]Proposal 4, we are fine to keep RACH transmission. |

    - RRM measurement based on EE processing

[Qualcomm][vivo][Huawei][CMCC][Samsung][DOCOMO][ZTE][Apple][OPPO][CATT][Spreadtrum][WILUS] discuss that considering  6GR DL WUS is based on OFDM sequence, it is possible to perform RRM measurement based on 6GR measurement signal, i.e., at least PSS/SSS by EE processing, and thus, dedicated reference signal for EE processing measurement may not be enovory to reduce system overhead. It is suggested to study RRM measurement based on 6GR measurement signal by EE processing.

In addition, the following issues are identified for study:

- Neighboring cell identification for neighboring cell RRM enovoryt [vivo][Huawei][NEC]

- Impact on the EE processing complexity to maintain acceptable measurement accuracy for neighboring cell RRM enovoryt [Huawei] [Apple]

- coverage and accuracy for both serving cell and neighboring cell RRM measurement [vivo] [ZTE][Apple][OPPO][Xiaomi]

- feasibility of neighbour cell measurement including intra-frequency and inter-frequency measurement [DOCOMO][ZTE][vivo][Ericsson][Apple][OPPO][CATT]

- [FL1]Proposal 7:

For RRC idle state, study serving cell RRM measurement based on 6GR measurement signal (e.g., PSS/SSS) by EE processing regarding at least the following:

- coverage (e.g., achievable SINR/SNR) and accuracy

For RRC idle state, study neighboring cell RRM measurement based on 6GR measurement signal (e.g., PSS/SSS) by EE processing, regarding at least the following aspects :

- Neighboring cell identification

- Coverage(e.g., achievable SINR/SNR) and accuracy

- Impact on the EE processing complexity

| Company | Y/N | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Y | We are generally OK with this proposal, but it may be merged together with proposal 5? |
| Nokia |  | Should this be merged with proposal 5? |
| LGE | Y | We are general fine with the proposal. Same comments as above. Suggest to merge Proposal 5 and 7. |
| Xiaomi |  | We generally agree with the proposal. As we analyzed in Section 3.6, the 6GR measurement signal also includes an LP-SS-like signal. We suggest the following modifications: - [FL1]Proposal 7 : For RRC idle state, study serving cell RRM measurement based on 6GR measurement signal (e.g., PSS/SSS/LP-SS-like signal) by EE processing regarding at least the following: - coverage (e.g., achievable SINR/SNR) and accuracy For RRC idle state, study neighboring cell RRM measurement based on 6GR measurement signal (e.g., PSS/SSS/LP-SS-like signal) by EE processing, regarding at least the following aspects : - Neighboring cell identification - Coverage(e.g., achievable SINR/SNR) and accuracy - Impact on the EE processing complexity |
| FW |  | We are OK with the study |
| InterDigital |  | Fine |
| Spreadtrum |  | We are fine to study serving cell and/or neighboring RRM measurement based on 6GR measurement signal (e.g., PSS/SSS) by EE processing in RRC idle mode. Entry and exit conditions for EE process mode and impact on performance of cell reselection are also need to be study. Therefore, we suggest to modified the proposal as follow: - [FL1]Proposal 7: For RRC idle state, study serving cell RRM measurement based on 6GR measurement signal (e.g., PSS/SSS) by EE processing regarding at least the following: - coverage (e.g., achievable SINR/SNR) and accuracy - Entry and exit conditions for EE process mode For RRC idle state, study neighboring cell RRM measurement based on 6GR measurement signal (e.g., PSS/SSS) by EE processing, regarding at least the following aspects : - Neighboring cell identification - Coverage(e.g., achievable SINR/SNR) and accuracy - Entry and exit conditions for EE process mode - Impact on the EE processing complexity - Impact on cell reselection |
| WILUS | Y |  |
| TCL |  | We do not know if RRM measurement should be discussed in this topic due to it seems some part been overlapped with OFDM-based DL-WUS design topic. For example, one proposal been proposed in AI 10.6.1.1 FL Proposal 3.4.3 Study feasibility of RRM offloading to EE processing state with respect to - RRC states - Serving and/or neighbour cell - Other aspects are not precluded |
| OPPO | Y |  |
| Ofinno | Y |  |
| vivo | Y |  |
| Lenovo | Y | Meaurment capability and measurement time budgets can be different for EE processing state and non-EE processing state. It is not clear whether there will be any entry and exit condition and should be discussed. |
| DOCOMO | Y | We suggest to study the feasibility of neighbor cell measurement. So we suggest the update of the proposal as follow. For RRC idle state, study neighboring cell RRM measurement based on 6GR measurement signal (e.g., PSS/SSS) by EE processing, regarding at least the following aspects : - Feasibility of neighbor cell measurement - Neighboring cell identification - Coverage(e.g., achievable SINR/SNR) and accuracy Impact on the EE processing complexity |
| Samsung |  | Agree in general. One comment is that all the example in bracket can be removed. |
| CATT |  | - In general OK. Just wonder why ‘Impact on the EE processing complexity’ is not applicable for the first one |
| Qualcomm | Y |  |
| Ericsson | Y |  |
| Google | Y |  |
| Mod |  | The proposal is updated to reflect companies’ comments and merged with proposal 5: - [FL1]Proposal 7r1: For RRC idle state, study serving cell RRM measurement based on 6GR measurement signal (e.g., PSS/SSS) by EE processing regarding at least the following: - Power saving gain - coverage (e.g., achievable SINR/SNR) and accuracy For RRC idle state, study neighboring cell RRM measurement based on 6GR measurement signal (e.g., PSS/SSS) by EE processing, regarding at least the following aspects : - Power saving gain - Neighboring cell identification - Coverage(e.g., achievable SINR/SNR) and accuracy - Impact on the EE processing complexity @ Xiaomi, considering 6GR DL WUS is based on OFDM sequence, it is possible to perform RRM measurement based on 6GR measurement signal, i.e., at least PSS/SSS by EE processing, and thus, dedicated reference signal for EE processing measurement may not be enovory to reduce system overhead. @ spreadtum, enovo the necessity of Entry and exit conditions for EE process mode is based on coverage study, which can be studied later. And for cell-reselection, it would be also next step study after neighboring measurement study. @TCL, after discussing with Magnus, measurement will be handled in this section. @DOCOMO, after study Neighboring cell identification and coverage(e.g., achievable SINR/SNR) and accuracy study, we can figure out the feasibility @CATT, since neighboring cell measurement involves measurement for more cells and neighboring cell identification, for which the complexity may increase compared to serving cell measurement |
| ZTE, Sanechips |  | OK with Proposal 7r1 with minor updates At least for For RRC idle state, …. |
| Apple |  | Generally fine with the direction but there are similar proposals in AI 10.6.1.1. Need to decide where to discuss. |
| Huawei, HiSilicon | Y | OK with the update. |
| MTK1 |  | We support the study of serving cell RRM measurement. However, regarding the second bullet (neighboring cell), we object to its inclusion at this stage. As stated in our Observation 3, neighbor cell monitoring complexity conflicts with the low power assumption of EE processing. The study should focus on serving cell measurement coverage and accuracy. [FL1]Proposal 7 (with modification): For RRC idle state, study serving cell RRM measurement based on 6GR measurement signal (e.g., PSS/SSS) by EE processing regarding at least the following: - coverage (e.g., achievable SINR/SNR) and accuracy For RRC idle state, study neighboring cell RRM measurement based on 6GR measurement signal (e.g., PSS/SSS) by EE processing, regarding at least the following aspects : - Neighboring cell identification - Coverage(e.g., achievable SINR/SNR) and accuracy - Impact on the EE processing complexity |
| Mod2 |  | - [FL2]Proposal 7: For RRC idle state, study serving cell RRM measurement based on 6GR measurement signal (e.g., PSS/SSS) by EE processing regarding at least the following: - Power saving gain - coverage (e.g., achievable SINR/SNR) and accuracy For RRC idle state, study neighboring cell RRM measurement based on 6GR measurement signal (e.g., PSS/SSS) by EE processing, regarding at least the following aspects : - Power saving gain - Neighboring cell identification - Coverage(e.g., achievable SINR/SNR) and accuracy - Impact on the EE processing complexity |
| vivo | Y | Considering 6GR EE processing is more capable compared to NR, at study phase, we support to study the measurement by EE processing for both serving and neighbor cell. |

    - Power saving for functionalities other than waking up and measurements

Companies also discuss to study the following new functionalities potentially to be indicated by DL WUS:

- SI updated and ETWS/CMAS indication  [Nokia][Spreadtrum][CATT][LGE][WILUS]

- System information delivery indication [Nokia]

- Cell common signals transmission manner indication [Nokia][LGE]

- If CA framework in idle/inactive state is supported, how the WUS operate with multi-carrier/cell [ZTE]

- Cell DTX/DRX pattern adaptation [Spreadtrum][NEC]

- PRACH resource adaptation [Spreadtrum][LGE]

- waking-up UE to receive 6G synchronization signal [TCL][Spreadtrum]

- PO adaptation[Samsung]

- Small data transmission[CMCC][ZTE][Spreadtrum]: To avoid uplink activity for DL small data, [CMCC] propose to study DL WUS to bring group/UE ID and small data together with two alternatives

- Alt 1: the DL WUS can be used to bring group/UE ID and small data together.

- Alt 2: the DL WUS can be used to bring group/UE ID and trigger UE to receive small payload packet.

Proponent companies are encouraged to provide more details of new functionalities.

| Company | Comments/suggestions |
| --- | --- |
| Huawei, HiSilicon | Instead of categorizing ‘new functionalities’, it is better to categorize ‘functionalities other than wakeup’. If so, first short message should be there. Also, we support to study PO adaptation. Maybe in this meeting we can list all the possible functionalites for further study, like ‘The following funcationalites other than wakeup are identified for further study…’ |
| TCL | We think DL-WUS could be used for reminding UE to receive 6GR SSB or other sync signal for UE sync, which is beneficial for UE to reduce the waiting time for SSB reception. If one UE is always on line to search SSB, which will consume more energy or take more complexity if no SSB transmission in current cell or the start time of SSB transmission is longer than X ms, e.g., X equals to 320ms. One way to solve the mentioned issue is to keep SSB transmission in each band or cell, however, it will bring more energy cost at base station side. Thus, we give another way to solve above mentioned issue according to send DL-WUS before SSB transmission Like proposal 6, we suggest considering DL-WUS used for waking up UE for 6G synchronization signal reception during initial access or RRC non-connected state. |
| OPPO | We can give the possible other indication than wake-up in a list. Then we can discuss to select most supported items based on discssuion further. If DL WUS can indicate presence of aperiodic reference signal, e.g., TRS, it could allow UE to quickly synchronize its receiver rather than waiting for the next 6GR sync signal, e.g., PSS/SSS or CSI-RS. If PDCCH Blind Decoding Reduction information could be carried by DL WUS, UE could monitor the PDCCH in a reduced blind decoding configuration to achieve more power saving. Thus, regarding the possible indication other indication than wake-up, we think following could also be studied. - Indicate the presence of Aperiodic reference signal for synchronization. - Providing information to reduce complexity of PDCCH blind detection after wake-up. |
| DOCOMO | The proposal is related to Proposal 4 in section 4.1.1. But the priority of listed functionalities in this proposal is lower than functionality listed in Proposal 4. |
| Samsung | Low priority. New use cases can be discussed in 10.6.3. |
| Qualcomm | Wouldn’t other functionality fall under the inactive 10.6.3 agenda item? |
| Mod | Companies are encouraged to share your views on these functionalities if not shared yet. |

- 6GR DL WUS operation in RRC connected

  - Use Case #1: DL LP-WUS triggering PDCCH monitoring

Almost all companies discuss and propose the first use case or function of DL WUS in RRC connected is to trigger PDCCH monitoring for UE power saving.

Evaluation results for connected state

- [HW] provides the evaluation results on energy saving gain, and UPT/latency/QoS satisfaction rate impact of 6G DL WUS in RRC connected state as shown in Figure 7 below and observed following

  - If the monitoring periodicity of 6G DL WUS is equal to C-DRX cycle of the reference setting.

    - For FTP traffic, 15.2%~40.1% energy saving gain can be obtained for various MR sleep states, while there is UPT/latency loss, and the QoS satisfaction is also reduced.

    - For IM traffic, 70.2% energy saving gain can be obtained for MR deep sleep, while there is UPT/latency loss.

  - If the monitoring periodicity of 6G DL WUS is smaller than C-DRX cycle of the reference setting.

    - For FTP traffic, 3.1%~21.1% energy saving gain can be obtained for various MR sleep states, while there is also UPT/latency gain.

    - For IM traffic, 64.2% energy saving gain can be obtained for MR deep sleep, while there is also UPT/latency gain.

    - For XR traffic, 6.3%~19.1% energy saving gain can be obtained for MR micro sleep and light sleep, while the UPT/latency performance is comparable.

- [Apple] provides the evaluation results on energy saving gain, and UPT impact between baseline scheme of “C-DRX + R16 WUS (DCP) + cross-slot scheduling + R17 PDCCH skipping (8 slot)” and 6G DL WUS scheme of “6G WUS replaces R16 DCP + 6G WUS is also enabled during C-DRX ON duration with different MR sleep state (deep or light or micro sleep)” and observed deeper sleep state during 6G WUS monitoring provides more UE energy saving gain, but also leads to lower average UPT. Hence the UE energy saving gain and the average UPT gain/loss depends on how 6G WUS operates and the sleep state during 6G WUS monitoring.

- [MTK] provides the evaluation results on energy saving gain, and latency for VoIP, FTP and IM traffic of 6G DL WUS in RRC connected state as shown in Figures below and observed following:

  - For VoIP traffic, with WUS monitoring period of 10ms (1/4 of the DRX cycle) and 10ms UE wake-up delay allowed, UE can achieve both power saving gain of 23.6% and shorter average packet latency than 6G baseline of 40ms C-DRX for PDCCH monitoring.

  - For FTP traffic, applying WUS monitoring during PDCCH skip duration can achieve 29.2% power saving gain for FTP traffic with minor increment in average packet latency (5.25ms increase from 48.6ms to 53.8ms).

  - For IM traffic, Enhancement 1 (C-DRX for WUS) and Enhancement 2 (WUS-assisted PDCCH skipping) are complementary. Applying both to IM traffic jointly achieves 50.1% power saving gain and reduces average packet latency from 147.6ms to 37.6ms (74.5% reduction).

- [QC] provides the evaluation results on energy saving gains using EE processing between two cases. In the first EE processing is used only to monitor DL WUS prior to a DRX on-duration; while in the second, EE processing is also used to perform measurements. It is observed that in both cases, there is some energy saving gain from introducing DL WUS and that gain grows significantly once measurements are performed in EE processing mode.

As shown above, for most cases, DL WUS can bring power saving benefits.

Besides, based on the agreed evalution assumptions as listed below for energy saving schemes for 6GR, FL suggests the following proposal:

RAN1 #122bis Agreement

At least the following NR metrics,

- Network energy saving gain relative to baseline for BS

- UE energy saving gain relative to baseline for UE

- Impact to UPT (User-Perceived Throughput), if applicable,

as well as the metrics

- Impact to latency, if applicable

- Impact to QoS/delay budget satisfaction rate, if applicable

are used for 6G energy efficiency evaluation.

RAN1 #123 Agreement

For evaluation purposes and relative comparison over different candidate energy saving schemes for 6GR, define the following baseline power saving configurations for UE for evaluation purpose for FR1 (including around 7GHz):

- 5G NR I-DRX (1.28s cycle) for idle mode

  - Group paging rate (for a PO): TBD

- 5G NR C-DRX settings of (cycle, on-duration timer, inactivity timer) are assumed for the following 6GR traffic models for connected mode:

  - VoIP: (40 ms, 8 ms, 10 ms)

  - FTP3: (160 ms, 8 ms, 100 ms)

  - Instant message : (320 ms, 8 ms, 100 ms)

  - XR: (16 ms, 10 ms, 4 ms)

- Companies can evaluate and report other traffic(s) and/or configuration(s) with justification

Note: The corresponding evaluation is not intended for energy efficiency comparison with 5G/NR.

- [FL1]Proposal 8: For 6GR DL WUS triggering PDCCH monitoring in RRC connected state,  study UE performance and network impact, for at least:

- UE energy saving gain

- Impact to UPT, if applicable

- Impact to latency, if applicable

- Impact to QoS/delay budget satisfaction rate, if applicable

- Network overhead

- Network energy consumption

- Othe impacts, if any

| Company | Y/N | Comments |
| --- | --- | --- |
| Huawei, HiSilicopn | Y |  |
| Nokia | Y | Our understanding of this proposal is that it is identifying the key metrics to be analyzed with subsequent WUS in Connected mode studies. It is not the intention of this specific proposal to limit/constrain the studies. |
| LGE | Y | Fine with the proposal |
| Xiaomi | Y |  |
| FW | Y |  |
| InterDigital | Y |  |
| Spreadtrum | Y |  |
| WILUS | Y |  |
| TCL | Y |  |
| OPPO |  | Few clarification, what is the Network overhead? Would the T/F resources be the Nwoverhead? Or it means the basestation complexiy and cost? |
| Ofinno | Y |  |
| vivo | Y |  |
| Lenovo | Y |  |
| DOCOMO | Y |  |
| Samsung | Y |  |
| CATT | Y |  |
| Qualcomm | Y |  |
| Google | Y | We emphasize the importance of evaluating ‘Impact to UPT’ and ‘Latency’ thoroughly, as user experience in RRC Connected state is highly sensitive to wake-up delays. |
| Mod |  | Seems this proposal is agreeble to all @Nokia, yes, the intention is to study WUS in Connected mode with applicable metrics. No intention on limiting study. |
| ZTE, Sanechips |  | For WUS monitoring without DRX, CSI and RRM procedure may be impacted. When and how UE perform CSI/RRM measurement and reporting needs to be discussed. And the following modification is proposed, - [FL1]Proposal 8: For 6GR DL WUS triggering PDCCH monitoring in RRC connected state, study UE performance and network impact, for at least: - UE energy saving gain - Impact to UPT, if applicable - Impact to latency, if applicable - Impact to QoS/delay budget satisfaction rate, if applicable - Network overhead - Network energy consumption - CSI report and RRM measurement - Othe impacts, if any |
| Apple |  | Generally fine with the proposal, but I wonder if the proposal intends to say we support DL WUS triggering PDCCH monitoring already, or we study it. Either way it is good to clarify. |
| MTK1 |  | We support this proposal. On the other hand, we suggest network overhead (resource and energy consumption) should be handled in DL-WUS design agenda while focusing on trade-off between UEPS and performance (UPT, latency, QoS) in this agenda item. Conseuqently, we suggest the following revision: [FL1]Proposal 8: For 6GR DL WUS triggering PDCCH monitoring in RRC connected state, study UE performance and network impact, for at least: - UE energy saving gain - Impact to UPT, if applicable - Impact to latency, if applicable - Impact to QoS/delay budget satisfaction rate, if applicable - Network overhead - Network energy consumption - Othe impacts, if any |
| mod |  | Slightly updated to relect companies’ comments - [FL1]Proposal 8r1: Study 6GR DL WUS triggering PDCCH monitoring in RRC connected state, for at least: - UE energy saving gain - Impact to UPT, if applicable - Impact to latency, if applicable - Impact to QoS/delay budget satisfaction rate, if applicable - Network overhead - Network energy consumption - Othe impacts, if any |
| Mod2 |  | Merge proposal 9 into proposal 8 - [FL2]Proposal 8: Study 6GR DL WUS triggering PDCCH monitoring with and without C-DRX in RRC connected state, for at least: - UE energy saving gain - Impact to UPT, if applicable - Impact to latency, if applicable - Impact to QoS/delay budget satisfaction rate, if applicable - Network overhead - Network energy consumption - Othe impacts, if any |
| vivo | Y |  |

In addition, following sub-use cases are proposed to be studied by companies:

Sub use case#1-1: DL WUS monitoring according to the DL WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. [Nokia], [CMCC], [QC], [ETRI]

- [HW]: WUS Option 1-1 and Option 1-2 either one has better performance than Rel-15 C-DRX, while both options have the similar energy consumption and latency performance, if they are configured with the same key parameters (e.g., monitoring periodicity, monitoring duration and etc) and RAN1 targets for supporting a single procedure.

- [ZTE] (scheme#1)

- [SPD]: Option 1-1 and Option 1-2 in NR can be used as a starting point.

- [Apple] with supporting separate config. Of measurement cycle,

Sub use case#1-2: PDCCH monitoring is triggered by DL-WUS only and not triggered by C-DRX cycle and drx-onDurationTimer when monitoring DL WUS.

Companies like [Nokia], [vivo], [CATT], [OPPO], [LG], [Apple], and [WILUS] have noted that for NR Rel-19 Option 1-2, even though C-DRX is configured, the PDCCH monitoring is controlled by LP-WUS, independent of the C-DRX cycle. C-DRX remains in use primarily because the measurement cycle and RRM/RLM measurement relaxation depend on the applied C-DRX or short C-DRX cycle, which alleviates UE measurement power consumption for NR WUS option 1-2.

Therefore, following sub use case#1-4 by removing the constraints imposed by NR option 1-2 is proposed to be studied by [ZTE] (Scheme#3), [CATT], [vivo], [CMCC], [OPPO], [TCL], [LG], [Apple?], [WILUS], allowing for increased flexibility as well as a streamlined and simplified design.

- [CATT] [CMCC], [OPPO], [TCL], [LG], [WILUS]

- [ZTE] (scheme#2), (Scheme#3)

- [SPD]: Option 1-1 and Option 1-2 in NR can be used as a starting point.

- [HW]: WUS Option 1-1 and Option 1-2 either one has better performance than Rel-15 C-DRX, while both options have the similar energy consumption and latency performance, if they are configured with the same key parameters (e.g., monitoring periodicity, monitoring duration and etc) and RAN1 targets for supporting a single procedure.

- [MTK]: modify C-DRX mechanism e.g., configure WUS MO instead of PDCCH MOs for the cycle and on-duration parameters

- [Apple] with supporting separate config. Of measurement cycle,

- [QC]: Study DL WUS triggering PDCCH monitoring outside C-DRX active time (e.g. compare it with using shorter C-DRX cycles).

- [Interdigital]: Support aperiodic UE DRX active time based on DL-WUS indication

- [vivo]: 6GR DL WUS is used without any DRX cycle configuration

Sub use case#1-3: DL WUS monitoring inside at least C-DRX active time according to the DL WUS monitoring configuration to trigger PDCCH monitoring (Studied in Rel-19 WUS operation).

- [Nokia]: revisiting ACTIVE time usage of WUS, considering the new EE processing state, expected payload improvements and the single OFDM-sequence based design of the WUS, that reuses the main radio.

- [Apple], [MTK], [QC]: consider WUS during C-DRX active time together with other NR features such as PDCCH skipping enabling WUS monitoring during C-DRX inactivity timer period/skipping duration

Based on the sub use cases proposed by companies, FL observes they can be divided into two candidate direction for functionality study, and thus, suggest the following:

- [FL1]Proposal 9: for RRC connected state, study DL WUS triggering PDCCH monitoring with and without C-DRX.

| Company | Y/N | Comments |  |  |
| --- | --- | --- | --- | --- |
| Huawei, HiSilicon | Y |  |  |  |
| Nokia | N | We support the study of WUS with DRX for the following reasons: DRX is proven and low overhead DRX provides an excellent fallback mechanism / PS scheme when WUS cannot be supported. As noted RAN2 is taking DRX as a baseline, thus we should first focus to it to avoid overlapping designs. As discussed, based on EE-processing the power benefits (over DRX based) are not evident. |  |  |
| LGE | Y | In Rel-19 NR LP-WUS, WUS triggering PDCCH monitoring with/without C-DRX has been introduced, which only makes fragmented specification fragment. In 6GR WUS, we think one key principle is to avoid duplicated functionality. In this regards, we think a single approach is sufficient for DL WUS operation in RRC CONNCECTED, and we prefer WUS without C-DRX for simplicity and echanismy. Given that some companies think WUS with C-DRX needs to be study, we are OK with the proposal to study which approach is suitable for RRC conncected. |  |  |
| Xiaomi |  | C-DRX is a successful mechanism, and DL WUS triggering PDCCH monitoring with DRX can make the entire design simpler. |  |  |
| FW | Y |  |  |  |
| InterDigital | Y |  |  |  |
| ETRI | Y | Even though we think DL WUS with DRX configuration is basic operation, we are OK to study DL WUS without DRX configuration also. |  |  |
| Spreadtrum | Y |  |  |  |
| WILUS | Y |  |  |  |
| TCL | Y |  |  |  |
| OPPO | Y | We think the DL-WUS should be able to work without configuration of DRX. So, both case should be studied. |  |  |
| Ofinno |  | In general we are okay to study with and without DRX configuration. One question is if “with DRX configuration” includes both DL WUS in active time and inactive time? |  |  |
| Vivo | Y | Reply to Ofinno that our understanding is at this stage “with DRX configuration” includes both DL WUS in active time and inactive time. |  |  |
| Lenovo | Y | We can take 5GNR discussion as baseline and should downselect only one solution from option1-1, option1-2 and option1-3 from 5GNR. Also consider the lower tier device type |  |  |
| DOCOMO |  | First of all, RAN2 may continue to use C-DRX for PDCCH monitoring for UE power saving. For the discussion of DL WUS operation in CONNECTED, we could consider that as basis. So for case of “without C-DRX”, it is better to clarify whether it is R19 Option 1-2 like operation, which considers to resue other timers of C-DRX, or other operation without considering other timers of C-DRX. Gerally, it is fine to study options. But from operator perspective, we prefer one option in 6GR for easy deployment. |  |  |
| Samsung | Y | Since C-DRX liked echanism is supported in RAN2, RAN1 can study the DL WUS operation with and without C-DRX to aglin with the intention in RAN2. |  |  |
| CATT |  | We first need to clarify the relation ship between WUS and DRX, |  |  |
| Qualcomm | N | Like Nokia, we would like to focus on interaction with DRX at this point. |  |  |
| Ericsson |  | We think clarification on C-DRX is needed, if it refers to 6G C-DRX design or 5G C-DRX. |  |  |
| Google | Y |  | We support studying both directions. However, we view ‘With C-DRX’ as a necessary baseline for robustness and link recovery (fallback). ‘Without C-DRX’ may be considered as an enhancement for specific traffic types but presents risks regarding reachability if WUS detection fails. |
| Mod |  |  | In NR WUS, both directions are supported with benefits. It’s worthy to study both of them. |  |
| ZTE, Sanechips | Y |  |  |  |
| Apple | Y |  |  |  |
| MTK1 |  |  | Given RAN2 may have discussion on 6G C-DRX, it is not clear what mechanism to be jointly considered. One the other hand, ignoring C-DRX may end up with duplicated functionality with RAN2 study. [FL1]Proposal 9: for RRC connected state, study DL WUS triggering PDCCH monitoring during and outside Active Time (duration with PDCCH monitoring). |  |

Other procedure details

- [QC]: Study DL WUS monitoring when multiple carriers are configured or active

- [InterDigital]: Support flexible configuration of DL-WUS monitoring allowing intra/inter-band MR and LR

- [QC]: Study DL WUS monitoring when multiple carriers are configured or active

- [Ofinno]: As part of the 6GR study on DL WUS, include multi-TRP and single cell multi-carrier as part of the study.

- [HW], [Google]: proposed to consider in 6G, how to adapt MR status/time gap to various traffic types, to achieve trade-off between traffic latency and power saving gain compared to NR Rel-19 where only a single capability of wake-up delay UE reports and gNB configures between DL WUS MO and PDCCH monitoring start point

- [HW], [WILUS]: proposed to study increasing the number of UE subgroups or UE specific indication compared to NR Rel-19 LP-WUS where up to 31 codepoints were designed

- [InterDigital]: Support activation/deactivation of DL-WUS monitoring based on at least channel measurement (e.g., entry/exit conditions) potentially with UE assistance.

- [Samsung]: RAN1 shall study whether the enable/disable of DL WUS operation should depend on RRC configuration for RRC CONNECTED state.

- [Ofinno]: RAN1 to assume that DL WUS monitoring will be supported both inside and outside the active BWP, while studying any impact to changes to BWP in 6GR compared with 5G BWP.

  - Use Case #2: Measurements by EE processing

Background of NR Rel-19 LP-WUS operation for measurements in connected state: Based on NR Rel-19 LP-WUS WID of RP-234056, in CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR.

Many companies have observed that 5G NR WUS operation in the connected state leads to compromised power savings because RRM, RLM and BFD measurements are still carried out using non-EE processing. It’s worth noting that EE processing for serving cell RRM measurement is already supported, but only in the idle/inactive state in NR Rel-19 WUS operation.

For 6GR, since EE processing uses an OFDM-based receiver and both EE and non-EE processing have the same noise figure, it’s important and advantageous to investigate RRM/RLM/BFD measurements for serving and RRM measurement for neighbouring cell by EE processing in the RRC-connected state. The study should focus on at least the benefits of power saving and the accuracy of these measurements, as proposed by [HW], [ZTE], [CATT], [vivo], [xiaomi], [Samsung], [Ericsson], [MTK], [Sony], [Lenovo], [Interdigital], [Apple], [QC], [WILUS]. Therefore, following sub use cases for measurements by EE processing can be studied:

- Sub use case#2-1: RRM measurement for serving cell

- Sub use case#2-2: RRM measurement for neighbor cell

- Sub use case#2-3: RLM/BFD measurement for serving cell

Evaluation results for power saving for measurement by EE processing

Sub use case#2-1: RRM measurement for serving cell

- [vivo] provide power saving benefits for RRM/RLM by EE processing for serving cell and observe following

  - Compared to serving cell measurement by non-EE processing, serving cell measurement by EE processing for 1 sample per measurement cycle can achieve 35.55%~56.77% power saving gain.

  - Compared to serving cell measurement by non-EE processing, serving cell measurement by EE processing for 2 samples per measurement cycle can achieve 50.99%~69.58% power saving gain.

- [MTK] provide the power breakdown analysis, reveals that when SSB measurement is offloaded to EE processing, the power consumption of the MR for SSB measurements is minimized, and the probability of the UE entering a deeper sleep state increases. The analysis implies the need for EE processing capability for synchronization and serving cell monitoring routines in order for UE to achieve lower power deep sleep states in connected mode.

- [QC] provides the evaluation results on energy saving gains using EE processing between two cases. In the first EE processing is used only to monitor DL WUS prior to a DRX on-duration; while in the second, EE processing is also used to perform measurements. It is observed that in both cases, there is some energy saving gain from introducing DL WUS and that gain grows significantly once measurements are performed in EE processing mode.

Sub use case#2-2: RRM measurement for neighbor cell

- [vivo] provide power saving benefits for RRM measurement for intra-frequency neighboring cell by EE or non-EE processing and RRM/RLM measurement for serving cell by EE processing and observe following:

  - Compared to serving cell and intra-frequency neighboring cell measurement by non-EE processing, RLM/RRM measurement for both serving and neighbor cell measured by EE processing can achieve 24.4%~32.58% power saving gain.

  - Compared to serving cell and intra-frequency neighboring cell measurement by non-EE processing, RLM/RRM measurement for serving cell only measured by EE processing, RRM measurement for neighbor cell is measured by non-EE processing can achieve 14.13%~18.87% power saving gain.

Sub use case#2-3: RLM/BFD measurement for serving cell

- [vivo] provide power saving benefits for RRM/RLM by EE processing for serving cell and observe following

  - Compared to serving cell measurement by non-EE processing, serving cell measurement by EE processing for 1 sample per measurement cycle can achieve 35.55%~56.77% power saving gain.

  - Compared to serving cell measurement by non-EE processing, serving cell measurement by EE processing for 2 samples per measurement cycle can achieve 50.99%~69.58% power saving gain.

- (merged to proposal 12)[FL1]Proposal 10: Study power saving gain of RRM measurement including both serving cell and neighboring cell in RRC connected performed by EE processing.

| Company | Y/N | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Y |  |
| Nokia | Y | As part of this study, we would like to understand what impact (if any?) the #NCs has on the EE processing state power consumption. It should also be clarified whether this is applied only for e.g. SS based measurements or also others. |
| LGE | Y | We are generally fine with the proposal. Same comments as above. Suggest to merge Proposal 10 and 12. |
| Xiaomi | Y |  |
| FW |  | OK |
| Spreadtrum | Y |  |
| WILUS | Y |  |
| TCL |  | Same comments like proposal 7 |
| OPPO | Y |  |
| Ofinno | Y | Suggest to clarify it is UE power saving gain that is being studied. |
| Vivo | Y |  |
| Lenovo | Y |  |
| DOCOMO |  | Although the proposal may be reasonable, we still think that before above proposal, it is better to discuss Propoal 12 firstly. |
| Samsung | Y |  |
| CATT |  | Why do we need to study this ? company can show the benefit if theyhave evluaton result. Don’t know the intention. |
| Qualcomm | Y |  |
| Ericsson | Y |  |
| Google | Y | Offloading RRM to EE processing during active data sessions (e.g., within C-DRX inactivity periods) is crucial for reducing power consumption in high-load scenarios. |
| MTK1 |  | FL1]Proposal 10: Study power saving gain of RRM measurement including both serving cell and neighboring cell in RRC connected performed by EE processing. |

- (merged to proposal 13)[FL1]Proposal 11: Study power saving gain of RLM/BFD measurement in RRC connected performed by EE processing.

| Company | Y/N | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Y | We are OK for this proposal. But when the power saving gain is evaluated, some assumptions should be clarified first. For example, whether the measurement samples are shared with RRM measurements or not. |
| Nokia | Y | As above, it should also be clarified whether this is applied only for e.g. SS based measurements |
| LGE | Y | We are generally fine with the proposal. Same comments as above. Suggest to merge Proposal 11 and 13. |
| Xiaomi | Y |  |
| InterDigital | Y |  |
| Spreadtrum | Y |  |
| WILUS | Y |  |
| OPPO | Y |  |
| Ofinno | Y | Suggest to clarify it is UE power saving gain that is being studied. |
| Vivo | Y |  |
| Lenovo | Y | Impact of/relevance to operation BW should be considered. |
| Samsung |  | Our concern is RLM/BFD measurement will bring more power consumption in EE processing power mode. If the impact is marginal, we are open to study. |
| CATT |  | Why do we need to study this ? |
| Qualcomm | Y |  |
| Ericsson | Y |  |
| Google | Y | Offloading RLM/BFD/RRM to EE processing during active data sessions (e.g., within C-DRX inactivity periods) is crucial for reducing power consumption in high-load scenarios. |
| MTK1 | Y | We support this proposal as it aligns with our view on exploiting EE processing for serving cell monitoring to enable lower power deep sleep states. Some refinement is suggested: [FL1]Proposal 11: Study power saving gain of offloading RLM/BFD measurement based on SSB in RRC connected performed by EE processing |

Measurement accuracy by EE processing

- [QC] observed that measurements can be performed in EE processing state without the need to inform the network or for special signals since noise figure is the same for EE and non-EE processing.

While,

- [HW] observed that the synchronization accuracy by detecting SSB are different for MR and LR due to different assumptions on RX number, it may result different coverage between LR and MR as shown in following figure.

Figure 3: The coverage performance difference between MR and LR [HW]

- [ZTE] proposed it is necessary to distinguish whether the reported measurements are obtained in EE mode or in normal mode, so as to ensure that the network can correctly interpret the results and facilitate subsequent scheduling

- [xiaomi] proposed to evaluate the accuracy of the WUR measurement may be evaluated to prevent compromising DL/UL scheduling performance.

- [Ericsson] proposed to study the feasibility and necessity of performing measurements in EE processing state of UE in Connected state.

- [vivo] proposed to study the SINR derivation using EE processing to meet RLM/BFD measurement accuracy requirement. Unlike non-EE processing, where SINR is measured in the frequency domain as a linear average of resource element power, EE processing assumes only time-domain method to reduce power consumption.

- [vivo] also provide the evaluation results for in terms of RSRP and SINR accuracy and it is observed for SINR measurement for serving cell by EE processing based on PSS/SSS sequences, where SINR is calculated by RSRP/(RSSI-RSRP), the SINR accuracy within 3dB for 1 sample @SINR=-6dB, and 2.5dB for 1 sample @SINR=-1.5dB can be achieved.

Based on above, following proposals are made

- [FL1]Proposal 12:

For RRC connected state, study serving cell RRM measurement based on 6GR measurement signal (e.g., PSS/SSS) by EE processing regarding at least the following:

- coverage  (e.g., achievable SINR/SNR) and accuracy

For RRC connected state, study neighboring cell RRM measurement based on 6GR measurement signal (e.g., PSS/SSS) by EE processing, regarding at least the following:

- Neighboring cell identification

- Coverage(e.g., achievable SINR/SNR) and accuracy

- Impact on the EE processing complexity

| Company | Y/N | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Y | Merge to proposal 10? |
| Nokia |  | We would also like to study/clarify whether this is only SS based or whether we also consider CSI-RS based measurement? |
| LGE | Y | We are generally fine with the proposal. Same comments as above. Suggest to merge Proposal 10 and 12. |
| Xiaomi |  | We generally agree with the proposal. We suggest the following modifications: - [FL1]Proposal 12: For RRC connected state, study serving cell RRM measurement based on 6GR measurement signal (e.g., PSS/SSS/LP-SS-like signal) by EE processing regarding at least the following: - coverage (e.g., achievable SINR/SNR) and accuracy For RRC connected state, study neighboring cell RRM measurement based on 6GR measurement signal (e.g., PSS/SSS/LP-SS-like signal) by EE processing, regarding at least the following: - Neighboring cell identification - Coverage(e.g., achievable SINR/SNR) and accuracy - Impact on the EE processing complexity |
| FW |  | OK |
| InterDigital |  | Y |
| Spreadtrum |  | We are fine to study serving cell and/or neighboring RRM measurement based on 6GR measurement signal (e.g., PSS/SSS) by EE processing. Entry and exit conditions for EE process mode and impact on performance of handover are also need to be study. Therefore, we suggest to modified the proposal as follow: - [FL1]Proposal 12: For RRC connected state, study serving cell RRM measurement based on 6GR measurement signal (e.g., PSS/SSS) by EE processing regarding at least the following: - coverage (e.g., achievable SINR/SNR) and accuracy - Entry and exit conditions for EE process mode For RRC connected state, study neighboring cell RRM measurement based on 6GR measurement signal (e.g., PSS/SSS) by EE processing, regarding at least the following: - Neighboring cell identification - Coverage(e.g., achievable SINR/SNR) and accuracy - Entry and exit conditions for EE process mode - Impact on the EE processing complexity - Impact on handover |
| WILUS | Y |  |
| OPPO | Y |  |
| vivo | Y |  |
| Lenovo | Y | SSB can be detected over a larger window to compensate for smaller # of RX. |
| DOCOMO | Y | Similar as Proposal 7, we also need to study the feasibility of neighbor cell measurement. We suggest the following update. - [FL1]Proposal 12: For RRC connected state, study serving cell RRM measurement based on 6GR measurement signal (e.g., PSS/SSS) by EE processing regarding at least the following: - coverage (e.g., achievable SINR/SNR) and accuracy For RRC connected state, study neighboring cell RRM measurement based on 6GR measurement signal (e.g., PSS/SSS) by EE processing, regarding at least the following: - Feasibility of neighboring cell identification - Neighboring cell identification - Coverage(e.g., achievable SINR/SNR) and accuracy - Impact on the EE processing complexity |
| Samsung |  | Agree in general. One comment is that all the example in bracket can be removed. |
| Qualcomm | Y |  |
| Ericsson | Y |  |
| mod |  | The proposal is updated to reflect companies’ comments and merged with proposal 10 - [FL1]Proposal 12 r1: For RRC connected state, study serving cell RRM measurement based on 6GR measurement signal (e.g., PSS/SSS) by EE processing regarding at least the following: - Power saving gain - coverage (e.g., achievable SINR/SNR) and accuracy For RRC connected state, study neighboring cell RRM measurement based on 6GR measurement signal (e.g., PSS/SSS) by EE processing, regarding at least the following: - Power saving gain - Neighboring cell identification - Coverage(e.g., achievable SINR/SNR) and accuracy - Impact on the EE processing complexity |
| ZTE, Sanechips | Y | OK with Proposal 12 r1: |
| Apple |  | OK in principle. I wonder what “power saving gain” refers to here. If it is the average PSG for idle/inactive UE, it needs to be evaluated together with DL WUS, not separately for RRM measurement. |
| Huawei, HiSilicon | Y | OK with the update |
| MTK1 |  | We support the study of serving cell RRM measurement. We strongly object to the study of neighboring cell identification and measurement by EE processing, as it conflicts with the low power assumption. We suggest to focus on the serving cell first. [FL1]Proposal 12: For RRC connected state, study serving cell RRM measurement based on 6GR measurement signal (e.g., PSS/SSS) by EE processing regarding at least the following: - coverage (e.g., achievable SINR/SNR) and accuracy For RRC connected state, study neighboring cell RRM measurement based on 6GR measurement signal (e.g., PSS/SSS) by EE processing, regarding at least the following: - Neighboring cell identification - Coverage(e.g., achievable SINR/SNR) and accuracy Impact on the EE processing complexity |
| Mod 2 |  | - [FL2]Proposal 12: For RRC connected state, study serving cell RRM measurement based on 6GR measurement signal (e.g., PSS/SSS) by EE processing regarding at least the following: - Power saving gain - coverage (e.g., achievable SINR/SNR) and accuracy For RRC connected state, study neighboring cell RRM measurement based on 6GR measurement signal (e.g., PSS/SSS) by EE processing, regarding at least the following: - Power saving gain - Neighboring cell identification - Coverage(e.g., achievable SINR/SNR) and accuracy - Impact on the EE processing complexity |
| vivo | Y |  |

- [FL1]Proposal 13: For RRC connected state, study RLM/BFD measurement based on 6GR measurement signal (e.g., PSS/SSS) signal by EE processing, regarding at least the following:

- Coverage(e.g., achievable SINR/SNR) and accuracy for SINR estimation by EE processing

- Impact on the EE processing complexity

| Company | Y/N | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Y | Merge to proposal 11? |
| Nokia |  | We would also like to study/clarify if the EE state can also support CSI-RS measurement? |
| LGE | Y | We are generally fine with the proposal. Same comments as above. Suggest to merge Proposal 11 and 13. |
| Xiaomi |  | We generally agree with the proposal. We suggest the following modifications: - [FL1]Proposal 13: For RRC connected state, study RLM/BFD measurement based on 6GR measurement signal (e.g., PSS/SSS/LP-SS-like signal) signal by EE processing, regarding at least the following: - Coverage(e.g., achievable SINR/SNR) and accuracy for SINR estimation by EE processing - Impact on the EE processing complexity |
| InterDigital | Y |  |
| Spreadtrum |  | We are fine to study RLM/BFD measurement based on 6GR measurement signal (e.g., PSS/SSS) signal by EE processing. Entry and exit conditions for EE process mode and impact on performance of RLM/BFD are also need to be study. Therefore, we suggest to modified the proposal as follow: - [FL1]Proposal 13: For RRC connected state, study RLM/BFD measurement based on 6GR measurement signal (e.g., PSS/SSS) signal by EE processing, regarding at least the following: - Coverage(e.g., achievable SINR/SNR) and accuracy for SINR estimation by EE processing - Entry and exit conditions for EE process mode - Impact on the EE processing complexity - Impact on performance of RLM/BFD |
| WILUS | Y |  |
| OPPO | Y |  |
| vivo | Y |  |
| Lenovo | Y | Impact of/relevance to operation BW should be considered. |
| Samsung |  | This proposal can be discussed when proposal 11 is agreed. |
| CATT |  | delay |
| mod |  | - Merged with proposal 11 - [FL1]Proposal 13r1: For RRC connected state, study RLM/BFD measurement based on 6GR measurement signal (e.g., PSS/SSS) signal by EE processing, regarding at least the following: - Power saving gain - Coverage(e.g., achievable SINR/SNR) and accuracy for SINR estimation by EE processing - Impact on the EE processing complexity |
| ZTE, Sanechips | Y |  |
| Apple |  | Should we study the feasibility first? |
| Huawei, HiSilicon | Y | OK with the update. |
| MTK1 | Y | We support this proposal. This proposal can be merged with Proposal 11. |
| Mod2 |  | - [FL2]Proposal 13: For RRC connected state, study RLM/BFD measurement based on 6GR measurement signal (e.g., PSS/SSS) signal by EE processing, regarding at least the following: - Power saving gain - Coverage(e.g., achievable SINR/SNR) and accuracy for SINR estimation by EE processing - Impact on the EE processing complexity |
| vivo | Y |  |

  - New Use Cases or functions for DL LP-WUS in connected state

As 6GR DL-WUS is expected to have larger payload size than NR Rel-19 WUS for which the payload size is up to 5 bits, following use cases or functions are proposed by companies.

      - DL LP-WUS facilitates PDCCH monitoring adaptation

Dynamic indication of UE active period

- [Nokia], [CATT], [ZTE]: consider indication via WUS of ACTIVE time duration or PDCCH monitoring window

- [ZTE], [NEC]: consider adaptation of C-DRX via WUS indication (e.g., cycle, drx-start offset, value of drx-onDurationTimer, etc.,).

Reduce blind decoding complexity

[SPD] [ZTE], [Interdigital], [LG], [Lenovo], [QC] proposed to study BD information (e.g., AL, CCE, PDCCH monitoring candidate) indication or PDCCH monitoring adaptation via WUS

      - DL LP-WUS indicating SCell waking up

[FutureWei], [SPD], [OPPO], [ZTE], [TCL] [Interdigital] proposed study the feasibility of DL WUS triggering finer Scell wake up (e.g., similar as dormancy indications) in Connected state.

- [Nokia]: WUS can be effectively utilized as a robust and power saving alternative to legacy SPS Activation/Deactivation DCIs.

- [SPD], [WILUS]: WUS can be used to transmit a small amount of essential control information and/or data

- [TCL]: Consider DL-WUS is used for on-demand transmission or clustering transmission with long periodicity or for UE-specific signal

- [ZTE]: DL LP-WUS triggering CSI measurement and report

Proponent companies are encouraged to provide more details of new functionalities.

| Company | Comments/suggestions |
| --- | --- |
| Huawei, HiSilicon | We are fine to discuss the above mentioned functionalites. Beside We think DL WUS can be used for indicating some network parameters/behavior updating, i.e., BWP, PDCCH monitoring periodicity, triggering of RS, beam updating and etc. |
| Nokia | Dynamic indication of DRX configuration via WUS (or other means), should be studied for potential performance gains and as an simpler alternative to supporting a DRXless mode of operation. One significant concern with using WUS to indicate any significant RRC config change, is the WUS’s robustness and security compared to the DCI/RRC style approach. |
| LGE | Regarding CSI measurement and report, we have similar view as ZTE proposal. Rel-19 NR LP-WUS has already specified it with C-DRX. If 6GR DL WUS operates independently of C-DRX, CSI reports on C-DRX active time no longer applies. Thus, we think it needs to be discussed. Furthermore, CSI measurement and report is not related to Scell waking up, so it should be under different bullet from “DL LP-WUS indicating Scell waking up”. |
| TCL | We are open for this proposal. Actually, DL-WUS could be used for other functionality in RRC connected state, including Scell ormancy or ON-duration parameter updating for DL/UL scheduling. |
| OPPO | Similar as for Idle mode, the list of functionalied should be given and further selected, if there are good supports. |
| Lenovo | Providing side information—such as whether PDCCH should be monitored in a slot, which search space to monitor, and which DCI format(s) and candidates to consider—can help reduce blind decoding and improve UE power efficiency. |
| DOCOMO | Benefit and feasibility of above should be firstly clarified. |
| Samsung | Low priority. New use cases can be discussed in 10.6.3. |
| Qualcomm | Generally, we are ok to consider additional functionality, but in our understanding that should take place in the inactive 10.6.3 agenda item. |
| Huawei, HiSilicon2 | Beside the aspects we mentioned in the previous round of reply, we notice that companies mentioned RRC inactive state in proposal 1. Based on this, we would like to remind the group that currently in RAN2 there are two alternatives under discussion: 1) inactive state, 2) sub-state of connected state. To avoid any missing of the study, we also suggest to study DL WUS to trigger sub-state transition for connected state. |
| MTK1 | We suggest to focus on the most essential functionalities with EE processing, including WUS monritoring and wake-up mechanism and serving cell monitoring. Additonal complexity may induce higher UE complexity/power consumption and network resource overhead. |

  - Co-existence with other features

[Apple] [Google]: Consider DL WUS and the other NR features such as C-DRX, Rel-16 DCP, PDCCH skipping, and SSSG switching together, to achieve a simplified/harmonized design and avoid duplicated functions for time domain adaptation of DL control monitoring

      - Co-existence with Network Energy Saving (NES) features

Background: In NR Rel-19, for RRC CONNECTED mode when LP-WUS is configured with Cell DTX, during Cell DTX inactive time, the UE is not expected to monitor LP-WUS both for Option 1-1 and 1-2.

[SPD], [TCL], [NEC], [InterDigital], [LG], [Sony], [Google], [WILUS] proposed to study WUS inter-action with NES features, for example

- Use DL WUS for adaptation of cell DTX/DRX

- UE WUS monitoring behavior during cell DTX/DRX active/inactive time

      - Co-existence with PDCCH skipping

Background: In NR Rel-19, for RRC CONNECTED mode LP-WUS can work with legacy PDCCH skipping mechanism without any enhancements.

[Nokia], [SPD], [ZTE], [vivo], [MTK], [LG], [NEC]: study DCI-based skip indication to resume DL WUS monitoring

      - Co-existence with SSSG switching

Background: In NR Rel-19, for RRC CONNECTED mode LP-WUS can work with legacy SSSG switching mechanism without any enhancements.

[Nokia], [SPD], [LG]: study WUS triggering/co-existing with SSSG switching (with or without additional indication of target SS)

Proponent companies are encouraged to provide more details of new functionalities.

| Company | Comments/suggestions |
| --- | --- |
| Huawei, HiSilicon | We support to disuss the co-existence of DL WUS with other features, including C-DRX, cell DTX/DRX, PDCCH skipping, SSSG switching and cross-slot scheduling. |
| Samsung | For the relationship between DL WUS and cell DTX/DRX, we are open to discuss it. Regarding the relationship between DL WUS and PDCCH skipping/SSSG switching, based on our understanding, it can be discussed in 10.6.3 that whether DL WUS can be used to indicate SSSG switching. In addition, whether to support SSSG switching or PDCCH skipping is related to 6G DL control discussion. |
| Qualcomm | In our view, DL WUS and sparse PDCCH (SSSG switching and PDCCH skipping) are complementary and work well tegether. We show the gain of combining the two in our revised contribution R1-2601537. |
| MTK1 | One intention to introduce WUS is to provide network flexibility in adjust the timing for scheduling data to UE, which can aggregate BS activity time and achieve NES. In this regard, whether/what is necessary for introducing cell DTX/DRX should be clarified without duplicated functionality and feature conflicts. |

- Appendix : Proposals from contributions

R1-2600046_Nokia

- For 6GR WUS based power saving, study attainable UE power saving in IDLE/Inactive with paging.

- For the 6GR study of UE operation with OFDM sequence-based DL WUS, assume that 6GR WUS monitoring is duty-cycled, with predefined time occasions.

- For 6GR study of DL WUS, assume that UE monitors WUS(es) in one time occasions that may contain one or more (continuous) WUS monitoring occasions one per paging cycle.

- Consider additional synchronization/reference signals prior paging occasion(s) to alleviate the impact of infrequent always-on synchronization signals.

- Feasibility and benefit of 6GR WUS to carry other type of indications, such as SI updated and ETWS/CMAS indication could be considered in 6GR WUS study.

- Feasibility and the benefit of 6GR WUS indicating the delivery of system information could be studied.

- RAN1 to investigate the benefit of using WUS operation to indicate the applied transmission approach for cell-common signaling.

From the discussions of Connected mode operation, we have the following observations and conclusions.

- From the RAN1 perspective, for the Connected RRC mode operation, a basic duty-cycled based C-DRX mode can be configured.

- For 6GR DL WUS based power saving, study attainable UE power saving in CONNECTED mode when used to trigger ACTIVE time for PDCCH monitoring.

- For 6GR DL WUS study if one or more modes of WUS operation to trigger active time are beneficial.RAN1 to study use of WUS in ACTIVE times.

- RAN1 to study use of WUS in ACTIVE times.

- RAN1 strives for a 6G suite of PDCCH monitoring adaptation features that avoids overlapping functionality.

- RAN1 studies WUS for supporting additional PDCCH monitoring adaptation features, including but not restricted to:

- Triggering ACTIVE times with additional indication eg of ACTIVE time duration

- Early termination of PDCCH skipping

- Triggering SSSG switching (with or without additional indication of target SS)

- RAN1 studies the application of WUS to other procedures, e.g. SPS.

R1-2600061_FUTUREWEI

RRC IDLE/INACTIVE Operation:

Proposal 1: Consider DL WUS of OFDM based sequence for a unified early paging indication functionality in 6G using codepoints with at least an all subgroups indication codepoint, while supporting a maximum of at least 31 subgroups per PO and association to a maximum of at least 4 POs.

RRC CONNECTED Operation:

Proposal 2: Consider how to support dormancy indication using DL WUS of OFDM based sequence with an increased wake-up indication capacity, compared to Rel-19 LP-WUS design, for a unified energy saving information notification functionality in RRC Connected state of 6G.

R1-2600126_Spreadtrum, UNISOC

Proposal 1: For 6GR, only one type of DL-WUS should be introduced in idle mode.

Proposal 2: DL WUS occasion can to be introduced and one DL WUS occasion consists of multiple DL WUS monitoring occasion.

Proposal 3: For wake up delay, NR mechanism can be a starting point.

Proposal 4: For DL WUS occasion configuration, NR mechanism can be as a starting point.

Proposal 5: UE in EE mode should be able to perform RRM measurements (at least for the serving cell) with the follow 6GR sync signal(s):

- SSB

- LP-SS (if introduced)

Proposal 6: For 6GR, only one type of DL-WUS should be introduced in connected mode.

Proposal 7: For LP-WUS operation in connected mode, Option 1-1 and Option 1-2 in NR can be used as a starting point.

R1-2600158_Huawei, HiSilicon

Proposals

- Study 6G DL WUS with finer wakeup granularity (than 5G LP-WUS) in RRC idle state.

- Study 6G DL WUS procedures in RRC connected state, taking the following into account:

- Support of larger WUS capacity, compared to 5G-A.

- Single solution to avoid duplicated design/procedure.

- Trade-off between latency and power saving gain.

- Measurements by LR, including RRM measurement.

- Study neighbour cell measurement performed by LR, considering the impact on the LR complexity to maintain acceptable measurement accuracy.

- Strive for unified synchronization signal for MR and LR, if feasible.

- If 6G DL WUS carries UE subgroup information, DL WUS triggers UE to monitor PO; if 6G DL WUS carries UE dedicated information, DL WUS triggers UE to perform RACH procedure.

- RAN1 study DL WUS in RRC connected state to trigger per UE fast transition from low-power sub-state to data-transmission sub-state.

- RAN1 study DL WUS monitoring procedure for triggering PDCCH monitoring in 6G RRC connected state with the objective of supporting a single procedure.

- In 6G, how to adapt MR status to various traffic types, and the trade-off between traffic latency and power saving gain should be considered.

- RAN1 study the possibility of using DL WUS to support functionalities other than wake-up, if beneficial. Considering the following aspects:

- Power impact both from network side and device side.

- Benefits of moving these functionalities from their legacy signal to 6G DL WUS.

- Coverage performance.

- Number of bits needed to carry the functionality.

R1-2600216_Ericsson

Proposal 1	The 6GR WUS should be supported in all RRC states (like 5G WUS in Idle/Inactive, Connected states) to provide energy saving for devices in different use cases and scenarios.

Proposal 2	Study the feasibility and necessity of including additional information in DL WUS considering the tradeoff with WUS duration and NW overhead.

Proposal 3	Consider 5G WUS monitoring procedure in Idle/Inactive state as baseline for 6G WUS monitoring.

Proposal 4	Study the impact from 6G paging design on LO-to-PO association, number of WUS UE subgroups, UE power saving gain, NW signaling overhead and related aspects.

Proposal 5	To avoid overlapping functionalities, design one single WUS tool for paging indication in Idle/Inactive state.

Proposal 6	Study the feasibility and necessity of performing neighbor cell measurement in energy efficient (EE) processing mode in Idle/Inactive state.

Proposal 7	For 6G WUS operation in Idle/Inactive states, entry/exit conditions for WUS monitoring are not needed.

Proposal 8	6G WUS operation in Connected mode depends on 6G C-DRX and related schemes design.

Proposal 9	Design only one option for 6G WUS monitoring in Connected state.

Proposal 10	Study the feasibility and necessity of performing measurements in EE processing state of UE in Connected state.

Proposal 11	To avoid overlapping functionalities, design one single WUS tool for PDCCH monitoring indication in Connected state.

R1-2600235_TCL

Proposal 1: Consider DL-WUS used for initial access or random access for waking-up UE to receive 6G synchronization signal.

Proposal 2: Consider DL-WUS used for PDCCH monitoring in RRC connected state and Paging PDCCH monitoring in RRC non-connected state.

Proposal 3: Consider DL-WUS used for on-demand transmission or clustering transmission with long periodicity or for UE-specific signal.

Proposal 4: Do not down-select DCI-WUS or SEQ-WUS in 6G SI due to the discussion/agreement lack of energy consumption model or WUR capability or UE types. Keep following potential DL-WUS schemes to further study or unify.

- DCI-WUS, e.g., DCP, PEI

- SEQ-WUS, e.g., LP-WUS, NWUS, GWUS

Proposal 5: Discuss DL-WUS types or sequence types for different RRC states or UE types, e.g.,

- RRC states：

  - RRC connected state: both DCI-WUS and SEQ-WUS, or only C-DRX configuration

  - RRC non-connected state: only SEQ-WUS

- UE types：

  - eMBB UE/Redcap: both DCI-WUS and SEQ-WUS

  - IoT: only SEQ-WUS, or only DRX configuration including e-DRX

Proposal 6: For DL-WUS scheduling or configuration, including resource location of DL-WUS and carrying messages at DL-WUS, consider following potential design as reference,

- Dedicated/broadcast RRC configuration

- Common signal(s) based scheduling with explicit or implicit way, e.g., 6G sync signal/channel, SIB X, etc.

- Dynamic scheduling, e.g., DCI indication following 2-stage DCI design

Proposal 7: Consider unify DL-WUS design if supported for diverse UE types for RRC states operation.

Proposal 8: Consider DRX based DL-WUS scheme by following items,

- RRC connected state: Only DL-WUS scheme, or only C-DRX scheme

- RRC non-connected state: I-DRX/e-DRX based DL-WUS scheme, or only DL-WUS scheme

Proposal 9: For cooperation energy savings from both BS and UE side, consider both Cell DTX/DRX and I-DRX/e-DRX in RRC non-connected to get energy efficiency gain.

Proposal 10: Discuss the impact of single/multiple carrier(s) configuration on DL-WUS design.

Proposal 11: Consider EE processing as one or multiple states for DL-WUS reception with various configurations or MR/WUR capabilities.

R1-2600313_CATT

Proposal 1: The sequence-based DL WUS shall be used for early indication of paging in 6GR, which is complementary to UE I-DRX in RRC IDLE/INACTIVE state.

Proposal 2: For the synchronization of DL WUS monitoring, a unified synchronization signal (PSS/SSS) for both EE processing and non-EE processing could be considered.

Proposal 3：Study the feasibility of offloading serving and neighbor cell measurements to the UE in EE processing mode for RRC_IDLE/INACTIVE state.

Proposal 4: Apart from paging early indication, the following functionalities of DL WUS could also be considered:

- Indicating TRS/CSI-RS availability

- Carrying Short message.

Proposal 5: If C-DRX is configured in 6GR, the sequence-based DL WUS could be designed to coordinate with C-DRX, however, the detailed design of DL WUS would be postponed until further progress is made in RAN2.

Proposal 6: The sequence-based DL WUS should be designed to support the following aspects:

- Indicating the start of the PDCCH monitoring window,

- Indicating the adjustment of PDCCH monitoring window duration.

Proposal 7: At a minimum, RRM measurements could be considered in the EE processing state for RRC_CONNECTED UEs.

R1-2600403_CMCC

Proposal 1: It is recommended that 6GR to support IDLE state DL WUS for paging indication.

Proposal 2: IoT UE should be considered for DL WUS in IDLE state.

Proposal 3: It is recommended 6GR to consider the following two sub use cases for CONNECTED DL WUS,

-  The DL WUS is used to indicate whether the UE is going to monitor the upcoming DRX ON duration (DCP-like).

-  The DL WUS is used to indication/activate PDCCH monitoring.

Proposal 4: It is recommended that 6GR to study DL WUS for considering DL small data transmission or triggering downlink small data reception in UE.

Proposal 5: Send LS to RAN2 and kindly ask RAN2 to take into consideration the above all use cases in proposal 1-4.

R1-2600443_Xiaomi

Proposals:

Proposal 1: The offloading of both serving and neighboring cell measurements from UE MR to UE WUR could be studied, and the related potential gains and loss need to be further investigation and evaluation.

R1-2600458_ZTE Corporation, Sanechips

Proposal 1: For group WUS, the following should be considered:

• Number of UEs in one group, and the number of groups.

• Position relationship between DL WUS monitoring occasions and paging occasions/ paging frames

• Wake up delay of DL WUS

• Beam relationship of DL WUS

Proposal 2: Both group WUS and UE specific WUS should be studied.

Proposal 3: For UE specific WUS, the following should be considered:

• Wake up delay of UE specific WUS

• Payload size/ carried WUS information bits

• Resource allocation

• Beam of UE specific WUS

Proposal 4: DL small data transmission in RRC inactive state triggered by UE specific WUS should be studied in 6G.

Proposal 5: If CA framework in idle/inactive state is supported, how the WUS operate with multi-carrier/cell should be studied including cross cell wake-up.

Proposal 6: For RRM measurement in idle/inactive states, the following aspects should be evaluated

•whether 6G synchronization signals (e.g., PSS/SSS) can be reused by a low-power modem

•whether LP-modem-based measurements can meet 6G performance requirements.

Proposal 7: Study the following three options for connect state:

• Scheme#1: WUS indicates whether or not activate a configured DRX ON.

• Scheme#2: WUS dynamically triggers a DRX active time.

• Scheme#3: WUS dynamic triggers PDCCH monitoring when DRX is not configured.

Proposal 8: Dynamic DRX parameters adaptation indicated by WUS can be considered.

Proposal 9: Power saving parameters other than DRX parameters can be indicated by WUS outside or inside DRX active time.

Proposal 10: CSI measurement and reporting triggered by WUS can be considered.

Proposal 11: Study efficient WUS function to trigger finer SCell wake-up and study the relationship with SCell dormancy.

Proposal 12: Study the measurement behavior of UE in connected state when WUS is enabled.

R1-2600518_vivo

Proposal 1: Study 6GR DL WUS procedure for both RRC idle and RRC connected states.

Proposal 2: Study a unified 6GR DL WUS procedure for both eMBB and IoT use cases.

Proposal 3: For RRC idle state, study DL WUS procedure for waking-up UE regarding at least the following aspects, by taking NR LP-WUS procedure as a starting point:

- WUS monitoring periodicity

- WUS monitoring occasions

- WUS indication granularity, e.g., sub-group level

- UE behaviors after waking-up, e.g., paging PDCCH monitoring

Proposal 4: Study the feasibility and benefit of FDMed LOs in addition to TDMed LOs from the perspective of network energy saving and  UE power saving in 6GR.

Proposal 5: In 6GR, study finer WUS indication granularity than R19.

- FFS the number of subgroups indicated by DL-WUS

Proposal 6: Study on-demand SSB/RS triggered by DL WUS for paging reception in 6GR.

Proposal 7: In 6GR, study the serving cell RRM measuemnt based on PSS/SSS by EE processing in RRC idle and connected states, including at least the following:

- Power saving gain for EE vs non-EE processing

- Side conditions and accuracy e.g., RSRP/RSRQ

Proposal 8: In 6GR, study neighbor cell measurement based on PSS/SSS by EE processing in RRC idle and connected states, including at least the following:

- Power saving gain for EE vs non-EE processing

- Feasibility of neighbor cell identification

- Side conditions and accuracy e.g., RSRP/RSRQ

Proposal 9: C-DRX is useful for periodic traffic and should be kept simple to maintain straightforward implementation and effectiveness for periodic traffic in 6GR.

Proposal 10: Study C-DRX cycle free DL WUS for waking-up UE in 6GR for connected mode UEs, NR Rel-19 WUS operation option 1-2 is considered as starting point, with following differences

- Separating L1/L3 measurements and reporting (including both EE and non-EE processing based) from C-DRX cycle and C-DRX active time for connected mode UEs in 6GR to maximize UE power saving while maintaining mobility robustness.

- 6GR DL WUS is used without any DRX cycle configuration

Proposal 11: In 6GR, study RLM/BFD based on PSS/SSS by EE processing, including at least the following

- Power saving gain for EE processing vs non-EE processing

- SINR estimation scheme in time domain

- Side condition and SINR accuracy for EE processing

R1-2600532_InterDigital, Inc.

Proposal 1: Support DL-WUS via low-power receiver-based operation as well as main radio-based operation at the UE from 6GR Day-1.

Proposal 2: Support flexible configuration of DL-WUS monitoring allowing intra/inter-band MR and LR.

Proposal 3: Support DL-WUS operation at least with UE DRX operation.

Proposal 4: Support fast adaptation of DL-WUS configuration based on NW energy saving operation and available network resources.

Proposal 5: Support activation/deactivation of DL-WUS monitoring based on at least channel measurement (e.g., entry/exit conditions) potentially with UE assistance.

Proposal 6: Support fallback operation (e.g., based on a timer) for DL-WUS monitoring.

Proposal 7: Energy saving with DL-WUS should be evaluated at least with different sleeping states:

- RRC CONNECTED mode: For micro sleep, light sleep and deep sleep states.

- RRC IDLE/INACTIVE modes: At least for deep sleep and ultra deep sleep states.

Proposal 8: Study DL-WUS monitoring behavior depending on different sleeping states (e.g., different activation times).

Proposal 9: Support LR based mobility and cell (re)selection including offloading neighbouring cell measurement.

Proposal 10: Support use of DL-WUS at least for controlling PDCCH monitoring behavior and monitoring complexity.

Proposal 11: Support fast carrier activation and deactivation procedures via DL-WUS.

Proposal 12: Support aperiodic UE DRX active time based on DL-WUS indication.

Proposal 13: Support use of DL-WUS at least for controlling paging monitoring.

Proposal 14: Support different wake-up procedures based on DL-WUS to support different latency requirements.

R1-2600557_LG Electronics

Proposal #1: For UEs in RRC non-connected mode, consider at least the following functionalities of DL WUS.

- Indication of UE’s paging monitoring (i.e., similar to Rel-19 LP-WUS for idle/inactive UEs)

- Adaptation indication of common signals/channels (e.g., paging, SS/PBCH, SI, PRACH)

- Short messages indication

Proposal #2: For UEs in RRC connected mode, consider DL WUS to indicate whether UE starts a PDCCH monitoring timer after a time offset, without configuring DRX (i.e., similar to Rel-19 LP-WUS option 1-2).

Proposal #3: For UEs in RRC connected mode, consider DL WUS to indicate SSSG switching and/or PDCCH skipping.

Proposal #4: For UEs in RRC connected mode, consider DL WUS to adjust parameters for PDCCH monitoring (e.g., the number of BD candidates, aggregation level, etc.).

Proposal #5: Study the harmonized design of DL WUS operation and cell DTX/DRX.

R1-2600610_Ofinno

Proposal 1: Support LP-WUS framework from 5G as baseline for DL WUS in 6GR with a focus on OFDM based sequence of the DL WUS.

Proposal 2: RAN1 to study including sending some control information via DL WUS as part of DL WUS energy efficiency studies.

Proposal 3: Support DL WUS in all RRC states.

Proposal 4: RAN1 to postpone detailed discussion of RRC state impact to DL WUS until RAN2 has made sufficient progress on RRC state modelling.

Proposal 5: RAN1 to focus DL WUS discussions on:

- Supported scenarios for DL WUS

- OFDM based sequence design of DL WUS

- Functionalities of DL WUS

Proposal 6: As part of the 6GR study on DL WUS, include multi-TRP and single cell multi-carrier as part of the study.

- FFS: any dedicated design for those scenarios. \

Proposal 7: RAN1 to assume that DL WUS monitoring will be supported both inside and outside the active BWP, while studying any impact to changes to BWP in 6GR compared with 5G BWP.

Proposal 8: RAN1 to carefully consider any RAN2 progress on DRX as part of DL WUS design.

Observation 2: DL WUS and PEI share similar goals of minimizing paging occasions the UE needs to monitor.

Proposal 9: RAN1 to study how to combine the features of PEI and DL WUS into the DL WUS design.

R1-2600669_NEC

Proposal 1: Support OFDM sequence-based DL WUS for paging monitoring indication as a UE mandatory feature, and do not support reduplicate PEI-like mechanism.

Proposal 2: Support both serving cell and neighbour cell measurement by LR in 6GR.

Proposal 3: Study synchronization signal design to facilitate neighbour cell measurement by LR, especially for the cell ID detection.

Proposal 4: Study mechanisms for separately or jointly triggering an active time of cell DTX/DRX and C-DRX by DL WUS.

Proposal 5: Support both the periodical C-DRX without DL WUS and C-DRX with DL WUS, study a unified C-DRX configuration to support the two cases.

Proposal 6: Study utilizing DL WUS for adaptation of cell DTX/DRX or C-DRX.

Proposal 7: Study joint design of PDCCH skipping and WUS, e.g., utilizing DL WUS to resume PDCCH monitoring without a predicted skipping period.

Proposal 8: Study a unified design of DL WUS for multiple use cases, e.g., define unified and scalable coding, modulation and resource mapping schemes for different use cases.

R1-2600707_Panasonic

Proposal 1: For WUS operation, it would be studied first to how to harmonize these features with DL WUS

Proposal 2: It should be studied first when/whether to monitor DL WUS and how to trigger the subsequent active period

- If the active period is similar as drxOnDuration, it would be duty-cycle operation. If the active period is not periodic, it would be non-duty cycle operation.

R1-2600770_Samsung

Proposal 1: As a starting point, the scope for Agenda 10.6.1.2 can focus on the DL WUS operation to trigger PDCCH monitoring in RRC states.

- DL WUS operation for other functionalities are not precluded and can be discussed in Agenda 10.6.3.

Proposal 2: RAN1 shall study the relationship between DL WUS and UE C-DRX (and/or Cell DTX/DRX) if supported and configured for RRC CONNECTED state.

Proposal 3: RAN1 shall study whether the enable/disable of DL WUS operation should depend on RRC configuration for RRC CONNECTED state.

Proposal 4: RAN1 shall study how to support a cell-specific transmission (in addition to UE-sub-group-based transmission) for DL WUS operation for RRC INACTIVE/RRC IDLE state.

Proposal 5: RAN1 shall study the mechanism to use DL WUS to trigger paging monitoring in the associated PO for UEs in RRC INACTIVE/RRC IDLE state.

- Whether and how to support periodic PO triggered by DL WUS and/or PO for paging adaptation triggered by DL WUS.

Proposal 6: RAN1 shall study how to reduce the resource overhead for DL WUS operation for UEs in RRC INACTIVE/RRC IDLE state.

Proposal 7: RAN1 shall study how to achieve time/frequency synchronization for reception of sequence-based DL WUS for both RRC states.

- 6GR synchronization signals (e.g., SSS) can be used as a starting point.

Proposal 8: RAN1 shall study how to support serving cell measurement and neighbor cell measurements during EE processing state for both RRC states to reduce the power consumption for RRM measurements in 6GR.

R1-2600798_Lenovo

Proposal 1: Study Neighbor cell measurement in EE processing state, at least for low tier delay tolerant devices.

Proposal 2: Study how to determine neighbor cell measurement capability and measurement time budget for an EE power state within a sleep-state.

Proposal 3: Study the feasibility of notification using EE state to minimize unnecessary PDCCH monitoring, reduce blind decoding complexity.

Proposal 4: Study Neighbor cell measurement in EE processing state, at least for low tier delay tolerant devices.

R1-2600842_Apple

Proposal 1: For 6GR idle/inactive mode, consider a unified WUS-based framework to replace NR PEI and LP-WUS.

- The framework should include the necessary support for (but not necessarily mandate) the implementation of low-power reception for LP-WUS.

Proposal 2: Consider the transmission of a reference signal for synchronization purpose before paging reception after a WUS is transmitted.

Proposal 3: Study the feasibility of performing neighbor cell measurement with low power processing mode, considering at least the following aspects:

- Intra- and inter-frequency measurement

- UE complexity

- Measurement accuracy

- As a starting point, assume neighbor cell measurement is performed based on regular SSB

Proposal 4: For 6GR connected mode, consider DL WUS and the other NR features such as C-DRX, Rel-16 DCP, PDCCH skipping, and SSSG switching together, to achieve a simplified/harmonized design and avoid duplicated functions for time domain adaptation of DL control monitoring.

Proposal 5: For 6GR connected mode, consider a unified WUS-based framework to replace NR DCP and LP-WUS.

Proposal 6: For 6GR connected mode, consider separate configuration of measurement cycle and PDCCH monitoring cycle (C-DRX periodicity or WUS monitoring periodicity).

Proposal 7: For 6GR connected mode, consider WUS during C-DRX active time together with other NR features such as cross-slot scheduling, PDCCH skipping, SSSG switching, and short C-DRX cycle.

R1-2600929_Sharp

Proposal 1: Support group-common DL WUS as the baseline for idle-mode operation.

Proposal 2: Support both CN-assigned subgrouping and UE-ID-based subgrouping for group-common DL WUS.

Proposal 3: Further study UE-specific DL WUS for idle mode, including its impact on UE complexity, system overhead, and coexistence with group-common DL WUS.

Proposal 4: Study the operation of DL WUS in relation to i-DRX for idle mode, including cooperation with i-DRX and the feasibility of partially or fully replacing i-DRX.

Proposal 5: Study a unified framework for UE PDCCH monitoring control in 6GR, including whether C-DRX or DL WUS should act as the baseline mechanism, and clarify the interaction and precedence among DL WUS, DRX, and other monitoring control features.

R1-2601016_ETRI

Proposal 1: DL WUS operation should be supported for UEs in both RRC Connected and RRC IDLE/INACTIVE states.

Proposal 2: DL WUS should be supported in the RRC Connected state to enable low-complexity wake-up indication and support the DCP functionality defined in NR Rel-16.

Proposal 3: DL WUS should be supported in the RRC IDLE/INACTIVE state to enable low-complexity wake-up indication while supporting the PEI functionality defined in NR Rel-17.

Proposal 4: DL WUS should support at least subgroup-based wake-up functionality for UEs in the RRC IDLE/INACTIVE state.

Proposal 5: Study a DL WUS–based wake-up mechanism to support cell-level DTX/DRX operation for on-demand delivery of cell-common signaling in the RRC IDLE/INACTIVE state.

Proposal 6: During DTX/DRX inactive time, the UE receives the SSB, or a part of the SSB, using a low-power receiver or an energy-efficient mode.

R1-2601141_Sony

Proposal 1 – RAN1 to consider DL-WUS solutions for both IDLE/INACTIVE and CONNECTED RRC states.

Proposal 2 – For DL-WUS operation in idle mode to allow joint UE and base-station power saving, RAN1 to consider the following:

- no additional low-power synchronization signal is needed for DL-WUS reception,

- DL-WUS is designed such that it can also be used as a synchronization signal to the subsequent channel when it is detected,

- Simple configuration for wake-up occasion and discontinuous reception of the WUR by allowing Wake-up occasion to become reference for channel monitoring.

Proposal 3 – RAN1 to consider mechanisms where the discontinuous reception operation of the wake-up monitoring occasions is adjusted according to the 6GR synchronization signal and its periodicity.

Proposal 4 – 6GR DL-WUS scheme design should strive to avoid shortcoming of 5G NR solutions in terms of maintaining synchronization and performing measurements.

Proposal 5 – RAN1 to study procedures to align the DL-WUS mechanisms including its wake-up and sleeping opportunities to the network discontinuous cell TX and RX.

Proposal 6 – RAN1 to include relative power models for different device types (e.g., IoT UE, normal UE) in the evaluation assumption or agree on the operating BW of the respective device.

R1-2601196_NTT DOCOMO, INC

Proposal 1:

- At least SSB (e.g., SSS) could be used for RRM measurement by WUR in IDLE/INACTIVE modes

Proposal 2:

- Study the need of exact WUR based measurement behaviour, e.g., neighbour cell measurement, considering the discussion in agenda item of 10.6.1.1.

Proposal 3:

- If the necessity and feasibility of neighbour cell measurement by WUR is identified, at least following issues can be studied.

  - Relaxation of MR to wake-up for neighbour cell measurement

  - Entry/Exit condition of neighbour cell measurement by WUR including the assessment of the need for Entry/Exit mechanism

Proposal 4:

- Study DL WUS in IDLE/INACTIVE states, e.g., OFDM-based DL-WUS with WUR.

Proposal 5:

- Replace PEI with DL WUS, if DL WUS has larger power saving gain and small drawbacks such as overhead and coverage.

Proposal 6:

- Study the feasibility of cell common information indication by DL WUS.

Proposal 7:

- Study DL WUS in CONNECTED state, e.g., OFDM-based DL-WUS with WUR.

- The DL-WUS design should be unified among IDLE/INACTIVE/CONNECTED states.

Proposal 8:

- Study the better integration of DL-WUS and C-DRX procedure for further UE power saving.

Proposal 9:

- Study which and how the other PDCCH monitoring adaptation methods are co-existence with DL-WUS operation.

R1-2601210_Google

Proposal 1: RAN1 study on the 6G DL-WUS assumes group-based indication as the starting point for RRC IDLE/INACTIVE state.

Proposal 2: 6G study on DL WUS design in RRC CONNECTED state should address the trade-off between latency and resource/power overhead.

Proposal 3: 6G study on DL WUS should consider supporting a dynamic indication of the time gap carried by the DL WUS.

Proposal 4: For enhancements on PDCCH monitoring adaptation, strive for a harmonized and simplified design across related features introduced in 5G (e.g., C-DRX, PDCCH skipping, SSSG switching, and LP-WUS).

Proposal 5: The 6G SI should investigate mechanisms to enable dynamic switching for a cell between “Sleep” mode and “Active” mode.

R1-2601286_Qualcomm Incorporated

Proposal 1: Study using DL WUS to indicate wake-up for an upcoming paging occasion and study details such as the gap between a WUS occasion and a paging occasion as well as UE grouping.

Proposal 2: Study DL WUS triggering wake-up for an upcoming DRX on-duration

Proposal 3: Compare the UE energy consumption of a shorter C-DRX cycle and of DL WUS triggering PDCCH monitoring outside C-DRX active time.

Proposal 4: Study DL WUS triggering PDCCH monitoring within a C-DRX active time.

Proposal 5: Study the time gap between a DL WUS and the action it triggers in CONNECTED mode for each supported scenario.

Proposal 6: Study the periodicity of DL WUS occasions in CONNECTED mode.

Proposal 7: Study DL WUS monitoring when multiple carriers are configured or active.

R1-2601421_WILUS Inc.

Proposal 1: Study Integration of DCP into DL-WUS Framework

  - Consider replacing the separate DCP (DCI format 2_6) with DL-WUS for integrated C-DRX On-duration control in 6G

  - Study incorporating UE/Group IDs in DL-WUS of OFDM based sequence to prevent unnecessary wake-ups

  - Consider a baseline operation where the C-DRX On-duration is skipped upon WUS non-detection (similar to Rel-19 LP-WUS Option 1-1) for high energy efficiency

Proposal 2: Study DL-WUS operation in Clustered Common Signal (CCS) Transmission

  - Study utilizing DL-WUS as a dynamic control signal in 6G to determine the activation status of the Clustered Common Signal (CCS) duration

  - Consider mechanisms where DL-WUS indicates the presence of paging messages for IDLE/INACTIVE UEs and the necessity of PDCCH monitoring for CONNECTED UEs, thereby preventing unnecessary monitoring overhead

  - Investigate schemes to achieve joint energy savings for both the network and the UE by allowing the UE to maintain a sleep state without monitoring the long CCS duration when no traffic is present

Proposal 3: Study and introduce C-DRX Free operation in connected mode, where PDCCH monitoring is dynamically triggered by DL-WUS.

Proposal 4: Study RRM Offloading and Measurement

  - Define the offloading of RRM, RLM, and BFD measurements to the Low-Power Receiver (LPR) as a baseline operation to maximize energy efficiency.

  - Include both serving and neighbor cell measurements across all RRC state (IDLE, INACTIVE, CONNECTED) and study the associated procedures

Proposal 5: Study LP-SS Configuration and Unified Design

  - Prioritize a Unified Design where the Low-Power Receiver (LPR) shares the reception of the existing MR SSB (PSS/SSS) to maximize resource efficiency and minimize overhead, avoiding separate LP-SS definition.

Proposal 6: Study Enhanced Payload & Functionalities in DL-WUS

  - Define DL-WUS as a unified low-power signaling delivering a small amount of essential control information to minimize unnecessary Main Radio processing and maximize system energy efficiency.

R1-2601367_NICT

Proposal-1: For WUS design, information obtainable from the UE should be examined and considered.

Proposal-2: Multiple WUS waveform profiles should be studied, and UEs should be able to indicate their supported profiles through RRC-level capability signaling, enabling the network to select the most appropriate waveform based on use-case and duty-cycle requirements.

Proposal-3: Depending on the UE’s use case, the UE should be allowed the operational flexibility to selectively refrain from monitoring WUS when appropriate. The applicability of this flexibility may require coordination with relevant RAN2 procedures (e.g., idle/inactive mode behavior and paging).