3GPP TSG-RAN WG1 Meeting #123	     R1-2509387

Dallas, USA, November 17 – 21, 2025

Agenda Item:	11.2

Source:	Moderator (Huawei)

Title:	FLS#3 on evaluation assumptions for 6GR air interface

Document for:	Discussion and Decision

# Introduction

Evaluation assumptions for 6GR air interface (in AI 11.2 from chair notes)

Discussions on models, scenarios, parameters, and methodology, metrics/criteria, as well as traffic model that can be commonly used for evaluating technology proposals.

[123-R20-6GR-Evaluation] Email discussion on Rel-20 6GR-Evaluation – Jinhuan (Huawei)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

For this meeting, the following issues are to be discussed in each of subsequent sections:

- The common evaluation assumptions for TN

  - Remaining BS antenna modelling

  - UE antenna modelling

  - SLS assumptions

    - Layout

    - BS/UE transmission power

    - UE distributions

    - Other parameters (less controversial).

- Traffic models

- Link budget template update

- General assumptions for NTN

  - Companies’ views based on the agreed template are summarized in the attachment.

  - High-level general issues are planned to be discussed in this meeting

    - Carrier frequency for Ku-band, Satellite antenna modelling, Satellite orbit type.

Kinder Note for the discussions in this summary:

- Different rounds with proposals with label [FL1, FL2, …] of discussion may be created in this summary for proposal update based on the online/offline progress. It’s companies’ discretion to provide your comments to either round but just being reminded that the discussion is supposed to carry on the proposal in the latest round in principle.

- Please use the following convention for uploading your comments:

  - Filename_v001_Moderator

  - Filename_v002_Moderator_CompanyA

  - Filename_v003_CompanyA_CompanyB

  - Filename_v004_CompanyB_CompanyC

  - Etc

# Remaining antenna modeling for TN

## BS antenna modelling

### Companies’ views

Companies’ views collected over post-122 email discussion were summarized in R1-2507292. In addition, the views included in the contribution are summarized as follows:

| Company | Views/proposals |
| --- | --- |
| Futurewei | Proposal 3: For base station antenna models, hybrid antenna architecture with a large number of antenna elements and moderate number of TXRUs, should be considered at least for: Bands at the higher end of the upper midband, e.g., ≥ 15 GHz. For around 15 GHz, at least support “Combination 3: 2048 elements, 128 TXRUs”. For around 15 GHz, further consider “Combination 4: 2048 elements, 64 TXRUs”. For around 7 GHz, further consider “Combination 6: 2048 elements, 128 TXRUs” and “Combination 7: 2048 elements, 64 TXRUs”. Carriers with very wide bandwidth, e.g., ≥ 100 MHz bandwidth. Antenna panel with a large number of antenna elements, e.g., > 1024 elements. |
| Nokia | Proposal 1: If found to be needed, RAN1 can clarify further optional antenna configurations at 700 MHz and 2GHz as follows: a. 700 MHz, Optional configuration: (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1, 2, 4), i.e., with 16 TXRUs b. 2 GHz: Outdoor, Combination 1(Optional): consider increasing the number AEs from 32 to 64, and number of TXRUs from 4 to 8, and assume (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 4). Proposal 2: For around 4 GHz carrier, we propose to consider the following BS antenna configurations as a baseline: a. For Indoors, Configuration 2: (M, N, P, Mg, Ng; Mp, Np) = (8, 8, 2, 1, 1; 2, 8), 128 AEs, 32 TXRUs. b. For Outdoors, Configuration 1: (M, N, P, Mg, Ng; Mp, Np) = (12, 8, 2, 1, 1; 4, 8), 192 AEs, 64 TXRUs. Proposal 3: For the BS antenna around 7GHz, to consider as baseline: a. For outdoor, either modified Combination 1, 768 AEs with 256 TXRUs instead of 128 with configurations (M, N, P, Mg, Ng; Mp, Np) = (24, 16, 2, 1, 1; 8, 16) or already listed Configuration 2 (M, N, P, Mg, Ng; Mp, Np) = (32, 16, 2, 1, 1; 8, 16), 1024 AEs, 256 TXRUs. b. For indoor deployments, Combination 3, (M, N, P, Mg, Ng; Mp, Np) = (16, 16, 2, 1, 1; 8, 8), 512 AEs, 128 TXRUs. Proposal 4: For the BS antenna around 15 GHz to consider: a. For indoor deployments, (M, N, P, Mg, Ng; Mp, Np) = (16, 16, 2, 1, 1; 8, 8), 512 AEs, 128 TXRUs, as baseline. b. For outdoor deployments, (M, N, P, Mg, Ng; Mp, Np) = (48, 32, 2, 1, 1; 1, 32), 3072 AEs, 64 TXRUs and (M, N, P, Mg, Ng; Mp, Np) = (48, 32, 2, 1, 1; 2, 32), 3072 AEs, 128 TXRUs. |
| ZTE | Proposed (24, 16, 2, 1, 1; 4, 16) for 7GHz outdoor combination1 and (48, 16 ,2, 1, 1; 8, 16) for combination3. (8, 4, 2, 1, 1; 2, 2) for 700MHz combination1 and (8, 2, 2, 1, 1; 1, 2) with (0.5, 0.8 0.5)λ for 2GHz outdoor combination1. Proposed antenna modelling for 15GHz: BS antenna modelling; Total number of antenna elements; Total number of TXRU; (M, N, P, Mg, Ng; Mp, Np); (dH,dV). Indoor. Combination 1; 128; 32; (8, 8, 2, 1, 1; 2, 8); (0.5, 0.5)λ. Combination 2; 512; 8; (8, 8, 2, 2, 2; 1, 1); (0.5, 0.5)λ. Outdoor. Combination 1; 2048; 16; (16, 8, 2, 4, 2; 1, 1); (0.5, 0.8)λ. Combination 2; 1536; 256; (48, 16 ,2, 1, 1; 8, 16); (0.5, 0.5)λ. |
| OPPO | Proposed (24, 16, 2, 1, 1; 8, 8) for 7GHz outdoor combination1 and (48, 16 ,2, 1, 1; 8, 16) for combination3. Proposed antenna modelling for 15GHz: BS antenna modelling; Total number of antenna elements; Total number of TXRU; (M, N, P, Mg, Ng; Mp, Np); (dH,dV). Indoor. Combination 1 (baseline); 128; 32; (8, 8, 2, 1, 1; 2, 8); (0.5, 0.5)λ. Combination 2 (optional); 256; 64; (16, 8, 2, 1, 1; 4, 8); (0.5, 0.5)λ. Outdoor. Combination 1 (baseline); 1024; 256; (32, 16, 2, 1, 1; 8, 16); (0.5, 0.5)λ. Combination 2 (optional); 2048; 256; (64, 16, 2, 1, 1; 8, 16); (0.5, 0.5)λ. Note1: A single TXRU is mapped per panel per subarray per polarization as mandatory option. Companies can provide results optionally, assuming fully connected TXRU mapping within a panel per polarization. Note2: Other combinations used in the simulation results are up to company to report.. |
| Huawei | Proposal 2: To address the TBD values - For 700MHz, (8, 4, 2, 1, 1; 1, 4) for 64 antenna elements and 8 TXRUs - For 2GHz, (8, 2, 2, 1, 1; 1, 2) for 32 antenna elements and 4 TXRUs Proposal 2: To address the TBD values for around 7GHz - Combination 1: (24,16,2,1,1; 4,16) for 768 antenna elements and 128 TXRUs - Combination 3: (48,16,2,1,1; 8,16) for 1536 antenna elements and 256 TXRUs |
| Samsung | Proposed the values to address the remaining details for antenna modelling for 700 MHz, 2 GHz, and 7 GHz. Proposed 15 GHz BS can be designed based on hybrid beamforming with 256 TXRUs and more antenna elements can be exploited to compensate more pathloss than 7 GHz BS antenna modelling; Total number of antenna elements; Total number of TXRU; (M, N, P, Mg, Ng; Mp, Np); (dH,dV). Indoor. Combination 1; 256; 256; (8, 16, 2, 1, 1; 8, 16); (0.5, 0.5)λ. Outdoor. Combination 1; 2048; 256; (32, 32, 2, 1, 1; 8, 16); (0.5, 0.8)λ. Combination 2; 2304; 256; (72, 16, 2, 1, 1; 8, 16); (0.5, 0.8)λ. |
| Intel | Proposal 1 - BS antenna configuration detailed assumptions for missing cases are amended as follows: - For 700 MHz Combination 2, antenna port mapping is (8, 4, 2, 1, 1; 1, 4) - For 2 GHz, Outdoor Combination 1, antenna port mapping is (8, 2, 2, 1, 1; 1, 2) - For 7 GHz, Outdoor Combination 1, antenna port mapping is or (24, 16, 2, 1, 1; 4, 16) - For 7 GHz, Outdoor Combination 3, antenna port mapping is (24, 32, 2, 1, 1; 4, 32) or (48, 16, 2, 1, 1; 8, 16) Proposal 2 - BS antenna configuration detailed assumptions for 15 GHz are as based on the offline proposal from RAN1#122bis, and are the following: BS antenna modelling; Total number of antenna elements; Total number of TXRU; (M, N, P, Mg, Ng; Mp, Np); (dH, dV). 15 GHz Indoor. Combination 1; 128; 32; (8, 8, 2, 1, 1; 2, 8); (0.5, 0.5) λ. Combination 2; 256; 256; (8, 16, 2, 1, 1; 8, 16); (0.5, 0.5) λ. Combination 3; 512; 256; (16, 16, 2, 1, 1; 8, 16); (0.5, 0.5) λ. 15 GHz Outdoor. Combination 1; 1536; 256; (24, 32, 2, 1, 1; 4, 32); (0.5, 0.8) λ. Combination 2; 2048; 256; (32, 32, 2, 1, 1, 4, 32); (0.5, 0.5) λ. Combination 3; 2048; 512; (32, 32, 2, 1, 1; 8, 32); (0.5, 0.5) λ. |
| Ericsson | • For Base station antenna modelling o For around 2GHz, use (M, N, P, Mg , Ng; Mp, Np) to be (4,4,2,1,1;1,4) for Outdoor Combination 1. For around 15Ghz, use the values in below Table BS antenna modelling (around 15GHz); Total number of antenna elements; Total number of TXRU; (M, N, P, Mg, Ng; Mp, Np); (dH,dV). Indoor. Combination 1; 128; 64; (8, 8, 2, 1, 1; 4, 8); (0.5, 0.5)λ. Combination 2; 128; 8; (4, 4, 2, 2, 2; 1, 1); (0.5, 0.5)λ. Outdoor. Combination 1; 2048; 256; (64, 16, 2, 1, 1; 8, 16) or (32, 32, 2, 1, 1; 4, 32); (0.5, 0.5)λ. Combination 2; 2048; 16; (16, 8, 2, 2, 4; 1, 1); (0.5, 0.5)λ. Note1: For comb. 1 a single TXRU is mapped per panel per subarray per polarization. For comb.2 a single TXRU is mapped per panel per polarization. Note2: Other combinations used in the simulation results are up to company to report.. |
| Qualcomm | Proposed the antenna modelling for 15GHz as follows: BS antenna modelling; Total number of antenna elements; Total number of TXRU; (M, N, P, Mg, Ng; Mp, Np); (dH,dV). Indoor. Combination 1; 128; 8; (8, 8, 2, 1, 1; 1, 1); (0.5, 0.5)λ. Combination 2; 512; 32; (8, 8, 2, 2, 2; 1, 1); (0.5, 0.5)λ. Outdoor. Combination 1; 1024; 128; (32, 16, 2, 1, 1; 8, 8); (0.5, 0.5)λ. Combination 2; 2048; 256; (16, 8, 2, 4, 2; 4, 4); (0.5, 0.5)λ. |
| DOCOMO | For the BS antenna configuration for around 15 GHz BS antenna modelling; Total number of antenna elements; Total number of TXRU; (M, N, P, Mg, Ng; Mp, Np); (dH,dV). Indoor. Combination 1 (for HBF); 128; 64; (8, 8, 2, 1, 1; 4, 8); (0.5, 0.5)λ. Combination 2 (for ABF); 128; 8; (4, 4, 2, 2, 2; 1, 1); (0.5, 0.5)λ. Outdoor. Combination 1 (for HBF); 2048; 256; (32, 32, 2, 1, 1; 8, 16) or (64, 16, 2, 1, 1; 8, 16) or (32, 32, 2, 1, 1; 4, 32); (0.5, 0.5)λ or (0.5, 0.8)λ. Combination 2 (for ABF); 2048; 16; (16, 8, 2, 4, 2; 1, 1); (0.5, 0.5)λ. Note1: A single TXRU is mapped per panel per subarray per polarization for combination 1. A single TXRU is mapped per panel per polarization for combination2. Note2: Other combinations used in the simulation results are up to company to report.. |

### Discussions

Summary on the views:

700MHz carrier frequency:

| For around 700 MHz carrier frequency, for BS antenna modelling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg, Ng; Mp, Np) | (dH,dV) | Mentioned by |
| Combination 2(Optional) | 64 | 8 | (8, 4, 2, 1, 1; 2, 2) | (0.5, 0.5)λ | ZTE |
| Combination 2(Optional) | 64 | 8 | (8, 4, 2, 1, 1; 1, 4) | (0.5, 0.5)λ | Huawei, Samsung, Intel |
| Combination 2(Optional) | 64 | 16 | (8, 4, 2, 1, 1; 2, 4) | (0.5, 0.5)λ | Nokia |

2GHz carrier frequency:

| For around 2GHz carrier frequency, for BS antenna modelling |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Indoor/Outdoor | BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg, Ng; Mp, Np) | (dH,dV) | Mentioned by |
| Outdoor | Combination 1(Optional) | 32 | 4 | (8, 2, 2, 1, 1; 1, 2) | (0.5, 0.5)λ | ZTE |
| Outdoor | Combination 1(Optional) | 32 | 4 | (8, 2, 2, 1, 1; 1, 2) | (0.5, 0.8)λ | Huawei, Samsung, Intel |
| Outdoor | Combination 1(Optional) | 32 | 8 | (4, 4, 2, 1, 1; 1, 4) | (0.5, 0.8)λ | Ericsson |
| Outdoor | Combination 1(Optional) | 64 | 8 | (8, 4, 2, 1, 1; 1, 4) | (0.5, 0.8)λ | Nokia |

4GHz carrier frequency:

| For around 4GHz carrier frequency, for BS antenna modelling |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Indoor/Outdoor | BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg, Ng; Mp, Np) | (dH,dV) | Mentioned by |
| Indoor | Combination 2(baseline) | 128 | 32 | (8, 8, 2, 1, 1; 2, 8) | (0.5, 0.5)λ | Nokia |
| Outdoor | Combination 1(baseline) | 192 | 64 | (12, 8, 2, 1, 1; 4, 8) | (0.5, 0.8)λ | Nokia |
|  | New Combination (baseline) | 256 | 128 |  |  | NEC |

7GHz carrier frequency:

| For around 7GHz carrier frequency, for BS antenna modelling |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Indoor/Outdoor | BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg, Ng; Mp, Np) | (dH,dV) | Mentioned by |
| Indoor | Combination 3(baseline) | 512 | 128 | (16, 16 ,2, 1, 1; 8, 8) | (0.5, 0.5)λ | Nokia |
| Outdoor | Combination 1 | 768 | 128 | (24, 16, 2, 1, 1; 4, 16) | (0.5, 0.8)λ | ZTE, Huawei, CMCC, Samsung, Intel |
| Outdoor | Combination 1(baseline) | 768 | 256 | (24, 16, 2, 1, 1; 8, 16) | (0.5, 0.8)λ | Nokia |
| Outdoor | Combination 1 | 768 | 128 | (24, 16, 2, 1, 1; 8, 8) | (0.5, 0.8)λ | OPPO |
| Outdoor | Combination 2(baseline) | 1024 | 256 | (32, 16, 2, 1, 1; 8, 16) | (0.5, 0.8)λ | Nokia, NEC |
| Outdoor | Combination 2 | 1024 | 256 | (32, 16, 2, 1, 1; 8, 16) | (0.5, 0.5)λ | OPPO |
| Outdoor | Combination 3 | 1536 | 256 | (48, 16 ,2, 1, 1; 8, 16) | (0.5, 0.8)λ | ZTE, Huawei, Samsung, Intel |
| Outdoor | Combination 3 | 1536 | 256 | (48, 16 ,2, 1, 1; 8, 16) | (0.5, 0.5)λ | OPPO |
| Outdoor | Combination 3 | 1536 | 256 | (24, 32, 2, 1, 1; 4, 32) | (0.5, 0.8)λ | Intel |
| Outdoor | New Combination | 2048 | 64 |  |  | Futurewei |
| Outdoor | New Combination | 2048 | 128 |  |  | Futurewei |

15GHz carrier frequency:

| For around 15GHz carrier frequency, for BS antenna modelling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Indoor/Outdoor | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg, Ng; Mp, Np) | (dH,dV) | Mentioned by |
| Indoor | 128 | 8 | (4, 4, 2, 2, 2; 1, 1) | (0.5, 0.5)λ | Ericsson, DOCOMO |
| Indoor | 128 | 8 | (8, 8, 2, 1, 1; 1, 1) | (0.5, 0.5)λ | Qualcomm |
| Indoor | 128 | 32 | (8, 8, 2, 1, 1; 2, 8) | (0.5, 0.5)λ | ZTE, OPPO, Intel |
| Indoor | 128 | 64 | (8, 8, 2, 1, 1; 4, 8) | (0.5, 0.5)λ | Ericsson, DOCOMO |
| Indoor | 256 | 64 | (16, 8, 2, 1, 1; 4, 8) | (0.5, 0.5)λ | OPPO |
| Indoor | 256 | 256 | (8, 16, 2, 1, 1; 8, 16) | (0.5, 0.5)λ | Samsung, Intel |
| Indoor | 512 | 8 | (8, 8, 2, 2, 2; 1, 1) | (0.5, 0.5)λ | ZTE |
| Indoor | 512 | 32 | (8, 8, 2, 2, 2; 1, 1) | (0.5, 0.5)λ | Qualcomm |
| Indoor | 512 | 128 | (16, 16, 2, 1, 1; 8, 8) | (0.5, 0.5)λ | Nokia |
| Indoor | 512 | 256 | (16, 16, 2, 1, 1; 8, 16) | (0.5, 0.5)λ | Intel |
| Outdoor | 1024 | 128 | (32, 16, 2, 1, 1; 8, 8) | (0.5, 0.5)λ | Qualcomm |
| Outdoor | 1024 | 256 | (32, 16, 2, 1, 1; 8, 16) | (0.5, 0.5)λ | OPPO |
| Outdoor | 1536 | 256 | (48, 16 ,2, 1, 1; 8, 16) | (0.5, 0.5)λ | ZTE |
| Outdoor | 1536 | 256 | (24, 32, 2, 1, 1; 4, 32) | (0.5, 0.8)λ | Intel |
| Outdoor | 2048 | 16 | (16, 8, 2, 4, 2; 1, 1) | (0.5, 0.5)λ | DOCOMO |
| Outdoor | 2048 | 16 | (16, 8, 2, 4, 2; 1, 1) | (0.5, 0.8)λ | ZTE |
| Outdoor | 2048 | 16 | (16, 8, 2, 2, 4; 1, 1) | (0.5, 0.5)λ | Ericsson |
| Outdoor | 2048 | 64 |  |  | Futurewei |
| Outdoor | 2048 | 128 |  |  | Futurewei |
| Outdoor | 2048 | 256 | (64, 16, 2, 1, 1; 8, 16) | (0.5, 0.8)λ | DOCOMO |
| Outdoor | 2048 | 256 | (64, 16, 2, 1, 1; 8, 16) | (0.5, 0.5)λ | OPPO, Ericsson, DOCOMO |
| Outdoor | 2048 | 256 | (32, 32, 2, 1, 1; 8, 16) | (0.5, 0.5)λ | DOCOMO |
| Outdoor | 2048 | 256 | (32, 32, 2, 1, 1; 8, 16) | (0.5, 0.8)λ | Samsung, DOCOMO |
| Outdoor | 2048 | 256 | (32, 32, 2, 1, 1, 4, 32) | (0.5, 0.8)λ | DOCOMO |
| Outdoor | 2048 | 256 | (32, 32, 2, 1, 1, 4, 32) | (0.5, 0.5)λ | Intel, Ericsson, DOCOMO |
| Outdoor | 2048 | 256 | (16, 8, 2, 4, 2; 4, 4) | (0.5, 0.5)λ | Qualcomm |
| Outdoor | 2048 | 512 | (32, 32, 2, 1, 1; 8, 32) | (0.5, 0.5)λ | Intel |
| Outdoor | 2304 | 256 | (72, 16, 2, 1, 1; 8, 16) | (0.5, 0.8)λ | Samsung |
| Outdoor | 3072 | 64 | (48, 32, 2, 1, 1; 1, 32) | (0.5, 0.8)λ | Nokia |
| Outdoor | 3072 | 128 | (48, 32, 2, 1, 1; 2, 32) | (0.5, 0.8)λ | Nokia |

Round-1 discussions:

(FL1) Proposal 2.1.2-1

Updating the BS antenna modelling agreed in the last meeting as follows:

- For around 700MHz carrier frequency, for BS antenna modelling,

- update the (8, 4, 2, 1, 1; x, y) to be (8, 4, 2, 1, 1; 1, 4).

- For around 2GHz carrier frequency, for BS antenna modelling,

- for outdoor combination 1 (i.e., 32AE/4TXRU), update the (M,N,P,Mg,Ng; Mp,Np) to be (8, 2, 2, 1, 1; 1, 2).

- For around 7GHz carrier frequency, for BS antenna modelling,

- for outdoor combination 1 (i.e., 768AE/128TXRU), update the (M,N,P,Mg,Ng; Mp,Np) to be (24, 16, 2, 1, 1; 4, 16).

- for outdoor combination 3 (i.e., 1536AE/256TXRU), update the (M,N,P,Mg,Ng; Mp,Np) to be (48, 16 ,2, 1, 1; 8, 16).

Any comments/suggestions, please leave them here.

| Company | Comments |
| --- | --- |
| ZTE | Thanks a lot for the great summary. We have the following comments: - For 2GHz, we suggest changing the vertical antenna spacing of Combination 1 from 0.8λ to 0.5λ, which is aligned with Combination 2 that was agreed as baseline in last meeting. - We agree with antenna config. @7GHz. |
| Samsung | We can support the proposed updates of antenna modelling from FL for 700MHz, 2GHz and 7 GHz. |

(FL1) Proposal 2.1.2-2

For 6GR evaluations, RAN1 to consider BS antenna modelling for around 15GHz carrier frequency as follows:

| BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg, Ng; Mp, Np) | (dH,dV) |
| --- | --- | --- | --- | --- |
| Indoor |  |  |  |  |
| Combination 1 | 128 | 64 | (8, 8, 2, 1, 1; 4, 8) | (0.5, 0.5)λ |
| Combination 2 | 128 | 8 | (4, 4, 2, 2, 2; 1, 1) | (0.5, 0.5)λ |
| Outdoor |  |  |  |  |
| Combination 1 | 2048 | 256 | (64, 16, 2, 1, 1; 8, 16) | (0.5, 0.5)λ |
| Combination 2 | 2048 | 16 | (16, 8, 2, 4, 2; 1, 1) | (0.5, 0.5)λ |
| Note1: A single TXRU is mapped per panel per subarray per polarization for combination 1. A single TXRU is mapped per panel per polarization for combination2. Note2: Other combinations used in the simulation results are up to company to report. |  |  |  |  |

Any comments/suggestions, please leave them here.

| Company | Comments |
| --- | --- |
| ZTE | We agree with this proposal. |
| NTT DOCOMO | Based on the discussion on SCS with 15 GHz in frame structure agenda (AI-11.3.2), it seems that 30/60kHz is majority view. Here, we think that to effectively and fully utilize ABF, at least SCS of 120kHz would be necessary. This is because smaller SCS would lead to larger symbol duration, then the number of times the analog beam can be swept within a given time interval decreases, as a result, more time to sweep the analog beam would be required and flexibility in SSB beam placement decreases. Therefore, in our view, if the SCS of 15GHz is agreed upon as 30kHz, we believe antenna configuration with ABF might not be beneficial for actual deployment. Considering that, we propose to change our view to make HBF baseline and ABF optional respectively. To be specific, we propose to delete/deprioritize combination 1 for both indoor and outdoor. |
| Samsung | For indoor scenarios, one-to-one mapping between AEs and ports is practical. And comparing with antenna configurations for 7GHz, more TXRU might be required due to more pathloss. Therefore, for evaluation purpose, we propose to below antenna configuration instead of Combination 1: Combination 1; 256; 256; (8, 16, 2, 1, 1; 8, 16); (0.5, 0.5)λ. For outdoor, in our view, hybrid beamforming based BS antenna configuration could be beneficial because more flexible precoding in frequency domain can be supported with 256 TXRUs. Therefore, we prefer to support hybrid-beamforming based antenna configuration. However, for the sake of progress, we could be fine with one updated Combination 1 for hybrid beamforming based BS and the other configuration for analog beamforming based BS as below: Combination 1; 2048; 256; (32, 32, 2, 1, 1; 8, 16); (0.5, 0.8)λ. Combination 2; 2048; 16; (16, 8, 2, 4, 2; 1, 1); (0.5, 0.5)λ. |

Round-2 discussions:

(FL2) Proposal 2.1.2-1

Updating the BS antenna modelling agreed in the last meeting as follows:

- For around 700MHz carrier frequency, for BS antenna modelling,

- update the (8, 4, 2, 1, 1; x, y) to be (8, 4, 2, 1, 1; 1, 4).

- For around 2GHz carrier frequency, for BS antenna modelling,

- for outdoor combination 1 (i.e., 32AE/4TXRU), update the (M,N,P,Mg,Ng; Mp,Np) to be (8, 2, 2, 1, 1; 1, 2)

- For around 7GHz carrier frequency, for BS antenna modelling,

- for outdoor combination 1 (i.e., 768AE/128TXRU), update the (M,N,P,Mg,Ng; Mp,Np) to be (24, 16, 2, 1, 1; 4, 16).

- for outdoor combination 3 (i.e., 1536AE/256TXRU), update the (M,N,P,Mg,Ng; Mp,Np) to be (48, 16 ,2, 1, 1; 8, 16).

ZTE’s change was not agreed by others, so the proposal is the same as in Round-1.

Any strong concerns, please leave them here.

| Company | Comments |
| --- | --- |

(FL2) Proposal 2.1.2-2rv1

For 6GR evaluations, RAN1 to consider BS antenna modelling for around 15GHz carrier frequency as follows:

| BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg, Ng; Mp, Np) | (dH,dV) |
| --- | --- | --- | --- | --- |
| Indoor |  |  |  |  |
| Combination 1 | 128 | 64 | (8, 8, 2, 1, 1; 4, 8) | (0.5, 0.5)λ |
| Combination 2 | 128 | 8 | (4, 4, 2, 2, 2; 1, 1) | (0.5, 0.5)λ |
| Combination 3 | 512 | 128 | (16, 16, 2, 1, 1; 8, 8) | (0.5, 0.5)λ |
| Outdoor |  |  |  |  |
| Combination 1 | 1024 | 64 | (32, 16, 2, 1, 1, 4, 8) | (0.5, 0.5)λ |
| Combination 2 | 2048 | 256 | (32, 32, 2, 1, 1; 4, 32) | (0.5, 0.5)λ |
| Combination 3 | 2048 | 16 | (16, 8, 2, 4, 2; 1, 1) | (0.5, 0.5)λ |
| Note1: A single TXRU is mapped per panel per subarray per polarization for combination 1. A single TXRU is mapped per panel per polarization for combination2. Note2: Other combinations used in the simulation results are up to company to report. |  |  |  |  |

Any strong concerns, please leave them here.

| Company | Comments |
| --- | --- |
| Nokia | Outdoor combination 1 seems to be an outlier for in comparison with 7GHz, where the minimum number of AEs is 768. We propose to consider an option with 3072 AEs and 128 TXRUs: (48, 32, 2, 1, 1; 2, 32) that scales directly from 7GHz configuration and also has more AEs in horizontal directions, i.e., corresponding to the around 30 cm width instead of 15 cm for Combination 1. Could proponents of ABF, please clarify if coherent RF combining across panels is assumed? |
| Interdigital | The antenna location index for Alt 2 and the note that state antenna location for Alt 2 is left up to companies to report seems contradictory. If we are going to leave antenna locations for Alt 2 open for companies to report, then there should not be any location index information for Alt 2. Our preference would be specify the locations such that companies can bring aligned results. With that said, the more important thing would be align the description of the note and the entries in the table. |

#### Agreement

Agreement

Updating the BS antenna modelling agreed in the last meeting as follows:

- For around 700MHz carrier frequency, for BS antenna modelling,

- update the (8, 4, 2, 1, 1; x, y) to be (8, 4, 2, 1, 1; 1, 4).

- For around 2GHz carrier frequency, for BS antenna modelling,

- for outdoor combination 1 (i.e., 32AE/4TXRU), update the (M,N,P,Mg,Ng; Mp,Np) to be (8, 2, 2, 1, 1; 1, 2)

- For around 7GHz carrier frequency, for BS antenna modelling,

- for outdoor combination 1 (i.e., 768AE/128TXRU), update the (M,N,P,Mg,Ng; Mp,Np) to be (24, 16, 2, 1, 1; 4, 16).

- for outdoor combination 3 (i.e., 1536AE/256TXRU), update the (M,N,P,Mg,Ng; Mp,Np) to be (48, 16 ,2, 1, 1; 8, 16).

Round-3 discussions:

#### (FL3) Proposal 2.1.2-2rv3

For 6GR evaluations, RAN1 to consider BS antenna modelling for around 15GHz carrier frequency as follows:

| BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg, Ng; Mp, Np) | (dH,dV) |
| --- | --- | --- | --- | --- |
| Indoor |  |  |  |  |
| Combination 1 | 512 | 128 | (16, 16, 2, 1, 1; 8, 8) | (0.5, 0.5)λ |
| Combination 2 | 128 | 8 | (4, 4, 2, 2, 2; 1, 1) | (0.5, 0.5)λ |
| Outdoor |  |  |  |  |
| Combination 1 | 2048 | 256 | (32, 32, 2, 1, 1; 4, 32) | (0.5, 0.5)λ |
| Combination 1bis | 2048 | 128 | () | (0.5, 0.5)λ |
| Combination 2 | 2048 | 32 | (16, 8, 2, 4, 4; 1, 1) | (0.5, 0.5)λ |
| Note1: A single TXRU is mapped per panel per subarray per polarization for combination 1. A single TXRU is mapped per panel per polarization for combination2. Note2: Other combinations used in the simulation results are up to company to report. |  |  |  |  |

## discussion points:##

- DOCOMO has concern for this note1 (Note1 was suggested by Futurewei or Nokia??) and prefer the following version.

  - Note1bis: A single TXRU is mapped per panel per subarray per polarization as mandatory option. Companies can provide results optionally, assuming a single TXRU is mapped per panel per polarization.

- Futurewei has strong opinion to keep 128TxRU for outdoor. Need the numbers for (M, N, P, Mg, Ng; Mp, Np) but no input at all for this combination before.

- Is (16, 8, 2, 4, 4; 1, 1) for 2048/32?

- Please the proponent and the opponent talk to each other and make a concrete and compromised proposal to me.

Any strong concerns, please leave them here.

| Company | Comments |
| --- | --- |

## UE antenna modelling

### Companies’ views

Companies’ views collected over post-122 email discussion were summarized in R1-2507292. In addition, the views included in the contribution are summarized as follows:

| Company | Views/proposals |
| --- | --- |
| Nokia | Proposal 7: RAN1 should use new directive handheld UE antenna model described in section 7.3 of Rel-19 version of TR 38.901 by default. Proposal 8: For handheld UE antenna modelling, RAN1 to select the following default configurations frequency: a. For around 700 MHz: 1/2T2R directional antennas, (4, 8) as described in section 7.3 in TR 38.901, b. For around 2 GHz, around 4GHz: 2T4R directional antennas, (1, 3, 5, 7) as described in section 7.3 in TR38.901, c. For around 7 GHz and around 15 GHz: 4T4/8R directional antennas, (1, 3, 5, 7) for 4 and (1,2,3,4,5,6,7,8) for 8 as described in section 7.3 in TR38.901. Proposal 10: For CPE antenna modelling, RAN1 to use CPE antenna placement from section 7.3 in TR 38.901: a. 2/4T4R with antenna locations (1, 3, 5, 7) b. 6/8T6/8R with antenna locations (1, 2, 6, 7, 8, 9) for 6 and (1,2,3,4,5,6,7,8) for 8. Proposal 11: For CPE antenna modelling, RAN1 to select CPE antenna configurations per frequency and device type (outdoor/indoor CPE) as described in Table 3 above. |
| ZTE | Proposed Handheld UT antenna model for different frequency bands and CPE antenna model for different frequency bands |
| CATT | Proposal 3: For the study on evaluation assumption in 6GR, For the UE antenna elements, the detailed designs are illustrated as follows: - Around 4 GHz - UE: Up to 8Tx/Rx ([4]Tx/Rx for handheld device); - Around 7 GHz - UE: Up to 8Tx/Rx ([4]Tx/Rx for handheld device). |
| InterDigital | Proposed the general antenna assumptions for UT for different carrier frequencies, i.e., Handheld UT model except for 30GHz which uses Model-1. |
| Xiaomi | Proposal 2: For handheld UEs, consider 1T2R at 700MHz or 2GHz, and 2T4R (as baseline) or even 3T6R (as optional) at 4GHz or 7GHz. Proposal 3: For IoT UEs, consider 1T1R at 700MHz or 2GHz, and 1T2R for 4GHz. Proposal 4: Consider the legacy UE antenna model, represented by a 5-element tuple (M, N, P, Mg, Ng), in the evaluation of 6GR. |
| OPPO | Proposal 2: For antenna modeling, consider different antenna configurations for different device types, and reuse the antenna modeling method in 38.901 (Rel-19). Proposal 3: For antenna radiation power pattern, the pattern in 38.901(Rel-19) can be baseline, and further consider practically measured antenna radiation pattern if needed. |
| Huawei | Proposal 4: For 6GR evaluation, the maximum number of UE antenna elements should at least consider 16. Proposal 5: Regarding the UE antenna assumptions considered for 6GR evaluations, no need to mention the device types. Table 2 can be a starting point for the discussions. |
| Samsung | Proposal #5: - For handheld antenna configuration, 1Tx should be a baseline assumption. - Considering practical device and need to wait for RAN Plenary’s decision on UE device types and the maximum number of UE antenna elements, support the UE antenna configurations in Table 2.1-6 for each frequency band as baseline. |
| Intel | Proposal 3 •Adopt Release-19 UE handheld antenna model as a baseline model for all TN scenarios. |
| Google | Proposal 5: Prioritize a single-panel UE configuration for evaluations in FR2/FR3. Proposal 6: Support the following for the 6GR evaluations for UL antenna configurations: - 2Tx/4Rx for > 1GHz and TDD band and 1Tx/2Rx for <= 1GHz - 1Tx/1Rx for wearables - 2Tx/4Rx for NTN bands - 4Tx/6Rx/8Rx are optional. |
| Sony | Proposal 5: UE antenna configuration should be defined depending on centre frequency and device types. - Around 700 MHz - IoT: 1TX/1RX - Smartphone/XR device: 1TX/2RX - FWA/CPE: 4TX/4RX - Around 2 GHz, 4 GHz, and 7 GHz - IoT: 1TX/1RX - Smartphone/XR device: 2TX/4RX - FWA/CPE: 4TX/4RX as baseline and 8TX/8RX as optional |
| Ericsson | • For UE antenna modelling, o Evaluations should consider realistic UE antenna modelling (e.g., non-uniform antenna positioning, use of directional antennas, power imbalance / insertion loss among antennas, and limited control of relative phase). UE/CPE models in TR38.901 can be considered as the starting point with further elaboration when needed. |
| MediaTek | UE Antenna configurations - Around 700 MHz for TN: - Option 1: 1 Tx and 1 Rx - Option 2: 1 Tx and 2 Rx - Option 3: 2 Tx and 4 Rx - Around 4 GHz for TN: - Option 1: 2 Tx and 4 Rx - Option 2: 4 Tx and 8 Rx - Around 7 GHz for TN: - Option 1: 2 Tx and 4 Rx - Option 2: 4 Tx and 8 Rx UE Antenna element gain pattern - for TN: Omnidirectional as baseline and directional based on Table 7.3-2 from TR 38.901 as optional UE Polarized antenna modelling - for TN: Linear polarization based on Model-2 of TR 38.901 section 7.3.2 |
| Qualcomm | Proposal 3: The Rel-19 defined handheld UT and CPE antenna model should be used as the default in system-level simulations. Legacy omnidirectional antenna could be used for link level simulation. Proposal 4: Study extension of CPE antenna model for larger array and more than 8 antennas. Proposal 5: Consider UE antenna configuration in Table 3 and Table 4 for 6GR evaluation. |

### Discussions

Summary on the views:

Configurations:

| Alt1: (M, N, P, Mg, Ng; Mp, Np) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Number of RXU | RXU | Number of TXU | TXU | Mentioned by |
| General | 2 | (1, 2, 1, 1, 1; 1, 2) | 1 | (1, 1, 1, 1, 1; 1, 1) | Huawei |
|  |  | (1, 1, 2, 1, 1; 1, 1) | / |  | Samsung |
|  | 4 | (1, 2, 2, 1, 1; 1, 2) | 2 | (1, 2, 1, 1, 1; 1, 2) | Huawei |
|  |  |  | 4 | (1, 2, 2, 1, 1; 1, 2) | Huawei, Apple |
|  |  |  | / |  | CMCC, Docomo, ZTE |
|  |  | (2, 2, 1, 1, 1, 2, 2) | 2 | (1, 2, 1, 1, 1, 1, 2) | OPPO, ETRI, vivo |
|  | 8 | (1, 4, 2, 1, 1; 1, 4) | 4 | (1, 2, 2, 1, 1; 1, 2) | Huawei |
|  |  |  | 4 | (1, 4, 1, 1, 1; 1, 4) | Huawei |
|  |  |  | 8 | (1, 4, 2, 1, 1; 1, 4) | Huawei |
|  |  | (2, 4, 1, 1, 1, 2, 4) | 4 | (2, 2, 1, 1, 1, 2, 2) | OPPO |
|  |  |  | 8 | (2, 4, 1, 1, 1; 2, 4) | Huawei |
|  |  | (2, 2, 2, 1, 1, 2, 2) | / |  | ZTE |
|  | 16 | (2, 4, 2, 1, 1; 2, 4) | 8 | (1, 4, 2, 1, 1; 1, 4) | Huawei |
|  |  |  | 8 | (2, 4, 1, 1, 1; 2, 4) | Huawei |
|  |  |  | 16 | (2, 4, 2, 1, 1; 2, 4) | Huawei |
|  |  | (4, 2, 2, 1, 1; 4, 2) | / |  | ZTE |
|  | 32 | (4, 4, 2, 1, 1; 4, 4) | 32 | (4, 4, 2, 1, 1; 4, 4) | Huawei |
| FR2 | 2 | (4, 1, 2, 1, 1, 1, 1) | 2 | (4, 1, 2, 1, 1, 1, 1) | Samsung |
|  | 4 | (2, 4, 2, 1, 2; 1, 1), (dg,H,dg,V) = (0, 0)λ |  |  | CMCC |
|  | 4 | (2, 4, 2, 1, 2, 1, 1), (dg,H,dg,V) = (0, 0)λ, Θmg,ng=90°, Ω0,1=Ω0,0+180° |  |  | ZTE |
|  | 8 | (4, 8, 2, 1, 2, 1, 2), (dg,H,dg,V) = (0, 0)λ, Θmg,ng=90°, Ω0,1=Ω0,0+180° |  |  | ZTE |
|  | 8 | (2, 4, 2, 1, 2, 1, 2), (dg,H,dg,V) = (0, 0)λ |  |  | CMCC |
|  | 8 | (2, 4, 2, 1, 2; 1, 2), (dg,H,dg,V) = (0, 0)λ Θmg,ng = 90°; Ω0,1=Ω0,0+180° |  |  | InterDigital |
| Companies all proposed (dH, dV) = (0.5, 0.5) λ |  |  |  |  |  |

| Alt2: Handheld UT model |  |  |  |  |
| --- | --- | --- | --- | --- |
| Number of RXU | RXU | Number of TXU | TXU | Mentioned by |
| 2 | (1, 5) | 1 | (1) | Huawei |
|  | (4, 8) | / |  | Nokia |
| 4 | (2, 4, 6, 8) | 2 | (2, 6) | Huawei |
|  | (1, 3, 5, 7) | 4 | (1, 3, 5, 7) | Huawei, Nokia |
|  |  | / |  | Nokia, Intel, Docomo |
| 6 | (1, 2, 6, 7, 8, 9) | 6 | (1, 2, 6, 7, 8, 9) | Nokia |
| 8 | (1, 2, 3, 4, 5, 6, 7, 8) | 4 | (1, 3, 5, 7) | Huawei, Nokia |
|  |  | 6 | (1, 2, 6, 7, 8, 9) | Nokia |
|  |  | 8 | (1, 2, 3, 4, 5, 6, 7, 8) | Huawei |
| 16 | (1, 2, 3, 4, 5, 6, 7, 8), dual polarization | 8 | (1, 2, 3, 4, 5, 6, 7, 8) | Huawei |
| 16 | (1, 2, 3, 4, 5, 6, 7, 8), dual polarization | 16 | (1, 2, 3, 4, 5, 6, 7, 8), dual polarization | Huawei |

700MHz carrier frequency:

| For around 700 MHz carrier frequency, for UT antenna modelling |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Total number of TXRU | Mentioned by | Alt1 | Alt2 |
| General | 1T1R | MediaTek | CMCC, Docomo | Samsung, Docomo, InterDigital, Intel |
|  | 1T2R | Samsung (baseline), MediaTek, Google(baseline), InterDigital |  |  |
|  | 2T2R | InterDigital |  |  |
|  | 2T4R | MediaTek |  |  |
|  | 4R | CMCC, Docomo |  |  |
|  | 4T/6R/8R | Google (optional) |  |  |
| low-end IoT UE/MTC | 1R | Ericsson |  |  |
|  | 1T1R | Xiaomi, Sony, Qualcomm, Google, OPPO |  |  |
|  | 1T2R | Qualcomm |  |  |
| Handheld | 1T2R | Xiaomi, Nokia, Sony, Qualcomm (baseline), Ericsson | Xiaomi, ZTE, OPPO | Qualcomm, ZTE, Nokia, Ericsson |
|  | 2T2R | Nokia, InterDigital |  |  |
|  | 2~4 | Tejas |  |  |
|  | 4 | ZTE |  |  |
|  | 1T4R | Qualcomm (optional) |  |  |
|  | 2T4R | OPPO |  |  |
| CPE/FWA | 2/4T4R | Nokia | ZTE, OPPO | Qualcomm, Nokia, Ericsson |
|  | 4T4R | Sony |  |  |
|  | 2T4R | Qualcomm, Ericsson |  |  |
|  | 4T8R | OPPO |  |  |
|  | 6/8T6/8R | Nokia |  |  |
|  | 8R | ZTE |  |  |

| For around 700 MHz carrier frequency, for UT antenna modelling |  | General | Handheld | CPE/FWA |
| --- | --- | --- | --- | --- |
| Polarization | Alt1: Model-1 in Section 7.3.2 of TR 38.901 | DOCOMO | Ericsson, ZTE | Ericsson, ZTE |
|  | Alt2: Model-2 in Section 7.3.2 of TR 38.901 | DOCOMO, MediaTek | Ericsson, ZTE | Ericsson, ZTE |
|  | Alt 3: Handheld UT model in Section 7.3.2 of TR 38.901 | DOCOMO | Ericsson, Interdigital, ZTE | Ericsson |
| Antenna gain pattern | Alt 1: Omnidirectional | MediaTek (baseline) | ZTE | ZTE |
|  | Alt2: According to Table 7.3-2 in TR 38.901 (radiation power pattern for handheld UT) | DOCOMO, MediaTek (optional) | Ericsson, Interdigital, Nokia | Ericsson |

2GHz carrier frequency:

| For around 2 GHz carrier frequency, for UT antenna modelling |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Total number of TXRU | Mentioned by | Alt1 | Alt2 |
| General | 1T2R | Samsung (baseline) | CMCC, Docomo | InterDigital, Docomo, Samsung, Intel |
|  | 2T4R | InterDigital, Google(baseline) |  |  |
|  | 4R | CMCC, Docomo |  |  |
|  | 4T/6R/8R | Google (optional) |  |  |
| low-end IoT UE/MTC | 1R | Ericsson |  |  |
|  | 1T1R | Xiaomi, Sony, Qualcomm, Google, OPPO |  |  |
|  | 1T2R | Qualcomm |  |  |
| Handheld | 1T2R | Xiaomi | Xiaomi, ZTE, OPPO | Qualcomm, ZTE, Nokia, Ericsson |
|  | 2T4R | Sony, Qualcomm (baseline), Nokia, OPPO |  |  |
|  | 4T4R | Qualcomm (optional), Ericsson |  |  |
|  | 4R | ZTE |  |  |
| CPE/FWA | 2/4T4R | Nokia | ZTE, OPPO | Qualcomm, Nokia, Ericsson |
|  | 4T4R | Sony (baseline), Qualcomm (baseline) |  |  |
|  | 4T8R | Qualcomm (optional), OPPO |  |  |
|  | 6/8T6/8R | Nokia |  |  |
|  | 8T8R | Sony (optional), Ericsson |  |  |
|  | 8R | ZTE |  |  |

| For around 2 GHz carrier frequency, for UT antenna modelling |  | General | Handheld | CPE/FWA |
| --- | --- | --- | --- | --- |
| Polarization | Alt1: Model-1 in Section 7.3.2 of TR 38.901 | DOCOMO | Ericsson, ZTE | Ericsson, ZTE |
|  | Alt2: Model-2 in Section 7.3.2 of TR 38.901 | DOCOMO | Ericsson, ZTE | Ericsson, ZTE |
|  | Alt 3: Handheld UT model in Section 7.3.2 of TR 38.901 | DOCOMO | Ericsson, Interdigital, ZTE | Ericsson |
| Antenna gain pattern | Alt 1: Omnidirectional |  | ZTE | ZTE, Nokia(indoor) |
|  | Alt2: According to Table 7.3-2 in TR 38.901 (radiation power pattern for handheld UT) | DOCOMO | Ericsson, Interdigital, ZTE, Nokia | Ericsson, Nokia(indoor/outdoor) |

4GHz carrier frequency:

| For around 4 GHz carrier frequency, for UT antenna modelling |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Total number of TXRU | Mentioned by | Alt1 | Alt2 |
| General | 1T4R | Samsung | CMCC, Docomo, ETRI | InterDigital, Samsung, Docomo, Intel |
|  | 2T4R | InterDigital, MediaTek, ETRI, Google (baseline) |  |  |
|  | 4R | CMCC, Docomo |  |  |
|  | 4T8R | MediaTek |  |  |
|  | 4T/6R/8R | Google (optional) |  |  |
|  | 16R | ETRI |  |  |
| low-end IoT UE/MTC | 1R | Ericsson |  |  |
|  | 1T1R | Sony, Qualcomm, Google, OPPO |  |  |
|  | 1T2R | Xiaomi, Qualcomm |  |  |
| Handheld | 2T4R | Xiaomi (Baseline), Sony, Nokia, OPPO | Xiaomi, ZTE, OPPO | Qualcomm, ZTE, Nokia, Ericsson |
|  | 2T6R | Qualcomm (baseline) |  |  |
|  | 3T6R | Xiaomi (Optional) |  |  |
|  | 4T4R | CATT, Ericsson |  |  |
|  | 4T8R | Qualcomm (optional) |  |  |
|  | 4R | ZTE |  |  |
| CPE/FWA | 2/4T4R | Nokia | ZTE, OPPO | Qualcomm, Nokia, Ericsson |
|  | 4T4R | Sony (baseline) |  |  |
|  | 4T8R | Qualcomm (baseline), OPPO |  |  |
|  | 6/8T6/8R | Nokia |  |  |
|  | 8T8R | Sony (optional), Qualcomm (optional), Ericsson, CATT |  |  |
|  | 8R | ZTE |  |  |

| For around 4 GHz carrier frequency, for UT antenna modelling |  | General | Handheld | CPE/FWA |
| --- | --- | --- | --- | --- |
| Polarization | Alt1: Model-1 in Section 7.3.2 of TR 38.901 | DOCOMO | Ericsson, ZTE | Ericsson, ZTE |
|  | Alt2: Model-2 in Section 7.3.2 of TR 38.901 | DOCOMO, MediaTek, ETRI | Ericsson, ZTE | Ericsson, ZTE |
|  | Alt 3: Handheld UT model in Section 7.3.2 of TR 38.901 | DOCOMO | Ericsson, Interdigital, ZTE | Ericsson |
| Antenna gain pattern | Alt 1: Omnidirectional | MediaTek (baseline) | ZTE | ZTE, Nokia (indoor) |
|  | Alt2: According to Table 7.3-2 in TR 38.901 (radiation power pattern for handheld UT) | DOCOMO, MediaTek (optional), ETRI | Ericsson, Interdigital, ZTE, Nokia | Ericsson, Nokia(indoor/outdoor) |

7GHz carrier frequency:

| For around 7 GHz carrier frequency, for UT antenna modelling |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Total number of TXRU | Mentioned by | Alt1 | Alt2 |
| General | 1T4R | Samsung (baseline) | CMCC, Docomo, ETRI | Samsung, InterDigital, Docomo, Intel |
|  | 2T4R | MediaTek, ETRI, Google (baseline) |  |  |
|  | 4T8R | InterDigital, MediaTek |  |  |
|  | 4T/6R/8R | Google (optional) |  |  |
|  | 4R | Docomo, CMCC |  |  |
|  | 16R | ETRI |  |  |
| low-end IoT UE/MTC | 1T1R | Sony, Google, OPPO |  |  |
|  | 1T2R | vivo |  |  |
| Handheld | 2T4R | Xiaomi (Baseline), vivo, Sony, NEC (baseline), OPPO | Xiaomi, ZTE, vivo, OPPO | Qualcomm, ZTE, Nokia, NEC, Ericsson |
|  | 3T6R | Xiaomi (Optional) |  |  |
|  | 4T4R | CATT, Qualcomm (optional), Nokia |  |  |
|  | 4T8R | Qualcomm (baseline), Ericsson, Nokia |  |  |
|  | 4R | ZTE |  |  |
|  | 6R | ZTE |  |  |
|  | 8R | ZTE |  |  |
| CPE/FWA | 2/4T4R | Nokia | ZTE, vivo, OPPO | Qualcomm, Nokia, Ericsson |
|  | 4T4R | Sony (baseline) |  |  |
|  | 4T8R | vivo, OPPO |  |  |
|  | 8T8R | ZTE, vivo, Sony (optional), Qualcomm (optional), Ericsson, CATT |  |  |
|  | 6/8T6/8R | Nokia |  |  |
|  | 8T16R | Qualcomm (baseline) |  |  |
|  | 8~16 | Tejas |  |  |
|  | 16R | ZTE |  |  |

| For around 7 GHz carrier frequency, for UT antenna modelling |  | General | Handheld | CPE/FWA |
| --- | --- | --- | --- | --- |
| Polarization | Alt1: Model-1 in Section 7.3.2 of TR 38.901 | DOCOMO | Ericsson, ZTE | Ericsson, ZTE |
|  | Alt2: Model-2 in Section 7.3.2 of TR 38.901 | DOCOMO, MediaTek, ETRI | Ericsson, ZTE | Ericsson, ZTE |
|  | Alt 3: Handheld UT model in Section 7.3.2 of TR 38.901 | DOCOMO | Ericsson, Interdigital, ZTE | Ericsson |
| Antenna gain pattern | Alt 1: Omnidirectional | MediaTek (baseline) | ZTE | ZTE, Nokia(indoor) |
|  | Alt2: According to Table 7.3-2 in TR 38.901 (radiation power pattern for handheld UT) | DOCOMO, vivo, MediaTek (optional), ETRI | Ericsson, Interdigital, ZTE, Nokia | Ericsson, Nokia(indoor/outdoor) |

15GHz carrier frequency:

| For around 15 GHz carrier frequency, for UT antenna modelling |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Total number of TXRU | Mentioned by | Alt1 | Alt2 |
| General | 1T4R | Samsung (baseline) | Docomo | Samsung, InterDigital, Docomo, Intel |
|  | 2T4R | Google |  |  |
|  | 4T8R | InterDigital |  |  |
|  | 4T/6R/8R | Google (optional) |  |  |
|  | 8R | Docomo |  |  |
| low-end IoT UE/MTC | 1T1R | Google, OPPO |  |  |
| Handheld | 2T4R | OPPO | ZTE, OPPO | Qualcomm, ZTE, Nokia |
|  | 4T4R | Nokia |  |  |
|  | 4T8R | InterDigital, Qualcomm (baseline), Nokia |  |  |
|  | 4T16R | Qualcomm (optional) |  |  |
|  | 4R | ZTE |  |  |
|  | 6R | ZTE |  |  |
|  | 8R | ZTE |  |  |
| CPE/FWA | 2/4T4R | Nokia | ZTE, OPPO | Qualcomm, Nokia |
|  | 6/8T6/8R | Nokia |  |  |
|  | 4T8R | OPPO |  |  |
|  | 8T8R | Qualcomm (optional) |  |  |
|  | 8T16R | Qualcomm (baseline) |  |  |
|  | 16R | ZTE |  |  |

| For around 15 GHz carrier frequency, for UT antenna modelling |  | General | Handheld | CPE/FWA |
| --- | --- | --- | --- | --- |
| Polarization | Alt1: Model-1 in Section 7.3.2 of TR 38.901 | DOCOMO | Ericsson, ZTE | ZTE |
|  | Alt2: Model-2 in Section 7.3.2 of TR 38.901 | DOCOMO | Ericsson, ZTE | ZTE |
|  | Alt 3: Handheld UT model in Section 7.3.2 of TR 38.901 | DOCOMO | Ericsson, Interdigital, ZTE |  |
| Antenna gain pattern | Alt 1: Omnidirectional |  | ZTE | ZTE |
|  | Alt2: According to Table 7.3-2 in TR 38.901 (radiation power pattern for handheld UT) | DOCOMO | Ericsson, Interdigital, ZTE, Nokia |  |

30GHz carrier frequency:

| For around 30 GHz carrier frequency, for UT antenna modelling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Number of Antenna Elements | Total number of TXRU | Mentioned by | Alt1 | Alt2 |
| General | 8 | 2T2R | Samsung | CMCC, InterDigital, Samsung, Docomo | Docomo, Intel, Qualcomm |
|  | 16 | 4T4R | Qualcomm |  |  |
|  | 32 | 8T8R | Qualcomm |  |  |
|  | 32 | 4R | CMCC |  |  |
|  | 8 | 8R | Docomo |  |  |
|  | 32 | 8R | CMCC, InterDigital |  |  |
| low-end IoT UE/MTC |  | 1T1R | Google, OPPO |  |  |
| Handheld | 8 | 2R | Ericsson, Samsung | ZTE, Ericsson, Samsung | Ericsson, Nokia |
|  | 32 | 4R | ZTE |  |  |
| CPE/FWA |  | 4T8R | OPPO | ZTE, Ericsson, OPPO | Ericsson, Nokia |
|  |  | 2/4T4R | Nokia |  |  |
|  |  | 6/8T6/8R | Nokia |  |  |
|  | 32 | 2R | Ericsson |  |  |
|  | 128 | 8R | ZTE |  |  |
|  | Up to 32 elements, 8 elements per panel (M, N, P) = (2, 2, 2) | 2T2R per panel Config 1 can be 2 panels on front and back Config 2 can be 4 panels on 4 edges | Qualcomm |  |  |

| For around 30 GHz carrier frequency, for UT antenna modelling |  | General | Handheld | CPE/FWA |
| --- | --- | --- | --- | --- |
| Polarization | Alt1: Model-1 in Section 7.3.2 of TR 38.901 | DOCOMO, Interdigital | Ericsson, ZTE | Ericsson, ZTE |
|  | Alt2: Model-2 in Section 7.3.2 of TR 38.901 | DOCOMO | Ericsson, ZTE | Ericsson, ZTE |
|  | Alt 3: Handheld UT model in Section 7.3.2 of TR 38.901 | DOCOMO | Ericsson | Ericsson |
| Antenna gain pattern | Alt 1: Omnidirectional |  |  |  |
|  | Alt2: According to Table 7.3-2 in TR 38.901 (radiation power pattern for handheld UT) | DOCOMO | Ericsson | Ericsson |
|  | Alt 3: Table A.2.1-8 in TR 38.802 |  | ZTE | ZTE |

Round-1 discussions:

(FL1) Proposal 2.2.2

For 6GR evaluation, RAN1 to model the UE antenna as follows:

| UE antenna modelling for RAN1 evaluations | Total number of antenna elements | Total number of TXRU | Alt 1: (M,N,P,Mg,Ng; Mp,Np), (dH,dV), (dg,H,dg,V) if any, or Alt 2: handheld device antenna model using candidate antenna locations as described in section 7.3 in TR38.901 |
| --- | --- | --- | --- |
| Combination1 | 2 | 1T2R, | 1T2R, Alt 1: - 1T: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) , (dH,dV)= (0.5, 0.5)λ - 2R: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 1, 1, 1; 1, 2), (dH,dV)= (0.5, 0.5)λ Alt 2: - 1T: (1) as described in section 7.3 in TR 38.901 - 2R: (1, 5) as described in section 7.3 in TR 38.901 |
| Combination2 | 4 | 2T4R, 4T4R | 2T4R, Alt 1: - 2T: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 1, 1, 1; 1, 2) , (dH,dV)= (0.5, 0.5)λ - 4R: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 2, 1, 1; 1, 2), (dH,dV)= (0.5, 0.5)λ Alt 2: - 2T: (2, 6) as described in section 7.3 in TR 38.901 - 4R: (2, 4, 6, 8) as described in section 7.3 in TR 38.901 4T4R, Alt 1: (M, N, P, Mg, Ng; Mp, Np)= (1, 2, 2, 1, 1; 1, 2), (dH,dV)= (0.5, 0.5)λ Alt 2: (1, 3, 5, 7) as described in section 7.3 in TR38.901 |
| Combination3 | 8 | 4T8R, 8T8R | 4T8R, Alt 1: - 4T: (M, N, P, Mg, Ng; Mp, Np)= (1, 2, 2, 1, 1; 1, 2) for dual polarization or (1, 4, 1, 1, 1; 1, 4) for single polarization, (dH,dV)= (0.5, 0.5)λ. - 8R: (M, N, P, Mg, Ng; Mp, Np)= (1, 4, 2, 1, 1; 1, 4) , (dH,dV)= (0.5, 0.5)λ Alt 2: - 4T: (1, 3, 5, 7) as described in section 7.3 in TR38.901 - 8R: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901 8T8R, Alt 1: (M, N, P, Mg, Ng; Mp, Np)= (1, 4, 2, 1, 1; 1, 4) for dual polarization or (2, 4, 1, 1, 1; 2, 4) for single polarization , (dH,dV)= (0.5, 0.5)λ Alt 2: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901 |
| Combination4 | 16 | 8T16R, 16T16R | 8T16R, Alt 1: - 8T: (M, N, P, Mg, Ng; Mp, Np)= (1, 4, 2, 1, 1; 1, 4) for dual polarization or (2, 4, 1, 1, 1; 2, 4) for single polarization, (dH,dV)= (0.5, 0.5)λ - 16R: (M, N, P, Mg, Ng; Mp, Np)= (2, 4, 2, 1, 1; 2, 4) , (dH,dV)= (0.5, 0.5)λ Alt 2: - 8T: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901, single polarization - 16R: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901, dual polarization 16T16R, Alt 1: (M, N, P, Mg, Ng; Mp, Np)= (2, 4, 2, 1, 1; 2, 4) , (dH,dV)= (0.5, 0.5)λ Alt 2: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901, dual polarization |
| Combination5 | 32 | 16T32R, 32T32R | 16T32R, - 16T: (M, N, P, Mg, Ng; Mp, Np)= (2, 4, 2, 1, 1; 2, 4) for dual polarization or (4, 4, 1, 1, 1; 4, 4) for single polarization, (dH,dV)= (0.5, 0.5)λ - 32R: (M, N, P, Mg, Ng; Mp, Np)= (4, 4, 2, 1, 1; 4, 4) , (dH,dV)= (0.5, 0.5)λ 32T32R, (M, N, P, Mg, Ng; Mp, Np)= (4, 4, 2, 1, 1; 4, 4) , (dH,dV)= (0.5, 0.5)λ |

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| vivo | Comment #1 For 4R, we propose (M, N, P, Mg, Ng; Mp, Np)=(2, 2, 1, 1, 1; 2, 2), (dH,dV)= (0.5, 0.5)λ, which means single polarization. It is not practical to enforce dual-pol in UE side. Further, similar to BS antenna arrays discussed in last meeting, we think it is better to mark one certain configuration as baseline. In our view, considering the current commercialization and future implementation, we support to make 2T4R as baseline for at least around 4GHz and 7GHz. Comment #2 For all the Alts 2 listed above, if they are to be agreed as possible options, we think it is important not to fix the candidate locations. For candidate antenna locations specified in Rel-19 TR 38.901 as depicted in Figure (a), we don’ t think it is a practical assumption. The design of the detailed antenna locations is complex in real world. It depends on many aspects not limited to communications. For instance, an example of the candidate antenna locations of one commercial smart phone is given in Figure (b), which is very different from the location set in Figure (a). Therefore, if handheld model specified in Rel-19 is to be used, let companies to report the candidate locations, not limiting to the 8 locations described in TR38.901. (a) (b) |
| ZTE | We still recommend discussing the UE antenna model separately for each frequency band, in the same way as for the BS. We also understand that the feature leader seems to prefer first defining all possible UE antenna configurations, and then deciding which configuration to use for each frequency band and UE type. Following this approach, we suggest first determining the number of transmit and receive antennas, and then specifying the antenna layout (e.g., UPA array parameters, or which antennas in the R19 UT model to use). Otherwise, it would be difficult to justify why the current proposal uses (2,4,6,8) for 2T4R and (1,3,5,7) for 4T4R. Finally, for FR2, we recommend reusing the 5G multi-panel configuration (e.g., two panels placed back-to-back), which is important for evaluating beam management schemes and better reflects practical deployments. |
| Xiaomi | For 1T2R case, in addition to (1, 2, 1, 1, 1; 1, 2), we should also consider the configuration of (1, 1, 2, 1, 1; 1, 1), i.e., dual-polarization. Under the single-polarization configuration, the reception quality of the two antennas will be relatively poor at certain angles, while the dual-polarization configuration can alleviate this issue to a certain extent. Therefore, we suggest the following revision: 1T2R, Alt 1: - 1T: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) , (dH,dV)= (0.5, 0.5)λ - 2R: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 1, 1, 1; 1, 2) for single polarization or (1, 1, 2, 1, 1; 1, 1) for dual polarization, (dH,dV)= (0.5, 0.5)λ Alt 2: - 1T: (1) as described in section 7.3 in TR 38.901 - 2R: (1, 5) as described in section 7.3 in TR 38.901 |
| MediaTek | In view of the combinations are applicable to all frequency ranges, including 700MHz, 1T1R should also be added as a candidate combination for evaluation assumption: UE antenna modelling for RAN1 evaluations; Total number of antenna elements; Total number of TXRU; Alt 1: (M,N,P,Mg,Ng; Mp,Np), (dH,dV), (dg,H,dg,V) if any, or Alt 2: handheld device antenna model using candidate antenna locations as described in section 7.3 in TR38.901. Combination0; 1; 1T1R,; 1T1R, Alt 1: - 1T: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) , (dH,dV)= (0.5, 0.5)λ - 1R: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) , (dH,dV)= (0.5, 0.5)λ. |
| NTT DOCOMO | Support |
| Ericsson1 | - Similar to ZTE comment, we also prefer to discuss the UE models separately for different carrier frequencies. - 1T1R should be included at least for 700 MHz and 2 GHz (this to take into account case for Massive Communication (IoT) in FR1) - Alt 2 (using 38.901) should be a default assumption at least for handheld UEs and this to be reflected in the proposal. - Similar to vivo comment, our preference is also to keep Alt 2 antenna candidate locations open at this stage. - The applicability of more than 8 antennas should be clarified (i.e., is it intended only for CPEs and possibly other high capability non-handheld/non-EMBB devices?) |
| Sony | We suggest adding combination of 1T1R. The combination of 1T1R is used for low-end IoT. |
| Samsung | For handheld device, we support 1TyR as baseline and 2TyR could be optional when we consider practical handheld UEs. And, we need to consider frequency band to determine the number of UE antennas. For low frequency band, larger antenna spacing is required but UE’s size is limited. Therefore, a smaller number of antennas for lower frequency band should be supported. Considering our general view on UE’s antenna configuration, we are okay with Combination1 for 700MHz and 2GHz. Regarding Combination2, we suggest that 1T4R should be supported as baseline because most of commercialized UEs have 1 Tx for UL. On the other hand, only high-end UEs have 2 Tx antennas for certain frequency band. Therefore, we should add 1T4R as baseline and 2T4R can be optionally considered for evaluation. And 4T4R should be considered for only CPE/FWA instead of handheld, so we propose to add the note such as “note: 4T4R is only considered for CPE/FWA evaluation”. Regarding Combination3, 4, 5, we should consider RAN plenary’s guidance for UE device type and maximum antenna elements. In addition, we should be careful to consider more than 2 Tx antennas for handheld because many antennas cannot be placed due to limited form-factor of handheld and many antennas to support multiple frequency bands and Wi-Fi/Bluetooth/other services are already occupying possible candidate locations for handheld. Therefore, we should discuss Combination 1 and Combination 2 including 1T4R as optional first and need to wait RAN plenary’s guidance for combinations with more than 4 antenna elements. |
| Qualcomm | A few comments. - 1T1R should be also considered for 700MHz, which can be used for IoT device. - For antenna modeling, we propose to make Alt. 2 as the default or the baseline assumption since it provides realistic antenna modeling. For Alt. 2, we don’t need to list the UE antenna index especially when the number of Tx antenna is less than the number of Rx antenna since this will imply that no Tx antenna selection will be supported. For Alt. 2, we suggest the antenna location for evaluation is up to company reporting. - For >4 antenna, we are not sure whether it is reasonable for 700MHz considering large antenna spacing and small UE form factor. |

Round-2 discussions:

(FL2) Proposal 2.2.2-rv1

For 6GR evaluation, RAN1 to model the UE antenna as follows:

- Note: Each of other topics could further decide to use which combination(s) for the evaluations.

- Note: The antenna locations in Alt 2 are considered for performance calibration. Other antenna locations in Alt 2 are also possible for evaluations.

- Note: The antenna locations in Alt 2 not following TR38.901 are up to companies to report.

- FFS: Alt1 or Alt2 is used for each of the combination.

| UE antenna modelling for RAN1 evaluations | Total number of antenna elements | Total number of TXRU | Alt 1: (M,N,P,Mg,Ng; Mp,Np), (dH,dV), (dg,H,dg,V) if any, or Alt 2: device antenna model using candidate antenna locations as described in section 7.3 in TR38.901 | Applicable carrier frequency |
| --- | --- | --- | --- | --- |
| Combination0 | 1 | 1T1R, | 1T1R, Alt 1: 1T: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) , (dH,dV)= (0.5, 0.5)λ 1R: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) , (dH,dV)= (0.5, 0.5)λ Alt 2: - 1T: (1) as described in section 7.3 in TR 38.901 | 700MHz, 2GHz, 4GHz, 7GHz, 15GHz, 30GHz |
| Combination1 | 2 | 1T2R, | 1T2R, Alt 1: - 1T: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) , (dH,dV)= (0.5, 0.5)λ - 2R: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 1, 1, 1; 1, 2) for single polarization or (1, 1, 2, 1, 1; 1, 1) for dual polarization, (dH,dV)= (0.5, 0.5)λ Alt 2: - 1T: (1) as described in section 7.3 in TR 38.901 - 2R: [(1, 5)] as described in section 7.3 in TR 38.901. | 700MHz, 2GHz, 4GHz, 7GHz, 15GHz, 30GHz |
| Combination2 | 4 | 2T4R, 4T4R | 2T4R, Alt 1: - 2T: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 1, 1, 1; 1, 2) , (dH,dV)= (0.5, 0.5)λ - 4R: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 2, 1, 1; 1, 2) for dual polarization or (2, 2, 1, 1, 1; 2, 2) for single polarization, (dH,dV)= (0.5, 0.5)λ Alt 2: - 2T: [(2, 6)] as described in section 7.3 in TR 38.901 - 4R: [(2, 4, 6, 8)] as described in section 7.3 in TR 38.901 4T4R, Alt 1: (M, N, P, Mg, Ng; Mp, Np)= (1, 2, 2, 1, 1; 1, 2), (dH,dV)= (0.5, 0.5)λ Alt 2: [(2, 4,6, 8)] as described in section 7.3 in TR38.901 | 2GHz, 4GHz, 7GHz, 15GHz, 30GHz |
| Combination3 | 8 | 4T8R, 8T8R | 4T8R, Alt 1: - 4T: (M, N, P, Mg, Ng; Mp, Np)= (1, 2, 2, 1, 1; 1, 2) for dual polarization or (1, 4, 1, 1, 1; 1, 4) for single polarization, (dH,dV)= (0.5, 0.5)λ. - 8R: (M, N, P, Mg, Ng; Mp, Np)= (1, 4, 2, 1, 1; 1, 4) , (dH,dV)= (0.5, 0.5)λ Alt 2: - 4T: [(1, 3, 5, 7)] as described in section 7.3 in TR38.901 - 8R: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901 8T8R, Alt 1: (M, N, P, Mg, Ng; Mp, Np)= (1, 4, 2, 1, 1; 1, 4) for dual polarization or (2, 4, 1, 1, 1; 2, 4) for single polarization , (dH,dV)= (0.5, 0.5)λ Alt 2: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901 | 4GHz 7GHz, 15GHz, 30GHz |
| Combination4 | 16 | 8T16R, 16T16R | 8T16R, Alt 1: - 8T: (M, N, P, Mg, Ng; Mp, Np)= (1, 4, 2, 1, 1; 1, 4) for dual polarization or (2, 4, 1, 1, 1; 2, 4) for single polarization, (dH,dV)= (0.5, 0.5)λ - 16R: (M, N, P, Mg, Ng; Mp, Np)= (2, 4, 2, 1, 1; 2, 4) , (dH,dV)= (0.5, 0.5)λ Alt 2: - 8T: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901, single polarization - 16R: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901, dual polarization 16T16R, Alt 1: (M, N, P, Mg, Ng; Mp, Np)= (2, 4, 2, 1, 1; 2, 4) , (dH,dV)= (0.5, 0.5)λ Alt 2: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901, dual polarization | 7GHz, 15GHz, 30GHz |

Any strong concerns, please leave them here.

| Company | Comments |
| --- | --- |
| Nokia | For 1T2R, an option with antenna locations (8,4) should be added because UE SNS model at frequencies below 1 GHz are defined for these candidate ports only. For 2T4R and 4R4T, (1,3,5,7) ports should be considred because they were calibrated in 38.901. For 6/8T6/8R, CPE device ports from Figure 7.3-6 from TR 38.901 should be considred, i.e., antenna locations (1, 2, 6, 7, 8, 9) for 6 and (1,2,3,4,5,6,7,8) for 8. |
| Qualcomm | Firstly, we want to clarify the proposed UE antenna configuration is only for FR1 and FR3 not for FR2 since FR2 should also consider multi-panel which is not included here. 1T1R and 1T2R should not be included for 7GHz, 15GHz and 30GHz. We don’t think there is any interest to support IoT device type at higher freq band. We propose to also add a new combination for 3T6R or 2T6R for 4GHz, 7GHz and 15GHz since 3T and 6R have been already supported in Rel-19. We think it is also useful to include it for 6GR evaluation For combination 4, we think 4T16R should also be included. The note on antenna location for Alt. 2 are confusing and misleading. If other locations can also be used for evaluation, why do we need to define the antenna location here? The proposal here will imply Tx antenna selection will not be considered for 6GR evaluation since Tx antenna is fixed according to the proposal. To move forward, we suggest to remove the forth column from the proposal and discuss it later since anyway we need to discuss how to use Alt.1 or 2 for each combination. |
| vivo | We suggest separating calibration from evaluation, since different antenna locations will affect performance, and the optimal one should be based on company report. We suggest revising the 2nd and 3rd Notes as follows: - Note: The antenna locations in Alt 2 are considered for performance calibration. Other antenna locations in Alt 2 are also possible for evaluations. - Note: The antenna locations in Alt 2 not following TR38.901 are up to companies to report. The antenna locations as described in section 7.3 in TR 38.901 are considered for evaluations. The antenna locations not included in section 7.3 in TR38.901 are also possible for evaluations, and which is up to company report. For 1T or 1R case, it is unnecessary to define (dH,dV)= (0.5, 0.5)λ, and we suggest removing it. When we define UE antenna modelling, both frequency range and UE type should be taken into account. We suggest focusing on handheld device type and below 7GHz firstly. - For combination 2, suggest taking 2T4R as baseline, 4T4R as optional. - For combination 3 and 4, postpone the discussion. |
| Ericsson2 | For “Alt 2: handheld device antenna model using candidate antenna locations as described in section 7.3 in TR38.901”, OK to remove ‘handheld’ so that both CPE and handheld are covered, but in that case we need “Alt 2: handheld device UT antenna model using candidate antenna locations as described in section 7.3 in TR38.901”, which will cover both, but not the base station model in section 7.3 (which can be a ‘device’). FR2 multi-panel configurations are missing as also commented by others. FR2 antenna configurations should be further discussed, e.g. how many TXRUs are present, how panels are modeled, and the total number of elements Suggest to limit current discussion to FR1 and add FFS for FR2 configurations. For Combination 0, 4th column suggest to update Alt2 as below for completeness Alt 2: - 1T: (1) as described in section 7.3 in TR 38.901 - 1R: (1) as described in section 7.3 in TR 38.901 |
| ZTE | Firstly, we suggest evaluating the multi-panel configuration at 30 GHz, as has been done in 5G NR. This configuration is essential for assessing beam-management–related schemes, where each Rx panel corresponds to one Rx beam. The current configuration does not capture this characteristic. Moreover, we suggest including a 3T6R configuration for evaluation. Since many companies are concerned that a smartphone cannot accommodate eight antennas, six antennas may serve as a practical compromise between performance and complexity. We propose adding the following configurations: Combination 5; 32; 2T2R; - 4 ports: (M, N, P, Mg, Ng, Mp, Np) = (2, 4, 2, 1, 2, 1, 1) - (dH, dV) = (0.5, 0.5)λ, (dH,g,dV,g) = (0, 0)λ, Θmg,ng=90°, Ω0,1=Ω0,0+180°; 30GHz. Combination 6; 128; 4T8R 8T8R; - 8 ports: (M, N, P, Mg, Ng, Mp, Np) = (4, 8, 2, 1, 2, 1, 2) - (dH, dV) = (0.5, 0.5)λ, (dH,g,dV,g) = (0, 0)λ,Θmg,ng=90°, Ω0,1=Ω0,0+180°; 30GHz. Combination 7; 6; 3T6R; as described in section 7.3 in TR38.901; 7GHz, 15GHz, 30GHZ. |

#### (FL3) Proposal 2.2.2-rv4

For 6GR evaluation, RAN1 to model the UE antenna as follows:

- Note: Each of other topics could further decide to use which combination(s) for the evaluations. Other combinations are not precluded for evaluations, e.g., 2T6R, 3T6R, 6T6R, 6T8R.

- Note: The antenna locations in Alt 2 are considered as an example and used for performance calibration. Other antenna locations in section 7.3 in TR38.901 are also possible for evaluations and up to companies to report.

- Note: The antenna locations in Alt 2 not included in section 7.3 in TR38.901 are up to companies to report.

- Note: The antenna element/location of T is a subset of the element/locations for R.

- FFS: Alt1 or Alt2 is used for each of the combination.

- FFS on UE antenna modelling for 30GHz.

- Note: The mapping between the combination and the device types will be separately discussed.

| UE antenna modelling for RAN1 evaluations | Total number of antenna elements | Total number of TXRU | Alt 1: (M,N,P,Mg,Ng; Mp,Np), (dH,dV), (dg,H,dg,V) if any, or Alt 2: UT device antenna model using candidate antenna locations as described in section 7.3 in TR38.901 | Applicable carrier frequency |
| --- | --- | --- | --- | --- |
| Combination0 | 1 | 1T1R, | Alt 1: 1T: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) 1R: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) Alt 2: - 1T - 1R | 700MHz, 2GHz |
| Combination1 | 2 | 1T2R, | Alt 1: - 2R: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 1, 1, 1; 1, 2) for single polarization or (1, 1, 2, 1, 1; 1, 1) for dual polarization, (dH,dV)= (0.5, 0.5)λ Alt 2: - 2R: [(1, 5), or (4, 8)] as described in section 7.3 in TR 38.901. | 700MHz, 2GHz, 4GHz |
| Combination2 | 4 | 1T4R, 2T4R, 4T4R | Alt 1: - 4R: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 2, 1, 1; 1, 2) for dual polarization or (2, 2, 1, 1, 1; 2, 2) for single polarization, (dH,dV)= (0.5, 0.5)λ Alt 2: - 4R: [(2, 4, 6, 8), or (1, 3, 5, 7)] as described in section 7.3 in TR 38.901 | 700MHz 2GHz, 4GHz, 7GHz, 15GHz |
| Combination3 | 8 | 4T8R, 8T8R | Alt 1: (M, N, P, Mg, Ng; Mp, Np)= (1, 4, 2, 1, 1; 1, 4) for dual polarization or (2, 4, 1, 1, 1; 2, 4) for single polarization , (dH,dV)= (0.5, 0.5)λ Alt 2: [(1, 2, 3, 4, 5, 6, 7, 8)] as described in section 7.3 in TR38.901 | 2GHz, 4GHz, 7GHz, 15GHz |
| Combination4 Note: This combination is not for smartphone. | 16 | 4T16R, 8T16R, 16T16R | Alt 1: - 16R: (M, N, P, Mg, Ng; Mp, Np)= (2, 4, 2, 1, 1; 2, 4) , (dH,dV)= (0.5, 0.5)λ Alt 2: - 16R: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901, dual polarization | 7GHz, 15GHz |

#Discussion points#

- Added the note for combination 4 that it is not for smartphone to address MediaTek’s strong concern.

- Any other strong concern literarily??

Any strong concerns, please leave them here.

| Company | Comments |
| --- | --- |

## Other views in TDoc

| Company | Views/proposals |
| --- | --- |
| Futurewei | Proposed to study energy consumption by different antenna models/architectures. Proposed to study mixed antenna architectures (with additional fully-digital 1-bit ADC receiver at the base station) for mmWave FR2. |

# SLS assumptions for TN

## Layout

### Companies’ views

Companies’ views collected over post-122 email discussion were summarized in R1-2507292. In addition, the views included in the contribution are summarized as follows:

| Company | Views/proposals |
| --- | --- |
| ZTE | Proposal 2-2-1: For dense urban and urban macro scenarios, co-channel HetNet deployment should be evaluated, where macro and micro layers share the same frequency. The evaluation assumptions are as follows. Scenario; Dense Urban; Urban Macro. Layout; Single layer: - Hex. Grid Two layers with same carrier frequency for macro and micro layers: - Macro layer + Micro layer: 4GHz+4GHz, or 7GHz+7GHz - Micro layer: Random or fixed locations. |
| CATT | Proposal 1: For the study on evaluation assumption in 6GR, adopts the following layouts: - Indoor-hotspot - single layer layout illustration with ISD = 20m, which is equivalent to 12 TRPs per 120m*50m; - Dense Urban - single layer layout illustration with hex grid; - two layers layout illustration which the macro grid is hex grid and the TRPs in the micro layer are randomly dropped; - Urban Macro - single layer layout illustration with hex grid; - two layers layout illustration which the macro grid is hex grid and the TRPs in the micro layer are randomly dropped; - Rural - single layer layout illustration with hex grid. |
| InterDigital | Parameters; Indoor Hotspot; Dense urban; Urban Macro; Rural; Suburban. Layout; TR38.901 v19.1.0 InH (Room Size: 120 m x 50 m x 3 m); Option 1: Single layer - Hex. Grid Option 2: Two layers - Macro layer: Hex. Grid - Micro layer: Random drop; Single layer - Hex. Grid; Single layer - Hex. Grid; Single layer - Hex. Grid. |
| DOCOMO | Parameters; Indoor hotspot; Dense urban; Rural; Urban macro; Sub-urban macro. Layout; Follow TR 38.914, i.e., Single layer: - Indoor floor (Open office), 120m x 50m; Follow TR 38.914, i.e., Single layer: - Macro layer: Hex. Grid Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop; Follow TR 38.914, i.e., Single layer - Macro layer: Hex. Grid; Follow TR 38.914, i.e., Single layer: - Macro layer: Hex. Grid Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop; Follow TR 38.914, i.e., Single layer Macro layer: Hex. Grid. |

### Discussions

Summary on the views:

Companies’ views are summarized in R1-2507292 collected from post-122 email discussion. In addition, companies shared the views in the contribution are also summarized in section 3.1.1.

Round-1 discussions:

(FL1) Proposal 3.1.2

For 6GR evaluation, the layout for system-level simulation is assumed as follows:

| Parameters | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- | --- | --- |
| Layout | Single layer - Indoor floor (Open office), (Room size: 120m x 50m) | Single layer: - Hex. Grid Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop | Single layer: - Hex. Grid | Single layer: - Hex. Grid Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop | Single layer: - Hex. Grid |

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| ZTE | For the two-layer layout, we also suggest considering a fixed-drop approach. In 5G NR evaluations, random drops require a minimum separation between micro BSs, and the methods for generating micro BS locations may vary significantly across companies, making it difficult to align results. Using fixed drops simplifies the simulation and addresses this issue. A fixed drop is illustrated as follows. |
| NTT DOCOMO | We are generally fine to the proposal. RAN1 can use the parameters of Layout decided by RANp (i.e., TR 38.914). |

Round-2 discussions:

(FL2) Proposal 3.1.2-rv1

For 6GR evaluation, the layout for system-level simulation is assumed as follows:

- Note: Single layer will be prioritized for the evaluations.

- Note: The carrier frequency for the corresponding layout for the two layers will be reported by companies for the evaluations.

| Parameters | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- | --- | --- |
| Layout | Single layer - Indoor floor (Open office), (Room size: 120m x 50m) | Single layer: - Hex. Grid Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop | Single layer: - Hex. Grid | Single layer: - Hex. Grid Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop | Single layer: - Hex. Grid |

Any strong concerns, please leave them here.

| Company | Comments |
| --- | --- |
| Nokia | There needs to be a minimum distance between macro and micro BSs (as for the 3GPP HetNet scenarios defined during LTE times). |
| Tejas Networks | Deployment Scenarios (38.914) are being discussed at RAN Plenary and is expected to be finalized by the upcoming meeting in December 2025. RAN1 should also plan for the study of possible new deployment scenarios being discussed at RAN Plenary. |

#### Agreement

Agreement

For 6GR evaluation, the layout for system-level simulation is assumed as follows:

- Note: Single layer will be prioritized for the evaluations.

- Note: The carrier frequency for the corresponding layout for the two layers will be reported by companies for the evaluations.

- FFS the minimum distance for random drop in two layers.

- Note: for system-level simulation of MIMO schemes, specific assumptions could be discussed under MIMO discussion

| Parameters | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- | --- | --- |
| Layout | Single layer - Indoor floor (Open office), (Room size: 120m x 50m) | Single layer: - Hex. Grid Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop | Single layer: - Hex. Grid | Single layer: - Hex. Grid Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop | Single layer: - Hex. Grid |

## BS/UE transmission power

### Companies’ views

Companies’ views collected over post-122 email discussion were summarized in R1-2507292. In addition, the views included in the contribution are summarized as follows:

| Company | Views/proposals |
| --- | --- |
| Nokia | Proposal 5: For the indoor deployments, for the total transmit power per BS, RAN1 to consider below 15 GHz: 24 dBm per 20 MHz, EIRP should not exceed 61 dBm, above 15 GHz: 23 dBm per 100 MHz, EIRP should not exceed 58 dBm. Proposal 6: For the outdoor deployments, macro layer, for the total transmit power per BS, RAN1 to consider: a. For around 700 and 2GHz MHz: 44 dBm per 20 MHz for dense urban and 49 dBm per 20 MHz otherwise. b. For around 4 GHz MHz: 44 dBm per 20 MHz for dense urban and 49 dBm per 20 MHz otherwise, EIRP should not exceed 88 dBm. c. For around 7 GHz and around 15 GHz: 44 dBm per 20 MHz for dense urban and 46 dBm per 20 MHz otherwise, EIRP should not exceed 88 dBm. d. For around 30 GHz: 33 dBm per 20 MHz, EIRP should not exceed 75 dBm. Proposal 9: For handheld UE modelling, RAN1 to consider the following UE transmit power: a. For below 7 GHz: 23, 26, 29 dBm b. For 7 GHz and above: 26, 29 dBm Proposal 12: For CPE modelling, RAN1 to support 23, 26, 29, 31 dBm power classes and up to the max conducted power that meets the maximum average band EIRP limit (subject to regulatory limitations and accounting for maximum average EIRP limit mitigations for both indoor and outdoor CPEs). |
| InterDigital | Parameters; Indoor Hotspot; Dense urban; Urban Macro; Rural; Suburban. Total transmit power per BS; Around 7GHz and below: 24dBm/20MHz; Around 7GHz and below for Macro layer: 44dBm/20MHz Around 7GHz and below for Micro layer: 33dBm/20MHz; Around 7GHz and below: 44dBm/20MHz; Around 7GHz and below: 49dBm/20MHz; Around 7GHz and below: 49dBm/20MHz. Around 15GHz and above:; Around 15GHz and above for Macro layer: Around 15GHz and above for Micro layer:; Around 15GHz and above:; Around 15GHz and above:; Around 15GHz and above:. UE power class; Around 7GHz and below: 23dBm; Around 7GHz and below: 23dBm; Around 7GHz and below: 23dBm; Around 7GHz and below: 23dBm; Around 7GHz and below: 23dBm. |
| Huawei | Proposal 7: Multiple options for BS Tx Power assumptions can be considered for 6GR evaluations allowing for different evaluation purposes. Proposal 8: Table 4 of BS Tx Power assumptions for 6GR evaluations can be the starting point for the discussions. |
| Samsung | Support the TRP for BS per deployment scenario per frequency band as in Table 2.3-1 including the transmission power. |
| Sony | Proposal 6: For UE power class, 23 dBm for FR1 and 26 dBm for FR3 should be considered. |
| MediaTek | UE power class: - Below 7 GHz for TN: PC3 as baseline and PC2 as optional - Around and above 7 GHz for TN: PC2 as baseline and PC3 as optional |
| Qualcomm | Proposed to consider the following assumption on total transmit power per BS for 6GR evaluation Indoor Hotspot; Dense Urban; Rural; Urban Macro; Sub-urban macro. Around 700MHz; N/A; Macro BS: 44 dBm per 20 MHz; 49 dBm per 20 MHz; Macro BS: 49 dBm per 20 MHz; 49 dBm per 20 MHz. Around 2GHz; 24 dBm per 20 MHz; Macro BS: 44 dBm per 20 MHz; 49 dBm per 20 MHz; Macro BS: 49 dBm per 20 MHz; 49 dBm per 20 MHz. Around 4GHz; 24 dBm per 20 MHz; Macro BS: 41 dBm per 20 MHz; 46 dBm per 20 MHz; Macro BS: 46 dBm per 20 MHz; 46 dBm per 20 MHz. Around 7GHz; 24 dBm per 20 MHz; Macro BS: 41 dBm per 20 MHz; 46 dBm per 20 MHz; Macro BS: 46 dBm per 20 MHz; 46 dBm per 20 MHz. Around 30GHz; 23 dBm per 100 MHz; 40 dBm per 100 MHz; 40 dBm per 100 MHz. Note: BS Tx power scales up with bandwidth proportionally. Note: The maximum allowed EIRP for each scenario will be defined. FFS values.. |
| DOCOMO | Parameters; Indoor hotspot; Dense urban; Rural; Urban macro; Sub-urban macro. Total transmit power per BS; 2GHz: 24dBm/20MHz 4GHz: 24dBm/20MHz 7GHz: 24dBm/20MHz; Macro layer: 700MHz, 2GHz, 4GHz, and 7GHz: 44dBm/20MHz Micro layer: 700MHz, 2GHz, 4GHz, and 7GHz: 33dBm/20MHz; 49dBm/20MHz; 49dBm/20MHz; 49dBm/20MHz. 15GHz: 23dBm/20MHz 30GHz: 23dBm/20MHz EIRP should not exceed 58 dBm; Macro layer: 15GHz and 30GHz: 40dBm/20MHz EIRP should not exceed 73dBm Macro layer: 15GHz and 30GHz: 33dBm/20MHz EIRP should not exceed 68dBm; -; 43dBm/20MHz, EIRP should not exceed 78 dBm; 49dBm/20MHz. UE power class; 23dBm, EIRP should not exceed 43 dBm; 23dBm, EIRP should not exceed 43 dBm; 23dBm EIRP should not exceed 43 dBm; 23dBm EIRP should not exceed 43 dBm; 23dBm EIRP should not exceed 43 dBm. 23dBm, EIRP should not exceed 43 dBm; 23dBm, EIRP should not exceed 43 dBm; -; 23dBm EIRP should not exceed 43 dBm; 23dBm EIRP should not exceed 43 dBm. |

### Discussions

Summary on the views:

Based on collected views over post-122 email discussion summarized in R1-2507292 and the views mentioned in the contributions for this meeting, all the views are summarized as follows:

Indoor Hotspot

| Frequency | Total transmit power per BS | Mentioned by |
| --- | --- | --- |
| Around 2GHz | 24dBm/20MHz | Huawei, OPPO, CATT, Ericsson, ZTE, Qualcomm, DCM, Samsung, MTK, Sony, Intel, Ofinno, Nokia, Futurewei |
| Around 4GHz | 24dBm/20MHz | Huawei, OPPO, CATT, Ericsson, ZTE, Qualcomm, DCM, Samsung, MTK, Sony, Intel, Ofinno, Nokia, Futurewei |
| Around 7GHz | 24dBm/20MHz | Huawei, OPPO, CATT, ZTE, Qualcomm, DCM, Xiaomi, Samsung, MTK, Sony, Intel, Ofinno, Nokia, Futurewei |
|  | 23dBm/80MHz | Ericsson |
| Around 15GHz | 23dBm/20MHz, EIRP not exceed 58dBm | ZTE, DCM, Intel, Nokia |
|  | 24dBm/20MHz | Samsung, |
|  | 23dBm/80MHz | Ericsson |
| Around 30GHz | 23dBm/20MHz, EIRP not exceed 58dBm | ZTE, Qualcomm, DCM, Intel |
|  | 26.6dBm/100MHz | Samsung, |
|  | 23dBm/80MHz | Ericsson |
|  | 23dBm/100MHz | Huawei, Nokia (EIRP<61dBm) |

| Frequency | UE power class | Mentioned by |
| --- | --- | --- |
| Around 7GHz and below | 23dBm (FDD and TDD) | Huawei, vivo, OPPO, Ericsson, ZTE, Qualcomm, DCM, Samsung, MTK, Sony, Intel, Nokia, Futurewei |
|  | 26dBm (TDD) | Huawei, vivo, OPPO, CATT, Ericsson, ZTE, Qualcomm, Samsung, MTK, Intel, Ofinno, Nokia, Futurewei |
|  | 29dBm (TDD) | Huawei, Qualcomm, Nokia, Futurewei |
|  | 31dBm (CPE/FWA) | ZTE, |
| Around 15GHz and above | 23dBm, EIRP not exceed 43dBm | Huawei, OPPO, Ericsson, DCM, Samsung, Intel, |
|  | 29dBm | Nokia, |
|  | 26dBm | Samsung, Nokia, |

Dense Urban

| Frequency | Total transmit power per BS | Mentioned by |
| --- | --- | --- |
| Around 700MHz | 44dBm/20MHz | Huawei, vivo, OPPO, CATT, Interdigital, ZTE, DCM, Samsung, Intel, Ofinno, Futurewei, ETRI, Qualcomm, Nokia |
|  | 33dBm/20MHz (micro layer) | Huawei, OPPO, Interdigital, ZTE, DCM, Intel, Ofinno, Nokia, Futurewei, |
|  | 46dBm/20MHz | Apple |
|  | 49dBm/20MHz | Huawei |
|  | 52dBm/20MHz | Ericsson, Nokia, |
| Around 2GHz | 44dBm/20MHz | Huawei, vivo, OPPO, CATT, Interdigital, ZTE, DCM, Samsung, Sony, Intel, Ofinno, Futurewei, ETRI, Qualcomm |
|  | 33dBm/20MHz (micro layer) | Huawei, OPPO, Interdigital, ZTE, DCM, Sony, Intel, Ofinno, Nokia, Futurewei, |
|  | 46dBm/20MHz | Apple, Huawei |
|  | 49dBm/20MHz | Ericsson, Nokia, |
|  | 52dBm/20MHz | Ericsson, |
| Around 4GHz | 44dBm/20MHz | Huawei, vivo, OPPO, CATT, Interdigital, ZTE, DCM, Samsung, MTK, Sony, Intel, Ofinno, Futurewei, ETRI, Nokia |
|  | 41dBm/20MHz | Qualcomm |
|  | 33dBm/20MHz (micro layer) | Huawei, OPPO, Interdigital, ZTE, DCM, Sony, Intel, Ofinno, Nokia, Futurewei, |
|  | 46dBm/20MHz | Qualcomm, Apple, |
|  | 49dBm/20MHz | Ericsson, |
| Around 7GHz | 44dBm/20MHz | Huawei, vivo, OPPO, CATT, Interdigital, DCM, Samsung, MTK, Sony, Intel, Ofinno, Futurewei, ETRI, Nokia (EIRP<88dBm) |
|  | 41dBm/20MHz | Qualcomm |
|  | 43dBm/20MHz | Huawei |
|  | 33dBm/20MHz (micro layer) | Huawei, OPPO, Interdigital, ZTE, DCM, Sony, Intel, Ofinno, Nokia, Futurewei, |
|  | 46dBm/20MHz | Qualcomm, Apple |
|  | 49dBm/20MHz | Ericsson, |
|  | 56dBm | ZTE |
| Around 15GHz | 40dBm/20MHz, EIRP not exceed 73dBm | Huawei, OPPO, Futurewei, |
|  | 33dBm/20MHz, EIRP not exceed 68dBm (micro layer) | Huawei, OPPO, Nokia, Futurewei, |
|  | 23dBm/20MHz, EIRP not exceed 58dBm | Intel, |
|  | 44dBm/20MHz | Samsung, |
|  | 46.2dBm/20MHz | Nokia, |
| Around 30GHz | 40dBm/20MHz, EIRP not exceed 73dBm | Huawei, OPPO, ZTE, Futurewei,, Qualcomm |
|  | 33dBm/20MHz, EIRP not exceed 68dBm (micro layer) | Huawei, OPPO, ZTE, Nokia, Futurewei, |
|  | 34dBm/20MHz | vivo, Ericsson, |
|  | 26.6dBm/100MHz | Samsung, |
|  | 32.6dBm/800MHz | Samsung, |
|  | 23dBm/20MHz, EIRP not exceed 58dBm | Intel, |
|  | 35dBm/20MHz | Nokia, |

| Frequency | UE power class | Mentioned by |
| --- | --- | --- |
| Around 7GHz and below | 23dBm (FDD and TDD) | Huawei, vivo, OPPO, Ericsson, Interdigital, ZTE, Qualcomm, DCM, Samsung, MTK, Sony, Intel, Nokia, Futurewei, Apple, ETRI |
|  | 26dBm (TDD) | Huawei, vivo, OPPO, CATT, Ericsson, ZTE, Qualcomm, Samsung, MTK, Intel, Ofinno, Nokia, Futurewei, Apple |
|  | 29dBm (TDD) | Huawei, Qualcomm, Nokia, Futurewei |
|  | 31dBm (CPE/FWA) | ZTE, Nokia |
| Around 15GHz and above | 23dBm, EIRP not exceed 43dBm | Huawei, vivo, OPPO, Ericsson, DCM, Samsung, Intel, Futurewei |
|  | 29dBm | Nokia, |
|  | 26dBm | Samsung, Nokia, Sony |
|  | 12dBm | Qualcomm, |
|  | 35dBm (CPE/FWA) | Samsung |

Rural

| Frequency | Total transmit power per BS | Mentioned by |
| --- | --- | --- |
| Around 700MHz | 49dBm/20MHz | Huawei, vivo, OPPO, CATT, Interdigital, ZTE, Qualcomm, DCM, Samsung, {Tejas Networks, CEWiT, IIT Madras}, MTK, Sony, Intel, Ofinno, Nokia, Futurewei |
|  | 52dBm/20MHz | Ericsson, |
| Around 2GHz | 49dBm/20MHz | Huawei, OPPO, Ericsson, Interdigital, ZTE, DCM, Samsung, MTK, Sony, Intel, Ofinno, Nokia(EIRP<88dBm), Futurewei |
|  | 52dBm/20MHz | Ericsson, |
| Around 4GHz | 49dBm/20MHz | Huawei, vivo, OPPO, CATT, Ericsson, Interdigital, ZTE, DCM, Samsung, {Tejas Networks, CEWiT, IIT Madras}, MTK, Sony, Intel, Ofinno, Nokia(EIRP<88dBm), Futurewei |
|  | 46dBm/20MHz | Qualcomm, |
| Around 7GHz | 49dBm/20MHz | Huawei, vivo, OPPO, Ericsson, Interdigital, ZTE, DCM, {Tejas Networks, CEWiT, IIT Madras}, MTK, Sony, Intel, Ofinno, Futurewei |
|  | 46dBm/20MHz | Nokia(EIRP<88dBm) |
|  | 56dBm/20MHz | ZTE, Samsung |
|  | 53dBm/100MHz | Qualcomm, |

| Frequency | UE power class | Mentioned by |
| --- | --- | --- |
| Around 7GHz and below | 23dBm (FDD and TDD) | Huawei, vivo, OPPO, Ericsson, Interdigital, ZTE, Qualcomm, DCM, Samsung, {Tejas Networks, CEWiT, IIT Madras}, MTK, Sony, Intel, Nokia, Futurewei |
|  | 26dBm (TDD) | Huawei, vivo, OPPO, CATT, Ericsson, ZTE, Qualcomm, Samsung, {Tejas Networks, CEWiT, IIT Madras}, MTK, Intel, Ofinno, Nokia, Futurewei |
|  | 29dBm (TDD) | Huawei, Qualcomm, Nokia, Futurewei |
|  | 31dBm (CPE/FWA) | ZTE, Nokia |

Urban Macro

| Frequency | Total transmit power per BS | Mentioned by |
| --- | --- | --- |
| Around 7GHz and below | 56dBm | ZTE |
|  | 52dBm/20MHz | Ericsson, Nokia, |
|  | 49dBm/20MHz | Huawei, vivo, OPPO, CATT, Ericsson, ZTE, Qualcomm, DCM, Xiaomi, MTK, Sony, Ofinno, Nokia, Futurewei, |
|  | 46dBm/20MHz | Huawei, Qualcomm, Nokia (EIRP<88dBm), Apple, Samsung |
|  | 44dBm/20MHz | Interdigital, Intel, Huawei |
|  | 33dBm/20MHz (micro layer) | Huawei, Sony, Ofinno, Nokia, |
| Around 15GHz and above | 43dBm/20MHz, EIRP not exceed 78dBm | Huawei, vivo, OPPO, ZTE, DCM, Futurewei, |
|  | 33dBm/20MHz, EIRP not exceed 68dBm (micro layer) | Huawei, |
|  | 34dBm/20MHz | Ericsson, |
|  | 44dBm/20MHz | Samsung |
|  | 23dBm/20MHz | Intel |
| Around 30GHz | 26.6dBm/100MHz | Samsung |
|  | 32.6dBm/800MHz | Samsung |
|  | 40dBm/100MHz | Qualcomm |

| Frequency | UE power class | Mentioned by |
| --- | --- | --- |
| Around 7GHz and below | 23dBm (FDD and TDD) | Huawei, vivo, OPPO, Ericsson, Interdigital, ZTE, Qualcomm, DCM, Xiaomi, Samsung, MTK, Sony, Intel, Nokia, Futurewei, Apple |
|  | 26dBm (TDD) | Huawei, vivo, OPPO, CATT, Ericsson, ZTE, Qualcomm, Samsung, MTK, Intel, Ofinno, Nokia, Futurewei, Apple |
|  | 29dBm (TDD) | Huawei, Qualcomm, Nokia, Futurewei, |
|  | 31dBm (CPE/FWA) | ZTE, Nokia, |
| Around 15GHz and above | 23dBm, EIRP not exceed 43dBm | Huawei, vivo, OPPO, Ericsson, DCM, Samsung, Intel, Futurewei, |
|  | 26dBm | Samsung, Nokia, |
|  | 29dBm | Nokia, |
|  | 35dBm (CPE/FWA) | Samsung, |

Suburban Macro

| Frequency | Total transmit power per BS | Mentioned by |
| --- | --- | --- |
| Around 7GHz and below | 56dBm | ZTE |
|  | 52dBm/20MHz | Ericsson, Nokia, |
|  | 49dBm/20MHz | Huawei, vivo, OPPO, Ericsson, Interdigital, ZTE, Qualcomm, DCM, MTK, Ofinno, Nokia, Futurewei, Apple |
|  | 46dBm/20MHz | Qualcomm, Nokia (EIRP<88dBm), |
|  | 44dBm/20MHz | Samsung, Intel, |
| Around 15GHz and above | 49dBm/20MHz | DCM, |
|  | 44dBm/20MHz | Samsung, |
|  | 23dBm/20MHz | Intel, |
| Around 30GHz | 34dBm/20MHz | Ericsson, |

| Frequency | UE power class | Mentioned by |
| --- | --- | --- |
| Around 7GHz and below | 23dBm (FDD and TDD) | Huawei, vivo, OPPO, Ericsson, Interdigital, ZTE, Qualcomm, DCM, Samsung, MTK, Intel, Nokia, Futurewei, Apple |
|  | 26dBm (TDD) | Huawei, vivo, OPPO, Ericsson, ZTE, Qualcomm, Samsung, MTK, Intel, Ofinno, Nokia, Futurewei, Apple |
|  | 29dBm (TDD) | Huawei, Qualcomm, Nokia, Futurewei, |
|  | 31dBm (CPE/FWA) | ZTE, Nokia |
| Around 15GHz and above | 23dBm, EIRP not exceed 43dBm | Huawei, OPPO, Ericsson, DCM, Nokia, |
|  | 26dBm (TDD) | Samsung, Nokia, |

Round-1 discussions:

(FL1) Proposal 3.2.2-1

For 6GR evaluation, the total transmit power per BS for system-level simulation is assumed as follows:

| Total transmit power per BS | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Sub-urban macro |
| --- | --- | --- | --- | --- | --- |
| Around 700MHz | NA | Macro BS: - Option1: 44 dBm per 20 MHz - Option2: 49 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 2GHz | 24 dBm per 20 MHz | Macro BS: - Option1: 44 dBm per 20 MHz - Option2: 46 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: - Option1: 49 dBm per 20 MHz - Option2: 46 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 4GHz | 24 dBm per 20 MHz | Macro BS: - Option1: 44 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: - Option1: 49 dBm per 20 MHz - Option2: 44 dBm per 20 MHz - Option3: 46 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 7GHz | 24 dBm per 20 MHz | Macro BS: - Option1: 44 dBm per 20 MHz - Option2: 43 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: - Option1: 49 dBm per 20 MHz - Option2: 43 dBm per 20 MHz - Option3: 46 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 15GHz | 23dBm per 20MHz | Macro BS: 40dBm per 20MHz Micro BS: 33 dBm per 20 MHz | NA | Macro BS: 43dBm per 20MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49dBm per 20MHz |
| Around 30GHz | 23 dBm per 20 MHz | Micro BS: 33 dBm per 20 MHz | NA | Micro BS: 33 dBm per 20 MHz | Macro BS: 33 dBm per 20 MHz |
| Note: BS Tx power scales up with bandwidth proportionally. Note: The maximum BS Tx power for each scenario will be defined. FFS: 56dBm for outdoor BS and xx for indoor BS. |  |  |  |  |  |

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| ZTE | ‘Micro BS: 33 dBm per 20 MHz’ is missing for 700MHz/2GHz/4GHz? Per FL’s summary, a lot of vendors and operators support to have them for all frequency range of dense urban and urban macro. That’s aligned with typical NW deployment as in a real-field network. ‘Micro BS: 33 dBm per 20 MHz’ for 700MHz/2GHz/4GHz should be added. |
| Samsung | Need to clarify what is the intention to make total transmit power common for all antenna configurations first. It is our understanding that BS Tx power may scale up with the number of antenna elements On top of that, we should discuss whether to consider an EIRP limit per frequency band. |
| Qualcomm | For 4GHz and 7GHz, we propose to have 3dB lower Tx power than 2GHz at the same deployment scenario. Using the same Tx power for both TDD and FDD is not aligned with the previous RAN1 assumption. For around 30GHz, our proposal is to follow the same assumption in TR 38.830, i.e., using 23 dBm per 100MHz not 23 dBm per 20MHz for indoor and 40dBm per 100MHz for outdoor. We think not only the maximum BS Tx power for each scenario but also the maximum EIRP limit need to be defined for each scenario. The maximum BS Tx power plus the total antenna gain should be exceed the maximum EIRP limit. |

(FL1) Proposal 3.2.2-2

For 6GR evaluation, the UE power class for system-level simulation is assumed as follows:

| UE power class | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Sub-urban macro |
| --- | --- | --- | --- | --- | --- |
| Around 700MHz | NA | 23dBm, 26dBm | 23dBm, 26dBm | 23dBm, 26dBm | 23dBm, 26dBm |
| Around 2GHz | 23dBm, 26dBm | 23dBm, 26dBm | 23dBm, 26dBm | 23dBm, 26dBm | 23dBm, 26dBm |
| Around 4GHz | 23dBm, 26dBm | 23dBm, 26dBm | 23dBm, 26dBm | 23dBm, 26dBm | 23dBm, 26dBm |
| Around 7GHz | 23dBm, 26dBm and 29dBm Note: CPE/FWA is [31dBm] Note: EIRP should not exceed 43 dBm | 23dBm, 26dBm and 29dBm Note: CPE/FWA is [31dBm] Note: EIRP should not exceed 43 dBm | 23dBm, 26dBm and 29dBm Note: CPE/FWA is [31dBm] Note: EIRP should not exceed 43 dBm | 23dBm, 26dBm and 29dBm Note: CPE/FWA is [31dBm] Note: EIRP should not exceed 43 dBm | 23dBm, 26dBm and 29dBm Note: CPE/FWA is [31dBm] Note: EIRP should not exceed 43 dBm |
| Around 15GHz | 23dB, 26dBm and 29dBm | 23dB, 26dBm and 29dBm | NA | 23dB, 26dBm and 29dBm | 23dBm EIRP should not exceed 43 dBm |
| Around 30GHz | 23dB, 12dBm, 26dBm, and 29dBm Note: CPE/FWA is [31dBm] Note: EIRP should not exceed 43 dBm | 23dB, 12dBm, 26dBm, and 29dBm Note: CPE/FWA is [31dBm] Note: EIRP should not exceed 43 dBm | NA | 23dB, 12dBm, 26dBm, and 29dBm Note: CPE/FWA is [31dBm] Note: EIRP should not exceed 43 dBm | 23dBm EIRP should not exceed 43 dBm |

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| vivo | For 29dBm power class, we don’t think there are such type of commercialized PAs in the market. Hence we suggest to remove it or at least make it optional. For 12dBm power class, we think more discussion are needed to understand its use case. |
| ZTE | We are generally fine with this proposal, with one suggestion. Since the typical carrier frequencies for CPE/FWA have not been clearly identified yet, we suggest keeping the CPE/FWA power note for other carrier frequencies. |
| OPPO | We also think at least for evaluation 29dBm is not typical configuration. |
| Samsung | We want to share the similar view on 29 dBm for UE and suggest to remove 29dBm. For around 30GHz, 35 dBm with EIRP limitation such as 55 dBm can be supported for CPE/FWA instead of 31 dBm. We are open to discuss whether 12 dBm power class is required. |
| Qualcomm | The UE number per TRxP will be dependent on the used traffic model and may not be listed here. For UMa, we propose to use the same assumption as before (i.e., 80% indoor and 20% outdoor) and remove the bracket. |

Round-2 discussions:

(FL2) Proposal 3.2.2-1rv1

For 6GR evaluation, the total transmit power per BS for system-level simulation is assumed as follows:

| Total transmit power per BS | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Sub-urban macro |
| --- | --- | --- | --- | --- | --- |
| Around 700MHz | NA | Macro BS: - Option1: 44 dBm per 20 MHz - Option2: 49 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 2GHz | 24 dBm per 20 MHz | Macro BS: - Option1: 44 dBm per 20 MHz - Option2: 46 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: - Option1: 49 dBm per 20 MHz - Option2: 46 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 4GHz | 24 dBm per 20 MHz | Macro BS: - Option1: 44 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: - Option1: 49 dBm per 20 MHz - Option2: 44 dBm per 20 MHz - Option3: 46 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 7GHz | 24 dBm per 20 MHz | Macro BS: - Option1: 44 dBm per 20 MHz - Option2: 43 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: - Option1: 49 dBm per 20 MHz - Option2: 43 dBm per 20 MHz - Option3: 46 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 15GHz | 23dBm per 20MHz | Macro BS: 40dBm per 20MHz Micro BS: 33 dBm per 20 MHz | NA | Macro BS: 43dBm per 20MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49dBm per 20MHz |
| Around 30GHz | 23 dBm per 20 MHz | Micro BS: 33 dBm per 20 MHz | NA | Micro BS: 33 dBm per 20 MHz | Macro BS: 33 dBm per 20 MHz |
| Note: BS Tx power scales up with bandwidth proportionally. Note: The maximum BS Tx power is 56dBm for outdoor and 33dBm for indoor. FFS: EIRP limit for 15GHz and 30GHz. FFS: whether to consider 3dB lower Tx power for 4GHz/7GHz than for 2GHz for the same deployment scenario. |  |  |  |  |  |

Any strong concerns, please leave them here.

| Company | Comments |
| --- | --- |
| Nokia | If we want to deploy a 192AE BS with AEG=8dBi at 2.5GHz (n41) with aggregate (sum EIRP across both polarizations) 88dBm/100MHz max DL EIRP then we need 50.2dBm per 20MHz max conducted tx power. This would match 3.45GHz UMa coverage for 500m ISD grid. Hence, we need more than 49 dBm per 20MHz option in Urban macro, i.e. 50.2dBm per 20MHz. Same is true for Rural and Sub-urban macro @2.5GHz. At 15 GHz, it is not very clear why only 43 dBm is proposed. We should also be targeting in Urban Macro around 49 dBm per 20 MHz like SMa. |
| Qualcomm | For indoor 30GHz, we can add another option of 23dBm per 100MHz which is based on TR 38.830. According to the note, the maximum BS Tx power is 56dBm for outdoor and 33dBm for indoor. Then for macro with 49dBm per 20MHz, the total Tx power will be 49 + 11.7 = 60.7 dBm for 300MHz BW exceeding the maximum value. Therefore, we propose the following changes to the note. Note: BS Tx power scales up with bandwidth proportionally under the limitation of the maximum BS Tx power which is 56dBm for outdoor and 33dBm for indoor EIRP limit for other bands should also be considered. It is better to include the maximum EIRP limit in the table. |
| Ericsson2 | For 700MHz, suggest to replace “49 dBm per 20 MHz” with “52dBm dBm per 20 MHz”. This aligns better with the 4T4R antenna assumption for this carrier frequency that was previously agreed. Regarding the “Note: The maximum BS Tx power is 56dBm for outdoor and 33dBm for indoor.”, for FR2 (i.e., around 30GHz) we suggest this to be 41dBm. |

(FL2) Proposal 3.2.2-2rv1

For 6GR evaluation, the UE power class for system-level simulation is assumed as follows:

- Note: 23dBm is for both FDD and TDD. 26dBm and 29dBm (optional) are for TDD only.

- FFS: 31dBm is for CPE/FWA, or 35 dBm with EIRP <55 dBm is for CPE/FWA

| UE power class | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Sub-urban macro |
| --- | --- | --- | --- | --- | --- |
| Around 700MHz | NA | 23dBm | 23dBm | 23dBm | 23dBm |
| Around 2GHz | 23dBm, 26dBm | 23dBm, 26dBm | 23dBm, 26dBm | 23dBm, 26dBm | 23dBm, 26dBm |
| Around 4GHz | 23dBm, 26dBm | 23dBm, 26dBm | 23dBm, 26dBm | 23dBm, 26dBm | 23dBm, 26dBm |
| Around 7GHz | 23dBm, 26dBm and 29dBm | 23dBm, 26dBm and 29dBm | 23dBm, 26dBm and 29dBm | 23dBm, 26dBm and 29dBm | 23dBm, 26dBm and 29dBm |
| Around 15GHz | 23dB, 26dBm and 29dBm Note: EIRP should not exceed 43 dBm | 23dB, 26dBm and 29dBm Note: EIRP should not exceed 43 dBm | NA | 23dB, 26dBm and 29dBm Note: EIRP should not exceed 43 dBm | 23dBm, 26dBm, and 29dBm EIRP should not exceed 43 dBm |
| Around 30GHz | 23dB, 26dBm, 29dBm Note: EIRP should not exceed 43 dBm | 23dB, 26dBm, 29dBm Note: EIRP should not exceed 43 dBm | NA | 23dB, 26dBm, 29dBm Note: EIRP should not exceed 43 dBm | 23dBm, 26dBm, and 29dBm EIRP should not exceed 43 dBm |

Any strong concerns, please leave them here.

| Company | Comments |
| --- | --- |

#### (FL3-pending?) Proposal 3.2.2-1rv2

For 6GR evaluation, the total transmit power per BS for system-level simulation is assumed as follows:

| Total transmit power per BS | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Sub-urban macro |
| --- | --- | --- | --- | --- | --- |
| Around 700MHz | NA | Macro BS: - Option1: 44 dBm per 20 MHz - Option2: 49 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 2GHz | 24 dBm per 20 MHz | Macro BS: - Option1: 44 dBm per 20 MHz - Option2: 46 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: - Option1: 49 dBm per 20 MHz - Option2: 46 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 4GHz | 24 dBm per 20 MHz | Macro BS: - Option1: 44 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: - Option1: 49 dBm per 20 MHz - Option2: 44 dBm per 20 MHz - Option3: 46 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 7GHz | 24 dBm per 20 MHz | Macro BS: - Option1: 44 dBm per 20 MHz - Option2: 43 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: - Option1: 49 dBm per 20 MHz - Option2: 43 dBm per 20 MHz - Option3: 46 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 15GHz | 23dBm per 20MHz | Macro BS: 40dBm per 20MHz Micro BS: 33 dBm per 20 MHz | NA | Macro BS: 49dBm per 20MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49dBm per 20MHz |
| Note: BS Tx power scales up with bandwidth proportionally under the limitation of the maximum BS Tx power is 56dBm for outdoor and 33dBm for indoor for the above carrier frequencies. |  |  |  |  |  |
| Around 30GHz | - Option1: 23 dBm per 20 MHz - Option2: 16dBm per 20MHz. | Micro BS: 33 dBm per 20 MHz | NA | Micro BS: 33 dBm per 20 MHz | Macro BS: 33 dBm per 20 MHz |
| Note: For around 30GHz, BS Tx power scales up with bandwidth proportionally under the limitation of EIRP 75dBm. |  |  |  |  |  |

#Not update from FL perspective, either take this one or discuss it in the plenary as Chair suggested#

Any strong concerns, please leave them here.

| Company | Comments |
| --- | --- |

#### (FL3) Proposal 3.2.2-2rv1

For 6GR evaluation, the UE power class for system-level simulation is assumed as follows:

- Note: 29dBm is not for smartphone.

  - It is for UE with more than one PA.

- FFS: 31dBm, or 35 dBm with EIRP <55 dBm

| UE power class | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Sub-urban macro |
| --- | --- | --- | --- | --- | --- |
| Around 700MHz | NA | 23dBm | 23dBm | 23dBm | 23dBm |
| Around 2GHz | 23dBm, 26dBm, 29dBm | 23dBm, 26dBm, 29dBm | 23dBm, 26dBm, 29dBm | 23dBm, 26dBm, 29dBm | 23dBm, 26dBm, 29dBm |
| Around 4GHz | 23dBm, 26dBm, 29dBm | 23dBm, 26dBm, 29dBm | 23dBm, 26dBm, 29dBm | 23dBm, 26dBm, 29dBm | 23dBm, 26dBm, 29dBm |
| Around 7GHz | 23dBm, 26dBm, 29dBm | 23dBm, 26dBm, 29dBm | 23dBm, 26dBm, 29dBm | 23dBm, 26dBm, 29dBm | 23dBm, 26dBm, 29dBm |
| Around 15GHz | 23dB, 26dBm, 29dBm | 23dB, 26dBm, 29dBm | NA | 23dB, 26dBm, 29dBm | 23dBm, 26dBm,29dBm |
| Around 30GHz | 23dB, 26dBm, 29dBm Note: EIRP should not exceed 43 dBm | 23dB, 26dBm, 29dBm Note: EIRP should not exceed 43 dBm | NA | 23dB, 26dBm, 29dBm Note: EIRP should not exceed 43 dBm | 23dBm, 26dBm, 29dBm EIRP should not exceed 43 dBm |

#Discussion points#

- Updated the note that 29dBm is not for smartphone to address some companies’ concern.

- Any other strong concern literarily??

Any strong concerns, please leave them here.

| Company | Comments |
| --- | --- |

## UE distribution

### Companies’ views

Companies’ views collected over post-122 email discussion were summarized in R1-2507292. In addition, the views included in the contribution are summarized as follows:

| Company | Views/proposals |
| --- | --- |
| CATT | Proposal 2: For the study on evaluation assumption in 6GR, for ISD, the UE speed and the number of UEs for each TRP, suggest the following typical values: - Indoor-hotspot: ISD is 20m; 100% Indoor (3km/h), 10 users per TRP; - Dense Urban: ISD is 200m; 80% indoor (3km/h), 20% outdoor (30km/h); 10 users per TRP; - Urban Macro: ISD is 500m; 80% indoor (3km/h), 20% outdoor (30km/h); 10 users per TRP; - Rural: ISD is 1732m; 50% outdoor vehicles (120km/h), 50% indoor (3 km/h); 10 users per TRP. |
| InterDigital | Parameters; Indoor Hotspot; Dense urban; Urban Macro; Rural; Suburban. UE distribution and UE speed; 100% indoor (3km/h) UE number per TRxP: avg. 10 users; 80% indoor (3km/h), 20% outdoor (30km/h) UE number per TRxP with single-layer only: Option 1: avg. 10 users Option 2: avg. 20 users; TR38.901 UMa 80% indoor (3km/h), 20% outdoor (30km/h) UE number per TRxP: avg. 10 users; TR38.901 RMa 50% indoor (3km/h), 50% outdoor (120km/h) UE number per TRxP: avg. 10 users; TR38.901 SMa 80% indoor (3km/h), 20% outdoor (40km/h) UE number per TRxP: avg. 10 users. |
| Huawei | Proposal 6: Multiple user densities per TRxP shall be considered for 6GR evaluation, e.g., chosen from [10, 20, 30, 50] for each of the deployment scenarios. |
| Samsung | Support Indoor/Outdoor ratio and UE speed according to deployment scenarios as in Table 2.3-2 Rural; Urban Macro, Dense Urban; Indoor Hotspot. Indoor; Ratio; 50%; Ratio; 80%; Ratio; 100%. UE speed; 3 km/h; UE speed; 3 km/h; UE speed; 3 km/h. Outdoor; Ratio; 50%; Ratio; 20%; Ratio; 0%. UE speed; 120 km/h; UE speed; 30 km/h; UE speed; -. |
| DOCOMO | Parameters; Indoor hotspot; Dense urban; Rural; Urban macro; Sub-urban macro. UE distribution and UE speed; Follow TR 38.914, i.e., 100% Indoor, 3km/h, [10] users per TRxP; Follow TR 38.914, i.e., Step1: Uniform/macro TRxP, 10 users per TRxP Step2: Uniform/macro TRxP + Clustered/micro TRxP, 10 users per TRxP [10] users per TRxP with single layer only 80% indoor (3km/h), 20% outdoor (30km/h); Follow TR 38.914, i.e., [15% outdoor vehicles (120km/h), 20% outdoor (3 km/h) and 70% indoor (3 km/h) 50% outdoor vehicles (120km/h), 50% indoor (3 km/h)] [10] users per TRxP; Follow TR 38.914, i.e., TBD [10] users per TRxP; Follow TR 38.914, i.e., [10% Outdoor pedestrian: 3km/h, 10% Outdoor in cars: 40km/h, 80% Indoor in houses: 3km/h] [10] users per TRxP. |

### Discussions

Summary on the views:

Based on collected views over post-122 email discussion summarized in R1-2507292 and the views mentioned in the contributions for this meeting, all the views are summarized as follows:

Dense Urban

| UE distribution and speed |  | Mentioned by |
| --- | --- | --- |
| UE number per TRxP | 10 | Huawei, vivo, OPPO, CATT, Ericsson, Interdigital (10/20), CMCC, DCM, MTK, Intel, Ofinno, Nokia, Futurewei, AT&T, ETRI |
|  | 30 | Huawei, Sony, Nokia (10/30/50), Futurewei, |
| UE location & speed | 20% outdoor incar 30km/h 80% indoor 3km/h | vivo (outdoor 30,60,120km/h), OPPO, CATT, Ericsson, ZTE, Qualcomm, CMCC, DCM, MTK, Intel, Ofinno, Futurewei, Apple, AT&T, ETRI |
|  | 20% outdoor incar 30km/h 60% outdoor 3km/h 20% indoor 3km/h | Ericsson, |
| O2I penetration loss | 20% high-loss 80% low-loss | Huawei, vivo, OPPO, CATT, Ericsson, Interdigital, ZTE, CMCC, DCM, Samsung, MTK, Sony, Ofinno, Nokia, Futurewei, Apple, ETRI |
|  | 50% high-loss 50% low-loss | Huawei, vivo, OPPO, ZTE, DCM, Nokia, |

Rural

| UE distribution and speed |  | Mentioned by |
| --- | --- | --- |
| UE number per TRxP | 10 | Huawei, vivo, OPPO, CATT, Ericsson, Interdigital, CMCC, DCM, {Tejas Networks, CEWiT, IIT Madras}, MTK, Sony, Intel, Ofinno, Nokia, Futurewei |
|  | 30 | Huawei, Nokia (optional 10/30/50), |
| UE location & speed | 50% outdoor incar 120km/h 50% indoor 3km/h | Huawei, vivo, OPPO, CATT, Ericsson, Interdigital, ZTE, Qualcomm, CMCC, DCM, MTK, Sony, Intel, Ofinno, Nokia, Futurewei |
|  | 15% outdoor incar 120km/h 15% outdoor 3km/h 70% indoor 3km/h | Ericsson, DCM, Ofinno, Nokia, |
|  | 40% Outdoor pedestrian 3km/h 20% Outdoor incar 30km/h or 60km/h 40% Indoor 0-3km/h | {Tejas Networks, CEWiT, IIT Madras} |
|  | 100% outdoor 0km/h | {Tejas Networks, CEWiT, IIT Madras} |
| O2I penetration loss | 100% low-loss | vivo, Ericsson, Interdigital, CMCC, DCM, {Tejas Networks, CEWiT, IIT Madras}, MTK, Sony, Ofinno, Nokia |
|  | 20% high-loss 80% low-loss | Futurewei, ZTE |
|  | 50% high-loss 50% low-loss | ZTE, |

Urban Macro

| UE distribution and speed |  | Mentioned by |
| --- | --- | --- |
| UE number per TRxP | 10 | Huawei, vivo, OPPO, Interdigital, CMCC, DCM, Xiaomi, MTK, Sony, Intel, Ofinno, Nokia, Futurewei, AT&T |
|  | 30 | Huawei, Nokia (10/30/50 per TRxP), |
| UE location & speed | 20% outdoor incar 30km/h 80% indoor 3km/h | Huawei, vivo (incar 30,60,120km/h), OPPO, Ericsson, Interdigital, ZTE, Qualcomm, CMCC, Xiaomi, MTK, Sony, Intel, Nokia, Futurewei, Apple |
| O2I penetration loss | 20% high-loss 80% low-loss | Huawei, vivo, CATT, Ericsson, Interdigital, ZTE, CMCC, DCM, Xiaomi, MTK, Sony, Ofinno, Nokia, Futurewei, Apple |
|  | 50% high-loss 50% low-loss | Huawei, vivo, CATT, ZTE, DCM, Intel, Ofinno, Nokia, Futurewei |

Suburban Macro

| UE distribution and speed |  | Mentioned by |
| --- | --- | --- |
| UE number per TRxP | 10 | Huawei, vivo, OPPO, Ericsson, Interdigital, CMCC, DCM, MTK, Ofinno, Nokia, Futurewei, Apple, AT&T |
|  | 30 | Huawei, Nokia (optional 30/50), Futurewei, |
| UE location & speed | 10% outdoor pedestrian 3km/h 10% outdoor incar 40km/h 80% indoor 3km/h | Huawei, vivo (incar 40,60,120km/h), Ericsson, ZTE, Qualcomm, DCM, MTK, AT&T |
|  | 20% outdoor 40km/h 80% indoor 3km/h | OPPO, Interdigital, Intel, Ofinno, Nokia, Apple, |
| O2I penetration loss | 100% low-loss | Huawei, vivo, CMCC, DCM, Ofinno, |
|  | 20% low-loss 80% low-loss A | Ericsson, |
|  | 100% low-loss A | Interdigital, Intel, Nokia |
|  | 5% high-loss 20% low-loss 75% low-loss A | Ericsson, |
|  | 20% high-loss 80% low-loss | ZTE, MTK, Futurewei, Apple |
|  | 50% high-loss 50% low-loss | ZTE, Futurewei, |

Round-1 discussions:

(FL1) Proposal 3.3.2

For 6GR evaluation, the UE distribution and UE speed for system-level simulation is assumed as follows:

| Parameters | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- | --- | --- |
| UE distribution and UE speed | 10 users per TRxP. 100% Indoor, 3km/h | Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per TRxP is [10, 30]. 80% indoor (3km/h); 20% outdoor(30km/h). | Single layer: Uniform/macro TRxP UE number per TRxP is [10, 30]. 50% indoor (3km/h); 50% outdoor(120km/h). | Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per TRxP is [10, 30]. [80% indoor (3km/h); 20% outdoor(30km/h).] | Single layer: Uniform/macro TRxP UE number per TRxP is [10, 30]. 10% Outdoor pedestrian: 3km/h; 10% Outdoor in cars: 40km/h; 80% Indoor in houses: 3km/h. |

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| vivo | Considering high speed support is one particular point mentioned in ITU 6G requirement, we think it is better for 3GPP to include higher UE speeds in its evaluations. Hence we suggest to add more higher speeds (e.g., 60km/h, 120 km/h) for outdoor UEs and let companies report the used UE speeds. |
| ZTE | The UE number for two-layer layout is related to the number of micro cells. For example, if there are 9 micro BSs per macro BS, having 30 UEs per TPxP seems too high. |
| MediaTek | For Urban Macro, single layer should be baseline and two layers should be optional for evaluation. |
| NTT DOCOMO | For UE distribution and UE speed, RAN1 can refer the RANp decision (i.e., TR 38.914), and no update from 38.914 is needed. Especially, based on the current RANp status, we prefer the UE number per TRxP as 10. |
| Samsung | We are fine with UE distribution for InH, Dense Urban, Rural and Urban Macro. |
| Qualcomm | The UE number per TRxP will be dependent on the used traffic model and may not be listed here. For UMa, we propose to use the same assumption as before (i.e., 80% indoor and 20% outdoor) and remove the bracket. |

Round-2 Discussions

(FL2) Proposal 3.3.2-rv1

For 6GR evaluation, the UE distribution and UE speed for system-level simulation is assumed as follows:

- UE number per TRxP will be dependent on the used traffic model. Other values may also be considered in the future evaluations.

| Parameters | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- | --- | --- |
| UE distribution and UE speed | 10 users per TRxP. 100% Indoor, 3km/h | Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per TRxP is [10, 30]. 80% indoor (3km/h); 20% outdoor(30km/h). | Single layer: Uniform/macro TRxP UE number per TRxP is [10, 30]. 50% indoor (3km/h); 50% outdoor(120km/h). | Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per TRxP is [10, 30]. 80% indoor (3km/h); 20% outdoor(30km/h). | Single layer: Uniform/macro TRxP UE number per TRxP is [10, 30]. 10% Outdoor pedestrian: 3km/h; 10% Outdoor in cars: 40km/h; 80% Indoor in houses: 3km/h. |

Any strong concerns, please leave them here.

| Company | Comments |
| --- | --- |
| Nokia | Fixing the number of UEs per cell to 10 or 30 as suggested by the FL primarily makes sense for cases with Full Buffer traffic models, while it makes less sense for more realistic traffic like e.g. XR. For the more realistic traffic models, the number of UEs per cell is varied depending on how much traffic the system is able to carry. For the full buffer cases, we recommend having 30 UEs as the default to represent high-load cases. |
| Tejas Networks | For Rural, instead of 50% indoor (3km/h); 50% outdoor(120km/h). Study 40% indoor (3km/h), 10% outdoor (3 Km/h), [20-30] % outdoor with 30-60 km/h, [20-30]% outdoor(120km/h). |

#### (FL3) Proposal 3.3.2-rv2

For 6GR evaluation, the UE distribution and UE speed for system-level simulation is assumed as follows:

- UE number per TRxP will be dependent on the used traffic model. Other values can also be considered in the future evaluations.

- Other velocities and/or other outdoor/indoor ratio will be dependent on the used traffic model or use cases. Other values can also be considered in each of other topics.

| Parameters | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- | --- | --- |
| UE distribution and UE speed | 10 users per TRxP. 100% Indoor, 3km/h | Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per TRxP is [10, 30]. Opt1: 80% indoor (3km/h); 20% outdoor(30km/h). Opt2: 40% indoor (3km/h) 40% outdoor (3km/h) 20% outdoor (30km/h) | Single layer: Uniform/macro TRxP UE number per TRxP is [10, 30]. Opt1: 50% indoor (3km/h); 50% outdoor(120km/h). Opt2: 20% indoor (3km/h) 40% outdoor (60km/h) 40% outdoor (120km/h) | Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per TRxP is [10, 30]. 80% indoor (3km/h); 20% outdoor(30km/h). | Single layer: Uniform/macro TRxP UE number per TRxP is [10, 30]. Opt1: 10% Outdoor pedestrian: 3km/h; 10% Outdoor in cars: 40km/h; 80% Indoor in houses: 3km/h. Opt2: |

#Discussion points#

- AT&T wanted to add one more option for Sma? What’s proposal for the Opt2?

- If people just have strong concern that it should be discussed in plenary first, please indicate it at the very beginning, so we would not have wasted everybody’s time on this procedure-wise problem!

Any strong concerns, please leave them here.

| Company | Comments |
| --- | --- |

## Other parameters

The other less controversial parameters are summarized based on the R1-2507292 capturing the companies’ views collected over post-122 email discussion.

### Discussions

Round-1 Discussions

(FL1) Proposal 3.4.1

For 6GR evaluation, the following are assumed for system-level simulation:

| Parameters | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- | --- | --- |
| ISD | 20m, equivalent to 12TRxPs per 120m x 50m | Macro layer: 200m | ISD 1: 1732m ISD 2: 5000m | Macro: 500m | ISD 1: 1299m ISD 2: 1732m |
| BS antenna height | 3m | 25m for macro cells and 10m for micro cells | 35 m | Macro: 25m | Alt 1: 35m Alt 2: 25m |
| BS noise figure | Around 7GHz and below: 5dB Around 15GHz and above: 7dB |  |  |  |  |
| UE antenna height | TR38.901 Indoor-Office Table 7.2-2 | TR38.901 UMi/UMa Table 7.2-1 | TR38.901 RMa Table 7.2-3 | TR38.901 UMa Table 7.2-1 | TR38.901 SMa Table 7.2-5 |
| UE Receiver | MMSE-IRC as the baseline |  |  |  |  |
| UE Power control parameter for UL | Company report |  |  |  |  |
| Channel model | TR 38.901 v19.1.0 Indoor-Office | TR 38.901 v19.1.0 UMa/UMi | TR 38.901 v19.1.0 RMa | TR 38.901 v19.1.0 UMa | TR 38.901 v19.1.0 SMa |
| Numerology | 15kHz SCS for FDD, 30kHz SCS for TDD |  |  |  |  |
| Scheduling | Proportional fairness (PF) |  |  |  |  |
| Inter-cell interference model | Explicitly and realistically modelled |  |  |  |  |
| [Inter-cell interference estimation model] | Alt 1: Ideal, calculated by ground truth channel matrix Alt 2: Realistic model, Company report, e.g., Wishart distribution-based model; retain only diagonal elements of interference Cov. Matrix. |  |  |  |  |
| Channel estimation assumption | Alt 1: Ideal Alt 2: Realistic, company report, e.g., apply gauss noise to real channel matrix, or random |  |  |  |  |
| Feedback assumption | Alt 1: Ideal Alt 2: Realistic, company report, e.g., consider feedback delay and overhead; codebook; |  |  |  |  |
| O2I penetration loss (X% high loss, Y% low loss) | NA | Two options are supported: Option 1: 80% low loss, 20% high loss; Option 2: 50% low loss, 50% high loss | 100% low loss | Two options are supported: Option 1: 80% low loss, 20% high loss; Option 2: 50% low loss, 50% high loss | 100% Low-loss A Model as TR 38.901 |
| Mechanic tilt | 180° in GCS (pointing to the ground) as baseline. Company can report if not follow the baseline. | 90° in GCS (pointing to the ground) as baseline. Company can report if not follow the baseline. | 90° in GCS (pointing to the ground) as baseline. Company can report if not follow the baseline. | 90° in GCS (pointing to the ground) as baseline. Company can report if not follow the baseline. | Baseline: 95 degrees for ISD = 1299m; 92 degrees for ISD = 1732m; Company can report if not follow the baseline. |
| Electronic tilt | 90° in LCS as baseline. Company can report if not follow the baseline. | Company report, e.g. 105 or 102 degrees in LCS. | Company report, e.g. 96 degree in LCS. | Company report, e.g. 102 degrees in LCS. | Company report, e.g. 102 degrees in LCS. |
| Handover margin (dB) | 0dB as baseline. 1dB and 3dB as optional configuration. | 0dB as baseline. 1dB and 3dB as optional configuration. | 0dB as baseline. 1dB and 3dB as optional configuration. | 0dB as baseline. 1dB and 3dB as optional configuration. | 0dB as baseline. 1dB and 3dB as optional configuration. |
| UE attachment | Based on RSRP from BS port 0 |  |  |  |  |
| Wrapping around method | No wrapping around | Geographical distance-based wrapping | Geographical distance-based wrapping | Geographical distance-based wrapping | Geographical distance-based wrapping |
| Multi-TRP operation, e.g., ideal or non-ideal backhaul/sync | NA | Alt 1: ideal backhaul/sync Alt 2: non-ideal backhaul/sync, company report |  |  |  |

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| vivo | Comment #1 It seems UE receiver noise figure is missing. For UE NF, we think it is better to make a decision on 7dB vs 9dB in RAN1. Comment #2 Some typos for Mechanic tilt in the Dense urban/Rural/Urban macro: 90° in GCS (pointing to horizontal direction) Further, the Mechanic tilt in Dense urban/Rural/Urban macro is same that of 90° in GCS as baseline. While in Suburban Macro scenario, the baseline is 92 or 95 degrees without explanation of the Coordinate System, we think it should be further clarified. |
| ZTE | We have the following comments: - The Numerology parameter is closely related to carrier frequency. Since 6GR evaluations will span multiple frequency ranges, this assumption should remain open and be further refined, e.g., in line with the discussion under AI 11.3 - For UE receiver, we support using MMSE-IRC receiver as the baseline, and we also support considering more advanced receivers such as R-ML, which are already adopted in current networks. - The Multi-TRP operation parameter should consider different TRP types. For example, in a two-layer layout, micro TRPs may be passive nodes used for signal reflection/enhancement, which are different from the active TRPs deployed in the macro layer. We believe such a deployment is beneficial for energy saving and network cost. We suggest the following updates to the parameters Numerology; In line with AI 11.3. UE receiver; MMSE-IRC or R-ML Receiver. Multi-TRP operation; TRP type - Single layer: active TRP only - Two layers: macro layer: active TRP micro layer: passive TRP for reflecting/enhancing signal, e.g., reconfiguration intelligent surface Sync model - Ideal - Non-ideal, company report Backhaul - Ideal - Non-ideal. |
| NTT DOCOMO | Regarding the Comment #1 mentioned by vivo, we also think UE noise figure should be determined. We propose the following: - Around 7GHz and below: 9 dB - Around 15GHz and above: 10 dB Other parameters are generally fine for us. |
| Samsung | Regarding interference estimation model, we should support realistic model for SLS evaluation to reflect more practical BS receiver and evaluate the performance with more antenna ports (especially, UL). |
| Qualcomm | For UE receiver, both MMSE-IRC and R-ML should be considered. Advanced receiver like R-ML has been widely used in the real-world UE devices and should be included also for evaluation. For numerology, we prefer company to report. At least for 30GHz, 120GHz SCS should be assumed. For penetration loss, we support using 50% low and 50% high for UMa and Umi. For mechanic tilt for UMa and Umi, if we assume it points to the ground then tilt would be 0 degree in GCS not 90 deg. For mTRP, we think non-ideal backhaul and sync should be assumed. |

Round-2 Discussions

(FL2) Proposal 3.4.1-rv1

For 6GR evaluation, the following are assumed for system-level simulation:

| Parameters | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- | --- | --- |
| ISD | 20m, equivalent to 12TRxPs per 120m x 50m | Macro layer: 200m | ISD 1: 1732m ISD 2: 5000m | Macro: 500m | ISD 1: 1299m ISD 2: 1732m |
| BS antenna height | 3m | 25m for macro cells and 10m for micro cells | 35 m | Macro: 25m | Alt 1: 35m Alt 2: 25m |
| BS noise figure | Around 7GHz and below: 5dB Around 15GHz and above: 7dB |  |  |  |  |
| UE antenna height | TR38.901 Indoor-Office Table 7.2-2 | TR38.901 UMi/UMa Table 7.2-1 | TR38.901 RMa Table 7.2-3 | TR38.901 UMa Table 7.2-1 | TR38.901 SMa Table 7.2-5 |
| UE noise figure | Around 7GHz and below: 9dB (baseline performance), 7dB (high performance) Around 15GHz and above: 13dB (baseline performance), 10dB (high performance) |  |  |  |  |
| UE Receiver | MMSE-IRC as the baseline, or R-ML Receiver |  |  |  |  |
| UE Power control parameter for UL | Company report |  |  |  |  |
| Channel model | TR 38.901 v19.1.0 Indoor-Office | TR 38.901 v19.1.0 UMa/UMi | TR 38.901 v19.1.0 RMa | TR 38.901 v19.1.0 UMa | TR 38.901 v19.1.0 SMa |
| Numerology | In line with AI 11.3 |  |  |  |  |
| Scheduling | Proportional fairness (PF) |  |  |  |  |
| Inter-cell interference model | Explicitly and realistically modelled |  |  |  |  |
| [Inter-cell interference estimation model] | Alt 1: Ideal, calculated by ground truth channel matrix Alt 2: Realistic model, Company report, e.g., Wishart distribution-based model; retain only diagonal elements of interference Cov. Matrix. |  |  |  |  |
| Channel estimation assumption | Alt 1: Ideal Alt 2: Realistic, company report, e.g., apply gauss noise to real channel matrix, or random |  |  |  |  |
| Feedback assumption | Alt 1: Ideal Alt 2: Realistic, company report, e.g., consider feedback delay and overhead; codebook; |  |  |  |  |
| O2I penetration loss (X% high loss, Y% low loss) | NA | Two options are supported: Option 1: 80% low loss, 20% high loss; Option 2: 50% low loss, 50% high loss | 100% low loss | Two options are supported: Option 1: 80% low loss, 20% high loss; Option 2: 50% low loss, 50% high loss | 100% Low-loss A Model as TR 38.901 |
| Mechanic tilt | 180° in GCS (pointing to the ground) as baseline. Company can report if not follow the baseline. | 90° in GCS (pointing to horizontal direction) as baseline. Company can report if not follow the baseline. | 90° in GCS (pointing to horizontal direction) as baseline. Company can report if not follow the baseline. | 90° in GCS (pointing to horizontal direction) as baseline. Company can report if not follow the baseline. | Baseline: 95° in GCS (pointing to horizontal direction) for ISD = 1299m; 92° in GCS (pointing to horizontal direction) for ISD = 1732m; Company can report if not follow the baseline. |
| Electrical tilt | 90° in LCS as baseline. Company can report if not follow the baseline. | Company report, e.g., 105° or 102° in LCS. | Company report, e.g., 96° in LCS. | Company report, e.g., 102° in LCS. | Company report, e.g., 102° in LCS. |
| Handover margin (dB) | 0dB as baseline. 1dB and 3dB as optional configuration. | 0dB as baseline. 1dB and 3dB as optional configuration. | 0dB as baseline. 1dB and 3dB as optional configuration. | 0dB as baseline. 1dB and 3dB as optional configuration. | 0dB as baseline. 1dB and 3dB as optional configuration. |
| UE attachment | Based on RSRP from BS port 0 |  |  |  |  |
| Wrapping around method | No wrapping around | Geographical distance-based wrapping | Geographical distance-based wrapping | Geographical distance-based wrapping | Geographical distance-based wrapping |
| Multi-TRP operation, e.g., ideal or non-ideal backhaul/sync | NA | Alt 1: ideal backhaul/sync Alt 2: non-ideal backhaul/sync, company report |  |  |  |

Any strong concerns, please leave them here.

| Company | Comments |
| --- | --- |
| Nokia | We support having both realistic interference covariance matrix estimation and channel estimation as the baseline. Such assumptions has been adopted also for past LTE studies, and we see no reasons for reverting back to ideal assumptions as it is well-known that especially the realistic estimation of the interference covariance matrix is an important performance determining effects. As for the feedback assumptions: For DL user-plane simulations, we suggest that UL feedback such as CSI and ACK/NACK is realistically modelled so it is only sent in UL slots, and with realistic modeling of timings in line with UE processing times (e.g. taking the 5G NR processing capabilities 2 as reference to start with). For cases with RLC AM and/or TCP, feedback such RLC status reports and TCP-ACK shall be also modeled in coherence with TDD frame structure and related processing latencies. Similarly, for UL user plane simulations, there must be realistic modeling of the availability of SRS, SR, BSR, DSR, etc. |
| Interdigital | For Sma scenario, vegetation assumption is missing. We think we can use 0% vegetation assumption. |
| Ericsson2 | Handover margin should be 3 dB as a default assumption; 0 dB assumes perfect cell selection, which is not so realistic. Tilt depends on the antenna system and beamforming assumptions, and should be defined appropriately for the feature / MIMO design. Regarding channel estimation methods allowing direct/explicit simulation of RS reception should be included, as this is a straightword way to address realistic channel estimation: “Alt 2: Realistic, company report, e.g., direct/explicit RS estimation, apply gauss noise to real channel matrix, or random” Multi-TRP assumptions can be left to MIMO agenda item, so that row can be deleted. |
| ZTE | Regarding BS antenna height, the BS height of micro cells should also be set in Urban Macro scenario. Multi-TRP operation should also be considered for the indoor hotspot scenario, since indoor environments typically suffer from more severe interference, and multi-TRP schemes (e.g., CJT) can effectively suppress such interference. In addition, the backhaul and synchronization assumptions should be discussed separately. We support ideal backhaul + non-ideal synchronization. Finally, different TRP types should be considered. In a two-layer layout, micro TRPs could be passive nodes used for signal reflection/enhancement, which are different from the active TRPs deployed in the macro layer. We believe such a deployment is beneficial for energy saving and network cost. The simulation assumptions can be updated as follows: BS antenna height; Urban macro 25m for macro cells and 10m for micro cells. Multi-TRP operation; TRP type - Active TRP - Passive TRP for reflecting/enhancing signal, e.g., NCR, reconfiguration intelligent surface (RIS). Note: Passive TRPs are used for two-layer layout Sync model - Ideal - Non-ideal, company report Backhaul - Ideal - Non-ideal. |

#### (FL3) Proposal 3.4.1-rv1

For 6GR evaluation, the following are assumed for system-level simulation:

| Parameters | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- | --- | --- |
| ISD | 20m, equivalent to 12TRxPs per 120m x 50m | Macro layer: 200m | ISD 1: 1732m ISD 2: 5000m | Macro: 500m | ISD 1: 1299m ISD 2: 1732m |
| BS antenna height | 3m | 25m for macro cells and 10m for micro cells | 35 m | 25m for macro cells and 10m for micro cells | Alt 1: 35m Alt 2: 25m |
| BS noise figure | Around 7GHz and below: 5dB Around 15GHz and above: 7dB |  |  |  |  |
| UE antenna height | TR38.901 Indoor-Office Table 7.2-2 | TR38.901 UMi/UMa Table 7.2-1 | TR38.901 RMa Table 7.2-3 | TR38.901 UMa Table 7.2-1 | TR38.901 SMa Table 7.2-5 |
| UE noise figure | Around 7GHz and below: 9dB (baseline performance), 7dB (high performance) Around 15GHz and above: 13dB (baseline performance), 10dB (high performance) |  |  |  |  |
| UE Receiver | MMSE-IRC as the baseline, R-ML Receiver as optional |  |  |  |  |
| UE Power control parameter for UL | Company report |  |  |  |  |
| Channel model | TR 38.901 v19.1.0 Indoor-Office | TR 38.901 v19.1.0 UMa/UMi | TR 38.901 v19.1.0 RMa | TR 38.901 v19.1.0 UMa | TR 38.901 v19.1.0 SMa, 0% vegetation. |
| Numerology | In line with AI 11.3 |  |  |  |  |
| Scheduling | Proportional fairness (PF) |  |  |  |  |
| Inter-cell interference model | Explicitly and realistically modelled |  |  |  |  |
| [Inter-cell interference estimation model] | Alt 1: Ideal, calculated by ground truth channel matrix Alt 2: Realistic model, Company report, e.g., Wishart distribution-based model; retain only diagonal elements of interference Cov. Matrix. |  |  |  |  |
| Channel estimation assumption | Alt 1: Ideal for benchmark Alt 2: Realistic, company report, e.g., direct/explicit RS estimation, apply gauss noise to real channel matrix, or random |  |  |  |  |
| Feedback assumption | Alt 1: Ideal Alt 2: Realistic, company report, e.g., consider feedback delay and overhead; codebook; |  |  |  |  |
| O2I penetration loss (X% high loss, Y% low loss) | NA | Two options are supported: Option 1: 80% low loss, 20% high loss; Option 2: 50% low loss, 50% high loss | 100% low loss | Two options are supported: Option 1: 80% low loss, 20% high loss; Option 2: 50% low loss, 50% high loss | 50% Low-loss A 50% Low-loss Model as TR38.901. |
| Mechanic tilt | 180° in GCS (pointing to the ground) for performance calibration Company can report other values for evaluations. | 90° in GCS (pointing to horizontal direction) for performance calibration Company can report other values for evaluations. | 90° in GCS (pointing to horizontal direction) for performance calibration Company can report other values for evaluations. | 90° in GCS (pointing to horizontal direction) for performance calibration. Company can report other values for evaluations. | 90° in GCS (pointing to horizontal direction) for performance calibration Company can report other values for evaluations. |
| Electrical tilt | 90° in LCS for performance calibration. Company can report other values for evaluations. | 90° in LCS for performance calibration. Company can report other values for evaluations. | 90° in LCS for performance calibration. Company can report other values for evaluations. | 90° in LCS for performance calibration. Company can report other values for evaluations. | 90° in LCS for performance calibration. Company can report other values for evaluations. |
| Handover margin (dB) | 3dB as baseline. 1dB and 0dB as optional configuration. | 3dB as baseline. 1dB and 0dB as optional configuration. | 3dB as baseline. 1dB and 0dB as optional configuration. | 3dB as baseline. 1dB and 0dB as optional configuration. | 3dB as baseline. 1dB and 0dB as optional configuration. |
| UE attachment | Based on RSRP from BS port 0 |  |  |  |  |
| Wrapping around method | No wrapping around | Geographical distance-based wrapping | Geographical distance-based wrapping | Geographical distance-based wrapping | Geographical distance-based wrapping |
| Multi-TRP operation, e.g., ideal or non-ideal backhaul/sync | Backhaul: ideal or non-ideal; sync: ideal or non-ideal; Company reports the assumptions of the non-ideal backhaul/non-ideal sync. |  |  |  |  |

#No update, it’s the same version as discussed in off-offline#

Any strong concerns, please leave them here.

| Company | Comments |
| --- | --- |

## Other views in TDoc

| Company | Views/proposals |
| --- | --- |
| Futurewei | Proposed to study multi-cell / multi-TRP deployments with nonideal backhaul or imperfect network synchronization. |
| Nokia | Proposed to include realistic spatial interference covariance estimation for multi-antenna receivers. For system-level simulations, we suggest adopting the well-known Wishart model. |
| vivo | Proposed a table of system level simulation assumptions as the starting point for 6GR evaluation for 7GHz including UE antenna modelling, UE transmit power, etc. Discussed in details and proposed to support the urban grid scenarios for 6GR evaluations. |
| CMCC | Attached a spreadsheet including SLS parameters for the scenarios of interest. Proposed simulation assumptions for the high speed scenario. |
| ZTE | Discussed the co-frequency networking for two-layer deployment and multi-layer heterogenous network with assisting node. Proposed to consider multi-TRP operation with CJT (targeting for FR1/around-7GHz) and NCJT transmission (targeting FR2/around 30GHz under ideal/non-ideal backhaul and ideal/non-ideal sync (in terms of frequency-domain and time-domain) Proposed to consider near-field and SNS channel models in TR38.901. Proposed sensing related scenarios, e.g., indoor-factory, highway. |
| CATT | Proposed that multi-TRP operation with ideal backhaul/ideal sync or ideal backhaul/non-ideal sync should be considered in 6GR evaluation. |
| AT&T | Proposed for 6GR evaluations, realistic assumptions are used for interference modeling, channel estimation, and traffic modelling. Proposed for SLS evaluations, the ISD for SMa deployment scenarios is 1732m. |
| Xiaomi | Proposed to support release 19 channel modelling including near-field and spatial non-stationarity in the evaluations involved in 6GR, esp. for the cases with larger-scale antenna arrays. |
| OPPO | Proposed that the channel model output of 7-24GHz channel modeling in Rel-19 can be applied for 6G evaluation. |
| Huawei | Discussed and proposed to add the urban grid scenario for sensing and communication. |
| Intel | Proposed to include UE-side spatial non-stationarity modelling as per section 7.6.14.2 in R19 TR 38.901 into 6GR evaluation assumptions. Proposed to optionally include Rural scenarios with ISD ~3 km and ~5 km, potentially for specific frequency bands only. |
| LGE | Discussed and proposed urban grid and highway should be studied so need to remove the brackets. |
| Tejas | Proposed to study Isolated Macro cell with cluster UE drop as one of the deployment scenarios for Rural in 6G. Proposed to add indoor-factor as one of scenarios. |
| Sony | Proposed that High-density of cells in indoor hotspot (e.g. 12/36 TRPs) and dense urban (e.g. 6 or 9 micro TRPs per macro TRP) should be considered for the evaluation of spectrum efficiency and user-experienced data rates. |
| T-Mobile | Proposed to integrate the FWA scenarios, parameters, and evaluation methodologies described herein into FWA evaluations done in RAN1. |
| Ericsson | Proposed to define simulation assumptions that support FWA service operation in RAN1, using scenarios already present in 38.914 and [11] as a starting point. Proposed to defer discussion of values of mechanical and electrical tilt, defining them later according to factors including scenario and base station antenna configuration, etc. |

# Traffic models

## New model 1- AI/ML services

### Companies’ views

| Company | Views/proposals |
| --- | --- |
| Futurewei | - Regarding the traffic model(s) for 6GR AI/ML services and traffic modelling for evaluations related to immersive communication services, while RAN1 is waiting for SA4 inputs, RAN1 can carry out study in the meantime. To be able to accurately reflect practical traffic for AI/ML services and immersive communication services, RAN1 study should be based on realistic traffic data/statistics, e.g., current and projected AI/ML token statistics, realistic traffic data provided by advanced XR vendors and haptics service providers, and so on. |
| Nokia | Proposal 15: RAN1 to prioritize reaching agreements on FTP traffic modes within RAN1, while waiting for the feedback from SA4 and other WGs on traffic models for AI, XR, Haptic feedback, etc. Proposal 25: Once clear guidance is received from SA4, RAN1 can come back to discuss modeling of AI traffic and potential extensions of XR traffic. Aspects related to e.g., “importance of tokens”, “potential encapsulation of tokens with other traffic”, “scheduling priority of tokens” are up to RAN2 to decide. |
| vivo | Proposal 9: For AI/ML service traffic model, further study the interaction scenarios and service types. Proposal 10: Further study other token communication traffic beyond LLM scenarios, e.g., AIML based JSCCM for audio codec. |
| CMCC | Proposal 3: For the traffic model(s) for 6GR AI/ML services, Take Option-1a as starting point: • Option-1a: The model is parameterized by Token, e.g., Token size, Token arrival rate, and Token delay budget. • Token is the minimum unit of data generated in the application layer. Proposal 4: For the traffic model(s) for 6GR AI/ML services, consider following aspects to model token: 1) The frame arrives with a fixed periodicity, which is given by the inverse of the frame rate. One frame consists of several fixed numbers of tokens. 2) One frame consisting of several numbers of tokens can be seen as one PHY layer packet. 3) Token arrival rate is the average number of tokens per second arrived for transmission. 4) The size of token is fixed value. 5) Token delay budget is a fixed value of time budget for a token to be transmitted over the air in uplink or downlink. |
| ZTE | Proposal 4-2-1: When discussing token size, it is necessary to clarify the input data type (e.g., text, image) and the AI/ML task type (e.g., general-purpose tasks, specific-purpose tasks). Proposal 4-2-2: Regarding traffic model(s) for generative AI, option 1b is supported with the following modification. - The traffic model consists of the following 3 service types: image-based GenAI, video-based GenAI, and chatbot. Each service type adopts FTP model 3 for simulation, which is characterized by packet size, arrival rate, latency requirement, and reliability requirement. - The parameter values for each service type are fixed and defined in Table 4-2-1. - FFS: Whether to select a single service type for simulation, or a combination of the 3 services. - FFS: If a combination of services is selected, whether each UE is assigned with mixed services. Proposal 4-2-3: Study the traffic model(s) for AI/ML model training/inference in 6GR evaluation. - The traffic model consists of the following 3 service types: training data collection, inference result transmission, and model download. Each service type adopts FTP model 3 for simulation, which is characterized by packet size, arrival rate, latency requirement, and reliability requirement. - The parameter values for each service type are fixed and defined in Table 4-2-2. - FFS: Whether to select a single service type for simulation, or a combination of the 3 services. - FFS: If a combination of services is selected, whether each UE is assigned with mixed services. |
| CATT | Proposal 8: The discussion on new traffic models should be postponed until the discussion of the common evaluation assumptions is completed. |
| NEC | Proposal 5: For AI services, adopt a flexible traffic model that differentiates between bursty and streaming uplink prompts, defined by the following parameters: • Uplink Traffic Profile, configured as either: o For bursty inputs (e.g., text, images): An Uplink Prompt Size. o For streaming inputs (e.g., real-time video): An Uplink Service Data Rate with periodic packet generation. • Token Generation Rate (downlink). • Token Size. • Token Success Rate. • E2E Delay Budget. |
| AT&T | Observation 1: Option-1a is not realistic for most of the AI/ML traffic seen in cellular networks, based on Generative AI/LLM services. Proposal 4: For traffic modeling of AI/ML services for 6GR evaluations, do not pursue option-1a. Observation 2: Reusing or extending either the FTP-3 model or the XR model for GenAI traffic fails to capture LLM characteristics of the GenerativeAI traffic. Proposal 5: For traffic modeling of AI/ML services for 6GR evaluations, do not pursue option-1c. Observation 3: Option-1b can be used to model all types of GenAI-based LLM traffic expected to be prevalent in 6G networks. Observation 4: Option-1b can be designed to account for the statistical distributions of the different phases of an LLM session, subsequently yielding realistic traffic behavior. Proposal 6: For traffic modeling of AI/ML services for 6GR evaluations, use Option-1b. |
| OPPO | Proposal 10：A new traffic model for AI service (e.g., token communication) should be introduced in 6G, at least including following aspects: - Introduce token communication as a new traffic type for RAN1 evaluation - Introduce token size, token packet size, token packet arrival, token packet success rate, assumptions on token importance Observation 1: Error in just one token (even within a tolerated token error rate of a target service) can trigger the retransmission of the entire TB or CBG, which is inefficient. Observation 2: To efficiently support AI services and token communications, studies and evaluations on framework for token communication evaluation are needed in RAN1. Proposal 11：Regarding the requirement for token communication, at least following aspects should be introduced: - Assumptions on downstream service - Downstream service requirement - Transmission requirement to guarantee the service requirement - Requirements for token communication (e.g., for token error rate) - Methodology for token error identification Proposal 12: A typical token communication traffic model is shown as Table 4: Table 4: A typical token communication traffic model for RAN1 evaluation New traffic type; Token communication, e.g., tokenized image. Token based Packet (Tokenized image); Original source data size (image); 256*256*3*8 bit. Tokenized source data size (tokenized image); 256*12 bit. Number of token per image; 256. Number of bits per token; 12. Tokenizer and Detokenizer model; Up to implementation, [e.g., by AI models like One-D-Piece]. Packet size Tokens to be transmitted in one packet; N image, N*256 tokens to be transmitted, i.e., N*256*12/8 Bytes FFS the value of N. Packet arrival; Case1: Packet arrival in Poisson distribution Case2: Packet arrival in Periodic distribution. Packet delay budget; The value of packet delay budget may vary for different services.. Packet success rate; [99% Packet success rate] The successful transmission of a packet is defined as the token success rate (e.g., by identify token errors within a packet, or token error rate is mapping to a BER threshold) within a packet meets the service-required threshold.. Assumptions on token importance; Case 1: tokens are treated without different importance Case 2: tokens are treated with different importance, e.g., with assumptions as below: - 256 tokens can be numbered from 0 to 255 in descending order of importance; - a lower number indicates higher criticality for information expression, while a higher number corresponds to detail information.. Requirement for token communication; Assumptions on downstream service; Case 1: a classification task, [ e.g., ImageNet dataset, with N images, belonging to M categories, FFS N, M, e.g., N=3000, M=1000] - FFS whether/how to define an aligned source data set - FFS whether/how to define an aligned classification algorithm/model Case 2: FFS others. Downstream service requirement; Top-5 accuracy > 90%. Requirements for token communication; X TER(token error rate)/BER(bit error rate)/BLER(block error rate) that could guarantee corresponded service requirement. FFS the value of X, e.g., 20% TER, or 5% BER. Y important tokens are selected for transmission that guarantee corresponded service requirement. FFS the value of Y, e.g., 25% tokens are selected based on their importance.. Methodology for token error identification; Methods for token level error identification are needed. |
| Huawei | Observation 1: It is Token success rate directly reflecting the AI/ML service quality and cannot be replaced by the packet success rate used by the XR traffic model. Proposal 13: The traffic model for AI/ML services can be derived based on the application layer simulation to extract the requirements of Token success rate (i.e., 1 – Token error rate), delay budget, Token arrival rate, and Token size from specific application layer models. Proposal 14: Model Option-1a (parameterized by Token arrival rate, Token size, Token success rate, and Token delay budget) should be used as the traffic model representing the AI/ML services, given only Token success rate has a direct impact on the quality of AI/ML services and it may be not the same as and cannot be represented by packet error rate. Correspondingly, Token arrival rate, Token size, and Token delay budget should be modelled together. Proposal 15: On top of Token related parameters, other parameters regarding packet arrival rate, packet success rate, and packet delay are not needed, assuming Tokens could be grouped together at application layer and considering the arrival rate, delay, and success rate can be defined based on Tokens. |
| Samsung | Observation: the traffic characteristics of representative AI/ML services are expected to be studied soon as future 6G work in the responsible working group - SA4. Proposal #16: - RAN1 should at least wait for SA4 response to make any possible decision on the traffic model for the evaluations of AI/ML services. Observation: XR traffic model can be considered as a starting point for studying AI/ML traffic model. Proposal #17: - If any, a traffic model for AI/ML services evaluation in RAN1 should be characterized by packetization parameters as convention, avoiding anything supposed to be unknown to PHY or unassociated with PHY aspects directly. - RAN1 strives for reusing or extending XR traffic model, provided similarities. |
| NVIDIA | Proposal 8: Consider the following enhancements to the existing XR model for performance evaluation during 6GR study for use case scenarios related to 6G services beyond immersive communication (e.g., AI/ML-based services) - - Packet generation: - Wide range of sizes, from <100 bytes (e.g., AI inference query) to ~100 MB (e.g., federated learning gradient updates). As one example, introduce a bimodal distribution to generate these two ranges of packet sizes. - Packet arrival rate: - Introduce burstiness and event-driven arrival (i.e., aperiodicity), enhancing the existing periodic/fixed packet arrival rate (e.g., ~60 fps for media). - Packet size deviation: - Instead of static percentage (e.g., standard deviation as ~10.5% of distribution mean), introduce AI-flow dependent separate deviation models (e.g., AI inference query vs. FL gradient updates). - Packet delay budget (PDB): - Define service-dependent PDB parameters, (i.e., different PDB parameters for different traffic flows, e.g., AI inference vs. gradient updates), enhancing existing fixed-per-service PDB for XR media (e.g., ~10ms for VR traffic model). Proposal 10: Study the feasibility of new token-based traffic model for performance evaluation during 6GR study for AI/ML based services based on token communications (e.g., GenAI applications). |
| Google | Proposal 1: Adopt the token-streamlined traffic model as the baseline for evaluating AI-specific use cases. Proposal 2: GenAI traffic properties as agreed in SA1/TR 22.870 can be used as baseline for token-streamlined traffic model in RAN1. |
| Ericsson | Generative AI applications are quite diverse and accordingly the traffic generated by such applications is also expected to be diverse. For example, increased use of real-time GenAI-driven video assistants and immersive interactions could significantly increase traffic (especially uplink) while other text/chat-style usage applications may have relatively less impact. From RAN1 perspective, Option-1c discussed in RAN1#122bis is a suitable starting point for the study on traffic modelling for such applications. The video traffic modeling in XR TR 38.838 could be used as a starting point to evaluate the impact of video traffic generated by such new applications. Potential modifications could be considered based on any input received for the LS sent in RAN1#122bis [12]. For other traffic types, the generic FTP Model1/FTP Model 3 with extensions could be used as a starting point. Proposal 4-3 - For the study on traffic models for generative AI services, for video related applications, use the video traffic modeling in XR TR 38.838 as starting point, and for other applications use the extensions being studied for FTP Model 1/FTP Model 3 as starting point. |
| Ofinno | Observation 1: There are large number of AL/ML use cases and services, which may lead to multiple AL/ML traffic models. Observation 2: The tokens being generated on application layer level will eventually be encapsulated into physical layer packets for transmission over a radio interface. Observation 3: From the perspective of physical layer evaluations of the AI/ML technology in 6GR, it is natural to consider Option-1b, e.g., by transforming tokens into physical layer packets. Observation 4: Depending on the input from SA4, Option-1c can also be considered for certain AL/ML services. |
| Qualcomm | Observation 1: Tokens cannot be transmitted directly in raw form over the network. Neither technically feasible nor resource and power efficient Observation 2: Tokens are typically encrypted and encapsulated within standards protocol layers, e.g., embedded in RLC/MAC frames for transmission over the air Observation 3: A traffic model parameterized by Token is not necessary. Proposal 8: The traffic model for tokenized generative AI applications can be effectively characterized using the general burst traffic model with mixed packet sizes • Bursts represent tokenized multimodal information and may arrive either periodically or according to a Poisson process. • Each token is mapped to a packet within the burst, and multiple tokens can be encapsulated into one packet, with its size being an integer multiple of the token size • Packet sizes within a burst can be uniform or variable. The packets within the burst arrive back-to-back with short inter-arrival time (e.g., 0.5-1msec) |
| DOCOMO | Proposal 4 - For the traffic model for AI/ML services, support Option-1c. - Option-1c: reusing or extending the FTP3/XR traffic model. First, we believe that traffic characteristic of “Token” would be different from parameter values of existing traffic model. Therefore, we are open to considering a traffic model that reflects the “Token” characteristic. On the other hand, the new parameters named “Token” should not be introduced, as “Token” itself is not visible in PHY layer. |

### Discussions

Background

The traffic model for AI/ML services (a representative AI/ML service is the generative AI) was discussed in the last meeting and LS was to SA4 requesting for potential input.

Meanwhile, RAN1 is discussing the following options for the model:

- Option-1a: The model is parameterized by Token, e.g., Token size, Token arrival rate, and Token delay budget.

- Option-1b: The model is characterized by the parameters of PHY layer packet, including e.g., packet size, arrival rates, latency requirement, reliability requirement, etc.

- Option-1c: reusing or extending the FTP-3/XR traffic model.

Observations from the submitted paper for this meeting

17 companies (Futurewei, Nokia, vivo, CMCC, ZTE, CATT, NEC, AT&T, OPPO, Huawei, Samsung, NVIDIA, Google, Ericsson, Ofinno, Qualcomm, DOCOMO) discussed the traffic model for AI/ML in the contributions to follow the agreement from the last meeting, and in particular,

- 12 companies (CMCC, ZTE, NEC, AT&T, OPPO, Huawei, NVIDIA, Google, Ericsson, Ofinno, Qualcomm, DOCOMO) further discussed and analysed the options or proposed other general ideas on how to model the traffic.

- A few companies (Nokia, CATT, Samsung) also suggested to wait for SA4 first. However, the 2 companies (Nokia, Samsung) also mentioned the potential extension of XR traffic model for RAN1’s study.

- 1 company (vivo) proposed to further study other token communication traffic beyond LLM scenarios, e.g., AI/ML based JSCCM for audio codec.

- 1 company (ZTE) proposed to study the traffic model(s) for AI/ML model training/inference.

- The support of the options among companies:

  - Option-1a: CMCC, NEC, OPPO (token contained within packets), Huawei.

  - Option-1b: ZTE, AT&T, NVIDIA, Ofinno.

  - Option-1c: Ericsson, Ofinno, DOCOMO.

  - Others: general burst traffic model with mixed packet sizes (Qualcomm)

Similarities and differences from the existing XR traffic model

Similarities

- GenAI services include video-based. XR traffic model mainly models the video traffic.

- Given video-based is one type of sources for AI/ML services, the XR model traffic including the periodic arrival, generation rate can be similarly used for the model for GenAI services, e.g., at least for video-based.

Differences

- GenAI services include image-based and chatbot besides video-based.

- The key differential characteristic of error tolerance of token cannot be reflected by XR traffic model, because the packet (for single stream) representing a video frame is counted as either success or failure. When a packet includes different numbers of Tokens, the packet error rate is not equal to the Token error rate which directly reflects the AI/ML services performance.

- XR traffic model does not reflect the packet importance in a more general approach, which is also the key characteristics for GenAI services.

The concerns raised in the contributions

- The existing services, e.g., video, have similar characteristics to the concerned token traffic streams, where not all packets are equally important and some have stricter delay constraints. Specifically, certain packets carry critical reference information, while others contain data that depends on previously transmitted packets. It can be well understood that the differing importance of video packets is reflected at higher protocol layers through mechanisms such as unequal error protection, QoS prioritization, adaptive frame dropping, and selective retransmissions. These approaches ensure that critical packets are more likely to reach the receiver intact, maintaining overall video quality even under degraded network conditions.

- Token bitstreams can be regarded as another form of codec bitstream traffic, with features related to importance, prioritization and dependency already considered in the existing XR traffic model and the similar PDU set information concept. In 5G user plane, PDU sets are defined as the minimum group of packets required to carry a video frame, and are assigned relative importance levels. This allows network operators to prioritize traffic based on the criticality of the service. Likewise, for token related to generative AI/ML services, we can simply do the same.

- For 6GR AI/ML service evaluations in RAN1, it is sufficient to model AI/ML traffic characteristics to parameters related packetization in terms of the future study output from SA, e.g., packet size, arrival rate and additional latency requirement (if necessary), as other conventional traffic models.

Harmonized idea for the model from FL perspective

- Most of companies mentioned the Token will be encapsulated so it is invisible in PHY. Hence, the packet relate parameters are still needed and could suffice as long as the relationship between Tokens and packets are clarified.

- Several companies also mentioned that the XR traffic model is very close to what the model would be like for the AI/ML services. With that, reusing the frame work of XR traffic model could be considered, e.g., defining the parameterized generic traffic model for both UL and DL. As for XR, the parameter values could be different for UL and DL, or even for different cases for DL, e.g., VR, cloud gaming, which will be detailed case by case.

- The model wouldn’t be quite different from the generic model defined for XR even though SA4’s study is still on-going. It should be ok to study the new AI/ML traffic by reusing the generic XR traffic model and new AI characteristic, and refine the parameter values later whenever SA4 updates RAN1 about SA4 study outcome.

Round-1 Discussions:

(FL1) Proposal 4.1.2

Working assumptions for RAN study:

The generic traffic model for AI/ML services is parametrized by:

- Token is the minimum unit of data generated at the application layer.

- Packet size: A packet models a set of Tokens.

  - Packet size is of integer multiple of the token size.

  - FFS whether modelled as a random variable following truncated Gaussian distribution.

  - FFS the candidate values of token size and the candidate token numbers in one packet.

- Packet arrival: The packet arrival rate depends on the generation rate of Token.

  - N multiple packets arrive together as a bust, e.g., Poisson process.

  - Within the burst, the N packets arrive periodically.

  - Note: N multiple packets include M Type-1 packets and (N-M) Type-2 packets. Type-1 and Type-2 packets may have different importance.

    - FFS: More than 2 importance levels.

    - FFS: N and M can vary among different bursts

    - FFS: the distributions of N and M.

- Packet delay budget: The latency requirement of the traffic in RAN side (i.e., air interface) is modelled as packet delay budget (PDB). The PDB is a limited time budget for a packet to be transmitted over the air from a base station to a UE.

- Token success rate requirement:

  - If packet delivery delay exceeds a given PDB, the packet is counted as failure.

    - For Type-2 packet: if any one of M Type-1 packets is counted as failure, all (N-M) Type-2 packets are counted as failure.

  - Token success rate = (Numerator A) divided by (Denominator B)

    - Numerator A is the number of Tokens included in the packets that successfully transmitted.

    - Denominator B is the number of Tokens included in all the packets transmitted.

- FFS the parameter values for detailed cases, e.g., image-based GenAI, video-based GenAI, and chatbot, etc.

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| vivo | First of all, we agree that Token is the basic processing unit of LLM, and it may be invisible to the mobile network. We prefer to first clarify the specific/interesting use cases before discussing the detailed traffic model, rather than defining a traffic model framework upfront. Second, currently at RAN1, perhaps we can only discuss what use cases we are considering, e.g., LLM, JSCCM audio, video or anything else. For the other details, we are not sure whether we can discuss them correctly and technically if we don’t have any feedback from SA4. Regarding the details, for specific GenAI use cases, we suggest drawing on the experience from XR study to provide traffic characteristics such as data burst size, data burst arrival time, and inter arrival time of the packets within a data burst. The definition of PDB is still under higher layer discussion. It may take dynamic values or vary among different packets, so we recommend collaborating with the related WGs. For the definition of evaluation metrics, we suggest deferring it to specific evaluations rather than discussing it as part of traffic model. |
| ZTE | First of all, we believe some more discussion on the procedure or how to do the down-selection is the first step for moving forward this topic. Option-2 has our preference. Then, for above FL proposal, we have some comments: - For textual data, a single packet may contain multiple Tokens. - The current proposal defines importance from a packet failure perspective, which contradicts the redundant nature of Tokens. - The mechanisms for importance and for handling successful/failed transmission should be discussed by RAN2. |
| OPPO1 | Firstly we think a generic traffic model for token based services is needed. For the detailed modeling for token success rate, the current description is ambiguous. - Packet success rate should be defined similar to XR, otherwise the definition of PDB would be meaningless. - For token success rate, it could be defined per packet or for all packets within a burst, which could be further discussed. Based on the above, we have the following suggestion: - Token success rate requirement: - Packet success rate: [xx%]. If packet delivery delay exceeds a given PDB, the packet is counted as failure. - Token success rate = (Numerator A) divided by (Denominator B) - Numerator A is the number of Tokens included in the packet[s] that successfully transmitted. - Denominator B is the number of Tokens included in the packet[s] transmitted. - Note: Different packet types may need different token success rates - Note: For Type-2 packet: if any one of M Type-1 packets is counted as failure, all (N-M) Type-2 packets may be counted as failure due to different importance of different packet types. |
| Xiaomi | - The characteristic of AI service is under RAN2/SA4 discussion already, we think we’d better wait for SA4 and RAN2’s study outcome rather than bring it to RAN study duplicatedly - Here we think the intention of the proposal is to study a general traffic model for AI service. The AI service refers to a wide range of applications rather than only tokens. For example, currently the delivered packets for LLM are text or video rather than tokens. Thus, only relying on the token to characterize the traffic model of AI service seems insufficient - Thirdly, it seems the traffic model is built on the top of token concept. But the concept of token is not crystally clear and we believe that different companies may have different understanding. To our observation, someone consider it is the processing unit for the large model processing and someone may consider it also refers to something like AI codec. Thus, if some category of AI service maybe characterized by Token related parameters, we should give a much clear definition for token first. |
| MediaTek | RAN1 has send out LS to SA4 and SA4 will attempt at drafting a reply to LS to RAN1 this week, it will be better to wait for SA4 to provide more info to RAN1. |
| NTT DOCOMO | We are generally fine with the FL proposal. |
| CMCC | Support to directly consider the concept of token and corresponding KPIs regarding token into RAN1 simulation. One clarification on the packet arrival modeling: “- Within the burst, the N packets arrive periodically”, if the N packets is confined in one burst, what is the meaning of “N packets arrive periodically”? does it means the time gap between two consecutive packets is constant? Another clarification on Token success rate requirement: if PDB exceeding process is only targeting on the level of packet, then the token success rate is same as packet success rate, if the token numbers per packet are same across different packets? |

Round-3 Discussions:

#### (FL3) Proposal 4.1.2-rv1

Working assumptions for RAN study:

The generic traffic model for AI/ML services is modelled by:

- Packet size:

  - A packet models a set of Tokens, when Token is generated at the application layer as the minimum unit of data.

    - Packet size is of integer multiple of the token size.

    - FFS the candidate values of token size and the candidate token numbers in one packet.

  - FFS whether the Packet size is modelled as a random variable following truncated Gaussian distribution.

- Packet arrival:

  - When a packet models a set of Token, the packet arrival rate depends on the generation rate of Token.

  - N multiple packets arrive together as a burst. The burst is modelled as a random variable following, e.g., Poisson process.

  - Within the burst, the N packets arrive periodically following the same average packet arrival periodicity.

  - Note: The N multiple packets include M Type-1 packets and (N-M) Type-2 packets. Type-1 and Type-2 packets may have different importance.

    - FFS: More than 2 importance levels.

    - FFS: N and M can vary among different bursts

    - FFS: the distributions of N and M.

- Packet delay budget: The latency requirement of the traffic in RAN side (i.e., air interface) is modelled as packet delay budget (PDB). The PDB is a limited time budget for a packet to be transmitted over the air from a base station to a UE.

- Packet success rate: [xx%]. If packet delivery delay exceeds a given PDB, the packet is counted as failure.

  - Note: For Type-2 packet: if any one of M Type-1 packets is counted as failure, all (N-M) Type-2 packets may be counted as failure due to different importance of different packet types.

- When a packet models a set of Token,

  - Token success rate requirement:

    - Token success rate = (Numerator A) divided by (Denominator B)

      - Numerator A is the number of Tokens included in the packet[s] that successfully transmitted.

      - Denominator B is the number of Tokens included in the packet[s] transmitted.

    - Note: Different packet types may need different token success rates.

- FFS the parameter values for detailed cases, e.g., image-based GenAI, video-based GenAI, and chatbot, etc.

##The proposal is the same for FL3, Companies can further comment if not done yet. Offline time is needed for at least clarifications from the proponents##

| Company | Comments |
| --- | --- |
| Nokia | We firmly suggest to await the feedback from SA4 and other WGs on traffic models for AI, XR, Haptic feedback, etc. Simply because RAN1 does not have the competence to decide on such traffic models, and risk taken unrealistic assumptions if concluding on such models in isolation from SA4 and other WGs that holds more insight on such traffic models. |
| Qualcomm | We suggest studying firstly whether the new model 2 and model 3 can be used for AIML service evaluation. Also, the new model for AIML service should be general and not dedicated to a particular GenAI application. At least till now we don’t see a strong need to introduce a model parameterized by token. Token is application layer data and is not visible to PHY/MAC. The model characterized by token size and token arrival rate would cause misunderstanding that application layer data can be processed directly at lower layer. We should ask RAN2 regarding whether token like application layer data can be visible and processed directly at the PHY/MAC before we discuss the token based traffic model. |
| AT&T | We are supportive of option1-b captured in the chair’s notes from RAN1#122bis. We do not see a strong reason to introduce a traffic model parametrized by tokens, or a token-based AI traffic model. We nevertheless see a great need for modeling AI/ML traffic in our network, stemming from mobile-based LLMs. Such traffic observed is not tokenized on the air interface. Tokenization happens at the receiver (server) end or at the edge cloud, which is beyond the air interface. We propose that this model carefully incorporates all the phases of a GenAI based LLM session (e.g. time-to-first-token, stream rate, think-time, etc.) in addition to using parameters like packet size, delay requirement, etc.. We cannot agree on the FL proposal for multiple reasons, amongst them: - We do not have enough indication how dominant the tokenized AI/ML data will be in the air interface, if any - Tokenization is a proprietary mechanism where different companies adopt different strategy. Finding a tokenization method which is reasonable across all the uses is non-trivial - Tokenization is an application layer construct, which is transparent to PHY/MAC and interpret the packet level performance to token level performance is non-trivial |
| Samsung | We share the views from Nokia and Qualcomm. It appears quite premature to discuss the generic traffic model for AI/ML services in RAN1 without specific guidance and requirements from responsible working group(s), especially from SA4. We would also emphasize that the tokenization, embedding and encoding processes between media data (video, audio, text) and tokens are highly dependent on the AI-media services and the ability of corresponding AI models, which may differ with large dynamics from perspective of application layer without standardized process. Hence, it is quite questionable to build up a generic traffic model for RAN1 evaluation based on the token concept. Moreover, in our view, token bitstreams should be regarded as another form of codec bitstream traffic, with features related to importance, prioritization and dependency already considered in e.g., the existing XR traffic model and the similar PDU set information concept. |

## New model 2-Immersive comm.

### Companies’ views

| Company | Views/proposals |
| --- | --- |
| Futurewei | Regarding the traffic model(s) for 6GR AI/ML services and traffic modelling for evaluations related to immersive communication services, while RAN1 is waiting for SA4 inputs, RAN1 can carry out study in the meantime. To be able to accurately reflect practical traffic for AI/ML services and immersive communication services, RAN1 study should be based on realistic traffic data/statistics, e.g., current and projected AI/ML token statistics, realistic traffic data provided by advanced XR vendors and haptics service providers, and so on. |
| Nokia | Proposal 15: RAN1 to prioritize reaching agreements on FTP traffic modes within RAN1, while waiting for the feedback from SA4 and other WGs on traffic models for AI, XR, Haptic feedback, etc. Proposal 26: RAN1 to adopt the single-stream packet-based XR traffic model for real-time video from TR 38.838 with default settings of 30 or 45 Mbps for DL and 10 Mbps for UL, assuming 60 fps. Proposal 27: RAN1 to adopt for the 6GR study the model of XR UL control information feedback from 38.838 that needs to be transmitted together with DL traffic. |
| vivo | Proposal 2: Support to introduce haptic services traffic model for immersive communication evaluation, as detailed in Table 2 above. Table 2: Statistical parameters for haptic services traffic model Parameters; Unit; Value. Parametric media format; Time-sampled media format. Packet size; byte; For active state1 Exponential distribution with λ= 0.003, with min = 62 bytes For idle state2 Fixed size, 1 byte; For active state1 Truncated gaussian distribution with mean3=R×T/8, std=20%×mean, range=[0.5×mean, 1.5×mean] For idle state2 Fixed size, 53 bytes. Packet arrival time (T); ms; Pareto distribution with min = 128, k = 0.5; Period, 128ms. Jitter; ms; Optional, follows the description in clause 5.1.1.2 in TR 38.838. PDB; ms; 30 (20ms CN PDB is subtracted according to TR 23.501 [24]). Packet Success rate; %; 90. Note 1: Active state means that there are least one haptic data packet. Note 2: Idle state means that there is without haptic data, only with overhead. Note 3: R is the data rate of the flow in Kbps.. Proposal 3: Support to introduce synchronization requirements model for immersive multi-modality XR services evaluation, as detailed in the Table 3 above. Table 3: Tolerable asynchronicity thresholds of multi-modality service Use case; Media; Tolerable asynchronicity threshold. Immersive multi-modality XR; audio-haptics; audio delay: 50 ms; haptic delay: 25 ms. video-haptics; visual delay: 15 ms; Haptic delay: 50 ms. |
| ZTE | Proposal 4-3-1: For 6G immersive communication services, reuse the existing XR traffic model structure with updated parameter values. RAN1 should postpone further discussions on traffic model parameters until receiving service characteristics from SA4. Additionally, we are open to the study of new traffic types, such as haptic traffic. |
| CATT | Proposal 8: The discussion on new traffic models should be postponed until the discussion of the common evaluation assumptions is completed. |
| InterDigital | Proposal 4: Support the traffic model haptics - Haptics traffic is defined as XR traffic packet generation with co-generated haptics packets. - Generation of haptics packets are determined by the following pseudo-code. - Haptics packets has packet delay budget (PDB) of either 12 msec or 30 msec, which can be selected as a traffic model parameter. If XR packet generation event, - If silent state, generate haptic packet with probability P2 - If still silent state, do nothing - If transitioned to haptics state, - Determine number of channels (exponential distribution with min and max values) - Packet size is determined to be ‘haptics unit size X number of channels’ - Generate packet - Note: P2 is determined by ‘fps/average silent period’, where fps is the fps parameter of XR traffic. If haptics state & 128 msec has passed since last haptics packet generation, - Transition to silent state with probability P1 - If still in haptics state, generate another haptic packet with same size as previous packet. - Note: P1 is determined by ‘1/average haptic frame between silent periods’. |
| Huawei | Proposal 16: Towards modelling the advanced XR (i.e., immersive gaming), extend the existing XR traffic model (i.e., CG DL stream model in clause 5.4.1 of TR 38.838 corresponding to XR cloud gaming use cases) with modifications in red on data rate, frame generation rate as in Table 8 and on the packet size distribution as in Table 9, considering the requirement defined in TR22.870. Table 8: Extended values to Table 5.4.1-1 TR 38.838 for immersive gaming use cases Parameters; unit; Baseline values for evaluation; Optional values for evaluation; Extended values for immersive gaming. data rate: R; Mbps; 30, 8; 45; 100, 300, 500. frame generation rate: F; fps or Hz; 60; 90,120. PDB; ms; 15; 10, 30; 15, or 10, 30. Table 9: Extended values to Table 5.1.1.1-1 TR 38.838 for immersive gaming use cases Parameter; unit; Baseline values for evaluation; Optional values for evaluation for single eye buffer; Extended values for immersive gaming. Mean: M; byte; R×1e6 / F / 8; R×1e6 / F / 8; R×1e6 / F / 8. STD; byte; 10.5% of M; 3 % of M; 25 % of M. Max; byte; 150% of M; 109% of M; 300% of M. Min; byte; 50% of M; 91% of M; 25% of M. R: data rate of the flow in Mbps. F: frame generation rate of the flow in fps. Note that the mean and STD apply before truncation applies. Note that the value of R, F depend on application.. Proposal 17: Towards modelling the advanced XR (i.e., UL-heavy video uploading for immersive services), extend the existing XR traffic model with modifications on data rate, frame generation rate as in Table 11 and on packet size distribution as in Table 12, considering the requirement defined in TR22.870. Table 10: Extended values to Table 5.6.1-1 TR 38.838 for UL heavy video uploading services Parameters; unit; extended values for UL-heavy video uploading. data rate: R; Mbps; 20, 60, 100. frame generation rate: F; fps or Hz; 15, 30. PDB; ms; 10, 15. Table 11: Extended values to Table 5.1.1.1-1 TR 38.838 for UL heavy video uploading services Parameter; unit; 1st candidate values for UL-heavy video uploading; 2nd candidate values for UL-heavy video uploading. Mean: M; byte; R×1e6 / F / 8; R×1e6 / F / 8. STD; byte; 10.5% of M; 25 % of M. Max; byte; 150% of M; 300% of M. Min; byte; 50% of M; 25% of M. |
| Samsung | Proposal #18: - Recommend to wait for further information from SA4, and to minimize the effort in RAN1 on defining the traffic model for advanced XR and haptics services via leveraging the existing model in TR 38.838 as much as possible. |
| NVIDIA | Observation 3: 6G network traffic is expected to be uplink-heavy, bursty and highly dynamic with the uprise in immersive (e.g., XR/cloud gaming) and AI-driven applications. Proposal 7: Consider the existing XR model as the baseline for performance evaluation during 6GR study for use case scenarios related to immersive communication. Proposal 11: Study traffic models for performance evaluation during 6GR study taking into consideration the unique characteristics of UL-heavy immersive and AI applications related traffic. |
| Ericsson | Generative AI applications are quite diverse and accordingly the traffic generated by such applications is also expected to be diverse. For example, increased use of real-time GenAI-driven video assistants and immersive interactions could significantly increase traffic (especially uplink) while other text/chat-style usage applications may have relatively less impact. From RAN1 perspective, Option-1c discussed in RAN1#122bis is a suitable starting point for the study on traffic modelling for such applications. The video traffic modeling in XR TR 38.838 could be used as a starting point to evaluate the impact of video traffic generated by such new applications. Potential modifications could be considered based on any input received for the LS sent in RAN1#122bis [12]. For other traffic types, the generic FTP Model1/FTP Model 3 with extensions could be used as a starting point. Proposal 4-3 - For the study on traffic models for generative AI services, for video related applications, use the video traffic modeling in XR TR 38.838 as starting point, and for other applications use the extensions being studied for FTP Model 1/FTP Model 3 as starting point. |
| Ofinno | Observation 5: The XR related traffic models defined in clause 5 of TR 38.838 include models for variety of XR related traffic and for both UL/DL directions. Observation 6: The existing XR related traffic models can likely be extended to create traffic models for immersive communication (e.g., advance XR services, haptics media services). |
| Qualcomm | Proposal 7: For modeling of immersive communication services, the XR traffic model is extended as the following • Burst with mixed packet sizes arrive periodically roughly at 60fps with random jitter • Each burst consists of multiple packets including both small and large packet sizes. FFS the packet sizes • The packets within the burst arrive back-to-back with short inter-arrival time (e.g., 0.5-1msec) • The number of packets per burst can be fixed or random with an upper limit. The distribution of different packet sizes within the burst can be determined based on a predefined probability. |
| DOCOMO | Proposal 3 - Regarding traffic model for immersive communication including haptics traffic, we support modification of the following parameters of XR traffic model. - Packet size distribution - Jitter and packet arrival rate between two adjacent packets - PDB - To represent typical immersive communication services and typical haptics services, the following XR traffic should be used. - Immersive communication services: single-stream XR traffic model - Haptics services: multi-stream XR traffic model |

### Discussions

Background

It was agreed in the last meeting that RAN1 studies the traffic modelling for evaluations related to immersive communication services including but not limited to advanced XR [e.g., TR22.870] and haptics services as follows:

Agreement Study traffic modelling for evaluations related to immersive communication services including but not limited to advanced XR [e.g., TR22.870] and haptics services, - XR traffic models (in TR 38.838) are considered as starting point. - FFS the detailed modifications on the parameters to the XR traffic model, e.g., higher packet size, higher packet arrival rate, higher packet size deviation, PDB, etc. - FFS how many models need to be defined and the corresponding representative use cases. - FFS how to incorporate haptics traffic (TR26.854). Send LS to SA4 requesting input if any on the relevant traffic characteristics, RAN1 can continue the study before SA4 potential response.

It is clear that the modelling for immersive communication services will be based on the XR traffic model and FFS the modifications if identified.

Observations from the submitted paper for this meeting

8 Companies (vivo, Interdigital, Huawei, NVIDIA, Ericsson, Ofinno, Qualcomm, DOCOMO) proposed to modify the XR traffic model parameters and 3 companies (Huawei, vivo, Interdigital) proposed the detailed modifications. 4 companies mentioned to wait for SA4’s input (Nokia, ZTE, CATT, Samsung). In particular,

- 1 company (Huawei) proposed to add a larger packet size and a higher frame generation rate for DL traffic accounting for the immersive gaming requiring higher resolution video.

- 2 companies (Huawei, NVIDIA) proposed to reflect the UL-heavy immersive and AI applications related traffic, which mainly models the video uploading traffic.

- 2 companies (vivo, InterDigital) proposed to model the haptic traffic paired with XR traffic in details, for which 1 company (Qualcomm) proposed to generalize the paired traffic to be correlated multi-stream model unlike the individually generated multi-stream model defined for XR.

The concerns raised in the contributions

- There an is ongoing discussion in ITU-R WP 5D on introduction of a new Composite Requirement for IMT-2030 targeting XR use case, and as proposed in the RAN LS, the candidate DL and UL bit rates are 30 and 10 Mbps, respectively, with a latency bound of 40 ms (including both DL and UL).

Modifications and motivation for the advanced XR traffic without Haptics

- Immersive gaming in 6GR as introduced in section 9.2 of TR 22.870 leveraging the higher display resolution up to 8K/12K to avoid the so-called “Screen Door Effect”. In addition, many current display devices of XR cloud gaming support 90fps, while the refresh rate up to 120 fps also needs to be considered since high frame rate could help reduce the probability of simulator sickness. Moreover, since H.265 coding for videos are usually utilized for better compression with little loss, an extended fluctuation range of the packet size is also envisioned.

- The real-time video uploading is the commonality for many immersive services and exhibits several key characteristics including: quasi-periodic data arrivals, large packet sizes, and low packet delay budget. Since commercial cameras usually perform frame extraction to delete the unnecessary information like the cameras on vehicles, the frame generation rates should include lower values such as 15/30 fps compared to the current one with 60fps of UL AR in clause 5.5.2 of TR 38.838. In addition, similar to the description in Section 3.3.1, the fluctuation range on the packet size are also diversified due to H.265 coding.

Modifications and motivation for the advanced XR traffic with Haptics

- The attributes of the traffic characteristics of haptic media services are packet size per channel, jitter, packet delay budget, packet loss rate, and throughput per channel.

- Need to study not only its periodic characteristic as modelled in existing XR traffic model in TR 38.838, but also its aperiodic traffic characteristic.

- Many are relatively low data rates that are paired with high data rate streams. Haptics traffic has much more stringent latency requirements as it is intended to be a feedback system. Synchronicity with paired data stream is an important feature of haptics.

Round-1 Discussions:

(FL1) Proposal 4.2.2

For 6GR evaluations related to immersive communications services, the following two amended XR models based on the existing XR traffic model (in TR 38.838) can be considered:

- Model-1: eXR model without Haptics

  - Regarding the statistical parameters for single stream CG traffic model defined in Table 5.4.1-1 TR 38.838, add values for immersive gaming regarding the data rate and the frame generation rate as in red:

| Parameters | unit | Baseline values for evaluation | Optional values for evaluation | Values for immersive gaming |
| --- | --- | --- | --- | --- |
| data rate: R | Mbps | 30, 8 | 45 | 100, 300, 500 |
| frame generation rate: F | fps or Hz | 60 |  | 90,120 |
| PDB | ms | 15 | 10, 30 | 15, or 10, 30 |

  - Regarding the statistical parameters for packet size following truncated Gaussian distribution in Table 5.1.1.1-1 TR 38.838, add values for immersive gaming regarding STD, Max, and Min values as in red:

| Parameter | unit | Baseline values for evaluation | Optional values for evaluation for single eye buffer | Values for immersive gaming |
| --- | --- | --- | --- | --- |
| Mean: M | byte | R×1e6 / F / 8 | R×1e6 / F / 8 | R×1e6 / F / 8 |
| STD | byte | 10.5% of M | 3 % of M | 25 % of M |
| Max | byte | 150% of M | 109% of M | 300% of M |
| Min | byte | 50% of M | 91% of M | 25% of M |
| R: data rate of the flow in Mbps. F: frame generation rate of the flow in fps. Note that the mean and STD apply before truncation applies. Note that the value of R, F depend on application. |  |  |  |  |

  - Regarding the statistical parameters for AR UL Model 1 defined in Table 5.5.2.1-1 TR 38.838, add values for UL-heavy video uploading regarding packet size, generate rate, data rate, and PDB values as in red:

| Parameters | unit | value | Values for UL video uploading |
| --- | --- | --- | --- |
| Packet size | byte | Follows clause 5.1.1.1 (i.e., mean packet size = R×1e6 / F / 8, STD/Min/Max=10.5/50/150%) | 1st candidate: Follows clause 5.1.1.1 (i.e., mean packet size = R×1e6 / F / 8, STD/Min/Max=10.5/50/150%) 2nd candidate: Follows clause 5.1.1.1 (i.e., mean packet size = R×1e6 / F / 8, STD/Min/Max=25/25/300%) |
| packet generation rate: F | Hz | 60 | 15, 30 |
| Jitter | ms | Optional, follows the description in clause 5.1.1.2 | Optional, follows the description in clause 5.1.1.2 |
| Data rate: R | Mbps | 10 (baseline), 20 (optional) | 20, 60, 100 |
| PDB | ms | 30 (baseline), 10 or 15 or 60 (optional) | 10, 15 |

- Model-2: eXR model with Haptics

  - Haptics traffic is defined as XR traffic packet generation with co-generated haptics packets.

  - Generation of haptics packets are determined by the following pseudo-code.

  - Haptics packets has packet delay budget (PDB) of either 12 msec or 30 msec, which can be selected as a traffic model parameter.

If XR packet generation event, - If silent state, generate haptic packet with probability P2 - If still silent state, do nothing - If transitioned to haptics state, - Determine number of channels (exponential distribution with min and max values) - Packet size is determined to be ‘haptics unit size X number of channels’ - Generate packet - Note: P2 is determined by ‘fps/average silent period’, where fps is the fps parameter of XR traffic. If haptics state & 128 msec has passed since last haptics packet generation, - Transition to silent state with probability P1 - If still in haptics state, generate another haptic packet with same size as previous packet. - Note: P1 is determined by ‘1/average haptic frame between silent periods’

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| vivo | For eXR model with Haptics, in addition to the PDB for haptics itself, we suggest also studying the PDB requirements for the multimodal integration of haptics, video, and audio, which is crucial for immersive user experience. These pseudo-codes are intermediate processes for generating haptics services. The final traffic model can still adopt traditional parameters such as packet size, arrival time, and etc. |
| ZTE | - The specific parameter values should not be determined at this stage, the reply from SA4 is needed. - It is too early to discuss pseudo-code for generating haptics traffic; consensus should first be reached on its characteristics. |
| NTT DOCOMO | Model-1: Ok. Model-2: Prefer to wait for input from SA4. |
| CMCC | Support to reuse XR models for immersive communication evaluation. But if we extend the values for immersive gaming/ UL video uploading, then the legacy values in TR 38.838 will be used or not? We think we can only consider the values for 6G immersive gaming/ UL video uploading. One clarification on the values for UL video uploading, regarding packet generation rate: F, why the value (15, 30) is smaller than legacy(60)? |

Round-3 Discussions:

#### (FL3) Proposal 4.2.2-rv1

For 6GR evaluations related to immersive communications services, the following two amended XR models based on the existing XR traffic model (in TR 38.838) can be considered:

- Model-1: eXR model without Haptics

  - Regarding the statistical parameters for single stream CG traffic model defined in Table 5.4.1-1 TR 38.838, add values for immersive gaming regarding the data rate and the frame generation rate as in red:

| Parameters | unit | Baseline values for evaluation | Optional values for evaluation | Values for immersive gaming |
| --- | --- | --- | --- | --- |
| data rate: R | Mbps | 30, 8 | 45 | 100, 300, 500 |
| frame generation rate: F | fps or Hz | 60 |  | 90,120 |
| PDB | ms | 15 | 10, 30 | 15, or 10, 30 |

  - Regarding the statistical parameters for packet size following truncated Gaussian distribution in Table 5.1.1.1-1 TR 38.838, add values for immersive gaming regarding STD, Max, and Min values as in red:

| Parameter | unit | Baseline values for evaluation | Optional values for evaluation for single eye buffer | Values for immersive gaming |
| --- | --- | --- | --- | --- |
| Mean: M | byte | R×1e6 / F / 8 | R×1e6 / F / 8 | R×1e6 / F / 8 |
| STD | byte | 10.5% of M | 3 % of M | 25 % of M |
| Max | byte | 150% of M | 109% of M | 300% of M |
| Min | byte | 50% of M | 91% of M | 25% of M |
| R: data rate of the flow in Mbps. F: frame generation rate of the flow in fps. Note that the mean and STD apply before truncation applies. Note that the value of R, F depend on application. |  |  |  |  |

  - Regarding the statistical parameters for AR UL Model 1 defined in Table 5.5.2.1-1 TR 38.838, add values for UL-heavy video uploading regarding packet size, generate rate, data rate, and PDB values as in red:

| Parameters | unit | value | Values for UL video uploading |
| --- | --- | --- | --- |
| Packet size | byte | Follows clause 5.1.1.1 (i.e., mean packet size = R×1e6 / F / 8, STD/Min/Max=10.5/50/150%) | 1st candidate: Follows clause 5.1.1.1 (i.e., mean packet size = R×1e6 / F / 8, STD/Min/Max=10.5/50/150%) 2nd candidate: Follows clause 5.1.1.1 (i.e., mean packet size = R×1e6 / F / 8, STD/Min/Max=25/25/300%) |
| packet generation rate: F | Hz | 60 | 15, 30 |
| Jitter | ms | Optional, follows the description in clause 5.1.1.2 | Optional, follows the description in clause 5.1.1.2 |
| Data rate: R | Mbps | 10 (baseline), 20 (optional) | 20, 60, 100 |
| PDB | ms | 30 (baseline), 10 or 15 or 60 (optional) | 10, 15 |

- Model-2: eXR model with Haptics

  - Haptics traffic is defined as XR traffic packet generation with co-generated haptics packets.

    - FFS on how to generate the multi-channel haptics packet including how to handle silent periods of haptics and the haptics packet sizes.

  - Haptics packets has packet delay budget (PDB) of either 12 msec or 30 msec, which can be selected as a traffic model parameter.

#The proposal was updated per InterDigital’s suggestion for FL3, Companies can further comment if not done yet. Offline time is needed for at least clarifications from the proponents#

| Company | Comments |
| --- | --- |
| Nokia | In our view, the baseline XR models shall be the default models used for 5G NR XR studies. That is, single-stream packet-based XR traffic model for real-time video from TR 38.838 with default settings of 30 or 45 Mbps for DL and 10 Mbps for UL, assuming 60 fps. Any potential justification for modifying the XR traffic model(s) shall come from SA4, and should not be initiated by RAN1 on its own. |
| Interdigital | In order to get further discussion on Model-2 eXR with haptics, it would be useful to further clarifh the FFS on details. Some asepcts to note for FFS would be, how to handle and generate of multi-channel haptics packet, probability function of haptics packet generation, how to generate haptic packets including how to handle silent periods of haptics, haptics packet sizes. |
| Qualcomm | For model 1, we suggest discussing it after receiving SA4 response since the proposed changes to packet size, frame generation rate and data rates are highly dependent on traffic characteristics. RAN1 could not make its own decision. Since the changes are only related to parameter values and the legacy modeling is reused, we don’t think it is urgent to make the decision. For model 2, we agree to discuss the FFS details. For the previous FL proposal, we want to clarify whether it is assumed that haptic packet is generated at the same time as XR packet generation. We think it may be too ideal assumption and at least a small offset (fixed or random) should be added between XR packet generation and haptic packet. Secondly, if the number of channels for haptic is larger than one, it should be also possible that the packet for each channel is generated independently and transmitted. We cannot assume all the channel data are aggregated and mapped to one packet for transmission. Lastly, we want to know what is the average haptic frame between silent periods for P1 determination? Is it same as the “average silent period”? If not, how can we determine these two parameter values? |
| Samsung | Suggest to wait for the response from SA4 for haptics as well. |

## New model 3- Multiple packet size+PDB

### Companies’ views

| Company | Views/proposals |
| --- | --- |
| Futurewei | Proposal 1: For extensions to FTP Model 1/FTP Model 3: Multiple packet sizes and associated time-domain behaviors (e.g., inter arrival time) Support at least 3 packet sizes. Support having fixed (and potentially more than one) packet size and fixed mean packet arrival rate for a given UE. At least FTP Model 3 should support multiple packet sizes. Support at least small, medium, and large packets (e.g., 0.1 MB, 0.5 MB, and 2 MB). Support different mean arrival rates to achieve different levels of resource utilization (RU) ratios of the network (e.g., low, normal, medium, high, extremely high loads). Packet delay budget (PDB) related parameters Different PDB parameters for different traffic flows can be considered. Support to drop packets when exceeding the budget, and report packet drop rate due to PDB. PDB should be applicable at least to the extension to FTP Model 3 with one packet size. PDB should be applicable at least to the extension to FTP Model 3 with multiple packet sizes. Different packet sizes should have different PDBs. |
| Nokia | Proposal 16: RAN1 to select FTP-3 as the default FTP mode using the following parametrization: default file size of 0.5 MB, variable file arrival rate resulting in Low, Medium, Large offered loads per cell (corresponding to average PRB utilization of approximately 10%, 30% and 60%). Proposal 17: We suggest an extended FTP-3 (eFTP-3) model that consists of superposition of two FTP-3 legacy models with different file sizes (B1 and B2) and average arrival rates (r1 and r2). The characteristic of the model is as follows: a. File sizes of B1 and B2 bits are generated according two independent Poisson process with average arrival rates of r1 and r2, b. B1=0.5 MB as the default setting for FTP-3, c. B2=K*B1 and r2=r1/K, where K is a natural number with default setting K=4, d. The file arrival rate r1 is varied to result in Low, Medium, Large offered loads per cell that corresponds to average PRB utilization of approximately 10%, 30% and 60%. e. Alternative parametrizations for B1, and K are also possible. Proposal 18: eFTP-3 traffic model can optionally have different parameters for different groups/classes of UEs in a simulation, for example, K = 0 (only legacy FTP-3) for some UEs and K = 4 for the rest. Proposal 19: A PDB metric may be additionally associated with the traffic model. Proposal 20: Packet discarding shall not be part of the traffic model definition. Such mechanisms are up to RAN2 to decide and can be included in the RAN simulation assumptions (but not as part of the traffic model itself). Proposal 21: It is sufficient to consider extensions to FTP Model 3 only. |
| vivo | Proposal 4: For FTP models, the use of FTP3 is baseline for 6GR evaluations, whereas the use of FTP1 is optional. Proposal 5: Do not support to enhance FTP model 1, as the motivation for PDB and multiple packets size extension remains unclear. Proposal 6: Support to introduce PDB related parameters to FTP Model 3 for 6G evaluation, as detailed in table 4 above. Table 4: Extended FTP Traffic Model 3 Parameter; Statistical Characterization. File size, S; 0.5 Mbytes. Packet arrival rate, λ; Poisson distributed with arrival rate λ Possible range of λ: [5]. PDB; Possible value of PDB: [20ms, 50ms, 100ms]. Proposal 7: It is up to company to report the details for handling the packet when it has exceeded the PDB. Proposal 8: Support to introduce multiple packet sizes and associated time-domain behaviors to FTP model 3 for 6G evaluation, which can be done by supporting at least 2 traffic flows for a UE as given in Table 5. Table 5: Mixed FTP Traffic Model 3 Parameter; Statistical Characterization. Large packet size traffic flow; Small packet size traffic flow. File size, S; 0.5Mbytes; 0.1M bytes. Packet arrival rate, λ; Poisson distributed with arrival rate λ1 Possible range of λ1: [FFS] Range of λ1 can further be adjusted; Poisson distributed with arrival rate λ2 Possible range of λ2: [FFS] Range of λ2 can further be adjusted. PDB; Possible value of PDB1: [20ms, 50ms, 100ms]; Possible value of PDB2: [10ms, 30ms, 80ms]. Note 1: When the mixed FTP traffic model 3 is used, the relationship λ1 >= λ2 and PDB1 >= PDB2 should hold. |
| CMCC | Proposal 5: For mixed-service traffic model, consider at least FTP3 extension from the following aspects: - How to model multiple packet sizes with the extension on FTP3. From our understanding, the following options can be further considered and down-selected: - Option 1: consider randomly-selection within multiple sizes in new packet generation. The number of packet sizes, specific values and probabilities in ETSI TS 103 786 can be taken as the starting point. - Option 2: consider parallel FTP3 traffic threads within a UE, where each traffic thread is related one packet size. The number of packet sizes and values in ETSI TS 103 786 can be taken as the starting point. - Whether/how to enhance the arrival rate characteristics, including, - The exponential distribution (according to existing FTP3 model) should be considered as baseline. - Whether the parameter of exponential distribution can be varied per packet generation can be further discussed. The number of corresponding parameters, specific values and probabilities in ETSI TS 103 786 can be taken as the starting point. - Whether other distributions can be considered instead of exponential distribution, e.g., uniform distribution in in ETSI TS 103 786, can be further discussed. Proposal 6: For PDB-related traffic model, consider at least FTP3 extension from the following aspects: - - A specific PDB parameter for each kind of service (within a FTP3 traffic model), and consider various kinds of services will be simultaneously transmitted while evaluation. - - The execution when packet is exceeding the budget need to be considered according to service type. For TCP-based service, the packet shall be transmitted. For UDP-based service, the packet can be dropped. - - The PDB aware metric can be set as the percentage of the packet transmission that fulfills PDB requirement as the starting point. |
| ZTE | Proposal 4-1-1: Extended FTP traffic model for 6GR evaluation is characterized as follows: - Each UE is only assigned with one of three service types according to pre-defined probabilities, and each services type is characterized by packet size, arrival rate, latency requirement and reliability requirement. - Parameters including packet size for a specific service type should be configured with fixed values. Three packet sizes are considered in the extended FTP traffic model. Multiple packet sizes are applicable to both FTP model 1 and FTP model 3. - Model parameter values are referenced in Table 4-1-1. - For packets that exceed the PDB requirement, discard them and treat the UPT of these packets as zero. - For unfinished packets due to expiration of the simulation time, it is proposed to incorporate them into metrics such as UPT by calculating: the number of served bits (possibly zero) of an unfinished packet divided by the served time (simulation end time - file arrival time). Table 4-1-1 Extended FTP traffic model for 6GR evaluation Parameter; Value. UE distribution; Each UE selects one of the three service types: - Service type-1: Small packet only - Service type-2: Medium packet only - Service type-3: Large packet only; - Service type-1, [80%] UE - Service type-2, [10%] UE - Service type-3, [10%] UE. Service type-1; Packet size; [~10] KB. Arrival rate; Poisson process with [~100 - 1000] packets per second. Latency requirement; < [~10] ms. Reliability requirement; > [99%]. Service type-2; Packet size; [~1] MB. Arrival rate; Poisson process with [~1 - 10] packets per second. Latency requirement; < [~100] ms. Reliability requirement; > [99%]. Service type-3; Packet size; [~10] MB. Arrival rate; Poisson process with [~0.1 - 1] packets per second. Latency requirement; < [~1] s. Reliability requirement; > [99%]. |
| NEC | Proposal 4: For MBB evaluations, adopt FTP Model 3 with an added PDB requirement. The model is defined as follows: • Packets are generated according to the FTP Model 3 process (e.g., 0.5 Mbytes packet size, 200 ms mean inter-arrival time). • Each packet is assigned a PDB (e.g., 50 ms or 200 ms) to reflect QoS requirements. • The evaluation metric shall be the QoS satisfaction rate (the percentage of packets successfully delivered within their PDB) in addition to average UPT. |
| Interdigital | Proposal 5: Support varying the offered traffic load for each UEs with FTP-3 traffic model - The offered traffic load for different UEs can be varied based on geometry. - Average Offered traffic load, λavg, is adjusted by adjusting the arrival rate. - Arrival rate of each user, λn=λavgTnn=1NTn - Estimate throughput per user, Tn=BNlog21+SNRn. Where SNRn is the geometry of the user, B is the system bandwidth, and N is the number of users in the cell. - All UEs will have same packet size. |
| Huawei | Proposal 9: Extending the FTP model to incorporate Packet delay budget (PDB) and one/multiple packet sizes should be based on FTP Model 3 instead of FTP Model 1. Proposal 10: As an extension to FTP Model 3, FTP Model 3a models one packet size, one mean inter-arrival time value and one packet delay budget (PDB): • One packet size value chosen from 40bytes, 0.5Mbytes, 0.1Mbytes or other value for different RU scenario, e.g., 1Mbytes. • The combination of packet delay budget (PDB) value and inter-arrival time value as below table inter-arrival time value from TR 38.864; packet delay budget (PDB) from TS 23.501. Combination 1 for ftp service; 200ms; 300ms. Combination 2 for VOIP service; 160ms; 100ms. Combination 3 for IM service; 2s; 100ms or 200ms. Proposal 11: For FTP Model 3 extended with PDB, the packets exceeding the packet delay budget value should be dropped. Proposal 12: Regarding the study on extending FTP model 3 considering mixed packet sizes, it’s more reasonable to consider fixed packet sizes, fixed combination of mean inter arrival time values and a PDB value for a given UE with the following assumptions in one simulation drop. • At most two packet size values chosen from 40bytes, 0.5Mbytes, 0.1Mbytes or other value for different RU scenario, e.g., 1Mbytes. • At most two combinations of packet delay budget (PDB) value and inter-arrival time value chosen from below table inter-arrival time value from TR 38.864; packet delay budget (PDB) from TS 23.501. Combination 1 for ftp service; 200ms; 300ms. Combination 2 for VOIP service; 160ms; 100ms. Combination 3 for IM service; 2s; 100ms or 200ms. |
| Samsung | Observation: - Packets with variable sizes are observed from real traffic logs, with small and medium packets occurring much more frequently than larger ones Proposal #10: - In addition to existing traffic models, consider optional 6GR evaluation with non-full buffer traffic models supporting variable packet sizes using FTP modeling framework. Proposal #11: - For traffic model supporting variable packet sizes, define three reference packet sizes (S_S,S_M,S_L ) corresponding to small, medium and large packet sizes along with associated packet arrival rates (λ_S,λ_M,λ_L ). - The proportion of the packet arrival rates (λ_S,λ_M,λ_L ) should be maintained to ensure that smaller packets are occurring much more frequently than larger ones. Proposal #12: - For traffic model supporting variable packet sizes, consider scheduling aspects to determine reference packet sizes (S_S,S_M,S_L ). Proposal #13: - For traffic model supporting variable packet sizes, consider the following options of packet assignment to the UE for joint SLS evaluation with different packet sizes: - Each UE can receive the packet of only one size from the reference packet sizes. - Each UE can receive packet of different sizes from reference packet sizes. Proposal #14: - For traffic model supporting variable packet sizes consider applicability for the following FTP traffic modelling framework: - For FTP traffic model 1, each UE can receive packet of only one size. - For FTP traffic model 3, each UE can receive packets of different sizes. Proposal #15: - Further clarify what kind of services under what conditions require a need of introducing additional PDB in FTP model for 6GR evaluation. |
| Intel | Observation 2 - FTP Model 3 allows packet backlogging which limits its applicability and results interpretability in high loading cases Proposal 8 - FTP Model 3 is updated to always have an associated PDB - A packet exceeding PDB during the transfer is dropped - The number / ratio of dropped packets is reported - UE packet throughput accounts all three types of packets, - Successfully transferred during the simulation time, - Dropped during the simulation time, - Unfinished during the simulation time. Proposal 9 - Introduce extension to FTP Model 3 (and Model 1) to support multiple traffic flows in a cell - N separate sets of packet size, packet arrival rate, and PDB are provided to the model, - Each of N separate sets are used to generate independent Poisson process of packet arrival between UEs (both FTP Model 3 and FTP Model 1), - The ratio of each of the N separate sets is also provided, each user in a cell is assigned to one of the N flow types, respecting the ratio - FFS if the flows are assigned based on UE long-term channel conditions - N = 2, other values are FFS |
| NVIDIA | Proposal 3: Consider “FTP model 1” as the optional non-full buffer traffic model for performance evaluation during 6GR study with the following enhancements – - File size: at least 3 different file sizes- - flarge (~100s of MB, e.g., AI/ML training dataset), - fmedium (~10s of MB, e.g., high-resolution TIFF/RAW image), - fsmall (~1MB, e.g., conventional eMBB data traffic). - User arrival rate distribution (λ): at least 3 different ranges of λ (users/s), viz., λsmall, λmedium and λlarge, with λmedium = β1 * λsmall and λlarge = β2 * λsmall, where β1 = fsmall / fmedium and β2 = fsmall / flarge respectively. As an example – - λlarge-100MB = [.002, .005, .007, .01, .012], - λmedium-10MB = [.025, .05, .075, .1, .12], - λsmall-1MB = [0.25, 0.5, 0.75, 1.0, 1.25]. Proposal 4: Consider “FTP model 3” as the de-facto non-full buffer traffic model for performance evaluation during 6GR study with the following enhancements – - File size: two variants- - Fixed file size = 0.5 MB. - Variable file size = log-normal (µ, σ) distribution to model a range of file sizes, from small <10 MB) to moderate (~10-100 MB) and large (>100 MB). - Inter-arrival time: two variants – - Single model (e.g., Poisson distribution (λ)) governing next packet arrival time. - Two independent models to characterize transmission time (e.g., as a function of file size) and reading time (e.g., using exponential distribution (λ)) in a sequential manner, instead of modelling jointly. - Packet delay budget (PDB): - Service-dependent PDB parameters, (i.e., different PDB parameters for different traffic flows), applicable to both fixed and variable packet sizes, where packets are dropped when packet delay exceeds PDB. Proposal 5: Consider existing VoIP model as the baseline for performance evaluation during 6GR study for use case scenarios with ‘voice-only’ traffic. Proposal 6: Consider the following enhancements to the existing VoIP model for performance evaluation during 6GR study for use case scenarios with mixed traffic (voice + data) – - Mixed traffic pattern: - Combine voice and FTP data generation into a single model instead of separate models for voice/FTP traffic flow generations. - Variable packet size, data rate and inter-arrival time: - Payload size modelling, depending on traffic pattern (voice-only vs. mixed) and service-based requirements (e.g., advanced voice quality for video call/gaming vs. regular voice quality for audio-only phone call. - Data rate modelling depending on the heterogeneity of traffic patterns and traffic volumes. - Burst/silence patterns modelling depending on the traffic types and QoS requirements. - Flexible codec and QoS parameters: - Adaptive codec instead of static voice codec (e.g., G.729A), catering to different traffic types (mixed traffic vs. voice-only traffic). - Variable latency, jitter, and packet loss requirements to cater to mixed traffic. |
| Ericsson | Proposal 4-1-1 - For 6GR evaluations, support the following framework for extension to FTP Model 1 - UEs are categorized into multiple (e.g., 2 or 3) classes - Each UE class c consumes a fraction (αc) of the total traffic, where cαc=1 - Session size is set as Sc for UE’s of class c (each UE generates only one session) - Packet within a session have size Pc - Sc/ Pc packets are generated in each session - Inter-arrival time between different packets in the session is Tp - The arrival rate λc for UE’s of class c is set based on the traffic fraction αc and the total offered load - A packet delay bound can be defined for at least some of the UE classes (e.g., for small packets) - Note1: For simplified evaluations, the packet size Pc can be set to be same as session size Sc (i.e., only one packet per session can be simulated). - Note2: For further simplified evaluations,the packet size Pc can be set to be same as session size Sc, and only only class can be assumed, in which case the model would be same as FTP Model 1 used in LTE/NR evaluations. Proposal 4-1-2 - For 6GR evaluations, support the following framework for extension to FTP Model 3 - UEs are categorized into multiple (e.g., 2 or 3) classes - Each UE class c consumes a fraction (αc) of the total traffic, where cαc=1 - Sessions of each UE arrive according to Poisson distribution with mean inter-arrival time T - Session size is set as Sc for UE’s of class c - Packets within a session have size Pc - Sc/ Pc packets are generated in each session - Inter-arrival time between packets in the session is Tp - Number of UEs dropped in the simulation for class c is set based on the traffic fraction αc and the total offered load - A packet delay bound can be defined for at least some of the UE classes (e.g., for small packets) - Note1: For simplified evaluations, the packet size Pc can be set to be same as session size Sc (i.e., only one packet per session can be simulated). - Note2: For further simplified evaluations,the packet size Pc can be set to be same as session size Sc, and only only class can be assumed, in which case the model would be same as FTP Model 3 used in LTE/NR evaluations. |
| Ofinno | Observation 7: The existing FTP model 1 and FTP model 3 don’t guaranteed any target quality of service. Observation 8: In reality the file size shared between a UE and an FTP server can have large range. Proposal 1: Introduce variable packet size in both FTP model 1 and FTP model 3. The variable packet size is statistically selected from two or more file sizes. Proposal 2: Associated each packet in both FTP model 1 and FTP model 3 with a packet delay budget (PDB), which depends on the packet size. |
| Qualcomm | Proposal 6: For extension of FTP model 3 for enabling mixed packet sizes, consider the following modeling - Total number of UEs per cell is fixed, e.g., 10 - Each UE has multiple bursts that arrive according to a Poisson process. A single arrival rate (or mean inter-arrival time between bursts) is used for all the UEs, determined based on the target offered load (resource utilization ratio). - Each burst contains N≥1 packets with mixed and variable sizes where N can be common for all UEs, or different based on the setting. - Two or three packet sizes are predefined. The packet sizes are chosen so that the large packet size is much larger than the smaller packet size. For example, the large packet size is 0.5 Mbytes and the smaller packet size is several hundred Kbits. - Packets within the burst are separated by a short interval T, typically between 0.5 and 1 ms. The interval can be fixed during the simulation - For N packets within a burst, UE chooses the packet size independently for each packet according to a predefined probability αi, i.e., iαi=1. The same probability distribution is used for all UEs. |
| DOCOMO | Proposal 5 • Regarding FTP model 1/3, no extension is necessary. |

### Discussions

Background

The last meeting agreed to study extensions to FTP Model 1/FTP Model 3 to incorporate multiple packet size and PDB related as follows:

Agreement Study extensions to FTP Model 1/FTP Model 3 to incorporate the following: - Multiple packet sizes and associated time-domain behaviors (e.g., inter arrival time) - FFS number of packet sizes (e.g., 2 or 3). - FFS whether to have fixed or variable packet size and packet arrival rate for a given UE. - FFS applicability of multiple packet sizes to only one or both of FTP Model 1/FTP Model 3. - FFS packet size and arrival rate characteristics. - Packet delay budget (PDB) related parameters - FFS PDB applicability to packets (e.g., one PDB parameter for only one traffic flow or different PDB parameters for different traffic flows). - FFS how to consider the PDB, e.g., whether to drop packets when exceeding the budget, PDB aware metric. - Note consider the following for PDB: - Applicability to the extension to FTP Model 1/ FTP Model 3 with one packet size. - Applicability or not to the extension to FTP Model 1/ FTP Model 3 with multiple packet sizes.

Observations from the submitted paper for this meeting

14 companies (Futurewei, Nokia, vivo, CMCC, ZTE, NEC, Interdigital, Huawei, Samsung, Intel, NVIDIA, Ericsson, Ofinno, Qualcomm) discussed how to extend the FTP Model 1/FTP Model 3 to incorporate the multiple packet sizes and/or the PDB related consideration. 3 companies (Nokia, vivo, Huawei) explicitly proposed to extend FTP Model 3 only and a few other companies also implicitly prioritized FTP Model 3 extension as well.

Regarding PDB

11 companies (Futurewei, Nokia, vivo, CMCC, ZTE, NEC, Huawei, Intel, NVDIA, Ericsson, Ofinno) discussed and proposed to consider PDB as part of the model. 1 company (Samsung) proposed to further clarify what kind of services under what conditions require a need of introducing additional PDB in FTP model for 6GR evaluation.

- Behaviour when packet extending the PDB

  - Packets are dropped: Futurewei, CMCC (UDP-based), ZTE, Huawei, Intel, NVDIA

  - Packets are transmitted: CMCC (TCP-based)

  - Up to company to report: vivo

  - Not part of model up to RAN2 to decide: Nokia

- PDB awareness metrics due to PDB

  - Report packet drop rate: Futurewei

  - Report the percentage of the transmitted packet fulfilling PDB requirement: CMCC, NEC

  - Report the number of served bits (possibly zero) of an unfinished packet divided by the served time (simulation end time - file arrival time): ZTE

  - UE packet throughput considering the three aspects: Intel

- Applicability to the number of packet size

  - At least one packet size: Futurewei, vivo, Huawei

  - Multiple packet sizes: Futurewei, vivo

  - Applicable to small packets: Ericsson

- PDB values:

  - Combinations with inter-arrival time: Huawei, ZTE

  - Particular values: vivo [20ms, 50ms, 100ms]

Regarding multiple packet sizes

How to support multiple packet sizes for the traffic modelling, divergent ideas are observed from the contributions, concerning

- Whether the multiple packet sizes are supported per cell or per UE

  - Per cell: Nokia, Samsung, Intel, Ericsson

  - Per UE: Futurewei, Nokia, vivo, ZTE, Huawei, Qualcomm, Samsung

- How many packet sizes

  - Two: Nokia, vivo, Intel, Qualcomm

  - Three: Futurewei, ZTE, Qualcomm

In particular,

- Nokia: Proposed the support based on two independent Poisson process. A natural number K balances the packet size ratio and the packet arrival ratio of the two Poisson process generators.

- CMCC: Proposed to take ETSI TS 103 786 as the starting point to model the number of packet sizes and the packet generation rate regarding the specific values and probabilities.

- Interdigital: Proposed to vary the offered traffic load for different UEs based on geometry. The more estimated throughput for the user the greater number of packets are generated for the user. All UEs will have same packet size.

- Samsung: Proposed to model variable packet sizes corresponding to small, medium and large packet sizes along with associated packet arrival rates. The proportion of the packet arrival rates should be maintained to ensure that smaller packets are occurring much more frequently than larger ones.

- NVIDIA: Proposed to extend FTP Model 1 with at least 3 different file sizes corresponding to small, medium and large packet sizes and at least 3 different user arrival rate ranges corresponding to the associated packet size. Proposed to extend FTP Model 3 with two variants for file sizes (fixed value and variable size following lognormal distribution) and two variants for the inter-arrival time (single Poisson or independent modes).

- Ericsson: Proposed the framework for extension to FTP Model 1 and the framework for extension to FTP Model 3 illustrated as later.

- Qualcomm: Proposed a flexible and scalable model to evaluate general burst traffic. a single parameter (i.e., the number of packets per burst) could control the UE common or specific traffic profile evaluation illustrated as later.

Two illustrated models from the submitted paper for this meeting

- The modelling from Ericsson

Figure 2.4.1.1-1 – Options for extending FTP Model 1

- eFTP1-option1 is the simplest possible extension to FTP Model 1 to incorporate multiple packet sizes. A couple drawbacks of this approach are 1) it does not provide sufficient flexibility to group packets into bursts and 2) it does not allow sufficient flexibility to set appropriate packet sizes.

- eFTP1-option2 addresses the above drawbacks without increasing simulation complexity.

Figure 2.4.1.2-1 – Options for extending FTP Model 3

- Similar to eFTP1-option2, FTP Model 3 can also be further extended to have more flexibility in packet sizes using eFTP3-option2 shown below.

- eFTP3-option2 provides more flexibility to group packets into bursts (sessions) and also more flexibility to set packet sizes. For example, the small packet size can be set to a few 10s or 100s of bytes (doing the same for eFTP-option 3 results in too many UEs for a given offered load). For evaluations that do not require such flexibility, the packet size P can be set to be same as session size S (i.e., only one packet per session), in which case eFTP3-option2 would be same as eFTP3-option1 discussed above. Finally, it should be noted that the model can be further simplified when needed by using only one session class, in which case the model would be same as FTP Model 3 used in LTE/NR evaluations.

- The modelling from Qualcomm

- The proposed model offers a flexible and scalable way to evaluate general burst traffic, and a single parameter (i.e., the number of packets per burst) could control the UE common or specific traffic profile evaluation.

Figure 4: Extended FTP model 3 for general burst traffic with multiple packets having variable sizes

- To implement the extended FTP model 3, the first step is to define multiple packet sizes. There are two main approaches to this:

  - One option is to treat packet size as a random variable drawn from a predefined distribution—such as a truncated Gaussian, which is commonly used in XR traffic models.

  - The other approach is to define a fixed set of packet sizes (e.g., small, medium, and large) and assign a selection probability to each.

- Next is to define the packet (or burst) arrive rate for the UE. The packet (or burst) arrival rate can be a single value shared across all packet sizes or vary depending on the size. While it’s reasonable to expect smaller packets to arrive more frequently than larger packets, using multiple arrival rates adds complexity to the evaluation. Specifically, it becomes harder to determine the right combination of arrival rates to achieve a target load level. we prefer using a single packet arrival rate across all packet sizes to keep the model manageable while still capturing key aspects of traffic diversity.

Round-1 Discussions:

(FL1) Proposal 4.3.2-1

Regarding extensions to FTP Model 3 to incorporate the packet delay budget (PDB),

- The latency requirement of the traffic in RAN side (i.e., air interface) is modelled as packet delay budget (PDB). The PDB is a limited time budget for a packet to be transmitted over the air from a BS to a UE for DL, or from a UE to a BS for UL.

- For a given packet, the delay of the packet incurred in air interface is measured from the time that the packet arrives at the BS to the time that it is successfully transferred to the UE for DL, or from a UE to a BS for UL. If the delay is larger than a given PDB for the packet, the packet is said to violate PDB, otherwise the packet is said to be successfully delivered.

- One PDB parameter applies to only one traffic flow modelled.

  - Candidate values are chosen from {10ms, 100ms, 200ms, 300ms, 1000s}

- If the delay is larger than a given PDB for the packet,

  - The packet is dropped.

  - UE packet throughput accounts all three types of packets,

    - Successfully transferred during the simulation time,

    - Dropped during the simulation time,

    - Unfinished during the simulation time.

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| vivo | For the candidate PDB values, we recommend adding 20ms and 50ms, mainly considering that some services are delay-sensitive (refer to TS 23.501). Meanwhile, we suggest removing 10ms and 1000s: the former is rarely used in actual services, while the latter is excessively large. |
| ZTE | The candidate PDB values depend on packet size, it’s too early to determined the candidate values of PDB at this stage. Moreover, dropped packets should not be counted in the UPT results. |
| NTT DOCOMO | We failed to see the necessity of enhancing FTP model. we generally think that the candidate study points are similar to or the same as those of single stream of existing XR traffic model. |
| Ericsson1 | - Our understanding is XR traffic models will cover evaluations related to video traffic while the FTP Model 1/FTP Model 3 extensions will be used for more general traffic not limited to video. - PDB is already used in XR traffic models. Then, regarding the question of how/why PDB should be used for FTP Model 1/3 extensions, we think it could be a suitable metric for small packets. - The discussion should consider extensions to both FTP1 and FTP3. FTP1 is generally used for MIMO related evaluations and FTP3 has been used for energy efficiency evaluations. It is important to reflect the impact of different packet size for MIMO evaluations like MU-MIMO, hybrid beamforming, frequency domain scheduling, etc. |
| CMCC | Support the first two main bullets. For the third main packet, we support the main sentence while the specific values for PDB can be further discussed. For the fourth main packet, we believe that the behavior when delay is larger than PDB is belong to the specific service type. For instance, for TCP-based service (e.g. file download, web browsing, mail service, etc) , the packet shall be transmitted or retransmitted with respect to sliding window mechanism. For UDP-based service (e.g. online meeting, video service), however, the packet can be dropped considering the best-effort strategy. |

Round-1 Discussions:

(FL1) Proposal 4.3.2-2

Regarding extensions to FTP Model 3 to incorporate multiple packet sizes, further discuss the following options:

- Option1: Fixing the numbers of UEs per cell in each simulation drop, [2, 3] independent traffic flows are modelled for each UE. Each of the traffic flow is modelled by FTP Model 3 with a fixed packet size S, a fixed inter-arrival rate λ, [and a PDB value].

  - The [2,3] sets of (S, λ) correspond to Low, Medium, Large traffic loads.

    - FFS detailed values of (S, λ).

  - Option1-1: [2, 3] independent traffic flows are transmitted for each UE during the simulation drop.

  - Option1-2: Each UE selects one traffic flow once, and the traffic flow are transmitted for the UE during the simulation drop.

    - FFS selection is random or others

- Option2: Fixing the numbers of UEs per cell in each simulation drop, [2, 3] independent traffic flows are modelled per cell.

  - Option2-1: UEs are categorized into multiple (e.g., 2 or 3) classes

    - Each UE class c consumes a fraction (αc) of the total traffic, where cαc=1

    - Packets of each UE arrive according to Poisson distribution with mean inter-arrival time T

    - FFS: Session of multiple Packets for UE’s of class c

      - Packets within a session have size Pc

      - Sc/ Pc packets are generated in each session

      - Inter-arrival time between packets in the session is Tp

    - Number of UEs dropped in the simulation for class c is set based on the traffic fraction αc and the total offered load.

    - FFS: A packet delay bound can be defined for at least some of the UE classes (e.g., for small packets)

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| vivo | We don’t think we should go to option 2. - In real world, it is not practical to distinguish a limit number of types of UEs by service traffic details. In real world, a UE will have multiple traffic types ongoing at a given time. To reflect the UE traffic in real world for FTP model 3, it is necessary to have variable packet size and packet arrival rate for a given UE. - Further, option 1 is a super set of option 2. If option 1 is supported, Option 2 can be naturally achieved. Hence there is no point to have option 2 as an option here. For details in Option 1: Why is it necessary to use 3 flows? Can't 2 flows reflect the traffic phenomena? |
| ZTE | We support option 2, and the option 1-2 appears to be similar to option 2. |
| OPPO | As discussed in our contribution, we support multiple packet sizes for FTP model 3. We fail to find the difference between Option 1-2 and Option2, though Option 2 has more details. Furthermore, if multiple traffic flows are modelled per UE, we think two classes are sufficient. |
| MediaTek | Detailed clarification for option 1-1 is needed, e.g., if the independent traffic flows are transmitted for each UE during the same simulation drop, are they parallel or with a certain order? |
| Ericsson1 | - Similar to previous comment, the discussion should consider extensions to both FTP1 and FTP3. FTP1 is generally used for MIMO related evaluations and FTP3 has been used for energy efficiency evaluations. It is important to reflect the impact of different packet size for MIMO evaluations like MU-MIMO, hybrid beamforming, frequency domain scheduling etc. - We prefer the direction of option2-1. We are, however, open to considering multiple packet sizes for same UE for FTP Model 3. |
| CMCC | From NW side, we thought that both options can realize the mixed-traffic characteristic. But form UE side, however, option 1 is more fit into reality. - Furthermore, we believe that two sub-options within Option 1 are workable and can reach the similar effect. We are open for both sub-options. |

Round-3 Discussions:

#### (FL3) Proposal 4.3.2-1rv1

Regarding extensions to FTP Model 3 to incorporate the packet delay budget (PDB),

- The latency requirement of the traffic in RAN side (i.e., air interface) is modelled as packet delay budget (PDB). The PDB is a limited time budget for a packet to be transmitted over the air from a BS to a UE for DL, or from a UE to a BS for UL.

- For a given packet, the delay of the packet incurred in air interface is measured from the time that the packet arrives at the BS to the time that it is successfully transferred to the UE for DL, or from a UE to a BS for UL. If the delay is larger than a given PDB for the packet, the packet is said to violate PDB, otherwise the packet is said to be successfully delivered.

- One PDB parameter applies to only one traffic flow modelled.

  - Candidate values are chosen from [{20ms, 50ms, 100ms, 200ms, 300ms, 1000ms}]

- If the delay is larger than a given PDB for the packet,

  - The packet is dropped.

  - UE packet throughput accounts all three types of packets,

    - Successfully transferred during the simulation time,

    - Dropped during the simulation time,

    - Unfinished during the simulation time.

#The proposal is the same for FL3, Companies can further comment if not done yet. Offline time is needed for at least clarifications from the proponents#

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| Nokia | Packet dropping is not a traffic model characteristic. Packet discarding is a PDCP protocol feature, and the traffic model should not model RAN features. It is up to RAN2 to define how packet discarding, say e.g. timer-based packet discarding, is handled in 6GR. We are of course okay to include potential Packet Discarding effects in 6GR simulations, but it should not be embedded as part of the Traffic Model, and RAN2 should naturally be consulted so RAN1 model potential Packet Discarding inline with how this functionality will be defined for 6GR. |
| Interdigital | It would be good to clarify what it means to “accounts UE packet throughtput”. From our understanding the bullet and sub-bullets are asking companies to provide information for all three types of packets, and do not mean to combine the results into the final throughput results. |
| Qualcomm | PDB values are dependent on application and traffic types. It is hard to define the values without any assumption on the traffic types. A large PDB value, e.g., hundreds of millisecond may not be meaningful. Regarding whether packet is dropped when exceeding PDB limit, we prefer company to report. Also we want to clarify the dropping rule here is only for evaluation purpose and will not reflect actual system behavior. |
| Samsung | We failed to see strong need to incorporate PDB for evaluating services following FTP protocol, which are not either delay sensitive or with massive connections. In addition, very high load scenarios typically should be avoided in simulations as real networks are unlikely to operate at loads too high to threaten network stability, otherwise full buffer traffic model should be used instead for high-load scenarios. Therefore, incorporating PDB in FTP model 3 seems not so meaningful but just imposes more complex evaluations. |

#### (FL3) Proposal 4.3.2-2rv1

Regarding extensions to FTP Model 3 to incorporate multiple packet sizes, further discuss the following options:

- Option1: Fixing the numbers of UEs per cell in each simulation drop, [2, 3] independent traffic flows are modelled for each UE. Each of the traffic flow is modelled by FTP Model 3 with a fixed packet size S, a fixed inter-arrival rate λ, [and a PDB value].

  - The [2,3] sets of (S, λ) correspond to Low, Medium, Large traffic loads.

    - FFS detailed values of (S, λ).

  - [2, 3] independent traffic flows are transmitted for each UE during the simulation drop.

- Option2: Fixing the numbers of UEs per cell in each simulation drop, [2, 3] independent traffic flows are modelled per cell.

  - UEs are categorized into multiple (e.g., 2 or 3) classes

    - Each UE class c consumes a fraction (αc) of the total traffic, where cαc=1

    - Packets of each UE arrive according to Poisson distribution with mean inter-arrival time T

    - FFS: Session of multiple Packets for UE’s of class c

      - Packets within a session have size Pc

      - Sc/ Pc packets are generated in each session

      - Inter-arrival time between packets in the session is Tp

    - Number of UEs dropped in the simulation for class c is set based on the traffic fraction αc and the total offered load.

    - FFS: A packet delay bound can be defined for at least some of the UE classes (e.g., for small packets)

#The proposal is the same for FL3, Companies can further comment if not done yet. Offline time is needed for at least clarifications from the proponents#

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| Nokia | Quite many variants of enhanced FTP-1 and FTP-3 models are proposed. Perhaps RAN1 needs to take a step back and ask the question of what is gained from all these enhanced models? Will the conclusions of what are the best RAN1 solutions for e.g. 6G MIMO, frame structure, channel coding, etc. change whether RAN1 perform the evaluations with todays base line FTP-3, or any of the proposals for enhanced FTP-1/FTP-3? Our expectation is that RAN1 will be able to draw solid conclusions on what are the best 6G PHY solutions based on simulations with today’s baseline FTP-3 model (complemented with XR models for certain cases), so there is likely little to gain from RAN1 perspective to develop and initiate more simulations with enhanced FTP models. Note that in addition to e.g. FTP-3, we will have XR models with variable frame sizes and semi-static arrival (as compared to fixed file-sizes and fully random arrival for FTP-3). So in our view, such models are sufficient for RAN1 to draw conclusions for PHY design options. |
| Interdigital | It would be good to address other options that were discussed by companies. Other options would be - Option 3) For each UE, determine traffic flow from [2, 3] traffic class. - Option 3A) Each traffic class consists of same packet size, but different arrival rate. The arrival rate for different traffic class is weighted such that weighted sum of arrival rate equals target packet arrival rate. - Option 3B) Each traffic class consists of different packet size, but same arrival rate. The packet size for different traffic class is weighted such that weighted sum of packet size is equal to target packet packet size. - Option 3C) Each traffic class consists of different packet size and different arrival rate. |
| Qualcomm | To our understanding, the difference between option 1 and option 2 is that option 1 assumes multiple traffic flows per UE while option 2 assumes multiple traffic flow per cell but single traffic flow per UE. If it is correct understanding, we want to know whether both options would be considered or only one will be down-selected as extension of FTP model 3. For option 1, we think the independent assumption for multiple traffic flows per UE is not necessary. We can just simplify say that multiple traffic flows are modeled per UE. Whether there is dependency between multiple traffic flows per UE could be further discussed. Also, it would be also possible that not all the UEs in the cell have multiple traffic flows. Some UEs may have multiple flows while others may have a single flow. Below are proposed revised option 1. - Option1: Fixing the numbers of UEs per cell in each simulation drop, [2, 3] multiple traffic flows are modelled for each UE. Each of the traffic flow is modelled by FTP Model 3 with a fixed packet size S, a fixed inter-arrival rate λ, [and a PDB value]. - The [2,3] sets of (S, λ) correspond to Low, Medium, Large traffic loads. - Option1-1: Multiple traffic flows have independent (S, λ). - FFS detailed values of (S, λ). - Option1-2: Multiple traffic flows have a single (or dependent) arrival rate λ but different packet sizes. - FFS where packet sizes are also dependent - FFS whether all UEs have the same number of traffic flows, or UE could select one or multiple traffic flows during the simulation drop For option 2, we want to clarify what is relevant traffic type for the modeling of session with multiple packets? And why multiple packets in a session are assumed to have the same size? We want to know the referred traffic type behind this assumption. For option 2, it is assumed that the packet size and arrival rate are fixed (or predefined) and only the number of UEs dropped for each class is determined based on the total offered load. We think it is not aligned with existing FTP model 3 where the number of UEs are fixed (or predefined) while the packet arrival rate is determined by the offered traffic load. We prefer to keep the same principle for the extension of FTP model 3 for mixed packet size. Lastly we want to propose our general model which offers a flexible and scalable way to evaluate general burst traffic, and a single parameter (i.e., the number of packets per burst) could control the UE common or specific traffic profile. We propose the following option for discussion - Option3: Fixing the numbers of UEs per cell in each simulation drop, multiple traffic flows (e.g., 2 or 3) are modelled per cell and independently configured for each UE. - Each traffic flow is modelled by a different packet size Si and a traffic fraction αi. - The different packet size Si correspond to Low, Medium, Large traffic load. - FFS detailed values of S. - Session of multiple Packets for each UE arrive according to Poisson distribution with mean inter-arrival time T - Nc packets are generated in each session - FFS Nc is common or different for each UE - Packet size is determined based on the traffic faction αi, where iαi=1 - Inter-arrival time between packets in the session is Tp |

## New model 4-bidirectional traffic

### Companies’ views

| Company | Views/proposals |
| --- | --- |
| Nokia | Proposal 22: RAN1 to include at least one closed-loop (bi-directional) traffic model into the 6GR study, e.g., to consider a scenario when the existing XR model is modelled with realistic TCP ACK feedback. Proposal 23: Default assumptions for simulation with TCP functionality shall be agreed for cases where this is enabled. Our suggestion is to rely on TCP CUBIC. Proposal 24: For simulations with TCP, we suggest that the transmission of the TCP-ACK over the RAN is simulated in UL explicitly, and possible CN latency is added to that. a. For cases where CN delay plays a role, bi-direction model can assume a fixed one-way CN delay of e.g. 5 ms, representing the latency between the application (traffic source/sink) and the gNB. b. Such parameters as L1/L2 processing latencies, HARQ processing, preparing and decoding of transmission, assumption on TTI size, etc. should not be a part of the traffic model definition, but simulation assumptions. |
| vivo | As for whether to evaluate the impact of bidirectional traffic flows, from the perspective of capacity and energy efficiency evaluation, bidirectional traffic can better reflect actual condition than unidirectional traffic. Good performance in one direction does not necessarily mean the system is functioning optimally. Therefore, it is essential to support the evaluation of bidirectional traffic in certain use cases, where UE and NW sides need to be considered jointly. |
| CMCC | Proposal 7: For traffic model on bidirectional traffic flow, RAN1 first discuss and clarify the following issues: - The delay modeling on core/transport/internet network, i.e., whether this fixed delay as 5~10ms is proper. - The impact on slow start, i.e., how much impact will be reflected on slow start by TCP ACK considering that majority transmission is small packet and large initial congestion window size in current TCP protocol. |
| ZTE | Proposal 4-4-1: Whether to model the TCP slow start mechanism in traffic models still requires sufficient justification. |
| Huawei | Proposal 18: Modelling the impact of bidirectional traffic flows caused by the TCP ACK needs to be well justified first. |
| Ericsson | Proposal 4-2 - 6G system simulation methodology should include realistic modelling of bidirectional traffic flows by considering impact of TCP slow start and TCP ACK latency on throughput. - In DL system simulations the UL TCP ACK delay can be modeled by combining the two components below - Component 1: Fixed delay (e.g., 5-10ms) to reflect Core/transport/internet network delays - Component 2: SR+ UL grant+UL transmission delay. - Suitable values can be chosen based on HARQ RTT, SR availability and TTI length for the corresponding evaluation - Ideal transmission of UL TCP ACK can be assumed instead of explicitly simulating UL. |
| Samsung | RAN1 does not consider to model the impact of TCP/IP protocol for 6GR evaluations in RAN1. |

### Discussions

Background

The modeling for bidirectional traffic was not discussed in the last meeting. The following are excerpted from the last meeting FLS#5 in R1-2507957.

(FL1) Question Whether to define a bidirectional traffic flow modelling with simplified modelling as follows: - In DL system simulations the UL TCP ACK delay can be modeled by combining the two components below - Component 1: Fixed delay (e.g., 5-10ms) to reflect Core/transport/internet network delays - Component 2: SR+ UL grant+UL transmission delay. - Suitable values can be chosen based on HARQ RTT, SR availability and TTI length for the corresponding evaluation - Ideal transmission of UL TCP ACK can be assumed instead of explicitly simulating UL. Company; Comments. ZTE; Before discussing how to model this, it is better to fully justify whether this issue has a significant impact on system performance. For example, the existing protocols already support a relatively large congestion window.. CMCC; We are generally fine with component 2. For component 1, we are not sure whether a fixed delay with 5-10ms is proper. Normally, the delay within core network and/or internet is much larger than the delay in access network. It is recommended to have some further discussion on this.. vivo; From the perspective of capacity and energy efficiency evaluation, bidirectional traffic can better reflect actual condition than unidirectional traffic. Good performance in one direction does not necessarily mean the system is functioning optimally. Therefore, it is essential to support the evaluation of bidirectional traffic in certain use cases, where UE and NW sides need to be considered jointly. If companies have doubts about the first component, at least the second component should be taken into account in the future simulation.. Samsung; The justification to have the proposed simplified modelling is not clear to us for now.. DOCOMO; We can accept simplified modeling as in Ericsson proposal.. Google; We should be prudent in making assumptions regarding higher layer protocols. The impact of evolving transport layer protocols is complex, and we should focus on Layer 1/2 aspects within RAN1 scope. Qualcomm; It is not clear to us whether the component 2 value is fixed or dynamic during the evaluation. If it is dynamic, how is the value decided... Huawei; Agree with ZTE proposals. We should justify this issue firstly when the existing protocols already support a relatively large congestion window.. Nokia1; We support having a case defined where the core network (CN) delay is fixed to a certain value (e.g. 10 ms), including effects of TCP flow-control and potential other retransmission mechanisms such as RLC AM, and of course HARQ. We further furthermore assume that RAN processing times (e.g. for HARQ, Scheduling) are explicitly modelled in SLS, including aspects such that ACK/NACKs in UL are sent only during slots with UL transmissions, as well as the same for CSI reports, SRS, TCP ACK, RLC Status Report. In this respect, we think that term Simplified should be removed at this stage, and other options than just a single value at Stage 2 can be considred.. Apple; We have concerns on the proposal. TCP is not the only relevant transport layer protocol..

From the comments/views collected in the last meeting, some companies have concern to support it or not convinced yet. Some companies still have questions regarding the first component 1 or the component 2 but no discussion in the last meeting.

Observations from the submitted paper for this meeting

The proponent Ericsson proposed the same as in the last meeting. Others’ views (Nokia, ZTE, Huawei, CMCC, Samsung) are the same as in the last meeting.

Rationale about the modelling (inherited from the last meeting)

The bidirectional traffic discussed the last meeting was mainly talking about the TCP protocol from transport layer and its impact on the wireless network performance. The TCP protocol has a slow start mechanism and is used to prevent network congestion.

Specifically, as explained in a couple of companies’ contributions (ZTE, Ericsson) as well, when the TCP connection starts, the congestion window (cwnd) is set to a small initial value, typically measured in units of maximum segment size (MSS). For each ACK received, the congestion window increases by one MSS, which implies cwnd size increases exponentially. During the slow start process, the performance is latency limited. Only when the cwnd size stabilizes can the UE's data rate reach its maximum. This means that a packet transported using TCP consists of a sequence of data chunks, where the separation between packets is determined by the latency of the acknowledgment in the opposite link direction.

Since DL throughput depends on packet size divided by the total transmission time, the DL throughput will be affected by the UL latency. With the high data rates to be supported, the intervals between data chunks will be comparable to the time it takes to transmit the bits. This is the motivation from the proponent to consider the bidirectional traffic flow impact on wireless network performance.

Illustration of slow start mechanism of TCP protocol

Companies’ view on whether to consider this bidirectional traffic flow impact:

-     Most companies (Futurewei, CMCC, ZTE, Huawei, Apple, DOCOMO) view that it should be well justified or clarified first and be prudent in assuming a higher layer protocol to be considered for evaluating wireless network performance.

-    Specific concerns mentioned in companies’ contributions:

- Transport layer protocol is out of 3GPP and has been evolving as well in another standard origination. For instance, as mentioned that the TCP protocol nowadays allows a larger initial congestion window size, which can mitigate significantly the performance bottleneck caused by latency-limited slow start phases. (ZTE, Huawei)

- It is unclear and should be clarified how to consider the latency on RAN MAC/RRC/core network layer/etc., and how to model the impact on ACK/NAK for TCP layer. (CMCC)

Handling plan for this meeting

The same question is asked for checking the general support on the proposal by collecting views over the summary first and may seek a chance for discussion.

#### (FL1) Question

Whether to define a bidirectional traffic flow modelling as follows:

- In DL system simulations the UL TCP ACK delay can be modeled by combining the two components below

  - Component 1: Fixed delay (e.g., 5-10ms) to reflect Core/transport/internet network delays

  - Component 2: SR+ UL grant+UL transmission delay.

    - Suitable values can be chosen based on HARQ RTT, SR availability and TTI length for the corresponding evaluation

    - Ideal transmission of UL TCP ACK can be assumed instead of explicitly simulating UL.

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| vivo | We are open to define bidirectional traffic model, at least modeling Component 2 is beneficial for studying UL transmission latency. |
| NTT DOCOMO | We are fine to define the bidirectional traffic flow modeling based on the above. |
| CMCC | For component 2 we are fine, but for component we think that the value should be further discussed, e.g., whether a fixed value of 5-10ms is feasible considering normally the delay within core/transport/internet network is normally much larger than the delay in air interface. Another issue is the impact on slow start as HW/ZTE mentioned. As companies’ illustration, around 96% of transmission in real world is small packet [27] while the initial congestion window size in current TCP protocol is relatively large (e.g. 14.6 MB) [6], how much impact will be reflected on slow start by TCP ACK may need to be further clarified. |
| Nokia | We are okay to assume a fixed CN signaling delay the TCP-ACK. However, for the RAN part (Components 2), we recommend having realistic modeling. Such realistic modeling should e.g. include effects such that a TCP-ACK sent in the UL is only transmitted in UL slots, and is subject to certain transmission delays (i.e. subject to air interface processing times) |
| Qualcomm | We should be cautious when deciding to model the higher layer protocol for evaluating the wireless performance. But if RAN1 could agree on a simple approach on modeling the UL TCP ACK delay it would be also okay for us. For the proposed model, we want to know the details on the component 2, e.g., whether these suitable values are predefined and how they are selected based on HARQ RTT, SR availability and TTI length. |
| Huawei, HiSilicon | We have some doubts on the necessity of introducing bi-directional traffic flow. Firstly, it is not accurate to only consider TCP ACK based bidirectional traffic, given there are a lot of other traffic types that behave differently than TCP. Secondly, even for TCP ACK based bi-directional traffic, there may be additional TCP-layer optimization to change the overall DL TCP packet delivery to lower layers or from CN to RAN, which cannot be covered by this model. Thirdly, it is complicated to implement in the simulation. Compared with the existing statistical traffic model, considering the impact from UL TCP ACK delay on DL introduces a lot of complexity, which is not well justified. As we said in our paper, the motivation should be well justified, and the benefit on radio layer design that would require such a sophisticated traffic modeling should be clearly understood. |
| Xiaomi | Similar comments as HW, e.g., the motivation for introducing this model seems not that clear? Which aspects of differences will it verify, and what benefits will it bring to system design? In addition, there seems to be no unified understanding within RAN1 regarding the proposed UL TCP delay. Intuitively, this model will introduce additional evaluation complexity, so RAN1 needs to clarify the motivation and reach a unified understanding of related issues. |
| Samsung | We do not support to model the impact of TCP/IP protocol for 6GR evaluations in RAN1, considering: - TCP/IP is a non-3GPP standard protocol, if any problem cause by TCP/IP, it should not be addressed by 3GPP air interface design. Accordingly, PHY evaluations rarely consider the modelling of TCP/IP in the past proactively due to less knowledge, unless required by other upper layer working groups. - The concerned TCP/IP “slow-start” mechanism may result in data rate limit due to TCP ACK latency, but this impact is constrained in the very initial process. Even if there was some concern on “slow-start” due to sporadic small packet transmissions with short live connection, such concern should not be extended universally to other cases with long live connections. And with the evolved TCP/IP protocol allowing sufficiently larger configurable window sizes, this pain has been relieved to most extent. - Accurate modelling of TCP impact is difficult, as slow-start is not the only mechanism for congestion control in TCP/IP protocol. A simplified modelling TCP/IP latency with a fixed value in RAN1 without a comprehensive modelling of TCP/IP protocol may lead to biased or inaccurate assessment of its impact to throughput. - The complexity aspects of simultaneous simulation of DL and UL should be carefully considered. The modeling the frequency-domain consistency of the channels would be needed, while it is not yet well established in the current RAN1 channel models. |

## Traffic model for IoT

### Companies’ views

| Company | Views/proposals |
| --- | --- |
| Nokia | - Proposal 28: RAN1 to consider adoption of the following models for massive IoT: - a. Instant messaging traffic for IoT applications can be modeled using FTP model 3 with an assumed packet size of 0.1 MB, following the proposal in TR 38.840. - b. Software updates in massive IoT can be effectively represented by the FTP model 1, accommodating the transfer of larger files as typically required for such updates. |
| Sony | Observation 1 : All of the 6G IoT traffic models proposed in RAN1#122bis are relevant for IoT applications in the 6G era but it is necessary to identify which models would be relevant for IMT-2030 and 3GPP internal requirements evaluation. Proposal 13 : When discussing the traffic model, RAN1 shall also clarify the requirements that 6G massive IoT needs to be verified against. Observation 2 : Automous reporting traffic model can be used for connection density requirement and battery life evaluation for 6G massive IoT. Observation 3 : Remote actuation traffic model can be used to evaluate the power saving performance and / or the connection density requirement for 6G massive IoT. Observation 4 : A firmware / software update traffic model can be used to evaluate the minimum required data rate that 6G massive IoT needs to support. Observation 5 : The Network triggered polling / reporting traffic model may not be a bottleneck traffic model of 6G massive IoT to meet any corresponding requirement. Observation 6 : The FTP3 traffic model with suitably scaled packet size (e.g. 0.1Mbyte) and interarrival time (e.g 200ms) would be relevant for 6G-IoT. Proposal 14 : The following existing traffic models defined in Table 5 – d) in Report ITU-R M.2412 should be used for 6G-IoT performance evaluations: - With layer 2 PDU (Protocol Data Unit) message size of 32 bytes: 1 message/day/device - 1 message/2 hours/device Packet arrival follows Poisson arrival process for non-full buffer system-level simulation This traffic model can be applied in the UL or DL. Note: The above existing traffic models are applicable to Autonomous reporting and Remote actuation. Proposal 15 : 6G-IoT is evaluated against the ability to support a [1Mbyte] firmware / software update in [10 seconds] |
| Ericsson | Proposal 4-4-1 - Based on RAN#109 agreement to support massive communication in 6G, include traffic model(s) suitable for massive communications for 6G evaluations. - Include the following massive communication (IoT) traffic types for 6G evaluations: - Triggered/polled reporting - Autonomous reporting (event-driven or periodic) - Remote actuation - Firmware/software upgrade Proposal 4-4-2 - Adopt the traffic models in tables 2.4.4-1 to 2.4.4-4 for 6G massive communication use cases. Proposal 4-4-3 - For comparability with 5G results and verify that 6G can meet the IMT-2030 connection density requirements, the mMTC traffic model from IMT-2020 (TR 37.910) may be used as a starting point. |

### Discussions

Background

The traffic modelling for IoT was not discussed in the last meeting. The following are excerpted from the last meeting FLS#5 in R1-2507957 but not much discussion in the last meeting.

(FL1) Question Whether need to study a new traffic model for 6GR IoT performance evaluation, considering the following factors: - Triggered/polled reporting - Autonomous reporting (event-driven or periodic) - Remote actuation - Firmware/software update - FFS the values for traffic characteristics (e.g., packet size, inter-arrival time, number of users, mobility pattern) considering 6G massive communication use cases Company; Comments. ZTE; The different processes above clearly exhibit distinct traffic characteristics. We suggest describing them using either an extended FTP model (for random arrivals) or an XR model (for periodic arrivals). The discussion may only need to focus on the specific value of model parameters.. Ericsson1; Since RAN#109 decided to support Massive Communication (IoT) for FR1, corresponding traffic models should be considered. We think the factors in the list (triggered/polled reporting, event-driven or periodic autonomous reporting, remote actuation, and firmware/software update) cover the most important aspects..

Observations from the submitted paper for this meeting

3 companies (Nokia, Sony, Ericsson) discussed the traffic model for IoT related use cases. In particular,

  - The original proponent Ericsson proposed the detailed parameters for each of model targeting a specific use case for IoT.

  - Nokia proposed using FTP Model 3 with 0.1 MB packet size and FTP model 1 for Software updates.

  - Sony proposed RAN1 shall also clarify the requirements that 6G massive IoT needs to be verified against when discussing the traffic model for IoT. Proposed to use the existing traffic models defined in Table 5 – d) in Report ITU-R M.2412 should be used for 6G-IoT performance evaluations and 6G-IoT is evaluated against the ability to support a [1Mbyte] firmware / software update in [10 seconds]

Rationale about the modelling (inherited from the last meeting)

What traffic models were used in the past

-   In TR 37.910 (“Study on self-evaluation towards IMT-2020 submission”), mMTC uses a traffic model with layer 2 PDU (Protocol Data Unit) message size of 32 bytes and 1 message/day/device or 1 message/2 hours/device, where the packet arrival follows Poisson arrival process for non-full buffer system-level simulation.

-    In TR 36.888 (“Machine-Type Communications (MTC) User Equipments (UEs) based on LTE”), the following traffic types have been considered for mMTC (Annex A and Annex A.1):

- Triggered reporting (command-response traffic)

- Autonomous reporting (exception/event-driven reports or periodic reports)

-   In TR 45.820 (“Cellular system support for ultra-low complexity and low throughput Internet of Things”), the following traffic types have been considered (Annex E.2):

- Autonomous reporting (exception reports or periodic reports)

- Network command

- Software update

Why a new model is needed?

-    Although the traffic types described in the above TRs may still be relevant for 6G massive communication, the traffic characteristics (e.g., packet size, inter-arrival time) may not be representative of what is expected for 6G massive communication or what has been observed in real deployments of legacy massive IoT solutions.

What the new aspect needs to be considered for the new traffic model for IOT

The following traffic models can be considered as representative of the applications expected for 6G massive communications for evaluation purposes:

- Network triggered/polled reporting

  - Application layer in the network triggering an UL application payload from the device, e.g., for sensor reading.

- Device autonomous reporting (event-driven or periodic)

  - Event-driven: An UL application payload triggered by an event in the device delivered (within a certain latency target) and a DL application ACK, e.g., for outage notifications from sensors.

  - Periodic: Periodic UL reporting from a device and a DL application ACK, e.g., for regular sensor reading.

- Remote actuation

  - An application server generates an application layer command to the device to perform an action (in the physical world) with an UL application ACK received (within a certain latency target), e.g., for disconnecting devices or triggering an emergency shutoff.

- Firmware/software update

  - All 6G massive communication devices are expected to require occasional firmware and/or software updates (e.g., every few months). Although updates are expected rather occasionally, file sizes are expected to be relatively large (e.g., for new version release) and certain updates (e.g., security patches) may need to be delivered to selected UE groups withing a certain update campaign timeframe (e.g. within a few days or weeks). Based on the above discussion, we have the following proposals.

Handling plan for this meeting

Ericsson proposed the detailed parameters for each of IoT service. We can check the general support on the proposal by collecting views over the summary first and may seek a chance for discussion.

Round-3 Discussions:

#### (FL3) Proposal 4.5.2-rv1

For 6GR evaluations related to IoT traffic communication,

- The following traffic types are considered:

- Triggered/polled reporting

- Autonomous reporting (event-driven or periodic)

- Remote actuation

- Firmware/software upgrade

- The following traffic models in Tables 1-4 are considered accordingly.

- For comparability with 5G results and verify that 6G can meet the IMT-2030 connection density requirements, the mMTC traffic model from IMT-2020 (TR 37.910) may be used as a starting point. This traffic model can be applied in UL or DL.

Table 1 – Network triggered reporting

| Parameter | Characterization |
| --- | --- |
| Packet size | DL trigger: [150] bytes UL payload: [1000] bytes |
| Inter-arrival time | [1] report/hour/UE Packet arrival process follows uniform distribution |
| Number of UEs per cell | [50,000] |
| Mobility pattern | 70% stationary, 30% non-stationary |

Table 2 – Autonomous reporting

| Parameter | Characterization |
| --- | --- |
| Packet size | UL payload: [1000] bytes DL ACK: [100] bytes |
| Inter-arrival time | Periodic reporting: - [1] report/hour/UE - Packet arrival process follows uniform distribution Event-driven reporting: - Reports from [5%] of the UEs within a 1-minute window (message storm in case of an outage) |
| Number of UEs per cell | [50,000] |
| Mobility pattern | 70% stationary, 30% non-stationary |

Table 3 – Remote actuation

| Parameter | Characterization |
| --- | --- |
| Packet size | DL payload: [500] bytes UL ACK: [100] bytes |
| Inter-arrival time | [2] commands/day/UE Packet arrival process follows Poisson distribution |
| Number of UEs per cell | [50,000] |
| Mobility pattern | 70% stationary, 30% non-stationary |

Table 4 – Firmware/software upgrade

| Parameter | Characterization |
| --- | --- |
| Packet size | DL payload: [2] Mbytes, transmitted using FTP3 traffic model with [0.1Mbyte] packet size and [200ms] interarrival time UL ACK: [100] kbytes |
| Inter-arrival time | [4] upgrades/year/UE Upgrade [10%] of the UEs within a 12-hour (night-time) window |
| Number of UEs per cell | [50,000] |
| Mobility pattern | 100% stationary |

#The proposal was updated based on Sony’s comment for FL3, Companies can further comment if not done yet. Offline time is needed for at least clarifications from the proponents#

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| NTT DOCOMO | We are open to study. |
| Ericsson1 | Since RAN#109 decided to support Massive Communication (IoT) for FR1, corresponding traffic models should be considered. We think the factors in the list (network triggered/polled reporting, event-driven or periodic autonomous reporting, remote actuation, and firmware/software update) cover the most important aspects. In addition, we had the following proposal in our contribution: - For comparability with 5G results and verify that 6G can meet the IMT-2030 connection density requirements, the mMTC traffic model from IMT-2020 (TR 37.910) may be used as a starting point. |
| Qualcomm | We are open to study traffic models for IoT but want to clarify firstly the objective for IoT evaluation. Considering the sparse traffic property and low data rate, the required throughput and offered load will be very low for IoT. The effort for simulating 50000 UEs per cell with large packet interarrival time such as several hours or longer would be very high . |
| Sony | We support the inclusion of IoT traffic models in the 6GR study. We see two main uses for IoT traffic models: - evaluation against requirements (in particular the IMT connection density requirement) - Comparison of technical proposals in RAN WGs For evaluation against requirements (particularly connection density), we think that proposal 4-4-3 from Ericsson (which is consistent with our proposal 14) is needed. Hence, we think that the following proposal should be added: Proposal 4_5_2_addition - For comparability with 5G results and verification that 6G can meet the IMT-2030 connection density requirements, the mMTC traffic model from IMT-2020 (TR 37.910) may be used as a starting point. This traffic model can be applied in UL or DL. We support the traffic models in tables 1,2,3. We consider these to be the traffic models for the cell (as opposed to the usual traffic model for the UE) – and we think that these traffic models for the cell are important for IoT. These traffic models for the cell include the overall traffic load in the cell and the correlation of traffic in the cell (e.g. in the case of autonomous event-driven reporting). For the firmware / software upgrade traffic model of Table 4, we think it is important to also capture a model of how data is served to each UE. The firmware / software upgrade needs to be provided to the UE at a sufficient data rate to allow (1) the upgrade to be completed before an application layer timeout, (2) the upgrade to be completed before the UE battery is depleted and (3) the base station is able to ,complete upgrades quickly and then return to sleep in order to save network power and energy. Hence, we propose the following update to Table 4: Table 4 – Firmware/software upgrade Parameter; Characterization. Packet size; DL payload: [2] Mbytes, transmitted using FTP3 traffic model with [0.1Mbyte] packet size and [200ms] interarrival time UL ACK: [100] kbytes. Inter-arrival time; [4] upgrades/year/UE Upgrade [10%] of the UEs within a 12-hour (night-time) window. Number of UEs per cell; [50,000]. Mobility pattern; 100% stationary. |

# Link budget

## Link budget template for 6GR

### Companies’ views

| Company | Views/proposals |
| --- | --- |
| Nokia | - Proposal 29: RAN1 to use methodology formulated in TR 38.830 as framework and as a baseline. Simplifications can be applied, e.g., before parameters for LLS with 6G parameters are available. |
| vivo | Proposal 13: Adopt MCL as the metric for link budget evaluation and introduce band specific MCL target. - If the target of 3.5GHz is MCL0, the target of 7GHz is MCL0 + X, where X depends on pathloss including penetration loss difference and BS antenna array difference between 3.5GHz and 7GHz. - Use Candidate 1 as the link budget template. Confirm the definition of MCL in row 22bis. - For BS antenna array difference, include BS antenna array components 2, 3 and 4 in calculating X. - For the parameter values, - reuse the parameter values agreed in Rel-17 CE as much as possible for 5G baseline, e.g., the BS antenna array with 192 antenna elements and 64 TxRUs; - reuse the values agreed as general assumptions in 11.2 for 6GR, e.g., the BS antenna array with 2048 antenna elements and 512 TxRUs; - use common values for UE side parameters between 5G and 6GR. - RAN1 to decide a value between {7dB, 9dB} for UE noise figure. |
| CMCC | Proposal 15: Link level simulation can be carried out to evaluate the SINR requirements for the specific channel and traffic. Link budget template from Rel-16 CE and IMT-2020 self-evaluations can be considered as a starting point. Observation 1: The MCL defined in TR38.913 only relates to the transmit power and the receiving sensitivity at receiver. It cannot reflect the impact of gNB’s antenna and the additional propagation loss due to the higher operation frequencies. Proposal 18: MCL with modification based on Rel-16 definition can be considered as the coverage performance metric. The definition of MCL is proposed as follows: - MCL = Total transmit power – Receiver sensitivity + gNB antenna gain (considering all elements in one polarization). Observation 2：The template in TR38.913 does not explicitly consider the carrier frequency and the assumption of gNB’s antenna element numbers, which are both important for the coverage performance. Proposal 20: The link budget template in TR38.913 cannot be used for working group’s study. Proposal 21: The definition of MCL in row (22bis) can be defined as - Option 1: aligned with TR38.913, - MCL = Total transmit power – Receiver sensitivity - Option 2: with updates - MCL = Total transmit power – Receiver sensitivity + gNB antenna gain (considering all elements in one polarization). |
| ZTE | Proposal 3-1-1: For link budget template: - Candidate 1: Reusing the link budget template from TR38.830 with followings: - The values of the parameters are based on the agreed simulation assumption. - Reuse the definition of MCL row. - Delete the “(30) Maximum range (based on (29) and according to the system configuration section of the link budget) (m)” row - Candidate 2: Reusing the Template as Table 7.10.1-1 from TR38.913 without any update. Note: The “required SNR” in the two candidate templates should be derived from link‑level simulations based on the baseline assumptions before performing further calculations. |
| CATT | Observation 1: Link budget template in Candidate 1 is more suitable if metric(s) other than MCL is agreed to evaluate coverage performance. Proposal 9: For link budget template in Candidate 1, RE power control dynamic range specified in TS 38.104 can be introduced as (3d) and added in the calculation the total transmit power for occupied bandwidth in row (3bis). Proposal 10: For link budget template in Candidate 1, total antenna gain at antenna gain component 3 & antenna gain component 4 in row (11bis) should be removed from the calculation of MCL in row (22bis). Proposal 11: For link budget template in Candidate 1, maximum range in row (30) is needed to evaluate co-site between existing 5G mid-band and around 7 GHz. Observation 2: Using maximum transmit power for downlink channels occupying fewer PRBs may exceed the BS power boosting capability. Proposal 12: PSD constraint for downlink and antenna gain component 2 at gNB side should be introduced in link budget template in Candidate 2 if Candidate 2 is adopted. |
| OPPO | Proposal 6: Support Candidate 1 (MPL) for the determination of link budget template for RAN1 study. |
| Huawei | Proposal 19: Table 12 is used for 6G Link budget template, where the MPL should be used as the basic performance metric for coverage analysis, towards studying the coverage of around 7GHz co-site deployed with 5G mid-band. Provided the parameter values as in the Table. Proposal 20: Interference density/margin should be obtained by SLS based on different scenarios. CDL channel model with actual number of TxRUs should be used in LLS for link budget analysis, especially for massive MIMO scenario. |
| Ericsson | Proposal 5-1 • Support link budget template Candidate 2 based on Maximum Coupling loss (Table 7.10.1-1: MaxCL calculation template in 38.913) for the purposes of determining coverage target(s) for 6G. We see no need for updating this template; it can be used as is. Proposal 5-2 • Study adaptations to the 38.830 link budget template to improve consistency of parameter selection among companies while maintaining accuracy. • Limit the use of link budgets as a deciding factor in physical layer design to where link level simulations and link budgets can accurately identify the net performance benefit of design alternatives to the system. Proposal 5-3 • Reflect Maximum coupling loss (MaxCL) calculation as described in Table 7.10.1-1in 38.913 (candidate 2 template) also in the link budget template for candidate 1 |
| MediaTek | Observation1: For same band link budget analysis, both Candidate 1 and Candidate 2 can be utilized. Observation 2: For 3.5GHz VS 7GHz link budget analysis, Candidate 1: Reusing the link budget template from TR38.830 is more applicable. Proposal 3: From RAN1 perspective, adopt Candidate 1: Reusing the link budget template from TR38.830 for 6GR coverage evaluation as the baseline. NOTE: Following RAN Plenary discussion on targets and associated assumptions, some parameters in the template may become redundant for some scenarios. |
| Qualcomm | Proposal 20: Reuse the MCL definition in TR 38.830 for 6GR link budget template - MCL = Total transmit power – Receiver sensitivity + gNB antenna gain (component 2) Provided parameter values in the table. Proposal 21: RAN1 discusses and agrees on parameters values in link budget template for 6GR coverage evaluation |
| DOCOMO | Observation 3 - It would be efficient to discuss “MCL in row (22bis) for Candidate 1, FFS parts” in this agenda. Proposal 1 - No update is necessary for candidate 1 and candidate 2 of link budget table. |
| Tejas | In this contribution, we propose some additions to Candidate 1 template to compute available path loss and maximum range. We would like to incorporate target data rate with a specified BLER (19bis) for data channel and link it with the required SNR (19) of the table. Our proposed changes (highlighted text) are added to the Candidate 1 template below |

### Discussions

Background

Last meeting discussed the link budget template for the coverage analysis together with the performance metric to be used for the study for the co-site deployment comparable coverage between mid-band and the new carrier frequency for 6GR. The discussion was relevant to the coverage target, which was clarified to be decided by RAN plenary meeting first.

The discussion later on in the last meeting focused on the link budget template itself and would continue to be the focus of the discussion in this agenda, mainly regarding whether the template needs any update.

Observations from the submitted paper for this meeting

12 companies (Nokia, vivo, CMCC, ZTE, CATT, OPPO, Huawei, Ericsson, MediaTek, Qualcomm, DOCOMO, Tejas) followed up the agreement and discussed whether the updates are needed for the templates agreed in the last meeting. Particularly,

Regarding the updates to the templates

- Whether update to the Candidate2 template is needed

  - No change: ZTE, Ericsson, DOCOMO

  - Yes: CATT (PSD constraint for downlink and antenna gain component 2 at gNB should be introduced)

- Whether update to the Candidate1 template is needed

  - Change1: regarding MCL(22bis)

    - Yes: CMCC (considering all elements in one polarization or align with MaxCL from TR38.913)

    - No: ZTE, vivo, CATT, Qualcomm, DOCOMO

  - Change2: regarding FFS on row(30) Maximum range

    - Yes: ZTE (delete the row)

    - No: CATT, DOCOMO

  - Change3: Add RE power control dynamic range specified in TS 38.104 as (3d) and added in the (3bis)

    - CATT

      - According to the RE power control dynamic range in TS 38.104, the minimum requirement for BS power boosting is specified for PDCCH and PDSCH under different modulation schemes, respectively. Since it is the baseline BS capability specified in RAN4, RE power control dynamic range should be considered when calculating the total transmit power for occupied bandwidth in row (3bis).

  - Change4: Reflect Maximum coupling loss (MaxCL) calculation as described in Table 7.10.1-1in 38.913 (candidate 2 template) also in the link budget template for candidate 1)

    - Ericsson

  - Change5: Incorporate target data rate with a specified BLER (19bis) for data channel and link it with the required SNR (19) of the table

    - Tejas

Regarding which template is chosen:

- Candidate 1 (Table from TR38.830): Nokia, vivo, CMCC, OPPO, Huawei, MediaTek

- Candidate 2 (Table from TR38.913): Ericsson, MediaTek

Regarding the parameter values to the template

2 companies (Huawei, Qualcomm) provided the parameter values to the Candidate1 template. 2 companies (vivo, ZTE) proposed to reuse the parameter values agreed in Rel-17 CE as much as possible for 5G baseline and the agreed values for the common assumptions in this agenda

Handling plan for this meeting

This agenda for this meeting will focus on discussing the potential update to the templates.

#### (FL1) Proposal 5.1.2

Update the link budget template for Candidate 1 as agreed in the last meeting as follows:

- Row(19) is updated to:  (19) Required SNR (dB) = SNR for Target data rate for the allocated bandwidth (3c) (Downlink and uplink data channels) with Y% BLER (e.g, Y=10).

- Row(30) is updated to: removing ‘FFS’ but keep the row(30) as in the table.

- Add one more row as (22bis-a): Maximum coupling loss (MaxCL) calculation as described in Table 7.10.1-1in 38.913 (candidate 2 template)

- FFS: Add more row as (3d) that is added into (3bis): RE power control dynamic range specified in TS 38.104.

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| CATT | Thanks FL for the great efforts on the summary. One clarification is on our proposal for adding the RE power control dynamic range is that the range is introduced for IoT devices. Consider that it is agreed that the template for link budget is used for all device types, it is essential to add such a parameter, which is already clearly captured in RAN4 spec’s requirement. The values can be reported by companies individually. |
| ZTE | - For the 1st bullet: Negative. The proposed interpretation for the required SNR is only applicable for data channel, which is too restrictive. We prefer to leave it open to allow different definition for other channel/reference signal. - For the 2nd bullet: Negative. We prefer to remove this row since the row(30) Maximum range is not a comparable metric to define the coverage including comparison cross deployments. - For the 3rd bullet: Not clear about the intention. If It’s just to merge the two templates for simplicity, it seems fine. But since the metric selection is still discussed under 11.1 by directly citing the agreed two templates, no need to merge it now to avoid the confusion. - For the FFS: No additional row for the RE level power control is needed, which is more related to the specific solution. |
| NTT DOCOMO | We are generally fine with the proposal. |
| CMCC | For the 1st bullet, there is no need to update the row(19) in the candidate 1 template. Since with different traffic, such as data rate and VoIP, different target BLER would be used. The target BLER(Y%) can be referred to the evaluation assumptions, which would be discussed before the link budget calculation. For the last bullet, it is not clear to us why RE power control dynamic range should be considered here. |
| Qualcomm | For FFS, it is not clear to us how the new row 3d is used for compute 3bis “Total transmit power for occupied bandwidth”. |
| Nokia | The proposed interpretation of SNR is appropriate only for the data channels, e.g., PUSCH, PDSCH. Is the intent to consider only those for the coverage evaluation, and not to consider other channels such as PUCCH, PDCCH, RACH? |
| Tejas Networks | We support the 1st bullet. We can add note saying “target data rate is applicable only for data channels” similar to the notes given in other rows of the link budget table. For control channels, the required SNR is specified as earlier. |
| Sharp | We are OK with the proposal. |

## Target and metrics for coverage

### Companies’ views

| Company | Views/proposals |
| --- | --- |
| Futurewei | Proposal 10: For 6GR upper midband in at least around 7 GHz based on existing 5G mid-band site grid: • The coverage range (distance in meters) is the most direct metric for coverage analysis. • One single value in MCL/MIL/MPL as a general coverage requirement may not be sufficient, though MPL may be the most direct among the three quantities. o Certain frequency-specific conditions (e.g., antenna panel gain, pathloss equation, penetration margin, etc.) need to be provided or fixed for coverage analysis. |
| Nokia | Proposal 30: RAN1 should consider all metrics: MCL, MIL, and MPL because they are not interchangeable and provide evaluation of different aspects of coverage. |
| vivo | Proposal 13: Adopt MCL as the metric for link budget evaluation and introduce band specific MCL target. - If the target of 3.5GHz is MCL0, the target of 7GHz is MCL0 + X, where X depends on pathloss including penetration loss difference and BS antenna array difference between 3.5GHz and 7GHz. - Use Candidate 1 as the link budget template. Confirm the definition of MCL in row 22bis. - For BS antenna array difference, include BS antenna array components 2, 3 and 4 in calculating X. - For the parameter values, - reuse the parameter values agreed in Rel-17 CE as much as possible for 5G baseline, e.g., the BS antenna array with 192 antenna elements and 64 TxRUs; - reuse the values agreed as general assumptions in 11.2 for 6GR, e.g., the BS antenna array with 2048 antenna elements and 512 TxRUs; - use common values for UE side parameters between 5G and 6GR. - RAN1 to decide a value between {7dB, 9dB} for UE noise figure. |
| CMCC | Proposal 8: Compared with 5G, similar or better coverage performance can be considered for 6G design. Proposal 9: For the coverage enhancements and the link budget calculation, sharing the 5G NR site grid of 2.6GHz should be also considered. Proposal 10: Both urban macro and rural scenarios can be considered for coverage performance evaluation. And urban macro scenario has higher priority. Proposal 11：Both requirements of wideband traffic with high data rate and VoIP, LPWA traffic should be considered for the coverage performance evaluations. Proposal 12: 7GHz can be considered as main operation frequency for the coverage evaluations. Proposal 13: The coverage performance or link budget should be compared between 7GHz with 2.6GHz and 3.5GHz respectively. Proposal 14: Both uplink and downlink channels in the idle mode and connected mode should be considered for the coverage evaluations. Proposal 16: The definition of MCL in TR38.830 containing only parts of antenna elements or TXUR numbers cannot be used in the coverage performance evaluation, even with the consideration of reusing the network grid of NR. Proposal 17: It is proposed that considering the pathloss, penetration loss and shadow fading in total as propagation loss for the coverage performance metric. Proposal 19: The MIL in TR38.830 can be used for the coverage evaluations at least in the working group. |
| Huawei | Proposal 19: Table 12 is used for 6G Link budget template, where the MPL should be used as the basic performance metric for coverage analysis, towards studying the coverage of around 7GHz co-site deployed with 5G mid-band. |
| Samsung | Proposal #20: - RAN1 uses MCL as by Candidate 2 (without modifications) as methodology for link budget analysis to provide the initial analysis of potentially achievable coverage targets to RAN#110, i.e., confirm the 144-145 dB (or better) coverage target for 6GR. - RAN1 uses MPL based on Candidate 1 (without modifications) as methodology for the link budget coverage analysis to compare the achievable coverage for 6GR in 7 GHz and NR 3.4-3.9 GHz assuming the same/shared site deployment. - Companies can further consider modifications to MPL (e.g., across bands) or MIL (e.g., within a band) based on Candidate 1 as methodology for link budget coverage analysis or identification of coverage bottlenecks. |

### Discussions

Background

Last meeting discussed the link budget template for the coverage analysis together with the performance metric to be used for the study for the co-site deployment comparable coverage between mid-band and the new carrier frequency for 6GR. The discussion was relevant to the coverage target, which was clarified to be decided by RAN plenary meeting first.

The discussion later on in the last meeting focused on the link budget template itself and would continue to be the focus of the discussion in this agenda.

Observations from the submitted paper for this meeting

Some companies (Futurewei, Nokia, vivo, CMCC, Huawei, Samsung, etc.) analyzed and proposed which metric will be used for the coverage analysis, either for setting the coverage target or for RAN1 evaluations, e.g., identifying the ‘bottleneck’ channel or the supported ranging distance, etc.

Handling plan for this meeting

As suggested, this agenda will focus on the link budget template itself and the metric(s) selection issue is turned in to RAN plenary or for agenda item 11.1 for the auxiliary discussion.

#### (FL1) Moderator’s suggestion

The metric(s) selection issue for coverage analysis is turned in to RAN plenary or for agenda item 11.1 for the auxiliary discussion.

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| Qualcomm | We are okay with the FL proposal on the metric selection. One remaining issue with the link budget template is the parameter value. If possible, we prefer to have some agreement which would be helpful for the link budget evaluation. |

# Assumptions for NTN

## Carrier frequency for NTN

### Companies’ views

| Company | Views/proposals |
| --- | --- |
| Futurewei | Proposal 5: For 6GR NTN evaluations, adopt the frequency range in the Ku-band as specified for 5G NTN (as presented in Table 3 and Table 4): Table 3: Satellite FR1-NTN operating bands Satellite operating band; Uplink operating band; Downlink operating band; Duplex mode. n248; 14000 MHz - 14500 MHz; 10700 MHz – 12750 MHz; FDD. n247; 13750 MHz - 14000 MHz; 10700 MHz – 12750 MHz; FDD. Table 4: Satellite FR2-NTN operating bands Satellite operating band; Uplink operating band; Downlink operating band; Duplex mode. n509; 14000 MHz - 14500 MHz; 10700 MHz – 12750 MHz; FDD. n508; 13750 MHz - 14000 MHz; 10700 MHz – 12750 MHz; FDD. |
| Xiaomi | Proposal 5: The detailed frequency range is 11 GHz for DL, 14 GHz for UL for Ku band simulation. |
| LGE | Proposed 14GHz for UL and 12GHz for DL for Ku band simulation. |
| Sharp | Proposed for Ku band, RAN1 assumes 14 GHz for UL and 11 GHz for DL. |
| Ofinno | Proposed to consider Ku-band frequencies around 14/12 GHz (UL/DL) for 6GR NTN evaluations. |

### Discussions

Summary on the views:

4 companies (Xiaomi, LGE, Sharp, Ofinno) proposed the detailed carrier frequency for Ku-band for 6GR NTN evaluations.

- DL: 11GHz (Xiaomi, Sharp), 12GHz (LGE, Ofinno)

- UL: 14GHz (Xiaomi, LGE, Sharp, Ofinno)

Round-3 discussions:

#### (FL3) Proposal 6.1.2

For 6GR NTN evaluations, the carrier frequency for Ku-band is 14GHz for UL and 11GHz for DL.

#The proposal was the same for FL3, Companies can further comment if not done yet. #

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| Xiaomi | OK |
| Nokia | In general OK, but we should also target for a single number for SCS to apply for this band. |
| Sharp | We support the proposal. |

## Antenna modelling for NTN

### Companies’ views

| Company | Views/proposals |
| --- | --- |
| ZTE | Proposal 2-6-1: 6GR evaluation for NTN satellite should consider the fully-connected mapping approaches. Proposal 2-6-2: In 6GR NTN scenario, if the phased antenna array used for non-GSO system, the antenna modeling in TR 38.901 can be used with consideration of polarization, e.g., circular polarization (LHCP/RHCP). Proposed the antenna modelling based on the template in an attachment. |
| Xiaomi | Proposed the antenna modelling based on the template in an attachment. |
| Huawei | Proposed the antenna modelling based on the template in the contribution. |
| ETRI | Proposed the antenna modelling based on the template in the contribution. |
| ESA, et al. | Proposed to add to the 6GR evaluation parameters spreadsheet the new phased-array parameters for NTN antenna model reported in R1-2509055 from Table 6 to Table 9. |
| Sharp | Proposal 2: Fully connected TXRU mapping within a panel per polarization as mandatory for NTN phased arrays. Proposal 3: The evaluation assumptions in TR38.821 can be used as a baseline for S-band and Ka-band. Proposal 4: The evaluation assumptions developed in RAN4 Rel-19 Ku band WI can be used as a baseline for Ku-band. Proposal 5: RAN1 assumes the equivalent antenna aperture of the reflector antenna as the form factor reference to derive antenna configuration for the phased array. Proposal 6: RAN1 derives the number of antenna elements N for the phased array, using the reference form factor (Table 1 can be a starting point). Proposal 7: A form factor 10 × 10 cm and 20 × 20 cm should be included in the evaluation assumptions for the Ku- and Ka-bands as well as the form factor equivalent to 60 cm equivalent antenna aperture. Proposal 8: TR38.811 is a starting point for reflector antenna modeling and TR38.901 is a starting point for phased array modeling. |
| MediaTek | UE Antenna configurations - S-band (i.e. 2 GHz) for NTN: - Option 1: 1 Tx and 1 Rx - Option 2: 2 Tx and 4 Rx - Ku-band ((FFS detailed frequency range) for NTN: - Option 1: 256 Tx and 256 Rx UE Antenna element gain pattern - for NTN: - Option 1: Omnidirectional - Option 2: directional UE Polarized antenna modelling - for NTN: - Option 1: Linear polarization based on Model-2 of TR 38.901 section 7.3.2 - Option 2: Circular polarization |
| Qualcomm | Proposal 17: For NTN, RAN1 to model the satellite antenna as an array. - TXRU virtualization model Option-2B in TR 36.897 (full-connection model) can be used as baseline. - The virtualization matrix W may include both amplitude and phase coefficients. - Antenna array parameters: - Mg = Ng = 1. - Other values FFS (may be different for different satellite types) - Total and per-beam transmit power (or PFD, when applicable) should be defined. - For simplified evaluations, the Bessel function in 38.811 can be used. Proposal 18: For NTN in FR1 (e.g. L/S-band), the UE antenna models are reused from TN “around 2GHz or 4GHz”. Proposal 19: For 6GR-NTN, RAN1 to discuss new satellite types, including the following: - Inclusion of orbits lower than 600km (VLEO) - Larger antenna gains than existing satellite types in TR 38.821 |
| CSCN | Proposed the antenna modelling based on the template in the contribution for 2GHz. |

### Discussions

Summary on the views:

Based on template agreed in the last meeting, companies’ views on NTN evaluation assumptions are summarized in the spreadsheet as attached.

Round-1 discussions:

(FL1) Proposal 6.2.2

For 6GR NTN evaluations, RAN1 to model the satellite antenna as an array.

- TXRU virtualization model Option-2B in TR 36.897 (full-connection model) can be used as baseline.

- The virtualization matrix W may include both amplitude and phase coefficients.

- Antenna array parameters:

- Mg = Ng = 1.

- Other values FFS (may be different for different satellite types)

- Total and per-beam transmit power (or PFD, when applicable) should be defined.

- FFS: For simplified evaluations, the Bessel function in 38.811 can be used.

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| CSCN | The intention of this proposal should be further clarified since some points are already covered in the NTN antenna model template. Furthermore, the detailed values of NTN antenna model and SLS parameters have been provided by companies, thus, we believe it's reasonable to start the discussion of detailed values based on these inputs. |
| ZTE | - For the 1st bullet: Agree; - For the 2nd bullet: Agree to take it as baseline; - For the 3rd bullet: No need to define the per beam transmit power since how to split the power per beam is up to implementation and can be defined per solution discussion. - For the FFS, agree to take the Bessel function. |
| Xiaomi | We understand that the third bullet "total and per-beam transmit power" is irrelevant to antenna modelling. It may be considered to discuss the third bullet in a separate proposal or revise it in the following manner. #### (FL1) Proposal 6.2.2 For 6GR NTN evaluations, RAN1 to model the satellite antenna as an array. - TXRU virtualization model Option-2B in TR 36.897 (full-connection model) can be used as baseline. - The virtualization matrix W may include both amplitude and phase coefficients. - Antenna array parameters: - Mg = Ng = 1. - Other values FFS (may be different for different satellite types) - Total and per-beam transmit power (or PFD, when applicable) should be defined. - FFS: For simplified evaluations, the Bessel function in 38.811 can be used. For 6GR NTN evaluations, total and per-beam transmit power (or PFD, when applicable) should be defined. |
| Qualcomm | Agree |
| Nokia | For the polarization discussion (especially on the UE side), it is understood that some companies prefer to also consider the VSAT, but for us it is a bit unclear if there for this case would be a common understanding that UE and satellite will use one – and only one – polarization. That is, either LHCP or RHCP would be used. The reason being that if the two ends of the link uses different polarization, there will be extremely large negative antenna gain to be modelled (and the link will fail). There would be no options for a UE to subsequently recover such a loss. |
| Sharp | We have similar views with Xiaomi. Tx power should be discussed as part of SLS assumptions. |

Round-3 discussions:

#### (FL3) Proposal 6.2.2

For 6GR NTN evaluations, RAN1 to model the satellite antenna as an array.

- TXRU virtualization model Option-2B in TR 36.897 (full-connection model) can be used as baseline.

- The virtualization matrix W may include both amplitude and phase coefficients.

- Antenna array parameters:

- Mg = Ng = 1.

- Other values FFS (may be different for different satellite types)

- FFS: For simplified evaluations, the Bessel function in 38.811 can be used.

Note: For 6GR NTN evaluations, total and per-beam transmit power (or PFD, when applicable) should be defined.

#The proposal was updated based on Xiaomi’s suggestion for FL3, Companies can further comment if not done yet. Please companies to address Nokia’s comment from the last round in the following table and share the view that whether Nokia’s comment affects stabling this proposal#

| Company | Comments |
| --- | --- |

## SLS assumptions for NTN

### Companies’ views

| Company | Views/proposals |
| --- | --- |
| Futurewei | Proposal 6: 6G radio study should consider VLEO orbits at an altitude ranging from 160 km to 350 km; and the traditional LEO/MEO/GEO orbits considered in the Rel-16 NR NTN and Rel-17 IoT NTN can be adopted as a starting point. Proposal 7: 6G radio study should include heterogeneous satellite constellations (or multi-orbit satellites), comprising a combination of VLEO, LEO, MEO, and GEO satellites. Proposal 8: 6G radio study adopts the NTN payload types in Rel-16 NR NTN and may include a semi-transparent payload (e.g., onboard transceiver units). Proposal 9: 6G radio study should include energy efficiency for 6G NTN including UE with and without a GNSS receiver. |
| Nokia | Proposal 31: RAN1 shall reuse NTN parameters from TR 38.811 and TR 38.821 as much as possible. The VLEO deployment scenario may be added to the Set-1 definitions from Table 6.1.1.1-1 of TR 38.821. |
| ZTE | Proposal 2-6-3: In 6GR NTN scenario, the UE configuration in TN can be reused as baseline at least for handheld. - Other NTN specific parameter, e.g., high-power UE can also be considered. Proposal 2-6-4: For 6G NTN, the simulation assumption in the template is recommended for evaluation within an attachment. |
| CATT | Proposed the SLS evaluation assumptions in the contribution based on the template. |
| Xiaomi | Proposed the SLS evaluation assumptions in the attachment based on the template. |
| Huawei | Proposed the SLS evaluation assumptions in the contribution based on the template. |
| LGE | Proposed bandwidth, layout, IBD, UE distribution and UE speed, and orbit types. Proposed equivalent satellite antenna aperture and satellite TX max gain, payload type, and aggregated satellite EIRP (Total) (dBW). |
| Huawei | Proposed the SLS evaluation assumptions in the contribution based on the template. |
| ESA, et al. | Proposed to add the new SAN characteristics in common TN/NTN evaluation parameters spreadsheet. Proposed for the evaluation of NTN-based 6GR access, consider the SAN characteristics reported in the R1-2509055 from Table 1 to Table 5. |
| Sharp | Proposal 9: Discuss whether RAN1 should define the different carrier frequency for HD-FDD from that for FD-FDD. |
| Sony | Proposed the SLS evaluation assumptions in the attachment based on the template. |
| Ericsson | Proposed the SLS evaluation assumptions in the contribution based on the template. |
| Ofinno | Proposed assumptions for orbit type, altitude, payload type, system bandwidth, simulation bandwidth, duplexing assuming FDD in all carrier frequencies. |
| MediaTek | Proposed the SLS evaluation assumptions in the contribution based on the template. wrt UE power class ‐ S-band (i.e. 2 GHz) for NTN: PC3 as baseline and PC2 as optional ‐ Ku-band ((FFS detailed frequency range) for NTN: [57-60] dBm as EIRP density |
| DOCOMO | Proposal 9 • At least, satellite orbit type and payload type should be determined before RAN1#124 (i.e., these parameters should be determined within the RAN1#123 or RAN#110). • Detailed evaluation assumption for NTN can be discussed based on the NTN template agreed in RAN1#122bis meeting under the topic of 6GR NTN starting RAN1#124. |
| CSCN | Proposal 2: VLEO-300km, LEO-600km, LEO-1200km, and GSO should be considered for 6GR evaluation. Proposed the SLS evaluation assumptions in the contribution based on the template for 2GHz. |

### Discussions

Summary on the views:

Based on template agreed in the last meeting, companies’ views on NTN evaluation assumptions are summarized in the spreadsheet as attached.

Round-1 discussions:

(FL1) Proposal 6.3.2

For 6GR NTN evaluations, RAN1 to consider the following Satellite orbit types:

- VLEO

- LEO

- GEO

Please check the templates and share your comments/views here if any:

| Company | Comments |
| --- | --- |
| CSCN | In general fine with the proposal. Some example orbital altitudes should be provided. For instance, 600km and 1200km used in NR-NTN could be further evaluated in 6GR LEO-NTN, in which 600km should be prioritized. For VLEO, an altitude of 300km could be considered. |
| ZTE | Prefer to also include the MEO. Additionally, The LEO-600 should be prioritized since it’s more typical scenarios compared to VLEO. |
| Xiaomi | Fine |
| NTT DOCOMO | We are generally fine with the proposal. |
| CMCC | Support |
| Qualcomm | We are OK with this approach, but we would like to ask companies if there is a need to differentiate the support of GEO for different bands. For instance, are companies interested in deploying GEO S/L-band for 6GR? On the satellite parameters, given the expected deployment timeline of 6GR and advances in space technology, we would expect to see satellites with better performance than those considered for NR. Aspects such as simultaneously active beams and satellite antenna gain should be better than those considered for NR. |
| Nokia | OK |
| Sharp | OK |

Round-3 discussions:

#### (FL3) Proposal 6.3.2

For 6GR NTN evaluations, RAN1 to consider the following Satellite orbit types:

- VLEO

- LEO

- GEO

#The proposal is the same for FL3, Companies can further comment if not done yet. #

| Company | Comments |
| --- | --- |

## Other general views in TDoc

| Company | Views/proposals |
| --- | --- |
| vivo | Proposed a table of Evaluation assumptions for 6G NTN evaluation including carrier frequency, BW, orbit, UE antenna, UE transmit power, etc. |
| Xiaomi | Proposed a table of Doppler and Delay Characteristics for NTN evaluation. |
| Tejas | Proposed for 6GR evaluation, consider HAPS as one of the payloads under NTN deployment scenarios. Consider Table 2.4-1 as the Evaluation parameters for HAPS payload. |
| Sharp | Proposed for NTN, the link budget template from TR38.830 should be employed to incorporate beamforming gain. Proposed RAN1 takes into account the beamforming gain reduction at the maximum satellite elevation angle in the link budget template for NTN phased array. |
| Sony | Proposal 7: 5G NTN simulation parameters can be re-used for 6G evaluations, including any modifications or extensions implemented by 5GA study. |

# Other common assumptions?

The earlier meeting has already clarified that the common evaluation assumptions, traffic models, and link budget will be discussed in this agenda. Other topic specific assumptions will be discussed in other individual agendas including those have opened for discussions including waveform, coding and modulation, frame structure, AI/ML, energy efficiency and those will be discussed from RAN1#124/124bis including initial access, sensing, etc.

It was also noted that additional assumptions can also be discussed in this agenda if the necessity is being identified.

Companies’ views are briefly summarized in the next subsection and the discussion is supposed to identify whether additional common evaluation assumptions will be further discussed in this agenda.

## Companies’ views

| Company | Views/proposals |
| --- | --- |
| vivo | Discussed evaluation methodology in details for sensing, e.g., use case, metrics, channel model, etc. |
| ZTE | Discussed sensing related use cases and scenarios. |
| Lenovo | Discussed power consumption model that needs to allow for the upper mid band spectrum operation, large antenna array, and larger bandwidth. Discussed MIMO related evaluations aspects, including larger number of antennas for both base station and UEs and other related optimization. Discussed and proposed the evaluation assumption for sensing related including use case, waveform, performance metrics. |
| NEC | Discussed power model for energy efficiency. Discussed AI/MI modeling, considering performance, overhead, complexity and also discussed the need to enhance the power model for AI/ML. |
| InterDigital | View that it might be useful to align some link level evaluation parameters as much as possible so a common set of link level evaluation parameters that each topic can leverage needs to be defined. Proposed a set of parameters for link-level evaluations. |
| Xiaomi | Discussed sensing related evaluations and metrics. |
| OPPO | Discussed evaluation methodology for energy efficiency, including power model for BS and UE. |
| Huawei | Discussed sensing related use case, evaluation methodology. |
| Samsung | Discussed specific assumptions for MIMO, e.g., sTRP/mTRP, MIMO layers, channel and interference estimation error, etc. Discussed specific assumptions for sensing, e.g., sensing modes, performance metrics, interference modelling, waveform, etc. Discussed a specific interference estimation modelling and proposed to use a realistic modelling e.g., Wishart distribution-based model according to TR 36.829 (Note: the parameters of the model should be reported by company). |
| LGE | Discussed MIMO related assumptions, e.g., antenna ports, rank numbers, MIMO layers, etc. Discussed sensing related scenarios. |
| NVIDIA | Discussed power consumption model and proposed to study the model based on AI-driven NW energy optimization and energy cost of running AI models. Discussed channel model and proposed Ray-tracing should be considered. Discussed and proposed the performance metrics for 6GR evaluations. |
| ETRI | Proposed to study cost-efficient network architectures leveraging NCR and/or RIS technologies from the early stage of 6GR. Discussed link-level evaluation assumptions for both TN and NTN. |
| Sony | Discussed power consumption models for BS and UE, performance metrics for 6GR evaluations, ISAC related performance metrics, link-level simulation assumptions. |
| Ericsson | Discussed channel model enhancement for e.g., sensing, accurate Doppler modelling for high speed scenario. Discussed evaluation assumptions for support of diverse device types. Discussed UE RF and PA modelling. Proposed RAN4 should be involved early in high level aspect of RF performance and makes final conclusion on relative RF performance of schemes studied by RAN1. |
| Apple | Discussed and proposed evaluation assumptions for initial access, ISAC related assumptions, NTN related, MIMO related. |
| Qualcomm | Discussed evaluation assumptions for MIMO related. |

## Discussions

Except those have been clarified in the earlier meeting agreements, there is nothing else that can be decided that should be discussed in this agenda at this meeting per moderator’s assessment.

#### (FL1) Moderator’s suggestion

Continue discussing the issues as agreed to be discussed in this agenda first. TBD on any other issues that needs to be discussed in this agenda.

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| Interdigital | It would be good if RAN1 would make further progress on general LLS assumptions that could be leveraged by simulation assumptions for each agena topic. |

# Proposals for offline/online

## 1st offline on Monday

(FL1) Proposal 2.1.2-1

Updating the BS antenna modelling agreed in the last meeting as follows:

- For around 700MHz carrier frequency, for BS antenna modelling,

- update the (8, 4, 2, 1, 1; x, y) to be (8, 4, 2, 1, 1; 1, 4).

- For around 2GHz carrier frequency, for BS antenna modelling,

- for outdoor combination 1 (i.e., 32AE/4TXRU), update the (M,N,P,Mg,Ng; Mp,Np) to be (8, 2, 2, 1, 1; 1, 2), change (0.5, 0.8)λ to (0.5, 0.5)λ.

- For around 7GHz carrier frequency, for BS antenna modelling,

- for outdoor combination 1 (i.e., 768AE/128TXRU), update the (M,N,P,Mg,Ng; Mp,Np) to be (24, 16, 2, 1, 1; 4, 16).

- for outdoor combination 3 (i.e., 1536AE/256TXRU), update the (M,N,P,Mg,Ng; Mp,Np) to be (48, 16 ,2, 1, 1; 8, 16).

(FL1) Proposal 2.1.2-2

For 6GR evaluations, RAN1 to consider BS antenna modelling for around 15GHz carrier frequency as follows:

| BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg, Ng; Mp, Np) | (dH,dV) |
| --- | --- | --- | --- | --- |
| Indoor |  |  |  |  |
| Combination 1 | 128 | 64 | (8, 8, 2, 1, 1; 4, 8) | (0.5, 0.5)λ |
| Combination 2 | 128 | 8 | (4, 4, 2, 2, 2; 1, 1) | (0.5, 0.5)λ |
| Combination 3 | 256 | 256 | (8, 16, 2, 1, 1; 8, 16) | (0.5, 0.5)λ |
| Outdoor |  |  |  |  |
| Combination 1 | 2048 | 256 | (64, 16, 2, 1, 1; 8, 16) | (0.5, 0.5)λ |
| Combination 1 | 2048 | 256 | (32, 32, 2, 1, 1; 8, 16) | (0.5, 0.8)λ |
| Combination 2 | 2048 | 16 | (16, 8, 2, 4, 2; 1, 1) | (0.5, 0.5)λ |
| Note1: A single TXRU is mapped per panel per subarray per polarization for combination 1. A single TXRU is mapped per panel per polarization for combination2. Note2: Other combinations used in the simulation results are up to company to report. |  |  |  |  |

(FL1) Proposal 2.2.2

For 6GR evaluation, RAN1 to model the UE antenna as follows:

- Note: Each of other topics could further decide to use which combination(s) for the evaluations.

- Note: The antenna locations in Alt 2 are at least considered for performance calibration. Other locations are also possible for evaluations.

| UE antenna modelling for RAN1 evaluations | Total number of antenna elements | Total number of TXRU | Alt 1: (M,N,P,Mg,Ng; Mp,Np), (dH,dV), (dg,H,dg,V) if any, or Alt 2: handheld device antenna model using candidate antenna locations as described in section 7.3 in TR38.901 |
| --- | --- | --- | --- |
| Combination0 | 1 | 1T1R, | 1T1R, Alt 1: 1T: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) , (dH,dV)= (0.5, 0.5)λ 1R: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) , (dH,dV)= (0.5, 0.5)λ Alt 2: - 1T: (1) as described in section 7.3 in TR 38.901 |
| Combination1 | 2 | 1T2R, | 1T2R, Alt 1: - 1T: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) , (dH,dV)= (0.5, 0.5)λ - 2R: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 1, 1, 1; 1, 2) for single polarization or (1, 1, 2, 1, 1; 1, 1) for dual polarization, (dH,dV)= (0.5, 0.5)λ Alt 2: - 1T: (1) as described in section 7.3 in TR 38.901 - 2R: (1, 5) as described in section 7.3 in TR 38.901. |
| Combination2 | 4 | 2T4R, 4T4R | 2T4R, Alt 1: - 2T: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 1, 1, 1; 1, 2) , (dH,dV)= (0.5, 0.5)λ - 4R: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 2, 1, 1; 1, 2) for dual polarization or (2, 2, 1, 1, 1; 2, 2) for single polarization, (dH,dV)= (0.5, 0.5)λ Alt 2: - 2T: (2, 6) as described in section 7.3 in TR 38.901 - 4R: (2, 4, 6, 8) as described in section 7.3 in TR 38.901 4T4R, Alt 1: (M, N, P, Mg, Ng; Mp, Np)= (1, 2, 2, 1, 1; 1, 2), (dH,dV)= (0.5, 0.5)λ Alt 2: (1, 3, 5, 7) as described in section 7.3 in TR38.901 |
| Combination3 | 8 | 4T8R, 8T8R | 4T8R, Alt 1: - 4T: (M, N, P, Mg, Ng; Mp, Np)= (1, 2, 2, 1, 1; 1, 2) for dual polarization or (1, 4, 1, 1, 1; 1, 4) for single polarization, (dH,dV)= (0.5, 0.5)λ. - 8R: (M, N, P, Mg, Ng; Mp, Np)= (1, 4, 2, 1, 1; 1, 4) , (dH,dV)= (0.5, 0.5)λ Alt 2: - 4T: (1, 3, 5, 7) as described in section 7.3 in TR38.901 - 8R: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901 8T8R, Alt 1: (M, N, P, Mg, Ng; Mp, Np)= (1, 4, 2, 1, 1; 1, 4) for dual polarization or (2, 4, 1, 1, 1; 2, 4) for single polarization , (dH,dV)= (0.5, 0.5)λ Alt 2: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901 |
| Combination4 | 16 | 8T16R, 16T16R | 8T16R, Alt 1: - 8T: (M, N, P, Mg, Ng; Mp, Np)= (1, 4, 2, 1, 1; 1, 4) for dual polarization or (2, 4, 1, 1, 1; 2, 4) for single polarization, (dH,dV)= (0.5, 0.5)λ - 16R: (M, N, P, Mg, Ng; Mp, Np)= (2, 4, 2, 1, 1; 2, 4) , (dH,dV)= (0.5, 0.5)λ Alt 2: - 8T: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901, single polarization - 16R: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901, dual polarization 16T16R, Alt 1: (M, N, P, Mg, Ng; Mp, Np)= (2, 4, 2, 1, 1; 2, 4) , (dH,dV)= (0.5, 0.5)λ Alt 2: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901, dual polarization |
| Combination5 | 32 | 16T32R, 32T32R | 16T32R, - 16T: (M, N, P, Mg, Ng; Mp, Np)= (2, 4, 2, 1, 1; 2, 4) for dual polarization or (4, 4, 1, 1, 1; 4, 4) for single polarization, (dH,dV)= (0.5, 0.5)λ - 32R: (M, N, P, Mg, Ng; Mp, Np)= (4, 4, 2, 1, 1; 4, 4) , (dH,dV)= (0.5, 0.5)λ 32T32R, (M, N, P, Mg, Ng; Mp, Np)= (4, 4, 2, 1, 1; 4, 4) , (dH,dV)= (0.5, 0.5)λ |

(FL1) Proposal 3.1.2

For 6GR evaluation, the layout for system-level simulation is assumed as follows:

| Parameters | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- | --- | --- |
| Layout | Single layer - Indoor floor (Open office), (Room size: 120m x 50m) | Single layer: - Hex. Grid Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop or fixed drop as xx | Single layer: - Hex. Grid | Single layer: - Hex. Grid Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop or fixed drop as xx | Single layer: - Hex. Grid |

(FL1) Proposal 3.2.2-1

For 6GR evaluation, the total transmit power per BS for system-level simulation is assumed as follows:

| Total transmit power per BS | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Sub-urban macro |
| --- | --- | --- | --- | --- | --- |
| Around 700MHz | NA | Macro BS: - Option1: 44 dBm per 20 MHz - Option2: 49 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 2GHz | 24 dBm per 20 MHz | Macro BS: - Option1: 44 dBm per 20 MHz - Option2: 46 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: - Option1: 49 dBm per 20 MHz - Option2: 46 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 4GHz | 24 dBm per 20 MHz | Macro BS: - Option1: 44 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: - Option1: 49 dBm per 20 MHz - Option2: 44 dBm per 20 MHz - Option3: 46 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 7GHz | 24 dBm per 20 MHz | Macro BS: - Option1: 44 dBm per 20 MHz - Option2: 43 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: - Option1: 49 dBm per 20 MHz - Option2: 43 dBm per 20 MHz - Option3: 46 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 15GHz | 23dBm per 20MHz | Macro BS: 40dBm per 20MHz Micro BS: 33 dBm per 20 MHz | NA | Macro BS: 43dBm per 20MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49dBm per 20MHz |
| Around 30GHz | 23 dBm per 20 MHz | Micro BS: 33 dBm per 20 MHz | NA | Micro BS: 33 dBm per 20 MHz | Macro BS: 33 dBm per 20 MHz |
| Note: BS Tx power scales up with bandwidth proportionally. Note: The maximum BS Tx power for each scenario will be defined. FFS: 56dBm for outdoor BS and [33dBm] for indoor BS. |  |  |  |  |  |

(FL1) Proposal 3.2.2-2

For 6GR evaluation, the UE power class for system-level simulation is assumed as follows:

- Note: 23dBm is for both FDD and TDD. 26dBm and 29dBm are for TDD only.

- Note: 31dBm is not targeted for smartphone both FDD and TDD.

| UE power class | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Sub-urban macro |
| --- | --- | --- | --- | --- | --- |
| Around 700MHz | NA | 23dBm | 23dBm | 23dBm | 23dBm |
| Around 2GHz | 23dBm, 26dBm | 23dBm, 26dBm | 23dBm, 26dBm | 23dBm, 26dBm | 23dBm, 26dBm |
| Around 4GHz | 23dBm, 26dBm | 23dBm, 26dBm | 23dBm, 26dBm | 23dBm, 26dBm | 23dBm, 26dBm |
| Around 7GHz | 23dBm, 26dBm and 29dBm Note: CPE/FWA is [31dBm] | 23dBm, 26dBm and 29dBm Note: CPE/FWA is [31dBm] | 23dBm, 26dBm and 29dBm Note: CPE/FWA is [31dBm] | 23dBm, 26dBm and 29dBm Note: CPE/FWA is [31dBm] | 23dBm, 26dBm and 29dBm Note: CPE/FWA is [31dBm] |
| Around 15GHz | 23dB, 26dBm and 29dBm Note: EIRP should not exceed 43 dBm | 23dB, 26dBm and 29dBm Note: EIRP should not exceed 43 dBm | NA | 23dB, 26dBm and 29dBm Note: EIRP should not exceed 43 dBm | 23dBm EIRP should not exceed 43 dBm |
| Around 30GHz | 23dB, 26dBm, and 29dBm Note: CPE/FWA is [31dBm] Note: EIRP should not exceed 43 dBm | 23dB, 26dBm, and 29dBm Note: CPE/FWA is [31dBm] Note: EIRP should not exceed 43 dBm | NA | 23dB, 26dBm, and 29dBm Note: CPE/FWA is [31dBm] Note: EIRP should not exceed 43 dBm | 23dBm EIRP should not exceed 43 dBm |

(FL1) Proposal 3.3.2

For 6GR evaluation, the UE distribution and UE speed for system-level simulation is assumed as follows:

| Parameters | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- | --- | --- |
| UE distribution and UE speed | 10 users per TRxP. 100% Indoor, 3km/h | Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per TRxP is [10, 30]. 80% indoor (3km/h); 20% outdoor(30km/h). | Single layer: Uniform/macro TRxP UE number per TRxP is [10, 30]. 50% indoor (3km/h); 50% outdoor(120km/h). | Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per TRxP is [10, 30]. [80% indoor (3km/h); 20% outdoor(30km/h).] | Single layer: Uniform/macro TRxP UE number per TRxP is [10, 30]. 10% Outdoor pedestrian: 3km/h; 10% Outdoor in cars: 40km/h; 80% Indoor in houses: 3km/h. |

(FL1) Proposal 3.4.1

For 6GR evaluation, the following are assumed for system-level simulation:

| Parameters | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- | --- | --- |
| ISD | 20m, equivalent to 12TRxPs per 120m x 50m | Macro layer: 200m | ISD 1: 1732m ISD 2: 5000m | Macro: 500m | ISD 1: 1299m ISD 2: 1732m |
| BS antenna height | 3m | 25m for macro cells and 10m for micro cells | 35 m | Macro: 25m | Alt 1: 35m Alt 2: 25m |
| BS noise figure | Around 7GHz and below: 5dB Around 15GHz and above: 7dB |  |  |  |  |
| UE antenna height | TR38.901 Indoor-Office Table 7.2-2 | TR38.901 UMi/UMa Table 7.2-1 | TR38.901 RMa Table 7.2-3 | TR38.901 UMa Table 7.2-1 | TR38.901 SMa Table 7.2-5 |
| UE noise figure | Around 7GHz and below: 9dB (baseline performance), 7dB (high performance) Around 15GHz and above: 13dB (baseline performance), 10dB (high performance) |  |  |  |  |
| UE Receiver | MMSE-IRC as the baseline |  |  |  |  |
| UE Power control parameter for UL | Company report |  |  |  |  |
| Channel model | TR 38.901 v19.1.0 Indoor-Office | TR 38.901 v19.1.0 UMa/UMi | TR 38.901 v19.1.0 RMa | TR 38.901 v19.1.0 UMa | TR 38.901 v19.1.0 SMa |
| Numerology | 15kHz SCS for FDD, 30kHz SCS for TDD |  |  |  |  |
| Scheduling | Proportional fairness (PF) |  |  |  |  |
| Inter-cell interference model | Explicitly and realistically modelled |  |  |  |  |
| [Inter-cell interference estimation model] | Alt 1: Ideal, calculated by ground truth channel matrix Alt 2: Realistic model, Company report, e.g., Wishart distribution-based model; retain only diagonal elements of interference Cov. Matrix. |  |  |  |  |
| Channel estimation assumption | Alt 1: Ideal Alt 2: Realistic, company report, e.g., apply gauss noise to real channel matrix, or random |  |  |  |  |
| Feedback assumption | Alt 1: Ideal Alt 2: Realistic, company report, e.g., consider feedback delay and overhead; codebook; |  |  |  |  |
| O2I penetration loss (X% high loss, Y% low loss) | NA | Two options are supported: Option 1: 80% low loss, 20% high loss; Option 2: 50% low loss, 50% high loss | 100% low loss | Two options are supported: Option 1: 80% low loss, 20% high loss; Option 2: 50% low loss, 50% high loss | 100% Low-loss A Model as TR 38.901 |
| Mechanic tilt | 180° in GCS (pointing to the ground) as baseline. Company can report if not follow the baseline. | 90° in GCS (pointing to horizontal direction) as baseline. Company can report if not follow the baseline. | 90° in GCS (pointing to horizontal direction) as baseline. Company can report if not follow the baseline. | 90° in GCS (pointing to horizontal direction) as baseline. Company can report if not follow the baseline. | Baseline: 95° in GCS (pointing to horizontal direction) for ISD = 1299m; 92° in GCS (pointing to horizontal direction) for ISD = 1732m; Company can report if not follow the baseline. |
| Electronic tilt | 90° in LCS as baseline. Company can report if not follow the baseline. | Company report, e.g., 105° or 102° in LCS. | Company report, e.g., 96° in LCS. | Company report, e.g., 102° in LCS. | Company report, e.g., 102° in LCS. |
| Handover margin (dB) | 0dB as baseline. 1dB and 3dB as optional configuration. | 0dB as baseline. 1dB and 3dB as optional configuration. | 0dB as baseline. 1dB and 3dB as optional configuration. | 0dB as baseline. 1dB and 3dB as optional configuration. | 0dB as baseline. 1dB and 3dB as optional configuration. |
| UE attachment | Based on RSRP from BS port 0 |  |  |  |  |
| Wrapping around method | No wrapping around | Geographical distance-based wrapping | Geographical distance-based wrapping | Geographical distance-based wrapping | Geographical distance-based wrapping |
| Multi-TRP operation, e.g., ideal or non-ideal backhaul/sync | NA | Alt 1: ideal backhaul/sync Alt 2: non-ideal backhaul/sync, company report |  |  |  |

## 1st online on Tuesday

(FL2) Proposal 2.1.2-1

Updating the BS antenna modelling agreed in the last meeting as follows:

- For around 700MHz carrier frequency, for BS antenna modelling,

- update the (8, 4, 2, 1, 1; x, y) to be (8, 4, 2, 1, 1; 1, 4).

- For around 2GHz carrier frequency, for BS antenna modelling,

- for outdoor combination 1 (i.e., 32AE/4TXRU), update the (M,N,P,Mg,Ng; Mp,Np) to be (8, 2, 2, 1, 1; 1, 2)

- For around 7GHz carrier frequency, for BS antenna modelling,

- for outdoor combination 1 (i.e., 768AE/128TXRU), update the (M,N,P,Mg,Ng; Mp,Np) to be (24, 16, 2, 1, 1; 4, 16).

- for outdoor combination 3 (i.e., 1536AE/256TXRU), update the (M,N,P,Mg,Ng; Mp,Np) to be (48, 16 ,2, 1, 1; 8, 16).

(FL2) Proposal 2.1.2-2rv2

For 6GR evaluations, RAN1 to consider BS antenna modelling for around 15GHz carrier frequency as follows:

| BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg, Ng; Mp, Np) | (dH,dV) |
| --- | --- | --- | --- | --- |
| Indoor |  |  |  |  |
| Combination 1 | 128 | 64 | (8, 8, 2, 1, 1; 4, 8) | (0.5, 0.5)λ |
| Combination 2 | 128 | 8 | (4, 4, 2, 2, 2; 1, 1) | (0.5, 0.5)λ |
| Combination 3 | 512 | 128 | (16, 16, 2, 1, 1; 8, 8) | (0.5, 0.5)λ |
| Outdoor |  |  |  |  |
| Combination 1 | 1024 | 64 | (32, 16, 2, 1, 1, 4, 8) | (0.5, 0.5)λ |
|  | 3072 | 128 | (48, 32, 2, 1, 1; 2, 32) | (0.5, 0.5)λ |
| Combination 2 | 2048 | 256 | (32, 32, 2, 1, 1; 4, 32) | (0.5, 0.5)λ |
| Combination 3 | 2048 | 16 | (16, 8, 2, 4, 2; 1, 1) | (0.5, 0.5)λ |
| Note1: A single TXRU is mapped per panel per subarray per polarization for combination 1. A single TXRU is mapped per panel per polarization for combination2. Note2: Other combinations used in the simulation results are up to company to report. |  |  |  |  |

(FL2) Proposal 2.2.2-rv2

For 6GR evaluation, RAN1 to model the UE antenna as follows:

- Note: Each of other topics could further decide to use which combination(s) for the evaluations.

- Note: The antenna locations in Alt 2 are considered for performance calibration. Other antenna locations in Alt 2 are also possible for evaluations.

- Note: The antenna locations in Alt 2 not included in section 7.3 in TR38.901 are up to companies to report.

- FFS: Alt1 or Alt2 is used for each of the combination.

- FFS on other combinations, e.g., 2T6R, 3T6R, 6T6R, 6T8R.

| UE antenna modelling for RAN1 evaluations | Total number of antenna elements | Total number of TXRU | Alt 1: (M,N,P,Mg,Ng; Mp,Np), (dH,dV), (dg,H,dg,V) if any, or Alt 2: UT device antenna model using candidate antenna locations as described in section 7.3 in TR38.901 | Applicable carrier frequency |
| --- | --- | --- | --- | --- |
| Combination0 | 1 | 1T1R, | 1T1R, Alt 1: 1T: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) , (dH,dV)= (0.5, 0.5)λ 1R: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) , (dH,dV)= (0.5, 0.5)λ Alt 2: - 1T: (1) as described in section 7.3 in TR 38.901 - 1R: (1) as described in section 7.3 in TR 38.901 | 700MHz, 2GHz |
| Combination1 | 2 | 1T2R, | 1T2R, Alt 1: - 1T: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) , (dH,dV)= (0.5, 0.5)λ - 2R: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 1, 1, 1; 1, 2) for single polarization or (1, 1, 2, 1, 1; 1, 1) for dual polarization, (dH,dV)= (0.5, 0.5)λ Alt 2: - 1T: [(1)] as described in section 7.3 in TR 38.901 - 2R: [(1, 5)] as described in section 7.3 in TR 38.901. | 700MHz, 2GHz, |
| Combination2 | 4 | 2T4R, 4T4R | 2T4R, Alt 1: - 2T: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 1, 1, 1; 1, 2) , (dH,dV)= (0.5, 0.5)λ - 4R: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 2, 1, 1; 1, 2) for dual polarization or (2, 2, 1, 1, 1; 2, 2) for single polarization, (dH,dV)= (0.5, 0.5)λ Alt 2: - 2T: [(2, 6)] as described in section 7.3 in TR 38.901 - 4R: [(2, 4, 6, 8), or (1, 3, 5, 7)] as described in section 7.3 in TR 38.901 4T4R, Alt 1: (M, N, P, Mg, Ng; Mp, Np)= (1, 2, 2, 1, 1; 1, 2), (dH,dV)= (0.5, 0.5)λ Alt 2: [(2, 4,6, 8)] as described in section 7.3 in TR38.901 | 2GHz, 4GHz, 7GHz, |
| Combination3 | 8 | 4T8R, 8T8R | 4T8R, Alt 1: - 4T: (M, N, P, Mg, Ng; Mp, Np)= (1, 2, 2, 1, 1; 1, 2) for dual polarization or (1, 4, 1, 1, 1; 1, 4) for single polarization, (dH,dV)= (0.5, 0.5)λ. - 8R: (M, N, P, Mg, Ng; Mp, Np)= (1, 4, 2, 1, 1; 1, 4) , (dH,dV)= (0.5, 0.5)λ - FFS values of Mg/Ng>1. Alt 2: - 4T: [(1, 3, 5, 7)] as described in section 7.3 in TR38.901 - 8R: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901 8T8R, Alt 1: (M, N, P, Mg, Ng; Mp, Np)= (1, 4, 2, 1, 1; 1, 4) for dual polarization or (2, 4, 1, 1, 1; 2, 4) for single polarization , (dH,dV)= (0.5, 0.5)λ - FFS values of Mg/Ng>1. Alt 2: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901 | 4GHz 7GHz, 15GHz, 30GHz |
| Combination4 | 16 | 8T16R, 16T16R | 8T16R, Alt 1: - 8T: (M, N, P, Mg, Ng; Mp, Np)= (1, 4, 2, 1, 1; 1, 4) for dual polarization or (2, 4, 1, 1, 1; 2, 4) for single polarization, (dH,dV)= (0.5, 0.5)λ - 16R: (M, N, P, Mg, Ng; Mp, Np)= (2, 4, 2, 1, 1; 2, 4) , (dH,dV)= (0.5, 0.5)λ - FFS values of Mg/Ng>1. Alt 2: - 8T: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901, single polarization - 16R: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901, dual polarization 16T16R, Alt 1: (M, N, P, Mg, Ng; Mp, Np)= (2, 4, 2, 1, 1; 2, 4) , (dH,dV)= (0.5, 0.5)λ Alt 2: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901, dual polarization | 7GHz, 15GHz, 30GHz |

(FL2) Proposal 3.1.2-rv2

For 6GR evaluation, the layout for system-level simulation is assumed as follows:

- Note: Single layer will be prioritized for the evaluations.

- Note: The carrier frequency for the corresponding layout for the two layers will be reported by companies for the evaluations.

- FFS the minimum distance for the two layers.

| Parameters | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- | --- | --- |
| Layout | Single layer - Indoor floor (Open office), (Room size: 120m x 50m) | Single layer: - Hex. Grid Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop | Single layer: - Hex. Grid | Single layer: - Hex. Grid Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop | Single layer: - Hex. Grid |

(FL2) Proposal 3.2.2-1rv1

For 6GR evaluation, the total transmit power per BS for system-level simulation is assumed as follows:

| Total transmit power per BS | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Sub-urban macro |
| --- | --- | --- | --- | --- | --- |
| Around 700MHz | NA | Macro BS: - Option1: 44 dBm per 20 MHz - Option2: 49 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 2GHz | 24 dBm per 20 MHz | Macro BS: - Option1: 44 dBm per 20 MHz - Option2: 46 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: - Option1: 49 dBm per 20 MHz - Option2: 46 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 4GHz | 24 dBm per 20 MHz | Macro BS: - Option1: 44 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: - Option1: 49 dBm per 20 MHz - Option2: 44 dBm per 20 MHz - Option3: 46 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 7GHz | 24 dBm per 20 MHz | Macro BS: - Option1: 44 dBm per 20 MHz - Option2: 43 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: - Option1: 49 dBm per 20 MHz - Option2: 43 dBm per 20 MHz - Option3: 46 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 15GHz | 23dBm per 20MHz | Macro BS: 40dBm per 20MHz Micro BS: 33 dBm per 20 MHz | NA | Macro BS: 49dBm per 20MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49dBm per 20MHz |
| Around 30GHz | 23 dBm per 20 MHz | Micro BS: 33 dBm per 20 MHz | NA | Micro BS: 33 dBm per 20 MHz | Macro BS: 33 dBm per 20 MHz |
| Note: BS Tx power scales up with bandwidth proportionally under the limitation of the maximum BS Tx power is 56dBm for outdoor and 33dBm for indoor. For around 30GHz, the maximum BS Tx power is 41dBm FFS: EIRP limit for 15GHz and 30GHz. |  |  |  |  |  |

(FL2) Proposal 3.2.2-2rv1

For 6GR evaluation, the UE power class for system-level simulation is assumed as follows:

- Note: 23dBm is for both FDD and TDD. 26dBm and 29dBm (optional) are for TDD only.

- FFS: 31dBm is for CPE/FWA, or 35 dBm with EIRP <55 dBm is for CPE/FWA

| UE power class | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Sub-urban macro |
| --- | --- | --- | --- | --- | --- |
| Around 700MHz | NA | 23dBm | 23dBm | 23dBm | 23dBm |
| Around 2GHz | 23dBm, 26dBm | 23dBm, 26dBm | 23dBm, 26dBm | 23dBm, 26dBm | 23dBm, 26dBm |
| Around 4GHz | 23dBm, 26dBm | 23dBm, 26dBm | 23dBm, 26dBm | 23dBm, 26dBm | 23dBm, 26dBm |
| Around 7GHz | 23dBm, 26dBm and 29dBm | 23dBm, 26dBm and 29dBm | 23dBm, 26dBm and 29dBm | 23dBm, 26dBm and 29dBm | 23dBm, 26dBm and 29dBm |
| Around 15GHz | 23dB, 26dBm and 29dBm Note: EIRP should not exceed 43 dBm | 23dB, 26dBm and 29dBm Note: EIRP should not exceed 43 dBm | NA | 23dB, 26dBm and 29dBm Note: EIRP should not exceed 43 dBm | 23dBm, 26dBm, and 29dBm EIRP should not exceed 43 dBm |
| Around 30GHz | 23dB, 26dBm, 29dBm Note: EIRP should not exceed 43 dBm | 23dB, 26dBm, 29dBm Note: EIRP should not exceed 43 dBm | NA | 23dB, 26dBm, 29dBm Note: EIRP should not exceed 43 dBm | 23dBm, 26dBm, and 29dBm EIRP should not exceed 43 dBm |

(FL2) Proposal 3.3.2-rv2

For 6GR evaluation, the UE distribution and UE speed for system-level simulation is assumed as follows:

- UE number per TRxP will be dependent on the used traffic model. Other values can also be considered in the future evaluations.

- Other velocities will be dependent on the used traffic model or use cases. Other values can also be considered in the future evaluations.

| Parameters | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- | --- | --- |
| UE distribution and UE speed | 10 users per TRxP. 100% Indoor, 3km/h | Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per TRxP is [10, 30]. 80% indoor (3km/h); 20% outdoor(30km/h). | Single layer: Uniform/macro TRxP UE number per TRxP is [10, 30]. 50% indoor (3km/h); 50% outdoor(120km/h). | Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per TRxP is [10, 30]. 80% indoor (3km/h); 20% outdoor(30km/h). | Single layer: Uniform/macro TRxP UE number per TRxP is [10, 30]. 10% Outdoor pedestrian: 3km/h; 10% Outdoor in cars: 40km/h; 80% Indoor in houses: 3km/h. |

(FL2) Proposal 3.4.1-rv1

For 6GR evaluation, the following are assumed for system-level simulation:

| Parameters | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- | --- | --- |
| ISD | 20m, equivalent to 12TRxPs per 120m x 50m | Macro layer: 200m | ISD 1: 1732m ISD 2: 5000m | Macro: 500m | ISD 1: 1299m ISD 2: 1732m |
| BS antenna height | 3m | 25m for macro cells and 10m for micro cells | 35 m | Macro: 25m | Alt 1: 35m Alt 2: 25m |
| BS noise figure | Around 7GHz and below: 5dB Around 15GHz and above: 7dB |  |  |  |  |
| UE antenna height | TR38.901 Indoor-Office Table 7.2-2 | TR38.901 UMi/UMa Table 7.2-1 | TR38.901 RMa Table 7.2-3 | TR38.901 UMa Table 7.2-1 | TR38.901 SMa Table 7.2-5 |
| UE noise figure | Around 7GHz and below: 9dB (baseline performance), 7dB (high performance) Around 15GHz and above: 13dB (baseline performance), 10dB (high performance) |  |  |  |  |
| UE Receiver | MMSE-IRC as the baseline, or R-ML Receiver |  |  |  |  |
| UE Power control parameter for UL | Company report |  |  |  |  |
| Channel model | TR 38.901 v19.1.0 Indoor-Office | TR 38.901 v19.1.0 UMa/UMi | TR 38.901 v19.1.0 RMa | TR 38.901 v19.1.0 UMa | TR 38.901 v19.1.0 SMa |
| Numerology | In line with AI 11.3 |  |  |  |  |
| Scheduling | Proportional fairness (PF) |  |  |  |  |
| Inter-cell interference model | Explicitly and realistically modelled |  |  |  |  |
| [Inter-cell interference estimation model] | Alt 1: Ideal, calculated by ground truth channel matrix Alt 2: Realistic model, Company report, e.g., Wishart distribution-based model; retain only diagonal elements of interference Cov. Matrix. |  |  |  |  |
| Channel estimation assumption | Alt 1: Ideal Alt 2: Realistic, company report, e.g., apply gauss noise to real channel matrix, or random |  |  |  |  |
| Feedback assumption | Alt 1: Ideal Alt 2: Realistic, company report, e.g., consider feedback delay and overhead; codebook; |  |  |  |  |
| O2I penetration loss (X% high loss, Y% low loss) | NA | Two options are supported: Option 1: 80% low loss, 20% high loss; Option 2: 50% low loss, 50% high loss | 100% low loss | Two options are supported: Option 1: 80% low loss, 20% high loss; Option 2: 50% low loss, 50% high loss | 100% Low-loss A Model as TR 38.901 |
| Mechanic tilt | 180° in GCS (pointing to the ground) as baseline. Company can report if not follow the baseline. | 90° in GCS (pointing to horizontal direction) as baseline. Company can report if not follow the baseline. | 90° in GCS (pointing to horizontal direction) as baseline. Company can report if not follow the baseline. | 90° in GCS (pointing to horizontal direction) as baseline. Company can report if not follow the baseline. | Baseline: 95° in GCS (pointing to horizontal direction) for ISD = 1299m; 92° in GCS (pointing to horizontal direction) for ISD = 1732m; Company can report if not follow the baseline. |
| Electrical tilt | 90° in LCS as baseline. Company can report if not follow the baseline. | Company report, e.g., 105° or 102° in LCS. | Company report, e.g., 96° in LCS. | Company report, e.g., 102° in LCS. | Company report, e.g., 102° in LCS. |
| Handover margin (dB) | 0dB as baseline. 1dB and 3dB as optional configuration. | 0dB as baseline. 1dB and 3dB as optional configuration. | 0dB as baseline. 1dB and 3dB as optional configuration. | 0dB as baseline. 1dB and 3dB as optional configuration. | 0dB as baseline. 1dB and 3dB as optional configuration. |
| UE attachment | Based on RSRP from BS port 0 |  |  |  |  |
| Wrapping around method | No wrapping around | Geographical distance-based wrapping | Geographical distance-based wrapping | Geographical distance-based wrapping | Geographical distance-based wrapping |
| Multi-TRP operation, e.g., ideal or non-ideal backhaul/sync | NA | Alt 1: ideal backhaul/sync Alt 2: non-ideal backhaul/sync, company report |  |  |  |

## 2nd online on Tuesday

(FL2) Proposal 2.1.2-2rv3

For 6GR evaluations, RAN1 to consider BS antenna modelling for around 15GHz carrier frequency as follows:

| BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg, Ng; Mp, Np) | (dH,dV) |
| --- | --- | --- | --- | --- |
| Indoor |  |  |  |  |
| Combination 1 | 512 | 128 | (16, 16, 2, 1, 1; 8, 8) | (0.5, 0.5)λ |
| Combination 2 | 128 | 8 | (4, 4, 2, 2, 2; 1, 1) | (0.5, 0.5)λ |
| Outdoor |  |  |  |  |
| Combination 1 | 2048 | 256 | (32, 32, 2, 1, 1; 4, 32) | (0.5, 0.5)λ |
| Combination 2 | 2048 | 32 | (16, 8, 2, 4, 4; 1, 1) | (0.5, 0.5)λ |
| Note1: A single TXRU is mapped per panel per subarray per polarization for combination 1. A single TXRU is mapped per panel per polarization for combination2. Note2: Other combinations used in the simulation results are up to company to report. |  |  |  |  |

(FL2) Proposal 2.2.2-rv3

For 6GR evaluation, RAN1 to model the UE antenna as follows:

- Note: Each of other topics could further decide to use which combination(s) for the evaluations. Other combinations are not precluded for evaluations, e.g., 2T6R, 3T6R, 6T6R, 6T8R.

- Note: The antenna locations in Alt 2 are considered as an example and used for performance calibration. Other antenna locations in section 7.3 in TR38.901 are also possible for evaluations and up to companies to report.

- Note: The antenna locations in Alt 2 not included in section 7.3 in TR38.901 are up to companies to report.

- Note: The antenna element/location of T is a subset of the element/locations for R.

- FFS: Alt1 or Alt2 is used for each of the combination.

- FFS on UE antenna modelling for 30GHz.

- Note: The mapping between the combination and the device types will be separately discussed.

| UE antenna modelling for RAN1 evaluations | Total number of antenna elements | Total number of TXRU | Alt 1: (M,N,P,Mg,Ng; Mp,Np), (dH,dV), (dg,H,dg,V) if any, or Alt 2: UT device antenna model using candidate antenna locations as described in section 7.3 in TR38.901 | Applicable carrier frequency |
| --- | --- | --- | --- | --- |
| Combination0 | 1 | 1T1R, | Alt 1: 1T: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) 1R: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) Alt 2: - 1T - 1R | 700MHz, 2GHz |
| Combination1 | 2 | 1T2R, | Alt 1: - 2R: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 1, 1, 1; 1, 2) for single polarization or (1, 1, 2, 1, 1; 1, 1) for dual polarization, (dH,dV)= (0.5, 0.5)λ Alt 2: - 2R: [(1, 5), or (4, 8)] as described in section 7.3 in TR 38.901. | 700MHz, 2GHz, 4GHz |
| Combination2 | 4 | 1T4R, 2T4R, 4T4R | Alt 1: - 4R: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 2, 1, 1; 1, 2) for dual polarization or (2, 2, 1, 1, 1; 2, 2) for single polarization, (dH,dV)= (0.5, 0.5)λ Alt 2: - 4R: [(2, 4, 6, 8), or (1, 3, 5, 7)] as described in section 7.3 in TR 38.901 | 700MHz 2GHz, 4GHz, 7GHz, 15GHz |
| Combination3 | 8 | 4T8R, 8T8R | Alt 1: (M, N, P, Mg, Ng; Mp, Np)= (1, 4, 2, 1, 1; 1, 4) for dual polarization or (2, 4, 1, 1, 1; 2, 4) for single polarization , (dH,dV)= (0.5, 0.5)λ Alt 2: [(1, 2, 3, 4, 5, 6, 7, 8)] as described in section 7.3 in TR38.901 | 2GHz, 4GHz, 7GHz, 15GHz |
| Combination4 | 16 | 4T16R, 8T16R, 16T16R | Alt 1: - 16R: (M, N, P, Mg, Ng; Mp, Np)= (2, 4, 2, 1, 1; 2, 4) , (dH,dV)= (0.5, 0.5)λ Alt 2: - 16R: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901, dual polarization | 7GHz, 15GHz |

(FL2) Proposal 3.2.2-1rv2

For 6GR evaluation, the total transmit power per BS for system-level simulation is assumed as follows:

| Total transmit power per BS | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Sub-urban macro |
| --- | --- | --- | --- | --- | --- |
| Around 700MHz | NA | Macro BS: - Option1: 44 dBm per 20 MHz - Option2: 49 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 2GHz | 24 dBm per 20 MHz | Macro BS: - Option1: 44 dBm per 20 MHz - Option2: 46 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: - Option1: 49 dBm per 20 MHz - Option2: 46 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 4GHz | 24 dBm per 20 MHz | Macro BS: - Option1: 44 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: - Option1: 49 dBm per 20 MHz - Option2: 44 dBm per 20 MHz - Option3: 46 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 7GHz | 24 dBm per 20 MHz | Macro BS: - Option1: 44 dBm per 20 MHz - Option2: 43 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: - Option1: 49 dBm per 20 MHz - Option2: 43 dBm per 20 MHz - Option3: 46 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 15GHz | 23dBm per 20MHz | Macro BS: 40dBm per 20MHz Micro BS: 33 dBm per 20 MHz | NA | Macro BS: 49dBm per 20MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49dBm per 20MHz |
| Note: BS Tx power scales up with bandwidth proportionally under the limitation of the maximum BS Tx power is 56dBm for outdoor and 33dBm for indoor for the above carrier frequencies. |  |  |  |  |  |
| Around 30GHz | - Option1: 23 dBm per 20 MHz - Option2: 16dBm per 20MHz. | Micro BS: 33 dBm per 20 MHz | NA | Micro BS: 33 dBm per 20 MHz | Macro BS: 33 dBm per 20 MHz |
| Note: For around 30GHz, BS Tx power scales up with bandwidth proportionally under the limitation of EIRP 75dBm. |  |  |  |  |  |

(FL2) Proposal 3.2.2-2rv1

For 6GR evaluation, the UE power class for system-level simulation is assumed as follows:

- 29dBm is an optional value.

  - Note: 29dBm is for UE with more than one PA.

- FFS: 31dBm, or 35 dBm with EIRP <55 dBm

| UE power class | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Sub-urban macro |
| --- | --- | --- | --- | --- | --- |
| Around 700MHz | NA | 23dBm | 23dBm | 23dBm | 23dBm |
| Around 2GHz | 23dBm, 26dBm, 29dBm | 23dBm, 26dBm, 29dBm | 23dBm, 26dBm, 29dBm | 23dBm, 26dBm, 29dBm | 23dBm, 26dBm, 29dBm |
| Around 4GHz | 23dBm, 26dBm, 29dBm | 23dBm, 26dBm, 29dBm | 23dBm, 26dBm, 29dBm | 23dBm, 26dBm, 29dBm | 23dBm, 26dBm, 29dBm |
| Around 7GHz | 23dBm, 26dBm, 29dBm | 23dBm, 26dBm, 29dBm | 23dBm, 26dBm, 29dBm | 23dBm, 26dBm, 29dBm | 23dBm, 26dBm, 29dBm |
| Around 15GHz | 23dB, 26dBm, 29dBm | 23dB, 26dBm, 29dBm | NA | 23dB, 26dBm, 29dBm | 23dBm, 26dBm,29dBm |
| Around 30GHz | 23dB, 26dBm, 29dBm Note: EIRP should not exceed 43 dBm | 23dB, 26dBm, 29dBm Note: EIRP should not exceed 43 dBm | NA | 23dB, 26dBm, 29dBm Note: EIRP should not exceed 43 dBm | 23dBm, 26dBm, 29dBm EIRP should not exceed 43 dBm |

(FL2) Proposal 3.3.2-rv2

For 6GR evaluation, the UE distribution and UE speed for system-level simulation is assumed as follows:

- UE number per TRxP will be dependent on the used traffic model. Other values can also be considered in the future evaluations.

- Other velocities and/or other outdoor/indoor ratio will be dependent on the used traffic model or use cases. Other values can also be considered in the future evaluations.

| Parameters | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- | --- | --- |
| UE distribution and UE speed | 10 users per TRxP. 100% Indoor, 3km/h | Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per TRxP is [10, 30]. Opt1: 80% indoor (3km/h); 20% outdoor(30km/h). Opt2: 40% indoor (3km/h) 40% outdoor (3km/h) 20% outdoor (30km/h) | Single layer: Uniform/macro TRxP UE number per TRxP is [10, 30]. Opt1: 50% indoor (3km/h); 50% outdoor(120km/h). Opt2: 20% indoor (3km/h) 40% outdoor (60km/h) 40% outdoor (120km/h) | Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per TRxP is [10, 30]. 80% indoor (3km/h); 20% outdoor(30km/h). | Single layer: Uniform/macro TRxP UE number per TRxP is [10, 30]. 10% Outdoor pedestrian: 3km/h; 10% Outdoor in cars: 40km/h; 80% Indoor in houses: 3km/h. |

(FL2) Proposal 3.4.1-rv1

For 6GR evaluation, the following are assumed for system-level simulation:

| Parameters | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- | --- | --- |
| ISD | 20m, equivalent to 12TRxPs per 120m x 50m | Macro layer: 200m | ISD 1: 1732m ISD 2: 5000m | Macro: 500m | ISD 1: 1299m ISD 2: 1732m |
| BS antenna height | 3m | 25m for macro cells and 10m for micro cells | 35 m | 25m for macro cells and 10m for micro cells | Alt 1: 35m Alt 2: 25m |
| BS noise figure | Around 7GHz and below: 5dB Around 15GHz and above: 7dB |  |  |  |  |
| UE antenna height | TR38.901 Indoor-Office Table 7.2-2 | TR38.901 UMi/UMa Table 7.2-1 | TR38.901 RMa Table 7.2-3 | TR38.901 UMa Table 7.2-1 | TR38.901 SMa Table 7.2-5 |
| UE noise figure | Around 7GHz and below: 9dB (baseline performance), 7dB (high performance) Around 15GHz and above: 13dB (baseline performance), 10dB (high performance) |  |  |  |  |
| UE Receiver | MMSE-IRC as the baseline, R-ML Receiver as optional |  |  |  |  |
| UE Power control parameter for UL | Company report |  |  |  |  |
| Channel model | TR 38.901 v19.1.0 Indoor-Office | TR 38.901 v19.1.0 UMa/UMi | TR 38.901 v19.1.0 RMa | TR 38.901 v19.1.0 UMa | TR 38.901 v19.1.0 SMa, 0% vegetation. |
| Numerology | In line with AI 11.3 |  |  |  |  |
| Scheduling | Proportional fairness (PF) |  |  |  |  |
| Inter-cell interference model | Explicitly and realistically modelled |  |  |  |  |
| [Inter-cell interference estimation model] | Alt 1: Ideal, calculated by ground truth channel matrix Alt 2: Realistic model, Company report, e.g., Wishart distribution-based model; retain only diagonal elements of interference Cov. Matrix. |  |  |  |  |
| Channel estimation assumption | Alt 1: Ideal for benchmark Alt 2: Realistic, company report, e.g., direct/explicit RS estimation, apply gauss noise to real channel matrix, or random |  |  |  |  |
| Feedback assumption | Alt 1: Ideal Alt 2: Realistic, company report, e.g., consider feedback delay and overhead; codebook; |  |  |  |  |
| O2I penetration loss (X% high loss, Y% low loss) | NA | Two options are supported: Option 1: 80% low loss, 20% high loss; Option 2: 50% low loss, 50% high loss | 100% low loss | Two options are supported: Option 1: 80% low loss, 20% high loss; Option 2: 50% low loss, 50% high loss | 50% Low-loss A 50% Low-loss Model as TR38.901. |
| Mechanic tilt | 180° in GCS (pointing to the ground) for performance calibration Company can report other values for evaluations. | 90° in GCS (pointing to horizontal direction) for performance calibration Company can report other values for evaluations. | 90° in GCS (pointing to horizontal direction) for performance calibration Company can report other values for evaluations. | 90° in GCS (pointing to horizontal direction) for performance calibration. Company can report other values for evaluations. | 90° in GCS (pointing to horizontal direction) for performance calibration Company can report other values for evaluations. |
| Electrical tilt | 90° in LCS for performance calibration. Company can report other values for evaluations. | 90° in LCS for performance calibration. Company can report other values for evaluations. | 90° in LCS for performance calibration. Company can report other values for evaluations. | 90° in LCS for performance calibration. Company can report other values for evaluations. | 90° in LCS for performance calibration. Company can report other values for evaluations. |
| Handover margin (dB) | 3dB as baseline. 1dB and 0dB as optional configuration. | 3dB as baseline. 1dB and 0dB as optional configuration. | 3dB as baseline. 1dB and 0dB as optional configuration. | 3dB as baseline. 1dB and 0dB as optional configuration. | 3dB as baseline. 1dB and 0dB as optional configuration. |
| UE attachment | Based on RSRP from BS port 0 |  |  |  |  |
| Wrapping around method | No wrapping around | Geographical distance-based wrapping | Geographical distance-based wrapping | Geographical distance-based wrapping | Geographical distance-based wrapping |
| Multi-TRP operation, e.g., ideal or non-ideal backhaul/sync | Backhaul: ideal or non-ideal; sync: ideal or non-ideal; Company reports the assumptions of the non-ideal backhaul/non-ideal sync. |  |  |  |  |

# References

R1-2507292	Post-122 email discussion on 6GR common evaluation assumptions	Moderator (Huawei)

R1-2508321	Evaluation assumptions for 6GR air interface	FUTUREWEI

R1-2508335	On Evaluation Assumptions for Study of 6G Radio Air Interface	Nokia

R1-2508431	Evaluation methodology and assumptions for 6GR air interface	vivo

R1-2508454	Discussion on evaluation assumptions for 6GR air interface	CMCC

R1-2508524	Discussion on evaluation assumptions for 6GR air interface	ZTE Corporation, Sanechips

R1-2508580	Discussion on evaluation assumptions for 6GR air interface	CATT

R1-2508620	Evaluation assumptions for 6GR air interface	Lenovo

R1-2508627	Discussion on 6G Evaluation Requirements	NEC

R1-2508632	Evaluation assumptions for 6GR air interface	InterDigital, Inc.

R1-2508638	Evaluation Assumptions for 6GR Air Interface	AT&T

R1-2508683	Discussion on evaluation assumptions for 6GR air interface	Xiaomi

R1-2508726	Evaluation assumption for 6GR air interface	OPPO

R1-2508734	Evaluation assumptions for 6GR air interface	Huawei, HiSilicon

R1-2508801	Evaluation assumptions for 6GR	Samsung

R1-2508865	Evaluation assumptions for 6GR	Intel

R1-2508909	Discussion on evaluation assumptions for 6GR air interface	LG Electronics

R1-2508934	Evaluation Assumptions for 6GR Air Interface	Tejas Network Limited, CEWiT, IIT Madras

R1-2508937	Evaluation assumptions for 6GR air interface	NVIDIA

R1-2508972	Discussion on evaluation assumptions for 6GR air interface	ETRI

R1-2509052	On Evaluation Assumptions for the 6GR air interface	Google

R1-2509055	NTN Characteristics for the Evaluation Assumptions for 6GR air interface	ESA, Thales, Viasat, Eutelsat, Airbus, SES, Hispasat

R1-2509062	Discussion on 6GR evaluation assumptions	Sharp

R1-2509073	Evaluation assumptions for 6GR air interface	Sony

R1-2509079	Fixed Wireless Access Scenarios	T-Mobile USA, Ericsson, Nokia, MediaTek

R1-2509109	Evaluation assumptions for 6GR air interface	Apple

R1-2509117	Evaluation assumptions for 6GR	Ericsson AB.

R1-2509132	Discussion on Evaluation assumptions for 6GR air interface	Ofinno

R1-2509142	Evaluation assumptions for 6GR air interface	MediaTek Inc.

R1-2509230	Evaluation assumptions for 6GR air interface	Qualcomm Incorporated

R1-2509281	Discussion on Evaluation assumptions for 6GR air interface	NTT DOCOMO, INC.

R1-2509335	Views on evaluation assumptions for 6GR air interface	CSCN

# Appendix – Existing traffic models

- FTP Model 1, FTP Model 2, and VOIP (in TR 36.814)

#### A.2.1.3 Traffic models Traffic models for system performance evaluations are given in Table A.2.1.3-1. System throughput studies shall be assessed using full-buffer traffic model capturing continuous traffic and non-varying interference. Additionally, evaluations with time-varying interference shall be carried out using bursty traffic models. Table A.2.1.3-1 proposes FTP traffic models to exercise system performance studies in bursty traffic. Table A.2.1.3-1. Traffic Models Traffic Models; Model Applies to. Full buffer; DL and UL. Continuous traffic.. Non-full buffer FTP models; DL and UL. Bursty traffic.. VoIP; DL and UL Real time services. ##### A.2.1.3.1 FTP traffic models Two FTP traffic models are considered as non-full buffer traffic models. Tables A.2.1.3-2 and A.2.1.3-3 show the parameters for FTP traffic model 1 and model 2, respectively. Figure A.2.1.3.1-1 and A.2.1.3.1-2 illustrate the user arrival of traffic model 1 and 2, respectively. Baseline model is Model 1 with file size of 2 Mbytes, however Model 1 with file size of 0.5 Mbytes and Model 2 with file size of 0.5 Mbytes can be also evaluated. Table A.2.1.3.1-1. FTP Traffic Model 1 Parameter; Statistical Characterization. File size, S; 2 Mbytes (0.5 Mbytes optional) (one user downloads a single file). User arrival rate λ; Poisson distributed with arrival rate λ. - Small file size of 0.5 Mbytes can be chosen to speed-up the simulation. - Simulations are run for various λ to find performance metrics covering at least the range of HM-NCT (See A.2.1.3.2) that leads to [10%, 50%] of RU (See A.2.1.3.2) in non-CoMP SU-MIMO. - Possible range of λ: [0.5, 1, 1.5, 2, 2.5] for 0.5 Mbytes, [0.12, 0.25, 0.37, 0.5, 0.625] for 2 Mbytes (See A.2.1.3.4 for more details). Range of λ can further be adjusted. - The same traffic should be simulated for CoMP and non-CoMP schemes. The above range of λ will cover RU from 10% to 50% for non-CoMP SU-MIMO Figure A.2.1.3.1-1: Traffic generation of FTP Model 1 Table A.2.1.3.1-2. FTP Traffic Model 2 Parameter; Statistical Characterization. File Size, S; 0.5 Mbytes. Reading Time, D; Exponential Distribution, Mean= 5 seconds PDF: λ = 0.2. Number of users, K; Fixed. - Simulations are run for various K to find performance metrics covering at least the range of HM-NCT that leads to [10%, 50%] of RU in non-CoMP SU-MIMO. - Possible range of K: [2, 5, 8, 10, 14] (See A.2.1.3.4 for more details). Range of K can further be adjusted. - The reading time D is the time interval between end of download of previous file and the user request for the next file. - The same traffic should be simulated for evaluating CoMP and non-CoMP schemes. The above range of K will cover RU from 10% to 50% for non-CoMP SU-MIMO. Figure A.2.1.3.1-2: Traffic generation of FTP Model 2

- FTP Model 3 (in TR 36.872)

FTP Model 3: based on FTP model 2 with the exception that packets for the same UE arrive according to a Poisson process and the transmission time of a packet is counted from the time instance it arrives in the queue 0.5Mbytes file size. The offered traffic is generated per macro cell geographical area when FTP model 1 is used.

- XR Traffic models (in TR 38.838)

# 5 Traffic models In this clause, we provide the DL and UL traffic models for VR, CG, and AR applications. Since DL/UL traffic models for these applications share similar characteristics, we first define a generic and parameterized DL / UL traffic model, which could be later used in defining VR, CG, AR applications. The traffic model defined in this clause is statistical traffic model, where packet size and packet arrival process are characterized by certain random variables. The described model is based on the input XR traffic study from SA4 [7][3][4]. ## 5.1 Generic DL traffic model ### 5.1.1 Single stream DL traffic model This clause provides a parameterized generic single stream DL traffic model. In this model, as shown in Figure 5.1-1, the XR DL traffic is modelled as a sequence of video frames arriving at gNB according to the considered video frame rates and random jitter. The size of each frame is also random according to a certain distribution. Figure 5.1.1-1: Single stream DL traffic model #### 5.1.1.1 Packet Size In this model, a packet models the set of IP packets belong to the same video frame. The video frame includes both left and right eye frame sharing the same buffer, which is referred to as 'single stream for dual eye buffer' or 'single eye buffer' throughout this document. The size of a packet is determined by the given data rates and frame rates, which is modelled as a random variable following truncated Gaussian distribution with following statistical parameters. Table 5.1.1.1-1: Statistical parameters for packet size following truncated Gaussian distribution Parameter; unit; Baseline values for evaluation; Optional values for evaluation for single eye buffer. Mean: M; byte; R×1e6 / F / 8; R×1e6 / F / 8. STD; byte; 10.5% of M; 3 % of M. Max; byte; 150% of M; 109% of M. Min; byte; 50% of M; 91% of M. R: data rate of the flow in Mbps. F: frame generation rate of the flow in fps. Note that the mean and STD apply before truncation applies. Note that the value of R, F depend on application.. Exploration to other distributions for packet size are left up to each company and could be reported with the modelling details. #### 5.1.1.2 Packet arrival In this model, the packet arrival rate is determined by the frame generation rate, e.g., 60fps. Accordingly, the average packet arrival periodicity is given by the inverse of the frame rate, e.g., 16.6667ms = 1/60fps. The periodic arrival without jitter gives the arrival time at gNB for packet with index k (=1,2,3….) as k/F*1000 [ms], where F is the given frame generation rates (per second). Note that this periodic packet arrival implicitly assumes fixed delay contributed from network side including fixed video encoding time, fixed network transfer delay, etc. However, in a real system, the varying frame encoding delay and network transfer time introduces jitter in packet arrival time at gNB which. In this model, the jitter is modelled as a random variable added on top of periodic arrivals. The jitter follows truncated Gaussian distribution with following statistical parameters shown in Table 5.1-2. Table 5.1.1.2-1: Statistical parameters for jitter Parameter; unit; Baseline value for evaluation; Optional value for evaluation. Mean; ms; 0. STD; ms; 2. Truncation range; ms; [-4, 4]; [-5, 5]. Note that the given parameter values and considered frame generation rates (60 or 120 in this model) ensure that packet arrivals are in order (i.e., arrival time of a next packet is always larger than that of the previous packet). Thus, the periodic arrival with jitter gives the arrival time for packet with index k (=1,2,3….) as offset + k/F*1000 + J [ms], where F is the given frame generation rates (per second) and J is a random variable capturing jitter. Note that actual traffic arrival timing of traffic for each UE could be shifted by the UE specific arbitrary offset. #### 5.1.1.3 Packet delay budget The latency requirement of XR traffic in RAN side (i.e., air interface) is modelled as packet delay budget (PDB). The PDB is a limited time budget for a packet to be transmitted over the air from a gNB to a UE. For a given packet, the delay of the packet incurred in air interface is measured from the time that the packet arrives at the gNB to the time that it is successfully transferred to the UE. If the delay is larger than a given PDB for the packet, then, the packet is said to violate PDB, otherwise the packet is said to be successfully delivered. The value of PDB may vary for different applications and traffic types. #### 5.1.1.4 Packet success rate requirement The performance requirement in terms of packet success rate is given as X (%). If packet delivery delay exceed a given PDB, then, the packet is counted as failure. Following values for packet success rate X are considered. Table 5.1.1.4-1: Packet Success Rate Requirement Parameter; unit; Baseline values for evaluation; Optional values for evaluation. Packet success rate requirement X for DL single stream; %; 99; 95, 99.99, etc.. Note that the Packet error rate (PER) in percentage is given as PER = 100 – X. #### 5.1.1.5 Dual eye buffer model This clause describes optional modification of packet size and frame rates for separate packet arrival for dual-eye buffer. In single eye buffer model, the frame for both eyes arrive at the same time as a single packet. Thus, mean packet size M is given as R×1e6 / F, where R is frame generation rate in Mbps and F is frame generation rate. Whereas, in dual eye buffer model of data rate R, the left and right eye frame arrive separately with a time offset, which makes the arrival process effectively equivalent to have two times of frame rates and half mean packet size of that of single eye buffer model. Accordingly, we have mean packet size M of dual eye buffer model is given as R×1e6 / (2×F) for dual eye buffer model. Table 5.1.1.5-1: Statistical parameter values for dual eye buffer packet size Parameter; unit; values for evaluation; Optional values for evaluation. Mean: M; byte; R×1e6 / (2×F) /8; R×1e6 / (2×F) / 8. STD; byte; 10.5% of M; 4% of M. Max; byte; 150% of M; 112% of M. Min; byte; 50% of M; 88% of M. R: data rate of the flow in Mbps F: frame generation rate of the flow in fps. ### 5.1.2 Multi-streams DL traffic model This clause provides optional multi-streams model for XR DL traffic. - Option 1: I-frame + P-frame - Option 1A: slice-based traffic model - Option 1B: Group-Of-Picture (GOP) based traffic model - Option 2: video + audio/data - Option 3: FOV + omnidirectional stream #### 5.1.2.1 Option 1 (I+P) For Option 1, two streams (I-stream and P-stream) are modelled according to Table 5.1-5. - Stream 1: I stream - Stream 2: P stream Depending on the video encoding scheme, two additional sub models – slice based, and Group of Picture (GOP)-based models are defined. - Slice-based: In this encoding scheme, a single video frame is divided into N slices. Out of N, one slice is I slice and remaining N-1 slices are P slices. N packets (one I and N-1 P) packets corresponds to one video frame arriving at the same time. - GOP-based: In this encoding scheme, a single video frame is either I frame or P frame. I frame is transmitted every K frames, where K is the GOP size, i.e., every group of picture. One video frame arrives at a time as a packet. Table 5.1.2.1-1: Statistical parameters for Option 1 multi streams DL traffic model Two data streams; Option 1A: slice-based; Option 1B: GOP-based. I-stream; P-stream; I-stream; P-stream. Packet modelling; Slice-level; Frame-level. Traffic pattern; Both streams are periodic at 60 fps with the same jitter model as for single stream.; Follow the GOP structure, where GOP size K = 8 with the same jitter model as for single stream.. Number of packets per stream at a time; 1; N-1; I-frame: 1 or 0 P-frame: 0 or 1 At each time instant, there is either only one I-stream packet or only one P-stream packet. N = 8: the number of slices per frame.. Average data rate per stream; RI=R*αN-1+α; RP=R*N-1N-1+α; RI=R*αK-1+α; RP=R*K-1K-1+α. - R: average data rate of a single stream video - α: average size ratio between one I-frame/slice and one P-frame/slice - α = 1.5, 2 (baseline) - α = 3 (optional). Packet size distribution; Truncated Gaussian distribution. Mean = RIFPS; Mean = RPFPS*(N-1); Mean = RI*KFPS; Mean = RP*KFPS*(K-1). - [STD, Max, Min]: [10.5, 150, 50]% of Mean packet size - FPS is the frame rate of the single stream video. Depends on application, see 6.3.1, 6.4.1, 6.5.1 for VR, CG, AR respectively.. PDB; Depends on application, see 6.3.1, 6.4.1, 6.5.1 for VR, CG, AR respectively.. #### 5.1.2.2 Option 2 (video + audio/data) For Option 2, two streams (video + audio/data) are modelled. - Stream 1: video - Stream 2: audio/data The stream 1 - video stream follows the generic single stream model given in clause 5.1.1. The stream 2 - audio/data a periodic traffic with following parameters. Table 5.1.2.2-1: Statistical parameter values for Option 2 multi streams model Parameters; unit; Baseline values for evaluation; Optional values for evaluation. Periodicity P; ms; 10. Data rate: R; Mbps; 0.756, 1.12. Packet size; byte; R×1e6 × P /1000 / 8. PDB; ms; 30; Other values can be optionally evaluated. Packet Success Rate; %; 99; 99.9. #### 5.1.2.3 Option 3 (FOV + omnidirectional view) For Option 3, following two streams are modelled. - Stream 1: FOV - Stream 2: omnidirectional view stream The detailed modelling of the two streams is left to company with the report of evaluation results. ## 5.2 Generic UL pose/control traffic In this clause, we provide the generic UL pose/control stream traffic model. A packet for UL pose/control arrives at UE periodically with following parameters. Table 5.2-1: Statistical parameters for the UL pose/control traffic Parameters; unit; Baseline values for evaluation; Optional value for evaluation. Periodicity; ms; 4; Other values can be optionally evaluated.. Jitter; ms; No jitter. Packet size; byte; 100. PDB; ms; 10. Packet Success Rate X; %; 99; 90, 95.

- Instant message (as in TR 38.840)

Traffic model used for the UE power saving scheme evaluation - Applications with the traffic model for the evaluation of the UE power saving scheme - FTP - FTP model 3 - Other bursty traffic arrival models can be considered - Web-browsing - Video streaming - Instant messaging - VoIP - Gaming - Background app sync For FTP, instant messaging, and VoIP application, the following traffic models and DRX configuration should be included for evaluation: FTP traffic; Instant messaging; VoIP. Model; FTP model 3; FTP model 3; As defined in R1-070674. Assume max two packets bundled.. Packet size; 0.5 Mbytes; 0.1 Mbytes. Mean inter-arrival time; 200 ms; 2 sec. DRX setting; Period = 160 ms Inactivity timer = 100 ms; Period = 320 ms Inactivity timer = 80 ms; Period = 40 ms Inactivity timer = 10 ms. Note: For ON duration setting, following reference DRX configurations as previously agreed. - For web-browsing, video streaming, and gaming applications, the traffic models and the delay requirements defined in R1-070674 can be used in the evaluation. The parameters (e.g. packet size) may be updated to be in line with EMBB traffic requirements. - For background app sync application, for power consumption evaluation purpose, it can be assumed that idle mode operations (inclusive of page detection, RRM, deep sleep and transition overhead) contributes to X% of the use case power. The remaining portion is contributed by intermittent RRC connections due to background activities (FFS: value of X) - Companies should report the assumptions made in the evaluation

# Appendix - Agreements

## Agreements from RAN1#122

Agreement

- The deployment scenarios in TR38.914 should be considered for evaluation assumption

- The common evaluation assumptions including the antenna modelling, general system-level simulation assumptions (including the carrier frequency, bandwidth and subcarrier spacing used for link-level simulation) for the deployment scenarios in TR38.914, link budget and traffic models will be discussed in AI 11.2

  - Other assumptions including for link-level simulation specific to each technical topic will be separately discussed under each individual agenda.

  - Note: Subcarrier spacing decision is up to AI 11.3.2.

Conclusion

- Template in R1-2506582 is to be used for collecting inputs from companies.

  - Additional NTN or TN assumptions, if any, or any necessary change of the parameters, are to be incorporated into the updated one of R1-2506582.

Agreement

- Study which of the following traffic models are to be used for 6G evaluations, e.g.,

  - Full buffer

  - FTP Model 1 (in TR 36.814)

  - FTP Model 2 (in TR 36.814)

  - FTP Model 3 (in TR 36.872)

  - XR Traffic models (in TR 38.838)

  - VoIP model (as in TR 36.814)

  - Instant message (as in TR 38.840)

- Study whether to introduce the following traffic models for 6G evaluations considering, e.g.,

  - FTP-3 variant with packet delay budget requirement

    - Details FFS

  - New traffic model considering a mixed/variable packet size and the associated time domain behaviors (e.g., time between adjacent packet arrivals, packet delay budget)

    - Details FFS

  - New traffic model(s) considering the new use cases or services, e.g., AI/ML services, immersive communication services, etc.

    - Details FFS

- Study whether to introduce new/additional approaches that can reflect the impact of bidirectional traffic flows on performance metrics (e.g., impact of UL TCP ACK latency on DL throughput/latency)

Note: Whether/how to consider the combination of traffic model and loading level will be studied under individual agendas.

## Agreements from RAN1#122bis

Agreement

For around 700MHz, for TXRU mapping at base station, it is adopted as mandatory option for simulation campaign that a single TXRU is mapped per panel per subarray per polarization.

Note: Companies can provide results optionally, assuming fully connected TXRU mapping within a panel per polarization.

Agreement

- For around 700MHz, 32 for total number of antenna element at base station, 4 for total number of TXRU at base station, (8, 2, 2, 1, 1; 1, 2) for (M,N,P,Mg,Ng; Mp, Np), and (0.5, 0.5)λ for (dH,dV) are assumed as the baseline combination.

- For around 700MHz, 64 for total number of antenna element at base station, 8 for total number of TXRU at base station, (8, 4, 2, 1, 1; x, y) for (M,N,P,Mg,Ng; Mp, Np), and (0.5, 0.5)λ for (dH,dV) are assumed as the optional combination.

Note: Other values/combinations are up to company to report

Agreement

For around 2GHz carrier frequency, for BS antenna modelling

| BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg , Ng; Mp, Np) | (dH,dV) |
| --- | --- | --- | --- | --- |
| Indoor |  |  |  |  |
| Combination 1(Optional) | 8 | 4 | (2, 2, 2, 1, 1; 1, 2) | (0.5, 0.5)λ |
| Combination 2 (Baseline) | 32 | 8 | (4, 4, 2, 1, 1; 1, 4) | (0.5, 0.5)λ |
| Outdoor |  |  |  |  |
| Combination 1(Optional) | 32 | 4 |  | (0.5, 0.8)λ |
| Combination 2 (Baseline) | 192 | 64 | (12, 8, 2, 1, 1; 4, 8) | (0.5, 0.5)λ |
| Note1: A single TXRU is mapped per panel per subarray per polarization as mandatory option. Companies can provide results optionally, assuming fully connected TXRU mapping within a panel per polarization. Note2: Other combinations used in the simulation results are up to company to report. |  |  |  |  |

Conclusion

The following existing traffic models could be used for 6GR performance evaluations,

- Full buffer

- FTP Model 1 (in TR 36.814)

- FTP Model 3 (in TR 36.872)

- XR Traffic models (in TR 38.838)

- VoIP model (as in TR 36.814)

- Instant message (as in TR 38.840)

- Note that which model(s) will be used can be further decided when performing simulations in each individual topic.

Agreement

For the study traffic model(s) for 6GR AI/ML services:

  - A representative AI/ML service is the generative AI, e.g., as defined in TR22.870.

Send LS to SA4 (cc RAN2, SA1, SA2) requesting input if any on traffic characteristics for AI/ML services.

Note: RAN1 is discussing the following options for the model:

  - Option-1a: The model is parameterized by Token, e.g., Token size, Token arrival rate, and Token delay budget.

    - Token is the minimum unit of data generated in the application layer.

    - How to associate Tokens to PHY layer packets.

    - How to reflect the variable importance of tokens.

    - Whether other parameters are additionally needed when tokens are encapsulated together into a packet, e.g., packet arrival rate, packet success rate, and packet delay.

  - Option-1b: The model is characterized by the parameters of PHY layer packet, including e.g., packet size, arrival rates, latency requirement, reliability requirement, etc.

  - Option-1c: reusing or extending the FTP-3/XR traffic model.

  - FFS other models/options need to be defined for other AI/ML services.

Agreement

Study traffic modelling for evaluations related to immersive communication services including but not limited to advanced XR [e.g., TR22.870] and haptics services,

- XR traffic models (in TR 38.838) are considered as starting point.

  - FFS the detailed modifications on the parameters to the XR traffic model, e.g., higher packet size, higher packet arrival rate, higher packet size deviation, PDB, etc.

- FFS how many models need to be defined and the corresponding representative use cases.

- FFS how to incorporate haptics traffic (TR26.854).

Send LS to SA4 requesting input if any on the relevant traffic characteristics, RAN1 can continue the study before SA4 potential response.

Agreement

Study extensions to FTP Model 1/FTP Model 3 to incorporate the following:

- Multiple packet sizes and associated time-domain behaviors (e.g., inter arrival time)

  - FFS number of packet sizes (e.g., 2 or 3).

  - FFS whether to have fixed or variable packet size and packet arrival rate for a given UE.

  - FFS applicability of multiple packet sizes to only one or both of FTP Model 1/FTP Model 3.

  - FFS packet size and arrival rate characteristics.

- Packet delay budget (PDB) related parameters

  - FFS PDB applicability to packets (e.g., one PDB parameter for only one traffic flow or different PDB parameters for different traffic flows).

  - FFS how to consider the PDB, e.g., whether to drop packets when exceeding the budget, PDB aware metric.

- Note consider the following for PDB:

  - Applicability to the extension to FTP Model 1/ FTP Model 3 with one packet size.

  - Applicability or not to the extension to FTP Model 1/ FTP Model 3 with multiple packet sizes.

Agreement

The attached templates for NTN in R1-2507956 are endorsed in principle.

Agreement

The following configurations for system-level simulations could be used for 6GR evaluation:

|  | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Sub-urban macro |
| --- | --- | --- | --- | --- | --- |
| Carrier frequency | Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz |
| Aggregated BW | Follow system bandwidth per carrier frequency in TR 38.914 as 1) Around 700 MHz: Up to 60 MHz 2) Around 2GHz: Up to 200 MHz 3) Around 4GHz: Up to 300 MHz 4) Around 7GHz: Up to 400MHz 5) Around 15GHz: Up to 400MHz 6) Around 30GHz: Up to 1GHz |  |  |  |  |
| Simulation BW | Around 700 MHz: 20MHz, 60MHz |  |  |  |  |
|  | Around 2 GHz: 20MHz, 100MHz, 200MHz |  |  |  |  |
|  | Around 4 GHz: 20MHz, 100MHz, 200MHz, 300MHz |  |  |  |  |
|  | Around 7 GHz: 20MHz, 100MHz, 200MHz, 400MHz |  |  |  |  |
|  | Around 15 GHz: 20MHz, 100MHz, 200MHz, 400MHz |  |  |  |  |
|  | Around 30GHz: 100MHz, 400MHz, 800MHz |  |  |  |  |
|  | Note: other simulation BW could be considered. |  |  |  |  |
| Note: The layout for each scenario will be separately discussed, including the carrier frequency combination for single layer and/or two layers. |  |  |  |  |  |

Agreement

Draft LS R1-2508183 is endorsed in principle.

Agreement

Final LS R1-2508184 is endorsed.

Agreement

For link budget template, consider the following candidates:

- Candidate 1: Reusing the link budget template from TR38.830, i.e., the following table with notes as follows:

- The values of the parameters are TBD.

- MCL in row (22bis) is TBD.

- FFS: whether/how/why to update

| System configuration |  |
| --- | --- |
| Channel for evaluation |  |
| Scenarios and Carrier frequency (GHz) |  |
| BS antenna heights (m) |  |
| UT antenna heights (m) |  |
| Cell area reliability (%) |  |
| Lognormal shadow fading std deviation (dB) |  |
| Tx Diversity |  |
| Number of SSB |  |
| Transmitter |  |
| (1) Number of transmit antenna elements |  |
| (2) Number of transmit TxRUs Note: this row is void (left empty) for uplink |  |
| (2a) Number of transmit chains modelled in LLS |  |
| (3) Total transmit power (dBm) Note: total transmit power for system bandwidth |  |
| (3a) System bandwidth for downlink, or occupied bandwidth for uplink (Hz) |  |
| (3b) Power Spectrum Density = (3) - 10 log( (3a) / 1000000 ) (dBm/MHz) Note: no PSD constraint for uplink |  |
| (3c) Bandwidth used for the evaluated channel (Hz) Note: (3c) is identical to the number of PRBs assigned to the channel evaluated. For uplink, (3a) = (3c) |  |
| (3bis) Total transmit power for occupied bandwidth = (3b) + 10 log ((3c) /1000000) (dBm) |  |
| (4) Total antenna gain at antenna gain component 3 & antenna gain component 4 of transmitter = (4a) – (4b) (dB) |  |
| (4a) Antenna gain at antenna gain component 3 & antenna gain component 4 of transmitter = (4c) + 10 log ((1) / (2)) (dB) for downlink, and = (4c) + 10 log ((1) / (2a)) (dB) for uplink |  |
| (4b) Antenna gain correction factor at antenna gain component 3 & antenna gain component 4 of transmitter (dB) |  |
| (4c) Gain of antenna element (dBi) |  |
| (5) Total antenna gain at antenna gain component 2 of transmitter = (5a) - (5b) (dB) Note: zero for uplink |  |
| (5a) Antenna gain at antenna gain component 2 of transmitter = 10 log((2)/(2a)) (dB) Note: zero for uplink |  |
| (5b) Antenna gain correction factor at antenna gain component 2 of transmitter (dB) Note: zero for uplink |  |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) |  |
| (9) EIRP = (3bis) + (4) + (5) – (8) dBm |  |
| Receiver |  |
| (10) Number of receive antenna elements |  |
| (10a) Number of receive TxRUs Note: this row is void (empty) for downlink |  |
| (10b) Number of receive chains modelled in LLS |  |
| (11) Total antenna gain at antenna gain component 3 & antenna gain component 4 of receiver = (11a) - (11b) (dB) |  |
| (11a) Antenna gain at antenna gain component 3 & antenna gain component 4 of receiver = (11c) + 10 log ((10)/(10a)) (dB) for uplink = (11c) + 10 log ((10)/(10b)) (dB) for downlink |  |
| (11b) Antenna gain correction factor at antenna gain component 3 & antenna gain component 4 of receiver (dB) |  |
| (11c) Gain of antenna element (dBi) |  |
| (11bis) Total antenna gain at antenna gain component 2 of receiver = (11bis-a) - (11bis-b) (dB) Note: zero for downlink |  |
| (11bis-a) Antenna gain at antenna gain component 2 of receiver = 10 log((10a)/(10b)) (dB) Note: zero for downlink |  |
| (11bis-b) Antenna gain correction factor at antenna gain component 2 of receiver (dB) Note: zero for downlink |  |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) |  |
| (13) Receiver noise figure (dB) |  |
| (14) Thermal noise density (dBm/Hz) |  |
| (15) Receiver interference density (dBm/Hz) |  |
| (16) Total noise plus interference density = 10 log (10^(( (13) + (14))/10) + 10^((15)/10)) (dBm/Hz) |  |
| (18) Effective noise power = (16) + 10 log ((3c)) (dBm) |  |
| (19) Required SNR (dB) |  |
| (20) Receiver implementation margin (dB) |  |
| (21) H-ARQ gain (dB) Note: Only applicable if HARQ is not considered in LLS |  |
| (22) Receiver sensitivity = (18) + (19) + (20) – (21) (dBm) |  |
| (22bis) MCL = (3bis) – (22) + (5) + (11bis) (dB) |  |
| (23) Hardware link budget, a.k.a. MIL = (9) + (11) + (11bis) − (12) − (22) (dB) Note: MIL can also be derived by (22bis) + (4) – (8) + (11) − (12) |  |
| Calculation of available pathloss |  |
| (25) Shadow fading margin (function of the cell area reliability and lognormal shadow fading std deviation) (dB) |  |
| (26) BS selection/macro-diversity gain (dB) |  |
| (27) Penetration margin (dB) |  |
| (28) Other gains (dB) (if any please specify) |  |
| (29) Available path loss = (23) – (25) + (26) – (27) + (28) (dB) |  |
| Range/coverage efficiency calculation |  |
| FFS: (30) Maximum range (based on (29) and according to the system configuration section of the link budget) (m) |  |

- Candidate 2: Template as Table 7.10.1-1 from TR38.913.

  - FFS: whether/how/why to update.

| Item | Value |
| --- | --- |
| Transmitter |  |
| (1) Tx power (dBm) |  |
| Receiver |  |
| (2) Thermal noise density (dBm/Hz) |  |
| (3) Receiver noise figure (dB) |  |
| (4) Interference margin (dB) |  |
| (5) Occupied channel bandwidth (Hz) |  |
| (6) Effective noise power = (2) + (3) + (4) + 10 log(5) (dBm) |  |
| (7) Required SINR (dB) |  |
| (8) Receiver sensitivity = (6) + (7) (dBm) |  |
| (9) MaxCL = (1) - (8) (dB) |  |

Agreement

For around 4GHz carrier frequency:

| BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg, Ng; Mp, Np) | (dH,dV) |
| --- | --- | --- | --- | --- |
| Indoor |  |  |  |  |
| Combination 1 | 32 | 32 | (4, 4, 2, 1, 1; 4, 4) | (0.5, 0.5)λ |
| Combination 2 | 128 | 32 | (8, 8, 2, 1, 1; 2, 8) | (0.5, 0.5)λ |
| Combination 3 | 256 | 64 | (16, 8, 2, 1, 1;4, 8) | (0.5, 0.5)λ |
| Outdoor |  |  |  |  |
| Combination 1 | 192 | 64 | (12, 8, 2, 1, 1; 4, 8) | (0.5, 0.8)λ |
| Combination 2 | 256 | 64 | (16, 8, 2, 1, 1; 4, 8) | (0.5, 0.8)λ |
| Combination 3 | 512 | 128 | (16, 16, 2, 1, 1; 4, 16) | (0.5, 0.5)λ |
| Note1: A single TXRU is mapped per panel per subarray per polarization as mandatory option. Companies can provide results optionally, assuming fully connected TXRU mapping within a panel per polarization. Note2: Other combinations used in the simulation results are up to company to report. |  |  |  |  |

For around 7GHz carrier frequency:

| BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg, Ng; Mp, Np) | (dH,dV) |
| --- | --- | --- | --- | --- |
| Indoor |  |  |  |  |
| Combination 1 | 64 | 32 | (4, 8, 2, 1, 1; 2, 8) | (0.5, 0.5)λ |
| Combination 2 | 256 | 64 | (16, 8, 2, 1, 1; 4, 8) | (0.5, 0.5)λ |
| Combination 3 | 512 | 128 | (16, 16, 2, 1, 1; 8, 8) | (0.5, 0.5)λ |
| Outdoor |  |  |  |  |
| Combination 1 | 768 | 128 | TBD | (0.5, 0.8)λ |
| Combination 2 | 1024 | 256 | (32, 16, 2, 1, 1; 8, 16) | (0.5, 0.8)λ |
| Combination 3 | 1536 | 256 | TBD | (0.5, 0.8)λ |
| Combination 4 | 2048 | 256 | (32, 32, 2, 1, 1, 8, 16) | (0.5, 0.5)λ |
| Combination 5 | 2048 | 512 | (64, 16, 2, 1, 1; 16, 16) | (0.5, 0.5)λ |
| Note1: A single TXRU is mapped per panel per subarray per polarization as mandatory option. Companies can provide results optionally, assuming fully connected TXRU mapping within a panel per polarization. Note2: Other combinations used in the simulation results are up to company to report. |  |  |  |  |

For around 30GHz carrier frequency:

| BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg, Ng; Mp, Np) | (dH,dV) |
| --- | --- | --- | --- | --- |
| Indoor |  |  |  |  |
| Combination 1 | 128 | 8 | (4, 4, 2, 2, 2; 1, 1) | (0.5, 0.5)λ |
| Combination 2 | 512 | 8 | (8, 8, 2, 2, 2; 1, 1) | (0.5, 0.5)λ |
| Combination 3 | 1024 | 8 | (16, 8, 2, 2, 2; 1, 1) | (0.5, 0.5)λ |
| Combination 4 | 768 | 2 | (24, 16, 2, 1, 1; 1, 1) | (0.5, 0.5)λ |
| Outdoor |  |  |  |  |
| Combination 1 | 2048 | 16 | (16, 8, 2, 4, 2; 1, 1) | (0.5, 0.5)λ |
| Combination 2 | 4096 | 32 | (16, 8, 2, 4, 4; 1, 1) | (0.5, 0.5)λ |
| Combination 3 | 1024 | 4 | (16, 16, 2, 2, 1; 1, 1) | (0.5, 0.5)λ |
| Note1: A single TXRU is mapped per panel per polarization as mandatory option. Companies can provide results optionally, assuming a single TXRU is mapped per panel per subarray per polarization as mandatory option. Note2: Other combinations used in the simulation results are up to company to report. |  |  |  |  |

Agreement

At least the following carrier frequencies could be considered (from RAN1 perspective) for 6GR NTN evaluations:

- L-band (i.e., 1.5GHz)

- S-band (i.e. 2 GHz)

- Ku-band (FFS detailed frequency range)

- Ka-band (i.e. 30 GHz for UL, 20GHz for DL)

## Agreements from RAN1#123

Agreement

Updating the BS antenna modelling agreed in the last meeting as follows:

- For around 700MHz carrier frequency, for BS antenna modelling,

- update the (8, 4, 2, 1, 1; x, y) to be (8, 4, 2, 1, 1; 1, 4).

- For around 2GHz carrier frequency, for BS antenna modelling,

- for outdoor combination 1 (i.e., 32AE/4TXRU), update the (M,N,P,Mg,Ng; Mp,Np) to be (8, 2, 2, 1, 1; 1, 2)

- For around 7GHz carrier frequency, for BS antenna modelling,

- for outdoor combination 1 (i.e., 768AE/128TXRU), update the (M,N,P,Mg,Ng; Mp,Np) to be (24, 16, 2, 1, 1; 4, 16).

- for outdoor combination 3 (i.e., 1536AE/256TXRU), update the (M,N,P,Mg,Ng; Mp,Np) to be (48, 16 ,2, 1, 1; 8, 16).

Agreement

For 6GR evaluation, the layout for system-level simulation is assumed as follows:

- Note: Single layer will be prioritized for the evaluations.

- Note: The carrier frequency for the corresponding layout for the two layers will be reported by companies for the evaluations.

- FFS the minimum distance for random drop in two layers.

- Note: for system-level simulation of MIMO schemes, specific assumptions could be discussed under MIMO discussion

| Parameters | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- | --- | --- |
| Layout | Single layer - Indoor floor (Open office), (Room size: 120m x 50m) | Single layer: - Hex. Grid Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop | Single layer: - Hex. Grid | Single layer: - Hex. Grid Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop | Single layer: - Hex. Grid |