3GPP TSG RAN WG1 Meeting #123		                                   R1-250xxxx

Dallas, USA, November 17th – 21st, 2025

Agenda Item:	11.4.1

Title:	FL summary#1 for 6G channel coding

Source:	Moderator (ZTE, Apple)

Document for:	Discussion, Decision

- Introduction

In RAN#109, the SID for 6GR has been updated in [1]. The detailed objectives of the study for 6GR channel coding are:

c) Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6GR requirements and characteristics with acceptable performance/complexity trade-off [RAN1].

In this document, summary of both data and control channel coding are provided.

The draft FL proposals will be found in each section with the following way of naming:

Proposal 3.1.1-1-v1: XXX … - ‘3.1.1’ is the section number in this document - ‘1’ is marked as the serial of proposals under this section, e.g. different proposals in this section will be numbered as {1, 2, 3, …} - ‘v1’ is the version of a proposal, and will be numbered as {v1, v2, v3, …}

- Proposals for online discussion

  - Proposals for xxx online

- Data channel coding

  - Evaluation methodology

    - Evaluation metrics

Summary of observations/proposals

| Source | Observation/Proposal |
| --- | --- |
| vivo | Observation 2: Besides increased data rates, other aspects motivating the data channel extension include improved efficiency and BLER performance. Typically, a smaller number of iterations can reduce energy consumption while negatively impacting the BER/BLER performance. Overall, it is desirable to reduce energy-/area-cost in supporting higher throughput, while maintaining reasonable performance similar to that of NR, such as similar SNR operation point of BLER@10E-1 in NR eMBB scenarios. Proposal 1: The LDPC extension in 6GR should carefully balance throughput, energy-/area-cost, and BLER performance. Proposal 7: Consider using the number of edges per information bit * number of iterations (required to reach a target BLER) to evaluate the complexity of a given LDPC extension design. |
| CMCC | In the last RAN1 meeting, companies discussed the evaluation metrics for decoding throughput of LDPC codes [5]. From our perspective, the decoding throughput of LDPC codes can be approximated as follows: Tput≈c×Kb×ZI×Titer [bps], where c is the number of LDPC decoder cores, Kb is the number of columns for information bits in a base graph, Z denotes the lifting size, I denotes the number of decoding iterations, and Titer denotes the decoding time per iteration. |
| CATT | For block paralleled decoder, we use the following equation to calculate throughput, which is similar with that in [12-13]: K denotes the length of information bits; fclk denotes the operating frequency; Here, we assume fclk = 1 GHz; I denotes the number of iteration; Here, we assume I = 8; WB denotes the weight of the base matrix; C denotes the number of decoding cores; T0 denotes the ratio of the weight of the last row of the base matrix to the number of decoding cores. As delineated by equation , elevating C enhances throughput; however, an excessively large C induces incremental clock cycle latency due to synchronization dependencies. Consequently, the selection of C necessitates a systematic trade-off analysis between throughput optimization and latency minimization. For instance, when C is configured within the range of 1 to 5 under a fixed iteration count of 8, the achievable throughput of 5G LDPC codes is presented in Figure 1. Observation 1：As shown in Figure 1, the 5G LDPC codes achieve the peak data rate of about 80 Gbps under the configuration of 5 decoding cores and 8 iterations. |
| Lenovo | Proposal 2: Data channel coding extensions should be designed to balance performance–complexity trade-offs while maximizing hardware reuse between 5G and 6G systems. Proposal 14: RAN1 to clarify the following point in SID and make consensus: - Evaluation/analysis metrics for computational and implementation complexity |
| OPPO | where the complexity on x-axis is calculated as a maximum iteration number {2 3 4 5 6 7 8 9 10 15 20 25} multiplying a normalized factor (i.e., number of edges per information bit). |
| Huawei | - For LDPC codes, the performance-complexity tradeoff can be optimized through improved decoding efficiency. - When evaluating LDPC extension for higher throughput, a fair comparison should be conducted considering: - BLER performance shall be thoroughly investigated and compared under the same computational complexity; - If throughput is to be reported, area efficiency should be reported together instead of throughput alone: - It is recommended that a unified area evaluation model is used for evaluating area efficiency; - Hardware throughput and latency: Considering both processing latency and the extra waiting delay in LDPC decoding. |
| Samsung | Proposal 4: LDPC decoding throughput is analyzed based on the following equation: Tput≈kb×ZI×Titer×NDEC [bps] where kb denotes the number of columns for information bits in a BG, Z is lifting size, I is the maximum number of decoding iterations, Titer is decoding cycle per iteration, and NDEC means the number of decoder blocks. - Performance: Target block error rate (BLER) [10-3] - Complexity: Number of one in BG/PCM, Average number of iterations (ANI) - Latency: Number of one in BG/Number of layers (w/ or w/o row merging), Average number of iterations (ANI) |
| ZTE | Proposal 1: The following metrics should be evaluated for 6GR LDPC design: - Performance requirements including throughput, BLER results - Complexity including computational complexity, hardware complexity Proposal 2: TB level BLER performance should be considered. Proposal 3: The decoding throughput for different LDPC codes should be compared under the condition of the same decoding parallelism. |
| Tejas | Proposal 3: To evaluate the data channel coding enhancements, we would like to propose the following metrics - LDPC throughput - BLER - Throughput Gain vs Complexity - Decoding complexity (number of computations and Chip area) - Decoding Latency - Energy efficiency |
| MediaTek | Proposal: Consider I×C as the estimation of total decoding cycles per CB - C: decoding cycle per iteration - C can be approximated by e/M, where e is the number of edge in a BG and M is the number of edges available to be processed simultaneously, if any - I: [Average number or max number] of iterations to achieve target BLER at target SNR - Target BLER: [0.01] - Target SNR=Reference SNR+[<0.5]dB - Reference SNR: The SNR where BG1 achieves target BLER under Layer BP with 20 iterations Proposal: To ensure BLER performance is acceptable for all MCS, consider the following metric to facilitate peak data rate evaluation for a decoder operating at maximum frequency fmax - minMCS=XCBsizefmaxImcs=X×Cmcs=X×RMCS=27RMCS=X×QAMMCS=27QAMMCS=X - Imcs=X is the minimum iterations to satisfy BLER performance requirement Observation 6: Bottleneck case to determine peak data rate of BG1 is MCS=20 based on peak data rate estimation minMCS=XCBsizefmaxImcs=X×Cmcs=X×RMCS=27RMCS=X×QAMMCS=27QAMMCS=X. |
| Qualcomm | Proposal 6: For 6GR LDPC code enhancement study, RAN1 shall discuss metrics to capture the decoding complexity in the complexity-performance tradeoff evaluations, e.g., the normalized number of decoding iterations with respect to one iteration of NR BG1. |
| NTT DOCOMO | Proposal 3 - RAN1 to clarify the following points in SID and make consensus - Evaluation/analysis metrics for performance/complexity trade-off Proposal 4 - Regarding the evaluation/analysis of “performance/complexity trade-off” for data channel coding, at least the following metrics should be considered - Performance: BLER, throughput, decoding latency - Complexity: computational complexity and complexity relevant to implementation aspects Proposal 5 - RAN1 to discuss how to define complexity relevant to implementation aspects Proposal 7 - The starting point of the formula for LDPC decoding throughput is Throughput∝Kb×ZI×Titer Kb is the number of information columns in LDPC BG, Z is the lifting size, I is the number of iterations, Titer is the number of decoding cycles per iteration. Proposal 8 - The starting point of the formula for LDPC decoding latency is Latency∝I×Titer I is the number of iterations, Titer is the number of decoding cycles per iteration. Proposal 9 - RAN1 to discuss to ensure fair evaluation and to specify a common implementation method (e.g., block parallel, row parallel, etc.) and/or decoding algorithm as evaluation assumptions Proposal 10 - If computational complexity is treated as a metric to be considered, the number of addition/comparison operations in decoding process should be evaluated - The starting point of the formula for LDPC computational complexity is Computational complexity=I∙M∙W I is the number of [average/maximum] iteration, M is the number of rows of the parity check matrix H, W is the number of calculations per row (may vary depending on the decoding algorithm assumed for evaluation) |
| Rakuten Mobile | Proposal 3: In studying channel coding for 6GR data channels beyond NR, use at least the following evaluation assumptions: - TR 38.901 channel model and its extensions for the new frequency range, NTN, and ISAC scenarios - Modulation Orders in the range from QPSK, 16-QAM, 64-QAM, 256-QAM and 1024-QAM - Payload Sizes - FFS: study typical sizes for small packets and large packets. - Code rates in range [1/3, 2/5, 1/2, 2/3, 3/4, 5/6, 8/9, 948/1024] - Non-adaptive HARQ assumption with [X] maximum retransmissions - Decoding algorithms: - BP and min-sum for LDPC / extended LDPC with detailed implementation including number of iterations in range [2, 3, 4, 5, 6, 8, 10, 20] - System Bandwidths - FFS: study typical bandwidths reflecting representative 6G configurations. Proposal 4: In studying channel coding for 6GR data channels beyond NR, use at least the following evaluation methodology: - Link-Level Simulations - System-Level Simulations - Note: to evaluate latency and reliability in realistic traffic scenarios with diverse packet sizes and HARQ configurations. - Hardware-Level Analysis - Note: to evaluate decoding complexity, including gate count, memory footprint, and power consumption. |

Summary of inputs

In RAN1#123 meeting, companies discussed the evaluation methodology for 6G data channel coding. In general, the evaluation metrics include BLER performance, decoding throughput/latency, computation/implementation complexity, memory, area efficiency, energy efficiency. Companies’ views are summarized as below.

- BLER performance: vivo, Huawei, Samsung, ZTE

  - vivo: Besides increased data rates, other aspects motivating the data channel extension include improved efficiency and BLER performance.

  - Huawei: BLER performance shall be thoroughly investigated and compared under the same computational complexity

  - Samsung: Since this fixed limit cannot scale with evolving system requirements, large TB transmissions are subject to extensive segmentation and consequent performance loss. Each CB is encoded and decoded independently with its own CRC, while the overall TB includes an additional TB CRC. The relationship between TB and CB error rates can be expressed as: ℙℰTB=1−1−ℙℰCBC, where ℙℰTB and ℙℰCB represent the TB and CB error rate, respectively, and C denotes the number of code blocks.

  - ZTE: Different LDPC coding schemes may define different maximum numbers of systematic columns and lifting sizes, which may lead to variations in the maximum information length. TB level BLER performance should be considered, which can be derived by first simulating CBER for the code‑block length after segmentation and then computing BLER using BLER = 1 − (1 − CBER)C, where C represents the number of code blocks obtained from segmentation.

Throughput: vivo, CMCC, CATT, Lenovo, Huawei, Samsung, ZTE, Tejas, Fujitsu, MediaTek, NTT DOCOMO

  - vivo: For a row decoder with row parallelism of 1, the throughput can be easily estimated by the following equation

where, L is the length of code block, I is the number of iterations, NLayer is the number of layers, Tpipis the pipelined processing clocks required for CNU/VNU message updating in each layer, fc is the operating frequency of the decoding processors.

  - CMCC: the decoding throughput of LDPC codes can be approximated as follows:

Tput≈c×Kb×ZI×Titer [bps],

where c is the number of LDPC decoder cores, Kb is the number of columns for information bits in a base graph, Z denotes the lifting size, I denotes the number of decoding iterations, and Titer denotes the decoding time per iteration.

  - CATT: For block paralleled decoder of LDPC code,

K denotes the length of information bits;

fclk denotes the operating frequency; Here, we assume fclk = 1 GHz;

I denotes the number of iteration; Here, we assume I = 8;

WB denotes the weight of the base matrix;

C denotes the number of decoding cores;

T0 denotes the ratio of the weight of the last row of the base matrix to the number of decoding cores.

  - Lenovo: It could be roughly expressed as:

- T≈Zc× Kb×f×PIiter×C

Where Zc is the lifting size, Kb is number of columns in the BG corresponding to information bits, f is the clock frequency, P is the degree of parallelism (e.g. number of decoder cores), Iiter is the number of iterations, and C is the number of decoding cycles per iteration.

  - Huawei: If throughput is to be reported, area efficiency should be reported together instead of throughput alone: It is recommended that a unified area evaluation model is used for evaluating area efficiency. Hardware throughput and latency: Considering both processing latency and the extra waiting delay in LDPC decoding.

- Tℎrougℎput=Kb×Z×fI×(Ep+md∗twait)×D.

where:

- f is the operating frequency

- D is the number of decoders

- Kb is the number of systematic columns,

- Z is the lifting size,

- I is the maximum number of iterations,

- p is the number of edges that can be processed in parallel,

- m is the number of rows in the LDPC base graph,

- d is the number of orthogonal rows processed simultaneously in block parallel decoding,

- and twait is the waiting time to complete the processing of a previous row before starting to process a new row.

  - Samsung: LDPC decoding throughput is analyzed based on the following equation:

Tput≈Kb×ZI×Titer×NDEC [bps]

where Kb denotes the number of columns for information bits in a BG, Z is lifting size, I is the maximum number of decoding iterations, Titer is decoding cycle per iteration, and NDEC means the number of decoder blocks.

  - ZTE: For LDPC code,

Throughput=f⋅c⋅kb⋅zI⋅Eb

Wherein, f is operating frequency, c is the number of decoders, b is the number of blocks processed simultaneously in block parallel decoding, kb is the number of systematic columns, z is lifting size, I is the maximum number of iterations, E is the number of '1' in the LDPC base graph.

  - Fujitsu: QC-LDPC codes achieve high decoding throughputs which can be approximately calculated by the formula as follows:

Throughput= Kb∙Z∙fTclock∙E∙I

Where Kb is the number of information nodes in BG, Z is lifting size, f is clock frequency, E is the number of ones in BG, I is the number of iterations, Tclock is the number of clock cycles needed for processing one block in BG

  - MediaTek: The Tput of QC-LDPC decoder for a given operating frequency rate f, BG associated with given code rate, and lifting size Z can be approximated as

Tput≈NI∙(Kb×Z)∙fI×C

where NI is the number of decoders, Kb is the number of information column in the BG, I is the number of iterations deployed at decoding algorithm, and C is the number of decoding cycles per iteration

To ensure BLER requirement is satisfied for all MCS, consider the following metric to facilitate peak data rate evaluation for a decoder operating at maximum frequency fmax

- minMCS=XCBsizefmaxImcs=X×Cmcs=X×RMCS=27RMCS=X×QAMMCS=27QAMMCS=X

  - Imcs=X is the minimum iterations to satisfy BLER performance requirement.

  - NTT DOCOMO:

Throughput∝Kb×ZI×Titer

Kb is the number of information columns in LDPC BG,

Z is the lifting size,

I is the number of iterations,

Titer is the number of decoding cycles per iteration.

Complexity: vivo, Lenovo, OPPO, Huawei, Samsung, ZTE, Tejas, MediaTek, Qualcomm, NTT DOCOMO

  - vivo: using the number of edges per information bit * number of iterations (required to reach a target BLER) to evaluate the complexity of a given LDPC extension design.

  - Lenovo: clarify the metrics of computational and implementation complexity

  - OPPO: a maximum iteration number multiplying a normalized factor (i.e., number of edges per information bit).

  - Huawei: The decoding complexity is as below.

| Coding Scheme | LDPC |
| --- | --- |
| Decoder | LMS |
| Addition | I∙M(2dc+2) |
| Comparison | I∙M(2dc-3) |
| Total Computational Complexity | I∙M(4dc-1) |

The notations in the table are:

- K is the number of information bits;

- N is the code length;

- L is the list size used by the SCL decoding;

- I is maximum number of iterations for the LMS decoding;

- M is the number of rows in the LDPC parity check matrix (after lifting);

- dc is the average degree of the parity nodes in the LDPC parity check matrix (after lifting).

  - Samsung: Number of one in BG/PCM, Average number of iterations (ANI)

  - ZTE: For LDPC code, the total computational complexity is as follows:

Operations=IM4dc−1= I4Ez−mb∙z

where N is the number of columns in the binary parity check matrix H, and M is the number of rows in the binary parity check matrix H, the size of the base graph is mb rows by nb columns, the size of the base graph is mb rows by nb columns.

For fair comparison, normalized computational complexity regarding to information bit size should be used for evaluating different designs of LDPC BG schemes, such as:

NormOperations=I4Ez−mb∙zkb∙z=I4E−mbkb

    - For LDPC code, the hardware complexity (converted to the number of adders) is as follows:

Hardwares=6p+round2⋅2⋅log2p+1⋅p−2log2p+1+14.7

    - For LDPC code, the required memory is as follows:

3M+N= z∙3mb+nb

  - MediaTek

The memory area can be roughly categorized into 4 blocks: LLR memory, R memory, Q memory, and sign memory.

The LLR memory: c×(8+8)×Zmax, where c is the number of columns (VNs) in the BG and 8 bits are assumed to represent each of LLRs.

The estimated R memory is r ×(8+8+ceil(log2©)) ×Zmax, where r is the number of rows in the BG.

The Q memory is w×(8)×Zmax, where w is the maximum row weight in the BG.

The sign memory is e×(1)×Zmax, where e is the number of edges(1s) in the BG.

For logic area, it can be generally assumed to be proportional to Zmax.

  - Qualcomm: RAN1 shall discuss metrics to capture the decoding complexity in the complexity-performance tradeoff evaluations, e.g., the normalized number of decoding iterations with respect to one iteration of NR BG1.

Niternormalized= (#edges in a given BG/Kbnew )×#iterations(#edges in NR BG at the same rate)/KbNR

KbNR and Kbnew denotes the number of information columns in the base graph for NR and new LDPC codes, respectively.

  - NTT DOCOMO: the formula for LDPC computational complexity is

Computational complexity=I∙M∙W

I is the number of [average/maximum] iteration,

M is the number of rows of the parity check matrix H,

W is the number of calculations per row (may vary depending on the decoding algorithm assumed for evaluation)

- Area efficiency: vivo, Huawei

  - vivo: provided in Table 2/3, calculated as throughput/number of gates

  - Huawei:

Area efficiency = Tℎrougℎput  (bit/second)Area (mm2 or #PE)

The area for LDPC decoder can be estimated as

Area=PEnum+Astore×D

Where PEnum=PEnumllr+PEnumqsn≈4×p+b1.33×Z⌈logZ⌉, Astore=3M+N+21.33×b−1×M, M is the number of check nodes, N is the number of variable nodes, and b is the number of parallel-processed block

- Latency: Samsung, Tejas, MediaTek, NTT DOCOMO

  - Samsung: number of one in BG/number of layers (w/ or w/o row merging), average number of iterations (ANI)

  - MediaTek: Consider I×C as the estimation of total decoding cycles per CB

- C: decoding cycle per iteration

  - C can be approximated by e/M, where e is the number of edge in a BG and M is the number of edges available to be processed simultaneously, if any

- I: Average number of iterations to achieve target BLER at target SNR

  - Target BLER: [0.01]

  - Target SNR=Reference SNR+[<0.5]dB

  - Reference SNR: The SNR where BG1 achieves target BLER under Layer BP with 20 iterations

  - NTT DOCOMO: for LDPC decoding latency is

Latency∝I×Titer

I is the number of iterations,

Titer is the number of decoding cycles per iteration.

- Energy efficiency: vivo, ZTE, Tejas, Qualcomm

Discussion

To evaluate the candidate 6G channel coding schemes, a common set of evaluation metrics is required. In RAN1#122 meeting, chairman’s guidance was to provide evaluation on BLER performance, throughput, complexity, and decoding latency. Furthermore, the agreed working assumption of 6G data channel coding in RAN1#122bis meeting is for higher throughput with acceptable performance-complexity tradeoff for both NW and UE. Therefore, these metrics need to be evaluated in the design of LDPC extension.

Table 3.1.1-1 Summary of companies’ input on evaluation metrics

| BLER | 19 sources vivo(*), CMCC, CATT(*), Lenovo(*), OPPO(*), Huawei(*), Samsung(*), ZTE(*), Tejas, SJTU(*), LGE(*), ETRI(*), ESA(*), Thales(*), Apple(*), Qualcomm(*), Ericsson(*), Fujistu(*), MTK(*) |
| --- | --- |
| Throughput | 11 sources vivo, CMCC, CATT, Lenovo, Huawei, Samsung(*), ZTE(*), Tejas, Fujitsu, MediaTek, NTT DOCOMO |
| Latency | 3 sources Samsung (*), Tejas, NTT DOCOMO |
| Complexity | 10 sources vivo, Lenovo, OPPO, Huawei(*), Samsung, ZTE(*), MediaTek, Tejas, Qualcomm, NTT DOCOMO |
| Area efficiency | 2 sources vivo (*), Huawei(*) |
| Energy efficiency | 4 sources vivo (*), ZTE(*), Tejas, Qualcomm (*) |
| Note:(*) resource with evaluation results |  |

As summarized in Table 3.1.1-1, comprehensive evaluation on the metrics has been provided in RAN1#123 meeting. FL suggests that at least BLER performance, decoding throughput/latency, complexity should be considered for the evaluation of 6G channel coding scheme. Meanwhile, other metrics, such as area efficiency, energy efficiency, are also important aspects that need to be considered in the implementation of LDPC decoder.

For throughput calculation, most companies proposed similar formulations which can be expressed as below.

Throughput=Kb×Z×fI×Titer×C,

Where Kb is the number of systematic columns, Z is the lifting size, f is the operating clock frequency, I is the number of decoding iterations, Titer is the decoding cycles per iteration, and C is the number of decoder cores.

Meanwhile, MediaTek and Samsung proposes to consider decoding cycles per code block, i.e., I×Titer. In FL’s understanding, the decoding throughput and decoding latency are interchangeable measures.

Furthermore, FL observes that normalization the throughput in relative to NR LDPC can eliminate different implementation, such as clock frequency.

From companies’ input, the complexity evaluation includes computational complexity, required number of processing elements, and required memory.

- For computation complexity, it is based on the operation number of additions, comparison, multiplication, etc., during LDPC decoding. 7 sources (vivo, OPPO, Huawei, Samsung, ZTE, Qualcomm, NTT DOCOMO ) provided computational complexity models, where 3 sources (vivo, OPPO, ZTE) suggested evaluating the complexity per information bit, 4 sources (vivo, OPPO, Samsung, NTT DOCOMO) suggested simplifying the model as the number of edges [per information bit] * number of iterations (required to reach a target BLER). 1 company (Qualcomm) suggested normalizing the complexity in relative to one iteration of NR BG decoding.

- For processing elements, it includes components such as adders, comparator, QSN (quasi-shift network) etc. 3 sources (vivo, ZTE, Huawei) provided the calculation of required number of processing elements.

- 4 sources (Huawei, Samsung, ZTE, MediaTek) provided required memory models/calculation.

For area efficiency, 2 sources (vivo, Huawei) provided the model, the methodology is similar, i.e., throughput/chip area.

For energy efficiency, 3 sources (vivo, ZTE, Qualcomm) provided the evaluation results of energy efficiency.

Based on the companies’ input, FL proposal for evaluation metrics is as below. Considering different companies may have different implementations, the detailed definitions/models of these metrics can be reported by companies.

Round 1

- Proposal 3.1.1-1-v1

Proposal 3.1.1-1-v1: For the evaluation of 6G data channel coding scheme(s) for higher throughput with acceptable performance-complexity tradeoff, at least the following metrics should be reported following Chair’s guidance in RAN1#122.

- BLER performance

- Complexity

- Decoding throughput/latency

In addition, the following metrics can be also reported

- Area efficiency

- Energy efficiency

Note: The detailed definitions/models of the metrics should be reported.

Please provide your comments as below.

| Company | Comments |
| --- | --- |
| CMCC | Support the first three bullets. For the last two bullets, we think it is not appropriate to evaluate area efficiency/energy efficiency in 3GPP because unified evaluation among different companies is hard to achieve. |
| CATT | We are OK with the KPI of BLER, complexity and throughput/latency. However, the area efficiency and energy efficiency have too many implementation specific aspects, which would be challenged to be aligned in understanding. |
| AccelerComm | Memory usage should also be included among the first three bullet points. |
| Xiaomi | Thanks for the good proposal, we are mostly fine except for the energy efficiency part. From our reading of the TDocs listed, the energy efficiency evaluation focus on the energy consumption linked with ET of DCI and power saving gain linked to reduced number of iterations with data channel encoding/decoding. We concur the contributions listed this metric for data channel as well, but the evaluation methodology is not clear to us, making it difficult to agree. Thus can be say FFS this metric, and companies can for sure report this metric as well if a methodology is developed for this metric as others. If in the meeting such methodology can be clarified as well, we can also discuss this point. Proposal 3.1.1-1-v1: For the evaluation of 6G data channel coding scheme(s) for higher throughput with acceptable performance-complexity tradeoff, at least the following metrics should be reported following Chair’s guidance in RAN1#122. - BLER performance - Complexity - Decoding throughput/latency In addition, the following metrics can be also reported - Area efficiency - FFS: Energy efficiency, Note: The detailed definitions/models of the metrics should be reported. |
| Lenovo | We support the proposal, however a consensus on the decoding throughput formula and complexity evaluation should be reached to allow for better alignment/calibration of evaluation metrics among all companies. |
| NTT DOCOMO | Even though there may be room for discussion of details, we are OK to support three first bullets as metrics for data channel coding. Complexity may contain computational complexity and complexity relevant to implementation aspects. In addition, further discussion for the detailed assumptions for BLER performance and decoding throughput/latency is needed. |
| ZTE, Sanechips | Agree. First of all, we think jointly considering the following three aspects: BLER performance, complexity, and decoding throughput is needed. These metrics are interdependent so that a careful trade-off analysis of these metrics is required. Secondly, since we need to consider performance-complexity trade-off for higher throughput LDPC enhancement, area efficiency or energy efficiency can be one of angle to analyze. To this end, we are okay to report area efficiency and energy efficiency. Regarding the comment for energy efficiency, it would be related to power saving gain from our perspective. And it can be also reflected by computational complexity. But based on other contributions, it can be evaluated by other methodologies. To this end, we think it is ok to report energy efficiency with detailed definition/models. |

    - Evaluation assumption

Summary of observations/proposals

| Source | Observation/Proposal |
| --- | --- |
| Nokia | Proposal 2: To facilitate the discussion on the data rate beyond NR range for the study of LDPC extension, RAN1 to consider discussing the max TBS supported in 5G NR Rel-15 per carrier. Proposal 4: Applicability of any potential LDPC extension to data rate within NR range should be well justified. In case it is justified, it should not be considered for low coding rate and low MCS, and should not overlap with the applicability range of BG2. |
| vivo | Observation 1: 6GR may target at least 2 times the peak data rates as NR does, which presents clear motivations for the extension to data channel coding. Table 4. Simulation assumptions for LDPC extensions Parameters; Values or assumptions. Channel; AWGN. Modulation; QPSK. Code rate; 1/2, 2/3, 4/5, 8/9 Note: Other values are reported by companies. HARQ (if applicable); IR-HARQ. Decoding algorithm; layered min-sum (baseline), layered BP Note1: scale and offset should be reported by companies if normalized min-sum is used Note2: layer scheduling should be reported by companies. Code block size (bits); In the range of 8K and [16K, 32K] Note: Intermediate CBS is determined based on Kb and Zc; for the design with different Kb, puncturing is used to meet a same CBS. Target BLER, CBLER; BLER or CBLER: 0.01 Note: CBLER is the baseline. If target CBS cannot be achieved, a single TB with multiple codeblocks is used, and BLER is used.. Table 5 shows our proposed evaluation assumptions for coding chain design. Table 5. Simulation assumptions for coding chain design Parameters; Values or assumptions. Channel; CDL-A, CDL-D DS=30/100/300ns, user speed =3,120km/h. Carrier Frequency; 6 GHz. SCS; 30 kHz. FFT size; 4096. Data Allocation; - 1, 60, 260 RBs - 2/6/12 OFDM symbols - DMRS pattern (follow NR). BS antenna configuration; 2 Tx/Rx ports as start point Other values are not precluded. UE antenna elements; 1Tx2Rx ports as start point Other values are not precluded. Modulation; QPSK, 256QAM. Code rate; QPSK: 120/1024, 308/1024, 602/1024 256QAM: 682.5/1024, 754/1024, 885/1024, 948/1024. Coding Scheme; LDPC. HARQ (if applicable); IR-HARQ. Decoding algorithm; layered min-sum Note1: scale and offset should be reported by companies if normalized min-sum is used Note2: layer scheduling should be reported by companies. Code block size (bits); up to 8448, 16896, 33792 Note: Smaller CBS is determined based on Kb and Zc. Channel estimation; Real estimation (Proponent should report DMRS pattern with RS overhead). Target BLER; 0.1. Proposal 8: Consider the parameters in Table 4 and Table 5 as simulation assumptions for evaluating respectively LDPC extension and coding chain design for data channel coding. |
| CATT | Evaluation cases: QPSK; K=9720~14400; CR=1/3~2/3 K=200,1024, 4576; CR=1/3, 2/3 Decoding algorithm: SPA |
| Lenovo | Observation 1: Channel Coding enhancements are required to satisfy 6G KPIs given new emerging services and corresponding requirements. Observation 2: New emerging services such as immersive communication, XR and next-gen IoT require higher peak data rates and more stringent requirements on spectral and energy efficiency. Proposal 1: Consider the evaluation assumptions in Table 2.1.2-1 in adopting a set of evaluation assumptions for data channel coding extensions. |
| AT&T | Proposal 1 Discussion on target peak data rates for 6GR is not a RAN WG1 related issue, and it should be discussed in RAN plenary meetings. Proposal 2 The channel coding study for both data and control channels should focus on improvements in complexity, reliability and migration efficiency with respect to NR incumbent networks. Proposal 3 Precise characterization of “NR range” notion used in the agreement for channel coding is to be provided in RAN1#123. Proposal 4 Reuse NR LDPC codes and NR Polar codes for 6GR data channel(s) and 6GR control channel(s), respectively, at least under the same NR conditions on code rate and code block length. Proposal 5 Proposed enhancements for 6GR channel coding beyond the NR range should only address critical issues related to performance, complexity at both the network side and the device side, as well as consider the potential migration needed from NR to 6GR deployment. Proposal 6 LDPC base graph selection based on data rate regimes for DL-SCH is not supported in 6GR. |
| Xiaomi | Proposal 3: For 6GR, channel coding evaluation shall be performed channel wise instead of scenario wise. - LDPC is the data channel candidate and the evaluation assumptions need to reflect the requirements for at least IC/hRLLC/MC - Polar is the control channel candidate and the evaluation assumptions need to reflect the requirements for at least IC/hRLLC/MC Proposal 4: For 6GR, the following evaluation assumptions can be used to check whether the channel coding candidates fulfill the 6GR requirements. - Evaluate the block error rate (BLER) performance versus SNR Evaluated Channel Type Evaluation Assumption; Data Channel; Control Channel. Modulation; QPSK; QPSK, 64 QAM, 256 QAM. Coding Scheme; LDPC; Polar. Code rate; 1/12, 1/6, 1/5, 1/3, 2/5, 1/2, 2/3, 3/4, 5/6, 8/9; 1/12, 1/6, 1/5, 1/3. Decoding algorithm**; min-sum; List decoding. Info. block length*** (bits w/o CRC); 20, 40,100, 200, 400, 600, 1000, 2000, 4000, 6000, 8000 Optional(12K, 16K, 32K, 64K). Channel*; AWGN. * Fading channels will be simulated in the next stage ** These algorithms are starting points for further study. Other variants of agreed algorithms can be used for encoding and decoding (Complexity details should be illustrated) *** At least these info. block length and code rate shall be evaluated. Other info. block lengths and code rates are not precluded. Similar info. and encoded block lengths should be used for the evaluation. Total coded bits = info. Block length/code rate. Note: these info. block length and code rate are only for initial performance evaluations. They are not interpreted as design targets or assumptions for complexity analysis.. - General guidelines - BLER simulations down to 10-1 , 10-3 is recommended (to observe the error floor) for IC/MC - BLER simulations down to 10-4 is recommended (to observe the error floor) for hRLLC |
| OPPO | Evaluation case: BLER performance under Kmax; QPSK, AWGN; R=1/3; LayerBP 25 Observation 3: For less iterations, the performances of BG1/2 with top-to-bottom scheduling decoding can hardly represent the baseline 5G code performance. So the comparison between 5G code and 6G code still needs to be done on BG1/2 with other specifically enhanced decoding scheduling. |
| Huawei | Evaluation case: different code rate/MCS. |
| Samsung | Observation 1: The per CC maximum DL throughput in a deployed NR modem chip can be considered to be approximately 3 Gbps. Proposal 1: The upper bound of the "data rate within the NR range" should be defined 3 Gbps as the maximum data rate supported by currently deployed modem implementations. Proposal 2: The 6GR network should support configurability that enables selection between the NR BGs and the HT-BG. - Evaluate the complexity and block error rate (BLER) performance versus SNR Channel; AWGN. Modulation; QPSK. Coding Scheme; LDPC code. Code rate; 1/3, 2/5, 1/2, 2/3, 3/4, 5/6, 8/9, 11/12. Decoding algorithm; Sum-product + Layered decoding. Decoding configurations; Number of iterations = 2 : 1 : 20 Precoder syndrome check (early termination) Decoding validity check = syndrome & CRC & genie check. Info. block length (bits w/o CRC); 100 : 100 : 8400 for NR maximum lifting size 8500 : 500 : 16500, 17000 : 1000 : 32000 for larger maximum lifting size. Evaluation metrics and criteria - Performance: Target block error rate (BLER) [10-3] - Complexity: Number of one in BG/PCM, Average number of iterations (ANI) - Latency: Number of one in BG/Number of layers (w/ or w/o row merging), Average number of iterations (ANI) |
| ZTE | Observation 29: The maximum bandwidth around 7GHz (including the upper 6GHz band) has been agreed as four times that of 5G FR1 and the peak efficiency of IMT-2030 has been agreed to be twice that of IMT-2020. The combination of these enhancements will result in 6G peak data rate per carrier that is eight times that of IMT 2020. - Considering the agreed maximum channel bandwidth and peak spectral efficiency, the target peak data rate in 6G can be 2, 4 or 8 times of the target peak data rate in 5G NR. Observation 30: NR data range should be considered as NR data rate supported per carrier which depends on capability of different device types and network implementation. To support portable devices for immersive communication, 6G channel coding should be designed to not only support high data rate, but also to ensure more efficient implementation and reduce power consumption for XR devices with strict form factor limitations and area efficiency requirements. - The data rate for a XR device (i.e. around 1Gbps in NR) should be enhanced to a higher data rate (e.g. 4Gbps) with the same form factor (i.e., the same chip area). Observation 31: To make sure efficient channel decoding processing for high cell throughput in network side for UL, it’s important to make sure all the 6G UEs are capable of new 6G channel coding even though each individual UE throughput is lower than 5G peak data rate. - Whether a base station needs to switch to a new channel coding method (to support beyond its original NR capability) depends on its decoder resource per carrier and the network load which should be determined by the network itself. Observation 32: Efficient 6G LDPC design with better performance-complexity trade-off should be applied within NR range. If a new decoder is implemented, it is more cost/energy efficient to make the best out of the new decoder by covering NR range as well. Table 1 Evaluation assumption for LDPC coding Parameter settings; Details. Channel; AWGN channel. Modulation scheme; QPSK. Target TBLER; TBLER=10−1. TBLER= 1 − 1 − CBLERC. Wherein TBSm is derived from TS38.214 based on 273RB, 256QAM, 948/1024 code rate, 4MIMO layers.. Target CBLER; CBLER=10−2, 10−4 (same as NR). Wherein information sizes are [100, 400], 1000, 2000, 4000, 6000, 8448.. Code rate; 1/3, 2/5,1/2,2/3,3/4,5/6,8/9,948/1024. Decoding algorithm; Layered BP Iteration times: 3~20 (To be reported by company) Decoding order: reversed order. Table 2 Evaluation assumption for channel coding chain Parameter settings; Details. Channel model; AWGN channel; Fading channel: CDL, TDL, Delay Spread of 30ns/100ns. Carrier frequency; 4GHz. Subcarrier spacing; 30kHz. FFT size; 4096. Channel estimation; IDEAL/MMSE. Target BLER; 10%. UE speed; 3 km/h. DMRS Overhead; 1 OFDM Other values are not precluded.. Data allocation; 13 OFDM symbols Other values are not precluded.. Bandwidth; 50MHz/100MHz, 133RBs/273RBs/260RBs Other values are not precluded.. HARQ; IR-HARQ. TBS; TBS is calculated by bandwidth, code rate, modulation order and number of layers as in 38.214.. Code rate / Modulation scheme; MCS Tables in TS38.214. Channel coding; 5G LDPC or new LDPC. Antenna configuration; 1Tx/4Tx/32Tx, 2Rx/4Rx, Close loop spatial multiplexing, NR type1 codebook or SVD. MIMO layer number; 1~4. MCS/RI; MCS adaption / RI adaptation. |
| Tejas | Proposal 1: Possible enhancements to current LDPC codes for transport block sizes larger than 5G NR are - Use 5G NR LDPC base graphs (BG1 and BG2) and increase lifting size beyond 384 for BG1 - Study reducing complexity and iterations for faster decoding convergence - Reuse 5G NR base graphs and lifting sizes - Increase parallelism (more number of codeblocks to be processed simultaneously) at both encoder and decoder - Design a new LDPC base graph for 6G Radio supporting larger codeblock sizes (> 8448) - Increase the number of systematic columns - Fast decoding convergence |
| SJTU, NERCDTV | Observation 1: Both the service demands from emerging 6G use cases and the intrinsic capabilities of the 6G network (e.g., wider bandwidth, higher MIMO layers) point to a required cell throughput that can easily exceed 40 Gbps. This represents a significant leap from the 20 Gbps design target of 5G, creating a clear and undeniable motivation for a fundamental evolution in data channel coding to handle these higher data rates efficiently. Proposal 1: To support the demanding data rates driven by new 6G services and enhanced network capabilities, the study on 6GR channel coding should target a design capable of efficiently handling throughputs in the range of at least 40 Gbps, with a primary focus on achieving this with a sustainable and area-efficient hardware implementation. Evaluation case: AWGN, QPSK, NMS, CR=1/3,Z=128, K=2816 |
| LGE | Evaluation case: BPSK, Zmax=384, CR 0.8462, flooding BP Proposal 3: Decision on simulation parameters for the comparison of different proposals for high throughput new base graph or modifications in current base graph, e.g., the range of code rates, the range of iteration and etc. |
| Fujitsu | Observation 1: - The error floors of 5G LDPC codes could basically satisfy the reliability requirement of 6GR which are all below 10−5 under different MCS indices. - The throughput of 5G LDPC codes may not meet the 6GR requirement of peak data rate. - The max decoding throughput that 5G LDPC codes can reach is about 20Gbps. - The code rates supported by 5G BG1/BG2, which are between 1/5 and 11/12, may be not enough for 6GR to support HARQ-disable/free transmissions. |
| ETRI, ESA, Thales | Proposal 2. RAN1 to consider the following evaluation assumptions for data channel coding evaluation of 6G channel coding study: Parameters; Values or assumptions. Channel; AWGN. Modulation; QPSK. Code rate; Variable code rates can be used.. HARQ; HARQ-disable (baseline), IR-HARQ (if needed). Code bock size; Variable code block sizes can be used.. Target BLER; CBLER = 10-2 for eMBB CBLER = 10-5 for NTN, HRLLC Note #1: To check potential error floor, down to CBLER = 10-6 may be necessary. Note #2: TBLER can be used when it is properly converted from CBLER = 10-2 and CBLER = 10-5.. Decoding algorithm of LDPC; Normalized min-sum, Layered offset min-sum Iteration times: 3~20. |
| Apple | Simulation case: AWGN, BPSK, CR=0.71, 0.88, K=8448, Layer BP 32 |
| MediaTek | Observation 1: Due to the 6G maximum SE decision made in RAN and newly introduced spectrum, 2x-4x peak data rate improvement from the 5G QC-LDPC design is anticipated in 6G. Proposal: Study 6G QC-LDPC design to achieve 2x-4x peak data rate of 5G with acceptable performance-complexity tradeoff for both NW side and UE side provided that additional performance and complexity trade-offs required from 2 to 4 times peak data rate increase is minimized Simulation case: CR=2/3, 3/4, 5/6, 8/9, 11/12. Decoding algorithm LBP |
| Ericsson | Proposal 1 Confirm the working assumptions from RAN1#122bis, i.e. • For 6G LDPC, for data rate within NR range, reuse of NR LDPC design is supported. • For 6G Polar code, for control information within NR range (larger than 11 bits), reuse of NR Polar code design is supported. Observation 1 While RAN1 may continue study of LDPC extension based on the interim assumption made in RAN1#122b, the target peak data rate for 6G coding needs to follow RAN plenary guidance. Proposal 2 The baseline for study of LDPC extensions for data rate beyond NR range (i.e. >20 Gbps) should be use of NR LDPC code as is (e.g. use of multiple parallel NR decoders to achieve higher peak data rate target). Proposal 6 For evaluation of candidates for LDPC extensions for high peak data rate (>20 Gbps), the following evaluation assumptions are used as starting point. • AWGN, QPSK, QC-LDPC • Code rate: at least from 0.75 up to ~0.925 • Layered min-sum (scaling factor =1) • Number of iterations: 10~15 • Information block length: > 8448 bits • Baseline for comparison: i. Two NR BG1 decoders in parallel to support peak data rates up to 40 Gbps • Metrics: BLER results reported together with at least the following parameters: code rate, information block size, SNR, decoder details, number of iterations, code construction, rate matching method, etc. |
| Qualcomm | Observation 2: 5G NR supports diverse UE device types and diverse UE tiers with very different peak data rate conditions. 5G Low tier modems and Redcap/eRedcap modems may only support peak data rate ranges between 20Mbps to 2.5Gbps in the downlink. Observation 4: New LDPC code may provide substantial benefits in implementation and energy efficiency for billions of devices in 6G, at a very small cost on the network side to support an encoder for the new LDPC code. Proposal 2: For 6G data channel coding, RAN1 should first focus on the design of the new LDPC code (with improved performance-complexity tradeoff). The decision whether the new LDPC code is applicable to data rate within NR range shall be made only after thorough investigations of the benefits of the new LDPC code. - A layer that is connected to the punctured node by a single edge has higher priority than a layer that is connected to the punctured node by double edges - A layer with lower degree has higher priority than a layer with higher degree. Observation 8: For layered decoding, the order in which the layers are decoded affects the performance at small number of iterations. Proposal 7: For 6GR LDPC code study, RAN1 shall consider layered decoding for the performance evaluations, where the decoding schedule (i.e., ordering of the layers to be processed) can be reported by the proponent company. Finally, as we discussed in section 2.4, for high throughput use cases, it is more important to evaluate the scenarios with higher order QAM and high coding rate. The MCS with QPSK and lower coding rate (e.g., ~1/3) is less critical. Therefore, in our view, the evaluation shall focus on large TB size scenarios, or equivalently lower CBLER target. Proposal 8: For 6GR LDPC code study and evaluation with improved performance-complexity tradeoff - RAN1 shall prioritize the evaluations of MCS values with higher order QAM and higher code rate. - Both 10−2 and 10−4 CBLER shall be evaluated (same as NR). Simulation case: AWGN, QPSK/256QAM, K=8448, CR=2/3, 0.88 |
| NTT DOCOMO | Observation 1 - Regarding the agreement of RAN1#122bis, the lower bound for the data rate range for the 6G LDPC study and the upper bound for the NR data rate range are not clearly defined Proposal 1 - RAN1 should clarify the definitions of NR data rate range and the data rate range for 6G LDPC study - NR data rate range - Upper bound: the peak data rate defined in TR38.913, i.e., 20 Gbps for DL, 10 Gbps for UL - Data rate range for 6G LDPC study - Lower bound: the peak data rate defined in TR38.913, i.e., 20 Gbps for DL, 10 Gbps for UL - Upper bound: 2 × the peak data rate defined in TR38.913 Proposal 2 - RAN1 to study the criteria for determining whether the data rate is within or beyond the NR range in the RAN1 specifications There should be several possible options for this method: - Option-1: Switching is by setting/activation/indication via RRC, MAC-CE, DCI, etc. - Option-2: Switching is based on whether the number of bits transmitted/received within a given time unit exceeds the criterion data rate. - The number of bits transmitted/received can be derived from, for example, TB size, CB size, coding rate, number of transmission/reception MIMO layers, MCS index, etc., across active carriers at each UE |
| Vodafone, AT&T, BT, Bouygues Telecom, Deutsche Telekom, Orange, Telecom Italia, Nokia, SK Telecom, Ericsson, T-Mobile, Rakuten Mobile | Observation 1: Peak data rates, even those achievable in the field, are becoming increasingly irrelevant for MNOs. Observation 2: Maximum theoretical peak rate assuming 400MHz contiguous bandwidth and a realistic number of spatial layers is still significantly below the IMT2020 requirements. Observation 3: Modifications on channel coding for data channels and control information may have direct CAPEX/OPEX impact with respect to the 5G NR incumbent networks. Proposal 1: Decision on peak target data rates for 6GR should be done by RAN Plenary only. Proposal 2: Agree on the following for data and control channel coding: - For data rate at least within NR range, reuse NR LDPC design - For control information at least within NR range (larger than 11 bits), reuse NR Polar code design - FFS: Clarification on the definition of "NR range" |

Summary of inputs

Based on the working assumptions/agreements in RAN1#122bis meeting, companies’ provided views on the aspects, such as target peak data rate for LDPC extension, data range of NR range, and the applicability within NR range, etc. Companies’ views are summarized as below.

The increase compared with target peak data rate of NR: vivo, Lenovo, ZTE, SJTU, MediaTek, Qualcomm, NTT DOCOMO

- 2x of 5G peak data rate: vivo (at least 2x), SJTU, NTT DOCOMO

- 5x: Lenovo

- Up to 8x: ZTE

- 2x~4x: MediaTek

- 2.5x: Qualcomm

- Decided by RAN-P: Vodafone, AT&T, BT, Bouygues Telecom, Deutsche Telekom, Orange, Telecom Italia, Nokia, SK Telecom, Ericsson, T-Mobile, Rakuten Mobile

Interpretation of data rate of NR range: Nokia, Samsung, ZTE, Ericsson, Qualcomm, NTT DOCOMO, Vodafone, AT&T, BT, Bouygues Telecom, Deutsche Telekom, Orange, Telecom Italia, SK Telecom, Rakuten Mobile, T-Mobile

- One carrier: Nokia (TBS), Samsung (3Gbps for 5G commercial UE), ZTE (different interpretation between UL and DL), Qualcomm (one carrier or multiple carriers depends on diverse UE device types, e.g., 20Mbps, 2.5Gbps)

- 20Gbps: Ericsson, NTT DOCOMO

- Clarification of NR range: Vodafone, AT&T, BT, Bouygues Telecom, Deutsche Telekom, Orange, Telecom Italia, Nokia, SK Telecom, Ericsson, T-Mobile, Rakuten Mobile, NTT DOCOMO

Applicability within NR range: Nokia, Samsung, ZTE, Qualcomm

- Non-overlapping with BG2: Nokia

- Configured by NW: Samsung, ZTE

- Focus on the new LDPC design: Qualcomm

FL observes that companies’ understanding of these aspects is diverse. Meanwhile, in FL’s understanding, the interpretation of NR range and the application of the new LPDC is also relevant to the LDPC design and companies’ views on the evaluation assumption about information block size and code rate. Regardless how NR range should be interpreted, the applicability of LDPC extension will be studied according to the agreement in RAN1#122bis. So, it is up to companies’ choice to provide evaluation results for both within and beyond NR range.

In RAN1#123 meeting, 19 sources (Nokia, vivo, CATT, Lenovo, Xiaomi, OPPO, Huawei, Samsung, ZTE, Tejas, SJTU, LGE, ETRI, ESA, Thales, MediaTek, Ericsson, Qualcomm, NERCDTV) discussed the evaluation assumptions for data channel coding or provided evaluation results. The views on information block size and code rate are summarized in Table 3.1.2-1, which are also diverse. Therefore, FL suggests the simulated information block size and code rate can be reported by companies. Furthermore, considering the checking point in 2026 June and more efficient usage of TUs, we can start to discuss the interpretation of NR range and the application of the new LPDC design after the design is clear.

Table 3.1.2-1 Summary of companies’ views on information block size and code rate

| Parameters | Values |
| --- | --- |
| Code rate | - 1/2, 2/3, 4/5, 8/9: vivo - 1/3~2/3: CATT (*) - 1/5;1/2; 2/3;3/4;8/9: Lenovo - 1/12, 1/6, 1/5, 1/3, 2/5, 1/2, 2/3, 3/4, 5/6, 8/9: xiaomi - 1/3, 2/5, 1/2, 2/3, 3/4, 5/6, 8/9, 11/12: Samsung - 1/3, 2/5,1/2,2/3,3/4,5/6,8/9,948/1024: ZTE - 1/3: SJTU (*) - 0.71,0.88: Apple (*) - 2/3, 3/4, 5/6, 8/9, 11/12: MediaTek (*) - 0.75 up to ~0.925: Ericsson - Higher modulation order and higher code rate: Qualcomm |
| Transport block size | - In the range of 8K and [16K, 32K]: vivo - 2000,4000,6000,8000,10K,12K, 16K: Lenovo - 20, 40,100, 200, 400, 600, 1000, 2000, 4000, 6000, 8000 Optional(12K, 16K, 32K, 64K): Xiaomi - 100:100: 8400 for NR maximum lifting size; 8500: 500: 16500, 17000: 1000:32000 for larger maximum lifting size: Samsung - CB size: [100, 400], 1000, 2000, 4000, 6000, 8448; TB size: derived from TS38.214 based on 273RB, 256QAM, 948/1024 code rate, 4MIMO layers: ZTE - [3276,4096,5461,6144,6826,7585] + 24-bit CRC: Huawei - 100: 100: 32K: Samsung - ≥8448: Tejas, Ericsson |

For other simulation assumptions, companies’ views are summarized as in Table 3.1.2-2.

Table 3.1.2-2 Summary of companies’ views on other parameters

| Parameters | Values or assumptions |
| --- | --- |
| Channel | - AWGN: vivo, Lenovo, Xiaomi, Samsung, ZTE, SJTU(*), ETRI, ESA, Thales, Ericsson |
| Modulation | - BPSK: CATT(*), LGE(*), Apple(*) - QPSK: vivo, CATT(*), Lenovo, Xiaomi, Samsung, ZTE, ETRI, ESA, Thales, Ericsson - 16QAM: Lenovo - 64QAM: Lenovo - 256QAM: Lenovo - 1024QAM: Lenovo - Higher modulation order and higher code rate: Qualcomm |
| HARQ | IR-HARQ: vivo, Lenovo, ETRI, ESA, Thales HARQ-less: ETRI, ESA, Thales |
| Target BLER | - 10-1: Xiaomi, ZTE(TB) - 10-2: vivo, ZTE(CB), ETRI, ESA, Thales, Qualcomm - 10-3: Xiaomi, Samsung - 10-4: Xiaomi, ZTE(CB), Qualcomm - 10-5: ETRI, ESA, Thales |
| Decoding algorithm of LDPC | - Min-sum: vivo(baseline), Lenovo, Xiaomi, ETRI, ESA, Thales, Ericsson - BP: vivo, CATT(*), OPPO(*), Samsung, ZTE, Apple(*), MediaTek(*) - Iteration times: 2:1:20: Samsung 3:1:20: ZTE, ETRI, ESA, Thales 10~15: Ericsson - Decoding order: Reported by companies: vivo Other specific order than natural order: OPPO Reversed order: ZTE |
| Note: (*) resource with evaluation results |  |

For the evaluation of LDPC code design, the suggested evaluation assumptions are as follows.

Table 3.1.2-3 Evaluation assumption for LDPC code

| Parameters | Suggested values or assumptions | FL comments |
| --- | --- | --- |
| Channel | AWGN | Similar to 5G, AWGN channel can be considered |
| Modulation | QPSK (baseline) Other modulation orders such as 16QAM, 64QAM, 256QAM can be also reported. | Similar to 5G, QPSK can be considered as baseline. In addition, higher modulation orders, i.e., 16QAM, 64QAM, 256QAM be evaluated for high throughput. |
| Code rate | Reported by company | Considering the diverse views, this value range is reported by company |
| Transport block size | Reported by company | Considering the diverse views, this value range is reported by company |
| Target BLER | BLER=10-2, 10-4 | Same as the evaluation assumptions in NR |
| Decoding algorithm of LDPC | Layered BP Iteration times: 2~20 Decoding order: reversed order | For min-sum decoding: - Scaling factor has impact on performance, the optimal scaling factor depends on code rate, etc. - Companies may have different implementations of min-sum decoding. For easy calibration among companies, FL suggests to use BP decoding. For decoding iteration times, compaies can report the iteration times between 2 and 20. For decoding order,reversed decoding provides better performance than natural order,the performance is also close to optimized order, which is suggested simplifying calibration. |

For the evaluation evaluations of data channel coding scheme, FL has the following proposal.

Discussion

Round 1

- Proposal 3.1.2-1-v1

Proposal 3.1.2-1-v1: For the study of 6G data channel coding scheme(s), the evaluation assumptions are as below.

| Parameters | Values or assumptions |
| --- | --- |
| Channel | AWGN |
| Modulation | QPSK (baseline) Other modulation orders, such as 16QAM, 64QAM, 256QAM, 1024QAM, can be also reported. |
| Code rate | Reported by company |
| Transport block size (bits w/o CRC) | Reported by company |
| Target BLER | BLER=10-2, 10-4 |
| Decoding algorithm of LDPC | Layered BP Iteration times: 2~20 Decoding order: reversed order |

Please provide your comments as below.

| Company | Comments |
| --- | --- |
| Xiaomi | For the target BLER, before agreeing on the detailed values, I think we need to differentiate the CB/TB level target firstly. For the proposed value from FL, is 10^(-4) targeting CB level while 10^(-2) targeting TB level? And since we agreed both BLER targets, do we need TBS and CBS both reported by companies as well? For the modulation part, we think one baseline higher order modulation, maybe 256QAM as evaluated by some companies, can be taken as well to mimic the real field effect for high throughput scenario. |
| CATT | In addition, Code block size and interleaver design should be included in the evaluation results |
| AccelerComm | Target BLER of 10-5 (or even 10-6) is needed for NTN applications |
| Lenovo | Agree. For the channel, we propose to consider adding fading channel. For clarification of our inputs (currently captured as “5x” in this FL version), we suggested considering higher throughputs in a range from 2.5 - 5x, to be confirmed by RAN plenary confirmation agreement. |
| NTT DOCOMO | We tend to think that it’s practical to evaluate mainly under the assumption set for higher throughput (e.g., high code rate, large TB/CB size) because the motivation of study for data channel extension is for the higher throughput case than NR data rate range here. Regarding decoding algorithm, further discussion may be needed. Regarding the target BLER, we guess that the proposed values ​​follow those of the NR study, but we’d like to confirm the intention of these values ​​(for example, the target BLER for data seems to be usually 10^-1). |
| ZTE, Sanechips | - We are fine with current table for evaluation assumptions for LDPC simulation. And we share some views on the parameters - Code rate: OK .We think the following values should be considered: 948/1024, 8/9, 5/6, 3/4, 2/3, 1/2, 2/5, and 1/3. The code rate of 948/1024 is the maximum code rate in 5G MCS table. For higher throughput, there is no doubt that the highest code rate should be considered. Besides, the code rate range of 1/3 to 8/9 should be also considered for comprehensive analysis, since they were also used for channel coding evaluation in 5G discussions. Considering the current situation, we are okay to up to company report the code rate. - Transport block size: OK. We need to consider large TBS in evaluations, since it is better to have a fair comparison between different BG designs. Besides, moderate transport block sizes (including e.g., 1000, 2000, 4000, 8448) should be also considered in order to evaluate the BG designs under different scenarios. Considering the current situation, we are okay to up to company report the transport block size. Regarding the comments on the target BLER, we think if we consider relatively large TBS (e.g., ~12k and ~16k) for a fair comparison among different BG designs, which would not be large number of CBs, the target BLER performance can either Transport block level BLER or Code block level BLER. |

  - Data channel coding scheme

In RAN1#122bis meeting, the working assumptions/agreements for data channel coding are as below.

Working Assumption for data channel coding - Study 6G data channel coding for higher throughput than 5G with acceptable performance-complexity tradeoff for both NW side and UE side, - Target peak data rate is assumed to be 2 times of the target peak data rate defined in TR38.913 Note: The other target throughput is up to company to report. Note: Applicability of the potential channel code will be further discussed. Agreement for data channel coding - For 6G channel coding, LDPC is used for data (including SIBs) and Polar code is used for L1 control information (larger than 11 bits, including PBCH) - For 6G LDPC - Working assumption: For data rate within NR range, reuse of NR LDPC design is supported - For data rate beyond NR range, study LDPC extension with acceptable performance-complexity tradeoff for both NW side and UE side - Note: Applicability of the potential LDPC extension to data rate within NR range will be further discussed

Summary of observations/proposals

| Source | Observation/Proposal |
| --- | --- |
| Nokia | Proposal 1: For the study of LDPC and Polar code extensions beyond NR range, - prioritize approaches that are compatible with NR code design, - prioritize approaches that demonstrably enhance key performance indicators such as throughput, reliability or power consumption with acceptable performance-complexity tradeoff, - deprioritize proposals aiming at enhancements beyond potential 6G requirements. Proposal 3: For LDPC extensions beyond 5G NR data rate range, in case an additional BG is studied, it should keep the same design principles as for 5G BGs, i.e. using QC-LDPC with dual-diagonal structure and sub-matrices construction as per 5G design principle. |
| Spreadtrum | Proposal 1: The LDPC extension should has minimal impact on either software or hardware, e.g. implementation based solutions should be given high priority. |
| vivo | Observation 3: Extending the lifting size to create larger code blocks or stacking more decoders to parallelly handle more code blocks increase the overall overheads linearly. Observation 4: Extending the lifting size may slightly decrease area efficiency. However, larger code blocks enabled by extended lifting sizes may bring some coding gains. Observation 5: A smaller number of iterations in the decoder can improve the throughput and/or reduce energy consumption at the cost of reduced BLER/BER performance. Proposal 2: Further study the LDPC coding design with performance-complexity tradeoffs. Observation 6: As Option 0, implementation-based LDPC extension supports higher data rates using more parallel encoders and decoders, which achieves same area-/energy-efficiency but no coding gain, compared with NR baseline. For this option, no specification efforts are needed. Observation 7: From the obtained SNR vs CBS curves, it is feasible to directly extend the lifting size beyond 384 and reuse the same CPM shift values of BG1. Observation 8: As Option 1, extending BG1 lifting size with legacy CPM shift values supports higher data rates using larger CBS and larger encoders/decoders, which achieves some coding gains but degrades area-/energy-efficiency, compared with NR baseline. For this option, small specification efforts are needed. Observation 9: As Option 2, extending BG1 lifting size with new CPM shift values have similar property as Option 1 regarding performance, throughput, and area-/energy-efficiency, while medium specification efforts are foreseen to specify the new CPM shift values. Observation 10: As Option 3, introducing a new BG can potentially reduce the number of iterations at the decoder and consequently improve the data rates and energy efficiency. However, great specifications are expected. Proposal 3: For the LDPC extension, further study the following options for the performance-complexity trade-offs in supporting higher data rates, - Option 0: Implementation-based extension with more parallel encoders and decoders - Option 1: Extend BG1 lifting size with legacy CPM shift values - Option 2: Extend BG1 lifting size with new CPM shift values - Option 3: New BG and new CPM shift values |
| CMCC | Observation 1: Expanding the maximum lifting sizes may lead to proportionally increased circuit gates in the LDPC decoder. Considering the limited chip area, it may not be cost-efficient to improve the peak throughput by significantly expanding the maximum lifting size. Proposal 5: For the study of LDPC extension to meet a higher throughput requirement for 6GR, the following can be considered but not limited to - Increased lifting size - New BG design including, decoding parallelism optimization, number of iterations reduction, and etc. |
| CATT | Proposal 2： NR LDPC codes should be reused in 6G data channel. - Alt1: Increase the number of systematic columns. - Alt2: Increase the number of decoder. - Alt3: Increase the maximum lifting size. - Alt4: Reduce the number of iterations. - Alt5: Reduce the number of none zero elements in the LDPC base graph. Proposal 3: In order to meet higher peak data rate requirements in 6G data channel, the potential enhancement of the LDPC codes in increasing the lifting size larger than 384 could be studied. - Option1: Keeping the base matrices of NR LDPC codes unchanged while increasing its lifting size. - Advantage：This option is compatible with NR LDPC codes, which have the decoder hardware implementation being optimized. - Disadvantage：This option would increase the code length, lead to higher power consumption, and constrain the scalability of the lifting size. - Option2: Decreasing the number of information bits in the base matrices of NR LDPC codes while simultaneously increasing its lifting size. - Advantage：This option achieves throughput enhancement while preserving the CB size. - Disadvantage：This option is not backward-compatible with existing NR LDPC codes and exhibits degraded performance under small block size conditions. Proposal 4: To support an increase in lifting size, there are two options to be considered. - Option1: Keeping the base matrices of NR LDPC codes unchanged while increasing its lifting size. - Option2: Decreasing the number of information bits in the base matrices of NR LDPC codes while simultaneously increasing its lifting size. |
| Lenovo | Observation 3: Increasing the lifting size values (beyond 384) for BG1 and BG2 could enable higher throughputs at the decoder. Observation 4: Large lifting sizes coupled with same number of systematic columns induce large code blocks being encoded at the transmitter and decoded at the receiver which could impact energy efficiency, increase memory area and reduce hardware efficiency. Proposal 3: Study the extension of lifting sizes proposed in Table 2.1.3-1 for both base graph 1 and base graph 2 as a candidate to increase decoding throughput. Proposal 4: RAN1 to study the degree of scalability of the lifting size increase. Proposal 5: Study solutions that balance throughput and energy efficiency/processing complexity. Option 2: Reduce the number of decoding iterations Observation 5: Reducing the number of decoding iterations can enable higher decoding throughput, and lower decoding power consumption. The power-saving benefit also applies in scenarios with legacy data rates. Observation 6: Reducing the number of iterations without simultaneously rethinking the BG design can impact BLER performance and consequently other metrics such as spectral efficiency and latency. Observation 7: Jointly considering larger lifting sizes and smaller number of decoding iterations could enable higher decoding throughput but at the expense of BLER performance and energy efficiency. Option 3: Reduce number of edges in BG1 Observation 8: In a Tanner graph, edges connect variable and check nodes, influencing both decoding complexity and code performance. Reducing the number of edges lowers computational cost and increases throughput but may increase error rates and short cycles if the graph becomes excessively sparse. Observation 9: Current LDPC BG1 and BG2 designs may not be well adapted to address some 6G KPIs and use cases in terms of throughput and energy-efficiency. Proposal 6: Extension of the LDPC base graph should comply with the 5G design principles—specifically, employing QC-LDPC codes with a dual-diagonal structure. Proposal 7: Scenarios involving ultra-high throughput should be paired with a redesigned base graph. Proposal 8: Evaluate new LDPC base graph design and/or corresponding lifting factors for targeted scenarios, ultra-high throughput and high energy-efficient operation. Proposal 9: Analize and study application of new LDPC base graph to different block sizes and coding rates, taking into account existing 5G NR base graphs and their application. |
| Xiaomi | Observation 1. The motivation to optimize decoding latency and error floor performance is questionable given the relevant 6GR requirements are the same as 5G. Observation 2. Implementation means exist to achieve 200Gbps peak data rate, if needed, using the 5G NR LDPC. Proposal 1: For within NR range, confirm the working assumption for 6G LDPC and 6G polar for within NR range as below - For data rate within NR range, reuse of NR LDPC design is supported - For control information within NR range (larger than 11 bits), reuse of NR Polar code design is supported Proposal 2: For beyond NR range, - Thorough and vigorous evaluation is needed to justify peak data rate oriented incremental enhancement with specification change for LDPC Designs motivated by increase of UCI/DCI payload size(s), if any, shall be triggered by relevant discussion instead of channel coding discussion |
| OPPO | Proposal 2: Study methods to support LDPC code block sizes larger than the maximum in 5G (Option 1-1), with the lifting sizes up to 2*Z for BG1 or 4*Z for BG2, where Z is the maximum lifting size in 5G. Observation 1: New protograph of LDPC can achieve better performance-complexity tradeoff with less-iteration decoding (Option 1-2) at high code rate. Observation 2: Our initial investigation does not reveal that edge reducing (Option 1-4) can achieve better performance-complexity tradeoff than 5G legacy code and Option 1-2 code. - Option 1-1 is to enlarge LDPC payload size by increasing the lifting size, so that the code throughput is increased. - Option 1-2 is to reduce the maximum number of decoding iterations by using a new BG, so that the decoding time can be reduced and therefore the code throughput is increased. - Option 1-3 is to enlarge LDPC payload size while maintaining the legacy lifting size. Similar to option 1-1, the peak rate can be increased with small coding gain. - Option 1-4 is to simplify the decoding operation (e.g., reduce MIPS) by reducing the number of edges in BG, so that the code throughput is increased. - Option 1-5 is to increase the number of fully orthogonal rows so that the decoding has higher chance to process multiple rows in parallel and therefore the code throughput is increased. Observation 3: For less iterations, the performances of BG1/2 with top-to-bottom scheduling decoding can hardly represent the baseline 5G code performance. So the comparison between 5G code and 6G code still needs to be done on BG1/2 with other specifically enhanced decoding scheduling. Observation 4: The following are observed for Option 1-3 (increasing the number of systematic bit nodes), - It can achieve 0.2dB gain at 4-iterations with larger protograph compared with 5G BG1 in supporting high code rate. - It may slightly decrease performance (less than 0.1dB) at large number of iterations. - It may raise the error floor compared with larger lifting size solution. - It may increase complexity if protograph is expanded too large. |
| Huawei | - For LDPC codes, quasi-cyclic (QC) structure is beneficial for hardware commonality between 5G and 6G. - For LDPC codes, fast convergence with reduced iteration number is simple and effective for higher throughput. - For LDPC codes, the different types of parallelism, i.e., single-block parallel, multi-block parallel, row parallel, and multi-decoder parallel decoding, can affect not only throughput but also hardware implementation. - Do not consider larger lifting value for LDPC extension considering the additional area overhead without any performance gain under the same area efficiency. Observation 4: For LDPC codes, puncturing can affect (i) code rate, and (ii) the convergence speed and performance. Observation 5: For LDPC codes, the choice of BG size has impact on the supported code rate range and also the performance. - For LDPC codes, the performance-complexity tradeoff can be optimized through improved decoding efficiency. Observation 7: The graph structure of LDPC codes has an impact on decoding convergence rate. Observation 8: For LDPC codes, orthogonality facilitates hardware implementation and reduces the decoding latency, but may lead to performance loss. Observation 9: For LDPC codes, fixed-code-rate BGs cannot support fine-granularity rate matching and IR-HARQ. Observation 10: Multi-edge BGs in LDPC codes are incompatible with practical backward-compatible decoders. While single-edge decoding offers a workaround, it yields performance inferior to the BG1 baseline. |
| Samsung | Observation 2: The cell coverage of a network equipment unit is heavily constrained by its baseband processing capacity, specifically LDPC decoding process being the most dominant contributor to the computational load. Observation 3: Enhancing the throughput of the LDPC decoding process is identified as a critical and highly impactful strategy for boosting per-unit cell capacity, which directly leads to significant reduction of total network expenditure in 6GR networks. Observation 4: The current NR LDPC code design may not be scalable to higher decoding throughput without incurring significant implementation complexity. Proposal 3: Study LDPC code extension with higher decoding throughput to support the higher data rates as well as increasing cell capacity. - Reducing the number of maximum iterations (I): - Design fast-convergence LDPC codes to reduce the required iterations while maintaining target performance. - Improve or develop advanced decoding algorithms and layer scheduling schemes to minimize iterations. - Minimizing decoding cycle (Titer): - Increase the operating frequency to decrease the cycle duration per clock. - Design a new BG with fewer 1s compared to NR BGs, reducing the absolute number of blocks the decoder must process. - Increasing maximum lifting size (Z): - Scale the lifting size to process more information bits in the same time period. - Maintain or slightly modify the existing NR LDPC code structure, if necessary. - Increasing the number of information columns (Kb): - Introduce a new BG with more information columns to process a larger number of bits in the same time period. - Increasing the number of decoder blocks (NDEC): - Utilize more decoder blocks in parallel. Observation 5: If the block-wise decoder of LDPC codes fully utilizes its parallelism characteristics, it takes the same processing time to decode a single block regardless of the lifting size. In other words, the decoding cycle per iteration is proportional to the number of 1s (density) in the BG. Observation 6: The proposed HT-BG design achieves nearly half the density of NR BG1 for most code rates, leading to reduced decoding latency and enabling higher throughput. Observation 7: A proposed HT-BG achieves up to a 140% increase in decoding throughput (≈2.4x vs. baseline) Proposal 5: RAN1 should study a new base graph for high-throughput LDPC code design that incorporates a larger lifting size and a reduced number of nonzero elements compared to existing NR base graphs. Observation 8: Enlarging the maximum lifting size reduces the decoding processing time required to achieve the same performance as NR, with performance gains becoming more pronounced as the lifting size increases. Proposal 6: RAN1 should investigate the feasibility of implementing larger lifting sizes in LDPC code designs to enhance decoding efficiency and reduce latency while maintaining or improving performance. |
| ZTE | Observation 1: Increasing the operating frequency, number of decoders, or number of blocks processed simultaneously in block parallel decoding will increase the complexity, cost and power consumption, which is contradictory to 6GR target of energy efficiency. Observation 2: Increasing the number of blocks processed simultaneously may lead to address conflicts, additional waiting time between adjacent rows and an increase in decoding delay. Observation 3: Increasing k_b can improve the decoding throughput and better performance for high code rate. Observation 4: Although increasing the maximum lifting size z can improve throughput, the decoding efficiency is degraded. Observation 5: For 5G LDPC BG1, when the maximum number of decoding iterations is equal to 10, the throughput is only 13.3Gbps; when the maximum number of decoding iterations is equal to 3, the throughput can be around 45Gbps. Observation 6: Reducing the number of iterations can yield two benefits: 1) increase the decoding throughput; 2) reduce decoding complexity and power consumption. Observation 7: Reduction in the number of '1' in the base graph can improve decoding throughput to some extent, but the decoding performance may be impacted. Observation 8: For 6GR, the reduction of the maximum number of iterations of LDPC decoder is the most effective, energy efficient, and economical method to improve the decoding throughput of LDPC codes. Observation 9: Based on 5G LDPC BG1, when the code rate is no larger than 1/3, the performance loss of 3 iterations is 1.8dB compared to 10 iterations; when the code rate is 0.926, the performance loss of 3 iterations is 1.7dB compared to 10 iterations. Observation 10: Based on 5G LDPC BG1, reducing the number of iterations results in a significant performance loss. Observation 11: Compared to 5G LDPC BG1, when the number of iterations is equal to 2 and code rate is larger than 2/3, the performance gain of a newly designed LDPC can reach more than 1dB; when the number of iterations is equal to 2 and code rate is less than 2/3, the maximum performance gain can reach 0.7dB. Observation 12: The decoding convergence speed of the newly designed LDPC code is faster than that of 5G LDPC codes. Observation 13: At the same information size, the decoding convergence speed of the newly designed LDPC code is faster than that of 5G LDPC codes. Proposal 4: LDPC coding with reduced decoding iterations (fast convergence LDPC) should be studied for 6GR. Proposal 5: Fast convergence LDPC design combined with other solutions can be used to achieve the throughput target in IMT-2030 as well as the trade-off between performance and complexity. |
| Tejas | Proposal 1: Possible enhancements to current LDPC codes for transport block sizes larger than 5G NR are - Use 5G NR LDPC base graphs (BG1 and BG2) and increase lifting size beyond 384 for BG1 - Study reducing complexity and iterations for faster decoding convergence - Reuse 5G NR base graphs and lifting sizes - Increase parallelism (more number of codeblocks to be processed simultaneously) at both encoder and decoder - Design a new LDPC base graph for 6G Radio supporting larger codeblock sizes (> 8448) - Increase the number of systematic columns - Fast decoding convergence Proposal 8: Study device specific data and control channel coding algorithms |
| SJTU, NERCDTV | Observation 2: The simulation results confirm that the proposed fast-convergence BG provides a superior performance-complexity trade-off. It achieves a significant ~1.6 dB gain at a very low iteration count of 2, which is critical for enabling massive throughput gains. Crucially, its performance rapidly converges with the robust 5G BG1 at higher iterations, demonstrating that this enhancement in speed comes without sacrificing peak performance and reliability. Observation 3: The proposed larger BG demonstrates remarkable low-iteration performance. The 0.7 dB gain at just 4 iterations is particularly notable, as rapid convergence is critical for increasing decoder throughput. This allows for significantly reduced processing time per codeword without sacrificing error-correction capability. Observation 4: The performance advantages of the larger BG design are scalable and robust. The gain at 4 iterations increases to 0.8 dB for this larger block size, confirming the design's effectiveness for the high-rate, large-payload transmissions central to 6G. This strong low-iteration performance directly translates to higher decoder throughput, making it a compelling solution. Proposal 2: Based on the demonstrated benefits for both fast convergence and scalability, it is proposed to study and specify a new LDPC BG with larger dimensions as the core enhancement for 6GR data channels. This study should focus on leveraging the expanded design space of a larger BG to create a code structure that is: - Inherently optimized for fast convergence, delivering significant performance gains with a reduced number of decoding iterations to meet throughput targets beyond 40 Gbps. - Scalable to larger transport block sizes with superior performance and decoding efficiency compared to the legacy NR BG structure, as validated by simulation results. Observation 5: While the proposed new base graph shows significant performance gains over 5G BG1 at low and medium code rates, it exhibits a performance loss at higher code rates. This indicates that a one-size-fits-all approach to BG design is sub-optimal for 6G. Proposal 3: The study of 6G LDPC codes should evaluate the use of multiple, rate-specific base graphs to ensure optimal performance across different operational scenarios. A dedicated BG should be considered for high code rate transmissions, while another BG could be optimized for low and medium code rate transmissions. |
| LGE | Observation 1: 2Z punctured information nodes are main cause of lower decoding speed/higher number of iteration for decoding convergence. Proposal 1: Modifying or changing the base graph and the puncturing patterns of 5G NR LDPC codes can improve the error rate performance at lower number of decoding iterations. Proposal 2: Study methods to improve decoding performance at lower iteration regime by keeping the 2Z or Z puncturing scheme in the base graph. |
| Fujitsu | Observation 1: - The error floors of 5G LDPC codes could basically satisfy the reliability requirement of 6GR which are all below 10−5 under different MCS indices. - The throughput of 5G LDPC codes may not meet the 6GR requirement of peak data rate. - The max decoding throughput that 5G LDPC codes can reach is about 20Gbps. - The code rates supported by 5G BG1/BG2, which are between 1/5 and 11/12, may be not enough for 6GR to support HARQ-disable/free transmissions. Observation 4: - The throughput and the error floor of 5G BG2 can be improved by BG optimization/redesign. - The throughput can be improved with row (quasi-)orthogonal structures in BG. - The error floor can be at least reduced to 10−5. Proposal 1: - For the higher throughput of 6G data channel coding, RAN1 to study the following directions: - Fast decoding convergence. - Lifting size enhancement. Observation 2: - The decoding parallelization/throughput of 5G LDPC codes is proportional to the lifting size. - The larger the lifting size is, the higher the throughput is. - The larger the lifting size is, the higher degree of decoding parallelization has. Proposal 2: - To satisfy the requirement of peak data rates in 6GR, 5G BG can be reused and the double/quadruple maximum lifting size can be considered. - FFS: whether to redesign cyclic shift values. Observation 3: - The decoding convergence can be accelerated by designing QC-LDPC code without punctured columns in BG. Proposal 3: - For the study of fast-decoding convergence, RAN1 to consider QC-LDPC design without punctured high-weight columns in BG. |
| C-DOT | - Proposal 1: For 6GR high throughput communication scenario - Data Channels: Consider using existing NR LDPC BG1 and BG2. For higher lifting sizes define BG3 with girth ≥ 6. - Control Channels Uplink): Consider alternate short block length codes for UCI payload of length 6-11 bits and continue reusing existing 5G NR codes for control channel, to maintain H/W compatibility. For UCI > 1706 bits consider using LDPC BG2, instead of modifications to uplink control channel process involving polar codes. |
| ETRI, ESA, Thales | Proposal 1. For channel coding evaluation, RAN1 to consider NTN use cases requiring operations at low error rates. The following target CBLER levels are proposed depending on use cases: - Target CBLER of 10-2: eMBB - Target CBLER of 10-5: NTN, HRLLC Proposal 3. RAN1 to study and investigate the feasibility of new and larger lifting sizes of LDPC codes, to provide performance gain. Proposal 4. RAN1 to study fast convergence (low/limited iteration) of LDPC decoding for higher throughput, and for energy efficiency in NTN applications. The following approaches can be considered: - Reducing the number of edges in LDPC BG - Increasing the number of systematic columns Observation 2: The error floor behaviors observed in BG2 can be mitigated by appropriately adjusting the shift values that define the LDPC code structure. Proposal 5. RAN1 to study enhancements to LDPC BG2 to mitigate/eliminate error floor behaviors observed in 5G NR. The following approaches can be considered: - Modification of BG2, including updates and adjustments of the shift values. - Introduce a new BG ensuring the absence of error floors Observation 3: By modifying BG2 of NR (e.g., reselection of shortening and puncturing), BLER performance of lower code rate can be improved at low error rates. Proposal 6. RAN1 to study low code rate enhancements for better reliable applications (e.g., NTN, HRLLC). The following approaches can be considered: - Modification of BG2, including updates of shortening and puncturing, and defining additional shift values - Introduce a new BG supporting low code rates |
| Apple | Proposal 12: It is proposed to consider new LDPC base graph design to support high throughput use cases by optimizing the performance in the small number of iterations, without compromising performance for large number of decoding iterations compared to 5G NR. Proposal 13: It is proposed to support high-throughput use cases by enabling a range of high code rates with the incremental redundancy code, rather than limiting the design to a single code rate. Proposal 14: It is proposed to consider supporting high throughput use cases by increasing the maximum lifting size up to 1024. |
| Spark NZ | Observation 1: Modifications on channel coding for data channels and control information may result in substantial SE improvements that may needed especially to meet the cell edge KPIs. Proposal 1: Agree on the following for data and control channel coding: Improvements to channel coding should be encouraged to meet the cell edge KPIs provided the complexity /power consumption does not make the improvements either unrealisable in practice or prove to be costly. |
| MediaTek | Observation 2: dB level BLER performance degradation is observed when reducing decoding iteration I from 20 to 5 for 4X peak data rate gain. Observation 3: Increasing maximum lifting size Z and/or reducing decoding cycles per decoding iteration C from current 5G design provides potential Tput enhancements. Observation 4: Reducing decoding cycles per decoding iteration C from current 5G design provides potential decoding latency benefit. Observation 5: Dual diagonal structure and Raptor code structures introduced in 5G QC-LDPC code can be leveraged in 6G QC-LDPC code design to achieve encoding latency reduction. Proposal: Study 6G QC-LDPC design to achieve 2x-4x peak data rate of 5G with acceptable performance-complexity tradeoff for both NW side and UE side provided that additional performance and complexity trade-offs required from 2 to 4 times peak data rate increase is minimized Proposal: To improve 2-4x peak data rate from 5G, study following design aspects of QC-LDPC with acceptable error correction performance and implementation complexity trade-off - Reduced number of iterations - Reduced BG edges - Parallelism structure - Maximum lifting size>384 - BG size adaptation for different code rates - Combination of above aspects |
| Ericsson | Proposal 1 Confirm the working assumptions from RAN1#122bis, i.e. • For 6G LDPC, for data rate within NR range, reuse of NR LDPC design is supported. • For 6G Polar code, for control information within NR range (larger than 11 bits), reuse of NR Polar code design is supported. Proposal 3 Maximum lifting size in study of LDPC code extensions for higher peak data rate should not be excessively large compared to 384, i.e. it should be less than or equal to [768]. Proposal 4 In study of LDPC code extensions for higher peak data rate, one potential base graph candidate to consider is a sub-graph of base graph 1, e.g. by using the sub-matrix corresponding to first 10 rows x first 32 columns of base graph 1. - Option 0 (Baseline): Reuse NR Base graph 1 and NR lifting sizes (e.g. use of multiple parallel NR decoders to achieve higher peak data rate target) - Option 1: BG3 which is a sub-graph of BG1 + new lifting sizes (between 384 and 768), e.g. - BG3 is a sub-graph of BG1, i.e. sub-matrix corresponding to first 10 rows x first 32 columns of base graph 1 - New lifting sizes: {480, 576, 672, 768} - Shift coefficients are defined for the maximum lift size 768, and modulo operation is used to obtain shift coefficients for the other lifting sizes (480, 576, 672, 768) Proposal 5 In study of LDPC code extensions for higher peak data rate, following can be considered as an option for base graph 3 (BG3) •BG3 (is a sub-graph of BG1) + new lifting sizes (between 384 and 768). An example setting is given below. i. BG3 is a sub-graph of BG1, i.e. sub-matrix corresponding to first 10 rows x first 32 columns of base graph 1 ii. New lifting sizes: 480, 576, 672, 768 iii. Shift coefficients are defined for the maximum lift size 768, and modulo operation is used to obtain shift coefficients for the other lifting sizes (480, 576, 672) |
| Qualcomm | Proposal 1: Study LDPC code enhancements with faster convergence for improved performance-complexity tradeoff to meet 6G requirements, including facilitating peak throughput scaling in 6GR. Observation 3: Fast converging LDPC may provide 15%~25% energy saving relative to 5G LDPC codes while achieving the same decoding performance for data rate within NR range. Observation 4: New LDPC code may provide substantial benefits in implementation and energy efficiency for billions of devices in 6G, at a very small cost on the network side to support an encoder for the new LDPC code. Observation 5: The (number of) punctured nodes affect the convergence speed of the decoder of QC-LDPC codes, and the total degree of the punctured nodes affects the asymptotic performance of the decoder of QC-LDPC codes. Observation 6: Using a single punctured node with double edges provides good performance in both the small decoding iteration and large decoding iteration regime. Proposal 3: For 6G LDPC code design, study base graph design with double edges connecting a pair of variable and check node. Proposal 4: Study LDPC code enhancements optimized for higher order modulation (including the SBPM bit mapping) in 6GR. Observation 7: NR LDPC code is not fully systematic. Proposal 5: Study (fully) systematic LDPC codes in the 6GR. Observation 9: For IoT devices, reducing the maximum lifting size Zmax could deliver significant area reduction for the LDPC encoder and decoder, without requiring hardware change at the gNB. Proposal 9: In 6GR, support reduced maximum CB size for IoT devices by reducing the maximum lifting size Zmax, for improved area efficiency and reduced cost at the UE. |
| NTT DOCOMO | Observation 2 - There are two main approaches to improving throughput - Referring to the throughput formula proposed by FL in the previous meeting as an example, Throughput∝Kb×ZI×Titer, the approaches involve either increasing the numerator or decreasing the denominator - Increasing the numerator: Larger code block length of LDPC code than 5G - Decreasing the denominator: Designing suitable BG structure for high throughput Proposal 12 - RAN1 to study the necessity of larger code block length of LDPC code than 5G Proposal 13 - RAN1 to study the necessity of designing suitable BG structure for high throughput Proposal 14 - RAN1 to study LDPC code extensions targeting the data rate range for 6G study and evaluate the necessity of them - Larger code block length than 5G (Increasing the numerator of throughput formula) - Option 1: Increase the lifting size - Designing suitable BG structure for high throughput (Decreasing the denominator of throughput formula), including BG selection rule - Option 2: Reduce the maximum number of iterations - Option 3: Increase the number of systematic columns - Option 4: Reduce the number of edges in LDPC BG - Option 5: Optimize parallelism - (Note) If the necessity of the above options is not approved, RAN1 will apply the implementation-based solution Observation 3 - By adopting larger code block length of LDPC code than 5G, - Throughput per decoder and overall decoder throughput can be improved - A little additional coding gain is expected - When study of larger code block length of LDPC code than 5G is conducted, following points can be considered - The magnitude of throughput improvement and associated coding gain, complexity, scalability of parallel processing, etc. Observation 4 - Regarding larger code block lengths of LDPC codes than in 5G, the following analysis can be made: - The throughput improvement may depend on the implementation perspective (e.g., the degree of decoding/decoder parallelism), and the achievable coding gain is not necessarily very large - The feasibility of adopting this option depends on the complexity relevant to implementation aspects. If this complexity when applying this option is significantly large (e.g., difficult to reuse 5G equipment for processing), the motivation to adopt this option would not be strong Observation 5 - By adopting a suitable BG structure for high throughput, - Throughput can be improved by shortening decoding time per code block, but throughput gains may depend on decoder implementation - User experience improvement, complexity reduction may be observed, but there may be compatibility issues with 5G encoding/decoding architectures - When study of designing suitable BG structure for high throughput is conducted, following points can be considered - Throughput, complexity, BLER performance changes, compatibility with 5G LDPC, and BG selection rules for different use cases, etc. Observation 6 - Regarding Options 0 to 5, which are candidate extension methods for LDPC, an analysis from the perspectives of throughput/latency, BLER performance, computational complexity, and architectural complexity can be made as shown in Table 1: |
| Vodafone, AT&T, BT, Bouygues Telecom, Deutsche Telekom, Orange, Telecom Italia, Nokia, SK Telecom, Ericsson, T-Mobile, Rakuten Mobile | Observation 1: Peak data rates, even those achievable in the field, are becoming increasingly irrelevant for MNOs. Observation 2: Maximum theoretical peak rate assuming 400MHz contiguous bandwidth and a realistic number of spatial layers is still significantly below the IMT2020 requirements. Observation 3: Modifications on channel coding for data channels and control information may have direct CAPEX/OPEX impact with respect to the 5G NR incumbent networks. Proposal 1: Decision on peak target data rates for 6GR should be done by RAN Plenary only. Proposal 2: In studying channel coding for 6GR data channels beyond NR, consider the following aspects: - Low Latency - High Reliability - Low-Complexity Devices - Extreme Coverage. Proposal 5: In studying channel coding for 6GR data channels beyond NR, use at least the following, consider the following directions: - Enhanced LDPC Designs - Utilize protograph-based and spatially coupled LDPC codes to improve error floor, support high code rates, and reduce decoding complexity. - Advanced Polar Code Structures - Apply CRC-aided SCL decoding and dynamic frozen bit selection to enhance performance at short block lengths and high code rates. - Hybrid Coding Schemes - Combine LDPC, Polar and potentially other codes in concatenated or layered designs to leverage complementary strengths for scenario-specific optimization. - Lightweight Codes for Massive IoT - Develop simplified LDPC or sparse graph codes with ultra-low complexity and minimal power consumption for constrained devices. - Codes for ISAC - Design structured codes that embed sensing information or enable joint decoding and sensing operations. - Flexible Rate-Matching Techniques - Introduce adaptive puncturing and shortening methods to support fine-grained code rate control across varying service requirements. - Low-Latency Decoding Architectures - Explore parallelizable decoding algorithms and hardware-friendly designs to meet stringent latency constraints in real-time applications. |
| AccelerComm | Observation 1: The chip area associated with the RAM of a layered belief LDPC decoder is proportional to its maximum supported lifting size Zmax. Observation 2: The chip area associated with the computational logic of a layered belief LDPC decoder is proportional to its parallelism P. Observation 3: Increasing the parallelism P of an LDPC decoder implementation only achieves a proportionally increased peak throughput if the maximum lifting size Zmax is also increased proportionately. Observation 4: Increasing the parallelism P and maximum lifting size Zmax of an LDPC decoder by the same proportion will also increase its chip area by that proportion. Observation 5: The same increase in peak throughput and chip area can be achieved by simply using a proportionately increased number of instances C of the baseline LDPC decoder implementation. Observation 6: Increasing the maximum lifting size Zmax enables support for longer information block lengths K, where improved BLER performance is achieved. Observation 7: At information block lengths within the range supported by the baseline maximum lifting size of Zmax=384, multiple instances of a baseline LDPC decoder implementation achieves a significantly higher throughput than a single instance of an LDPC decoder implementation having a proportionately increased Zmax and parallelism P. Proposal 1. Do not increase the maximum lifting size of the 5G NR LDPC code above Zmax=384, unless this is deemed to be necessary for achieving BLER performance improvement. |

Summary of inputs

In RAN1#123 meeting, companies discussed the solutions for 6G data channel coding schemes. Companies’ views on LDPC code for higher throughput and other purposes are summarized as below.

Higher throughput

LDPC code structure

- Dual diagonal structure: Nokia, Lenovo, MediaTek

- QC structure: CATT, Lenovo, HUAWEI, MediaTek

- Raptor-like structure for flexible rates: Lenovo, MediaTek

- Different BG sizes for different code rate regions: SJTU, MediaTek

To support LDPC extension for higher throughput and performance-complexity tradeoff, companies’ views and evaluation results are summarized as below

- Option 1: Reduce the maximum number of iterations, e.g., fast convergence LDPC: (12 sources) vivo, CMCC, OPPO, ZTE, Tejas, SJTU, LGE, Fujitsu, Apple, MediaTek, Qualcomm, NTT DOCOMO

  - Design details

    - New BG/PCM design: vivo, CMCC, OPPO, ZTE, SJTU, LGE, Apple, Qualcomm

    - New puncturing pattern for systematic column: LGE, Fujitsu, Qualcomm

  - BLER performance:

    - vivo observed that for NR BG1, reducing iteration times from 50 to 5 is at the cost of 2.4dB CBLER loss for K=8448 bits and code rate 1/3 under AWGN channel and NMS decoding. Meanwhile, new BG design can achieve at most 1dB gain at 1% BLER with a small number of iterations for K=16896 bits and code rates of 1/2, 3/4, 5/6, 8/9 under AWGN channel, QPSK and NMS reversed decoding.

    - OPPO observed for K=8448 bits and coding rates of 1/3, 2/3, 5/6, a new code B of dimension 31×46 has a performance gain over 5G LDPC on high code rate (~0.2db at iteration 4), which gets fading out as code rate decreases, under AWGN channel, QPSK and LBP with {2 3 4 5 6 7 8 9 10 15 20 25} iterations and specific scheduling order.

    - ZTE observed for 5G BG1, when the code rate is no larger than 1/3, the performance loss of 3 iterations is 1.8dB compared to 10 iterations; when the code rate is 0.926, the performance loss of 3 iterations is 1.7dB compared to 10 iterations. Meanwhile, compared to 5G LDPC BG1, when the number of iterations is equal to 2 and code rate is larger than 2/3, the performance gain of a new LDPC can reach more than 1dB; when the number of iterations is equal to 2 and code rate is less than 2/3, the maximum performance gain can reach 0.7dB.

    - SJTU observed for NR BG1, reducing the maximum iteration times from 10 to 3 can cause a performance loss of over 2.9dB at 1% BLER for Z=256 and R=2/3 and 1/3. Meanwhile, new BG design of 46*68 achieves a significant ~1.6 dB gain at a very low iteration count of 2 for K=2816 bits and R=1/3, and the new BG design of 92*136 achieves 0.7dB(0.8dB) at 4 iterations for K=5632(11264) and R=1/3.

    - LGE observed that for K=8424 and N=10752, the performance of new design at the lower iteration region improves significantly, while it comes with a tradeoff of small performance loss at larger iterations under BPSK, FBP and 1%BLER.

    - Fujitsu observed that for 5G BG1, the decoding is not converged when the maximum number of iterations is 15, and the performance difference between 8 decoding iterations and 30 decoding iterations is larger than 0.5dB. While for new design, the decoding is almost converged when the maximum number of iterations is 8, and the performance difference between 8 decoding iterations and 30 decoding iterations is less than 0.2dB

    - Apple observed that for K=8448 bits and coding rate being 0.71, new design shows better performance than 5G BG under the same iteration times under AWGN channel, BPSK and LBP decoding.

    - MediaTek observed >0.5dB loss for BG1 at high code rates and >1dB loss at low code rates when reducing decoding iteration from 20 to 5 for NR BG1 with K=8448 and coding rates of 11/12, 8/9, 5/6, 3/4, 2/3. MediaTek also observed that new BG provides better BLER performance than BG1 across all iterations at code rates except a minor degradation (~0.1dB) at code rare 2/3 and 5/6 at high iterations.

    - Qualcomm observed new design could potentially improve performance in the small iteration regime without degrading the performance at large number of iterations under AWGN channel, QPSK and K=8448, CR=0.88, and the performance gain is 0.3~1dB under 256QAM.

    - Lenovo observed with NR BG1, less iteration times (from 15 to 8 and 4) has worse performance for rate=0.5.

    - Huawei observed for NR BG1 with R=0.926, the maximum iteration times cannot decrease too much, otherwise it will result in more than 3dB loss. Huawei also observes the proposed new design show better BLER performance than NG BG1 for QPSK and R=0.917, 0.8 under the same decoding complexity, at equivalent iterations 3, 5, and 7.

  - Throughput

    - vivo, Lenovo, Huawei, ZTE, SJTU, and MediaTek observed that reducing the maximum number of iteration times can improve the data rate.

    - ZTE observed with SNR loss no more than 0.21dB, new BG design can achieve 2.25x ~ 3.6xdecoding throughput of 5G BG1.

    - MediaTek observed that 4x peak data rate gain can be obtained when reducing decoding iteration from 20 to 5 for NR BG1.

    - Huawei observed that fast convergence with reduced iteration number is simple and effective for higher throughput. Huawei also observed that new design shows better performance vs area efficiency than NR BG1 for MCS15, 17, 19, 23, 25, 27.

  - Complexity

    - ZTE observed with SNR loss no more than 0.21dB, the decoding computational complexity of new LDPC code is only 0.28 ~ 0.45 times that of 5G LDPC code.

    - Huawei observed that new design shows better complexity-performance trade-off than NR BG1 for MCS15, 17, 19, 23, 25, 27.

  - Energy efficiency

    - vivo observed that reducing the maximum iteration times from 50 to 5 for BG1 can reduce 96% energy consumption.

    - ZTE observed that the reduction of the maximum number of iterations of LDPC decoder is the most effective, energy efficient, and economical method to improve the decoding throughput of LDPC codes.

    - Lenovo observed that reducing the number of decoding iterations can lower decoding power consumption, and the power-saving benefit also applies in scenarios with legacy data rates. While reducing the number of iterations without new BG design can impact BLER performance and consequently other metrics such as spectral efficiency and latency.

    - Qualcomm observed reducing iteration times provides 25% energy saving relative to NR LDPC code at the SNR that achieves 10% CBLER, and generally 15%-25% energy saving at higher SNR values for K=8448 and R=0.88 under TDL-C channel and 256 QAM.

- Option 2: Increase lifting size: (13 sources)vivo, CMCC, CATT, Lenovo, OPPO, Samsung, Tejas, Fujitsu, Apple, MediaTek, Ericsson, NTT DOCOMO, C-DOT

  - No: Huawei, AccelerComm

  - Design details

    - Same number of systematic columns as NR: vivo (2*Z or 4*Z), CMCC, CATT, OPPO (2*Z or 4*Z), Tejas, Fujitsu (2*Z or 4*Z), NTT DOCOMO, Lenovo (416, 448, 480), Samsung (2*Z, 4*Z)

    - Decrease the number of systematic columns: CATT (BG size 32*50), Samsung(kb=11), Ericsson (first 10*32 sub-matrix of BG1, with lifting size of 480, 576, 672, 768)

  - BLER performance

    - vivo observed that larger lifting size provides better TBLER performance.

    - CATT observed for K=200, 1024, 4576 and R=1/3, 2/3, new BG show worse performance than 5G under SPA decoding and QPSK, but gap narrowed as K increases.

    - Lenovo observed that for BG1 and R=0.5, increasing Zc enables larger code block sizes and results in higher coding gains.

    - OPPO observed that for Kmax corresponding to Zmax and coding rate of 1/3, BG1 with lager lifting size has better performance, while BG2 with larger Zmax show error floor loss but it can be optimized by updating cyclic coefficient under AWGN channel and LBP with 25 iterations.

    - Samsung observed for K=8448, QPSK and R=1/2,2/3,3/4,5/6,8/9,11/12, new design show 0.2~1.95dB gain at 0.1% BLER under the equal throughput compared with NR BG1. Besides, for kb=22 and a wide range of code rates, NR BG1 with larger lifting size show better performance gain.

    - Fujisu observed that for 5G BG2, doubled and quadrupled maximum lifting size show good BLER performance for redesigned CPVs at mediate/low code rates, but serious error floors are observed for high code rates. While for 5G BG1, doubled and quadrupled maximum lifting size show good BLER performance without any error floor.

    - Apple proposed to enlarge Z up to 1024 and observed that for coding rate of 0.71, increasing lifting size from 384 to 1024 show better BLER performance under AWGN channel, BPSK and 32 LBP iterations.

    - Ericsson observed new design has better/similar SNR performance at 1%, 0.1%CB-BLER for R=8/9, 5/6, 3/4 and LMS decoding (scaling factor=1).

    - NTT DOCOMO observed BLER performance of A=16896 and Z=768 is better than that of A=8448 and Z=384 for R=8/9 under AWGN channel, QPSK and flooding MS decoding with 32 iterations.

    - AccelerComm observed that increasing the maximum lifting size Zmax enables support for longer information block lengths K, where improved BLER performance is achieved.

  - Throughput

    - vivo, Lenovo, Huawei, Samsung, Fujitsu, Apple, MediaTek, AccelerComm observed that larger lifting size provides higher throughput.

    - Apple observed that for coding rate of 0.71, by extending the maximum lifting size to 1024, system throughput can potentially be increased by a similar factor, i.e., approximately 1024/384 ≈ 2.67 under AWGN channel, BPSK and 32 LBP iterations.

    - MediaTek observed that double the max lifting size Zmax from 5G design can directly double the peak data rate.

    - Huawei observed that a doubled lifting size can increase throughput by up to 2 times in the case of single-block parallelism.

    - Samsung observed for K=8448, QPSK and R=1/2,2/3,3/4,5/6,8/9,11/12, new design provides 21%~140% throughput gain under the same BLER performance.

  - Complexity

    - CMCC observed it may not be cost-efficient considering the limited chip area.

    - Lenovo observed that large lifting sizes coupled with same number of systematic columns could impact energy efficiency, increase memory area and reduce hardware efficiency.

    - ZTE observed the hardware complexity of increasing the maximum lifting size is higher than multiple decoders, indicating that though the throughput is improved, the decoding efficiency is degraded.

    - MediaTek observed the side-effect of large Z is the implementation cost.

    - NTT DOCOMO observed that the feasibility of adopting larging lifting size depends on the complexity relevant to implementation aspects. If the complexity is significantly large, the motivation to adopt this option would not be strong.

    - Huawei observed larger lifting value leads to additional area overhead without any performance gain under the same area efficiency since it may increase the non-linear complexity of the QSN and decreases chip area efficiency.

    - AccelerComm observes that the chip area associated with the RAM of a layered belief LDPC decoder is proportional to its maximum supported lifting size Zmax, and increasing the parallelism P and maximum lifting size Zmax of an LDPC decoder by the same proportion will also increase its chip area by that proportion.

  - Area efficiency

    - vivo observed that larger lifting size increases the overall overheads linearly and decreases the area/energy efficiency compared with NR baseline

    - Huawei observed larger lifting value leads to additional area overhead without any performance gain under the same area efficiency since it may increase the non-linear complexity of the QSN and decreases chip area efficiency.

  - Energy efficiency

    - vivo observed that larger lifting size increases the overall overheads linearly and decreases the area/energy efficiency compared with NR baseline

- Option 3: Parallelism optimization, e.g., improving the orthogonality between rows of BG:(2 sources) CMCC, MediaTek

  - BLER performance

    - MediaTek observed that new BG provides better BLER performance than BG1 across all iterations at code rates except a minor degradation (~0.1dB) at code rare 2/3 and 5/6 at high iterations.

    - Huawei observed that orthogonality facilitates hardware implementation and reduces the decoding latency, but may lead to performance loss. Besides, multi-block-parallel (e.g., two blocks with unchanged lifting size Z) decoding achieves 20% higher area efficiency (throughput/area) compared with single-block-parallel (e.g., one block with doubled lifting size 2Z) decoding assuming the same area.

    - Huawei proposed a new BG design considering parallelism via multi-block processing, and observes the better BLER performance than NG BG1 for QPSK and R=0.917, 0.8 under the same decoding complexity, at equivalent iterations 3, 5, and 7.

  - Throughput

    - MediaTek observed parallelism structure within the BG can enable the decoder to exploit the parallelism and boost the throughput

  - Complexity/area efficiency

    - Huawei observed that new BG design considering parallelism via multi-block processing shows better complexity-performance trade-off and performance vs area efficiency than NR BG1 for MCS15, 17, 19, 23, 25, 27.

- Option 4: Increase the number of systematic columns: (5 sources) ZTE, Tejas, SJTU, MediaTek, NTT DOCOMO

  - Design details

    - New BG: ZTE (new BG with dimension of 96*144, kb=48) SJTU (new BG with dimension of 92*136, kb=44), MediaTek (kb=44 for larger code rate, kb=22 for smaller code rate)

  - BLER performance

    - OPPO observed that increasing the number of systematic columns can achieve 0.2dB gain at 4 iterations with larger photograph compared with 5G BG1 in high code rate, while it may slightly decrease performance (less than 0.1dB) at large number of iterations, raise the error floor compared with larger lifting size solution and increase complexity if photograph is expanded too large.

    - ZTE observed that compared to 5G LDPC BG1, when the number of iterations is equal to 2 and code rate is larger than 2/3, the performance gain of new design can reach more than 1dB; when the number of iterations is equal to 2 and code rate is less than 2/3, the maximum performance gain can reach 0.7dB.

    - Huawei observed for K=8448, R=0.926, larger BG size can provide more than 0.5dB performance gain compared with NR BG1 with 5 iterations.

    - SJTU observed new BG design achieves 0.7dB(0.8dB) at 4 iterations for K=5632(11264) and R=1/3.

    - MediaTek observed that new BG provides better BLER performance than BG1 across all iterations at code rates except a minor degradation (~0.1dB) at code rare 2/3 and 5/6 at high iterations.

  - Throughput

    - ZTE observes with SNR loss no more than 0.21dB, new BG design can approach 2.25 ~ 3.6 times decoding throughput than that of 5G BG1.

  - Complexity

    - ZTE observed with SNR loss no more than 0.21dB, the decoding computational complexity of the new LDPC code is only 0.28 ~ 0.45 times that of the 5G LDPC code.

    - Huawei observed that the decoding of larger BG becomes much more complex than for the original BG codes, especially at low code rates.

- Option 5: Reduce the number of edges in the LDPC base graph: Samsung, MediaTek, Ericsson, NTT DOCOMO

  - Design details

    - MediaTek (adaptive BG sizes according to code rates), Samsung (new BG design of kb=11 and Zmax=768), Ericsson (first 10*32 sub-matrix of BG1, and with new lifting sizes {480, 576, 672, 768}).

  - BLER performance

    - OPPO observed that BG2 with larger lifting size (sparser than BG1) has no obvious gain over 5G BG1 codes in the performance (1% BLER)-complexity metric.

    - Samsung observed for K=8448, QPSK and R=1/2,2/3,3/4,5/6,8/9,11/12, new design show 0.2~1.95dB gain at 0.1% BLER under the same throughput compared with NR BG1.

    - MediaTek observed that new design provides better BLER performance than BG1 across all iterations at code rates except a minor degradation (~0.1dB) at code rare 2/3 and 5/6 at high iterations, for K=8448 and coding rates being 11/12, 8/9, 5/6, 3/4, 2/3.

    - Ericsson observed the proposed design has better/similar SNR performance at 1%, 0.1%CB-BLER for R=8/9, 5/6, 3/4 and LMS decoding (scaling factor=1).

  - Throughput

    - Samsung observed for K=8448, QPSK and R=1/2,2/3,3/4,5/6,8/9,11/12, new design 21%~140% throughput gain under the same BLER performance compared with NR BG1.

  - Complexity

    - MediaTek observed new design can provide implementation benefits and provides benefit to daily use scenario in terms of decoding latency.

- Option 6: implementation-based solution, e.g., more decoders or blocks to be processed simultaneously: (5 sources) Spreadtrum, vivo, Xiaomi, Tejas, NTT DOCOMO

  - Throughput

    - CATT observed that the 5G LDPC codes achieve the peak data rate of about 80 Gbps under the configuration of 5 decoding cores and 8 iterations.

  - Complexity

    - ZTE observed increasing the number of decoders or of blocks processed simultaneously increases the complexity, cost and power consumption, which is contradictory to 6GR target of energy efficiency.

Other motivations than higher throughput

In addition to higher throughput, companies also discussed about other motivations and the views are summarized as below.

- Motivation to optimize decoding latency, error floor performance is unclear: Xiaomi

- To optimize the error floor performance for the scenarios, such as URLLC, NTN service: Fujitsu, ETRI, ESA, Thales, Rakuten Mobile

  - Fujitsu observed that introducing row (quasi)-orthogonality in new BG2, the error floor of new BG2 is at least reduced to 10−5, and there is slight performance loss of 0~0.15dB at the waterfall region. Meanwhile, the throughput can be improved with row (quasi-)orthogonal structures in BG.

  - ETRI, ESA, Thales observed that for BG2 and low code rate of 1/9, doubling the lifting size can improve the BLER performance under QPSK and SPA with 50 iterations. Besides, ETRI, ESA, Thales also observed that updating the BG2 shift values can improve the error floor for 0.263 code rate, 1120 code length under QPSK and SPA with 50 iterations. ETRI, ESA, Thales also observed that better BLER performance at low error rate (especially 0.5 dB gain at error rate of 10-6) can be achieved with appropriate selection of shortening and puncturing positions.

- To improve device energy efficiency for the scenarios, such as IoT/Redcap UEs

  - New BG: Apple, Rakuten Mobile

    - Apple observed energy savings can be achieved for IoT devices by compressing the base graph and compressing the base graph 5x times shows SNR loss at 1% BLER for K=160, CR=0.2 under AWGN channel, BPSK and 7 LBP iterations, but further optimizations can close this gap.

  - Reduce the maximum lifting size: Qualcomm

    - Qualcomm observed for IoT devices whose target throughput may be 20~200Mbps, reducing the maximum lifting size Zmax can reduce maximum CB size and then deliver significant area reduction for the LDPC encoder and decoder, without requiring hardware change at the gNB.

- To optimize data channel coding for the scenarios, such as ISAC service: Rakuten Mobile

Based on companies’ discussions, for higher throughput, FL proposes to study the solutions with acceptable performance-complexity tradeoff. Furthermore, FL suggest considering QC-LDPC code per companies’ input.

Furthermore, FL suggests to study whether/how to enhance LDPC code for other motivations.

Discussion

Round 1

Proposal 3.2-1-v1

Proposal 3.2-1-v1: For the study of LDPC code for higher throughput with acceptable performance-complexity tradeoff, the following options can be considered

  - Option 1: Reduce the maximum number of iterations, e.g., fast convergence LDPC code

  - Option 2: Increase the maximum lifting size to [2 or 4]*384

  - Option 3: Optimize parallelism, e.g., improve orthogonality between rows of LDPC BG

  - Option 4: Increase the number of systematic columns

  - Option 5: Reduce the number of edges in LDPC BG

  - Option 6: Implementation based solutions

  - Other options are not precluded.

  - The above options can be combined.

  - The LDPC code is quasi-cyclic LDPC (QC-LDPC)

  - FFS: whether to use 5G LDPC BG(s) or define new LDPC BG(s)

Please provide your comments as below.

| Company | Comments |
| --- | --- |
| CMCC | Prefer Option 1/3/5. Before the BG design, the down select between new BG and enhancement based on 5G NR BG should be considered first. |
| Xiaomi | Yes |
| CATT | We are OK with the list of options. |
| AccelerComm | Option 2 is not well justified for achieving higher throughput - it is more hardware efficient to use multiple instances of a 5G LDPC encoder/decoder. |
| Lenovo | It is infeasible to study all options given the timeline and expected progress on coding A.I. We therefore propose to down-select and keep alternatives 1, 2 and 5 and further decide whether these options require the design of a new BG or can be enhancements to existing BGs. |
| NTT DOCOMO | We prefer to add the modification to the main bullet in order to reflect the statement of current agreement. For the study of LDPC code for higher throughput than NR range with acceptable performance-complexity tradeoff, the following options can be considered. This modification can be applied for the same statements in the following proposals. In addition, further clarification for this “NR range” is needed. We prefer to reuse NR BG1 for higher throughput than NR range as much as possible, but new LDPC BG can be also studied if it allows easily the leverage of existing NR equipment. |
| ZTE, Sanechips | Okay with the proposal. |

Proposal for conclusion3.2-2-v1

Proposal for conclusion 3.2-2-v1: For the study of LDPC code for higher throughput with acceptable performance-complexity tradeoff, companies are encouraged to

- Provide the LDPC BG(s) and PCM(s) in the excel spreadsheet

- Provide the required SNR for target BLER using the following template in the excel spreadsheet

|  | Qm1, R1 | Qm1, R2 | Qm2, R3 | Qm2, R4 |
| --- | --- | --- | --- | --- |
| Transport block size 1 | SNR_1_1 | SNR_2_1 | SNR_3_1 | SNR_4_1 |
| Transport block size 2 | SNR_1_2 | SNR_2_2 | SNR_3_2 | SNR_4_2 |
| Transport block size 3 | SNR_1_3 | SNR_2_3 | SNR_3_3 | SNR_4_3 |
| Transport block size 4 | SNR_1_4 | SNR_2_4 | SNR_3_4 | SNR_4_4 |

Please provide your comments as below.

| Company | Comments |
| --- | --- |
| Xiaomi | This is clear for a given TB BLER target corresponding to a certain TBS, maybe the sheet updates to consider the CB part as well if we are to assess the CB level target and performance. |
| AccelerComm | A common set of block sizes, modulation orders, coding rates and target BLERs should be agreed |
| Lenovo | Okay. Good to further clarify whether a common set of modulation order, rate and target BLER should be applicable to ensure some calibration across companies. |
| NTT DOCOMO | Comparing the evaluation result and cross checking can be important, so specifying the report format is OK but detailed discussion is needed. According to the current agreements, the main purpose of LDPC extension is for higher throughput than NR data rate range with acceptable performance-complexity tradeoff. Considering this situation, we tend to think that the evaluation under too low code rate and/or too small transport block size is not practical. These ranges should be carefully discussed in discussion of evaluation assumptions. |
| ZTE, Sanechips | The required SNR performance under different numbers of iterations needs to be considered, where the fields in the first row can be {Qm1, R1, Iteration1}, {Qm1, R1, Iteration2}. Qm1, R1, Iteration1; Qm1, R1, Iteration2. Transport block size 1; SNR_1_1; SNR_2_1. Transport block size 2; SNR_1_2; SNR_2_2. Transport block size 3; SNR_1_3; SNR_2_3. Transport block size 4; SNR_1_4; SNR_2_4. In addition, we think similar template can be added to collect the evaluation of other metrics than BLER. |

Proposal 3.2-3-v1

Proposal 3.2-3-v1: Study whether/how to optimize the error floor performance for LDPC code.

Please provide your comments as below.

| Company | Comments |
| --- | --- |
| CMCC | The motivation of error floor performance optimization should be clarified first, given that the reliability targets of 5G and 6G are the same. |
| Xiaomi | We don’t think this is quite aligned with the motivation agreed from last meeting and prefers not having this proposal for now. |
| AccelerComm | Agree with this proposal. Error floors in BG2 at high coding rate are problematic for NTN applications. |
| Lenovo | The motivation to further study LDPC error floor performance improvements is currently unclear. It is generally fine to explore first the need for such enhancements. However, the need should be motivated by a set of clear service-level requirements, e.g., BLER, beyond those supported by 5G, otherwise the need for this stream of work becomes questionable. |
| NTT DOCOMO | We tend to think there is no motivation for LDPC extension for the other aspects than achieving higher data rate than NR data rate range with acceptable performance-complexity tradeoff. |

Proposal 3.2-4-v1

Proposal 3.2-4-v1: Study whether/how to enhance LDPC for 6G low complexity/power scenarios.

Please provide your comments as below.

| Company | Comments |
| --- | --- |
| CMCC | Appropriate data channel coding scheme for 6G low complexity/power scenarios can be studied, but the design target scenarios and principle need to be clarified first. For example, the target scenario is IoT? The design principle is reusing eMBB data channel coding scheme, or designing new LDPC coding scheme, e.g. new LDPC BG? |
| Xiaomi | We don’t think this is quite aligned with the motivation agreed from last meeting and prefers not having this proposal for now. |
| AccelerComm | Support for a reduced Zmax would be beneficial for low complexity/power scenarios |
| Lenovo | Support the proposal. If we are targeting IoT devices, we suggest adding low code rates (below 0.2) and low complexity/power scenarios. |
| NTT DOCOMO | We tend to think there is no motivation for LDPC extension for the other aspects than achieving higher data rate than NR data rate range with acceptable performance-complexity tradeoff. |

  - Data channel coding channel chain

Summary of observations/proposals

| Source | Observation/Proposal |
| --- | --- |
| Lekha | Observation 1: Machine learning models can adaptively optimize decoding strategies for LDPC codes based on observed channel conditions, error patterns, and historical data. Proposal 1: Recent advances and promising directions in existing coding schemes, are expected to meet the various requirements of 6G networks. The enhancements for LDPC codes including machine learning-based LDPC decoders and autoencoders using end-to-end learning approaches present promising advancements. |
| vivo | Observation 11: When incorporating high order modulations to support higher data rates, there exists significant reliability (channel capacity) imbalance among the bits in a QAM symbol. Therefore, it is necessary to consider coding chain enhancements to better achieve higher target data rates. Proposal 4: To better achieve higher target data rates, the imbalance between different bits in QAM symbols should be considered in the mapping between coded bits and modulated symbols, for example, in the form of a better bit interleaver design. Observation 12: Compared with NR design, the cross-codeblock interleaver design, e.g., MGCM can better leverage the unequal sub-channel capacity of QAM modulated symbols, by jointly modulating multiple different (e.g., coding rates/schemes) coded blocks. Proposal 5: For data channels, consider the cross-codeblock interleaver design, e.g., MGCM, to better leverage the unbalanced capacity of different bit subchannels in QAM modulated symbols. Proposal 6: Further study the MGCM design as a solution for joint coding and modulation, considering at least the use case where two SCH data blocks are coded by LDPC using different coding rates. |
| Lenovo | Proposal 10: Evaluate optimized interleaver designs and refined bit-to-symbol mapping to support enhancements of LDPC data channel coding for higher order modulation schemes (e.g 1024-QAM, 4096-QAM). |
| Huawei | Packet-level coding - The performance of this technique is highly dependent on a careful design of the bit-puncturing module and requires iterative strategies like successive interference cancellation. Furthermore, its effectiveness is limited in cases of severe channel degradation or inaccurate channel estimation, where an entire transport block may be corrupted. In such scenarios, packet coding is unable to correct the widespread errors. QC-block interleaving - QC-block interleaving can offer performance gains in scenarios with low code rates and high modulation orders - Observations show that an interleaver provides gain under BP decoding may introduce losses under MS decoding. - This variability may complicate nested descriptions and hardware implementation, requiring careful evaluation. - If the technique is used for HARQ, the combined effects of initial transmission interleaving and retransmission interleaving must be carefully assessed, as they cannot be simply compared to a single large interleaver. |
| ZTE | Observation 14: For TB error rates (0.1, 0.2, or 0.3), the probability of only one or two code block errors is high, i.e., it rarely happens in the case when there are more than 2 code block errors. Observation 15: Using packet coding has a performance gain of about 0.5dB. Proposal 6: Packet coding should be studied for 6GR channel coding enhancement. Observation 1: In 5G, bit interleaving is only performed within a code block, resulting in the inability to obtain frequency diversity gains and degrading the performance of the individual CBs and increasing the error rate of entire transport block, and further decreasing the throughput. Observation 2: Compared with 5G NR legacy without frequency domain interleaving, the proposed intra-CBG interleaving method has shown 0.5-7dB performance gain for closed-loop MIMO scheme with up to 4 layers, which is aligned with the 6G KPI of higher throughput/spectral efficiency. Observation 3: Compared with 5G NR legacy without frequency domain interleaving, the proposed intra-CBG interleaving method has shown up to about 12.6% and 24.5% throughput gain for closed-loop MIMO scheme with up to 4 layers at bandwidth of 50MHz and 100MHz, respectively. Proposal 7: Intra-CBG interleaving should be studied in 6GR for better performance and higher throughput. Observation 19: Compared to NR LDPC coded modulation scheme, multi-group coded modulation (MGCM) scheme has 0.5~1.45dB performance gain while maintaining the transmission efficiency for MCS16/17/26/27 over TDL-A channel. Proposal 8: Coded modulation scheme coordinating multiple code blocks and modulation sub-channel reliability can be studied for 6GR with higher throughput/spectral efficiency. Observation 20: Compared to 5G NR bit interleaving, the new interleaving has a performance gain up to 0.2dB at the information length ranging from 1000 bits to 8500 bits under 16QAM and 1/3 code rate. Proposal 9: New bit interleaving scheme considering distribution characteristics of LDPC base graph can be studied for 6GR. |
| Tejas | Proposal 2: For larger transport block sizes, to achieve better data channel coding gains, we propose the following enhancements to the data coding chain - Inter codeblock coding - Interleaver enhancements - Joint source and channel coding |
| InterDigital | Observation 1: Inter-CB coding for erasure correction in combination with LDPC for bit error correction can be beneficial to improve reliability and reduce latency for diverse use cases and deployments scenarios with potential for bursty errors or data puncturing. Observation 2: For both downlink and uplink communication, joint inter-CB and channel coding outperforms LDPC-only with 10 - 1000 times reliability gain in the presence of interfering signals. Observation 3: When DMRS is affected by interference, for both downlink and uplink communication, joint inter-CB and channel coding can lower the error floor. Proposal 2: Study inter-CB coding for data channels as a candidate technique for channel coding enhancement. Proposal 3: In addition to AWGN, deep fading and bursty interference should be considered for the performance evaluation of the joint inter-CB and channel coding scheme. It is shown in our simulation results that deep fading and bursty interference are such scenarios where the joint inter-CB and channel coding scheme is most beneficial. Proposal 4: The performance of joint inter-CB and channel coding should be evaluated based on key metrics such as throughput, spectrum efficiency, reliability, and/or latency, where for fair comparison, the overall effective code rate value should be the same for legacy LDPC channel coding and joint inter-CB and channel coding schemes. |
| LGE | Observation 2: Inter-CB outer coding with outer LDPC codes shows large performance gain compared to 5G NR for high speed scenarios. Proposal 4: Study LDPC codes as outer codes in 6GR. Observation 3: GC-LDPC codes can significantly improve the error rate performance. Proposal 5: Study GC-LDPC codes as inter-CB coding scheme. |
| Apple | Proposal 15: It is proposed to consider packet level coding of code blocks to improve both retransmission rate and spectral efficiency. |
| Qualcomm | Proposal 4: Study LDPC code enhancements optimized for higher order modulation (including the SBPM bit mapping) in 6GR. Proposal 10: Study LDPC code design for iterative receivers in 6GR if compelling use cases can be identified to justify the complexity & performance tradeoffs. |
| CEWiT | Proposal 2: Consider NR CW-to-Layer mapping schemes as baseline in 6G. Proposal 3: The need to support higher number of codewords in 6G and appropriate extension to CW-to-Layer mapping schemes for additional codewords should be studied. Proposal 4: Enhance the existing CW-to-Layer mapping schemes to accommodate multiple device types |
| Rakuten Mobile | Proposal 5: In studying channel coding for 6GR data channels beyond NR, use at least the following, consider the following directions: - Enhanced LDPC Designs - Utilize protograph-based and spatially coupled LDPC codes to improve error floor, support high code rates, and reduce decoding complexity. - Advanced Polar Code Structures - Apply CRC-aided SCL decoding and dynamic frozen bit selection to enhance performance at short block lengths and high code rates. - Hybrid Coding Schemes - Combine LDPC, Polar and potentially other codes in concatenated or layered designs to leverage complementary strengths for scenario-specific optimization. - Lightweight Codes for Massive IoT - Develop simplified LDPC or sparse graph codes with ultra-low complexity and minimal power consumption for constrained devices. - Codes for ISAC - Design structured codes that embed sensing information or enable joint decoding and sensing operations. - Flexible Rate-Matching Techniques - Introduce adaptive puncturing and shortening methods to support fine-grained code rate control across varying service requirements. - Low-Latency Decoding Architectures - Explore parallelizable decoding algorithms and hardware-friendly designs to meet stringent latency constraints in real-time applications. |

Summary of inputs

In RAN1#123, 8 sources (vivo, Lenovo, ZTE, Tejas, InterDigital, LGE, Apple, Qualcomm) discussed the solutions for 6G data channel coding chain enhancements. Companies’ views are summarized as below.

5 sources (ZTE, Tejas, InterDigital, LGE, Apple) discussed inter-CB coding and observed that in the case of large transport block transmission, e.g., for high throughput, increased bandwidth, etc., the number of code block within one transport block increases. In this case, inter-CB coding is proposed by companies to improve the performance. Companies’ views and evaluation results are summarized as below.

- Inter-CB coding: (5 sources) ZTE, Tejas, InterDigital, LGE, Apple

- ZTE observed that compared with the legacy coding chain without packet coding, the packet coding has a PER performance gain of about 0.5dB at 10% BLER for TBS=84000 and R=0.89 under AWGN and QPSK.

- InterDigital observed that for both downlink and uplink communication, joint inter-CB (2/3 rate) and channel coding outperforms LDPC-only with 10 - 1000 times reliability gain (the error floor can be order of magnitude lower at the same SNR) in the presence of interference for MCS 10/17 under TDL channel for TBS of 44040 and 43032 for DL and UL respectively.

- LGE observed that

  - Inter-CB outer coding with outer LDPC codes shows up to more than 3dB performance gain compared to 5G NR for high speed scenarios, wherein 100km/h and 200 km/h are assumed along with 2*2 MIMO, CDL-A channel with DS=100ns, QPSK, 16QAM, 64QAM and 256QAM, 273 PRBs, and 0.3333~0.8678 code rate.

  - GC-LDPC code shows about 0.174dB gain at 10% TBER compared with 5G NR LDPC BG1 under AWGN channel and the number of GC-CBG being 3, the number of CBs in GC-CBG being 10, CB size of 8448, and code rate of 0.5.

- Apple observed that for the two-packet coding scheme with effective rate 22/31 derived from 5G NR rate 22/30, it shows ~0.5dB SNR gain compared to 5G NR scheme at rate 22/30, and ~0.15dB gain compared to the equivalent 5G NR scheme at rate 22/31, under the AWGN channel, BPSK, K=8448, Z=384 and LBP decoding with 32 iterations..

In 5G, a bit interleaver is applied after data channel coding. In RAN1#123, 5 sources (vivo, Lenovo, ZTE, Tejas, Qualcomm) discussed to enhance the bit interleaver to further improve the performance by exploiting the bit reliability within modulation symbol or frequency diversity gain. Companies’ views are summarized as below.

- Interleaver in LDPC coding chain: (5 sources) vivo, Lenovo, ZTE, Tejas, Qualcomm

- vivo observed that compared with NR design, the cross-codeblock interleaver design (e.g., MGCM) can achieve a throughput gain of 7% under CDL-A channel, 260RBs with 14 OFDM symbols and [3,11] DMRS pattern, and MS decoding.

- ZTE observed that

  - Compared with legacy 5G without interleaving, intra-CBG interleaving method has a performance gain of 0.5-7dB under the same throughput, and has up to about 12.6% and 24.5% throughput gain under the same SNR for closed-loop MIMO scheme with up to 4 layers at bandwidth of 50MHz and 100MHz, respectively, wherein the CDL-A channel with 30ns, RI adaptation and adaptive MCS, 64QAM MCS table, 4Tx antennas are assumed.

  - Compared with NR LDPC coded modulation scheme, multi-group coded modulation (MGCM) scheme has 0.5~1.45dB performance gain while maintaining the transmission efficiency for MCS16/17/26/27 over CDL-A channel.

  - Compared with 5G NR bit interleaving, new sub-block interleaving has up to 0.2dB performance gain at 1% BLER for the information length ranging from 1000 bits to 8500 bits under 16QAM, 1/3 code rate and AWGN channel.

- Qualcomm observed that under AWGN channel, 5.33bps spectrum efficiency and 256QAM, new LDPC code optimized for higher order modulation (including the SBPM bit mapping) with PS QAM provides >0.5dB performance gain at 1%BLER in the small iteration regime compared to the NR LDPC code with PS QAM, and the new design can achieve the same performance as NR LDPC with uniform QAM while the decoding complexity is >5x lower (assuming NR LDPC runs 25 iterations).

In addition, companies’ views about data channel coding chain in other aspects are summarized as below

- AI/ML based LDPC decoder and autoencoder: Lekha

- Channel coding design for ISAC: Rakuten Mobile

- Flexible rate matching scheme: Rakuten Mobile

- Hybrid Coding Schemes (Combine LDPC, Polar and potentially other codes in concatenated or layered designs): Rakuten Mobile

- Low-Latency Decoding Architectures: Rakuten Mobile

- Joint channel and source coding: Qualcomm (Non-AI/ML based), Tejas

- Study LDPC code design for iterative receivers: Qualcomm

- Study (fully) systematic LDPC codes to facilitate PAS: Qualcomm

- Enhance CW-to-Layer mapping: CEWiT

Discussion

Round 1

Based on companies’ discussion, FL observes the evaluation results provided by proponent companies for data channel coding chain enhancements (including inter code block coding, bit interleaver enhancement, etc) show BLER and/or throughput performance gain. Therefore, the FL proposal for data channel coding chain enhancements is as below.

Proposal 3.3-1-v1

Proposal 3.3-1-v1: For the study of 6G data channel coding chain enhancements, proponent is encouraged to provide at least the following

- Details and expected spec impact of data channel coding chain enhancements, including inter-code block coding, enhancement of bit interleaver

  - Other solutions are not precluded

- Evaluation metrics, including BLER/throughput performance

  - Other metrics can be also reported

- Evaluation assumptions, including at least

  - Channel type

  - Resource allocation

  - Modulation and coding scheme

  - Channel estimation method

  - Tx/Rx antenna configuration

Please provide your comments as below.

| Company | Comments |
| --- | --- |
| CMCC | The applicable scenario of these solutions is not clear, e.g., when the number of code blocks within a TB significantly increases, the inter-code block coding can be considered. |
| CATT | The enhancement of channel coding chain needs to be evaluated in the fading channel. |
| IDC | Support. It is recommended to report BLER (at TB level) and throughput at the same overall effective code rate as the LDPC-only baseline when assessing any chain enhancement (e.g., inter-CB coding, interleaver). Include deep fading and bursty-interference cases in addition to AWGN, as these are where inter-CB + LDPC shows the largest benefit, representing the motivation and necessity. |
| Lenovo | Agree with this proposal, yet the evaluation assumptions could be aligned and consolidated into the Proposal 3.1.2-1-v1 |
| NTT DOCOMO | According to the current agreements, the main purpose of LDPC extension is for higher data rate than NR data rate range with acceptable performance-complexity tradeoff. Considering this situation, if the study for the coding chain is conducted, we tend to think that the motivation for the study of coding chain should be limited to higher data rate than NR data rate range with acceptable performance-complexity tradeoff. At this point, we tend to think there is no need to study extension of the coding chain. |
| ZTE, Sanechips | Support FL’s proposal. We think study of 6G data channel coding chain should be considered. According to our contribution R1-2508821, in 6G, we observe that due to the larger transmission bandwidth and higher spectral efficiency, the transport block size will be very large, resulting in a large number of code blocks. A large number of code blocks is much more likely to cause degradation in BLER performance. We think that inter-CB coding can provide performance gains for the transmission of large transport blocks. Additionally, enhancing the bit interleaver is beneficial for performance under high-order modulation. To this end, at least these enhancements should be considered for the 6G data channel coding chain to improve the BLER/throughput performance. |

- Control channel coding

  - Channel coding on DCI

Summary of observations/proposals

| Source | Observation/Proposal |
| --- | --- |
| Spreadtrum | Proposal 2: For control information within NR range, reuse of NR Polar code design. |
| vivo | Observation 14: Due to lacking information on the real needs input by other agendas, the motivations to consider a larger UCI/DCI payload size cannot be justified by the channel coding agenda alone. Proposal 10: UCI/DCI payload size extension should be discussed and confirmed by other agendas, e.g., control channel design. Observation 15: From the perspective of latency reduction, there is no motivation to enhance early termination, because - PDCCH decoding is not the latency bottleneck. - Early termination does not always happen, and thus PDCCH decoding timeline cannot be shortened for accommodating the worst-case decoding latency. Observation 16: Even if the real TSCCR (considering the probability of receiving a non-intended DCI and the overall TSCCR) is assumed to be as high as 45% all the time, the most optimistic estimate of the overall gain to UE energy saving is capped by around 10%. Observation 17: From the perspective of UE energy saving, there is no motivation to enhance early termination. Observation 18: It is feasible to directly extend DCI payload upper bound to, e.g., 200 bits, by removing the D-CRC. |
| CMCC | Proposal 2: For the DCI with payload sizes beyond NR range, the following enhancements of polar codes can be considered: - Extend the maximum mother code length of polar codes to 1024 for DCI. - Introduce segmentation for DCI. For example, an inner code can be applied across the polar-encoded sub-blocks, generating additional bits Proposal 3: If segmented polar coding is supported, enhancement such as outer/inner code across sub-blocks could be considered to provide additional error protection across the sub-blocks. |
| CATT | Observation 7: In the 5G NR DCRC Polar scheme, the early termination rate during decoding failure events can reach nearly 100%. For frames where early termination occurs, the average decoding complexity is reduced by 20% to 50%. Even when the DCI size exceeds 140 bits, the average decoding complexity can still be reduced by approximately 15%. Observation 8: Under favorable channel conditions, the decoding failure rate for 5G NR Polar codes is low, resulting in an overall low early termination rate. The overall TSCCR achieved by early termination design is often less than 1%. However, this should not be interpreted as an indication that early termination provides negligible gain, especially in scenarios where multiple UEs share common PDCCH transmission resources. Proposal 10: Support for early termination should be maintained, reusing the Distributed CRC (DCRC) mechanism as a baseline, if not fully reused. Proposal 11: Given that the DCRC scheme contributes to a reduction in average decoding complexity, the enhancement of the PDCCH early termination scheme should be considered |
| Lenovo | Proposal 11: Determine the scope of control channel coding enhancements based on one of: Alternative 1: Study and evaluate further Polar coding enhancements for control channel coding beyond 5G NR Polar. Alternative 2: Reuse 5G NR Polar control channel coding and revisit need for any necessary minor enhancements following other A.I. progress (e.g., control channel design, MIMO CSI feedback, etc.) |
| Xiaomi | Proposal 1: For within NR range, confirm the working assumption for 6G LDPC and 6G polar for within NR range as below - For data rate within NR range, reuse of NR LDPC design is supported - For control information within NR range (larger than 11 bits), reuse of NR Polar code design is supported Proposal 2: For beyond NR range, - Thorough and vigorous evaluation is needed to justify peak data rate oriented incremental enhancement with specification change for LDPC - Designs motivated by increase of UCI/DCI payload size(s), if any, shall be triggered by relevant discussion instead of channel coding discussion Proposal 4 Reuse NR LDPC codes and NR Polar codes for 6GR data channel(s) and 6GR control channel(s), respectively, at least under the same NR conditions on code rate and code block length. |
| OPPO | Observation 5: By following the RAN1 analysis assumptions for Rel-17 DSS and Rel-18 MC-enhancement, the total number of DCI information bits has a good chance to exceed 140 or even 200. Proposal 3: Study solutions to increase DCI payload size in 6G, where the principle of the solution should hold against the uncertainty of the exact DCI payload size upper-bound. Proposal 4: Study the following schemes of interleaving to enable the maximum DCI payload size to be larger than the one (140+24) in 5G. - Scheme 1: Follow the legacy design principle to re-define the interleave table (i.e., ΠILmaxm defined in TS38.212) to support a maximum Polar code payload size that is larger than (140+24). - Scheme 2: Only apply the legacy interleaving over the last (140+24) bits. |
| Huawei | Observation 15: For PDCCH, high reliability, reliability - measured by BLER and FAR - is the top priority and, as a pre-requisite requirement for ET improvement, early termination schemes must not compromise performance in either of these metrics. Observation 16: The 5G NR DCRC-polar scheme meets the pre-requisite requirement of no compromise of BLER and FAR performance, and can reduce average decoding complexity by 20%~40%, as verified by extensive simulations. Observation 17: NR DCRC was chosen in 5G after extensive simulations covering hundreds of different cases that are all evaluated down to an FAR of 10⁻⁷ due to fact that it does not sacrifice the reliability performance (measured by BLER and FAR) and meanwhile can early terminate the decoding as much as possible. Observation 18: RNTI-FAR is avoidable by gNB during RNTI assignment. RNTI-FAR reduction via split-reduced SCL decoding Observation 19: Assuming bad pair of RNTIs allocation could happen, the conditional RNTI-FAR can be reduced under a low level by the decoder side via split-reduced SCL decoding. Proposal 4: Confirm the working assumption with revision to reuse NR polar codes for control information within NR range (larger than 11 bits). Observation 20: The maximum DCI size in 6GR might be still within the NR payload size range. Even if there is any increase, the maximum DCI size is moderate, e.g., up to 200 bits. Observation 21: For potential larger DCI payload size, there exist scalable DCRC interleaver generation procedures that can ensure the NR same sequence for the existing DCI payload size region, and also provide future-proof (support arbitrary large Kmax in 6G and beyond). - For larger DCI payloads, increasing the maximum code length to 1024 bits by reusing the existing 1024 sequence for UCI provides 0.3 to 1.1 dB coding gain. Observation 25: The partial polarization in the two-stage DCI decoding incurs significantly higher construction complexity. The segmented CRC has a significantly higher FAR than the 6G requirement. Observation 26: PAC-Polar provides limited performance gain and brings challenges to reuse NR SCL decoder. Observation 27: The reliability of control channels is the top priority, so sacrificing error correction performance through sequence optimization is not a viable option |
| Samsung | Observation 16: The NR D-CRC interleaver pattern is defined up to 164 bits, strictly limiting the maximum DCI size to 140 bits. Observation 17: A simple augmentation of the interleaver pattern is not a forward-compatible solution, as it will face the same hard limit problem. The 6GR solution must not be coupled to a pre-defined payload limit. Proposal 11: RAN1 should study and design a new, scalable polar-family code construction for DCI that is not constrained by a fixed-length D-CRC interleaver, allowing it to support arbitrary DCI sizes. Observation 19: Within the NR DCI range, TPAC achieves more than double the TSCCR of NR D-CRC polar codes with comparable or slightly better error-correction performance. Observation 20: The TPAC is inherently scalable, supports arbitrarily large DCI payloads, and is independent of any interleaver. Its design study can proceed without waiting for a 6GR DCI size to be finalized. Proposal 12: RAN1 should consider PAC codes as an enhanced, scalable polar coding scheme to ensure early termination gains without any constraints on the DCI size. |
| Tejas | Proposal 4: Study enhancements to current DCI channel coding for DCI payloads larger than 140 bits. Proposal 6: Study polar codes for higher order QAM’s for control channel for better SE |
| Fujitsu | Proposal 6: - If a larger payload size of control information is identified for 6G DL, RAN1 to study the scalable interleaving design of distributed CRC for DL polar codes. |
| Apple | Observation 1: Among the three categories of CRC behavior previously studied, the random noise and random QPSK symbols categories are not highly sensitive to the CRC polynomial or to the placement of CRC bits within the DCI payload. This is because, for these two categories, each decoding decision is essentially random, akin to a coin flip. As long as the 24 CRC bits are statistically (linearly) independent, the probability of passing all 24 checks remains 2−24. Considering the number of list candidates in the SCL decoder, having 24 linearly independent CRC bits allows the UE to easily meet the 1.5×2−21 FAR requirement. Therefore, minor alterations to the CRC parity-check matrix do not affect compliance with these two categories, provided the matrix remains full rank. Observation 2: The Undetected Error Rate (UER) is highly sensitive to both the location of the CRC bits and the CRC polynomial. This is because an undetected error occurs when the UE is initialized with LLR values corresponding to a valid DCI, but decoding errors remove the correct trajectory from the SCL decoder’s candidate list. In such cases, passing the CRC check is no longer a purely random event as in the previous two categories. Many of the surviving decoding trajectories were initially aligned with the correct codeword, and thus, for distributed CRC bits appearing early in the payload, there is a high probability that all surviving trajectories will pass those bits correctly. Consequently, CRC bits that appear later within the payload are more effective in improving the UER performance. Proposal 6: It is proposed to simplify the CRC precoding for PDCCH configurations beyond the 5G NR range in order to enhance CRC-assisted early termination while maintaining compliance with all FAR category requirements. Observation 3: In 6G, scheduling becomes more joint and multi-cell aware, potentially spanning different bands and TRPs under a single control command. Therefore, DCI payloads in 6G should be designed with scalability in mind. Observation 4: In 6G, substantially larger DCI payload sizes are anticipated as a result of multi-carrier, multi-TRP, and beam-aware scheduling, often extending well beyond the 140-bit limit defined in 5G NR. This trend motivates the need to re-examine the requirements and design scalable solutions beyond the 5G NR range to ensure efficient and reliable control channel design. Observation 5: In 6G, it is expected that DCI formats designed for joint or dual-format decoding will converge toward similar payload sizes to facilitate efficient joint blind decoding at the UE. Proposal 7: It is proposed for 6G to study scalable and backward-compatible polar coding solutions such as segmentation to support increased control payload sizes without imposing significant complexity or implementation burden on either the UE or the network operators. Proposal 8: It is proposed for 6G to study efficient segmentation strategies for PUCCH that minimize BLER degradation. Proposal 9: It is proposed for 6G to study efficient segmentation strategies for PDCCH that leverage two-stage DCI decoding to minimize power consumption and decoding latency in the UE. Observation 4: The FAR performance under PPP and two-stage DCI decoding is consistently better than that of plain segmentation (i.e., τ=0). This improvement arises because, while all segments remain padded with the RNTI, decoding often terminates early upon detecting an RNTI mismatch in the primary segment. As a result, the decoder avoids generating false positives that could otherwise occur when processing the remaining segments. Proposal 10: It is proposed for 6G to combine inter-segment coding schemes, such as the Partially Polarized Polar (PPP) approach, with DCI segmentation to recover coding gain and further reduce the decoding complexity of the primary segments in PDCCH. Proposal 11: It is proposed to investigate the use of dynamic frozen bits in the design of polar codes beyond 5G NR range, to further enhance performance of control channel codes. |
| MediaTek | Observation 7: Due to the limitation of maximum 140bit payload sizes and the support of only QPSK QAM, the 5G Polar code design for DCI is limited on scheduling flexibility and resource efficiency to support 6G applications. Observation 8: 6G’s wider bandwidth substantially increases DCI FDRA bit requirements and current 140bits limit is not sufficient. Observation 9: The 5G distributed CRC design for DCI delivers limited early termination benefit at the UE, resulting in increased power consumption and latency. Proposal: Study a new data integrity check mechanism with the aim to improve payload size scalability and early termination rate over 5G distributed CRC design Observation 10: The 5G RNTI scrambling mechanism for DCI is not optimized for RNTI-based false alarm rate among UEs with different RNTIs and endanger DCI reliability especially in denser and more demanding 6G scenarios. Proposal: Study new mechanism of RNTI scrambling to reduce RNTI-FA rate Proposal: Study high order QAM polar code for UCI performance enhancement and DCI SE enhancement. |
| Ericsson | Observation 3 Radical changes to NR polar coding within NR range are not expected to provide significant overall system-level benefits. Observation 4 The distributed CRC interleaver used for the downlink in NR provides rather minimal (if any) early termination benefits at the decoder while increasing implementation complexity. Observation 5 Simplifying the distributed CRC interleaver used for downlink DCI in NR, (e.g. making it identity) can reduce decoder complexity. Observation 6 The necessity of extending channel code design for 6G control channels beyond the NR control channel coding schemes is unclear. This can be revisited based on additional needs (if identified) in other 6GR discussions. Observation 7 The NR polar coding scheme is quite flexible and can be reused or adapted for potential control channel extension needs, e.g. removing distributed CRC interleaver for PDCCH enables extending DCI payload size to larger than 140 bits (if needed). Observation 8 Regarding polar code extension for downlink control channels, removal/simplification of distributed CRC interleaver enables extending DCI payload size to larger than 140 bits. Proposal 8 For polar coding enhancement within NR range, the enhancement (if any) should be limited to simplifying distributed CRC interleaver by making it identity for the downlink. No motivation is identified for modifying polar code for UCI within NR range. |
| Qualcomm | Observation 10: For broadcast PDCCH, reducing the CRC from 24 bits to 16 bits may provide 1 dB coverage improvement with minimum changes at the transmitter. |
| NTT DOCOMO | Proposal 15 - The discussion regarding the necessity of larger DCI/UCI payload sizes than those in 5G should be left to the control discussion on AI 11.9. Whether to specify control channel coding enhancements related to larger DCI/UCI payload sizes should be determined based on the outcome of control discussion on AI 11.9. - For any agreement/conclusion related to larger DCI/UCI payload sizes, keep “For larger DCI/UCI payload sizes than those in 5G NR, which may/may not be supported,” in the main bullet until concluded in AI 11.9. Proposal 16 - Details of UCI/DCI and its container should be discussed separately in the control discussion on AI 11.9. Proposals/agreements/conclusions on AI 11.4.1 do not determine the definition and details of UCI/DCI and its container. The definition and details of UCI/DCI and its container will follow the agreements reached in the control discussion. - For any agreement/conclusion related to DCI/UCI, add/keep “if treated as layer 1 information” in the main bullet until concluded in AI 11.9. Proposal 18 - For control channel coding, 5G Polar code should be reused as much as possible unless there are critical motivations and clear gains enough to justify the enhancement Proposal 19 - For control channel coding, 5G polar transform should be reused if extensions of the polar transform part have a significant complexity increase Proposal 20 - 5G Polar code (including 5G interleaving (CRC distribution), 5G polar sequence plus polar transform plus concatenated coding) should be adopted for DCI with payload size between 12 bits and 140 bits if DCI is treated as layer 1 information Proposal 23 - RAN1 to identify detailed issues raised from DCI beyond the NR payload size and analyse the motivations to address these issues Proposal 24 - For L1 downlink control information beyond the NR payload size range (which may/may not be supported), RAN1 should study possible solutions below: - Option 1: Apply code block segmentation - Option 2: Remove the distributed CRC interleaver - Study whether to increase the maximum code block length - Option 3: Redefine the distributed CRC interleaver pattern - Study whether to leverage NR interleaver pattern - Study whether to increase the maximum code block length - Option 4: Use a new code construction scheme - Other options are not precluded |
| Google | Observation 2: The main reason of having a larger DCI size is based on multi-carrier enhancement, where a single DCI can schedule transmissions on multiple carriers. The only benefit for this feature is to reduce control channel blocking probability, which might not be a critical issue in the day-one 6GR. Proposal 1: RAN1 studies DCI and UCI size enhancement for coding schemes, only if requirements have been identified in 6GR data and control channels agenda items. |
| Vodafone, AT&T, BT, Bouygues Telecom, Deutsche Telekom, Orange, Telecom Italia, Nokia, SK Telecom, Ericsson, T-Mobile, Rakuten Mobile | Proposal 2: Agree on the following for data and control channel coding: - For data rate at least within NR range, reuse NR LDPC design - For control information at least within NR range (larger than 11 bits), reuse NR Polar code design - FFS: Clarification on the definition of "NR range" |

Summary of inputs

In NR, the largest supported DCI payload size is 140 bits. This limitation is because the interleaver size is 164 (i.e., 140 bits DCI payload+24 bits CRC).

In RAN1#122bis meeting, the agreements for control channel are as below

Agreement for control channel coding - For 6G channel coding, LDPC is used for data (including SIBs) and Polar code is used for L1 control information (larger than 11 bits, including PBCH) - For 6G Polar code - Working assumption: For control information within NR range (larger than 11 bits), reuse of NR Polar code design is supported - For control information beyond NR range, study Polar code extension with acceptable performance-complexity tradeoff for both NW side and UE side - Note: Necessity for control information beyond NR range is to be further discussed - Polar code maximum mother code length is kept as 1024. - FFS: further motivation(s) for potential extension/enhancement until RAN1#123

In RAN1#123 meeting, companies’ views on 6GR DCI channel coding are summarized as below:

For control channel coding on DCI within NR range

- 16 sources (Nokia, vivo, AT&T, Xiaomi, Huawei, Ericsson, NTT DOCOMO, Rakuten Mobile, Vodafone, BT, Bouygues Telecom, Deutsche Telekom, Orange, Telecom Italia, SK Telecom, T-Mobile) suggested using NR design within NR range

- Issue D-within-1: early termination

- 4 sources (Huawei, Ericsson, Samsung, MediaTek) discussed early termination issue for PDCCH decoding

  - Option 1 use NR D-CRC Polar code: Nokia, vivo, AT&T, Xiaomi, Huawei, Ericsson, NTT DOCOMO, Rakuten Mobile, Vodafone, BT, Bouygues Telecom, Deutsche Telekom, Orange, Telecom Italia, SK Telecom, T-Mobile

    - Huawei observed 5G NR DCRC-polar scheme has no compromise of BLER and FAR performance and can reduce average decoding complexity by 20%~40%. Furthermore, Huawei observed that frozen bits can be also used for PDCCH early decoding termination without specification change

  - Option 2: simplify D-CRC: Ericsson

    - Ericsson observed that for K = 80 and K=140, the third CRC bit appears after 75%~78% of bits are already decoded in the Polar list decoding, which means that most of the decoding time is already consumed.

  - Option 3: terminated polarization-adjusted convolutional (TPAC) coding scheme: Samsung

    - Samsung observed that TPAC codes achieve a TSCCR of 60-65%, while the TSCCR for NR D-CRC polar codes remains below 30%

    - Huawei observed that PAC-Polar provides limited performance gain and brings challenges to use NR SCL decoder.

  - Option 4: new data integrity check mechanism: MediaTek

    - MediaTek observed that >50% information bits are processed before decoding termination with 5G D-CRC design

- Issue D-within-2: RNTI FAR issue

- 2 sources (Huawei, MediaTek) discussed RNTI FAR issue for DCI

  - Huawei observed RNTI-FAR is avoidable by gNB RNTI assignment or UE-specific scrambling. Furthermore, it can be reduced by split-reduced SCL decoding

  - MediaTek observed that the probability of sending a DCI to one of the UE (target UE) but mistakenly detected by any of 10 other UEs is between 10−3~10−4

- Issue D-within-3: higher modulation order

- 2 sources (Tejas, MediaTek) discussed higher modulation order for Polar code

  - For 16QAM, MediaTek observed 0.2-0.3 dB gain over 5G BICM from MLC framework. By incorporating shaping bits for 16QAM, >0.5dB gain is observed across the examined payload sizes.

- Issue D-within-4: CRC overhead

- 1 company (Qualcomm) discussed to reduce CRC size for DCI

  - Qualcomm observed that for broadcast PDCCH, reducing the CRC from 24 bits to 16 bits may provide 1 dB coverage improvement with minimum changes at the transmitter.

For control channel coding on DCI beyond NR range

- 12 sources (Nokia, vivo, CMCC, Lenovo, Xiaomi, OPPO, Huawei, Samsung, Apple, MediaTek, NTT DOCOMO, Google) discussed the maximum DCI payload size,

- The maximum DCI payload size could be more than 140 bits: CMCC, OPPO, Samsung, Apple, MediaTek

  - Indicate TPMI per subband: CMCC

  - Single DCI scheduling multiple UEs/cells/PDSCH: CMCC, OPPO, Samsung, Apple

  - Wider bandwidth: MediaTek

- Unclear of maximum DCI payload size increase: Ericsson

- The maximum DCI payload size may not be larger than 140 bits: Huawei

- Depend on other agendas: vivo, Lenovo, Xiaomi, NTT DOCOMO, Google

- Issue D-beyond-1: channel coding for DCI payload size >140bits

- 11 sources (vivo, CMCC, CATT, OPPO, Huawei, Samsung, Fujitsu, Apple, MediaTek, Ericsson, NTT DOCOMO) coding schemes for DCI with payload size beyond 140bits

  - Option 1: Code block segmentation (e.g., inter-segment coding scheme): CMCC, Apple(Partially Polarized Polar, PPP, approach), NTT DOCOMO

  - Option 2: Remove interleaver: vivo, Ericsson, NTT DOCOMO

    - vivo observed that by applying the UCI polar encoding scheme to DCI, the performance curve is generally smooth and confirmed the feasibility

  - Option 3: Revisit/redefine interleaver pattern design: OPPO, Huawei, Fujitsu, NTT DOCOMO

    - OPPO suggested applying the legacy D-CRC interleaver over the last (140+24) bits.

    - Huawei observed that the original D-CRC interleaver in 5G is designed for 200-bit DCI.

  - Option 4: PAC code: Samsung, Apple

    - Samsung observed that TPAC codes achieve a TSCCR of 60-65%, while the TSCCR for NR D-CRC polar codes remains below 30%, and stated that TPAC is inherently scalable, supports arbitrarily large DCI payloads, and is independent of any interleaver.

    - Huawei observes that PAC-Polar provides limited performance gain and brings challenges to use NR SCL decoder.

  - Option 5: 2-stage DCI: Apple

    - Huawei observed that the partial polarization in the two-stage DCI decoding incurs significantly higher construction complexity

    - Apple observed that FAR performance under PPP and two-stage DCI decoding is consistently better than that of plain segmentation

  - Option 6: Use 1024-length Polar sequence for DL: CMCC, Huawei

    - Huawei observed 0.3 to 1.1 dB gain from Nmax=1024 over NR design, with payload sizes ranging from 141 to 300 bits.

- Issue D-beyond-2: PDCCH early decoding termination

- 8 sources (vivo, OPPO, Huawei, Samsung, Fujitsu, Apple, MediaTek, Ericsson) discussed PDCCH early decoding termination for DCI beyond NR range

  - Option 1: Apply a longer D-CRC interleaver pattern: OPPO, Huawei, Fujitsu

    - Huawei pointed out that the original D-CRC interleaver in 5G is designed for 200-bit DCI

    - OPPO, Huawei, Fujitsu suggested considering a scalable D-CRC interleaver to support large DCI

  - Option 2: segmented CRC: Apple

    - Apple observed that FAR requirements are satisfied for all categories at the operating SNR values of interest for segmented CRC

    - Huawei observed that the segmented CRC has significantly higher FAR compared to D-CRC scheme

  - Option 3: terminated polarization-adjusted convolutional (TPAC) coding scheme: Samsung

    - Samsung observed that TPAC codes achieve a TSCCR of 60-65%, while the TSCCR for NR D-CRC polar codes remains below 30%

    - Huawei observes that PAC-Polar provides limited performance gain and brings challenges to use NR SCL decoder.

  - Option 4: new data integrity check mechanism: MediaTek

    - MediaTek observed that >50% information bits are processed before decoding termination with 5G D-CRC design

- Issue D-beyond-3: higher modulation order

- 2 sources (Tejas, MediaTek) discussed higher modulation order for Polar code

  - For 16QAM, MediaTek observed 0.2-0.3 dB gain over 5G BICM from MLC framework. By incorporating shaping bits for 16QAM, >0.5dB gain is observed across the examined payload sizes.

Discussion

Round 1

Based on companies’ input, FL observes that the proposed motivations for DCI channel coding enhancement include larger payload size, early termination for PDCCH decoding, RNTI FAR improvement, performance improvement, and higher modulation order. FL has following observations.

Observation 4.1-1-v1

Observation 4.1-1-v1: [20 sources] discussed the channel coding for DCI with payload size within NR range (i.e., no larger than 140 bits).

- [16 sources] suggested using NR Polar code

- [4 sources] discussed early termination issue for PDCCH decoding

  - [1 source] observed 5G NR D-CRC Polar code has no compromise of BLER and FAR performance, and can reduce average decoding complexity by 20%~40%. Furthermore, it is also observed that frozen bits can be also used for PDCCH early decoding termination without specification change

  - [1 source] suggested removing D-CRC interleaver

  - [1 source] suggested studying terminated polarization-adjusted convolutional (PAC) code, which achieve a total saved computational complexity ratio (TSCCR) of 60-65%, while the TSCCR for NR D-CRC polar codes is below 30%. While [1 source] observed that PAC-Polar provides limited performance gain and brings challenges to use NR SCL decoder.

  - [1 source] suggested studying a new data integrity check mechanism for better early termination performance.

- [2 sources] discussed RNTI FAR issue for DCI

  - [1 source] observed that RNTI FAR can be up to 10−3~10−4

  - [1 source] observed RNTI-FAR can be avoided by proper RNTI assignment, UE-specific scrambling, and can be also reduced by split-reduced SCL decoding

- [2 sources] discussed higher modulation order for DCI

  - For 16QAM modulation, [1 source] observed 0.2-0.3 dB gain for MLC framework over 5G BICM, with shaping bits, >0.5dB gain is observed.

- [1 source] discussed CRC overhead issue for DCI

  - [1 source] observed that for broadcast PDCCH, reducing the CRC from 24 bits to 16 bits may provide 1 dB coverage improvement.

Please provide your comments as below.

| Company | Comments |
| --- | --- |
| CMCC | Regarding the channel coding for DCI with payload size within NR range, we suggest using NR polar code. |
| Xiaomi | This observation list seems to well reflect the contribution submitted to this meeting. But we think discussing or agreeing to such observations is not helpful and even misleading in the sense that we are drawing observations based on a stabilized motivation and agreed set of evaluation assumptions, which are clearly not the case. For within NR range, we think the majority still wants re-using NR design, the motivation for change is not clear or agreed, on which we can discuss by this meeting according to the FFS made in previous meeting. |
| CATT | Most of observed issues were discussed in 5G. We don’t see the need of repeating the discussion in 6GR. |
| AccelerComm | We suggest using the NR polar code for DCI with payload size within NR range. |
| IDC | Suport using the NR polar code for DCI with payload size within NR range. |
| Lenovo | We are open to DCI enhancements that might reduce the complexity of PDCCH blind decoding and allow better performance. However, based on current agreements and working assumption, the benefit of treating these enhancements in the channel coding A.I. is unclear given that solutions may be dependent/impact on the DCI design. Hence, we prefer to defer this considering control channel design progress. |
| NTT DOCOMO | Regarding the channel coding for DCI with payload size within NR range, we suggest that NR Polar code (including NR interleaving (CRC distribution), NR polar sequence plus polar transform plus concatenated coding) should be adopted if DCI is treated as layer 1 information. |
| ZTE, Sanechips | For the fourth bullet, it should be clarified whether it is to be discussed in channel coding or modulation? |

Observation 4.1-2-v1

Observation 4.1-2-v1:  [12 sources] discussed the necessity and channel coding for DCI with payload size beyond NR range (i.e., larger than 140 bits).

- For the necessity of DCI with payload size beyond NR range:

  - [5 sources] observed the DCI with payload size can exceed NR range considering single DCI scheduling multiple UEs/cells/PDSCHs, wider bandwidth, indication of TPMI per subband

  - [2 sources] observed the necessity is unclear

  - [5 sources] observed the necessity depends on other agendas

- For the coding scheme for DCI (including early termination) with payload size beyond NR range:

  - [3 sources] suggested code block segmentation

  - [3 sources] suggested removing D-CRC interleaver

  - [4 sources] suggested defining D-CRC interleaver for DCI payload size larger than 140bits

  - [2 sources] suggested polarization-adjusted convolutional (PAC) code, wherein [1 source] observed total saved computational complexity ratio (TSCCR) of 60-65% for terminated PAC code, while the TSCCR for NR D-CRC polar codes is below 30%. While [1 source] observed that PAC code provides limited performance gain and brings challenges to use NR SCL decoder.

  - [1 source] suggested 2-stage DCI

  - [2 sources] suggested 1024-length Polar code sequence for DL

- [2 sources] suggested higher modulation order for DCI.

  - For 16QAM modulation, [1 source] observed 0.2-0.3 dB gain for MLC framework over 5G BICM, with shaping bits, >0.5dB gain is observed.

Please provide your comments as below.

| Company | Comments |
| --- | --- |
| CMCC | For the coding scheme for DCI with payload size beyond NR range, on top of code block segmentation, concatenated coding schemes can be further considered, where an outer/inner code is introduced to provide additional error protection across the sub-blocks after segmentation while preserving the inherent benefits of short polar codes. |
| Xiaomi | This observation list seems to well reflect the contribution submitted to this meeting. But we think discussing or agreeing to such observations is not helpful and even misleading in the sense that we are drawing observations based on a stabilized motivation and agreed set of evaluation assumptions, which are clearly not the case. For beyond NR range, we think the motivation part needs to be verified in control agenda. In previous meeting, there is some confusion on this red sentence “further discussed”, we suggest updating this sentence as follows to make things aligned and facilitate future study. This is our proposal for this topic, Update the RAN1#122bis note as follows, - Note: Necessity for control information beyond NR range is to be further discussed triggered by control agenda. Agreement for control channel coding - For 6G channel coding, LDPC is used for data (including SIBs) and Polar code is used for L1 control information (larger than 11 bits, including PBCH) - For 6G Polar code - Working assumption: For control information within NR range (larger than 11 bits), reuse of NR Polar code design is supported - For control information beyond NR range, study Polar code extension with acceptable performance-complexity tradeoff for both NW side and UE side - Note: Necessity for control information beyond NR range is to be further discussed - Polar code maximum mother code length is kept as 1024. - FFS: further motivation(s) for potential extension/enhancement until RAN1#123 |
| CATT | We would like to clarify the purpose of these observations from companies. We don’t see any clear proposals for discussion. |
| AccelerComm | Table 5.3.1-1 of TS 38.212 v1.0.0 (from September 2017) specifies a D-CRC interleaver design that supports payload sizes of up to 200 bits. It includes the 140-bit interleaver of 5G NR as a special case. This should be reconsidered if it is agreed that payload sizes of up to 200 bits should be supported. |
| Lenovo | As discussed during RAN1#122bis, we suggest to revisit (as needed) the polar code design (beyond NR range) based on the outcome of other agenda items i.e., control channels. |
| NTT DOCOMO | The discussion regarding the necessity of larger DCI/UCI payload sizes than those in 5G should be left to the control discussion on AI 11.9. Whether to specify control channel coding enhancements related to larger DCI/UCI payload sizes should be determined based on the outcome of control discussion on AI 11.9. For any agreement/conclusion related to larger DCI/UCI payload sizes, keep “For larger DCI/UCI payload sizes than those in 5G NR, which may/may not be supported,” in the main bullet until concluded in AI 11.9. From the discussion schedule perspective, we do not object to starting the study of possible solutions related to larger DCI payload in advance. The main discussion point here may be the balance between the effect of early termination and the spec impacts. Regarding this discussion point, we believe that the current proposed candidates should be treated equally and studied carefully. |
| ZTE, Sanechips | For the fourth bullet, it should be clarified whether it is to be discussed in channel coding or modulation? |

  - Channel coding on UCI

Summary of observations/proposals

| Source | Observation/Proposal |
| --- | --- |
| Nokia | Observation 1: NR range of UCI payload is sufficient for 6G needs. Any extension beyond NR range should be well justified, taking into account further inputs from other agenda items. Observation 2: For the study of Polar code extensions for UCI payload beyond NR range, if necessary to support larger payloads, using more segments should be properly evaluated, especially in terms of encoding/decoding complexity and latency. Observation 3: The potential new 6G requirements for control information may (or may not) be the payload beyond 5G NR range. Any other motivation(s) for extension/enhancement other than the potential payload beyond 5G NR range is out of the scope of the study. |
| Spreadtrum | Proposal 2: For control information within NR range, reuse of NR Polar code design. Proposal 3: If the final maximum UCI payload size exceeds 1706 bits, high priority should be given to more than 2 segments. |
| vivo | Proposal 9: Confirm the working assumption regarding the polar coding with further clarification For the control information with the payload size within NR range (larger than 11 bits), reuse of NR Polar code design without introducing any enhancement. Observation 14: Due to lacking information on the real needs input by other agendas, the motivations to consider a larger UCI/DCI payload size cannot be justified by the channel coding agenda alone. Proposal 10: UCI/DCI payload size extension should be discussed and confirmed by other agendas, e.g., control channel design. |
| CMCC | Proposal 1: For the UCI with payload sizes beyond NR range, enhancements to the polar code segmentation scheme can be considered, e.g., increasing the maximum number of segments. |
| CATT | Observation 2: The 5G NR segmentation rule results in significant performance loss up to about 0.5dB over AWGN channel and does not effectively estimate the crossover point between segmented and non-segmented performance. Observation 3: Performance degradation is observed for large UCI payload size, even when the size does not exceed 1706. Observation 4: 5G NR segmentation scheme suffers significant performance loss and is not a robust solution for UCI segmentation. Observation 5: Compared to 5G NR segmentation scheme, Scheme 2 demonstrates better performance without increasing complexity. Proposal 6: NR segmentation rules should be re-optimized based on simulation results and only applied when a clear performance advantage is demonstrated. Proposal 7: Segmentation schemes with more than two segments should be considered, both to better support larger UCI in 6G and to improve performance for size below 1706. Proposal 8: The following two-segment rule based on linear function should be used for UCI segmentation If R<=1/5 Ksegthr=370 Else if R>1/5 Ksegthr=832*R+200 End Observation 6: Enhancement of segmentation by increasing the number of segments can improve BLER performance significantly for large UCI payload size compared with the NR segmentation scheme, with the gain ranging from 0.43 dB to 1.88 dB at BLER=1e-2 across all examined cases. Proposal 9: Segmentation scheme with >=2 segments should be considered for UCI segmentation. |
| Lenovo | Proposal 11: Determine the scope of control channel coding enhancements based on one of: Alternative 1: Study and evaluate further Polar coding enhancements for control channel coding beyond 5G NR Polar. Alternative 2: Reuse 5G NR Polar control channel coding and revisit need for any necessary minor enhancements following other A.I. progress (e.g., control channel design, MIMO CSI feedback, etc.) |
| AT&T | Proposal 4 Reuse NR LDPC codes and NR Polar codes for 6GR data channel(s) and 6GR control channel(s), respectively, at least under the same NR conditions on code rate and code block length. |
| Xiaomi | Proposal 1: For within NR range, confirm the working assumption for 6G LDPC and 6G polar for within NR range as below - For data rate within NR range, reuse of NR LDPC design is supported - For control information within NR range (larger than 11 bits), reuse of NR Polar code design is supported Proposal 2: For beyond NR range, - Thorough and vigorous evaluation is needed to justify peak data rate oriented incremental enhancement with specification change for LDPC - Designs motivated by increase of UCI/DCI payload size(s), if any, shall be triggered by relevant discussion instead of channel coding discussion |
| Huawei | Observation 23: Even with aggressive massive MIMO configurations, the estimated maximum UCI size typically remains well within the 1706-bit limit, and can be effectively managed by polar codes already used in 5G NR. Observation 24: A simple and unified segmentation rule is sufficient to achieve close-to-optimal CB segmentation for larger UCI payload size (>1706 bits). Proposal 5: If the maximum UCI payload size would exceed 1706 bits, more than 2 CB segmentations should be considered for UCI with payload size lager than 1706 bits. |
| Samsung | Observation 9: In 6G, there are need for larger UCI size more than 1706 bits such as CSI feedback for energy saving the possibility of joint UCI feedback, and more. Observation 10: NR polar codes are constrained by a maximum mother code size of Nmax=1024 and a segmentation limit of at most two code blocks. This fixed architecture results in significant performance degradation for large UCI payloads within the existing NR UCI range. In 6GR, uplink control payloads are expected to be even larger, exacerbating the performance loss caused by the limited scalability. Observation 11: Extending segmentation beyond two CBs provide significant, stable coding gains and is necessary to support payloads larger than 2026 bits. Proposal 7: RAN1 should introduce an extended segmentation scheme and allow more than two code block segments to support large UCI payloads reliably. Observation 12: The NR segmentation and mother code size selection rule can abruptly halve the mother code size from N to N/2 when the segmentation is triggered. This abrupt halving results in a critical performance loss of 0.5 dB between pre- and post-segmentation scenarios, which can be resolved by a simple, code-rate-aware modification to the segmentation trigger. Proposal 8: RAN1 should refine the existing segmentation rule for the NR range to avoid such drastic halving phenomenon to ensure a graceful, stable performance curve as payload size increases. Observation 13: A straightforward extension of segmentation, where each of the C segments appends its own full-length CRC (e.g., 11 bits), creates a total CRC overhead that scales linearly with C, introducing a substantial rate loss and thus performance degradation. This overhead is unnecessary, as the per-CB CRC length can be reduced as C increases while maintaining the same target FAR. Observation 14: TPAC codes replace CRC with termination zero-padding, which the convolutional pre-transform converts into parity bits for error detection. This approach achieves comparable FAR performance while eliminating CRC overhead, and its flexible termination length provides additional coding gains in multi-segment scenarios. Proposal 9: RAN1 should investigate an overhead-efficient error-detection scheme that satisfies the target FAR. Proposal 10: We propose that RAN1 consider a PAC coding scheme with flexible allocation of zero-padding bits to offset increasing parity-bit overhead under segmentation. |
| ZTE | Observation 22: Performance degradation is more serious if UCI payload size further increases (i.e., larger than 1706 bits) when using the 5G polar code with 2 segments. Proposal 10: Polar coding enhancement should be considered for large UCI sizes (i.e., for the cases with the UCI size within and beyond 5G range). Observation 24: Beyond the 5G range: when the information length ranges from 1706 to 2000bits, increasing the number of segments outperforms 5G segmentation at a target BLER of 0.01. The observed gains are - Code rate 1/12: 2.14– 5.03 dB - Code rate 1/6: 2.12 – 5.07dB - Code rate 1/3: 2.30 – 4.93 dB - Code rate 1/2: 1.92 – 4.95dB - Code rate 2/3: 1.44 – 5.09 dB - Code rate 3/4: 0.66 – 4.19 dB - Code rate 5/6: 0 – 3.25 dB Proposal 11: Polar coding enhancement for segmentation, e.g., increase the maximum number of segments for UCI, should be considered in 6GR. |
| Tejas | Proposal 5: Study following channel coding enhancements for large UCI payloads (greater than 1000 bits) - Polar code enhancements - LDPC codes Proposal 6: Study polar codes for higher order QAM’s for control channel for better SE |
| LGE | Proposal 6: Polar code enhancement can be considered as a topic for 6G channel coding study, including supports for large UCI payload. Proposal 7: Study polar code enhancements to efficiently support larger payload sizes than 5G NR, while reusing the existing 5G NR polar sequences. |
| Fujitsu | Proposal 5: - To reduce the performance degradation for large payload size and/or low code rates, RAN1 to consider a new CB segmentation and a larger code length for polar codes. |
| MediaTek | Proposal: Study high order QAM polar code for UCI performance enhancement and DCI SE enhancement. |
| Ericsson | Observation 6 The necessity of extending channel code design for 6G control channels beyond the NR control channel coding schemes is unclear. This can be revisited based on additional needs (if identified) in other 6GR discussions. Observation 9 Regarding polar code extension for uplink control channels, whether the UCI payloads would exceed the NR range (e.g. 1706 bits payload) is unclear and there exist alternate solutions that do not require increase of Polar code complexity. |
| NTT DOCOMO | Proposal 15 - The discussion regarding the necessity of larger DCI/UCI payload sizes than those in 5G should be left to the control discussion on AI 11.9. Whether to specify control channel coding enhancements related to larger DCI/UCI payload sizes should be determined based on the outcome of control discussion on AI 11.9. - For any agreement/conclusion related to larger DCI/UCI payload sizes, keep “For larger DCI/UCI payload sizes than those in 5G NR, which may/may not be supported,” in the main bullet until concluded in AI 11.9. Proposal 16 - Details of UCI/DCI and its container should be discussed separately in the control discussion on AI 11.9. Proposals/agreements/conclusions on AI 11.4.1 do not determine the definition and details of UCI/DCI and its container. The definition and details of UCI/DCI and its container will follow the agreements reached in the control discussion. - For any agreement/conclusion related to DCI/UCI, add/keep “if treated as layer 1 information” in the main bullet until concluded in AI 11.9. Proposal 18 - For control channel coding, 5G Polar code should be reused as much as possible unless there are critical motivations and clear gains enough to justify the enhancement Proposal 19 - For control channel coding, 5G polar transform should be reused if extensions of the polar transform part have a significant complexity increase Proposal 21 - 5G Polar code (including 5G code block segmentation, 5G polar sequence plus polar transform plus concatenated coding) should be adopted for UCI with payload size between 12 bits and 1706 bits if UCI is treated as layer 1 information Proposal 25 - For L1 uplink control information beyond the NR payload size range (which may/may not be supported), RAN1 should study enhanced code block segmentation scheme that optimizes the balance between simplicity of the segmentation rules and error performance as a possible solution |
| Google | Observation 1: For larger UCI size, it is not necessary to change the maximum code block size and reliability table of 5G polar coding scheme. Proposal 1: RAN1 studies DCI and UCI size enhancement for coding schemes, only if requirements have been identified in 6GR data and control channels agenda items. |
| C-DOT | - Proposal 1: For 6GR high throughput communication scenario - Data Channels: Consider using existing NR LDPC BG1 and BG2. For higher lifting sizes define BG3 with girth ≥ 6. - Control Channels Uplink): Consider alternate short block length codes for UCI payload of length 6-11 bits and continue reusing existing 5G NR codes for control channel, to maintain H/W compatibility. For UCI > 1706 bits consider using LDPC BG2, instead of modifications to uplink control channel process involving polar codes. - Proposal 2: For 6GR hyper reliable low latency communications - Data Channels: Consider using existing LDPC BG1 and BG2. - Control Channels (Uplink): Consider alternate short block length codes for UCI payload of length 6-11 bits and continue reusing existing 5G NR codes for control channel, to maintain H/W compatibility. For UCI > 1706 bits consider using LDPC BG2, instead of modifications to uplink control channel process involving polar codes. |
| Vodafone, AT&T, BT, Bouygues Telecom, Deutsche Telekom, Orange, Telecom Italia, Nokia, SK Telecom, Ericsson, T-Mobile, Rakuten Mobile | Proposal 2: Agree on the following for data and control channel coding: - For data rate at least within NR range, reuse NR LDPC design - For control information at least within NR range (larger than 11 bits), reuse NR Polar code design - FFS: Clarification on the definition of "NR range" |

Summary of inputs

In NR, Polar code is used in UCI channel coding when payload size is larger than 11. Furthermore, 2 segments are applied when (payload size ≥ 360 ∩ encoded bit length  ≥ 1088) ∪ payload size ≥ 1013. And the largest supported UCI payload size is 1706 bits).

In RAN1#122bis meeting, the agreements for control channel are as below

Agreement for control channel coding - For 6G channel coding, LDPC is used for data (including SIBs) and Polar code is used for L1 control information (larger than 11 bits, including PBCH) - For 6G Polar code - Working assumption: For control information within NR range (larger than 11 bits), reuse of NR Polar code design is supported - For control information beyond NR range, study Polar code extension with acceptable performance-complexity tradeoff for both NW side and UE side - Note: Necessity for control information beyond NR range is to be further discussed - Polar code maximum mother code length is kept as 1024. - FFS: further motivation(s) for potential extension/enhancement until RAN1#123

In RAN1#123 meeting, companies’ views on 6GR UCI channel coding are summarized as below:

For control channel coding on UCI within NR range

- 16 sources (Nokia, vivo, AT&T, Xiaomi, Huawei, Ericsson, NTT DOCOMO, Rakuten Mobile, Vodafone, BT, Bouygues Telecom, Deutsche Telekom, Orange, Telecom Italia, SK Telecom, T-Mobile) suggested using NR design can within NR range.

- Issue U-within-1: BLER performance

- 3 sources (CATT, Samsung, ZTE) observed BLER performance degradation even when UCI payload size does not exceed 1706 bits due to excessive repetition.

- Option 1: use NR design: Nokia, vivo, AT&T, Xiaomi, Huawei, Ericsson, NTT DOCOMO, Rakuten Mobile, Vodafone, BT, Bouygues Telecom, Deutsche Telekom, Orange, Telecom Italia, SK Telecom, T-Mobile

- Option 2: more than 2 segments for UCI payload size within NR range: CATT, Samsung, ZTE

  - CATT observed that the segmentation gain increases ranging from 0.43 dB to 1.88dB as the payload size increases from 1260 to 1644, when more segments are used.

  - Samsung observed at a target BLER of 1e-3, segmenting a UCI payload of 1427 bits into four CBs achieves more than 1 dB of coding gain, segmenting a UCI payload of 1706 bits into five CBs achieves more than 2 dB of coding gain.

  - ZTE observed that for information lengths ranging from 1024 to 1706 bits, BLER gains by increasing the number of segments at a target BLER of 0.01 are: Code rate 1/12: 0 .35– 2.07 dB; Code rate 1/6: 0.34 – 2.07dB; Code rate 1/3: 0.57 – 2.17 dB; Code rate 1/2: 0 – 1.86dB; Code rate 2/3: 0 – 1.34 dB; Code rate 3/4: 0 – 0.63 dB.

- Option 3: re-optimize NR segmentation rules: CATT, Samsung

  - CATT observed that the 5G NR segmentation rule does not effectively estimate the crossover point between segmented and non-segmented performance and results in up to about 0.5dB performance loss over AWGN channel.

  - Samsung observed that the NR segmentation and mother code size selection rule can abruptly halve the mother code size from N to N/2 and result in 0.5 dB performance loss.

- Option 4: terminated polarization-adjusted convolutional (TPAC) coding scheme: Samsung

  - Samsung observed that using TPAC with 8 zero-padding can reduce CRC overhead and deliver up to 2.2 dB coding gain compared with NR segmentation.

  - Huawei observed that PAC-Polar provides limited performance gain and brings challenges to use NR SCL decoder.

- Issue U-within-2: higher modulation order

- 2 sources (Tejas, MediaTek) discussed higher modulation order for Polar code

  - For 16QAM, MediaTek observed 0.2-0.3 dB gain over 5G BICM from MLC framework. By incorporating shaping bits for 16QAM, >0.5dB gain is observed across the examined payload sizes.

For control channel coding on UCI beyond NR range,

- 14 sources (Nokia, Spreadtrum, vivo, CMCC, Lenovo, Xiaomi, Huawei, Samsung, ZTE, Tejas, LGE, Ericsson, NTT DOCOMO, Google) provided the views regarding the maximum UCI payload size in 6G,

- the maximum UCI payload size may exceed 1706 bits: CMCC, Samsung, ZTE, LGE, Tejas

  - increase the CSI feedback payload size by deployments of wider bandwidths such as the around 7GHz: CMCC, ZTE, LGE

  - increase the CSI feedback payload size by MIMO systems with more antenna ports: CMCC, ZTE, LGE

  - CSI feedback for energy saving: Samsung

  - increase CSI ports with non-AI/ML and AI/ML feedback methods: Tejas

- the maximum UCI payload size may not exceed 1706 bits: Huawei

- Unclear of maximum DCI payload size increase: Ericsson

- depend on other agendas: Nokia, Spreadtrum, vivo, Lenovo, Xiaomi, NTT DOCOMO, Google

- Issue U-beyond-1: BLER performance

- 10 sources (Spreadtrum, CMCC, CATT, Huawei, Samsung, LGE, Fujitsu, Apple, NTT DOCOMO, C-DOT) discussed coding schemes for UCI with payload size beyond 1706 bits, 4 sources (CMCC, CATT, Samsung, ZTE) observed performance degradation for larger UCI payload size,

- Option 1: more than 2 segments for UCI payload size beyond NR range: (9 sources) Spreadtrum, CMCC, CATT, Huawei, Samsung, ZTE, Fujitsu, Apple, NTT DOCOMO

  - Samsung observed extending segmentation beyond two CBs provide more than 4~5dB for payloads larger than 2026 bits.

  - ZTE observed that for the information length ranges from 1706 to 2000bits, BLER gains by increasing the number of segments at a target BLER of 0.01 gains are: Code rate 1/12: 2.14– 5.03 dB; Code rate 1/6: 2.12 – 5.07dB; Code rate 1/3: 2.30 – 4.93 dB; Code rate 1/2: 1.92 – 4.95dB; Code rate 2/3: 1.44 – 5.09 dB; Code rate 3/4: 0.66 – 4.19 dB; Code rate 5/6: 0 – 3.25 dB.

  - Nokia pointed out that encoding/decoding complexity and latency should be also evaluated with more segments.

- Option 2: terminated polarization-adjusted convolutional (TPAC) coding scheme: Samsung

  - Samsung observed that using TPAC with 8 zero-padding can reduce CRC overhead and deliver up to 2.2 dB coding gain compared with NR segmentation.

  - Huawei observed that PAC-Polar provides limited performance gain and brings challenges to SCL decoder.

- Option 3: (N, K) Polar code based on N/2 Polar sequence: LGE

  - LGE observed that when using the (N, K) polar code based on the N/2 polar sequence scheme, with K = 1706 and E = 2048, it achieves approximately a 0.2 dB gain over NR segmentation scheme at a BLER of 1%.

- Option 4: LDPC BG2 can be considered: C-DOT

  - FL comments: this is no consistent with the agreements in RAN1#122bis meeting

- Issue U-beyond-2: higher modulation order

- 2 sources (Tejas, MediaTek) discussed higher modulation order for Polar code

  - For 16QAM, MediaTek observed 0.2-0.3 dB gain over 5G BICM from MLC framework. By incorporating shaping bits for 16QAM, >0.5dB gain is observed across the examined payload sizes.

Discussion

Based on companies’ input, FL observes both for UCI payload size within and beyond NR range, companies’ views on motivation for UCI channel coding enhancements are similar, i.e., BLER performance, higher modulation order.

Based on companies’ input, FL has the following observations.

Round 1

Observation 4.2-1-v1

Observation 4.2-1-v1: [21 sources] discussed the channel coding for UCI with payload size within NR range (i.e., no larger than 1706 bits).

- [16 sources] suggested using NR Polar code

- [3 sources] observed BLER performance degradation with NR segmentation scheme.

  - [3 sources] suggested new segmentation scheme (e.g., more than 2 segments, new segmentation rule), which provides up to 2dB BLER performance gain compared to NR segmentation scheme.

  - [1 source] suggested terminated polarization-adjusted convolutional (TPAC) code, which provides up to 2.2dB BLER performance gain compared to NR segmentation scheme. [1 source] observed PAC-Polar provides limited performance gain and brings challenges to SCL decoder

- [2 sources] suggested higher modulation order for UCI.

  - For 16QAM modulation, [1 source] observed 0.2-0.3 dB gain from MLC framework over 5G BICM, with shaping bits, >0.5dB gain is observed.

Please provide your comments as below.

| Company | Comments |
| --- | --- |
| CMCC | Regarding the channel coding for UCI with payload size within NR range, we suggest using NR polar code. |
| Xiaomi | Please kindly check our reply to observation 4.1.1-v1. We think the situation is similar, a vast majority focuses on re-using, we don’t need these observations since we didn’t have any unified evaluation assumptions agreed yet. |
| CATT | We are OK with the direction of proposed study but prefer to have proposal for discussions |
| AccelerComm | We suggest using NR polar code for UCI with payload size within NR range. |
| Lenovo | We support using NR Polar codes for UCI and we are open to discuss any enhancements including segmentation and higher order modulations |
| NTT DOCOMO | Regarding the channel coding for UCI with payload size within NR range, we suggest that NR Polar code (including NR code block segmentation, NR polar sequence plus polar transform plus concatenated coding) should be adopted if UCI is treated as layer 1 information. |
| ZTE, Sanechips | We observed that even within the NR range, the segmentation rules of 5G NR also incur performance loss. If more segments is only applied when exceeding the NR range, there will be a significant performance gap at the boundary, i.e., the BLER performance for UCI=1706bits may be about 2dB worse than UCI=1707 bits.. To resolve this issue, ,more segmentation can also be evaluated and considered within NR range. For the third bullet, it should be clarified whether is to be discussed in channel coding or modulation. |

Observation 4.2-2-v1

Observation 4.2-2-v1: [14 sources] discussed the necessity and channel coding for UCI with payload size beyond NR range (i.e., larger than 1706 bits).

- For the necessity of UCI with payload size beyond NR range:

  - [5 sources] observed the UCI with payload size can exceed NR range considering CSI feedback payload size for wider bandwidth, more antenna ports

  - [2 source] observed the necessity is unclear

  - [8 sources] observed the necessity depends on other agendas

- For the coding scheme for UCI with payload size beyond NR range:

  - [9 sources] suggested applying more than 2 segments for UCI with payload size beyond NR range, wherein [2 sources] observed 2~5dB BLER performance gain by more segments for UCI payload size ranges from 1706 to 2026bits. [1 source] suggested evaluating encoding/decoding complexity and latency with more segments.

  - [1 source] suggested terminated polarization-adjusted convolutional (TPAC) code. [1 source] observed PAC-Polar provides limited performance gain and brings challenges to SCL decoder.

  - [1 source] suggested Polar encoding based on N/2 Polar code sequence, which approximately provides 0.2 dB gain over NR segmentation scheme for K = 1706 and E = 2048, BLER of 1%.

- [2 sources] suggested higher modulation order for UCI.

  - For 16QAM modulation, [1 source] observed 0.2-0.3 dB gain for MLC framework over 5G BICM, with shaping bits, >0.5dB gain is observed.

Please provide your comments as below.

| Company | Comments |
| --- | --- |
| CMCC | For the coding scheme for UCI with payload size beyond NR range, on top of more segmentations, concatenated coding schemes can be further considered, where an outer/inner code is introduced to provide additional error protection across the sub-blocks after segmentation while preserving the inherent benefits of short polar codes. |
| Xiaomi | Please kindly check our reply in 4.1.2-v1, we think the following update can be made for progress on this topic. Update the RAN1#122bis note as follows, - Note: Necessity for control information beyond NR range is to be further discussed triggered by control agenda. |
| CATT | We would prefer to have a proposal for the direction of further study. |
| AccelerComm | We suggest the use of more segments for UCI with payload size beyond NR range. |
| Lenovo | We support using NR polar code for UCI. Enhancements beyond NR range depend on the outcome of other agenda items, like control channel design. |
| NTT DOCOMO | The discussion regarding the necessity of larger DCI/UCI payload sizes than those in 5G should be left to the control discussion on AI 11.9. Whether to specify control channel coding enhancements related to larger DCI/UCI payload sizes should be determined based on the outcome of control discussion on AI 11.9. For any agreement/conclusion related to larger DCI/UCI payload sizes, keep “For larger DCI/UCI payload sizes than those in 5G NR, which may/may not be supported,” in the main bullet until concluded in AI 11.9. From the discussion schedule perspective, we do not object to starting the study of possible solutions related to larger UCI payload in advance. We are OK to study enhanced code block segmentation scheme that optimizes the balance between simplicity of the segmentation rules and error performance as a possible solution. |
| ZTE, Sanechips | For the first bullet, the UCI with payload size may exceed the NR range considering CSI feedback payload size for wider bandwidth and more antenna ports. Although the final UCI size depends on other agendas, the channel coding agenda should provide a backup solution that can be directly used if other agendas determine that the UCI will exceed NR range. For the second bullet, we observed that outside the NR range, the segmentation rules of 5G NR result in more severe performance loss. Outside the NR range, more segmentation options can be evaluated and considered. For the third bullet, it should be clarified whether is to be discussed in channel coding or modulation. |

  - Small code block

Summary of observations/proposals

| Source | Observation/Proposal |
| --- | --- |
| Nokia | Proposal 6: For 6G, block codes for small block lengths should be kept the same as in 5G. |
| vivo | Observation 19: The UCI transmitted by AI-generated sequences can outperform legacy RM based schemes for different receiving antenna assumptions. For the target BLER@10E-2, in the case with CDL-C fading channels, more than 1dB BLER gain is observed if realistic channel estimation is used in RM decoding. Proposal 13: Further study the feasibility and potential of applying AI-generated sequences to transmit the UCI with the payload size spanning from 3 to [11] bits, where the upper side boundary can be discussed separately. |
| CMCC | Proposal 4: If the RM code used for 3~11-bit UCI payload is replaced by an optimized coding scheme in 6GR, the range of small UCI, especially the upper limit, i.e., 11 bits, could be reconsidered. |
| Lenovo | Proposal 13: RAN1 to study short block codes enhancements targeting better BLER performance for K payload sizes (K > 11 bits) for control channels. - FFS: the maximum bound on K value. |
| ZTE | Observation 25: In 5G NR, UCI associated with different priority index may be transmitted in PUCCH or PUSCH at higher code rate. Observation 26 UCI at higher code rate can improve resource efficiency for data transmission (e.g., HRLLC packet) in PUSCH, when it is multiplexed in PUSCH. Observation 27 5G RM code at higher code rate has inferior performance and even suffers from error floor due to the limited minimum hamming distance. Proposal 13 RM code can be enhanced for 6GR. |
| Tejas | Proposal 7: Study small block length codes for DCI payload sizes < 12 bits |
| EURECOM | Observation 1: Coverage enhancement is one of the key KPIs in 6G. Previous studies of coverage enhancements showed that significant performance improvements in the transmission of small UCI payloads are possible. Observation 2: The performance of 3GPP Short Block-Length codes is far from optimal and there is significant room for improvement. Proposal 1: Study novel encoding/modulation schemes for transmission of short packages. Observation 3: For short block lengths, DMRS introduce a significant amount of sub-optimality and potential novel coding strategies should aim to reduce this overhead. Observation 4: The proposed transmission scheme has low complexity because detection in time and frequency domain can be efficiently separated. Observation 1: DMRS-less transmission schemes provide significant room for PAPR reduction. Observation 5: Simulations of novel coding strategies in UCI transmission show significant performance improvements over NR Short Block-Length Codes. |
| Ericsson | Proposal 7 For channel coding of up to K=11 bits, the corresponding NR channel codes are reused, i.e. • K=1: repetition code • K=2: simplex code • 3<=K<=11: LTE/NR Reed-Mueller code |
| Qualcomm | Observation 11: 5G NR Reed-Muller based linear block code has the problem of rank deficiency and decoding ambiguity for some (N, K) cases. Proposal 12: 6GR shall study new code design (including rate matching) for the small payload sizes for uplink control channel. FFS the maximal payload size K and coded bits N values supported by this new code. Proposal 13: 6GR should study conventional (i.e., non-AI/ML) channel coding designs that facilitate exploitation of side information available at the transmitter/receiver about the source information. |
| C-DOT | - Proposal 1: For 6GR high throughput communication scenario - Data Channels: Consider using existing NR LDPC BG1 and BG2. For higher lifting sizes define BG3 with girth ≥ 6. - Control Channels Uplink): Consider alternate short block length codes for UCI payload of length 6-11 bits and continue reusing existing 5G NR codes for control channel, to maintain H/W compatibility. For UCI > 1706 bits consider using LDPC BG2, instead of modifications to uplink control channel process involving polar codes. - Proposal 2: For 6GR hyper reliable low latency communications - Data Channels: Consider using existing LDPC BG1 and BG2. - Control Channels (Uplink): Consider alternate short block length codes for UCI payload of length 6-11 bits and continue reusing existing 5G NR codes for control channel, to maintain H/W compatibility. For UCI > 1706 bits consider using LDPC BG2, instead of modifications to uplink control channel process involving polar codes. |

Summary of inputs

In NR, for UCI payload size <= 11 bits, short linear codes are used. Specifically, repetition for 1-bit UCI, (3,2) simplex code for 2-bit UCI, and RM coding for 3–11bit UCI.

Regarding the channel coding performance for small code block, 4 sources (vivo, ZTE, EURECOM, Qualcomm) mentioned that Reed-Muller code has performance issue, due to of rank deficiency and decoding ambiguity for some cases, especially at higher code rate. The potential solutions to enhance performance for small code block are summarized as below

- 7 sources (vivo, CMCC, Lenovo, ZTE, Tejas, EURECOM, Qualcomm) discussed the potential solutions to improve the performance for short block code

  - AI-generated sequences for UCI with payload size from 3 to [11] bits: vivo

    - vivo observed it can outperform legacy RM based schemes for different receiving antenna assumptions. For the target BLER@10E-2, with CDL-C fading channels, more than 1dB BLER gain is observed compared with RM code.

  - Enhancement on basis sequence selection and rate matching pattern of 5G RM code: ZTE

    - ZTE observed that it can improve BLER performance significantly, i.e. 0.7dB and 1.4dB BLER performance gain under the case of K=3 and 4 at medium code rate respectively; no error floor or 0.7~1.2dB BLER performance gain under the case of K=6~8 and 11 at higher code rate, for the target BLER@10E-2, in AWGN channel.

  - VHC (vertical and horizontal code) schemes: EURECOM

    - EURECOM observed the proposed scheme has low complexity because detection in time and frequency domain can be efficiently separated. At 1%BLER, the VHC ‘low PAPR’ scheme shows a 3dB gain of the B0 bits and a 1dB gain overall compared to the standard ‘PF3, CD’ receiver. The performance is about the same as the ‘PF3, NCD’ but providing significantly lower PAPR.

  - new linear code (including rate matching): Qualcomm

    - Qualcomm observed 0.1~1dB performance gain can be achieved over NR RM-based code, in terms of required SNR at 1% BLER, for N = 12, 16, 20, 24, 32, and for variable K values.

On the other hand, Nokia, Ericsson, and C-DOT suggested reusing 5G coding schemes for small code block.

Discussion

Round 1

Based on companies’ input, FL observes BLER performance gain from enhanced schemes for small UCI. Meanwhile, some companies also propose to reuse 5G RM code. Therefore, FL suggests to considering the following proposal.

Proposal 4.3-1-v1

Proposal 4.3-1-v1: For 6G small UCI channel coding, study the following aspects:

- Use of 5G RM code

- Enhanced coding scheme (including rate matching)

- Use of sequence

- FFS: the range of small UCI, e.g., 3~11 bits

Please provide your comments as below.

| Company | Comments |
| --- | --- |
| CMCC | Unified coding schemes for PUCCH with small payload sizes (1-11bits) are preferred, to avoid excessive variety of codes (e.g., Reed-Muller, Simplex, and Repetition coding schemes). |
| Xiaomi | If we would like to study enhancements on this direction, we don’t need to study “use of 5G RM code” since the RM code has been deployed and used in real field and thus there is no need for further study. For the bullet use of sequence, is it part of channel coding agenda or should we discuss it in conjunction with UL control agenda as well. From our understanding, this sequence approach was proposed in Rel-17 focusing the coverage enhancement by eliminating the DMRS and applying non-coherent receiver at the Rx side, could the complexity and channel structure e.g. with or without DMRS be discussed here without comprehensive discussion on PUCCH channel structure? |
| CATT | We are OK with the proposed study but would like to avoid any repetitive discussions on topics been discussed in 3G/4G/5G before. |
| EURECOM | We are in favor of studying enhanced coding schemes since there is room for improvement. Those enhanced schemes may replace 5G RM codes or exist alongside them and may be used in novel DMRS-less PUCCH formats. Concerning the proposal, in a first instance we should decide if we study enhancements and if those novel coding schemes may replace legacy RM codes or exist in parallel. Hence, we propose to delete the first and third bullet and, if not understood, mention that the 5G RM codes are the baseline for evaluating the enhancements. |
| AccelerComm | We support the use of 5G RM code for small UCI. |
| Lenovo | Agree. Good to further clarify however the scope of “use of sequence” |
| NTT DOCOMO | We prefer to reuse NR coding scheme for small block size (i.e., Repetition code if K=1, Simplex code if K=2, NR Reed-Mueller code if 3<=K<=11) |
| ZTE, Sanechips | For the second bullet, we think that the enhanced coding scheme should be based on 5G RM code. And we are okay to add “others are not precluded” to this proposal. Therefore, the proposal is modified as follows: Proposal 4.3-1-v1: For 6G small UCI channel coding, study the following aspects: - Use of 5G RM code - Enhanced coding scheme based on 5G RM code (including rate matching) - Use of sequence - Others are not precluded. - FFS: the range of small UCI, e.g., 3~11 bits |

  - Channel coding on PBCH

Summary of observations/proposals

| Source | Observation/Proposal |
| --- | --- |
| Spreadtrum | Proposal 2: For control information within NR range, reuse of NR Polar code design. Proposal 3: If the final maximum UCI payload size exceeds 1706 bits, high priority should be given to more than 2 segments. |
| vivo | Proposal 9: Confirm the working assumption regarding the polar coding with further clarification For the control information with the payload size within NR range (larger than 11 bits), reuse of NR Polar code design without introducing any enhancement. |
| CMCC | Hence, similar to 5G NR, a fixed polar code also can be used for PBCH channel coding in 6GR. |
| AT&T | Proposal 4 Reuse NR LDPC codes and NR Polar codes for 6GR data channel(s) and 6GR control channel(s), respectively, at least under the same NR conditions on code rate and code block length. |
| Xiaomi | Proposal 1: For within NR range, confirm the working assumption for 6G LDPC and 6G polar for within NR range as below - For data rate within NR range, reuse of NR LDPC design is supported - For control information within NR range (larger than 11 bits), reuse of NR Polar code design is supported |
| Qualcomm | Observation 10: For broadcast PDCCH, reducing the CRC from 24 bits to 16 bits may provide 1 dB coverage improvement with minimum changes at the transmitter. Proposal 11: In 6GR, support reduced CRC size (e.g., 16 bit) for broadcast PDCCH and PBCH for coverage enhancement. |
| NTT DOCOMO | Proposal 22 5G Polar code (including 5G interleaving (CRC distribution), 5G polar sequence plus polar transform plus concatenated coding) should be adopted for PBCH |
| Vodafone, AT&T, BT, Bouygues Telecom, Deutsche Telekom, Orange, Telecom Italia, Nokia, SK Telecom, Ericsson, T-Mobile, Rakuten Mobile | Proposal 2: Agree on the following for data and control channel coding: - For data rate at least within NR range, reuse NR LDPC design - For control information at least within NR range (larger than 11 bits), reuse NR Polar code design FFS: Clarification on the definition of "NR range" |

Summary of inputs

In 5G, the payload size of PBCH is fixed as 32 bits, and the encoded bit length is fixed as 864bits.

In RAN1#122bis meeting, the agreements for PBCH channel are as below.

Agreement for PBCH - For 6G channel coding, LDPC is used for data (including SIBs) and Polar code is used for L1 control information (larger than 11 bits, including PBCH) - For 6G Polar code - Working assumption: For control information within NR range (larger than 11 bits), reuse of NR Polar code design is supported - For control information beyond NR range, study Polar code extension with acceptable performance-complexity tradeoff for both NW side and UE side - Note: Necessity for control information beyond NR range is to be further discussed - Polar code maximum mother code length is kept as 1024. - FFS: further motivation(s) for potential extension/enhancement until RAN1#123

In RAN1#123 meeting, companies’ views on channel coding for PBCH are summarized as below:

- 16 sources (Nokia, vivo, AT&T, Xiaomi, Huawei, Ericsson, NTT DOCOMO, Rakuten Mobile, Vodafone, BT, Bouygues Telecom, Deutsche Telekom, Orange, Telecom Italia, SK Telecom, T-Mobile) suggested using NR design can within NR range

- 1 company (Qualcomm) supported reduced CRC size (e.g., 16 bit) for broadcast PDCCH and PBCH for coverage enhancement

Based on companies’ input, FL has the following observation.

Discussion

Round 1

Observation 4.4-1-v1

Observation 4.4-1-v1: [17 sources] discussed the channel coding for PBCH,

- [16 source] suggested NR Polar code

- [1 source] discussed CRC overhead issue and observed that reducing the CRC from 24 bits to 16 bits may provide 1 dB coverage improvement.

Please provide your comments as below.

| Company | Comments |
| --- | --- |
| CMCC | Support NR Polar code for PBCH. |
| Xiaomi | We think this issue is similar to DCI/UCI within NR range discussion. Please kindly check our replies to observation 4.1.1-v1 and 4.2-1-v1. |
| CATT | We support reusing NR polar code for PBCH. CRC 24bits are used for lower the false alarm level and should not be changed. |
| AccelerComm | We support NR polar code for PBCH. |
| IDC | Support NR Polar code for PBCH. |
| Lenovo | We agree to continue using existing NR Polar code for PBCH. Specific enhancements beyond legacy may be however necessary based on other A.I./RAN plenary decisions on carrier bandwidth (e.g., 3 vs. 5 Mhz carrier optimizations) support in 6GR |
| NTT DOCOMO | Regarding PBCH, we suggest that NR Polar code (including NR interleaving (CRC distribution), NR polar sequence plus polar transform plus concatenated coding) should be adopted |

  - Control coding chain

Summary of observations/proposals

| Source | Observation/Proposal |
| --- | --- |
| Lekha | Observation 2: A learning based polar code design learns optimal polar code constructions from decoding performance, achieving improved error rates and adaptability across channels and decoders. Observation 3: A learning–assisted SC decoder learns to correct error-prone decisions in the decoding process, improving reliability without major complexity increases. This hybrid approach integrates deep learning with classical decoding for practical, scalable performance gains. Observation 4: Learning based adaptive polar coding predicts optimal code rates for precoded polar codes based on SNR, rate, and list size, enabling adaptive throughput optimization without exhaustive simulations. This ML-based approach allows for real-time, scalable, and reliability-aware polar code adaptation. Proposal 2: Machine learning enhances polar code design, decoding, and adaptation by learning bit reliabilities, correcting SC decoding errors, and predicting optimal code rates for varying conditions. These data-driven methods offer scalable, efficient, and robust improvements in communication system performance. |
| vivo | Observation 20: In the scenario of UCI multiplexed with UL-SCH, the cross-codeblock interleaver design, e.g., MGCM, can greatly simplify the RE allocation pattern, and meanwhile achieve greater throughput (larger than 10% in some simulated scenarios) for UL-SCH or deliver more UCI payload compared to NR baseline. Proposal 14: For the use case of multiplexing UCI and UL-SCH, consider the cross-codeblock interleaver design, e.g., MGCM, to better leverage the unbalanced capacity of different bit subchannels in QAM modulated symbols. |
| Fraunhofer | Observation 1: The message bits from a DCI format comprises zero-padding bits, reserved bits and in some cases, bit indices of fixed value, which are typically known to the UE. Proposal 1: To improve the reliability of the PDCCH, RAN1 shall study the strategic assignment of “low-priority” bits of a DCI format such as the zero-padded bits, bits with fixed value(s), etc. to bit-indices in the N-bit vector for polar encoding based on the reliability of the bit-indices. |
| Qualcomm | Proposal 13: 6GR should study conventional (i.e., non-AI/ML) channel coding designs that facilitate exploitation of side information available at the transmitter/receiver about the source information. |
| Apple | Observation 5: In 6G, it is expected that DCI formats designed for joint or dual-format decoding will converge toward similar payload sizes to facilitate efficient joint blind decoding at the UE. |

Summary of inputs

In RAN1#123 meeting, companies’ views for control channel coding chain enhancement can be summarized as below:

- Cross-codeblock interleaver design (e.g., MGCM) can better leverage the unbalanced capacity of different bit subchannels in QAM modulated symbol for the multiplexing between UCI and UL-SCH: vivo

  - vivo observed that MGCM can greatly simplify the RE allocation pattern and achieve greater throughput (larger than 10% in some simulated scenarios) for UL-SCH or deliver more UCI payload compared to NR baseline.

- Conventional channel coding designs that facilitate exploitation of side information available at the transmitter/receiver about the source information: QC

- Proper assignment of “low priority” bits for DCI: Fraunhofer

- Machine learning based Polar code design, decoder: Lekha

Discussion

Round 1

Based on companies’ input, FL suggests to considering the following proposal.

Proposal 4.5-1-v1

Proposal 4.5-1-v1: For the study of 6G control channel coding chain enhancements, proponent is encouraged to provide at least the following

- Details and expected spec impact of control channel coding chain enhancements

- Evaluation metrics, including BLER, throughput performance

  - Other metrics can be also reported

- Evaluation assumptions, including at least

  - Channel type

  - Resource allocation

  - Modulation and coding scheme

  - Channel estimation method

  - Tx/Rx antenna configuration

Please provide your comments as below.

| Company | Comments |
| --- | --- |
| CMCC | The motivation of 6G control channel coding chain enhancements should be clarified first before providing the details. |
| Xiaomi | We are open for such discussion if we can add a bullet reflecting the RAN1#122bis agreement to this. If the group thinks this proposal does not intend to revert or update the RAN1#122bis agreement, we can be flexible not to add that as well. Proposal 4.5-1-v1: For the study of 6G control channel coding chain enhancements, proponent is encouraged to provide at least the following - Details and expected spec impact of control channel coding chain enhancements - Evaluation metrics, including BLER, throughput performance - Other metrics can be also reported - Evaluation assumptions, including at least - Channel type - Resource allocation - Modulation and coding scheme - Channel estimation method - Tx/Rx antenna configuration This coding chain discussion needs to take the following agreement made in RAN1#122bis as baseline. - For 6G channel coding, LDPC is used for data (including SIBs) and Polar code is used for L1 control information (larger than 11 bits, including PBCH) - For 6G Polar code - Working assumption: For control information within NR range (larger than 11 bits), reuse of NR Polar code design is supported |
| CATT | We like to clarify what the issues of control channel coding chain are and need to be resovled. |
| Lenovo | Okay. |
| NTT DOCOMO | At this point, we tend to think there is no need to study extension of the coding chain for the other motivations than larger DCI/UCI payload. |

  - Evaluation methodology

    - Evaluation metrics

Summary of observations/proposals

| Source | Observation/Proposal |
| --- | --- |
| Nokia | Observation 2: For the study of Polar code extensions for UCI payload beyond NR range, if necessary to support larger payloads, using more segments should be properly evaluated, especially in terms of encoding/decoding complexity and latency. Proposal 5: For 6G polar codes evaluation, a baseline receiver should use successive cancellation list (SCL) decoding with a list size of L=8 or L=16. The key performance metrics for this evaluation are the overall and undetected error probabilities. |
| Xiaomi | Proposal 4: For 6GR, the following evaluation assumptions can be used to check whether the channel coding candidates fulfill the 6GR requirements. Evaluate the block error rate (BLER) performance versus SNR |
| Samsung | Evaluation metrics and criteria - Performance: Target transport block error rate (BLER) [10-2 or 10-3] - Complexity - Latency - False Alarm Rate (FAR) - Total saved computational complexity ratio (TSCCR*) for early termination gain *TSCCR = 1- No. of information bits decoded with early termination / No. of information bits decoded without early termination |
| ZTE | Performance metrics: BLER, FAR |
| Apple | Proposal 1: It is proposed to adjust the FAR requirements on a per-category basis and to study and define all FAR categories under consistent evaluation assumptions as part of the transition to 6G. Proposal 2: It is proposed to include the operating SNR assumption in the FAR requirements for those FAR categories that are sensitive to channel SNR. Proposal 3: It is proposed to include path-metric-assisted early termination as part of the decoder assumptions used for FAR evaluation. Proposal 4: It is proposed to include path-metric-assisted early termination as part of the PDCCH decoder assumptions used for BLER evaluation. Proposal 5: It is proposed to study and define a standardized early-termination scheme as part of the PDCCH decoder assumptions, with the objective of relaxing processing constraints for both UEs and network operators as 6G evolves. |
| MediaTek | Proposal: Consider following metric and methodology to facilitate early termination rate evaluation - Early termination rate: RET=Ekterminatek+m - k : payload size - m : number of total data integrity check bits - kterminate: the smallest number of decoded information bits (including data integrity check bits) when none of the candidates in the list can pass the current CRC check(s) after decoding kterminate informations bits - E[kterminate] is estimated under SCL with the assumption of list size=8 and pure AWGN noise is transmitted. Proposal: Consider following methodology to facilitate DCI RNTI false alarm evaluation. - Given a target UE RNTItarget, random payload with size K, aggregation level AL, and AWGN channel - Alt1:Evaluate PRNTIWorst=maxRNTI ≠RNTItarget Pr∃u∈ℒ s.t ϕu,RNTI passes CRC checks - ℒ is the list of 8 candidates from SCL decoding - ϕu,RNTI is the function that recovers the original unscrambled information sequence from a candidate u in the list and RNTI - Opration @SNR with BLER=1% - Alt2: Evaluate Pr∃u∈ℒ s.t ϕu,RNTIi passes CRC checks for i∈1,2,…,N - ℒ is the list of 8 candidates from SCL decoding - ϕu,RNTIi is the function that recovers the original unscrambled information sequence from a candidate u in the list and RNTIi - {RNTIi, for i∈1,2,…,N } are distinct and generated uniformly from all possible RNTI ≠RNTItarget for each DCI transmission - Opration @SNR with BLER=1% |
| NTT DOCOMO | Proposal 17 - Regarding the evaluation/analysis of “performance/complexity trade-off” for control channel coding, at least the following metrics should be considered - Performance: BLER, FAR, decoding latency - Complexity: computational complexity and complexity relevant to implementation aspects |
| Rakuten Mobile | Proposal 6: To evaluate channel coding for 6GR control channels beyond NR rage, use the following metrics: - BLER - FAR - Decoding latency - Hardware complexity - Device power consumption. Proposal 7: In studying channel coding for 6GR control channels beyond NR rage, consider the following aspects: - Low latency - High reliability: - Initial access robustness - Number of connectivity - Operating frequency. Proposal 9: In studying channel coding for 6GR control channels, use to the following evaluation methodology: - Link-Level Simulation - Complexity Analysis - Latency Evaluation - Implementation Feasibility - Evaluate efficiency and scalability for real-time deployment. |

Summary of inputs

10 sources (Nokia, Lenovo, AT&T, Xiaomi, Samsung, ZTE, Apple, MediaTek, NTT DOCOMO, Rakuten Mobile) discussed evaluation metrics for control channel coding. Companies’ views are summarized as below.

- BLER performance: Nokia, Lenovo, Xiaomi, Samsung, ZTE, Apple, NTT DOCOMO, Rakuten Mobile

- FAR: Nokia, Lenovo, Samsung, ZTE, Apple, NTT DOCOMO, Rakuten Mobile

- Complexity: Nokia, Lenovo, AT&T, Samsung, NTT DOCOMO, Rakuten Mobile

- Decoding latency: Nokia, Samsung, NTT DOCOMO, Rakuten Mobile

- Early termination rate: Samsung, Apple, MediaTek

- DCI RNTI false alarm: MediaTek

- Migration efficiency: AT&T

- Device power consumption: Rakuten Mobile

Discussion

Round 1

Based on companies’ input, FL has the following proposal.

- Proposal 4.6.1-1-v1

Proposal 4.6.1-1-v1: For the evaluation of 6G control channel coding, at least the following metrics are considered

- BLER performance

- FAR

- Complexity

- Early termination

- DCI RNTI false alarm

Please provide your comments as below.

| Company | Comments |
| --- | --- |
| Xiaomi | Similar to the data channel proposal structure, can we formulate the proposal as follows, the first three metrics seem to be the main discussion point for performance-complexity trade off correct? Proposal 3.1.1-1-v1: For the evaluation of 6G control channel coding, at least the following metrics are considered - BLER performance - FAR - Complexity In addition, the following metrics can be also reported - Early termination - DCI RNTI false alarm Note: The detailed definitions/models of the metrics should be reported. |
| CATT | OK |
| Lenovo | Generally okay, however early termination aspects could be included in complexity. |
| NTT DOCOMO | We are OK to support the first three bullets. If there is a common understanding that the main discussion point of the channel coding for DCI with payload size beyond NR range is the balance between the effect of early termination and the spec impacts, we are also OK to consider metric relevant to early termination but further detailed discussion may be needed. |

    - Evaluation assumptions

Summary of observations/proposals

| Source | Observation/Proposal |
| --- | --- |
| Nokia | Proposal 5: For 6G polar codes evaluation, a baseline receiver should use successive cancellation list (SCL) decoding with a list size of L=8 or L=16. The key performance metrics for this evaluation are the overall and undetected error probabilities. |
| CATT | K; [248:16:488, 508:16:2012] + 11-bit CRC. R; 0.12:0.02:0.4, 0.45:0.05:0.95. modulation; QPSK. channel; AWGN. Nmax; 1024(5G NR). |
| Lenovo | Table 2.2.1-1: 6GR control channel coding evaluation assumptions. Coding Scheme; Polar Codes. Code Rates; 1/8;1/3,1/5;1/2; 2/3; 3/4. Information bits lengths w/o CRC; 20, 40,100, 200, 400, 600, 1000, 2000, 4000. Channel; AWGN. Decoding Algorithm; SCL/SSC (L=8). Modulation; QPSK, 16-QAM. Proposal 12: Consider the evaluation assumptions in Table 2.2.1-1 in adopting a set of evaluation assumptions for control channel coding extensions. |
| Xiaomi | Proposal 3: For 6GR, channel coding evaluation shall be performed channel wise instead of scenario wise. - LDPC is the data channel candidate and the evaluation assumptions need to reflect the requirements for at least IC/hRLLC/MC - Polar is the control channel candidate and the evaluation assumptions need to reflect the requirements for at least IC/hRLLC/MC Proposal 4: For 6GR, the following evaluation assumptions can be used to check whether the channel coding candidates fulfill the 6GR requirements. - Evaluate the block error rate (BLER) performance versus SNR Evaluated Channel Type Evaluation Assumption; Data Channel; Control Channel. Modulation; QPSK; QPSK, 64 QAM, 256 QAM. Coding Scheme; LDPC; Polar. Code rate; 1/12, 1/6, 1/5, 1/3, 2/5, 1/2, 2/3, 3/4, 5/6, 8/9; 1/12, 1/6, 1/5, 1/3. Decoding algorithm**; min-sum; List decoding. Info. block length*** (bits w/o CRC); 20, 40,100, 200, 400, 600, 1000, 2000, 4000, 6000, 8000 Optional(12K, 16K, 32K, 64K). Channel*; AWGN. * Fading channels will be simulated in the next stage ** These algorithms are starting points for further study. Other variants of agreed algorithms can be used for encoding and decoding (Complexity details should be illustrated) *** At least these info. block length and code rate shall be evaluated. Other info. block lengths and code rates are not precluded. Similar info. and encoded block lengths should be used for the evaluation. Total coded bits = info. Block length/code rate. Note: these info. block length and code rate are only for initial performance evaluations. They are not interpreted as design targets or assumptions for complexity analysis.. - General guidelines - BLER simulations down to 10-1, 10-3 is recommended (to observe the error floor) for IC/MC - BLER simulations down to 10-4 is recommended (to observe the error floor) for hRLLC |
| Samsung | - Evaluate the flexibility and block error rate (BLER) performance versus SNR Channel; AWGN. Modulation; QPSK. Coding Scheme; Polar code. Code rate; 1/8, 1/6, 1/5, 1/3, 2/5, 1/2, 2/3, 3/4, 5/6. Decoding algorithm; SCL decoding with list-8. Info. block length (bits w/o CRC); DL: 12:1:300 UL: 12:1:20, 20:4:256, 264:8:512, 528:16:2000, 2032:32:3984. Evaluation metrics and criteria - Performance: Target transport block error rate (BLER) [10-2 or 10-3] - Complexity - Latency - False Alarm Rate (FAR) - Total saved computational complexity ratio (TSCCR*) for early termination gain |
| ZTE | - Proposal: The evaluation assumption in Table 9 can be considered as starting point. Table 9: Evaluation assumption for Polar code Channel; AWGN. Modulation; QPSK. Coding Scheme; Polar code. Code rate for UL; 1/12, 1/6, 1/3, 1/2, 2/3, 3/4, 5/6. Coded bit length for DL; 108*[1,2,4,8,16]. Decoding algorithm; SCL decoding (L=8). Info. block length (bits w/o CRC); DL: 12:4:[200] UL: 12:4:140, 140:4: 256, 264:8:512, 528:16: [2000]. Performance metrics; BLER, FAR. |
| Rakuten Mobile | Proposal 8: In studying channel coding for 6GR control channels beyond NR rage, consider at least the following assumptions: - TR 38.901 channel models as baseline, with extensions for new frequency ranges, NTN, and ISAC scenarios - Payload sizes - Small block lengths [<11 bits] - Large code lengths [>1024 bits] - Code Rates in range [1/12, 1/6, 1/3, 1/2, 2/3] - Decoding algorithms - SSC and SCL with detailed implementation including number of iterations in range [2, 3, 4, 5, 6, 8, 10, 20] and list size if applicable - Control channel types - Include representative PDCCH-like, PUCCH-like, and PBCH-like formats. |

Summary of inputs

7 sources (Nokia, CATT, Lenovo, Xiaomi, Samsung, ZTE, Rakuten Mobile) discussed the evaluation assumptions for control channel coding. Companies’ views are summarized as follows.

Table 4.6.2-1 Evaluation assumptions for control channel coding scheme

| Channel | - AWGN: CATT, Lenovo, Xiaomi, Samsung, ZTE |
| --- | --- |
| Modulation | - QPSK: CATT, Lenovo, Xiaomi, Samsung, ZTE - 16QAM: Lenovo - 64QAM: Xiaomi - 256QAM: Xiaomi |
| Coding Scheme | - Polar code: Lenovo, Xiaomi, Samsung, ZTE |
| Code rate for UL | - 0.12:0.02:0.4, 0.45:0.05:0.95: CATT - 1/8, 1/3, 1/5, 1/2, 2/3, 3/4: Lenovo - 1/12, 1/6, 1/5, 1/3: Xiaomi - 1/12,1/8,1/6,1/5,1/4,1/3, 1/2,3/5,2/3, 3/4, 5/6,7/8,0.93: Huawei - 1/8, 1/6, 1/5, 1/3, 2/5, 1/2, 2/3, 3/4, 5/6: Samsung - 1/12, 1/6, 1/3, 1/2, 2/3, 3/4, 5/6: ZTE - 1/12, 1/6, 1/3, 1/2, 2/3: Rakuten Mobile |
| Coded bit length for DL | - 108*[1,2,4,8,16]: ZTE |
| Decoding algorithm | - SCL/SSC (L=8): Lenovo, Rakuten Mobile - List decoding: Xiaomi - SCL(4, 8, 16): Huawei - SCL decoding with list-8: Samsung, ZTE |
| Info. block length (bits w/o CRC) | - CATT: - [248:16:488, 508:16:2012] - Lenovo: - 20, 40,100, 200, 400, 600, 1000, 2000, 4000 - Xiaomi: - 20, 40,100, 200, 400, 600, 1000, 2000, 4000, 6000, 8000 Optional(12K, 16K, 32K, 64K) - Samsung: - DL: 12:1:300 - UL: 12:1:20, 20:4:256, 264:8:512, 528:16:2000, 2032:32:3984 - ZTE: - DL: 12:4:[200] - UL: 12:4:140, 140:4: 256, 264:8:512, 528:16: [2000] |
| Target BLER | - 10-2 or 10-3: Samsung |

FL observes that the main divergence among companies’ proposals on evaluation assumptions is code rate and information block size. As a starting point, the FL proposal presented at RAN1#122 can be used, with updates to the upper bound and number of encoded bits for DL, determined by the aggregation levels.

Furthermore, companies also discussed the potential solutions to small block enhancement(3~11bits), the corresponding evaluation assumptions also need to be discussed.

Therefore, the FL proposal about evaluation assumptions for control channel coding is as follows.

Discussion

Round 1

- Proposal 4.6.2-1-v1

Proposal 4.6.2-1-v1: For the evaluation assumptions for 6G control channel coding scheme, following evaluation assumptions are considered.

| Channel | AWGN |
| --- | --- |
| Modulation | QPSK Other modulation orders in UL can be reported by company |
| Code rate for UL | 1/12, 1/6, 1/3, 1/2, 2/3,3/4, 5/6 |
| Coded bit length for DL | 108*[1,2,4,8,16] |
| Decoding algorithm for Polar code | SCL decoding (list size=8) |
| Info. block length (bits w/o CRC) | UL: 3:1:11, 12:4: 256, 264:8:512, 528:16: [X] DL: 12:4: [Y] X, Y values are to be reported by company |

Please provide your comments as below.

| Company | Comments |
| --- | --- |
| Xiaomi | We can be OK focusing on QPSK as the modulation order. It seems we don’t have information bits for UL/DL rows? Is the intention leaving to companies’ report? For the coded bit length for DL, why don’t we use the 164 bits (140 bits + 24 bits CRC) as the basis for scaling? |
| CATT | OK |
| AccelerComm | CRC-aided SCL decoding was assumed as the baseline polar decoding algorithm during 5G standardisation. |
| Lenovo | Agree |
| NTT DOCOMO | According to the current agreements, the main target of Polar code extension is for control information beyond the NR payload size range. Considering this situation, we tend to think that the evaluation within NR payload size range is not needed. |
| ZTE, Sanechips | For UL, X should be at least equal to 2000. |

- Proposals for offline discussions

  - Proposals for Mon Offline

References

- RP-252912 	Study on 6GR Radio.

- RP-252946 	Draft Reply LS to ITU-R_WP5D_TEMP_167 = RP-243202 on Minimum requirements related to technical performance for IMT-2030 radio interface(s)

- R1-2508338	Channel Coding in 6G Radio Air Interface	Nokia

- R1-2508358	Channel Coding for 6GR Interface	Lekha Wireless Solutions

- R1-2508389	Discussion on channel coding for 6GR	Spreadtrum, UNISOC

- R1-2508434	Discussion on Channel Coding for 6GR air interface	vivo

- R1-2508457	Discussion on channel coding for 6GR interface	CMCC

- R1-2508597	Channel coding for 6G network	CATT

- R1-2508622	Channel Coding for 6G	Lenovo

- R1-2508640	Views on Channel Coding for 6GR	AT&T

- R1-2508686	Discussion on 6GR Channel Coding	Xiaomi

- R1-2508729	Discussion on 6G channel coding	OPPO

- R1-2508737	Channel coding for 6GR air interface	Huawei, HiSilicon

- R1-2508804	Discussion on channel coding for 6GR	Samsung

- R1-2508821	Discussion on channel coding for 6GR	ZTE Corporation, Sanechips

- R1-2508826	Channel coding for 6GR Air Interface	Tejas Network Limited

- R1-2508839	Discussion on Channel Coding for Small Block Lengths	EURECOM

- R1-2508842	Discussion on channel coding for 6GR	Shanghai Jiao Tong University.

- R1-2508870	Channel coding enhancements for 6GR air interface	InterDigital, Inc.

- R1-2508910	Channel coding study for 6G	LG Electronics

- R1-2508931	Discussion on channel coding for 6GR	Fujitsu

- R1-2508975	Discussion on 6GR channel coding	ETRI, ESA, Thales

- R1-2509047	Study of channel coding aspects in 6G Radio	Fraunhofer IIS, Fraunhofer HHI

- R1-2509112	Considerations of 6GR Channel Coding	Apple

- R1-2509136	Channel Coding	Spark NZ

- R1-2509145	Channel coding for 6GR interface	MediaTek Inc.

- R1-2509169	Channel coding for 6GR interface	Ericsson

- R1-2509233	Channel coding for 6GR	Qualcomm Incorporated

- R1-2509284	Discussion on Channel coding for 6GR	NTT DOCOMO, INC.

- R1-2509299	Discussion on channel coding for 6GR air interface	Google Korea LLC

- R1-2509351	Views on Channel Coding for 6G	CEWiT

- R1-2509370	Discussion on Channel Coding for 6GR	Rakuten Mobile, Inc

- R1-2509378	Discussion on 6G channel coding	C-DOT

- R1-2509398	On channel coding aspects	Vodafone, AT&T, BT, Bouygues Telecom, Deutsche Telekom, Orange, Telecom Italia, Nokia, SK Telecom, Ericsson, T-Mobile US, Rakuten Mobile

- R1-2508841   LDPC decoder throughput and chip area AccelerComm

Annex A: SID objectives

------------------------------------------------------------------omitted------------------------------------------------------------------

- Physical Layer structure for 6GR,

  - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4]

  - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1]

  - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1]

  - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4]

  - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2]

  - MIMO operation [RAN1, RAN4]

  - Duplexing [RAN1, RAN4]

  - Initial access [RAN1, RAN2, RAN4]

    - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4]

    - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4]

  - 6GR spectrum utilization and aggregation.  [RAN1, RAN2, RAN4]

  - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4]

  - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1].

    - RAN4 can be involved, if necessary, based on the LS from RAN1

------------------------------------------------------------------omitted------------------------------------------------------------------

Interim results shall be delivered as per the milestones below, in coordination with the RAN Plenary 6G Study [RP-250810].

TSG#112 (June/2026):

RAN1 to provide interim assessment on the following areas:

- Waveform, modulation, channel coding: scope of enhancements beyond NR baseline ((2) a, c)

- Channel bandwidth (min and max), frame structure, numerology ((2) b, d)

- Basic sync signal structure and associated periodicity(ies) ((2) h)

For objectives where RAN4 may be impacted, RAN1 shall coordinate with RAN4 early to enable the above assessment by June 2026.

RAN3 to provide interim study results to allow TSGs to make a decision on:

- RAN-CN interface: P2P vs SBI

- RAN internal interfaces: CU-DU split, CP-UP split.

RAN plenary to make a decision on additional 6G-6G aggregation beyond 6G CA: 6G-6G DC. RAN plenary will task relevant RAN WGs for any specific technical analysis, as needed.

NOTE: It is planned to decide on Release-21 timeline in June/2026.

------------------------------------------------------------------omitted------------------------------------------------------------------

Annex B: RAN1 agreements for 6G channel coding

Chairman guidance in RAN1#122

Data channel

For 6GR data channel coding,

- Evaluations can be provided in form of BLER results.

- Evaluation/analysis on throughput, complexity, and decoding latency can be provided

- Other metrics are not precluded.

- Proponent companies to provide their target scenarios and requirements, evaluation assumptions and methodologies for respective evaluation/analysis, e.g., decoding algorithm and details, information sizes, code rates, HARQ scheme, channel type, modulation order, target BLER, etc.

- Proponent companies to provide details of channel coding extension compared with NR channel coding.

- Proponent companies to provide justification for the channel coding extension, and how to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off, compared with data channel codes as defined in 5G NR.

Control channel

For 6GR control channel coding,

- Evaluations can be provided in form of BLER and FAR results.

- Evaluations/analysis can be provided for complexity, decoding latency,

  - Other metrics are not precluded.

- Proponent companies to provide evaluation assumptions and methodologies for respective evaluation.

- Proponent companies to provide details of channel coding extension compared with NR channel coding

- Proponent companies to provide justification for the channel coding extension, compared with control channel codes as defined in 5G NR.

Agreements in RAN1#122bis

Channel coding

Agreement

- For 6G channel coding, LDPC is used for data (including SIBs) and Polar code is used for L1 control information (larger than 11 bits, including PBCH)

- For 6G LDPC

  - Working assumption: For data rate within NR range, reuse of NR LDPC design is supported

  - For data rate beyond NR range, study LDPC extension with acceptable performance-complexity tradeoff for both NW side and UE side

    - Note: Applicability of the potential LDPC extension to data rate within NR range will be further discussed

- For 6G Polar code

  - Working assumption: For control information within NR range (larger than 11 bits), reuse of NR Polar code design is supported

  - For control information beyond NR range, study Polar code extension with acceptable performance-complexity tradeoff for both NW side and UE side

    - Note: Necessity for control information beyond NR range is to be further discussed

    - Polar code maximum mother code length is kept as 1024.

  - FFS: further motivation(s) for potential extension/enhancement until RAN1#123

Data channel

Working Assumption

- Study 6G data channel coding for higher throughput than 5G with acceptable performance-complexity tradeoff for both NW side and UE side,

  - Target peak data rate is assumed to be 2 times of the target peak data rate defined in TR38.913

Note: The other target throughput is up to company to report.

Note: Applicability of the potential channel code will be further discussed.

Agreements in RAN1#123