3GPP TSG-RAN WG1 Meeting #123	R1-2509405

Dallas, TX, USA, November 17th – 21st 2025

Agenda Item:	11.5

Source:	Moderators (Ericsson, MediaTek)

Title:	Summary #2 of Energy Efficiency Study

Document for:	Discussion, Decision

Introduction

This summary synthesizes observations and proposals from contributions submitted to 3GPP TSG RAN WG1 #123, for EE for 6G Radio (6GR) in AI 11.5 and as described in the SID. The contributions address network energy savings (NES), user equipment power saving (UEPS), and joint NW-UE EE, emphasizing a holistic, day-one integration to overcome 5G’s limitations, such as backward compatibility constraints and fragmented feature deployment, etc.

Work plan

The EE work in the 6G Radio SI has been scheduled for three meetings and the objective is to come up with recommendations to consider for further studies in different agenda items. That means that after the present meeting, only two meetings remain. Rapporteur’s work plan for AI 11.5 Energy Efficiency is as follows [1]:

RAN1#122 (8 TU) - Energy efficiency - Identify candidate technologies for NW power saving, UE power saving, and joint mechanisms taking both NW and UE into account for power saving. RAN1#122bis (10 TU) - Energy efficiency - Continue identifying candidate technologies for NW power saving, UE power saving, and joint mechanisms taking both NW and UE into account for power saving. RAN1#123 (10 TU) - Energy efficiency - Complete identifying candidate technologies for NW power saving, UE power saving, and joint mechanisms taking both NW and UE into account for power saving, to be distributed to respective related agenda.

In FL’s understanding, the term candidate technology represents a deliverable for the initial energy efficiency phase of the 6G radio SI.

The Chairman has also further clarified to the FLs that information about when and how to use the power models is valuable to for future work, i.e.,:

- Whether future evaluations should be qualitative or quantitative,

- Whether metrics and modes are used for system or link level simulations

The FL’s detailed deconstruction of the work plan is to divide the work into the following parts:

RAN1 #122: Agree on topics for discussion (during 3 meeting EE phase) as candidate technologies for NW/UE/joint power savings (i.e., in other AIs). In parallel, start discussion on energy efficiency models, metrics and scenarios for future use.

RAN1 #122bis: Agree on candidate technologies for NW/UE/joint power savings, evolve/refine agreed topics for discussion, if needed, and agree on further topics for discussion. Evolve discussions on models, metrics and scenarios. Discuss and capture, if available, observations on issues and evaluation results.

RAN1 #123: Additional agreements on candidate technologies for NW/UE/joint power savings. Finalize models metrics and scenarios for evaluation of energy efficiency. Discuss and capture, if available, observations on issues and evaluation results to assist further studies in specific agenda items.

Proposals for online discussion

Proposal 3.3.2.1a:

Include the following non-sleep states as 6G UE power consumption model

- Other power state(s) is not precluded

- FFS: Configuration and relative power value(s) for EE processing in FR2 (including 24.25 GHz – 52.6 GHz)

- Pending agreement in 11.2 whether to evaluate 15 GHz

| Power State | Characteristics | Relative Power in FR1 (including ~7GHz) | Relative Power in FR2 (including 24.25 GHz – 52.6 GHz) |
| --- | --- | --- | --- |
| EE Processing | Processing DL WUS of OFDM-based sequence(s) for wake-up indication and, if applicable, other 6GR signal(s) of OFDM-based sequence(s) for synchronization and/or measurement, in an energy efficient manner, based on the following configuration: FR1 (including ~7GHz) - 5 MHz BW, 1-RX / 2-RX for reception; FFS: Power values for 2-RX, i.e., X1, X2, X3 - Reception time is up to a slot - Residue CFO up to [5] ppm and residue timing offset up to [2] us - Noise Figure (NF) = 6GR UE NF FR2 (including 24.25 GHz – 52.6 GHz) - FFS Note: No implication on which configuration(s) to be supported by 6GR EE-processing can only be performed during a sleep state with additional relative power value added w.r.t. the sleep state and without triggering UE transition out of the sleep state. | 10 / X1 (during micro sleep) 12 / X2 (during light sleep) 15 / X3 (during other sleep) | TBD |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 | 175 |
| SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell. TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. | 100 | 175 |
| PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. | 300 | 350 |
| UL | Long PUCCH or PUSCH. FFS: Power values for more Tx power levels, i.e., X4, X5, X6, X7 | 250 (0 dBm) X4 (10 dBm) X5 (15 dBm) X6 (20 dBm) 700 (23 dBm) X7 (26 dBm) | 350 (FFS Tx power level) |

TR 38.869:

| Power State | Relative Power (unit) | Ramp-up and down transition energy (Note1): (unit multiplied by ms) | Ramp-up time | Time for sync/re-sync |
| --- | --- | --- | --- | --- |
| Ultra-deep sleep | 0.015 | For evaluation, at least for FR1 MR ultra-deep sleep state, (Ramp-up and down transition energy, ramp-up time) is as follows, - Alt 1: (15000, 400ms) as baseline - Alt 2: (40000, 800ms) Company to report which alternative they use for which use cases. |  | For MR, at least for FR1 evaluation, - Number of SSBs for sync/re-sync for MR is up to 10 - Companies to report timeline and energy consumption |

Proposal 3.3.2.2a

Include the following sleep states for 6G UE power consumption model:

- Note: Ultra Deep Sleep and EE sleep is not intended for connected mode

| Power State | Characteristics | Relative Power |
| --- | --- | --- |
| Ultra Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing or frequency may not be maintained. | [0.05] or [0.05 + A1] |
| EE Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing or frequency may not be maintained. | [0.7] or [0.7 + A2] |
| Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | [1] or [1 + A3] |
| Light Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. | 20 |
| Micro sleep | Immediate transition is assumed for power saving study purpose from or to a non-sleep state | 45 |

| Sleep type | Additional transition energy: (Relative power x ms) | Total transition time** |
| --- | --- | --- |
| Ultra deep sleep | [40000] | [1600] ms |
| EE sleep | [4500] | [200] ms |
| Deep sleep | 450 | 20 ms |
| Light sleep | 100 | 6 ms |
| Micro sleep | 0 | 0 ms* |
| * Immediate transition is assumed for power saving study purpose from or to a non-sleep state ** Ramp-up time is no less than half of the total transition time, and time for sync/re-sync is not included |  |  |

Agreements: (RAN1#102, Aug. 2020) - The performance metrics described in TR38.840 section 8.2 is reused for power saving evaluation of Rel-17 DCI-based power saving adaptation during ActiveTime. - The following Rel-15 / 16 features is recommended of the power consumption as reference for baseline. Company can report the feature(s) being used in the baseline. - DRX - C-DRX cycle 40msec for VoIP - 10ms IAT, 8ms On-duration - Assume max two packets bundled - C-DRX cycle 160msec for FTP - Alt 1: 20 msec IAT, 8ms On-duration - Alt 2: short DRX - 20 ms [or 40ms as optional] IAT, 8ms On-duration - 20 ms for short DRX cycle, 4 cycles - Note: 100 msec IAT, 8ms On-duration can also be used with sufficient justifications that available Rel-15/16 Techniques being used to reduce UE power saving (Omitted)

Conclusion (RAN1#122b, Oct, 2025) The following existing traffic models could be used for 6GR performance evaluations, - Full buffer - FTP Model 1 (in TR 36.814) - FTP Model 3 (in TR 36.872) - XR Traffic models (in TR 38.838) - VoIP model (as in TR 36.814) - Instant message (as in TR 38.840) - Note that which model(s) will be used can be further decided when performing simulations in each individual topic.

With the above information, moderator would like to suggest company to consider the following updated proposal, that also try to integrate companies’ inputs:

Proposal 3.1.1.1a

For evaluation purposes and relative comparison over different candidate energy saving schemes for 6GR, define the following baseline energy saving configurations:

Baseline for BS:

- SSB with 20ms periodicity

- SIB1, if available, company to report assumed periodicity from 20 ms,  160 ms

- RO, if available, with 20 ms periodicity

- Companies to report sa, sp and sf values for BS processing of the above signal(s)/channel(s).

- Companies can evaluate and report other configuration(s) with justification

Baseline for UE:

- I-DRX (1.28s cycle) for idle mode

  - Group paging rate (for a PO): 10%

- C-DRX settings of (cycle, on-duration timer, inactivity timer) are assumed for the following traffic models for connected mode:

  - VoIP: (40 ms, 8 ms, 10 ms)

  - FTP3: (160 ms, 8 ms, 20 ms)

  - Instant message: (320 ms, 8 ms, 40 ms)

- Companies can evaluate and report other configuration(s) with justification

Note: The corresponding evaluation is not intended for energy efficiency comparison with 5G/NR.

Evaluation Methodology and Assumptions

Energy Efficiency Metric(s) and Evaluation Methodology

Companies’ Views. Please Unfold for Reference

| CompanyName | Views/Proposals |
| --- | --- |
| Nokia | Proposal 13: For the evaluation of 6G vs 5G NES, only deployed NES features should be considered for the 5G baseline. Proposal 14: Details of QoS/delay budget satisfaction rate metric (e.g. packet delay budget for the investigated traffic or if any other QoS attribute is considered) for 6G energy efficiency evaluation need to be further clarified, and whether or not it is applied to all the traffic types. |
| vivo | Proposal 1: Consider the following as baseline setting for evaluating UE energy efficiency improvement techniques - For RRC idle, I-DRX cycle of 1.28s - For RRC connected, C-DRX cycle of 160ms, inactivity timer (100, 40)ms - FR1 Onduration:8ms - FR2 Onduration:4ms |
| CMCC | Proposal 4: Power consumption evaluation should consider both Tx and Rx at the same time. Proposal 5: RAN1 further discuss and down-select one of the following options for EE metric definition: - Option 1: RAN1 further discuss the EE definition within RAN1#123 meeting, and send LS to RAN plenary for the final information before RAN#110 meeting. - Option 2: RAN1 focus on power model design for both NW and UE within RAN1#123 meeting, and send LS to RAN plenary for the final information before RAN#110 meeting, if any. It is up to RAN plenary to define the definition of EE and the relative values. Proposal 6: The EE definition for 3GPP internal evaluation should not be divergent from the ITU-R ones. RAN1 should take the EE definition in ITU-R as baseline if decide to further discuss the definition of EE. Proposal 7: RAN1 take the following as starting point if decide to further discuss the definition of EE: Energy efficiency is an important metric of sustainability. Network energy efficiency is the capability of a RIT/SRIT to support radio access network energy saving in relation to the traffic capacity provided. UE energy efficiency is the capability of the RIT/SRIT to support UE modem power saving in relation to the traffic characteristics. This metric is defined for the purpose of evaluation in the Immersive communication usage scenario. The metric is the relative energy savings between the selected load case(s) and a reference case. For network energy efficiency, the reference case is when network is fully loaded. For UE energy efficiency, the reference case is when UE is fully-loaded/has full-buffer service. Proponents should report the number of UEs under each BS and traffic model used for evaluation. The relative target of certain metric (e.g. threshold value for the metric, or certain performance loss compared with the benchmark) can be set. Constant that certain percent (e.g. 90%) of UE fulfils the target should be achieved while evaluating the relative power saving for each load case. The certain metric can be selected according to service type: For Vo6G (i.e. VoNR-like) service, the certain metric is latency. For burst-buffer service, the certain metric is QoS or UPT. For XR service, the certain metric is latency. |
| Fraunhofer IIS | Proposal 1: The 6GR study on EE should retain the approach in TR 38.864 for assessing an energy saving (ES) technique, i.e. contrasting the achievable ES gain along with observed impact on performance measured as other KPIs. Proposal 2: For ES features targeting UE power saving, while UE power saving gain is the key metric, potential impact on NW energy consumption should be added to the list of KPIs to be computed for the assessment. Proposal 3: Potential of other relevant EE metrics that can quantify the energy consumption per unit value of a given KPI of interest (e.g., bits/Joule) should be justified to complement or replace the agreed basic approach for EE evaluation. |
| CATT | Proposal 1: For evaluation purposes and relative comparison over different candidate energy saving schemes for 6GR, define the following baseline energy saving configurations: Baseline for BS: - SSB with 20ms periodicity - SIB1 and RO, if applicable, with 20 ms periodicity - Companies can report additional configuration if justified Baseline for UE: - I-DRX ([1.28s] cycle) for idle mode - Group paging rate (for a PO): 10% - P-RNTI is detected but PDSCH decoding results in no match - C-DRX (320ms,160ms,40ms cycle, on-duration 10ms, 8ms,4ms, inactivity timer 10ms,40ms,80ms) for connected mode - Different parameters can be used to adapt to different traffic loads - Companies can report the configuration value. Companies can report additional configuration if justified Proposal 2: FTP3 model should be considered for 6GR energy efficiency evaluation. Specifically, FTP3 (0.5MB as packet size, 200ms as mean inter-arrival time), FTP3 IM (0.1MB as packet size, 2s as mean inter-arrival time) can be considered in the evaluation. Each company may report which traffic model is used among the agreed three traffic models in their evaluations. Proposal 3: The traffic load for different load scenarios should be considered. The parameters in the following table are recommended: - Idle/empty load: Include cell-specific signals and channels, and L = 0 - low load: Include cell-specific signals and channels, and 0 < L≤15 - Light load: Include cell-specific signals and channels, and 15 < L≤30 - Medium load: Include cell-specific signals and channels, and 30 < L≤50 Note: a load (L)% of a cell is a percentage of resources used for UE specific PDSCH/PUSCH. |
| China Telecom | Observation 5：For 6GR EE metrics, it is necessary to reflect the overall system benefit and design differently based on core scenarios, as each scenario has distinct power consumption characteristics and optimization priorities. Proposal 6: 6G should jointly optimize spectrum efficiency, energy efficiency, QoS, and latency using adaptive RAN designs, evaluated through a unified system metric (e.g., $\ {\Delta EE}_{sys}$) that holistically balances throughput, latency, and power consumption across diverse service requirements. |
| AT&T | Proposal 3: Metric(s) that ensure energy saving mechanisms at the NW side have no detrimental impact on the UE side, and vice versa, are introduced in the RAN WG1 study of EE in 6GR interface Proposal 11: Energy Efficiency metric(s) are included as 6GR key performance metrics from day 1. Proposal 12: An energy efficiency metric based on aggregate throughput normalized by the total system power at the transmitter and/or receiver side, including power needed to operate different transmitter/ receiver modules, is considered as a starting point for evaluation of energy efficiency in 6GR air interface. Proposal 13: Study idle mode energy efficiency metrics for UE EE, network EE, and joint UE and NW EE. Proposal 14: Study relevant baseline schemes for network and UE energy efficiency assessment, including NW and UE configurations, network load(s), and frequency ranges. Note: Strive to simplify the evaluation assumptions whenever applicable for a given scenario. |
| xiaomi | Observation 3: Qualitative analysis is not excluded and can be used to make specific observations in certain evaluation scenarios. Observation 4: The adopted metrics can comprehensively reflect the energy saving gains and performance impacts while the introduction of EE increases the evaluation complexity without new observation dimensions. Proposal 20: The following FL proposal is adopted for baseline setting. Proposal 3.1.2.3a in [6]: For evaluation purposes and relative comparison over different candidate energy saving schemes for 6GR, define the following baseline energy saving configurations: Baseline for BS: - SSB with 20ms periodicity - SIB1 and RO, if applicable, with 20 ms periodicity - Companies can report additional configuration if justified Baseline for UE: - I-DRX ([1.28s] cycle) for idle mode - C-DRX ([160ms] cycle, on-duration [8ms], inactivity timer [20ms]) for connected mode - Companies can report additional configuration if justified Note: The corresponding evaluation is not intended for energy efficiency comparison with 5G/NR. Proposal 21: For joint NW-UE energy efficiency evaluation, report both NW and UE energy saving gains and performance impacts, as well as qualitative analysis at least in the following aspects: - Trade-offs between NW and UE energy saving - Joint benefits and synergies |
| OPPO | Proposal 7: The 5G basic metrics in Table 5 can be reused, while for power consumption evaluation, apart from separate evaluation for BS and UE, 6GR may also consider a metric reflecting joint BS and UE power consumption. |
| Huawei | Proposal 4: For some FTP3 traffic with delay requirement, the network energy consumption should be evaluated and compared with a given QoS satisfaction rate - QoS satisfaction rate is defined as the percentage of satisfied UEs over the total number of UEs - QoS satisfaction rate value can be FFS. Proposal 5: Besides evaluating overall power consumption considering the traffic model and QoS requirement, when evaluate the UE energy-efficiency, the following three cases focusing on specific aspects of power consumption are also to be investigated at least for calibration purpose: - only PDCCH monitoring case during UE connected state - PDSCH/PUSCH case focusing on the power consumption of data transmission or reception - only in IDLE state. |
| Tejas Network Limited | Observation 1: For joint UE and network energy efficiency computation, a baseline configuration can be defined. And the energy efficiency of the joint UE and Network energy saving scheme can be computed considering the base line scheme. - The time duration for computing the energy consumption for the baseline scheme and the energy saving scheme should be same for both UE and the base station. - Joint UE and network energy consumption is sum of the UE energy consumption and base station energy consumption. - FFS: Any other metrics to report. Example: Latency, UPT, scheduling delay. |
| Samsung | Proposal 1: For 6GR energy efficiency evaluation, at least loaded case and connected mode, daily averaged ESG/UPT shall be derived to compare energy efficiency performance. - Daily averaged ESG/UPT is calculated by scaling ESG/UPT for each traffic load by time ratio of each traffic load in daily traffic patterns - FFS: the time ratio of each traffic load in daily traffic patterns Proposal 2: For evaluation purposes and relative comparison of different candidate energy saving schemes for 6GR, NR Rel-15 basic functionalities shall be used as baseline energy saving configuration for both network and UE. |
| ZTE Corporation | Proposal 16: The load scenario defined in Table 2-9 can be considered as a starting point. Observation 2: Considering the available DL/UL resources can define the resource load reasonably. Observation 3: Traffic load is a statistical parameter measured on the cell side, instead of UE's perspective. Proposal 17: FTP traffic can be used as the starting point for the evaluation of the 6G energy efficiency. Observation 4: The energy efficiency metric based on average data rate may not be used for empty load cases and no data rate optimization cases. Observation 5: One of the EE gain directions would be data rate improvement, including higher modulation order, coding and MIMO, instead of energy saving techniques. |
| Ericsson | Observation 8: According to a dataset with more than 100 live networks, the average NR cell load over 15-minute sampled periods is as follows: 21.0% of the sampled periods have load below 1%, 33.2% of the sampled periods have load below 3%, 40.8% of the sampled periods have load below 5%, 53.2% of the sampled periods have load below 10%, 61.2% of the sampled periods have load below 15%, 66.9% of the sampled periods have load below 20%, 74.6% of the sampled periods have load below 30%. Observation 9: Considering the traffic data presented in Observation 8, time-domain energy savings techniques have great potential for NES. Proposal 11: Adopt NR Rel-15 as baseline for 6GR network and UE energy efficiency evaluations. Proposal 12: Update the BS load level definitions to the following: Empty load: L = 0%, Low load: 0 < L ≤ 5%, Light load: 5 < L ≤ 15%, Medium load: 15 < L ≤ 35% |
| LG Electronics | Proposal #1: Performance metrics for 6GR EE evaluation for a selected load case are compared to baseline BS/UE settings for the same load case. |
| HONOR | Observation1: Due to the late introduction of network energy-saving features, the design is conservative and the prospects for commercial deployment are not ideal. Observation2: The energy-saving features were designed independently, therefore, an optimal network energy-saving solution cannot be provided. Observation 3: Network energy-saving features usually affect the performance experience of the UE. Observation 4: There is overlap between features, leading to design complexity and a significant investment of effort. Observation 5: The design of UE energy-saving features did not fully consider the impact on network energy-saving, thus leading to an improvement in UE energy-saving affecting the effectiveness of network energy-saving. Observation 9: The terminal experience plays a crucial role in the commercialization of 6G, especially in the early stages. Proposal 1: The initial version of 6G needs to incorporate energy-saving factors into the design issues that need to be addressed. Proposal 2：Consider the design of energy-saving features as a whole to avoid overlapping and redundancy between features. Proposal 3: Joint consideration and design of UE energy saving and network energy saving to avoid mutual interference that prevents both from functioning simultaneously. Proposal 4: The energy-saving project can be researched according to the following steps: - First, conduct a comprehensive analysis of various energy-saving fields, prioritize them, and select the fields with high energy-saving gains for research. - Second, analyze the high-priority fields, consider all factors within these fields comprehensively, and provide the optimal solution. - Finally, conduct a comprehensive analysis of the solutions for each field and present an optimal overall energy-saving solution. Proposal 5: The design of energy-saving should not affect the performance experience of the UE. |
| Ofinno | Observation 1: The prior energy efficiency studies performed by 3GPP have different evaluation assumptions and methodologies as well as varying baselines. Observation 2: New spectrum for 6GR should be considered as part of the energy efficiency study. Proposal 7: RAN1 should discuss evaluation assumptions for energy efficacy and strive for common evaluation assumptions (e.g., baseline setting(s)) for all energy efficiency items (e.g., both NES and UE power saving). |
| Hanbat National University | Proposal 8: Adopt the power model frameworks of TR 38.864 (gNB) and TR 38.840 (UE). Initiate a study to define updated 6G-specific parameters and scaling rules (e.g., for wider bandwidths). Proposal 9: Define primary metrics: 1) Network EE as Throughput/Power (bits/Joule) at various traffic loads, and 2) UE EC as Average Power (mW) in RRC_IDLE, INACTIVE, and CONNECTED states. Proposal 10: Define the baseline for 6G EE evaluation as an NR Rel-17 system with standard power saving. The 6G scheme under test shall be the baseline plus the single new 6G feature being evaluated. |
| Sony | Proposal 1: Optimization methods for energy efficiency between the network and UE should be studied based on the differences in NES and UEPS requirement levels according to the use case, network deployment, and UE type. Proposal 2: Capture Figure 1 in the TR exhibiting different energy saving and power saving states and transition between these states. |
| Apple | Observation 1 -- Lessons learned from NR UE power-saving features: Numerous UE-side power-saving mechanisms have been specified since Release-15. Yet, most remain either un-deployed or under-utilized because: - They were added late in the standard, resulting in low device penetration in the market; - Networks have little incentive to enable features that benefit only the UE, as they can increase network complexity and degrade network performance. Observation 2: NES enhancements started in Rel-18, however, due to backward compatibility issues, techniques in 5G has limitations considering legacy UEs, which limits the energy saving gain. This is especially true for any enhancements that affect the common signals/channels. Observation 3: On the other hand, NES features have large impact on UE, may increase UE complexity, impact UE time/sync performance, increase latency and/or memory under different techniques. Proposal 1: 6G NES features should be well justified considering impact to UE. Proposal 4: Loaded scenarios should be the main scenarios for evaluations. |
| MediaTek Inc | Proposal 1: For sustainability, define the energy efficiency metric as: EE ≜ (Average data rate (bits/sec))/(Average power consumption (Joule/sec or an equivalent unit)) Proposal 2: To study and compare different energy saving schemes, define energy efficiency gain metrics for BS or UE as: EER_BS or UE (EE Ratio for BS or UE) = (EE of applying given energy saving scheme(s) to BS or UE)/(EE of the baseline BS or UE setting); EEG_BS or UE (EE Gain for BS or UE; %) = (EER_BS or UE - 1) × 100 Proposal 3: To quantify joint EE improvement for both BS and UE, define the following joint EE metrics: EER_Joint (Joint EE Ratio) ≜ √(EER_BS^α · EER_UE^β), α + β = 2; EEG_Joint (Joint EE Gain; %) = (EER_Joint - 1) × 100 Proposal 4: Study energy saving schemes that lead to significant improvement in EER_Joint with (α,β) = (1, 1). Proposal 5: For 6GR energy efficiency evaluation, at least the following load cases are evaluated: Empty (L=0), low (0<L≤15%), light (15%<L≤30%), and medium (30%<L≤50%), where L represents resource utilization (RU) ratio. FFS: whether/how high/full load scenarios should be included, e.g., for IMT2030 related metrics Proposal 6: For evaluation purposes and relative comparison over different candidate energy saving schemes for 6GR, define the following baseline energy saving configurations: Baseline for BS: SSB with 20ms periodicity; SIB1 and RO, if applicable, with 20 ms periodicity; Companies can report additional configuration if justified. Baseline for UE: I-DRX ([1.28s] cycle) for idle mode; C-DRX ([160ms] cycle, on-duration [8ms], inactivity timer [20ms]) for connected mode; Companies can report additional configuration if justified. Note: The corresponding evaluation is not intended for energy efficiency comparison with 5G/NR. |
| Qualcomm Incorporated | Proposal 1: Energy efficiency should be considered across network and devices, in conjunction with user experience and network system capacity. Proposal 2: Study how to ensure successful deployment of energy-saving features in 6GR. Proposal 19: Include updated traffic models reflecting 6GR target application as baseline for evaluation, including bursty EMBB traffic and periodic video for AR/XR applications. Proposal 20: Energy efficiency studies to consider a mix of traffic types for the UE and a mix of loading states for the network. Proposal 21: 6GR network energy studies to also evaluate a scheme's reduction in total network energy, not only in a specific load state. Proposal 22: where applicable, use the following features as the baseline for energy efficiency comparison: DRX, PDCCH skipping, sparse PDCCH monitoring and SSSG switching, BWP switching, SCell activation from NR Rel-15, simultaneous activation of multiple-CCs, and Rel-18/19 LP-WUS. |
| NTT DOCOMO | Proposal 5: For joint EE evaluations, NOT support introduction of normalized joint metric. Proposal 6: In terms of whether to include Cell DTX/DRX for baseline scheme, it should be up to companies' decisions. Companies can report whether Cell DTX/DRX operation is included for their evaluation. Proposal 7: Prefer to define baseline BS configurations for SCell/capacity cell/carrier, which are different from PCell's. For SCell or capacity cell/carrier operation, the periodicity of always-on signal(s)/channel(s) can be 160 ms Proposal 15: For EE design principle: Maximize EE gain by introducing the related features in Day 1 where no NBC issue needs to be considered; EE gain should be jointly realized for both NW and UE, i.e., EE for one entity should not degrade the other entity's EE performance; Impact on system performance should be minimized by e.g., supporting immediate resumption of necessary operation. Proposal 16: The proposed direction for joint energy efficiency in 6G, design 6GR EE to achieve all of the following: Longer sleep as much as possible; Immediate/dynamic wake-up and serving once needed; No conflict between UE and NW, for EE as well as performance |
| TCL | Observation 3: The amount of information (bits) transmitted per unit of energy (Joule) as baseline parameter of energy efficiency has been defined in IMT-2020. Observation 4: Simulation results highlight how deep sleep and dynamic resource adaptation in Config B significantly cut energy use during idle or light load periods, aligning with 3GPP's defined sleep states (micro/light/deep) for NR base stations. Observation 5: Single metric like bit/J need to be complemented with other aspects, and other potential parameters may be considered for 6G study like active data energy efficiency, idle power consumption, or wake-up/transition efficiency, UPT, User Equipment Power Saving Gain, Computing Efficiency and Latency etc. Observation 6: 6G might also consider the end-to-end energy efficiency of the communication system. However, end-to-end metrics are harder to allocate and implement since operators and users manage different portions of the power budget, and business models are separate. Observation 7: Other metrics related energy efficiency could be used for evaluation, like peak energy efficiency ratio, latency energy ratio, coverage energy ratio, idle power ratio and energy power density. Proposal 2: Additional complementary metrics for 6G use cases could be considered, e.g., computing efficiency, wake-up/transition efficiency, etc. Proposal 3: Support following method for joint NW-UE energy efficiency evaluation: Option 2 (Geometric mean): EE_Joint = (EE_NW^α · EE_UE^β)^(1/(α+β)) with α+β=2. Rationale: Balanced improvement on both sides; prevents one-sided optimization; study schemes with (α,β)=(1,1) for equal weighting Proposal 4: Consider how to handle or manage the conflict between 6G EE and UPT/QoS/Latency. Proposal 5: For 6G energy efficiency evaluation, at least the following load cases are evaluated: Empty/Idle Load (L=0); Low Load (0<L≤10%); Light Load (10%<L≤30%); Medium Load (30%<L≤50%); High Load (L>50%) Proposal 6: Consider one unified metrics could be used for 6G EE evaluation by combined with peak energy efficiency ratio, latency energy ratio, coverage energy ratio, idle power ratio and energy power density, which relates to UPT, QoS and latency, etc. Proposal 7: For evaluation or study target, consider whether/how to introduce unified metrics like energy efficiency level (EEL) to define NW/UE or joint NW and UE power consumption. |

Summary and Discussion

Moderator thanks companies’ valuable inputs. To complete the evaluation methodology, the following proposal is suggested for companies’ check and comment:

Proposal 3.1.1.1 (1st round; high priority):

For evaluation purposes and relative comparison over different candidate energy saving schemes for 6GR, define the following baseline energy saving configurations:

Baseline for BS:

- SSB with 20ms periodicity

- SIB1 and RO, if applicable, with 20 ms periodicity

- Companies can report additional configuration if justified

Baseline for UE:

- I-DRX ([1.28s] cycle) for idle mode

- C-DRX ([160ms] cycle, on-duration [8ms], inactivity timer [20ms]) for connected mode

- Companies can report additional configuration if justified

Note: The corresponding evaluation is not intended for energy efficiency comparison with 5G/NR.

Companies’ views on Proposal 3.1.1.1 (1st round)

| Company | View |
| --- | --- |
| CMCC | Support the proposal with one small comment on the value of C-DRX. During Rel-18 NES and Rel-16~17 UE PS phase, we have the following table for traffic model: Therefore, the reason of changes on the duration of inactivity timer need to be clarified, or we can simply reuse the configuration on legacy. |
| Samsung | OK in general. |
| LG Electronics | OK |
| vivo | Regarding baseline for BS, we support the proposal here. Besides, SS#0, Coreset#0 and RO configuration should be also reported when evaluation. Regarding baseline for UE, C-DRX, we suggest to reuse the values in TR38.840 - For RRC connected, C-DRX cycle of 160ms, inactivity timer (100, 40) ms - FR1 Onduration:8ms - FR2 Onduration:4ms We suggest the following changes for UE baseline: - C-DRX ([160ms] cycle, on-duration [8ms, 4ms], inactivity timer [20ms100ms,40ms]) for connected mode |
| Nokia | OK |
| Ericsson | Ok in general |
| TCL | Seems okay. But for I-DRX for idle mode at UE side, other values with longer cycle should be included for LPWA devices if no eDRX considered in this proposal. |
| Lenovo | I-DRX is some how related to paging cycle and it affects both the EE and PS which maybe different for different device types e.g., e-DRX. We can add paging cycle to the Baseline BS model for EE consideration and I-DRX to the UE side. BS sends paging message to multiple UEs so its impact should be captured, however from UE point of view, UE monitors a selected paging occasion and its impact is bit different. |
| CEWiT | Fine in general |
| DCM | Support the baseline for BS. Regarding baseline for UE in connected mode, the reason for the following settings needs to be clarified - a common baseline setting across different frequency ranges, - the inactivity timer value of 20 ms which is different from value captured in TR38.840 |
| MediaTek | We are in principle fine with the proposal. A few comments are listed below. - For CDRX, to our understanding, inactive time of 20ms was agreed for R17 UE power saving evaluation thought it was not captured to TR. - We agree with vivo that detailed configuration of SSBs (e.g. how many SSBs), SIB1 (e.g. how many slots), and ROs (e.g. which preamble format and how many symbols/slots) shall be reported by companies. We observe that different power calcuation assumption (specifically about valuse of scaling factors sa and sp) would lead to different results and observation. If we cannot align the evaluation assumptions on sa and sp, we probably should agree that detailed parameters for power caculation should be reported by companies. |
| Qualcomm | Generally ok with the proposal, except for SIB1 periodicity, we propose to have it at 160ms instead of 20ms. For CDRX, we propose to also include additional cycle values, e.g. 40 and 80ms. |
| ZTE, Sanechips | If C-DRX is considered as baseline technique, does it mean LP-WUS also should operate with C-DRX? If no, we’d better to clarify ‘Baseline for UE’ only means the technique used for comparison. Additionally, if C-DRX is used as baseline scheme for comparison, with different configuration, the power saving gain would be different. For example, LP-WUS would have different power saving for different traffic and C-DRX configuration. If we only focus on one configuration, we do not think it is fair. Last, we would suggest to compare all the PS schemes based on the baseline scheme. If C-DRX with one configuration is baseline scheme, then we can not fairly compare C-DRX and other schemes, e.g., LP-WUS. Currently, we still consider whether C-DRX is an efficient PS scheme compared with others. Therefore, always-on monitoring should be the baseline scheme for comparison, and all the other schemes for EE improvement like C-DRX, LP-WUS, should be compared with always-on monitoring at the same level, to see which one is the most efficient. |
| CATT | Regarding baseline for BS, we support the proposal. Regarding baseline for UE, we support to reuse 38.840 for the I-DRX configuration and C-DRX configuration. Baseline for UE: • I-DRX ([1.28s] cycle) for idle mode − Group paging rate (for a PO): 10% - P-RNTI is detected but PDSCH decoding results in no match • C-DRX (320ms,160ms,40ms cycle, on-duration 10ms, 8ms,4ms, inactivity timer 10ms,40ms,80ms) for connected mode − Different parameters can be used to adapt to different traffic loads − Companies can report the configuration value. Companies can report additional configuration if justified |
| Apple | Similar view as CMCC on the C-DRX related parameters. For the RO, we think there is no default value for RO configuration, therefore we prefer to leave it for companies to report the configuration. |
| Sharp | OK in general. |
| Tejas | BS baseline Configuration: Detailed configuration of number of SSBs, number of SIB1 slots, and PRACH preamble needs to be reported. Fine with baseline configuration for UE. |
| Xiaomi | OK in principle. We also share the views as vivo and MTK that detailed configuration of SSBs, SIB1, and ROs shall be reported by companies. |
| Huawei, HiSilicon03 | The period of SSB/SIB1/RO channel maybe different in 6GR at least for 7GHz considering the coverage enhancement requirement where some time domain repetition seems necessary. The baseline periodicity should be determined in IA agenda. |
| NEC | Support |
| Mod2 | Moderator would like to provide some information for the DRX timer as well as traffic models being considered in evaluation agenda item: Agreements: (RAN1#102, Aug. 2020) - The performance metrics described in TR38.840 section 8.2 is reused for power saving evaluation of Rel-17 DCI-based power saving adaptation during ActiveTime. - The following Rel-15 / 16 features is recommended of the power consumption as reference for baseline. Company can report the feature(s) being used in the baseline. - DRX - C-DRX cycle 40msec for VoIP - 10ms IAT, 8ms On-duration - Assume max two packets bundled - C-DRX cycle 160msec for FTP - Alt 1: 20 msec IAT, 8ms On-duration - Alt 2: short DRX - 20 ms [or 40ms as optional] IAT, 8ms On-duration - 20 ms for short DRX cycle, 4 cycles - Note: 100 msec IAT, 8ms On-duration can also be used with sufficient justifications that available Rel-15/16 Techniques being used to reduce UE power saving (Omitted). Conclusion (RAN1#122b, Oct, 2025) The following existing traffic models could be used for 6GR performance evaluations, - Full buffer - FTP Model 1 (in TR 36.814) - FTP Model 3 (in TR 36.872) - XR Traffic models (in TR 38.838) - VoIP model (as in TR 36.814) - Instant message (as in TR 38.840) - Note that which model(s) will be used can be further decided when performing simulations in each individual topic.. With the above information, moderator would like to suggest company to consider the following updated proposal, that also try to integrate companies’ inputs: Proposal 3.1.1.1a For evaluation purposes and relative comparison over different candidate energy saving schemes for 6GR, define the following baseline energy saving configurations: Baseline for BS: - SSB with 20ms periodicity - SIB1, if available, company to report assumed periodicity from 20 ms, 160 ms - RO, if available, with 20 ms periodicity - Companies to report sa, sp and sf values for BS processing of the above signal(s)/channel(s). - Companies can evaluate and report other configuration(s) with justification Baseline for UE: - I-DRX (1.28s cycle) for idle mode - Group paging rate (for a PO): 10% - C-DRX settings of (cycle, on-duration timer, inactivity timer) are assumed for the following traffic models for connected mode: - VoIP: (40 ms, 8 ms, 10 ms) - FTP3: (160 ms, 8 ms, 20 ms) - Instant message: (320 ms, 8 ms, 40 ms) - Companies can evaluate and report other configuration(s) with justification Note: The corresponding evaluation is not intended for energy efficiency comparison with 5G/NR. |

BS power consumption model updates

General

FL comments and proposals

In FL’s understanding, there is strong consensus among companies to base 6GR BS power models on the energy consumption models, including reference configurations, power states, relative power values, transition times, transition energy, scaling methods, active DL transmission, active UL reception and load levels. This does not prohibit making changes to the models or even defining new models. As a baseline in order to save valuable time agreements, FL makes the following overarching proposal:

FL Proposal 3.2.1.1 (1st round; high priority):

As a starting point, the BS energy consumption model and load level scenarios from TS 38.864 also applies to 6GR EE model.

FL comment after 1st round:

FL’s conclusion based on companies’ comments below: the intention is to have a general agreement on model aspects that we may not have time to discuss. However, vivo makes a good point, RAN1 has agreed to update current models, implicitly agreeing to use them as a starting point. This proposal is downprioritized.

Companies’ comments

Companies’ comments on the above proposal are appreciated.

| Company | Comment |
| --- | --- |
| CMCC | For energy consumption model, - We believe that the current 2 categories are belong to different types of implementation that both exist in current NR network. Therefore, directly reuse NR power model to evaluate NES gain for 6G technique does not reflect all situations, especially considering the upgrading of base station equipment. - From our point view, at least the value of transition time (especially for Cat 2) should be revisited considering it will strongly impact the NES gain and design on default always-on SS periodicity in 6GR. For load level, we can fine with reusing the legacy one. |
| Samsung | Support |
| LG Electronics | OK |
| vivo | We think there is no need to discuss this proposal. According to the plan, this is the last meeting to finalize 6GR EE power model so it seems spending time to discuss starting point is not necessary. Almost all companies propose the update based on that from TS 38.864 since our previous agreements below imply this already: Agreement Study how to reuse and update reference configurations in TR 38.864 for 6G BS. Agreement Study how/whether to reuse or update the power model in TR 38.864 for evaluating BS power consumption for 6G BS. |
| Nokia | OK |
| Ericsson | We support the CAT1 BS energy consumption model. For load (treated in FL Proposal 3.2.6.1), we think it should be updated based on real data sets. We propose the following: Empty load: L = 0%, Low load: 0 < L ≤ 5%, Light load: 5 < L ≤ 15%, Medium load: 15 < L ≤ 35%, High load: L>35%. |
| TCL | Support |
| Lenovo | We also agree with vivo that this proposal is redundant and doesn’t add any value. |
| CEWiT | Support |
| DCM | If this proposal is about supporting the structure of BS energy consumption model similar to what is provided in TR38.864 (i.e., BS reference configuration/power states/relative power values/total transition time/additional transition energy/scaling), we can support such direction, but we think we have already had agreement in the 122 meeting, so we do not think this is necessary. If the proposal is about reusing all the values in TR38.864 for existing sets, then we would like to propose revisiting the relative power values at least for Active DL. We think that the relative power of active DL is too underestimated. For the case of active DL power of existing sets, at least we should update the set 1 FR1 value for the following analysis. Based on our understanding, power it takes for DL transmission from NW can be determined based on the ratio of actual and maximum number of TxRUs for antenna part, and the ratio between actual and maximum DL transmission power for PA (dynamic joint) part. As each set (i.e., set 1/2/3/4) has different maximum number of TxRUs and maximum DL transmission power, we should take into account for active DL at least for FR1 and FR3. We think that following parameter value update should be taken into account based on the following compensated active DL power; P4updated=P3+P4-P3×A×TxRUstotalset1TxRUstotalset2+(1-A)×DLPWtotalset1DLPWtotalset2, where, TxRUstotalset1 or set2 is total number of TxRUs for set 1 or set 2 and DLPWtotalset1 or set2 are the total DL power level for set 1 or set 2, which are both captured in Table 5.1-1 in TR38.864. Also, A can be 0.4 as specified in TR as baseline. Based on the above calculation, the following adjustment can be made. Power state; Set 1. CAT 1 BS; CAT 2 BS. Deep sleep; 1; 1. Light sleep; 25; 2.1. Micro sleep; 55; 5.5. Active DL; 775; 90.3. Active UL; 110; 6.5. |
| MediaTek | Not sure the implication if this proposal is agreed. Does it mean that we do not need to discuss proposals in 3.2.3-3.2.6 if this proposal is agreed? Or does it mean that 6GR energy efficiency evaluation would consider any agreed updates/models from 3.2.3-3.2.6 on top of the BS power models from TR 38.864? |
| Qualcomm | We are generally ok with the proposal, but have the same question as MediaTek |
| Huawei, HiSilicon | We would like to address the other proposals first. |
| ZTE, Sanechips | In 6G day1, with progress in manufacturing technology, there is a chance to reconsider the power model which could more accurately reflect the 6G system characteristics. Considering the 6G BS implementation will be more energy efficient than NR, at least the the transition time of light and deep sleep state of BS Cat 2 should be reduced. For the load level, we could be fine with the legacy one and also open to some updates e.g., Ericsson’s suggestion. |
| CATT | We support the proposal. |
| Apple | Support |
| Sharp | Support |
| Tejas | Need clarification on “the BS energy consumption model from TR 38.864 also applies to 6GR EE model.”. Does it mean that we will be reusing the power states, reference configuration sets, Relative power values P, transition time T and Additional transition energy E defined in TR 38.864 for evaluating the 6G energy efficiency consumption? If yes, we don’t support this proposal. The transition time T, defined for cat2 base station (Assuming this proposal support both cat and cat2 BS) needs to be revisited. |
| Xiaomi | We understand that this proposal does not seem to facilitate discussions on other proposals(we still need to check them one by one), nor does it affect the subsequent discussions of other proposals. So, is this proposal not particularly necessary? |
| OPPO | OK for NW. For UE side, suggest simple reuse the NR configuration for power saving. |

Reference configuration sets

Companies’ views

RAN1 agreed to expand the existing BS power model reference configuration with Set 4 for ∼7 GHz operation, but some parameters were initially bracketed for confirmation. Companies largely focused their proposals on fixing the values for BW, SCS, and the Total number of DL TX/UL Rx RUs, while also confirming the Total DL power level. While consensus exists that the properties will be TDD with 1 TRP, the specific numerical selections vary across delegates. A common trend is the anticipation of larger array sizes (128-512) to compensate for higher frequency path loss in this range. FUTUREWEI suggested confirming 400 MHz BW, 30 kHz SCS, and 512 DL/UL TX/Rx RUs for Set 4, while also proposing considering an additional Set 5 for more moderate RU counts like 128 or 64. MediaTek and OPPO also proposed confirmation of 400 MHz BW and 256 TX/Rx RUs. However, several other companies proposed slightly more restrained baselines, such as 100 MHz BW, which CATT, Ericsson, NTT, ZTE, and Apple favored as the default, with 200 MHz or 400 MHz considered optional. For antenna configuration, 128 TX/Rx RUs was suggested as the baseline by CATT and Ericsson, whereas Lenovo, ZTE, MediaTek, Xiaomi, and OPPO proposed or supported confirming 256 TX/Rx RUs. The Total DL power level consensus is strong, with 56 dBm being the most frequently confirmed or proposed value. TCL/ITRI included 62 dBm as an optional value for DL power level.

While RAN1 agreed to expand the existing BS power model reference configuration with Set 4, specific relative power values for both CAT 1 and CAT 2 were proposed by several companies, for CAT 2 sometimes tied to an updated category or adjusting existing values.

FL comments and proposals

In RAN1 #122-bis, RAN1 made the following agreement:

Agreement

For evaluation purposes, expand the existing BS power model reference configuration with a set for ~7 GHz operation with the following parameters:

| Property | Configuration for Set 4 around 7 GHz |
| --- | --- |
| Duplex | TDD |
| BW | [100, 200, 400] MHz |
| SCS | [30 kHz, 60 kHz] |
| Number of TRP | 1 |
| Total number of DL TX RUs | [128, 256] |
| Total DL power level | [56] dBm |
| Total number of UL Rx RUs | [128, 256] |

Note: Bracketed values to be confirmed. Other values are not precluded.

The above configuration has no implication on supported BW, SCS for 6GR.

To synthesize the most common proposals for Set 4, the following table presents an “average configuration” based on the input from companies regarding the outstanding bracketed parameters (in red):

Table 1: Reference configuration for BS Set 4 (~7 GHz)

|  | Set 4 around 7 GHz |
| --- | --- |
| Duplex | TDD |
| System BW | 200 MHz |
| SCS | 30 kHz |
| Number of TRP | 1 |
| Total number of DL TX RUs | 128 |
| Total DL power level | 56 dBm |
| Total number of UL Rx RUs | 128 |

In FL’s view, in order to get a complete agreement, BS Sets 1,2,3 should also be included. That results in the following FL Proposal:

FL Proposal 3.2.2.1 (1st round; high priority):

The following reference configuration for BS power consumption is defined:

|  | Set 1 FR1 | Set 2 FR1 | Set 3 FR2 | Set 4 around 7 GHz |
| --- | --- | --- | --- | --- |
| Duplex | TDD | FDD | TDD | TDD |
| System BW | 100 MHz | 20 MHz | 100 MHz | 100 MHz |
| SCS | 30 kHz | 15 kHz | 120 kHz | 30 kHz |
| Number of TRP | 1 | 1 | 1 | 1 |
| Total number of DL TX Rus | 64 | 32 | 2 | 256 |
| Total DL power level | 55 dBm | 49 dBm | 33 dBm* | 56 dBm |
| Total number of UL Rx Rus | 64 | 32 | 2 | 256 |
| *Note: EIRP limit is 63 dBm for the reference configuration. The EIRP value is scaled with the number of TxRUs. |  |  |  |  |

FL comment after 1st round:

FL has updated the above table according to companies’ comments. It is an even race between 100 MHz and 200 MHz but considering a few companies prefers 400 MHz, 200 MHz is proposed. Regarding #antenna elements, companies seem to prefer 128. That results in the below updated proposal:

FL Proposal 3.2.2.1b (1st round; high priority):

The following reference configuration for BS power consumption is defined:

|  | Set 1 FR1 | Set 2 FR1 | Set 3 FR2 | Set 4 around 7[, 15]** GHz |
| --- | --- | --- | --- | --- |
| Duplex | TDD | FDD | TDD | TDD |
| System BW | 100 MHz | 20 MHz | 100 MHz | 100 MHz |
| SCS | 30 kHz | 15 kHz | 120 kHz | 30 kHz |
| Number of TRP | 1 | 1 | 1 | 1 |
| Total number of DL TX Rus | 64 | 32 | 2 | 256 |
| Total DL power level | 55 dBm | 49 dBm | 33 dBm* | 59 dBm |
| Total number of UL Rx Rus | 64 | 32 | 2 | 256 |
| *Note: EIRP limit is 63 dBm for the reference configuration. The EIRP value is scaled with the number of TxRUs. **Pending agreement in 11.2 to evaluate 15 GHz. |  |  |  |  |

Provided the above proposal is agreed, FL furthermore proposes the following power state values for Set 4 based on averaging of companies proposed values, even if FL acknowledges that these may have been provided for a different configuration of Set 4:

FL Proposal 3.2.2.2 (1st round; high priority):

The following relative power levels applies to Set 4 for a CAT 1 BS and a CAT 2 BS:

| Power state | Set 4 |  |
| --- | --- | --- |
|  | CAT 1 BS | CAT 2 BS |
| Deep sleep | 1 | 1 |
| Light sleep | 25 | 2.2 |
| Micro sleep | 58 | 5.4 |
| Active DL | 440 | 45 |
| Active UL | 130 | 7.8 |

FL comment after 1st round

Companies have very differing views on power consumption, approximately a 6 dB range for Active DL/UL. More discussion may be needed but no need to update the proposal for that. Remaining values seem agreeable to companies.

Companies’ comments

Companies’ comments on the above proposals are appreciated.

| Company | Comment |
| --- | --- |
| CMCC | We prefer 200 MHz as the default System BW for Set 4. |
| Samsung | 200MHz for set 4 |
| LG Electronics | OK |
| vivo | Regarding the first proposal, we have the following comments: 1st comment: We suggest not to use FR1, FR2 terminology now since it is not clear now for 6GR. Similar with other agendas, we use spectrum range explicitly, i.e., Sub-6GHz to replace FR1, 24.25-52.6GHz to replace FR2 2nd comment: Regarding system bandwidth, we prefer 400MHz as the reference configuration since 400MHz is already agreed at least for network side. If using 100MHz as reference configuration, up scaling is needed 3rd comment: Regarding DL power level, we suggest to put in bracket at this moment until there is agreement in 11.2 agenda. Regarding the second proposal, we suggest to discuss this after reference configuration value for Set 4 is settled. |
| Nokia | Regarding FL Proposal 3.2.2.1, the controversial parts are system bandwidth and number of DL/UL Tx/Rx RUs. Before agreeing on these two values, we need to understand whether or not the legacy scaling factor would work well with scaling-up and scaling-down, e.g. is it applicable for the scaling up from 100MHz to 200/400MHz? Regarding FL Proposal 3.2.2.2, we want to understand where/how do we get these values from? Are the proposed values the averaging values among companies proposed values |
| Spreadtrum | We prefer 200MHz BW for Set 4. |
| Ericsson | 3.2.2.1: OK. We have a slight preference for 128 TX/RX RUs (set 4) 3.2.2.2: For CAT 1 BS, Active DL is too high, we propose: Active DL 320 In TR 38.864, same Light/Deep sleep power values are used for all sets. This should be applied to Set 4 as well, thus the Light sleep value for CAT2 should be 2.1 (as in Set 1-3). |
| TCL | - For set 4 parameters configuration in FL proposal 3.2.2.1, we support the baseline setting. But for total DL power level, to maintain range, the maximum EIRP may need to be ~3 dB higher for 7–8 GHz cells than for 3.5 GHz cells if considering 100MHz system BW(but as most companies’ comment, we also prefer 200MHz system BW). The Set 4 power level of 56 dBm is a baseline that, when paired with a 256-element array, yields an EIRP on the order of 59 dBm – in line with regulatory limits for macro base stations. Thus, we think [56, 59]dBm should be carefully down-selected. - For FL proposal 3.2.2.2, it seems too early for discussion due to no agreement to support which BS category (Cat.1 or Cat. 2 or both or new Cat.) to be used for EE evaluation. Thus, we suggest to postpone discuss this proposal until BS category being decided. |
| Lenovo | Transition time is missing in the Proposal 3.2.2.2 ? |
| DCM | For reference configuration, we support the values for set4. For the relative power values, we think that the relative power value for active DL should be larger than what is proposed from the FL proposal. As we see that the total number of TxRUs are 4 times larger than what is listed in set1. Considering that factor, we think that relative power value for active DL power should be at least 4 times larger than what is listed in set 1 (i.e., 1120 for cat. 1 and 128 for cat. 2). Power state; Set 4. CAT 1 BS; CAT 2 BS. Deep sleep; 1; 1. Light sleep; 25; 2.2. Micro sleep; 58; 5.4. Active DL; 1120; 128. Active UL; 130; 7.8. |
| MediaTek | For Set 4, we also propose 200MHz BW. |
| Qualcomm | We are ok with the proposals. |
| ZTE, Sanechips | Support the reference configuration in FL Proposal 3.2.2.1. And the note ‘The above configuration has no implication on supported BW, SCS for 6GR.’ from previous agreement should also be copied here. For FL Proposal 3.2.2.2, we are open to discuss the relative power values. It is noted that for the power values of cat 2 in FL Proposal 3.2.2.2, the power value of light sleep state is greater than that of value of set 1, while the power value of micro sleep state is less than that of value of set 1, which does not conform to the actual situation. |
| CATT | For FL Proposal 3.2.2.1, we prefer 128Tx RUs and Rx RUs as the default System Tx RUs and Rx RUs for Set 4. For FL Proposal 3.2.2.2, the reference configuration should be discussed firstly. And the principle of determination for power consumption values should be discussed before the power consumption values discussion. |
| Apple | Support using 100MHz as the system BW for reference configuration. For TXRU, we prefer 128. |
| Tejas | Active DL power for set 4 reference configurations should be essentially high at least by 4 times as compared to the reference configuration set 1. |
| Xiaomi | 200MHz for set 4 |
| OPPO | We are ok for proposal 3.2.2.1, we think that for the evaluation purpose, this proposal is ok. But if companies want to change the system BW to 200MHz, we are also fine. For proposal 3.2.2.2, we have a general comment. From the suggested power unit values, it seems that Cat 2 BS is already a low power consumption BS. For such BS, does it still have power saving issue ? or why 6G NES should consider such BS ? We would like to hear opinions from the proponents. |
| Huawei, HiSilicon03 | We prefer 200/400M BW with the following power values for 3.2.2.2 Power state; Set 4. CAT 1 BS; CAT 2 BS; Huawei. Deep sleep; 1; 1; 1. Light sleep; 25; 2.2; 4.5. Micro sleep; 58; 5.4; 10. Active DL; 440; 45; 50. Active UL; 130; 7.8; 12. |
| Futurewei | We prefer 400MHz as the system BW for set 4 or to at least consider upscaling. |
| Vodafone | We would prefer to include an additional reference configuration for SBFD base station and study any updates to reflect SBFD operation. In our contribution R1-2509399, co-signed by multiple operators there is clear interest to have a comprehensive study of this technology so we do not understand why it was not reflect in any way in this feature lead summary. |

BS model categories

Companies’ views

RAN1 agreed to study whether to downselect between CAT 1 and CAT 2 BS models, or if new categories should be introduced. Several companies explicitly proposed continuing to use both existing categories or defining a new one: Samsung, Tejas, and NTT proposed maintaining both CAT 1 and CAT 2 to reflect the diversity in realistic deployments and current versus future network capabilities. Ericsson favors using CAT 1 as the baseline because its faster transition times (6 ms for light sleep, 50 ms for deep sleep) enable better modeling of advanced networks that can switch dynamically into energy-saving states, reflecting anticipated 6G hardware trends. Conversely, Nokia, MediaTek, and Apple favor using CAT 2 as the baseline, arguing that it better reflects the state-of-the-art or currently deployed BS hardware. Meanwhile, vivo and Xiaomi advocated for adopting a single, energy-efficient BS power model for 6GR NES study to ensure focused evaluation, while Lenovo and HONOR proposed introducing a new category specifically for the ~7GHz frequency range or recommending a new category with optimized values.

FL comments and proposals

In FL’s view, it is desirable for future work with a single model, but such a model must also reasonably reflect reality. Considering this, it seems like companies think CAT 2 is pessimistic in a 10-year timeframe whereas CAT 1 is optimistic for the near future. Due to polarized views, selecting one of the two as the baseline, even if slightly updated, seems futile. Before proceeding, FL would appreciate companies’ preferences with a poll:

FL Poll 3.2.3.1 (1st round; high priority):

What are companies’ preference for future BS model categories among the following alternatives?

- Only use CAT 1 for future EE assessments

- Only use CAT 2 (possibly modified) for future EE assessments

- Use both CAT 1 and CAT 2 (possibly modified) for future EE assessments

- Define a new model in-between CAT 1 and CAT 2.

FL comment after 1st round:

Based on companies’ comments, it is clear that only the option with CAT1 + CAT2 is agreeable. Regarding the interpretation of “both”, it is not FL’s intention to change the existing practice, i.e., companies may provide results based on either CAT1 or CAT2. How this will be used in future work is up to that AI to decide, i.e., whether to require power saving technologies to provide benefits for both or only for one of the BS categories.

FL Conclusion 3.2.3.1 (1st round; high priority):

There is no consensus in RAN1 to downselect betweenCAT1 and CAT2 BS power models.

Companies’ comments

Companies’ comments on the above poll are appreciated. Use color-coding (green, yellow, red) as exemplified on the first line for Example, Inc., that prefers CAT 2, can accept both models and a new model in-between but cannot accept only CAT 1.

| Company | CAT 1 only | CAT 2 only | CAT 1 + CAT 2 | New model |
| --- | --- | --- | --- | --- |
| Example, Inc. |  |  |  |  |
| CMCC |  |  |  |  |
| Samsung |  |  |  |  |
| LG Electronics |  |  |  |  |
| vivo |  |  |  |  |
| Nokia |  |  | We propose to modify this column as “CAT1 or CAT2” instead. As discussed, practically different companies have different preferences of 6G hardware choices, either CAT1 or CAT2 shall be considered for future EE assessments based on companies choices, and companies may provide the simulation results based on their preferred BS category. For NOKIA, we proposed CAT2-plus with 6G hardware improvement over CAT2, as shown in our Tdoc. We will focus on providing the simulation results for CAT2-plus only. |  |
| Spreadtrum |  |  |  |  |
| Ericsson |  |  |  |  |
| TCL |  |  |  |  |
| CEWiT |  |  |  |  |
| DCM |  |  |  |  |
| MediaTek |  |  |  |  |
| Qualcomm | No | OK (1st preference) | OK | No, unless another model is introduced for an enhanced CAT1 |
| ZTE, Sanechip |  |  |  |  |
| Apple |  |  |  |  |
| Sharp |  |  |  |  |
| Tejas |  |  |  |  |
| Xiaomi |  |  |  |  |
| OPPO |  |  |  |  |

CAT 2 model updates

Companies’ views

Several companies support adopting the existing 5G CAT 2 BS model as the baseline for 6GR NES evaluation because it better reflects state-of-the-art and currently deployed BS hardware, there is a wide consensus that the legacy CAT 2 model requires significant modification to reflect anticipated 6G hardware advancements and support new features. The long transition times characteristic of CAT 2 (e.g., 640ms for light sleep, 10s for deep sleep) are noted to severely limit NES gains and render the model unsuitable for supporting critical low-latency mechanisms like UL WUS. To address this, Nokia proposed defining a new category, "Category 2-plus," which specifically incorporates hardware advances by suggesting improvements over 5G CAT 2, such as reducing the transition time for light sleep from 640ms to 100ms and deep sleep from 10s to 5s. Furthermore, Nokia proposed a general improvement in power efficiency, including an e.g., 10% improvement of the power level of deep sleep compared to 5G CAT 2. Similarly, ZTE proposed using a 6G model that reduces CAT 2 transition times significantly to 100ms for light sleep and [1s] for deep sleep, while Tejas and CATT also explicitly proposed that if CAT 2 is utilized, its existing transition latency should be reduced to reflect 6G capability enhancements.

FL comments and proposals

Companies find legacy CAT 2 slightly outdated and for that reason propose updates to it. Without touching the question whether CAT 1 or CAT 2 should be selected, it is FL understanding that an update of CAT 2 is supported by companies. For that reason, the following is proposed:

FL Proposal 3.2.4.1 (1st round; high priority):

If selected as a 6GR BS model, CAT 2 is updated to CAT 2.1 with the following updates regarding,

  - Total transition time T for reference configuration Set 1, Set 2, Set 3 and Set 4.

| Power state | BS Category 2.1 |
| --- | --- |
| Deep sleep | [2] s |
| Light sleep | [100] ms |

  - Additional transition energy E for reference configuration Set 1, Set 2, Set 3 and Set 4.

| Power state | BS Category 2.1 |
| --- | --- |
| Deep sleep | 3400 |
| Light sleep | 170 |

FL comment after 1st round:

In FL’s view, if these updates are agreed, CAT2.1 replaces CAT2 in evaluations with the understanding that CAT2 is an NR version and CAT2.1 is a 6GR version of the same architecture. In this sense, CAT1 can (reasonably) be considered to have been more future proof than CAT2. Transition times are agreed in a first step, and transition energy can be adjusted in a second step. No update to proposal is needed.

Companies’ comments

Companies’ comments on the above proposal are appreciated.

| Company | Comment |
| --- | --- |
| CMCC | Support the proposal |
| Samsung | Before discuss this proposal, we need to conclude for the proposal 3.2.2.1. |
| LG Electronics | We have several comments for clarification: - If Cat 2.1 is agreed, is Cat 2 replaced with Cat 2.1? Or, are there three categories for BS evaluation (i.e., Cat 1, Cat 2, and Cat 2.1)? - If transition time energy is updated for Cat 2, we think relative power value for power state (e.g., micro sleep, light sleep) also needs to be updated. |
| vivo | We think the transition time should be determined first. For transition energy, it depends on the outcome of power value and transition time. For the proposed transition time of modified Cat 2, we prefer to improve it a bit further since this model aims to be used for more than 10 years for 6GR base station. |
| Nokia | OK |
| Ericsson | OK. It should be up to the proponents of CAT2 to propose updates of CAT2 that match their product plans. Regarding transition energy E: The relative power values for micro-sleep and light-sleep must first be set before we can decide about the transition energies. (The transition energy is essentially a function of the transition time and the relative power levels.) |
| TCL | Generally agree with this proposal. In our understanding, CAT 2.1 here might represent more advanced hardware than state-of-the-art deployed BS hardware but values need to be carefully considered especially for transition energy E above listed. |
| Lenovo | Ok |
| DCM | We can support this proposal. |
| MediaTek | We share a similar view with vivo. Transition time has a huge impact on time-domain enhancements of extending default SSB periods. Hence, for Cat2.1, we would like to echo vivo that proponents can consider more advanced transition time. |
| Qualcomm | We do not support the proposal. It is not clear how the enhanced value was derived and why only Cat 2 could be improved but not Cat 1 |
| ZTE, Sanechips | Support the proposal. Considering the 6G BS implementation will be more advanced, the transition time and the additional transition energy should be reduced. 100ms for light sleep, and 1-3s for deep sleep are in line with the actual situation of 6GR. And the additional transition energy can be calculated according to the transition time. To LG, the relationship among CAT2.1, CAT2, CAT 1 discussed in FL Poll 3.2.2.1 (1st round; high priority) |
| Apple | We think the current Cat 2 BS model represents a starting point of the BS capability which is important to understand the NES gain of proposed schemes based on the current 5G BS capability at the beginning of 6G era. If updates are needed, it is proposed to consider a Cat 3 BS instead. |
| Sharp | The same view with LG. |
| Tejas | Support |
| Xiaomi | Similar comments as vivo and MTK. |
| OPPO | Please see our questions to proposal 3.2.2.2. |

Scaling rules updates for BS operating with reduced capabilities (LPR/WUR/LP mode)

Companies’ views

The introduction of a BS operating with reduced capabilities regarding BW and #antenna elements in use, also referred to as a BS LPR, BS WUR or BS LP mode is proposed as a key enabler for NES, particularly during low/no load scenarios. The applicability of the LP mode centers on handling essential but lightweight control and access signals, primarily acting as a receiver (LP-RX) for PRACH or UL WUS sequences. It is also proposed to function as a transmitter (LP-TX) for "light Sync signals" or DL WUS. A common denominator is that the sequences may be determined in advance, thereby reducing the complexity for the BS, in particular the BB. For power state modeling, companies including FUTUREWEI, CATT, vivo, and TCL/ITRI proposed introducing new power states specifically to reflect LPR operation, such as "Active UL using low power state" for UL WUS reception or "Active DL using low power state" for light sync transmission. TCL/ITRI proposed a new Active LPR state with a relative power value of [1.1]. For antenna configuration, the LP mode is designed to use a smaller set of active antennas or TX/RX RUs than the MR, sometimes using ultra-narrow bandwidths (e.g., 5MHz/10MHz). This reduced hardware use is reflected in proposals by MediaTek, which suggest applying an additional scaling factor (s’a ∈ {1/2, 1/3}) on top of existing scaling factors for LPR power consumption. Huawei proposes to introduce scaling for static power regarding #antenna elements and bandwidth, Pstatic{D,U}L=sa*sf*P3. Crucially, the LP mode’s interaction with the MR introduces trade-offs: detection of a request (like UL WUS) triggers the MR wake-up, leading to an extended UE radio access delay because the MR must transition from deep sleep to active status, which takes approximately 40 ms for a CAT 1 BS. A major challenge highlighted by Ericsson is that DL/UL link performance (specifically for SSB, SIB1, and PRACH) must be equivalent to the MR to maintain consistent cell coverage and uplink RX sensitivity, otherwise the UE would have to transmit at higher power. Because meeting these constraints requires substantial parts of the radio (like PAs and local oscillators) to remain semi-active, the actual energy savings opportunities are questioned. Furthermore, cost aspects are a concern, as an LP mode requiring additional hardware increases costs for operators, though some companies argue the LP mode can be achieved by operating a subset of the MR hardware. Due to these unknowns, Ericsson explicitly proposed not updating the BS power model to include a LP mode unless a common understanding of its functionality can be reached.

FL comments and proposals

Regarding LPR, RAN1 was close to agreeing on the following proposal in the last meeting:

Study and evaluate BS aspects for NES regarding common signal transmission at least for initial access and the impact on UE performance and user experience, including, e.g.,

Power domain, e.g., Tx power,

Frequency domain, e.g., BW,

Spatial domain, e.g., #antennas,

Sleep state transitions.

It is FL’s view that the above agreement is not sufficiently complete for inclusion as is in the BS model. Since this is the last meeting for AI 11.5 EE, a more complete agreement is needed if the LP mode is to be supported. TS 38.864 already includes scaling for both BW and #antennas but further scaling could be justified since functionality would be limited to receiving or transmitting sequences, i.e., previously determined signals, e.g., sync, PBCH, PRACH. However, even a single active antenna requires substantial RF functionality up and running, why, in FL’s opinion, a linear scaling with, e.g., BW or #antennas is unreasonable. In lack of a better solution (offline discussions are particularly encouraged in this matter), it is assumed that 50 % is fixed and 50% is scalable of PstaticDL for a BS operating in a LP mode.

It is furthermore FL’s understanding that the remainder of the antenna elements and also the BB will assume some sleep mode. Consequently, there must also be some transition time and energy assumptions for a BS transitioning to/from its LP mode of operation. Considering parts of the BS are active, in FL’s view, that value should be closer to waking up from light sleep than deep sleep. For that reason, FL proposes:

FL Proposal 3.2.5.1 (1st round; high priority):

A BS LP mode includes transmission and reception of predetermined deterministic sequences. The following scaling methods are used to allow modeling of a BS operating in a LP mode:

PDL=PstaticDL+PdynamicDLPstaticDL=1+sa*sf2P3PdynamicDL=sa*Pdyn,ante+sf*spηsf,  sp*Pdyn,jointPUL=PstaticUL+PdynamicULPstaticUL=1+sa*sf2P3PdynamicUL=sa*Pdyn,anteULPdyn,anteUL=P5-P3

The same transition time and energy values as for light sleep are used for BS transitioning to/from its LP mode.

FL comment after 1st round:

This is a controversial topic that may need more discussion. In FL’s view, it is reasonable to modify the static power considering the reduced functionality. How it should be done can be discussed, FL proposal above use a 50/50 split, whereas Huawei in their comments below propose a more scalable approach:

1-sa*Pi for remaining partsa*P3 for LP transmission

In FL’s view, Huawei’s update provides a clearer relation between the different states and also naturally allows for different implementations, e.g., sleep levels of remaining HW. FL proposal is updated to reflect this.

FL Proposal 3.2.5.1 (1st round; high priority):

A BS LP mode includes transmission and reception of predetermined deterministic sequences. The following scaling methods are used to allow modeling of a BS operating in a LP mode:

PDL=PstaticDL+PdynamicDLPstaticDL=1+sa*sf2P3PdynamicDL=sa*Pdyn,ante+sf*spηsf,  sp*Pdyn,jointPUL=PstaticUL+PdynamicULPstaticUL=sa*P3+1-sa*PiPdynamicUL=sa*Pdyn,anteULPdyn,anteUL=P5-P3

where Pi is the relative power values (P1 or P2) that the remaining HW is assumed to be in. The same transition time and energy values (correspondingly scaled (1-sa) as for the assumed power state applies for BS transitioning to/from its LP mode.

Companies’ comments

Companies’ comments on the above proposal are appreciated.

| Company | Comment |
| --- | --- |
| CMCC | Fine with the structure. |
| Samsung | This structure is fine. Even in LP mode, it is preferred to define the power within the same framework rather than defining the power for a separate receiver. |
| LG Electronics | Proposal looks fine as a starting point. In addition, we have the following comments. - Additional scaling factor is required not only for static power but also for dynamic DL/UL power - Regarding BS’s RX sensitivity issue brought up by Ericsson, in our view, it can be dependent on how UL WUS is designed |
| vivo | We are open to discuss additional power scaling rule in a LP mode. But we have the following comments: Comment #1: In our understanding, here LP mode means static power can be optimized when reduction of antenna/frequency in a long time instead of the deterministic sequence transmission. Actually there is no need to mention what LP mode includes which is up to BS implementation, i.e., a BS LP mode includes transmission and reception of predetermined deterministic sequences. The difference with normal power scaling here lies in that static power can be also optimized when reduction of antenna or bandwidth in a long time scale, e.g. zero load. Comment #2: There is no need to define the transition to/from LP mode in BS power model. When/How to transit between LP mode and normal mode belongs to technical discussion. Comment #3: Why to determine the scaling factor like this for static power should be clarified. Besides, it seems FL doesn’t have proposals on additional power state that is related to LP mode as well. The proposal of introducing energy efficient state is due to the deterministic sequence communication, e.g., WUS monitoring. In this case, BS can do some optimization on both RF and BB part that is less than normal state even there is no reduction of antenna and frequency. We suggest to add another proposal on additional power state. |
| Ericsson | Does this new scaling mechanism apply to all states of the power model, or is this a new state in the power model? For which sequences/signals would the scaling apply to? Can it be used if e.g. SSB, SIB1 are frequency multiplexed with each other or other transmissions? We wonder if there need to be new transition times associated with this. |
| TCL | We know there is one urgent requirement to discuss BS LP mode in last meeting. For this proposal, although BS LP mode defined for transmission and reception of predetermined deterministic sequences, from our perspective, more explanations might be necessary before giving LP mode (i.e., PstaticDL updating based on TR 38.864), e.g., functionalities of LPR like FL’s conclusion (i.e., acting as a receiver (LP-RX) for PRACH or UL WUS sequences. It is also proposed to function as a transmitter (LP-TX) for "light Sync signals" or DL WUS) could be as explanation or note in this proposal. |
| Lenovo | Before defining BS LPR mode, some more clarification on its operation needs to be clarified. How BS enters LPR mode and how it affects the ideal mode UEs and does UE performing the initial access is transparent or non-transparent to the BS LPR mode. We prefer postponing such evaluation to a later phase until the BS power consumption models are agreed. |
| DCM | We do not support this proposal. We are skeptical of scaling the static DL power based on BW and number of active TxRUs since as in TR38.864, static value is defined as static part of power for BS in active, which is not scaled based on reference configuration, meaning, the static part is not involved RF unit. Therefore, there is no need to scale the static power based on the actual BW and the number of TxRUs for the static part that is irrelevant from RF effects. From FL summary, LP mode has much more small BW (5/10 MHz) and small active antenna count. In that case, we can apply scaling rule based on the same rule as what is defined in TR38.864 such that when NW tries to send sync signal with lower power, dynamic power might be scaled. |
| Qualcomm | OK with the proposal |
| Huawei, HiSilicon | We believe the following proposal is useful for motivating the relevant study, thus needs to be captured Study and evaluate BS aspects for NES regarding common signal transmission at least for initial access and the impact on UE performance and user experience, including, e.g., Power domain, e.g., Tx power, Frequency domain, e.g., BW, Spatial domain, e.g., #antennas, Sleep state transitions. Regarding the scaling modification, the understanding of our LP mode seems not accurate. We assume shared HW/components and take 25% TRXU used for LP mode as an example. Then, the remaining 75% components are expected to be deep/light sleep, instead of micro sleep, thus the total BS power consist of two parts, one from the LP mode part which works in active or micro sleep state, and the other from the remaining 75% BS components, which is kept in sleep state. For illustration, the following explains how it works. In this case, the static power part can be modified as below, where the Pi is the power state of deep or light sleep state and the same transition time and energy values as for the corresponding deep or light sleep state are used for BS transitioning to/from its LP mode. PDL=PstaticDL+PdynamicDLPstaticDL=1-sa*Pi for remaining partsa*P3 for LP transmissionPdynamicDL=sa*Pdyn,ante+sf*spηsf, sp*Pdyn,joint |
| ZTE, Sanechips | First, we want to clarify what is the LPR function? For example, whether LPR at BS side is used to transmit DL signal or receive UL signal like DL WUS which has only one direction link. Second, we try to understand if LPR is introduced, what schemes we are gonna to evaluate? If there is no scheme fully depending on the LPR, we do not think it is needed to introduce this, since it can be up to implementation. For example, we all know AI could be used for BS power saving, do we spend some time to introduce a model and evaluate the power saving gain ? |
| CATT | We are OK in general for BS LP mode. One comment is that additional scaling for dynamic DL/UL power should be considered. |
| Apple | We would like to understand better why the static power for LP mode is modeled as proposed, especially the linear scaling w.r.t. the TxRU and the BW. It seems too optimistic. On the transition: “The same transition time and energy values as for light sleep are used for BS transitioning to/from its LP mode” It is not clear to us what transition it refers to, whether it means the transition from LP mode to the regular power mode. If this is the case, the transition energy should probably be much higher than the light sleep mode because the BS (or a significant part of the BS) is not in light sleep mode. |
| Tejas | Support |
| Xiaomi | The LP reception on the base station side is a function not covered in previous studies. Before discussing its related scaling methods, we also believe it is necessary to first clarify the assumptions underlying the signals/channels associated with these models, as well as their general procedure and how these designs and procedures affect the components. |
| OPPO | Support the proposal. We are also open to further discuss the scaling scheme and transition time/energy. |

Network loads for power savings evaluations

Companies’ views

Companies largely agree that 6GR energy efficiency evaluations must use a load-dependent methodology, requiring quantitative analysis for both empty load (L=0) and partial load cases. The commonly proposed baseline for defining Low, Light, and Medium load scenarios relies on the Resource Utilization (RU) ratio (L) based on UE-specific PDSCH/PUSCH resources: Low load is generally proposed as 0 < L ≤ 15%, Light load as 15% < L ≤ 30%, and Medium load as 30% < L ≤ 50%. However, Ericsson provided field data suggesting that the majority of real-world traffic cases are concentrated below 15% load, proposing a shift to updated definitions to better reflect operational reality: Low load as 0 < L ≤ 5%, Light load as 5 < L ≤ 15%, and Medium load as 15 < L ≤ 35%, see Figure 1. Additionally, several companies have the view that the "fully loaded case" (L=100) must serve as the reference against which energy saving gains are calculated, and TCL proposed explicitly including High Load (L > 50%) as a mandatory evaluation case. Finally, Samsung and Qualcomm advocated for calculating daily averaged ESG and UPT by weighting results from different load cases based on their time ratio in daily traffic patterns, ensuring a fair comparison of schemes optimized for varying load conditions across a full day of operation.

Figure 1: CDF of 15 min load quanta over time. (Ericsson)

FL comments and proposals

Based on Ericsson’s data, a BS has a lower activity level than 15% in approximately 60% of the time. Hence, it is reasonable to adjust the load levels to better reflect reality. Furthermore, although high loads in themselves are contradictory to NES since then the BS is working at capacity, a high load level can be included for completeness. FL propose the following compromise:

FL Proposal 3.2.6.1 (1st round; high priority):

The following load levels are defined for BS power savings evaluations:

Empty load:	L = 0%,

Low load:		0 < L ≤ 10%,

Light load:		10 < L ≤ 20%,

Medium load:	20 < L ≤ 40 %,

High load:		L > 40 %.

FL comment after 1st round:

Most companies agree to update load values whereas a few oppose, and one company wants to decrease load values even further. It seems to FL that the above proposal is the feasible compromise, if any. No need to update the proposal further.

Companies’ comments

Companies’ comments on the above proposal are appreciated.

| Company | Comment |
| --- | --- |
| CMCC | Fine with the proposal. |
| Samsung | The proposal is fine. Additionally, let's evaluate the EE using daily average metrics based on this traffic load, at least for loaded cases, as we proposed. |
| LG Electronics | OK |
| vivo | We prefer to reuse the definition of load level as we already discussed this in 5G NR. |
| Nokia | OK |
| Ericsson | We prefer: Empty load: L = 0%, Low load: 0 < L ≤ 5%, Light load: 5 < L ≤ 15%, Medium load: 15 < L ≤ 35%. The load-level definitions of TR 38.864 are skewed in comparison with field data from live NR networks. More than 60% of the time, the load is less than or equal to 15%, why it is misleading to call it “low load”. |
| Google | This FL proposal is fine to us. |
| TCL | OK |
| Lenovo | Ok |
| DCM | For load rate from empty to medium load, we are fine to support the proposed values. As for whether to allow high load for completeness, we need to discuss the necessity of introducing the high load first. In our view, we think that the NES characteristics between medium load and high load will not be significantly different. Thus, we do not think high load should be introduced, which was not even introduced in NR and will cause unnecessary work on a simulation and a comparison of evaluation results from companies. |
| MediaTek | First of all, thank Ericsson for sharing the collected field data. But since it is averaged over many nodes, we don’t know the detailed distributions among different cells (e.g. coverage cells vs capacity cells) and in different frequency ranges (e.g. FDD, TDD in FR2, and FR2). It would be highly appreciated if E/// can further share the detailed distribution. It is not clear to us why this proposal is needed. If it is intended choose certain values of loading for evaluations, can we simply say that, for example, only empty, low, and light loading scenarios are evaluated? We can more specific to discuss the evaluated L values. |
| ZTE, Sanechips | Fine with the proposal. Considering that the load is defined as a percentage of resources used for UE specific PDSCH/PUSCH without considering the uplink/downlink TDD pattern and the resources occupied by common signal/channel, even with a moderate traffic load, the actual downlink resource usage already accounts for a large proportion. We are OK to change the load levels as Ericsson suggested. |
| CATT | We suggest to reuse the traffic load of 5G NR in 38.864. |
| Apple | In our understanding, the load represents the cell resource utilization ratio, rather than the percentage of time that a cell is in this state. Therefore, we propose to keep the original definition of load levels. However, we think it is useful to consider the percentage of the time for each load level. We support using the daily average metric proposed by Samsung, in this metric, the duration/percentage of a loaded scenario will be reflected. |
| Xiaomi | OK |
| OPPO | OK |
| Futurewei | We are OK with the proposal. |

UE Power Consumption Model Updates

Observations and proposals about UE power consumption model.

Companies’ Views. Please Unfold for Reference

| ### CompanyName | ### Views/Proposals |
| --- | --- |
| ### FUTUREWEI | ### Observation 1: The NR UE power consumption model in TR38.840 and TR38.869 may be revisited or simplified to reflect improvements in 6GR UEs. For example, the LP-WUR turned off power state and MR ultra-deep sleep state can be combined into a single UE sleep state. Observation 2: Given focus in 6GR day 1 on DL WUS of OFDM-based sequence, the relative power value for LP-WUR turned on state can be P_ON=1 corresponding to Rel-19 OFDM based WUR with time domain sequence detection and additional 2.5dB of noise figure compared to MR. Proposal 1: Under the extended UE power consumption model, it should be clarified whether the MR and WUR can be active simultaneously, i.e., simultaneous operation of two active power states (energy efficient and non-energy-efficient). Proposal 2: Consider the power states and corresponding relative power values, transition times, and transition energies in Table 2 for the extension of UE power consumption model in TR38.840 for UE operation with DL WUS of OFDM-based sequence. Proposal 3: Discuss whether UE's support of 400MHz bandwidth around 7GHz is captured in the UE power consumption model as update to bandwidth scaling and/or multi-carrier scaling. Further, discuss the impact of potentially sparser sync raster and anchor/non-anchor carrier designs on the model for RRM and CC scaling. |
| ### Nokia | ### Observation 4: To advance the discussion on how to decouple, weight and scale RF and BB separately, there should be a common understanding of what is covered by RF and BB. Observation 5: Scaling of the dynamic parts of the RF and BB parts of the UE power model is expected to vary depending on the bandwidth. For lower bandwidth operation, joint scaling may be sufficient, whereas for higher bandwidths, independent scaling could be considered. Observation 6: Adaptation/restriction of MIMO layers, PDSCH allocation (and TBS) could be accounted via scaling of the dynamic BB power consumption. Scaling can be also based on the 'relative processing bandwidth' change. Observation 7: Consider at least for the BW related scaling, having separate static and dynamic components. For lower bandwidth operation (≤ 100MHz) the NR UE power model ratio between static and dynamic could be considered. Proposal 6: RAN1 agree on the partition of RF and BB processing for the 6GR UE power consumption model at least for higher bandwidth operation. Proposal 7: The 6GR UE power consumption model scaling supports static and dynamic components which can be scaled either separately or jointly depending on the operational bandwidth. Proposal 8: 6GR UE power consumption model for [20MHz - 100MHz] bandwidth operation uses scaling for the static and dynamic parts that is down-selected from: Option 1: A joint scaling factor for RF and BB (like the NR UE power model); Option 2: Independent scaling factor for dynamic part of RF and BB based on (X/100)^θ, where θ=[0.3] for RF scaling and θ= [1.2~1.25] for BB scaling. Proposal 9: 6GR UE power consumption model for [>100MHz] bandwidth for single carrier and for intra-band CA operation uses the following scaling factor for dynamic part of RF and BB: Determine the scaling factor for dynamic part of RF and BB based on (X/100)^θ, where θ=[0.6] for RF scaling and θ= [1.2~1.25] for BB scaling, and X is the total bandwidth for single carrier and intra-band CA operation. Proposal 10: 6GR UE power consumption model, for [>100MHz] bandwidth and for inter-band CA operation uses the following scaling factor for dynamic part of RF and BB: Determine the scaling factor for dynamic part of RF based on (#CC)^θ where θ=[1.0] for RF scaling and BB based on (X/100)^θ, where θ= [1.2~1.25] for BB scaling, and X is the total bandwidth for inter-band CA operation. Proposal 11: For 6GR UE power consumption model in combination of RF and BB dynamic parts, use weighting as follows; For low bandwidth operation [≤100MHz] assume weight γ=0.6 for RF (and (1- γ) for BB respectively); For high bandwidth operation [>100MHz] assume γ=0.5 for RF (and (1- γ) for BB respectively). Observation 8: A 6GR WUS receiver based on an energy efficient state of the main radio, can take advantage of main radio capabilities and components to improve the WUS coverage and spectral efficiency. Observation 9: Unlike PDCCH/PDSCH monitoring after deep sleep, the transition energy from the energy efficient, rx-ready deep or light sleep mode to monitor WUS or SSS for measurements requires the activation of a limited number of RX chain(s) with, reduced sampling clock and BB components. Observation 10: For evaluation of 6GR WUS, consider the potential power saving gain over the whole cell area. Proposal 12: The 6GR UE power consumption model supports Rx-Ready deep/light and micro sleep states that support fast and low power transition to new WUS or SSS monitoring power states as indicated by the table in the Nokia contribution. |
| ### Spreadtrum | ### Proposal 1: Adopt the following Table 2 as UE consumption model for 6GR EE evaluation. Proposal 2: For power scaling rules for UE, the follow principles should be considered: Linear model for simplify evaluation; Scaling only for non-sleep states; 100MHz BW as baseline, with scaling up/down. |
| ### vivo | ### Observation 1: For the power scaling rule for split RF and BB proposed in [2], the total relative power value of certain channel increases as the proportion of BB, i.e., 1-α increases for the same BW of RF and BB. Proposal 2: RF and BB split and associated scaling rule for RF and BB is not explicitly captured in 6GR UE power model. Proposal 3: Study BW scaling rule and the CA scaling rule based on similar UE implementation to keep consistency. Proposal 4: Dynamic and static split and associated scaling rule for dynamic and static is not explicitly captured in 6GR UE power model. Proposal 5: Regarding the relative power values of the sleep states for evaluation purposes of 6GR UE EE, - The existed relative power values of ultra-deep sleep, deep sleep, and light sleep are reused regardless of BW without additional scaling. - For micro sleep: - The existed relative power value according to the reference BW (100MHz) is reused to BW no larger than 100MHz. - Study new relative power values for bandwidth larger than 100MHz, i.e., 200MHz, 400MHz BW. - Study reduced relative power value for bandwidth smaller than 100MHz, i.e., 20MHz BW. - Study switching between micro sleep at small BW and non-sleep state at large BW, including switching delay and power. Proposal 6: Regarding the relative power values of the non-sleep states, i.e., PDCCH only, SSB, PDCCH and PDSCH, PDSCH only, and measurements, etc., for 200MHz and 400MHz, study new relative power value(s) or study up-scaling rule from the reference BW. Proposal 7: Regarding the relative power values of the non-sleep states, i.e., PDCCH only, SSB, PDCCH and PDSCH, PDSCH only, and measurements, etc., study reduced relative power value (s) for certain small BW(s), e.g., 20MHz BW, and two sets of relative power value(s), i.e., both the reduced relative power value (s) and the existed power value (s) are used. - Except for the certain small BW(s) such as 20MHz BW with reduced relative power value(s), the existed down-scaling rule is re-used for other BW(s) no more than 100MHz - FFS: how to use two sets of relative power value(s) and applied use cases, switch delay for different sets Proposal 8: Consider power consumption model for 6GR UE with DL WUS of OFDM-based sequence in frequency up to around 7GHz as below: - FFS the power consumption model for 6GR UE with DL WUS of OFDM-based sequence for other frequency ranges |
| ### CMCC | ### Proposal 3: For UE power model in 6GR, consider the following scaling method: The UE power can be divided into static part and dynamic part: P_total = aP_1 + bP_2, where P_1, P_2 are the relative powers for static and dynamic part, and a, b are the scaling factors. For scaling on frequency/spatial/power/processing domains, the specific method is as follows: For maximum BW for dynamic scheduling (static or semi-static manner), both static part and dynamic part are impacted. For BW in dynamic scheduling for DL (dynamic manner), only dynamic part is impacted. For BW in dynamic scheduling for UL (dynamic manner), no scaling for power-limited scenario. For antenna for DL, only dynamic part is impacted. For antenna for UL, only dynamic part is impacted. For transmit power for UL, only dynamic part is impacted. For processing clock/complexity, only static part is impacted. |
| ### CATT | ### Proposal 14: The 100MHz system BW is the preferred reference configuration for operation around 7GHz. The existing scaling rule in TR 38.840 can be re-used for 7GHz. Proposal 15: If the 200MHz system BW is supported for 7GHz, the scaling rule for 200MHz can be based on the scaling rule in TR 38.840. Proposal 16: The following three power states can be introduced for UE operation with DL WUS of OFDM-based sequence: - Sleep state#1: Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing or frequency cannot be maintained. Relative Power: [0.1] - Sleep state#2: Additional sleep power outside the occasions for DL WUS monitoring while in a sleep state. Relative Power: [0.2] - DL WUS: DL WUS monitoring, which can be performed while in a sleep state. Relative Power: [10/20/30] |
| ### xiaomi | ### Proposal 26: For reference configurations, RAN1 needs to confirm whether configuration information of PDCCH and PDSCH (, e.g., control channel region, K0, PDCCH BD, Number of RBs for TRS, etc.) are also reused. Proposal 27: For BW scaling, a unified BW scaling method should be considered for both down scaling and up-scaling. Proposal 28: For the modelling of LR functionality, RAN1 needs to ensure that the model can accommodate different implementation approaches, such as shared or independent hardware between LR and MR. Either a unified or an independent modelling method can be considered for this purpose. |
| ### OPPO | ### Proposal 3: For EE evaluation purpose, eMBB device can be the basic evaluation reference configuration. It is not precluded to further include other 6GR device types depending on other sessions/RAN progress. Proposal 6: For 6GR eMBB UE, the TR 38.840/ TR 38.869 model is the starting point for UE power model with following. - Introducing new power state between Deep-sleep and Ultra Deep-sleep, with power value of 0.1, transition energy as 5000 and the total transition time of 100ms/150ms. - Extending power scaling for the bandwidth adaptation low to 3MHz and up to 200MHz, up to 7GHz frequency band. - Reusing the scaling for number of PDCCH blind detection. - The model further support for low/high capability mode (Mandatory Baseline/Full Functionality set). - The power of low/high capability can be defined with scaling factor from high capability. |
| ### Huawei | ### Proposal 6: In 6GR study, UE power consumption should be split into dynamic power consumption and static power consumption, where the value of static power consumption is the same as that of micro-sleep for the same BW and Rx/Tx number. Proposal 7: In 6GR study, UE power consumption model of MR in Table 1 is used for reference configuration. Proposal 8: In 6GR study, UE power consumption model of LPR in Table 2 can be used. Proposal 9: In 6GR UE power model study, the template in Table 3 is used for further collection on scaling factors. |
| ### Samsung | ### Proposal 4: For the UE power consumption model, introduce static/dynamic power partitioning for active UE states, reusing the conceptual framework from the network power model. The specific scaling values for static and dynamic components are FFS. Proposal 5: For evaluation purposes, update the NR UE power consumption model for receiving DL WUS of OFDM-based sequence as follows: - Define DL-WUS monitoring as a distinct power state with power value reflecting shared receiver operation in reduced-power mode - Include measurement power state for 6GR sync signal performed in reduced-power mode |
| ### ZTE Corporation | ### Proposal 6: Table 2-4 can be used as a starting point for reference configuration for UE power consumption model. Proposal 7: Table 2-5, Table 2-6 can be used as a starting point for power states, additional transition energy and total transition time for UE power consumption model. Proposal 8: Table 2-7 can be used as a starting point for low power modem for UE power consumption model. Proposal 9: For downlink power states other than sleep state, the power is determined according to: P(a,b,c) = P_micro-sleep + (P_t -- P_micro-sleep)ab*t; Wherein, P_t is relative power of PDSCH or reference signal reception under the reference configuration, a is UE antenna scaling factor, calculated as a = A_use/A_ref, wherein, A_use is the number of UE antenna used in a slot, A_ref is the number of UE antenna in reference configuration; b is system bandwidth scaling factor, calculated as b = B_use/B_ref, wherein, B_use is the reception bandwidth in one slot, B_ref is the system bandwidth in reference configuration; t is time domain scaling factor, calculated as t = T_use/T_ref, wherein, T_use is the occupied time domain resource of a power state in one slot, T_ref is the occupied time domain resource in reference configuration. Proposal 10: The power of PDCCH with different numbers of PDCCH candidates is calculated according to: P(d) = {P_micro-sleep, c = 0; c×Pt + 0.7×(1-c)×Pt, c > 0}; Wherein, P_t is the power of PDCCH for reference configuration; c is PDCCH candidate scaling factor, calculated as c = C_use/C_ref, wherein, C_use is the number of PDCCH candidate, C_ref is the number of PDCCH candidate in reference configuration. Proposal 11: When multiple power states in one slot, the power of the slot can be calculated according to: P_sum = Σ(i=1 to 14)P(i); Wherein, P(i) is the power of i-th symbol. Proposal 12: For downlink power states, the power for intra-band CA scenario is determined according to: P_sum = Pmicrosleep + Σ(i=1 to CA_num)(P(i) - Pmicrosleep) for intra-band CA; P_sum = Σ(i=1 to CA_num)P(i) for inter-band CA; Wherein, CA_num is the number of carriers in CA scenario, P(i) is total power in a time interval of carrier i. Time interval is one slot with the smallest SCS. Proposal 13: No scaling on sleep power state. Proposal 14: For the scaling of UE power model, further study the following aspects: Whether or not to consider the impact of diverse device type; Whether or not to apply scaling on processing timeline. Proposal 15: The power consumption of AI model can be reflected by FLOPs approximately. |
| ### Ericsson | ### Observation 7: The power consumption for the low power state can be within range [1-10] units assuming 1Rx and 5MHz bandwidth. Proposal 8: For UE power model in 6GR, the following should be considered: • Extending the bandwidth scaling to cover all bandwidths from less than 10 MHz up to larger than 100 MHz, including any number of receiving antennas, with a single power model. • UE active power (for non-low-power mode) after multiple scaling should not be lower than the micro-sleep power. Proposal 9: The UE power model and evaluation should include UL power levels of 26 dBm and 29 dBm. Proposal 10: A 'low power state' can be added for the purpose of modelling DL WUS monitoring and serving cell measurement in RRC idle mode. |
| ### LG Electronics | ### Proposal #5: Confirm to reuse the existing UE power consumption model FR1 reference configurations in TR 38.840 for operation sub-6 GHz as well as around 7GHz, i.e., no need to introduce around 7 GHz specific update for UE power reference configuration. Proposal #6: For UE power consumption scaling rule, different scaling factors can be applied to RF and BB powers, respectively. Further study the necessity of applying scaling factors to static power. |
| ### HONOR | ### Observation 7: Support larger single carrier bandwidth, such as 200M carrier bandwidth. Observation 8: Due to the support of more service types, the system design will become more complex. Proposal 15: Reuse the existing FR1 UE reference configuration, but update the bandwidth, modulation scheme, number of Tx, and transmission power. |
| ### Apple | ### Proposal 5: Reuse the power values for different power states in TR 38.840. Proposal 6: Reuse the scaling rules in TR 38.840 as the baseline, except for the scaling w.r.t. the DL BWP, which is updated to: - PDSCH: scaling of X MHz = [0.5] + [0.5] (X - 20) / 80 for X up to 100 MHz<<<LINEBREAK>>>- PDCCH: scaling of X MHz = [0.9] + [0.1] (X - 20) / 80 for X up to 100 MHz - Power model needed for specific enhancements, if necessary, can be discussed case-by-case. Proposal 7: For ~7GHz, use the same power model and scaling rules as FR1. Proposal 8: For the monitoring of DL WUS of OFDM-based sequence, consider the following power model: - Power values for the active monitoring of DL WUS: {[10, 15, 20]} - Power for the sleep state of DL WUS monitoring: [0.4] - Transition time (ramp-up + ramp-down): {[8]} ms - Transition energy (unit x ms): power value for the active monitoring of DL WUS * transition time |
| ### MediaTek Inc | ### Observation 3: Because of the introduction of 200 MHz UE BW, bandwidth adaptation can require longer time for the adaptation of larger BW difference. The power consumption for a given BW can be different w.r.t. the allowed bandwidth adaptation time. Proposal 11: Update scaling w.r.t. bandwidth adaptation for 6GR by taking into account the allowed adaptation time. If the allowed adaptation time is similar to NR, the scaling of bandwidth adaptation can be similar; otherwise, more conservative scaling should be applied. The following table is one suggested update: Bandwidth adaptation (DL): If adaptation time of at least [2] ms is allowed, scaling by {0.325, 0.4, 0.55, 1.0, 1.4, 1.7} for {10, 20, 40, 100, 160, 200} MHz; Else, {0.5, 0.55, 0.65, 1.0, 1.6, 2.0} for {10, 20, 40, 100, 160, 200} MHz; FFS: Scaling for other target bandwidth values; Above scaling is applicable for FR1 only. In case scaling is needed for FR2, companies can report the assumed scaling factor. Proposal 12: For 6GR CA, a multi-carrier cell can apply more dynamic adaptation in frequency domain, and the following worst case power consumption scaling is suggested: CA (DL): 2CC is 2x1CC; 4CC is 4x1CC (i.e. 2x 2CC); Above refers to the worst case CA configuration in terms of power consumption. Activation/deactivation delay follows RAN4 specification; FFS transition energy; Applicable for FR1 and FR2 Proposal 13: The following antenna related scaling rules are suggested for 6GR UE: Antenna scaling (DL): 8Rx power is 2x 4Rx power for FR1; 6Rx power is 1.4x 4Rx power for FR1; 2Rx power is 0.7x 4Rx power for FR1; 1Rx power is 0.7x 2Rx power for FR2; Assume same number of antenna elements per Rx chain. Antenna scaling (UL): 2Tx (4Tx) power is 1.4x 1Tx (2Tx) power at 0dBm. 1.2x at 23dBm FR1 only; 4Tx support is not considered for FR2. Observation 4: Micro sleep power (and light sleep) power can be impacted due to larger UE BW for 6G. Proposal 14: For 6GR EE evaluation, introduce scaling on at least micro sleep, as suggested below: Scaling on micro sleep: Scaling micro sleep power by 0.7, 1, and 1.7 when the active bandwidth after transition out of micro sleep is ≤ 20 MHz, within (20 MHz, 160 MHz), and ≥ 160 MHz, respectively; Applicable for FR1 and FR2. FFS: Scaling on light sleep Proposal 15: The model the benefit of relaxed PDSCH processing time, apply the following scaling for 6GR: PDSCH processing time relaxation: Scaling UE power consumption by 0.7 if 2x or longer PDSCH processing time can be accommodated; Applicable for FR1 and FR2 Proposal 16: The following power consumption model is introduced to include WUS related operations: FFS: the power values for FR2. Power States with Relative Power: Ultra Deep Sleep (0.1); EE Sleep (0.7); Deep Sleep (1); Light Sleep (20); Micro sleep (45); EE Processing (15) - Monitoring 6GR signals of OFDM-based sequence(s), e.g. WUS, PSS and/or SSS, in an energy efficient manner, based on the following configuration: [12]-RB and [1]-RX for reception; CFO up to [5] ppm and timing offset up to [2] us; EE-processing can coexist with a sleep state with its power consumption added on top of the sleep power and without triggering transition out of the sleep state; PDCCH-only (100); SSB or CSI-RS proc. (100); PDCCH + PDSCH (300); UL (250 (0 dBm), 700 (23 dBm), 1100 (26 dBm)). Additional transition energy and total transition time: Ultra deep sleep: [45000], [2000] ms; EE sleep: [4500], [200] ms; Deep sleep: 450, 20 ms; Light sleep: 100, 6 ms; Micro sleep: 0, 0 ms (Immediate transition is assumed for power saving study purpose from or to a non-sleep state); Ramp-up time is no less than half of the total transition time |
| ### Qualcomm Incorporated | ### Observation 1: The UE power model from 38.840 needs updating to correctly estimate UE power trends for the bandwidth values of interest for 6G. Observation 2: Super-linear UE power scaling is not limited to very large bandwidth and depends on the target baseline and maximum operating points of a UE. Observation 3: Monitoring of an OFDM signal can be performed without a performance loss and at a significant reduction in power compared to SSB monitoring in 38.840. Observation 4: The main source of energy savings of performing RRM in an energy efficient operation mode is the ability to remain in a sleep, not the reduced power per measurement. Observation 5: RRM relaxation is not needed to perform RRM in an energy-efficient operation mode. Proposal 3: Bandwidth, multi-CC, and rank scaling in the UE power model should correctly capture the super-linear power scaling. Proposal 4: Update scaling rules in the UE power model to independently scale RF and baseband power with bandwidth, number of CCs, and rank for baseline 100 MHz power states. Proposal 5: RAN1 to consider whether to introduce separate power models for RedCap (and eRedCap) devices or scale them from the regular UE power model. Proposal 6: Adopt Tables 1, 2, and 3 as part of the UE power model for FR1 evaluations. Proposal 7: Do not introduce measurement relaxation when RRM is performed in an energy-efficient mode. Proposal 8: For RedCap-like devices, reuse the NR RedCap power model as baseline with the 6GR scaling rules in Table 5. Proposal 9: Use scaling rules to evaluate UE energy efficiency around 7 GHz. Proposal 10: Adopt Table 4 as part of the UE power model for FR2 evaluations. Proposal 11: 6GR should provide UE power models to evaluate the gains from decoupling the activation/de-activation of secondary cells in the downlink and uplink. Proposal 12: 6GR UE power model should include the effects of both increasing and decreasing PDCCH blind decodes and monitored CCEs on UE power. Proposal 13: For uplink evaluations in 6GR, the RAN scheduling model should take the information provided by a UE about its power variations in time and frequency into account. |
| ### NTT DOCOMO | ### Proposal 8: The UE power consumption model should also cover the frequency around 15 GHz: Reuse the existing UE power consumption model and reference configuration in TR 38.840 for frequency around 15 GHz; RAN1 to consider whether to reuse the power model and reference configuration for FR1 or FR2; FFS: updates on SCS, power value and transition time Proposal 9: For evaluation purposes, UE power consumption model should cover diverse device types, including both high-tier UEs such as eMBB UEs and low-tier UEs that typically have reduced capability and RF/BB processing complexity. Proposal 10: For the energy efficiency evaluation of 6G low-tier UEs, study potential update(s) of the power model in TR 38.840 w.r.t. baseline power consumption, BW and antenna setting. Baseline power consumption refers to the relative power for each power state Proposal 11: Define two sets of baseline power consumption for DL-WUS state and EE-Meas state, shown in Table 4, by considering at least the following two categories of WUR architecture: Architecture 1 ("Add-on LR"): The LR (Low-power Receiver) and the MR (Main Receiver) are separate; Architecture 2 ("Integrated LR"): The LR functionality is implemented by the MR operating in low-power mode Observation 2: The power model in TR 38.840 is not adequate to evaluate UE power consumption with a variety of PDCCH monitoring configurations (e.g., CORESET size, number of CCEs / BDs). Proposal 13: For evaluation purposes, study extending NR UE power consumption scaling w.r.t. PDCCH monitoring configuration. Proposal 14: Reuse performance metrics captured in TR38.840. |
| ### TCL | ### Proposal 14: For 6GR UE power model, reuse the power model in TR 38.840 for FR1 as a starting point, and further study the necessity of introducing new power states, such as the power state for WUS/WUR monitoring, and the power state for EE-RRM measurement. Proposal 15: For 6GR UE power model, reuse the power model in TR 38.840 for FR2 as a starting point, and further study the necessity of introducing new power states, such as the power state for WUS/WUR monitoring, and the power state for EE-RRM measurement. Proposal 16: For 6GR UE power model, study the necessity of introducing power model for frequency around 7GHz and/or 15GHz. Proposal 17: For 6GR UE power model, study the necessity of introducing power model for diverse device types, such as low-tier UE, high-tier UE, etc. Proposal 18: For 6GR UE power model, study the necessity of introducing power scaling for bandwidth adaptation, number of PDCCH blind detection, number of Rx/Tx antennas, etc. |

Summary and Discussion

Moderator would like to thank companies’ valuable inputs. For 1st round of discussion, the following will be prioritized:

- Power states and power values, including those related to DL WUS operation and one small update related to higher UL transmission power.

- Scaling rules including those for new settings beyond NR R16 UE power consumption model, including wider UE BW, more number of antennas, higher UL transmission power, and higher T-put/processing speed requirement, etc.

Note that, depending on the discussion, more aspects may be further covered.

Power states and power values

In RAN1#122-bis meeting the following is agreed to guide companies’ input on power states and power values related to UE operation with DL WUS of OFDM-based sequence:

Agreement For evaluation purposes, study extending NR UE power consumption model for UE operation with DL WUS of OFDM-based sequence, regarding the following aspects: - Power state(s), sleep and non-sleep, and corresponding characteristics and power value(s) - Transition energy and time for each of sleep state(s) - Companies to report the assumption(s) for achieving the proposed power value(s), e.g., time/frequency domain detection, noise figure assumption(s), synchronization assumption(s), BW/antenna assumption(s), etc.

While NR Rel-18 study on WUR considered dedicated receiver architectures with ultimate low power consumption, there is emerging interest in considering alternative implementation where WUR is a low power/EE operation of MR. From UE power model perspective, there requires at least a new non-sleep power state which focus on processing of OFDM-based sequence(s) for wake-up indication and, if applicable, synchronization and/or measurement. At the same time, to allow the flexibility of both shared or separated receiver design for WUR, it is suggested the new non-sleep power state can coexist with a sleep state without trigger UE transition out the sleep state. This also implies such power state should be subject to a limited processing capability so that most of the modem components remain in the sleep state, which is related to the assumption in 3rd bullet of the above agreement. Based on the considerations, the following proposal is proposed for companies check and comment. Note, to complete UE power consumption model, all non-sleep states from TR 38.840 are also included.

Proposal 3.3.2.1 (1st round; high priority):

Include the following non-sleep states as 6G UE power consumption model

- FFS: Configuration and relative power value(s) for EE processing in FR2 (including 24.25 GHz – 52.6 GHz)

| Power State | Characteristics | Relative Power in FR1 (including ~7GHz) | Relative Power in FR2 (including 24.25 GHz – 52.6 GHz) |
| --- | --- | --- | --- |
| EE Processing | Processing 6GR signals of OFDM-based sequence(s) for wake-up indication and, if applicable, synchronization and/or measurement, in an energy efficient manner, based on the following configuration: FR1 (including ~7GHz) - [12]-RB and [2]-RX for reception - CFO up to [5] ppm and timing offset up to [2] us - Noise Figure (NF) = 6GR UE NF or 6GR UE NF + [2] dB FR2 (including 24.25 GHz – 52.6 GHz) - FFS EE-processing can coexist with a sleep state with its power consumption added on top of the sleep power and without triggering transition out of the sleep state. | [10] if NF = 6GR UE NF [4] if NF = 6GR UE NF + [2] dB | TBD |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 | 175 |
| SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell. TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. | 100 | 175 |
| PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. | 300 | 350 |
| UL | Long PUCCH or PUSCH. | 250 (0 dBm) 700 (23 dBm) 1100 (26 dBm) | 350 (FFS Tx power level) |

Companies’ views on Proposal 3.3.2.1 (1st round)

- Note: Structure and description are critical part to consider first; values can be updated further

| Company | View |
| --- | --- |
| CMCC | Support the structure but whether 2 kinds of NF is needed? |
| Samsung | For power value in EE processing, it can be scaled from the reference configuration (e.g., 2Rx). (e.g., if 1RX is assumed, lower value can be used based on the scaling rule), or only agreed value can be assumed for link budget calculation as well as power evaluation assumption. Further, the operation of measurement or synchronization itself consumes power. It seems that there are no conditions for CFO or timing offset here. This can be addressed in a sequence design that meets the conditions later. |
| LG Electronics | We have two comments: - Reusing the terminology “DL WUS of OFDM based sequence” in the previous RAN1 agreement is preferred - The sentence “ EE-processing can coexist with a sleep state with its power consumption added on top of the sleep power and without triggering transition out of the sleep state.” needs further clarification. Our preference is not to consider coexistence with EE-processing state and other sleep state(s). |
| Vivo | We suggest the following changes in red color: Processing 6GR DL WUS signals of OFDM-based sequence(s) for wake-up indication, if applicable, synchronization and/or measurement, in an energy efficient manner, based on the following configuration: FR1 (including ~7GHz) - [12]-RB and [2 1]-RX for reception - Residual CFO up to [5] ppm and residual timing offset up to [2] us - Noise Figure (NF) = 6GR UE NF or 6GR UE NF + [2] dB FR2 (including 24.25 GHz – 52.6 GHz) - FFS EE-processing can coexist with be performed by a UE during a sleep state with its additional power consumption added on top of the sleep power and without triggering the UE transition out of the sleep state. - FFS whether the additional power consumption for EE-processing is the same or different across different sleep states. |
| Spreadtrum | We support to introduce EE processing power state but whether 2 kinds of NF is necessary need more discussion. |
| Ericsson | The purpose of EE sleep state and the new ultra deep sleep state is not clear. The characteristic of each sleep state should reflect the status/function of the state. The transition time/energy of the new EE processing state to/from other states is not defined. Does it follow the transition of the corresponding sleep state? The values for Ultra-deep sleep should follow TR 38.869: |
| TCL | We support introducing the power state of EE processing, but we have following comments: - regarding the configuration for EE processing, we prefer 1 Rx considering low power reception. - Regarding the relative power for EE processing, the relative power for micro sleep should be as baseline. the relative power for EE processing should be larger than that for micro sleep. For the UL transmission power, the relative power would be different depending on different transmission power. The relative power for values other than 0/23/26 dBm can be further defined. Linear interpolation or determination by the power range can be considered. |
| Huawei, HiSilicon | We are fine to add a new power state for DL WUS reception. However, the name ‘EE processing’ may be misleading, for which we suggest to change it to ‘DL WUS’ or ‘DL WUS proc.’. For the content, we agree with vivo to change it to 1RX and residual CFO/TO. For the NF, we think it is better to take it the same as 6GR UE, since majority view is too have a ‘full coverage’ for DL WUS. For the last paragraph, we would like to first clarify how to understand the meaning. - Understanding 1: When the energy consumption is calculated, the power consumption per slot is calculated as ‘DL WUS reception + micro/light/deep sleep’. - Understanding 2: When the energy consumption is calculated, the power consumption per slot is calculated as ‘DL WUS reception’. With understanding 1, FL’s wording is OK, and vivo’s change is not necessary. With understanding 2, vivo’s change may be needed. For the detailed values, we think 10 and 4 may be too small. Some larger values should be taken. In addition, in our view, the power consumption of DL WUS should be based on particular assumptions. Thus the scaling rule of ‘MR’ may not be applied to this power state. One note is need such as ‘Scaling rule is not applied to this power state’. Beside the new state power, we also think the power values for the existing non-sleep states should be reviewed to match 6G UE implementation. |
| CEWiT | The sentence “Processing 6GR signals of OFDM-based sequence(s) for wake-up indication and, if applicable, synchronization and/or measurement, in an energy efficient manner” needs clarification. Since the synchronization and/or measurement is also included we feel this mode should be able to perform both DL and UL WUS operations. |
| DCM | We are fine with the structure. The two NF types are distinguished by whether the WUR type is add-on or integrated. Regarding the description of EE-processing coexisting with a sleep state, it should be clarified that it is also allowed for EE-processing not to coexist with any sleep state. Additionally, clarification is needed regarding which sleep states defined in Proposal 3.3.2.2 can coexist with EE-processing. Our view is that EE-processing cannot coexist with Ultra Deep Sleep or EE Sleep. |
| Qualcomm | We do not think a degraded NF is necessary based on our analysis. We are concerned that this could hinder deployment of the wake-up sequence by signaling that it would degrade performance and impose more overhead on the system. 10 is too low as a power state for EE processing. In 38.869, most companies reported 10—20 for time-domain processing and 10—30 for frequency-domain processing. We propose picking a value in the middle and view [15] as a better starting point. Our preference for the number of antennas is 4. |
| ZTE, Sanechips | If there is no WUS monitoring, does it mean the power consumption in this WUR modem is ignored? We would be fine if the WUR off state power is small enough, e.g., <=0.05 compared with deep sleep. Additionally, currently the WUS function should not be limited to wake-up indication and, synchronization and/or measurement. Therefore, some wording change highlighted as blue is preferred. Processing 6GR signals of OFDM-based sequence(s) for wake-up indication and, if applicable, synchronization and/or measurement, and others if any, in an energy efficient manner Last, all the values for EE processing should be removed currently, which is quite misleading. |
| CATT | We share similar with LGE, ‘EE-processing can coexist with a sleep state with its power consumption added on top of the sleep power and without triggering transition out of the sleep state.’ should be further clarified. |
| Apple | We think a few candidate values should be provided for EE processing, given that it depends on the assumptions we make. It is risky to use a single value at this early stage. We propose [10, 15, 20] for NR NF, and [4, 10, 15] for NR NF + 2 dB. |
| Xiaomi | We support this direction: adding a row to the existing table to indicate the LP status. But the problem is that the sentence "EE-processing can coexist with a sleep state" is not very clear. Which sleep state can it coexist with? One interpretation is: if LR and MR share hardware, then the state during EE processing is micro-sleep; if LR and MR are implemented with independent hardware, then the state during EE processing is deep-sleep? |
| Futurewei | We are OK with the proposal. |
| Mod2 | Moderator thanks company’s valuable offline and online discussion. Given chair still mark the whole proposal yellow, moderator updated the description that can better reflect companies’ inputs: - 12-RB 5MHz BW - Reception time assumption for the power value: Suggest to define a power level over a slot time as other power states; scaling of 0.7 (similar to cross-slot scheduling scaling where we can receive only 2 symbols for PDCCH) is used for the same reception time can be less than 8 symbols. - UE chip vendors’ views on this value will be very important! - Final FFS is suggested to be dropped to keep simple model Power State; Characteristics; Relative Power in FR1 (including ~7GHz); Relative Power in FR2 (including 24.25 GHz – 52.6 GHz). EE Processing; Processing DL WUS of OFDM-based sequence(s) for wake-up indication and, if applicable, other 6GR signal(s) of OFDM-based sequence(s) for synchronization and/or measurement, in an energy efficient manner, based on the following configuration: FR1 (including ~7GHz) - 5 MHz BW, 1-RX / 2-RX for reception; FFS: Power value for 2-RX, i.e., X1 - Reception time over 14 symbols. Scaling of 0.7 can be applied if reception time is no longer than 8 symbols. - Residue CFO up to [5] ppm and residue timing offset up to [2] us - Noise Figure (NF) = 6GR UE NF FR2 (including 24.25 GHz – 52.6 GHz) - FFS Note: No implication on which configuration(s) to be supported by 6GR EE-processing can only be performed during a sleep state with additional power consumption added on top of the sleep power and without triggering transition out of the sleep state. - Additional power consumption is the relative power value defined. FFS: Whether/what different value across different sleep states; 20 / X1; TBD. |

When longer transition time is allowed, lower sleep power consumption can be achieved. On the other hand, (much) larger additional transition energy is included due to (much) complicated procedure to ‘hibernate/wake up’ more UE components (even including what outside modem system). With EE_processing, UE can avoid the large transition energy while remaining periodically monitoring WUS from BS, which enables UE to leverage the benefit of a deeper sleep.

Ultra deep sleep is investigated in Rel-18 NR WUR study, which is beneficial for devices that can tolerate long wake-up latency (400 ms – 800 ms). On the other hand, the long latency makes it challenging to be used by mobile/phone devices. In this regard, an intermediate gear of deeper sleep between ultra-deep sleep and deep sleep can be considered. The following proposal is therefore suggested for companies check and comment

Proposal 3.3.2.2 (1st round; high priority):

Include the following sleep states for 6G UE power consumption model:

| Power State | Characteristics | Relative Power |
| --- | --- | --- |
| Ultra Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing or frequency may not be maintained. | [0.015] |
| EE Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing or frequency may not be maintained. | [0.5] |
| Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | 1 |
| Light Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. | 20 |
| Micro sleep | Immediate transition is assumed for power saving study purpose from or to a non-sleep state | 45 |

| Sleep type | Additional transition energy: (Relative power x ms) | Total transition time** |
| --- | --- | --- |
| Ultra deep sleep | [45000] | [1600] ms |
| EE sleep | [4500] | [200] ms |
| Deep sleep | 450 | 20 ms |
| Light sleep | 100 | 6 ms |
| Micro sleep | 0 | 0 ms* |
| * Immediate transition is assumed for power saving study purpose from or to a non-sleep state ** Ramp-up time is no less than half of the total transition time |  |  |

Figure: Illustration of UE power consumption at state transition

Companies’ views on Proposal 3.3.2.2 (1st round)

- Note: Structure and description are critical part to consider first; values can be updated further

| Company | View |
| --- | --- |
| CMCC | Support to define a deeper sleep mode than legacy deep sleep mode, but whether we need 2 values need to be further considered. E.g., by considering that the default value of I-DRX is 1.28 s (1280 ms), is the current UDS mode with 1600 ms transition time meaningful? |
| Samsung | We want to explicitly clarify the target device for ultra deep sleep. If ultra deep sleep is targeting for IoT device, it is still unclear why the much longer total transition time is needed for ultra-deep sleep. From our understanding, longer transition time may be required for eMBB device, because it should support more complex processing and high capability. In addition, we want to clarify the transition energy from EE processing to non-sleep state. If UE detects WUS during EE processing, which transition energy can be applied? In addition, if UE changes state sleep state (no WUS monitoring) to EE processing, which transition energy can be applied? this aspect should be clarified for further evaluation. |
| LG Electronics | From the structure-wise perspective, we are fine with the proposal. One clarification question: Why is the sentence “Ramp-up time is no less than half of the total transition time” necessary? |
| vivo | 1）For ultra deep sleep, we suggest to keep the same values for additional transition energy and total transition time as in TR 38.869 unless the need of new values for 6G are clearly justified. 2）We support a new sleep, e.g., EE sleep in-between deep sleep and ultra deep sleep to reduce the waking up latency, especially for eMBB UEs. 3）To avoid ambiguity on a sleep state in which UE can perform other 6GR signal/channel processing, we suggest to add a note which is common to all the sleep states in the table for clarification: Note1: During a sleep state, UE does not process any other 6GR signal/channels except that UE may process DL WUS of OFDM-based sequence, and related sync and measurement if applicable, corresponding to a EE-processing power. |
| Spreadtrum | We agree to introduce a deeper sleep type (i.e., EE-sleep) for low-power wake-up signal, which has lower power consumption compared to the deep sleep defined in 5G-A. We think that the proposed EE-sleep can be retained while deep sleep can be removed. In addition, we think that a unified sleep mode should be defined for all 6G device types. Ultra deep sleep is almost impossible for eMBB devices. Therefore, we prefer to remove the Ultra deep sleep. |
| TCL | Based on TR 38.869 on LP-WUS/WUR, the total time for main radio transition from ultra-deep sleep to active/micro sleep state is the sum of ramp-up time and time for sync/re-sync. In the above table, is sync/re-sync included in the total transition time for ultra-deep sleep? |
| DCM | We are fine with the structure. But followings should be clarified - Regarding the total transition time, especially for ultra deep sleep - whether the time required for sync/re-sync is included or not. The reason why values of Additional transition energy and Total transition time are different from TR 38.869 values. |
| Qualcomm | On ultra-deep sleep, we support reusing the faster transition time from NR: 400ms ramp-up time (800ms total). We’re not clear on why the longer value from NR was used. We are open to further discussing “EE sleep”, but we need to better understand how the 0.5 value is achieved. |
| ZTE, Sanechips | We have similar feeling that the UDS is useless. |
| CATT | We are OK for introducing these two sleeps. The ultra-deep sleep was introduced in Rel-18 WUS/WUR. The terminology for ultra-deep sleep should be changed for 6G. And the relative power can be around 0.1 which is larger than that in Rel-18 WUS/WUR. |
| Apple | The rationale behind the EE sleep power, transition time and energy is not clear to us. Does it imply that the EE mode may also go into a deep-sleep state, but needs 200 ms to wake-up? This seems very long. |
| Xiaomi | We understand that EE Sleep means the UE neither needs to receive high-power channel signals such as PDCCH nor low-power signals like LP-SS. If that’s the case, this state should be the most energy-efficient one—so why is there still an Ultra Deep Sleep state? Under what conditions can this state be entered? |
| OPPO | Update of Ultra-deep sleep is OK for us The EE-sleep, we prefer smaller transition time. If 100ms is no Ok, may be something in middle like 150ms. |
| Futurewei | Initially, our understanding was that the purpose of the EE sleep is to allow the UE’s transition to/out of the EE processing state, but the assumptions on transition time/energy here is not aligned with that understanding, so we prefer to clarify first the purpose of the EE sleep state. |
| Mod2 | Moderator thanks companies’ inputs and would like to provide more information regarding the proposal: - From companies’ contribution, achieve the relative power value for ultra-deep sleep looks challenging, particularly for smart phone devices, due to, e.g., other components than modem should to be coordinated. Apply longer ramp up time and/or larger power value are the possible solutions. - To address Samsung’s and QC’s concerns, we can put power value 0.1 with target ramp up time of 400 ms. Note that since NR UE power model use transition time instead of ramp-up time, 800 ms transition time can be used and put a note for ramp-up time design that will be further studied as wake-up latency - Since ramp-up time doesn’t include pre-sync time, 400 ms ramp-up time + up to 10 SSBs may contribute too long paging latency. Therefore another sleep state with larger power value while shorter ramp-up time can be introduced - Why 0.5? A average of 0.2 (Samsung) and 0.7 (MTK) ~ 0.5 is used - If companies think to discuss and decide the values later, we can put variables first. TR 38.869: Power State; Relative Power (unit); Ramp-up and down transition energy (Note1): (unit multiplied by ms); Ramp-up time; Time for sync/re-sync. Ultra-deep sleep; 0.015; For evaluation, at least for FR1 MR ultra-deep sleep state, (Ramp-up and down transition energy, ramp-up time) is as follows, - Alt 1: (15000, 400ms) as baseline - Alt 2: (40000, 800ms) Company to report which alternative they use for which use cases.; For MR, at least for FR1 evaluation, - Number of SSBs for sync/re-sync for MR is up to 10 - Companies to report timeline and energy consumption. With the above further information, moderator suggests companies to further check the following updated proposals: Proposal 3.3.2.2a Include the following sleep states for 6G UE power consumption model: Power State; Characteristics; Relative Power. Ultra Deep Sleep; Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing or frequency may not be maintained.; [0.1]. EE Sleep; Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing or frequency may not be maintained.; [0.5]. Deep Sleep; Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained.; 1. Light Sleep; Time interval for the sleep should be larger than the total transition time entering and leaving this state.; 20. Micro sleep; Immediate transition is assumed for power saving study purpose from or to a non-sleep state; 45. Sleep type; Additional transition energy: (Relative power x ms); Total transition time**. Ultra deep sleep; [15000]; [800] ms. EE sleep; [2500]; [100] ms. Deep sleep; 450; 20 ms. Light sleep; 100; 6 ms. Micro sleep; 0; 0 ms*. * Immediate transition is assumed for power saving study purpose from or to a non-sleep state ** Ramp-up time is no less than half of the total transition time, and time for sync/re-sync is not included. |

Scaling rules

In this subsection, UE power consumption scaling w.r.t. bandwidth is considered first, where the following 4 alternatives can be identified [Huawei] [Qualcomm] [Nokia] [MTK]. The common part of the scaling designs lies in larger scaling factors for >100MHz. At the same time, processing/switch time requirements are considered, and with more relaxed processing time or reduced data rate, a lower factor can be achieved. With the above targets, the following proposal is therefore proposed for companies to check and comment the following proposal.

Proposal 3.3.3.1 (1st round; high priority):

For the scaling rule w.r.t. UE RF BW (and BB BW/processing ratio) for 6G UE power consumption model, down select one from the following alternatives:

Alt 1 (Scaling regarding static and dynamic portions): P=α∙Pstatic+β∙Pdynamic

| Scaling | α | β |
| --- | --- | --- |
| Power scaling for PDCCH or SSB/CSI-RS due to scalable design for bandwidth, where both static power and dynamic power are scaled along with bandwidth. γ is the scaling ratio of the bandwidth relative to the reference bandwidth. | {a00,a01,…,a0n} for {γ0,γ1,…,γn} E.g., {0.4, 0.6, 1, 2, 4.5} for {20%, 50%,100%, 200%, 400%} Note: the switching delay of BW adaptation is T1 (e.g., around 5ms) | {b00,b01,…,b0n} for {γ0,γ1,…,γn} E.g., {0.5, 0.7, 1, 2, 4} for {20%, 50%, 100%, 200%, 400%} |
| Power scaling for PDCCH+PDSCH due to scalable design for bandwidth, where both static power and dynamic power are scaled along with bandwidth. γ is the scaling ratio of the bandwidth relative to the reference bandwidth. |  | {b10,b11,…,b1n} for {γ0,γ1,…,γn}. E.g., {0.2, 0.5, 1, 3, 8} for {20%, 50%, 100%, 200%, 400%} |
| Power scaling for micro-sleep due to scalable design for bandwidth, where static power is scaled along with bandwidth. γ is the scaling ratio of the bandwidth relative to the reference bandwidth. |  | - |
| Power scaling for PDCCH or SSB/CSI-RS due to bandwidth scaling, where dynamic power is scaled along with bandwidth while static power is based on particular assumption of BB utilization. γ is the scaling ratio of the bandwidth relative to the reference bandwidth. | {a00,a01,…,a0n}, for {γ0,γ1,…,γn}, assuming particular BB utilization E.g., {1, 1, 1}, for {20%, 50%, 100%}, assuming BB utilization corresponding to 100MHz {2, 2, 2, 2}, for {20%, 50%, 100%, 200%}, assuming BB utilization corresponding to 200MHz {4.5, 4.5, 4.5, 4.5, 4.5} for {20%, 50%, 100%, 200%, 400%}, assuming BB utilization corresponding to 400MHz Note: the switching delay of BW adaptation is T2, where T1>T2. | b00,b01,…,b0n for γ0,γ1,…,γn E.g., {0.5, 0.7, 1, 2, 4} for {20%, 50%, 100%, 200%, 400%} |
| Power scaling for PDCCH+PDSCH due to bandwidth scaling, where dynamic power is scaled along with bandwidth while static power is based on particular assumption of BB utilization. γ is the scaling ratio of the bandwidth relative to the reference bandwidth. |  | b10,b11,…,b1n for γ0,γ1,…,γn E.g., {0.2, 0.5, 1, 3, 8} for {20%, 50%, 100%, 200%, 400%} |
| Power scaling for micro-sleep due to bandwidth scaling, where static power is based on particular assumption of BB utilization. γ is the scaling ratio of the bandwidth relative to the reference bandwidth. |  | - |
| Power scaling for PDCCH due to antenna scaling, where δ is the scaling ratio of the antenna chain numbers over the reference Rx chains. Static power is assumed unchanged since RF part contributes little to the static part. | 1, for {δ0,δ1,…,δm} | b20,b21,…,b2m for δ0,δ1,…,δm E.g., {0.7, 1, 1.3, 2} for {50%, 100%, 150%, 200%} |
| Power scaling for PDSCH due to antenna scaling, where δ is the scaling ratio of the antenna chain numbers over the reference Rx chains. Static power is assumed unchanged since RF part contributes little to the static part. |  | b30,b31,…,b3m for δ0,δ1,…,δm E.g., {0.5, 1, 1.5, 3} for {50%, 100%, 150%, 200%} |

Alt 2: Scaling regarding RF BW and BB (T-put ratio)

Scaling with BW XMHz and the baseband maximum throughput S. The maximum baseband throughput is also affected by the maximum rank. The reference configuration BW is Xref and the reference baseband maximum throughput is Sref.

With a shorter switching time, TS, the scaling factor is:

sf=0.4+0.6X-2080ifX≤Xrefα⋅XXref0.9+1-α⋅X⋅SXref⋅Sref1.4otherwise

RAN1 to decide α from {0.4, 0.5, 0.6}

If a longer switching time TL is allowed, the scaling factor becomes: γ⋅sf, where γ=γ0<1 when X≤Xth<Xref, and γ=1 otherwise. RAN1 to decide γ0 and Xth.

Alt 3: Combined Static/dynamic and RF/BB scaling

For single carrier operation and inter-band CA operation;

Ptot=P*α+P*1-α*γ*X100θRF+1-γ*X100θBB.

For intra-band CA operation;

Ptot=P*α+P*1-α*γ*CCθRF+1-γ*X100θBB.

Where α=0.4,θBB=1.2 1.25,  and

| BW range |  | θRF | γ |
| --- | --- | --- | --- |
| ≤100MHz |  | 0.3 | 0.6 |
| >100MHz | single carrier or intra-band CA | 0.6 | 0.5 |
|  | inter-band CA | 1.0 |  |

Alt 4: Additional scaling for >100 MHz and shorter BW adaptation delay than NR

If adaptation time of at least [ T ] ms is allowed, scaling by

{0.325, 0.4, 0.55, 1.0, Y1, Y2} for {10, 20, 40, 100, 160, 200} MHz

Else,

{Z1, Z2, Z3, Z4, Z5, Z6} for {10, 20, 40, 100, 160, 200} MHz

FFS: Scaling for other target bandwidth values

Above scaling is applicable for FR1 (including ~7GHz).

In case scaling is needed for FR2 (including 24.25 GHz – 52.6 GHz), companies can report the assumed scaling factor.

Companies’ views on Proposal 3.3.3.1 (1st round)

- Note: Structure and description are critical part to consider first; values can be updated further

| Company | View |
| --- | --- |
| CMCC | We think both Alt 1-4 are meaningful can may reflect to the certain HW design. But given consideration that there is only 1 meeting left, it is difficult to converge based on one single polynomial. Therefore, we suggest to use a series of stepped values instead which is much easier to make the progress, e.g., Use the structure of Alt 1. Alt 2-4 can be reflected in each row of Alt 1. Also, we think that Alt 1 need a new row for UL power scaling, and whether different channel/signal needs a separately design need to be further discussed. |
| Samsung | We agree with CMCC's proposal. It would be good to first agree on the alt1 structure and then consider remaining Alt 2-4. |
| LG Electronics | The concept of splitting BB part and RF part is acceptable to us. However, in Alt 1, the reasoning of applying scaling factor to static power should be further justified. Rather, static power in Alt 1 seems like BB part of dynamic power. |
| vivo | Firstly, we understand the investigation into RF, BB, dynamic, or static help to learn how the power changes w.r.t. BW adaptation, but we don’t see the need to explicitly capture both RF/BB split and dynamic/static split in the scaling rules, and thus, we don’t support alt 1, alt 2, or alt 3. We suggest to reflect the scaling to the total relative power value w.r.t. BW adaptation. Secondly, for the BW adaptation below 100MHz, there exist scaling rules which can be reused, and thus, we suggest to reuse the exist scaling rules as baseline, and further study new scaling rules if needed. Thirdly, for the BW adaptation above 100MHz, since it is not covered by exist scaling rules, we can study whether existed scaling rules can be reused or new scaling rules are needed. Further, for the BW adaptation above 100MHz, if we consider it is up-scaled from 100MHz according to the existed power value for 100MHz bandwidth, we don’t think the up-scaled values to larger BW than 100MHz are dependent on switching time since in any way the UE shall switch both RF and BB to fit larger BW than 100MHz, by assuming the exited values for 100MHz is based on the RF and BB setting targeted for 100MHz. Based on the above analysis, we suggest the following changes to Alt 4, we separate it into two proposals for >100MHz and <100MHz Alt 4-1: For >100 MHz: and shorter BW adaptation delay than NR If adaptation time of at least [ T ] ms is allowed, scaling by {0.325, 0.4, 0.55, 1.0, Y1, Y2} for {10, 20, 40, 100, [160], [200]} MHz Else, {Z1, Z2, Z3, Z4, Z5, Z6} for {10, 20, 40, 100, 160, 200} MHz FFS: Scaling for other target bandwidth values Alt 4-2: For <=100 MHz: and shorter BW adaptation delay than NR - The existed scaling rule is reused as baseline, i.e., If adaptation time of at least [ T ] ms is allowed, scaling by {0.325, 0.4, 0.55, 0.85,1.0, Y1, Y2} for {10, 20, 40, 80, 100, 160, 200} MHz - Study whether there is a need for reduced power value(s) than existed one(s) as below: - {Z1, Z2, Z3, 1.0 Z4, Z5, Z6} for {10, 20, 40, 100, 160, 200} MHz - FFS the applied condition for the existed set of values and new set of values if supported FFS: Scaling for other target bandwidth values |
| Spreadtrum | Just as FL said, the common part of the scaling designs lies in larger scaling factors for >100MH. The proposal can focus on the common part. For simplicity, we think that only two scaling factor values need to be defined for >100MHz. Therefore, we prefer the following alt. Alt 5: Additional scaling for >100 MHz {0.325, 0.4, 0.55, 1.0, {Y1}, Y2} for {10, 20, 40, 100, [Z], 200 } MHz |
| Ericsson | We prefer a simple scaling rule as it is difficult to converge on detailed parameters. The existing scaling rules in TR 38.840 should be reused as much as possible as considerable time was spent on the model during Rel-16. The scaling rule should also cover small BWs (e.g., <=10 MHz). A simplified version of Alt2 (RF and BB separation) can be considered. |
| TCL | A unified formula that including all potential scaling factors (including changes of time/frequency/spatial domain) can be considered for the power scaling rules. |
| Huawei, HiSilicon | We share the same view as CMCC and Samsung that Alt 1 could be the starting point for further discussion. For example, some further simplification can be considered based on Alt 1. For LG’s question, the UE’s static power can be reduced for some condition. For example, if the UE knows the scheduled BW/antenna is within a range without quick switching, UE can turn off some components (e.g., memory, processing units and etc.) or lower down the voltage to reduce the static power. The ‘static’ does not mean it is not changed. Instead, it means the part of power is always consumed no matter the UE is operating or not (i.e., standby). With different BW/antenna configuration, different static power can be assumed. Though we prefer Alt 1, to better formulate the whole proposal, we’d like also to discuss the updates on Alt 4. For vivo’s update on Alt 4, first, we don’t see the need to split this alternative into two parts, which makes the discussion too complicated. Second, the original intention is that the second set of values should be larger than the first set, since a shorter transition time applies for the first set. But vivo’s change is in the opposite way. |
| CEWiT | We share similar views with TCL for simpler design. |
| DCM | In our understanding, the main differences among Alt1-4 are based on the scaling objects being considered. Alt1 focuses on scaling regarding dynamic/static, Alt2 is scaling regarding RF/BB, Alt3 on scaling regarding both RF/BB and dynamic/static, and Alt4 on scaling regarding processing time. It would be more helpful for making progress if we can reach a consensus on the scaling objects before discussing the specific models. |
| Qualcomm | While we understand the desire for simple scaling, the existing scaling does not correct capture the UE behavior and needs to be updated with a more detailed rule. Our first preference is Alt 2. We would also be ok with Alt 3 if it is updated to capture the baseband throughput scaling with some updates to θBB . We note that regardless of the alternative, the final results could be captured as a table or a list of scaling values. We provided an example of the table represented in the appendix in our contribution (R1-2509235), which is reproduced below. |
| ZTE, Sanechips | - Regarding static+dynamic For Alt1, our proposal is not captured, wherein the static part is not scaled. Therefore, there are two alternatives for Alt1. Alt 1 (Scaling regarding static and dynamic portions): Alt1-1: P=Pstatic+μ∙Pdynamic Alt1-2: P=α∙Pstatic+β∙Pdynamic Additionally, if the static part could be scaled, the corresponding latency, e.g., from micro sleep to being active, should be increased. Therefore, we do not think Alt 1-2 make sense to us. - Regarding BB+RF Which active transmission is applied based on BB+RF separate scaling should be clarified. - Regarding combined scaling If we have clear understanding on alt 1 and alt 2, we could further consider combined scaling rule. |
| CATT | Similar as vivo, we believe that introducing some typical BW can reflect the need of RF/BB and static/dynamic split. |
| Apple | - We do not see strong need to separate static and dynamic power, or separate RF and baseband power. Even though separating these and applying different scaling rules can make sense intuitively, it is not possible to have a power model that matches different implementations very well. The existing model, in our view, is already reasonably good. Therefore, we prefer to take the existing model as the baseline. - For the proposals here, we are open to discuss case by case, without affecting the baseline model. |
| Xiaomi | - We also hope to simplify the discussion without getting overly detailed. Alt1 seems to cover multiple factors, and we are prefer to proceed to the next step based on this approach. |
| OPPO | We are OK for the discrete number for the power value. However, we should include the smaller bandwidths 3/5MHz, as well as 200MHz. Note this is not the mandated UE minimum UE capability. It is only for modeling the UE bandwidth operated in smaller bandwidth and the power consumption. |

In the above scaling design, the consideration of >100 MHz BW is one important motivation. For micro sleep, it is required that UE can back to any non-sleep state in next slot. When the non-sleep state corresponds to a 200 MHz processing or a 100 MHz processing, there is less components can be turned off during micro sleep for the 200 MHz case. In this regard, there should reasonably be larger micro sleep power consumption for a larger BW than 100 MHz. To make the model also matched to NR Redcap model regarding micro sleep power, a scaling for BW ≤ 20 MHz can be considered. By the above, the following proposal is suggested for companies’ check and comment. Note that, similar scaling may be applied to light sleep, but regarding its longer transitional, the difference may not be significant.

Proposal 3.3.3.2 (1st round; high priority):

Include the following scaling rule on micro sleep for 6G UE power consumption model:

| Scaling for FR1 | Proposal | Comment |
| --- | --- | --- |
| Scaling on micro sleep | Scaling micro sleep power by [0.4], 1, and [1.7] when the active bandwidth after transitioning out of micro sleep is ≤ [20] MHz, within ([20] MHz, [160] MHz), and ≥ [160] MHz, respectively | Applicable for FR1 and FR2. FFS: Scaling on light sleep |

Companies’ views on Proposal 3.3.3.2 (1st round)

- Note: Structure and description are critical part to consider first; values can be updated further

| Company | View |
| --- | --- |
| CMCC | Fine for the new scaling rule. |
| Samsung | OK |
| LG Electronics | In our view, the static power is not dependent on BW, by definition. One way could be to let companies report scaling value applied to static power, rather than making an agreement for this topic. |
| vivo | Considering micro-sleep is standby mode where UE is ready for either transmission or reception, it is highly related to the scaling rules for the non-sleep states w.r.t. BW adaptation. We suggest to hold on this issue until there is progress on scaling rules for the non-sleep states regarding BW>100MHz or BW<100MHz. |
| TCL | We support the scaling on micro sleep. Regarding the scaling rules, in addition to the bandwidth mentioned in the table, the number of antennas also needs to be taken into consideration. Furthermore, the power of Redcap for sleep states is lower than that in TR 38.840 due to the reduced bandwidth and Rx, considering 6G would support diverse device types, power scaling rules for all sleep states should be considered |
| Huawei, HiSilicon | We support this scaling framework, which is the same concept as the scaling on Proposal 3.3.3.1, i.e., static power reduction. In addition, for the micro-sleep case, similar impact on the transition time (i.e., the ‘T’ in Proposal 3.3.3.1) may also be considered. We can further discuss this part. |
| Qualcomm | We agree that microsleep should be scaled. Our preference is to use the scaling derived from the previous proposal if possible. |
| ZTE, Sanechips | Static power should not be scaled, otherwise, it is not static power. Or, if the static power is scaled down or up, the corresponding wake-up time also should be adjusted. For example, for a UE supporting 100MHz, the static power in micro sleep is not changed since the UE need to preheat the hardware and make sure the UE could keep short latency and turn into active transmission. If the static power is scaled down, more hardware is shut-off and the corresponding latency should be increased. |
| CATT | The reason that why we need a new rule for micro sleep is not clear, we should discuss firstly. |
| Xiaomi | Similar views as vivo. |
| OPPO | In generally OK, but need some further confirmation, after the previous proposal. |

When the processing requirement is higher, higher power values are expected in the previous scaling rules. On the other hand, then the processing requirement/time is relaxed, there can also be a lower power consumption value. The following is therefore proposed for companies’ check and comment:

Proposal 3.3.3.3 (1st round; high priority):

Include the following scaling rule on non-sleep power states for 6G UE power consumption model:

| Scaling for FR1 | Proposal | Comment |
| --- | --- | --- |
| PDSCH processing time relaxation | Scaling UE power consumption by [0.7] if [2]x or longer PDSCH processing time can be accommodated | Applicable for FR1 and FR2 |

Companies’ views on Proposal 3.3.3.3 (1st round)

- Note: Structure and description are critical part to consider first; values can be updated further

| Company | View |
| --- | --- |
| CMCC | Fine for the new scaling rule but we think this can be included on Proposal 3.3.3.1, e.g. adding a new rule for scaling on static power only, and also such definition can be extended to use case like PDCCH decoding. |
| Samsung | If the scaling rules for RF/BB are agreed upon in the static/dynamic structure, it seems that they will be naturally reflected. No need for additional agreement. |
| LG Electronics | We don’t have a strong preference/objection. However, it would be better to discuss this issue after PDSCH processing time is defined. |
| vivo | Regarding this proposal, one question is that is it assumed that 6GR baseline processing time is the same as 5G baseline, i.e. processing capability #1? Further, Whether and how multiple processing timeline will be defined/supported in 6GR is not clear yet, it is hard to discuss the power impact due to processing timeline relaxation |
| Ericsson | In TR 38.840, scaling rule exists for cross-slot scheduling which is effectively PDSCH processing time relaxation. |
| TCL | Support. |
| Huawei, HiSilicon | We are open to discuss this scaling. But we also agree with Samsung that this can be discussed after Proposal 3.3.3.1. |
| Qualcomm | We support the proposal and propose to add additional values, e.g. 3x and 4x processing time scaling and to discuss the power scaling numbers. |
| ZTE, Sanechips | The relationship between BB/RF separation and this proposal should be clarified. In our understanding, PDSCH relaxing also belong to BB scaling for PDSCH. |
| CATT | The necessity of introducing a new scaling should be discussed firstly. |
| Xiaomi | Similar comments as Samsung. |
| OPPO | OK |

The following proposal is to include the scaling for larger DL and/or UL antenna number. Analogous scaling to NR scaling rules are proposed for companies check and comment:

Proposal 3.3.3.4 (1st round; high priority):

Include the following scaling rule on non-sleep power states for 6G UE power consumption model:

| Scaling for FR1 | Proposal | Comment |
| --- | --- | --- |
| Antenna scaling (DL) | 8Rx power is [2]x 4Rx power for FR1 6Rx power is [1.4]x 4Rx power for FR1 2Rx power is 0.7x 4Rx power for FR1 1Rx power is 0.7x 2Rx power for FR2 | Assume same number of antenna elements per Rx chain |
| Antenna scaling (UL) | 2Tx (4Tx) power is 1.4x 1Tx (2Tx) power at 0dBm. 1.2x.at 23dBm, and [1.1]x.at 26dBm for FR1 (including ~7GH) | 4Tx support is not considered for FR2. |

Companies’ views on Proposal 3.3.3.4 (1st round)

- Note: Structure and description are critical part to consider first; values can be updated further

| Company | View |
| --- | --- |
| CMCC | Fine for the DL scaling rule but we think this can be included on Proposal 3.3.3.1. For UL, whether the power can be scaled needs further discussion. |
| Samsung | Does this apply to overall power or only to dynamic power? |
| LG Electronics | OK |
| vivo | We don’t see the 8RX scaling is needed in power model since it is related to FWA, for which power consumption evaluation may not be necessary. |
| Ericsson | Consistent scaling factor can be considered for doubling the number of Rx antennas. That is, 1.4x for 8Rx relative to 4Rx. For 6Rx it can be 1.2x relative to 4Rx. |
| TCL | Support. |
| Huawei, HiSilicon | We are OK to introduce addition scaling for RX. But for the values of the scaling factor is larger than our investigation. Per our understanding, the change of RX number mainly impact the dynamic power, which only a part of the overlal power. Based on our investigation, scaling factors should be ~1.1 and ~1.5 for 6RX and 8RX. For TX scaling, the necessity should be further discussed, since usually for TX states the PA is the dominant part. The impact of TX number could be marginal. |
| Qualcomm | If the antenna scaling is also accompanied by scaling of MIMO layers, there will be an impact to baseband power and we propose to use an approach similar to Proposal 3.3.3.1 |
| ZTE, Sanechips | The comment column at second row should be removed, since whether 4Tx is supported is decided by other agenda. For the exact values in the table, it could be kept open for now. |
| CATT | Similar views as above proposal, the necessity of introducing a new scaling should be discussed firstly. |
| Xiaomi | We also question the necessity of supporting 8Rx in the power model. Additionally, Alt1 of Proposal 3.3.3.1 already includes antenna scaling, making this proposal seem somewhat redundant. We can revisit whether this proposal is needed after Proposal 3.3.3.1 has been discussed. |
| OPPO | In generally OK, but need some further confirmation, after the previous proposal. |

Cell-Specific designs

PRACH and paging

Companies’ views

Companies generally agree that the periodic and widely distributed nature of legacy Paging Frames (PFs)/POs and ROs in 5G NR significantly restricts the BS's ability to enter deep sleep states. The primary solution proposed by numerous delegates (including FUTUREWEI, Nokia, Panasonic, CMCC, CATT, Apple, InterDigital, and vivo) is Clustered Provisioning in the time domain, which aggregates these signals near the SSB burst. Simulation results confirm that this clustering, particularly when combined with longer SSB periodicities (e.g., 80ms or 160ms), generates substantial NES gains by creating longer continuous periods (up to 136ms) where the BS can transition into deep sleep.

For PRACH, Nokia observed that time adaptation allows clustered ROs functionality, resulting in NES gain, and proposed that 6GR consider this PRACH adaptation by design rather than as an add-on constrained by legacy NR UEs. Several companies (NEC, ITL, NTT, CATT) proposed moving beyond the limitations of the Rel-19 PRACH adaptation (which relied on legacy configuration tables) by defining new PRACH configuration tables from scratch in 6GR to support a greater diversity of dense or "bursty RO patterns" that are optimized for NES.

For Paging, the goal is to define PF/PO occasions that are configured within a shorter period than NR and allow flexible placement of PFs/POs that are closely tied to common signals like SSB. Both PRACH and Paging mechanisms are also targeted for on-demand activation (CATT, InterDigital, TCL), where resources remain inactive by default until explicitly triggered, and the feasibility of spatial domain adaptation of PRACH resources based on UE density/beams should be studied. Finally, clustering must be balanced, as condensing ROs significantly increase RA latency for the UE.

FL comments and proposals

PRACH and paging has been iterated for two meetings without any online discussions so far. FL picks up where the discussion ended in RAN1 #122-bis with the following proposal, with PO and RO capacity scaling and a clarification being newcomers to the list:

FL Proposal 4.1.1:

Study and evaluate energy efficiency of PRACH and paging resources with respect to:

- PRACH and paging resource clustering with sync signals and SIB-1

- NW and UE energy savings potential,

- PRACH and paging periodicity and relation with sync signal or SIB-1 periodicity,

- Impact on latency and potential mitigation techniques,

- Preamble sequence set size and paging procedures,

- PO and RO capacity scaling,

- Etc.

Companies’ comments

Companies’ comments on the above proposal are appreciated.

| Company | Comment |
| --- | --- |
| CMCC | We support the proposal expect the new bullet “PO and RO capacity scaling”. This new concept needs to be clarified at first. |
| FAI | Support |
| Samsung | In general, it would be better to split the proposals into proposals for paging and RACH – there is no pre-assumption that they two shall use the same energy efficiency scheme. After splitting, “PO capacity scaling” may need further clarification. |
| vivo | We have the following comments or questions: Comment#1: We prefer to have separate proposals for RO and Paging since they may have different purposes and issues. Comment#2: For the first bullet: Q1: On clustering, there are two different understanding on clustering: one is clustering of PRACH/Paging resources; the other is clustering with sync signals and SIB-1. Which understanding is included here or both understanding? Q2: Is the intention to only consider clustered RO/PO? For example, time uniformed PO will not be considered in 6GR? Comment#3: For the second bullet: There may not be UE energy saving here. We suggest the following updates: NW energy saving potential and UE power consumption impact Comment#4: For the last bullet: What does RO and PO capacity scaling mean here? What’s the relation with NES? |
| Spreadtrum | We general are fine with the proposal. But the bullet of “PO and RO capacity scaling” is not clear for us. |
| Ericsson | Agree with CMCC about “PO and RO capacity scaling”. Also ok to split into one proposal for paging and another for RACH. The proposals can be simplified and focus on the energy efficiency aspects, other aspects will anyway be studied in initial access agenda. |
| Google | Generally fine with the proposal, but we are also unsure about the RO and PO capacity scaling. |
| TCL | Agree with this proposal. |
| CEWiT | Support this proposal. |
| DCM | We would like to clarify what does PO and RO capacity scaling mean. If this is about adaptation of RO/PO resources, we think it would be better to capture as such. If this is about anything other than that, we would like to clarify that. Also, we do not think that these techniques should combine when studying, but to speed up our discussion, this formulation can be fine with adding note saying that this proposal does not mean that PO and RO should be studied altogether. Our proposal, Study and evaluate energy efficiency of PRACH and paging resources with respect to: - PRACH and paging resource clustering with sync signals and SIB-1 - NW and UE energy savings potential, - PRACH and paging periodicity and relation with sync signal or SIB-1 periodicity, - Impact on latency and potential mitigation techniques, - Preamble sequence set size and paging procedures, - PO and RO adaptation in time domain, - PO and RO capacity scaling, - Etc. Note: PRACH and Paging study do not necessarily have to consider altogether. |
| Qualcomm | The last 2 bullets are unclear; more clarification is needed or they can be removed for now. We suggest the following update. FL Proposal 4.1.1: Study and evaluate energy efficiency of PRACH and paging resources at least with respect to: - PRACH and paging resource clustering with sync signals and SIB-1 - NW and UE energy savings potential, - PRACH and paging periodicity and relation with sync signal or SIB-1 periodicity, - Impact on RACH latency and potential mitigation techniques, - Impact on paging performance and potential mitigation techniques, - Preamble sequence set size and paging procedures, - PO and RO capacity scaling, Etc. |
| ZTE, Sanechips | We are OK with the proposal with some modifications. Study and evaluate energy efficiency of PRACH and paging resources with respect to: - PRACH and paging resource clustering with sync signals and SIB-1 - NW and UE energy savings potential, - PRACH and paging periodicity and relation with sync signal or SIB-1 periodicity, - Impact on latency and potential mitigation techniques, - Preamble sequence set size and paging procedures, - PO and RO capacity scaling, - The relationship between PO and RO - The PO/RO adaptation mechanism base on SSB adaption - Etc. |
| CATT | On-demand activation of RO/PO and RO/PO adaptation should both be studied in 6GR, which is benefit to mitigate the impact on latency. They should be included in the proposal. In addition, the concept of “PO and RO capacity scaling” is not clear to us. |
| Apple | We would like to understand what is the detailed mechanism of PO and RO capacity scaling. |
| Sharp | It is not clear for the new item ‘PO and RO capacity scaling’. We need clarify it. |
| Xiaomi | We agree with Samsung’s modification and echo companies view that ‘PO and RO capacity scaling’ is lack of clarity. Furthermore, we are not quite sure how preamble set and paging procedure are related to energy efficiency. Further clarification is appreciated. Considering the third sub-bullet, it may be better to delete the first sub-bullet to leave thing open and future proof. |
| OPPO | In the agreement copied below made in the last meeting, clustered provisioning of RO/PO has already been captured, seems the first 4 sub-bullets are talking about the same thing. In addition, we also think “PO and RO capacity scaling” is unclear. ” Agreement Study and evaluate NW energy savings and the impact on UE performance and user experience with respect to 20ms and longer periodicities of sync signal(s) at least for initial access with the following consideration, but not limited to: BS assumptions: - Cell-common signaling (e.g., sync signal(s), broadcast PDCCH, SIB-1, SIB, paging, PRACH), e.g., - Clustered provisioning of different cell-common signaling, - On-demand provisioning of different cell-common signaling, - UE-specific signaling (for low, light, medium loads), e.g., - Clustered provisioning with cell-common signaling, - Unclustered provisioning with cell-common signaling, …… |
| Futurewei | Agree with companies suggestion to split the discussion on Paging and RACH and to further clarify the meaning of “PO and RO capacity scaling”. |
| NEC | We support this study along with the split companies are suggesting on paging and RACH. We also see a need to evolve mechanisms like Cell DTX/DRX to be tightly coupled with the adaptation of common channels and signals, including PRACH. Therefore, studying the clustering and joint adaptation of these resources is critical. |
| Panasonic | We are in general okay with the proposal. Regarding the last bullet of “• PO and RO capacity scaling,”, as the meaning and details are not clear, we prefer to start from PO and/or RO adaptation. Then how it impacts the capacity and how it may achieve the gain can be further discussed. |

Cell DTX/DRX

Companies’ views

Companies view Cell DTX/DRX as a core NES mechanism that requires significant enhancement and expansion in 6GR beyond its restrictive 5G scope. There is a consensus that the primary limitation of the 5G implementation was its restriction to RRC_CONNECTED state UEs, along with its inability to mute periodic common signals like SSB, SIB-1, and PRACH, which prevents the BS from entering deep sleep states. Consequently, many companies (FUTUREWEI, CMCC, LG, OPPO, Spreadtrum, ZTE, CEWiT, WILUS, CATT, Fujitsu, and HONOR) explicitly propose studying the extension of Cell DTX/DRX applicability to all RRC states, often recommending that common signals be turned off or adapted/clustered during the Cell DTX non-active period to maximize NES gain. A central design principle for 6GR is the joint/aligned design of Cell DTX/DRX and UE DRX, which is necessary to avoid complexity, reduce signaling overhead, and ensure mutual energy benefits for both the network and the UE; however, Ericsson proposed that Cell DTX/DRX should not be used as a common tool for time-domain adaptations across both Idle/Inactive and Connected modes due to differing transmission characteristics, and instead, the alignment mechanisms should be led by RAN2. Furthermore, vivo points out that the introduction of the LP-WUS is recognized as key to achieving joint EE, as it can be utilized for the activation/deactivation/adaptation of the Cell DTX/DRX pattern itself, providing a dynamic control mechanism that is decoupled from rigid C-DRX timers and allows for deeper sleep states when traffic is low.

FL comments and proposals

The Cell DTX/DRX topic has been iterated for two meetings without any online time for discussion. FL repeats the last proposal from RAN1 #122-bis as a starting point for the discussion, with an addition that applies to both UEPS and NES:

FL Proposal 4.2.1:

Study joint design of Cell DTX/DRX and UE DRX regarding, e.g.,

- Applicability to IDLE/INACTIVE resp. CONNECTED state,

- Signals and channels that are affected (turned off) during non-active duration, including idle/inactive and connected modes,

- Common (idle mode) signal adaptation and clustering during inactive duration,

- Impact on UE latency and synchronization, and other performance in connected mode,

- Flexible DTX/DRX pattern,

- Interference issues in multi-TRP scenarios,

- Fast UL wake-up mechanism for uplink data reception during gNB inactive period,

- Lower-layer features for UE PDCCH monitoring and measurement operations.

Companies’ comments

Companies’ comments on the above proposal are appreciated.

| Company | Comment |
| --- | --- |
| CMCC | We support to further discuss such joint configuration for both NW and UE energy saving, and we also think that instead of discuss case by case, it is recommended to consider the joint configuration of NES feature and UE PS feature as a universal structure, i.e., to consider a mutual beneficial feature combination framework that can enable to different kinds of scenarios. Therefore, we suggest the following revision: FL Proposal 4.2.1-rev1: Study joint design of Cell DTX/DRX and UE DRX regarding, e.g., - Applicability to IDLE/INACTIVE resp. CONNECTED state, - Signals and channels that are affected (turned off) during non-active duration, including idle/inactive and connected modes, - Common (idle mode) signal adaptation and clustering during inactive duration, - Impact on UE latency and synchronization, and other performance in connected mode, - Flexible DTX/DRX pattern, - Interference issues in multi-TRP scenarios, - Fast UL wake-up mechanism for uplink data reception during gNB inactive period, - Lower-layer features for UE PDCCH monitoring and measurement operations. - Extend such joint configuration mechanism to a mutual beneficial feature combination framework. |
| FAI | Support |
| Samsung | 1) At this stage of the SI, it is not clear that we should study joint operation of cell DTX/DRX and UE DRX, as this presumes that both features will be supported, It is better to reword the main sentence of the proposal to remove UE DRX and add as one of the features to consider for cell DTX/DRX Study joint design of Cell DTX/DRX and UE DRX regarding, e.g. considering the following aspects, - UE DRX operation, - … 2) Not clear what is the intention of “Fast UL wake-up mechanism”, is this for cell DRX or cell DTX or both. In case of cell DRX how will the base station receive the wake-up signal if it is in cell DRX? Instead, above bullet of “Impact on UE latency and synchronization” can cover this concerned bullet. |
| vivo | We have the following comments: Comment#1: we don’t think cell DTX/DRX must be working together with UE DRX. So we suggest to put joint operation with UE DRX in sub-bullet. Comment#2: For the third sub-bullet, inactive here should be active? Comment#3: What does flexible DTX/DRX pattern mean here? Does it mean adaptation of DTX/DRX patterns? Comment#4: What’s the interference issues mean? Suggest to remove this Comment#5: Not clear what does the last bullet mean. So we suggest the following updates: Study joint design of Cell DTX/DRX and UE DRX regarding, e.g., - Applicability to IDLE/INACTIVE resp. CONNECTED state, - Signals and channels that are affected (turned off) during non-active duration, including idle/inactive and connected modes, - Common (idle mode) signal adaptation and clustering during inactive duration, - Impact on UE latency and synchronization, and other performance in connected mode, - Flexible DTX/DRX pattern, - Interference issues in multi-TRP scenarios, - Fast UL wake-up mechanism for uplink data reception during gNB inactive period, - Lower-layer features for UE PDCCH monitoring and measurement operations. |
| Nokia | For the main bullet, we can make it more genetic by saying “Study of cell DTX/DRX for 6G EE”, anything related to UE DRX will be discussed later after RAN2 had made related discussions/decisions, including the joint design of cell DTX/DRX and UE DRX. Let’s first focus on the design of cell DTX/DRX for 6G EE without the impact of UE DRX. |
| Spreadtrum | We are fine with the proposal. |
| Ericsson | This study should be driven in RAN2. RAN1 should focus on the integration of lower-layer UE transmission/reception-related features including PDCCH monitoring and measurement operations into the DRX/DTX framework |
| Google | The following subbullet confuses us. Perhaps better wordings would be “mechanisms of fast waking up NW for uplink data reception during NW inactive period”. At least, we should not use “gNB” since we are talking about 6G. In addition, we feel it is more related to the section 4.3 UL WUS. - Fast UL wake-up mechanism for uplink data reception during gNB inactive period, |
| TCL | Thanks for FL’s effort on this proposal. We support this proposal generally. For 5th bullet, it seems not clear for us how to define flexible DTX/DRX pattern. In our understanding, switching from one DTX/DRX pattern to another DTX/DRX pattern needs to consider special conditions or related configurations, which also will be discussed in RAN 2. |
| CEWiT | Fine with the proposal in general. However, the sub-bullet “Lower-layer features for UE PDCCH monitoring and measurement operations.” is not clear to us and needs clarification. |
| DCM | First of all, we think that the discussion on Cell DTX/DRX and UE DRX should be done in RAN2 since this includes the optimization on UE DRX as well, which was mainly discussed in RAN2 in NR. In operator’s perspective, UE DRX has already been deployed, so this feature should be a baseline. However, cell DTX/DRX should be reviewed in studying necessity of separated features and potential extension. In that sense, this discussion should be made in RAN2 domain. For the last bullet, we would like to clarify the intention of it. If this bullet tries to capture how to reduce RS transmission from NW and to relax UE measurements during non-active period, we are fine with this bullet. Study joint design of Cell DTX/DRX and UE DRX regarding, e.g., - Applicability to IDLE/INACTIVE resp. CONNECTED state, - Signals and channels that are affected (turned off) during non-active duration, including idle/inactive and connected modes, - Common (idle mode) signal adaptation and clustering during inactive duration, - Impact on UE latency and synchronization, and other performance in connected mode, - Flexible DTX/DRX pattern, - Interference issues in multi-TRP scenarios, - Fast UL wake-up mechanism for uplink data reception during gNB inactive period, Lower-layer features for UE PDCCH monitoring and measurement operations. |
| Qualcomm | We suggest simplifying the proposal as follows: FL Proposal 4.2.1: Study joint design of Cell DTX/DRX and UE DRX regarding, e.g., - Applicability to RRC state(s) IDLE/INACTIVE resp. CONNECTED state, - UE behaviors during the inactive time of cell DTX/DRX and/or inactive time of UE DRX. - Signals and channels that are affected (turned off) during non-active duration, including idle/inactive and connected modes, - Common (idle mode) signal adaptation and clustering during inactive duration, - Impact to UE/NW - Impact on UE latency and synchronization, and other performance in connected mode, - Flexible DTX/DRX pattern, - Interference issues in multi-TRP scenarios, - Fast UL wake-up mechanism for uplink data reception during gNB inactive period, - Lower-layer features for UE PDCCH monitoring and measurement operations. |
| ZTE, Sanechips | We are OK to discuss the cell DTX/DRX and UE DRX. However, it is noted that the joint design of Cell DTX/DRX and UE DRX is a specific technical scheme and can be further discussed only when both cell DTX/DRX and UE DRX are considered. Before discussing the joint design, the scheme of the cell DTX/DRX and clustering transmission should be clarified. If cluster transmission is specified and Cell DRX/DTX is not needed, then the joint design of cell DRX/DTX and C-DRX does not exist. Based on this, the proposal is suggested as follows: Study and evaluate joint design of Cell DTX/DRX and UE DRX regarding, e.g., - Applicability to IDLE/INACTIVE resp. CONNECTED state, - Signals and channels that are affected (turned off) during non-active duration, including idle/inactive and connected modes, - Common (idle mode) signal adaptation and clustering during inactive duration, - Impact on UE latency and synchronization, and other performance in connected mode, - Impact on UE DRX - The relationship with clustered transmission and adaptation of SSB\PO\RO - Flexible DTX/DRX pattern, - Interference issues in multi-TRP scenarios, - Fast UL wake-up mechanism for uplink data reception during gNB inactive period, - Lower-layer features for UE PDCCH monitoring and measurement operations. |
| CATT | We have following comments: Comment#1: Please clarify the meaning of “resp.”. Is that Cell DTX/DRX in Connected state is extended to IDLE/INACTIVE state? Does it contain the pattern of Cell DTX/DRX in Connected state and/or the activation mechanism? The mechanism of Cell DTX/DRX in CONNECTED state can be extended to IDLE/INACTIVE state. However, detailed parameter and UE behaviors in IDLE/INACTIVE state should be further discussed, but not directly applied. Comment#2: For the signals and channels during non-active duration might be turned off or with the sparse periodicity or clustered transmission for adaptation, thus, “turned off” is only an example. Comment#3: The wording “non-active duration” in the second sub-bullet and “inactive duration” in the third sub-bullet should be aligned. Thus, we preferred updated proposal as following: FL Proposal 4.2.1: Study joint design of Cell DTX/DRX and UE DRX regarding, e.g., - Applicability to IDLE/INACTIVE resp. CONNECTED state, - Signals and channels that are affected (e.g. turned off or sparse periodicity) during non-active duration, including idle/inactive and connected modes, - Common (idle mode) signal adaptation and clustering during inactive non-active duration, - Impact on UE latency and synchronization, and other performance in connected mode, - Flexible DTX/DRX pattern, - Interference issues in multi-TRP scenarios, - Fast UL wake-up mechanism for uplink data reception during gNB inactive period, Lower-layer features for UE PDCCH monitoring and measurement operations. |
| Apple | We prefer to separate the discussion for CONNECTED and IDLE/INACTIVE mode. In general, we are supportive of joint design of cell DTX/DRX and UE DRX/DTX in CONNECTED mode. However, for IDLE/INACTIVE mode, we think this is naturally determined by configuration of PRACH and Paging. We suggest to the following updates to discuss separately for CONNECTED and IDLE/INACTIVE mode: Study joint design of Cell DTX/DRX and UE DTX/DRX in CONNECTED mode regarding,e.g. - Applicability to IDLE/INACTIVE resp. CONNECTED state - Signals and channels that are affected (e.g. turned off, sparse tx/rx) during non-active duration - Common (idle mode) signal adaptation and clusteringduring inactive duration, - Impact on UE latency and synchronization, and other performance in connected mode, - Flexible DTX/DRX pattern, - Interference issues in multi-TRP scenarios, - Fast UL wake-up mechanism for uplink data reception during gNB inactive period, Study applicability of cell DTX/DRX impact to common signals/channels for UEs in IDLE/INACTIVE mode, |
| Xiaomi | We are not sure how the last three sub-bullets connect to energy efficiency. More clarification is appreciated. |
| OPPO | OK |
| Futurewei | We are OK |
| NEC | We support this proposal. In our view, since Cell DTX/DRX and UE C-DRX are configured separately in NR, it can be difficult to ensure their active periods overlap appropriately. For 6G, it is beneficial to jointly design the two features, for instance by configuring them jointly in a single configuration. This would allow the durations and positions of the overlapping active time to be controlled well to better fit traffic characteristics. |
| Panasonic | We are in general okay to study joint design for Cell DTX/DRX and UE DRX. However, we would like to stress following aspects: - We are okay to study a unified framework for IDLE and CONNECTED state. But not sure yet how to apply a same pattern/setting to both RRC states. Thus, the first bullet needs to be clarified and updated to: - Applicability to IDLE/INACTIVE and/or CONNECTED states - On the 4th bullet, as latency may potentially impacted for all kinds of service, just limiting to “UE latency” and “connected mode” is not preferable. We propose to modify to - Impact on latency, synchronization, and other performance, if applicable - On the 5th bullet, we do not understand how to interpret the “flexible DTX/DRX pattern” at this moment considering it may imply both Cell and UE side and even both RRC states. Therefore, we prefer the below update: - DTX/DRX pattern adaptation - On the 6th bullet, we think we should look at the scenario first and then to see whether any issue. - Whether/how to apply to multi-TRP scenarios and potential issues. - We do not think the 7th bullet regarding fast UL wake-up is needed for this proposal, as it is too detailed level and can anyway be studied as part of the scheme. - On the last bullet, PDCCH monitoring aspect is already covered by the second bullet so no need to mention. For measurement, we think it should be studied but no need to highlight whether it is low-layer or high-layer measurement, as L1/L3 measurement (in NR context) may both be affected. So we propose to change to:  Measurement operations |

UL WUS

Companies’ views

Some companies view UL WUS as a vital mechanism for NES), designed to allow a BS, operating in a deep sleep state, to detect a UE's need for communication using a specialized BS LPR/WUR/LP mode. The primary purpose of UL WUS is to enable UE-triggered on-demand provisioning of common signals, specifically to request OD-SSB and OD-SIB1, particularly when the network has adopted longer periodicities for these broadcasts, see Figure 2. While the largest energy gains are anticipated when used by UEs in RRC Idle or Inactive states during initial access or cell reselection, its applicability is generally studied for UEs in any RRC state, though Ericsson questioned its utility for connected mode UEs since the network is already partially active. However, the use of UL WUS presents several key challenges, especially in standalone cell scenarios where configuration cannot be obtained from an assisting cell. Challenges include designing a lightweight mechanism to efficiently provide the UL WUS configuration to the UE before it has received SIB1/SSB, ensuring the UE can achieve necessary downlink time/frequency synchronization for UL WUS transmission when periodic SSB is absent, and mitigating the inherent trade-off involving increased UE power consumption due to the need to transmit the signal and potentially increased access latency due to the time required for the MR to wake up.

Figure 2: Illustration of standalone OD-SIB1 case1a (left) and case2a (right). (vivo)

FL comments and proposals

In FL’s view, RAN1 must first have a common understanding of the purpose with UL WUS before discussing it further. This includes what actions are to follow upon receiving the UL WUS and in what RRC states a UE can transmit the UL WUS. In this respect, FL’s view is that UL WUS may be used at least for requesting SIB-1 by UEs in idle mode, possibly also SSBs although thar use case is weaker due to the inherent transmission issues for a UE that is out of sync. A sync signal request may also be useful for UEs in connected mode, but the underlying issues may also be addressed, e.g., by mandating more frequent sync signals for UEs in connected mode.

FL Proposal 4.3.1:

Study UL WUS for requesting on-demand sync signals and/or on-demand SIB-1, at least for UEs in RRC IDLE during initial access and cell reselection.

Companies’ comments

Companies’ comments on the above proposal are appreciated.

| Company | Comment |
| --- | --- |
| CMCC | Support the proposal |
| FAI | Support |
| Samsung | Postpone until the applicable scenarios are discussed, the SSB design details are known, and respective NES over periodic signaling can be evaluated. |
| vivo | Inactive state should also be included here. So we suggest the following updates: Study UL WUS for requesting on-demand sync signals and/or on-demand SIB-1, at least for UEs in RRC IDLE/INACTIVE during initial access and cell reselection. |
| Nokia | OK |
| Ericsson | At RAN1#122-bis it was agreed to study both on-demand sync signals and on-demand SIB-1. Do we need this proposal? |
| Google | We are supportive of this proposal. |
| TCL | Except for the function of requesting on-demand sync signals and/or on-demand SIB-1, other potential functions should not excluded in 6G day-1. Thus, we suggest to revise the main bullet as Study UL WUS at least for requesting on-demand sync signals and/or on-demand SIB-1, at least for UEs in RRC IDLE during initial access and cell reselection. |
| CEWiT | Share similar views with TCL |
| DCM | In our view, we should at least study UL WUS for requesting on-demand SIB-1, at least for UEs in RRC IDLE as this has proved to be positive effect on NES during Rel-19 discussion. On the other hand, for the case of UL WUS for OD-SSB we should first discuss what scenarios to be applicable for the use of this technique. Also, we should study NW triggered based OD-SSB transmission. |
| Qualcomm | We are fine with the proposal |
| ZTE, Sanechips | Similar update as vivo. |
| CATT | We are OK to study UL WUS, but the purpose of UL WUS can be further discussed. FL Proposal 4.3.1: Study and evaluate the UL WUS operation for NW for 6GR EE improvement, following UL WUS purpose can be considered: - UE-triggered NW wake-up considering, e.g., PRACH reception - requesting on-demand sync signals and/or on-demand SIB-1, at least for UEs in RRC IDLE during initial access and cell reselection |
| Apple | Similar view as SS. |
| Sharp | OK. |
| Xiaomi | Considering we already have agreement saying we need to study on-demand signal and channel, it seems UL WUS study is already part of it. This proposal can be deprioritized. |
| OPPO | Support |
| Huawei, HiSilicon03 | We suggest the following modifications. Study UL WUS for requesting on-demand sync signals and/or on-demand SIB-1, UE identification and scheduling with BS low power consumption, RACH response considering different RRC states at least for UEs in RRC IDLE during initial access and cell reselection. |
| Futurewei | We are OK |
| NEC | Support. UE-triggered on-demand SSB is a promising solution, and a potential mechanism for this is via an UL-WUS. We propose that the study of UE-triggered on-demand SSB should investigate the design of a UL-WUS, mechanisms for providing its configuration to UEs, and procedures for achieving initial downlink synchronization for its transmission. |
| Panasonic | The study should focus on the scenarios and procedures first rather than the UL-WUS itself. So, we propose following updates: Study scenarios and procedures for requesting on-demand sync signals and/or on-demand SIB-1, at least for UEs in RRC IDLE during initial access and cell reselection. |

Multi-carrier and CA

Companies’ views

Some companies’ view is that the existing CA framework in 5G, with its one-to-one mapping between a cell and a carrier, is inefficient for 6GR NES because it requires multiple carriers to transmit always-on common signals (vivo). The fundamental consensus is that 6GR must minimize the number of carriers/cells transmitting these signals (Samsung), leading to a focus on the Sync signal-less carriers/cells/TRPs mechanism (Ofinno, CMCC, Fraunhofer, OPPO, Ericsson, KT). This concept should be retained and standardized. To achieve this, companies widely support adopting an Anchor/Non-Anchor or Coverage/Capacity Carrier Structure (Futurewei, vivo, CATT, Lenovo, Samsung, Ofinno, MediaTek, TCL, CAICT), where the Anchor/Coverage carrier (often a low-frequency carrier) handles essential synchronization and system information transmissions, allowing the Non-Anchor/Capacity carriers (often high-bandwidth SCells) to turn off their TX/RX components or transmit only long-period AO-SSs by default, see Figure 3. To maximize the operational benefits of this structure, fast SCell activation and deactivation/dormancy procedures are necessary (vivo, CMCC, CATT, Samsung, Apple, TCL), allowing capacity carriers to be swiftly brought online when traffic demands spike and returned to sleep promptly; however, NR procedures for this feature often had duplications and limitations. Furthermore, enhancing energy efficiency involves dynamically adapting carrier roles, with several proposals suggesting the support of UL-only cell and DL-only cell configurations (vivo, ZTE, TCL), where the network can dynamically allocate carriers as full duplex, DL-only, or UL-only based on traffic needs. Finally, enhancements also include the study of Single Cell with Multiple Carriers (SCMC) as an alternative framework to legacy CA, which allows multiple carriers to map to the same logical cell, reducing overhead and enabling common signal/channel sharing only on one carrier.

Figure 3: CA framework in idle/inactive mode. (ZTE)

FL comments and proposals

In RAN1 #122-bis, RAN1 made the following agreement:

Agreement

Study and evaluate multi-carrier/cells/TRPs mechanisms for 6GR NES, considering, e.g.,:

- Sync signal-less carriers/cells/TRPs for at least intra-band and collocated inter-band multi-carrier/cell/TRPs, including potential extensions to additional deployments and scenarios,

- RRC states,

- UE energy consumption and complexity,

- Other mechanisms/aspects/signals/channels are not precluded.

Companies have been discussing a follow-up to the above agreement focusing on an Anchor/Non-Anchor or Coverage/Capacity Carrier Structure. Without preceding multi-carrier specification work too much, FL thinks that a more specific proposal regarding this concept can be justified. Hence, FL proposes the following:

FL Proposal 4.4.1:

Study and evaluate anchor cell/carrier provisioning of common signals/channels on behalf of a non-anchor cell/carrier for NES, considering e.g.,

- Common signals/channels to be provided by the anchor cell/carrier,

- Fast activation/deactivation of Non-Anchor/Capacity carriers,

- Dynamic adaptation of carrier directivity (DL-only/UL-only/TDD)

Companies’ comments

Companies’ comments on the above proposal are appreciated.

| Company | Comment |
| --- | --- |
| CMCC | Support in general. Moreover, our understanding is that such deployment and mechanism can also be easily extended to muti-TRP scenario. Therefore, we suggest the following revision: FL Proposal 4.4.1-rev1: Study and evaluate anchor cell/carrier/TRP provisioning of common signals/channels on behalf of a non-anchor cell/carrier/TRP for NES, considering e.g., - Common signals/channels to be provided by the anchor cell/carrier/TRP, - Fast activation/deactivation of Non-Anchor/Capacity carriers/TRPs, - Dynamic adaptation of carrier directivity (DL-only/UL-only/TDD) |
| FAI | Support |
| Samsung | Do not support dynamic adaptation of carrier directivity. Need to focus on solutions that are likely to be implemented. |
| vivo | We have the following comments: Comment#1: We think anchor cell/carrier is providing configuration of common signals/channels instead of the signals/channels themselves. Comment#2: For the last bullet, it may be too early to say dynamic adaptation. Suggest to remove dynamic. So, we suggest the following updates: Study and evaluate anchor cell/carrier provisioning of common signals/channels configurations on behalf of a non-anchor cell/carrier for NES, considering e.g., - Common signals/channels configurations to be provided by the anchor cell/carrier, - Fast activation/deactivation of Non-Anchor/Capacity carriers, - Dynamic adaptation of carrier directivity (DL-only/UL-only/TDD) |
| Nokia | Please clarify how the third bullet point relate to the main bullet point? |
| Spreadtrum | We are fine with the proposal. |
| Ericsson | Do we need this proposal? We already have agreements to study: - On-demand SIB1 (which includes the deployment case where an anchor cell provides SIB1 of a non-anchor cell) - On-demand sync-signal (e.g. for fast activation/deactivation of non-anchor cell) Sync-signal less carriers (where a reference anchor cell provides the sync) |
| Google | Support in principle. But we should add a note stating that naming of anchor/non-anchor carriers and coverage/capacity carriers is just for discussion purpose. |
| TCL | Generally okay with this proposal. For second bullet, one of key to ensure fast activation/deactivation of Non-anchor/capacity carriers is to consider traffics at both of NW and UE sides. In our understanding, it seems another issue that should not included in this proposal due to main bullet focus on the discussion of anchor cell/carrier provisioning. |
| CEWiT | Support in general. |
| DCM | For the last bullet, we do not think it is necessary to include TDD. When we dynamically change the TDD for a certain cell/carrier, this causes significant interferences to other cells/carriers. Therefore, introducing such technique will cause severe damage to the entire network operation. |
| Qualcomm | We do not support the proposal. There is no definition for terminologies such as anchor/non-anchor cell/carrier in 3GPP. We suggest further discussing the motivation and what to achieve that leads to this proposal. |
| ZTE, Sanechips | We are OK with minor modification. Study and evaluate anchor cell/carrier provisioning of common signals/channels on behalf of a non-anchor cell/carrier for NES, considering e.g., - Common signals/channels to be provided by the anchor cell/carrier, - Fast activation/deactivation of Non-Anchor/Capacity carriers/cells, - Dynamic adaptation of carrier directivity (DL-only/UL-only/TDD) |
| CATT | We prefer to add TRP in the proposal, which is similar as the agreement of the last meeting. And not to support dynamic adaptation of carrier directivity. |
| Apple | We have already agreed to study sync signal-less carriers in the previous meeting so it is not clear what is additionally studied here. For fast activation/deactivation of SCells, we do no think this is limited to the the case where anchor cell is providing common signals for the non-anchor cell. We suggest to consider a separate proposal to study the fast activation/deactivation of SCells. |
| Xiaomi | Agree with Samsung. Furthermore, with first two subbullets, what is the difference between PCell/SCell and Anchor cell/Non-anchor cell? If no difference, we don’t see the necessity to have this proposal. |
| OPPO | It is better to clarify that the “common signals/channels” includes at least sync signal. It is unclear whether carrier direction can be dynamically changes as they are restricted by regional spectrum regulations, therefore, we also suggest to remove the last sub-bullet. |
| Huawei, HiSilicon03 | Support in general as well as CMCC update. |
| Futurewei | OK with proposal, but prefer to remove “Dynamic” |
| NEC | We support studying this topic. |
| Panasonic | We believe this feature is for both UE and network EE, as it also reduce the UE side efforts and power consumption on measurement and synchronization. We are also okay with the proposals from CMCC on TRP and from DCM on TDD. Therefore, Study and evaluate anchor cell/carrier/TRP provisioning of common signals/channels on behalf of a non-anchor cell/carrier/TRP for NES and UE power saving, considering e.g., - Common signals/channels to be provided by the anchor cell/carrier/TRP, - Fast activation/deactivation of Non-Anchor/Capacity carriers/cells/TRPs, - Dynamic adaptation of carrier directivity (DL-only/UL-only/TDD) |

SSB

Companies’ views

Companies widely agree that the mandatory 20 ms SSB periodicity assumed by UEs for initial cell search in 5G NR is a fundamental bottleneck that prevents BS from entering deeper sleep states during low or zero traffic, severely limiting NES. Consequently, there is consensus to study and evaluate longer periodicities of sync signals. Proposals frequently target increasing the default SSB periodicity for 6GR Initial Access to 80 ms or 160 ms, with results showing potential NES gains reaching 77% to 85% for BS CAT 1 in unloaded scenarios; however, this benefit is significantly smaller for the CAT 2 BS model. Companies acknowledge that this extension significantly increases UE cell search latency, complexity, and power consumption. To mitigate these adverse impacts, companies propose: Clustered Provisioning of SSB bursts (i.e., multiple repetitions within a short time window, see Figure 4) to maximize instantaneous detection probability and reduce buffer requirements; implementing OD-SSB, often triggered by the UE, to provide rapid synchronization when required, applicable to all RRC states and cell types; defining a Sparser Synchronization Raster to reduce the frequency domain search complexity for the UE; and adopting SSB adaptation to allow dynamic adjustment of periodicity based on traffic load. Furthermore, proposals include decoupling the essential PSS/SSS synchronization signals from the longer-periodicity PBCH transmission to enhance energy efficiency while maintaining rapid synchronization capabilities.

Figure 4: Example of 3 SSB repetitions within 5 ms. (OPPO)

Summary and Discussion

In RAN1 #122-bis, RAN1 made the following agreement:

Agreement

Study and evaluate NW energy savings and the impact on UE performance and user experience with respect to 20ms and longer periodicities of sync signal(s) at least for initial access with the following consideration, but not limited to:

BS assumptions:

Cell-common signaling (e.g., sync signal(s), broadcast PDCCH, SIB-1, SIB, paging, PRACH), e.g.,

  - Clustered provisioning of different cell-common signaling,

  - On-demand provisioning of different cell-common signaling,

UE-specific signaling (for low, light, medium loads), e.g.,

  - Clustered provisioning with cell-common signaling,

  - Unclustered provisioning with cell-common signaling,

UE impact:

Cell search complexity and latency, including frequency search latency,

UE Power consumption,

Sync signal detection, coverage and tracking performance,

RRM, mobility,

Beam management,

Other properties are not precluded,

Improvements to address identified impact, e.g.,

  - Additional sync signal needs,

  - Adaptation of sync signal transmission periodicity,

  - Sparser synch raster.

Although there are solid technical justifications for the proposals in this area, it is FL’s view that they should not be discussed as part of EE but belong to the more detailed work with initial access. Hence, FL makes the following conclusion:

FL Conclusion 4.5.1:

Companies are encouraged to bring their detailed views on SSB design for extended SSB periodicity to the initial access AI starting in RAN1 #124.

Companies’ comments

Companies’ comments on the above proposal are appreciated.

| Company | Comment |
| --- | --- |
| CMCC | Support the proposal. |
| FAI | Support |
| Samsung | OK with FL’s conclusion. |
| vivo | Support the proposal |
| Spreadtrum | We are fine with the proposal. |
| Ericsson | OK |
| Google | OK with the conclusion, but should we change “SSB” to “sync signal”? |
| TCL | OK |
| CEWiT | Support |
| DCM | support |
| Qualcomm | We are fine with the proposal |
| ZTE, Sanechips | OK |
| CATT | OK |
| Apple | OK |
| Sharp | OK |
| Xiaomi | Support. |
| OPPO | Support |
| Futurewei | OK |

SIB-1 enhancements

Companies’ views

Companies generally agree that the frequent, mandatory periodic SIB-1 transmission, aligned with a 20ms SSB periodicity, must be minimized or replaced, as it is a major factor preventing BSs from entering deep sleep states and severely limits NES potential. RAN1 agreed to formally study and evaluate on-demand and/or periodic SIB-1 transmission regarding its NES potential, acquisition delay, complexity, and coverage. The core enhancement proposed is the extension of the OD-SIB1 mechanism—which showed NES gains of up to approximately 40% in simulation results—to be supported in Standalone cell/carrier scenarios, overcoming the 5G NR limitation that required an assisting cell (Cell A) to provide the necessary UL WUS configuration. To facilitate standalone OD-SIB1, proposals suggest embedding the necessary UL WUS configuration information within the SSB structure. Furthermore, SIB-1 transmission should be clustered with other cell-common signals (SSB, PRACH, Paging) to maximize contiguous sleep periods, and enhancements are sought to allow UE assistance information (e.g., indicating preferred SSB beams) to trigger OD-SIB1 transmissions, thereby conserving energy by optimizing beam transmission while mitigating coverage risks for cell-edge UEs, cf. Figure 5.

Figure 5: 160ms SSB period and clustered common signal. (Huawei)

Summary and discussion

In RAN1 #122-bis, RAN1 made the following agreement:

Agreement

Study and evaluate on-demand and/or periodic SIB-1 transmission with respect to

NW energy savings potential and UE power consumption impact,

SIB-1 acquisition delay,

NW and UE complexity,

Coverage,

Applicable deployment scenarios, e.g.,:

  - Standalone cell/carrier,

  - Multiple TRPs/cells/carriers.

It is FL’s view that much of the discussion is either included in other topics or is on such a level of detail that it belongs to the more detailed study of SIB-1. Hence, FL makes the following conclusion:

FL Conclusion 4.5.1:

Companies are encouraged to bring their detailed views on SIB-1 design to the initial access AI starting in RAN1 #124.

Companies’ comments

Companies’ comments on the above proposal are appreciated.

| Company | Comment |
| --- | --- |
| CMCC | Support the proposal. |
| FAI | Support |
| Samsung | OK |
| vivo | Support the proposal |
| Spreadtrum | We are fine with the proposal. |
| Ericsson | OK |
| Google | Support this FL proposal. |
| TCL | OK |
| CEWiT | Support this FL proposal. |
| DCM | support |
| Qualcomm | We are fine with the proposal |
| ZTE, Sanechips | OK |
| CATT | Support |
| Apple | OK |
| Sharp | OK |
| Xiaomi | Support. |
| OPPO | Support |
| Futurewei | OK |

UE/Group-Specific Designs

UE DRX

Companies’ Views. Please Unfold for Reference

| CompanyName | Views/Proposals |
| --- | --- |
| FUTUREWEI | Proposal 11: Consider the adoption from day one of duty-cycled based operations (iDRX, eDRX, cDRX) and DL WUS of OFDM based sequence with at least PEI/DCP functionality replacement as baseline UE energy saving mechanisms in 6G. |
| Nokia | Proposal 29: RAN1 defer UE power saving discussions on the following topics to RAN2: UE DRX for all RRC states and related enhancements; SDT; UE feedback procedures. |
| vivo | Proposal 9: C-DRX in 6GR should be kept simple to maintain straightforward implementation and effectiveness for periodic traffic. Proposal 10: RAN1 should study and evaluate UE operation without C-DRX configuration for 6GR UE EE improvement, including PDCCH monitoring and L1/L3 measurement. |
| CATT | Proposal 36: The DL WUS should be designed to replace the UE duty-cycled operation for flexible indication for RRC Connected state UE. Proposal 37: The unified DL WUS should be further studied, which is complementary with UE IDRX in RRC IDLE/INACTIVE state. |
| xiaomi | Proposal 5: Enhanced DRX operation can be considered to better cater to UE energy saving purpose. At least relevant operation of the following timers can be considered: - drx-InactivityTimer - drx-RetransmissionTimerDL - drx-RetransmissionTimerUL |
| ZTE Corporation | Proposal 32: Dynamic adaptation of DRX parameters, e.g., by WUS, can be considered. Proposal 33: Dynamic activation of the starting position of DRX ON based on LP WUS can be considered. |
| Ericsson | Observation 30: NR defines multiple overlapping mechanisms across MAC and PHY for PDCCH monitoring control, resulting in redundant and partly conflicting behavior. Observation 31: In NR, the effective UE DRX activity is determined by both MAC-layer C-DRX timers and PHY-layer PDCCH monitoring configurations. Observation 32: NR does not support pre-configuration of multiple PDCCH monitoring modes tied to DRX states (e.g., different monitoring patterns/bandwidths for OnDuration vs. drx-InactivityTimer). Such modes must be dynamically (de-)activated via PHY-layer DCI signaling. Observation 33: Frequent DCI-based reconfigurations (e.g., BWP switching or PDCCH monitoring adaptations) may increase control channel overhead, particularly when managing large UE populations. Proposal 23: For 6GR, support low-overhead transitions between preconfigured PDCCH monitoring sets without requiring DCI-based signaling for each switch, thereby reducing control load and latency. Proposal 24: For 6GR connected DRX, allow configuration of multiple DRX window types (analogous to NR C-DRX "OnDuration", "drx-InactivityTimer"), each associated with individual PHY-layer configurations (e.g., search space, TDRA configuration, bandwidth) preconfigured via RRC. Proposal 25: For 6GR, retain the ability for additional lower-layer control (e.g., search space, enhanced cross-slot, bandwidth) when necessary, enabling a combination of preconfigured monitoring patterns across different windows and real-time adaptability. Proposal 26: For 6GR, allow WUS-based triggering of specific "OnDuration" window configurations, enabling energy-efficient receiver reactivation with appropriate PHY setting. |
| Fujitsu | Proposal 6: The UE behaviors are different for UEs in different RRC states, which requires different power saving strategies. - For UEs in RRC_CONNECTED state, the strategy is to reduce power consumption for PDCCH blind decoding - For UEs in RRC_IDLE state, the strategy is to minimize the unnecessary wake-ups before paging occasions. |
| Sony | Proposal 8: RAN1 to study UE DRX operation in coordination to DL WUS, i.e., DRX operation adjusted for LP-WUS DRX operation. |
| Apple | Proposal 15: Time domain adaptation techniques for PDCCH monitoring, including C-DRX, Rel-16 DCP, PDCCH skipping, SSSG switching, and LP-WUS, should be considered together for a simplified/harmonized design. Proposal 16: Schemes that enable UE to inform NW of early termination of C-DRX related timers or UL skipping can be considered for UE power saving. |
| Nordic Semiconductor ASA | Proposal 5: Study UE DRX operations in RRC idle and connected modes, including interaction between LP-WUS and DRX. |
| ITL | Proposal 6: - Study flexible C-DRX configurations with dynamic real-time adjustments. - Consider eDRX for idle/inactive modes as a baseline functionality in 6G. |
| WILUS Inc | Proposal 3: Study unified wake-up mechanism, joint NW--UE coordination, and evolved DRX framework for UE DRX in 6GR. |
| TCL | Proposal 31: For 6GR, study the UE DRX mechanism, such as DRX parameter adaptation, joint design of UE DRX and Cell DTX/DRX, etc. |

Summary and Discussion

Moderator thanks companies’ valuable inputs, and the following proposal is for companies’ check/comment:

Proposal 5.1.2.1 (1st round; medium priority):

Study and evaluate UE duty-cycled operations (e.g., iDRX, cDRX, eDRX, and/or DL WUS monitoring) for 6GR EE improvement, considering at least the following aspect:

- Coordination and avoiding duplication between UE operations based on DRX and DL WUS

- Potential simplification on UE DRX for periodic traffic

Companies’ views on Proposal 5.1.1.1 (1st round)

| Company | View |
| --- | --- |
| CMCC | Support the proposal. |
| FAI | Support |
| Samsung | As already pointed out by Chair in Prague meeting, we may need RAN2 progress first. Or what is the RAN1 focus for now? |
| vivo | - DL WUS in main bullet is DL WUS of OFDM based sequence, right? - About the last bullet, we think current DRX mechanism is simple and straightforward for periodic traffic. Not sure the intention of the “potential simplification” and it is desirable that any UE power saving techniques is simple, not only for DRX. Therefore, we propose following modifications:. Study and evaluate UE duty-cycled operations (e.g., iDRX, cDRX, eDRX, and/or DL WUS of OFDM based sequence monitoring) for 6GR EE improvement, considering at least the following aspect: - Simplification and avoiding duplicated functionalities - Coordination of different features, if any - Potential simplification on UE DRX for periodic traffic |
| Nokia | Given proposals for the above topics primarily impact RAN2 specifications and that RAN2 has already started discussing these topics, we feel RAN1 should defer any further discussions in these areas unless there is special justification. |
| Ericsson | Agree with Nokia. |
| Google | Similar views as Samsung. |
| TCL | Support |
| Huawei, HiSilicon | We share the similar view as Samsung. It can be left to RAN2. |
| CEWiT | Similar views as Samsung. |
| DCM | In general, UE DRX cycle itself should be discussed and leaded by RAN2 as done in NR. So, the main bullet and second sub-bullet should be discussed by RAN2. For the first sub-bullet, we can discuss it in RAN1 as the wake-up mechanism topic. |
| Qualcomm | We support the proposal without the sub-bullets Study and evaluate UE duty-cycled operations (e.g., iDRX, cDRX, eDRX, and/or DL WUS monitoring) for 6GR EE improvement, considering at least the following aspect: - Coordination and avoiding duplication between UE operations based on DRX and DL WUS - Potential simplification on UE DRX for periodic traffic |
| ZTE, Sanechips | For 6GR EE improvement, the DRX adaptation also should be part of the study. Study and evaluate UE duty-cycled operations (e.g., iDRX, cDRX, eDRX, and/or DL WUS monitoring) for 6GR EE improvement, considering at least the following aspect: - Coordination and avoiding duplication between UE operations based on DRX and DL WUS - Potential simplification on UE DRX for periodic traffic - Potential adaptation of DRX parameters |
| CATT | Considering the different relationships between DL-WUS and UE duty-cycled operation for Connected state and Idle/Inactive state, we suggest to add the following note: Proposal 5.1.2.1 (1st round; medium priority): Study and evaluate UE duty-cycled operations (e.g., iDRX, cDRX, eDRX, and/or DL WUS monitoring) for 6GR EE improvement, considering at least the following aspect: - Coordination and avoiding duplication between UE operations based on DRX and DL WUS - Potential simplification on UE DRX for periodic traffic Note: DRX study for idle/inactive mode and connected mode may have different considerations |
| xiaomi | We are supportive on the direction to further enhance UE duty-cycled operations. However, the sub bullets is a little bit misleading as they are not the major aspects on EE. We propose to delete sub bullets and update the main bullet a little bit. Proposal 5.1.2.1 -v1 1st round; medium priority): Study and evaluate L1 procedures related to UE duty-cycled operations (e.g., iDRX, cDRX, eDRX, and/or DL WUS monitoring) for 6GR EE improvement, considering at least the following aspect: |
| OPPO | We are OK for the motivation, but would be good to have whole agreement by RAN2 first. We may simply apply some timer mechanism for DL-WUS, for study purpose. |
| Futurewei | This can be left to RAN2 |
| NEC | We support this proposal. In NR, different mechanisms and configurations exist for UE C-DRX depending on whether LP-WUS is supported. We believe it may not be necessary to define two different mechanisms in 6G. We propose to study a unified design of UE C-DRX with or without LP-WUS to avoid defining multiple mechanisms or configurations. |

Wake-Up Mechanism

Companies’ Views. Please Unfold for Reference

| CompanyName | Views/Proposals |
| --- | --- |
| FUTUREWEI | Observation 11: LP-WUS can offer significant energy saving benefits and lower latency associated with duty-cycled operations. Further, considering a standalone DL WUS of OFDM based sequence (i.e., not reliant on OOK waveform), LP-WUS coverage comparable to a 6GR control channel should be achievable. Observation 12: There is a functional overlap of Rel-19 LP-WUS with Rel-17 PEI and Rel-16 DCP. An enhanced LP-WUS mechanism with scalable design can serve as a unified energy efficiency solution, i.e., taking over the Rel-17 PEI and Rel-16 DCP functionalities, common for all device types. |
| Spreadtrum | Proposal 11: Study DL WUS without C-DRX configuration in connected mode in 6G day-1. |
| vivo | Observation 3: Enhanced DL WUS of OFDM based sequence could achieve normal eMBB coverage and extended IoT coverge target (144 dB and 154dB MCL respectively), with DL WUS duration not exceeding one slot. Observation 4: For RRC idle/inactive mode, DL WUS to trigger paging monitoring only requires 0.03% system overhead to achieve 144dB MCL coverage, and 0.43% system overhead to achieve 154dB MCL. Observation 5: For RRC idle/inactive mode, DL WUS requires less system overhead than PEI. Observation 6: For RRC connected mode, DL WUS to trigger PDCCH monitoring only requires 0.046% system overhead. Observation 7: For RRC connected mode, DL WUS and DCP consumes similar system overhead. Observation 8: Offloading both serving cell and neighbor cell measurements to LR can achieve 37.7%-70.3% PSG compared to NR. UE LP-WUR is required to achieve RRM offloading. Observation 9: Up to 70% energy efficiency gain can be obtained by DL WUS with enhanced RRM measurement compared with PEI. Observation 10: DL WUS of OFDM based sequence can flexibly achieve the best balance between the power saving and UPT/latency. Observation 11: DL WUS of OFDM based sequence results in marginal network energy consumption increase over PEI. Proposal 13: For DL WUS of OFDM based sequence, study coverage enhancement to achieve 6GR coverage target for all channels, e.g., normal eMBB coverage of 144dB MCL and extended IoT coverage target 154dB. Proposal 14: Study further measurement relaxation and offloading for both serving and neighbor cell measurements for UEs enabled with DL-WUS of OFDM-based sequence in RRC idle[/inactive] modes in 6GR EE agenda, including - Higher measurement relaxation factor compared with UEs without UEs enabled with DL-WUS of OFDM-based sequence - Offloading both serving and neighbor cell measurements from UE MR to LP-WUR - Cell (re-)selection triggered by LP-WUR measurements Proposal 15: Study LP-WUR measurement for RRC Connected mode in 6GR EE agenda, at least for relaxation and offloading of MR measurement, including L1 and L3 measurements, performend by LR based on configured measurement cycle(s). Proposal 16: Study synchronization signal design for synchronization and RRM measurement by DL WUR of OFDM based sequence in 6GR - Unified PSS/SSS signal that serves both MR for initial access and LP-WUR for synchronization and RRM measurement - LP-SS dedicated for LP-WUR synchronzatio/RRM measurement Proposal 17: For DL WUS of OFDM based sequence for 6GR EE improvement, - Study DL WUS operation with I-DRX for RRC idle[/inactive] state, taking over the PEI functionalities. - Study C-DRX free DL WUS operation for connected mode UE, including at least the following aspects - UE PDCCH monitoring (active time) triggred by DL WUS according to DL WUS monitoring, providing flexible PDCCH monoioring beyond DCP functionalities - PDCCH skipping indication terminates UE PDCCH monitoring (active time) and resumes DL WUS monitoring Proposal 26: For paging with fast UE identification, the following aspects should be made clear: - Energy efficiency gain compared to sub-group based paging/waking up indication - The system overhead impact - FAR impact to direct RACH |
| Panasonic | Proposal 14: To investigate the wake-up mechanism scenarios with and without Cell DTX/DRX. To investigate the merits and feasibility of utilizing wake-up indication as the first step of the two-step PDCCH procedure design for data channel scheduling. To investigate the LP-WUS/LP-SS integration with 6GR and the support of MRSS. To study cell DTX/DRX framework in MRSS. |
| CATT | Proposal 50: Study and evaluate the UL WUS/WUR operation for NW for 6GR EE improvement, focusing particularly on the PRACH designed for LP-Rx and UL-WUS. |
| China Telecom | Proposal 5: Support increased payload size per WUS to support more paging subgroups and enable indication of new functionalities beyond wake-up, including but not restricted to the following functionalities: - Carrying other indication(s) than sub-group wake-up based on larger payload size in WUS - Receiving downlink broadcast/control information and small data |
| Lenovo | Proposal 21: Study low power radio architecture in 6GR by considering - A unified design catering to diverse device types - Minimize the impact on network power consumption - Improved coverage and reduced transmission/reception duration Proposal 22: Study the feasibility of low power radio usage in 6GR for idle mode paging occasion monitoring and evaluate the paging latency, FAR etc.,. |
| NEC | Proposal 15: The study of UE power saving in idle mode can take the following 5G NR features as a start point: Dynamic indication of paging monitoring based on PEI and/or LP-WUS; RRM measurement relaxation and RRM measurement offloading to LP-WUR. Proposal 16: Study enhancement of LP-WUS and LP-WUR for other purposes, e.g., receiving downlink broadcast/control information and even small data, to further reduce the active time of MR. |
| InterDigital | Observation 5: PDCCH based energy saving mechanisms provide significant energy saving in practical network conditions considering activation/deactivation latency, high transition energy and additional implementation complexity. Observation 6: OFDM sequence-based wake-up signal allows energy efficient transmission regardless of radio type (i.e., MR or LR) with minimized impact compared to OOK based WUS. Observation 7: As OFDM sequence-based WUS can be received regardless of the radio type, there's no need to introduce a separate configuration framework. Proposal 16: Support enhancement of PDCCH based energy saving mechanisms in NR by consolidating the fragmented specification enhancements across multiple releases within a unified framework. Proposal 17: Support OFDM sequence-based DL WUS. Proposal 18: Support OFDM sequence-based WUS as a type/format of PDCCH allowing multiplexing and configuration within the existing control channel framework. |
| Huawei | Proposal 22: DFT-s-OFDM can be considered for DL WUS design to enhance coverage performance. Proposal 24: Power consumption reduction technologies for UE's DL/UL data transmission should be studied, at least including: - How to minimize unnecessary padding bits transmission and meanwhile keep the latency benefit of pre-scheduling for network side - Enhanced uplink scheduling mechanism for energy efficiency, e.g. grant free enhancement - UE antenna adaptation mechanism - Time domain aggregated transmission for DL/UL data, which is beneficial for network and UE. Proposal 25: Preamble sequence with larger pool size and paging/wake-up procedure with fast UE identification should be studied to enable fast state transition for system re-entry. |
| Samsung | Observation 4: OOK modulation used in LP-WUS leads to large resource overhead and limited coverage, and requires a separate receiver for DL WUS reception, which leads to increased network energy and UE implementation complexity. Observation 14: The benefit of defining C-DRX and WUS-based activation of PDCCH monitoring should be further studied to prevent overlapping or redundant functionalities for the same target. Proposal 12: Study the following candidate use-cases for DL WUS of OFDM based sequence: - Trigger for PDCCH monitoring (replace DCP and PEI in NR) - Short message, like SI change information, ETWS and CMAS indication, and so on - Carrier/sub-band/SCell activation and/or deactivation or, generally, indication of cell/carrier 'state' - Change the cell DTX/DRX state or adaptation of UE DRX Proposal 13: For evaluation purposes and relative comparison of DL WUS of OFDM based sequence and other candidate energy saving schemes for 6GR, NR PDCCH-based WUS can be used as a reference. Proposal 14: Study whether a DL WUS of OFDM based sequence can provide T/F synchronization and/or whether existing synchronization signals (e.g. SSS-only) can be reused for DL WUS reception. Proposal 15: The coverage target for DL WUS of OFDM based sequence depends on whether a same coverage is supported for all the signals/channels or not in 6GR. - If all 6GR signals/channels are designed to have same coverage, DL WUS should be designed to achieve the target coverage. - If DL WUS is designed to have a separate coverage, the coverage for PDCCH can be used as a starting point. Proposal 16: The signal structure for DL WUS of OFDM based sequence should be designed based on the target coverage, latency, reliability and resource overhead. Proposal 17: Study link performance of CP-OFDM based waveform for DL WUS of OFDM based sequence design. Proposal 18: The existing sequence types supported in NR can serve as a starting point for the design of DL WUS of OFDM based sequence. Proposal 19: DL WUS of OFDM based sequence should be evaluated for all device types. |
| ZTE Corporation | Observation 13: Group wake-up introduces a probability of false alarms, leading to unnecessary UE wake-up, ineffective blind detection, energy waste and increased network overhead. Observation 14: UE-specific WUS can reduce the false-alarm rate, eliminate the need of monitoring paging DCIs triggered by wake-up transmitted for other UEs in the group, and reduce the resources allocated for paging DCI and paging PDSCH. Proposal 34: Regarding the functionality, UE specific and group specific WUS in idle/ inactive mode and connected mode can be studied. Proposal 35: DL WUS coverage should be better than 6GR's bottleneck channel, and strive to reuse sync signal for WUR measurement and sync. |
| Ericsson | Proposal 27: The OFDM-based WUS in 6GR is expected to provide significant energy saving and it should replace the main functionality of Rel-16 DCP and Rel-17 PEI which provide limited gains. Proposal 28: The design of 6GR WUS should ensure full cell coverage by targeting the PDCCH coverage. Proposal 29: The 6GR WUS should be supported in all RRC states (i.e., Idle/Inactive, Connected) to provide energy saving benefits for different use cases and scenarios. |
| LG Electronics | Proposal #19: Study PEI to lessen UE's efforts on paging monitoring with potential enhancements. Proposal #20: Study DL WUS to control UE's wake-up during the next active time. |
| Fujitsu | Proposal 5: At least the following features should be considered in the study phase of 6G to meet different power saving requirements for different device types: - LP-WUS, wake up signal from the network to device. - The LP-WUS design adopted in 5G needs to be re-evaluated. The primary design objective is to ensure the coverage of LP-WUS signal equivalent to that of normal cells, while minimizing any negative impact on spectral efficiency. - Wake-up signal from devices to the network. - Which assistance can be required by sending this wake-up signal. - DRX mechanism. |
| ETRI | Proposal 17: Study efficient PDCCH monitoring adaptation mechanisms for 6GR that ensure coordination with DTX/DRX and WUS frameworks, while maintaining a minimal and non-redundant set of functions. |
| Ofinno | Proposal 2: Support LP-WUS framework for 6GR as baseline. Consider control channel monitoring adaptation and MIMO adaptation as well. Proposal 15: Support LP-WUS framework for 6GR as baseline. Proposal 16: RAN1 to study including sending some control information via LP-WUS as part of LP-WUS energy efficiency studies. Proposal 17: RAN1 to consider studying the following features for 6GR: - PEI - RRM measurement relaxation - Cross slot scheduling - DRX adaptation - PDCCH skipping - Dormancy |
| Hanbat National University | Proposal 6: Study and evaluate how/whether an enhanced DL WUS can effectively unify and replace Rel-17 PEI in IDLE/INACTIVE and Rel-16 DCP in CONNECTED states. |
| MediaTek Inc. | Observation 22: NR idle mode UE power consumption primarily consists of sleep/wake-up overhead (~81%) and SSB synchronization/measurement (~17%). Observation 23: Reducing idle UE power consumption requires achieving deeper sleep and measurement offloading for UE's MR. Observation 24: WUR offloading can reduce MR sleep and SSB-related power consumption. Observation 25: To maximize WUR power saving gain, minimize the probability of MR wake-up as shown in Figure 13. Proposal 28: Study the following 6G design requirements to maximize UE EE gain with WUR offloading: MR operations: Measurement (Serving cell measurements over SSB), Synchronization (Synchronization over SSB), PDCCH/PO (Paging indication monitoring); WUR offloading: SSB design allows WUR meas. and sync with: 1 RX, initial CFO 20 ppm and residue CFO 5 ppm; OFDM-based WUS with full coverage; Subgrouping to minimize MR paging false alarm. Observation 26: Combining WUS/WUR as a sequence-based DL control format monitored in one SSSG (e.g., SSSG #0) and the conventional PDCCH in another SSSG (e.g., SSSG #1) can improve UE energy efficiency when a feasible SSSG switch delay is applied. Observation 27: For FTP/video traffic, the WUR-assisted SSSG switch achieves ~41% UE energy savings; for XR traffic, it achieves ~21% energy savings with only a slight increase in latency. The feature has almost no negative impact on BS power consumption: For FTP/video, the NES gains are -2.27% and 0.14% for Cat-1 BS and Cat-2 BS, respectively. For XR traffic, the NES gains are -1.48% and -1.70% for Cat-1 BS and Cat-BS, respectively. Proposal 29: Study SSSG switching enhancement integrating WUS/WUR benefits for UE power saving. |
| Sharp | Proposal 6: Study the operation mechanism of DL WUS, including whether it should work in conjunction with other features (e.g., DRX) or be capable to operating independently. Proposal 7: Support the study of both definitional approaches for the 6G DL WUS---that is, as a separate signal/channel, or as a part/type of a common signal/channel---without precluding either option. |
| NTT DOCOMO | Observation 9: Evaluation results indicate that DL WUS of OFDM-based sequence achieves significant UE energy saving with acceptable coverage and overhead under representative traffic and load conditions. Proposal 31: Study and evaluate DL WUS of OFDM-based sequence for 6G UE EE improvement, taking into account the following aspects: Coverage target aligned with 6GR control channel coverage; Signal design including sequence type, time-frequency resource allocation, and transmission parameters; Support for multiple RRC states (IDLE/INACTIVE/CONNECTED); Integration with UE DRX operations or DRX-free operation; Enhanced functionalities including sub-group wake-up, UE-specific wake-up, and potential carrying of additional information Proposal 32: Considering the DL WUS design for 6GR EE, the following directions should be studied: Unified WUS framework supporting multiple use cases and RRC states; Scalable design enabling flexible trade-off between coverage, latency, and overhead; Enhanced payload capacity for carrying additional indications beyond wake-up; Integration with synchronization and measurement framework for LP-WUR operation; Joint optimization with other UE power saving techniques Proposal 33: Study LP-WUR (Low-Power Wake-Up Receiver) architecture and operation for 6G UE EE improvement, taking into account the following aspects: LP-WUR power consumption model and architecture; Synchronization and measurement capabilities using LP-WUR; RRM measurement offloading to LP-WUR for both serving and neighbor cells; LP-SS (Low-Power Synchronization Signal) design for LP-WUR operation; Integration with DL WUS framework Proposal 34: Study enhanced paging mechanisms for 6G UE EE improvement in IDLE/INACTIVE states, taking into account the following aspects: Integration with DL WUS for paging indication; Paging sub-grouping mechanisms with reduced false alarm rate; On-demand paging resource allocation; Joint optimization with SSB periodicity adaptation and cell DTX/DRX. |
| ITL | Proposal 10: - Consider OFDM-based LP-WUS/WUR as an integral part of 6G baseline features for wake-up signaling. |
| WILUS Inc | Proposal 4: Study simplified and integrated PDCCH monitoring mechanism primarily by minimizing overlapping 5G PDCCH monitoring solutions (such as DCP and PEI). |
| TCL | Observation 34: For 6GR, the DL WUS can be considered to reduce the UE power consumption. Observation 35: For 6GR, the DL WUS may increase the latency and the signaling overhead. Observation 36: For 6GR, the DL WUS may degrade the coverage performance. Observation 37: For 6GR, the DL WUS may increase the false alarm rate. Proposal 32: For 6GR, study the DL WUS mechanism, such as DL WUS design, DL WUS configuration, DL WUS operation, etc. Proposal 33: For 6GR, study the impact of DL WUS on latency, signaling overhead, coverage performance, and false alarm rate, and study the methods to mitigate the impact. |

Summary and Discussion

Moderator thanks companies’ valuable inputs, and the following proposal is for companies’ check/comment:

Proposal 5.2.1.1 (1st round; medium priority):

Study and evaluate enhanced functionalities of DL WUS of OFDM-based sequence for 6G EE improvement, including but not restricted to the following:

- Whether/how to replace UE DRX (i.e., DRX-free operation) or integrate with UE DRX operations

- Coverage enhancement ([154] dB MCL)

- Whether/how to offload neighbor cell measurement

- Whether/how to carry other indication(s) than sub-group wake-up indication

- Whether/how to design UE-specific WUS

#### Companies’ views on Proposal 5.2.1.1 (1st round)

| Company | View |
| --- | --- |
| CMCC | Support the proposal. |
| FAI | Support |
| Samsung | We think the following RAN1#122bis agreement for DL WUS is general enough: Agreement Study and evaluate DL WUS of OFDM based sequence and corresponding mechanisms for 6GR EE improvement, regarding at least the following aspects: - Coverage target for DL WUS (e.g., same as PDCCH, common sync signal, or other) - Measurements and/or synchronization. - System overhead and network energy consumption/UE energy saving for UE operation with the DL WUS. - RRC states - Other functionalities |
| vivo | Clarification is needed on the relation between this proposal and the agreement in last meeting. Currently they seem to be partially overlap in several aspects, e.g. coverage, measurement, etc. Is the intention to provide further details of the study from last meeting agreement, or to include additional aspects beyond last meeting agreement? Agreement Study and evaluate DL WUS of OFDM based sequence and corresponding mechanisms for 6GR EE improvement, regarding at least the following aspects: - Coverage target for DL WUS (e.g., same as PDCCH, common sync signal, or other) - Measurements and/or synchronization. - System overhead and network energy consumption/UE energy saving for UE operation with the DL WUS. - RRC states - Other functionalities |
| Nokia | In general, we are ok with the direction of the proposal, though we suggest: - Coverage enhancement as the most important aspect by making that the 1st bullet - Remove the “[154] db MCL” target for the coverage enhancement bullet - companies should study what is viable/needed first given other trade-offs - Add a new bullet to cover “Spectral efficiency” - Clarify if “carry other indication(s)” means “in addition to” wake-up indication or “alternative use” of WUS payload or both?. Just as important as coverage, is the spectral efficiency. A WUS that can meet the 154dB target that requires lots of REs, is far less likely to be deployed. |
| Ericsson | - “Coverage enhancement” should be changed to Coverage target and should follow AI 11.1 and 11.2. Consider MCL = 143 dB for “normal coverage” as in TR 38.913. For “extended coverage”, MCL = 153 dB can be considered. - Please update bullet: Whether/how to offload/perform neighbor cell measurement |
| Google | Indeed, it seems we already have similar agreement. |
| TCL | Support |
| Huawei, HiSilicon | Generally OK, but can also live without this proposal, since we already had progress in RAN1#122bis. |
| CEWiT | Support |
| DCM | For the second sub-bullet, coverage enhancement is OK but the reason of value “154” needs to be clarify. For the fifth sub-bullet, wording of “UE-specific WUS” should be clarified. Does this mean WUS transmitted to UE(s)/subgroup(s)? |
| ZTE, Sanechips | No for the forth sub-bullet. First, subgroup indication is not the best power saving indication for LP-WUS. Instead, UE specific WUS has the most power saving gain. Assuming subgroup wake-up indication as baseline function is not fair before we have the evaluation results. Second, LP-WUS only carries a UE specific wake-up indication in connected mode. It is not correct to say, besides subgroup indication, we may have other indication. Therefore, to be more general, we’d like to update the forth bullet as follows Whether/how to carry other indication(s) and/or sub-group wake-up indication |
| CATT | Similar views with HW, we do not need this proposal. Further details can be discussed in further RAN1 meetings. |
| Xiaomi | Share the view from Samsung. |
| OPPO | We could have other solutions progress first. We need some real evaluation for DL-WUS before new agreement. |
| Futurewei | OK |
| NEC | We support studying enhanced functionalities for DL WUS. We propose to study the enhancement of LP-WUS and LP-WUR for other purposes beyond simple wake-up, such as receiving downlink broadcast/control information and even small data, to further reduce the active time of the main receiver. We also propose to study mechanisms to further exploit the LP-WUR for cell edge UEs, for example by offloading more RRM measurement activities, including neighbor cell measurement, to the LP-WUR. |
| Panasonic | Support |

PDCCH Monitoring and Adaptation

Companies’ Views. Please Unfold for Reference

| CompanyName | Views/Proposals |
| --- | --- |
| Nokia | Observation 17: PDCCH monitoring/adaptation schemes that enable devices to avoid having to power up from a power saving state to decode a scheduling attempt will generally save more power than schemes that focus on power saving during the decoding of the scheduling attempt. Proposal 20: Methods enabling reducing the overall PDCCH monitoring for UE power saving should be prioritized over schemes that attempt to make PDCCH processing more efficient. Observation 18: SSSG switching can support adaptation of PDCCH monitoring occasions in a wider variety of ways when compared with other NR PDCCH adaptation schemes (e.g. PDCCH skipping). Observation 19: Additional uses of the WUS, can be studied provided they prioritise: Decoder complexity; Spectral efficiency; Coverage; Resilience to loss/false detection. Observation 20: Rather than configuring and adjusting separate settings related to PDCCH monitoring such as monitoring bandwidth and periodicity, adjusting these settings jointly in a coordinated manner may lead to more optimal UE power saving. Proposal 21: Study mechanisms to reduce PDCCH monitoring occasions for 6GR EE improvement, based on the design principles from NR UE power saving techniques (e.g. C-DRX, SSSG switching, WUS) and considering what can be harmonized and simplified, prioritizing PS schemes that meet at least one of the following criteria: Are robust and low overhead; Are flexible enough to be considered to replace/enhance other schemes. Proposal 22: Study mechanisms enabling joint adaptations of PDCCH monitoring, including bandwidth and search space, to optimize UE power saving. |
| Spreadtrum | Proposal 14: Providing blind decoding information by DL-WUS or DCI before one or more PDCCH monitoring occasions to reduce PDCCH monitoring occasions can be studied. |
| vivo | Observation 2: Adapting the PDCCH monitoring frequency/period via PDCCH skipping or SSSG switching yields similar power savings, but PDCCH skipping can provide longer skipping durations to obtain more power saving gains. Observation 15: Restricting the maximum CORESET bandwidth and/or minimum Search Space period lowers static power but limits network scheduling flexibility and impacts eMBB performance. These shemes are more appropriate for IoT devices. Proposal 24: More discussion is needed to understand the scheme and the source(s) contributing to the UE power saving and develop power models to quantify the static power savings when restricting maximum CORESET size and minimum search space period. Proposal 11: Study mechanism(s) to reducet PDCCH monitoring for 6GR UE EE improvement based on the design principle from NR PDCCH skipping and SSSG switching in the 6GR EE or DL control agenda. |
| Panasonic | Proposal 13: To investigate how PDCCH monitoring adaptation is integrated with Cell DTX/DRX framework to avoid function duplication. Proposal 15: Besides time domain adaptation to reduce PDCCH monitoring, frequency domain, spatial domain and blind decoding reduction should also be investigated. |
| CATT | Observation 11: The enhanced PDCCH monitoring adaptation schemes based on the NR system assume that the initial PDCCH monitoring incurs high power consumption. Proposal 38: The initial PDCCH monitoring configuration in 6GR should be configured with sparse or infrequent PDCCH monitoring occasions for energy efficiency. Proposal 39: On-demand PDCCH monitoring occasions based on dynamic signaling indication should be further studied as part of PDCCH monitoring adaptation in 6GR. |
| Lenovo | Proposal 23: Study the feasibility of reducing the device power consumption in the connected mode by avoiding unnecessary PDCCH monitoring, minimizing blind decoding attempts. |
| NEC | Proposal 23: Study dynamic indication of PDCCH monitoring skipping and SSSG switching based on LP-WUS, e.g., introduce LP-WUS to trigger or terminate a PDCCH monitoring skipping or SSSG switching. |
| xiaomi | Proposal 2: The following mechanisms be considered as starting point for 6GR energy saving: - PDCCH skipping - SSSG switching - DCI carried by PDSCH Proposal 3: PDCCH monitoring reduction within a configured search space should be considered for 6GR energy saving. |
| OPPO | Proposal 21: The design of DL control in 6G shall natively support energy efficient operations. PDCCH should consider two stage indications - First stage with compact indication and for detection of main control information with limited detection effort. - Second stage corresponds to full detection with full information of control, by facilitation of the First stage. Proposal 24: The energy saving procedures are naturally enabled in the baseline 6G operation with: - PDCCH monitoring adaptation. - Lower Power Wake-up as first-stage for PDCCH monitoring. |
| Huawei | Proposal 23: Study at least the following aspect for new PDCCH mechanism and design, taking into account low detection complexity, low power consumption and impact on system performance - Avoid unnecessary extension of drx-InactivityTimer caused by pre-scheduling, while considering not to reduce the system's latency as much as possible - Appropriate PDCCH CORESET bandwidth, considering potential impact on system performance such as PDCCH blocking rate, scheduling flexibility and scheduling delay etc. - Appropriate PDCCH monitoring density, considering potential impact on latency - PDCCH Rx adaptation, considering potential impact on coverage - Appropriate number of DCI formats, considering potential impact on scheduling flexibility and DCI size alignment etc. |
| Samsung | Observation 15: The PDCCH monitoring skipping and SSSG switching framework can serve as a starting point for 6GR. Proposal 30: Assess the impact of energy saving mechanisms for PDCCH monitoring individually on the UE and on base station, and analyze any trade-offs with other KPIs. Proposal 31: Study mechanisms for PDCCH monitoring for 6GR energy efficiency including: - Reducing PDCCH monitoring, e.g., based on SSSG switching or PDCCH skipping, while avoiding overlapping functionalities by harmonizing and simplifying the design. - Reducing number of blind decodes per PDCCH monitoring occasion. - Maximizing commonality between IDLE/INACTIVE and CONNECTED states. |
| ZTE Corporation | Proposal 36: Mechanism for dynamically indicating the number of PDCCH monitoring occasions or the PDCCH blind decoding information (e.g., the search space set, CORESET, DCI format) can be considered. |
| LG Electronics | Proposal #17: Study PDCCH monitoring adaptation mechanisms introduced in NR and to be enhanced for 6GR. |
| Sony | Proposal 11: RAN1 to study UEPS mechanisms with respect to monitoring adaptation, in CONNECTED mode, restricting the search space and time duration in accordance with the design of 6GR. |
| MediaTek | Observation 27: For FTP/video traffic, the WUR-assisted SSSG switch achieves ~41% UE energy savings; for XR traffic, it achieves ~21% energy savings with only a slight increase in latency. The feature has almost no negative impact on BS power consumption: For FTP/video, the NES gains are -2.27% and 0.14% for Cat-1 BS and Cat-2 BS, respectively. For XR traffic, the NES gains are -1.48% and -1.70% for Cat-1 BS and Cat-BS, respectively. |
| Qualcomm Incorporated | Observation 13: Limited adaptation mechanisms in NR for PDCCH monitoring may not provide sufficient energy savings. Observation 14: PDCCH monitoring in NR may not be optimized for diverse traffic patterns and UE conditions. Observation 15: PDCCH monitoring in NR may lead to unnecessary power consumption for UEs with sparse traffic. Observation 16: PDCCH skipping in NR may not provide sufficient energy savings compared to sparse PDSCH transmission. Proposal 29: Study enhanced PDCCH monitoring adaptation mechanisms in 6GR to improve energy efficiency for diverse traffic patterns and UE conditions. Proposal 30: Study mechanisms to enable more flexible and efficient PDCCH monitoring in 6GR based on traffic characteristics and UE conditions. Proposal 32: RAN1 should study the schemes for RF and baseband adaptation for PDCCH monitoring and data reception/transmission in multi-carrier scenarios. |
| NTT DOCOMO | Observation 7: Evaluation results indicate that PDCCH monitoring adaptation achieves significant UE energy saving with limited throughput impact under representative traffic and load conditions. Observation 8: Rel-18 PDCCH monitoring adaptation framework provides basic functionality but has limitations in terms of configuration flexibility and adaptation granularity. Proposal 26: Study and evaluate PDCCH monitoring adaptation techniques for 6G UE EE improvement, taking into account the following aspects: Time-domain adaptation (e.g., monitoring periodicity, skipping patterns); Frequency-domain adaptation (e.g., monitoring bandwidth, search space configuration); Spatial-domain adaptation (e.g., number of antenna ports for PDCCH reception); Blind decoding complexity reduction; Dynamic adaptation mechanisms with reduced signaling overhead; Joint optimization with other UE power saving techniques (e.g., C-DRX, BWP switching) Proposal 27: Considering the PDCCH monitoring adaptation design for 6GR EE, the following directions should be studied: Unified framework supporting multiple adaptation dimensions (time/frequency/spatial); Scalable configuration enabling fine-grained adaptation based on traffic patterns and UE conditions; Enhanced signaling mechanisms for dynamic adaptation with minimal overhead; Integration with AI/ML-based traffic prediction for proactive adaptation. |
| ITRI | Proposal: The following aspect can be considered for energy efficiency by RAN1: PDCCH Monitoring and Adaptation |
| ASUSTeK | Observation 1: One of lessons learned from 5G's power saving technics is that same/similar function is fragmented into different solutions and their interaction/interworking is non-trivial. Observation 2: Different triggering mechanisms to adapt PDCCH monitoring has been considered in 5G and remain strong candidates in 6G. Observation 3: Implicit triggering mechanisms could be energy efficient by its nature, while the reliability of adaptation needs to be further study. Observation 4: The following two types of PDCCH monitoring adaptation could be considered as candidates for 6G: On-off adaptation; Sparse-dense adaptation Observation 5: Multiple solution for a same type of adaptation is not preferred. Proposal: RAN1 further study the following aspects for adaptation of control channel monitoring: 1. Triggering mechanisms and their applicable scenarios 2. Consider at least one solution for the following two types of adaptation and their applicable scenarios: - On-off adaptation - Sparse-dense adaptation |
| Nordic Semiconductor ASA | Proposal 7: Study adaptation methods in spatial/power, frequency, and time domains for energy efficiency enhancement in 6GR, by leveraging existing 5G adaptation solutions. at least for downlink control channel monitoring. Proposal 8: Study a harmonized adaptation for downlink control channel monitoring, by leveraging existing C-DRX, SSSG switching, and OFDM-based WUS techniques as a starting point. |
| CEWiT | Proposal 5: Study mechanism for reducing blind decoding attempts at UE including dynamic indication of blind decoding info (search space set, CORESET, DCI format, PDCCH occasion count etc), and mechanisms to provide blind decoding information before PDCCH monitoring occasions. |
| Google | Proposal 4: On enhancements for PDCCH monitoring adaptation, strive for a harmonized and simplified design across related features introduced in 5G (e.g., C-DRX, PDCCH skipping, SSSG switching and LP-WUS). |
| ITL | Proposal 9: - Consider SSSG switching as baseline in 6G, while re-evaluating the role of PDCCH skipping. |
| TCL | Observation 29: For 6GR, the PDCCH monitoring adaptation can be considered to reduce the UE power consumption. Proposal 28: For 6GR, study the PDCCH monitoring adaptation mechanism, such as PDCCH monitoring periodicity adaptation, PDCCH monitoring bandwidth adaptation, PDCCH monitoring skipping, etc. Proposal 29: For 6GR, study the impact of PDCCH monitoring adaptation on latency, throughput, and reliability, and study the methods to mitigate the impact. |

Summary and Discussion

Moderator thanks companies’ valuable inputs, and the following proposal is for companies’ check/comment:

Proposal 5.3.1.1 (1st round; medium priority):

Study mechanisms to reduce PDCCH monitoring power consumption in 6GR, considering but not restricted to the following:

- Reducing of PDCCH monitoring occasions

- Reducing blind decodes per PDCCH monitoring occasion

- Reducing total PDCCH monitoring complexity by multi-carrier/cell/slot scheduling

#### Companies’ views on Proposal 5.3.1.1 (1st round)

| Company | View |
| --- | --- |
| CMCC | Support the proposal. |
| FAI | Support |
| Samsung | OK |
| vivo | 1st sub-bullet is fine as its power saving benefit has been well justified in 5G studies 2nd and 3rd bullet, the power saving benefit was not conclusive from 5G studies thus requires further justifications. In addition, reducing blind decodes and PDCCH monitoring complexity is tightly coupled with 6GR PDCCH design and therefore suggest to studies the 2nd and 3rd sub-bullets in the DL control agenda. |
| Nokia | The 3rd bullet should either be clarified or deleted. For example, does the 3rd bullet include “cross-slot” scheduling (proposal 5.5.1.1) or is it something else? |
| Ericsson | Reduction of PDCCH monitoring occasions should be prioritized as it provides significantly larger power saving compared to others. Suggestion: Study mechanisms to reduce PDCCH monitoring occasion in time domain in 6GR. In addition, at least the following schemes can be considered: - Reducing blind decodes per PDCCH monitoring occasion - Reducing total PDCCH monitoring complexity by multi-carrier/cell/slot scheduling |
| Google | Support this FL proposal. We also share similar views with Ericsson. |
| TCL | Support |
| Huawei, HiSilicon | We support to study reducing PDCCH monitoring power. But currently the sub-bullets only cover limited aspects. More sub-bullets are expected. Study mechanisms to reduce PDCCH monitoring power consumption in 6GR, considering but not restricted to the following: - Reducing of PDCCH monitoring occasions - Reducing blind decodes per PDCCH monitoring occasion - Reducing total PDCCH monitoring complexity by multi-carrier/cell/slot scheduling - Reducing of PDCCH BW (e.g., CORESET) - Reducing of PDCCH receving antenna |
| CEWiT | Support |
| DCM | For first and second sub-bullet, we prefer to study them since reducing of PDCCH monitoring occasion in time/frequency domain has dominant UEPS gain compared to other schemes. For third sub-bullet, clarification is needed since it is unclear what scheme is indicated. |
| ZTE, Sanechips | OK in general. Seems second subbullet and third subbullet is overlapping. Considering, the control channel design may discuss the complexity, in EE AI, we may not need to discuss it. |
| CATT | OK |
| Xiaomi | Generally fine. |
| OPPO | Support |
| NEC | We support this study. We see value in taking 5G NR features like dynamic indication of PDCCH monitoring skipping and SSSG switching as a starting point. For 6G, we propose to study the use of LP-WUS to further enhance these mechanisms. Since LP-WUS monitoring consumes very little energy, it could be used to trigger or terminate a PDCCH monitoring skipping period or an SSSG switch with relatively short latency, reducing the need for the gNB to precisely predict traffic patterns. |
| Panasonic | Support |

RRM/RLM/BFD

Companies’ Views. Please Unfold for Reference

| CompanyName | Views/Proposals |
| --- | --- |
| FUTUREWEI | Observation 17: RRM measurement relaxation for neighboring cells and relaxation/offloading to LP-WUR for serving cell based on stationarity, low mobility, and/or not-at-cell-edge criteria are critical elements for the significant UE energy saving benefits in 6GR. Observation 18: Considering the enhancement of Sync signal(s) and LP-WUS designs for better coverage and/or capacity for LP-WURs, RRM measurements of serving and neighboring cells can be offloaded to LP-WUR. Proposal 14: Consider the adoption from day one of the relaxation of RRM measurements and the offloading of serving and neighboring cells RRM measurements to LP-WUR in 6GR. |
| Spreadtrum | Proposal 12: LP-WUR for serving/neighboring cell RRM measurement can be studied in all RRC states in 6G day-1. Proposal 13: Whether to use WUS for "synchronization and measurements" need more discussion and clarification. |
| vivo | Proposal 18: Study RRM relaxation for serving cell and/or neighbor cell measurement for UE without enabled DL WUS of OFDM-based sequence in 6G EE agenda. Proposal 19: Study RRM Relaxation and RLM/BFD Relaxation for UE without enabled DL WUS of OFDM-based sequence in 6GR EE agenda. |
| Panasonic | Proposal 18: To investigate on-demand/adaptive SS and additional TRS to improve the RRM/RLM/BFD measurement accuracy with less measurement occasions span and processing time. Proposal 19: To investigate more refined RRM/RLM/BFD relaxation design considering UE location, mobility/speed, SINR condition, UE beams and so on. Measurement activity can be dynamically controlled in the CONNECTED mode for a certain cell in the multi-carrier operation. |
| NEC | Proposal 17: Study mechanisms to further exploit LP-WUR for cell edge UEs, e.g., offloading more RRM measurement activities to LP-WUR, and perform neighbour cell measurement by LP-WUR, etc. Proposal 18: Study a harmonized SSB design to facilitate neighbour cell measurement by both MR and LP-WUR. |
| Samsung | Observation 16: Progress on WUS design and consultation with RAN4 on RRM measurement requirements is needed before RAN1 can start discussing using the DL WUS of OFDM-based sequence for RRM measurements. |
| Sony | Proposal 9: RAN1 to study UEPS mechanism and adaptation related to RRM measurements including RRM and RLM relaxation, and offloading measurement to low-complexity receiver used for WUS detection. Proposal 10: For 6GR UEPS techniques, RAN1 to study UL RS-based mobility for Energy Efficiency improvements in the measurement framework. |

Summary and Discussion

Moderator thanks companies’ valuable inputs, and the following proposal is for companies’ check/comment:

Proposal 5.4.1.1 (1st round; medium priority):

Study and evaluate unified PSS/SSS signal serving both MR and WUR for synchronization and RRM measurements.

#### Companies’ views on Proposal 5.4.1.1 (1st round)

| Company | View |
| --- | --- |
| CMCC | Fine the proposal. Whether such unified design is feasible should be considered from detect performance, coverage and overhead for MR. |
| FAI | Support |
| Samsung | MR and WUR may mislead implying a separate UE receiver architecture. Suggest to reword: Study and evaluate unified PSS/SSS signal serving both MR and WUR regardless of UE power saving states for synchronization and RRM measurements. |
| vivo | We are fine with the proposal. |
| Nokia | Support |
| Ericsson | Support. It is important to have unified sync signal to avoid additional NW energy consumption, overhead, and specification efforts. |
| Google | Fine with FL proposal and SS’s revision. |
| TCL | It is not sure right now whether to support separate or sharing design for MR and WUR. Thus, we support this proposal to consider unified PSS/SSS serving both MR and WUR at least from decoding or measurement complexity perspectives, as well as coverage performance and signaling overhead. |
| Huawei, HiSilicon | OK. Samsung’s update is OK for us. |
| DCM | Support. We do not prefer additional synchronization signal for WUR. |
| ZTE, Sanechips | Support. If we have the terminology WUR here, it is better to use this also for the evaluation, e.g., EE processing changed as WUR processing. |
| Xiaomi | Samsung’s version is OK for us. |
| OPPO | OK with intention, should we just the cell acquisition channel should take into account of the low power receiver requirements? |
| NEC | Support |
| Panasonic | We are supportive of studying. Just minor modification as below, as the PSS/SSS is NR term. Study and evaluate unified PSS/SSS synchronization signal serving both MR and WUR for synchronization and RRM measurements. |

Cross-Slot Scheduling/Early Indication

Companies’ Views. Please Unfold for Reference

| CompanyName | Views/Proposals |
| --- | --- |
| Nokia | Proposal 27: RAN1 deprioritize studying enhanced techniques for cross-scheduling for 6GR EE given: Increased in scheduler complexity especially when managing multiple k0 offsets that maybe changing dynamically; The non-trivial indication and potential costs (time and resources) of changing k0 offsets; Increased UE receiver complexity; Alternative schemes that reduce the potential benefits of cross-scheduling techniques. |
| vivo | Proposal 12: Identify and analyze the barriers that prevent commercialization of cross-slot scheduling in NR networks before study the enhancements on cross-slot scheduling in 6GR. Observation 14: For the case when C-DRX and PDCCH skipping is used, - Opt3 (100 MHz RF + 100 MHz BB) consumes the lowest power compare with Opt1 (400 MHz RF + 100 MHz BB) and Opt2 (400 MHz RF + 400 MHzBB BW) for all alpha values. - Opt1 (400 MHz RF + 100 MHz BB) consumes more power than Opt2 (400 MHz RF + 400 MHzBB BW) if the RF power ratio, alpha is at least 0.7 for both 16 and 8 UEs per cell; While Opt1 (400 MHz RF + 100 MHz BB) consumes less power than Opt2 (400 MHz RF + 400 MHzBB BW) if the RF power ratio, alpha is below 0.7 for both 16 and 8 UEs per cell. - The UPT for Opt2 (400 MHz RF + 400 MHzBB BW) is increased 35%~36% compared with Opt1 (400 MHz RF + 100 MHz BB) for 8 and 16 UEs per cell. - The UPT for Opt3 (100 MHz RF + 100 MHz BB) is the lowest for both 16 and 8 UEs per cell. Proposal 22: Before considering the option of decoupling RF/BB operation, carefully evaluate power savings, performance, network, and specification impacts by comparing following options: - Option 2: (WB RF+WB BB w/o restriction): Both RF and BB use wide bandwidth (WB) without any scheduling restriction to maximize instantaneous data rate and UE total sleep time. - Option 3 (NB RF+NB BB w/o restriction): UE operates on a smaller BWP to meet the user experience data rate needs, with reduced RF and BB BW Proposal 23: More discussion is needed to understand the schemes, - In case decoupling PDCCH/PDSCH power states aligns with BW and MIMO/antenna adaptation in Type 1 schemes, this scheme can be discussed there. - In case PDCCH monitoring for multiple CCs overlaps with cross-carrier scheduling and Cell management e.g., fast SCell (de)activation/dormancy, it can be discussed in spectrum agenda. |
| Panasonic | Proposal 16: To investigate two-step DCI framework to balance the control channel overhead, scheduling complexity and UE side PDCCH decoding energy efficiency. Proposal 17: To investigate two-step DCI framework to facilitate adaptation between more simplified data scheduling scheme for UE side energy efficiency (e.g., coarser frequency domain resource allocation, cross-slot scheduling and so on) and more advanced transmission/scheduling scheme for higher throughput. |
| InterDigital | Proposal 13: Support application of different minimum PDSCH scheduling offsets considering UE status to ensure reliable decoding while minimizing unnecessary standby power consumption. |
| xiaomi | Proposal 4: For 6GR UE energy saving, the following 5G mechanisms can be considered as starting point and further developed in 6GR: - BWP switching with potential simplification - Cross-slot scheduling |
| OPPO | Proposal 22: Cross-slot scheduling needs to be supported in 6G. - Cross-slot scheduling can be one of power saving methods for mandatory baseline functionality set. |
| ZTE Corporation | Proposal 37: Mechanisms, such as cross-slot scheduling and PDCCH skipping, can be further studied together with other 6GR energy saving features. |
| Ericsson | Observation 34: NR Rel-16 "Enhanced Cross-Slot Scheduling" can provide significant energy savings by dynamic adaptation of PDCCH to PDSCH time offset. However, the impact on the scheduling complexity should be taken into account. Proposal 30: Further study Enhanced Cross-Slot Scheduling scheme in 6GR considering impact on the scheduling complexity and its overlapping functionalities with other UE energy saving schemes (e.g., WUS). |
| MediaTek | Observation 29: NR Rel-16 cross-slot scheduling allows UE PDCCH monitoring power reduction based on a priori PDSCH scheduling knowledge. Handling both cross-slot and same-slot scheduling UEs in one cell complicates BS scheduler design. Observation 30: Data early indication based on 2-stage DCI design achieves up to 30% power saving for UE PDCCH-only monitoring while prevents the complication of handling same-slot and cross-slot scheduling UEs. Proposal 30: Study early data indication mechanisms for UE power consumption optimization regarding data availability/amount, while preventing different scheduling information provision timing. |
| Qualcomm | Proposal 26: Study the UE monitoring PDCCH in reduced-power state and utilize cross-slot scheduling to provide the UE with sufficient time to receive any PDSCH. Proposal 28: Study a mechanism where the minimum scheduling offset is > 0 by default and is automatically adjusted based on data arrival. |
| ITL | Proposal 8: - Consider Rel-16 enhanced cross-slot scheduling as baseline functionality in 6G. |

Summary and Discussion

Moderator thanks companies’ valuable inputs, and the following proposal is for companies’ check/comment:

Proposal 5.5.1.1 (1st round; medium priority):

Study and evaluate mechanisms of indication of (partial) scheduling information prior to PDSCH slot for 6G EE improvement, regarding at least the following aspects:

- Minimization of NW scheduling complexity and impact

- Which channel/signal for the early indication, considering performance and signaling overhead

- Impact on latency and UE processing timeline

FFS: Specific types of (partial) scheduling information to be indicated (e.g., time scheduling, maximum throughput, resource allocation)

#### Companies’ views on Proposal 5.5.1.1 (1st round)

| Company | View |
| --- | --- |
| CMCC | Fine to further study but as companies discussed, one potential issue that impact the commercial deployment of cross-slot scheduling is the impact on scheduler complexity. We prefer to add this one in the proposal. Proposal 5.5.1.1 (1st round; medium priority): Study and evaluate mechanisms of indication of (partial) scheduling information prior to PDSCH slot for 6G EE improvement, regarding at least the following aspects: - Minimization of NW scheduling complexity and impact - Which channel/signal for the early indication, considering performance and signaling overhead - Impact on latency and UE processing timeline - Impact on scheduler complexity. FFS: Specific types of (partial) scheduling information to be indicated (e.g., time scheduling, maximum throughput, resource allocation) |
| FAI | Support |
| Samsung | Deprioritize. Need justification from very limited commercial NR deployments for cross-slot scheduling. |
| vivo | We have concern on extending the cross-slot scheduling to be more general for other scheduling information. Before knowing the hurdle for network side to use cross-slot scheduling,the related proposal should be de-prioritized. . |
| Nokia | In general, we would prefer these schemes deprioritised, for 2 reasons NW scheduler impacts Limited power savings compared to other techniques. If the clear majority want study in this area, we would like wording added to the opening sentence, to emphasise the need for real “power consumption reduction of” mechanisms … |
| Google | We are fine with the proposal. CMCC’s revision seems to be sort of covered by the first subbullet. |
| TCL | Support |
| Huawei, HiSilicon | We support to study the direction of cross-slot scheduling. If companies have concern on the details, we can keep the main-bullet only and clarify it to be cross-slot scheduling. Study and evaluate mechanisms of cross-slot scheduling indication of (partial) scheduling information prior to PDSCH slot for 6G EE improvement, regarding at least the following aspects: - Impact Minimization of NW scheduling complexity and impact - Which channel/signal for the early indication, considering performance and signaling overhead - Impact on latency and UE processing timeline |
| CEWiT | Support |
| DCM | We are OK to study but if UEPS gain is limited or any NW scheduling remains, cross-slot scheduling should be deprioritized. |
| ZTE, Sanechips | Open to study, and also impact on throughput also should be considered. |
| Xiaomi | We are fine with the direction to further study and evaluate cross slot scheduling mechanism. However, the current main bullet is too broad to show the connections with cross-slot scheduling. We need to make the main bullet more specific. |
| OPPO | We need more clear definition of the mechanism before agree. |
| Panasonic | Support |

UE UL Power Saving for Connected Mode

Companies’ Views. Please Unfold for Reference

| CompanyName | Views/Proposals |
| --- | --- |
| Nokia | Proposal 28: RAN1 deprioritizes studying the following techniques: Enhanced uplink scheduling for reducing unnecessary uplink transmission, including UL skipping; Autonomous PUSCH transmission mode without grant; Releasing unused configured grant resources. |
| vivo | Observation 13: Uplink power consumption is almost independent of UL channel/BWP bandwidth when the UE total transmit power is kept constant. Proposal 25: Study UL DTX mechanism to shorten the total UL transmission time in 6GR scheduling agenda or EE agenda, e.g., uplink DTX mode requested by the UE and/or configured by the network. Proposal 27: For the techniques not mainly driven by UE energy efficiency as listed below, suggest no further discussion in 11.5 and to be directly discussed in the corresponding agendas. - Pre-scheduling and UL skipping in scheduling agenda - wideband transmission with low PAPR in waveform agenda - no late changes to UL in scheduling agenda - Autonomous PUSCH transmission in scheduling agenda - inactive SDT in initial accessagenda |
| NEC | Proposal 9: Study mechanisms to provide the gNB with awareness of the latency experienced by UL packets at the UE, to enable more energy-efficient UL scheduling decisions that can create or prolong gNB microsleep opportunities. |
| InterDigital | Proposal 14: Support energy-efficient uplink scheduling schemes for bursty transmission of data and control information. Proposal 15: Support uplink transmission scheme where UE selects and indicates a TB size and/or UCI size to the network prior to transmission. |
| OPPO | Proposal 25: 6GR UL control should consider sparse UCI transmission to improve UL efficiency and reduce power consumption. |
| Samsung | Observation 17: Clustering UL transmissions can reduce UE and BS energy consumption by maximizing the time in sleep states. Observation 18: CG-PUSCH and uplink skipping enable UE energy savings while requiring base station complexity. Proposal 32: Study mechanisms to reduce UE energy consumption from UL transmission perspective and the associated base station energy efficient procedure. |
| ZTE Corporation | Observation 15: Separate transmission of SR and BSR results in scheduling delay and increases UE power consumption. Proposal 38: A specific UCI indicating both SR and BSR can be considered. Proposal 39: Rapid release of unused SPS or CG resources can be studied for energy efficiency. |
| Sony | Proposal 16: Study UEPS techniques designed for UL-heavy use cases. |
| Qualcomm Incorporated | Observation 21: Configured grants in NR have limited flexibility and may not be suitable for all traffic types. Observation 22: Configured grants in NR may not provide sufficient energy savings for bursty traffic. Observation 23: Configured grants in NR may lead to resource wastage when traffic is not present. Observation 24: Configured grants in NR may not adapt well to changing traffic conditions. Observation 25: UL transmission timing in NR may not be optimized for energy efficiency. Proposal 33: Study autonomous PUSCH transmission enhancements in 6GR to improve energy efficiency and reduce latency for bursty traffic. Proposal 34: Study mechanisms to enable UE to autonomously select transmission parameters (e.g., MCS, resource allocation) for PUSCH transmission in 6GR. |
| Nordic Semiconductor ASA | Proposal 9: Study mechanisms to enhance UL transmission efficiency from UE point of view. |
| TCL | Observation 30: For 6GR, the UE UL power saving can be considered to reduce the UE power consumption. Observation 31: For 6GR, the UE UL power saving may increase the latency and the signaling overhead. Observation 32: For 6GR, the UE UL power saving may degrade the throughput performance. Observation 33: For 6GR, the UE UL power saving may increase the complexity. Proposal 30: For 6GR, study the UE UL power saving mechanism, such as UE UL DTX, UL skipping, UE transmission with low PAPR, UCI reporting enhancement, aggregated/efficient data transmission, no late changes to UL, Autonomous PUSCH transmission, etc. |

Summary and Discussion

Moderator thanks companies’ valuable inputs, and the following proposal is for companies’ check/comment:

Proposal 5.6.1.1 (1st round; medium priority):

Study and evaluate power consumption reduction mechanism(s) related to UE's UL data transmission with or without scheduling grant.

#### Companies’ views on Proposal 5.6.1.1 (1st round)

| Company | View |
| --- | --- |
| CMCC | Support in general. Moreover, we also consider that the transmission overhead on control signalling (e.g. periodic L1-RSRP, ACK reporting) will cause certain overhead and some features have been proved to have benefit on overhead reduction and have been already introduced in NR (e.g., UE-initiated BM in Rel-19 MIMO). We believe such aspect can be also considered in 6GR design and suggest the following revision: Proposal 5.6.1.1-rev1 (1st round; medium priority): Study and evaluate power consumption reduction mechanism(s) related to UE's UL data transmission with or without scheduling grant. |
| FAI | Support |
| Samsung | OK |
| vivo | The scope is too extensive and we think it should be discussed in control or scheduling agenda |
| Nokia | Other than UL DTX, which is likely to be covered within the NES cell DTX study, we do not see much support for study in this domain, and hence we would deprioritise this proposal completely. |
| Google | We support CMCC’s revision. |
| TCL | Fine with this proposal. Power consumption reduction mechanism(s) for UL’s UL transmission may increase the latency and the signaling overhead, as well as may degrade throughput performance. Thus, more explanations are recommended for this proposal, e.g., consider other potential impact on UL transmission performances (e.g., coverage or latency). |
| Huawei, HiSilicon | We are OK for this proposal. |
| CEWiT | Support |
| DCM | We do not prefer to study if UEPS gain is limited. Moreover, any clarification is needed since this scheme is not discussed in NR. |
| ZTE, Sanechips | OK |
| Xiaomi | OK |
| OPPO | OK |

BW Adaptation

Companies’ Views. Please Unfold for Reference

| CompanyName | Views/Proposals |
| --- | --- |
| FUTUREWEI | Observation 19: A BWP switch, especially one that changes numerology, could alter the timing context and thus influence the assignment or continuity of HARQ Process IDs for configured grants. Observation 20: Bandwidth adaptation using BWP switching for low load and bursty traffic can provide energy saving gains but may lead to increased scheduling/traffic latency and subsequently lower UE perceived throughput, particularly for inactivity timer based switching. Observation 21: High flexibility of BWP design adds to inherent complexity of the 5G NR, and increases validity and testing complexity of the feature. Proposal 15: Study UE operating bandwidth solutions for 6GR to improve UE EE considering minimization of data interruption and maximization of reliability when switching between narrowband and wideband BWPs. Proposal 16: Study improving BWP switching latency by simplifying BWP design parameters, e.g., limiting BWP update to mainly bandwidth and frequency location, to mitigate BWP switching impact on latency/throughput while maintaining energy saving gains. |
| Nokia | Proposal 23: The study of BW adaptation for 6GR EE focuses on the UE potential power savings of reducing the DL and UL active bandwidth. Defer the discussion on the mechanisms for more effectively adapting the UE's DL and UL active bandwidth to agenda item 11.1 and subsequent mapping to dedicated agenda item from RAN1#125 onwards. Proposal 24: RAN1 to deprioritize EE mechanisms that increase the design complexity (e.g., sub-CC) when BW adaptation is studied for EE. |
| Spreadtrum | Proposal 15: Support to study UE-specific and cell/group-wise BW adaptation indication in 6G day-1. |
| vivo | Observation 12: NR configuring every parameter as "BWP-dedicated" causes excessive configraution overhead and slow BWP switching time Observation 16: When UE DL BW adaptation is used on top of basic C-DRX (without wake-up sginals), 17%-18% additional energy efficiency gain can be observed for 0.1Mbye-0.5Mbyte packet size of FTP traffic, compared with C-DRX only. Observation 17: When UE DL BW adaptation is used on top of LP-WUS, maginal energy efficiency gain, or loss can be observed for FTP traffic with different packet sizes, compared with LP-WUS only. Proposal 20: Study efficient BWP adaptation framework at least for DL BW and/or MIMO layer adapataion in 6GR EE or spectrum-related agenda to achieve UE power saving and better commercial practicality, considering at least following: - Identify the parameters that has major impact to UE power saving - Minimize the number of BWP-dedicated parameters Proposal 28: Study the accumulative energy efficiency gain of 6GR EE techniques when they are performed simultaneously, i.e., performing frequency or spatial domain techniques on top of the time domain techniques. |
| CMCC | Proposal 22: RAN1 to further study a faster and more flexible adaptation scheme on frequency resource in 6GR (e.g., how to reduce the RF retuning time and refreshing time for HW/SW during adaptation). The details on related signal design/procedure can be discussed in the upcoming spectrum utilization agenda. |
| Panasonic | Proposal 21: BWP framework should be studied to support below functionalities - To support multi-carrier/spectrum operation, where each cell or cell combination can be represented by a BWP - To support multi-TRP operation, where one or more TRPs can be logically represented by a BWP - To support 6GR mobility, where a serving cell/TRP and optional beam information can be logically represented by a BWP |
| CATT | Observation 14: If the BWP concept is still adopted in 6GR, BWP switching could remain a potential solution for frequency-domain resource adaptation. Proposal 48: Adaptation within the BWP with a wide range of bandwidth or discontinuous spectrum resource should be considered in the 6GR, and whether the discontinuous spectrum resource belongs to a BWP is FFS. Proposal 49: From the power saving perspective, whether simplifying BWP configuration and BWP switching procedure can obtain the obvious power saving gain should be further studied. |
| OPPO | Proposal 20: Multiple functionality-sets transforming is supported in bandwidth adaptation mechanism with dynamic change in set of parameters. - A mandatory baseline functionality set of parameters is commonly supported by all UEs, which could correspond to the minimum capable bandwidth. - Simplified bandwidth parameter with only RB number, without SCS change. - Limited number of parameters related to main capability factors is supported. |
| Huawei | Proposal 26: Study enhanced BWP mechanism for UE power saving purpose, including coarse BWP granularity, longer switching delay to reduce static power while considering the impact on system performance as well. |
| Samsung | Observation 12: The BWP framework in NR was aimed to achieve UE power saving via adaptation of RF/BB bandwidth adaptation, but the NR BWP design is generally recognized as ineffective primarily because the long switching delay reduced the effectiveness and usefulness of multi-BWP operation. Proposal 29: Study RF/BB bandwidth adaptation mechanisms for 6GR, including the need for supporting a BWP operation, in conjunction with other 6GR functionalities related to UE power savings: - Reduced/No association with RRC reconfiguration - No change of SCS |
| ZTE Corporation | Observation 16: The BWP switching and dormancy BWP can reduce the unnecessary utilization of frequency domain resources and power consumption. Proposal 40: Faster BWP switching by restricting the parameter set that varies between BWP can be considered. Proposal 41: Improving the reliability of BWP switching through verification indication in DCI should be considered. |
| LG Electronics | Proposal #18: Study efficient indication (e.g., via group-common DCI) for BWP switching. |
| ETRI | Proposal 18: Study UE bandwidth adaptation mechanisms to improve UE energy efficiency in 6GR. |
| Sony | Proposal 12: RAN1 to study enhancement to bandwidth adaptation mechanisms based on 6GR design. |
| Apple | Proposal 24: Study simplified UE bandwidth adaptation for UE power saving, considering at least a minimum set of bandwidth-dependent configuration parameters, and same numerology across different bandwidth configurations. |
| MediaTek Inc. | Observation 31: The BWP framework is not only beneficial for UE bandwidth adaptation but can also enable multi-domain adaptation for network energy saving and potentially realize joint BS/UE energy efficiency improvement. By jointly adapting across time, frequency, and spatial domains, BWP switching with a common indication provides significant gains: BS energy efficiency improves by 38%~87%, and UE energy efficiency improves by 20%~54% for cell loads between 5% and 45%. Proposal 31: Study and evaluate enhanced BW adaptation for 6GR EE improvement, regarding at least the following aspects: UE-specific indication for UE energy saving (Fast and robust L1 indication); Cell/group-wise indication for network energy saving. |
| Qualcomm Incorporated | Observation 6: BWP switching in NR is not fast enough to adapt to bursty traffic and can lead to increased latency and reduced throughput. Observation 7: BWP switching in NR requires reconfiguration of many parameters which increases complexity and latency. Observation 8: BWP switching in NR can cause interruptions to data transmission and reception. Observation 9: BWP switching in NR may not be suitable for all traffic types and scenarios. Observation 10: BWP switching in NR may not provide sufficient energy savings for all use cases. Observation 17: Wideband operation can provide benefits for high data rate services but may increase UE power consumption. Observation 18: Wideband operation may not be suitable for all traffic types and scenarios. Observation 19: Wideband operation may require additional UE complexity and power consumption. Observation 20: Wideband operation may not provide sufficient energy savings for bursty traffic. Proposal 23: Study enhanced BWP framework for 6GR that enables faster and more flexible bandwidth adaptation with reduced latency and improved energy efficiency. Proposal 25: Study mechanisms to reduce BWP switching latency and interruption time in 6GR. Proposal 26: Study simplified BWP configuration in 6GR to reduce complexity and improve energy efficiency. Proposal 27: Study adaptive BWP switching in 6GR based on traffic characteristics and UE conditions. Proposal 28: Study BWP switching enhancements in 6GR to support diverse traffic types and use cases. Proposal 31: Study wideband operation enhancements in 6GR to balance performance and energy efficiency for diverse traffic types and scenarios. Proposal 32: Study mechanisms to enable efficient wideband operation in 6GR with reduced UE complexity and power consumption. |
| NTT DOCOMO | Proposal 28: Study and evaluate bandwidth adaptation techniques for 6G UE EE improvement, taking into account the following aspects: Fast and robust BWP switching mechanisms with reduced latency; Simplified BWP configuration framework with minimal parameter set; Dynamic bandwidth adaptation based on traffic patterns and channel conditions; Joint optimization with other adaptation mechanisms (e.g., PDCCH monitoring, antenna adaptation); Separate adaptation of RF bandwidth and baseband processing bandwidth. |
| CEWiT | Proposal 6: Study and evaluate enhanced BW adaptation mechanism for 6G EE improvement, considering at least the following aspects: Adaptation of bandwidth-dependent parameters; Faster adaptation without BWP switching/shorter data interruption by restricting parameter set; L1-triggerred adaptation with Improved reliability; Cell/group-wise indication for network energy saving use case |
| ITL | Proposal 7: - Study a simplified UE bandwidth adaptation scheme in 6G, targeting efficient receiver operation with low complexity and fast switching. |
| WILUS Inc | Proposal 2: Study simplified BWP adaptation by focusing on a minimum set of parameters with major power-saving impact, while reducing the number of BWP-dedicated parameters to enable faster and more reliable switching. |
| TCL | Observation 39: For 6GR, the BW adaptation can be considered to reduce the UE power consumption. Proposal 34: For 6GR, study the BW adaptation mechanism, such as BWP configuration/RRC simplification, BWP switch time reduction, BWP signaling overhead reduction, DCI indication reliability, etc. |

Summary and Discussion

Moderator thanks companies’ valuable inputs, and the following proposal is for companies’ check/comment:

Proposal 5.7.1.1 (1st round; medium priority):

Study and evaluate enhanced BW adaptation for 6GR EE improvement, regarding at least the following aspects:

- UE-specific indication for UE energy saving

  - Fast and robust L1 indication

- Cell/group-wise indication for network energy saving

#### Companies’ views on Proposal 5.7.1.1 (1st round)

| Company | View |
| --- | --- |
| CMCC | Support the proposal |
| FAI | Support |
| Samsung | No need to consider at least for now. It can also be accommodated by other means such as WUS, or SSSG switching, or for NES. |
| vivo | We think fast means to enable fast BW adaptation, not fast L1 indication, following change is made. Study and evaluate enhanced BW adaptation for 6GR EE improvement, regarding at least the following aspects: - Mechanisms to achieve faster UE-specific BW adaptation for UE EE improvements - Also consider the simplification and reliability for the indication - Cell/group-wise indication for network energy saving |
| Nokia | OK – but suggest adding the following bullet which from an EE perspective may cross multiple AI. - Joint Adaptation with other features impacting PDCCH monitoring (BW, coreset, SSSG, Rank) |
| Ericsson | In our view, details (sub-bullets) should be avoided. It is sufficient to have: “Study and evaluate enhanced BW adaptation for 6GR EE improvement.” |
| Google | Fine with this proposal. |
| TCL | Fine with this proposal |
| Huawei, HISiicon | We are OK with the directions. Besides, mechanism to reduce UE static power should also be studied. Study and evaluate enhanced BW adaptation for 6GR EE improvement, regarding at least the following aspects: - UE-specific indication for UE energy saving - Fast and robust L1 indication - Cell/group-wise indication for network energy saving - Reducing static power with new switching delay |
| CEWiT | Support the FL version of proposal |
| DCM | We are OK to study but if UEPS gain is limited, BW adaptation should be deprioritized. |
| ZTE, Sanechips | OK |
| CATT | Agree with vivo’s comments. |
| Xiaomi | The main bullet itself is good enough. |
| OPPO | Support |
| Futurewei | We agree with vivo’s updated version. |
| NEC | Support |
| Panasonic | Support |

Rank/Max MIMO Layer for UE TX/RX Antenna Adaptation

Companies’ Views. Please Unfold for Reference

| CompanyName | Views/Proposals |
| --- | --- |
| vivo | Observation 18: When UE Rx antenna adaptation is used on top of basic C-DRX (without wake-up sginals), 3.71%-13.20% additional energy efficiency gain can be observed for 0.1Mbye-0.5Mbyte packet size of FTP traffic, compared with C-DRX only. Observation 19: When UE Rx antenna adaptation is used on top of LP-WUS trigger PDCCH monitoring, the additional energy efficiency gain compared with LP-WUS only case is small. |
| ZTE Corporation | Proposal 42: Adaptive UE Rx antenna on/off for UE energy saving can be further studied. |
| Sony | Proposal 13: RAN1 to study antenna adaptation in joint optimization with other UEPS schemes. |
| NTT DOCOMO | Proposal 30: Study and evaluate UE antenna adaptation techniques for 6G UE EE improvement, taking into account the following aspects: Dynamic adaptation of UE Rx antenna ports/chains based on channel conditions and traffic patterns; Fast switching mechanisms with reduced latency; Joint optimization with other UE power saving techniques (e.g., PDCCH monitoring adaptation, BWP switching); Signaling mechanisms for antenna adaptation with minimal overhead; Impact on UE complexity and implementation. |

Summary and Discussion

Moderator thanks companies’ valuable inputs. Since there are diverse views on this enhancement direction, Moderator would like to suggest proponents to propose it in MIMO agenda next meeting.

Energy Efficient MIMO

Companies’ Views. Please Unfold for Reference

| CompanyName | Views/Proposals |
| --- | --- |
| FUTUREWEI | Proposal 19: Consider the following antenna architectures for energy saving: Antenna architecture with adaptable spatial elements; Hybrid Antenna Architectures for large antennas (e.g., consider 32 and 64 TXRUs for up to 2048 antenna elements at around 15 GHz); Mixed antenna architectures for mmWave FR2 BM (e.g., consider adding 128 or 256 RXRUs with 1-bit ADC to base station receiver). Proposal 20: Consider the following potential enhancements related to MIMO transmission, CSI acquisition, and FR2 BM: Overhead reduction for MIMO CSI acquisition; MIMO transmission and CSI acquisition with adaptable spatial elements; MIMO transmission and CSI acquisition with hybrid antenna architectures; Support of beam (re)acquisition with mixed antenna architectures for mmWave FR2. Proposal 21: Study early link/CSI acquisition related to power-domain adaptation for 6GR energy efficiency improvement. |
| Nokia | Observation 21: The 5G multi-CSI framework for spatial and power adaptations for NES has some key limitations and shortcomings such as: High energy consumption and overhead for CSI-RS transmissions and CSI reporting of sub-configurations, and configuration overhead, especially with increased number of CSI-RS ports; It does not exploit SRS-based MIMO operation. Proposal 25: Study spatial and power adaptation (of base station Tx antenna ports/chains and power), for network EE improvement. Proposal 26: Study methods for reduced CSI-RS overhead and relaxed CSI measurements and measurement reporting based on the needs of the actual conditions. Observation 22: A given AI/ML model can jointly tackle CSI-RS overhead reduction and NES aspects (multi-CSI framework) if the model is trained to output multiple CSI sub-configurations. Observation 23: The AI/ML use case of CSI prediction with sparse/low overhead CSI-RS, as listed in 11.6, is highly relevant for achieving network energy saving in 6G. In addition to the already listed KPIs, we should include NES gain as an additional KPI for this use case as proposed in [10]. |
| CMCC | Proposal 23: Support the following techniques related to MIMO be further considered in 6GR: Spatial domain: Multi-CSI based adaptation in spatial domain. Power domain: Multi-CSI based adaptation in power domain. |
| CATT | Observation 12: The network can obtain significant energy saving gain mainly from the adaptation of TxRUs from 64 to 32TxRUs, and obvious decrement of energy saving gain from 32TxRUs to 16 TxRUs and from 16TxRUs to 8TxRUs. Observation 13: The network energy saving technique of TRP adaptation in multi-TRP operation can provide 19.7%-28.7% network energy saving gain for low, light and medium system loads. Proposal 40: The corresponding CSI reporting schemes for TxRU adaptation mechanisms can be studied, e.g. CSI compression. Proposal 41: In 6GR, it is necessary to study TxRU adaptation mechanisms for periodic CSI-RS transmission. Proposal 42: Study mechanisms to minimize the impact of TxRU adaptation on the transmission power and coverage performance of common signals/channels. Proposal 43: It is necessary to support the dynamic TRP adaptation for spatial domain energy saving mechanism of network in 6GR. Proposal 44: Study CSI-RS transmission and CSI reporting mechanisms tailored for dynamic TRP ON/OFF adaptation in 6GR, ensuring fast and efficient link adaptation while maintaining energy savings for both the network and the UE. Proposal 45: Study SSB transmission mechanisms for dynamic TRP ON/OFF adaptation in 6GR to enable full low-power operation of OFF-TRPs while maintaining necessary synchronization and system information coverage. |
| China Telecom | Observation 4: Adaptive BS antenna switching is beneficial for NES and requires UE feedback to enable optimal spatial adaptation according to traffic load and service requirements. Proposal 4: Study and evaluate adaptive BS antenna switching in 6GR based on UE feedback to achieve holistic energy efficiency gains across diverse deployment scenarios and service requirements. |
| Lenovo | Observation 5: Spatial domain (SD-NES) framework defined in 5G NR has multiple issues/limitations, including limitation in supported CB types, large CSI feedback overhead, and increased UE power consumption. Observation 6: Identifying spatial non-stationarity (SnS) behaviour across BS antennas is necessary to maintain uniform channel behaviour across transmissions from BS, as well as saving energy in case a subset of BS antennas are blocked from the UE perspective. Proposal 17: Study the necessity/feasibility of deriving a new framework for 6G that enables flexible MIMO configuration by the BS, while neither increasing UE complexity nor the corresponding CSI feedback overhead. Observation 7: Given the lower per element cost and energy consumption and the expected larger array dimensions for FR3 deployments, it is of interest to study the impact of LR transmission and receiver chains from the perspective of feasibility, spectral and energy efficiency and to identify potential physical layer enhancements. Proposal 18: RAN1 to study MIMO architectures with support of low-resolution convertors from the perspective of compliance with RAN4 requirements, as well as the potential enhancements associated with beam management and reference signals adjusted to the low-resolution MIMO operation. Proposal 15: Study the feasibility of network energy saving from various PAPR/CM reduction techniques such as DL waveform using DFT-s-OFDM, selective mapping, tone reservation. |
| NEC | Observation 6: While extremely large antenna arrays (XL-MIMO) are a key enabler for 6G performance, the associated increase in the number of RF chains, ADCs, and DACs presents a significant energy consumption challenge. The power draw of the RF front-end scales with the number of antenna ports, making spatial domain energy saving techniques a fundamental requirement for a sustainable network design. Proposal 12: Study spatial domain energy saving techniques for XL-MIMO, including: Dynamic antenna adaptation mechanisms that support coordinated adaptation across multiple TRPs; A lean CSI framework with scalable feedback mechanisms designed to efficiently support a large number of antenna patterns with minimal overhead. |
| AT&T | Proposal 9: For 6GR interface, evaluate the tradeoff between performance, energy efficiency, UE complexity and CSI feedback overhead focusing on scenarios with high-resolution DL precoding configured. Proposal 10: For 6GR interface, study the feasibility of deriving a DFT-based precoding framework in which a precoder associated with a set of ports can be straightforwardly used to derive an orthonormal sub-precoder associated with a subset of the set of ports. |
| InterDigital | Proposal 10: Support CSI reporting adaptation considering network energy saving by including dynamic power offset adaptation, antenna port adaptation, on-demand CSI-RS activation and state-dependent CSI reporting. |
| xiaomi | Proposal 17：Dynamic adaptation of TRP on/off can be considered for 6GR energy saving. - UE behavior associates with dynamic adaptation of TRP on/off should be considered. |
| OPPO | Proposal 12: Multi-hypotheses CSIs for spatial domain NES (e.g., type 1 SD and type 2 SD in Rel-18) can be studied within CSI framework of 6G MIMO, considering the following aspects: - Collaboration with CSI-RS overhead reduction - CSI compression with reduced CSI feedback overhead Proposal 13: Support beam management enhancement in 6G for energy saving, including: - Minimize the resources/transmission for beam sweeping - UE initiated multi-beam operation to reduce beam measurement and report |
| Samsung | Observation 6: Rel-18 NR SD NES has introduced a few CSI-related optimizations to turn off antenna sub-array(s), for CONNECTED UEs. However, - There has been no analysis or simulation in Rel-18 to quantify the benefits in terms of the trade-off between network energy saving and user throughput; and - Refinement on CSI framework in Rel-18 NR NES (sub-Configurations) and its associated timeline and priority rule refinement may not be needed and can be achieved by multiple CSI Reporting Configurations and triggers. Observation 7: From our initial SLS study, when data traffic is sufficiently low, the EE improvement from SD NES is marginal (around 3% EE gain in low load). Observation 8: There is an overlap between SD NES and AI/ML use case agenda items for CSI-RS overhead reduction, where they provide CSI-RS port adaptation with a subset of the CSI-RS ports for CSI-RS transmission occasion(s). Proposal 21: Support Proposal 5.10.2.1a in the FL summary as it guides companies to a proper direction of study for SD/PD adaptation techniques for 6G NW EE improvement. Proposal 22: Do not support to study 'multi-CSI reporting' in conjunction with SD/PD adaptation techniques, at least due to the following issues: error propagation, intricate inter-dependency across report types/occasions, and overly complex and cumbersome priority rules. Proposal 23: Support to assess the benefits of SD NES schemes using both of UPT reduction and energy saving gain via SLS, and to consider only the schemes that are validated by the empirical assessment as candidate schemes. Proposal 24: Categorize enhancements to support sub-array turn-off into two types: A. CSI-RS overhead adaptation (e.g. number of ports, T/F density), including sTRP and mTRP scenarios, striving for a single unified solution a. Discuss how to handle overlap with AI/ML use case discussion to avoid duplication and contradicting outcome between the two agenda items B. CSI reporting a. Avoid the unnecessary sub-Configuration concept from Rel-18 NR SD NES Proposal 25: For enhancements that are proven beneficial, study (during the early SI phase) how such enhancements can be integrated into CSI-RS and CSI designs from day-one with minimum additional features. |
| ZTE Corporation | Proposal 44: Adaptive BS antenna switching should be supported in 6GR. Proposal 45: Dynamic TRP on/off can be studied. Proposal 46: Multi-CSI report in spatial domain (including both dynamic antenna and TRP on/off) should be considered in 6GR. Proposal 47: Overhead reduction for multi-CSI report for NES can be considered. Proposal 48: Multi-CSI report and the overhead reduction for multi-CSI report in power domain should be considered. |
| Ericsson | Observation 25: NR energy-saving techniques in spatial and power domains, including transceiver muting and adaptive Tx power based on CSI feedback for multiple CSI hypotheses, can provide significant energy saving with minimal KPI degradation. |
| LG Electronics | Proposal #14: Study spatial element (e.g., antenna element, antenna port) and transmit power adaptation for NW, by taking CSI enhancement of Rel-18 NES as a starting point. Proposal #15: Study dynamic TRP adaptation based on a pattern or NW indication. |
| Fujitsu | Proposal 4: Study the possible methods to enable adaptation of spatial elements and transmit power for both UE specific and common channels/signals. - The relevant discussions include CSI reporting framework design, AI/ML based channel estimation in spatial and power domain, and beam management mechanism considering spatial and power domain adaptation on synchronization signals. |
| ETRI | Proposal 20: Study dynamic TRX-chain or antenna adaptation for incorporation into the 6G CSI framework. Proposal 21: Study dynamic TRP-level on--off operation as part of the 6G beam management framework. |
| Apple | Proposal 23: Spatial/power domain adaptation schemes can be discussed in the MIMO agenda assuming NR Rel-18 NES schemes as starting point, taking into account of UE complexity reduction solutions. |
| MediaTek | Observation 32: Sustainable and scalable radio networks with large antenna arrays require: Energy usage scaling with traffic loading rather than antenna count Maximum energy efficiency Observation 33: CSI frameworks providing desired energy saving/efficiency properties feature: Reduced CSI-RS/SRS ports relative to antenna count CSI-RS/SRS overhead scaling with traffic loading Minimized CSI feedback compression/quantization loss Observation 34: Increasing CSI-RS/SRS transmission periodicity supplemented with opportunistic/on-demand RSs (e.g., DMRS) for CSI tracking secures limited performance degradation while achieving >25% BS power saving gain by extending 128-port CSI-RS periodicity from 20 ms to 80 ms for Cat-1 BS in low load scenario. Observation 35: Reducing CSI-RS ports through techniques like compressed sensing complements time-domain approaches in addressing energy consumption challenges. Observation 36: Direct channel feedback approaches can increase spectral efficiency under fixed overhead compared to NR CSI feedback. Proposal 32: Pursue lean CSI frameworks in time and spatial domains to reduce 6G channel information acquisition energy consumption through: Reduced CSI-RS/SRS ports relative to antenna count CSI-RS/SRS overhead scaling with traffic loading Minimized CSI feedback compression/quantization loss |
| NTT DOCOMO | Observation 4: Evaluation results indicate that BS antenna port adaptation achieves energy saving with limited throughput impact under representative traffic and load conditions. Observation 5: Rel-18 multi-CSI framework supports basic NES operation but results in significant overhead when multiple logical antenna port/element configurations need to be simultaneously supported. Proposal 21: Study and evaluate spatial and power domain adaptation techniques for 6G NW EE improvement, taking into account the following aspects: BS Tx antenna ports/chains and Tx power adaptation; CSI reporting (including CSI framework design) for dynamic antenna/TRP/power adaptation with overhead reduction; CSI-RS overhead reduction and adaptation; UE energy saving and complexity Proposal 22: Considering the CSI reporting/framework design for 6GR EE, the following directions should be studied: Scalable CSI reporting where the UE reports sub-PMIs reusable across sub-configurations, enabling overhead reduction and scalable NES for large MIMO arrays; Dynamic updating of CSI configuration parameters (e.g., number of ports, port mapping, number of reported CSIs), and separation of long-term and short-term CSI components. |
| CEWiT | Observation 2: Spatial domain adaptation independently by the BS without prior CSI reporting provide an extra energy saving of 13.92% and 8.7% in low and medium loads respectively when compared to spatial adaptation with CSI reporting. Proposal 4: Spatial and power domain adaptations should be supported in 6G for network energy savings, with areas for enhancements considering: Optimal CSI framework and reporting; Application to both UE specific and cell specific signals e.g., SSB; Multi-TRP adaptations; Enhancements based on AI/ML; Co-ordination with longer synch signal period based on traffic |
| Google | Proposal 3: Study CSI-related enhancement for 6G EE improvement, considering at least the following aspects: CSI-RS overhead reduction; CSI report enhancement with reduced CSI feedback overhead; Power adaptation for CSI-RS operations; Enhanced CSI triggering mechanisms. |
| TCL | Observation 41: For 6GR, the energy efficient MIMO can be considered to reduce the BS power consumption and the UE power consumption. |

Summary and Discussion

Moderator thanks companies’ valuable inputs, and the following proposal is for companies’ check/comment:

Proposal 5.10.1.1 (1st round; medium priority):

Study and evaluate spatial and power domain adaptation techniques for 6G NW EE improvement, taking into account the following aspects:

- BS Tx antenna ports/chains and Tx power adaptation

- CSI reporting (including CSI framework design) for dynamic antenna/TRP/power adaptation with overhead reduction

- CSI-RS overhead reduction and adaptation

- UE energy saving and complexity

#### Companies’ views on Proposal 5.9.1.1 (1st round)

| Company | View |
| --- | --- |
| CMCC | Support the proposal |
| FAI | Support |
| Samsung | OK |
| vivo | General direction is fine, but for details, It is more proper to be discussed in MIMO session. Study and evaluate spatial and power domain adaptation techniques for 6G NW EE improvement, taking into account the following aspects: - BS Tx antenna ports/chains adaptation - BS Tx power adaptation - Corresponding adaptation of CSI-RS transmissions, and CSI-reporting aspects - UE energy saving and complexity |
| Nokia | OK |
| Ericsson | OK |
| Google | Support this FL proposal. |
| TCL | Support |
| Huawei, HiSilicon | OK |
| CEWiT | Support |
| DCM | We support this proposal |
| ZTE, Sanechips | Beam management also belongs to spatial domain power saving, suggest to include Study and evaluate spatial and power domain adaptation techniques for 6G NW EE improvement, taking into account the following aspects: - BS Tx antenna ports/chains and Tx power adaptation - CSI reporting (including CSI framework design) for dynamic antenna/TRP/power adaptation with overhead reduction - Beam management - CSI-RS overhead reduction and adaptation - UE energy saving and complexity |
| Xiaomi | OK |
| OPPO | OK |
| NEC | Support |

UAI or UE Feedback

Companies’ Views. Please Unfold for Reference

| CompanyName | Views/Proposals |
| --- | --- |
| CMCC | Proposal 24: RAN1 to discuss a signaling framework to allow "Mutual beneficial feature combination". And for these features (UE feature A + NW feature B) in the combination, the following rules are followed: Neither A nor B can be enabled by the network unless the UE declares support for both features. The network can activate feature A or B only if the UE supports both. If the UE supports only one feature, both shall remain deactivated. Proposal 25: Examples for Mutual beneficial feature combination for power saving purpose can be further discussed and considered as follows: Time-domain pattern-based adaptation (C-DRX + Cell DTX/DRX). Wake-up receiver (LP-WUS/WUR + on-demand operation including UL-WUS trigger). Measurement Relax (RRM Relaxation + SSB Periodicity adaptation). Spatial domain (MIMO layer adaptation + Cell spatial-domain adaptation). |
| InterDigital | Proposal 3: Support fast adaptation of UE configuration based on NW energy saving operation and available network resources. |
| xiaomi | Proposal 8: UE assistance information for energy saving can be considered in 6GR. |
| Ericsson | Proposal 21: Techniques for UE-assisted power and spatial adaptation via multi-hypotheses CSI should be included as baseline functionality in 6GR. |
| Sony | Proposal 14: RAN1 to study enhancement on how to accommodate preferred UEPS mechanism using UAI reporting. |
| Apple | Proposal 26: Study UE-initiated adaptation based on e.g. UE knowledge of traffic and RF condition, including which configuration adaptation can be triggered by a UE and the corresponding procedures. Some examples include: - Frequency-domain adaptation such as SCell activation/deactivation and BWP switching - Time domain adaptation for PDCCH monitoring |
| MediaTek Inc | Observation 37: UAI/UE feedback can save UE power consumption by 10-20% in connected state by reducing unnecessary feedback reporting. Proposal 33: Study UAI/UE feedback for UE power saving in connected state considering, e.g., Purpose with UAI/UE feedback; Applicable deployments and scenarios; UAI/UE feedback configuration; UE reporting requirements. |
| Google | Proposal 8: 6G SI study on energy efficiency should pursue a more flexible approach for network to accommodate UE's preference from UAI reporting and dynamically update/adjust related configuration(s). |
| TCL | Observation 40: For 6GR, the UAI/UE feedback can be considered to reduce the UE power consumption and the BS power consumption. Proposal 35: For 6GR, study the UAI/UE feedback mechanism, such as UE assistance information, UE feedback configuration, UE reporting requirements, etc. |

Summary and Discussion

Moderator thanks companies’ valuable inputs, and the following proposal is for companies’ check/comment:

Proposal 5.10.1.1 (1st round; medium priority):

Study and evaluate NW indication and UE EE feedback and for joint BS and UE power saving, including but not restricted to:

- NW-to-UE indication to assist estimation of UE power saving gain (e.g., data availability, traffic information)

- UE-to-NW feedback: User assistance information with UE power saving gain for the indicated setting(s) and/or UE knowledge of traffic (DL and/or UL)

#### Companies’ views on Proposal 5.10.1.1 (1st round)

| Company | View |
| --- | --- |
| CMCC | Fine for the proposal |
| FAI | Support |
| Samsung | NW-to-UE indication can be accomplished by “existing” means (e.g. PDCCH skipping/SSSG switching, …) |
| vivo | We agree that current UAI mechanism is not sufficient thus good to have some study for better NW and UE collaboration. We propose following modifications Study and evaluate NW and UE collaboration for joint BS and UE power saving, including but not restricted to: - NW-to-UE assist information to facilitate UE estimation of UE power saving gain (e.g., data availability, traffic information) - UE-to-NW assistance information for UE power saving gain and/or UE knowledge of traffic (DL and/or UL) |
| Nokia | OK |
| Ericsson | Ok in general. UEs should not always by default be forced to operate with overly demanding configurations when it is not needed. UE feedback on the actual service demands/requirements can guide the NW to decide on an appropriate configuration. - UE-to-NW feedback: User assistance information with UE power saving gain for the indicated setting(s) and/or UE knowledge of traffic (DL and/or UL) and service demands. |
| Google | Support this proposal. Re. UE-to-NW feed, one important thing is NW’s response on UAI or dynamic adjustment corresponding to UE’s preference. Currently, UAI has been ignored without any feedback. |
| TCL | Fine with some revise suggestions. For two bullets, in our understanding, it is not only for UE power saving gain but also considering joint BS and UE power saving. Study and evaluate NW indication and UE EE feedback and for joint BS and UE power saving, including but not restricted to: - NW-to-UE indication to assist estimation of joint BS and UE power saving gain (e.g., data availability, traffic information) UE-to-NW feedback: User assistance information with joint BS and UE power saving gain for the indicated setting(s) and/or UE knowledge of traffic (DL and/or UL) |
| Huawei, HiSilicon | Generally OK. But the ‘indicated setting(s)’ is confusing. Study and evaluate NW indication and UE EE feedback and for joint BS and UE power saving, including but not restricted to: - NW-to-UE indication to assist estimation of UE power saving gain (e.g., data availability, traffic information) - UE-to-NW feedback: User assistance information with UE power saving gain for the indicated preferred setting(s) and/or UE knowledge of traffic (DL and/or UL) |
| CEWiT | Fine for the proposal |
| ZTE, Sanechips | We would suggest to make it simple and high level, since currently what does the following mean: e.g., data availability, traffic information, setting(s) and/or UE knowledge of traffic (DL and/or UL) and service demands, Study and evaluate NW and UE collaboration for joint BS and UE power saving, including but not restricted to: - NW-to-UE assistance information - UE-to-NW assistance information |
| Xiaomi | OK |
| OPPO | A bit earlier to conclude this ‘assistant’ information. |

Others

Other power saving techniques

Collection of observations or proposals not in any of the above sub-sections.

| Company | Comment |
| --- | --- |

General principle

Collection of observations or proposals on the general principle of the EE work.

| Company | Comment |
| --- | --- |
| Samsung | Clarification question on current TR skeleton: What is FLs’ plan to capture in section 6 (Energy efficiency)? We assume individual EE technique is captured in each corresponding section (e.g., IA, control, MIMO, …). |

Contacts

Below is a contact list for companies’ delegates following the energy efficiency topic in the 6G Radio SI:

| Company | Delegates | Email address(es) |
| --- | --- | --- |
| Apple | Sigen Ye; Dan Wu; Hong He; Seunghee Han | sigen_ye@apple.com; dan_wu4@apple.com; hhe5@apple.com; seunghee.han@apple.com; |
| ASUSTeK | Eddie Lin | eddie1_lin@asus.com |
| AT&T | Ahmed Hindy, Ralf Bendlin | Ahmed.hindy@att.com; rb691m@att.com; |
| CATT | Shupeng Li; Miaomiao Liu | lsp@catt.cn; liumiaomiao@catt.cn; |
| CEWiT | Deepak Agarwal; Deepak PM | deepak@cewit.org.in; deepakpm@cewit.org.in; |
| CMCC | Xiaodong Shen; Minghan Jiao | shenxiaodong@chinamobile.com; jiaominghan@chinamobile.com; |
| DCM | Takashi Ikeuchi; Taichi Shichijo; Naoya Shibaike; Shinya Kumagai | takashi.ikeuchi.gs@nttdocomo.com; taichi.shichijou.ma@nttdocomo.com; naoya.shibaike.eg@nttdocomo.com; shinya.kumagai.yw@nttdocomo.com; |
| Ericsson | Magnus Åström; Gustav Lindmark; Mohammad Mozaffari; Yanpeng Yang | magnus.astrom@ericsson.com; gustav.lindmark@ericsson.com; mohammad.mozaffari@ericsson.com; yanpeng.yang@ericsson.com; |
| ETRI | Sunghyun Moon; Junghoon Lee | sh.moon@etri.re.kr; jh.lee@etri.re.kr; |
| Fainity | ChieMing | chieming@fainnov.com; |
| Fainity | YenHua Li | yenhua@fainnov.com; |
| Fraunhofer | Geordie George; Gustavo Costa; Nazanin Vatanian; Elke Roth-Mandutz | geordie.george@iis.fraunhofer.de; gustavo.wagner.oliveira.da.costa@iis.fraunhofer.de; nazanin.vatanian@iis.fraunhofer.de; elke.roth-mandutz@iis.fraunhofer.de; |
| Fujitsu | Lei Zhang | zhanglei@fujitsu.com; |
| Futurewei | George Calcev; Hussain Elkotby | gcalcev@futurewei.com; helkotby@futurewei.com; |
| Google | Alex Liou | alexliou@google.com; |
| Huawei, HiSilicon | Yi Wang; Yifan Xue; Xiaolei Tie; Yan Cheng; Matthew Webb | wangyi6@huawei.com; xueyifan1@huawei.com; tiexiaolei@hisilicon.com; chengyan.cheng@huawei.com; matthew.webb@huawei.com; |
| IIT Kanpur | Dheeraj Naidu Amudala; Jyotirmay Saini | dheeraja@iitk.ac.in; jsaini@iitk.ac.in; |
| Lenovo | Karthikeyan Ganesan; Ali Ramadan Ali Yuantao Zhang | kganesan@lenovo.com; aali@lenovo.com; zhangyt18@lenovo.com |
| LG Electronics | Seonwook Kim; Suckchel Yang; Sechang Myung; Youngdae Lee | Sseonwook.kim@lgepartner.com; suckchel.yang@lge.com; sechang.myung@lge.com; youngdae.lee@lge.com; |
| MediaTek | Chiou-Wei Tsai; Weide Wu | cw.tsai@mediatek.com; weide.wu@mediatek.com; |
| NEC | Pravjyot Singh Deogun, Liu Yun | pravjyot.deogun@emea.nec.com; |
| Nokia | Naizheng Zheng; David Bhatoolaul; Cássio Ribeiro, Ganesh Venkatraman, Jorma Kaikkonen | naizheng.zheng@nokia-sbell.com; david.bhatoolaul@nokia.com; cassio.ribeiro@nokia.com; ganesh.venkatraman@nokia.com; jorma.kaikkonen@nokia.com; |
| Ofinno | Ryan Keating; Yunjung Yi | rkeating@ofinno.com; yyi@ofinno.com; |
| OPPO | Zhisong Zuo; Hao Lin; Zhenshan Zhao | zuozhisong@oppo.com; lin.hao@oppo.com; zhaozhenshan@oppo.com; |
| Orange | Raphael Visoz | raphael.visoz@orange.com; |
| Panasonic | Hongchao Li; Suzuki Hidetoshi; Iwata Ayako; Kuruvatti, Nandish; Horiike, Naoto | Hongchao.Li@eu.panasonic.com; suzuki.hidetoshi@jp.panasonic.com; iwata.ayako@jp.panasonic.com; Nandish.Kuruvatti@eu.panasonic.com; Naoto.Horiike@eu.panasonic.com; |
| Qualcomm | Gabi Sarkis; Hung Ly; Diana Maamari | gsarkis@qti.qualcomm.com; hdly@qti.qualcomm.com; dmaamari@qti.qualcomm.com; |
| Samsung | Youngbum Kim; Hongbo Si; Emad Farag; Qi Xiong, Sa Zhang | youngbum.kim@samsung.com; hongbo.si@samsung.com; e.farag@samsung.com; q1005.xiong@samsung.com; sa.zhang@samsung.com |
| Sharp | Hiroki Takahashi; Juan Liu; Emily Lai | takahashi.hiroki@mail.sharp; juan.liu@cn.sharp-world.com; emily.ch.lai@sharp-world.com.tw; |
| Spreadtrum | Yu Ding; Huan Zhou; Zhenzhu Lei; Hualei Wang; Lei Gu | Yu.Ding@unisoc.com; Huan.Zhou@unisoc.com; Reven.Lei@unisoc.com; Hualei.Wang@unisoc.com; Lei.gu@unisoc.com; |
| TCL | Rongling Jian; Wenwen Huang; Yuanqing Yang | rongling.jian@tcl.com; wenwen5.huang@tcl.com; yuanqing4.yang@tcl.com; |
| vivo | Xin Qu; Lihui Wang; Gen Li; Huan Wang; Xueming Pan | quxin@vivo.com; wanglihui@vivo.com; reagan.li@vivo.com; wanghuan@vivo.com; panxueming@vivo.com; |
| Xiaomi | Lei Wang; Sicong Zhao | wanglei25@xiaomi.com; zhaosicong@xiaomi.com; |
| ZTE, Sanechips | Youjun Hu; Xuan Ma; Mengzhu Chen | hu.youjun1@zte.com.cn; ma.xuan1@zte.com.cn; chen.mengzhu@zte.com.cn; |
| Sony | Yuu Ichikawa; Naoki Kusashima | Yuu.Ichikawa@sony.com; Naoki.Kusashima@sony.com; |

All email addresses (separated by “;”)

magnus.astrom@ericsson.com; gustav.lindmark@ericsson.com; mohammad.mozaffari@ericsson.com; yanpeng.yang@ericsson.com; alexliou@google.com; rongling.jian@tcl.com; wenwen5.huang@tcl.com; yuanqing4.yang@tcl.com; Hongchao.Li@eu.panasonic.com; suzuki.hidetoshi@jp.panasonic.com; iwata.ayako@jp.panasonic.com; Nandish.Kuruvatti@eu.panasonic.com; Naoto.Horiike@eu.panasonic.com; gsarkis@qti.qualcomm.com; hdly@qti.qualcomm.com; dmaamari@qti.qualcomm.com; zhanglei@fujitsu.com; chieming@fainnov.com; naizheng.zheng@nokia-sbell.com; david.bhatoolaul@nokia.com; cassio.ribeiro@nokia.com; takahashi.hiroki@mail.sharp; juan.liu@cn.sharp-world.com; emily.ch.lai@sharp-world.com.tw; wangyi6@huawei.com; xueyifan1@huawei.com; tiexiaolei@hisilicon.com; chengyan.cheng@huawei.com; matthew.webb@huawei.com; shenxiaodong@chinamobile.com; jiaominghan@chinamobile.com; lsp@catt.cn; liumiaomiao@catt.cn; sh.moon@etri.re.kr; jh.lee@etri.re.kr; pravjyot.deogun@emea.nec.com; wanglei25@xiaomi.com; zhaosicong@xiaomi.com; quxin@vivo.com; wanglihui@vivo.com; reagan.li@vivo.com; wanghuan@vivo.com; panxueming@vivo.com; hu.youjun1@zte.com.cn; ma.xuan1@zte.com.cn; chen.mengzhu@zte.com.cn; youngbum.kim@samsung.com; hongbo.si@samsung.com; e.farag@samsung.com; q1005.xiong@samsung.com; dheeraja@iitk.ac.in; sigen_ye@apple.com; dan_wu4@apple.com; hhe5@apple.com; seunghee.han@apple.com; kganesan@lenovo.com; aali@lenovo.com; geordie.george@iis.fraunhofer.de; gustavo.wagner.oliveira.da.costa@iis.fraunhofer.de; nazanin.vatanian@iis.fraunhofer.de; elke.roth-mandutz@iis.fraunhofer.de; gcalcev@futurewei.com; helkotby@futurewei.com; takashi.ikeuchi.gs@nttdocomo.com; taichi.shichijou.ma@nttdocomo.com; naoya.shibaike.eg@nttdocomo.com; shinya.kumagai.yw@nttdocomo.com; deepak@cewit.org.in; deepakpm@cewit.org.in; rkeating@ofinno.com; yyi@ofinno.com; cw.tsai@mediatek.com; weide.wu@mediatek.com; Yu.Ding@unisoc.com; Huan.Zhou@unisoc.com; Reven.Lei@unisoc.com; Hualei.Wang@unisoc.com; Lei.gu@unisoc.com; Sseonwook.kim@lgepartner.com; suckchel.yang@lge.com; sechang.myung@lge.com; youngdae.lee@lge.com; zuozhisong@oppo.com; lin.hao@oppo.com; zhaozhenshan@oppo.com; jsaini@iitk.ac.in; raphael.visoz@orange.com; Ahmed.hindy@att.com; rb691m@att.com; Yuu.Ichikawa@sony.com; Naoki.Kusashima@sony.com; eddie1_lin@asus.com; zhangyt18@lenovo.com; sa.zhang@samsung.com; ganesh.venkatraman@nokia.com; jorma.kaikkonen@nokia.com;

References

RP-251881, “New SID: Study on 6G Radio,” NTT DOCOMO (Moderator), RAN #108, June 2025.

R1-2508323, “Discussion on 6G Energy Efficiency Techniques,” FUTUREWEI, RAN1 #123, November 2025.

R1-2508340, “Energy Efficiency in 6G Radio,” Nokia, RAN1 #123, November 2025.

R1-2508391, “Discussion on energy efficiency for 6GR,” Spreadtrum, UNISOC, RAN1 #123, November 2025.

R1-2508436, “Discussion on 6G energy efficiency,” vivo, RAN1 #123, November 2025.

R1-2508459, “Discussion on Energy Efficiency of 6GR interface,” CMCC, RAN1 #123, November 2025.

R1-2508477, “6G Study on Energy Efficiency,” Fraunhofer IIS, Fraunhofer HHI, RAN1 #123, November 2025.

R1-2508478, “On 6GR design for energy efficiency,” Panasonic, RAN1 #123, November 2025.

R1-2508581, “Discussion on energy efficiency of 6GR,” CATT, RAN1 #123, November 2025.

R1-2508616, “Discussion on 6GR energy efficiency,” China Telecom, RAN1 #123, November 2025.

R1-2508624, “Discussion on 6GR Energy Efficient design,” Lenovo, RAN1 #123, November 2025.

R1-2508629, “Discussion on Physical Layer Design for Energy Efficiency in 6G,” NEC, RAN1 #123, November 2025.

R1-2508642, “Views on Energy Efficiency for 6GR,” AT&T, RAN1 #123, November 2025.

R1-2508646, “Energy Efficiency in 6GR Air Interface,” InterDigital, Inc., RAN1 #123, November 2025.

R1-2508688, “Discussion on energy efficiency for 6GR,” Xiaomi, RAN1 #123, November 2025.

R1-2508731, “Energy Saving for 6GR air interface,” OPPO, RAN1 #123, November 2025.

R1-2508739, “Views on energy saving for 6GR,” Huawei, HiSilicon, RAN1 #123, November 2025.

R1-2508745, “Energy Efficiency,” Tejas Networks Ltd., RAN1 #123, November 2025.

R1-2508806, “Discussion on energy efficiency for 6GR,” Samsung, RAN1 #123, November 2025.

R1-2508823, “Discussion on energy efficiency for 6GR,” ZTE Corporation, Sanechips, RAN1 #123, November 2025.

R1-2508864, “6GR Energy Efficiency,” Ericsson, RAN1 #123, November 2025.

R1-2508879, “Considerations for 6G energy efficiency,” KT Corp., RAN1 #123, November 2025.

R1-2508912, “Discussion on energy efficiency for 6GR,” LG Electronics, RAN1 #123, November 2025.

R1-2508932, “Discussion on energy efficiency for 6GR,” Fujitsu, RAN1 #123, November 2025.

R1-2508977, “Discussion on energy efficiency in 6GR,” ETRI, RAN1 #123, November 2025.

R1-2508995, “Discussion on energy efficiency,” HONOR, RAN1 #123, November 2025.

R1-2509027, “Views on 6GR Energy Efficiency,” Ofinno, RAN1 #123, November 2025.

R1-2509045, “Discussion on energy efficiency in 6GR,” Hanbat National University, RAN1 #123, November 2025.

R1-2509077, “Consideration on 6GR Energy Efficiency,” Sony, RAN1 #123, November 2025.

R1-2509114, “Views on 6G energy efficiency,” Apple, RAN1 #123, November 2025.

R1-2509147, “Energy efficiency for 6GR,” MediaTek Inc., RAN1 #123, November 2025.

R1-2509177, “Study on energy efficiency for 6GR,” Sharp, RAN1 #123, November 2025.

R1-2509235, “Energy Efficiency in 6GR,” Qualcomm Incorporated, RAN1 #123, November 2025.

R1-2509286, “Discussion on Energy Efficiency for 6GR,” NTT DOCOMO, INC., RAN1 #123, November 2025.

R1-2509309, “Discussion on Energy efficiency,” ITRI, RAN1 #123, November 2025.

R1-2509314, “Energy efficiency for 6GR,” ASUSTeK, RAN1 #123, November 2025.

R1-2509331, “Energy Efficiency in 6GR,” Nordic Semiconductor ASA, RAN1 #123, November 2025.

R1-2509353, “Views on Energy Efficiency,” CEWiT, RAN1 #123, November 2025.

R1-2509360, “Discussion on 6G energy efficiency,” Google, RAN1 #123, November 2025.

R1-2509361, “Energy Efficiency in 6G Radio,” ITL, RAN1 #123, November 2025.

R1-2509383, “Discussion on Energy Efficiency for 6G Radio,” WILUS Inc., RAN1 #123, November 2025.

R1-2509384, “Discussion on energy efficiency for 6GR,” TCL, RAN1 #123, November 2025.

R1-2509399, “On 6GR energy efficiency power model updates for SBFD,” Vodafone, Telecom Italia, Deutsche Telekom, Nokia, Charter, RAN1 #123, November 2025.

R1-2509414, “Discussion on energy efficiency and energy saving,” CAICT, RAN1 #123, November 2025.