3GPP TSG RAN WG1 #123								     R1-250XXXX

Dallas, USA, Nov 17th – 21st, 2025

Agenda item:	11.6

Source: 	Samsung (Moderator)

Title:		Moderator summary #` on AI/ML for 6GR

Document for: 	Decision

# Introduction

In RAN 1 #122, the following agreement was made for use case identification:

Agreement For 6GR AI/ML use cases identification/categorization, for each (sub-)use case proposed, proponent companies are encouraged to study and report the following: - Definition of each (sub-)use case, including at least AI/ML model input/output - The evaluation assumption, methodology, KPIs, benchmark, and preliminary simulation results - Assumption on training types, e.g., - offline training, online training/finetuning - Label construction (if applicable), including whether/how to obtain label data for model training - Assumption on model location for inference, e.g., UE-sided model, NW-sided model, and two-sided model - Collaboration/interaction between UE and NW, e.g., no collaboration/interaction UE/Network collaboration targeting at separate or joint ML operation - High level potential specification impact

This paper summarized the proposed use cases for 6GR study on AI/ML.

# Use cases

#### Proposal 1: potential agenda for further study

For the identified /categorized 6GR AI/ML use cases, the following corresponding agendas can be considered for potential further study

Note: whether/when to study can be decided in the corresponding agenda.

| Use cases |  |  | Related agendas |
| --- | --- | --- | --- |
| Low overhead CSI-RS or CSI prediction with AI/ML |  |  | MIMO: reference signal->CSI-RS |
| Low overhead DMRS with AI/ML receiver |  |  | MIMO: reference signal->DMRS |
| CSI compression and feedback |  |  | MIMO: CSI feedback |
| AI/ML for beam management and extension |  |  | MIMO: Beam management Initial access Mobility |
| Low overhead SRS with AI/ML |  |  | MIMO: SRS |
| AI-enabled UL precoder indication |  |  | MIMO: UL MIMO |
| AI/ML for (de)modulation |  |  | Modulation |
| AI/ML based waveform for PAPR reduction |  |  | Waveform |
| AI/ML based HARQ-ACK feedback |  |  | Channel coding |
| PDCCH related |  | Prior-Information-Aided DCI Decoding | RAN 4: Demod DL control |
|  |  | Lossless DCI Compression |  |
| Power control related |  | UL closed-loop power control | Power control |
|  |  | Pathloss prediction |  |
| RACH related design: |  | Early contention resolution in RACH | Initial access RAN 4 |
|  |  | Low PAPR PRACH sequence design |  |
| AI-based non-linearity handling at transmitter or receiver |  |  | RAN 4: RF requirement MIMO: reference signal -> DMRS Power control |
| AI/ML based SRS power imbalance compensation |  |  | RAN 4 |
| Improved scheduling/HARQ for token traffic |  |  | [RAN 2 scheduling/HARQ] |
| Digital twin related use cases | Sensing based RAN digital twin construction with NW-side AI/ML model |  | [ISAC] |
|  | AI/ML-enabled RAN digital twin with distributed model |  |  |
|  | Site Specific Learning for AI/ML and RAN Digital Twin |  |  |
| AI for positioning |  |  | [ISAC] |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | For the TBD part, or even some use cases that has proposed agenda, they may be implementation. The spec impact is not very clear. Let’s me collect input to see how to proceed the further discussion. 1. I don’t think we need to spend time on this, if this cannot be agreed, we can drop this proposal. 2. if some of use cases can be agreed, for the use case that may not have clear understanding by the group on where to discuss, we can remove them from the table and add one sentence to address the concern, e.g., For other use cases, proponent can directly propose to corresponding agendas. Note: there is no intention to exclude/deprioritize any AI/ML use cases |

## CSI prediction and CSI-RS overhead reduction

#### Proposal 2.1: study aspects

For the potential study on CSI prediction and CSI-RS overhead reduction with AI/ML, at least the following aspects are to be considered:

- KPIs:

  - Performance related: SGCS, NMSE, Throughput, CSI-RS overhead, NES gain

  - Model complexity

  - Overhead for fine-tuning, when applicable.

  - Inter-vendor collaboration overhead and complexity for two-sided model, when applicable.

- Other aspects for study:

  - Label quality impact CSI prediction

  - Performance with practical impairments

  - Generalization

    - Sub-Case A/B/D/E: deployment scenarios, antenna configurations, spatial domain factors (e.g., port number/patterns), frequency domain factors (e.g., bandwidth, subcarrier spacing)

    - Sub-Case C bandwidth and frequency gap between the source and target frequency bands

  - Feasibility/performance for finetuning, if applicable

  - Scalability, e.g., for frequency domain factor (e.g., bandwidth) and spatial domain factor (e.g., number of antenna ports)

  - Coexistence with non-AI capable UE

  - (Spec impact) CSI-RS pattern design and/or configuration

  - (Spec impact) signal/procedure for

    - data collection

    - inference

    - fine-tuning, if applicable and feasible

    - performance monitoring

    - inter-vendor collaboration, if applicable and justified

    - handling of NW-side additional conditions, if applicable

  - Note: the study and analysis consider the model location (i.e., UE-sided model, NW-sided model)

| Company | Comment |
| --- | --- |
| FL | The proposal is to listing the potential AI/ML specific consideration. For spec impact part, based on our experience in NR, we need some LCM procedure. |

## DMRS design with AI receiver

Companies have different receiver assumption, which may result in different input/output/label, and impact on generalization or study. Below is a brief summary on receiver assumptions:

#### Proposed observation 2.2-1: (Clarification on receiver assumptions for each sub-use cases )

For 6GR AI/ML use cases identification/categorization, for the use case of low overhead DMRS with AI/ML receiver, the following receiver assumptions are reported:

For Sub-case A: Sparse orthogonal DMRS in frequency and/or time domain

- Option 1: AI-based denoise on DMRS REs only

  - Note: channel interpolation, equalization, demodulation rely on non-AI

  - Input: estimated channel on DMRS REs (e.g., LS)

  - Output: denoised channel on DMRS REs only

  - Label: Self-supervise (received signal on adjacent RE as label)

  - Complexity: 267K Para/4.35M Flops for 8PRB

  - Reported by: Samsung

- Option 2: AI-based channel estimation for data REs (with or without denoise on DMRS REs)

  - Note: equalization, demodulation rely on non-AI

  - Input:

    - Option A: Received signal on DMRS REs, [and DMRS sequence/pattern or for a given DMRS sequence/pattern]

    - Option B: Estimated channel on DMRS REs (e.g., LS or MMSE)

  - Output: Estimated channel on target REs (data REs with/without DMRS REs)

  - Label:

    - Option X: Ideal channel information on target REs (all)

    - Option Y: Estimated channel in high SNR range (e.g. 30dB) (1)

  - Complexity:

    - CMCC: 1M Para/30M Flops for 10PRB

    - Qualcomm: 800K Para/230M Flops for 4PRB 4layer

    - ZTE: 7M Flops for 8PRB 4layer

    - Samsung: 150/190/340K Para, /2.4/5.9/3.7M Flops 8 PRB2layer

    - vivo: 4.84M Flops 8PRB 4layer

  - Reported by: CMCC, Apple, Huawei, Qualcomm, Spreadtrum, ZTE, Samsung(1), vivo, Fujitsu

- Option 3: AI based channel prediction for target REs

  - Note: (interpolation), equalization, demodulation rely on non-AI

  - Input: estimated channel on DMRS REs (e.g., MMSE)

  - Output: estimated channel on target REs ((full or) partial data REs)

  - Label: estimated channel on target REs (e.g., MMSE)

  - Complexity: 2~3M Para/up to 1M Flops for 10PRB 2T2R

  - Reported by: xiaomi

- Option 4: AI-based joint {channel estimation + equalization + demodulation/LLS calculation}

  - Input:

    - Option A: Received signal on DMRS REs and data REs (all)

      - Additionally: noise variance (1,4)

    - Option B: Estimated channel on DMRS REs (e.g., MMSE) (1)

  - Output:

    - Option M: LLRs (1,2,3,4,5, 6,7,8,9,10,11)

    - Note: decoding is based on non-AI

    - Option N: Estimated constellation points (4)

    - Note: LLS calculation and decoding is based on non-AI

  - Label:

    - Option X: Known sequence/data (all other than 10)

    - Option Y: label free (10)

  - Complexity:

    - Samsung: 11M Para/84M Flops for 2PRB 2T2R

    - Lenovo: 6M Para/ for [xx]PRB 1 layer

    - MediaTek: 128M Flops for 24PRB 1layer

    - InterDigital (option Y): 0.01/0.129/1.05M Flops for 1/8/32 Rx

    - LGE: 4.88M Para/4.32MFLops [xx]PRB 16Rx

  - Reported by: Nokia (1), Ericsson (2), CATT (3), Samsung (4), Kyocera (5), Lenovo (6), MTK (7), Docomo (8), NVIDIA (9), Interdigital (10), LGE (11)

For Sub-case B: Superimposed pilot

- Option 2: AI-based channel estimation for data REs

  - Note: equalization, demodulation rely on non-AI

  - Input:

    - Option A: Received signal on superimposed signal (all)

    - Option B: Estimated channel on superimposed signal (e.g., LS) (1)

  - Output: Estimated channel on target REs (data REs)

  - Label: Idea channel information on target REs (all) or power delay profile (1)

  - Complexity:

    - Lenovo: 3M Para/ for [xx]PRB 32T4R

    - InterDigital: 3.177K/12.7K Flops for 52PRB, SISO/8T2R

    - Vivo: 744M Flops (time domain), 6.12M Flops (Doppler domain), 8PRB, 2layers, 16T4R

  - Reported by: vivo (1), Lenovo, Huawei, OPPO, InterDigital

- Option 4: AI-based joint {channel estimation + equalization + demodulation/LLS calculation}

  - Input: Received signal on DMRS REs and data REs (all)

  - Output:

    - Option M: LLRs (2, 4, 5)

    - Note: decoding is based on non-AI

    - Option N: Estimated constellation points (1,3)

    - Note: LLS calculation and decoding is based on non-AI

  - Label: Known sequence/data

  - Complexity:

    - xiaomi: 0.4M Para/ 450.89M Flops for 10PRB 2layer 2T2R

  - Reported by: ZTE (1), Huawei (2), Xiaomi (3), Docomo (4), Kyocera (5)

- Option 5 : AI-based {channel estimation} + AI-based {equalization + demodulation/LLS calculation}

For Model 1: Channel estimation

  - Input: Received signal on superimposed signal (all)

  - Output: Estimated channel on target REs (data REs)

  - Label: Idea channel information on target REs (all)

For Model 2: equalization + demodulation/LLS calculation

    - Input: Received signal on DMRS REs and data REs (all)

    - Output: LLRs (all)

    - Note: decoding is based on non-AI

  - Label: Known sequence/data

  - Complexity:

    - CMCC: 0.84M Para/ 674M Flops for 10PRB 2layer

    - OPPO: 0.08M/0.11M Para / 10.67M(8PRB 1layer) 90.44M Flops (8PRB 4layer)

  - Reported by: CMCC, OPPO, NVIDIA, LGE

For Sub-case C: DMRS free

- Option 2: AI-based channel estimation for data REs

  - Note: equalization, demodulation rely on non-AI

  - Input: Received signal on superimposed signal

  - Output: Estimated channel on target REs (data REs)

  - Label: Idea channel information on target REs

  - Reported by: Huawei

- Option 4: AI-based joint {channel estimation + equalization + demodulation/LLS calculation}

  - Input: Received signal on DMRS REs and data REs (all)

  - Output: LLRs

  - Label: Known sequence/data

  - Complexity:

    - MediaTek: 10,856M Flops for 2T4R; 128M Flops for 1T8R 6PRB

    - Lenovo: 6M Flops (xx PRB 1layer, 32T4R)

  - Reported by: NVIDIA, MediaTek, Lenovo

(a) AI Receiver Option 1

(b) AI Receiver Option 2A

(c) AI Receiver Option 2B

(d) AI Receiver Option 3

(e) AI Receiver Option 4

(f) AI Receiver Option 5

| Company | Comment |
| --- | --- |
| FL | To facilitate potential further study, clarification on AI receiver is helpful. The intention of this proposal is to capture the above as observation, similar as the tables we agreed in previous meeting. For the highlighted part, please help to double check if my understanding is correct or not, or help to fill in the number of any (delete is also ok) |

#### Proposal 2.2-2: study aspects

For the potential study on low overhead DMRS with AI/ML receiver, at least the following aspects are to be considered:

- KPIs:

  - Performance related: Throughput, DMRS overhead, BLER

  - Receiver complexity and latency

  - Overhead for fine-tuning, when applicable.

  - Inter-vendor collaboration overhead and complexity for two-sided model, when applicable.

- Benchmark

  - FFS on DMRS pattern assumption, receiver assumption, channel estimation assumption

- Other aspects for study:

  - Label quality impact for DMRS design with AI receiver

  - Performance with uplink MIMO and downlink MU-MIMO including imperfect precoder

  - Scalability, e.g., frequency domain allocation, time domain allocation, number of layers, number of Tx/Rx, modulation, DMRS pattern/sequence

  - Performance under non-ideal factor, at least including realistic Rnn estimation, TO, FO, phase noise, inference, other distortions (e.g., PA, IQ imbalance).

  - Performance with practical inter-vendor collaboration solution for two-sided model, when appliable

  - Generalization

    - Generalization performance of AI receiver over different channels

    - Generalization performance of AI receiver over different non-ideal factors, if the trained model is used to overcome non-ideal factors

  - Coexistence with the UE with non-AI based receiver, if applicable

  - (Spec impact) DMRS design and performance impact

    - Time-domain and frequency-domain sparse DMRS pattern for Sub-case A

    - DMRS sequence/pattern design, power allocation for Sub-case B

  - (Spec impact) Whether/what signal/procedure is needed for

    - data collection

    - inference

    - performance monitoring

    - inter-vendor collaboration, if applicable and justified

  - Note: the study and analysis consider the model location (i.e., UE-sided model, NW-sided model)

| Company | Comment |
| --- | --- |
| FL | The proposal is to listing the potential AI/ML specific consideration for AI receiver. For spec impact part, it is not clear on whether LCM related procedure is needed or not. |

#### Proposal 2.2-3: Labeling acquisition requirement

For the potential study on low overhead DMRS with AI/ML receiver, the following can be considered for label acquisition:

- For receiver option 1 and option 3: estimated channel

  - Label can be obtained by via conventional LS or MMSE

- For receiver option 2 and option 5 (model for channel estimation):

  - Option X: genie channel information on target REs

    - Label can be generated according to a certain channel model

      - FFS on the generalization performance to practical/unseen channel model to verify the feasibility

  - [Option Y: Estimated channel on target REs in high SNR

    - Label can be obtained by known DMRS/data received in high SNR range

      - FFS on the performance, and how to verify the quality of the label]

- For receiver option 4 and option 5(model for equalization +demodulation/LLS calculation):

  - Option X: Known sequence/data

    - Label can be generated by the known TB/DMRS at receiver, e.g., TB passed CRC

  - Option Y: label free

    - FFS on the feasibility and performance

| Company | Comment |
| --- | --- |
| FL | To facilitate potential further study, clarification on label acquisition is helpful. This is to provide some considerations on label acquisition, which is unique to AI receiver than other use cases. |

## CSI compression and feedback

#### Proposal 2.3: study aspects

For the potential study on CSI compression and feedback, at least the following aspects are to be considered:

- KPIs

  - Intermediate KPIs: SGCS, NMSE

  - System KPIs: throughput, overhead

  - Model complexity at NW side

  - Model/computation complexity and computation latency at UE side

  - Inter-vendor collaboration overhead and complexity for two-sided model, if applicable.

  - Overhead of downloadable basis/codebook/compression matrix for NW-sided model, if applicable.

- Study aspects:

  - Performance under fading channel with practical channel estimation (for Sub-case A and Sub-case B)

  - Generalization performance

    - Different NW-sided assumptions

    - Different UE side additional condition, when applicable

    - Different UL channel (for Sub-case A and Sub-case B)

    - Note: for all Sub-Cases: generalization issue studied in NR are referred as baseline

  - Scalability: e.g., number of antenna ports, subband number, UL resources for UCI (when applicable), number of payloads(when applicable)

    - Note: for all Sub-Cases: Scalability studied in NR are referred as baseline

  - Performance impacted by non-ideal factors, such as channel estimation error, PA non-linearity, phase noise, IQ imbalance, interference measurement

  - For Sub-case A and Sub-case B: whether/how to introduce explicit reliability check mechanism

  - (Spec impact)

    - For Sub-case B and Sub-case C: singling/procedure for downloadable compression matrix,

    - For Sub-case B: necessary signaling/ procedure to support JSCM transmission

  - (Spec impact) signal/procedure is needed for

    - data collection

    - inference

    - performance monitoring

    - inter-vendor collaboration, if applicable and justified

| Company | Comment |
| --- | --- |
| FL | The proposal is to listing the potential AI/ML specific consideration for AI receiver. For spec impact part, it is not clear on whether LCM related procedure is needed or not. |

## AI for beam management and extension

#### Proposal 2.4: study aspects

For the potential study on beam management and extension with AI/ML, at least the following aspects are to be considered:

- KPIs:

  - Performance related: RS overhead, prediction accuracy, throughput

  - Model complexity

- Other aspects for study:

  - Generalization

    - Sub-Case A: deployment scenarios, multi-cell/TRP configurations

    - Sub-Case B: frequency domain position

    - Sub-Case E: UE location

    - Note: for all Sub-Cases: generalization issue studied in NR are referred as baseline

  - Coexistence with non-AI capable UE

  - (Spec impact) Initial access related to beam prediction (Sub-Case D)

  - (Spec impact) signal/procedure for

    - data collection

    - inference

    - performance monitoring

    - handling of NW-side additional conditions, if applicable

  - Note: the study and analysis consider the model location (i.e., UE-sided model, NW-sided model)

| Company | Comment |
| --- | --- |
| FL | The proposal is to listing the potential AI/ML specific consideration for AI receiver. For spec impact part, it is not clear on whether LCM related procedure is needed or not. |

## AI based (De-)Modulation

#### Proposed 2.5:

For the potential study on AI-based (de)modulation [and precoding],

- For constellation design, no AI specific aspects have been identified and constellation designed with AI can be potentially study together with non-AI constellation.

- For AI receiver with constellation (and precoding) design, if justified, at least the following aspects are to be considered:

  - KPIs:

    - Performance related: Throughput, BLER

    - Receiver complexity and latency

    - Inter-vendor collaboration overhead and complexity for two-sided model, when applicable.

  - Other aspects for study:

    - Performance with (MU-)MIMO

    - Scalability, e.g., different frequency domain allocation, different time domain allocation, different number of layers, different modulation, different DMRS pattern/sequence

    - Performance gain under non-ideal factor, at least including realistic Rnn estimation, TO, FO, phase noise, inference, other distortions (e.g., PA, IQ imbalance).

    - Performance with practical inter-vendor collaboration solution for two-sided model, when appliable

    - Generalization

      - Generalization performance of AI receiver over different channels

      - Generalization performance of AI receiver over different non-ideal factors, if the trained model is used to overcome non-ideal factors

    - Coexistence with the UE with non-AI based receiver, if applicable

    - (Spec impact) modulation design

    - (Spec impact) Whether/what signal/procedure is needed for

      - data collection

      - inference

      - performance monitoring

| Company | Comment |
| --- | --- |
| FL | In general, I think if proposal 1 can be agreed, no need to spend time on this. AI receiver needs to be justified. |

## AI for DPoD/DPD/non-linearity compensation

#### Proposed 2.6:

For the potential study on AI for DPoD/DPD/non-linearity compensation, at least the following aspects are to be considered:

- Note: The study on performance/complexity are expected to be led by RAN 4.

- Other aspects for RAN 1 study:

  - For AI at receiver:

    - Feasibility/performance for finetune

    - (Spec impact) potential DMRS design/configuration impact with performance evaluation

    - (Spec impact) potential power control procedure impact

    - (Spec impact) Whether/what signal/procedure is needed for

      - data collection

      - inference

      - performance monitoring

      - online finetune, if applicable and feasible

  - For AI at transmitter:

    - Feasibility/performance for finetune

    - (Spec impact) Whether/what signal/procedure is needed for

      - data collection

      - inference

      - performance monitoring

      - online finetune, if applicable and feasible

| Company | Comment |
| --- | --- |
| FL | To facilitate potential further study, listing RAN 1 led part is helpful |

## AI for TMPI

#### Proposal 2.7:

For the potential study on AI-enabled UL precoder indication, at least the following aspects are to be considered:

- KPIs:

  - Performance related: SCGS, BLER, Throughput

  - Model complexity

  - Inter-vendor collaboration overhead and complexity for two-sided model, when applicable.

- Other aspects for study:

  - Generalization

    - deployment scenarios, UE antenna configuration

  - Coexistence with non-AI-enabled UL precoder

  - (Spec impact) The signaling/procedure related to the download/upload of UL codebooks/compressed UL precoder

  - (Spec impact) Whether/what signal/procedure is needed for

    - data collection

    - inference

    - performance monitoring

    - inter-vendor collaboration, if applicable and justified

| Company | Comment |
| --- | --- |
| FL | I think this can be directly discussed in UL MIMO. If this is not related to two-sided model. this use case can be treated as AI-enabled use case. Proposal 1 may be enough. |

## AI for SRS

#### Proposal 2.8-A: study aspects

For the potential study on low overhead SRS with AI/ML, at least the following aspects are to be considered:

- Note: this use case focus on SRS based channel acquisition only.

- KPIs:

  - Performance related: SCGS, throughput, [PAPR, Cross-correlation between SRS sequences]

  - Model complexity

- Other aspects for study:

  - Performance (including generalization performance) under fading channel with practical channel estimation (for Sub-case A and Sub-case B)

  - Coexistence with non-AI designed sequence/pattern, if applicable

  - (Spec impact) SRS design and performance

    - Time-domain and frequency-domain sparse SRS pattern for Sub-case A

    - SRS sequence/pattern design for Sub-case B

  - (Spec impact) The signaling/procedure related to the download/upload of SRS sequence/pattern

  - (Spec impact) Whether/what signal/procedure is needed for

    - data collection

    - inference

    - performance monitoring

#### Proposal 2.8-B:

For SRS design, AI/ML receiver can be considered as an implementation choice.

- FFS on whether additional procedure for data collection/performance monitoring is needed for not.

| Company | Comment |
| --- | --- |
| FL | I think this can be directly discussed in SRS design If this is not related to two-sided model. this use case can be treated as AI-enabled use case. Proposal 1 may be enough. |

## AI for PAPR reduction for waveform

#### Proposal 2.9 A:

For the potential study on AI-based waveform for PAPR reduction, at least the following aspects are to be considered:

- KPIs:

  - Performance related: BLER, CCDF of PAPR (UL), throughput (DL),

  - Model complexity

  - Inter-vendor collaboration overhead and complexity for two-sided model, when applicable

- Benchmark

  - FFS: Non-AI or implementation-based method

- Other aspects for study:

  - Generalization

    - The number of subcarriers

    - Modulation order

  - Coexistence with DFT-s-OFDM and CP-OFDM, e.g., UL MU-MIMO scenario

  - (Spec impact) Whether/what signal/procedure is needed for

    - data collection

    - inference

    - performance monitoring

    - inter-vendor collaboration, if applicable and justified

#### Proposed conclusion 2.9B:

AI based PAPR reduction for waveform can be study together with non-AI low PAPR waveform design.

| Company | Comment |
| --- | --- |
| FL | Unless two-sided model can be justified. I don’t see any special LCM related procedure is needed. conclusion should be enough. Anyway, I think if proposal 1 can be agreed, no need to spend time on this. |

## AI for Sequence based HARQ-ACK

#### Proposal 2.10: study aspects

For the potential study on AI/ML based HARQ-ACK feedback (up to11bit), at least the following aspects are to be considered:

- KPIs

  - BLER

  - Receiver complexity

- Study aspects:

  - Performance under fading channel with practical channel estimation

  - Generalization performance

    - Different UL channel

    - Different ACK/NACK probability

  - Scalability: e.g., number HARQ-ACK payload, UL resources for UCI

  - Performance impacted by non-ideal factors, such as channel estimation error, PA non-linearity, phase noise, IQ imbalance, interference measurement

  - (Spec impact)

    - Learned sequences/modulated symbols design for up to 11 bits HARQ-ACK

    - Whether/how to introduce explicit reliability check mechanism

    - Necessarily and singling/procedure for downloadable learned sequences/modulated symbols

    - Necessarily and signaling/procedure to support learned sequences/modulated symbols transmission, if needed

  - (Spec impact) whether/what signal/procedure is needed for

    - data collection

    - inference

    - performance monitoring

| Company | Comment |
| --- | --- |
| FL | The proposal is to listing the potential AI/ML specific consideration for AI receiver. For spec impact part, it is not clear on whether LCM related procedure is needed or not. |

## AI for PDCCH

#### Proposal 2.11: study aspects

For the potential study on AI/ML for PDCCH related use case, at least the following aspects are to be considered:

- Validation on the pre-requisite assumption before detailed study.

- Note: the study is not expected to be start before finishing PDCCH design.

- KPIs

  - BLER, DCI overhead

  - Decoding complexity and latency

  - Inter-vendor collaboration overhead and complexity for UE-sided and NW-sided model, if applicable.

- Study aspects:

  - Performance under fading channel with practical channel estimation

  - Generalization performance over different DL channel, missing of historical PDCCH

  - Scalability: DCI payload, aggregation level, DCI format

  - Performance impacted by non-ideal factors, e.g., interference measurement

  - (Spec impact) Whether/what signal/procedure is needed for

    - data collection

    - inference

    - performance monitoring

    - Inter-vendor collaboration overhead and complexity for UE-sided and NW-sided model, if applicable.

#### Proposed conclusion 2.12:

AI/ML for PDCCH related use case can be discussed in RAN 4 and/or RAN 1when PDCCH design has been finished.

| Company | Comment |
| --- | --- |
| FL | If this is UE-sided model, I feel like this is implementation issue… which can be ran 4 for requirement. Pre-requisite assumption may need to be validated in Ran 1. UE-sided/NW-sided model should be justified first. Anyway, I think if proposal 1 can be agreed, no need to spend time on this. |

## (P)RACH related design

#### Proposal 2.12A: study aspects

For the potential study on early contention resolution in RACH, at least the following aspects are to be considered:

- KPI

  - Prediction accuracy of UE multiplicity, first-attempt success probability, and false alarm detection rate

  - System gain, e.g., RACH access delay

  - Receiver complexity

  - Addition signaling overhead

  - Note: The collision probability of more than one UE to select the same PRACH needs to be validated.

- Study aspects:

  - Performance under fading channel

  - Performance impacted by non-ideal factors, such as timing advance, TO/FO, PA non-linearity, phase noise, IQ imbalance, interference measurement

  - Generalization performance

    - Different UL channel

    - Different received power from different UE

  - (Spec impact) necessary signaling/procedure related to random access, if any

  - (Spec impact) Whether/what signal/procedure is needed for

    - data collection

    - inference

    - performance monitoring

| Company | Comment |
| --- | --- |
| FL | I feel this can be led by RAN 4. Not sure what is the clear RAN 1 impact. I think if proposal 1 can be agreed, no need to spend time on this. |

#### Proposed conclusion 2.12:

AI based lower PAPR design for PRACH can be study together with non-AI PRACH design.

| Company | Comment |
| --- | --- |
| FL | I don’t see any special LCM related procedure is needed. conclusion should be enough. I think if proposal 1 can be agreed, no need to spend time on this. |

## AI for power control

#### Proposed conclusion 2.13:

For AI/ML for power control related use case, the study outcome of L1-RSRP prediction can be considered.

Proponent can propose necessary enhancement to corresponding agenda, e.g., power control.

| Company | Comment |
| --- | --- |
| FL | One subcase is NW-sided model which can be handled by implementation, I don’t feel any special study is needed. another use case is related to L1-RSRP, which is more like extension of beam management. Therefore, I don’t see the necessary of special handle. Any other view? In addition, I think if proposal 1 can be agreed, no need to spend time on this. |

## AI/ML based SRS power imbalance compensation

#### Proposal 2.14A: study aspects

For the potential study on AI/ML based SRS power imbalance compensation, at least the following aspects are to be considered:

- Note: The study on performance/complexity are expected to be led by RAN 4.

- Study aspects:

  - (Spec impact) Whether/what signal/procedure is needed for

    - data collection

    - inference

    - performance monitoring

#### Proposed conclusion 2.14-B:

For AI/ML based SRS power imbalance compensation, AI/ML receiver can be considered as an implementation choice, or up to RAN 4.

- FFS on whether additional procedure for data collection/inference/performance monitoring is needed for not.

| Company | Comment |
| --- | --- |
| FL | I feel this can be led by RAN 4. Not sure what is the clear RAN 1 impact. Maybe the proposed conclusion B is sufficient? Or proposal 1 is enough |

## AI for positioning

#### Proposed conclusion 2.15:

AI for positioning related use case can be considered in position related design.

| Company | Comment |
| --- | --- |
| FL | Ericsson proposed to consider AI for Pos. Maybe proposal 1 is enough |

## RAN for token traffic

#### Proposal 2.16: study aspects

For the potential study on improved scheduling/HARQ for token traffic, at least the following aspects are to be considered:

- Evaluation methodology:

  - Traffic model (depending on the outcome of traffic model discussion in agenda 11.2)

  - Note: AI/ML models are located in the application layer. RAN1 don’t need to discuss the model related.

  - Note: The coordination with other WGs on the performance evaluations is needed

- KPI: supported number of UEs, achievable throughput

- Benchmark: NR scheduling/HARQ mechanism without knowledge of token traffic

- Study aspects: Improved RAN solutions to facilitate Token communication, e.g., awareness of Token traffic unit and its characteristic at RAN, scheduling and HARQ mechanism

#### Proposed conclusion 2.16-B:

For proposed token related use case, AI/ML models are located in the application layer, which is out of the discussion scope of RAN 1.

Token traffic related system design/optimization can be further study after defining token related traffic model. Proponent can propose the token related design/optimization to corresponding agenda.

| Company | Comment |
| --- | --- |
| FL | Some new RAN for AI use cases have been submitted. We can fix the name later. Two potential directions. Personally, I am not so confident on direction A, maybe we can conclude for option B? Or proposal 1 is enough. |

## Digital twin related use cases

#### Proposal 2.17-A: study aspects

For the potential study on digital twin related use cases, at least the following aspects need to be clarified:

- Note: digital twin related use cases at least including:

  - Sensing based RAN digital twin construction with NW-side AI/ML model

  - AI/ML-enabled RAN digital twin with distributed model

  - Site Specific Learning for AI/ML and RAN Digital Twin

- Label:

  - how to obtain/verify the ground-truth label for digital twin related information, when applicable.

- KPI:

  - BLER, throughput

  - FFS on other KPIs

- Benchmark

  - The performance without the help of digital twin

  - FFS on other benchmark

- Other study aspects

  - How to verify the performance of digital twin

  - Generalization performance over different scenarios, FFS on how to define different scenarios

  - Note: whether/how to support the generation of digital twin is up to ISAC

#### Proposal 2.17-B:

Whether/how to support the generation of digital twin is up to ISAC or other agenda.

Digital twin related use case can be discussed later.

| Company | Comment |
| --- | --- |
| FL | It is hard to proceed the study on digital twin related use case study without knowing defining the function/performance of digital twin. Therefore, I suggest to go for proposal 2.11-B. we can come back when digital twin itself has been clarified. Maybe proposal 1 is enough. |

# Framework and evaluation

## Evaluation and KPIs

Several companies discussed aspect on EVM and KPIs. Several companies proposed for comprehensive evaluation of AI/ML use cases by considering KPIs including system performance, system and model complexity, inter-vendor collaboration complexity, power consumption. In addition to intermediate and system KPIs that were adopted in 5G NR, companies proposed new KPIs such power consumption and inference latency to be considered in 6GR.

#### Proposal 3.1 (RAN 1 #122b) :

For evaluation of AI/ML use cases in 6GR, consider

- Performance related metrics, including intermediate (model) performance KPIs, link level KPIs (e.g., BLER) and system level KPIs (e.g., throughput, overhead)

  - Where the overhead including the overhead associated with inferencing, performance monitoring and (re)training

- AI/ML Model related metrics, including model/computational complexity, inference latency, training latency (when applicable),

  - FFS on whether/how to measure power consumption

- Inter-vendor collaboration when applicable

- Generalization performance under a wide range of conditions

  - FFS on whether and how to consider realistic deployment scenarios

Note: Detailed metrics to be discussed per use case.

| Company | Comment |
| --- | --- |
| FL | Leftover from last meeting. The red part may need to explained by proponent companies |
| Lenovo | Model storage requirement should also be considered in the second bullet. And just a wording suggestion: - Inter-vendor collaboration complexity analysis when applicable |
| NTT DOCOMO | We suggest a further clarification for the Note in the last line, Note: Which types of KPI (intermediate, link-level, system-level) will be adopted and detailed metrics to be discussed per use case. |
| Futurewei | In the case that the model requires online fine-tuning, the overhead should also include the fine-tuning aspects. We suggest the following modification in the sub-bullet of the first main bullet: - Where the overhead including the overhead associated with inferencing, performance monitoring and (re)training and fine-tuning (if any). |
| Huawei, HiSilicon | 1, the system level KPI should also consider the metric to evaluate the efficiency of serving AI/ML traffic type (e.g., tokens) For evaluation of AI/ML use cases in 6GR, consider - Performance related metrics, including intermediate (model) performance KPIs, link level KPIs (e.g., BLER) and system level KPIs (e.g., throughput, overhead) - Where the overhead including the overhead associated with inferencing, performance monitoring and (re)training - FFS performance metric for AI/ML services such as token communication |
| CATT, CICTCI | (1) Training latency is difficult to evaluate as it is highly implementation or even hardware dependent. And it does not matter for most use cases unless they rely on fast online training. Suggest to remove it. (2) For two-sided model use cases (e.g. CSI compression related), we think ‘scalability’ should also be looked into, just the same as CSI compression study in NR. |
| SPRD | For the last bullet, what is the meaning of “a wide range of conditions”? |
| Xiaomi | - For the AI/ML related metrics, it is unclear how to measure the inter-vendor collaboration - For the power consumption, with the increase of AI use cases, the power consumption will be increased accordingly. From that sense, guranteeing the power efficiency is critical to the user throughput. Thus, power consumption should be one criteria for the use case evaluation. As for the how to measure the power consumption, we don’t think it could be simply reflected by the model complexity, it also affected by the inference frequency. Even when there is no task, certain base electronic current is also needed to maintain the AI engine on, that would also cause some energy consumption - As for the training latency, we share similar view with CATT |
| Nokia | Direction is fine. Better to discuss in the offline session for wording changes. |
| SK Telecom | For the third bullet, the wording needs to be improved like: “Complexity for inter-vendor collaboration when applicable”. For the last bullet, it is a bit unclear what it implies. What should be considered for generalized performance? |
| CMCC | If model (re)training or model fine-tuning is adopted, then model can be changed when UE moved into different areas. So, generalization performance is not needed under a wide range of conditions, maybe generalization under one or several cells is enough. - Generalization performance under a wide range of conditions |
| Tejas | Fine with the direction |
| IIT Kanpur | We agree with Futurewei’s comment regarding the inclusion of overhead aspects during fine-tuning, should the model require fine-tuning. |
| OPPO | Fine with this direction. But for some details, such as inference latency or training latency, we don’t really know how to evaluate it. Btw, training latency in Rel-18 study, it can be considered as not time critical, so why would we study it again for 6GR. |
| Ericsson | Regarding performance related metrics bullet, suggest adding a sub-bullet: - Conclusions shall be drawn based on final performance metric (e.g., throughput) instead of intermediate KPIs In addition, please add a bullet - RAN4 interoperability testing feasibility and complexity |
| InterDigital | We support the sub-bullet in red. The OH considering inferencing, training/retraining, and performance monitoring are critical aspects that determine the overall benefits of specifying an AI/ML-based solution for a functionality. In addition, we also would like to second the suggestion from Ericsson on consideration of final performance metrics. |

#### Proposal 3.1(new) :

For study/evaluation of AI/ML use cases in 6GR, consider

- Performance related metrics, including, intermediate (model) performance KPIs, link level KPIs (e.g., BLER) and system level KPIs (e.g., throughput, overhead), etc

  - including the overhead associated with inferencing, performance monitoring, online fine-tuning (if any)

  - Note: conclusions shall be drawn based on final performance metric (e.g., throughput) instead of intermediate KPIs

- AI/ML Model related metrics, including model/computation complexity/power consumption, inference latency, training latency (for online training/fine-tuning, when if applicable), scalability, etc

  - FFS on whether/how to measure power consumption

- Inter-vendor collaboration complexity and potential performance impact, when applicable

- Generalization performance under a wide range of conditions

  - FFS on whether and how to consider realistic deployment scenarios

Note: Which types of KPI (intermediate, link-level, system-level) will be adopted and detailed metrics to be discussed per use case.

Note: RAN4 interoperability testing feasibility and complexity is up to RAN 4

| Company | Comment |
| --- | --- |
| FL | Leftover from last meeting. The red part may need to explained by proponent companies |

## LCM framework

Many companies proposed enhancement on NR’s LCM, encompassing aspects such as data and model management, including model transfer, applicability of the associated ID, support for localized models, advanced training methods, e.g., online and federated learning, meta-learning for handling network-side additional conditions. Moreover, a number of companies proposed to 5G NR’s LCM framework including functionality-based LCM as a starting point. Enhancement on AI/ML processing unit framework was proposed by a few companies, e.g., 1 company (Samsung) proposed to introduce AI/ML memory unit (MU) on the concurrently activated AI/ML feature/models

#### Proposal 3.2(RAN 1 #122b):

For 6G LCM framework for AI/ML for air interface, consider the 5G NR AI/ML LCM framework as adopted in 5G Rel-19 specifications (e.g., TS38.331) as a starting point.

- Study the necessity of potential enhancements for LCM, and if justified, the enhancement details. The examples to study include:

  - Data and model management

  - Handling of additional conditions

  - Enablers for continuous (online) on-device model training/finetuning

  - framework for AI/ML processing and memory

[Note: the study strived to provide a unified LCM across use cases]

| Company | Comment |
| --- | --- |
| FL | Some modification according to Ericsson’s proposal |
| Lenovo | Generally fine with the proposal. One suggestion is to list some aspects, e.g., data collection for online training, model validation, under the third subbullet. |
| NTT DOCOMO | We are generally fine with directions in the main bullet, but feel the examples are not clearly defined currently. We also suggest keeping the last note, which should be an essential enhancement of 6GR AI/ML compared with 5GA. |
| Futurewei | We are ok with the proposal in general with a small comment regarding online model fine-tuning. We believe this should be applicable on either UE (side) or NW, so we suggest removing the wording of “on-device”: - Enablers for continuous (online) on-device model training/finetuning |
| Fujitsu | If the examples to study are listed in the proposal, then performance monitoring should be included which is important part of LCM. The following change is suggested. - Study the necessity of potential enhancements for LCM, and if justified, the enhancement details. The examples to study include: - Data and model management - Performance monitoring - Handling of additional conditions - Enablers for continuous (online) on-device model training/finetuning - framework for AI/ML processing and memory |
| CATT, CICTCI | We think ‘continuity of UE-side AI/ML features across cells’ should be studied. This is to guarantee the UE-side AI/ML model/functionality can be smoothly functioning/switching among different cells around handover time. |
| SPRD | For the last example, it is necessary to clarify what components are included in the framework for AI/ML processing. |
| Huawei, HiSilicon | We do not think RAN1 can determine the 6GR AI/ML LCM framework on behalf of RAN2 – notice that TS38.331 spec is RAN2 spec, they are more appropriated to determine whether to reuse 5G or make a new architecture. For 6G LCM framework for AI/ML for air interface, consider the 5G NR AI/ML LCM framework as adopted in 5G Rel-19 specifications (e.g., TS38.331) as a starting point. |
| Ofinno | Generally fine. On model management, is it correct understanding to include performance monitoring aspects also? |
| Xiaomi | - For the LCM Framework, we share similar view with Huawei. The determination of LCM framework is RAN2 work. 2. For the third sub bullet, we share similar view with Futurewei, the “on-device” should be removed 3. For the Note part, we suggest to remove the bracket, it is quite important principle for the LCM framework |
| Nokia | Ok with the proposal with Futurewei update. Mentioning of e.g., TS38.331 is not needed. |
| SK Telecom | We think defining the unified AI/ML framework across use cases is very important in order to avoid any excessive framework, and thus prefer to keep the last note. Regarding the examples, it seems still necessary to be further discussed, and maybe the wording would be better to be open a bit more. |
| CMCC | We are generally supportive of this proposal. But if we consider two-sided model, 5G Rel-20 specifications are also needed as a starting point, and the 5G NR AI/ML LCM framework is not only described in TS 38.331, but also TS 38.214, TS 38.212, etc. For 6G LCM framework for AI/ML for air interface, consider the 5G NR AI/ML LCM framework as adopted in 5G Rel-19/20 specifications (e.g., TS38.331) as a starting point. |
| LG Electronics. | We are generally fine, but we may need to further study on efficient and faster model switching by considering fall back operation to non-AI and allowing performance monitoring for inactive model. So, we suggest following: - Study the necessity of potential enhancements for LCM, and if justified, the enhancement details. The examples to study include: - Data and model management - Handling of additional conditions - Enablers for continuous (online) on-device model training/finetuning - Faster and more efficient model/functionality switching (e.g. via fall back operation to non-AI, performance monitoring for inactive model) - framework for AI/ML processing and memory |
| TCL | Agree |
| Tejas | We are supportive to this proposal |
| IIT Kanpur | We are fine with the proposal. |
| NEC | “5G NR AI/ML LCM framework as adopted in 5G Rel-19 specifications” is use case dependent and is very CSI framework dependent. It may not fit 6G use cases especially those non-MIMO use cases. We think it should be removed for this early disucssion. |
| vivo | @Futurewei, online model training/finetuning is different from on device. The major difference is the latency required. For online model training, it may require stringent timeline but on device training/finetuning may not necessarily have stringent timeline. If you want to add NW side online model training/finetuning, it can be explicitly added. But on device finetuning should not be removed. |
| OPPO | The LCM of 5G-A supports CSI use case and positioning. If we take the CSI report configured in TS 38.331 as a starting point, it may not be applicable to other non-CSI use cases, such as DMRS OH reduction. At the beginning of 6G, it would be a good chance to strive for a unified LCM framework. |
| InterDigital | We support the suggested change in the first bullet from Huawei. In fact, we suggest to simplify the first and second bullets as: - For 6G LCM framework for AI/ML for air interface, study potential enhancements relative to 5G NR AI/ML LCM framework. We are fine to keep the examples (including performance monitoring bullet suggested by Fujitsu) but do not think they are critical since they encompass all of LCM aspects. |
| Samsung | Support |
| MediaTek | We wish to add a bullet point: “leveraging cross-use case similarities” e.g., CSI-related use cases could have same model input data (e.g., CSI-RS measurements) and some parts of the model architecture could also be reused across CSI-related use cases. |

#### Proposal 3.2:

For 6G LCM framework for AI/ML for air interface, consider the 5G NR AI/ML LCM framework as adopted in 5G Rel-19 specifications (e.g., TS38.331) as a starting point.

- Study the necessity of potential enhancements for LCM, and if justified, the enhancement details. The examples to study include:

  - Data and model management (led by RAN 2)

  - Performance monitoring

  - Handling of additional conditions, e.g., continuity of UE-side AI/ML features across cells

  - Enablers for continuous (online) on-device model training/finetuning

  - Framework for AI/ML processing and memory

  - Leveraging cross-use case similarities for a unified LCM

[Note: the study strived to provide a unified LCM across use cases]

| Companies | Comment |
| --- | --- |
| FL | FL：1) delete adopted… for more open study 2) mentioned LCM, 3) UE-sided model 4) delete on device (maybe on NW), add last bullet and delete the notes. Also, in my understanding, other than data collection, all other aspects maybe discussed per use case. I understand companies want to strive to a common “framework”. However, how can the of AI receiver be the same as CSI feedback or BM? But I support MTK’s comment on similarities across use cases. Not sure whether this proposal is needed or workable. |

## Data collection framework

A number of companies discussed data collection framework in their contribution. The following summarizes the discussion points

- Enhancement in the data collection framework for future-proof and unified (across working groups) design.

- Scope and restrictions, e.g., whether to restrict data collection to use cases or to support generic purpose data collection.

- Whether to introduces a new AI/ML data management plane

Some of the proposals may not be under the realm of RAN1. However, RAN1 may identify requirements which may consequently suggest enhancement in the relevant working group. With this in mind, the RAN1 study may focus in identifying requirements that may lead to data collection framework enhancement.

#### Conclusion 3.3(RAN 1 #122bis):

For AI/ML study in 6GR, RAN1 to study on the content and format for data collection for each use case.

| Company | Comment |
| --- | --- |
| Lenovo | It should be clarified that the data collection in this conclusion at least includes the date collection for model training (at least for offline training). Both NW-side and UE-side data collection should be considered. |
| NTT DOCOMO | Support. |
| Fujitsu | Is this proposal applicable to offline training, online training and finetuning? |
| CATT, CICTCI | Support. But another question is that, does RAN1 need to analyze data collection framework in physical layer in high-level? For example, should RAN1 analyze the pros and cons if CSI related AI/ML data collection is imbedded in CSI reporting framework in physical layer? |
| SPRD | OK |
| Nokia | Let RAN2 discuss this. |
| SK Telecom | Support in principle for studying this but not sure if this would be the scope of RAN1. |
| CMCC | Support. When we discuss the requirements of data collection, both offline training, online training can be considered subject to each use case. |
| LG Electronics | Agree in principle, but what is the motivation of this conclusion. Anyway, when we discuss each use case, this part can be also studied. |
| TCL | Agree |
| vivo | Support |
| OPPO | Open to discuss. At least the content of data collection of RAN1 studied use cases should fall into RAN1 territory. |
| Ericsson | Support in general. Suggesting updating “each use case” to “each use case led by RAN1” |
| Samsung | Support |

#### Conclusion 3.3(new):

For AI/ML study in 6GR, RAN1 to study on the content and format for data collection at least for offline training for each use case.

| Company | Comment |
| --- | --- |

# Proposals for online

# Appendix

## RAN 1 #122

Agreement

For 6GR AI/ML use cases identification/categorization, for each (sub-)use case proposed, proponent companies are encouraged to study and report the following:

- Definition of each (sub-)use case, including at least AI/ML model input/output

- The evaluation assumption, methodology, KPIs, benchmark, and preliminary simulation results

- Assumption on training types, e.g.,

  - offline training, online training/finetuning

  - Label construction (if applicable), including whether/how to obtain label data for model training

- Assumption on model location for inference, e.g., UE-sided model, NW-sided model, and two-sided model

- Collaboration/interaction between UE and NW, e.g.,

no collaboration/interaction

UE/Network collaboration targeting at separate or joint ML operation

- High level potential specification impact

## RAN1 #122bis

### Low overhead CSI-RS or CSI prediction with AI/ML

Observation

For 6GR AI/ML use cases identification/categorization, [24 sources] provided preliminary simulation results and analysis on low overhead CSI-RS or CSI prediction with AI/ML.

- [23 sources] provided preliminary simulation results and analysis on frequency and/or spatial domain CSI prediction with sparse/low overhead CSI-RS with AI/ML. Detailed evaluation assumptions (model input/output/label/benchmark/KPI/ training type) and initial analysis can be found in Table A.

- [6 sources] provided preliminary simulation results (or by citing to NR study for CSI time domain prediction) and analysis on CSI time domain prediction with AI/ML wherein [3 sources] assumed Rel-19 CSI prediction while [3 sources] assumed differently. Detailed evaluation assumptions (model input/output/label/benchmark/KPI training type) and initial analysis can be found in Table B.

- [4 sources] provided preliminary simulation results and analysis on CSI prediction cross carrier/band/frequency block with AI/ML. Detailed evaluation assumptions (model input/output/label/benchmark/KPI/training type) and initial analysis can be found in Table B.

- [2 sources] provided preliminary simulation results and analysis on CSI prediction across analog beams with AI/ML. Detailed evaluation assumptions (model input/output/label/benchmark/KPI training type) and initial analysis can be found in Table B.

- [ 1source] provided preliminary simulation results and analysis on, CSI prediction with linear projection as pre-processing. Detailed evaluation assumptions (model input/output/label/benchmark/KPI training type) and initial analysis can be found in Table B.

- and time domain CSI prediction combining CSI-RS and DMRS measurements (MediaTek).

Note: whether/how to capture the observation in the TR is a separate discussion.

Table A

| Sub-use case | Sub-Case A: Frequency and/or spatial domain CSI prediction with sparse/low overhead CSI-RS with AI/ML |
| --- | --- |
| Reported companies | (23) Ericsson1, ZTE2, vivo3, OPPO, Xiaomi, CMCC, Huawei4, Samsung, Fujitsu, Apple, Qualcomm5, Kyocera6, Nokia7, {Spreadtrum, UNISOC}8, Interdigital9, Lenovo, LGE10, DoCoMo11, CEWiT, IITM, IIT Kanpur, Tejas, {CATT, CICTCI}12 |
| Model input (for decoder of 2-sided model, when applicable) | 1. Measurement of channel with sparse/low overhead CSI-RS (majority) 1a. Additional long-term multi-path power/angle/delay info information as assistance information4 2. Reported CSI for NW-sided model3,4,5 |
| Model output (for decoder of 2-sided model, when applicable) | 1. Full channel matrix (majority) 2. Eigenvector 3 for NW-sided model 3. Channel matrix/eigenvector with different/targeted antenna on/off patterns3, 12 |
| Label | 1. Estimated/ideal channel matrix based on full CSI-RS density(majority) 2. Ideal precoding matrix with full dimension3 3. Estimated/ideal channel matrix/eigenvector with different/targeted antenna on/off patterns3, 12 |
| Training types | Offline training(majority) Online finetuning for UE-sided model (for NW-sided model + UE sided model without training collaboration)4 |
| KPI | NMSE, SGCS, throughput, ratio of CSI-RS overhead |
| Benchmark | 1. non-AI based on full CSI-RS 2. non-AI based on sparse CSI-RS |
| Model location for inference | UE-sided model NW-sided model2,3, 4,5,6 Two-sided model3 NW-sided model + UE-sided model without training collaboration4 |
| Collaboration/interaction between UE and NW | As UE-sided model in NR As NW-sided model in NR As two-sided model for CSI compression4 in NR |
| Potential spec impact | 1.Sparse CSI-RS design and corresponding feedback (especially for NW-sided model) 2. Signalling/ procedure related to LCM 3. Inter-vendor collaboration for two-sided model, when applicable |

Table B

| Sub-use case | Sub-Case B: CSI time domain prediction (as Rel-19 CSI prediction or extension) | Sub-case C: CSI prediction cross carrier/band/frequency band | Sub-Case D: CSI prediction across analog beams | Sub-Case E: prediction with linear projection as pre-processing |
| --- | --- | --- | --- | --- |
| Reported Companies | (6) Ericsson2, BJTU, Samsung, MediaTek3, LGE, vivo1 | (4) Samsung, Apple, LGE, DoCoMo1 | (2) Samsung, vivo1 | (1) Huawei |
| Model input | 1. Channel matrix over K CSI-RS occasions 2. Measurements of interference over K CSI-RS occasions1 3. Channel matrix over K CSI-RS occasions with >20ms periodicity3 4 Channel matrix with one P CSI-RS with 20ms periodicity and K-1 AP CSI-RS2 | Channel matrix of carrier/band/frequency block A | Channel matrix of Set B of beams | K past CSI information after linear projecting |
| Model output | 1. Channel matrix of future instances 2. Interference in future instances1 | Channel matrix of carrier/band/frequency block B | Channel matrix of Set A of beams | Predicted CSI information after linear projecting at a future time instance |
| Label | Measurement in future time occasions. | Channel matrix of carrier/band/frequency block B | Channel matrix of Set A of beams | Ground-truth CSI information after linear projecting, based on the measurement at the future time instance |
| Training types assumption | offline training | offline training | offline training | Online finetuning |
| KPI | NMSE, SGCS, throughput, [ratio of CSI-RS overhead] | SGCS, NMSE, throughput, ratio of CSI-RS overhead | SGCS, NMSE, throughput, ratio of CSI-RS overhead | SGCS |
| Benchmark |  | 1.Ground truth of target frequency block 2. Sample and hold | Ground truth of Set A of beams | 1.Non-AI based CSI prediction 2.AI-based CSI prediction based on CSI information without linear projection |
| Model location for inference | UE-sided model NW-sided model1 | UE-sided model NW-sided model1 | UE-sided model NW-sided model1 Two-sided model1 | UE-sided model |
| Collaboration/interaction between UE and NW | As UE-sided model in NR As NW-sided model in NR1 | As UE-sided model in NR | As UE-sided model in NR | Similar to UE-sided model in NR |
| Potential spec impact | 1. As AI based CSI prediction in NR 2. Reporting content, signalling and procedure for LCM for extension cases1 | 1. Cross carrier/frequency switching procedure enhancement based on predicted CSI 2. signalling/ procedure related to LCM | 1.CSI-RS configuration for predicted beams 2. signalling/ procedure related to LCM | Signaling/ procedure related to LCM considering online finetuning |

### Low overhead DMRS with AI/ML receiver

Observation

For 6GR AI/ML use cases identification/categorization, [23 sources] provided preliminary simulation results and analysis on low overhead DMRS with AI/ML receiver.

- [22 sources] provided preliminary simulation results and analysis on sparse orthogonal DMRS in frequency and/or time domain with AI/ML receiver.

- [11 sources] provided preliminary simulation results and analysis on superimposed pilot with AI/ML receiver.

- [5 sources] provided preliminary simulation results and analysis on DMRS free with AI/ML receiver.

- Detailed evaluation assumptions (model input/output/label/benchmark/KPI/ training type) and initial analysis can be found in Table C.

Note: whether/how to capture the observation in the TR is a separate discussion.

Table for detailed assumptions of each category will be added.

Table C, For low overhead DMRS with AI/ML receiver

| Sub-use case | Sub-case A: Sparse orthogonal DMRS in frequency and/or time domain | Sub-case B: Superimposed pilot | Sub-case C: DMRS free |
| --- | --- | --- | --- |
| Reported companies | (23) Nokia1, Futurewei2, Ericsson3, ZTE4, {Spreadtrum, UNISOC}5, Interdigial6, vivo7, xiaomi8, CMCC9, {CATT, CICTCI}10, Fujitsu11, Apple12, Samsung13, Kyocera14, Lenovo15, Huawei16, Qualcomm 17, Ofinno18, NVIDIA19, MediaTek20, Lekha21, LGE22, DocoMo23 | (12) vivo 1, CMCC2, ZTE3, Lenovo4, Huawei5, OPPO6, NVIDIA7, LGE8，Xiaomi9 , InterDigital10 , DocoMo11 Kyocera12 | (5) InterDigital1, Huawei2, NVIDIA3, MediaTek4, Lenovo5 |
| Model input | 1. Received signal/estimated channel at DMRS and received signal on data 1,13, 22,15,3,17,10,4, 20,6,18,23 1a. additionally noise variance 1,13 2. Received signal/estimated channel at DMRS2,7, 8,11,12,13,16,5,23 | 1. Received signal and DMRS sequence (superimposed signal) (Majority) 2. Estimated channel (in delay doppler domain) from the received signal of target REs (superimposed signal) 1 For Tx side of two-sided model: modulated symbols and DMRS symbol5 | Received signal For Tx side of two-sided model: coded bit2,5 |
| Model output | 1. Estimated channel at target data and/or DMRS REs2,4,5,7,8,9,11, 12, 13,16,17,18,19,21,22,23 1a. Estimated noise variance 12 2. LLRs1,2,3, 4, 6,10, 13,15,19,20, 22 3. Filtering coefficients for channel estimation 7 | 1. Estimated channel at target data REs1,3,4,5,6,8 2.LLR2,3, 5,6,7,8,11,12 3.Estimated modulation symbols9 For Tx side of two-sided model: superimposed signal5 | 1.LLR (majority) 2.Estimated channel2 For Tx side of two-sided model: modulated data symbols 5,2 |
| Label | 1. Ideal channel information 2,5,7,8,9,11,12,13,15,16,17,18,22,23 2. Known sequence/data1,2,3, 4,10, 13,15,16,20,22 3. Label free (unsupervised)6, 21 4. Estimated channel using legacy DMRS pattern with legacy receiver8 5. Estimated channel of adjacent RE (self-supervised)13 | 1. Known sequence/data 2,3,11,12 2. Ideal channel information1,8 3.Transmitted modulation symbols9 | 1. Known sequence/data 2 ideal channel information2 3. Label free1 |
| Training types assumption | offline training | offline training | offline training |
| KPI | MSE, BLER, throughput | MSE, BLER, throughput | MSE, BLER, throughput |
| Benchmark | With ideal channel information With conventional receiver with sparse or legacy DMRS | With ideal channel informal With conventional receiver with legacy DMRS overhead | With ideal channel information With conventional receiver with legacy DMRS overhead |
| Model location for inference | UE-sided model for DL or NW-sided model for UL | UE-sided model for DL NW-sided model for UL Two-sided model5 | UE-sided model for DL1 NW-sided model for UL3,4 Two-sided model2,5 |
| Collaboration/interaction between UE and NW | Similar to UE-sided or NW-sided model as NR | Similar to UE-sided or NW-sided model as NR Similar to two-sided model as NR | Similar to UE-sided model as NR Similar to NW-sided model as NR Similar to two-sided model as NR |
| Potential spec impact | 1. DMRS design 2. RAN 4: Demod requirement 3. Signalling/ procedure related to LCM for UE and/or NW sided model Etc. | 1. DMRS design 2. RAN 4: Demod requirement 3. Signalling/ procedure related to LCM for UE and/or NW sided model or two-sided model (including inter-vendor calibration), when applicable Etc. | 1. RAN 4: Demod requirement 2. Signalling/ procedure related to LCM for UE and/or NW sided model or two-sided model (including inter-vendor calibration), when applicable 3. Constellation design and related signalling/procedure Etc. |

### CSI compression and feedback

Observation

For 6GR AI/ML use cases identification/categorization, [13 sources] provided preliminary simulation results and analysis on CSI compression and feedback.

- [10 sources] provided preliminary simulation results and analysis on CSI compression with joint source and channel coding (JSCC)

- [11 sources] provided preliminary simulation results and analysis on CSI compression with joint source, channel coding and modulation (JSCM)

- [2 sources] provided preliminary simulation results and analysis on CSI feedback with downloadable basis/codebook.

- [3 sources] provided preliminary simulation results (or cite to NR AI/ML for CSI compression simulation results) and analysis on CSI reconstruction with CSI feedback with SRS (assuming separate source and channel coding).

- Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in in Table D.

Note: whether/how to capture the observation in the TR is a separate discussion.

Table D

| Sub-use case | Sub-case A: CSI compression with JSCC | Sub-case B: CSI compression with JSCM | Sub-case C: DLable basis/codebook | Sub-case D: CSI reconstruction with CSI feedback with SRS (assuming SSCC) |
| --- | --- | --- | --- | --- |
| Reported companies | (10) ZTE1, Samsung2, vivo3, {Pengcheng, ZGC}, Lenovo, OPPO, MediaTek4, Fujitsu, BJTU5, {BUPT, ZGC}6 | (11) BJTU1, Samsung2, OPPO3,{Pengcheng, ZGC}4,vivo, CMCC, ZTE, {BUPT, ZGC}7, Fujitsu8, Apple, Lenovo | (2) ZTE1, Samsung | (3) Qualcomm, vivo, Samsung |
| Model input of decoder or model output of encoder, when applicable | 1. Compressed CSI bits 1a. additionally estimated channel based on SRS2,3 1b. (for training), assuming the model input via error bits caused by in UL transmission after legacy channel decoding4 | 1. Compressed CSI complex values via UE-sided model 2. Compressed CSI complex values via a projection matrix1,2,3 3. Received signal at sparse CSI-RS and CSI-RS sequence 1,4 | 1.Amplitudes and phases obtained by a look up table based on feedback CSI bits 2. Selected basis1 | 1. Compressed CSI bits 2. Estimated channel based on SRS |
| Model output of decoder or model input of encoder, when applicable | 1. (Reconstructed) Eigenvectors 2. (Reconstructed) Explicit H1,2,3,4 | 1. (Reconstructed) Eigenvectors 2. (Reconstructed) Explicit H2 | Reconstructed Eigenvectors | (Reconstructed) Eigenvectors |
| Label | 1.Eigenvectors 2.Explicit H1,2,3,4 | 1.Eigenvectors 2.Explicit H2 | Eigenvectors | Eigenvectors |
| Training types | Offline training | Offline training | Offline training | Offline training |
| KPI | SGCS, NMSE, SE, UE complexity | SGCS, NMSE, SE, UE complexity | UPT vs overhead | SGCS, UPT |
| Benchmark | eType II NR separate source and channel coding | eType II NR separate source and channel coding JSCM with two-sided model1,2,3 | eType II | eType II NR AI/ML CSI compression without SRS SRS without CSI feedback |
| Model location for inference | Two-sided model | Two-sided model NW-sided model1,2,3 | NW-sided model | Two-sided model |
| Collaboration/interaction between UE and NW | Similar to two-sided model in NR | Similar to two-sided model in NR For NW-sided model: no collaboration or Similar to NW-sided model in NR | No collaboration or Similar to NW-sided model in NR | Similar to two-sided model in NR |
| Potential specification impact | 1. Necessary signalling/ procedure to support JSCC 2. Signalling/ procedure related to LCM for two-sided model including inter-vendor collaboration | 1. Necessary signalling/ procedure to support JSCM 2. Projection matrix design for NW-sided model, when applicable 3. Signalling/ procedure related to LCM with NW-sided model or two-sided model including inter-vendor collaboration, when applicable 4. RAN4 requirements, e.g., EVM | 1. Downloadable basis/codebook related signalling/ procedure 2. Signalling/ procedure related to LCM with NW-sided model | 1. Necessary signaling/procedure to support lower overhead and/or simpler CSI feedback 2. Signalling/ procedure related to LCM for two-sided model including inter-vendor collaboration |

### AI/ML for beam management and extension

Observation

For 6GR AI/ML use cases identification/categorization, [13 sources] provided preliminary simulation results and analysis on AI/ML for beam management and extension.

- [7 sources] provided preliminary simulation for DL Tx beam management and analysis on inter-cell/inter-TRP/M-TRP DL Tx beam prediction and management.

  - Besides, [5 sources] citing to NR study for DL Tx beam management and analysis on inter-cell/inter-TRP/M-TRP DL Tx beam prediction and management.

- [3 sources] provided preliminary simulation results and analysis on cross frequency DL Tx beam prediction.

- [2 sources] provided preliminary simulation results and analysis on Tx-Rx beam pair prediction.

- [2 sources] provided preliminary simulation results for beam management and analysis on beam prediction for initial access.

  - Besides ,[5 sources] citing to NR study for beam management and analysis on beam prediction for initial access.

- [1 source] provided preliminary simulation results and analysis on DL Tx beam prediction for spatial and/or temporal domain with additional local UE information.

- [1 source] provided preliminary simulation results and analysis on reinforcement learning-based approach beam selection

- Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in in Table E.

Note: whether/how to capture the observation in the TR is a separate discussion.

able E-1 AI/ML for beam management and extension

| Sub-use case | Sub-case A: Inter-Cell/M-TRP DL Tx beam prediction and management | Sub-Case B: Cross frequency DL Tx beam prediction | Sub-Case C: Tx-Rx beam pair prediction | Sub-Case D: Beam prediction for initial access | Sub-Case E: DL Tx beam prediction for spatial and/or temporal domain with additional local UE information | Sub-Case F: reinforcement learning-based approach beam selection |
| --- | --- | --- | --- | --- | --- | --- |
| Reported companies | (7) Nokia, ZTE, xiaomi, CEWiT, DoCoMo, , Lenovo, BJTU (5) Qualcomm, Samsung, LGE, NVIDA, CEWiT (citing to NR study) | (3) Futurewei1, xiaomi2, Apple3 | (2) Ericsson, Nokia | (2) Huawei, vivo (5) Qualcomm, Samsung, LGE, ZTE, Apple (citing to NR study) | (1) Huawei | (1) Nokia |
| Model input | Measurements from Set B of one or more TRPs/Cells | Measurements in frequency A | Measurements from Set B DL Tx-Rx beam pairs. | Measurements from Set B of SSB | Measurements from Set B And additional local UE information (moving direction and speed) as UE side model input | Measurements from a set of DL Tx beam scheduling stats (at the NW), Cross corelation among DL Tx beams |
| Model output | Predicted best beam information and/or predicted measurements from Set A of target cell/TRP(s) [of current or future time instance] | Predicted cell/beam related information of frequency B [of current or future time instance] | Predicted best DL Tx-Rx beam pairs information from Set A DL Tx-Rx pairs. | Predicted best DL Tx beam information (and/or predicted measurements from Set A [of current or future time instance] | Predicted Best beam indexes (probability of each Tx beam in Set A to be the Top-1 Tx beam) and/or Predicted measurements from Set A [of current or future time instance] | Selected beam index for scheduling UE(s) |
| Label | Measurements [or Top beams] of Set A of target cell/TRP(s) | Measurements on cell(s)/beam(s) of frequency B | Measurements [or Top beams pairs] of Set A Tx-Rx pair | Measurements [or Top beams] of Set A | Measurements [or Top beams] of Set A | label-free (online learning) |
| Training types assumption | offline training | offline training | offline training | offline training | offline training; online finetuning (for UE side model) | Online learning |
| KPI | Prediction cell/beam/measurement accuracy, Throughput, RS overhead reduction | Prediction beam/measurement accuracy, RS overhead reduction | Prediction beam/measurement accuracy, RS overhead reduction | Prediction beam accuracy | Prediction beam/measurement accuracy | Throughput, End to end packet latency |
| Benchmark | Based on Set A Based on Set B | Measurements on cell(s)/beam(s) of frequency B | Based on Set A Based on Set B | Based on Set A Based on Set B | NR beam prediction with AI/ML | Beam with largest RSRP (from the set) consider as the scheduling beam |
| Model location for inference | UE-sided model or NW-sided model | UE-sided model or NW-sided model | UE-sided model | UE-sided model or NW-sided model | NW-sided model + UE-sided model without training collaboration | NW-sided model |
| Collaboration/interaction between UE and NW | As UE-sided or NW-sided model in NR | As UE-sided or NW-sided model in NR | As UE-sided model in NR | Similar to UE-sided or NW-sided model in NR | As UE-sided or NW-sided model in NR | No collaboration |
| Potential spec impact | 1. Inter-Cell/M-TRP beam prediction related singling/procedure 2. Signalling/ procedure related to LCM for NW-sided model or UE-sided model | 1. Cross frequency DL Tx beam prediction related signalling /procedure 2. Signalling/ procedure related to LCM for NW-sided model or UE-sided model | 1.Signalling/ procedure related to LCM for UE-sided model | 1. Initial access related to beam prediction 2. Signalling/ procedure related to LCM for NW-sided model or UE-sided model | 1. As NR AI for BM; 2. Signalling/ procedure related to NW-sided model + UE-sided model. 3. Signalling/ procedure related to online finetuning, if any | 1. Signalling/ procedure related to exploration phase (to mitigate the impact of exploration). |

### AI/ML for (de)modulation

Observation

For 6GR AI/ML use cases identification/categorization, [5 sources] provided preliminary simulation results and analysis on (de)modulation.

- [5 sources] provided preliminary simulation results and analysis on modulation constellation design with the help of AI, and with non-AI or AI receiver.

- [3 sources] provided preliminary simulation results and analysis on AI-based modulation and precoding with two-sided model.

- Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in Table F.

Note: whether/how to capture the observation in the TR is a separate discussion.

Table F For (de)modulation

| Sub-use case | Sub-use case A: AI-based (de)modulation | Sub-use case B: AI-based modulation and precoding |
| --- | --- | --- |
| Reported companies | (5)ZTE1, OPPO2, vivo3, Lenovo4, Xiaomi5 | (3) ZTE, OPPO, Lenovo |
| Model input | For constellation design 1. Coded bits 1,2,3,4,5 2. Channel characterization and modulation order4 For AI receiver 1.Received signal2,3,4 | Encoder: Coded bits Decoder: Estimated symbols |
| Model output | For constellation design 1. Learned constellation 1,2,3,45 2. Probability of constellation points 4 For receiver 1. LLR2,3,4 | Encoder: modulated symbols after layer mapping Decoder: Soft LLR |
| Label | Known coded bits | Known coded bits |
| Training types | Offline training | Offline training |
| KPI | BLER | BLER |
| Benchmark | Uniform QAM with legacy receiver | Uniform QAM with legacy receiver and NR layer mapping |
| Model location for inference | 1.NA (AI for constellation design with legacy receiver) 1,2,3,4,5 2.Receiver-sided model2,3,4 | Two-sided model |
| Collaboration/interaction between UE and NW | NA or Similar to NW-sided model or UE-sided model in NR | Similar to two-sided model in NR |
| Potential specification impact | 1. Constellation design and related signaling/procedure 2. Signaling/ procedure related to LCM for NW-sided model or UE-sided model 3. RAN4 requirements, e.g., EVM | 1. Modulation design and layer mapping design 2. Signaling/ procedure related to LCM for two-sided model including inter-vendor collaboration 3. RAN4 requirements, e.g., EVM |

### AI-based none-linearity handling at transmitter or receiver

Observation

For 6GR AI/ML use cases identification/categorization, [5 sources] provided preliminary simulation results and analysis on AI-based none-linearity handling at transmitter or receiver.

- [5 sources] provided preliminary simulation results and analysis on AI-based DPoD/None-linearity compensation at receiver.

- [2 sources] provided preliminary simulation results and analysis on AI-based DPD at transmitter.

- Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in Table G.

Note: whether/how to capture the observation in the TR is a separate discussion.

| Sub-use case | Sub-use case A: AI-based DPoD/None-linearity compensation | Sub-use case B: AI-based DPD |
| --- | --- | --- |
| Reported companies | (5) Samsung1, Ericsson2, OPPO3, vivo4, Huawei5 | (2) vivo2, Huawei1 |
| Model input | 1. Received signal1,3,4,5 | Time domain samples before pre-distortion |
| Model output | 1. Compensated signal in time domain1,2,4,5 2. Soft bits2,3 | Time domain samples after pre-distortion |
| Label | 1. DMRS1 2. Known bit sequence2,3,4 3. time domain samples from known sequence5 | Time domain samples |
| Training types | Online training/finetune1 Offline training | Offline training Online training/finetune2 |
| KPI | BLER, MPR, EVM, throughput | BLER, EVM, MPR |
| Benchmark | Without compensation | No DPD |
| Model location for inference | NW-sided model | UE-sided model |
| Collaboration/interaction between UE and NW | Similar to NW-sided model as NR | Similar to UE-sided model as NR |
| Potential specification impact | 1. RAN 4 requirements, e.g. EVM 2. DMRS/Sequence design/selection, Tx power determination 3. Signaling/ procedure related to LCM for NW-sided model | 1. RAN4 requirements, e.g. EVM 2. Tx power determination 3. Signaling/ procedure related to LCM for UE-sided model |

### Low overhead SRS with AI/ML

Observation

For 6GR AI/ML use cases identification/categorization, [4 sources] provided preliminary simulation results and analysis on low overhead SRS with AI/ML

[1 source] provided preliminary simulation results and initial analysis on low PAPR SRS sequence design with help of AI/ML

Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and analysis in Table I.

Note: whether/how to capture the observation in the TR is a separate discussion.

Table I SRS with AI/ML

| Use case | Low overhead SRS with AI/ML | Low PAPR SRS sequence design |
| --- | --- | --- |
| Reported companies | (4) {Spreadtrum, UNISOC}, vivo, Huawei, Kyocera | (1) vivo |
| Model input | Measurement of channel with low overhead SRS of frequency/temporal domain | Sequence index |
| Model output | Estimated channel | Learn sequences |
| Label | Ideal channel information | Label free |
| Training types | Offline training | Offline training |
| KPI | SCGS, throughput | PAPR, SGCS, Cross-correlation between SRS sequences |
| Benchmark | With legacy SRS With ideal channel information | Legacy SRS sequence |
| Model location for inference | NW-sided model | NW-sided model or Without model for inference |
| Collaboration/interaction between UE and NW | Similar to NW-sided model in NR | No collaboration for no model Similar to NW-sided model in NR |
| Potential specification impact | 1.Sparse SRS design 2. Signalling/ procedure related to LCM for NW-sided model | 1. SRS design 2. Signaling/procedure related to DLable/ULable SRS sequence, when applicable 3. Signalling/ procedure related to LCM for NW-sided model, when applicable |

### AI-enabled UL precoder indication

Observation

For 6GR AI/ML use cases identification/categorization, [3 sources] provided preliminary simulation results and analysis on AI-enabled UL precoder indication with detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in Table H.

Note: whether/how to capture the observation in the TR is a separate discussion.

Table H AI-enabled UL precoder indication

| Use case | AI-enabled UL precoder indication |
| --- | --- |
| Reported companies | (3) vivo1, Fujitsu2, Samsung3 |
| Model input of decoder or model output of encoder | UL precoder indicator/compressed UL precoder |
| Model output of decoder or model input of encoder | (Reconstructed) eigenvectors of UL channel |
| Label | Estimated eigenvectors of UL channel based on SRS measurement |
| Training types | offline training online finetune1 |
| KPI | SCGS, BLER |
| Benchmark | NR TPMI codebook |
| Model location for inference | No model for inference 1,3 Two-sided model1,2 |
| Collaboration/interaction between UE and NW | Similar to one-sided model in NR 1,3 Similar as two-sided model in NR1,2 |
| Potential specification impact | 1.The signaling/procedure related to the download/upload of UL codebooks/compressed UL precoder 2. LCM procedure to facilitate the training of the downloadable/uploadable UL codebooks when no model for inference, 1,3 3. Signalling/ procedure related to LCM for for two-sided model including inter-vendor collaboration, when applicable1,2 |

### AI/ML based waveform for PAPR reduction

Observation

For 6GR AI/ML use cases identification/categorization, [3 sources] provided preliminary simulation results and analysis on AI/ML based waveform for PAPR reduction with detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis in Table J.

Note: whether/how to capture the observation in the TR is a separate discussion.

Table J AI/ML based waveform for PAPR reduction

| Use case | AI/ML based waveform for PAPR reduction |
| --- | --- |
| Reported companies | (3) vivo1, Samsung2, Huawei3 |
| Model input | Symbols in frequency domain |
| Model output | For model output of encoder for UE-sided/NW-part of two-sided model: transformed/precoded symbols in frequency domain For output of decoder for NW-part of two-sided model: 1. LLR1,3 2. Symbols in frequency domain2 |
| Label | Label free2,3 Known bit sequences or its LLR1,3 |
| Training types | offline training |
| KPI | BLER, CCDF of PAPR(UL), throughput (DL) |
| Benchmark | DFT-s-OFDM |
| Model location for inference | Two-sided model UE-sided model (for frequency domain shaping)1 |
| Collaboration/interaction between UE and NW | Similar to two-sided model in NR No collaboration for UE-sided model1 |
| Potential specification impact | 1. Signaling/ procedure related to LCM for two-sided model including inter-vendor collaboration, when applicable 2. Signaling/ procedure related to LCM for UE-sided model 1 3. Related RAN4 requirements |

### AI/ML based HARQ-ACK feedback

Observation

For 6GR AI/ML use cases identification/categorization, [2 sources] provided preliminary simulation results and analysis on AI/ML based HARQ-ACK feedback with detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis in Table K.

Note: whether/how to capture the observation in the TR is a separate discussion.

Table K. AI/ML based HARQ-ACK feedback

| Use case | AI/ML based HARQ-ACK feedback |
| --- | --- |
| Reported companies | (3) Qualcomm, vivo |
| Model input | HARQ ACK/NACK bit sequence |
| Model output | Learned sequences/modulated symbols |
| Label | HARQ-ACK/NACK bit sequence |
| Training types | Offline training |
| KPI | BLER |
| Benchmark | NR RM code for up to 11bits with Maximum Likelihood (ML) receiver |
| Model location for inference | No model for inference |
| Collaboration/interaction between UE and NW | No collaboration |
| Potential specification impact | 1.Learned sequence/modulated symbols design 2.Downloadable sequence/modulated symbols related signalling/ procedure for HARQ-ACK 3. Related RAN4 requirements |

### improved scheduling/HARQ for token traffic

Observation

For 6GR AI/ML related service, for 6GR AI/ML use cases identification/categorization, [2 sources] provided preliminary simulation results and analysis on improved scheduling/HARQ for token traffic

Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis in Table L.

Note: whether/how to capture the observation in the TR is a separate discussion.

Table L for improved scheduling/HARQ for token traffic

| Use case | Improved scheduling/HARQ for token traffic |
| --- | --- |
| Reported companies | (2) Huawei1, OPPO2 |
| Model input | Tokenizer model: • Input: Raw data (e.g., image/video/audio, etc.) De-tokenizer model: • Input: Tokens |
| Model output | Tokenizer model: • Output: Tokens (e.g., tokenized image/video/audio) De-tokenizer model: • Output: Inference results for downstream tasks/Raw data (e.g., image/video/audio, etc.) |
| Label | Training at OTT, transparent to RAN1,2 |
| Training types | Offline training at OTT, transparent to RAN |
| KPI | Supported number of UEs, achievable throughput |
| Benchmark | NR scheduling/HARQ mechanism without knowledge of Token traffic |
| Model location for inference | The tokenizer model is at UE or NW/OTT server (e.g., an encoder). The de-tokenizer model is at NW/OTT server or UE (e.g., a decoder). |
| Collaboration/interaction between UE and NW | NA |
| Potential specification impact | • Service awareness in RAN • Token error identification, new scheduling and HARQ |

### Other use cases

Observation

For 6GR AI/ML use cases identification/categorization,

[one source] provided preliminary simulation results and analysis on pathloss prediction in the spatial, temporal, and/or frequency domain, to use the predicted pathloss in UL (PUSCH/PUCCH/PRACH/SRS) power control.

[one source] provided preliminary simulation results and analysis on UL closed-loop power control with an NW-sided AI/ML model, where the model predicts the optimal power adjustment (or TPC command index) for the UE,

[one source] provided preliminary simulation results and analysis on prior-information-aided DCI decoding,

[one source] provided preliminary simulation results and analysis on lossless DCI compression,

[one source] provided preliminary simulation results and analysis on early contention resolution in RACH,

[one source] provided preliminary simulation results and analysis on sensing based RAN digital twin construction with NW-side AI/ML model,

[one source] provided preliminary simulation results and analysis on AI/ML-enabled RAN digital twin with distributed model,

[one source] provided preliminary simulation results and analysis on AI/ML based SRS power imbalance compensation,

[one source] provided preliminary simulation results and analysis on Site Specific Learning for AI/ML and RAN Digital Twin,

Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in in Table M

Note: whether/how to capture the observation in the TR is a separate discussion.

Table M -1

| Use Case | Prior-Information-Aided DCI Decoding | Lossless DCI Compression | UL closed-loop power control with an NW-sided AI/ML model, where the model predicts the optimal power adjustment (or TPC command index) for the UE. | Pathloss prediction in the spatial, temporal, and/or frequency domain, to use the predicted pathloss in UL(PUSCH/PUCCH/PRACH/SRS) power control. |
| --- | --- | --- | --- | --- |
| Reported companies | (1)CMCC | (1)CMCC | (1) Nokia | (1) Nokia |
| Model input | LLR after demodulation at current transmission, and historical AI/ML based predicted LLR | Historical DCI payload | UL SINR measurement, UE Tx power estimate (derived from Pcmax, P0, PL alpha, pathloss measurement), and PUSCH allocation size | L1-RSRP measurements from a sub-set/set of RSs/beams (Set B). input can consider history of measurements |
| Model output | Decoded DCI payloads, and predicted LLR for next transmission | Predicted DCI payload | Predicted TPC command index | Predicted pathloss value(s) (or predicted L1-RSRP(s)) for a set of RSs/beams (Set A). output can consider future instances |
| Label | DCI payload sequences | DCI payload sequences | Optimal TPC command index (offline learning) label-free (online learning) | Pathloss value(s) (or L1-RSRP(s)) for a set of RSs/beams (Set A) |
| Training types | Offline training at the UE side | Offline training at the NW side, and model delivery to UE side | Offline and Online learning | Offline training |
| KPI | BLER performance | BER and sample-level prediction accuracy; DCI overhead reduction | UL throughput. | Pathloss prediction accuracy, throughput, RS overhead reduction, Complexity. |
| Benchmark | Traditional DCI decoder | Traditional DCI design | 1. UL Power control with optimized OLPC parameters 2. UL Power control with optimized OLPC parameters and possibly legacy CLPC algorithms (with 5G TPC tables). | Pathloss estimation based on Set A Pathloss estimation based on Set B |
| Model location for inference | UE-sided model | UE-sided model + NW-sided model | NW-sided model | UE-sided model NW-sided model |
| Collaboration/interaction between UE and NW | Similar to UE-sided model in NR | Model transfer from NW to UE | None | As UE-sided or NW-sided mode in nRl |
| Potential specification impact | 1. Signalling/configuration design for prior-information-aided DCI decoder. 2. Signalling/ procedure related to LCM for UE-sided model | 1. Signalling/configuration design for Lossless DCI Compression. 2. Signalling/ procedure related to LCM including model transfer | Configurability of the values in TPC command tables or an extended TPC command table (compared to NR). | 1. Pathloss prediction related signalling/procedure 2. Signalling/ procedure related to LCM for UE-sided or NW-sided model 3. RAN4 performance requirements and test cases, including defining new requirements related to pathloss reference signal (PL-RS) measurement and activation delays of TCI state(s). |

Table M-2

| Sub-use case | Early contention resolution in RACH | Sensing based RAN digital twin construction with NW-side AI/ML model | AI/ML-enabled RAN digital twin with distributed model | AI/ML based SRS power imbalance compensation | Site Specific Learning for AI/ML and RAN Digital Twin |
| --- | --- | --- | --- | --- | --- |
| Reported Companies | (1) Ofinno | (1) Huawei | (1) Huawei | (1) Huawei | (1) DeepSig |
| Model input | Received PRACH signal (e.g., preamble waveform) | Point cloud sensed by the BS with mono-static sensing and sensed/reported by UEs with bi-static sensing | UE-part models: local sparse point cloud NW-part model: latent space information from multiple UEs | UL measured channel matrix from SRS with IL imbalance | Received signal/estimated channel at DMRS and received signal on data, and the channel information generated by digital twin |
| Model output | Predicted number of UEs that transmitted the same preamble for given PRACH resources | 3D point cloud representing the static environment | UE-part models: compressed latent space information NW-part model: global point cloud | DL channel matrix with IL compensated | Decoded bit |
| Label | Ground-truth number of UEs that transmitted the same preamble | Ground truth point cloud | Ground truth point cloud | UL SRS measurement without IL (assuming it is compensated by UE at certain conditions) or DL CSI-RS measurement | Ground truth of target bit |
| Training types assumption | Offline training | Offline training | Offline training (adopted in simulation) Online finetuning (can be optionally considered) | offline training | Offline training |
| KPI | Prediction accuracy of UE multiplicity, RACH access delay, first-attempt success probability | Sensing accuracy metric: root mean square error (RMSE) of point cloud. RMSE=1ni=1nxi-xi2 is the square root of the average of the squared errors between each sensed point (xi in forms of coordinates) and ground truth point (xi in forms of coordinates) in the point cloud including n points with {x, y, z} dimensions. | 1. Overhead metric: Feedback bits per point 2. Sensing accuracy metric: intersection-over-union (IoU), edge detection probability | SGCS | BLER, throughput |
| Benchmark | First-attempt success rate based on legacy PRACH receiver | BS side mono-static sensing only to construction RAN digital twin | 1. Single UE sensing (to justify sensing accuracy metric of using distributed model). 2. Raw data transmission (to justify overhead metric of using distributed model). | 1. SRS without IL imbalance; 2. non-AI based SRS IL imbalance compensation | Legacy receiver without the help of digital twin |
| Model location for inference | NW-side model | NW-side model | Distributed model: a NW-side model paired with multiple UE-side models. | NW-sided model | NW-sided model |
| Collaboration/interaction between UE and NW | Similar to NW-sided model in NR | Similar to NW-sided model in NR | Similar to two-sided model: UE reporting of compressed sensing results for inference. Inter-vendor training collaboration between NW side and UE side. | Similar to NW-sided model in NR | Similar to NW-sided model in NR |
| Potential spec impact | 1. Signaling/procedure related to Mgs.3 grant for more than one UEs selected the same PRACH sequence 2. Signalling/procedure related to LCM for NW-sided model | 1. Signaling/procedure related to bi-static sensing results reported from UE 2. Signalling/procedure related to LCM for NW-sided model | 1. Sensing results reported from UE in forms of compressed latent message 2. Signalling/procedure related to LCM for two-sided model including inter-vendor collaboration | 1. Inference: UE reporting on the IL range for ensuring generalization 2. Signalling/procedure related to LCM for NW-sided model | 1. Signalling/procedure related to LCM for NW-sided model |

## Contact information

| Company | Delegate(s) | Email address |
| --- | --- | --- |
| Moderator | Feifei Sun | Feifei.sun@samsung.com |
| Google | Yushu Zhang | yushuzhang@google.com |
| Ofinno | Jaehoon Chung | jchung@ofinno.com |
| Sharp | Yinan Zhao | Yinan.zhao@cn.sharp-world.com |
| Fainity | Chia-Hung Lin | chlin@fainnov.com |
| Lenovo | Bingchao Liu Vahid Pourahmadi Srinivas Kothapalli | liubc2@lenovo.com vpourahmadi@lenovo.com vkothapalli@lenovo.com |
| CATT | Yongqiang FEI | feiyongqiang@catt.cn |
| SK Telecom | Hyunho Lee | hho.lee@sk.com |
| CMCC | Xiaodong Shen Yi Zheng Yuhua Cao | shenxiaodong@chinamobile.com zhengyi@chinamobile.com caoyuhua@chinamobile.com |
| NVIDIA | Xingqin Lin | xingqinl@nvidia.com |
| Fujitsu | WANG Guotong (David) | wangguotong@fujitsu.com |
| Nokia | Keeth Jayasinghe | keeth.jayasinghe@nokia.com |
| ZTE | Xingguang, Wenfeng, Yunqi | wei.xingguang@zte.com.cn liu.wenfeng@zte.com.cn sun.yunqi@zte.com.cn |
| Huawei, HiSilicon | Yuan Li | liyuan3@huawei.com |
| Ericsson | Yufei Blankenship Jingya Li Siva Muruganathan | yufei.blankenship@ericsson.com jingya.li@ericsson.com siva.muruganathan@ericsson.com |
| NEC | Peng Guan Pravjyot Deogun Yi Jiang | Guan_peng@nec.cn pravjyot.deogun@EMEA.NEC.COM y-jiang_ct@nec.com |
| Panasonic | Henry Tran Tetsuya Yamamoto Hidetoshi Suzuki | xuantuong.tran@sg.panasonic.com yamamoto.tetsuya001@jp.panasonic.com suzuki.hidetoshi@jp.panasonic.com |
| NTT DOCOMO | Kosuke Shima Wang Xin Zhang Zhibo | kousuke.shima.nr@nttdocomo.com wangx@docomolabs-beijing.com.cn zhangzb@docomolabs-beijing.com.cn |
| Xiaomi | Qin MU | muqin@xiaomi.com |
| Qualcomm | Hamed Pezeshki Taesang Yoo Yi Huang | hamedp@qti.qualcomm.com taesangy@qti.qualcomm.com yihuang@qti.qualcomm.com |
| OPPO | Jeffrey Cao Wendong Liu | caojianfei@oppo.com liuwendong1@oppo.com |
| ETRI | Youngjoon Yoon Minhyun Kim | youngjoon.yoon@etri.re.kr minhyun.kim@etri.re.kr |
| Spreadtrum | Shijia shao Zhe yu Mimi chen | Shijia.shao@unisoc.com Zhe.yu@unisoc.com Mimi.chen@unisoc.com |
| CEWiT | Dhivagar Baskaran Shiv Shankar | dhivagar.b@cewit.org.in shivshankar@cewit.org.in |
| TCL | Pu Yuan Tianqi Wu | pu.yuan@tcl.com tianqi1.wu@tcl.com |
| IITK | Chethan Ranganatha | chethanr@iitk.ac.in |
| Tejas Networks | Pavan Kalyan D | pavankalyand@tejasnetworks.com |
| IIT Madras | Anil Kumar Yerrapragada Jeeva Keshav S | anilkumar@5gtbiitm.in jeevak@5gtbiitm.in |
| LGE | Haewook Park | haewook.park@lge.com |
| InterDigital | Fumihiro Hasegawa | fumihiro.hasegawa@interdigital.com |
| AT&T | Salam Akoum | Salam.akoum@att.com |
| Rakuten | Awn Muhammad Abhinav Kishore Mattewada | awn.muhammad@rakuten.com abhinavkis.mattewada@rakuten.com |
| Samsung | Ameha Abebe Zhe Chen Qi Xiong | amehat.abebe@samsung.com tom.chenzhe@samsung.com q1005.xiong@samsung.com |
| Orange | Raphael Visoz | raphael.visoz@orange.com |
| Vivo | Peng Sun | sunpeng@vivo.com |
| MediaTek | Reubengeorge Stephen | Reubengeorge.Stephen@mediatek.com |
| Deepsig | James Lansford | jameslansford@deepsig.io |
| Lekha | Ipshita Panda | ipshita.panda@Lekhawireless.com Sashiganth M sashiganth.m@Lekhawireless.com Sreenath Ramanath sreenath@lekhawireless.com |
| Apple | Huaning Niu Weidong Yang | huaning_niu@apple.com wyang23@apple.com |
| Kyocera | Omar Sleem | omar.sleem@kyocera.com |

# Reference

- R1-2508324	Discussion on AI/ML in 6GR interface	FUTUREWEI

- R1-2508341	Views on AI/ML Operation and Use Cases for 6G Radio Air Interface	Nokia

- R1-2508351	Discussion on AI/ML-driven use cases for 6GR	BJTU

- R1-2508365	AI/ML Use Cases for 6GR Air Interface	Ericsson

- R1-2508366	AI/ML in 6GR interface	Kyocera

- R1-2508392	Discussion on AIML in 6GR interface	Spreadtrum, UNISOC

- R1-2508437	Discussion on AI/ML in 6GR interface	vivo

- R1-2508460	Discussion on AI/ML in 6GR interface	CMCC

- R1-2508515	AI/ML for 6G Air Interface	InterDigital, Inc.

- R1-2508539	Discussion on AI/ML in 6GR air interface	TCL

- R1-2508549	Discussion on AIML in 6GR interface	NEC

- R1-2508582	AI/ML in 6GR interface	CATT, CICTCI

- R1-2508643	AI/ML use cases and framework for 6GR Air Interface	AT&T

- R1-2508689	Discussion on AI/ML in 6GR interface	Xiaomi

- R1-2508697	Discussion on AI-based Smart Radio for 6G Air Interface	ZTE Corporation, Sanechips

- R1-2508732	AIML use cases for 6GR air interface	OPPO

- R1-2508740	Views on AI/ML in 6GR air interface	Huawei, HiSilicon

- R1-2508753	Discussion on AI/ML in 6GR interface	LG Electronics

- R1-2509516 (R1-2508807)	AI/ML Use cases and framework for 6GR	Samsung

- R1-2508827	AI/ML Use Cases for 6G RAN	Tejas Network Limited

- R1-2508916	Discussion on AI/ML in 6GR interface	Lekha Wireless Solutions

- R1-2508933	Discussion on AI/ML in 6GR	Fujitsu

- R1-2508947	AI/ML in 6GR Air Interface	Google

- R1-2508978	Discussion on AI/ML in 6GR interface	ETRI

- R1-2508999	Discussion on AI/ML in 6GR interface	Panasonic

- R1-2509001	Views on improved scheduling/HARQ for token traffic	CAICT

- R1-2509004	AI/ML in 6GR	Lenovo

- R1-2509008	AIML in 6GR air interface	KAIST

- R1-2509021	AI and ML in 6GR air interface	NVIDIA

- R1-2509022	Discussion on AI/ML in 6GR	IMU

- R1-2509039	Views on AI/ML in 6GR interface	Ofinno

- R1-2509046	Discussion on AI ML in 6GR air interface	Hanbat National University

- R1-2509050	Discussion on AI/ML for 6GR interface	Ruijie Networks Co. Ltd

- R1-2509078	Use cases for AI/ML in 6GR Interface	Sony

- R1-2509115	On AI/ML for 6G air interface	Apple

- R1-2509116	Discussion on AI/ML in 6GR interface	AUMOVIO

- R1-2509148	AI/ML in 6GR Air Interface	MediaTek Inc.

- R1-2509184	Discussions on AI/ML in 6GR interface	Sharp

- R1-2509236	AI/ML in 6GR air interface	Qualcomm Incorporated

- R1-2509251	Discussion on AI/ML in 6GR interface	Transsion Holdings

- R1-2509287	Discussion on AI/ML for 6GR air interface	NTT DOCOMO, INC.

- R1-2509302	Use cases for AI/ML in 6GR interface	KT Corp.

- R1-2509354	AI/ML in 6GR Interface	CEWiT

- R1-2509373	Discussion on AI/ML in 6GR Interface	Indian Institute of Tech (M)

- R1-2509400	On new use cases for AI/ML in 6GR interface	Vodafone, Deutsche Telekom, Nokia

- R1-2509411	Discussion on AI/ML-enabled use cases for 6GR	BUPT, ZGC Institute of Ubiquitous-X Innovation and Application, Pengcheng Laboratory

- R1-2509417	New use cases for AI/ML in 6GR interface	Pengcheng Laboratory, ZGC Institute of Ubiquitous-X Innovation and Application

- R1-2509433	Discussion on AI/ML use cases for 6GR air interface	IIT Kanpur

- Reference for RAN 1#122bis

- R1-2506742	Discussion on AI/ML in 6GR interface	FUTUREWEI

- R1-2506757	Views on AI/ML Operation and Use Cases for 6G Radio Air Interface	Nokia

- R1-2506762	AI/ML Use Cases for 6GR Air Interface	Ericsson

- R1-2506778	Discussion on AI-based Smart Radio for 6G Air Interface	ZTE Corporation, Sanechips

- R1-2506780	Discussion on AI/ML-driven use cases for 6GR	BJTU

- R1-2506784	Discussion on AI/ML in 6GR air interface	TCL

- R1-2506820	Discussion on AIML in 6GR interface	Spreadtrum, UNISOC

- R1-2506821	AI/ML for 6G Air Interface	InterDigital, Inc.

- R1-2506904	Discussion on AI/ML in 6GR interface	vivo

- R1-2506951	AI/ML in 6GR interface	Kyocera

- R1-2506995	Discussion on AI/ML in 6GR interface	Xiaomi

- R1-2507020	Discussion on AI/ML in 6GR interface	CMCC

- R1-2507029	Discussion on AI/ML in 6GR interface	Lekha Wireless Solutions

- R1-2507064	Views on AI/ML in 6GR air interface	Huawei, HiSilicon

- R1-2507107	Views on AI/ML in 6GR interface	CATT, CICTCI

- R1-2507182	AIML use cases for 6GR air interface	OPPO

- R1-2507203	AI/ML in 6GR interface	Tejas Network Limited

- R1-2507259	AI/ML Use cases and framework for 6GR	Samsung

- R1-2507288	Discussion on AI/ML in 6GR	Fujitsu

- R1-2507305	Discussion on AIML in 6GR interface	NEC

- R1-2507377	Discussion on AI/ML in 6GR interface	Panasonic

- R1-2507378	AI/ML Use Cases for 6G	NTU

- R1-2507400	Discussion on AI/ML in 6GR interface	LG Electronics

- R1-2507409	Views on AI/ML in 6GR air interface	SK Telecom

- R1-2507433	AI and ML in 6GR air interface	NVIDIA

- R1-2507471	Views on AI/ML in 6GR interface	Ofinno

- R1-2507488	Discussion on AI/ML Use-cases in 6GR	Lenovo

- R1-2507492	Use cases for AI/ML in 6GR interface	KT Corp.

- R1-2507512	Discussion on AI/ML in 6GR interface	ETRI

- R1-2507522	AI/ML in 6GR Air Interface	Google

- R1-2507525	New use cases for AI/ML in 6GR interface	Pengcheng Laboratory, ZGC Institute of Ubiquitous-X Innovation and Application

- R1-2507547	Discussion on AI/ML in 6GR interface	Continental Automotive

- R1-2507601	Discussion on the potential AI/ML use cases for 6GR interface	Sony

- R1-2507604	Discussion on AI/ML for 6GR interface	Ruijie Networks Co. Ltd

- R1-2507605	Discussion on AI/ML Use Cases for 6GR	DeepSig Inc

- R1-2507613	AI/ML Framework and Use Cases for 6GR Air Interface	MediaTek Inc.

- R1-2507633	Discussion on AI/ML in 6GR interface	Transsion Holdings

- R1-2507683	Discussion on 6G AI/ML use cases	Apple

- R1-2507727	AI/ML in 6GR air interface	Qualcomm Incorporated

- R1-2507752	AI/ML use cases and framework for 6GR Air Interface	AT&T

- R1-2507776	Discussions on AI/ML in 6GR interface	Sharp

- R1-2507779	Discussion on AI/ML in 6GR	Shanghai Jiao Tong University, Toyota

- R1-2507821	Discussion on AI/ML for 6GR air interface	NTT DOCOMO, INC.

- R1-2507826	AI/ML in 6GR air interface	KAIST

- R1-2507827	Discussion on AI/ML-enabled use cases for 6GR	BUPT, ZGC Institute of Ubiquitous-X Innovation and Application

- R1-2507842	Discussion on AI/ML in 6GR air interface	Hanbat National University

- R1-2507872	Discussion on AI/ML in 6GR Interface	Indian Institute of Tech (M)

- R1-2507883	On new use cases for AI/ML in 6GR interface	Vodafone, Deutsche Telekom

- R1-2507907	AI/ML in 6GR Interface	CEWiT

- R1-2507910	Discussion on AI/ML in 6GR -Physical Layer	Rakuten Mobile, Inc

- R1-2507940	Discussion on AI/ML in 6GR air interface	IIT Kanpur