3GPP TSG RAN WG1 #123	    								R1-250xxxx

Dallas, USA, 17th-21st November 2025

Agenda item:	11.6

Source: 	Samsung (Moderator)

Title:		Updates on observation for 6GR AI/ML use cases identification/categorization

Document for: 	Decision

# Introduction

In RAN 1 #122bis, some observation for use cases identification/categorization, this Tdoc is to further update the use cases based on the input from RAN 1 #123.

Please provide updated based on the submitted Tdoc in RAN 1 #122, #122bis, #123.

# Updated observations

## Low overhead CSI-RS or CSI prediction with AI/ML

Observation 2.1

For 6GR AI/M use cases identification/categorization, [24 sources] provided preliminary simulation results and analysis on low overhead CSI-RS or CSI prediction with AI/ML.

- [24 sources] provided preliminary simulation results and analysis on frequency and/or spatial domain CSI prediction with sparse/low overhead CSI-RS with AI/ML. Detailed evaluation assumptions (model input/output/label/benchmark/KPI/ training type) and initial analysis can be found in Table A.

- [7 sources] provided preliminary simulation results (or by citing to NR study for CSI time domain prediction) and analysis on CSI time domain prediction with AI/ML wherein [3 sources] assumed Rel-19 CSI prediction while [3 sources] assumed differently. Detailed evaluation assumptions (model input/output/label/benchmark/KPI training type) and initial analysis can be found in Table B.

- [4 sources] provided preliminary simulation results and analysis on CSI prediction cross carrier/band/frequency block with AI/ML. Detailed evaluation assumptions (model input/output/label/benchmark/KPI/training type) and initial analysis can be found in Table B.

- [3 sources] provided preliminary simulation results and analysis on CSI prediction across analog beams with AI/ML. Detailed evaluation assumptions (model input/output/label/benchmark/KPI training type) and initial analysis can be found in Table B.

- [1 source] provided preliminary simulation results and analysis on CSI prediction with linear projection as pre-processing. Detailed evaluation assumptions (model input/output/label/benchmark/KPI training type) and initial analysis can be found in Table B.

Note: whether/how to capture the observation in the TR is a separate discussion.

Table A low overhead CSI-RS or CSI prediction with AI/ML

| Sub-use case | Sub-Case A: Frequency and/or spatial domain CSI prediction with sparse/low overhead CSI-RS with AI/ML |
| --- | --- |
| Reported companies | (24) Ericsson1, ZTE2, vivo3, OPPO, Xiaomi, CMCC, Huawei4, Samsung, Fujitsu, Apple, Qualcomm5, Kyocera6, Nokia7, {Spreadtrum, UNISOC}8, Interdigital9, Lenovo, E10, DoCoMo11, CEWiT, IITM, IIT Kanpur, Tejas, {CATT, CICTCI}12, MediaTek |
| Model input (for decoder of 2-sided model, when applicable) | 1. Measurement of channel with sparse/low overhead CSI-RS (majority) 1a. Additional long-term multi-path power/angle/delay info information as assistance information4 1b. Received SRS info as assistance information for NW-sided model6 2. Reported CSI for NW-sided model3,4,5 2a. For spatial domain CSI prediction cross on-off pattern, the reported CSI can be associated with full port channel3 |
| Model output (for decoder of 2-sided model, when applicable) | 1. Full channel matrix (majority) 2. Eigenvector 3 for NW-sided model 3. Channel matrix/eigenvector with different/targeted antenna on/off patterns3, 12 |
| Label | 1. Estimated/ideal channel matrix based on full CSI-RS density(majority) 2. Ideal precoding matrix with full dimension3 3. Estimated/ideal channel matrix/eigenvector with different/targeted antenna on/off patterns3, 12 |
| Training types | Offline training(majority) Online finetuning for UE-sided model (for NW-sided model + UE sided model without training collaboration)4 |
| KPI | NMSE, SGCS, throughput, ratio of CSI-RS overhead, NES gain, CSI feedback overhead reduction3 |
| Benchmark | 1. non-AI based on full CSI-RS 2. non-AI based on sparse CSI-RS |
| Model location for inference | UE-sided model NW-sided model2,3, 4,5,6,10 Two-sided model2, 3 NW-sided model + UE-sided model without training collaboration4 |
| Collaboration/interaction between UE and NW | As UE-sided model in NR As NW-sided model in NR As two-sided model for CSI compression4 in NR |
| Potential spec impact | 1.Sparse CSI-RS design/configuration and corresponding feedback (especially for NW-sided model) 2. Signalling/ procedure related to LCM 3. Inter-vendor collaboration for two-sided model, when applicable |

Table B low overhead CSI-RS or CSI prediction with AI/ML (cont.)

| Sub-use case | Sub-Case B: CSI time domain prediction (as Rel-19 CSI prediction or extension) | Sub-case C: CSI prediction cross carrier/band/frequency band | Sub-Case D: CSI prediction across analog beams, or across different antenna to port virtualization | Sub-Case E: prediction with linear projection as pre-processing |
| --- | --- | --- | --- | --- |
| Reported Companies | (7) Ericsson2, BJTU, Samsung, MediaTek3, LGE, vivo1, Lenovo | (4) Samsung, Apple, LGE, DoCoMo1 | (3) Samsung, vivo1, Apple2 | (1) Huawei |
| Model input | 1. Channel matrix over K CSI-RS occasions 2. Measurements of interference over K CSI-RS occasions1 3. Channel matrix over K CSI-RS occasions with >20ms periodicity3 4 Channel matrix with one P CSI-RS with 20ms periodicity and K-1 AP CSI-RS2 | Channel matrix of carrier/band/frequency block A | 1.Channel matrix/precoding matrix of Set B of beams, 2. Channel matrix of antenna to port virtualization pattern 12 | K past CSI information after linear projecting |
| Model output | 1. Channel matrix of future instances 2. Interference in future instances1 | Channel matrix of carrier/band/frequency block B | 1.Channel matrix/precoding matrix of Set A of beams 2. Channel matrix of antenna to port virtualization pattern 22 | Predicted CSI information after linear projecting at a future time instance |
| Label | Measurement(s) of future time occasion(s). | Channel matrix of carrier/band/frequency block B | 1.Channel matrix of Set A of beams, 2. Channel matrix of antenna to port virtualization pattern 2 2 | Ground-truth CSI information after linear projecting, based on the measurement at the future time instance |
| Training types assumption | offline training | offline training | offline training | Online finetuning |
| KPI | NMSE, SGCS, throughput, [ratio of CSI-RS overhead] | SGCS, NMSE, throughput, ratio of CSI-RS overhead, SGCS ratio when e-type II codebook is used for PMI feedback, NES gain. | SGCS, NMSE, throughput, ratio of CSI-RS overhead, NES gain. | SGCS |
| Benchmark | - Sample and hold 2. Ground truth of future time occasions | 1.Ground truth of target frequency block 2. Sample and hold | Ground truth of Set A of beams | 1.Non-AI based CSI prediction 2.AI-based CSI prediction based on CSI information without linear projection |
| Model location for inference | UE-sided model NW-sided model1 | UE-sided model NW-sided model1 | UE-sided model NW-sided model1 Two-sided model1 | UE-sided model |
| Collaboration/interaction between UE and NW | As UE-sided model in NR As NW-sided model in NR1 | As UE-sided model in NR | As UE-sided model in NR As two-sided model in NR1 | Similar to UE-sided model in NR |
| Potential spec impact | 1. As AI based CSI prediction in NR 2. Reporting content, signalling and procedure for LCM for extension cases1 | 1. Cross carrier/frequency switching procedure enhancement based on predicted CSI 2. signalling/ procedure related to LCM | 1.CSI-RS configuration for predicted beams or different antenna to port virtualization 2. signalling/ procedure related to LCM | Signaling/ procedure related to LCM considering online finetuning |

## Low overhead DMRS with AI/ML receiver

Observation 2.2

For 6GR AI/ML use cases identification/categorization, [23 sources] provided preliminary simulation results and analysis on low overhead DMRS with AI/ML receiver.

- [25 sources] provided preliminary simulation results and analysis on sparse orthogonal DMRS in frequency and/or time domain with AI/ML receiver.

- [12 sources] provided preliminary simulation results and analysis on superimposed pilot with AI/ML receiver.

- [9 sources] provided preliminary simulation results and analysis on DMRS free with AI/ML receiver.

- Detailed evaluation assumptions (model input/output/label/benchmark/KPI/ training type) and initial analysis can be found in Table C.

Note: whether/how to capture the observation in the TR is a separate discussion.

Table C For low overhead DMRS with AI/ML receiver

| Sub-use case | Sub-case A: Sparse orthogonal DMRS in frequency and/or time domain | Sub-case B: Superimposed pilot | Sub-case C: DMRS free |
| --- | --- | --- | --- |
| Reported companies | (25) Nokia1, Futurewei2, Ericsson3, ZTE4, {Spreadtrum, UNISOC}5, Interdigial6, vivo7, xiaomi8, CMCC9, {CATT, CICTCI}10, Fujitsu11, Apple12, Samsung13, Kyocera14, Lenovo15, Huawei16, Qualcomm 17, Ofinno18, NVIDIA19, MediaTek20, Lekha21, LGE22, DocoMo23, BJTU24, IIT Kanpur25 | (12) vivo1, CMCC2, ZTE3, Lenovo4, Huawei5, OPPO6, NVIDIA7, LGE8，Xiaomi9 , InterDigital10 , DocoMo11 Kyocera12 | (9) BJTU1, Huawei2, NVIDIA3, MediaTek4, Lenovo5, SKT6 , Nokia7, DocoMo8, NTT9 |
| Model input | 1. Received signal/estimated channel at DMRS and/or data REs and received signal on data 1,2,6,13, 22,15,3,17,10,4, 20,6,18,23,19,8, 24 1a. additionally noise variance 1,13,24 2. Received signal/estimated channel at DMRS2,7, 8,11,12,13,16,5,23,25 | 1. Received signal and DMRS sequence (superimposed signal) (Majority) 2. Estimated channel (in delay and/or doppler domain) from the received signal (superimposed signal) 1 For Tx side of two-sided model: modulated symbols and DMRS symbol5 | Received signal3, 4,6-9 For Tx side of two-sided model: coded bit2,5,1 |
| Model output | 1. Estimated channel at target data and/or DMRS REs2,4,5,7,8,9,11, 12, 13,16,17,18,19,21,22,23,25 1a. Estimated noise variance 12 2. LLRs1,2,3, 4, 6,10, 13,14,15,19,20, 22,24 3. Filtering coefficients for channel estimation 7 4. Estimated modulation symbols8 5. Equalizer coefficients6 | 1. Estimated channel at target data REs1,3,4,5,6,8, 10 2.LLR2,3, 5,6,7,8,9,11,12 3.Estimated modulation symbols9 For Tx side of two-sided model: superimposed signal5 | 1.LLR (majority) 2.Estimated channel2 For Tx side of two-sided model: modulated data symbols 5,2,1 |
| Label | 1. Ideal channel information 2, 4, 5,7,8,9,11,12,13,15,16,17,18,22,23,25 2. Known sequence/data1,2,3, 4,8, 10, 13,14,15,16,20,22,19, 24 3. Label free (unsupervised)6, 21 4. Estimated channel using legacy/new DMRS pattern with legacy receiver4,8,13 5. Estimated channel of adjacent RE (self-supervised)13 | 1. Known sequence/data 2,3,11,12,7,9 2. Ideal channel information1,8, 10 3.Transmitted modulation symbols9 4. Estimated channel information using orthogonal DMRS4 | 1. Known sequence/data1,3,1 2 Ideal channel information2 |
| Training types assumption | offline training | offline training | offline training |
| KPI | MSE, BLER, throughput, raw BER | MSE, BLER, throughput | MSE, BLER, throughput |
| Benchmark | With ideal channel information With conventional receiver with sparse or legacy DMRS | With ideal channel informal With conventional receiver with legacy DMRS overhead | With ideal channel information With conventional receiver with legacy DMRS overhead |
| Model location for inference | UE-sided model for DL or NW-sided model for UL | UE-sided model for DL NW-sided model for UL Two-sided model5 | NW-sided model for UL3,4,6-9 Two-sided model2,5,1 |
| Collaboration/interaction between UE and NW | Similar to UE-sided or NW-sided model as NR | Similar to UE-sided or NW-sided model as NR Similar to two-sided model as NR | Similar to UE-sided model as NR Similar to NW-sided model as NR Similar to two-sided model as NR |
| Potential spec impact | 1. DMRS design 2. RAN 4: Demod requirement 3. Signalling/ procedure related to LCM for UE and/or NW sided model Etc. | 1. DMRS design (Including power allocation between DMRS and data) 2. RAN 4: Demod requirement 3. Signalling/ procedure related to LCM for UE and/or NW sided model or two-sided model (including inter-vendor calibration), when applicable Etc. | 1. RAN 4: Demod requirement 2. Signalling/ procedure related to LCM for UE and/or NW sided model or two-sided model (including inter-vendor calibration), when applicable 3. Constellation design and related signalling/procedure Etc. |

## CSI compression and feedback with AI

Observation 2.3

For 6GR AI/ML use cases identification/categorization, [13 sources] provided preliminary simulation results and analysis on CSI compression and feedback with AI.

- [10 sources] provided preliminary simulation results and analysis on CSI compression with joint source and channel coding (JSCC)

- [12 sources] provided preliminary simulation results and analysis on CSI compression with joint source, channel coding and modulation (JSCM)

- [2 sources] provided preliminary simulation results and analysis on CSI feedback with downloadable basis/codebook.

- [4 sources] provided preliminary simulation results (or cite to NR AI/ML for CSI compression simulation results) and analysis on CSI reconstruction with CSI feedback with SRS (assuming separate source and channel coding).

  - Besides, [1 source] citing to NR study

- Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in in Table D.

Note: whether/how to capture the observation in the TR is a separate discussion.

Table D CSI compression and feedback with AI

| Sub-use case | Sub-case A: CSI compression with JSCC | Sub-case B: CSI compression with JSCM | Sub-case C: DLable basis/codebook | Sub-case D: CSI reconstruction with CSI feedback with SRS (assuming SSCC) |
| --- | --- | --- | --- | --- |
| Reported companies | (10), Samsung2, vivo3, {Pengcheng, ZGC}, Lenovo, OPPO, MediaTek1, Fujitsu, BJTU, {BUPT, ZGC}, CMCC | (12) BJTU1, Samsung2, OPPO3, {Pengcheng, ZGC}4, vivo5, CMCC, ZTE, {BUPT, ZGC}, Fujitsu, Apple6, Lenovo, MediaTek7 | (2) ZTE1, Samsung | (4) Qualcomm1, vivo2, Samsung3, ZTE4 (1)LGE5 (citing to NR study) |
| Model input of decoder or model output of encoder, when applicable | 1. Compressed CSI bits (all) 1a. additionally estimated channel based on SRS2,3 | UE-part model output/NW-part input: 1. Compressed CSI complex values for UE-part model(all) NW-sided input: 1. Compressed CSI complex values via a projection matrix 1,2,3,,7 | 1.Amplitudes and phases obtained by a look up table based on feedback CSI bits 1a. Selected basis1 | 1. Compressed CSI bits 1a CSI measurement and channel matrix feedback for subset of CSI-RS ports3 2. Estimated channel based on SRS |
| Model output of decoder or model input of encoder, when applicable | 1. (Reconstructed) Eigenvectors (all) 2. (Reconstructed) Explicit H1,2,3 | NW-part/side output or UE-part input: 1. (Reconstructed) Eigenvectors(all) 1a: SNR as additional input to encoder 6 2. (Reconstructed) Explicit H2 UE-part input: 3. Received signal at sparse CSI-RS and CSI-RS sequence 1,4 | Reconstructed Eigenvectors | (Reconstructed) Eigenvectors Explicit H3,4,5 |
| Label | 1.Eigenvectors(all) 2.Explicit H1,2,3 | 1.Eigenvectors(all) 2.Explicit H2 | Eigenvectors | 1. Eigenvectors1 2. Explicit H2,3,4,5 |
| Training types | Offline training | Offline training | Offline training | Offline training |
| KPI | SGCS, NMSE, SE, UE complexity | SGCS, NMSE, SE, UE complexity | SGCS, UPT vs overhead | SGCS, NMSE, UPT |
| Benchmark | eType II NR separate source and channel coding NR AI/ML CSI compression with separate source and channel coding and link adaptation1 | eType II NR separate source and channel coding JSCM with two-sided model1,2,3 NR AI/ML CSI compression with separate source and channel coding and link adaptation7 | eType II | eType II NR AI/ML CSI compression without SRS SRS without CSI feedback |
| Model location for inference | Two-sided model | Two-sided model NW-sided model1,2,3,7 | NW-sided model | Two-sided model |
| Collaboration/interaction between UE and NW | Similar to two-sided model in NR | Similar to two-sided model in NR For NW-sided model: no collaboration or Similar to NW-sided model in NR | No collaboration or Similar to NW-sided model in NR | Similar to two-sided model in NR |
| Potential specification impact | 1. Necessary signalling/ procedure to support JSCC 2. Signalling/ procedure related to LCM for two-sided model including inter-vendor collaboration | 1. Necessary signalling/ procedure to support JSCM 2. Projection matrix design for NW-sided model, when applicable 3. Signalling/ procedure related to LCM with NW-sided model or two-sided model including inter-vendor collaboration, when applicable 4. RAN4 requirements, e.g., EVM | 1. Downloadable basis/codebook related signalling/ procedure 2. Signalling/ procedure related to LCM with NW-sided model | 1. Necessary signaling/procedure to support lower overhead and/or simpler CSI feedback 2. Signalling/ procedure related to LCM for two-sided model including inter-vendor collaboration |

## AI/ML for beam management and extension

Observation 2.4

For 6GR AI/ML use cases identification/categorization, [13 sources] provided preliminary simulation results and analysis on AI/ML for beam management and extension.

- [7 sources] provided preliminary simulation for DL Tx beam management and analysis on inter-cell/inter-TRP/M-TRP DL Tx beam prediction and management.

  - Besides, [6 sources] citing to NR study for DL Tx beam management and analysis on inter-cell/inter-TRP/M-TRP DL Tx beam prediction and management.

- [6 sources] provided preliminary simulation results and analysis on cross frequency DL Tx beam/beam pair prediction.

- [2 sources] provided preliminary simulation results and analysis on Tx-Rx beam pair prediction.

- [3 sources] provided preliminary simulation results for beam management and analysis on beam prediction for initial access.

  - Besides, [8 sources] citing to NR study for beam management and analysis on beam prediction for initial access.

- [1 source] provided preliminary simulation results and analysis on DL Tx beam prediction for spatial and/or temporal domain with additional local UE information.

- [1 source] provided preliminary simulation results and analysis on reinforcement learning-based approach beam selection

- [2 sources] provided preliminary simulation results and analysis on weak DL Tx beam prediction for MU-MIMO operation

- Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in in Table E.

Note: whether/how to capture the observation in the TR is a separate discussion.

Table E AI/ML for beam management and extension

| Sub-use case | Sub-case A: Inter-Cell/M-TRP DL Tx beam prediction and management | Sub-Case B: Cross frequency DL Tx beam / beam pair prediction | Sub-Case C: Tx-Rx beam pair prediction | Sub-Case D: Beam prediction for initial access | Sub-Case E: DL Tx beam prediction for spatial and/or temporal domain with additional local UE information | Sub-Case F: reinforcement learning-based approach beam selection | Sub-Case G: Weak DL Tx beam prediction for MU-MIMO operation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reported companies | (7) Nokia, ZTE, xiaomi, CEWiT, DoCoMo, , Lenovo, BJTU (6) Qualcomm, Samsung, LGE, NVIDIA, CEWiT, {Spreadtrum, UNISOC} (citing to NR study) | (6) Futurewei1, xiaomi2, Apple3, Google, LGE, Qualcomm | (2) Ericsson, Nokia | (3) Huawei, vivo, ZTE (8) Qualcomm, Samsung, LGE, Apple Google {Spreadtrum, UNISOC}, Nokia, Lenovo (citing to NR study), | (1) Huawei | (1) Nokia | (2) Google, Panasonic |
| Model input | Measurements from Set B of one or more TRPs/Cells | Measurements in frequency A | Measurements from Set B DL Tx-Rx beam pairs. | Measurements from Set B of SSB | Measurements from Set B And additional local UE information (moving direction and speed) as UE side model input | Measurements from a set of DL Tx beam scheduling stats (at the NW), Cross correlation among DL Tx beams | L1-RSRP for Set B beams |
| Model output | Predicted best beam information and/or predicted measurements from Set A of target cell/TRP(s) [of current or future time instance] | Predicted cell/beam/beam pair related information of frequency B [of current or future time instance] | Predicted best DL Tx-Rx beam pairs information from Set A DL Tx-Rx pairs. | Predicted best DL Tx beam information (and/or predicted measurements from Set A [of current or future time instance] | Predicted Best beam indexes (probability of each Tx beam in Set A to be the Top-1 Tx beam) and/or Predicted measurements from Set A [of current or future time instance] | Selected beam index for scheduling UE(s) | Predicted DL Tx beam index(es) from Set A beams with lowest RSRPs |
| Label | Measurements [or Top beams] of Set A of target cell/TRP(s) | Measurements on cell(s)/beam(s)/beam pair(s) of frequency B | Measurements [or Top beams pairs] of Set A Tx-Rx pair | Measurements [or Top beams] of Set A | Measurements [or Top beams] of Set A | label-free (online learning) | Measurements from Set A beams |
| Training types assumption | offline training | offline training | offline training | offline training | offline training; online finetuning (for UE side model) | Online learning | Offline training |
| KPI | Prediction cell/beam/measurement accuracy, Throughput, RS overhead reduction | Prediction beam/beam pair/measurement accuracy, RS overhead reduction | Prediction beam/measurement accuracy, RS overhead reduction, throughput, L1-RSRP difference | Prediction beam accuracy, 95th‑percentile Top‑1 RSRP error | Prediction beam/measurement accuracy | Throughput, End to end packet latency | Beam prediction accuracy |
| Benchmark | Based on Set A Based on Set B | Measurements on cell(s)/beam(s)/pair (pairs) of frequency B | Based on Set A Based on Set B Rel-19 DL Tx beam prediction | Based on Set A Based on Set B | NR beam prediction with AI/ML | Beam with largest RSRP (from the set) consider as the scheduling beam | based on Set A/B beams |
| Model location for inference | UE-sided model or NW-sided model | UE-sided model or NW-sided model | UE-sided model | UE-sided model or NW-sided model | NW-sided model + UE-sided model without training collaboration | NW-sided model | UE-side model or NW-sided model |
| Collaboration/interaction between UE and NW | As UE-sided or NW-sided model in NR | As UE-sided or NW-sided model in NR | As UE-sided model in NR | Similar to UE-sided or NW-sided model in NR | As UE-sided or NW-sided model in NR | No collaboration | Similar to UE-sided or NW-sided model in NR |
| Potential spec impact | 1. Inter-Cell/M-TRP beam prediction related singling/procedure 2. Signalling/ procedure related to LCM for NW-sided model or UE-sided model | 1. Cross frequency DL Tx beam/Tx-Rx beam pair prediction related signalling /procedure 2. Signalling/ procedure related to LCM for NW-sided model or UE-sided model | 1.Signalling/ procedure related to LCM for UE-sided model | 1. Initial access/Random access related to beam prediction 2. Signalling/ procedure related to LCM for NW-sided model or UE-sided model | 1. As NR AI for BM; 2. Signalling/ procedure related to NW-sided model + UE-sided model. 3. Signalling/ procedure related to online finetuning, if any | 1. Signalling/ procedure related to exploration phase (to mitigate the impact of exploration). | 1. UE reports the recommended beams 2. Signalling/ procedure related to LCM for NW-sided model or UE-sided model |

## AI/ML for (de)modulation

Observation 2.5

For 6GR AI/ML use cases identification/categorization, [6 sources] provided preliminary simulation results and analysis on AI/ML for (de)modulation.

- [6 sources] provided preliminary simulation results and analysis on modulation constellation design with the help of AI, and with non-AI or AI receiver.

- [3 sources] provided preliminary simulation results and analysis on AI-based modulation and precoding with two-sided model.

- Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in Table F.

Note: whether/how to capture the observation in the TR is a separate discussion.

Table F AI/ML for (de)modulation

| Sub-use case | Sub-use case A: AI-based (de)modulation | Sub-use case B: AI-based modulation and precoding |
| --- | --- | --- |
| Reported companies | (6)ZTE1, OPPO2, vivo3, Lenovo4, Xiaomi5, Futurewei6 | (3) ZTE, OPPO, Lenovo |
| Model input | For constellation design 1. Coded bits 1,2,3,4,5,6 2. Channel characterization and modulation order4 For AI receiver 1.Received signal2,3,4,6 | Encoder: Coded bits Decoder: Estimated symbols |
| Model output | For constellation design 1. Learned constellation 1,2,3,4,5,6 2. Probability of constellation points 4 For receiver 1. (Soft) LLR2,3,4,6 | Encoder: modulated symbols after layer mapping Decoder: Soft LLR |
| Label | Known coded bits | Known coded bits |
| Training types | Offline training | Offline training |
| KPI | BLER | BLER |
| Benchmark | Uniform QAM with legacy receiver | Uniform QAM with legacy receiver and NR layer mapping |
| Model location for inference | 1.NA (AI for constellation design with legacy receiver) 1,2,3,4,5 2. NA (constellation learned offline jointly with AI receiver and applied during inference)6 3.Receiver-sided model2,3,4,6 | Two-sided model |
| Collaboration/interaction between UE and NW | NA or Similar to NW-sided model or UE-sided model in NR | Similar to two-sided model in NR |
| Potential specification impact | 1. Constellation design and related signaling/procedure 2. Signaling/ procedure related to LCM for NW-sided model or UE-sided model 3. RAN4 requirements, e.g., EVM | 1. Modulation design and layer mapping design 2. Signaling/ procedure related to LCM for two-sided model including inter-vendor collaboration 3. RAN4 requirements, e.g., EVM |

## AI-based none-linearity handling at transmitter or receiver

Observation 2.6

For 6GR AI/ML use cases identification/categorization, [5 sources] provided preliminary simulation results and analysis on AI-based none-linearity handling at transmitter or receiver.

- [5 sources] provided preliminary simulation results and analysis on AI-based DPoD/None-linearity compensation at receiver.

- [3 sources] provided preliminary simulation results and analysis on AI-based DPD at transmitter.

- Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in Table G.

Note: whether/how to capture the observation in the TR is a separate discussion.

Table G AI-based none-linearity handling at transmitter or receiver

| Sub-use case | Sub-use case A: AI-based DPoD/None-linearity compensation | Sub-use case B: AI-based DPD |
| --- | --- | --- |
| Reported companies | (5) Samsung1, Ericsson2, OPPO3, vivo4, Huawei5 | (3) vivo2, Huawei1, Nokia3 |
| Model input | 1. Received signal1,3,4,5 | 1. Time domain samples before pre-distortion1,2,3 2. Time domain samples for calibration signals.2 |
| Model output | 1. Compensated signal in time domain1,2,4,5 2. Soft bits2,3 | 1.Time domain samples after pre-distortion1,2,3 2. Fitted PA model2 |
| Label | 1. DMRS1 2. Known bit sequence2,3,4 3. time domain samples from known sequence5 | 1. Time domain samples1,2,3 2. Real PA distortion2 |
| Training types | Online training/finetune1 Offline training | Offline training Online training/finetune2 |
| KPI | BLER, MPR, EVM, throughput | BLER, EVM, MPR |
| Benchmark | Without compensation GMP model based NC1 | No DPD |
| Model location for inference | NW-sided model | UE-sided model |
| Collaboration/interaction between UE and NW | Similar to NW-sided model as NR | Similar to UE-sided model as NR |
| Potential specification impact | 1. RAN 4 requirements, e.g. EVM 2. DMRS/Sequence design/selection, Tx power determination 3. Signaling/ procedure related to LCM for NW-sided model | 1. RAN4 requirements, e.g. EVM 2. Tx power determination, SRS design and/or PUSCH scheduling design for data collection 3. Signaling/ procedure related to LCM for UE-sided model |

## AI/ML for SRS

Observation 2.7

For 6GR AI/ML use cases identification/categorization, [5 sources] provided preliminary simulation results and analysis on AI/ML for SRS

- [5 sources] provided preliminary simulation results and analysis on low overhead SRS with AI/ML

- [1 source] provided preliminary simulation results and initial analysis on low PAPR SRS sequence design with help of AI/ML

- [1 source] provided preliminary simulation results and initial analysis on AI/ML based SRS power imbalance compensation

Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and analysis in Table H.

Note: whether/how to capture the observation in the TR is a separate discussion.

Table H AI/ML for SRS

| Use case | Sub-Case A: Low overhead SRS with AI/ML | Sub-Case B: Low PAPR SRS sequence design | Sub-Case C: AI/ML based SRS power imbalance compensation |
| --- | --- | --- | --- |
| Reported companies | (5) {Spreadtrum, UNISOC}, vivo, Huawei1, Kyocera, LGE | (1) vivo | (1) Huawei |
| Model input | Measurement of channel with low overhead SRS of frequency/temporal/spatial domain 1a. assistance info of estimated channel of DMRS1 | Sequence index | UL measured channel matrix from SRS with IL imbalance |
| Model output | Estimated channel | Learn sequences | DL channel matrix with IL compensated |
| Label | Ideal channel information | Label free | UL SRS measurement without IL (assuming it is compensated by UE at certain conditions) or DL CSI-RS measurement |
| Training types | Offline training | Offline training | offline training |
| KPI | SCGS, throughput | PAPR, SGCS, Cross-correlation between SRS sequences | SGCS |
| Benchmark | With legacy SRS With ideal channel information | Legacy SRS sequence | 1. SRS without IL imbalance; 2. non-AI based SRS IL imbalance compensation |
| Model location for inference | NW-sided model | NW-sided model or Without model for inference | NW-sided model |
| Collaboration/interaction between UE and NW | Similar to NW-sided model in NR | No collaboration for no model Similar to NW-sided model in NR | Similar to NW-sided model in NR |
| Potential specification impact | 1.Sparse SRS design 1a. Indication of the association between DMRS port and SRS port1 2. Signalling/ procedure related to LCM for NW-sided model | 1. SRS design 2. Signaling/procedure related to DLable/ULable SRS sequence, when applicable 3. Signalling/ procedure related to LCM for NW-sided model, when applicable | 1. Inference: UE reporting on the IL range for ensuring generalization 2. Signalling/procedure related to LCM for NW-sided model |

## AI-enabled UL precoder indication

Observation 2.8

For 6GR AI/ML use cases identification/categorization, [5 sources] provided preliminary simulation results and analysis on AI-enabled UL precoder indication with detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in Table I.

Note: whether/how to capture the observation in the TR is a separate discussion.

Table I AI-enabled UL precoder indication

| Use case | AI-enabled UL precoder indication |
| --- | --- |
| Reported companies | (5) vivo1, Fujitsu2, Samsung3, LGE4, ZTE5 |
| Model input of decoder or model output of encoder | UL precoder indicator/compressed UL precoder |
| Model output of decoder or model input of encoder | (Reconstructed) eigenvectors of UL channel |
| Label | Estimated eigenvectors of UL channel based on SRS measurement |
| Training types | offline training online finetune1 |
| KPI | SCGS, BLER |
| Benchmark | NR TPMI codebook |
| Model location for inference | No model for inference 1,2,3 Two-sided model1,2,4, 5 |
| Collaboration/interaction between UE and NW | Similar to one-sided model in NR 1,2,3 Similar as two-sided model in NR1,2,4, 5 |
| Potential specification impact | 1.The signaling/procedure related to the download/upload of UL codebooks/compressed UL precoder 2. LCM procedure to facilitate the training of the downloadable/uploadable UL codebooks when no model for inference, 1,2,3 3. Signalling/ procedure related to LCM for two-sided model including inter-vendor collaboration, when applicable1,2 |

## AI/ML based waveform for PAPR reduction

Observation 2.9

For 6GR AI/ML use cases identification/categorization, [3 sources] provided preliminary simulation results and analysis on AI/ML based waveform for PAPR reduction with detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis in Table J.

Note: whether/how to capture the observation in the TR is a separate discussion.

Table J AI/ML based waveform for PAPR reduction

| Use case | AI/ML based waveform for PAPR reduction |
| --- | --- |
| Reported companies | (3) vivo1, Samsung2, Huawei3 |
| Model input | Symbols in frequency domain |
| Model output | For model output of encoder for UE-sided/NW-part of two-sided model: transformed/precoded symbols in frequency domain For output of decoder for NW-part of two-sided model: 1. LLR1,3 2. Symbols in frequency domain2 |
| Label | Label free2,3 Known bit sequences or its LLR1,3 |
| Training types | offline training |
| KPI | BLER, CCDF of PAPR(UL), throughput (DL) |
| Benchmark | DFT-s-OFDM |
| Model location for inference | Two-sided model UE-sided model (for frequency domain shaping)1,3 |
| Collaboration/interaction between UE and NW | Similar to two-sided model in NR No collaboration for UE-sided model1 |
| Potential specification impact | 1. Signaling/ procedure related to LCM for two-sided model including inter-vendor collaboration, when applicable 2. Signaling/ procedure related to LCM for UE-sided model 1,3 3. Related RAN4 requirements |

## AI/ML based HARQ-ACK feedback

Observation 2.10

For 6GR AI/ML use cases identification/categorization, [2 sources] provided preliminary simulation results and analysis on AI/ML based HARQ-ACK feedback with detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis in Table K.

Note: whether/how to capture the observation in the TR is a separate discussion.

Table K. AI/ML based HARQ-ACK feedback

| Use case | AI/ML based HARQ-ACK feedback |
| --- | --- |
| Reported companies | (2) Qualcomm, vivo |
| Model input | HARQ ACK/NACK bit sequence |
| Model output | Learned sequences/modulated symbols |
| Label | HARQ-ACK/NACK bit sequence |
| Training types | Offline training |
| KPI | BLER |
| Benchmark | NR RM code for up to 11bits with Maximum Likelihood (ML) receiver |
| Model location for inference | No model for inference |
| Collaboration/interaction between UE and NW | No collaboration |
| Potential specification impact | 1.Learned sequence/modulated symbols design 2.Downloadable sequence/modulated symbols related signalling/ procedure for HARQ-ACK 3. Related RAN4 requirements |

## AI/ML for PDCCH

Observation 2.11

For 6GR AI/ML use cases identification/categorization, [2 sources] provided preliminary simulation results and analysis on AI/ML for PDCCH

- [2 sources] provided preliminary simulation results and analysis on prior-information-aided DCI decoding,

- [one source] provided preliminary simulation results and analysis on lossless DCI compression,

Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in in Table L

Note: whether/how to capture the observation in the TR is a separate discussion.

Table L AI/ML for PDCCH

| Use Case | Sub-Case A: Prior-Information-Aided DCI Decoding | Sub-Case B: Lossless DCI Compression |
| --- | --- | --- |
| Reported companies | (2) CMCC, OPPO | (1) CMCC |
| Model input | Historical DCI payloads (including last successfully decoded DCI payload) | Historical DCI payload Received compressed DCI payload |
| Model output | Predicted LLR for intended DCI payload | Predicted DCI payload |
| Label | DCI payload sequences | DCI payload sequences |
| Training types | Offline training at the UE side | Offline training at the NW side, and model delivery to UE side |
| KPI | BLER performance | BER and sample-level prediction accuracy; DCI overhead reduction |
| Benchmark | Traditional DCI decoder | Traditional DCI design |
| Model location for inference | UE-sided model | UE-sided model + NW-sided model |
| Collaboration/interaction between UE and NW | Similar to UE-sided model in NR | Model transfer from NW to UE |
| Potential specification impact | 1. Signalling/configuration design for prior-information-aided DCI decoder. 2. Signalling/ procedure related to LCM for UE-sided model | 1. Signalling/configuration design for Lossless DCI Compression. 2. Signalling/ procedure related to LCM including model transfer |

## AI/ML for power control

Observation 2.12

For 6GR AI/ML use cases identification/categorization, [2 sources] provided preliminary simulation results and analysis on AI/ML for power control

[2 sources] provided preliminary simulation results and analysis on pathloss prediction in the spatial, temporal, and/or frequency domain, to use the predicted pathloss in UL (PUSCH/PUCCH/PRACH/SRS) power control.

[one source] provided preliminary simulation results and analysis on UL closed-loop power control with an NW-sided AI/ML model, where the model predicts the optimal power adjustment (or TPC command index) for the UE,

Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in in Table M

Note: whether/how to capture the observation in the TR is a separate discussion.

Table M AI/ML for power control

| Use Case | Sub-Case A: UL closed-loop power control with an NW-sided AI/ML model, where the model predicts the optimal power adjustment (or TPC command index) for the UE. | Sub-Case B: Pathloss prediction in the spatial, temporal, and/or frequency domain, to use the predicted pathloss in UL(PUSCH/PUCCH/PRACH/SRS) power control. |
| --- | --- | --- |
| Reported companies | (1) Nokia | (2) Nokia, Google (Cite R19 results) |
| Model input | UL SINR measurement, UE Tx power estimate (derived from Pcmax, P0, PL alpha, pathloss measurement), and PUSCH allocation size | L1-RSRP measurements from a sub-set/set of RSs/beams (Set B). input can consider history of measurements |
| Model output | Predicted TPC command index | Predicted pathloss value(s) (or predicted L1-RSRP(s)) for a set of RSs/beams (Set A). output can consider future instances |
| Label | Optimal TPC command index (offline learning) label-free (online learning) | Pathloss value(s) (or L1-RSRP(s)) for a set of RSs/beams (Set A) |
| Training types | Offline and Online learning | Offline training |
| KPI | UL throughput. | Pathloss prediction accuracy, throughput, RS overhead reduction, Complexity. |
| Benchmark | 1. UL Power control with optimized OLPC parameters 2. UL Power control with optimized OLPC parameters and possibly legacy CLPC algorithms (with 5G TPC tables). | Pathloss estimation based on Set A Pathloss estimation based on Set B |
| Model location for inference | NW-sided model | UE-sided model NW-sided model |
| Collaboration/interaction between UE and NW | None | As UE-sided or NW-sided mode in nRl |
| Potential specification impact | Configurability of the values in TPC command tables or an extended TPC command table (compared to NR). | 1. Pathloss prediction related signalling/procedure 2. Signalling/ procedure related to LCM for UE-sided or NW-sided model 3. RAN4 performance requirements and test cases, including defining new requirements related to pathloss reference signal (PL-RS) measurement and activation delays of TCI state(s). |

## AI/ML for RACH related design

Observation 2.13

For 6GR AI/ML use cases identification/categorization, [3 source] provided preliminary simulation results and analysis on AI/ML for RACH related design

- [2 sources] provided preliminary simulation results and analysis on early contention resolution in RACH,

- [1 source] provided preliminary simulation results and analysis on Low PAPR sequence design for PRACH

Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in in Table N

Note: whether/how to capture the observation in the TR is a separate discussion.

Table N AI/ML for RACH related design

| Sub-use case | Sub-Case A: Early contention resolution in RACH | Sub-Case B: Low PAPR sequence design for PRACH |
| --- | --- | --- |
| Reported Companies | (2) Ofinno1, Samsung2 | vivo |
| Model input | 1.Received PRACH signal (e.g., preamble waveform)1 2. Detected PRACH signal with PDP or truncated PDP in time domain for each preamble detection window2 | Sequence index |
| Model output | Predicted number of UEs that transmitted the same preamble for given PRACH resources | Learned sequences |
| Label | Ground-truth number of UEs that transmitted the same preamble | Label free |
| Training types assumption | Offline training | Offline training |
| KPI | Prediction accuracy of UE multiplicity, RACH access delay, first-attempt success probability, and false alarm detection rate | PAPR/DCM, Cross-correlation between sequences |
| Benchmark | First-attempt success rate based on legacy PRACH receiver | Legacy PRACH sequence |
| Model location for inference | NW-side model | Without model for inference |
| Collaboration/interaction between UE and NW | Similar to NW-sided model in NR | No collaboration |
| Potential spec impact | 1. Signaling/procedure related to random access for, including Mgs.3 grant for more than one UEs selected the same PRACH sequence 2. Signalling/procedure related to LCM for NW-sided model | Standardized sequence table per required sequence length |

## Site Specific Learning for AI/ML using RAN Digital Twin

Observation 2.14

For 6GR AI/ML use cases identification/categorization, [3 sources] provided preliminary simulation results and analysis on Site Specific Learning for AI/ML using RAN Digital Twin,

Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in in Table O

Note: whether/how to capture the observation in the TR is a separate discussion.

Table O Site Specific Learning for AI/ML using RAN Digital Twin

| Sub-use case | Site Specific Learning for AI/ML using RAN Digital Twin |
| --- | --- |
| Reported Companies | (3) DeepSig1, NVIDIA2, Huawei3 |
| Model input | 1.Received signal/estimated channel at DMRS and received signal on data, and the channel information generated by digital twin1,3 2.Noisy least square channel estimate on DMRS REs2 (The model is trained/finetuned with digital twin generated data) |
| Model output | 1.Decoded bit1,3 2.Denoised channel estimate on DMRS REs2 (The model is trained/finetuned with digital twin generated data) |
| Label | Ground truth of target bit1,3 Ground truth of channel2 |
| Training types assumption | Offline training |
| KPI | BLER, throughput |
| Benchmark | Legacy receiver without the help of digital twin |
| Model location for inference | NW-sided model |
| Collaboration/interaction between UE and NW | Similar to NW-sided model in NR |
| Potential spec impact | Signalling/procedure related to LCM for NW-sided model (including the generation of RAN digital twin3) |

## Digital twin construction related use cases

Observation 2.15

For 6GR AI/ML use cases identification/categorization, [1 source] provided preliminary simulation results and analysis on digital twin construction related use cases,

- [1 source] provided preliminary simulation results and analysis on sensing based RAN digital twin construction with NW-side AI/ML model,

- [1 source] provided preliminary simulation results and analysis on AI/ML-enabled RAN digital twin with distributed model,

Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in in Table P

Note: whether/how to capture the observation in the TR is a separate discussion.

Table P Digital twin construction related use cases

| Sub-use case | Sub-Case A: Sensing based RAN digital twin construction with NW-side AI/ML model | Sub-Case B: AI/ML-enabled RAN digital twin with distributed model |
| --- | --- | --- |
| Reported Companies | (1) Huawei | (1) Huawei |
| Model input | Point cloud sensed by the BS with mono-static sensing and sensed/reported by UEs with bi-static sensing | UE-part models: local sparse point cloud NW-part model: latent space information from multiple UEs |
| Model output | 3D point cloud representing the static environment | UE-part models: compressed latent space information NW-part model: global point cloud |
| Label | Ground truth point cloud | Ground truth point cloud |
| Training types assumption | Offline training | Offline training (adopted in simulation) Online finetuning (can be optionally considered) |
| KPI | Sensing accuracy metric: root mean square error (RMSE) of point cloud. RMSE=1ni=1nxi-xi2 is the square root of the average of the squared errors between each sensed point (xi in forms of coordinates) and ground truth point (xi in forms of coordinates) in the point cloud including n points with {x, y, z} dimensions. | 1. Overhead metric: Feedback bits per point 2. Sensing accuracy metric: intersection-over-union (IoU), edge detection probability |
| Benchmark | BS side mono-static sensing only to construction RAN digital twin | 1. Single UE sensing (to justify sensing accuracy metric of using distributed model). 2. Raw data transmission (to justify overhead metric of using distributed model). |
| Model location for inference | NW-side model | Distributed model: a NW-side model paired with multiple UE-side models. |
| Collaboration/interaction between UE and NW | Similar to NW-sided model in NR | Similar to two-sided model: UE reporting of compressed sensing results for inference. Inter-vendor training collaboration between NW side and UE side. |
| Potential spec impact | 1. Signaling/procedure related to bi-static sensing results reported from UE 2. Signalling/procedure related to LCM for NW-sided model | 1. Sensing results reported from UE in forms of compressed latent message 2. Signalling/procedure related to LCM for two-sided model including inter-vendor collaboration |

## AI for positioning

Observation 2.16

For 6GR AI/ML use cases identification/categorization, [1 source] provided preliminary simulation results and analysis on AI for positioning, and detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in in Table Q

Note: whether/how to capture the observation in the TR is a separate discussion.

Table Q AI for positioning

| Sub-use case | Sub-Case A: Positioning Case 1 | Sub-Case B: Positioning Case 3a | Sub-Case C: Positioning Case 3b |
| --- | --- | --- | --- |
| Reported Companies | (1) Ericsson | (1) Ericsson | (1) Ericsson |
| Model input | Time domain samples (PDP, DP), or timing/power measurements of multiple paths | Time domain samples (PDP, DP), or timing/power measurements of multiple paths | Time domain samples (PDP, DP), or timing/power measurements of multiple paths |
| Model output | UE location coordinates | Intermediate positioning measurements (e.g., ToA, LOS/NLOS indicator) | UE location coordinates |
| Label | UE location coordinates provided by PRU or LMF | ToA; binary value of LOS/NLOS indicator; Label provided by LMF based on UE location coordinates | UE location coordinates provided by PRU or LMF |
| Training types assumption | Offline training | Offline training | Offline training |
| KPI | CDF percentiles of horizonal accuracy | CDF percentiles of horizonal accuracy | CDF percentiles of horizonal accuracy |
| Benchmark | DL-TDOA | UL-TDOA | UL-TDOA |
| Model location for inference | UE-side model | gNB-side model | LMF-side model |
| Collaboration/interaction between UE and NW | LMF provides to UE all assistance information from legacy UE-based DL-TDOA, other than info #7 (Geographical coordinates of the TRPs) | gNB and LMF coordinate to configure SRS transmission by UE | gNB and LMF coordinate to configure SRS transmission by UE |
| Potential spec impact | Signaling related to the AI/ML functionality; Training data generation; Data collection; Model monitoring | Training data generation; Data collection; Model monitoring | Training data generation; Data collection; Model monitoring |

## For AI/ML related service

Observation 2.17

For 6GR AI/ML related service, for 6GR AI/ML use cases identification/categorization,

- [8 sources] provided preliminary simulation results and analysis on improved scheduling/HARQ for token traffic,

- [1 source] provided preliminary simulation results and analysis on transmission for audio token traffic

Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis in Table R.

Note: whether/how to capture the observation in the TR is a separate discussion.

Table R for AI/ML related service

| Use case | Sub-Case A: Improved scheduling/HARQ for token traffic | Sub-Case B: Transmission for audio token traffic |
| --- | --- | --- |
| Reported companies | (8) Huawei1, OPPO2, Google3, CMCC4, Lenovo5, CAICT6, Pengcheng Lab7,BUPT8 | (1)vivo1 |
| Model input | Tokenizer model: • Input: Raw data (e.g., image/video/audio/text, etc.) De-tokenizer model: • Input: Tokens | Tokenizer model: • Input: Raw data (e.g., audio, etc.) De-tokenizer model: • Input: Received tokens |
| Model output | Tokenizer model: • Output: Tokens (e.g., tokenized image/video/audio/text) De-tokenizer model: • Output: Inference results for downstream tasks/Raw data (e.g., image/video/audio, etc.) | Tokenizer model: • Output: Tokens (e.g., tokenized audio), in the form of bits or modulated symbols De-tokenizer model: • Output: reconstructive raw data (e.g., audio, etc.) |
| Label | Training at OTT, transparent to RAN | Original audio data |
| KPI | Supported number of UEs, achievable throughput | Coverage gain for acceptable PSEQ/MOS level, supported number of UEs |
| Benchmark | NR scheduling/HARQ mechanism without knowledge of Token traffic | Legacy design without aware of audio token traffic Note: AI/ML based source coding for both benchmark and proposed use cases |
| Model location for inference | Transparent to RAN | The tokenizer model is at UE The de-tokenizer model is at NW/OTT server |
| Collaboration/interaction between UE and NW | Transparent to RAN | Collaboration/interaction for two-sided model |
| Potential specification impact | • Service awareness in RAN • Token error identification, new scheduling and HARQ | • Service awareness in RAN • New scheduling and HARQ • New protocol design cross layers for audio |