3GPP TSG-RAN WG1 Meeting #123	Tdoc R1-2509515

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	8.1

Source:	Moderator (Ericsson)

Title:	Summary #2: maintenance of specification support for positioning accuracy enhancement

Document for:	Discussion, Decision

Introduction

This document summarizes the discussions during RAN1#123 for the agenda item 8.1, where the topic is the maintenance of Rel-19 specification support for AI/ML positioning accuracy enhancement.

Assistance info for model inference

Issue 2-1. Request associated ID to be included in PRU info

- InterDigital, Inc. (R1-2508644) Proposal 3: The UE shall be able to request for TRP-LocationInfo-Implicit to be included in NR-PRU-DL-Info

1st round discussion

Based on InterDigital (R1-2508644), the potential issue is:

- “the associated ID may always be provided to the UE which may lead to an increase in signaling overhead and creates potentially frequently communication between the LMF and gNB for determination of the network side conditions.”

In FL’s understanding, if this is a concern, RAN2 can address it in their CR to 37.355 on LPP signaling, and RAN1 does not need to handle it.

Companies can share views whether the issue pointed out by InterDigital needs to be addressed by RAN1.

Question 2.1.1: Should RAN1 address the issue described in InterDigital (R1-2508644)?

| Company | Comments |
| --- | --- |
| HW/HiSi | We believe that this issue can be addressed by RAN1 together with the Issue 2-3, i.e., the UE may request the PRU information with associated ID (Issue 2-1) by indicating the associated ID for which it expects to receive the PRU information (Issue 2-3). If the UE does not indicate any associated ID in the request of the PRU information, then the LMF does not need to provide the associated ID with the PRU information. |
| Nokia | Agree with the direction proposed by InterDigital. In our view, all aspects related to Associated ID should be solved by RAN1. The solution raised by RAN1 may impact 38.214 and 37.355 (RAN2). The UE should be able to request the TRP-LocationInfo-Implicit included in NR-PRU-DL-Info has a direct impact on Data collection received from PRUs, which is an essential element to build datasets to enable AIML Positioning. So, we agree with HW and InterDigital. |
| vivo | We support to discuss this issue. We also think this can be addressed via a TP we proposed in section 2.3. |
| Ericsson | The issue seem to be up to RAN2, in order to optimize the signaling overhead and avoid repeating associated ID information in the assistance data. In our view the issue is out of RAN1 scope. |
| Qualcomm | This is an optimization of signaling which needs to be handled by RAN2. |
| InterDIgital | We support this proposal. Since this shall be an explicit request sent by the UE, we don’t consider the proposal to be an optimization issue related to signaling. |

2nd round discussion

The conclusion presented in 1st round is shown below for further discussion.

Conclusion 2.1.2

- For Rel-19 AI/ML positioning, there is no consensus in RAN1 to discuss whether/how the UE sends a request to network to include TRP-LocationInfo-Implicit in NR-PRU-DL-Info.

| Company | Comments |
| --- | --- |
| NEC | Support this conclusion. If the intention of requesting “associated ID” is to reduce the overhead (as mentioned by InterDigital), this request is unnecessary as the nr-TRP-LocationInfo-Implicit(including associated ID) is optional in NR-PositionCalculationAssistance, thus the associated ID may NOT always be provided to the UE. If the intention of requesting “associated ID” is to require consistent data from PRU, it can be discussed in Issue 2-3, and this issue is not existing. |
| vivo | Given on-going related RAN2 discussion/work, we don’t think a conclusion in RAN1 is needed at this time. As that may give RAN2 the impression that RAN1 concluded on this matter which in turn may impact RAN2 discussion. |
| Nokia | We do not support the conclusion. However, we support the proposal done by InterDigital. In our view this explicit request is beneficial for data collection and monitoring. In our view it is not an optimization related signalling. To Fl and other companies that used the argument that it is an optimization, would you explain/provide details of the optimization that you are raising several times? |
| Qualcomm | We agree with the FL conclusion. We don’t consider it an essential correction for the UE to send a request for specific TRP-Implicit-ID. |
| DOCOMO | Support this conclusion. |

Issue 2-2. Consistency by AD provided explicitly

Nokia (R1-2508948) section 3.2 provided the following text proposal to TS 38.214.

| Summary of change | Describe the UE behavior on receiving explicit assistance data (e.g. nr-DL-PRS-ResourceID-r16) from the network for DL AI/ML positioning method. |
| --- | --- |
| Consequences if not approved | For Rel-19 AI/ML positioning, If explicit AD is provided, the UE may not be able to ensure consistency between training and inference. |
| Suggested text proposal | TS 38.214 V19.1.0 - 5.1.6.5 PRS reception procedure ================= text omitted=============================== The UE expects that it will be configured with dl-PRS-ID each of which is defined such that it is associated with multiple DL PRS resource sets. The UE expects that one of these dl-PRS-ID along with a nr-DL-PRS-ResourceSetID and a nr-DL-PRS-ResourceID-r16 can be used to uniquely identify a DL PRS resource. The UE may be configured by the network with nr-PhysCellID, nr-CellGlobalID, and nr-ARFCN [17, TS 37.355] associated with a dl-PRS-ID. - If nr-PhysCellID or nr-CellGlobalID is provided, and if nr-PhysCellID, nr-CellGlobalID and nr-ARFCN associated with the dl-PRS-ID, For DL AI/ML positioning, UE may assume that DL PRS across two configurations are consistent if both configurations contain the same dl-PRS-ID(s), nr-DL-PRS-ResourceSetID(s) and a nr-DL-PRS-ResourceID(s) associated with the same nr-PhysCellID or same nr-CellGlobalID and nr-ARFCN. - If nr-PhysCellID or nr-CellGlobalID is provided, and if nr-PhysCellID, nr-CellGlobalID and nr-ARFCN associated with the dl-PRS-ID, if provided, are the same as the corresponding information of a serving cell, the UE may assume that the DL PRS is transmitted from the serving cell; ================= text omitted=============================== |

1st round discussion

In FL’s view, for assistance information provided explicitly, the UE can check for consistency, and this is up to UE implementation. There is no need to describe it in 38.214.

Furthermore, there are many more types of assistance information, as shown below. It is not clear why Nokia’s TP should be adopted, which only address DL-PRS configuration.

|  | Information |
| --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement |
| 2 | Timing relative to the serving (reference) TRP of candidate NR TRPs |
| 3 | DL-PRS configuration of candidate NR TRPs |
| 4 | Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation |
| 5 | SSB information of the TRPs (the time/frequency occupancy of SSBs) |
| 6 | Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB |
| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |
| 8 | Fine Timing relative to the serving (reference) TRP of candidate NR TRPs |
| 9 | PRS-only TP indication |
| 10 | The association information of DL-PRS resources with TRP Tx TEG ID |
| 11 | LOS/NLOS indicators |
| 12 | On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation |
| 13 | Validity Area of the Assistance Data |
| 14 | PRU measurements together with the location information of the PRU |
| 15 | Data facilitating the integrity results determination of the calculated location |
| 16 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) |
| 17 | Expected Angle Assistance information |
| 18 | PRS priority list |

Companies can share views whether the TP by Nokia should be adopted by RAN1.

Question 2.2.1: Should RAN1 adopt the text proposal in Nokia (R1-2508948) section 3.2?

| Company | Comments |
| --- | --- |
| HW/HiSi | We do not support this text proposal, as the UE can check for consistency based on implementation and hence, there is no need to describe it in 38.214. |
| Nokia | This TP is based on agreements done during the work item of Rel. 19, it is not based on additional preferences that companies may provide at this stage. Answering FL question, we proposed „DL-PRS configuration“ for consistency because TS 38.214 Section 5.1.6.5 is related only to PRS procedures. |
| vivo | No |
| Ericsson | We should not have wording refering to the purpose in our specification. Additionally, the UE assumption for associated ID is already captured in the paragraphs following the TP. |
| Qualcomm | In principle, two different PRS configurations may assign common cell IDs as well as common TRP IDs and RS set/RS IDs. The proposed TP does not seem to help ensure consistency. Further justification needed on how it can ensure consistency at PRS configuration level. |
| Nokia | To FL, as commented before, this TP is not based on a preference of one specific company, it is based on RAN1 agreements done with the participation of all companies. There is no RAN1 agreement or conclusion saying that it is up to implementation or similar. So, we request to FL to raise this TP during the online discussion and we will provide the proper explanation to vice-chair. To Qualcomm, the proposed TP is mentioning that the PRS configuration is associated to nr-PhysCellID or same nr-CellGlobalID and nr-ARFCN. So, the PRS configuration may be consistent at least in certain Cell-ID. |

2nd round discussion

The conclusion presented in 1st round is shown below for further discussion.

Conclusion 2.2.2

- For Rel-19 AI/ML positioning, there is no consensus in RAN1 to further discuss the UE behavior on receiving explicit assistance data from the network.

| Company | Comments |
| --- | --- |
| Nokia | We do not support the conclusion. |
| Qualcomm | We support the conclusion. It is not an essential correction to specify/clarify further the UE behavior regarding this aspect. |
| DOCOMO | In my understanding, how to check for consistency with explicit indication is up to implementation. |

Issue 2-3. Consistency between training and inference

Nokia (R1-2508948) section 3.3 provided the following text proposal to TS 38.214.

| Reason for change | In RAN1#122, it was agreed that the UE may be provided with NR-TRP-LocationInfo-Implicit associated with NR-PRU-DL-Info. However, this alone does not help the UE if it is not expecting to receive NR-PRU-DL-Info associated with the provided TRP-LocationInfo-Implicit (e.g., when the UE has not collected before the PRU data associated with the provided TRP-LocationInfo-Implicit). Moreover, this may result in DL resource wastage as it risks signaling PRU data that is not useful at the UE. To this end, the UE can report the one or more NR-TRP-LocationInfo-Implicit for which it expects to receive the associated NR-PRU-DL-Info. The network can then provide the PRU data that is associated with only the reported NR-TRP-LocationInfo-Implicit. |
| --- | --- |
| Summary of change | Clarify that the UE may report NR-TRP-LocationInfo-Implicit for which it expects to receive the associated NR-PRU-DL-Info. |
| Consequences if not approved | For Rel-19 AI/ML positioning, the UE may not be able to ensure consistency between training and inference. Moreover, there is a risk of DL resource wastage by providing PRU data that is not useful at the UE. |
| Suggested text proposal | TS 38.214 V19.1.0 ##### 5.1.6.5 PRS reception procedure ================= text omitted=============================== The UE may be provided with NR-PRU-DL-Info which contains measurement(s) performed by a positioning reference unit (PRU) [20, TS 38.305], the timestamps associated with the measurement(s), and the location information of the PRU. The UE may be provided with NR-TRP-LocationInfo-Implicit [17, TS 37.355] associated with NR-PRU-DL-Info. The UE may report NR-TRP-LocationInfo-Implicit [17, TS 37.355] for which it expects to receive the associated NR-PRU-DL-Info. ================= text omitted=============================== |

1st round discussion

In FL’s understanding, NR-TRP-LocationInfo-Implicit is used by LMF to send to UE the network-side additional information of associated ID. The UE does not have the network’s associated ID information (i.e., NR-TRP-LocationInfo-Implicit) without it being provided by LMF to UE. Thus, it is not clear how the UE can report NR-TRP-LocationInfo-Implicit to network for which it expects to receive the associated NR-PRU-DL-Info.

Also, similar to issue 2-1 in section 2.1, if there is a need to change the LPP signaling, RAN2 can address it in their CR to 37.355, and RAN1 does not need to handle it.

Companies can share views whether the TP by Nokia should be adopted by RAN1.

Question 2.3.1: Should RAN1 adopt the text proposal in Nokia (R1-2508948) section 3.3?

| Company | Comments |
| --- | --- |
| HW/HiSi | We support the text proposal. Before the provision of PRU information, the UE may have been provided with NR-TRP-LocationInfo-Implicit, e.g., during the training of a model at the UE. Thus, the UE may already be aware of an associated ID that is associated with a model at the UE and hence, the UE can request PRU information with this associated ID, such that the provided PRU measurements can be used with the UE’s model, e.g., for further training/monitoring. Note that the LMF may not be aware with which associated ID, a UE may have trained a model. As mentioned in our reply to Issue 2-1, we believe that this issue can be addressed by RAN1 together with the Issue 2-1, i.e., the UE may request the PRU information with associated ID (Issue 2-1) by indicating the associated ID for which it expects to receive the PRU information (Issue 2-3). If the UE does not indicate any associated ID in the request of the PRU information, then the LMF does not need to provide the associated ID with the PRU information. |
| Nokia | Agree with HW. We also support the TP. |
| vivo | We think it’s beneficial for UE to request NR-TRP-LocationInfo-Implicit when request assistant data. However, we think the wording should be refined to clearly indicating this is a UE request. We have the following wording suggestion: The UE may report NR-TRP-LocationInfo-Implicit [17, TS 37.355] through NR-DL-AIML-RequestAssistanceData [17, TS 37.355] for which it expects to receive the associated NR-PRU-DL-Info. |
| Ericsson | Similar to proposal 2-1, it is not clear what RAN1 can do on such issue. If ran2 see the signalling as necessary, it can handle it. |
| Qualcomm | First, it is not clear whether there is a waste in resources or not. The same claim can be casted even for legacy PRU design. Any optimizations on reporting overhead can be handled by RAN2. Second, UE can indicate explicit TRP locations, TRP IDs, and/or cell IDs whose PRU related measurements need to be provided. It should not be limited to implicit TRP location indication. |
| Nokia | Ok with the wording proposed by vivo. To Ericsson and Qualcomm, the TP is impacting TS 38.214. So, what RAN2 may do or not is not part oft he discussion. The purpose is to provide practical usage of NR-TRP-LocationInfo-Implicit for data collection with PRU, which is the most reliable entity to generate dataset points. If required, we may explain offline to provide further clarification. |
| ZTE | We think Nokia’s TP lacks the justification of necessity at the maintenance stage. Current spec could function despite of the potential DL resource wastage, if any. Besides, it could be resolved by UE implementation to assure the consistency between training and inference. |

2nd round discussion

For the conclusion presented in 1st round, there is a suggestion to change to “no consensus to endorse the TP”. Thus it is revised below for further discussion.

Conclusion 2.3.2

- For Rel-19 AI/ML positioning, there is no consensus in RAN1 to endorse the TP to 38.214 where the UE may report NR-TRP-LocationInfo-Implicit for which it expects to receive the associated NR-PRU-DL-Info.

| Company | Comments |
| --- | --- |
| NEC | The issue is existing. If it not intends to endorse the TP to TS38.214, does it mean the similar TP will be reflected in TS37.355? |
| vivo | First of all, this is UE report NR-TRP-LocationInfo-Implicit (associated ID) in the request NR-DL-AIML-RequestAssistanceData. Given on-going related RAN2 discussion/work, we don’t think a conclusion in RAN1 is needed at this time. As that may give RAN2 the impression that RAN1 concluded on this matter which in turn may impact RAN2 discussion. |
| HW/HiSi | We believe it is beneficial for the UE to indicate the associated ID, for which it would like to receive the associated PRU information, in the request of the PRU information as assistance data for Case 1. |
| Nokia | We do not support the conclusion. This type of conclusions are not helping the progress and discussion. In our view, the discussion should be limited to show our views on proposed TPs and nothing else. we request companies that are using the argument that it may be solved by implementation to provide further explanation to support their views. To companies indicating that RAN2 may solve it, we do not agree with such asssmption because RAN1 was the lead working group and because RAN2 did not introduced the concept of Associated ID (NR-TRP-LocationInfo). |
| Qualcomm | We support the conclusion. We don’t consider it an essential correction the UE to indicate specific associated ID. The LMF can make sure that the PRU-INFo that are provided to the UE are derived using the same associatedID as that being used at the EU. |

Issue 2-4. Usage of associated ID

Nokia (R1-2508948) section 3.4 provided the following text proposal to TS 38.214.

| Reason for change | The recently introduced specification text on associated ID in TS 38.214 Section 5.1.6.5 is not in line with the agreements, and further creates ambiguities, in the following ways: - As per the latest draft CR R2-2508587 for TS 37.355, Associated ID within a TRP-LocationInfo-Implicit-Element IE is always provided with the corresponding cell ID using nr-CellGlobalID. However, the currently introduced text in TS 38.214 implies that the cell ID is optional, and can be either nr-CellGlobalID or nr-PhysCellID. - In the currently introduced text in TS 38.214, it is not clear what is meant by geographical coordinates of the TRP(s) being “consistent”, i.e., with regards to what. For DL AI/ML positioning, UE would need to evaluate the consistency of TRP locations between training and inference by comparing their associated ID(s) assumed for training and inference, namely across two different configurations. - As per the LS response to RAN2 (R1-2508026), a cell might have multiple associated IDs, where each associated ID may represent one or more TRP(s) within the cell. However, the currently introduced text on associated ID in TS 38.214 can be interpreted as if a cell could have only a single associated ID. - When a cell has multiple associated IDs, it would be impossible for UE to evaluate any consistency without knowing which TRP(s) is(are) represented by each associated ID for a given cell. As per legacy specifications, DL-PRS-ID is used to identify the associated TRP(s) in TRP-LocationInfoElement, whereas the newly introduced TRP-LocationInfo-Implicit-Element contains only the associated ID and the cell ID. Therefore, simply integrating DL-PRS-ID(s) for the group of TRP(s) within the TRP-LocationInfo-Implicit-Element would resolve the mapping issue between the TRP(s) and the associated ID. |
| --- | --- |
| Summary of change | Correction of the text in TS 38.214 Section 5.1.6.5 to align with the agreements and to avoid any ambiguities in interpretation. |
| Consequences if not approved | - Specification text will conflict with TS 37.355, making signaling of cell ID together with an associated ID optional. - It is also not clear what is meant by “consistent” assumed by the UE. - A cell may only have a single associated ID and not multiple associated IDs, which was not agreed by RAN1. - UE will not be able to evaluate the consistency of TRP locations between training and inference for DL AI/ML positioning. |
| Suggested text proposal | 5.1.6.5 PRS reception procedure ================= text omitted=============================== The UE may be configured by the network with TRP-LocationInfo-Implicit-Element [17, TS 37.355]. For DL AI/ML positioning, UE may assume that the geographical coordinates of group of TRP(s) across two configurations are consistent if both configurations contain the same nr-AIML-AssociatedID for the same nr-CellGlobalID, together with the same DL-PRS-ID(s) corresponding to the group of TRP(s). ================= text omitted=============================== |

1st round discussion

In FL understanding, Nokia’s text proposal above has similar intention as the text proposal endorsed in RAN1#123bis:

TS 38.214 V19.1.0 ##### 5.1.6.5 PRS reception procedure < Unchanged parts are omitted > If the UE assumes that the DL PRS is not transmitted from a serving cell, and if nr-PhysCellID is provided, and is the same as physical cell ID of the SS/PBCH block from a non-serving cell of the same band as the DL PRS, the UE may assume that the DL PRS and the SS/PBCH block are transmitted from the same non-serving cell. The UE may be configured by the network with TRP-LocationInfo-Implicit-Element [17, TS 37.355]. - If TRP-LocationInfo-Implicit-Element is provided, and if nr-PhysCellID or nr-CellGlobalID is provided with nr-AIML-AssociatedID, for TRP(s) in the given cell, the UE may assume that the geographical coordinates of the TRP(s) are consistent for a same nr-AIML-AssociatedID.

The endorsed TP above does not have severe problems as described in Nokia contribution. It is unclear why Nokia thinks the endorsed TP is not inline with agreements.

Furthermore, Nokia’s TP links the associated ID to DL-PRS-ID(s). However, RAN1 agreed that “an associated ID is configured per-cell (e.g., NCGI-r15)”. In FL’s view, the associated ID should not be linked to DL-PRS-ID(s) for a group of TRP(s). Thus it is not convincing why Nokia’s TP should replace the endorsed TP.

Companies can share views whether the TP by Nokia should be adopted to replace the TP endorsed in RAN1#122bis.

Question 2.4.1: Should RAN1 adopt the text proposal in Nokia (R1-2508948) section 3.4 to replace the TP endorsed in RAN1#122bis?

| Company | Comments |
| --- | --- |
| HW/HiSi | We do not agree with this text proposal. As agreed in the RAN1#121 agreement, it is clear that the UE can check for consistency regarding the TRP locations based on the associated ID. |
| Nokia | Our intention was to complement and clarify the agreement done in RAN1#122bis, if you check, the TP includes the term „Group of TRPs “ based on the last LS reply to RAN2, in addition, we include the term „together with the same DL-PRS-ID(s) corresponding to the group of TRP(s)“ to provide details that each group of TRPs is composed by a set of TRPs and these TRPs should be identified by DL-PRS-ID(s) during data collection and during inference. whitout this detail, the TRP-LocationInfo-Implicit-Element IE may be useless for the purpose that RAN1 introduced the concept of Associated ID for AIML positioning. Clarifying to HW, our intention is not to change the agreement done in RAN1#122, our intention is only to complement it to provide a feasible and useful framework for checking consistency. We request FL and other companies to consider again the risk of not included such detail. |
| vivo | No |
| Ericsson | We do not think this TP is necessary. |

2nd round discussion

The conclusion presented in 1st round is shown below for further discussion.

Conclusion 2.4.2

- For Rel-19 AI/ML positioning, there is no consensus in RAN1 to further discuss any correction to the TP endorsed in RAN1#122bis:

TS 38.214 V19.1.0 ##### 5.1.6.5 PRS reception procedure < Unchanged parts are omitted > If the UE assumes that the DL PRS is not transmitted from a serving cell, and if nr-PhysCellID is provided, and is the same as physical cell ID of the SS/PBCH block from a non-serving cell of the same band as the DL PRS, the UE may assume that the DL PRS and the SS/PBCH block are transmitted from the same non-serving cell. The UE may be configured by the network with TRP-LocationInfo-Implicit-Element [17, TS 37.355]. - If TRP-LocationInfo-Implicit-Element is provided, and if nr-PhysCellID or nr-CellGlobalID is provided with nr-AIML-AssociatedID, for TRP(s) in the given cell, the UE may assume that the geographical coordinates of the TRP(s) are consistent for a same nr-AIML-AssociatedID.

| Company | Comments |
| --- | --- |
| NEC | Agree this conclusion. We also think the TP from last meeting can show the same meaning as Nokia’s. |
| HW/HiSi | Ok |
| Nokia | Clarification to NEC: Our intention was to complement and clarify the agreement done in RAN1#122bis, if you check, the TP includes the term „Group of TRPs “ based on the last LS reply to RAN2, in addition, we include the term „together with the same DL-PRS-ID(s) corresponding to the group of TRP(s)“ to provide details that each group of TRPs is composed by a set of TRPs and these TRPs should be identified by DL-PRS-ID(s) during data collection and during inference. whitout this detail, the TRP-LocationInfo-Implicit-Element IE may be useless for the purpose that RAN1 introduced the concept of Associated ID for AIML positioning. In addition, we do not support the conclusion. |
| Qualcomm | We do not think this TP is necessary. |
| DOCOMO | Support this conclusion. |

Other

Issue 3-1. Pairing of Part A and Part B for model training

Huawei, HiSilicon (R1-2508497) have the following proposals on pairing of Part A and Part B for model training:

Proposal 4: In the request of Part B for Case 3a data collection, the gNB indicates to the LMF the time stamp for which Part B is requested. • The gNB can perform channel measurement to derive and store Part A according to the time instance for the requested Part B, and pair Part A with Part B after Part B is delivered. Proposal 5: In the request of Part B for Case 1 data collection, when Part A is generated by the UE and Part B is delivered from LMF to UE, to enable the pairing of Part A and Part B at UE side, the UE indicates to the LMF the time stamp for which Part B is requested. • The UE can perform channel measurement to derive and store Part A according to the time instance for the requested Part B, and pair Part A with Part B after Part B is delivered.

1st round discussion

In RAN1#122bis meeting, Huawei/HiSilicon (R1-2506934) also discussed the same issue. It was proposed that LMF indicate to gNB (Case 3a) or UE (Case 1) the anticipated time stamp in which Part B is expected to be generated. Based on companies’ feedback, majority (6 out 9) didn’t support it and the issue was not further discussed in RAN1#122bis.

For RAN1#123, Huawei/HiSilicon (R1-2508497) proposals are revised to: gNB (Case 3a) or UE (Case 1) can indicate to LMF the time stamp for which part B is requested.

The same issue has been discussed numerous times in RAN1 without convergence. In FL’s understanding, Huawei/HiSilicon (R1-2508497) proposals are not essential corrections for the maintenance phase.

Companies are invited to provide views on whether the new proposals by Huawei/HiSilicon should be adopted.

Question 3.1.1: Whether RAN1 should support the proposals for pairing Part A and Part B?

- Huawei/HiSilicon (R1-2508497): In the request of Part B for Case 3a or Case 1 data collection, gNB (Case 3a) or UE (Case 1) indicates to LMF the time stamp for which part B is requested.

| Company | Comments |
| --- | --- |
| HW/HiSi | Without any alignment of the time stamp of Part B provided by the LMF, it is not clear how the gNB may be able to pair a provided Part B with a Part A at the gNB, e.g., as Part B may be outdated. Hence, a provided Part B to the gNB may be useless. The alignment can be simply achieved by the gNB indicating the time stamp in the request of Part B sent to the LMF as also proposed by E/// and acknowledged by other companies in RAN3 at the last RAN3 meeting (see excerpt below from RAN3#129b Chair’s notes), but which may require a RAN1 agreement. |
| vivo | There’s time stamp already in Part A and Part B. It’s not clear to us what else is needed as pairing could already be handled by the training entity. |
| Ericsson | In our view such discussion was already taken and not agreed in earlier meetings. |
| Qualcomm | The concerns raised by HW can be resolved by requesting scheduled location signaling. Such optimizations can be handled by RAN2 |
| Nokia | Agree with the concern raised by HW, so as a compromise, we suggest the following wording in the proposal. Proposal (new Text): From RAN1 point of view, in the requested of Part B for Case 3a data collection, the gNB and LMF align the maximum delta in time domain for pairing Part A and Part B. |

2nd round discussion

Regarding Issue 3-1, vivo shared the measurement report in 38.455 (NRPPa), which contains a field “Positioning Data Collection Needed”. This can be taken into account in 2nd round discussion.

9.1.4.4          MEASUREMENT REPORT

This message is sent by the NG-RAN node to report positioning measurements for the target UE.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Response List |  | 1 |  |  | YES | reject |
| >TRP Measurement Response Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>TRP Measurement Result | M |  | 9.2.37 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| Positioning Data Collection Needed | O |  | 9.2.106 |  | YES | ignore |

The conclusion presented in 1st round is shown below for further discussion.

Conclusion 3.1.2

- For Rel-19 AI/ML positioning, there is no consensus in RAN1 to further discuss the enhancement for pairing Part A and Part B for data collection.

| Company | Comments |
| --- | --- |
| NEC | We still think Huawei’s proposal is benefit to assist the gNB/UE to pair the data with high accuracy/quality. Considering we are in the maintenance stage, we can also accommodate the majority view if there is a consensus among other companies that such optimization is unnecessary. |
| HW/HiSi | The alignment of the time stamp of Part A and the time stamp of Part B can be achieved for example by including a value of a time stamp in the IE Positioning Data Collection Needed, i.e., in the request of Part B sent from the gNB to the LMF: which was precisely the TP proposed in RAN3 in the last RAN3 meeting, but may require a RAN1 agreement. This enables the LMF to be aware of the time stamp for which the gNB would like to have a Part B. We do not view this as an optimization, as otherwise it is not clear how can it be guaranteed that a provided Part B may be able to be paired with a Part A at the gNB. |
| Nokia | We do not support the Conclusion. This type of conclusion precludes the discussion on proposals and TPs. |
| Qualcomm | We support the conclusion |

Issue 3-2. Ambiguity in UE behavior on using PRU information

Nokia (R1-2508948) section 3.1 provided the following text proposal to TS 38.214.

| Summary of change | Addition of description related to usage of PRU information under TS 38.214 section 5.1.6.5 for DL AI/ML positioning method. |
| --- | --- |
| Consequences if not approved | Ambiguity in UE behaviour on using PRU information for DL AI/ML positioning method. |
| Suggested text proposal | TS 38.214 V19.1.0 5.1.6.5 PRS reception procedure ================= text omitted=============================== Within each window indicated by NR-DL-PRS-MeasurementTimeWindowsConfig, the UE expects that the indicated DL PRS resource sets across all dl-PRS-IDs are from one DL PRS positioning frequency layer, and that the number of indicated DL PRS resource sets associated with each dl-PRS-ID are the same. The UE may be provided with NR-PRU-DL-Info which contains measurement(s) performed by a positioning reference unit (PRU) [20, TS 38.305], the timestamps associated with the measurement(s), and the location information of the PRU. For DL AI/ML positioning method, the UE may use the information contained in NR-PRU-DL-Info to indicate the content of monitoring outcome at least in the form of NR-DL-AIML-TargetDeviceErrorCauses [17, 37.355]. ================= text omitted=============================== |

1st round discussion

In FL’s understanding, the PRU information may be used for various purposes, including training data collection and generating monitoring outcome. However, it is not essential correction to describe the various purposes of PRU information in 38.214. Also, it is not clear why Nokia’s text proposal is only concerned with monitoring outcome, but not other purposes that PRU information can be used.

Companies can share views whether the TP by Nokia should be adopted by RAN1.

Question 3.2.1: Should RAN1 adopt the text proposal in Nokia (R1-2508948) section 3.1?

| Company | Comments |
| --- | --- |
| HW/HiSi | We do not agree with the text proposal, as the purpose of the provided PRU information does not need to be disclosed and the provided PRU information can also be used for other purposes. |
| Nokia | Answering FL concern, we are concerned on monitoring, because RAN1 made several agreements on monitoring. One of these agreements indicated that „at least an indication that the target UE cannot perform the Case 1 positioning method“, which was interpreted by RAN2 with the IE NR-DL-AIML-TargetDeviceErrorCauses. So, this TP is essentially following a RAN1 agreement done during the WI Rel. 19. To clarify the previous comments, the intention is to indicate that the UE may use the information received from PRU to do label-based monitoring and the outcome is reported by using the IE NR-DL-AIML-TargetDeviceErrorCauses |
| vivo | No |
| Ericsson | Not required in ran1 specifications. |
| Qualcomm | The purpose of PRU data can be left for implementation; there is no need to explicitly link it to a specific purpose. In addition, there was no explicit agreement with the proposed TP. |

2nd round discussion

The conclusion presented in 1st round is shown below for further discussion.

Conclusion 3.2.2

- For Rel-19 AI/ML positioning, there is no consensus in RAN1 to further discuss the usage of PRU information.

| Company | Comments |
| --- | --- |
| NEC | Agree with this conclusion. It can be captured in RAN2’s specification. |
| HW/HiSi | Ok |
| Nokia | We do not support the Conclusion. |
| Qualcomm | We support the conclusion |
| DOCOMO | Support. |

(Closed) Issue 2-4. APU

Google (R1-2508940) provided the following proposal on APU framework:

Proposal 4-1: Support the APU framework for UE-side AI/ML based poisoning • Support the UE to report which APU pool the AI/ML based positioning occupies • Support the UE to report the number of APUs for an inference operation for AI/ML based positioning • The priority for AI/ML based positioning is lower than the priority for CSI report

FL view

In RAN1#122bis, Google had submitted the same proposal on APU. In FL summary of RAN1#122bis (R1-2508009 [1]), companies shared their views on whether RAN1 should discuss the APU framework for Rel-19 AI/ML positioning. According to feedback received [1], no company supported to discuss this issue.

Hence, in FL’ view, this issue does not need to be further discussed for maintenance of Rel-19 AI/ML positioning.

Proposals for online sessions

Proposals for first online session

Conclusion 3.3

- No further discussion in RAN1 on APU framework for Rel-19 AI/ML positioning.

Reason: The same proposal has been presented in several meetings. Based on companies’ feedback in last meeting (RAN1#123bis), no other company supported to discuss this.

Conclusion 3.1

- For Rel-19 AI/ML positioning, there is no consensus in RAN1 to further discuss enhancement for pairing Part A and Part B for data collection.

Reason: The same issue has been discussed in several meetings and no agreement could be made. In this meeting,

- Two companies (HW/HiSi, Nokia) believe this issue need to be addressed. Nokia suggest a new proposal than HW/HiSi.

- Three companies (vivo, Ericsson, Qualcomm) do not agree.

Conclusion 3.2

- For Rel-19 AI/ML positioning, there is no consensus in RAN1 to further discuss the usage of PRU information.

Reason: 4 companies other than Nokia provided feedback. No other company supported Nokia’s TP.

Conclusion 2.2

- For Rel-19 AI/ML positioning, there is no consensus in RAN1 to further discuss the UE behavior on receiving explicit assistance data from the network.

Reason: 4 companies other than Nokia provided feedback. No other company supported Nokia’s TP.

Conclusion 2.4

- For Rel-19 AI/ML positioning, there is no consensus in RAN1 to further discuss correction of the TP endorsed in RAN1#122bis:

TS 38.214 V19.1.0 ##### 5.1.6.5 PRS reception procedure < Unchanged parts are omitted > If the UE assumes that the DL PRS is not transmitted from a serving cell, and if nr-PhysCellID is provided, and is the same as physical cell ID of the SS/PBCH block from a non-serving cell of the same band as the DL PRS, the UE may assume that the DL PRS and the SS/PBCH block are transmitted from the same non-serving cell. The UE may be configured by the network with TRP-LocationInfo-Implicit-Element [17, TS 37.355]. - If TRP-LocationInfo-Implicit-Element is provided, and if nr-PhysCellID or nr-CellGlobalID is provided with nr-AIML-AssociatedID, for TRP(s) in the given cell, the UE may assume that the geographical coordinates of the TRP(s) are consistent for a same nr-AIML-AssociatedID.

Reason: 3 companies other than Nokia provided feedback. No other company supported Nokia’s TP.

Conclusion 2.1

- For Rel-19 AI/ML positioning, there is no consensus in RAN1 to discuss whether/how the UE send a request to network to include TRP-LocationInfo-Implicit in NR-PRU-DL-Info.

Reason:

- 4 companies (InterDigital, Nokia, HW/HiSi, vivo) support to discuss this issue.

- 2 companies (Ericsson, Qualcomm) believe this is up to RAN2.

Note: HW/HiSi and vivo commented that Issue 2-1 and 2-3 can be discussed together.

Conclusion 2.3

- For Rel-19 AI/ML positioning, there is no consensus in RAN1 to discuss whether/how the UE report NR-TRP-LocationInfo-Implicit for which it expects to receive the associated NR-PRU-DL-Info.

Reason:

- 3 companies (Nokia, HW/HiSi, vivo) suggest to have a TP to 38.214.

- 3 companies (Ericsson, Qualcomm, ZTE) do not think a TP to 38.214 is needed. Reasons: It can be handled by RAN2 (Ericsson, Qualcomm); it can be handled by UE implementation (ZTE).

Note: HW/HiSi and vivo commented that Issue 2-1 and 2-3 can be discussed together.

Proposals for second online session

References

R1-2508009, “Summary #2: maintenance of specification support for positioning accuracy enhancement,” Moderator (Ericsson), RAN1#122bis, October 2025.

R1-2508497	Maintenance of Rel-19 AI/ML for air interface	Huawei, HiSilicon

R1-2508644	Maintenance on Rel-19 AI/ML	InterDigital, Inc.

R1-2508940	Maintenance for AI/ML for NR Air Interface	Google

R1-2508948	Maintenance on AI/ML for NR Air Interface	Nokia