3GPP TSG RAN WG1 #123			R1-250XXXX

Dallas, USA, Nov 17th – 21st, 2025

Agenda:	8.4

Source: 	Moderator (CMCC)

Title:	Summary #1 for maintenance on solutions for A-IoT in NR

Document for:	Discussion & Decision

Introduction

In RAN#108 meeting, the RAN1 work of the work item on solutions for Ambient IoT (Internet of Things) in NR [1] has been declared as completion, and the physical layer specification TS 38.291 j00 has been endorsed [2].

This contribution provides a summary of the submitted contributions on the text proposals for TS 38.291, remaining issues for discussion, and outcomes in RAN1#122bis meeting.

Online/offline proposals

TBD

Remaining issues

- [High] 3.1 Time drift during padding chips

- 3.1.1 Summary of inputs

In the last RAN1 meeting, the potential overlap of Msg1 time resources when X = 2 due to the time drift during postamble and padding chips has been discussed, as illustrated in the following figure.

Unfortunately, no consensus has been reached and the conclusion guided the group to make a decision in this RAN1 meeting. The latest FL proposal is also recapped in the following.

Conclusion When X = 2, for the potential overlap of the 1st and 2nd Msg1 time resource due to the time drift during postamble and padding chips: - RAN1#123 will make a decision on whether to specify a solution for this issue, or to leave it to be avoided or handled by reader implementation. - [H] Proposal 5.2-v3 When X = 2, for the potential overlap of the 1st and 2nd Msg1 time resource due to the time drift during postamble and padding chips, down-select to one of the following options: - Opt. 1: RAN1 does not pursue any optimization for this issue, which is avoided or handled by reader implementation. - Opt. 2: RAN1 pursues solution for this issue: - Opt. 2a: Device assumes that there is a transition edge at the end of R2D. - Opt. 2b: RAN1 changes the definition of TR→D so that the timeline determination refers to the end of PRDCH. Both Toffset,1 and Toffset,2 additionally include the time error during postmable and padding. - For the 1st Msg1 time resource, TR→D is updated as TR→D=Toffset+Tpadding+Tpostamble; - For the 2nd Msg1 time resource, TR→D is updated as TR→D=1.25Toffset+MchipD2RTchipD2R+Tpadding+Tpostamble - Send an LS to RAN4 to update the definition of TR→D. - Opt. 2c: RAN1 changes the definition of Toffset,2 to additionally include the time error during postamble and padding. For the 2nd Msg1 time resource, TR→D is updated as TR→D=1.25Toffset+MchipD2RTchipD2R+Tpadding+Tpostamble

Based on the submitted contributions in this meeting, 15 companies (vivo, CMCC, Huawei, NEC, CATT, Xiaomi, OPPO, Samsung, ZTE, LGE, Nokia, Ofinno, InterDigital, Qualcomm, Docomo) provide their analysis.

- No optimization (i.e., Opt. 1): 9 companies (vivo, CMCC, Huawei, CATT, Xiaomi, OPPO, ZTE, LGE, InterDigital) are proponents of Option 1. 1 company (Nokia) are fine with Option 1 or Option 2a. 1 company (Samsung) can live with Option 1 if Option 2a is not agreeable. Proponents provide the following observations/reasons:

  - vivo, CMCC and Xiaomi give very detailed analysis on whether or in which cases the two Msg1 time resources may overlap. It is observed that the overlap does not always occur, and even if there is overlap, the overlap affects only a few chips and is not significant. CATT also provide similar observations.

  - The minimum scaling factors satisfied the equation are αmin=0.222 and βmin=1.222. The factors α and β are extended to 0.25 and 1.25, respectively. It already leaves some margin.

  - Reader can perform reasonable scheduling to avoid the overlapping.

  - When overlap occurs, the result of this overlap may be the failure of up to two otherwise successful Msg1 transmissions. This error case is already handled by the existing random access procedure which can resolve the issue at the cost of efficiency loss.

To sum up, proponents of Option 1 believe that the issue is an efficiency optimization issue, which does not break the system. Therefore, no need to pursue any optimization and spec change.

- Need optimization (i.e., Opt. 2): 5 companies (NEC, Samsung (1st preference), Ofinno, Qualcomm, Docomo). 1 company (Nokia) are fine with Option 1 or Option 2a. Proponents of Option 2 think that RAN1 should find a solution for the overlap issue to not restrict reader implementation, and the solution should ensure that the timing offset avoids overlap for the worst case. Preferences on different solutions are summarized as follows:

  - Opt. 2a: 3 companies (NEC, Samsung, Nokia). Proponents of Option 2a think that it is the simplest solution, which establishes a clear timing reference (the end of R2D) for the device timeline determination without altering any existing parameters. However, 2 companies (Ofinno, Qualcomm) are against this option, as it is not aligned with the prior RAN1 agreement (i.e., padding is left to reader implementation), or it requires more RAN4 work.

  - Opt. 2b: 1 company (Qualcomm (1st preference)). The proponent of Option 2b thinks that it does not require RAN4 work, and it is a complete technical solution to fix both TR→D for 1st and 2nd Msg1 time resource and avoid overlapping issue. However, 2 companies (Samsung, Ofinno) are not fine with this option, as it changes fundamental timeline definitions and may require LS to RAN4.

  - Opt. 2c: 2 companies (Ofinno, Qualcomm (2nd preference)). Proponents of Option 2c think that it can resolve the issue with limited specification work and does not require RAN4 work.

- 3.1.2 Round 1 discussion

From the inputs, there is a clear majority to support Option 1. The analysis show that the overlap issue happens probabilistically and partially. Even a reader does not avoid the unreasonable scheduling and the overlap does occur, the result leads to only efficiency loss.

In contrast, 5 companies insist that RAN1 should solve the issue to not restrict reader implementation. But FL observes potential arguments to each solution:

- For Option 2a, although it may be a clear solution from RAN1 perspective, it against the previous RAN1 agreement that the padding pattern is up to reader implementation. In addition, to go with Option 2a, it is not aligned with RAN4 definition of TSFO, which is the length in microseconds from the last transition edge of PRDCH intended for the device in the corresponding R2D transmission to the starting time of the D2R transmission. Therefore, LS to RAN4 for the spec alignment may be required and more RAN4 work may be expected.

- For Option 2b/2c, they all solve the potential overlap but at the cost of increasing the overhead, which reduces the efficiency to improve the efficiency. In addition, for Option 2b, it enhances TR→D for both the 1st and 2nd Msg1 resources. But currently, TR→D for the 1st Msg1 is no smaller than 30 us and it has already considered the timing drift during postamble and padding, which introduces repeated overhead for the 1st Msg1 transmission.

To make progress, FL thinks that with three sub-options under Option 2 on the table, it would be difficult to move forward. Based on the inputs, at least FL can see a slight preference on Option 2a over Option 2b/2c. So, at the first stage, FL plans to collect views on two things, especially for proponents of Option 2:

- Arguments that RAN1 has to address this potential overlap issue in maintenance phase;

- Whether you can live with Option 2a, if RAN1 addresses this issue

Then, in the next stage, we can further try to down-select between Option 1 and Option 2a (if possible to converge).

      - [H] Question 3.1-v1

Please companies provide your views on the questions.

- Q1: Besides FL’s observation/summary, any further points that RAN1 has to resolve this issue in the maintenance phase.

- Q2: If RAN1 addresses this issue, can you live with Option 2a, i.e., Device assumes that there is a transition edge at the end of R2D.

| Company | Comments |
| --- | --- |

      - [H] Proposal 3.1-v1

When X = 2, for the potential overlap of the 1st and 2nd Msg1 time resource due to the time drift during postamble and padding chips, down-select to one of the following options:

- Opt. 1: RAN1 does not pursue any optimization for this issue, which is avoided or handled by reader implementation.

- Opt. 2: RAN1 pursues solution for this issue, and

  - Opt. 2a: Device assumes that there is a transition edge at the end of R2D.

| Company | Comments |
| --- | --- |

Text proposals for TS 38.291

- [High] 4.1 Device behavior during TR→D

- 4.1.1 Summary of inputs

In the last meeting, the issue that a device should not be required to monitor R2D messages when preparing a D2R transmission was raised by NEC and Ofinno, most companies believed that this is a valid issue to be addressed from specification perspective, and corresponding text proposals were discussed.

Unfortunately, the proposed text proposal has not been agreed and an agreement was made to specify the device behavior, with an FFS point to further resolve specification change in RAN1#123.

Agreement A device is not required to monitor R2D during TR→D for preparing D2R transmission - FFS in RAN1#123: how to capture this behaviour in the specifications

From reviewing the submitted contributions in this meeting, 12 companies (CMCC, Huawei, NEC, Xiaomi, OPPO, Samsung, ZTE, LGE, Nokia, Ofinno, InterDigital, Qualcomm) provide text proposals.

- Text proposal from R1-2508442, CMCC

In [6], CMCC analyzes conditions when a reader can possibly schedule a R2D message during TR→D, and explains that all the potential conditions are not allowed based on the current PHY and MAC specifications. Therefore, even it is explicitly agreed that device is not required to monitor R2D when preparing D2R transmission during TR→D, from specification perspective, it can simply say that a device is not required to monitor R2D during TR→D.

| Reasons for change | Capture the RAN1 agreement that a device is not required to monitor R2D during TR→D for preparing D2R transmission. |
| --- | --- |
| Summary of change | Add a bullet to Clause 7.2.3 that a device is not required to monitor R2D for a duration TR→D, as defined in Clause 7.1.2, after the end of chip χR2D= χendR2D when the device receives a corresponding R2D transmission ending in chip χendR2D=MchipR2D-1. |
| Consequences if not approved | The RAN1 agreement is not captured in the specification. |
| Text proposal | ### 7.2.3 Monitoring of R2D A device is not required to monitor R2D: - when performing D2R transmission - for a duration TD→Rmin, as specified in TS 38.191 [5], after the end of chip χD2R=χendD2R when the device performs a D2R transmission ending in chip χendD2R=MchipD2R-1. - for a duration TR→D, as defined in Clause 7.1.2, after the end of chip χR2D= χendR2D when the device receives a corresponding R2D transmission ending in chip χendR2D=MchipR2D-1. |

- Text proposal from R1-2508503, Huawei

In [7], Huawei proposes that the simplest way to capture the RAN1 agreement is to refer the calculation of duration and start/end points of TR→D fully to Clause 7.1.2, where each of the relevant transmissions is fully-defined in-context.

<Unchanged parts omitted> ### 7.2.3 Monitoring of R2D A device is not required to monitor R2D: - when performing D2R transmission - for the time between chip χR2D= χendR2D and chip χD2R=0 as determined in Clause 7.1.2. - for a duration TD→Rmin, as specified in TS 38.191 [5], after the end of chip χD2R=χendD2R when the device performs a D2R transmission ending in chip χendD2R=MchipD2R-1. <Unchanged parts omitted>

- Text proposal from R1-2508541, NEC

| Reasons for change | Base on the current TS, a D2R transmission is performing starting an amount of time TR→D after the end of chip χR2D= χendR2D. For the time duration TR→D, device cannot monitor R2D when it is preparing to perform D2R transmission. |
| --- | --- |
| Summary of change | Add clarified conditions that device is not required to monitor R2D for a time duration. |
| Consequences if not approved | Device cannot prepare D2R transmission when it needs to monitor R2D transmission during the time duration. |
| Text proposal | 7.2.3 Monitoring of R2D A device is not required to monitor R2D: - when performing D2R transmission - for a duration TR→D after the end of chip χR2D= χendR2D when the device is going to perform a D2R transmission starting an amount of time TR→D after the end of chip χR2D= χendR2D - for a duration TD→Rmin, as specified in TS 38.191 [5], after the end of chip χD2R=χendD2R when the device performs a D2R transmission ending in chip χendD2R=MchipD2R-1. |

- Text proposal from R1-2508662, Xiaomi

TP#1

### 7.2.3 Monitoring of R2D A device is not required to monitor R2D: - when performing D2R transmission - for a duration TD→Rmin, as specified in TS 38.191 [5], after the end of chip χD2R=χendD2R when the device performs a D2R transmission ending in chip χendD2R=MchipD2R-1. - when is preparing D2R transmission

TP#2

### 7.2.3 Monitoring of R2D A device is not required to monitor R2D: - when performing D2R transmission or is preparing D2R transmission - for a duration TD→Rmin, as specified in TS 38.191 [5], after the end of chip χD2R=χendD2R when the device performs a D2R transmission ending in chip χendD2R=MchipD2R-1.

- Text proposal from R1-2508709, OPPO

===========================Text Proposal 1 =============================== 7.2.3 Monitoring of R2D A device is not required to monitor R2D: - After receiving a PRDCH intended for the device in an R2D transmission ending in chip χendR2D=MchipR2D-1, until the device performs a corresponding D2R transmission with chip χD2R=0 starting an amount of time TR→D after the end of chip χR2D= χendR2D according to the configuration received from higher layers. - when performing D2R transmission - for a duration TD→Rmin, as specified in TS 38.191 [5], after the end of chip χD2R=χendD2R when the device performs a D2R transmission ending in chip χendD2R=MchipD2R-1. ==============================End of Text Proposal 1=======================

- Text proposal from R1-2508778, Samsung

In [12], Samsung points out that skipping monitoring R2D transmission during the entirety of TR->D was not the intent of Rel-19 design. The device should monitor R2D transmission as long as it can avoid missing any important R2D command, given Rel-19 A-IoT is based on asynchronous timing, and to provide flexibility to the reader scheduler.

--- (Text Proposal Start) --- 7.2.3 Monitoring of R2D A device, when performing or in preparation for D2R transmission, shall monitor other R2D transmissions according to [TS 38.293]. A device is not required to monitor R2D: - when performing or in preparation for D2R transmission - for a duration TD→Rmin, as specified in TS 38.191 [5], after the end of chip χD2R=χendD2R when the device performs a D2R transmission ending in chip χendD2R=MchipD2R-1. --- (Text Proposal End) ---

- Text proposal from R1-2508813, ZTE

7.2.3 Monitoring of R2D A device is not required to monitor R2D: - when it is going to perform D2R transmission or is performing D2R transmission. - for a duration TD→Rmin, as specified in TS 38.191 [5], after the end of chip χD2R=χendD2R when the device performs a D2R transmission ending in chip χendD2R=MchipD2R-1.

- Text proposal from R1-2508893, LGE

7.2.3 Monitoring of R2D A device is not required to monitor R2D: - when performing D2R transmission or preparing for D2R transmission - for a duration TD→Rmin, as specified in TS 38.191 [5], after the end of chip χD2R=χendD2R when the device performs a D2R transmission ending in chip χendD2R=MchipD2R-1.

- Text proposal from R1-2508996, Nokia

### 7.2.3 Monitoring of R2D A device is not required to monitor R2D: - when performing D2R transmission and a time duration for preparation of the D2R transmission - for a duration TD→Rmin, as specified in TS 38.191 [5], after the end of chip χD2R=χendD2R when the device performs a D2R transmission ending in chip χendD2R=MchipD2R-1.

- Text proposal from R1-2509024, Ofinno

| Reasons for change | Current specification states an AIoT device should monitor the R2D channel during the time offset from an R2D transmission until a corresponding D2R transmission (e.g., during TR→D). The AIoT device can’t both monitor R2D during TR→D and prepare for D2R transmission (e.g., due to Rx-Tx switching). |
| --- | --- |
| Summary of change | Add another condition to Section 7.2.3 that the device should not monitor R2D during the time offset from an R2D transmission to a corresponding D2R transmission (i.e., during TR→D). |
| Consequences if not approved | Infeasible device behaviour prior to a D2R transmission. |
| Text proposal | ### 7.2.3 Monitoring of R2D A device is not required to monitor R2D: - when performing D2R transmission and for a duration TR→D, as defined in Clause 7.1.2, after the end of chip χR2D= χendR2D when the device receives a corresponding R2D transmission ending in chip χendR2D=MchipR2D-1. - for a duration TD→Rmin, as specified in TS 38.191 [5], after the end of chip χD2R=χendD2R when the device performs a D2R transmission ending in chip χendD2R=MchipD2R-1. |

- Text proposal from R1-2509118, InterDigital

| Reasons for change | Current specification states an AIoT device should monitor the R2D channel during the time offset from an R2D transmission until a corresponding D2R transmission (e.g., during TR→D). The AIoT device can’t both monitor R2D and prepare for D2R transmission (e.g., due to Rx-Tx switching). |
| --- | --- |
| Summary of change | Add another condition to Section 7.2.3 that the device should not monitor R2D during the time offset from an R2D transmission to a corresponding D2R transmission (i.e., during TR→D). |
| Consequences if not approved | Device will be required to perform simultaneous R2D reception and D2R transmission. |
| Text proposal | ### 7.2.3 Monitoring of R2D A device is not required to monitor R2D: - when performing D2R transmission and for a duration TR→D, as defined in Clause 7.1.2, after the end of chip χR2D= χendR2D when the device receives a corresponding R2D transmission ending in chip χendR2D=MchipR2D-1. - for a duration TD→Rmin, as specified in TS 38.191 [5], after the end of chip χD2R=χendD2R when the device performs a D2R transmission ending in chip χendD2R=MchipD2R-1. |

- Text proposal from R1-2509202, Qualcomm

| Reasons for change | To capture the agreed behaviour in the specifications. Agreement A device is not required to monitor R2D during TR→D for preparing D2R transmission FFS in RAN1#123: how to capture this behaviour in the specifications |
| --- | --- |
| Summary of change | Clarify that the device is not required to monitor R2D until the D2R transmission (which includes the D2R transmission time and TR→D). |
| Consequences if not approved | The AIoT device behavior to monitor R2D is not aligned the RAN1 agreement. |
| Text proposal | ### 7.2.3 Monitoring of R2D A device is not required to monitor R2D: - after the end of a R2D transmission until the end of the corresponding when performing D2R transmission as defined in Clause 7.1.2. - for a duration TD→Rmin, as specified in TS 38.191 [5], after the end of chip χD2R=χendD2R when the device performs a D2R transmission ending in chip χendD2R=MchipD2R-1. |

- 4.1.2 Round 1 discussion

From the inputs, the text proposals can be categorized into different groups:

- For one categorization (Category 1), text proposals (e.g., Xiaomi, Samsung, ZTE) simply use similar wording in the agreement, i.e., preparing D2R transmission, or is going to perform D2R transmission. In FL’s personal view, the wording may not be clear enough from specification perspective.

- For the other categorization (Category 2), text proposals (e.g., CMCC, Huawei, OPPO) capture the device behavior that a device is not required to monitor R2D during TR→D. Most companies mention that it well reflects the RAN1 agreement. Regarding the argument in [12] that Rel-19 design allows a device to monitor R2D during TR→D when it finishes preparing the D2R transmission, it is analyzed by 1 source [6] that such reader scheduling does not exist in Rel-19 use cases.

FL observes that 1 source [12] explicit mention that Rel-19 design allows that a device can monitor R2D during TR→D when it finishes preparing the D2R transmission, and therefore has concerns on the text proposals from Category 2. However, also there is another 1 source [6] provides detailed discussion and believes that Rel-19 does not allow such kind of R2D scheduling during TR→D. Therefore, before directly discussing the wording of the text proposal, FL would like to ask companies to first provide views on the issue mentioned by [12], and whether it is valid in Rel-19 reader scheduling. Then, FL can further come up with the text proposal.

      - [H] Question 4.1-v1

Please companies provide your views on the question.

Do you think that Rel-19 design allows a device to monitor (or allows a reader to schedule) R2D during TR→D when a device finishes preparing the D2R transmission. If so, please provide at least one valid example of the scheduling case.

| Company | Comments |
| --- | --- |

- 4.2 Others

- 4.2.1 Summary of inputs

Based on the submitted contributions in this meeting, the following text proposals on other issues have been proposed.

- Text proposal from R1-2508541, NEC

| Reasons for change | When Security Parameter is present in CFA paging message, the D2R responding message cannot be determined in prior and CRC pre-calculation is not available. Additionally, the maximum size of the first D2R message for CFRA in this case is larger than 128 bits which is not aligned with the assumption in agreement. For this case, the time offset between the end of R2D transmission and the start of D2R transmission should be the same as for command case. |
| --- | --- |
| Summary of change | Clarify the case to determine time offset for CFA procedure is only for CFA procedure when Security Parameter field is absent. |
| Consequences if not approved | Time offset in specification is not reasonable when Security Parameter is present in CFA paging message. |
| Text proposal | 7.1.2 Device procedure for transmission time determination If the D2R transmission is for a Random ID message - if after chip χR2D= χendR2D there are potential access occasion(s), as defined in TS 38.391 [3], for the transmission which are earlier in time than the access occasion selected for the transmission - the device shall set TR→D=1.25Toffset+MchipD2RTchipD2R - otherwise - the device shall set TR→D=Toffset else if the D2R transmission corresponds to a R2D Random ID Response message or to a contention-free access procedure when Security Parameter field is absent as specified in TS 38.391 [3] - the device shall set TR→D=Toffset+Δcode where Δcode has the value given in Table 7.1.2-1 if Rcode indicates that channel coding is used, and TR→D=Toffset if no channel coding is used otherwise - the device shall set TR→D=Toffset+Δcode where Δcode has the value given in Table 7.1.2-2 if Rcode indicates that channel coding is used, and TR→D=Toffset if no channel coding is used |

- Text proposal from R1-2509171, Qualcomm

### 7.1.2 Device procedure for transmission time determination <Unchanged parts omitted> If the D2R transmission is for a Random ID message - if after chip χR2D= χendR2D there are potential access occasion(s), as defined in TS 38.391 [3], for the transmission which are earlier in time than the access occasion selected for the transmission - the device shall set TR→D=1.25Toffset+MchipD2RTchipD2R - otherwise - the device shall set TR→D=Toffset else if the D2R transmission corresponds to a R2D Random ID Response message or to a contention-free random access procedure - the device shall set TR→D=Toffset+Δcode where Δcode has the value given in Table 7.1.2-1 in case of NTBSD2R≤16 and in Table 7.1.2-2 in case of NTBSD2R>16 if Rcode indicates that channel coding is used, and TR→D=Toffset if no channel coding is used Table 7.1.2-1 Tchip' [μs]; Δcode [μs]. τ; 0. τ/2. τ/4. τ/8. τ/16. τ/32; τ. τ/64. τ/128. τ/192. otherwise - the device shall set TR→D=Toffset+Δcode where Δcode has the value given in Table 7.1.2-2 if Rcode indicates that channel coding is used, and TR→D=Toffset if no channel coding is used Table 7.1.2-2 D2R block size [bytes]; Δcode [μs]. NTBSD2R≤32; 2τ. 32<NTBSD2R≤64; 4τ. 64<NTBSD2R≤125; 8τ. <Unchanged parts omitted>

- 4.2.2 Round 1 discussion

FL thinks that the text proposal from NEC does not belong to RAN1 discussion, and the text proposal from Qualcomm (submitted under AI 5 and related to reply LS from RAN2 to SA2 on the extension of permanent ID) can be pending for now, and RAN1 can further discuss, if necessary, once SA2 reaches the agreement to extend the permanent ID.

Please companies to provide your views on whether the proposed text proposals are necessary.

| Company | Comments |
| --- | --- |

References

- RP-250796, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, 3GPP TSG RAN Meeting #107.

- TS 38.291, Ambient IoT Physical layer v19.0.0.

- TS 38.391, Ambient IoT Medium Access Control Protocol v19.0.0.

- TS 38.191, Ambient IoT device radio transmission and reception v19.0.0

- R1-2508407	Discussion on potential overlapping between TDMed Msg1 transmissions	vivo

- R1-2508442	Maintenance on Rel-19 Ambient IoT	CMCC

- R1-2508503	Maintenance on Rel-19 Ambient IoT	Huawei, HiSilicon

- R1-2508541	Maintenance on Solutions for Ambient IoT in NR	NEC

- R1-2508572	Remaining issues on Rel-19 A-IoT	CATT

- R1-2508662	Discussion on remaining issues for Ambient IoT in NR	Xiaomi

- R1-2508709	Maintenance on Rel-19 Ambient IoT	OPPO

- R1-2508778	Maintenance issues on Rel-19 A-IoT	Samsung

- R1-2508813	Discussion on remaining issues of Rel-19 Ambient IoT	ZTE Corporation, Sanechips

- R1-2508893	Maintenance on Rel-19 Ambient IoT	LG Electronics

- R1-2508996	Rel-19 AIoT (Ambient_IoT_Solutions) maintenance	Nokia

- R1-2509024	Maintenance of AIoT	Ofinno

- R1-2509118	Maintenance of Rel-19 Ambient IoT	InterDigital, Inc.

- R1-2509202	Maintenance on Ambient IoT	Qualcomm Incorporated

- R1-2509259	Maintenance on solutions for Ambient IoT in NR	NTT DOCOMO, INC.