3GPP TSG RAN WG1 #123		R1-250xxxx

Dallas, USA, Nov 17th – 21st, 2025

Source: 	Moderator (Apple)

Title:                     FL Summary #2 of NR Mobility enhancement Phase 4

Agenda item:	8.8

Document for:	Discussion and Decision

# 1 Introduction

This document summarizes the contributions made under the agenda item 8.8 related to the Rel-19 work item ‘NR mobility enhancements Phase 4’.

# 2. Contact people

For potential offline discussion, companies/delegates are encouraged to enter the contact information in the table below:

| Name | Company | Email address |
| --- | --- | --- |
| Hong He | Apple (FL) | hhe5@apple.com |
| Jae-Nam Shim | Ofinno | jshim@ofinno.com |

# 3.	CSI acquisition

## Issue 3-1: UE assumption for calculation of CQI/PMI/RI

In RAN1#122bis, the following was agreed and implemented in [11].

Agreement

- For the purpose of deriving the CQI for target candidate cell, the UE shall assume the following:

  - The parameters are derived from the initial BWP configuration of the target cell.

It was pointed out in [Ericsson, 1] that the UE assumption for calculating the LTM CQI report is ambiguous. When the UE is configured with multiple CSI-ReportConfigs in the serving cell, the current text in TS 38.214 does not clearly specify which paragraph(s) should be applied to generate the LTM CSI report. A TP is prepared to clarify the specification:

Text proposal # 3-1:

##### 5.2.2.5.1 UE assumptions for CQI/PMI/RI calculation If configured to report CQI index, in the CSI reference resource, or in each of the slot(s) associated with a CQI in the predicted CSI, as defined in Clause 5.2.1.4.2, the UE shall assume the following for the purpose of deriving the CQI index, and if also configured, for deriving PMI and RI: - The first 2 OFDM symbols are occupied by control signaling. - The number of PDSCH and DM-RS symbols is equal to 12. - The same bandwidth part subcarrier spacing configured as for the PDSCH reception. - For the LTM-CSI-ReportConfig, for a candidate cell given under LTM-Candidate, the PDSCH configuration corresponds to the initial BWP configuration of the candidate cell. - The bandwidth as configured for the corresponding CQI report. - The IAB-MT shall only assume the frequency resources as indicated by the DL TX power adjustment MAC CE, if indicated for the slot of the CSI reference resource by DL Tx Power Adjustment MAC CE as described in [10, TS 38.321]. - The reference resource uses the CP length and subcarrier spacing configured for PDSCH reception. - For the LTM-CSI-ReportConfig, for a candidate cell given under LTM-Candidate, the PDSCH configuration corresponds to the initial BWP configuration of the candidate cell. - No resource elements used by primary or secondary synchronization signals or PBCH. - Redundancy Version 0. - The ratio of PDSCH EPRE to CSI-RS EPRE is as given in Clause 5.2.2.3.1. - In addition, the IAB-MT shall apply the provided DL TX power adjustment, if indicated for the slot of the CSI reference resource by DL Tx Power Adjustment MAC CE as described in [10, TS 38.321]. - Assume no REs allocated for NZP CSI-RS and ZP CSI-RS. - Assume the same number of front-loaded DM-RS symbols as the maximum front-loaded symbols configured by the higher layer parameter maxLength in DMRS-DownlinkConfig. - For the LTM-CSI-ReportConfig, for a candidate cell given under LTM-Candidate, the DMRS-DownlinkConfig is provided in the PDSCH configuration corresponds to the initial BWP configuration of the candidate cell. - Assume the same number of additional DM-RS symbols as the additional symbols configured by the higher layer parameter dmrs-AdditionalPosition. - For the LTM-CSI-ReportConfig, for a candidate cell given under LTM-Candidate, the dmrs-AdditionalPosition is provided in the DMRS-DownlinkConfig in the PDSCH configuration corresponds to the initial BWP configuration of the candidate cell. - Assume the PDSCH symbols are not containing DM-RS. - Assume PRB bundling size of 2 PRBs. - The PDSCH transmission scheme where the UE may assume that PDSCH transmission would be performed with up to 8 transmission layers as defined in Clause 7.3.1.4 of [4, TS 38.211]. For CQI calculation, the UE should assume that PDSCH signals on antenna ports in the set [1000,…, 1000+ν-1] for ν layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…, 3000+PCSI-RS-1], as given by y3000(i)⋯y3000+PCSI-RS-1(i)=W(i)x0(i)⋯xν-1(i) Where x(i)=x(0)(i)...x(ν-1)(i)T is a vector of PDSCH symbols from the layer mapping defined in Clause 7.3.1.4 of [4, TS 38.211], PCSI-RS∈1,2,4,8,12,16,24,32,48,64,128 is the number of CSI-RS ports. If only one CSI-RS port is configured, W(i) is 1. If the higher layer parameter reportQuantity in CSI-ReportConfig or in LTM-CSI-ReportConfig for which the CQI is reported is set to either 'cri-RI-PMI-CQI' or 'cri-RI-LI-PMI-CQI', W(i) is the precoding matrix corresponding to the reported PMI applicable to x(i). If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-CQI', W(i) is the precoding matrix corresponding to the procedure described in Clause 5.2.1.4.2. If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-i1-CQI', W(i) is the precoding matrix corresponding to the reported i1 according to the procedure described in Clause 5.2.1.4.2. The corresponding PDSCH signals transmitted on antenna ports [3000,…,3000 + PCSI-RS - 1] would have a ratio of EPRE to CSI-RS EPRE equal to the ratio given in Clause 5.2.2.3.1. - For a CSI-ReportConfig or an LTM-CSI-ReportConfig, if a UE is configured with codebookType set to 'typeI-SinglePanel-r19' and with the higher layer parameter typeI-softScalingRank1-2-r19, for υ=1, and υ=2, if supported by UE capability, the UE can assume that the PDSCH signal for each layer mapped to a vector of group Gs(x1,x2) would have the same ratio of EPRE to CSI-RS EPRE for all K CSI-RS resources, equal to sx1,x22/υ times the powerControlOffset (in linear scale) of the respective CSI-RS resource, where Gs(x1,x2) and sx1,x22 are described in Clause 5.2.2.2.1a; otherwise, the UE can assume that the PDSCH signals for υ layers would have the same ratio of EPRE to CSI-RS EPRE for all K CSI-RS resources, equal to the powerControlOffset of the respective CSI-RS resource. - For a CSI-ReportConfig or an LTM-CSI-ReportConfig, if a UE is configured with 48, 64 and 128 antenna ports, obtained by aggregating K CSI-RS resources for channel measurement, and codebookType set to 'typeI-MultiPanel-r19', for CQI calculation, the UE should assume that PDSCH signals on antenna ports in the set [1000,…,1000+υ-1], for υ layers, would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…,3000+P-1] of each of the K CSI-RS resources, as given by y03000(i)⋮y03000+P-1(i)y13000(i)⋮y13000+P-1(i)⋮yK-13000(i)⋮yK-13000+P-1(i)=Wix0i⋮xυ-1i where W(i) is the precoding matrix corresponding to the procedure described in Clause 5.2.2.2.2a. A UE can assume that the PDSCH signals for υ layers would have the same ratio of EPRE to CSI-RS EPRE for all CSI-RS resources k=0,…,K-1, equal to the powerControlOffset of the respective CSI-RS resource. - For a CSI-ReportConfig , if a UE is configured with one or more SRS resource sets with higher layer parameter usage set to 'antennaSwitching', with a total of PSRS=6 or 8 ports across the resources intended for xT6R or xT8R, respectively, if the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-CQI' and the UE is configured with the higher layer parameter SRSPortGrouping-r19, the UE can assume that SRS port group 0 corresponds to codeword 0 and comprises the even PSRS/2 ports, and that SRS port group 1 corresponds to codeword 1 and comprises the odd PSRS/2 ports out of the total PSRS ports. The SRS ports are indexed in an ascending order according to SRS resource ID and port number within each SRS resource, for one SRS resource set, or according to SRS resource set ID, SRS resource ID in a set and port number within each SRS resource, for multiple aperiodic SRS resource sets. - For a CSI-ReportConfig , that contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, - If a sub-configuration indicates a CSI-RS antenna port subset using the higher layer bitmap parameter portSubsetIndicator or portSubsetIndicator-r19, as described in clause 5.2.1.4.2, for CQI calculation, antenna ports corresponding to all bits with value of 1 in portSubsetIndicator are mapped to consecutive antenna ports starting at CSI-RS antenna port 3000 in increasing order of the bit position in portSubsetIndicator. The UE should assume that PDSCH signals on antenna ports in the set [1000,…, 1000+ν-1] for ν layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000, …, 3000+P-1] T, as given by y3000i⋯y3000+P-1i=Wix0i⋯xν-1i where P corresponds to the number of bits with value 1 in the bitmap portSubsetIndicator and xi=[x0i…xν-1i]T , and Wi are as previously described in this Clause, and the corresponding PDSCH EPRE to CSI-RS EPRE is as previously defined in this Clause if the sub-configuration does not indicate a power offset powerOffset. - If a sub-configuration indicates a list of NZP CSI-RS resources, provided by nzp-CSI-RS-ResourceList and does not indicate a power offset powerOffset, for CQI calculation for the sub-configuration the UE follows the procedure previously described in this Clause. - If a sub-configuration indicates a power offset powerOffset, for CQI calculation, the UE shall assume the corresponding PDSCH signals transmitted on the antenna ports of a CSI-RS resource would have a ratio of EPRE to CSI-RS EPRE equal to the difference between powerControlOffset of the CSI-RS resource, given in Clause 5.2.2.3.1, and powerOffset, where the difference is expected to take one of the values that can be configured for powerControlOffset of the CSI-RS resource, given in Clause 5.2.2.3.1, and is also expected to take a value that is no larger than the value of powerControlOffset.

| Company | View/Positions (Please indicate Option) | Comments (If a TP is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| --- | --- | --- |
| vivo |  | The codebook type of LTM CSI report is typeI-SinglePanel, rather than typeI-SinglePanel-r19 and typeI-MultiPanel-r19. Therefore, “or an LTM-CSI-ReportConfig” should be removed as follows. For a CSI-ReportConfig or an LTM-CSI-ReportConfig, if a UE is configured with codebookType set to 'typeI-SinglePanel-r19' and with the higher layer parameter typeI-softScalingRank1-2-r19, for υ=1, and υ=2, if supported by UE capability, the UE can assume that the PDSCH signal for each layer mapped to a vector of group Gs(x1,x2) would have the same ratio of EPRE to CSI-RS EPRE for all K CSI-RS resources, equal to sx1,x22/υ times the powerControlOffset (in linear scale) of the respective CSI-RS resource, where Gs(x1,x2) and sx1,x22 are described in Clause 5.2.2.2.1a; otherwise, the UE can assume that the PDSCH signals for υ layers would have the same ratio of EPRE to CSI-RS EPRE for all K CSI-RS resources, equal to the powerControlOffset of the respective CSI-RS resource. - For a CSI-ReportConfig or an LTM-CSI-ReportConfig, if a UE is configured with 48, 64 and 128 antenna ports, obtained by aggregating K CSI-RS resources for channel measurement, and codebookType set to 'typeI-MultiPanel-r19', for CQI calculation, the UE should assume that PDSCH signals on antenna ports in the set [1000,…,1000+υ-1], for υ layers, would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…,3000+P-1] of each of the K CSI-RS resources, as given by y03000(i)⋮y03000+P-1(i)y13000(i)⋮y13000+P-1(i)⋮yK-13000(i)⋮yK-13000+P-1(i)=Wix0i⋮xυ-1i. |
| Nokia |  | Agree with vivo that the first two changes are not needed, as the parameters mentioned in the paragraph are not applicable for LTM-CSI-ReportConfig. We understand the motivation behind the overall proposal, but we also think these changes are not essential. When the specification states “For a UE configured with a CSI-ReportConfig”, the subsequent details should apply to that CSI-ReportConfig. However, if the majority of companies find this part of the specification ambiguous from an implementation perspective, we are fine with supporting the last two changes. |
| NTT DOCOMO |  | We support the proposal. |
| Google |  | Agree with vivo’s views on the first two changes. We also don’t see what’s difference between the last two changes and original wordings. |
| ZTE |  | For this TP, we have the following comments: Comment#1 for the change of the following paragraph: For early CSI in Rel-19 LTM, we only agree that reportQuantity is set to 'cri-RI-PMI-CQI', not including 'cri-RI-LI-PMI-CQI'. Where x(i)=x(0)(i)...x(ν-1)(i)T is a vector of PDSCH symbols from the layer mapping defined in Clause 7.3.1.4 of [4, TS 38.211], PCSI-RS∈1,2,4,8,12,16,24,32,48,64,128 is the number of CSI-RS ports. If only one CSI-RS port is configured, W(i) is 1. If the higher layer parameter reportQuantity in CSI-ReportConfig or in LTM-CSI-ReportConfig for which the CQI is reported is set to either 'cri-RI-PMI-CQI' or 'cri-RI-LI-PMI-CQI', W(i) is the precoding matrix corresponding to the reported PMI applicable to x(i). If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-CQI', W(i) is the precoding matrix corresponding to the procedure described in Clause 5.2.1.4.2. If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-i1-CQI', W(i) is the precoding matrix corresponding to the reported i1 according to the procedure described in Clause 5.2.1.4.2. The corresponding PDSCH signals transmitted on antenna ports [3000,…,3000 + PCSI-RS - 1] would have a ratio of EPRE to CSI-RS EPRE equal to the ratio given in Clause 5.2.2.3.1.. The following update is proposed for your reference. Where x(i)=x(0)(i)...x(ν-1)(i)T is a vector of PDSCH symbols from the layer mapping defined in Clause 7.3.1.4 of [4, TS 38.211], PCSI-RS∈1,2,4,8,12,16,24,32,48,64,128 is the number of CSI-RS ports. If only one CSI-RS port is configured, W(i) is 1. If the higher layer parameter reportQuantity in CSI-ReportConfig or in LTM-CSI-ReportConfig for which the CQI is reported is set to either 'cri-RI-PMI-CQI' or 'cri-RI-LI-PMI-CQI', or in LTM-CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-PMI-CQI', W(i) is the precoding matrix corresponding to the reported PMI applicable to x(i). If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-CQI', W(i) is the precoding matrix corresponding to the procedure described in Clause 5.2.1.4.2. If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-i1-CQI', W(i) is the precoding matrix corresponding to the reported i1 according to the procedure described in Clause 5.2.1.4.2. The corresponding PDSCH signals transmitted on antenna ports [3000,…,3000 + PCSI-RS - 1] would have a ratio of EPRE to CSI-RS EPRE equal to the ratio given in Clause 5.2.2.3.1.. Comment#2 for the changes of the following two paragraphs: Firstly, we think that typeI-SinglePanel supported for early CSI of Rel-19 LTM is applicable to both Rel-15 and Rel-19. with this consideration, the change in the follow first paragraph seems to be meaningful. Even so, we still have one question: whether the PDSCH-related configuration/assumption for CQI/PMI/RI calculation for the current case still needs to be clarified as being determined based on the initial BWP of the candidate cell. Last one for the change in the follow second paragraph, we think that such change is not needed since 'typeI-MultiPanel-r19' or “typeI-MultiPanel” is not supported for Rel-19 LTM. For a CSI-ReportConfig or an LTM-CSI-ReportConfig, if a UE is configured with codebookType set to 'typeI-SinglePanel-r19' and with the higher layer parameter typeI-softScalingRank1-2-r19, for υ=1, and υ=2, if supported by UE capability, the UE can assume that the PDSCH signal for each layer mapped to a vector of group Gs(x1,x2) would have the same ratio of EPRE to CSI-RS EPRE for all K CSI-RS resources, equal to sx1,x22/υ times the powerControlOffset (in linear scale) of the respective CSI-RS resource, where Gs(x1,x2) and sx1,x22 are described in Clause 5.2.2.2.1a; otherwise, the UE can assume that the PDSCH signals for υ layers would have the same ratio of EPRE to CSI-RS EPRE for all K CSI-RS resources, equal to the powerControlOffset of the respective CSI-RS resource. - For a CSI-ReportConfig or an LTM-CSI-ReportConfig, if a UE is configured with 48, 64 and 128 antenna ports, obtained by aggregating K CSI-RS resources for channel measurement, and codebookType set to 'typeI-MultiPanel-r19', for CQI calculation, the UE should assume that PDSCH signals on antenna ports in the set [1000,…,1000+υ-1], for υ layers, would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…,3000+P-1] of each of the K CSI-RS resources, as given by y03000(i)⋮y03000+P-1(i)y13000(i)⋮y13000+P-1(i)⋮yK-13000(i)⋮yK-13000+P-1(i)=Wix0i⋮xυ-1i where W(i) is the precoding matrix corresponding to the procedure described in Clause 5.2.2.2.2a. A UE can assume that the PDSCH signals for υ layers would have the same ratio of EPRE to CSI-RS EPRE for all CSI-RS resources k=0,…,K-1, equal to the powerControlOffset of the respective CSI-RS resource.. |
| Samsung |  | For the first two proposed changes, we share similar views to vivo; For the last two proposed changes, we do not think they are needed – there should not be any ambiguities from the original descriptions in 214. |
| CATT |  | We have similar views as that of vivo for the first two changes. Accordingly, the last two changes seem not necessary. |
| Huawei, Hislicon |  | For the first two proposals by adding LTM-CSI-ReportConfig, we shared the view with vivo. For the last proposal of “For a CSI-ReportConfig , ”, we do not see the difference and prefer not to change. For the 3rd proposal, a LTM UE will be not configured with SRS on candidate cell. there is no ambiguity. |
| Ericsson |  | Thanks for the comments. We all know how early CSI would work, and here we need to forget that, and just read the specification. After performing the RRC reconfiguration, the UE will look at its configuration and determine which assumptions to make when calculating CQI. Here, the initial BWP will contain both a CSI-ReportConfig and an LTM-CSI-ReportConfig, and the UE would go through each paragraph in 5.2.2.5.1, and apply the assumption where its configuration matches the condition in the paragraph. Since we have now introduced special handling for LTM in a few places, the natural assumption is that in all other places, we should apply exactly the same assumptions for early CSI as for normal CSI. Again – here we must forget what we know, and just read the spec. On vivo’s comment, the UE can be configured with 'typeI-SinglePanel-r19' to perform early CSI reporting. The running CR states: LTM-CodebookConfig-r19 ::= CHOICE { twoToThirtyTwoPorts CodebookConfig, moreThanThirtyTwoPorts CodebookConfig-r19, ... } So the UE can be configured with CodebookConfig-r19, and hence 'typeI-SinglePanel-r19. So the first change seems needed, just as ZTE remarked. The second change in the draft CR is not needed – this only applies for CSI report config – it is not possible to configure the UE with multipanel codebook for early CSI. @Nokia: Happy to see that you understand the motivation of the clarification. Then I am a little surprised that you make the following interpretationA: For a UE configured with a CSI-ReportConfig”, the subsequent details should apply to that CSI-ReportConfig. The statement says “For a UE”, not “For a CSI-ReportConfig”. We should remember that the condition that the UE is configured with a CSI-ReportConfig is not enough to apply the condition in the paragraph: the CQI should also be calculated for that CSI-ReportConfig. @ZTE: It is true that reportQuantity cannot be set to 'cri-RI-LI-PMI-CQI' for early CSI, but I don’t think that matters here. @Huawei: In fact, the UE can very well be configured with SRS on that candidate cell, but even if it is, the UE should not use that when calculating the early CSI. The current specification states that it should apply the corresponding paragraph. |

### [2nd round]

Text proposal # 3-1:

##### 5.2.2.5.1 UE assumptions for CQI/PMI/RI calculation < Unchanged parts are omitted > - For a CSI-ReportConfig or an LTM-CSI-ReportConfig, if a UE is configured with codebookType set to 'typeI-SinglePanel-r19' and with the higher layer parameter typeI-softScalingRank1-2-r19, for υ=1, and υ=2, if supported by UE capability, the UE can assume that the PDSCH signal for each layer mapped to a vector of group Gs(x1,x2) would have the same ratio of EPRE to CSI-RS EPRE for all K CSI-RS resources, equal to sx1,x22/υ times the powerControlOffset (in linear scale) of the respective CSI-RS resource, where Gs(x1,x2) and sx1,x22 are described in Clause 5.2.2.2.1a; otherwise, the UE can assume that the PDSCH signals for υ layers would have the same ratio of EPRE to CSI-RS EPRE for all K CSI-RS resources, equal to the powerControlOffset of the respective CSI-RS resource. - For a CSI-ReportConfig, if a UE is configured with 48, 64 and 128 antenna ports, obtained by aggregating K CSI-RS resources for channel measurement, and codebookType set to 'typeI-MultiPanel-r19', for CQI calculation, the UE should assume that PDSCH signals on antenna ports in the set [1000,…,1000+υ-1], for υ layers, would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…,3000+P-1] of each of the K CSI-RS resources, as given by y03000(i)⋮y03000+P-1(i)y13000(i)⋮y13000+P-1(i)⋮yK-13000(i)⋮yK-13000+P-1(i)=Wix0i⋮xυ-1i where W(i) is the precoding matrix corresponding to the procedure described in Clause 5.2.2.2.2a. A UE can assume that the PDSCH signals for υ layers would have the same ratio of EPRE to CSI-RS EPRE for all CSI-RS resources k=0,…,K-1, equal to the powerControlOffset of the respective CSI-RS resource. - For a CSI-ReportConfig, if a UE is configured with one or more SRS resource sets with higher layer parameter usage set to 'antennaSwitching', with a total of PSRS=6 or 8 ports across the resources intended for xT6R or xT8R, respectively, if the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-CQI' and the UE is configured with the higher layer parameter SRSPortGrouping-r19, the UE can assume that SRS port group 0 corresponds to codeword 0 and comprises the even PSRS/2 ports, and that SRS port group 1 corresponds to codeword 1 and comprises the odd PSRS/2 ports out of the total PSRS ports. The SRS ports are indexed in an ascending order according to SRS resource ID and port number within each SRS resource, for one SRS resource set, or according to SRS resource set ID, SRS resource ID in a set and port number within each SRS resource, for multiple aperiodic SRS resource sets. - For a CSI-ReportConfig that contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, - If a sub-configuration indicates a CSI-RS antenna port subset using the higher layer bitmap parameter portSubsetIndicator or portSubsetIndicator-r19, as described in clause 5.2.1.4.2, for CQI calculation, antenna ports corresponding to all bits with value of 1 in portSubsetIndicator are mapped to consecutive antenna ports starting at CSI-RS antenna port 3000 in increasing order of the bit position in portSubsetIndicator. The UE should assume that PDSCH signals on antenna ports in the set [1000,…, 1000+ν-1] for ν layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000, …, 3000+P-1] T, as given by y3000i⋯y3000+P-1i=Wix0i⋯xν-1i where P corresponds to the number of bits with value 1 in the bitmap portSubsetIndicator and xi=[x0i…xν-1i]T , and Wi are as previously described in this Clause, and the corresponding PDSCH EPRE to CSI-RS EPRE is as previously defined in this Clause if the sub-configuration does not indicate a power offset powerOffset. - If a sub-configuration indicates a list of NZP CSI-RS resources, provided by nzp-CSI-RS-ResourceList and does not indicate a power offset powerOffset, for CQI calculation for the sub-configuration the UE follows the procedure previously described in this Clause. - If a sub-configuration indicates a power offset powerOffset, for CQI calculation, the UE shall assume the corresponding PDSCH signals transmitted on the antenna ports of a CSI-RS resource would have a ratio of EPRE to CSI-RS EPRE equal to the difference between powerControlOffset of the CSI-RS resource, given in Clause 5.2.2.3.1, and powerOffset, where the difference is expected to take one of the values that can be configured for powerControlOffset of the CSI-RS resource, given in Clause 5.2.2.3.1, and is also expected to take a value that is no larger than the value of powerControlOffset. < Unchanged parts are omitted >

| Company | View/Positions (Please indicate Option) | Comments (If a TP is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| --- | --- | --- |

## Issue 3-2: SP CSI-RS Deactivation/Release

When the semi-persistent NZP CSI-RS resources and CSI-IM resources are deactivated was discussed in RAN2#131 and the following agreement has been achieved [12].

SP CSI-RS resource for target cell is deactivated after CSI reporting in the first UL transmission. How to capture it in MAC is up to MAC CR rapporteur.

The TP 3-2 was proposed in [2] to implement this RAN2 agreement

Text proposal # 3-2:

Reasons of change: When the semi-persistent CSI-RS resources and CSI-IM are released/deactivated after CSC is not defined in RAN1 specification. Summary of change: Define when the semi-persistent CSI-RS and CSI-IM resources are released/deactivated after CSC. Consequences if not approved: When the semi-persistent CSI-RS and CSI-IM resources are released/deactivated after CSC is unclear.

### 5.2.4a CSI Reporting for LTM < Unchanged parts are omitted > After a UE receives an LTM Cell Switch Command MAC CE [10, TS 38.321] providing a candidate cell (given by Target Configuration ID field), and a [ltm-eCSI-ReportConfig] is configured for the candidate cell, the UE can measure corresponding NZP CSI-RS resources and shall transmit a CSI report to the candidate cell. The semi-persistent NZP CSI-RS resources and CSI-IM resources associated with the ltm-CSI-ReportConfig corresponding to the candidate cell are deactivated after UE transmits CSI report. For RACH-less LTM cell switch [23, TS 38.300], the UE shall transmit the CSI report to the candidate cell using the first PUSCH corresponding to a dynamic grant or a configured grant [6, TS 38.213]. For RACH-based LTM cell switch using a contention-free random access procedure [23, TS 38.300], the UE shall transmit the CSI report to the candidate cell using the PUSCH scheduled by the RAR UL grant or MsgA PUSCH. For RACH-based LTM cell switch using a contention-based random access procedure [23, TS 38.300], the UE shall transmit the CSI report to the candidate cell using the first PUSCH corresponding to a dynamic grant or a configured grant after the HARQ-ACK transmission corresponding to Msg4 or MsgB. If a valid CSI is not available, the UE shall transmit a CSI report which contains a CQI corresponding to the lowest CQI index. [Depending on the UE capability] the UE may start measuring corresponding NZP CSI-RS resources before receiving the LTM Cell Switch Command MAC CE [10, TS 38.321]. < Unchanged parts are omitted >

| Company | View/Positions (Please indicate Option) | Comments (If a TP is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| --- | --- | --- |
| vivo | Yes |  |
| Ericsson | Yes | We prefer to clarify that it is the UE that deactivates the SP-CSI-RS and CSI-IM: After transmitting the CSI report, ,the UE deactivates the semi-persistent NZP CSI-RS resources and CSI-IM resources associated with the ltm-CSI-ReportConfig corresponding to the candidate cell. |
| Nokia | Yes, but | (Minor comment) It appears that the other text is taken from an older version (V19.0.0). For completeness, it would be appropriate to reflect the other RAN2 agreement regarding the deactivation of CSI-RSs for the candidate cells other than the target cell as well. I.e., ### 5.2.4a CSI Reporting for LTM < Unchanged parts are omitted > If a valid CSI is not available, the UE shall transmit a CSI report which contains a CQI corresponding to the lowest CQI index. Depending on the UE capability, the UE may measure NZP CSI-RS resources and CSI-IM resources if configured corresponding to a ltm-CSI-ReportConfig before receiving the LTM Cell Switch Command MAC CE [10, TS 38.321]. After receiving the LTM Cell Switch Command MAC CE [10, TS 38.321], the semi-persistent NZP CSI-RS resources and CSI-IM resources associated with the ltm-CSI-ReportConfig corresponding to the candidate cells, except the target cell, are deactivated. |
| NTT DOCOMO | Yes |  |
| Google | Yes |  |
| ZTE | No | We don’t need to duplicate it in RAN1 since it has been reflected in RAN2 spec. |
| Samsung | No | We do not think that it is essential to reflect it in RAN1 specification. |
| CATT | No | Not needed in RAN1 spec. |
| Moderator |  | @ZTE, @Samsung @CATT Could you clarify where this is captured in the RAN2 specification, given that measurement resource activation/deactivation is defined in the RAN1 specification? Why do you think it is not needed? @Ericsson, ‘the deactivation’ is not only UE behavior, instead aligned behavior between UE and network. The proposed change maybe understood as ‘UE autonmous behavior’, which is not intended. Also, I checked the 3GPP spec, no such behavior is defined/used. @All, please check whether Nokia’s version is acceptable to move forward. ### 5.2.4a CSI Reporting for LTM < Unchanged parts are omitted > If a valid CSI is not available, the UE shall transmit a CSI report which contains a CQI corresponding to the lowest CQI index. Depending on the UE capability, the UE may measure NZP CSI-RS resources and CSI-IM resources if configured corresponding to a ltm-CSI-ReportConfig before receiving the LTM Cell Switch Command MAC CE [10, TS 38.321]. After receiving the LTM Cell Switch Command MAC CE [10, TS 38.321], the semi-persistent NZP CSI-RS resources and CSI-IM resources associated with the ltm-CSI-ReportConfig corresponding to the candidate cells, except the target cell, are deactivated. |
| Huawei, HiSilicon | Yes | Also fine with the revision from Ericsson and Nokia |

## Issue 3-3: Resource Setting for Early CSI Report

[CATT, 4] proposed to clarify the interference measurement resources when two resource settings are configured in one LTM CSI report setting.

Text proposal # 3-3:

| Reason for change: | For CSI acquisition in LTM, it is specified that resource setting for channel measurement contains configuration of a NZP CSI-RS Resource set which comprises a list of NZP CSI-RS resources associated with a list of candidate cells. When two resource settings are configured for one LTM CSI report setting, the same design should be applied to CSI-IM resources as well to ensure the NZP CSI-RS resource for channel measurement and CSI-IM for interference measurement for a candidate cell are one-to-one mapping. This part is missing in section 5.2.4a of TS38.214. |
| --- | --- |
| Summary of change: | In section 5.2.4a of TS38.214, complement that when two resource settings are configured for CSI acquisition in LTM, the resource setting for interference measurement contains a CSI-IM Resource set which comprises a list of CSI-IM resources associated with a list of candidate cells. Remove the bracket of CSI-IM resources for the following sentence: When two Resource Settings are configured, the first Resource Setting (given by higher layer parameter ltm-ResourcesForChannelMeasurement) provides a list of NZP CSI-RS resources for channel measurement, and the second Resource Setting (given by higher layer parameter ltm-ResourceForInterferenceMeasurements), provides a list of [CSI-IM resources] for interference measurement. The UE is not expected to be configured with more than 128 NZP CSI-RS ports in the CSI-RS resource set contained within the Resource Setting. |
| Consequences if not approved: | For early CSI acquisition in LTM, the interference measurement resources are not clear to UE if two resource settings are configured in one LTM CSI report setting. |

---------------------------- Start of text proposal to TS 38.214 v19.1.0 -----------------------------

5.2.4a 	CSI Reporting for LTM

A UE configured with LTM-Config can be provided configurations for CSI acquisition, by up to one Reporting Setting, ltm-CSI-ReportConfig, for a candidate cell. Each Reporting Setting ltm-CSI-ReportConfig is associated with either one or two Resource Settings

-	When one Resource Setting (given by higher layer parameter ltm-ResourcesForChannelMeasurement) is configured, it provides a list of NZP CSI-RS resources for both channel and interference measurements. The UE is not expected to be configured with more than 128 NZP CSI-RS ports in the CSI-RS resource set contained within the Resource Setting.

-	When two Resource Settings are configured, the first Resource Setting (given by higher layer parameter ltm-ResourcesForChannelMeasurement) provides a list of NZP CSI-RS resources for channel measurement, and the second Resource Setting (given by higher layer parameter ltm-ResourceForInterferenceMeasurements), provides a list of [CSI-IM resources] for interference measurement. The UE is not expected to be configured with more than 128 NZP CSI-RS ports in the CSI-RS resource set contained within the Resource Setting.

The Resource Setting given by higher layer parameter ltm-ResourcesForChannelMeasurement, LTM-CSI-ResourceConfig, contains configuration of a ltm-NZP-CSI-RS-ResourceSet which comprises of a list of Z ≥ 1 NZP CSI-RS resource indices (given by ltm-CSI-RS-ResourceList) and a list of Z LTM-CandidateIds (given by ltm-CandidateIdList) referring to candidate cells associated with the NZP CSI-RS resource indices. For CSI acquisition associated with a Reporting Setting, ltm-CSI-ReportConfig, the UE is expected to measure the NZP-CSI-RS resources in ltm-CSI-RS-ResourceList associated with the LTM-CandidateId that is equal to the LTM-CandidateId of the LTM-Candidate under which the Reporting Setting is configured.

The Resource Setting given by higher layer parameter ltm-ResourcesForInterferenceMeasurement, LTM-CSI-ResourceConfig, contains configuration of a ltm-CSI-IM-ResourceSet which comprises of a list of Z ≥ 1 CSI-IM resource indices (given by ltm-CSI-IM-ResourceList) and a list of Z LTM-CandidateIds (given by ltm-CandidateIdList) referring to candidate cells associated with the CSI-IM resource indices. For CSI acquisition associated with a Reporting Setting, ltm-CSI-ReportConfig, the UE is expected to measure the CSI-IM resources in ltm-CSI-IM-ResourceList associated with the LTM-CandidateId that is equal to the LTM-CandidateId of the LTM-Candidate under which the Reporting Setting is configured.

The UE shall expect the following configuration provided by ltm-CSI-ReportConfig:

-	For the frequency granularity of the CSI report, the CQI format indicator is Wideband CQI.

-	For the frequency granularity of the CSI report, the PMI format indicator is Wideband PMI.

-	The codebook type is typeI-SinglePanel.

-	The reportQuantity is set to ‘cri-RI-PMI-CQI’.

After a UE receives an LTM Cell Switch Command MAC CE [10, TS 38.321] providing a candidate cell (given by Target Configuration ID field), and a ltm-CSI-ReportConfig is configured for the candidate cell, the UE can measure corresponding NZP CSI-RS resources and CSI-IM resources if configured, and shall transmit a CSI report to the candidate cell.

For RACH-less LTM cell switch [23, TS 38.300], the UE shall transmit the CSI report to the candidate cell using the first PUSCH corresponding to a dynamic grant or a configured grant [6, TS 38.213].

For RACH-based LTM cell switch using a contention-free random access procedure [23, TS 38.300], the UE shall transmit the CSI report to the candidate cell using the PUSCH scheduled by the RAR UL grant or MsgA PUSCH.

For RACH-based LTM cell switch using a contention-based random access procedure [23, TS 38.300], the UE shall transmit the CSI report to the candidate cell using the first PUSCH corresponding to a dynamic grant or a configured grant after the HARQ-ACK transmission corresponding to Msg4 or MsgB.

If a valid CSI is not available, the UE shall transmit a CSI report which contains a CQI corresponding to the lowest CQI index. Depending on the UE capability, the UE may measure NZP CSI-RS resources and CSI-IM resources if configured corresponding to a ltm-CSI-ReportConfig before receiving the LTM Cell Switch Command MAC CE [10, TS 38.321].

----------------------------- End of text proposal to TS 38.214 v19.1.0 ---------------------------------

| Company | View/Positions (Please indicate Option) | Comments (If a TP is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| --- | --- | --- |
| vivo | No | According to TS38.331-j00, the CSI Resource Setting for interference measurement only includes CSI-IM resources associated with the candidate cell where the LTM-CSI-ReportConfig associated with the CSI Resource Setting is configured. UE can obtain CSI by measuring NZP-CSI-RS resources and CSI-IM resources associated with the candidate cell where the LTM-CSI-ReportConfig is configured. Therefore, the current specification is clear and the TP is unnecessary. |
| Ericsson | No | Agree with vivo: the TP does not match what is in RRC |
| Nokia | No | The first change on the removal of backet from [CSI-IM resources] is OK. The second change seems not needed. Unfortunately, RAN2 agreed to use a different design for ltm-NZP-CSI-RS-ResourceSet and ltm-CSI-IM-ResourceSet. ltm-CSI-IM-ResourceSet only contains CSI-IM resourced from one candidate cell. |
| NTT DOCOMO | No | We share the same view as above companies. |
| Ofinno | No | Same view with vivo. |
| Google | No |  |
| ZTE | No | Same view as the above companies. |
| Samsung | Open | The interference measurement behaviors/assumptions for when one resource setting and two resource settings are provided are completely different based on RAN1 and RAN2 designs; it was RAN1 to first agree to introduce CSI-IM, but it is unclear how RAN2 understood it assuming potentially more than one candidate cell for interference measurement – we prefer to at least discuss this issue |
| CATT | Yes | Since ltm-CSI-IM-ResourceSet only contains CSI-IM resourced from one candidate cell, for SP CSI-RS/CSI-IM, multiple MAC-CEs are needed to activate all the CSI-IMs of different candidate cells, which seems not reasonable. We prefer to discuss this issue. |
| Huawei, HiSilicon | No | Agree with the view from companies above. |

## Issue 3-4: Active CSI resource counting for Candidate Cell

A TP was proposed in [Samsung, 5] to correct the active resource counting before receiving the LTM CSC to align the intended UE’s operation(s).

Text proposal # 3-4:

38.214-j10 5.2.1.6 CSI processing criteria <Unchanged parts are omitted> For a report setting ltm-CSI-ReportConfig configured with reportQuantity is set to 'cri-RI-PMI-CQI': - If the UE is capable of performing CSI measurement for CSI acquisition for candidate cell(s) before receiving the LTM Cell Switch Command MAC CE [10, TS 38.321]: - For a periodic CSI-RS, the CSI-RS resource and CSI-RS ports within the CSI-RS resource are counted as active in a duration of time starting when the periodic CSI-RS resource is configured by higher layer signaling until, and ending at the end of when the reception of the LTM cell switch command MAC CE is applied or a RRC reconfiguration message when the periodic CSI-RS configuration is released, whichever occurs first. - For a semi-persistent CSI-RS, the CSI-RS resource and CSI-RS ports within the CSI-RS resource are counted as active in a duration of time starting from the end of when the activation command MAC CE is applied until, and ending at the end of when the reception of the LTM cell switch command MAC CE or a deactivation command MAC CE is applied, whichever occurs first. - If the UE is only capable of performing CSI measurement for CSI acquisition for a candidate cell (given by Target Configuration ID field in the LTM Cell Switch Command MAC CE) after receiving the LTM Cell Switch Command MAC CE [10, TS 38.321], no CSI-RS resource and CSI-RS ports are counted as active before the reception of the LTM Cell Switch Command MAC CE [10, TS 38.321]. <Unchanged parts are omitted>

| Company | View/Positions (Please indicate Option) | Comments (If a TP is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| --- | --- | --- |
| Ericsson | No? | No functional difference from the text currently in 38.214-j10 |
| Nokia |  | It seems we already discussed this during the offline session on this topic. Seems not essential. |
| NTT DOCOMO |  | We are ok, but it seems not essential. |
| Google | No | Current SPEC is clear. |
| ZTE |  | We are open to this issue. |
| Samsung | Yes | We would like to first note that the proposed changes are different from previous discussions – previously, it was proposed to remove the legacy operation(s)/condition(s). Furthermore, the current descriptions in 214 are not correct regarding legacy operation (so function-wise, there are differences): - For MAC CE command, it should be at the end of when it is APPLIED not when it is received - For periodic CSI-RS, it should be when the corresponding CSI-RS configuration is released (exactly the same as another part in 214 for legacy operation); RRC reconfiguration message is not correct, which can mean reconfiguring/releasing anything but periodic CSI-RS configuration. |
| CATT | No | Not necessary. |
| Huawei, HiSilicon |  | No functional difference. May be can leave editor to choose how to write? |
| Samsung2 |  | The legacy descriptions for periodic and semi-persistent CSI-RS are copied (and highlighted) below. As we explained already, function-wise they are different from the original wording for the corresponding active resource counting in 214 for LTM CSI acquisition: “ In any slot, the UE is not expected to have more active CSI-RS ports or active CSI-RS resources in active BWPs than reported as capability. NZP CSI-RS resource is active in a duration of time defined as follows. For aperiodic CSI-RS, (excluding the case in which the corresponding aperiodic CSI Reporting Setting is configured with the higher layer parameter [RRC_name-r19] and is linked to another aperiodic CSI Reporting Setting with reportQuantity-r19 set to 'csi-pai-r19'), starting from the end of the PDCCH containing the request and ending at the end of the scheduled PUSCH containing the report associated with this aperiodic CSI-RS. When the PDCCH candidates are associated with a search space set configured with searchSpaceLinkingId, for the purpose of determining the NZP CSI-RS resource active duration, the PDCCH candidate that ends later in time among the two linked PDCCH candidates is used. For semi-persistent CSI-RS, starting from the end of when the activation command is applied, and ending at the end of when the deactivation command is applied. For periodic CSI-RS, starting when the periodic CSI-RS is configured by higher layer signalling, and ending when the periodic CSI-RS configuration is released. ” |

# 4. Conditional LTM

## Issue 4-1: TCI-State determination for C-LTM

In RAN1 122Bis meeting, the following was agreed for TCI-state activation:

Agreement For C-LTM, after the LTM cell switch is triggered, - For RACH-less LTM, the UE determines an indicated TCI state in CandidateTCI-State or CandidateTCI-UL-State whose QCL RS has the same value as the RS signaled from higher layer to lower layer that meets the C-LTM execution condition. - Upon RACH-less CLTM procedure being triggered, activated Candidate TCI state(s), other than the indicated TCI state, should be deactivated.

Based on the highlighted text above, [Huawei, 2] identifies an issue for a UE that supports only ‘SSB’ as a Type-D QCL source RS. In this case, the UE is unable to locate an RS with the same value as the one signaled by the higher layer that satisfies the C-LTM execution condition. Moderator think this problem also exists when a UE is configured with CSI-RS as QCL source RS for TCI-state, while the C-LTM resource is configured with SSB.

A TP was prepared in [Huawei, 2] to resolve this problem.

Text proposal # 4-1:

Reasons of change: For a UE not supporting CSI-RS for BM as Type-D QCL source RS of LTM TCI state, it is not possible for the UE to find a TCI state whose QCL RS has the same value as RS signalled from higher layer that meets the C-LTM execution condition. Summary of change: Additionally allow UE to determine a TCI state whose QCL RS is QCLed with the RS signalled from higher layer that meets the C-LTM execution condition. Consequences if not approved: A UE not supporting CSI-RS for BM as Type-D QCL source RS of LTM TCI state cannot perform C-LTM.

# 21 L1/L2-triggered mobility procedures < Unchanged parts are omitted > For RACH-based LTM cell switch [19, TS 38.300], the UE applies the CandidateTCI-State for receptions on the candidate cell, and applies a spatial domain filter corresponding to the CandidateTCI-State or the CandidateTCI-UL-State for transmissions on the candidate cell, that are after the completion of the random access procedure associated with the PRACH transmission on the candidate cell and before a new TCI state is applied for the candidate cell. For RACH-less LTM cell switch [19, TS 38.300], the UE applies the CandidateTCI-State for receptions on the candidate cell and applies a spatial domain filter corresponding to the CandidateTCI-State or the CandidateTCI-UL-State for transmissions on the candidate cell before a new TCI state is applied for the candidate cell. After RACH-based conditional LTM cell switch, all activated TCI states are deactivated. For RACH-less conditional LTM cell switch, the UE determines a TCI state in CandidateTCI-State or CandidateTCI-UL-State to apply for receptions or transmissions on the candidate cell, where the QCL RS of the TCI state is same as or is QCLed with the SS/PBCH block or the CSI-RS resource selected in the RACH-less conditional LTM cell switch as described in clause 5.36.3 [11, TS 38.321]. After RACH-less conditional LTM cell switch, all activated TCI states, other than the TCI state, are deactivated. < Unchanged parts are omitted >

| Company | View/Positions (Please indicate Option) | Comments (If a TP is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| --- | --- | --- |
| vivo | Yes |  |
| Ericsson | Yes |  |
| Nokia | Yes |  |
| NTT DOCOMO | Yes |  |
| Ofinno | Yes |  |
| Google | Yes |  |
| ZTE | No | During the discussion of the corresponding proposal at the last meeting, our company had already proposed considering the case mentioned in this TP in the FL summary. But companies that provided comments deemed it unnecessary, which can be found in the following screenshot. Based on this, we don’t think that this TP is necessary. From our perspective, if UE does not support CSI-RS as QCL RS, RS type for CLTM event evaluation should not be configured as CSI-RS during CLTM configuration. |
| Samsung | No | We do not think the proposal deals with typical scenario, and is overoptimizing things, which should be avoided during maintenance phase. Such corner case can be avoided by NW’s implementation of configuring/not configuring C-LTM for a particular UE having a particular QCL assumption. |
| CATT | Yes |  |
| Moderator |  | My interpretation of the previously discussed issue (as noted above) differs from what this CR aims to address. In our last meeting, the scenario we examined was that when multiple TCI-states are associated with the same measurement resource, the UE has the discretion to select one. Also, companies mind may change meeting by meeting. That said, let’s focus on the specific issue identified and whether this CR provides a solution, rather than debating whether the issue was previously discussed but not agreed. Even if it has been discussed before, companies now appear to support this CR. Our focus should remain on the technical aspects, not on whether the topic has already been raised. |
| Huawei, HiSilicon | Yes | Basically, there can be four possible cases: Case 1: The QCL RS of the TCI-state is CSI-RS, and the RS selected by high layer is CSI-RS; Case 2: The QCL RS of the TCI-state is SSB, and the RS selected by high layer is SSB; Case 3: The QCL RS of the TCI-state is CSI-RS, and the RS selected by high layer is SSB; Case 4: The QCL RS of the TCI-state is SSB, and the RS selected by high layer is CSI-RS; The current spec only covers Case 1 and Case 2. Case 3 and Case 4 are not supported. Case 3 is a common configuration and should be supported. Case 4 is a corner case and it is not possible to find a TCI-state with QCL RS (SSB) QCLed with the CSI-RS selected by high layer. So, it is better to preclude this case by gNB implementation, i.e., gNB will not configure CSI-RS to do RSRP measurement when the QCL RS is SSB. Hence, the spec only need to cover case 1, 2, and 3. To cover case 1, 2, and 3, we suggest following modification. After RACH-based conditional LTM cell switch, all activated TCI states are deactivated. For RACH-less conditional LTM cell switch, the UE determines a TCI state in CandidateTCI-State or CandidateTCI-UL-State to apply for receptions or transmissions on the candidate cell, where the QCL RS of the TCI state is same as the resource selected in the RACH-less conditional LTM cell switch or is QCLed with the SS/PBCH block selected in the RACH-less conditional LTM cell switch as described in clause 5.36.3 [11, TS 38.321]. After RACH-less conditional LTM cell switch, all activated TCI states, other than the TCI state, are deactivated. |

## Issue 4-2: Power Control for Uplink Transmission in C-LTM

[Ofinno, 7] proposed reusing the candidate TCI state associated with the high-layer RS that triggered the C-LTM. Specifically, that candidate TCI state provides the pathloss reference RS, p0, α, and the closed-loop control entity/index/target for UL transmissions on the target cell until an explicit reconfiguration applies.

The corresponding TP is prepared and listed below:

Text proposal # 4-2:

Reason for change: The current specification does not define how uplink power control parameters are derived when the UE selects a candidate TCI state for C-LTM. Summary of change: Update TS 38.213 to clarify that uplink power control shall use the pathloss RS, p0, and α associated with the candidate TCI state indicated by MAC CE or selected by the UE for C-LTM. Consequences if not approved: UEs may apply inconsistent uplink power control after C-LTM, causing misaligned pathloss estimation and degraded mobility performance.

==================================unchanged omitted=================================== # 7 Uplink Power control Uplink power control determines a power for PUSCH, PUCCH, SRS, and PRACH transmissions. A UE does not expect to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions as described in clauses 7.1.1, 7.2.1, and 7.3.1, except for SRS transmissions configured by SRS-PosResourceSet as described in clause 7.3.1. If the UE is provided a number of RS resources for pathloss estimation for PUSCH/PUCCH/SRS transmissions that is larger than 4, the UE maintains for pathloss estimation RS resources corresponding to RS resource indexes qd as described in clauses 7.1.1, 7.2.1, and 7.3.1. If an RS resource updated by MAC CE, as described in clauses 7.1.1, 7.2.1 and 7.3.1, is one from the RS resources the UE maintains for pathloss estimation for PUSCH/PUCCH/SRS transmissions, the UE applies the pathloss estimation based on the RS resources starting from the first slot that is after slot k+3∙Nslotsubframe, μ+2μ∙kmac where k is the slot where the UE would transmit a PUCCH or PUSCH with HARQ-ACK information for the PDSCH providing the MAC CE, μ is the SCS configuration for the PUCCH or PUSCH, respectively, that is determined in the slot when the MAC CE command is applied and kmac is a number of slots for SCS configuration μ=0 provided by kmac or kmac=0 if kmac is not provided. A PUSCH/PUCCH/SRS/PRACH transmission occasion i is defined by a slot index ns,fμ within a frame with system frame number SFN, a first symbol S within the slot, and a number of consecutive symbols L. For a PUSCH transmission with repetition Type B, a PUSCH transmission occasion is a nominal repetition [6, TS 38.214]. In the remaining of this clause, if a UE is provided TCI-State in dl-OrJointTCI-StateList or TCI-UL-State or CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE, or CandidateTCI-State or CandidateTCI-UL-State selected by the UE for the conditional LTM cell switch, as described in clause 21, and for each indicated one or two TCI-State or TCI-UL-State or CandidateTCI-State or CandidateTCI-UL-State of a PUSCH, PUCCH, or SRS transmission occasion as described in [6, TS 38.214] - in clauses 7.1.1, 7.2.1, and 7.3.1, the RS index qd for obtaining the downlink pathloss estimate for PUSCH, PUCCH, and SRS transmission is provided by pathlossReferenceRS-Id-r17 associated with or included in the indicated TCI-State or TCI-UL-State except for SRS transmission that is not provided followUnifiedTCI-StateSRS, or by pathlossReferenceRS-Id included in CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE, or by pathlossReferenceRS-Id included in CandidateTCI-State or CandidateTCI-UL-State selected by the UE for the conditional LTM cell switch - in clause 7.1.1, if p0AlphaSetforPUSCH is provided, or if p0AlphaSetforPUSCH-SBFD is provided and for a PUSCH transmission in SBFD symbols as described in clause 11.1, the values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l are provided by p0AlphaSetforPUSCH or by p0AlphaSetforPUSCH-SBFD, respectively, associated with the indicated TCI-State or TCI-UL-State, or by p0AlphaSetforPUSCH associated with the CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE, or by p0AlphaSetforPUSCH associated with CandidateTCI-State or CandidateTCI-UL-State selected by the UE for the conditional LTM cell switch - in clause 7.2.1, if p0AlphaSetforPUCCH is provided, or if p0AlphaSetforPUCCH-SBFD is provided and for a PUCCH transmission in SBFD symbols, the values of PO_UE_PUCCHqu and the PUCCH power control adjustment state l are provided by p0AlphaSetforPUCCH or by p0AlphaSetforPUCCH-SBFD, respectively, associated with the indicated TCI-State or TCI-UL-State, or by p0AlphaSetforPUCCH associated with the CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE, or by p0AlphaSetforPUSCH associated with CandidateTCI-State or CandidateTCI-UL-State selected by the UE for the conditional LTM cell switch - in clause 7.3.1, if p0AlphaSetforSRS is provided, - if followUnifiedTCI-StateSRS is provided for a SRS resource set, the values of PO_UE_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS or, if p0AlphaSetforSRS-SBFD is provided and for an SRS transmission in SBFD symbols, by p0AlphaSetforSRS-SBFD, associated with the indicated TCI-State or TCI-UL-State, or by p0AlphaSetforSRS associated with the CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE, or by p0AlphaSetforPUSCH associated with CandidateTCI-State or CandidateTCI-UL-State selected by the UE for the conditional LTM cell switch - else, if followUnifiedTCI-StateSRS is not provided for a SRS resource set and for a SRS resource from the SRS resource set, the values of PO_UE_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS or, if p0AlphaSetforSRS-SBFD is provided and for an SRS transmission in SBFD symbols, by p0AlphaSetforSRS-SBFD, associated with TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set, a RS index qd for obtaining a pathloss estimate for the SRS transmission is provided by pathlossReferenceRS-Id-r17 associated with or included in the TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set, and a value of a PLb,f,c,offset, when TCI-State or a TCI-UL-State includes pl-Offset, is provided by pl-Offset associated with TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set PO_SRS,b,f,cqs is the sum of the component PO_UE_SRS,b,f,cqs and a component p0 provided by SRS-ResourceSet corresponding to the SRS resource set. ==================================unchanged omitted=================================== ### 7.1.1 UE behaviour ==================================unchanged omitted=================================== - For a PUSCH (re)transmission configured by ConfiguredGrantConfig, j=1, PO_NOMINAL,PUSCH,f,c1 is provided by p0-NominalWithoutGrant, or PO_NOMINAL,PUSCH,f,c1=PO_NOMINAL,PUSCH,f,c0 if p0-NominalWithoutGrant is not provided. ==================================unchanged omitted=================================== - else, PO_UE_PUSCH,b,f,c1 is provided by p0 obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig that provides an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet, or by sdt-P0-PUSCH for a PUSCH (re)transmission as described in clause 19.1, or by rrc-P0-PUSCH for a PUSCH (re)transmission as described in clause 22.1, or by p0 of p0AlphaSetforPUSCH associated with the CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE, or by p0 of p0AlphaSetforPUSCH associated with CandidateTCI-State or CandidateTCI-UL-State selected by the UE for the conditional LTM cell switch for a configured grant Type-1 PUSCH (re)transmission as described in clause [21.1], for active UL BWP b of carrier f of serving cell c ==================================unchanged omitted=================================== - For αb,f,cj ==================================unchanged omitted=================================== - else αb,f,c(1) is provided by alpha obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig providing an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet, or by sdt-Alpha for a PUSCH (re)transmission as described in clause 19.1, or by rrc-Alpha for a PUSCH (re)transmission as described in clause 22.1, or by alpha of p0AlphaSetforPUSCH associated with the CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE, or by alpha of p0AlphaSetforPUSCH associated with CandidateTCI-State or CandidateTCI-UL-State selected by the UE for the conditional LTM cell switch for a configured grant Type-1 PUSCH (re)transmission as described in clause [21.1], for active UL BWP b of carrier f of serving cell c ==================================unchanged omitted===================================

| Company | View/Positions (Please indicate Option) | Comments (If a TP is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| --- | --- | --- |
| vivo | Yes |  |
| Ericsson | Yes | Propose to include that this is only for RACH-less CLTM – for RACH-based CLTM, the UE uses the selected SSB |
| Nokia | Yes | Agree with the proposal from Ericsson. |
| NTT DOCOMO | Yes |  |
| Ofinno | Yes | We also agree with Ericsson. To address the comment, we may need to insert “RACH-less” in proposed TP, i.e., instead of “ CandidateTCI-State or CandidateTCI-UL-State selected by the UE for the conditional LTM cell switch”, we may say “ CandidateTCI-State or CandidateTCI-UL-State selected by the UE for the RACH-less conditional LTM cell switch” |
| Google | Yes | We agree with Ericsson that this change should only apply for RACH-less CLTM. |
| ZTE |  | Agree with Ericsson’s view. |
| Samsung | Yes | Fine with RACH-less CLTM only. |
| CATT | Yes | We share similar view as Ericsson that the change should only apply for RACH-less CLTM. |
| Moderator |  | @ Ofinno, can u please provide a complete CR for further review? Only single sentence to change or many places? |
| Ofinno | Yes | Thank you for capturing our concern. Here is our updated TP based on the discussion above. Updated part is highlighted in yellow. Reason for change: The current specification does not define how uplink power control parameters are derived when the UE selects a candidate TCI state for C-LTM. Summary of change: Update TS 38.213 to clarify that uplink power control shall use the pathloss RS, p0, and α associated with the candidate TCI state indicated by MAC CE or selected by the UE for C-LTM. Consequences if not approved: UEs may apply inconsistent uplink power control after C-LTM, causing misaligned pathloss estimation and degraded mobility performance.. ==================================unchanged omitted=================================== # 7 Uplink Power control Uplink power control determines a power for PUSCH, PUCCH, SRS, and PRACH transmissions. A UE does not expect to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions as described in clauses 7.1.1, 7.2.1, and 7.3.1, except for SRS transmissions configured by SRS-PosResourceSet as described in clause 7.3.1. If the UE is provided a number of RS resources for pathloss estimation for PUSCH/PUCCH/SRS transmissions that is larger than 4, the UE maintains for pathloss estimation RS resources corresponding to RS resource indexes qd as described in clauses 7.1.1, 7.2.1, and 7.3.1. If an RS resource updated by MAC CE, as described in clauses 7.1.1, 7.2.1 and 7.3.1, is one from the RS resources the UE maintains for pathloss estimation for PUSCH/PUCCH/SRS transmissions, the UE applies the pathloss estimation based on the RS resources starting from the first slot that is after slot k+3∙Nslotsubframe, μ+2μ∙kmac where k is the slot where the UE would transmit a PUCCH or PUSCH with HARQ-ACK information for the PDSCH providing the MAC CE, μ is the SCS configuration for the PUCCH or PUSCH, respectively, that is determined in the slot when the MAC CE command is applied and kmac is a number of slots for SCS configuration μ=0 provided by kmac or kmac=0 if kmac is not provided. A PUSCH/PUCCH/SRS/PRACH transmission occasion i is defined by a slot index ns,fμ within a frame with system frame number SFN, a first symbol S within the slot, and a number of consecutive symbols L. For a PUSCH transmission with repetition Type B, a PUSCH transmission occasion is a nominal repetition [6, TS 38.214]. In the remaining of this clause, if a UE is provided TCI-State in dl-OrJointTCI-StateList or TCI-UL-State or CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE, or CandidateTCI-State or CandidateTCI-UL-State selected by the UE for the RACH-less conditional LTM cell switch, as described in clause 21, and for each indicated one or two TCI-State or TCI-UL-State or CandidateTCI-State or CandidateTCI-UL-State of a PUSCH, PUCCH, or SRS transmission occasion as described in [6, TS 38.214] - in clauses 7.1.1, 7.2.1, and 7.3.1, the RS index qd for obtaining the downlink pathloss estimate for PUSCH, PUCCH, and SRS transmission is provided by pathlossReferenceRS-Id-r17 associated with or included in the indicated TCI-State or TCI-UL-State except for SRS transmission that is not provided followUnifiedTCI-StateSRS, or by pathlossReferenceRS-Id included in CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE, or by pathlossReferenceRS-Id included in CandidateTCI-State or CandidateTCI-UL-State selected by the UE for the RACH-less conditional LTM cell switch - in clause 7.1.1, if p0AlphaSetforPUSCH is provided, or if p0AlphaSetforPUSCH-SBFD is provided and for a PUSCH transmission in SBFD symbols as described in clause 11.1, the values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l are provided by p0AlphaSetforPUSCH or by p0AlphaSetforPUSCH-SBFD, respectively, associated with the indicated TCI-State or TCI-UL-State, or by p0AlphaSetforPUSCH associated with the CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE, or by p0AlphaSetforPUSCH associated with CandidateTCI-State or CandidateTCI-UL-State selected by the UE for the RACH-less conditional LTM cell switch - in clause 7.2.1, if p0AlphaSetforPUCCH is provided, or if p0AlphaSetforPUCCH-SBFD is provided and for a PUCCH transmission in SBFD symbols, the values of PO_UE_PUCCHqu and the PUCCH power control adjustment state l are provided by p0AlphaSetforPUCCH or by p0AlphaSetforPUCCH-SBFD, respectively, associated with the indicated TCI-State or TCI-UL-State, or by p0AlphaSetforPUCCH associated with the CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE, or by p0AlphaSetforPUSCH associated with CandidateTCI-State or CandidateTCI-UL-State selected by the UE for the RACH-less conditional LTM cell switch - in clause 7.3.1, if p0AlphaSetforSRS is provided, - if followUnifiedTCI-StateSRS is provided for a SRS resource set, the values of PO_UE_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS or, if p0AlphaSetforSRS-SBFD is provided and for an SRS transmission in SBFD symbols, by p0AlphaSetforSRS-SBFD, associated with the indicated TCI-State or TCI-UL-State, or by p0AlphaSetforSRS associated with the CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE, or by p0AlphaSetforPUSCH associated with CandidateTCI-State or CandidateTCI-UL-State selected by the UE for the RACH-less conditional LTM cell switch - else, if followUnifiedTCI-StateSRS is not provided for a SRS resource set and for a SRS resource from the SRS resource set, the values of PO_UE_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS or, if p0AlphaSetforSRS-SBFD is provided and for an SRS transmission in SBFD symbols, by p0AlphaSetforSRS-SBFD, associated with TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set, a RS index qd for obtaining a pathloss estimate for the SRS transmission is provided by pathlossReferenceRS-Id-r17 associated with or included in the TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set, and a value of a PLb,f,c,offset, when TCI-State or a TCI-UL-State includes pl-Offset, is provided by pl-Offset associated with TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set PO_SRS,b,f,cqs is the sum of the component PO_UE_SRS,b,f,cqs and a component p0 provided by SRS-ResourceSet corresponding to the SRS resource set. ==================================unchanged omitted=================================== ### 7.1.1 UE behaviour ==================================unchanged omitted=================================== - For a PUSCH (re)transmission configured by ConfiguredGrantConfig, j=1, PO_NOMINAL,PUSCH,f,c1 is provided by p0-NominalWithoutGrant, or PO_NOMINAL,PUSCH,f,c1=PO_NOMINAL,PUSCH,f,c0 if p0-NominalWithoutGrant is not provided. ==================================unchanged omitted=================================== - else, PO_UE_PUSCH,b,f,c1 is provided by p0 obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig that provides an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet, or by sdt-P0-PUSCH for a PUSCH (re)transmission as described in clause 19.1, or by rrc-P0-PUSCH for a PUSCH (re)transmission as described in clause 22.1, or by p0 of p0AlphaSetforPUSCH associated with the CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE, or by p0 of p0AlphaSetforPUSCH associated with CandidateTCI-State or CandidateTCI-UL-State selected by the UE for the RACH-less conditional LTM cell switch for a configured grant Type-1 PUSCH (re)transmission as described in clause [21.1], for active UL BWP b of carrier f of serving cell c ==================================unchanged omitted=================================== - For αb,f,cj ==================================unchanged omitted=================================== - else αb,f,c(1) is provided by alpha obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig providing an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet, or by sdt-Alpha for a PUSCH (re)transmission as described in clause 19.1, or by rrc-Alpha for a PUSCH (re)transmission as described in clause 22.1, or by alpha of p0AlphaSetforPUSCH associated with the CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE, or by alpha of p0AlphaSetforPUSCH associated with CandidateTCI-State or CandidateTCI-UL-State selected by the UE for the RACH-less conditional LTM cell switch for a configured grant Type-1 PUSCH (re)transmission as described in clause [21.1], for active UL BWP b of carrier f of serving cell c ==================================unchanged omitted===================================. |
| Huawei, HiSilicon | Yes |  |

## Issue 4-3: TCI-State Deactivation

The following was agreed in RAN1 122bis meeting:

Agreement - After RACH-based CLTM is performed, all activated candidate TCI states are deactivated. Agreement For C-LTM, after the LTM cell switch is triggered, - For RACH-less LTM, the UE determines an indicated TCI state in CandidateTCI-State or CandidateTCI-UL-State whose QCL RS has the same value as the RS signaled from higher layer to lower layer that meets the C-LTM execution condition. - Upon RACH-less CLTM procedure being triggered, activated Candidate TCI state(s), other than the indicated TCI state, should be deactivated.

As highlighted in yellow, it was agreed that TCI-states are deactivated when the LTM cell switch is triggered, rather than upon ‘completion’. [Google, 10] proposes the following TP to correct spec and align with RAN1 agreement.

Text proposal # 4-3:

Reason for change: As agreed, for both RACH-based and RACH-less CLTM, activated TCI states (except the determined TCI state, if present) are deactivated, after RACH-based or RACH-less CLTM is performed. However, current SPEC wordings can mislead reader that activated TCI states (except the determined TCI state, if present) are deactivated, after RACH-based or RACH-less CLTM is completed, which prolongs the time of deactivating TCI states. Summary of change: Clarify that for both RACH-based and RACH-less CLTM, activated TCI states (except the determined TCI state, if present) are deactivated, after RACH-based or RACH-less CLTM is performed. Consequences if not approved: Incorrect behavior, which is not aligned with corresponding agreements.

# 21 L1/L2-triggered mobility procedures < Unchanged parts are omitted > After RACH-based conditional LTM cell switch is performed, all activated TCI states are deactivated. For RACH-less conditional LTM cell switch, the UE determines a TCI state in CandidateTCI-State or CandidateTCI-UL-State to apply for receptions or transmissions on the candidate cell, where a QCL RS index of the TCI state is same as a SS/PBCH block index or a CSI-RS resource index selected in the RACH-less conditional LTM cell switch as described in clause 5.36.3 [11, TS 38.321]. After RACH-less conditional LTM cell switch is performed, all activated TCI states, other than the TCI state, are deactivated. < Unchanged parts are omitted >

| Company | View/Positions (Please indicate your support: Yes, No in this column) | Comments (If a TP text is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| --- | --- | --- |
| vivo | Yes |  |
| Ericsson | No | Even with the proposed interpretation, there is no difference in UE behaviour, since the UE cannot receive any indication between the triggering and completion of LTM |
| Nokia |  | Even with the proposed wording, the exact timing is unclear. But if the proposed change makes it less ambiguous – we are OK with the changes. |
| NTT DOCOMO |  | We are fine, but it seems not essential. |
| Ofinno | Yes, but | We agree with the intention but prefer to use ‘triggered’ instead of ‘performed’. |
| Google |  | @Ericsson: There is difference. It’s not about whether there is indication between the triggering and completion of LTM. It’s about when UE can deactivate TCI states and stop using UE’s memory on storing these activated TCI states. @Offino: Re. the agreement for RACH-based CLTM, “performed” is used. We just try to align the wordings. We are fine if majority thinks “triggered” is better wording. Agreement - After RACH-based CLTM is performed, all activated candidate TCI states are deactivated. |
| ZTE | No | The same meaning. Besides, even without update, it will not affect understanding. |
| Samsung | No | Not essential – without the change, the spec is not broken |
| CATT | Yes |  |
| Huawei, HiSilicon | Not essential |  |

## Issue 4-4: TCI-State application Timing for C-LTM

[Ofinno, 7] proposes the following TP to clarify the duration for which the determined TCI state is applied for both reception and transmission.

Text proposal # 4-4-1:

Reason for change: The currently agreed CR (R1-2508229) does not explicitly define the application duration for the TCI state determined by the UE after a RACH-less CLTM cell switch. Summary of change: Add “before a new TCI state is applied for the candidate cell” as an ending condition of the candidate TCI state application. Consequences if not approved: ### UE may apply mismatched TCI states, resulting in incorrect cell measurements and degraded mobility performance.

==================================unchanged omitted=================================== # 21 L1/L2-triggered mobility procedures ==================================unchanged omitted=================================== The UE does not expect to be indicated quasi co-location 'typeA' properties when a SS/PBCH block is configured as a source RS of the TCI state. The UE applies the CandidateTCI-State and/or CandidateTCI-UL-State, if indicated by the MAC CE, no later than TLTM-RRC-processing+TLTM-processing+Tfirst-RS+TRS-proc+3 msec after the last symbol of a PUCCH or PUSCH with HARQ-ACK information for the PDSCH providing the MAC CE, where TLTM-RRC-processing, TLTM-processing, Tfirst-RS and TRS-proc are defined in [10, TS 38.133]. For RACH-based LTM cell switch [19, TS 38.300], the UE applies the CandidateTCI-State for receptions on the candidate cell, and applies a spatial domain filter corresponding to the CandidateTCI-State or the CandidateTCI-UL-State for transmissions on the candidate cell, that are after the completion of the random access procedure associated with the PRACH transmission on the candidate cell and before a new TCI state is applied for the candidate cell. For RACH-less LTM cell switch [19, TS 38.300], the UE applies the CandidateTCI-State for receptions on the candidate cell and applies a spatial domain filter corresponding to the CandidateTCI-State or the CandidateTCI-UL-State for transmissions on the candidate cell before a new TCI state is applied for the candidate cell. After RACH-based conditional LTM cell switch, all activated TCI states are deactivated. For RACH-less conditional LTM cell switch, the UE determines a TCI state in CandidateTCI-State or CandidateTCI-UL-State to apply for receptions or transmissions on the candidate cell before a new TCI state is applied for the candidate cell, where a QCL RS index of the TCI state is same as a SS/PBCH block index or a CSI-RS resource index selected in the RACH-less conditional LTM cell switch as described in clause 5.36.3 [11, TS 38.321]. After RACH-less conditional LTM cell switch, all activated TCI states, other than the TCI state, are deactivated. ==================================unchanged omitted===================================

While, [ZTE,3] proposed to define TCI-state application time for C-LTM with the corresponding TP below:

Text proposal # 4-4-2:

Reason for change: The application time of the determined TCI state has not been discussed and defined yet in TS 38.213. Summary of change: Define the application time of TCI state for RACH-less based CLTM, e.g., UE applies the determined TCI state no later than TCLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc after RACH-less based CLTM, where TCLTM-RRC-processing, TLTM-processing, Tfirst-RS, TRS-proc are defined in TS 38.133. Consequences if not approved: The application time of TCI state for RACH-less based CLTM is unclear for UE. Proposal 1: For RACH-less based CLTM, UE applies the determined TCI state no later than TCLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc after RACH-less based CLTM, where TCLTM-RRC-processing, TLTM-processing, Tfirst-RS, TRS-proc are defined in TS 38.133. Text proposal 1: Adopt the following text change in Clause 21 of TS 38.213.

#### 21 L1/L2-triggered mobility procedures #### *** Unchanged parts are omitted *** After RACH-based conditional LTM cell switch, all activated TCI states are deactivated. For RACH-less conditional LTM cell switch, the UE determines a TCI state in CandidateTCI-State or CandidateTCI-UL-State to apply for receptions or transmissions on the candidate cell, where a QCL RS index of the TCI state is same as a SS/PBCH block index or a CSI-RS resource index selected in the RACH-less conditional LTM cell switch as described in clause 5.36.3 [11, TS 38.321]. After RACH-less conditional LTM cell switch, all activated TCI states, other than the TCI state, are deactivated. The UE applies the TCI state in CandidateTCI-State or CandidateTCI-UL-State, no later than TCLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc after RACH-less based CLTM, where TCLTM-RRC-processing, TLTM-processing, Tfirst-RS, TRS-proc are defined in [10, TS 38.133]. *** Unchanged parts are omitted ***

| Moderater Question 4-4: Which of two TPs is preferred for TCI-state application time or none of them or both? Please provide a simple justification for the choice in ‘Comment’ Column |  |  |
| --- | --- | --- |
| Company | View/Positions | Comments |
| vivo |  | Prefer Text proposal # 4-4-1. |
| Ericsson | No | TP#4-4-1: not needed, this is as normal. TP #4-4-2: I don’t think this is correct, the UE shall apply the TCI state directly after the RACH-less based CLTM, i.e., directly after transmitting the first UL message. The RRC reconfiguration must be complete when the UE starts transmitting on the target. There is little point in stating the time between the triggering of CLTM and the application of the new TCI state in the target, since the NW does not know when the triggering happens. |
| Nokia |  | Support TP #4-4-1 But in TP #4-4-2: the reference point, i.e., after RACH-less based CLTM, is not very clear. |
| NTT DOCOMO | TP#4-4-1: Fine TP#4-4-2: No | TP#4-4-1: We are fine to align with R18 LTM description. TP #4-4-2: We don’t think that this TP is necessary since this timing would not be useful since there is nothing to do before first UL data transmission (i.e., CG-PUSCH) in case of conditional LTM different from R18 LTM. Also, NW would not know the timing of CLTM triggering. |
| Ofinno |  | Support TP #4-4-1. TP is based on the existing spec for Rel-18 LTM so to be consistent, we think it is needed. For TP #4-4-2, we are open to discuss. Just minor comment is, “after triggering the RACH-less based CLTM” would be more accurate. |
| Google |  | TP #4-4-1: Support TP#4-4-2: We don’t think the timing is needed to NW. In addition, UE can not transmit first UL on CG-PUSCH before the TCI state is ready to apply. Also, it seems, for “RACH-less based CLTM”, “based” is redundant. |
| ZTE |  | For TP#4-4-1: Fine. For TP#4-4-2: The intention of this TP is to align with the design logic for starting time of TCI state application in Rel-18 LTM. Further, to ensure our understandings are better aligned, we provide the following comparison of defining TCI state application time between Rel-18 LTM and Rel-19 RACH-less CLTM. Rel-18 LTM; Rel-19 RACH-less CLTM CS. Determination of TCI state for Tx/Rx after cell switching completion; LTM CSC MAC CE indicates aTCI state; TCI state whose QCL RS is same as RS that meets CLTM execution condition. It means that once there is at least one beam satisfying CLTM execution condition, RACH-less CLTM CS is triggered. Starting time of applying TCI state; The UE applies the CandidateTCI-State and/or CandidateTCI-UL-State, if indicated by the MAC CE, no later than TLTM-RRC-processing+TLTM-processing+Tfirst-RS+TRS-proc+3 msec after the last symbol of a PUCCH or PUSCH with HARQ-ACK information for the PDSCH providing the MAC CE, where TLTM-RRC-processing, TLTM-processing, Tfirst-RS and TRS-proc are defined in [10, TS 38.133]. The above yellow highlighted part means that the earliest point at which the UE can apply the indicated beam is after the HARQ-ACK corresponding to the LTM CSC MAC CE(even if there is no first UL transmission at this time), and the latest point is the ending time corresponding to TLTM-RRC-processing+TLTM-processing+Tfirst-RS+TRS-proc+3 msec relative to the HARQ-ACK corresponding to the LTM CSC MAC CE. The latest point is determined based on LTM cell switch delay defined by TS 38.133. DLTM = THARQ + 3ms + TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU Note that TLTM-IU is not counted in the application time of TCI state since it is the time for the uncertainty on transmitting the first uplink transmission on the target cell; In our view, the rule similar to Rel-18 LTM should also be defined for RACH-less CLTM. Specifically, the definition of the determined TCI state application time can also refer to CLTM cell switch delay, i.e., DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU. However, different with Rel-18 LTM, we think that the real time where RACH-less CLTM is triggered is the ending time corresponding to TRRC + TEvent_DU + Tmeasure according to the definition of Tmeasure , i.e., Tmeasure is the time for the UE to measure and realize the condition for CLTM cell switch is fulfilled. So based on the above considerations, we understand that the earliest point at which the UE can apply the determined TCI state is after TRRC + TEvent_DU + Tmeasure , while the latest point is the ending time corresponding to TCLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc Regarding the above understanding and viewpoints, it would be greatly appreciated if you could share your insights. Thanks!. |
| Samsung |  | For TP#4-4-1: fine with the TP as it aligns with a corresponding description in Rel-18 LTM operation For TP#4-4-2: open to discuss, but such application timeline issue seems to exist in other relevant parts throughout 214 as well |
| CATT |  | For TP#4-4-1: Fine. |
| Huawei, HiSilicon |  | Fine with 4-4-1. For 4-4-2, gNB may not know when the CLTM start at UE side. it is difficult the count the timeline. |

## Issue 4-5: Others

[ZTE, 3] proposed clarifying the UE’s QCL assumption for UL and DL channels or signals after RACH-based CLTM and before a TCI state is applied for the target cell. The corresponding TP is listed below:

Text proposal # 4-5:

Reason for change: QCL assumption of UL and DL channel or signal after RACH-based CLTM and before a TCI state is applied for the target cell has not been specified yet. Summary of change: Define QCL assumption of UL and DL channel or signal after RACH-based CLTM and before a TCI state is applied for the target cell, i.e., - For DL, UE assumes that DM-RS of PDSCH and DM-RS of PDCCH, and the CSI-RS applying the indicated TCI state are quasi co-located with the SS/PBCH block or the CSI-RS resource the UE identified during the random access procedure initiated by RACH-based CLTM. - For UL, UE assumes that the UL TX spatial filter, if applicable, for dynamic-grant and configured-grant based PUSCH and PUCCH, and for SRS applying the indicated TCI state is the same as that for a PUSCH transmission scheduled by a RAR UL grant or a MsgA PUSCH transmission during random access procedure initiated by RACH-based CLTM. Consequences if not approved: QCL assumption of UL and DL channel or signal after RACH-based CLTM and before a TCI state is applied for the target cell is unclear for UE.

#### 21 L1/L2-triggered mobility procedures #### *** Unchanged parts are omitted *** After RACH-based conditional LTM cell switch, all activated TCI states are deactivated. For the case that after RACH-based conditional LTM cell switch and before a TCI state is applied for the target cell, - UE assumes that DM-RS of PDSCH and DM-RS of PDCCH, and the CSI-RS applying the indicated TCI state are quasi co-located with the SS/PBCH block or the CSI-RS resource the UE identified during the random access procedure initiated by RACH-based conditional LTM cell switch. - UE assumes that the UL TX spatial filter, if applicable, for dynamic-grant and configured-grant based PUSCH and PUCCH, and for SRS applying the indicated TCI state is the same as that for a PUSCH transmission scheduled by a RAR UL grant or a MsgA PUSCH transmission during random access procedure initiated by RACH-based conditional LTM cell switch. For RACH-less conditional LTM cell switch, the UE determines a TCI state in CandidateTCI-State or CandidateTCI-UL-State to apply for receptions or transmissions on the candidate cell, where a QCL RS index of the TCI state is same as a SS/PBCH block index or a CSI-RS resource index selected in the RACH-less conditional LTM cell switch as described in clause 5.36.3 [11, TS 38.321]. After RACH-less conditional LTM cell switch, all activated TCI states, other than the TCI state, are deactivated. *** Unchanged parts are omitted ***

Moderator notes that this issue was discussed in the RAN1 122bis meeting (P4-3-1) without reaching concensus. Most companies preferred not to capture in the specification, as legacy rule is applied for this case. If the situation remains unchanged, moderator will close the discussion after the 1st round.

| Company | View/Positions (Please indicate your support: Yes, No in this column) | Comments (If a TP text is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| --- | --- | --- |
| vivo | Yes |  |
| Ericsson | No? | Agree with the moderator, this is legacy behaviour. |
| Nokia | No | For RACH-based cell switch, the legacy specification is already clear. |
| NTT DOCOMO | No | Agree with the moderator. |
| Google |  | We are open to discuss. |
| ZTE |  | Actually, we didn’t see this issue being discussed in the last meeting. If we missed anything, please let me know. Thanks :) Besides, we agree with the views from above companies, that is, the behavior mentioned in the TP is same as legacy behavior. But we want to emphasize that the condition for applying the behavior is different with the legacy. From our perspective, if we want to avoid repeatedly elaborating on the same UE behavior under different scenarios in the existing spec, we could add some descriptions based on the existing spec (clause 5.1.5 of TS 38.214) to accommodate RACH-based CLTM scenario, such as the following update is proposed as a starting point for further discussion. 5.1.5 Antenna ports quasi co-location ... After a UE receives an initial higher layer configuration of dl-OrJointTCI-StateList where more than one TCI-State can be used as an indicated TCI state and before application of an indicated TCI state from the configured TCI states: - The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH that are not received during the RACH procedure, and the CSI-RS applying the indicated TCI state are quasi co-located with the reference signal(s) in the CandidateTCI-State indicated in the LTM Cell Switch Command MAC CE [10, 38.321] if applicable, otherwise. - The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH and the CSI-RS applying the indicated TCI state are quasi co-located with the SS/PBCH block the UE identified during the initial access procedure, or the random access procedure initiated by RACH-based conditional LTM cell switch. After a UE receives an initial higher layer configuration of dl-OrJointTCI-StateList where more than one TCI-State can be used as an indicated TCI state or an initial higher layer configuration of ul-TCI-StateList where more than one TCI-UL-State can be used as an indicated TCI state and before application of an indicated TCI state from the configured TCI states: - The UE determines the UL TX spatial filter, if applicable, for dynamic-grant based PUSCH that is not transmitted during the RACH procedure and configured-grant based PUSCH and PUCCH that are not transmitted during the RACH procedure, and for SRS applying the indicated TCI state, from the CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE [10, 38.321] if applicable, otherwise. - The UE assumes that the UL TX spatial filter, if applicable, for dynamic-grant and configured-grant based PUSCH and PUCCH, and for SRS applying the indicated TCI state, is the same as that for a PUSCH transmission scheduled by a RAR UL grant or a MsgA PUSCH transmission during the initial access procedure, or the random access procedure initiated by RACH-based conditional LTM cell switch.. |
| Samsung | No | Agree with moderator’s assessment |
| CATT | No | Agree with the moderator. |
| Huawei, HiSilicon |  | As it is agreed by RAN2, it can be up to RAN2 on how to capture in their spec. |
| ZTE2 |  | Offline discussion with some companies, they tend to discuss and address this issue for both RACH-based and RACH-less based CLTM. Based on this, we provide the following updated draft TP for further discussion. 5.1.5 Antenna ports quasi co-location ... After a UE receives an initial higher layer configuration of dl-OrJointTCI-StateList where more than one TCI-State can be used as an indicated TCI state and before application of an indicated TCI state from the configured TCI states: - The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH that are not received during the RACH procedure, and the CSI-RS applying the indicated TCI state are quasi co-located with the reference signal(s) in the CandidateTCI-State indicated in the LTM Cell Switch Command MAC CE [10, 38.321] if applicable, - The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH that are not received during the RACH procedure, and the CSI-RS applying the indicated TCI state are quasi co-located with the reference signal(s) in the CandidateTCI-State that is same as the RS resource selected in the RACH-less conditional LTM cell switch or is QCLed with the SS/PBCH block selected in the RACH-less conditional LTM cell switch as described in clause 5.36.3 [11, TS 38.321] if applicable, - The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH and the CSI-RS applying the indicated TCI state are quasi co-located with the SS/PBCH block the UE identified the random access procedure initiated by RACH-based conditional LTM cell switch if applicable, otherwise. - The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH and the CSI-RS applying the indicated TCI state are quasi co-located with the SS/PBCH block the UE identified during the initial access procedure. After a UE receives an initial higher layer configuration of dl-OrJointTCI-StateList where more than one TCI-State can be used as an indicated TCI state or an initial higher layer configuration of ul-TCI-StateList where more than one TCI-UL-State can be used as an indicated TCI state and before application of an indicated TCI state from the configured TCI states: - The UE determines the UL TX spatial filter, if applicable, for dynamic-grant based PUSCH that is not transmitted during the RACH procedure and configured-grant based PUSCH and PUCCH that are not transmitted during the RACH procedure, and for SRS applying the indicated TCI state, from the CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE [10, 38.321] if applicable, - The UE determines the UL TX spatial filter, if applicable, for dynamic-grant and configured-grant based PUSCH and PUCCH, and for SRS applying the indicated TCI state, from the CandidateTCI-State or CandidateTCI-UL-State that is same as the RS resource selected in the RACH-less conditional LTM cell switch or is QCLed with the SS/PBCH block selected in the RACH-less conditional LTM cell switch [10, TS 38.321] if applicable, - The UE determines the UL TX spatial filter, if applicable, for dynamic-grant based PUSCH that is not transmitted during the RACH procedure and configured-grant based PUSCH and PUCCH that are not transmitted during the RACH procedure, and for SRS applying the indicated TCI state, is the same as that for a PUSCH transmission scheduled by a RAR UL grant or a MsgA PUSCH transmission during the random access procedure initiated by RACH-based conditional LTM cell switch [10, TS 38.321] if applicable, otherwise. - The UE assumes that the UL TX spatial filter, if applicable, for dynamic-grant and configured-grant based PUSCH and PUCCH, and for SRS applying the indicated TCI state, is the same as that for a PUSCH transmission scheduled by a RAR UL grant or a MsgA PUSCH transmission during the initial access procedure.. |

### [2nd round]

Company is suggested to check the following TP and provide feedback.

5.1.5 Antenna ports quasi co-location ... After a UE receives an initial higher layer configuration of dl-OrJointTCI-StateList where more than one TCI-State can be used as an indicated TCI state and before application of an indicated TCI state from the configured TCI states: - The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH that are not received during the RACH procedure, and the CSI-RS applying the indicated TCI state are quasi co-located with the reference signal(s) in the CandidateTCI-State indicated in the LTM Cell Switch Command MAC CE [10, 38.321] if applicable, - The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH that are not received during the RACH procedure, and the CSI-RS applying the indicated TCI state are quasi co-located with the reference signal(s) in the CandidateTCI-State that is same as the RS resource selected in the RACH-less conditional LTM cell switch or is QCLed with the SS/PBCH block selected in the RACH-less conditional LTM cell switch as described in clause 5.36.3 [11, TS 38.321] if applicable, - The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH and the CSI-RS applying the indicated TCI state are quasi co-located with the SS/PBCH block the UE identified the random access procedure initiated by RACH-based conditional LTM cell switch if applicable, otherwise. - The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH and the CSI-RS applying the indicated TCI state are quasi co-located with the SS/PBCH block the UE identified during the initial access procedure. After a UE receives an initial higher layer configuration of dl-OrJointTCI-StateList where more than one TCI-State can be used as an indicated TCI state or an initial higher layer configuration of ul-TCI-StateList where more than one TCI-UL-State can be used as an indicated TCI state and before application of an indicated TCI state from the configured TCI states: - The UE determines the UL TX spatial filter, if applicable, for dynamic-grant based PUSCH that is not transmitted during the RACH procedure and configured-grant based PUSCH and PUCCH that are not transmitted during the RACH procedure, and for SRS applying the indicated TCI state, from the CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE [10, 38.321] if applicable, - The UE determines the UL TX spatial filter, if applicable, for dynamic-grant and configured-grant based PUSCH and PUCCH, and for SRS applying the indicated TCI state, from the CandidateTCI-State or CandidateTCI-UL-State that is same as the RS resource selected in the RACH-less conditional LTM cell switch or is QCLed with the SS/PBCH block selected in the RACH-less conditional LTM cell switch [10, TS 38.321] if applicable, - The UE determines the UL TX spatial filter, if applicable, for dynamic-grant based PUSCH that is not transmitted during the RACH procedure and configured-grant based PUSCH and PUCCH that are not transmitted during the RACH procedure, and for SRS applying the indicated TCI state, is the same as that for a PUSCH transmission scheduled by a RAR UL grant or a MsgA PUSCH transmission during the random access procedure initiated by RACH-based conditional LTM cell switch [10, TS 38.321] if applicable, otherwise. - The UE assumes that the UL TX spatial filter, if applicable, for dynamic-grant and configured-grant based PUSCH and PUCCH, and for SRS applying the indicated TCI state, is the same as that for a PUSCH transmission scheduled by a RAR UL grant or a MsgA PUSCH transmission during the initial access procedure.

| Company | View/Positions (Please indicate Option) | Comments (If a TP is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| --- | --- | --- |
| Nokia |  | There are some corrections required to ZTE’s latest proposal. Also for completeness we need to include all the scenarios specified for LTM. Please see the proposed changes below: 5.1.5 Antenna ports quasi co-location ... After a UE receives an initial higher layer configuration of dl-OrJointTCI-StateList where more than one TCI-State can be used as an indicated TCI state and before application of an indicated TCI state from the configured TCI states: - The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH that are not received during the RACH procedure, and the CSI-RS applying the indicated TCI state are quasi co-located with the reference signal(s) in the CandidateTCI-State indicated in the LTM Cell Switch Command MAC CE [10, 38.321] if applicable. - The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH that are not received during the RACH procedure, and the CSI-RS applying the determined TCI state in CandidateTCI-State indicated TCI state are quasi co-located with the reference signal(s) in the CandidateTCI-State that is same as the RS resource selected in the RACH-less conditional LTM cell switch or is QCLed with the SS/PBCH block selected in for the RACH-less conditional LTM cell switch as described in clause 5.36.3 [11, TS 38.321] [6, TS 38.213]), if applicable. - The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH and the CSI-RS applying the indicated TCI state are quasi co-located with the SS/PBCH block the UE identified during the random access procedure initiated by RACH-based conditional LTM cell switch [10, TS 38.321], if applicable, otherwise., - The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH and the CSI-RS applying the indicated TCI state are quasi co-located with the SS/PBCH block the UE identified during the initial access procedure. After a UE receives an initial higher layer configuration of dl-OrJointTCI-StateList where more than one TCI-State can be used as an indicated TCI state or an initial higher layer configuration of ul-TCI-StateList where more than one TCI-UL-State can be used as an indicated TCI state and before application of an indicated TCI state from the configured TCI states: - The UE determines the UL TX spatial filter, if applicable, for dynamic-grant based PUSCH that is not transmitted during the RACH procedure and configured-grant based PUSCH and PUCCH that are not transmitted during the RACH procedure, and for SRS applying the indicated TCI state, from the CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE [10, 38.321] if applicable. - The UE determines the UL TX spatial filter, if applicable, for dynamic-grant and configured-grant based PUSCH and PUCCH, and for SRS applying the determined TCI state in CandidateTCI-State or CandidateTCI-UL-State indicated TCI state, from the CandidateTCI-State or CandidateTCI-UL-State that is same as the RS resource selected in the RACH-less conditional LTM cell switch or is QCLed with the SS/PBCH block selected in the RACH-less conditional LTM cell switch [10, TS 38.321] [6, TS 38.213]), if applicable. - The UE determines the UL TX spatial filter, if applicable, for dynamic-grant based PUSCH that is not transmitted during the RACH procedure and configured-grant based PUSCH and PUCCH that are not transmitted during the RACH procedure, and for SRS applying the indicated TCI state, is the same as that for a PUSCH transmission scheduled by a RAR UL grant or a MsgA PUSCH transmission during the random access procedure initiated by RACH-based conditional LTM cell switch [10, TS 38.321], if applicable, otherwise., - The UE assumes that the UL TX spatial filter, if applicable, for dynamic-grant and configured-grant based PUSCH and PUCCH, and for SRS applying the indicated TCI state, is the same as that for a PUSCH transmission scheduled by a RAR UL grant or a MsgA PUSCH transmission during the initial access procedure. #### *** Unchanged parts are omitted *** When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' should be applied starting from the first slot that is after 𝑛+3𝑁𝑠𝑙𝑜𝑡𝑠𝑢𝑏𝑓𝑟𝑎𝑚𝑒,μ+2𝜇2𝜇𝐾𝑚𝑎𝑐∙𝑘mac where µ is the SCS configuration for the PUCCH and 𝜇𝐾𝑚𝑎𝑐 is the subcarrier spacing configuration for 𝑘𝑚𝑎𝑐 with a value of 0 for frequency range 1 and for FR2-NTN, and 𝑘mac is provided by K-Mac or 𝑘mac=0 if K-Mac is not provided. If tci-PresentInDCI is set to 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET scheduling the PDSCH, and the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than timeDurationForQCL if applicable, after a UE receives an initial higher layer configuration of TCI states and before reception of the activation command, - the UE assumes that DM-RS of ports of PDSCH of a serving cell are quasi co-located with the reference signal(s) in the CandidateTCI-State indicated in the LTM Cell Switch Command MAC CE [10, 38.321], except during RACH procedure for RACH-based LTM, if applicable, - The UE assumes that DM-RS ports of PDSCH of a serving cell are quasi co located with the reference signals(s) in determined TCI state in CandidateTCI-State for the RACH-less conditional LTM cell switch [6, TS 38.213]), if applicable, - The UE assumes that DM-RS ports of PDSCH of a serving cell are quasi co- located with the SS/PBCH block determined in the random access procedure initiated by RACH-based conditional LTM cell switch [10, TS 38.321], if applicable, otherwise - the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the SS/PBCH block determined in the initial access procedure with respect to qcl-Type set to 'typeA', and when applicable, also with respect to qcl-Type set to 'typeD'. |

# 5. Co-existence of ‘intra-DU’ LTM and SBFD features

In RAN2 #131 meeting, it is agreed that coexistence of SBFD and intra-DU LTM is supported [11] as captured below.

- Support co-existence of SBFD with intra-DU LTM. Whether to support the co-existence between SBFD and other LTM cases is not discussed in the Rel-19 SBFD WI.

In the RAN1 #122bis meeting, the feature combination was discussed twice but no consensus was reached. Most companies opposed supporting the new feature since RAN1 is now in the maintenance phase. The moderator noted that WG2 is leading this work item and that the agreement was made during RAN2’s normal phase. Therefore, the moderator recommended focusing on reviewing the TP content to ensure nothing is missing, rather than debating support for the feature.

A contribution [Ofinno, 7] examined the potential specification implications to support the combined operation of the LTM and SBFD features, and subsequently proposed the following TP:

Text proposal # 5-1:

Reason for change: Current specification does not clarify how an SBFD-aware UE shall derive CSI reports for LTM when one of the candidate cells operates SBFD. Symbol type configuration for LTM CSI reports is not specified. Summary of change: Update TS 38.214 to clarify that for LTM CSI reporting, the symbol type shall be explicitly configured, and the UE shall follow the same SBFD-aware behavior as for other CSI reports. Consequences if not approved: ### UE may derive and report CSI using resources with mismatched symbol types, potentially leading to incorrect cell measurements and degraded mobility performance.

==================================unchanged omitted=================================== #### 5.2.1.1 Reporting settings Each Reporting Setting CSI-ReportConfig is associated with a single downlink BWP (indicated by higher layer parameter BWP-Id) given in the associated CSI-ResourceConfig for channel measurement and contains the parameter(s) for one CSI reporting band: codebook configuration including codebook subset restriction, time-domain behavior, frequency granularity for CQI and PMI, measurement restriction configurations, and the CSI-related quantities to be reported by the UE such as the layer indicator (LI), L1-RSRP, L1-SINR, CRI, SSBRI (SSB Resource Indicator), CapabilityIndex, TDCP, L1-SRS-RSRP, L1-CLI-RSSI, SRS-RSRP-MRI, CLI-RSSI-MRI, CSI-PAI, P-CRI, P-SSBRI, P-L1-RSRP, RS-PAI, CJTC-Dd, CJTC-F, CJTC-Dd-F and CJTC-P. Each Reporting Setting ltm-CSI-ReportConfig is associated with a LTM-CSI-ResourceConfig for channel measurement and contains the parameters(s) for time-domain behavior provided by ltm-ReportConfigType, the number of cells and the number of reference signals per candidate cell provided by nrOfReportedCells, and nrOfReportedRS-PerCell, respectively, when ltm-ReportConfigType set to ‘periodic’ or ‘semiPersistentOnPUCCH’ or ‘semiPersistentOnPUSCH’ or ‘aperiodic’, comprising L1 measurement results associated with current SpCell if spCellInclusion is configured, and the CSI-related quantities to be reported by the UE provided by reportQuantity, if configured. If a UE is configured with SBFD symbols for a serving cell and a ltm-CSI-ReportConfig with the higher layer parameter symbolType, the UE only considers the CSI-RS occasions within either SBFD symbol(s) or non-SBFD symbol(s) of the corresponding LTM candidate cell(s) as indicated by symbolType for the CSI derivation for the CSI report. ==================================unchanged omitted===================================

| Moderater Question 5: Is the TP 5-1 above is acceptable? - If the answer is ‘no’, please decrible the expected implication of taking no action in RAN1, given that leading WG RAN2 has already agreed to support this feature (e.g., if no RAN1 CR is agreed. The feature would not be supported or the Gnb would restrict CSI-RS on non-SBFD sysmobls) |  |  |
| --- | --- | --- |
| Company | View/Positions (Yes or No) | Comments (If a TP is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| Ericsson | Yes |  |
| Nokia |  | There was no discussion or agreement on SBFD in the RAN2 mobility sessions. We are concerned that this may open the door to additional changes in other LTM aspects. Should we send an LS to RAN2 to ask for their views? |
| NTT DOCOMO | Fine, but | We are fine with this TP, but there is little concern that other additional changes would be needed if we open the door. |
| Ofinno | Yes | Although it is true that the RAN2 agreement is not from the RAN2 mobility session, but as FL mentioned, the agreement was achieved by RAN2 normal phase, so we think should be supported. If only concern is additional specification change in LTM aspects, we can make a separate conclusion that there is no more RAN1 spec impact for support SBFD for mobility enhancement point of view. |
| Google |  | We are open to discuss. |
| ZTE |  | Perhaps, whether this issue needs to be discussed in RAN1 should be triggered by RAN2, and it may even require the involvement of people from the SBFD topic to ensure that any related changes to the spec are correct and reasonable. |
| Samsung |  | Need further discussions on this – whether our understanding/interpretation of RAN2 agreement is correct or event relevant here. |
| CATT |  | Open to discuss. |
| Huawei, HiSilicon |  | Such cross WI feature may need new TEI according to guidance from Chair. |

# 6.	Other Text Proposals (TPs)

It was suggested in [Lenovo, 6] that the UE should perform PDSCH rate-matching around the P/SP CSI-RS resources of candidate cells to support L1 measurement and early CSI acquisition. FL considers this to be a new functionality, which does not align with the objectives of the maintenance phase. Moreover, CSI-RS resources for mobility have been available since Rel-15, and the PDSCH does not apply rate matching around them. The PDSCH performance of the serving cell may be substantially reduced when multiple LTM candidate cells are present. Therefore, the underlying motivation and justification for this proposal require further clarification.

Text proposal # 6-1:

#### 7.3.1.5 Mapping to virtual resource blocks The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols yp0, …, yp(Msymbap-1) conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with yp0 to resource elements k',lp,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission; - the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214]; - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled Ues as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS, which is according to clause 7.4.1.5 and not configured by the TRS-ResourceSet IE, if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI for multicast, G-CS-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource. The non-zero-power CSI-RS includes the non-zero-power CSI-RS resource from serving cell and the non-zero-power CSI-RS resources associated with LTM-Candidate. - not used for PT-RS according to clause 7.4.1.2; - not declared as ‘not available for PDSCH according to clause 5.1.4 of [6, TS 38.214]. The mapping to resource elements (k',l)p,μ allocated for PDSCH according to [6, TS 38.214] and not reserved for other purposes shall be in increasing order of first the index k' over the assigned virtual resource blocks, where k'=0 is the first subcarrier in the lowest-numbered virtual resource block assigned for transmission, and then the index l.

| Company | View/Positions (Please indicate your support: Yes, No in this column) | Comments (If a TP text is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| --- | --- | --- |
| Ericsson | No | Agree with moderator: we do not rate match around signals transmitted in other cells. |
| NTT DOCOMO | No | Agree with moderator. |
| CATT | No | Agree with moderator. |

# 7. Proposals for Online Discussion

## Moderator Propsoal 7-1:

- Endorse the following TP to TS38.214 Section 5.2.1.6

5.2.1.6 CSI processing criteria <Unchanged parts are omitted> For a report setting ltm-CSI-ReportConfig configured with reportQuantity is set to 'cri-RI-PMI-CQI': - If the UE is capable of performing CSI measurement for CSI acquisition for candidate cell(s) before receiving the LTM Cell Switch Command MAC CE [10, TS 38.321]: - For a periodic CSI-RS, the CSI-RS resource and CSI-RS ports within the CSI-RS resource are counted as active in a duration of time starting when the periodic CSI-RS resource is configured by higher layer signaling until, and ending at the end of when the reception of the LTM cell switch command MAC CE is applied or a RRC reconfiguration message when the periodic CSI-RS configuration is released, whichever occurs first. - For a semi-persistent CSI-RS, the CSI-RS resource and CSI-RS ports within the CSI-RS resource are counted as active in a duration of time starting from the end of when the activation command MAC CE is applied until, and ending at the end of when the reception of the LTM cell switch command MAC CE or a deactivation command MAC CE is applied, whichever occurs first. - If the UE is only capable of performing CSI measurement for CSI acquisition for a candidate cell (given by Target Configuration ID field in the LTM Cell Switch Command MAC CE) after receiving the LTM Cell Switch Command MAC CE [10, TS 38.321], no CSI-RS resource and CSI-RS ports are counted as active before the reception of the LTM Cell Switch Command MAC CE [10, TS 38.321]. <Unchanged parts are omitted>

## Moderator Propsoal 7-2:

- Endorse the following TP to TS38.213 Section 21

Reason for change: As agreed, for both RACH-based and RACH-less CLTM, activated TCI states (except the determined TCI state, if present) are deactivated, after RACH-based or RACH-less CLTM is performed. However, current SPEC wordings can mislead reader that activated TCI states (except the determined TCI state, if present) are deactivated, after RACH-based or RACH-less CLTM is completed, which prolongs the time of deactivating TCI states. Summary of change: Clarify that for both RACH-based and RACH-less CLTM, activated TCI states (except the determined TCI state, if present) are deactivated, after RACH-based or RACH-less CLTM is performed. Consequences if not approved: Incorrect behavior, which is not aligned with corresponding agreements.

# 21 L1/L2-triggered mobility procedures < Unchanged parts are omitted > After RACH-based conditional LTM cell switch is triggered, all activated TCI states are deactivated. For RACH-less conditional LTM cell switch, the UE determines a TCI state in CandidateTCI-State or CandidateTCI-UL-State to apply for receptions or transmissions on the candidate cell, where a QCL RS index of the TCI state is same as a SS/PBCH block index or a CSI-RS resource index selected in the RACH-less conditional LTM cell switch as described in clause 5.36.3 [11, TS 38.321]. After RACH-less conditional LTM cell switch is triggered, all activated TCI states, other than the TCI state, are deactivated. < Unchanged parts are omitted >

## Moderator Propsoal 7-3:

- Endorse the following TP to TS38.213 Section 21

Reason for change: The currently agreed CR (R1-2508229) does not explicitly define the application duration for the TCI state determined by the UE after a RACH-less CLTM cell switch. Summary of change: Add “before a new TCI state is applied for the candidate cell” as an ending condition of the candidate TCI state application. Consequences if not approved: ### UE may apply mismatched TCI states, resulting in incorrect cell measurements and degraded mobility performance.

==================================unchanged omitted=================================== # 21 L1/L2-triggered mobility procedures ==================================unchanged omitted=================================== The UE does not expect to be indicated quasi co-location 'typeA' properties when a SS/PBCH block is configured as a source RS of the TCI state. The UE applies the CandidateTCI-State and/or CandidateTCI-UL-State, if indicated by the MAC CE, no later than TLTM-RRC-processing+TLTM-processing+Tfirst-RS+TRS-proc+3 msec after the last symbol of a PUCCH or PUSCH with HARQ-ACK information for the PDSCH providing the MAC CE, where TLTM-RRC-processing, TLTM-processing, Tfirst-RS and TRS-proc are defined in [10, TS 38.133]. For RACH-based LTM cell switch [19, TS 38.300], the UE applies the CandidateTCI-State for receptions on the candidate cell, and applies a spatial domain filter corresponding to the CandidateTCI-State or the CandidateTCI-UL-State for transmissions on the candidate cell, that are after the completion of the random access procedure associated with the PRACH transmission on the candidate cell and before a new TCI state is applied for the candidate cell. For RACH-less LTM cell switch [19, TS 38.300], the UE applies the CandidateTCI-State for receptions on the candidate cell and applies a spatial domain filter corresponding to the CandidateTCI-State or the CandidateTCI-UL-State for transmissions on the candidate cell before a new TCI state is applied for the candidate cell. After RACH-based conditional LTM cell switch, all activated TCI states are deactivated. For RACH-less conditional LTM cell switch, the UE determines a TCI state in CandidateTCI-State or CandidateTCI-UL-State to apply for receptions or transmissions on the candidate cell before a new TCI state is applied for the candidate cell, where a QCL RS index of the TCI state is same as a SS/PBCH block index or a CSI-RS resource index selected in the RACH-less conditional LTM cell switch as described in clause 5.36.3 [11, TS 38.321]. After RACH-less conditional LTM cell switch, all activated TCI states, other than the TCI state, are deactivated. ==================================unchanged omitted===================================

##### 5.2.2.5.1 UE assumptions for CQI/PMI/RI calculation If configured to report CQI index, in the CSI reference resource, or in each of the slot(s) associated with a CQI in the predicted CSI, as defined in Clause 5.2.1.4.2, the UE shall assume the following for the purpose of deriving the CQI index, and if also configured, for deriving PMI and RI: - The first 2 OFDM symbols are occupied by control signaling. - The number of PDSCH and DM-RS symbols is equal to 12. - The same bandwidth part subcarrier spacing configured as for the PDSCH reception. - For the LTM-CSI-ReportConfig, for a candidate cell given under LTM-Candidate, the PDSCH configuration corresponds to the initial BWP configuration of the candidate cell. - The bandwidth as configured for the corresponding CQI report. - The IAB-MT shall only assume the frequency resources as indicated by the DL TX power adjustment MAC CE, if indicated for the slot of the CSI reference resource by DL Tx Power Adjustment MAC CE as described in [10, TS 38.321]. - The reference resource uses the CP length and subcarrier spacing configured for PDSCH reception. - For the LTM-CSI-ReportConfig, for a candidate cell given under LTM-Candidate, the PDSCH configuration corresponds to the initial BWP configuration of the candidate cell. - No resource elements used by primary or secondary synchronization signals or PBCH. - Redundancy Version 0. - The ratio of PDSCH EPRE to CSI-RS EPRE is as given in Clause 5.2.2.3.1. - In addition, the IAB-MT shall apply the provided DL TX power adjustment, if indicated for the slot of the CSI reference resource by DL Tx Power Adjustment MAC CE as described in [10, TS 38.321]. - Assume no REs allocated for NZP CSI-RS and ZP CSI-RS. - Assume the same number of front-loaded DM-RS symbols as the maximum front-loaded symbols configured by the higher layer parameter maxLength in DMRS-DownlinkConfig. - For the LTM-CSI-ReportConfig, for a candidate cell given under LTM-Candidate, the DMRS-DownlinkConfig is provided in the PDSCH configuration corresponds to the initial BWP configuration of the candidate cell. - Assume the same number of additional DM-RS symbols as the additional symbols configured by the higher layer parameter dmrs-AdditionalPosition. - For the LTM-CSI-ReportConfig, for a candidate cell given under LTM-Candidate, the dmrs-AdditionalPosition is provided in the DMRS-DownlinkConfig in the PDSCH configuration corresponds to the initial BWP configuration of the candidate cell. - Assume the PDSCH symbols are not containing DM-RS. - Assume PRB bundling size of 2 PRBs. - The PDSCH transmission scheme where the UE may assume that PDSCH transmission would be performed with up to 8 transmission layers as defined in Clause 7.3.1.4 of [4, TS 38.211]. For CQI calculation, the UE should assume that PDSCH signals on antenna ports in the set [1000,…, 1000+ν-1] for ν layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…, 3000+PCSI-RS-1], as given by y3000(i)⋯y3000+PCSI-RS-1(i)=W(i)x0(i)⋯xν-1(i) Where x(i)=x(0)(i)...x(ν-1)(i)T is a vector of PDSCH symbols from the layer mapping defined in Clause 7.3.1.4 of [4, TS 38.211], PCSI-RS∈1,2,4,8,12,16,24,32,48,64,128 is the number of CSI-RS ports. If only one CSI-RS port is configured, W(i) is 1. If the higher layer parameter reportQuantity in CSI-ReportConfig or in LTM-CSI-ReportConfig for which the CQI is reported is set to either 'cri-RI-PMI-CQI' or 'cri-RI-LI-PMI-CQI', W(i) is the precoding matrix corresponding to the reported PMI applicable to x(i). If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-CQI', W(i) is the precoding matrix corresponding to the procedure described in Clause 5.2.1.4.2. If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-i1-CQI', W(i) is the precoding matrix corresponding to the reported i1 according to the procedure described in Clause 5.2.1.4.2. The corresponding PDSCH signals transmitted on antenna ports [3000,…,3000 + PCSI-RS - 1] would have a ratio of EPRE to CSI-RS EPRE equal to the ratio given in Clause 5.2.2.3.1. - For a CSI-ReportConfig or an LTM-CSI-ReportConfig, if a UE is configured with codebookType set to 'typeI-SinglePanel-r19' and with the higher layer parameter typeI-softScalingRank1-2-r19, for υ=1, and υ=2, if supported by UE capability, the UE can assume that the PDSCH signal for each layer mapped to a vector of group Gs(x1,x2) would have the same ratio of EPRE to CSI-RS EPRE for all K CSI-RS resources, equal to sx1,x22/υ times the powerControlOffset (in linear scale) of the respective CSI-RS resource, where Gs(x1,x2) and sx1,x22 are described in Clause 5.2.2.2.1a; otherwise, the UE can assume that the PDSCH signals for υ layers would have the same ratio of EPRE to CSI-RS EPRE for all K CSI-RS resources, equal to the powerControlOffset of the respective CSI-RS resource. - For a CSI-ReportConfig, if a UE is configured with 48, 64 and 128 antenna ports, obtained by aggregating K CSI-RS resources for channel measurement, and codebookType set to 'typeI-MultiPanel-r19', for CQI calculation, the UE should assume that PDSCH signals on antenna ports in the set [1000,…,1000+υ-1], for υ layers, would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…,3000+P-1] of each of the K CSI-RS resources, as given by y03000(i)⋮y03000+P-1(i)y13000(i)⋮y13000+P-1(i)⋮yK-13000(i)⋮yK-13000+P-1(i)=Wix0i⋮xυ-1i where W(i) is the precoding matrix corresponding to the procedure described in Clause 5.2.2.2.2a. A UE can assume that the PDSCH signals for υ layers would have the same ratio of EPRE to CSI-RS EPRE for all CSI-RS resources k=0,…,K-1, equal to the powerControlOffset of the respective CSI-RS resource. - For a CSI-ReportConfig , if a UE is configured with one or more SRS resource sets with higher layer parameter usage set to 'antennaSwitching', with a total of PSRS=6 or 8 ports across the resources intended for xT6R or xT8R, respectively, if the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-CQI' and the UE is configured with the higher layer parameter SRSPortGrouping-r19, the UE can assume that SRS port group 0 corresponds to codeword 0 and comprises the even PSRS/2 ports, and that SRS port group 1 corresponds to codeword 1 and comprises the odd PSRS/2 ports out of the total PSRS ports. The SRS ports are indexed in an ascending order according to SRS resource ID and port number within each SRS resource, for one SRS resource set, or according to SRS resource set ID, SRS resource ID in a set and port number within each SRS resource, for multiple aperiodic SRS resource sets. - For a CSI-ReportConfig , that contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, - If a sub-configuration indicates a CSI-RS antenna port subset using the higher layer bitmap parameter portSubsetIndicator or portSubsetIndicator-r19, as described in clause 5.2.1.4.2, for CQI calculation, antenna ports corresponding to all bits with value of 1 in portSubsetIndicator are mapped to consecutive antenna ports starting at CSI-RS antenna port 3000 in increasing order of the bit position in portSubsetIndicator. The UE should assume that PDSCH signals on antenna ports in the set [1000,…, 1000+ν-1] for ν layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000, …, 3000+P-1] T, as given by y3000i⋯y3000+P-1i=Wix0i⋯xν-1i where P corresponds to the number of bits with value 1 in the bitmap portSubsetIndicator and xi=[x0i…xν-1i]T , and Wi are as previously described in this Clause, and the corresponding PDSCH EPRE to CSI-RS EPRE is as previously defined in this Clause if the sub-configuration does not indicate a power offset powerOffset. - If a sub-configuration indicates a list of NZP CSI-RS resources, provided by nzp-CSI-RS-ResourceList and does not indicate a power offset powerOffset, for CQI calculation for the sub-configuration the UE follows the procedure previously described in this Clause. - If a sub-configuration indicates a power offset powerOffset, for CQI calculation, the UE shall assume the corresponding PDSCH signals transmitted on the antenna ports of a CSI-RS resource would have a ratio of EPRE to CSI-RS EPRE equal to the difference between powerControlOffset of the CSI-RS resource, given in Clause 5.2.2.3.1, and powerOffset, where the difference is expected to take one of the values that can be configured for powerControlOffset of the CSI-RS resource, given in Clause 5.2.2.3.1, and is also expected to take a value that is no larger than the value of powerControlOffset.

# References

R1-2508463	Maintenance of NR mobility enhancements Phase 4	Ericsson

R1-2508505	Measurements related enhancements for LTM	Huawei, HiSilicon

R1-2508531	Maintenance on Mobility Phase 4	ZTE Corporation, Sanechips

R1-2508575	Maintenance on measurements related enhancements for LTM	CATT

R1-2508783	Maintenance on other Rel-19 topics	Samsung

R1-2508953	Maintenance on the measurements for LTM	Lenovo

R1-2509031	Maintenance on mobility enhancement phase 4	Ofinno

R1-2509086	FL Summary #1 of NR Mobility enhancement Phase 4	Moderator (Apple)

R1-2509087	FL Summary #2 of NR Mobility enhancement Phase 4	Moderator (Apple)

R1-2509357	Maintenance on Mobility Phase 4	Google

R1-2508257	Corrections on NR mobility enhancements phase 4, Nokia, RAN1#122bis, October 2025

R2-2506201	Report from session on R18 SL, R18/19 MOB and R19 NES, Vice Chairman, August 25th – 29th, 2025

R2-2506202	Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others, RAN2 Vice Chairman (CATT)