3GPP TSG RAN WG1 #124		R1-2601560

Gothenburg, SE, Feb. 9th ~ 13th, 2026

Source: 	Moderator (Apple)

Title:                     FL Summary #2 of NR Mobility enhancement Phase 4

Agenda item:	8.8

Document for:	Discussion and Decision

# 1 Introduction

This document summarizes the contributions made under the agenda item 7 related to RAN2 LS on CSI-IM resources/sets in LTM-Candidate.

# 2. CSI-IM resources/sets in LTM-Candidate

In [1], the following questions are asked by RAN2:

The mobility tab of the consolidated Rel-19 higher layers parameters list in R1-2508202 includes in LTM-Candidate: - a new field ltm-CSI-IM-ResourceSetToAddModList-r19, which is a list of 1 to maxNrofCSI-IM-ResourceSets (which is 64) of CSI-IM-ResourceSet; - a new field ltm-CSI-IM-ResourceToAddModList-r19, which is a list of 1 to maxNrofCSI-IM-Resources (which is 32) of CSI-IM-Resource. However: - the ltm-CSI-ReportConfig of an LTM-Candidate seems to be the only field that can refer to a CSI-IM-ResourceSet configured using ltm-CSI-IM-ResourceSetToAddModList-r19 in that LTM-Candidate; - a CSI-IM-ResourceSet cannot refer to more than 8 CSI-IM-Resource. Therefore, RAN2 would like to ask RAN1 whether it is the correct understanding that, in one LTM-Candidate, the network needs to configure at most one CSI-IM-ResourceSet and at most 8 CSI-IM-Resource.

Company positions are summarized in Table 1. Note that Spreadtrum [2] discussed the ASN.1 CSI-IM resource set configuration and proposes a RAN1 clarification to resolve them.

Table 1: Summary of company views on CSI-IM resource set configuration

|  | Supported companies |
| --- | --- |
| Opt.1: Confirm RAN2 understanding i.e., in one LTM-Candidate, the network needs to configure at most one LTM-CSI-IM-ResourceSet and each LTM-CSI-IM-ResourceSet can contain a list of at most 8 CSI-IM resources | Nokia [5], vivo [6], Huawei [8], |
| Opt.2: Keep current structure and no need to restrict the gNB to configure only one CSI-IM-ResourceSet. | ZTE [3], CATT [4], Samsung [7] |

In TS 38.331, CSI-IM resource sets follow a two-levels configuration model.

- Level-1: The UE is initially configured with up to 64 CSI-IM-ResourceSets per LTM candidate cell.

- Level-2: Subsequently, a single CSI-IM-Resource set is linked to an ltm-CSI-ReportConfig via a CSI-IM set ID.

Since only a single CSI-IM resource set is utilized for a candidate cell's CQI report in level-2, the requirement to support up to 64 sets in the level-1 configuration step lacks justification, rather causing additional signaling overhead.

While arguments have been made to retain this structure for "MIMO reuse" (Option 2), in moderator’s view, there is a fundamental difference in these two use cases. In MIMO, multiple CSI reports are configured to account for varied transmission assumptions. In contrast, LTM is currently defined for a single CSI report per candidate cell, rendering the additional resource sets unmotivated and computationally inefficient for the UE.

| ## Moderator Proposal - Reply RAN2 LS with the following reponse (i.e., Opt.1): - Confirm RAN2 understanding i.e., in one LTM-Candidate, the network needs to configure at most one LTM-CSI-IM-ResourceSet and each LTM-CSI-IM-ResourceSet can contain a list of at most 8 CSI-IM resources according to UE capability |  |  |
| --- | --- | --- |
| Company | View/Positions (Yes vs. NO) | Comments |

In addition, a draft LS reply was prepared to implement Moderator proposal above:

# 1. Overall Description RAN1 thanks RAN2 for the LS R1-2600006 (R2-2509335) on CSI-IM resources/sets in LTM-Candidate. and would like to provide the following response: Question: RAN2 would like to ask RAN1 whether it is the correct understanding that, in one LTM-Candidate, the network needs to configure at most one CSI-IM-ResourceSet and at most 8 CSI-IM-Resource? Answer: RAN1 confirms the RAN2 understanding i.e., in a LTM-Candidate in which ltm-CSI-ReportConfig is configured, the network needs to configure at most one CSI-IM-ResourceSet containing at most 8 CSI-IM-Resource in the CSI-IM-ResourceSet according to UE capability. # 2. Actions To RAN2 ACTION: RAN1 respectfully asks RAN2 to take the above information into account.

| Company | View/Positions (Yes vs. NO) | Comments |
| --- | --- | --- |

# 3. Proposals for Wensday’s Online Discussion

# References

R1-2600006	LS on CSI-IM resources/sets in LTM-Candidate	RAN2, Huawei

R1-2600246	Discussion on RAN2 LS on CSI-IM resources/sets in LTM-Candidate	Spreadtrum, UNISOC

R1-2600277	Draft reply LS to RAN2 on on CSI-IM resources/sets in LTM-Candidate	ZTE Corporation, Sanechips

R1-2600285	Draft reply LS on CSI-IM resources/sets in LTM-Candidate	CATT

R1-2600374	Discussion on LS on CSI-IM resources/sets in LTM-Candidate	Nokia

R1-2600468	Draft reply LS on CSI-IM resourcessets in LTM-Candidate	vivo

R1-2600718	Draft reply LS on CSI-IM resources/sets in LTM-Candidate	Samsung

R1-2601440	Draft reply to LS on CSI-IM resources/sets in LTM-Candidate	Huawei, HiSilicon