| 3GPP TR 38.765 V0.3.0 (2026-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group RAN Access Network; Study on Integrated Sensing And Communication (ISAC) for NR (Release 20) |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Performance metrics	7

4.1	Definitions of performance metrics	7

4.2	Performance objectives	8

5	Measurements	8

6	Performance evaluation	10

6.1	Evaluation methodologies	10

6.2	Evaluation Assumptions	10

6.3	Performance evaluation results	11

6.3.1	Baseline configuration 1	11

6.3.2	Baseline configuration 2	11

6.3.3	[Configuration x]	11

7	Network architecture	11

8	RAN-CN procedures and signalling	12

9	Conclusions	12

Annex <A>: Evaluation assumptions	13

Annex <B>: Evaluation results	16

Annex <X>: Change history		17

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

Current 5G-Advanced network design focuses primarily on data transmission, and although RAT-based positioning is supported, the specifications do not offer the in-built capability to detect objects that are not connected to the network. Integrated Sensing and Communication (ISAC) involves the simultaneous use of radio frequency (RF) signals for both sensing and communication purposes. If sensing capabilities are integrated into the design of the system, sensing may be offered as a service alongside communications. This integration can lead to improved spectrum efficiency, reduced latency, and enhanced reliability in various applications.

TR 22.837 identifies a very wide range of ISAC use cases, and requirements for such are defined in TS 22.137, in particular, detection and/or tracking of UAV (Uncrewed Aerial Vehicles). SA2 also has an ongoing Rel-20 study on Architecture Enhancement to support Integrated Sensing and Communication (FS_Sensing_ARC, SP-250401).

In Release 19, a channel modelling framework for ISAC to enable evaluation of sensing techniques for such use cases was added to the existing communication models in TR38.901. Both radar cross-section (RCS) and mobility of sensing targets and other objects in the environment was studied in the scope of channel modelling. However, there is a need for further study of ISAC techniques, including network architecture, procedures, signalling, and corresponding performance evaluations for UAV sensing target use cases.

This study item aims to study the following aspects for Integrated Sensing and Communication (ISAC):

Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1]

-	Identify and study metrics, measurements, and relevant measurement quantization for UAV use case

-	As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations.

-	For other waveform and reference signals, companies are to share relevant information

-	No UE impacts

-	Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions.

-	FR1 frequency range is prioritized.

Study the procedures, signaling between RAN and CN to support ISAC [RAN3]

Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3]

-	Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts.

-	No inter-gNB coordination will be studied.

-	Coordination with SA2 as necessary.

1	Scope

The present document is intended to capture the output of study item for "Study on Integrated Sensing And Communication (ISAC) for NR" [2]. CP-OFDM is considered as baseline waveform. The purpose of this TR is to document the following investigations for NR ISAC.

-	Performance evaluation of gNB-based mono-static sensing for UAV use case

-	Performance metrics and related performance objectives

-	Evaluation assumptions

-	Evaluation results

-	Measurements and quantization

-	Procedures and signalling between RAN and CN to support ISAC

-	Network architecture for gNB-based mono-static sensing for UAV use cases

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-252819: "Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR".

[3]	3GPP R1-2600081: "Performance metric, methodologies, and initial evaluation results for ISAC"

[4]	3GPP R1-2600105: "Discussion on evaluation assumptions and performance evaluations on ISAC"

[5]	3GPP R1-2600183: "Discussion of ISAC evaluation in 5GA"

[6]	3GPP R1-2600331: "Discussion on evaluation methodology and evaluation results for R20 ISAC for NR"

[7]	3GPP R1-2600381: "Discussion on sensing measurement and evaluation result"

[8]	3GPP R1-2600446: "Discussion on performance evaluation for ISAC"

[9]	3GPP R1-2600495: "Evaluation methodology and performance evaluation  for 5G-A ISAC"

[10]	3GPP R1-2600574: "Evaluation assumptions and performance evaluation of ISAC for NR"

[11]	3GPP R1-2600620: "Evaluation assumptions and performance evaluations for ISAC NR"

[12]	3GPP R1-2600622: "Performance evaluations of ISAC for NR"

[13]	3GPP R1-2600689: "Discussion on ISAC evaluation assumptions and performance evaluation"

[14]	3GPP R1-2601459: "Discussion on ISAC for NR"

[15]	3GPP R1-2600747: "Discussion on ISAC Evaluation assumptions and performance evaluation"

[16]	3GPP R1-2600846: "Discussion on ISAC Performance Evaluation"

[17]	3GPP R1-2600973: "Discussion on 5G-A ISAC evaluation"

[18]	3GPP R1-2601031: "Discussion on NR ISAC evalution assumption and performance evaluation"

[19]	3GPP R1-2601055: "On ISAC performance evaluations and assumptions"

[20]	3GPP R1-2601125: "On 5G NR ISAC evaluation assumptions and performance evaluation"

[21]	3GPP R1-2601264: "Evaluation Assumptions and performance evaluation for UAV gNB-monostatic sensing"

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

CPI	Coherent Processing Interval

STX	Sensing Transmitter

SRX	Sensing Receiver

UAV	Uncrewed Aerial Vehicles

TRP	Transmission-Reception Point

LCS	Local coordinate system

GCS	Global coordinate system

4	Performance metrics

4.1	Definitions of performance metrics

In the evaluation of NR ISAC, the following performance metrics are considered:

-	Horizontal/vertical positioning accuracy is defined as the absolute value of the difference between the estimated horizontal/vertical position and the corresponding true position of a sensing target.

NOTE:	There should be only one estimated horizontal/vertical position corresponding to the true position of a sensing target.

-	Velocity accuracy is defined as the absolute value of the difference between the estimated velocity and the corresponding true velocity of a sensing target. For single TRP monostatic sensing, both the radial velocity accuracy and the 3D velocity accuracy can be estimated. The true radial velocity is the projection of true 3D velocity on the direction from TRP to target for TRP monostatic.

NOTE:	There should be only one estimated velocity corresponding to the true velocity of a sensing target.

-	Missed detection probability is defined as the conditional probability of not detecting the presence of a target when the target is actually present in the simulation area.

Pmd=n=0N-1DnMnN

where,

-	Dn is the number of missed targets in the drop n, i.e., the true target not associated with any detected object.

-	Mn is the number of true targets in the drop n.

-	N is total number of drops with at least one target per drop

-	False alarm probability Type 1 is defined for cases without true target dropped in simulation area. An object detected when there is no target present in the simulation area is considered a false alarm.

Pf1=n=0N-1QnN

where,

-	Qn is equal to 1 if at least one object is detected when there is no target dropped in the simulation area in the drop n, otherwise Qn is equal to 0.

-	N is the total number of drops without target in the simulation area.

-	False alarm probability Type 2 is defined for cases with true target dropped in simulation area. An object detected but not associated with any true target in the simulation area is considered as a false alarm.

Pf2=0≤n<NMn'≠0Dn'Mn'K

where,

-	Dn' is the number of detected objects but not associated with any true targets in the drop n.

-	Mn' is the total number of detected objects in the drop n.

-	K is number of drops with at least one detected object.

-	N is the total number of drops with target(s) in the simulation area.

NOTE:	Both False alarm probability types are mandatory.

Sensing resolution, sensing service latency and refreshing rate are not considered as performance metrics for the evaluation of NR ISAC.

For the purpose of performance metric calculation, association of the detected object(s) and the true target(s) should fulfil at least the following conditions:

-	One true target is associated with at most one detected object.

-	One detected object is associated with at most one true target.

-	The same association applies to miss detection, false alarm probability Type 2 and positioning/velocity accuracy.

Companies should report the method used for association of the detected object(s) and the true target(s).

4.2	Performance objectives

The following performance objectives are adopted for the evaluation of UAV use case with gNB-based monostatic sensing.

Table 4.2-1: Performance objectives

| Performance metrics | Values |
| --- | --- |
| Missed detection Probability | 5% |
| False Alarm Probability Type 1 | 5% |
| False Alarm Probability Type 2 | 5% |
| Horizontal Positioning Accuracy | 10 m with confidence level 90% |
| Vertical Positioning Accuracy | 10 m with confidence level 90% |
| Velocity Accuracy | 5 m/s with confidence level 90% |
| NOTE: Confidence level of the X% represents X percentile point of the cumulative distribution function (CDF) of the estimation errors |  |

5	Measurements

5.1	Definitions of Levels/Options

From physical layer perspective, the following measurements that may be reported from RAN are identified in the study of NR ISAC.

-	Level A: Raw data per Tx antenna port per OFDM symbol per RX antenna port per TRP for a given time stamp

-	Option A1: Amplitude and phase samples in time/delay domain of the estimated channel, i.e., Amplitude and phase values of channel impulse response

-	Option A2: Amplitude and phase per subcarrier in frequency domain of the estimated channel

-	Level B: Amplitude and phase profile of delay, and/or Doppler, and/or angle per TRP for a given time stamp by using window(s) of the consecutive samples in at least one profile of delay, and/or Doppler and/or angle domain

-	Option B1: Delay-Doppler profile per Tx antenna port per Rx antenna port, which includes the amplitude and phase samples distributed across different delays and Doppler shifts.

-	Option B2: Delay-Angle profile per Tx antenna port per OFDM symbol, which includes the amplitude and phase samples distributed across different delays and spatial angles (e.g., Angle of Arrival).

-	Option B3: Delay-Doppler-Angle profile per Tx antenna port, which includes the amplitude and phase samples distributed across delay, Doppler, and angle domains.

-	Option B4: Delay profile per Tx antenna port per OFDM symbol per Rx antenna port, which includes the amplitude and phase samples distributed across different delays.

NOTE:	Level B is applicable for either LCS or GCS

-	Level C: per detected path/point measurements per Tx antenna port per TRP which may be reflected/scattered from scattering point(s).

-	Option C1: Delay/range, Doppler/radial velocity, one or multiple 3D angles, metric related to power, and confidence metric per detected path for a given time stamp. A path is associated with one set {delay/range, Doppler/velocity, metric related to power, and confidence metric}, and one or multiple 3D angles.

-	Option C2: Doppler/radial velocity, position, metric related to power, and confidence metric per detected path for a given time stamp. A path is associated with one set {Doppler/velocity, metric related to power, and confidence metric}, one or multiple positions

-	Option C3: Delay/range, Doppler/radial velocity, 3D angle, metric related to power, and confidence metric per detected path for a given time stamp. A path is associated with one delay, and one or multiple sets {Doppler/velocity, 3D angle, metric related to power, and confidence metric}

-	Option C4: Delay/range, Doppler/radial velocity, 3D angle, metric related to power, and confidence metric per detected point for a given time stamp. A point is associated with one range/delay, one Doppler/velocity, one 3D angle

-	Option C5: Position, radial velocity, metric related to power, and confidence metric per detected point for a given time stamp. A point is associated with one position, one velocity.

NOTE:	Position can be defined in either LCS or GCS. Angle can be defined in either LCS or GCS. 3D angle refers to a pair of horizontal and vertical angles.

-	Level D: Object/target level measurement per gNB.

-	Option D1: Only one value set {position, 3D velocity, metric related to power (if applicable), confidence metric} in GCS and optional radial velocity for a given time stamp is reported for a detected object/target. The association of multiple measurements across different time stamps for the same detected object/target is not reported.

-	Option D2: Only one value set {position, 3D velocity, metric related to power (if applicable), confidence metric} in GCS for a given time stamp is reported for a detected object/target. The association of multiple measurements across different time stamps for the same detected object/target is reported.

-	Option D3: One or more value sets {position, 3D velocity, metric related to power (if applicable), confidence metric} in GCS for a given time stamp are reported for a detected object/target. The association of multiple measurements across different time stamps for the same detected object/target is reported.

-	Option D4: One or more value sets {position, 3D velocity, metric related to power (if applicable), confidence metric} in GCS and optional radial velocity for a given time stamp are reported for a detected object/target. The association of multiple measurements across different time stamps for the same detected object/target is not reported.

NOTE:	From physical layer perspective, gNB can associate with one or multiple TRPs for Level D.

NOTE:	The metric related to power may be different from those defined for communication and between Level C and Level D

NOTE:	For Level A/B/C, Tx antenna port means reference signal antenna port for sensing purpose.

The definition on time stamp from existing specifications, e.g., TS 38.455, TS 38.355, TS 37.355 are reused as baseline.

5.2	Measurement quantization

From physical layer perspective, the following methods can be considered to define the quantization for the measurements of Level A/B, e.g., floating point, uniform scalar, non-uniform scalar. Other quantization method is not precluded.

From physical layer perspective, on measurement quantization of Level C/D for position/velocity measurement and confidence metric, quantization methods in existing specifications, e.g., TS 23.032, TS 38.455, TS 38.355, TS 37.355 can be considered.

From physical layer perspective, on measurement quantization of Level C for delay/angle measurement, quantization methods in existing specifications, e.g., TS 38.133 can be considered. For Doppler measurement, same number of quantized bits as radial velocity can be considered.

5.3	Assistance information

For Level A/B, at least the following examples of assistance information are identified as needed to assist the further processing at Sensing Function. Additional assistance information is not precluded. The assistance information may or may not be provided by RAN.

-	Level A

-	Location of TRP

-	Step size in delay domain if samples in delay domain is reported

-	Time information related to OFDM symbols used for measurement

-	Antenna configuration and/or beam information

-	Level B

-	Location of TRP

-	LCS to GCS transformation information

-	Information related to sampling of delay, Doppler and/or angle profile

For Level C, at least the following examples of assistance information are identified as needed to assist the further processing at Sensing Function. The assistance information may or may not be provided by RAN.

-	Location of TRP if at least delay/range, Doppler/radial velocity or angle is reported

-	If a measurement is reported in LCS, LCS to GCS transformation information

For Level D, if radial velocity is not reported, no assistance information is identified from physical layer perspective.

5.4	Payload size estimation

The payload size estimation of each measurement level/option for NR ISAC can be done by the following formulas.

-	Level A: Msubcarrier* Ncpi *Mtxport*Nrxport*Ntrp*(Nquan1+ Nquan2)

where,

-	Msubcarrier: number of subcarriers in an OFDM symbol

-	Ncpi: number of OFDM symbols within CPI

-	Mtxport: number of reference signal antenna port for sensing purpose

-	Nrxport: number of Rx antenna ports

-	Ntrp: number of TRPs for payload size calculation

-	Nquan1: number of quantized bits for amplitude

-	Nquan2: number of quantized bits for angle

-	Level B:

-	B1: Mdelay *Ndoppler *Mtxport* Nrxport *Ntrp *(Nquan1+ Nquan2)

-	B2: Mdelay * Ncpi *Mtxport*Nangle *Ntrp *(Nquan1+ Nquan2)

-	B3: Mdelay *Ndoppler *Mtxport*Nangle *Ntrp *(Nquan1+ Nquan2)

-	B4: Mdelay *Ncpi*Mtxport*Nrxport *Ntrp *(Nquan1+ Nquan2)

where,

-	Mdelay: number of delay samples in the window

-	Ndoppler: number of Doppler samples in the window

-	Nangle: number of angle samples in the window

-	Level C:

-	C1/C2/C3: Np1*Ntrp *i=1NparamNquan,i

-	C4/C5: Np2*Ntrp *i=1NparamNquan,i

where,

-	Np1 is total number of paths

-	Np2 is total number of points

-	Nparam is 5 including e.g.,

-	C1/C3/C4: delay, Doppler, horizontal angle, vertical angle, power measurement

-	C2/C5: position (x, y, z), radial velocity, power measurement

-	Nquan,i: number of quantized bits

-	Ntrp: number of TRPs for payload size calculation

-	Level D:

-	D1/D2: Nobject*Ngnb *i=1NparamNquan,i

-	D3/D4: Mpoints* Ngnb *i=1NparamNquan,i

where,

-	Nobject is total number of targets

-	Mpoints is total number of scattering points of all targets

-	Ngnb is fixed to 1

The following assumptions on the parameters can be used to estimate payload size of different Levels/Options of measurements for NR ISAC for calculation purpose.

Table 5.4-1: Assumptions on payload size estimation

| Parameters | Assumptions | Relation to maximum payload calculation |
| --- | --- | --- |
| Nsubcarriers | 3276 for 100MHz, 30kHz SCS |  |
| Ncpi | Min 64, medium 160, max 320 symbols per CPI |  |
| Mtxport | Min 1, medium 4, max 9 | See note 1 |
| Nrxport | 64 |  |
| Ntrp | 3 |  |
| Ngnb | 1 |  |
| Nquan1, i.e., amplitude | Min 16, medium 16, max 32 |  |
| Nquan2, i.e., phase | Min 16, medium 16, max 32 |  |
| Nquan,i | Min 16, medium 16, max 32 |  |
| Mdelay | Min 288 (CP length, UMa) Medium/max: 467 for 2*570m (UMa) Max (RMa): 1436 for 2*1732m |  |
| Ndoppler | same as Ncpi |  |
| Nangle | Level B2/B3: 32, 360 | see note 1, 2 |
| Np | 30, 60, 120 | to calculate min/medium/max for Level-C see note 1 |
| Nobject for D1/D2 | 15 | see note 1 |
| Mpoints for D3/D4 | 20 | see note 1 |
| NOTE 1: value obtained based on evaluation assumptions. NOTE 2: 32 assumes non-uniform sampling. 360 assumes uniform oversampling. |  |  |

-	For Level C

-	Nparam=5 parameters for measurements

-	For Level D

-	Nparam=7 parameters for measurements

-	for D2/D3/D4, one more parameter of target ID is assumed

NOTE:	For Level B2/B3, minimum value and maximum value can be different between non-uniform sampling case and uniform oversampling case

NOTE:	From physical layer perspective, it does not require all the parameters to take the minimum values at the same time or the maximum values at the same time

NOTE:	The above does not take into account the potential impact from measurement quantization on the sensing performance.

NOTE:	The above does not take into account the potential impact of different Levels/Options on the sensing performance.

6	Performance evaluation

6.1	Evaluation methodologies

The following general procedure for performance evaluation of NR ISAC is provided.

1)	Simulation parameter configuration

2)	Sensing scenario generation, including the deployment of sensing Tx/Rx (STXs/SRXs)

3)	Dropping N target(s), where N is equal to 0 or larger than 0

4)	Channel generation and STX/SRX determination for the targets

5)	Sensing signal generation and passing the sensing signal to the generated channel

6)	Sensing signal processing at each SRX, optionally, sensing signal processing based on fusion from multiple STXs/SRXs

-	E.g., optionally, sensing signal processing based on tracking

7)	Sensing performance metric calculation.

6.2	Evaluation Assumptions

The evaluation assumptions for the evaluation of UAV use case with gNB-based monostatic sensing are provided in Annex A. In order to define the sensing resource ratio used in the evaluation of NR ISAC, three kinds of resources are defined

-	Type_1: Resources that are used for sensing signal transmission

-	Type_2: Part of Type_1 resources that are used for communication purpose

NOTE:	It is possible the Type_2 resource doesn’t exist.

-	Type_3: Resources that are not used for sensing signal transmission, and cannot be used for communication purpose due to sensing operation

Two options are provided to calculate the sensing resource ratio. Both options should be reported by companies.

-	Option 1: (Type_1 + Type_3) resources over all radio DL and UL resources

-	Option 2: (Type_1 - Type_2 + Type_3) resources over all radio DL and UL resources

NOTE:	If Type_2 resource doesn’t exist, two options are the same

In the evaluation on NR ISAC, company should report which sensing RS resources are considered as Type 2 resource and related reason.

Two baseline configurations of the following parameters are defined to respectively analyse the evaluation results. All combinations of parameters other than the two baseline configurations are categorized as 'Other configurations'.

Table 6.2-1: Baseline configurations of evaluation assumptions

|  | Parameters | Baseline configuration 1 | Baseline configuration 2 |
| --- | --- | --- | --- |
| 1 | Scenario | (UMa-AV, 500m) | (UMa-AV, 500m) |
| 2 | Sensing Tx/Rx operating simultaneously or not | Sensing Tx/Rx operating simultaneously | Sensing Tx/Rx operating simultaneously |
| 3 | Carrier frequency | 4 or 4.9GHz | 4 or 4.9GHz |
| 4 | Max BS Tx power | 52 dBm | 37 dBm |
| 5 | BS antenna configuration | (M, N, P, Mg, Ng, Mp, Np) Tx: (8,8,2,1,1;4,8) Rx: (8,8,2,1,1;4,8) dH,dV = (0.5, 0.8)λ, +45°/-45° polarization | (M, N, P, Mg, Ng, Mp, Np) Tx: (8,8,2,1,1;4,8) Rx: (8,8,2,1,1;4,8) dH,dV = (0.5, 0.8)λ, +45°/-45° polarization |
| 6 | Number of targets per sector in the center site | N=5 | N=5 |
| 7 | Target vertical distribution | 25-300m | 25-300m |

6.3	Performance evaluation results

### 6.3.0	Introduction

In Clause 6.3, a set of reported performance metrics obtained by a combination of all evaluation parameters from a company, i.e., a row in the excel sheets “Baseline 1”, “Baseline 2” and “Other configurations” in R1-2601610, is referred as a result from a source.

NOTE:	The reported results do not compare different quantization of measurement report Level A/B/C/D.

In the evaluation, the Tx beam(s) at TRP are generated based on the BS antenna configurations provide in Table A-1 for sensing purpose. The number of horizontal Tx beams (m) and the number of vertical Tx beams (n) are provided in the format “m x n”. The value m or n is equals to 1 respectively indicates wide Tx beam in horizontal or vertical direction. Otherwise, directional Tx beam is respectively applied. The performance of communication is not evaluated.

6.3.1	Baseline configuration 1

17 sources ([3, 4, 5, 6, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 19, 20, 21]) report 46 results using baseline configuration 1. 21 results from 11 sources ([3, 5, 6, 8, 9, 13, 15, 16, 17, 20, 21]) show that all performance objectives can be met simultaneously, of which 15 results from 9 sources ([3, 5, 6, 8, 9, 15, 16, 17, 21]) model a target in the channels of multiple or all TRP. 22 results from 9 sources ([4, 8, 10, 11, 12, 14, 15, 16, 19]) show that not all performance objectives can be met simultaneously, of which 20 results from 8 sources ([4, 8, 10, 11, 12, 15, 16, 19]) model a target in the channel of single TRP.

The 46 results are categorized into 6 cases as provided in Table 6.3-1.

Table 6.3-1: 6 Cases for baseline configuration 1

|  | Number of TRPs of which the channel of a target is modelled | Sensing resource ratio | Number of horizontal and vertical Tx beams (m x n) |
| --- | --- | --- | --- |
| Case 1-1 | Multiple or all | <=10% | 1x1 |
| Case 1-2 | Multiple or all | <=10% | 1x2, 1x6, 2x4 |
| Case 1-3 | Multiple or all | >10% | 1x1 |
| Case 1-4 | Single | <=10% | 1x1 |
| Case 1-5 | Single | <=10% | 1x6 |
| Case 1-6 | Single | >10% | 1x1 |

For Case 1-1, with CPI up to 160ms, and self-interference model X = -Inf or 5, 8 results from 3 sources ([8, 14, 21]) provide evaluation results.

-	6 results from 2 sources ([8, 21]) show that all performance objectives can be met simultaneously

-	For Horizontal Positioning Accuracy @90%, the reported values range from 1.01 to 2.01 m

-	For Vertical Positioning Accuracy @90%, the reported values range from 0.56 to 1.64 m

-	For the 3D velocity Accuracy @90%, with the reported values range from 0.35 to 0.58 m/s

-	For Missed Detection Probability, the reported values range from 1.01% to 4.00%

-	For False Alarm Probability Type 1, the reported values range from 0.00% to 0.00%

-	For False Alarm Probability Type 2, the reported values range from 0.00% to 3.50%

-	2 results from 1 source ([14]) show that only the velocity Accuracy @90% cannot meet the performance objective. The reported values range from 52.19 to 56.28 m/s.

For Case 1-2, with CPI up to 64ms, and self-interference model X = -Inf or 5, 7 results from 5 sources ([3, 5, 6, 9,17]) provide evaluation results showing that all performance objectives can be met simultaneously.

-	For Horizontal Positioning Accuracy @90%, the reported values range from 0.697 to 2.50 m

-	For Vertical Positioning Accuracy @90%, the reported values range from 0.24 to 2.40 m

-	For 3D velocity Accuracy @90%, the reported values range from 0.0219 to 1.41 m/s

-	For Missed Detection Probability, the reported values range from 0.00% to 4.70%

-	For False Alarm Probability Type 1, the reported values range from 0.00% to 0.00%

-	For False Alarm Probability Type 2, the reported values range from 0.00% to 2.70%

For Case 1-3, with CPI up to 128 ms, and self-interference model X = -Inf or 5, 2 results from 2 sources ([15, 16]) provide evaluation results showing that all performance objectives can be met simultaneously.

-	For Horizontal Positioning Accuracy @90%, the reported values range from 2.05 to 4.30 m

-	For Vertical Positioning Accuracy @90%, the reported values range from 2.26 to 2.3 m

-	For 3D velocity Accuracy @90%, the reported values range from 0.48 to 2.8 m/s

-	For Missed Detection Probability, the reported values range from 1.40% to 2.60%

-	For False Alarm Probability Type 1, the reported values range from 0% to 0%

-	For False Alarm Probability Type 2, the reported values range from 1.60% to 2.30%

For Case 1-4, with CPI up to 160ms, and self-interference model X = {-Inf, 0, 5,10}, 20 results from 6 sources ([8, 10, 12, 13, 15, 19]) provide evaluation results.

-	3 results from 1 source ([13]) show that all performance objectives can be met simultaneously. The self-interference is modelled as X= -Inf.

-	For Horizontal Positioning Accuracy @90%, the reported values range from 2.45 to 3.73 m

-	For Vertical Positioning Accuracy @90%, the reported values range from 1.28 to 1.95 m

-	For Radial velocity Accuracy @90%, the reported values range from 3.08 to 4.63 m/s

-	For Missed Detection Probability, the reported values range from 4.31% to 5.00%

-	For False Alarm Probability Type 1, the reported values range from 0.00% to 0.67%

-	For False Alarm Probability Type 2, the reported values range from 1.15% to 3.37%

-	17 results from 5 sources ([8, 10, 12, 15, 19]) show that not all performance objectives can be met simultaneously

-	3 results from 3 sources ([8, 12, 15]) show that Missed Detection Probability and False Alarm Probability Type 2 cannot meet the performance objectives

-	For Missed Detection Probability, the reported values range from 9.21% to 23.05%

-	For False Alarm Probability Type 2, the reported values range from 6.03% to 36.41%

-	10 results from 2 sources ([10, 15]) show that only Missed Detection Probability cannot meet the performance objective. The reported values range from 5.17% to 12.40%

-	3 results from 1 source ([15]) show that only False Alarm Probability Type 2 cannot meet the performance objective. The reported values range from 5.70% to 8.10%

-	1 result ([19]) shows that Horizontal Positioning Accuracy @90%, Missed Detection Probability, False Alarm Probability Type 1, False Alarm Probability Type 2 cannot meet the performance objectives. The result is generated with clutter mobility and low power clusters enabled.

-	For Horizontal Positioning Accuracy @90%, the reported value is 11.1 m

-	For Missed Detection Probability, the reported value is 20.00%

-	For False Alarm Probability Type 1, the reported value is 91.00%

-	For False Alarm Probability Type 2, the reported value is 79.00%

For Case 1-5, with CPI up to 60ms, and self-interference model X = -Inf, 1 result from 1 source ([5]) provides evaluation results showing that all performance objectives can be met simultaneously

-	For Horizontal Positioning Accuracy @90%, the reported value is 4.60 m

-	For Vertical Positioning Accuracy @90%, the reported value is 3.30 m

-	For the radial velocity Accuracy @90%, the reported value is 0.21 m/s

-	For Missed Detection Probability, the reported value is 1.20%

-	For False Alarm Probability Type 1, the reported value is 0.00%

-	For False Alarm Probability Type 2, the reported value is 4.50%

For Case 1-6, with CPI up to 128ms, and self-interference model X = -Inf, 5 results from 5 sources ([4, 11, 13, 16, 20]) provide evaluation results.

-	2 results ([13, 20]) show that all performance objectives can be met simultaneously

-	For Horizontal Positioning Accuracy @90%, the reported values range from 1.33 to 3.83 m

-	For Vertical Positioning Accuracy @90%, the reported values range from 0.99 to 1.56 m

-	For the radial velocity Accuracy @90%, the reported values range from 0.34 to 4.17 m/s

-	For Missed Detection Probability, the reported values range from 1.60% to 3.85%

-	For False Alarm Probability Type 1, the reported values range from 0.00% to 0.50%

-	For False Alarm Probability Type 2, the reported values range from 1.60% to 2.46%

-	3 results ([4, 11, 16]) show that Missed Detection Probability and False Alarm Probability Type 2 cannot meet the performance objectives

-	For Missed Detection Probability, the reported values range from 9.0% to 35.00%

-	For False Alarm Probability Type 2, the reported values range from 7.62% to 32.80%

6.3.2	Baseline configuration 2

15 sources ([3, 5, 6, 7, 8, 9, 10, 11, 14, 15, 16, 17, 18, 20, 21]) report 29 results using baseline configuration 2. 13 results from 9 sources ([3, 5, 6, 8, 9, 17, 18, 20, 21]) show that all performance objectives can be met simultaneously, of which 11 results from 7 sources ([3, 6, 8, 9, 17, 18, 21]) model a target in the channels of multiple TRP. 16 results from 9 sources ([7, 8, 10, 11, 14, 15, 16, 18, 21]) show that not all performance objectives can be met simultaneously, of which 12 results from 7 sources ([10, 15, 7, 8, 11, 18, 16]) model a target in the channel of single TRP.

The 29 results are categorized into 6 cases as provided in Table 6.3-2.

Table 6.3-2: 6 Cases for baseline configuration 2

|  | Number of TRPs of which the channel of a target is modelled | Sensing resource ratio | Number of horizontal and vertical Tx beams (m x n) |
| --- | --- | --- | --- |
| Case 2-1 | Multiple or all | <=10% | 1x1 |
| Case 2-2 | Multiple or all | <=10% | 1x2, 1x6, 2x4 |
| Case 2-3 | Multiple or all | >10% | 1x1 |
| Case 2-4 | Single | <=10% | 1x1 |
| Case 2-5 | Single | <=10% | 1x6 |
| Case 2-6 | Single | >10% | 1x1 |

For Case 2-1, with CPI up to 160ms, and self-interference model X = -Inf or 5, 6 results from 3 sources ([8, 14, 21]) provide evaluation results.

-	3 results from 2 sources ([8, 21]) show that all performance objectives can be met simultaneously

-	For Horizontal Positioning Accuracy @90%, the reported values range from 2.29 to 4.84 m

-	For Vertical Positioning Accuracy @90%, the reported values range from 1.38 to 3.04 m

-	For 3D velocity Accuracy @90%. the reported values range from 0.44 to 0.78 m/s

-	For Missed Detection Probability, the reported values range from 3.06% to 4.96%

-	For False Alarm Probability Type 1, the reported values range from 0.00% to 0.00%

-	For False Alarm Probability Type 2, the reported values range from 1.39% to 3.94%

-	3 results from 2 sources ([14, 21]) show that not all performance objectives can be met simultaneously

-	1 result ([14]) shows that 3D velocity Accuracy @90% and Missed Detection Probability cannot meet the performance objectives

-	For the true velocity Accuracy @90%, the reported value is 45.01 m/s

-	For Missed Detection Probability, the reported value is 6.76%

-	1 result ([14]) shows that only 3D velocity Accuracy @90% cannot meet the performance objectives. The reported value is 49.44 m/s

-	1 result ([21]) shows that only Missed Detection Probability cannot meet the performance objectives. The reported value is 10.0%

For Case 2-2, with CPI up to 160ms, and self-interference model X = -Inf or 5, 6 results from 4 sources ([3, 6, 9,17]) provide evaluation results showing that all performance objectives can be met simultaneously

-	For Horizontal Positioning Accuracy @90%, the reported values range from 1.58 to 3.695 m

-	For Vertical Positioning Accuracy @90%, the reported values range from 0.72 to 1.997 m

-	For 3D velocity Accuracy @90%, the reported values range from 0.0217 to 1.77 m/s

-	For Missed Detection Probability, the reported values range from 1.27% to 4.67%

-	For False Alarm Probability Type 1, the reported values range from 0.00% to 0.00%

-	For False Alarm Probability Type 2, the reported values range from 0.00% to 4.20%

For Case 2-3, with CPI up to 160ms, and self-interference model X = -Inf or 5, 3 results from 2 sources ([16, 18]) provide evaluation results.

-	2 results from 1 source ([18]) show that all performance objectives can be met simultaneously

-	For Horizontal Positioning Accuracy @90%, the reported values range from 0.57 to 0.58 m

-	For Vertical Positioning Accuracy @90%, the reported values range from 0.68 to 0.91 m

-	For 3D velocity Accuracy @90%, the reported values range from 0.02 to 0.03 m/s

-	For Missed Detection Probability, the reported values range from 0.30% to 2.40%

-	For False Alarm Probability Type 1, the reported values range from 0.00% to 0.00%

-	For False Alarm Probability Type 2, the reported values range from 0.30% to 0.50%

-	1 result ([16]) shows that only Missed Detection Probability cannot meet the performance objectives

-	For Missed Detection Probability, the reported value is 6.76%

For Case 2-4 with CPI up to 160ms, and self-interference model X = {-Inf, 0, 5}, 5 results from 3 sources ([8, 10, 15]) provide evaluation results showing that not all performance objectives can be met simultaneously

-	1 result ([8]) shows that Missed Detection Probability and False Alarm Probability Type 2 cannot meet the performance objectives

-	For Missed Detection Probability, the reported value is 16.0%

-	For False Alarm Probability Type 2, the reported value is 32.13%

-	4 results from 2 sources ([10, 15]) show that only Missed Detection Probability cannot meet the performance objectives

-	For Missed Detection Probability, the reported values range from 12.6% to 23.0%

For Case 2-5, with CPI up to 160ms, and self-interference model X = -Inf, 5 results from 2 sources ([5, 7]) provide evaluation results showing that not all performance objectives can be met simultaneously

-	1 result ([5]) shows that all performance objectives can be met simultaneously

-	For Horizontal Positioning Accuracy @90%, the reported value is 5.2 m

-	For Vertical Positioning Accuracy @90%, the reported value is 3.4 m

-	For the radial velocity Accuracy @90%, the reported value is 0.22 m/s

-	For Missed Detection Probability, the reported value is 3.20%

-	For False Alarm Probability Type 1, the reported value is 0.00%

-	For False Alarm Probability Type 2, the reported value is 4.60%

-	4 results from 1 source ([7]) shows that not all performance objectives can be met simultaneously

-	1 result ([7]) shows that Missed Detection Probability and False Alarm Probability Type 2 cannot meet the performance objectives

-	For Missed Detection Probability, the reported value is 12.0%

-	For False Alarm Probability Type 2, [1] sources (7(1)) report value is 5.71%

-	3 results ([7]) show that Missed Detection Probability cannot meet the performance objectives. The reported values range from 30.6% to 42.0%

For Case 2-6, with CPI up to 160ms, and self-interference model X = -Inf or 5, 4 results from 4 sources ([11, 16, 18, 20]) provide evaluation results.

-	1 result ([20]) shows that all performance objectives can be met simultaneously

-	For Horizontal Positioning Accuracy @90%, the reported value is 1.27 m

-	For Vertical Positioning Accuracy @90%, the reported value is 0.94 m

-	For the radial velocity Accuracy @90%, the reported value is 0.34 m/s

-	For Missed Detection Probability, the reported value is 1.30%

-	For False Alarm Probability Type 1, the reported value is 0.00%

-	For False Alarm Probability Type 2, the reported value is 1.40%

-	3 results from 3 sources ([11, 16, 18]) show that Missed Detection Probability and False Alarm Probability Type 2 cannot meet the performance objectives

-	For Missed Detection Probability, the reported values range from 12.30% to 37.80%

-	For False Alarm Probability Type 2, the reported values range from 6.59% to 24.60%

6.3.3	Other configurations

11 sources ([3, 7, 8, 10, 12, 13, 15, 16, 17, 18, 20]) report 55 results using other configurations. 34 results from 8 sources ([3, 8, 13, 15, 16, 17, 18, 20]) show that all performance objectives can be met simultaneously, of which 27 results from 5 sources ([3, 8, 16, 17, 18]) model a target in the channels of multiple TRP. 17 results from 6 sources ([10, 12, 15, 16, 17, 18]) show that not all performance objectives can be met simultaneously, of which 13 results from 5 sources ([10, 12, 15, 16, 18]) model a target in the channel of single TRP.

The 55 results are categorized into 12 cases as provided in Table 6.3-3.

Table 6.3-3: 12 Cases for other configurations

|  | Scenario | Number of TRPs of which the channel of a target is modelled | Frequency (GHz) | BS Tx power | Sensing resource ratio | Number of horizontal and vertical Tx beams (m x n) |
| --- | --- | --- | --- | --- | --- | --- |
| Case 3-1 | UMa-AV | Multiple or all | 6 | 52 dBm | <=10% | 1x1 |
| Case 3-2 | UMa-AV | Multiple or all | 4, 4.9, 6 | 52 dBm | <=10% | 1x2, 2x4 |
| Case 3-3 | UMa-AV | Multiple or all | 4 | 52 dBm | >10% | 1x1 |
| Case 3-4 | UMa-AV | Multiple or all | 6 | 37 dBm | <=10% | 1x1 |
| Case 3-5 | UMa-AV | Multiple or all | 4, 4.9, 6 | 37 dBm | <=10% | 1x2, 5, 2x4 |
| Case 3-6 | UMa-AV | Multiple or all | 4.9 | 37 dBm | >10% | 1x1 |
| Case 3-7 | UMa-AV | Single | 4, 6 | 52 dBm | <=10% | 1x1 |
| Case 3-8 | UMa-AV | Single | 4, 6 | 52 dBm | >10% | 1x1 |
| Case 3-9 | UMa-AV | Single | 4.9 | 37 dBm | <=10% | 3x3, 3x1 |
| Case 3-10 | UMa-AV | Single | 4.9, 6 | 37 dBm | >10% | 1x1 |
| Case 3-11 | RMa-AV | Multiple or all | 4, 4.9 | 37, 56 dBm | <=10% | 5, 2x4 |
| Case 3-12 | UMi-AV | Single | 30 | 30 dBm | <=10% | 10x6, 10x11, 20x6, 20x11 |
| NOTE: In the 12 Cases, at least one parameter as listed in Table 6.2-1 is different from the two baseline configurations |  |  |  |  |  |  |

For Case 3-1, with CPI up to 160ms, and self-interference model X = -Inf, 1 result from 1 source ([8]) provides evaluation results showing that all performance objectives can be met simultaneously

-	For Horizontal Positioning Accuracy @90%, the reported value is 0.86 m

-	For Vertical Positioning Accuracy @90%, the reported value is 0.52 m

-	For 3D velocity Accuracy @90%, the reported value is 0.29 m/s

-	For Missed Detection Probability, the reported value is 0.89%

-	For False Alarm Probability Type 1, the reported value is 0.00%

-	For False Alarm Probability Type 2, the reported value is 0.00%

For Case 3-2, with CPI up to 160ms, and self-interference model X =-Inf or 5, 5 results from 2 sources ([3, 17]) provide evaluation results showing that all performance objectives can be met simultaneously

-	For Horizontal Positioning Accuracy @90%, the reported values range from 0.26 to 1.092 m

-	For Vertical Positioning Accuracy @90%, the reported values range from 0.12 to 0.754 m

-	For 3D velocity Accuracy @90%, the reported values range from 0.021 to 0.534 m/s

-	For Missed Detection Probability, the reported values range from 0.80% to 3.50%

-	For False Alarm Probability Type 1, the reported values range from 0.00% to 0.007%

-	For False Alarm Probability Type 2, the reported values range from 0.17% to 4.10%

For Case 3-3, with CPI up to 160ms, and self-interference model X = -Inf, 1 result from 1 source ([16]) provides evaluation results showing that all performance objectives can be met simultaneously

-	For Horizontal Positioning Accuracy @90%, the reported value is 4.3 m

-	For Vertical Positioning Accuracy @90%, the reported value is 2.3 m

-	For 3D velocity Accuracy @90%, with value is 2.8 m/s

-	For Missed Detection Probability, the reported value is 2.60%

-	For False Alarm Probability Type 1, the reported value is 0.00%

-	For False Alarm Probability Type 2, the reported value is 1.60%

For Case 3-4, with CPI up to 160ms, and self-interference model X = -Inf, 1 result from 1 source ([8]) provides evaluation results showing that all performance objectives can be met simultaneously

-	For Horizontal Positioning Accuracy @90%, the reported value is 2.47 m

-	For Vertical Positioning Accuracy @90%, the reported value is 1.91 m

-	For 3D velocity Accuracy @90%, the reported value is 0.4 m/s

-	For Missed Detection Probability, the reported value is 4.07%

-	For False Alarm Probability Type 1, the reported value is 0.00%

-	For False Alarm Probability Type 2, the reported value is 0.43%

For Case 3-5, with CPI up to 160ms, self-interference model X = 5, 9 results from 2 source ([3, 17]) provide evaluation results showing that all performance objectives can be met simultaneously

-	For Horizontal Positioning Accuracy @90%, the reported values range from 0.84 to 3.307 m

-	For Vertical Positioning Accuracy @90%, the reported values range from 0.51 to 1.981 m

-	For 3D velocity Accuracy @90%, the reported values range from 0.031 to 0.804 m/s

-	For Missed Detection Probability, the reported values range from 1.50% to 4.90%

-	For False Alarm Probability Type 1, the reported values range from 0.00% to 0.002%

-	For False Alarm Probability Type 2, the reported values range from 0.00% to 4.60%

For Case 3-6, with CPI up to 160ms, and self-interference model X =5, 2 results from 1 source ([18]) provide evaluation results showing that all performance objectives can be met simultaneously

-	For Horizontal Positioning Accuracy @90%, the reported values range from 0.62 to 0.87 m

-	For Vertical Positioning Accuracy @90%, the reported values range from 1.22 to 1.75 m

-	For 3D velocity Accuracy @90%, the reported values range from 0.02 to 0.04 m/s

-	For Missed Detection Probability, the reported values range from 0.50% to 2.60%

-	For False Alarm Probability Type 1, the reported values range from 0.00% to 0.00%

-	For False Alarm Probability Type 2, the reported values range from 0.40% to 1.10%

For Case 3-7, with CPI up to 160ms, and self-interference model X = {-Inf, 0, 5}, 8 results from 4 sources ([10, 12, 13, 15]) provide evaluation results

-	3 results from 2 sources ([13, 15]) show that all performance objectives can be met simultaneously

-	For Horizontal Positioning Accuracy @90%, the reported values range from 2.63 to 3.68 m

-	For Vertical Positioning Accuracy @90%, the reported values range from 1.07 to 2.19 m

-	For the radial velocity Accuracy @90%, the reported values range from 0.25 to 4.51 m/s

-	For Missed Detection Probability, the reported values range from 4.60% to 4.91%

-	For False Alarm Probability Type 1, the reported values range from 0.00% to 0.00%

-	For False Alarm Probability Type 2, the reported values range from 2.38% to 4.70%

-	5 results from 3 sources ([10, 12, 15]) show that not all performance objectives can be met simultaneously

-	1 result ([12]) shows that Missed Detection Probability and False Alarm Probability Type 2 cannot meet the performance objectives

-	For Missed Detection Probability, the reported value is 22.74%

-	For False Alarm Probability Type 2, the reported value is 9.52%

-	3 results from 2 sources ([10, 15]) show that only Missed Detection Probability cannot meet the performance objectives

-	For Missed Detection Probability, the reported values range from 5.60% to 8.83%

-	1 result ([15]) shows that only False Alarm Probability Type 2 cannot meet the performance objectives. The reported value is 5.80%

For Case 3-8 with CPI up to 160ms, and self-interference model X =-Inf, 3 results from 3 sources ([13, 16, 20]) provide evaluation results

-	2 results from 2 sources ([13, 20]) show that all performance objectives can be met simultaneously

-	For Horizontal Positioning Accuracy @90%, the reported values range from 1.26 to 4.75 m

-	For Vertical Positioning Accuracy @90%, the reported values range from 0.98 to 1.74 m

-	For the radial velocity Accuracy @90%, the reported values range from 0.28 to 4.62 m/s

-	For Missed Detection Probability, the reported values range from 1.50% to 4.44%

-	For False Alarm Probability Type 1, the reported values range from 0.00% to 0.00%

-	For False Alarm Probability Type 2, the reported values range from 1.50% to 2.33%

-	1 result ([16]) shows that Missed Detection Probability and False Alarm Probability Type 2 cannot meet the performance objectives

-	For Missed Detection Probability, the reported value is 23.50%

-	For False Alarm Probability Type 2, the reported value is 25.40%

For Case 3-9, with CPI up to 160ms, and self-interference model X = -Inf, 4 results from 1 source ([7]) show that only position performance objectives can be met

-	For Horizontal Positioning Accuracy @90%, the reported values range from 0.85 to 1.03 m

-	For Vertical Positioning Accuracy @90%, the reported values range from 0.58 to 1.05 m

For Case 3-10, with CPI up to 160ms, and self-interference model X = -Inf or 5, 2 results from 2 sources ([18, 20]) provide evaluation results

-	1 result ([20]) shows that all performance objectives can be met simultaneously

-	For Horizontal Positioning Accuracy @90%, the reported value is 1.32m

-	For Vertical Positioning Accuracy @90%, the reported value is 0.96m

-	For the radial velocity Accuracy @90%, the reported value is 0.28 m/s

-	For Missed Detection Probability, the reported value is 1.30%

-	For False Alarm Probability Type 1, the reported value is 0.00%

-	For False Alarm Probability Type 2, the reported value is 1.30%

-	1 result ([18]) shows that Missed Detection Probability and False Alarm Probability Type 2 cannot meet the performance objectives

-	For Missed Detection Probability, the reported value is 17.60%

-	For False Alarm Probability Type 2, the reported value is 20.10%

For Case 3-11, with CPI up to 160ms, and self-interference model X = 5, 12 results from 1 source ([17]) provide evaluation results.

-	4 results ([17]) models sensing Tx/Rx operating simultaneously with maximum BS Tx power 37dBm, and show that Missed Detection Probability and False Alarm Probability Type 2 cannot meet the performance objectives

-	For Missed Detection Probability, the reported values range from 16.40% to 29.00%

-	For False Alarm Probability Type 2, the reported values range from 13.70% to 15.80%

-	8 results ([17]) models hybrid use of sensing Tx/Rx operating simultaneously with maximum BS Tx power 37dBm and not simultaneously with maximum BS Tx power 56dBm, and show that all performance objectives can be met simultaneously

-	For Horizontal Positioning Accuracy @90%, the reported values range from 0.778 to 2.853 m

-	For Vertical Positioning Accuracy @90%, the reported values range from 0.161 to 1.395 m

-	For 3D velocity Accuracy @90%, the reported values range from 0.146 to 0.617 m/s

-	For Missed Detection Probability, the reported values range from 0.10% to 4.90%

-	For False Alarm Probability Type 1, the reported values range from 0.00% to 0.00%

-	For False Alarm Probability Type 2, the reported values range from 0.00% to 4.90%

For Case 3-12, with CPI up to 10ms, and self-interference model X = {0, 10, 20}, 7 results from 1 source ([15]) provide evaluation results.

-	1 result ([15]) shows that all performance objectives can be met simultaneously. The results are obtained assuming 20 horizontal Tx beam and 11 vertical Tx beam at TRP and self-interference model X=0.

-	For Horizontal Positioning Accuracy @90%, the reported value is 0.26 m

-	For Vertical Positioning Accuracy @90%, the reported value is 0.25 m

-	For the radial velocity Accuracy @90%, the reported value is 1.36 m/s

-	For Missed Detection Probability, the reported value is 4.80%

-	For False Alarm Probability Type 1, the reported value is 0.00%

-	For False Alarm Probability Type 2, the reported value is 4.20%

-	6 results ([15]) show that not all performance objectives can be met simultaneously

-	1 source ([15]) show that Missed Detection Probability and False Alarm Probability Type 2 cannot meet the performance objectives

-	For Missed Detection Probability, the reported value is 17.60%

-	For False Alarm Probability Type 2, the reported value is 5.50%

-	5 sources ([15]) show that only Missed Detection Probability cannot meet the performance objectives. The reported values range from 7.30% to 23.00%

7	Network architecture

Editor’s note: This section is to capture the study outcome of network architecture. Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. No inter-gNB coordination will be studied.

8	RAN-CN procedures and signalling

Editor’s note: This section is to capture the study outcome of procedures and signaling aspects between RAN and CN for gNB-based monostatic sensing.

9	Conclusions

The performance for UAV sensing use case with gNB monostatic sensing is evaluated based on the evaluation assumptions in Annex A (including UMa-AV, Sensing Tx/Rx operating simultaneously, FR1), with detailed assumptions and modelling reported by the companies as captured in Annex B. Among all the reported evaluation results as captured in Annex B,

-	Baseline 1 (high BS TX power 52dBm with 80dB antenna isolation)

-	By utilizing measurements from multiple or all TRPs, results from 9/10 sources achieve the performance objectives.

-	By utilizing measurements from single TRP, results from 3/11 sources achieve the performance objectives.

-	Baseline 2 (low BS TX power 37dBm with 65dB antenna isolation)

-	By utilizing measurements from multiple or all TRPs, results from 7/9 sources achieve the performance objectives.

-	By utilizing measurements from single TRP, results from 2/9 sources achieve the performance objectives

The study in physical layer has identified a number of Levels/Options for measurement report as described in Clause 5. Different Levels/Options for measurement report are associated with different payload overhead, assistance information and other higher layer aspects.

Annex <A>: Evaluation assumptions

In this clause, the evaluation assumptions for the evaluation of UAV use case with gNB-based monostatic sensing are provided. When sensing Tx/Rx operates simultaneously, the assumptions are summarized in Table A-1.

Table A-1: Evaluation assumptions

| Parameters |  | Assumptions |  |
| --- | --- | --- | --- |
|  |  | FR1 | FR2-1 (Optional) |
| Scenario |  | UMa-AV, optional RMa-AV | UMi-AV |
| Carrier frequency |  | 4 or 4.9 GHz Optional for FR1: 6 GHz | 30 GHz |
| System bandwidth |  | 100 MHz | 400 MHz |
| Numerology |  | SCS = 30 kHz | SCS = 120 kHz |
| BS Layout |  | Hexagonal grid, 7 macro sites, 3 sectors per site. 3 sectors with 30, 150, 270 degrees |  |
| Inter-BS (2D) distance |  | UMa-AV: 500 m, optional 1000 m RMa-AV: 1732 m | 200 m |
| Wrap-round |  | No wrap-round |  |
| BS antenna height |  | UMa-AV: 25 m RMa-AV: 35 m | 10 m |
| BS antenna configuration |  | (M, N, P, Mg, Ng; Mp, Np) for 4GHz, 4.9GHz - Configuration A - Tx: (8,8,2,1,1;4,8) - Rx: (8,8,2,1,1;4,8) - Configuration B as optional - Tx: (12,16,2,1,1;2,16) - Rx: (12,16,2,1,1;2,16) (M, N, P, Mg, Ng; Mp, Np) for 6GHz - Configuration A - Tx: (8,8,2,1,1;4,8) - Rx: (8,8,2,1,1;4,8) - Configuration B as optional - Tx: (16,16,2,1,1;4,16) - Rx: (16,16,2,1,1;4,16) dH,dV = (0.5, 0.8)λ, +45°/-45° polarization Optional: dH,dV = (0.5, 0.5)λ, +45°/-45° polarization | (M, N, P, Mg, Ng; Mp, Np) - Configuration A - Tx: (16,16,2,1,1;1,1) - Rx: (16,16,2,1,1;1,1) dH,dV = (0.5, 0.5 or 0.8)λ, +45°/-45° polarization |
| BS antenna radiation pattern |  | Table 9 in Report ITU-R M.2412 |  |
| BS antenna mechanic tilt |  | 90° in GCS (pointing to horizontal direction) |  |
| BS antenna electrical tilt |  | Option 1: no electrical tilt Option 2: 102° in GCS |  |
| Polarized antenna model |  | Model-2 in clause 7.3.2 in TR 38.901 |  |
| Antenna isolation |  | 65 dB, 80 dB | 80 ~ 100 dB |
| Max BS Tx power |  | 37 dBm, 52 dBm. See note 1 | 30 dBm |
| Inter-site interference |  | Not modelled |  |
| Self-interference |  | The residual leakage interference/noise is modelled e.g., by additional additive white Gaussian noise, -94+X dBm in 100 MHz, X is up to company report. Companies to provide details on their modelling. See note 2 |  |
| Co-site inter-sector interference |  | Not modelled |  |
| Adjacent channel interference |  | Not modelled |  |
| BS receiver noise figure |  | 5 dB | 7 dB |
| Sensing target | Target type | UAV with small size (0.3m x 0.4m x 0.2m) |  |
|  | 3D distribution | N targets per sector in the center site only - N = 5 - Optional: N is uniformly distributed from 1 to 10 Horizontal plane: Uniformly distributed in a sector Vertical plane: Uniformly distributed between 25m and 300m, optionally between 1.5m and 300m. |  |
|  | Mobility | Horizontal speed: uniformly distributed between 0 and 180km/h Vertical speed: 0km/h |  |
|  | Minimum BS-target 3D distance | 10 m |  |
|  | Minimum target-target (3D) distance | 10 m |  |
|  | Outdoor/indoor proportion | 100% outdoor |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | Orientation | Random in horizontal domain |  |
|  | RCS model | RCS model 1 for UAV with small size |  |
| gNB-target link |  | TRP-UAV link in Table 7.9.3-2 in TR 38.901, using Clause B.1.3 in TR 36.777 |  |
| Concatenation of TX-target and target-RX links |  | Up to company choice between two options for concatenation defined in Step 9 in clause 7.9.4.1 in TR 38.901 |  |
| The power threshold for path dropping after concatenation for target channel |  | -25 dB and -40 dB are respectively used for the two options for concatenation |  |
| NOTE 1: The above options are calculated with BS_maxpower = BS Rx saturation power + antenna isolation by assuming the BS Rx saturation power = -28dBm and the antenna isolation = 65dB and 80dB, respectively. NOTE 2: X = -Infinity corresponds to not modelling self-interference. |  |  |  |

Besides the evaluation parameters provided in Table A-1, the following assumptions are up to company report:

-	To model self-interference, value of X to derive the power of the additional additive white Gaussian noise to model the residual leakage interference/noise.

-	Length of Coherent Processing Interval (CPI).

-	Tx beam information (number of Tx beams, wide/narrow Tx beam) being used at TRP.

-	RE mapping of sensing RS, and assumed TDD UL/DL configuration if applicable.

-	Sensing resource ratio.

-	High-level sensing signal/data processing method, e.g., 2D FFT, MUSIC, and any other methods.

-	Optionally, the maximum BS Tx power when it is assumed that Tx and Rx don’t operate simultaneously. Companies should report how the maximum BS Tx power is derived.

-	Sensing signal processing and ISAC channel generation.

-	Whether a same target is modelled in the ISAC channel of single, multiple or all STXs/SRXs?

-	Company should report how to determine the single or multiple STXs/SRXs for a target.

-	If the evaluation results are derived by measurement reports from multiple/all STXs/SRXs, companies should report how measurement reports from multiple/all STXs/SRXs are used.

-	Beam set at TRxP for FR2-1.

-	Additional configuration information for FR2-1.

-	How target trajectory is modeled if evaluation results on UAV tracking is reported.

Annex <B>: Evaluation results

The evaluation results of UAV use case with gNB-based monostatic sensing can be found in R1-2601668. The related detailed assumptions and modelling reported by the companies are provided in R1-2601669.

Annex <X>: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-10 | RAN1 #122-bis | R1-2507422 |  |  |  | TR Skeleton | 0.0.1 |
| 2025-11 | RAN1 #123 | R1-2509522 |  |  |  | RAN1 agreements till RAN1 #122bis | 0.1.1 |
| 2025-12 | RAN1 #123 | R1-2509632 |  |  |  | RAN1 agreements in RAN1 #123 | 0.2.0 |