3GPP TSG-RAN WG2 Meeting #132	R2-2508044

Dallas, USA, Nov. 17th-21st, 2025

Source:	vivo

Title:	Discussion on 6GR UE capability framework

Agenda Item:	10.2.1

Document for:	Discussion and Decision

- Introduction

At the RAN2#131-bis meeting, RAN2 had an initial discussion on UE capability and reached the following agreements:

Agreements on UE capabilities for next meeting

For next meeting companies should focus on identifying the critical problems with UE capability framework and how to address them.  Identify what actions to be triggered with other WGs.

Companies can identify use cases to justify dynamic capability change in connected mode and analyse impacts to network side and network behaviour upon reception of updated capabilities.   Understand if there is impact to other WGs.

Companies can focus on the practical IODT problems in the field.   Possible ways to address them.

In this contribution, we will provide our further analysis of the critical problems with the UE capability framework.

- Discussion

  - Signaling overhead issue

2.1.1 General

NR's radio capability has signaling overhead issues and was discussed in Rel-16, i.e., "Study on optimizations of UE radio capability signalling"[1]. The sources for NR's radio capability signaling issues include the following aspects:

- Unnecessary UE capability reporting: NR only supports filtering at the granularity of RAT and Band, e.g., the gNB can request the UE to provide capabilities for a restricted set of bands. Although a fallback mechanism has been introduced, i.e., the UE may omit a subset of the requested band combinations in accordance with UE capability fallback behavior, many reported capabilities remain unnecessary, such as those not supported by the network or not preferred to be enabled/configured.

- The excessive flexibility: For example, in LTE, UE capability information can be indicated via UE categories, which define a combined uplink and/or downlink capability, such as the maximum number of transport block bits within a TTI and the maximum number of supported layers for spatial multiplexing. To enhance signaling flexibility and forward compatibility, UE capabilities in NR do not rely on UE categories. In other words, UE categories associated with fixed peak data rates are only defined for marketing purposes and are not signaled to the network.

As a consequence, the UECapabilityInformation message may exceed the maximum PDCP SDU size, and segmentation will be performed. This may introduce additional latency during the initial connection establishment procedure. Meanwhile, the unnecessary decomposition of features into overly granular sub-features complicates device development, network planning, and deployment, and leads to difficulties in quick deployment as well as market fragmentation.

- Unnecessary UE capability reporting and excessive UE capability flexibility result in significant signaling overhead of UE capability information report in NR.

6G should learn from the lessons of NR by reducing the signaling overhead of UE capability with reasonable UE flexibility. The following principles could be considered:

- Avoid unnecessary UE capability reporting. One of the most straightforward approaches is to reuse and enhance NR's filtering and fallback mechanisms by introducing finer-grained filtering. For instance, the network can indicate a restricted set of features it supports or prefers to configure.

- Balance signaling overhead and flexibility. This can be achieved by reducing the number of options for features and parameters. For example, different device types may be associated with different mandatory features and minimum capabilities.

- The UE capability information in 6G should consider reducing signaling overhead with the following principles:

-	Avoid unnecessary UE capability reporting. E.g., reuse and enhance filtering and fallback mechanisms by introducing finer-grained filtering.

-	Balance signaling overhead and flexibility. E.g., different device types can be associated with different mandatory features and minimum capabilities.

2.1.2 RF-related capability

Furthermore, as illustrated in Figure 2.1-1, the UE capability parameters have a hierarchical structure with multiple granularities.  Typically, finer-grained capabilities, such as Per BC/Per FS/Per FSPC capabilities, are based on the UE feature list tables provided by RAN1 and RAN4.

- Per UE: the associated parameter is signalled in field decoupled with band, e.g., measAndMobParameters.

- Per Band:  the associated parameter is signalled in field coupled with a specific band, e.g., fields in BandNR.

- Per BC:  the associated parameter is signalled per band combination, e.g., ca-ParametersNR in BandCombination.

- Per FS:  the associated parameter is signalled per feature set (per band per band combination), e.g., fields in FeatureSetDownlink.

- Per FSPC: the associated parameter is signalled per feature set per component carrier (per CC per band per band combination), e.g., fields in FeatureSetDownlinkPerCC.

Figure 2.1-1:	UE capability parameters with multiple granularities

However, the Per BC/Per FS/Per FSPC design, results in the rf-Parameters, featureSets, and featureSetCombinations occupying a significant proportion of the UE radio capability. This observation was also noted in [1] as follows:

## 5.1 UE Radio Capability size estimation <omitted> In practical deployment, the supported bands/band combinations for a device is limited, and the eNB/gNB can for example request the UE to provide UE radio capabilities for a restricted set of band combinations. According to LTE experience and analysis of the NR capability, the estimated practical size of rf-Parameters is about 11~12 kbytes based on ~1024 band combinations and up to 4 bands per band combination. The size of featureSetCombinations is highly variable depending on the number of feature set combinations and the number of feature sets per band, the size of featureSets is variable depending on the number of DL/UL feature sets per band and the number of DL/UL feature sets per CC. However, in some cases either the size of featureSetCombinations or featureSets can significantly exceed the size limitation of PDCP PDU. For example, with 256 feature set combinations, 8 feature sets per band, and 4 bands per band combination, the feature set combination list occupies ~22 kbytes.

As can be seen from Figure 2.1-1, the reasons for the above issues are primarily as follows:

- The excessive number of bands and band combinations leads to an almost linear growth in the size of rf-Parameters with the increase in band combinations.

- Feature sets are grouped per band and are associated with both uplink and downlink capabilities. The excessive flexibility in capabilities for each band makes it difficult for a single featureSetCombination to be reused across multiple BandCombinations.

- The RF-related capabilities of NR occupy a significant proportion of UE radio capability with the following reasons:

- Excessive number of bands and band combinations leads to a huge size of rf-Parameters.

- Feature sets are grouped per Band, and are associated with both uplink and downlink capabilities, making it difficult for a single featureSetCombination to be reused across multiple BandCombination.

Therefore, it’s essential to study how to optimize the RF-related capability signaling. One straightforward way is to optimize the structure of Feature Set combination as illustrated in Figure 2.1-2, including:

- Group multiple bands as one Band Group.

- Introducing Feature Sets per Band Group.

- Decoupling of uplink and downlink Feature Sets.

Figure 2.1-2: Feature set combinations with decoupled UL/DL feature sets per band group

In addition, coordination with RAN1/RAN4 on the optimization of RF related capability signaling is needed.

- Study the feasibility of optimizing the RF-related capability signaling, including:

- Introducing Band Group that includes multiple Bands.

- Introducing Feature Sets per Band Group.

- Decoupling of uplink and downlink Feature Sets.

Note: Coordinate with RAN1/RAN4, when needed.

2.1.3 RACS

UE radio capability signalling optimization (RACS) was introduced in NR based on the outcome of [1]. RACS operates by assigning a UE Radio Capability ID to represent a set of UE radio capabilities. As the radio capability ID is per UE, excessive capability flexibility makes it nearly impossible for the same UE Radio Capability ID to be reused across different UE types. This results in increased storage overhead for the UCMF (UE Radio Capability Management Function) to store the mappings between UE Radio Capability IDs and corresponding UE radio capability information.  As a result, the RACS feature is not well deployed in NR.

- UE Radio Capability ID in RACS is a per-UE identifier, making it nearly impossible for the same UE Radio Capability ID to be reused across different UE types.

Based on our understanding, if a UE is assigned a capability ID instead of specific UE radio capabilities, it can effectively reduce the air interface signaling overhead for UE radio capability interactions. Therefore, optimizing the RACS should be considered, such as introducing finer-grained capability IDs. Specifically, sets of features can be grouped together, with each group associated with a separate ID. If multiple UEs share certain capability groups, the same ID can be reused. Additionally, if a UE's capabilities change (e.g., due to RF capability modifications), only the IDs corresponding to the changed capabilities need to be updated.

As the RACS is leading by other WGs (e.g., SA2/CT1), coordination with SA2/CT1 is needed to ask them to take the enhancement into consideration.

- Study the enhancement for RACS, e.g., group sets of features together, and each group is associated with a separate ID. Coordinate with SA2/CT1 to take the enhancement into consideration.

  - Dynamic capability change

NR supports the reporting of temporary capability restrictions, e.g., due to hardware sharing, IDC problem, or overheating. These temporary restrictions are indicated to the gNB via the UEAssistanceInformation message rather than through UE capability update. However, since temporary capability restrictions are reported as UE preferences, how the network treats them for further configuration is up to network implementation.

Furthermore, with the ongoing evolution of UE types and form factors, such as the growing popularity of foldable phones, the RF capabilities (e.g., MIMO layers) may change depending on the device’s current state. In such cases, changes in UE capability should not be reported merely as preferences or assistance information, but rather as actual capability updates. The network should respect the updated capability parameters when configuring and scheduling the UE.

Therefore, we propose a unified UE capability update framework for the above various dynamic UE capability update cases. And the network needs to respect the updated UE radio capability and reconfigure the UE.

- How the network treats the UAI (e.g., IDC problem, overheating, low power) for further configuration is up to network implementation, while network needs to respect the UE radio capability when configuring or scheduling the UE.

- The UE capability framework in 6G should support dynamic and efficient UE capability update, including the following scenarios, e.g., factor switching (foldable phone and MUSIM) and exceptional UE issues (IDC problem, overheating, low power).

  - IODT problems in the field

In NR, for mandatory features with capability signalling, the capability information indicates whether the feature has been successfully tested.  We understand that this approach remains valid in 6G. Specifically, a smartphone must pass required tests to obtain certification. These tests are subject to varying standards and requirements across different regions or countries, and the features included in the tests are not entirely consistent. Furthermore, during international roaming, cross-regional or cross-country IODT may not be comprehensive. In these cases, mandatory features with capability parameters can indicate whether the smartphone has passed IODT under the current PLMN.

- To avoid the potential IODT issues crossing different regions and PLMNs, maintain the type of feature ‘mandatory with capability signalling’ in 6G.

On the other hand, in NR, we have also observed an imbalance between network and UE support for mandatory features. For example, support for the RRC_INACTIVE state is mandatory for NR UEs, while this feature has not been widely deployed on the network side. We understand it’s necessary to study how to ensure alignment between network and UE support for mandatory features from a standardization perspective, so as to better facilitate the implementation and deployment of mandatory features. One potential approach is to include mandatory features of network in the specification, which should be treated as mandatory items in IODT.

- Study how to ensure alignment between network and UE support for mandatory features, e.g., include mandatory features of network in the specification, which should be treated as mandatory items in IODT.

At the RAN2#131bis meeting, RAN2 discussed the following IODT issues:

- Issue 1: The UE must wait for the IODT with the "slowest" network vendor before activating a capability in the operator's network.

- Issue 2: Even after a feature is activated, interoperability issues may still arise when a new version of network equipment or a UE is deployed or used.

Regarding Issue 1, it is recommended that operators complete comprehensive IODT during the procurement of network equipment or when upgrading to a new feature.

Issue 2 is primarily due to different interpretations of the protocol across companies. From our understanding, RAN2 should try to optimize and refine the protocol descriptions to prevent implementation discrepancies resulting from ambiguous specifications.

In general, we think the above issues can be addressed by implementation and no need to be further studied in RAN2. Furthermore, we understand that some IODT issues may be related to other WGs, e.g., RAN4, RAN5. If RAN2 reaches a consensus on any IODT issue related to other WGs to be addressed by 3GPP, RAN2 should trigger corresponding discussions within RANP.

- If RAN2 reaches a consensus on any IODT issue related to other WGs (e.g., RAN4, RAN5) to be addressed by 3GPP, RAN2 should trigger corresponding discussions within RANP.

- Conclusion

In this paper, we discuss the 6G UE capability framework. we have the following observations and proposals:

- Signaling overhead issue

- Unnecessary UE capability reporting and excessive UE capability flexibility result in significant signaling overhead of UE capability information report in NR.

- The UE capability information in 6G should consider reducing signaling overhead with the following principles:

-	Avoid unnecessary UE capability reporting. E.g., reuse and enhance filtering and fallback mechanisms by introducing finer-grained filtering.

-	Balance signaling overhead and flexibility. E.g., different device types can be associated with different mandatory features and minimum capabilities.

- The RF-related capabilities of NR occupy a significant proportion of UE radio capability with the following reasons:

- Excessive number of bands and band combinations leads to a huge size of rf-Parameters.

- Feature sets are grouped per Band, and are associated with both uplink and downlink capabilities, making it difficult for a single featureSetCombination to be reused across multiple BandCombination.

- Study the feasibility of optimizing the RF-related capability signaling, including:

- Introducing Band Group that includes multiple Bands.

- Introducing Feature Sets per Band Group.

- Decoupling of uplink and downlink Feature Sets.

Note: Coordinate with RAN1/RAN4, when needed.

- UE Radio Capability ID in RACS is a per-UE identifier, making it nearly impossible for the same UE Radio Capability ID to be reused across different UE types.

- Study the enhancement for RACS, e.g., group sets of features together, and each group is associated with a separate ID. Coordinate with SA2/CT1 to take the enhancement into consideration.

- Dynamic capability change

- How the network treats the UAI (e.g., IDC problem, overheating, low power) for further configuration is up to network implementation, while network needs to respect the UE radio capability when configuring or scheduling the UE.

- The UE capability framework in 6G should support dynamic and efficient UE capability update, including the following scenarios, e.g., factor switching (foldable phone and MUSIM) and exceptional UE issues (IDC problem, overheating, low power).

- IODT problems

- To avoid the potential IODT issues crossing different regions and PLMNs, maintain the type of feature ‘mandatory with capability signalling’ in 6G.

- Study how to ensure alignment between network and UE support for mandatory features, e.g., include mandatory features of network in the specification, which should be treated as mandatory items in IODT.

- If RAN2 reaches a consensus on any IODT issue related to other WGs (e.g., RAN4, RAN5) to be addressed by 3GPP, RAN2 should trigger corresponding discussions within RANP.

- Reference

- TR 37.873		Study on optimizations of UE radio capability signalling		3GPP