3GPP TSG-RAN WG2 Meeting #132	R2-2508063

Dallas, USA, Nov. 17th - 21st, 2025

Source:	vivo

Title:	Discussion on measurement event prediction

Agenda Item:	9.3.3

Document for:	Discussion and Decision

- Introduction

R20 AI mobility WID[1] has been approved to specify both RRM measurement prediction and measurement event prediction:

- Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]: - Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported. - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation.

In this contribution, we will discuss LCM for RRM measurement event prediction, including UE capability, applicability, inference configuration/reporting, etc.

- Discussion

## General

The WID [1] includes the following scenarios and/or sub-cases for UE sided model (covering RRM and measurement event prediction): intra-frequency temporal domain case A, intra-frequency temporal domain case B and inter-frequency domain prediction for co-located case. As summarized in the study report [2], the main specification impact stems from the introduction of RRM measurement prediction, while the additional impact for measurement event prediction is limited. Therefore, we suggest first discussing the LCM for the RRM prediction in UE sided model, and subsequently addressing the additional impact for RRM event prediction within the same LCM framework.

For measurement event prediction, two methods have been studied: indirect and direct prediction. Currently, there is no consensus on the feasibility of performance monitoring of direct event prediction. It should be resolved before proceeding with specifying direct event prediction. Thus, we recommend to focus on the indirect event prediction before the feasibility of direct event prediction is justified.

- Take the LCM framework of RRM prediction as the baseline for measurement event prediction, with a focus on indirect event prediction.

## UE capability

UE should report its UE capability for RRM measurement prediction and measurement event prediction. In the indirect measurement event prediction approach, the measurement result is first predicted by an RRM measurement prediction model. Therefore, some basic capability information--such as supported scenarios--can be defined in the RRM measurement prediction capability report, for example:

- intra-frequency temporal domain case A,

- intra-frequency temporal domain case B,

- inter-frequency domain prediction for co-located case.

For measurement event prediction, event types A1 to A6 are within the WID scope. As this is the first release to support measurement event prediction, a general UE capability for measurement event prediction is sufficient. It’s unnecessary to define per-event-type capabilities at this stage. Additional event-related capabilities, such as supported prediction window, may be considered later. These details can be discussed after the inference configuration and report procedure are clarified.

- UE reports its capability for measurement event prediction as a feature group. No need to differentiate capability for different event type (i.e., A1-A6). The finer capabilities(e.g., prediction window length) can be discussed after the detailed inference configuration parameters are concluded.

## Applicability report

Our focus is on indirect measurement event prediction, which leverages an underlying RRM measurement prediction model. Consequently, the applicability report defined for RRM prediction can be reused. This includes reports for both full and partial configuration approaches, thereby we propose:.

- RRM prediction and measurement event prediction share the same applicability report procedures.

## Inference configuration/report

Inference configuration/report

Event types A1~A6 are within the WID[1] scope. Thre event definitions, based on 38331-j00, are as follows:

- Event A1:	Serving becomes better than absolute threshold;

- Event A2:	Serving becomes worse than absolute threshold;

- Event A3:	Neighbour becomes amount of offset better than PCell/PSCell;

- Event A4:	Neighbour becomes better than absolute threshold;

- Event A5:	PCell/PSCell becomes worse than absolute threshold1 AND Neighbour/SCell becomes better than another absolute threshold2;

- Event A6:	Neighbour becomes amount of offset better than SCell;

The inference configuration for measurement event prediction should provide in the existing reportConfigNR, which includes event-related parameters, such as event type, threshold or offset, and timeToTrigger etc.Inference configuration can reuse the RRM signaling procedure.

Furthmore, we need to consider how to configure prediction window for measuremet event predicction. Prediction window is a time window during which the measurement event may be triggered. Considering a single framework is pursued for both indirect event prediction and direct event prediction, the prediction window for measurement event is needed. And considering the RRM prediction and measurement event prediction could be configured simultaneously or separately, an independent prediction window for measurement event is more suitable. Hence, we propose:

- Inference configuration for measurement event prediction is included in reportConfigNR, which may include a dedicated prediction window in addition to the existing event type, threshold, offset, timeToTrigger, etc.

Leaving condition

In legacy measurement event reporting procedure, UE can be configured to report if leaving condition is fulfilled for a certain event. A similar mechanism should be considered for measurement event prediction, i.e., UE predicts whether the leaving condition is met. If the UE has previously reported a predicted measurement event and the leaving condition is predicted to be fulfilled, it may also report the leaving of the event. Furthermore, Even if a legacy measurement event is triggered based on actual measurement results, UE may pre-check the predicted leaving condition before sending the actual measurement event reporting. If the leaving condition is predicted to be met shortly, the UE may cancel the event reporting. This approach helps improve reporting accuracy, avoid false reports, and prevent incorrect handovers. Therefore, we propose that:

- Support leaving condition prediction in measurement event prediction to avoid unnecessary handover.

Event-triggered inference

In indirect measurement event prediction for intra-frequency temporal domain case A, continuous measurement results in PW are predicted by continuous historical measurement results (s) in OW via an RRM measurement prediction model. Afterwards, predicted and optionally actual historical measurement result(s) of the same cell are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model.

At least the following 2 candidate options can be considered inference timing:

- Option 1: Continuous inference. Consistent with Case A, a measurement event prediction result will be derived for each sampling period/measurement period;

- Option 2: Event-triggered inference. Inference is executed when UE moves to the edge of the serving cell, e.g., the entering condition for an event is met based on actual measurement.

To support measurement event prediction with lower power consumption, Event-triggered inference is preferred(i.e., Option 2). In this case, the network does not need to config a separate prediction window for measurement event prediction.

Therefore, we propose:

- For measurement event prediction based on Case A, UE can be configured to perform event-triggered inference, e.g., inference can be executed when the entering condition for an event is met based on actual measurement.

- Conclusion

Based on the above analysis, we make the following proposals:

General

- Take the LCM framework of RRM prediction as the baseline for measurement event prediction, with a focus on indirect event prediction.

UE capability

- UE reports its capability for measurement event prediction as a feature group. No need to differentiate capability for different event type (i.e., A1-A6). The finer capabilities(e.g., prediction window length) can be discussed after the detailed inference configuration parameters are concluded.

Applicability Report

- RRM prediction and measurement event prediction share the same applicability report procedures.

Inference configuration/report

- Inference configuration for measurement event prediction is included in reportConfigNR, which may include a dedicated prediction window in addition to the existing event type, threshold, offset, timeToTrigger, etc.

- Support leaving condition prediction in measurement event prediction to avoid unnecessary handover.

- For measurement event prediction based on Case A, UE can be configured to perform event-triggered inference, e.g., inference can be executed when the entering condition for an event is met based on actual measurement.

- Reference

- RP-252899 Revision of WID on AI mobility

- TR 38.744 Study on AI mobility