3GPP TSG-RAN WG2 Meeting #132			         R2-2508147

Dallas, TX, USA, 17 – 21 November 2025

Agenda Item:	10.3.2.3

Source: 	LG Electronics Inc.

Title:         	Initial and System Access for 6G

Document for: 	Discussion and Decision

Introduction

In this contribution, we propose to study some aspects for initial and system access for 6G.

Discussion

2.1	SS/SI transmission for single/multi-carriers

RAN2 made the following agreements on system information for 6GR:

RAN2 Agreements on System Information - System information design should consider energy efficiency and low complexity for both network and UE. - System information design should ensure that the public warning requirements are met. - Study RAN2 aspects of system information designs (e.g. on-demand SIB and SSB, periodicity of SSBs) - Study how to improve flexibility and extensibility of scheduling SI design - Study mechanisms to improve SI update mechanism - Study aspects related to SIB1 size. Understand the issues and desired content/size from RAN2 point of view. - Study areas specific SIB design. Understand 5G pain points and what improvements can be considered. NOTE: this is a starting list

RAN1 made the following agreements on sync signal(s) and system information:

RAN1 Agreement Study and evaluate NW energy savings and the impact on UE performance and user experience with respect to 20ms and longer periodicities of sync signal(s) at least for initial access with the following consideration, but not limited to: BS assumptions: - Cell-common signaling (e.g., sync signal(s), broadcast PDCCH, SIB-1, SIB, paging, PRACH), e.g., - Clustered provisioning of different cell-common signaling, - On-demand provisioning of different cell-common signaling, - UE-specific signaling (for low, light, medium loads), e.g., - Clustered provisioning with cell-common signaling, - Unclustered provisioning with cell-common signaling, UE impact: - Cell search complexity and latency, including frequency search latency, - UE Power consumption, - Sync signal detection, coverage and tracking performance, - RRM, mobility, - Beam management, - Other properties are not precluded, - Improvements to address identified impact, e.g., - Additional sync signal needs, - Adaptation of sync signal transmission periodicity, - Sparser synch raster. RAN1 Agreement Study and evaluate on-demand sync signal(s) mechanisms for 6GR energy efficiency, considering, e.g.,: - On-demand sync signal(s) for single cell/carrier, multi-carrier/cell, multi-TRP, - Network-triggered and UE-triggered on-demand sync signal(s), - Idle and/or connected modes, RAN1 Agreement Study and evaluate on-demand and/or periodic SIB-1 transmission with respect to - NW energy savings potential and UE power consumption impact, - SIB-1 acquisition delay, - NW and UE complexity, - Coverage, - Applicable deployment scenarios, e.g.,: - Standalone cell/carrier, - Multiple TRPs/cells/carriers.

In NR, UE assumes 20 msec as default synchronization signal (SS) periodicity during initial access procedure. The NR study on NES in TR 38.864 showed that substantial NES gains (up to 84 %) can be achieved by adopting longer SS periodicity. Therefore, it would be beneficial in terms of NES if default SS periodicity can be increased to the value greater than 20 msec for 6G.

However, even if NW transmits SS with a longer periodicity (e.g., 160 msec), UE may require frequent SS transmission in some cases. For instance, when UE attempts to access a cell, or finds a new beam during a beam recovery procedure, it can be more efficient for UE to receive SS with a shorter periodicity (e.g., 20 msec). Depending on whether NW is aware of such UE’s situation, NW-initiated (similar to Rel-19 OD-SSB) or UE-initiated on-demand SS (or SS periodicity adaptation) can be considered.

Alternatively, SS can be separately transmitted from PBCH. For instance, SS-burst (which consists of different SS beams) is periodically transmitted while PBCH-burst (which consists only of PBCH with different beams) can be transmitted in response to UE’s request (i.e., on-demand manner). For another instance, SS-burst is transmitted with a shorter periodicity (e.g., 20 msec) while PBCH-burst is transmitted with a longer periodicity (e.g., 160 msec). It is noted that NES gains can be obtained by reducing PBCH transmissions.

Observation 1a:	Long periodicity of SS/PBCH/SIB1/SI transmissions can save network energy while short periodicity can reduce cell access latency.

In addition to adjusted periodicity of common signal/channel transmissions, TX power on/off or TX power reduction of common signal/channel transmissions can be considered for network energy saving. For example, when there are small cells within a macro cell coverage or when inter-frequency multi-cells are overlapped on the same area, TX power of TRP or clustered beams of a cell could be increased/reduced or turned on/off according to up and down of the amount of user traffic/density without coverage holes. Moreover, TX power on/off of TRP or clustered beams can work in a TDM manner e.g. for NTN beam hopping. Harmonized TN/NTN design of TX power on/off can be considered.

Observation 1b:	Tx power on/off or Tx power reduction of common signal/channel transmissions for TRP or clustered beams of a cell can save network energy.

In the last meeting, RAN1 and RAN2 made the following agreements for 6GR:

RAN1 Agreements Study and evaluate multi-carrier/cells/TRPs mechanisms for 6GR NES, considering, e.g.,: - Sync signal-less carriers/cells/TRPs for at least intra-band and collocated inter-band multi-carrier/cell/TRPs, including potential extensions to additional deployments and scenarios, - RRC states, - UE energy consumption and complexity, - Other mechanisms/aspects/signals/channels are not precluded. Study and evaluate on-demand sync signal(s) mechanisms for 6GR energy efficiency, considering, e.g.,: - On-demand sync signal(s) for single cell/carrier, multi-carrier/cell, multi-TRP, - Network-triggered and UE-triggered on-demand sync signal(s), - Idle and/or connected modes, Study and evaluate on-demand and/or periodic SIB-1 transmission with respect to - NW energy savings potential and UE power consumption impact, - SIB-1 acquisition delay, - NW and UE complexity, - Coverage, - Applicable deployment scenarios, e.g.,: - Standalone cell/carrier, - Multiple TRPs/cells/carriers. RAN2 Agreements - Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed. - Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc). Understand the deployment scenarios and problems to address - System information design should consider energy efficiency and low complexity for both network and UE. - Study RAN2 aspects of system information designs (e.g. on-demand SIB and SSB, periodicity of SSBs)

For 6GR, we can take into account the legacy scenarios where SSB or SIB1 is transmitted on one carrier based on RACH configuration provided on another carrier such as Cell A in NR NES. If multi-carrier deployment is considered for initial and system access in 6GR, considering that RAN1 agreed to study sync signal-less carriers/cells/TRPs for at least intra-band and collocated inter-band multi-carrier/cell/TRPs for multiple RRC states, OD-SSB as well as OD-SIB1 can be studied even for UEs in idle/inactive mode to extend network energy saving gain to all RRC states.

In our view, multi-carrier scenarios for 6G study on initial access and system information can be summarized in Table 1. In multi-carrier, the network can save network energy (and radio resources) for Type-B carriers based on SS, PBCH, SIB1 and/or SI provided on Type-A carrier similar to Cell A in NR NES. In MC scenario 1 and 3, UEs can perform initial and system access on Type-B carrier relaying on SS/PBCH provided on Type-A carrier. In MC scenario 1, 2, 3 and 4, UEs can perform initial and system access on Type-B carrier relaying on SIB1/SI provided on Type-A carrier. If multi-carriers belong to a single cell, a cell-defining sync signals can be periodically transmitted on the sparse sync raster for a Type-A carrier while on-demand sync signals can be provided on off-sync raster as Type-B carrier(s).

Table 1: Multi-carrier scenarios for 6G study on initial access and system information

|  | Type-A carrier | Type-B carrier(s) |
| --- | --- | --- |
| MC Scenario 1 | - SS/PBCH - SIB1/SI | - No SS/PBCH/SIB1/SI |
| MC Scenario 2 | - SS/PBCH - SIB1/SI | - AO-SS with long periodicity and/or OD-SS |
| MC Scenario 3 | - SS/PBCH - SIB1/SI | - No SS/PBCH - (AO and/or OD-) SIB1/SI |
| MC Scenario 4 | - SS/PBCH - SIB1/SI | - AO-SS/PBCH with long periodicity and/or OD-SS/PBCH - (AO and/or OD-) SIB1/SI |

The network can also provide paging and/or RACH resources only on a few carriers among multiple carriers, such as Type-A carrier and/or a few Type-B carriers, and so it can save network energy and radio resources by avoiding duplicated paging and reserved RACH resources over some carriers. Considering possibility of decoupling of UL and DL, we can also study the scenarios where carriers for paging and RACH are same or different.

From cell modelling perspective, we think that MC scenarios in Table 1 can belong to a single cell or multiple different cells. For example, multiple carriers in MC scenario 1 and 2 can be possibly modelled as a single cell while multiple carriers in MC scenario 3 and 4 as different cells e.g. when SIB1s on different carriers provide different Cell IDs.

Proposal 1:	Study the following aspects for both single carrier and multi-carrier scenarios to save network energy and common signals/channel resources for 6GR:

- SS/PBCH transmission can be provided with long periodicity and/or on demand on one or more carriers of multiple carriers

- SS/PBCH transmission can be turned on/off or with a reduced TX power for TRP or clustered beams on one or more carriers of multiple carriers

- Paging can be provided only on one or more carriers of multiple carriers

- RACH can be provided only on one or more carriers of multiple carriers

Proposal 2:	Discuss which multi-carrier scenarios in Table 1 are considered for 6G study on initial access and system information:

Table 1: Multi-carrier scenarios for 6G study on initial access and system information

|  | Type-A carrier | Type-B carrier(s) |
| --- | --- | --- |
| MC Scenario 1 | - SS/PBCH - SIB1/SI | - No SS/PBCH/SIB1/SI |
| MC Scenario 2 | - SS/PBCH - SIB1/SI | - AO-SS with long periodicity and/or OD-SS |
| MC Scenario 3 | - SS/PBCH - SIB1/SI | - No SS/PBCH - (AO and/or OD-) SIB1/SI |
| MC Scenario 4 | - SS/PBCH - SIB1/SI | - AO-SS/PBCH with long periodicity and/or OD-SS/PBCH - (AO and/or OD-) SIB1/SI |

2.2	SIB1 information and transmission

SIB1 size can vary depending on multiple conditions. For example, feature-dependent common channel configurations can affect SIB size e.g. when different RACH configurations are required for different features such as RedCap, NTN and SDT. If multiple features are introduced with different RACH configurations, a large size of SIB1 would be required at that cell.

The size of the UAC information also affects SIB1 size. When it comes to UAC information, the SIB1 size scales e.g. with the number of access categories, barring info sets and PLMNs. Thus, a larger size of SIB1 is expectedly required in higher congestion where many UEs will try to perform RACH for initial access.

Considering a larger size of SIB1 may lead to smaller cell coverage and need for coverage extension, we should carefully design SIB1 information structure. We may avoid 6G SIB1 PDSCH always carrying all SIB1 information inherited from NR SIB1 information. For example, since UE camping on a cell would not need to immediately acquire common channel configuration for suitability check, the network could only include cell access information in SIB1. As for LTE, RACH configuration and access barring info are included in SIB2, not SIB1. As another example, a normal type of UEs may not need RedCap or CE specific RACH configuration in SIB1. Thus, RedCap specific RACH configuration could be only included in on-demand SIB provided to RedCap UEs. CE specific RACH configuration could be transmitted in SIB1 towards extended coverage.

Thus, we propose to study 6G SIB1 consisting of multiple SIB1 modules i.e. different SIB1 information groups e.g. cell access info, SI scheduling info, common channel configurations for different features or UE types, UE timer/constants and UAC info. SIB1 transmission and scheduling and SIB1 update and maintenance can be designed based on per-SIB1 module. Through this approach, different modules can be transmitted by different SIB1 transmissions. SIB1 size can be scalable depending on which modules are included in a SIB1 transmission at a time.

Proposal 3:	Study a scalable size of modular SIB1 construction/transmission for 6GR

- SIB1 consists of multiple SIB1 modules i.e. different SIB1 information groups (e.g. cell access info, SI scheduling info, common channel configurations for different features or UE types, UE timer/constants and UAC info)

- SIB1 scheduling /transmission and SIB1 update/maintenance can be designed based on per-SIB1 module.

2.3	System information transmission

The on-demand transmission mechanism for system information was introduced in NR to reduced broadcast overhead. However, only UE’s request procedure was specified to achieve it, while many aspects still followed the legacy SI transmission mechanism. As a result, the on-demand approach has not been able to fully provide its potential benefits.

Observation 3a:	Lack of integration between OD-SIB1 operation and OD-SI operation is a pain point for NR.

In NR, on-demand SIB1 has been specified after on-demand SI. Thus, on-demand SIB1 operation and on-demand SI operation are not so well integrated. For instance, UE can know the time window when OD-SIB1 is transmitted while UE cannot for OD-SI. Besides, there is the case when SIB1 is not broadcast while SI is broadcast. However, necessity of this case is not clear. The case seems eventually there and became something to be handled in the specification due to separate standard work in different releases.

Considering on-demand SSB is also on the table for 6G discussion, we can discuss a single framework to efficiently support on-demand SSB/SIB1/SI. The transmissions, procedures and RACH resources for on-demand SSB/SIB1/SI can be handled based on a unified approach.

In NR, the transmission of a requested SI message is constrained by pre-configured SI window. After a UE requests a specific SI message, the network can only schedule its transmission within the next available SI window associated with the SI message. This fixed scheduling approach forces the UE to wait for the start of the next SI window, resulting in an avoidable latency. This limitation constrains the usage of the system information, which is critical for future latency-sensitive features.

The fixed SI windows also impose a significant constraint on network energy saving. They force the network to remain active during the pre-defined SI windows, preventing the full use of energy-saving mechanisms like Cell Discontinuous Transmission. A more flexible 6G framework should support dynamically scheduled SI transmissions. This would not only reduce UE latency by decoupling SI delivery from fixed SI window but also enable the network to more effectively implement energy saving modes, aligning with 6G efficiency goals.

Observation 3b. 	In NR, the on-demand SI procedure focused on a UE request procedure, while network-side delivery largely retained legacy broadcast mechanism (fixed SI window and whole-message transmission even for update). Consequently, potential benefits from on-demand, such as dynamic scheduling for faster required-SI delivery and signalling-efficient SI delivery based on delta signalling, have not been achieved.

In NTN, it takes long time to accomplish the initial access due to long propagation delay. Additionally, the satellites move fast along their orbits, the time that UE stays in the particular beam coverage of a satellite is short. To successfully complete a connection within the satellite’s visibility, the UE has to complete quickly the initial connection setup procedure. However, NR NTN didn’t consider fast initial access specially. In 6GR NTN, as the new service scenarios, e.g., voice call or emergency alert, etc., are emerged, fast initial access should be addressed. In NR NTN, the UE can start the initial access procedure after receiving SS/PBCH, SIB1, and SIB19. Specifically, to obtain NTN configuration including the ephemeris information and common TA information for PRACH transmission, the UE needs to wait to receive SIB19 while the SIB19 can be received after obtaining the SI scheduling information from SIB1. Moreover, since the SI scheduling information is provided by the RRC message, it will take some time (e.g., few or few tens of msec) for the UE to acquire where the UE tries to detect the SIB19. It implies that the UE may not receive the NTN configuration right after the SIB1 reception. Considering that the SS/PBCH and common channel periodicities could be increased due to the limited active beam ratio at the satellite, this kind of steps would cause additional latency for the reception of the NTN configurations. Before receiving the NTN configuration, the UE cannot pre-compensate the TO/FO due to the satellite moving, so it cannot start the PRACH transmission. Also, the UE needs to pre-compensate the timing drift and the Doppler drift based on the received SS/PBCH. Moreover, since the satellites (especially for LEO) move fast along their orbits, it is important to ensure that the UE completes the initial connection setup procedure within the satellite’s visibility. Otherwise, the UE may need to restart the DL synchronization by detecting SS/PBCH from the target satellite since the information related to the satellite switching is currently included in SIB19. Considering that the VLEO scenario could be further taken into account in 6GR, it would be necessary to carefully treat these cases.

In 5G NR, depending on the system information change notifications and valueTag in SIB1, the UE can determine whether to consider the stored SIB as valid for the cell or try to receive the updated SIB for the cell. However, due to the different validity timeline of the NTN configurations, such mechanism cannot be used for the NTN configurations. Instead, for the NTN configurations, another timer (e.g., ntn-UlSyncValidityDuration) is used for the UE to reacquire the new NTN configurations. According to the NR NTN, UE should attempt to re-acquire SIB19 before the end of the duration indicated by ntn-UlSyncValidityDuration and epochTime. Since the SIB19 is not distinguishable from other SIBs (due to the same SI-RNTI), the UE would try to receive SIBs other than SIB19 even though the UE does not need to do based on the SI change notification. In short, the existing 5G NR SI reacquisition may not work properly or be inefficient in NTN scenario. In 6GR, it would be necessary to carefully investigate the SI (re)acquisition by taking the NTN characteristics into account.

Observation 3c	The system information required for initial access to NR NTN is defined separately from that for TN and is received within an independent time window, which results in additional latency and UE energy consumption.

In NR, system information is assumed to be same across different beams at a cell. Such restriction can be reconsidered for 6GR. For example, considering a single beam for NTN covers a large area, different common channel configurations can be provided for different beams. As another example, different common channel configurations can be provided for different TRPs of a cell.

Observation 3d	Beam/TRP specific system information can be considered e.g. for better common channel configurations in TN and NTN.

Proposal 4	Study followings for SI acquisition/update

- Unified and integrated approach among different on-demand mechanisms e.g. on-demand SI and on-demand SSB/PBCH/SIB1

- OD-SI transmission based on dynamic scheduling

- SI update based on delta SI

- SI update for fast (re)acquisition of NTN-related SI to reduce initial access latency and/or improve UE energy saving under NTN.

- Beam/TRP specific SI acquisition

2.4	Paging

In NR, various enhancements, e.g. OD-SI, SIB1 and SSB, have been introduced to reduce always-on signaling overhead. By contrast, paging has mainly enhanced to improve UE power saving, e.g. Paging Early Indication and Low Power Wake-Up Signal. Though a minor enhancement was specified for network energy saving in release-19, its effectiveness remains very limited due do the need to maintain support for legacy UEs.

Observation 4a	 Limited flexibility due to static PO configuration is a pain point for NR.

All configuration required for paging reception is provided through SIB1, including even the parameters introduced for PO adaptation in Release 19. Since SIB1 carries information that must be updated simultaneously by all UEs, its update is only allowed at predefined instances, i.e., the start of the next modification period. Consequently, SIB1 is not well suited for (re-)configuring parameters such as paging related, which requires more flexible and timely optimization. This restricts the network’s ability to optimize the number and distribution of paging occasions in response to real-time paging load. A more flexible mechanism would allow adaptive configuration of paging occasions, thereby minimizing the network’s active time for paging transmission.

Observation 4b	 Using Two different paging configurations for legacy NR UEs and Rel-19 NES UEs is a pain point for NR.

Moreover, Rel-19 NES introduced paging adaptation in time domain. In NR, since legacy NR UEs and Rel-19 NES UEs coexist, NW may need to individually configure two different paging configurations where one is for legacy NR UEs and the other is for Rel-19 NES UEs, which is clearly inefficient from NW operational point of view. Therefore, more flexible paging configuration for paging frame (PF) and for paging occasion (PO) than Rel-15 needs to be considered from 6G Day-1. For instance, PFs/POs can be allocated in a compressed manner in time domain, instead of evenly distributed PFs in time domain.

Observation 4c	 paging transmissions with all beams over multiple cells of a large area and coarse-grained UE location knowledge from network side lead to NW energy consumption and resource inefficiency in NR.

Paging DCI/PDSCHs normally happen over multiple cells of a large area, but UE will respond to the paging only for a single cell. In addition, paging DCI/PDSCHs are always broadcast with all beams over multiple cells of a large area. From network energy saving perspective and resource efficiency perspective, such legacy operation is not so a proper way to transmit paging.

Considering this pain point, paging transmissions can be based on request by UE actually present at a cell for network energy saving. In addition, paging can be transmitted only with specific beams corresponding to PRACH on the request to further reduce NW energy consumption for paging transmission and improve resource efficiency.

In NR, mechanisms such as PEI and LP-WUS have been introduced for the purpose of UE power saving. These allow the network to notify UEs whether paging exists for a given paging group, thereby avoiding unnecessary wake-ups and improving UE power saving. Despite these enhancements, each cell has no knowledge of which paging-group UEs are camped on it. Consequently, paging messages for all paging groups must be transmitted blindly from all cells, regardless of the actual presence of UEs. This results in significant inefficiency and unnecessary signalling overhead.

To address this limitation, 6G should consider new mechanisms that enable the network to more efficiently identify the presence and location of UEs. Such mechanisms would allow the network to minimize redundant paging transmissions while preserving UE power-saving benefits. However, achieving this goal may require UEs to report their presence or location more frequently. While this could enable more optimized paging transmission, it inevitably increases UE power consumption. Network energy saving should not come at the cost of excessive UE power consumption, and the impact on UEs should be treated as a critical consideration when specifying any paging-related enhancements.

Observation 4d	It is beneficial to study flexible paging occasion configuration/adaptation and enabling finer-grained UE presence awareness for on-demand paging to reduce unnecessary paging transmissions.

In NR, paging reception and random access execution are tightly coupled within a single serving cell. A UE in RRC_IDLE receives a paging message from its serving cell and is then expected to initiate random access in that same cell. While simple, this design creates inefficiencies in multi-frequency deployments. When multiple cells across different frequencies are available for camping, the network must broadcast paging across all of them to guarantee UE reachability. Conversely, if paging transmission were restricted to a single frequency, UEs would converge on that frequency for paging, leading to access congestion and unbalanced resource utilization.

To overcome this limitation, 6G could decouple the paging reception cell from the access execution cell. In this design, a UE would receive paging on one frequency but be able to perform random access on a different cell operating on another frequency. This allows the network to reduce paging transmissions, limiting them to M out of N frequencies, thereby saving radio resources and network energy. At the same time, access attempts can remain distributed across multiple frequencies, preventing overload on a single carrier and improving overall system stability.

A key challenge in this approach is avoiding access delays. If a UE searches for an access cell on inter-frequency only after receiving paging or generating MO data, the access procedure would be delayed, compared to the legacy design. To prevent this, the UE should pre-identify candidate access cells and pre-acquire the required system information in advance. With this preparation, the UE can immediately initiate access on the selected cell, even if it differs from the paging reception cell. This enhanced UE behaviour improves paging efficiency, reduces signalling overhead, and supports balanced access distribution across frequencies. It also enables more flexible scheduling of system information.

Observation 4e	Coupling of paging reception cell and access execution cell in NR lead to redundant paging transmission across all candidate frequencies to guarantee UE reachability and potential initial access congestion on frequencies where paging is transmitted in multi-carrier deployments.

Proposal 5	Study the following aspects for paging in single/multi-carrier scenarios:

- Paging reception and initial access execution across different frequencies to allow flexible paging transmission across multiple frequencies while supporting distributed and low-latency access across multiple frequencies

- Improving flexibility in paging occasion configuration/adaptation, and enabling finer-grained UE presence awareness for on-demand paging to reduce unnecessary paging transmissions.

2.5	RACH and RRC connection establishment

In NR, in multi-carrier deployment where inter-frequency cells are overlaid, UE selects one frequency based on reselection priority given by the network and then performs initial access to a cell on that frequency. With this operation, the network cannot move this UE to another frequency based on varying load situation for better distribution over multi-carriers unless the UE completes RRC connection establishment. This situation can also happen in multi-RAT deployment. We think that this pain point can be alleviated based on enhancement to initial access assuming that NR cell and 6GR cell in MRSS or multi-carriers can be well coordinated.

Observation 5a	It is beneficial to study load distribution for NR initial access mechanisms in multi-carrier deployment and MRSS deployment

Early SCell setup has been specified to accelerate use of a SCell for faster user data transmission/reception leading to a short stay of a RRC connection, which seems beneficial for efficient management of network energy and UE power by releasing/suspending the RRC connection immediately after data transmission/reception. This is obviously based on CA. If multi-carrier single cell deployment is considered in 6G, this mechanism needs to be extended for non-CA based spectrum utilization.

Meanwhile, multi-TRP operation has been introduced in NR to enhance reliability and robustness and increase throughput and capacity. However, MTRP can be set up only after complete of RRC connection establishment. This delay prevents the network from using advanced MIMO techniques for initial data transmission. Thus, early MTRP setup can be studied for fast reliable data transmission/reception leading to releasing/suspending the RRC connection immediately after data transmission/reception.

In addition, by allowing UEs in RRC_IDLE/INACTIVE to better interact with multiple TRPs in a cluster, we can extend cell coverage and provide a scalable cell deployed with multiple TRPs for robust initial access transmissions as well as reliable data transmissions afterwards. The TRPs can belong to a same cell or different cells depending on network implementation.

We think that 6GR can allow UEs in RRC_IDLE/INACTIVE to manage measurements and common channels for multiple TRPs, in order to reduce cell change/reselection at a boundary of a single TRP. In addition, considering that aggregating multiple TRPs can lead to reliable data transmission possibly with a higher overall data transmission rate, we can study early setup of MTRP during initial access or right after initial access which can reduce the time it takes to configure multiple TRPs, leading to faster user data transmission/reception with higher reliability and data rate for UE and a short stay of a RRC connection, which is beneficial for efficient management of network energy and UE power by releasing/suspending the RRC connection immediately data transmission/reception.

Moreover, early multi-carrier setup for initial access, e.g. fast SCell setup in NR, can be also considered in multi-carrier scenarios. Both early MTRP setup and multi-carrier setup can provide faster user data transmission/reception and will lead to a short stay of a RRC connection, which is beneficial for efficient management of network energy and UE power by releasing/suspending the RRC connection immediately data transmission/reception.

Furthermore, one of new deployments considered for 6G is MRSS. Considering that one of MRSS options is to share SSB/PBCH between 5G and 6G cell, we can study how to select 5G cell and 6G cell on the same frequency for initial access in this MRSS deployment.

Observation 5b	It is beneficial to study data transmission/reception with lower latency and higher reliability based on fast setup and immediate release/suspension of a RRC connection for network/UE energy saving.

The 5G NR supports two types of RACH procedures: the 4-step RACH designed initially for robust access, and the 2-step RACH introduced in NR Rel-16 for low-latency and small data transmissions. These procedures use distinct PRACH resources or RACH occasions to enable separation and efficient operation. In 6G, both procedures are expected to be relevant — e.g. 4-step RACH for extended uplink coverage and 2-step RACH for fast access in small cells. Both 4-step and 2-step RACH procedures can be studied in the early phase of 6G study.

Observation 5c	2-step RACH is beneficial for low-latency connection setup and small data transmissions.

In the meantime, after adoption of 2-step RA in NR Rel-16, RA procedure becomes complicated due to selection of RA type and fallback from 2-step RA to 4-step RA. The situation becomes even worse after the early indication in the RA preamble is introduced by various features. To support early indication of various features, RA partitioning is introduced in NR Rel-17, and this makes the RA procedure even more complicated. We think such complexity in RA procedure should be avoided in 6G, because the RA procedure is the most essential function in mobile system. A neat design of RA procedure is pre-requisite of success of mobile system.

Further, since multiple features for RA partitioning framework are supported in 5G NR, the number of resource partitions increases exponentially. As a result, the RA resource congestion problem becomes worse due to reduced number of allocated preambles and RACH occasions for each partition. Even though the backoff procedure is defined to alleviate the RA resource congestion, it is not sufficient to handle the RA congestion problem with RA partitioning, since there would be the case that some of partitions are highly overloaded while the other partitions are not crowded. Therefore, further mechanisms to handle the RA congestion problem should be studied in 6G, in order to efficiently use the RA resource with the RA partitioning for different purposes. For example, the network may indicate to retransmit the Random Access preamble using different set of Random Access resources in RAR, if the network detects a congestion in the selected set of Random Access resource.

Observation 5d	We should consider simplified feature combinations and mitigation of RA overload situations for RA partitioning design.

The NR Rel-19 NES introduced PRACH adaptation which will be generally beneficial for 6G network saving energy and efficient resource management. Unlike NR Rel-19 where PRACH resources configured for legacy NR UEs can coexist with PRACH resources configured for NR Rel-19 NES UEs, PRACH adaptation for 6G can be applied to PRACH resources provided in a single configuration, assuming 6G UEs can support PRACH adaptation feature from 6G Day-1. Moreover, considering resource overhead for pre-allocated PUSCH resources and collision risk in dense UE environments for 2-step RACH, adaptation of msgA PUSCH part can be taken into account in 6G. For example, MsgA PUSCH part can be deactivated in case RACH occasions are shared by both 4-step RACH and 2-step RACH

Observation 5e	RACH resource adaptation dose not need to be limited to additional RACH resources and can be applicable to any of RACH resources configured in 6GR.

For initial access, CCCH message size i.e. Msg3/A size is fixed for RRC setup request and RRC resume request in NR. For 6G study, it is natural to discuss determination of Msg3/A size for initial access. We think that it is worth studying support for a larger CCCH message size of Msg3/A for 6G initial access, compared to Msg3/A size in NR, considering early indication e.g. for UE capability or UE type indication and early report e.g. measurement for fast proper setup.

Observation 5f	It would be beneficial to study possibility of a larger Msg3/A size, e.g. including early indication and/or early report, which could support early initial setup with appropriate configurations.

Proposal 6	Study RACH and RRC connection establishment with the following aspects:

- Support of both 2-step RACH and 4-step RACH

- Better distribution across different carriers and RATs in multi-carrier scenarios and MRSS

- Fast/reliable data transmissions with early RRC connection setup

- Simplification of feature combinations

- Mitigation of RA overload situations

- Adaptation of PRACH and MsgA PUSCH resources

- Larger Msg3/A size for initial access

2.6	Access Control

The Unified Access Control (UAC) in NR does not fully reflect the actual usage patterns of modern smartphones. For example, the same application may run in the foreground at one moment and in the background at another, but NR UAC handles both cases identically. In addition, UAC does not take into account detailed traffic attributes, such as data volume, which could be used to prioritize small, time critical transmissions when the network is congested.

For 6G, access control should be enhanced to better match real smartphone usage and detailed traffic attributes. By adapting access control policies to the actual traffic attributes, the network can improve fairness, reduce access congestion, and allocate resources more efficiently to meet UE needs, eventually enhancing both system performance and user experiences.

Observation 6a	NR UAC cannot distinguish between foreground and background operation of the same application and lack of consideration for detailed traffic attributes (e.g., data volume) limits the efficiency of scheduling under high network load.

Universal Access Control (UAC) is a comprehensive L3 framework introduced in NR for controlling access attempts from multiple UEs trying to be connected to the network. Whether a UE is allowed to initiate RACH procedure for initial access in UAC is based on access category determined based on a type of service, access identity and barring parameters broadcasted in SIB1. The NAS layer maps access attempts to these categories and identities while the RRC layers perform the access barring check.

On the other hand, RACH backoff in NR is a L2 mechanism used to control and delay PRACH transmissions during a RACH procedure. It helps to manage network congestion and reduce collision probability during the initial access phase.

When it comes to access attempt control in NR, one of the pain points is the lack of integration and coordination between L3 UAC and L2 backoff. Compared to UAC based on access category as well as access identity, RACH backoff provides relatively coarse-grained control over access attempts, limiting its effectiveness in differentiated or policy-driven access management provided by UAC. Thus, a RACH procedure triggered by an access attempt which passed the access barring check in RRC layer can still be blindly and stochastically delayed by backoff mechanism in MAC layer, regardless of its access category.

We think that 6G still needs to provide RRC-based access control and RACH backoff mechanism, but 6G can provide better integration and coordination between RRC-based access control and RACH backoff mechanism for initial access.

Observation 6b	Integration and coordination between RRC-based access control and RACH backoff mechanism are supported only in limited cases in NR. Better integration and coordination can be studied for 6G initial access.

Proposal 7	Study the following aspects for 6G access control:

- Fine-grained Access Control capable of differentiating foreground and background applications, and reflecting detailed traffic attributes (data volume, etc.).

- Unified approach for RACH backoff and RRC access control for initial access

Conclusion

SS/SI transmission for single/multi-carriers

Observation 1a:	Long periodicity of SS/PBCH/SIB1/SI transmissions can save network energy while short periodicity can reduce cell access latency.

Observation 1b:	Tx power on/off or Tx power reduction of common signal/channel transmissions for TRP or clustered beams of a cell can save network energy.

Proposal 1:	Study the following aspects for both single carrier and multi-carrier scenarios to save network energy and common signals/channel resources for 6GR:

- SS/PBCH transmission can be provided with long periodicity and/or on demand on one or more carriers of multiple carriers

- SS/PBCH transmission can be turned on/off or with a reduced TX power for TRP or clustered beams on one or more carriers of multiple carriers

- Paging can be provided only on one or more carriers of multiple carriers

- RACH can be provided only on one or more carriers of multiple carriers

Proposal 2:	Discuss which multi-carrier scenarios in Table 1 are considered for 6G study on initial access and system information:

Table 1: Multi-carrier scenarios for 6G study on initial access and system information

|  | Type-A carrier | Type-B carrier(s) |
| --- | --- | --- |
| MC Scenario 1 | - SS/PBCH - SIB1/SI | - No SS/PBCH/SIB1/SI |
| MC Scenario 2 | - SS/PBCH - SIB1/SI | - AO-SS with long periodicity and/or OD-SS |
| MC Scenario 3 | - SS/PBCH - SIB1/SI | - No SS/PBCH - (AO and/or OD-) SIB1/SI |
| MC Scenario 4 | - SS/PBCH - SIB1/SI | - AO-SS/PBCH with long periodicity and/or OD-SS/PBCH - (AO and/or OD-) SIB1/SI |

SIB1 information and transmission

Proposal 3:	Study a scalable size of modular SIB1 construction/transmission for 6GR

- SIB1 consists of multiple SIB1 modules i.e. different SIB1 information groups (e.g. cell access info, SI scheduling info, common channel configurations for different features or UE types, UE timer/constants and UAC info)

- SIB1 scheduling /transmission and SIB1 update/maintenance can be designed based on per-SIB1 module.

System information transmission

Observation 3a:	Lack of integration between OD-SIB1 operation and OD-SI operation is a pain point for NR.

Observation 3b. 	In NR, the on-demand SI procedure focused on a UE request procedure, while network-side delivery largely retained legacy broadcast mechanism (fixed SI window and whole-message transmission even for update). Consequently, potential benefits from on-demand, such as dynamic scheduling for faster required-SI delivery and signalling-efficient SI delivery based on delta signalling, have not been achieved.

Observation 3c	The system information required for initial access to NR NTN is defined separately from that for TN and is received within an independent time window, which results in additional latency and UE energy consumption.

Observation 3d	Beam/TRP specific system information can be considered e.g. for better common channel configurations in TN and NTN.

Proposal 4	Study followings for SI acquisition/update

- Unified and integrated approach among different on-demand mechanisms e.g. on-demand SI and on-demand SSB/PBCH/SIB1

- OD-SI transmission based on dynamic scheduling

- SI update based on delta SI

- SI update for fast (re)acquisition of NTN-related SI to reduce initial access latency and/or improve UE energy saving under NTN.

- Beam/TRP specific SI acquisition

Paging

Observation 4a	 Limited flexibility due to static PO configuration is a pain point for NR.

Observation 4b	 Using Two different paging configurations for legacy NR UEs and Rel-19 NES UEs is a pain point for NR.

Observation 4c	 paging transmissions with all beams over multiple cells of a large area and coarse-grained UE location knowledge from network side lead to NW energy consumption and resource inefficiency in NR.

Observation 4d	It is beneficial to study flexible paging occasion configuration/adaptation and enabling finer-grained UE presence awareness for on-demand paging to reduce unnecessary paging transmissions.

Observation 4e	Coupling of paging reception cell and access execution cell in NR lead to redundant paging transmission across all candidate frequencies to guarantee UE reachability and potential initial access congestion on frequencies where paging is transmitted in multi-carrier deployments.

Proposal 5	Study the following aspects for paging in single/multi-carrier scenarios:

- Paging reception and initial access execution across different frequencies to allow flexible paging transmission across multiple frequencies while supporting distributed and low-latency access across multiple frequencies

- Improving flexibility in paging occasion configuration/adaptation, and enabling finer-grained UE presence awareness for on-demand paging to reduce unnecessary paging transmissions.

RACH and RRC connection establishment

Observation 5a	It is beneficial to study load distribution for NR initial access mechanisms in multi-carrier deployment and MRSS deployment

Observation 5b	It is beneficial to study data transmission/reception with lower latency and higher reliability based on fast setup and immediate release/suspension of a RRC connection for network/UE energy saving.

Observation 5c	2-step RACH is beneficial for low-latency connection setup and small data transmissions.

Observation 5d	We should consider simplified feature combinations and mitigation of RA overload situations for RA partitioning design.

Observation 5e	RACH resource adaptation dose not need to be limited to additional RACH resources and can be applicable to any of RACH resources configured in 6GR.

Observation 5f	It would be beneficial to study possibility of a larger Msg3/A size, e.g. including early indication and/or early report, which could support early initial setup with appropriate configurations.

Proposal 6	Study RACH and RRC connection establishment with the following aspects:

- Support of both 2-step RACH and 4-step RACH

- Better distribution across different carriers and RATs in multi-carrier scenarios and MRSS

- Fast/reliable data transmissions with early RRC connection setup

- Simplification of feature combinations

- Mitigation of RA overload situations

- Adaptation of PRACH and MsgA PUSCH resources

- Larger Msg3/A size for initial access

Access Control

Observation 6a	NR UAC cannot distinguish between foreground and background operation of the same application and lack of consideration for detailed traffic attributes (e.g., data volume) limits the efficiency of scheduling under high network load.

Observation 6b	Integration and coordination between RRC-based access control and RACH backoff mechanism are supported only in limited cases in NR. Better integration and coordination can be studied for 6G initial access.

Proposal 7	Study the following aspects for 6G access control:

- Fine-grained Access Control capable of differentiating foreground and background applications, and reflecting detailed traffic attributes (data volume, etc.).

- Unified approach for RACH backoff and RRC access control for initial access