3GPP TSG-RAN WG2 Meeting #132	                                                             R2-2508149

Dallas, USA, Nov. 17th-21st

Source:	CATT

Title:	Views on 6GR design approaches and services

Agenda Item:	10.2.3

Document for:	Discussion and Decision

- Introduction

A new study item for 6G Radio has been approved in RAN#108 [1] and updated in RAN#109 [2]. RAN2 is requested to study radio interface protocol architecture and procedures for 6GR. In this contribution, we share our views on the outline and highlight of 6G radio interface protocol architecture and procedures, including: general considerations of 6GR design, deployment scenarios, support of diverse device types and supporting existing and new services. Details are elaborated in the following section.

- Discussion

  - General considerations of 6GR design

The user-centric network is the center of the 6G network design to target the 6G system design of improving the user experiences and meeting the QoS requirement of wireless services, minimizing the total cost of operation and maximizing the spectrum efficiency. The 6G system needs to have an intelligent system design and breakthrough in elevating the spectrum efficiency to provide the user with optimal service experience and diverse service types. The improvement of the spectrum efficiency needs to have sophisticate functions in radio resource and interference management scheme to dynamically allocate the resource with optimal link adaptation for each user. The AI/ML are the functions to enable the 6G system with emulated human intelligent and should be incorporated as the function for the radio resource and interference management with optimal link adaptations.

UE centric network is a comprehensive design, which involves many aspects across different layers, including, e.g., UE-specific service subscription, optimal mobility, smart link selection, customized energy saving, efficient control and scheduling, etc. To approach the target of UE centric, the following aspects can be considered:

- Common UE ID in an area used for access procedure;

- Common configuration across cells per UE for optimal mobility.

Observation 1: The 6G network is designed as user-centric network to optimize the user experience, which involves, e.g., UE-specific service subscription, optimal mobility, smart link selection, customized energy saving, efficient control and scheduling.

Some important KPIs for 6GR should be considered as following aspects:

- Energy efficiency: Energy efficiency is an important metric of sustainability, which is introduced as a new technical performance requirement in ITU-R for IMT-2030 [3]. It includes both network and device energy efficiency. Some aspects need to be considered to improve the energy efficiency including system information design, initial access design, UP and CP design.

- Spectrum efficiency: The requirements of spectrum efficiency for 6GR was agreed to support 3xIMT-2020 spectrum efficiency, which is an important KPI to improve system performance compared with 5G [3]. To improve the spectrum efficiency, more antennas need to be considered for both BS and UE. RAN2 also need to consider the efficient design for User Plane and Control Plane protocol and procedure.

- Service continuity: Mobility interruption time shall be sufficiently minimized to support a seamless application performance. The minimum requirement for mobility interruption time is discussed during RAN#109 and agreed as 0ms at least for mobility between TRxPs of a single base station [3]. To achieve the 0ms mobility interruption time, some mobility technologies can be consider in 6GR, e.g., LTM.

- Latency: The latency of User Plane and Control Plane was discussed during RAN#109. 4ms for IC and 1ms for HRLLC were agreed for User Plane latency and 20ms was agreed for Control Plane latency [3]. To achieve these latency requirements, some User Plane and Control Plane functionalities and procedures should be designed to reduce the latency, e.g., ARQ/HARQ functionality.

- Security: Security is one of an important KPI for communication system. For 6GR, 5G AS security for SRB/DRB can be taken as baseline. Furthermore, MAC CE and System information security should be considered in 6G.

Observation 2: 6GR design should consider some important KPIs, e.g., energy efficiency, spectrum efficiency, service continuity, latency and security.

The advantage of 5G design is flexible and configurable. Many options can be configured to support various use cases and scenarios. However, this will cause 5G design is very complicated. Thus, for 6GR, simplified and efficient is the most important design principle, including protocol architecture design, functionality design, procedure design and etc.

Observation 3: Compared with 5G, 6GR design should consider more simplified and efficient design.

Proposal 1: From RAN2 point of view, the design target of 6GR should take into account:

- User-centric network design for optimized user experience;

- Substantial gains in term of KPIs, e.g., energy efficiency, spectrum efficiency, service continuity, latency and security;

- Simplicity and efficiency design.

  - Support of diverse device types

In 4G LTE and 5G NR, several UE types have been introduced, such (e)MBB UE, and IoT UE (including (e)MTC, NB-IoT, and (e)Redcap). And the IoT specific enhancement, such as complexity reduction, coverage enhancement and power consumption reduction, were introduced based on the basic system which is designed firstly. That procedure has caused complexity of the system design.

One of the 6G SID [1]objectives is:

- Target scalable and forward compatible design for diverse device types.

To support the scalability and forward compatibility for diverse device types, and learn the lesson from 4G/5G above, we should support diverse device types in 6G day1.

Observation 4: Diverse device types should be supported from 6G day1 to support the scalability and forward compatibility, and avoid the negative impacts on the system design complexity.

Furthermore, unified system design should be considered to avoid specific design for each device type.

In RAN1# 122bis meeting, RAN1 has achieved the following agreements:

Agreement

- High-level aspects to consider for the 6GR sync signal structure include, but not limited to

  - Common design for diverse device types

From RAN2 point of view, we should also pursue common design for different device types, especially for the common signaling transmission or procedure, for example:

- SSB/SIB transmission: common SSB/SIB transmission should be used for all devices. Thus, the UE of different type should at least support 5MHz channel bandwidth.

- Paging transmission: for example, the same method of determining PO should be used, or the UE of different device types can be paged by the same paging message.

- RACH procedure: early identification of device types is used in 4G LTE and 5G NR, which causes the PRACH resources fragmentation or the Msg3 message complexity.  In 6G day1, the necessity of early identification of device type can be re-evaluated.

- UAC: as the same in 4G LTE and 5G NR, the network can also control the access of each device type separately, according to the network load or other policy.

Proposal 2: Support multiple device types from 6G day1 with common protocol design especially on common signaling transmission or procedure like, SSB/SIB transmission, paging transmission, RACH procedure and UAC.

  - Supporting existing and new services

6GR design should consider support of existing and new services to meet the technical performance requirements for IMT-2030. Our general views on Positioning, AI and Immersive communication are shared as following.

Positioning

3GPP TSG RAN has provided further input as outcome from 3GPP TSG RAN#109 in September 2025. Positioning has been included in the Technical Performance Requirements (TPRs) without the accurate requirement in detail [5].

Table 1: Technical Performance Requirements (TPRs)

| Proposed TPR items | Proposed requirement values |
| --- | --- |
| - Positioning | - Quantitative TPR - Define requirements for both outdoor and indoor scenarios - Usage scenario: ISAC |
| - Sensing-related capabilities |  |

Since 6G networks must maintain backward compatibility with essential 5G positioning use cases, ensuring service continuity for E911 and regulatory emergency call positioning from Day-1. And considering 6G Day-1 will focus on 6G SA option, i.e. not like early 5G, EN-DC was the main architecture option which E-UTRA was used to support the positioning service for regulatory cases, so positioning functionality should be supported in 6G Day-1.

Observation 5: Positioning is required in TPR and positioning functionality for regulatory emergency call case is required in 6G Day-1.

The LPP (LTE Location Protocol) between the target device and location server, and the NRPPa (NR Positioning Protocol A) between NG-RAN node and the location server, can be directly reused in 6G to enable Day-1 positioning capabilities. This ensures immediate operational continuity without requiring a total new protocol development, leveraging existing 5G infrastructure for early 6G deployments.

The Rel-16 NR positioning architecture, developed through coordination between SA2 and RAN3 groups, can be leveraged as a foundational framework for 6G.

RAT-independent methods and 6GR E-CID (Enhanced Cell ID) will be supported as baseline options in 6G, ensuring broad applicability for indoor and outdoor scenarios. RAT-dependent methods will be discussed in future specification work based on RAN1 progress, allowing for flexibility and optimization as 6G technology matures.

The potential features and technologies inherited from 5G positioning will be addressed in the 6G study. This includes evaluating backward compatibility and the evolutionary path from 5G to 6G, ensuring a smooth transition while addressing future needs like RAT-independent methods and 6GR E-CID.

Observation 6: The 6G study will address the potential technologies inherited from 5G positioning, including protocols, architecture, methods and assessing backward compatibility to ensure a smooth evolutionary path from 5G to 6G.

Proposal 3: To ensure 6G positioning deployment, it is proposed to reuse positioning protocols (e.g., LPP/NRPPa) for 6G Day-1, leveraging the NR positioning architecture in 5G as the foundational framework, prioritizing RAT-independent methods and 6GR E-CID positioning method.

AI

It is expected that AI/ML will play an important role in 6G for tackling fundamental challenges and achieving improvements in mobility robustness, energy efficiency, and other critical KPIs. No matter which use case (e.g., 5G existing use cases or future new use cases) is supported in 6G, , study on AI/ML can comprise aspects of LCM framework and data transfer framework.

Toward LCM framework, current 5G LCM framework in TR38.843 can be as a starting point. It includes multiple modules of model training, model inference, performance monitoring, model storage .etc. Based on this framework, 5G AI use cases enable closed-loop management across the entire life cycle. Whether enhancement is needed based on this framework, e.g. to unify the management of one-side model and two-side model, or to unify the management of potential new 6G use cases, can be further study.

Observation 7: 5G LCM framework in TR38.843 can be a starting point also for 6G AI/ML LCM framework.

Toward AI/ML data transfer framework, data collections for UE-side data collection and model transfer have been discussed from Rel-18 to Rel-19. However, among the numerous options, a universal final solution has not been down-selected. For UE-side data collection, there are some limitations based on legacy CP/UP solution for large volumes of data. For model transfer, the sharing of dataset/model parameters from NW-side to UE or UE-side training entity was discussed by RAN1 for two-sided models, and two alternatives of OTA and non-OTA solutions for the sharing of dataset/model parameters are preliminarily provided by RAN1. From RAN2 perspective, RAN2 should focus on the OTA solution where the control of data transfer can be held on the RAN side. For both OTA-based model transfer in DL and AI/ML data transfer in UL, new data characteristics such as large data volume, various starting and endpoints, and various QoS requirements are involved. Thus, a new information transfer framework other than CP and UP should be studied in 6G. The new information transfer framework should be a unified framework for both UL data transfer and DL model transfer, as well as for other new service types with similar data transfer requirements, e.g., sensing.

Observation 8: Both OTA-based model transfer in DL and AI/ML data transfer in UL involve new data type characteristics such as large data volume, various starting and endpoints, and various QoS requirements.

Proposal 4: To design ‌a unified data transfer framework (other than CP/UP) for both UL data transfer and DL model transfer‌ in AI/ML, as well as for other new service types (e.g., sensing) with similar data transfer requirements.

Immersive communication

Immersive communication extends the enhanced Mobile Broadband (eMBB) and includes the typical use cases of communication for immersive XR, remote multi-sensory telepresence, and holographic communications. Supporting mixed traffic of video, audio, and other environment data in a time-synchronized manner is an integral part of immersive communications, including also stand-alone support of voice [4].

In TR38.914, many KPIs are defined considering immersive communication usage scenario, such as peak data rate, peak spectral efficiency, 5th percentile user data rate, 5th percentile user spectral efficiency, average spectral efficiency, area traffic capacity, user plane latency, control plane latency, mobility, mobility interruption time, and composite requirement [3]. The composite requirement is specifically targeted for immersive communication, which refers to simultaneously satisfying data rate, latency, packet success probability, and the number of users per TRxP. It combines the KPIs of data rate, latency and reliability for UE and the system capacity for NW. The minimum performance requirement of composite requirement summarized in [3] is shown in Table 2.

Table 2: Composite requirement

| [Test environment] | Data rate (Mbit/s) | Latency (ms) | Packet success probability | Number of users (TRxP) |
| --- | --- | --- | --- | --- |
| Dense Urban - Immersive Communication | DL = 30 Mbps/UL=10 Mbps | DL=10 ms / UL=30 ms (or DL+UL total 40 ms) | 99% | 6 |

The consistent user experience should be considered for immersive communication especially because it requires consistent high data rate and low latency anywhere. Note that the 6G system shall support service continuity for immersive communication such as mixed reality gaming between indoor and outdoor, and between Dense Urban and Rural deployments. 5G had considered XR mobility but didn’t address it specially. In [3], the KPI of mobility is defined as the maximum mobile station speed at which a defined QoS can be achieved (in km/h), and traffic channel link data rates for mobility are required as Table 3. Therefore, 6G should consider mobility for immersive communication from Day1.

Table 3: Traffic channel link data rates normalized by bandwidth

| [Test environment] | Normalized traffic channel link data rate (bit/s/Hz) | Mobility (km/h) |
| --- | --- | --- |
| Indoor Hotspot – IC | 2.25 | 10 |
| Dense Urban – IC | 1.68 | 30 |
| Rural – IC | 0.88~1.2 | 120 |
|  | 0.495~0.675 | 500 |

Compared to 5G XR, the required data rate is improved significantly. XR service data rates are 30/45Mbps downlink and 10 Mbps uplink for AR scenarios in Rel-17 XR evaluation; while 6G requirements with increased video resolution and frame rate can reach Gbps levels. Meanwhile, there are many new device types of immersive communications, such as HMD/AR glasses, haptics gloves, smart watches, and holographic projectors, etc. Due to lightweight and comfortable user experience design,  energy efficiency is important to the devices. 5G XR had researched power saving but only non-integer DRX period was considered in RAN2. In 6G design, both high data rate and energy efficiency need to be considered for UE.

Proposal 5: For immersive communication, RAN2 can initiate research from several aspects: the composite requirement, mobility requirement with high data rate, and the energy efficiency for the UE with high data volume.

Immersive communication has many attractive characteristics. Some characteristics have potential RAN2 impacts are:

- Service awareness: 6G immersive communication has the similar and extended service awareness requirements compared with 5G XR, such as the packets formats of data burst, PDU sets and service characteristics of variable packet size, multiple importance levels, jitter, etc. Service awareness is important to guarantee the service continuity and improve user experience. 6G should support service aware QoS management and scheduling from Day1.

- Adaptive QoS: Referring to the Potential New Requirements in TR22.870, [PR 9.8.6-2], [PR 9.11.6-2], [PR 9.12.6-3], and [PR 9.13.6-1] mention adaptive QoS from different aspects. The 6G network shall support QoS adjustment based on network status, user experience, QoE policies, and media traffic requirements, etc. In 5G, RAN nodes attend some QoS management such as alternative QoS for GBR and RAN feedback for adaptation of Burst Arrival Time and Periodicity. Thus, 6G RAN involvement will benefit to adaptive QoS further.

- Multi-modality: The synchronized multi-modal data streams require synchronized data transmission within intra-UE and inter-UE, referring to [PR 9.3.6-4], [PR 9.7.2-1], [PR 9.10.6-1], [PR 9.12.6-2], [PR 9.12.6-4], and [PR 9.12.6-5] in TR22.870. While multi-modality was discussed in 5G XR, 5GS lacked native support, even for single-UE multi-modality. RAN could not accurately determine synchronization delay requirements related to packet sequencing. Although the synchronized multi-modal data streams could be guaranteed roughly at the application layer, RAN-level solution is essential for fine-grained management, so as to maximize system capacity for immersive communication.

- Error tolerance: In 5G XR, error tolerance considering FEC has been discussed. 6G will support error-tolerant communication by leveraging AI technique further, and RAN impacts should be considered, referring to [PR 9.11.6-4] in TR22.870.

- HOL: The issue of Head-of-Line (HOL) blocking mainly related to QUIC (Quick UDP Internet Connections) protocol was proposed in some companies’ papers last meeting. The QUIC protocol integrates the reliability of the TCP protocol and the transmission speed/efficiency of the UDP protocol. It is expected to be applied in immersive communications. Compared to the TCP protocol, QUIC protocol improves the HOL issue through multiple independent streams. This may have impacts on the flow mapping mechanism in SA2 and RAN2.

Above issues may have impacts on RAN2 design or require RAN2’s involvement for performance improvements. However, it is difficult to identify RAN2 work in this stage. More information and requirements from SA2 and SA4 are needed.

Proposal 6: RAN2 should consider service awareness, adaptive QoS, multi-modality, error tolerance, and HOL for immersive communication and coordination with SA2 and SA4 is needed.

Sensing

In 5G-A, TRP mono-static sensing for UAV use case is supported. In RAN#109, it is agreed that 6GR should study the sensing modes, including TRP monostatic, TRP-TRP bistatic, TRP-UE DL, UE-TRP UL, UE-UE bistatic and UE monostatic. For the sensing modes only involving TRP, there is no RAN2 impact. For the sensing modes involving UE, UE acts as a sensing signal transmitter and/or receiver. Thus sensing signal related configuration needs to be provided to the UE so that the UE can perform sensing. In addition, the UE acting as a sensing signal receiver needs to provide sensing data to the network. Thus a UE sensing data collection mechanism needs to be studied.

At the last meeting, RAN2 agreed on the timeline for initiating the sensing study

RAN2 will start sensing study in April 2026

Once the sensing study starts, RAN2 shall focus on studying procedures for UE sensing signal configuration and UE sensing data collection, targeting UE-involved sensing modes.

Proposal 7: In order to support the sensing modes involving UE, RAN2 should study procedures of UE sensing signal configuration and UE sensing data collection.

- Conclusion

In this contribution, we provide our general views on 6G Radio Access Technology including below observations and proposals:

General considerations of 6GR design

Observation 1: The 6G network is designed as user-centric network to optimize the user experience, which involves, e.g., UE-specific service subscription, optimal mobility, smart link selection, customized energy saving, efficient control and scheduling.

Observation 2: 6GR design should consider some important KPIs, e.g., energy efficiency, spectrum efficiency, service continuity, latency and security.

Observation 3: Compared with 5G, 6GR design should consider more simplified and efficient design.

Proposal 1: From RAN2 point of view, the design target of 6GR should take into account:

- User-centric network design for optimized user experience;

- Substantial gains in term of KPIs, e.g., energy efficiency, spectrum efficiency, service continuity, latency and security;

- Simplicity and efficiency design.

Support of diverse device types

Observation 4: Diverse device types should be supported from 6G day1 to support the scalability and forward compatibility, and avoid the negative impacts on the system design complexity.

Proposal 2: Support multiple device types from 6G day1 with common protocol design especially on common signaling transmission or procedure like, SSB/SIB transmission, paging transmission, RACH procedure and UAC.

Positioning

Observation 5: Positioning is required in TPR and positioning functionality for regulatory emergency call case is required in 6G Day-1.

Observation 6: The 6G study will address the potential technologies inherited from 5G positioning, including protocols, architecture, methods and assessing backward compatibility to ensure a smooth evolutionary path from 5G to 6G.

Proposal 3: To ensure 6G positioning deployment, it is proposed to reuse positioning protocols (e.g., LPP/NRPPa) for 6G Day-1, leveraging the NR positioning architecture in 5G as the foundational framework, prioritizing RAT-independent methods and 6GR E-CID positioning method.

AI

Observation 7: 5G LCM framework in TR38.843 can be a starting point also for 6G AI/ML LCM framework.

Observation 8: Both OTA-based model transfer in DL and AI/ML data transfer in UL involve new data type characteristics such as large data volume, various starting and endpoints, and various QoS requirements.

Proposal 4: To design ‌a unified data transfer framework (other than CP/UP) for both UL data transfer and DL model transfer‌ in AI/ML, as well as for other new service types (e.g., sensing) with similar data transfer requirements.

Immersive communication

Proposal 5: For immersive communication, RAN2 can initiate research from several aspects: the composite requirement, mobility requirement with high data rate, and the energy efficiency for the UE with high data volume.

Proposal 6: RAN2 should consider service awareness, adaptive QoS, multi-modality, error tolerance, and HOL for immersive communication and coordination with SA2 and SA4 is needed.

Sensing

Proposal 7: In order to support the sensing modes involving UE, RAN2 should study procedures of UE sensing signal configuration and UE sensing data collection.

- References

- RP-251881, Study on 6G Radio.

- RP-252912, Revised SID: Study on 6G Radio.

- RP-252882, 3GPP TS 38.914 v0.2.0, Study on 6G Scenarios and Requirements.

- Recommendation ITU-R M.2160-0 Framework and overall objectives of the future development of IMT for 2030 and beyond, ITU, 2023.11.

- RP-252946 Draft Reply LS to ITU-R_WP5D_TEMP_167 = RP-243202 on Minimum requirements related to technical performance for IMT-2030 radio interface(s) (to: SA; cc: -; contact: CMCC)