3GPP TSG-RAN WG2 Meeting #132										R2-2508210

Dallas, USA, November 17th-21st, 2025

Agenda Item:	10.3.2.3

Source: 	Sharp

Title:  	Considerations on Initial and System Access and Others

Document for: 	Discussion

# Introduction

In RAN2_131bis meeting, RAN2 had following agreements on System Information [1]:

Agreements on System Information

-	System information design should consider energy efficiency and low complexity for both network and UE.

-	System information design should ensure that the public warning requirements are met.

-	Study RAN2 aspects of system information designs (e.g. on-demand SIB and SSB, periodicity of SSBs)

-	Study how to improve flexibility and extensibility of scheduling SI design

-	Study mechanisms to improve SI update mechanism

-	Study aspects related to SIB1 size.  Understand the issues and desired content/size from RAN2 point of view.

-	Study areas specific SIB design.  Understand 5G pain points and what improvements can be considered.

NOTE: this is a starting list

This document discusses 6GR Initial and System Access and Others.

# Discussion

## Synchronization signal and system information

In NR, synchronization signals and essential system information are transmitted periodically. For initial cell selection, the UE assumes that SS/PBCH block (SSB) is transmitted with 20ms periodicity, which has implications for network power consumption.

Once a cell is detected, the UE can obtain neighbour cell/frequency information, including SMTC, enabling efficient detection and measurement of neighbour cells. This mechanism contributes to both UE energy saving and network energy saving (NES). Furthermore, On-demand SSB and On-Demand SIB1 mechanisms were introduced for NES purpose.

For 6GR, considering the above mechanisms, it is important to study a framework for broadcasting synchronization signals and system information that ensures efficient initial access while improving overall energy efficiency. Although NR has multiple independent mechanisms for different purposes, 6GR study should discuss all together to prepare a comprehensive solution.

Proposal 1: Study a framework for broadcasting synchronization signals and system information that ensures efficient initial access and improved energy efficiency.

For energy efficient synchronization signals and system information, we can consider NR, those solutions were designed in backward compatibility manner, so the gain had some limitations. For example, on-demand SIB1 is only supported for non-stand-alone cell operation in NR, i.e. the UE has to obtain OD-SIB1 configuration from Cell A before it requests SIB1 in NES cell. In our view, RAN2 should study to utilize OD-SIB1 for stand-alone cell operation in 6GR to extend application scenarios, i.e. the UE can obtain OD-SIB1 configuration from stand-alone NES cell.

Proposal 2: RAN2 to study On-demand SIB1 in both non-standalone cell operation (i.e. OD-SIB1 configured by Cell A) and stand-alone cell operation (i.e. OD-SIB1 configured by stand-alone NES cell).

For the similar reason, On-demand SSB and On-demand SIB1 have limitations on RRC states. On-demand SIB1 is supported in RRC_IDLE/INACTIVE and RRC_CONNECTED with T331 running, and On-demand SSB is only supported in RRC_CONNECTED. We should consider redesigning and generalizing those NR mechanisms in 6GR so that they are not limited to specific RRC states.

Proposal 3: RAN2 to study On-demand SSB and On-demand SIB1 applicable across all RRC states.

## System information acquisition

System Information (SI) framework in NR introduced major enhancements over LTE. Such enhancements include on-demand SI request and area-specific SI. The on-demand SI request reduces periodic transmission of non-essential SIBs, resulting in energy saving in gNB. Whereas the area-specific SI decreases UE’s necessity to acquire non-essential SIBs every time it camps on a new cell. We believe that such enhancements will continue to be beneficial in 6GR, and the 6GR SI framework should be based on the framework of NR.

Proposal 4: NR SI framework is a baseline for 6GR SI framework.

That said, there are areas of concerns in the NR SI framework. For example, when a SIB gets updated, the UE will be notified by the Short Message, but the UE will not know which SIB (except ETWS/CMAS) is updated until it re-acquires SIB1. This may cause unnecessary acquisition of SIB1 and an additional delay for acquiring the updated SIB. Thus, it is desired that the system information update notification for 6GR carries more graduality on type of updated information.

Proposal 5: The system information update notification for 6GR carries more graduality on type of updated information.

The other example of concern is with regard to modification periods for on-demand SI. In NR, if an SI message is requested at the early time of a modification period, the gNB is supposed to broadcast the SI message at least until the end of the modification period. Depending on the length of the modification period, this may result in energy waste at the gNB. On the other hand, if the on-demand request is made at time very close to the end of the modification period, the UE may not have enough number of SI-windows to ensure acquisition of the requested SI message. In our view, the on-demand SI acquisition in 6GR should not be constrained by the modification period.

Proposal 6: The on-demand SI acquisition in 6GR should not be constrained by the modification period.

## Cell status/cell restrictions

In NR, the IDLE/INACTIVE mode procedures for cell status and cell reservations have been expanded in a patchy manner, whenever a new feature was introduced. Since Release 15, we introduced cell barring status for ATG, 2Rx XR, eRedCap, VSAT (fixed and mobile), NES, NTN, half duplex RedCap, IAB, NCR and Mobile IAB. The consequence of this patchy approach was a need to specify complicated rules of how to treat those cell status bits (e.g., what to consider and what to ignore) based on the device capabilities, consuming a lot of time in online/offline discussion time to reach consensus. In 6GR, we should study a simple but flexible approach implemented at the first place to accommodate with future extensions.

Proposal 7: Study a simple but flexible approach for cell status/cell restrictions to accommodate with future extensions.

# Conclusion

Based on the discussion given above, RAN2 is requested to discuss the observations and agree the following proposals:

Proposal 1: Study a framework for broadcasting synchronization signals and system information that ensures efficient initial access and improved energy efficiency.

Proposal 2: RAN2 to study On-demand SIB1 in both non-standalone cell operation (i.e. OD-SIB1 configured by Cell A) and stand-alone cell operation (i.e. OD-SIB1 configured by stand-alone NES cell).

Proposal 3: RAN2 to study On-demand SSB and On-demand SIB1 applicable across all RRC states.

Proposal 4: 5G SI framework is a baseline for 6G SI framework.

Proposal 5: The system information update notification for 6G carries more graduality on type of updated information.

Proposal 6: The on-demand SI acquisition in 6G should not be constrained by the modification period.

Proposal 7: Study a simple but flexible approach for cell status/cell restrictions to accommodate with future extensions.

# References

[1] RAN2_131bis meeting notes.