3GPP TSG RAN WG2 #132         	                                                                   R2-2508235

Dallas, USA, Nov 17th – 21st, 2025

Source:	CATT

Title:	Discussion on L2 Retransmission and Scheduling

Agenda Item:	10.3.1.3

Document for:	Discussion and Decision

- Introduction

In RAN2#131bis, 6G scheduling was initially discussed and reached the following agreements:

Agreements

- Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

- Study need for scheduling enhancements to address the SR/BSR/DSR latency

- Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

In this contribution, the following three issues will be further discussed:

- Issue 1：How to reduce the L2 retransmission latency/efficiency?

- Issue 2：How to design the SR/BSR/DSR?

- Issue 3：How to support configured grant in 6G?

- Discussion

  - L2 retransmission latency/efficiency

In 5G Rel-19 XR, in order to ensure the timely RLC retransmission, the following were agreed:

Agreement in RAN2#128 - Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them. Agreement in RAN2#129 - Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC

The autonomous retransmission and/or polling enhancement can reduce the L2 retransmissions latency, which is important for delay-critical service, e.g., XR in 5G and immersive communication in 6G. Hence, in 6G, when discussing how to reduce the L2 retransmission latency, the mechanisms introduced in Rel-19 XR can be used as an candidate.

Observation 1:  The RLC timely retransmission mechanisms introduced in 5G Rel-19 XR can be regarded as a candidate for reducing L2 retransmission latency.

Furthermore, in RAN2#131bis meeting, it was agreed that RAN2 should study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ. In 4/5G, RLC retransmission is mainly based on RLC status report. If tightly interworking between HARQ/ARQ are considered, RLC retransmission based on HARQ can be considered. Considering the HARQ mechanisms in UL and DL are different, hence, the analysis is performed for UL and DL separately:

- DL transmission

For DL HARQ transmission, UL HARQ feedback is supported in 5G. NW can perform RLC retransmission based on HARQ NACK feedback. For example, after a number of HARQ retransmission, if the HARQ feedback of this HARQ process is still NACK, RLC retransmission can be triggered. It can reduce the RLC retransmission latency compared with the RLC retransmission based on RLC status report.

Regarding to the HARQ feedback, if HARQ NACK to ACK error happens, it should rely on RLC status report or timely RLC retransmission in Rel-19 to recover from the error; and if HARQ ACK to NACK error happens, unnecessary RLC retransmission will be triggered, it will result resource waste.

- UL transmission

For UL HARQ transmission, no DL HARQ feedback channel is supported in 5G. If the same physical channel design was reused in 6G, it should consider how to trigger RLC retransmission based on HARQ. One possible solution is that UE can trigger RLC retransmission based on UL grant for HARQ process retransmission, e.g., if UE has received UL grant for HARQ process retransmission for a number of times, it can trigger RLC retransmission.

Observation 2:  RLC retransmission based on HARQ information (e.g., HARQ FB for DL transmission or UL grant of HARQ retransmission) can be regarded as another candidate for reducing L2 retransmission latency.

Proposal 1: In order to improve L2 ARQ latency/efficiency, the following two candidates can be considered as baseline:

- RLC timely retransmission introduced in 5G Rel-19 XR.

- RLC retransmission based on HARQ (e.g., HARQ FB for DL transmission or UL grant of HARQ retransmission).

  - SR/BSR/DSR design

In 5G, the UL scheduling information includes SR, BSR, DSR and PHR. PHR mainly depends on RAN1, hence, only SR/BSR/DSR are discussed here. Regarding to the SR/BSR/DSR, some lessons can be learnt from 5G:

- For the BSR and DSR, they are both used for buffer status reporting. The difference between them is that only delay-critical data is reported in DSR. These two functions is a bit repetitive;

- Before UE gets UL grant, it should first send SR, and when it gets UL grant, it can further report BSR. This will increase the UL latency.

Observation 3: In 5G, the functions of BSR and DSR are repetitive which will increase the MAC CE overhead and reduce the resource efficiency.

Observation 4: SR/BSR will increase the UL scheduling latency which is not friendly to delay-sensitive services.

Regarding to Observation 3, it is suggested to consider unified design for BSR and DSR, e.g., one MAC CE contains both delay-critical data and non-delay critical data.

Proposal 2: Unified design of BSR/DSR can be considered in 6G.

Regarding to Observation 4, the UL scheduling latency will impact the UE experience, especially for delay-sensitive services. In RAN2#131bis meeting, it was agreed that RAN2 to study the need for scheduling enhancements to address the SR/BSR/DSR latency. In our understanding, it is necessary to discuss this issue and solve it in order to meet the latency requirement of delay-sensitive services.

From RAN2 perspective, there are three candidates for this issue, listed below:

- Option 1: New SR design allows SR to include additional information, e.g., buffer info.

- Option 2: Using configured grant to transmit BSR/data, latency between SR and BSR can be omitted.

- Option 3: Using contention-based PUSCH to carry BSR/data.

For Option 1, it has already been proposed and studied in 5G, but it was not introduced finally due to it impacts to RAN1.

For Option 2, in 5G, type 1 configured grant is introduced for reducing SR/BSR latency. If UE has delay-sensitive services, network can configure these delay-sensitive services to use the type 1 configured grant through configuring LCH mapping restrictions.

For Option 3, contention-based PUSCH had also been discussed in LTE and 5G. But it was not finally specified due to its complexity, e.g., when to choose the contention based resource, how to solve the collision and etc.  Its benefits and shortages should be carefully studied compared with Option 2.

Proposal 3: It is necessary to study enhancements to address the SR/BSR/DSR latency. And the following three candidates can be as a starting point:

- Option 1: New SR design allows SR to include additional information, e.g., buffer info.

- Option 2: Using configured grant to transmit BSR/data.

- Option 3: Using contention-based PUSCH to carry BSR/data.

  - Configured grant

In RAN2#131bis, it agreed to support at least the following scheduling schemes: dynamic grant and configured grant. Further study configured grant.

In 5G, there are two kinds of configured grant, Type 1 and Type 2, as defined in TS38.300:

- With Type 1, RRC directly provides the configured uplink grant (including the periodicity). - With Type 2, RRC defines the periodicity of the configured uplink grant while PDCCH addressed to CS-RNTI can either signal and activate the configured uplink grant, or deactivate it; i.e. a PDCCH addressed to CS-RNTI indicates that the uplink grant can be implicitly reused according to the periodicity defined by RRC, until deactivated.

In 6G, there are four possible way forward:

- Option 1:  Only support Type 1 configured grant.

- Option 2:  Only support Type 2 configured grant.

- Option 3:  Support both Type 1 configured grant and Type 2 configured grant.

- Option 4:  A unified  configured grant design.

Considering Type 1 CG mainly focus on delay-sensitive service and Type 2 is aiming to reduce PDCCH overhead. The purposes of these two types of configured grant are  different,  it is not reasonable to merge them into one unified configured grant design. Hence, a unified configured grant is not proper. Option 4 can be excluded.

Furthermore, as analyzed in section 2.2, the function of Type 1 configured grant is duplicated with CB-PUSCH. Whether to support Type 1 configured grant depends on which candidate is selected in section 2.2. Considering this is study item, we can take both Type 1 configured grant and Type 2 configured grant as starting point.

Proposal 4: In 6G, as a starting point, both Type 1 and Type 2 configured grant can be further studied.

Furthermore, in 5G NR, the following configured grant enhancements were introduced:

- In Rel-16 IIOT, multiple active configured grant configurations (either Type 1 or Type 2) can be active in one BWP was introduced in order to satisfy the requirements of different services which with different traffic patterns.

- In Rel-18 XR, multi-PUSCH occasions was introduced for one configured grant configuration to solve the jitter issue.

Hence, in 6G, it should also discuss whether the above enhacements should be supported in 6G DAY1. From the perspective of services, it is inevitable that different services with different traffic patterns, hence, it is necessary to support multiple active configured grant configurations in one BWP. Furthermore, immersive communication is supported in 6G DAY1, hence, the jitter issue still exists. Hence, multi-PUSCH occasions should be supported.

Proposal 5: In 6G, multiple active configured grant configurations in one BWP and multi-PUSCH occasions should be supported.

- Conclusion

According to the above analysis, we raise the observations and proposals below:

Reducing L2 retransmission latency/efficiency

Observation 1:  The RLC timely retransmission mechanisms introduced in 5G Rel-19 XR can be regarded as a candidate for reducing L2 retransmission latency.

Observation 2:  RLC retransmission based on HARQ information (e.g., HARQ FB for DL transmission or UL grant of HARQ retransmission) can be regarded as another candidate for reducing L2 retransmission latency.

Proposal 1: In order to improve L2 ARQ latency/efficiency, the following two candidates can be considered as baseline:

- RLC timely retransmission introduced in 5G Rel-19 XR.

- RLC retransmission based on HARQ (e.g., HARQ FB for DL transmission or UL grant of HARQ retransmission).

SR/BSR/DSR design

Observation 3: In 5G, the functions of BSR and DSR are repetitive which will increase the MAC CE overhead and reduce the resource efficiency.

Observation 4: SR/BSR will increase the UL scheduling latency which is not friendly to delay-sensitive services.

Proposal 2: Unified design of BSR/DSR can be considered in 6G.

Proposal 3: It is necessary to study enhancements to address the SR/BSR/DSR latency. And the following three candidates can be as a starting point:

- Option 1: New SR design allows SR to include additional information, e.g., buffer info.

- Option 2: Using configured grant to transmit BSR/data.

- Option 3: Using contention-based PUSCH to carry BSR/data.

Configured grant

Proposal 4: In 6G, as a starting point, both Type 1 and Type 2 configured grant can be further studied.

Proposal 5: In 6G, multiple active configured grant configurations in one BWP and multi-PUSCH occasions should be supported.