3GPP TSG-RAN WG2#132	R2-2508271

Dallas, USA, Nov 17th – 21st 2025

Source: 	Fujitsu

Agenda item:	10.4

Title:	Discussion on 6GR Mobility

Document for:	Discussion and decision

Introduction

The SID of Rel-20 Study on 6G Radio was approved in RAN#108 and updated in RAN#109 [1].

(1) Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

The SID also describes the following objective about mobility and related RRM for 6GR.

(6) Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

In this contribution, we would like to provide our views on 6GR mobility aspects in terms of possible improvements/enhancements from NR.

Discussion

Mobility for idle/inactive mode

For RRC_IDLE/RRC_INACTIVE state in 5G NR, UE based mobility, e.g. cell reselection is supported. In addition, measurement is required to support mobility mechanisms.

For cell reselection in 5G NR, the following aspects are considered:

- Cell reservation, e.g. cell lists

- Cell-ranking criterion R

- Reselection priorities: e.g. absolute NR frequency priority, slice-based reselection priority, Service specific prioritization, cell-type specific prioritization

- Mobility states and scaling, e.g. normal-mobility state, medium-mobility state and high-mobility state

Cell reselection in NR can be considered as a baseline for 6GR. NR also introduces RRM measurement relaxation for IDLE/INACTIVE mode UE to achieve power saving. Energy efficiency should be considered in 6GR. So, RAN2 can take energy efficiency into account for 6GR cell reselection procedure.

Proposal 1: RAN2 to consider NR design as the baseline for 6GR cell reselection, including:

- cell-ranking criterion R

- priority based intra/inter-frequency reselection

- mobility speed-based scaling

Connected mode mobility

Supporting features in 6GR

In 5G, RAN2 had been seeking for mobility with less interruption time and robustness, and have introduced many types of connected mode mobility, which are Conditional Handover (CHO), Dual Active Protocol Stack (DAPS) and L1/L2 Triggered Mobility (LTM) in addition to baseline L3 HO, as shown in Table 1.

Table 1: Summary of NR mobility mechanisms

|  | RRC involved mobility |  |  |  | Mobility without RRC involved |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | basic HO | DAPS HO | RACH-less HO | CHO | LTM | CLTM |
| preparation | One cell is provided to the UE |  |  | Multiple cells are provided to the UE |  |  |
| Pre-execution | N/A |  |  |  | - TCI state (de)activation - Early TA acquisition - Early CSI acquisition and reporting |  |
| Execution | - RRC signalling triggered; - RACH-based | - RRC signalling triggered; - RA-based; - DL data with SgNB | - RRC signalling triggered; - No RA | - Evaluation of L3 event(s) - RACH-based | - MAC CE triggered; - RA-based or no RACH | - Evaluation of L1 or L3 event(s) - RA-based or no RA |
| completion | RACH completion (expect RACH-less HO) |  |  |  | Completion of RACH or the first UL TX via CG (or DG) |  |

As summarized in Table 1, the following aspects related to mobility are included:

- Pre-configuration for candidate cell

- Early DL synchronization

- Early UL synchronization

- Early CSI acquisition and reporting

- Condition evaluation

- Handover command via MAC CE or RRC signaling

- UL transmission in target via RAR, CG or DG

- Subsequent mobility without reconfiguration of candidates (LTM-like)

At last meeting, RAN2 agreed to study mobility with the following requirements in 6G mobility design: minimized interruption time and service continuity, robustness, energy efficiency for both UE and network [2]. Besides, we think that resource efficiency and implementation complexity for both UE and network are important. On the other hand, RAN2 will study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition). So, we will study these features by considering mobility related requirements.

Table 2: Analysis of mobility related features

|  | Minimized interruption time | robustness | energy efficiency | resource efficiency | complexity |
| --- | --- | --- | --- | --- | --- |
| Pre-config | Reduce TRRC |  |  |  |  |
| Early DL Sync | Reduce TDL-sync |  | for both UE and NW |  |  |
| Early UL sync via RACH | Reduce TRA and TRAR |  | RA for multiple times | reserved CG resource |  |
| UE measured TA | Reduce TRA and TRAR |  |  |  | UE complexity to calculate candidate TA |
| Early CSI acquisition | no service degradation |  | if CSI-RS is used |  |  |
| Conditional execution |  | improved HO success rate | if CSI-RS is used for evaluation | Need reserved resources |  |
| CG for 1st tx | reduce Tfirst-data |  |  | reserved CG resource |  |
| DG for 1st tx | need to wait for NW scheduling |  |  | no reserved UL grant | NW complexity |
| Subsequent HO | near 0ms interruption time |  |  | Need reserved resources | both UE and NW complexity |
| - Note: Meaning for each component of mobility latency is same as the description in Annex G [3] |  |  |  |  |  |

As a starting point, an analysis of mobility related features from requirements point of view is captured in Table 2. In this table, mobility related features including pre-configurations, early DL synchronization, early UL synchronization via RACH, UE measured TA, early CSI acquisition and reporting, conditional execution, configured grant-based 1st transmission in target, dynamic grant-based 1st transmission in target and subsequent HO are evaluated. In addition to agreed mobility requirements, i.e. minimized interruption time and service continuity, robustness, and energy efficiency for both UE and network, resource efficiency and complexity at UE/network for mobility related features are also considered.

It is difficult to have a conclusion without any analysis and thus a table can be made to evaluate listed features considering different requirements.

Proposal 2: RAN2 to make a table (Table 2) for an analysis of mobility related features at least including pre-configurations, early DL synchronization, early UL sync via RACH, UE measured TA, early CSI acquisition, conditional execution, subsequent HO, from requirements point of view.

Mobility framework

During RAN 6G workshop, some companies proposed a unified framework and signalling structure for multiple mobility mechanisms. According to SID [1], 6GR should minimize the adoption of multiple options for the same functionality, focusing on practical user experience and thus RAN2 should avoid specifying so many kinds of mobility mechanism in 6GR. So, we think that it is reasonable to apply a unified design, which will at least support variable existing services/needs and provide better forward compatibility to address the future services/needs.

However, there are different understandings of the unified framework based on companies’ contributions submitted at last RAN2 meeting. One direction is to design a unified mobility configuration, i.e. common RRC signaling at least for CHO, LTM and CLTM. Another direction is a single framework for mobility procedure based on different components/features, e.g. pre-configuration, execution, early UL/DL synchronization. Also, a modular mobility procedure using feature set was mentioned.

In our opinion, at the early stage of 6GR study, there is no need to choose one direction for further study. We can study different directions and down select at later stage or during WI.

Proposal 3: In 6GR, for network-controlled mobility in RRC_CONNECTED state, RAN2 will study the following aspects:

- Direction 1: a unified/simplified mobility configuration

- Direction 2: a unified/modular mobility procedure considering features in P2

RLM/RLF and recovery

RLM/RLF

In NR standalone scenario, the UE performs RLM on PCell. The configured RLM-RS resources can be all SSBs, or all CSI-RSs, or a mix of SSBs and CSI-RSs. The UE is not required to perform RLM outside the active DL BWP.

Regarding the RLF related operation in NR, upon receiving N310 consecutive "out-of-sync" indications for the PCell from lower layers, the timer T310 is started. Upon receiving N311 consecutive "in-sync" indications for the PCell from lower layers while T310 is running, the UE will stop timer T310. Upon T310 expiry in PCell or upon random access problem indication from MCG MAC or upon indication from MCG RLC that the maximum number of retransmissions has been reached, the UE considers radio link failure (RLF) detected for MCG.

For 6GR, we assume that the legacy RLM and RLF scheme can be used as the baseline. Some enhancements can be considered, since the legacy RLM is only performed on the PCell. If RLF is detected on the PCell, the UE will perform RRC re-establishment regardless of the channel quality of SCells.

In RAN2#131bis meeting, it is agreed to study spectrum aggregation of multiple pieces of spectrum and to study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc.) [2]. A new scenario of spectrum aggregation is to have one cell with multiple carriers. In this scenario, how to perform RLM and the condition to declare RLF on the cell can be further studied. For example, it can be studied on which carrier(s) RLM is performed and whether RLF on the cell is declared when RLF is detected on all the carriers in the cell.

Proposal 4: The RLM and RLF scheme in NR is taken as the baseline for 6GR. RAN2 to further study the enhancements on RLM and RLF in case of a single cell with multiple carriers.

Fast recovery

In NR, in case of non-DC, the UE will initiate RRC re-establishment when failure happens, e.g. RLF, HOF and integrity check failure. In Rel-16, CHO was introduced and if CHO is configured, handover towards CHO candidate cells can be performed if the selected cell is one of CHO candidates during the cell selection after HO/CHO failure or RLF, which is fast recovery. In Rel-18, RAN2 introduced LTM for mobility and LTM candidate cells can be used for the fast recovery as well.

With this mechanism, interruption time after failure can be shortened. However, applicable cells/ scenarios for the fast recovery are limited. For example, when inter-CU LTM cell switch fails, the cell towards which UE performs the fast recovery needs to be within the same CU as the original target cell due to lack of new security key. Therefore, fast recovery might not be so efficient in NR although it is better than nothing. We think that RAN2 should study more efficient recovery mechanisms in 6G.

Proposal 5: RAN2 to study more efficient recovery mechanisms in 6G.

Inter-RAT mobility

In RAN#109, RAN plenary has approved to study whether any additional migration option(s) is needed [1]. This study will be started from March 2026, and RAN will make a decision in September 2026. It means that RAN2 should focus on the standalone, MRSS and inter-RAT mobility between NR-6G unless RAN plenary requests the additional migration option(s).

Observation 1: RAN2 is required to start studying the following 6G migration scenarios:

- Standalone, MRSS and inter-RAT mobility between NR-6G

From RAN2 perspective, it may be useful to study and identify initial 6G deployment scenarios. For example, it is relevant to inter-RAT mobility functions supported in Day1.

Regarding inter-RAT mobility, cell reselection/handover between E-UTRA and 5G NR, redirection to E-UTRA and handover with SRVCC operation to UTRA are supported in 5G NR. To support inter-RAT mobility, inter-RAT measurement is needed. In 5G NR, inter-RAT event B1/B2 is used for measurement reporting.

For inter-RAT cell reselection, the following use cases are supported in 5G NR:

- Cell reselection between NR RRC_IDLE and E-UTRA RRC_IDLE

- Cell reselection from NR RRC_INACTIVE to E-UTRA RRC_IDLE

In addition, when mobility from other RAT fails, RRC connection establishment procedure is initiated for failure recovery.

In our view, network-controlled mobility and UE-based mobility should be supported between 6GR and 5G NR at the initial phase of 6G deployment. Measurement and failure recovery are supported as well.

Proposal 6: Inter-RAT measurement, cell reselection, handover and HOF recovery between 6GR and 5G NR are supported. Details are further studied after some progress is made in CP study and intra-6GR mobility.

Measurement framework

The UE will perform measurements for cell (re)selection in RRC_IDLE/INACTIVE mode. Also, the UE will perform measurements for cell/beam level mobility, radio link monitoring and beam failure detection/recovery. Basically, measurements can be used for these purposes in 6GR.

In NR, both L1 and L3 based measurements are supported. The measurement models are shown below:

Figure 9.2.4-1: Measurement Model [3]

Figure 9.2.4-2: LTM Event-triggered Measurement Model [3]

Based on the measurement model(s) and related specifications, measurement involves the following aspects:

- Measurement configuration: RRC configures parameters for measurement and reporting. In NR, measurement object, reporting configuration and measurement ID are configured for L3 based RRM measurement while RS information and reporting configuration associated with RS information are used for L1 based measurement. RRC also configures the RS to be measured for radio link monitoring and beam failure detection/recovery.

- Performing measurements: e.g. beam consolidation/selection for cell quality, L1/L3 filtering, measurement gap or autonomous gap (for inter-frequency/RAT measurement)

- Evaluation of reporting criteria: e.g. periodically reporting, Event Ax, Bx, …

- Measurement reporting: in NR, different mechanisms are introduced for measurement reporting. For example, L3 measurement results are transmitted by RRC signaling while L1 measurement results can be included in MAC CE.

We think that these aspects should be considered in RAN2, except measurement RS (e.g. SSB, CSI-RS and potential UL RS for measurement) and L1 filtering.

In addition, there are some measurements related enhancement in NR, e.g. measurement relaxation for normal UE power saving/(e)RedCap UE, early measurement reporting for fast SCell activation. Energy saving and throughput should be considered in 6G. So, we can take these into account at the beginning.

Proposal 7: RAN2 to study a 6GR measurement framework for RRC_CONNECTED mode UE considering cell/beam level mobility, including measurement configuration, performing measurement, evaluation of reporting criteria and measurement reporting.

Conclusion

In this paper we presented some views on study directions for 6GR mobility aspects and have come up with the following observation and proposals:

Proposal 1: RAN2 to consider NR design as the baseline for 6GR cell reselection, including:

- cell-ranking criterion R

- priority based intra/inter-frequency reselection

- mobility speed-based scaling

Proposal 2: RAN2 to make a table (Table 2) for an analysis of mobility related features at least including pre-configurations, early DL synchronization, early UL sync via RACH, UE measured TA, early CSI acquisition, conditional execution, subsequent HO, from requirements point of view.

Proposal 3: In 6GR, for network-controlled mobility in RRC_CONNECTED state, RAN2 will study the following aspects:

- Direction 1: a unified/simplified mobility configuration

- Direction 2: a unified/modular mobility procedure considering features in P2

Proposal 4: The RLM and RLF scheme in NR is taken as the baseline for 6GR. RAN2 to further study the enhancements on RLM and RLF in case of a single cell with multiple carriers.

Proposal 5: RAN2 to study more efficient recovery mechanisms in 6G.

Observation 1: RAN2 is required to start studying the following 6G migration scenarios:

- Standalone, MRSS and inter-RAT mobility between NR-6G

Proposal 6: Inter-RAT measurement, cell reselection, handover and HOF recovery between 6GR and 5G NR are supported. Details are further studied after some progress is made in CP study and intra-6GR mobility.

Proposal 7: RAN2 to study a 6GR measurement framework for RRC_CONNECTED mode UE considering cell/beam level mobility, including measurement configuration, performing measurement, evaluation of reporting criteria and measurement reporting.

References

- RP-252912, Revised SID: Study on 6G Radio

- RAN2#131bis Chair Notes _final, RAN2 Chair (InterDigital)

- 3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2", v19.0.0