3GPP TSG-RAN WG2 Meeting #132	R2-2508323

Dallas, USA, 17 – 21 November 2025

Agenda item:	8.0.2

Source:	Nokia

Title:	Discussion on per-band per-BC capabilities

WID/SID:	NR_MIMO_Ph5-Core, NR_AIML_air-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

After RAN2#131 the LS in R2-2506565 was sent to RAN1 enquiring about the relationship between per-band and per-band combination capabilities for features that are defined by RAN1 as “per band and per-BC”. In that LS, RAN2 provided several cases for analysis, with the agreed understanding in RAN2. Now RAN1 has replied back in R2-2508008 (R1-2507981), basically confirming RAN2’s understanding of per-band per-BC capabilities. However, we think there is one ambiguity that still remains between per-band and per-BC capabilities, which we raise for further discussion in this contribution.

# 2	Discussion

Question 1: what is the relationship between per band and per BC capabilities i.e. what is the final capability in the following cases?

- Case 1: when UE indicates both per band and per BC capability

- Case 2:  when per BC capability is indicated but one of the bands in the BC doesn’t indicate per band capability

- Case 3:  when the UE reports the per band capability but does not include the per BC capability for a certain BC

…

Regarding Case 3, RAN2 understands the UE supports the feature as indicated in the per band capability without further per BC limitations. Please indicate if this understanding is not correct.

In the LS to RAN1, RAN2 asked about the below cases. We focus our discussion on Case 3.

Case 3:

- RAN2’s understanding is generally correct

- RAN1 will review by RAN1 #123 for which FGs special rules/handling may need to be defined in Rel. 19

In their reply LS, RAN1 confirmed RAN2’s understanding:

In other words, if a UE indicates that it supports a per-band per-BC capability for a set of bands {nA,nB,nC} but does not indicate the same capability for any of the BCs, CA_nA-nB, CA_nA-nC, CA_nB-nC, CA_nA-nB-nC, then that implies that the UE supports the feature without further restriction in each of CA_nA-nB, CA_nA-nC, CA_nB-nC, CA_nA-nB-nC.

However, this leads to another question: how do we handle the case where a UE supports a per-band per-BC capability in a set of bands e.g. {nA,nB,nC}, but does not support the capability in a CA combination of those bands, e.g. in at least one of  {CA_nA-nB, CA_nA-nC, CA_nB-nC, CA_nA-nB-nC}? In other words, if a UE supports the feature in single CC but does not support it in CA, how can it indicate that to the network?

Normally, not signalling a capability in a BC means that the capability is not supported for the BC, but that contradicts the understanding confirmed by RAN1 for per-band per-BC capabilities. This seems to imply that for any per-band per-BC capability, if a UE supports the capability for a set of bands, then the UE must also support the capability in any BC composed of those bands. However, it is quite unlikely that UE implementations would be able to support that requirement in all cases!

Observation 1: For a per-band per-BC capability, according to the understanding confirmed by RAN1, there is no way for a UE to indicate that it supports the capability in a set of bands (per-band) while not supporting the capability in a CA band combination composed of those bands (not per-BC). This forces the UE to always support the feature in CA.

Proposal 1: RAN2 should discuss the following alternatives for per-band per-BC capabilities:

-	Alt-1: If a UE supports such capabilities in a set of bands separately (per-band) then the UE must always support those capabilities in any CA combination composed of the respective bands (per-BC). No further enhancement is needed for this case.

-	Alt-2: A UE may support such capabilities in a set of bands separately (per-band) while not supporting the capabilities in a CA combination composed of the respective bands (not per-BC). Further discussion is needed on how to handle this case.

# 3	Conclusion

In this document we have observed and proposed the following:

Observation 1: For a per-band per-BC capability, according to the understanding confirmed by RAN1, there is no way for a UE to indicate that it supports the capability in a set of bands (per-band) while not supporting the capability in a CA band combination composed of those bands (not per-BC). This forces the UE to always support the feature in CA.

Proposal 1: RAN2 should discuss the following alternatives for per-band per-BC capabilities:

-	Alt-1: If a UE supports such capabilities in a set of bands separately (per-band) then the UE must always support those capabilities in any CA combination composed of the respective bands (per-BC). No further enhancement is needed for this case.

-	Alt-2: A UE may support such capabilities in a set of bands separately (per-band) while not supporting the capabilities in a CA combination composed of the respective bands (not per-BC). Further discussion is needed on how to handle this case.