3GPP TSG-RAN WG2 Meeting #132	R2-2508327

Dallas, Texas, USA, 17-21 November 2025

Agenda item:	10.2.2

Source:	Samsung

Title:	On 6G TN-NTN integration

Document for:	Discussion & Decision

# Introduction

In RAN2#131bis, RAN2 initiated a new study item focused on developing a new radio access technology for 6G. The NTN-related points in the 6G study item are the following [1]:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] . . . j) Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

In this contribution we discuss a first set of principles for 6G TN/NTN integration.

# Discussion

## 6G NTN discussions across WGs

In RAN1, NTN specific 6G work not start before RAN1#124 (February 2026).

11.2 NTN Placeholder only. No contributions before RAN1#124.

We believe that it is important to highlight this, as direction of NTN in 6G will likely be driven by enhancements introduced for 6G in RAN1.

Observation 1: RAN1 has yet to start their study on NTN.

In RAN2#131bis, NTN was discussed in Agenda Item 10.2 General aspects and the following was agreed:

For next meeting companies should identify essential requirements for NTN and identify which functionality should be considered in the common design with TN and what are NTN specific and whether they are essential.

## Background on Release 17 NR and IoT NTN

NR NTN and IoT NTN were introduced in 2022, in 3GPP Release 17. NR NTN is built on top of Release 15 NR, which was introduced in 2018, while IoT NTN is built on top Release 13 eMTC and NB-IoT, which was introduced in 2015.

To identify what aspects of 6G NTN that shall be part of the common design, and what is specific for NTN, it is good to compare Release 17 NR NTN and Release 15 NR. In Table 1, some of the important protocol procedures of the NR standard can be seen.

Table 1. Procedures of NR Rel-15 vs NR NTN Rel-17.

| Procedure | Sub-procedure | Difference (NR Rel-15 vs NR NTN Rel-17) | Comment |
| --- | --- | --- | --- |
| Access procedures | Sub-features: - RRC Setup - RRC Resume | None |  |
| Security | Sub-features: - Initial AS security activation - Counter check | None |  |
| Configuration | Sub-features: - RRC configuration | None |  |
| Radio Link Failure | Sub-features: - Trigger RLF - RRC re-establishment | None |  |
| UE capabilities | Sub-features: - UE capability procedures - UE capability content | None | NR NTN has new features and capabilities, but the framework remains the same |
| Access control | Sub-features: - Unified Access Control - Barring | Minor difference | Barring of non-NTN capable UEs |
| System information | Sub-features: - Content and acquisition of SIB - SIBs to acquire | Minor difference | SIB19 is introduced, which needs to be re-acquired regularly |
| Mobility | Sub-features: - Measurement configuration framework - Network-triggered handovers - Measurement reporting and CHO triggering - Cell reselection measurement triggering | Moderate difference | UE uses neighbour satellite cell ephemeris to measure. SMTC adjustment operates differently. Network-triggered release 15 enhancements operates in the same way, but with ephemeris provided. Measurements reports and CHO can optionally be triggered based on time and location. Cell re-selection measurements can optionally be triggered based on location and time. |
| Synchronization | Sub-features: - TA and frequency pre-compensation - Kmac, Koffset etc - Timer extension and adjustment | Significant difference | UE performs performs autonomous TA and frequency pre-compensation for UL synchronization. New timing offsets, and adjustment of timers based on the propagation delays. |

During the discussions in RAN2#131bis, there seems to be an understanding that NR NTN as specified in Release 17 is entirely different from NR Release 15. However, by reviewing Table 1, we find that the major key difference lies in how a UE synchronizes and the features that were standardized to support synchronization. In NR NTN, the UE uses position of the satellite and the UE to perform autonomous TA and frequency pre-compensation for the uplink, due to the distance to the satellite and the movement of the satellite. It is undoubtedly so that these aspects are very different compared to Release 15 NR, but compared to all other important features that make up the basic operation of an NR connection and an NR network, this is still a very small part. This means that the main aspect that make up the difference between TN and NTN in 6G is still likely to be synchronization.

Observation 2: Aspects of 6G design that are not common between TN and NTN will likely be synchronization, which is a result of satellite movement and long distance to the UE.

Synchronization was studied in the NTN study item in 3GPP Release 16. The key aspect to decide upon was whether a UE shall use GNSS to synchronize or not. The key difference from a RAN2 perspective, at least for the random access procedure, was well-understood [2]. Therefore, we do not think that it is particularly useful for RAN2 to discuss protocol-level issues of GNSS vs GNSS-less synchronization without RAN1 having studied the physical layer issues.

Proposal 1: RAN2 to agree that synchronization (GNSS, GNSS-less access or other) shall only be discussed in RAN2 once (and if) RAN1 has started studying it.

## 5G NTN and impact on 6G NTN

In the last meeting, it was discussed what features from 5G NTN, which includes IoT and NR NTN, that shall be introduced for 6G.

6G will address a large variety of device types from Day 1, from IoT devices up to typical high-performance smartphones. Both the IoT use case and high-performance use case is well-deployed in current terrestrial networks. This is very different from current non-terrestrial networks where IoT NTN and NR NTN are currently being tested and deployed, which means that its various use cases are and will continue to be explored and developed.

The amount of work on NTN since Release 17 in RAN2 and other WGs has been extensive, and the number of features will naturally be a large list. The list of features from IoT NTN and NR NTN from Release 17 to 20 from a RAN2 perspective can be seen in Table 2.

Table 2. NTN features introduced from a RAN2 perspective

| Rel-17 | Feature | IoT/NR NTN | Mandatory/Optional | Comments |
| --- | --- | --- | --- | --- |
|  | Synchronization: - Autonomous TA and frequency pre-compensation - Timer adjustments - (Re)-acquisition of SIB19/SIB31 | IoT and NR | M |  |
|  | Cell barring | IoT and NR | M |  |
|  | Coarse location reporting | IoT and NR | N/A |  |
|  | Multiple TAC | IoT and NR | M |  |
|  | Timing Advance Reporting | IoT and NR | O |  |
|  | Mobility enhancements: - Location/time-based CHO - Location/time-based measurement events - Location/time-based cell reselection | NR | O | Introduced for IoT NTN in Rel-18 |
|  | HARQ enhancements | NR | O | Introduced for IoT NTN in Rel-18 |
|  | Discontinuous coverage | IoT | O | Further enhanced in Rel-18 |
| Rel-18 | Uplink Coverage enhancements | NR | O |  |
|  | Network-verified UE location | NR | O |  |
|  | NTN->TN idle mode relaxation | NR | O |  |
|  | RACH-less for NTN | NR | O |  |
|  | Unchanged PCI (Satellite switch with resync) | NR | O |  |
|  | Connected mode GNSS measurement handling | IoT | O |  |
|  | Connected mode measurements | IoT | O |  |
|  | TN to NTN idle mode assistance signalling support | IoT and NR | O |  |
| Rel-19 | Downlink coverage enhancements |  | O |  |
|  | Broadcast service support | NR | O |  |
|  | Regenerative payload | IoT and NR | O |  |
|  | Store and forward | IoT | O |  |
|  | Contention-based Msg3 EDT (DSA) | IoT | O |  |
|  | Support for PWS for NB-IoT NTN | IoT | O |  |
|  | Support for IoT NTN TDD | IoT | O |  |
|  | Inter-RAT TN to NTN idle mode mobility enhancements: - LTE TN to NR NTN - LTE TN to NB-IoT NTN | IoT and NR | O | TEI19 partly |
|  | Redirection from TN to NTN: - LTE TN to NB-IoT NTN - IoT TN to IoT NTN - NR TN to NR NTN | IoT and NR | O | TEI19 |
|  | NB-IoT NTN <-> NR NTN cell selection assistance information | IoT and NR | O | TEI19 |
| Rel-20 | LTE TN to NR NTN handover | NR | O |  |
|  | Voice over NB-IoT NTN | IoT | O |  |
|  | GNSS resilience (study) | NR | O |  |

As seen in Table 2, an extensive amount of features have been introduced. Due to the large amount of features, we believe that any type of feature from Release 17 to Release 20, which are not essential for the basic operation of NTN, needs to be thoroughly motivated. It would need to be thoroughly motivated not only for technical reasons, but also to consider the IoT NTN and NR NTN use case. In the last meeting, some companies for instance considered support of discontinuous coverage and Store and Forward operation to be introduced from Day-1. These type of features are specific to constrained networks, constraints which one may expect that a 6G network shall not be under.

Observation 3: It is not feasible nor desirable to move all the features from Release 17 to Release 20 5G NTN to 6G NTN.

Proposal 2: Any NTN-specific optimization from Rel-17 to Rel-20 NTN being proposed for 6G NTN, shall be well-motivated and shall consider the overlap with NR NTN and IoT NTN use case.

## Proposed new features for 6G NTN

In the last meeting, several contributions discussed enhancements specific for 6G NTN, which do not exist in NR NTN nor IoT NTN [3]. Some of these features are, excluding GNSS-free access:

- Extended coverage

- Massive messaging

- Multi-orbit architecture

- NTN TDD

- Positioning, Navigation and Timing (PNT) services

- Integrated Communication And Sensing

While the above list only captures a small amount of potential enhancements, it should be realized that the number of potential enhancements for 6G NTN compared to 5G NTN are very large. Given that it has taken several releases of 5G NTN to standardize several features seen as very important, it should be clear that not all potential enhancements can or shall be pursued for 6G day 1. While many of the enhancements listed above can be technically motivated to enhance 6G NTN over 5G NTN, the main strength of 6G NTN over 5G NTN shall be that 6G NTN is based the solid foundation of 6G. 6G itself shall be a lean, energy-efficient, high capacity standard with support for a wide range of devices.

Observation 4: The main enhancement of 6G NTN compared to 5G NTN should be that it will be built on top of the solid foundation of 6G.

Thus similar to the careful consideration of introducing 5G NTN features to 6G NTN, we believe that 6G NTN should be even more carefully consider the features introduced specifically for 6G NTN.

Proposal 3: Any NTN-specific enhancement for 6G shall be very well-motivated and shall consider the overlap with NR NTN and IoT NTN use cases.

## 6G TN NTN integration

The main guidance that RAN has on 6G NTN, is that it shall support TN-NTN integration. In terms of TN-NTN integration in 5G NTN some features have been introduced. In Table 2, these are TN to NTN signalling assistance support, NTN-TN idle mode relaxation and TN to NTN redirection. With regards to TN-NTN connected mode mobility, there is the case where it is generally understood that a handover from TN to NTN is supported from Release 17. However, there are no requirements for a UE in a terrestrial cell to measure a non-terrestrial cell. This will be supported for LTE TN to NR NTN in Release 20.

TN-NTN integration thus implies that some sort of basic mobility between TN to NTN and NTN to TN shall be supported from Day-1. This does for instance not mean that seamless or 0-ms interruption mobility needs to be supported, but rather that a basic mechanism shall exist which enables it.

Proposal 4: A 6G RAT shall have support for basic mobility between TN and NTN.

# Conclusion

In this contribution we have the following observations:

Observation 1: RAN1 has yet to start their study on NTN.

Observation 2: Aspects of 6G design that are not common between TN and NTN will likely be synchronization, which is a result of satellite movement and long distance to the UE.

Observation 3: It is not feasible nor desirable to move all the features from Release 17 to Release 20 5G NTN to 6G NTN.

Observation 4: The main enhancement of 6G NTN compared to 5G NTN should be that it will be built on top of the solid foundation of 6G.

Based on the observations, RAN2 is requested to discuss and agree the following proposals:

Proposal 1: RAN2 to agree that synchronization (GNSS, GNSS-less access or other) shall only be discussed in RAN2 once (and if) RAN1 has started studying it.

Proposal 2: Any NTN-specific optimization from Rel-17 to Rel-20 NTN being proposed for 6G NTN, shall be well-motivated and shall consider the overlap with NR NTN and IoT NTN use case.

Proposal 3: Any NTN-specific enhancement for 6G shall be very well-motivated and shall consider the overlap with NR NTN and IoT NTN use cases.

Proposal 4: A 6G RAT shall have support for basic mobility between TN and NTN.

# References

RP-252912, “Revised SID: Study on 6G Radio”, NTT Docomo, CMCC, AT&T, Vodafone, Beijing, CN, September 2025.

3GPP TR 38.811 “Study on New Radio (NR) to support non-terrestrial networks”, V16.2.0, Release 16.

R2-2507644, 6G Radio Access Technology general aspects for NTN, Thales, Airbus, Echostar, Novamint, Fraunhofer IIS, RAN2#131bis, Prague, Czechia, October 2025.