3GPP TSG-WG2 Meeting #132	R2-2508337

Dallas, USA, November 17 – 21, 2025

Agenda Item:	8.2.2

Source:	Ericsson

Title:	Open issues for TS 38.391

Document for:	Discussion/Decision

# 1	Introduction

The list of MAC open issues is updated after RAN2#131bis [2]. In this contribution, we discuss and provide our view on following topics:

- Issue 3-7: Checking whether there is new case for “no upper layer data available” other than long writing operation which may impact to how to set MDI field.

- Issue 1-3: Paging ID length impact from new SA2 LS in S2-2507793.

- New issue 6-1: Whether there is RAN2 further impact regarding CT1 LS C1-256624 on handling of inventory and command collision.

- New issue 6-2: Confirm to apply the no data available case to more general delayed NAS cases, e.g., inventory response message.

- New issue 6-3: Whether there is AS impact to support Temp ID, e.g., including upper layer ID type in paging message. Or, assume CT4 will address this based on SA2 LS S2-2509655.

# 2	Discussion

## 2.1 Paging ID length (Issue 1-3)

RAN2 sent an LS to SA3 to ask for the whether the security parameter is mandatory in every paging message. In addition, RAN2 assumes the security parameter is a MAC information element and agreed on 1-bit flag SPPI to indicate security parameter present or not indication. Regarding paging ID length field in the list of A-IoT MAC open issues [2], while waiting for the reply from SA3, we think there are some aspects to consider in RAN2:

- First, 8-bit paging ID length field is not currently not sufficient to indicate the actual length of device identification in some cases.

  - For individual paging, permanent device ID can be used if privacy is not used. It is agreed that the permanent device ID itself can be up to 496 bits. In this case, 8-bit paging ID length is not sufficient if the unit is in bits.

  - As indicated in the SA2 LS [3], the filtering information would be 360 bits for the case of 256-bit device identification information. Detailed structure of filtering information and permanent device ID have been agreed in CT4 and SA2, as in [4].

- Second, RAN2 has assumed that upper layer SDU is byte-aligned. The current A-AoT NAS specification has octet-aligned information elements. Therefore, we assume that paging ID will be byte aligned.

It is therefore proposed to change the unit of paging ID length field to byte instead of units. In addition, 8-bit paging ID length field can indicate up to 256 bytes (2048 bits) of paging ID, which already exceeds the 1000 bits maximum TBS in Rel-19. Therefore we think 7 bits are sufficient for paging ID length field. In addition, it is still feasible to achieve byte alignment considering other bit e.g., the bit indicating whether the security parameter is present or absent. This applies for both CFRA and CBRA paging messages.

- 8-bit paging ID length field is not sufficient with permanent device ID larger than 256-bit.

Paging ID length field indicates length of paging ID in unit of bytes (instead of unit of bits).

- 8-bit paging ID length field can indicate up to 2048 bits paging ID, which is way above 1000-bit maximum TBS.

Paging ID length field is 7 bits.

## 2.2 On security parameter in paging

In last meetings, RAN2 sent and LS to SA3 [5] asking whether the security parameter is mandatory in all paging messages. From TS33.369 [8], we understand that the security parameter is mandatory, i.e., RAN2 can proceed with that assumption to speed up and finalized Rel-19 A-IoT. Regarding the optionality bit security parameter present indication (SIPI). We think no need to consider the optionality of this field. Instead, the value of this field can be set to 1 always in Rel-19. The value 0 can be reserved for possible usage in next release.

- The security parameter in A-IoT paging message is mandatory, i.e., it cannot be omitted in subsequent paging messages.

Security parameter present indication (SIPI) field is always present, and its value is set to 1 in Rel-19.

## 2.3 AIoT NAS-AS interaction (Issue 3-7)

In last meetings, RAN2 made following agreements related to interaction between NAS and AS layers:

Agreements

- RAN2 confirms, in addition to delayed response, it is valid that in some cases A-IoT NAS doesn’t provide a response at all.

- For cases other than integrity failure, AS will indicate no NAS response expected to reader.   FFS how (e.g. using 0 SDU & MDI, or new indication).

- For integrity failure, for now RAN2 assumes that there is no AS response to the reader.  Ask SA3 ccCT1 whether a similar mechanism (e.g. AS response to the reader) can be used to indicate to reader no NAS response due to integrity failure.

From RAN2’s perspective, any other cases than integrity failure, device AS sublayer will send a D2R message to the reader indicating that no NAS response is expected. However, CT1 has concluded that in addition to the integrity failure case, device NAS also indicates to the AS when a received NAS message was ignored (C1-256623 [10], TS24.369, clause 6.2), i.e., the received message is too short. All the other cases than these two cases, NAS submits a NAS PDU to AS layer and the same handling at MAC sublayer is assumed. In short, there are following cases upon reception of a NAS message according to TS24.369:

(1) Message too short: Ignore and indicate to AS layer

(2) Unknown or unforeseen message type: Ignore and reply with a STATUS message

(3) Non-semantical IE errors: Ignore and reply with a STATUS/REJECT message

(4) Unknown and unforeseen IEs, non-imperative part errors, incorrect contents: Ignore and reply with STATUS message

(5) Integrity failure: indicate to AS layer (RAN2 assumes no indication to the reader)

Thus, it is our understanding that other cases than integrity failure refer to the case of too short NAS message where NAS ignores and indicates to the AS layer.

- Other cases than integrity failure that need special handling at device AS layer refer to too short NAS message case where device NAS ignores received message and indicates to AS layer.

It remains to specify how to handle the too short message case at AS layer, i.e., whether to also indicate no NAS response expected to reader or not. In case no indication to reader is needed, this is same handling as the integrity failure. Thus, the last agreement (below) needs to be revised with respective changes to implement in the MAC CR.

Agreements

- For cases other than integrity failure, AS will indicate no NAS response expected to reader.   FFS how (e.g. using 0 SDU & MDI, or new indication).

If the too short message case is handled in the same way as the integrity failure at AS layer, RAN2 to revise the previous agreement, i.e., for integrity failure and too short message indicated by NAS, no need of indication to reader about no NAS response is expected.

Alternatively, the too short message can be handled differently from the case of integrity failure, i.e., with an indication to the reader that no NAS response is expected. In this case, there is no need to revise the previous agreement. In addition, we think a simple and natural way to realize the indication that no NAS response is expected is to make use of the MDI value 0 together with SDU length value of 0. However, this value combination was used as an indication to the reader in case of late NAS message in previous meeting. We slightly prefer this option and propose to re-purpose the combination of MDI and SDU length value, i.e., MDI = 0 and SDU length = 0 to indicate no NAS response expected to reader in case of integrity failure, whereas MDI = 1 and SDU length = 0 to indicate late NAS response to reader.

We think it is simple and natural to make use of the MDI value 0 together with SDU length value of 0 to indicate to the reader that no NAS response is expected. With this, there is no need to introduce a new indication. However, this value combination was used as an indication to the reader in case of late NAS message in previous meeting. We propose to re-purpose the combination of MDI and SDU length value as an indication for the late NAS message, i.e., MDI = 1 and SDU length = 0.

If the too short message case is handled differently from the integrity failure at AS layer, device AS sets the value of MDI and SDU length in the D2R Upper layer transfer message as below:

- Set value of MDI = 0 and SDU length = 0 to indicate no NAS response expected to reader.

- Set value of MDI = 1 and SDU length = 0 to indicate late NAS message.

Moreover, for clarity, we think RAN2 can confirm that no special handling is needed in AS layer for the cases upper layer provide the STATUS/REJECT message.

To confirm that the case upper layer provides STATUS and REJECT message to AS has no AS impact, i.e., the AS treats this case as if the upper layer command has been successfully responded.

The text proposal for this option is provided in section 5.

Update A-IoT MAC specification with text changes for handling NAS-AS interaction as proposed in section 5.

## 2.4 Handling of inventory and command collision (Issue 6-1)

RAN2 received an LS from CT1 on handling of inventory and command collision [11] with content copied as below:

1. Overall Description:

CT1 notices that in R2-2504935 (LS on parallel service request), RAN2 has agreed on the following on support of parallel service requests:

Agreements on parallel service request

- Rel-19 devices are not expected to receive parallel service request for overlapping reader scenario based on network implementation. Capture this in RAN2 stage 2 specification.

- The Rel-19 device always responds to the new service indicated by the received paging message applicable for that device. Capture this in RAN2 stage 3 specification.

Based on above RAN2 agreement, CT1 has discussed the following two scenarios:

a)	Collision between inventory procedure and inventory procedure: The AIoT NAS layer receives a new inventory from lower layer when the AIoT NAS layer is performing the inventory procedure

b)	Collision between inventory procedure and command procedure: The AIoT NAS layer receives a new inventory from lower layer when the AIoT NAS layer is performing the command procedure

CT1 concluded that there can be an attack scenario of false AIoT pagings that would abort any ongoing valid paging or command, and thereby temporarily or continuously block valid inventory procedure and command procedure execution by the AIoT devices.

Hence, CT1 kindly ask RAN2 to provide their view on whether it is possible for the AIoT device to respond to the new AIoT paging after completion of an ongoing inventory or command when collision happens.

2. Actions:

To RAN2 group.

ACTION: 	CT1 kindly ask RAN2 to provide their view on whether it is possible for the AIoT device to respond to the new AIoT paging after completion of an ongoing inventory or command when collision happens.

We note that this topic was discussed in RAN2. In particular, in RAN2#130, RAN2 agreed no parallel paging case in Rel-19. Thus, we think this issue can be revisited in the Rel-20 work.

Agreements on parallel service request

- Rel-19 devices are not expected to receive parallel service request for overlapping reader scenario based on network implementation.   Capture this in stage 2 specification.

- The Rel-19 device always responds to the new service indicated by the received paging message applicable for that device.  Capture this in stage 3 specification.

- Send LS to RAN3 to notify them of agreements 1 and 2

- Parallel service request for overlapping reader scenario can be addressed in Rel-20

RAN2 replies to CTI that

        - RAN2 sticks to the agreement that parallel service request is not expected in Rel-19.

        - RAN2 takes no further actions to the CT1 LS (C1-2506624).

## 2.5 Generic delayed NAS message (Issue 6-2)

We understand the handling for late NAS message is applicable to any cases, including the delayed inventory response due to long time to perform authentication for the CFR procedure, as mentioned in the MAC open issue summary document [2]. We think it is necessary to clarify the device behaviour upon receiving a follow-up R2D message in CFA case, as suggested by companies and rapporteur in [2].

Confirm to apply the no data available case to more general delayed NAS cases.

## 2.6 Possible AS impact to support Temp ID in paging message (Issue 6-3)

We think type of device ID, i.e., whether/what type of temporary ID or permanent device ID is used as the paging ID in case of individual paging is transparent to AS layer. It is possible to reduce overhead in case the paging ID is constant in size, e.g., temporary ID is used. However, to realize such possible optimization, AS layer needs to receive an indication from upper layer about the type of device ID. We think the benefit is not worth for such interaction between AS and NAS and respective changes in the paging message format, e.g., another indication bit to indicate if paging ID length field is present or not. Therefore, we propose not to pursuit further optimization around paging message structure in Rel-19.

- Paging ID length field is mandatory.

Not pursuit further optimization around paging message structure in Rel-19.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	8-bit paging ID length field is not sufficient with permanent device ID larger than 256-bit.

Observation 2	8-bit paging ID length field can indicate up to 2048 bits paging ID, which is way above 1000-bit maximum TBS.

Observation 3	The security parameter in A-IoT paging message is mandatory, i.e., it cannot be omitted in subsequent paging messages.

Observation 4	Other cases than integrity failure that need special handling at device AS layer refer to too short NAS message case where device NAS ignores received message and indicates to AS layer.

Observation 5	Paging ID length field is mandatory.

Proposal 1	Paging ID length field indicates length of paging ID in unit of bytes (instead of unit of bits).

Proposal 2	Paging ID length field is 7 bits.

Proposal 3	Security parameter present indication (SIPI) field is always present, and its value is set to 1 in Rel-19.

Proposal 4	If the too short message case is handled in the same way as the integrity failure at AS layer, RAN2 to revise the previous agreement, i.e., for integrity failure and too short message indicated by NAS, no need of indication to reader about no NAS response is expected.

Proposal 5	If the too short message case is handled differently from the integrity failure at AS layer, device AS sets the value of MDI and SDU length in the D2R Upper layer transfer message as below:

1.	Set value of MDI = 0 and SDU length = 0 to indicate no NAS response expected to reader.

2.	Set value of MDI = 1 and SDU length = 0 to indicate late NAS message.

Proposal 6	To confirm that the case upper layer provides STATUS and REJECT message to AS has no AS impact, i.e., the AS treats this case as if the upper layer command has been successfully responded.

Proposal 7	Update A-IoT MAC specification with text changes for handling NAS-AS interaction as proposed in section 5.

Proposal 8	RAN2 replies to CTI that

a.	RAN2 sticks to the agreement that parallel service request is not expected in Rel-19.

b.	RAN2 takes no further actions to the CT1 LS (C1-2506624).

Proposal 9	Confirm to apply the no data available case to more general delayed NAS cases.

Proposal 10	Not pursuit further optimization around paging message structure in Rel-19.

# 4	References

R2-250xxxx, “Summary of A-IoT MAC open issues (outcome of [POST131bis][017][AIoT] MAC CR )”, RAN2-13.

S2-2507793, “LS on AIoT Device Permanent ID Length”, SA2-170, Goteborg, Sweden August 25-29, 2025

S2-2509655, “Reply LS on Structure updates of AIoT Identifiers”, SA2-171, Wuhan, China, October 13-29, 2025

R2-2507920, “LS on Security parameter in A-IoT paging”, RAN2, RAN2#131bis, Prague, Czech Republic, 13th – 17th October, 2025

TS24.369 A-IoT NAS specifications, v1.1.0 (2025-10).

TS33.369 A-IoT security aspects.

C1-2506624, “LS on handling of inventory and command collision”, CT1, CT1#157, Sophia Antipolis, France, 13-17 October 2025.

# 5	Text proposals

The below text proposals are based on the Rel-19 spec V19.0.0.

## 5.4	A-IoT upper layer data procedure

### 5.4.1	General

The purpose of this procedure is for a device to transmit or receive upper layer data.

### 5.4.2	D2R message transmission

Upon initiation of the procedure corresponding to the A-IoT access procedure as specified in clause 5.3 or reception of an R2D Upper Layer Data Transfer message which contains either the Data SDU field or the Received Data Size field set to 0, the A-IoT MAC entity shall:

1>	apply the D2R Scheduling Info field, received in the A-IoT Paging message with Access Type set to CFA or in the Random ID Response message or in the R2D Upper Layer Data Transfer message containing either the Data SDU field or the Received Data Size field set to 0, whichever initiated the procedure;

1>	if upper layer has indicated integrity check failure:

2>	consider the ongoing procedure unsuccessful.

1>	else if upper layer has indicated that too short message was received and ignored:

2>	generate the D2R Upper Layer Data Transfer message, as follows:

3>	include the D2R Message Type field;

3>	set the More Data Indication field to 0;

3>	set the SDU Length field to 0;

3>	if the size of the resulting MAC PDU including no upper layer data is smaller than the resource size given by the D2R TBS field in the D2R Scheduling Info field:

4>	include the MAC Padding field;

2>	instruct the physical layer to transmit the D2R Upper Layer Data Transfer message and indicate the L1 parameters to the physical layer, as specified in clause 6.2.1.6.

1>	else if upper layer data is available to be transmitted (i.e., including the STATUS/REJECT message case):

2>	if the size of the resulting MAC PDU including the total upper layer data is smaller than or equal to the resource size given by the D2R TBS field in the D2R Scheduling Info field:

3>	generate the D2R Upper Layer Data Transfer message, as follows:

4>	include the D2R Message Type field;

4>	set the More Data Indication field to value 0;

4>	include SDU Length field and Data SDU field;

4>	if the size of the resulting MAC PDU including the total upper layer data is smaller than the resource size given by the D2R TBS field in the D2R Scheduling Info field:

5>	include the MAC Padding field;

3>	instruct the physical layer to transmit the D2R Upper Layer Data Transfer message and indicate the L1 parameters to the physical layer, as specified in clause 6.2.1.6;

2>	else (the size of the resulting MAC PDU including the total upper layer data is larger than the resource size given by the D2R TBS field in the D2R Scheduling Info field):

3>	initiate the segmentation procedure for the upper layer data SDU as specified in 5.4.4;

1>	else (i.e., upper layer data is not available to be transmitted):

2>	generate the D2R Upper Layer Data Transfer message, as follows:

3>	include the D2R Message Type field;

3>	set the More Data Indication field to 1;

3>	set the SDU Length field to 0;

3>	if the size of the resulting MAC PDU including no upper layer data is smaller than the resource size given by the D2R TBS field in the D2R Scheduling Info field:

4>	include the MAC Padding field;

2>	instruct the physical layer to transmit the D2R Upper Layer Data Transfer message and indicate the L1 parameters to the physical layer, as specified in clause 6.2.1.6.

NOTE:	It is up to reader’s implementation to avoid segmentation for the D2R Upper Layer Data Transfer message for inventory response.

3>	initiate the following D2R message transmission, as specified in clause 5.4.2.