3GPP TSG-RAN WG2 Meeting #132	R2-2508461

Dallas, USA, Nov. 17th-21st, 2025

Agenda Item:	10.3.1.2 QoS, QoE and Service-awareness

Source:	NEC

Title:	Discussion on 6GR QoS and Service-awareness

Document for:	Discussion, Decision

Introduction

At the previous RAN2 meeting, the 6G study was initiated. Some initial discussions on QoS and frame structure were held under the 6G user plane sub-agenda. The related agreement reached at the last RAN2 meeting is highlighted as follows:

Agreements on UP

1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality. Discussions should focus on standalone architecture, until told otherwise by plenary. When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2.  UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB).

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic). Understand the issues and shortcoming with the current NR QoS. Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

This contribution further discusses QoS-related issues in accordance with the RAN2 chair’s instructions.

Discussion

Focused traffic types

The following new traffic types, which differ from legacy voice and eMBB/URLLC/massive connection, are under consideration:

- Immersive communication (XR/holographic/multi-sensory)

  - Multi-modal flows (video/audio/haptic/pose/sensor) with strict end-to-end latency targets and inter-modal synchronization requirements

  - Packet-set importance differentiation within a flow (e.g., XR PDU sets, PSI) and potential sub-flow granularity

- HRLLC-oriented traffic

  - Very low user-plane latency and high reliability targets, e.g., a user plane latency of 1 ms and reliability of 1-10⁻⁵ [2]

- Mobile AI traffic (including generative AI)

  - Uplink-heavy, latency-sensitive bursts (periodic or irregular), driven by prompts and inference tasks (e.g., images/audio/sensor uploads)

  - Token-based AI streams: sequences where tokens exhibit error tolerance and variable importance; anticipated uplink burstiness and latency needs

- Sensing/ISAC-related data traffic

  - Periodic/aperiodic sensing reports and potential joint communication/sensing scheduling needs at MAC

  - Latency/reliability profiles distinct from eMBB

Since the characteristics and requirements of mobile AI traffic and sensing/ISAC-related data are not yet clear, we believe it is reasonable to proceed with our study by focusing on immersive communication and HRLLC-oriented traffic firstly.

RAN2 to proceed with 6G study by focusing on immersive communication and HRLLC-oriented traffic.

Protocol-related design targets

To meet immersive communication and HRLLC requirements in 6G, the following protocol-related design targets should be considered and further discussed, based on lessons learned from 5G:

- Protocol stack simplification and functional integration: minimize protocol processing latency and complexity by integrating or consolidating L2 functions (e.g., sequence numbering, retransmission, segmentation/reassembly) and removing unnecessary protocol layers or redundant mechanisms.

  - for HRLLC, reduces the number of processing hops and buffering points, directly lowering end-to-end latency and improving reliability. Enables deterministic and predictable data handling, essential for ultra-low latency and high reliability.

  - for Immersive Communication, reduces protocol overhead and complexity, supporting high-throughput, real-time, and multi-modal data flows. Facilitates flexible adaptation to diverse traffic patterns (e.g., video, audio, haptics) and rapid protocol evolution for new immersive use cases.

  - for mobile AI traffic and sensing/ISAC-related data traffic, provide a flexible foundation that can readily accommodate them in future, enabling efficient adaptation to emerging service requirements and rapid protocol evolution.

- Fast and deterministic retransmission control: enable immediate, event-driven retransmission (e.g., HARQ-only or tightly coordinated ARQ/HARQ), and support fine-grained retransmission (e.g., sub-TB or code block level).

  - for HRLLC, avoids unpredictable timer-based delays, ensuring retransmission latency is always within strict HRLLC budgets. Supports mission-critical applications where packet loss or delay is unacceptable.

  - for Immersive Communication, minimizes jitter and latency spikes, which are critical for seamless user experience in XR/VR/AR. Ensures timely recovery from packet loss in high rate, bursty, or interactive media streams.

- Resource-efficient reliability mechanisms: provide selective duplication (e.g., per-packet or per-PDU set) and/or network coding, dynamically activated based on service requirements or channel conditions.

  - for HRLLC, achieves high reliability without excessive resource consumption, adapting to real-time needs. Enables flexible trade-off between reliability and efficiency, crucial for industrial and safety-critical scenarios.

  - for immersive communication, supports graceful degradation and adaptive reliability for different media types (e.g., prioritize key video frames or haptic feedback). Allows differentiated protection for multi-modal streams, optimizing user-perceived quality.

- Dynamic, service-aware QoS and scheduling: support dynamic QoS parameter adaptation (latency, reliability, etc.) and service-aware scheduling/resource allocation, leveraging application-level information.

  - for HRLLC, accommodates bursty, heterogeneous, and mission-critical traffic with strict and variable requirements. Ensures that the most critical packets always receive the required treatment.

  - for immersive communication, enables real-time adaptation to user interaction, scene changes, or network conditions. Supports multi-modal synchronization and differentiated QoS for audio, video, and sensor data.

- Header and Processing Efficiency: Reduce protocol overhead and processing load by introducing mechanisms such as upper layer SDU concatenation, unified or simplified header formats, and hardware-friendly design.

  - for HRLLC, minimizes per-packet overhead, especially important for small, frequent HRLLC packets. Reduces processing time and energy consumption, supporting both low latency and high throughput.

  - for immersive communication, Increases effective payload rate for high-bandwidth, real-time media streams. Reduces device power consumption and supports longer immersive sessions.

RAN2 to consider following protocol-related design targets in 6G RAN:

- Protocol stack simplification and functional integration

- Fast and deterministic retransmission control

- Resource-efficient reliability mechanisms

- Dynamic, service-aware QoS and scheduling

- Header and processing efficiency

Application/service-awareness QoS management

In 5G, the fundamental unit for QoS management is the “QoS flow.” Each QoS flow is characterized by a set of QoS parameters (such as 5QI, ARP, GBR, MBR, etc.) and is mapped to a Data Radio Bearer (DRB) in the RAN. This enables fine-grained, per-service differentiation and resource allocation.

In Rel-18, PDU Set QoS Parameters (e.g., PSDB, PSER) are further introduced to support PDU Set-based XR traffic, for which PDU Sets belonging to the same QoS flow may have different importance (PSI).

However, the current 5G QoS management has the following limitations:

-	Lack of intra-QoS flow QoS differentiation: In 5G, PDU Sets within a QoS flow shall apply the uniform QoS treatment except for PSI-based discarding in case of congestion. Even if one PDU Set has higher importance than another, both will be forwarded with identical QoS treatment. As a result, the QoS granularity of the current QoS framework is too coarse-grained to provide intra-QoS flow QoS differentiation.

-	Lack of consideration for inter-QoS flow dependencies: According to TS 22.847 [2], immersive multi-modal applications for a UE may require the synchronization or coordination of multiple data flows to improve user experience. While a Multi-modal Service ID (MMSID) can be provided to RAN to enable multi-modality awareness, it’s not actually used for inter-QoS flow packet-level synchronization. In other words, the current 5G QoS framework does not take inter-QoS flow dependencies into consideration, thus failing to support inter-QoS flow synchronization or coordination.

-	Lack of application/service-awareness in RAN: 6G aims to support diverse service types, including immersive services, AI-driven services, and more. These services exhibit varying traffic characteristics and entail diverse QoS requirements. However, the RAN is unaware of application/service-specific attributes—particularly real-time throughput, latency, and jitter, etc.—and thus cannot leverage such information to optimize data scheduling.

-	Lack of intra-DRB differentiated treatments in RAN: Beyond PSI-based discarding, PDU Sets of varying importance within a DRB apply uniform treatment, indicating a lack of intra-DRB packet differentiation in the RAN.

The current 5G QoS management has the following limitations:

-	Lack of intra-QoS flow QoS differentiation.

-	Lack of consideration for inter-QoS flow dependencies.

-	Lack of application/service-awareness in RAN.

-	Lack of intra-DRB differentiated treatments in RAN.

As 6G are expected to support a wide range of traffic types—including HRLLC, immersive communication, AI-mobile, and sensing—there is a growing need for QoS management that is not only flexible and dynamic, but also deeply aware of application and service requirements. Traditional QoS frameworks, which primarily focus on static parameter mapping, are insufficient for the complex, multi-modal, and real-time demands of next-generation services.

6G QoS management should be flexible, dynamic and deeply aware of application and service requirements.

Given that 6GR QoS can be application/service awareness, then packets within a QoS flow may also have distinct QoS requirements, such as varying importance, or there are QoS requirements across different QoS follows, such as multi-modal synchronization requirements. As we observed in Observation 1, the current 5G QoS framework is too coarse to handle intra-QoS flow QoS differentiation and meet QoS requirements across different QoS flows. Consequently, 6G should enhance the QoS framework to support finer QoS granularity intra-QoS flow QoS differentiation and inter-QoS flow synchronization or coordination.

6G should enhance the QoS framework to support intra-QoS flow QoS differentiation and inter-QoS flow synchronization or coordination.

Moreover, given that a QoS flow is mapped to a DRB and multiple QoS flows may share the same DRB, the 6G RAN should enhance intra-DRB treatment to enable finer-grained packet handling. For example, higher importance packets may be duplicated to improve their reliability over the Uu interface; alternatively, LCP may be enhanced by mapping different importance levels to different LCHs.

6G should enhance the intra-DRB treatment to enable finer-grained packet handling.

Regarding application/service-awareness in the RAN, it is essential to collaborate with SA2/SA4 to discuss the following aspects:

-	Which characteristics of the traffic should be visible by RAN.

-	How this information is provided to RAN (e.g., via QoS profiles).

-	How RAN utilizes this information to optimize scheduling strategies.

RAN2 to collaborate with SA2/SA4 to study application/service-awareness in the RAN.

Conclusion and Proposal

This contribution discussed QoS-related issues, and we have the following observations and proposals:

Observation 1	The current 5G QoS management has the following limitations:

-	Lack of intra-QoS flow QoS differentiation.

-	Lack of consideration for inter-QoS flow dependencies.

-	Lack of application/service-awareness in RAN.

-	Lack of intra-DRB differentiated treatments in RAN.

Observation 2	6G QoS management should be flexible, dynamic and deeply aware of application and service requirements.

Proposal 1 	RAN2 to proceed with 6G study by focusing on immersive communication and HRLLC-oriented traffic.

Proposal 2 	RAN2 to consider following protocol-related design targets in 6G RAN:

-	Protocol stack simplification and functional integration

-	Fast and deterministic retransmission control

-	Resource-efficient reliability mechanisms

-	Dynamic, service-aware QoS and scheduling

-	Header and processing efficiency

Proposal 3	     6G should enhance the QoS framework to support intra-QoS flow QoS differentiation and inter-QoS flow synchronization or coordination.

Proposal 4 	6G should enhance the intra-DRB treatment to enable finer-grained packet handling.

Proposal 5 	RAN2 to collaborate with SA2/SA4 to study application/service-awareness in the RAN.

Reference

- 6G WID

- 3GPP TR 38.914 V0.2.0, "Study on 6G Scenarios and Requirements"