3GPP TSG-RAN WG2 Meeting#132	    			                                                        R2-2508491

Dallas, USA, Nov. 17th – 21th, 2025

Agenda item:	10.2.2

Source:	ETRI

Title:	Discussion on TN/NTN Integration

Document for:	Discussion and Decision

# Introduction

In the last RAN2 meeting, there was an initial discussion on 6GR NTN design. As a result of the discussion, the following agreements were made[1].

For next meeting companies should identify essential requirements for NTN and identify which functionality should be considered in the  common design with TN and what are NTN specific and whether they are essential.

In this paper, we discuss the requirements and challenges for 6G NTN based on a comparison between 5G Non-Terrestrial Networks (NTN) and 6G NTN. We then present our views on which 5G NTN solutions could serve as baselines for a harmonized 6G radio design integrating both TN and NTN.

# Discussions

## Requirements for 6G NTN

NTN differ fundamentally from traditional Terrestrial Networks (TN) because of the unique propagation environment, long transmission distances, and highly dynamic nature of satellite and aerial platforms.

Following Table 1 shows the comparison between 5G NTN and 6G NTN.

| Category | 5G NTN | 6G NTN |
| --- | --- | --- |
| Position in the system | Considered an extension of 5G networks (“NTN as an add-on”) | Designed as a native and integral part of 6G architecture |
| Main objective | To complement terrestrial coverage in uncovered areas (e.g., sea, mountains, disaster zones) | To achieve truly ubiquitous global connectivity across all environments |
| Role | Auxiliary layer supporting terrestrial 5G | Equal and unified component within the overall 6G system |
| Orbital/platform type | LEO, MEO, GEO satellites + HAPS. | vLEO (very-low-Earth orbit), LEO/MEO/GEO, UAVs, HAPS, and stratospheric platforms integrated. |
| Network Architecture | NTN segment connected to the terrestrial 5G Core (dual connectivity) | Fully converged “Unified 6G Core” combining terrestrial and non-terrestrial layers |
| Timing / positioning dependency | Strong reliance on GNSS for synchronization | GNSS-resilient operation with network-assisted timing and positioning |
| Target services | Enhanced mobile broadband extension, IoT-NTN, public-safety connectivity | Global ultra-connectivity integrating communications, sensing, and positioning |

Table 1. Comparison between 5G NTN and 6G NTN

Based on difference between 5G NTN and 6G NTN, following technical challenges should be considered in 6G NTN.

- Propagation & Path Loss

NTN links experience significantly greater free-space path loss than terrestrial systems due to the long distances between transmitters and receivers. For example, a GEO satellite is about 36,000km from Earth, which results in a round-trip propagation delay of roughly 500 to 600 milliseconds. As a result, the dominant channel effects in NTN are not multipath fading, as in terrestrial systems, but rather severe attenuation, long delay spread, and large Doppler shifts. To compensate for these impairments, NTN systems must use highly directional beamforming, high-gain antennas, and repeated transmissions to ensure reliable signal reception.

- Doppler shift & Fast moving cell

Doppler shifts and moving-cell effects are major challenges. Low Earth Orbit (LEO) satellites move at speeds exceeding 7 km per second, which can cause frequency shifts of tens of kilohertz relative to User Equipment(UE). This motion makes it difficult to maintain carrier synchronization, especially when using the standard 5G NR numerologies.

- Long Transmission Delay

Another crucial issue is long transmission latency. Because of the great long distances involved, NTN round-trip times can vary from a few milliseconds (for HAPS) to hundreds of milliseconds (for GEO). This affects procedures that rely on short feedback cycles, such as HARQ acknowledgments, RLC retransmissions, and random-access timing.

- NTN Coverage and Mobility managment

NTN systems should also manage cell movement and handover differently. Satellite beams sweep across the Earth’s surface, meaning that a single UE may need to hand over between cells every few minutes. This behavior requires location-based cell reselection and tracking mechanisms that predict beam movement using satellite ephemeris data. Handover triggers must be also redefined to handle the constantly changing coverage footprint. In addtion, since a sevice continuity is important issue between TN and NTN, it is crucial  to provide neighbor cell information efficienty.

- Synchronization and Timing Control

Time and frequency synchronization also become more complex. Unlike terrestrial networks, which can rely on GPS or network-based synchronization, NTNs require continuous adjustments to maintain alignment as satellites move.

- RAN Architecture

At the RAN level, network architecture and gateway management present additional difficulties. Terrestrial systems use stable fiber backhaul connections, but NTNs rely on feeder links between gateways and satellites, often in Ka/Q bands, and on inter-satellite links (ISLs) for space-based routing. These links must support frequent switch-overs as LEO satellites move rapidly across the sky.

- UE Design Constraints

From a hardware perspective, UE design is constrained by limited antenna gain and transmit power—especially for handheld devices. For instance, a smartphone typically has an antenna gain of about –5.5 dBi, which is insufficient to maintain a strong uplink connection over satellite distances.

- Interference management

Interference management are critical in NTN due to the vast coverage areas of satellite beams and the potential overlap with terrestrial spectrum allocations. Dynamic beam coordination and adaptive resource management are required to mitigate inter-beam interference and ensure coexistence with terrestrial systems. In addition to that, considering multi-orbit operatiion of satellites on different orbits, resource management is essential for efficient resource usage and interference mitigation.

Proposal 1. Among the above technical challenges, long transmission delay, NTN coverage and mobility management, and synchronization and timing control can be considered as key aspects for TN/NTN integration in 6GR, from the RAN2 point of view.

## Functionalities for the common design with TN

In this section, we discuss 5G NTN functionalities that can serve as references for TN/NTN common design. For each technical challenge, we first introduce the corresponding solutions defined in 5G NTN and then discuss whether these solutions can be studied as baselines for 6G NTN.

### Long transmission delay

HARQ enhancements

In 5G NR NTN, the HARQ mechanism faces major challenges due to long and variable propagation delays between UEs and satellites. Traditional HARQ—limited to 16 parallel processes and short feedback timing—is inefficient when round-trip times reach tens or hundreds of milliseconds. To address this, 3GPP Release 17 introduced HARQ-related adaptations such as extending the number of HARQ processes to avoid transmission stalling. Release 18 focuses on further enhancements, including HARQ feedback disabling for IoT-NTN, to reduce latency and power consumption. These updates enable flexible retransmission control for both broadband NR-NTN and IoT-NTN. Overall, HARQ enhancements ensure robust link performance and spectral efficiency despite the large RTT and dynamic conditions of satellite-based 5G networks.

Observation 1. HARQ enhancement can be useful for NTN.

L2 UP parameter enhancements

In 5G NR NTN, Layer 2 user-plane  (UP) parameters were enhanced to handle long propagation delays and dynamic satellite links. Traditional RLC and PDCP configurations are inefficient for large round-trip times, so Release 17 introduced extended timer ranges, larger buffer sizes, and flexible reordering windows. These adjustments prevent premature retransmissions and packet loss caused by delay variations or beam switching. Release 18 continues to enhance L2 UP performance with adaptive buffering and delay-aware control for moving beams in IoT-NTN. Overall, these enhancements ensure reliable data delivery and efficient throughput in satellite-based NTN environments.

Observation 2. L2 UP parameter enhancement can be useful for NTN.

Proposal 2. The following technical solutions can serve as baselines for 6G NTN design to address long transmission latency:

- HARQ enhancements

- L2 UP parameter enhancements

### NTN Coverage and Mobility management

Multiple Tracking Area Codes (TACs)

Tracking Area (TA) represents a group of cells used for mobility and location management. Each TA is identified by a Tracking Area Code (TAC), and a UE typically registers with a single TA when idle. When the UE moves out of this area, it performs a Tracking Area Update (TAU) to inform the core network of its new location. In NTN, the radio footprint of a cell (or beam) moves dynamically across the Earth’s surface. A UE connected to an NTN may experience frequent transitions from one beam or satellite to another, even while physically stationary, which results in inefficient TAU operations. Therefore, a multiple TAC mechanism was introduced, enabling a UE to register with several tracking areas simultaneously. This minimizes TAU frequency, reduces signaling overhead, and maintains stable paging and service continuity as satellites move.

Observation 3. Multiple TAC can be useful for NTN mobility.

RACH-less HO

In 5G NR NTN, RACH-less handover allows a UE to switch between beams or satellites without performing a full random-access (RACH) procedure. Traditional RACH-based handovers cause long interruption times due to large satellite propagation delays. To minimize this, the network pre-configures uplink timing advance and resources for the target cell, enabling the UE to transmit immediately after handover. This greatly reduces latency and signaling overhead, ensuring seamless mobility in fast-moving LEO or MEO scenarios. 3GPP recognized RACH-less handover as part of NR-NTN mobility enhancements in Releases 18. Overall, it improves handover speed, reliability, and service continuity in dynamic NTN environments.

Observation 4. The RACH-less HO can be useful for NTN mobility.

Time and Location based HO

In 5G NR NTN, time- and location-based handover (HO) enables seamless mobility despite moving satellites and dynamic beam footprints. Traditional signal-strength-based HO is often too slow for NTN because beams shift even when the UE is stationary. Instead, the network predicts when a satellite or beam will lose visibility and triggers HO based on time (predicted visibility window) or location (UE’s position relative to the beam boundary). Using satellite ephemeris and Global Navigation Satellite System (GNSS) data, the UE or network can anticipate coverage loss and prepare the target connection early. This proactive approach minimizes radio link failures and reduces unnecessary handovers. 3GPP has recognized this in studies and ongoing NTN mobility work, integrating time and location triggers into the HO framework. Overall, it ensures stable connectivity and efficient mobility management in multi-beam, multi-satellite NTN environments.

Observation 5. The Time- and Location-based HO can be useful for NTN mobility.

Assistance information for NTN mobility

AS the assistance inforation for NTN-NTN mobility, SIB 19 was introduced to broadcast NTN-specific parameters such as ephemeris data, reference locatio, t-service and neighbor NTN cell list. For TN and NTN mobility, 5G NR provides assistance information such as NTN cell information within TN cells and TN coverage information within NTN cells via SIB25. Based on this information, UEs can (re)select appropriate cells for connection setup. As this assistance information is crucial for supporting TN-NTN mobility, RAN2 should study mechanisms for providing assistance information in 6G NTN.

Observation 6. The Assistance information can be useful for TN-NTN and NTN-NTN mobility.

Satellite switch with re-sync

In 5G NR NTN, satellite switch with re-synchronization enables a UE to transition between satellites or beams with minimal interruption. Instead of performing a full handover, the UE re-synchronizes its timing and frequency to the new satellite while maintaining the same cell or PCI. The network provides advance information—such as timing advance and frequency offset—so the UE can pre-align before the switch. This reduces latency and avoids the long delays caused by random access procedures. 3GPP is standardizing this mechanism as part of the NR-NTN mobility enhancement work. Overall, it ensures continuous connectivity and seamless service as satellites move or coverage areas shift.

Observation 7. The Satellite switch with re-sync can be useful for HO overhead reduction in transparent architecture.

SMTC enhancement

In 5G NR NTN, the SMTC mechanism was enhanced to manage long propagation delays and moving beam footprints caused by satellite motion. Unlike terrestrial systems with fixed timing, NTN requires flexible measurement windows to track shifting SSB arrival times. Release 17 extended SMTC parameters with larger offsets, multiple configurations, and visibility-based activation. Release 18 introduced dynamic reconfiguration, enabling the network to adjust SMTC timing as beam geometry changes and to coordinate it with measurement gaps. Release 19 further refines SMTC by extending the number of SMTC configurations and introducing location-based SMTC. Overall, these enhancements allow accurate synchronization, lower power consumption, and seamless mobility for NTN users.

Observation 8. The SMTC enhancement can be useful for measurement in NTN.

Proposal 3. The following technical solutions can serve as baselines for 6G NTN design for NTN coverage and mobility management:

- Multiple TACs

- RACH-less HO

- Time and Location based HO

- Assistance information for TN-NTN mobility

- Satellite switch with re-sync

- SMTC enhancement

### Synchronization and Timing Control

Uplink timing adjustment

In 5G NR NTN, uplink Timing Advance (TA) compensates for the long propagation delays between the UE and the satellite. Because NTN links—particularly those involving GEO and MEO orbits—exhibit much larger round-trip times (RTTs) than terrestrial networks, Release 17 introduced extended TA ranges and an uplink timing adjustment mechanism incorporating a Reference Point (RP), common TA, and UE-specific TA. The UE first pre-compensates for delay using satellite position information and then fine-tunes the timing based on network feedback. These enhancements maintain uplink synchronization despite large and varying delays, ensuring stable and efficient NTN operation.

Observation 9. The uplink timing adjustment mechanism can be useful for synchronization and timing control.

Proposal 4. The following technical solution can serve as a baseline for 6G NTN design concerning synchronization and timing control:

- Uplink timing adjustment

### Considerations on Rel-20 5G-Advanced (5GA) NTN

In 5G NTN, GNSS is often relied upon for UE timing, frequency, position, and Doppler compensation. However, GNSS signals may be temporarily unavailable, degraded, or inaccurate due to interference, jamming, spoofing, or operation in indoor or under canopy environments. Consequently, Release 20 includes a work item to assess and enhance the NTN’s resilience to GNSS failure or degraded GNSS availability—specifically, how NTN access and connected-mode procedures can operate without fully relying on GNSS. Therefore, 6G NTN design should consider take into account the ongoing work on GNSS resilience in 5G-Advanced NTN.

Proposal 5. The TN/NTN integration should take into account the GNSS resilience aspects studied in Rel-20 5GA NTN.

# Conclusions

Based on the discussion in the contribution, we have the following observations and proposals:

Observation 1. HARQ enhancement can be useful for NTN.

Observation 2. L2 UP parameter enhancement can be useful for NTN.

Observation 3. Multiple TAC can be useful for NTN mobility.

Observation 4. The RACH-less HO can be useful for NTN mobility.

Observation 5. The Time- and Location-based HO can be useful for NTN mobility.

Observation 6. The Assistance information can be useful for TN-NTN and NTN-NTN mobility.

Observation 7. The Satellite switch with re-sync can be useful for HO overhead reduction in transparent architecture.

Observation 8. The SMTC enhancement can be useful for measurement in NTN.

Observation 9. The uplink timing adjustment mechanism can be useful for synchronization and timing control.

Proposal 1. Among the above technical challenges, long transmission delay, NTN coverage and mobility management, and synchronization and timing control can be considered as key aspects for TN/NTN integration in 6GR, from the RAN2 point of view.

Proposal 2. The following technical solutions can serve as baselines for 6G NTN design to address long transmission latency:

- HARQ enhancements

- L2 UP parameter enhancements

Proposal 3. The following technical solutions can serve as baselines for 6G NTN design for NTN coverage and mobility management:

- Multiple TACs

- RACH-less HO

- Time and Location based HO

- Assistance information for TN-NTN mobility

- Satellite switch with re-sync

- SMTC enhancement

Proposal 4. The following technical solution can serve as a baseline for 6G NTN design concerning synchronization and timing control:

- Uplink timing adjustment

Proposal 5. The TN/NTN integration should take into account the GNSS resilience aspects studied in Rel-20 5GA NTN.

# References

- Chair’s Note, “3GPP TSG-RAN WG2 Meeting #131bis”, Oct. 2025