3GPP TSG-RAN WG2 Meeting #132	R2-2508541

Dallas, U.S, Nov 17-21, 2025

Agenda item:	10.2.2

Source:	Sony

Title:	Discussion on 6GR TN/NTN integration

Document for:	Discussion

# 1 Introduction

It was discussed in RAN2#131bis meeting [1] that RAN2 should identify essential requirements for NTN and identify which functionality should be considered in the common design with TN and what are NTN specific and whether they are essential.

In this paper, we mainly discussed 6G NTN on NTN-TN integration, NTN mobility and RACH, identify the baseline and what needs to study further.

# 2 Discussion

2.1 NTN-TN tight integration: Fast switching between TN and NTN

5G NTN does not support seamless handover from 5G NTN to TN and vice versa. Sharing of UE capabilities between TN and NTN is a technical problem. However, lack of coordination between TN and NTN MNOs is the cause. The main use case for 6G NTN, from our point of view, is to provide global coverage and NTN complementing TN coverage. To achieve this, a tight coordination is required between TN and NTN MNOs. NTN coverage can cover TN coverage holes as shown in Figure 1. We therefore propose that a tighter integration of NTN-TN should be studied.

Figure 1: NTN coverage covering TN coverage holes.

Proposal 1     RAN2 to study NTN-TN tighter integration.

2.2 NTN mobility

In NR NTN, additional trigger conditions e.g., a time-based trigger condition and a location-based trigger condition upon which UE may execute CHO to a candidate cell were introduced in Rel-17 NR NTN. And further in Rel-18 NR NTN, the restriction that time-based and location-based trigger should be jointly configured with RRM measurement in Rel-17 NR NTN was lifted, as a result, the time-based or location-based trigger conditions may be configured independently from the RRM measurement-based trigger conditions for CHO in NTN. And we believe time-based and location-based CHO trigger are useful features to support the NTN handover and it should be the baseline for 6G NTN handover.

Proposal 2	Time-based and location-based conditional handover is the baseline for 6G NTN.

On the other hand, we foresee some issues to be addressed in 6G NTN. The first is the signalling storm during handover. All UEs in an NTN cell are expected to experience a change of cell due to new satellite taking over every few seconds in a LEO constellation. Normally networks are not dimensioned to handle handover of all connected UEs in a cell within a short period of time. As a result, a significant chunk of HO or CHO signalling takes place during the Handover Preparation phase. Handover preparation signalling takes place between network nodes without any UE involvement. Most handovers are predictable in NTN because these occur due to the movement of cells and at regular intervals. Only a very small number of handovers are expected to be due to UE mobility. A significant chunk of HO signalling takes place during HO preparation phase and HOs are mainly due to cell movement. The oncoming cells are expected to support all UEs in a geographical area i.e. resource reservation is mainly to cater the traffic emanating from a geographical area and different from the scenario where resources are reserved due to unpredictable movement of UEs in Terrestrial Networks.

Proposal 3	6G NTN needs to address the signalling storm issue during handover due to the movement of the NTN cells.

Furthermore, the UL-based mobility as described in a companion contribution [2] can be explored to further enhance the NTN handover performance, in order to e.g., improve the energy saving gain from UE side (avoid performing RRM measurements), improve the handover reliability (due to long RTT, the real radio environment of the UE may have changed after UE sends the measurement report).

Proposal 4	RAN2 to study UL-based mobility for 6G NTN.

2.3 NTN RACH

Support of different device types requires the base station to know the capabilities of these devices earlier in the process of initial access procedure. This has led to a common solution of RACH resource partitioning and broadcasting these partitioning of resources. RACH partitioning was introduced for features like Redcap, slicing, SBFD, SDT etc. Resource partitioning is not a good practice from resource utilisation and feature implementation point of view and should be avoided in our view.

In order to avoid any early recognition of UE capabilities during initial access, common initial access procedure should be designed, and all device types should be able to perform initial access to any base station. UE capability for such diverse device types should be signalled later. UE may be able to indicate its capability either in MSG3 or MSG5 or later. This principle should be applicable to NTN UEs as well.

In NR NTN, in order to accommodate the propagation delay in NTN, some timing relationships are enhanced by a common timing advance and two offsets Koffset and kmac, and kmac is used in RACH to determine the start time of RAR window/Msg B window after a Msg1/Msg A transmission. And for the serving cell, the network will broadcast valid ephemeris information and common TA parameters and UE will use them to perform synchronisation before and during RACH procedure. And we believe these features should be the baseline for 6G NTN.

Proposal 5	Common RACH design in 6G NR should be applicable for 6G NTN UEs. Common TA and kmac should be the baseline for pre-compensation before the RACH and during RACH procedure respectively.

# 3 Conclusion

In this contribution, we have the following proposals:

Proposal 1    RAN2 to study NTN-TN tighter integration.

Proposal 2	Time-based and location-based conditional handover is the baseline for 6G NTN.

Proposal 3	6G NTN needs to address the signalling storm issue during handover due to the movement of the NTN cells.

Proposal 4	RAN2 to study UL-based mobility for 6G NTN.

Proposal 5	Common RACH design in 6G NR should be applicable for 6G NTN UEs. Common TA and kmac should be the baseline for pre-compensation before the RACH and during RACH procedure respectively.

# 4 References

- Minutes RAN2#131bis, Prague.

- R2-2508548. Discussion on 6GR Rel-20 mobility aspects, Sony.