3GPP TSG-RAN WG2 Meeting #132	R2-2508545

Dallas, U.S, Nov 17-21, 2025

Agenda item:	10.3.2.1 (Control plane)

Source:	Sony

Title:	Discussion on Inactive state for IOT devices

Document for:	Discussion and Decision

# 1 Introduction

At RANP#108, the new SID on 6G radio study RP-251881 was agreed [1], with objective on radio interface protocol architecture and procedures for 6GR as follows.

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

At RAN2#131bis, the following agreements were made:

Continue study - Idle mode – - Connected mode – - Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW. Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements. We can consider the pros/cons of different modelling, after having some discussions on the functionality.

In this paper, we discuss inactive state from IOT devices’ point of view.

# 2 Discussion

## 2.1 IoT Inactive state

All NR RRC states: RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED should be supported as baseline for IoT devices. Inactive state was not implemented in NR but with features like SDT (Small Data Transmission), we think inactive state can offer better results for UE and network energy efficiency.

RAN2#131bis discussed inactive state and primarily two solutions were discussed. One based on NR inactive state and another solution where, inactive state is a sub-state of connected state.

NR inactive state allows UE based mobility by using idle mode procedures like cell selection and reselection within an RNA. UE context is stored in the gNB and UE is in connected mode from CN point of view. New serving gNB can fetch UE context from the previous gNB. A GTP-U tunnel exists between RAN and CN and the security context is available in the RAN. RNA update procedure is required once the UE moves out of RNA.

For inactive to be a sub-state of connected mode, our understanding is that the UE remains in connected mode but transition into a power saving state within connected mode. In our view, UE context in RAN includes more information than a UE context in NR inactive state.

Based on this understanding of two solutions, we analyse inactive state for IOT devices. The main characteristics of 6G-IOT devices will be that these devices may transmit smaller amount of data as compared to a smartphone but at the same time, there will be large number of such IOT devices as compared to smartphones in a network. IOT devices may support SDT like feature for small amount of data.

We think one issue related to IOT devices is the storage capacity of UE contexts and associated resource allocation in the RAN during inactive state. In our view, NR inactive state is a lightweight approach, as compared to when inactive is a sub-state of connected mode, for storing UE context and reserving resources for a UE in RAN. NR inactive state requires at least an active GTP-U tunnel between RAN and core network and associated PDCP state machine to be active for receiving data. In addition, security information should be stored. This may impact the dimensioning of the gNB and may have a finite limit on the number of IOT-UE contexts it can store. In our view, this number will be less when inactive is a sub-state of connected mode.

Observation 1	gNB may have a capacity to store a finite number of UE contexts due to storage limitations and the amount of resources required for maintaining resources for inactive UEs like e.g. a GTP-U tunnel and PDCP state machine for individual IOT device. This storage of number of UE contexts may be less if, inactive is a sub-state of connected mode, because of maintaining more information and resources as part of the UE context.

The other issue is related to the length of ID addressing the UE in inactive state. For NR inactive state, I-RNTI and full I-RNTI is used. This ID should uniquely identify a UE within RAN and if inactive is a sub-state of connected mode, then this could be C-RNTI. But if C-RNTI is used then a further study is required if UE performs cell reselection and its current C-RNTI is not valid.

Observation 2	A unique identity in inactive state is required. This could be either I-RNTI (for NR inactive state) or C-RNTI (if inactive is a sub-state of connected mode). How to ensure C-RNTI uniqueness during mobility should be studied further.

We should wait for SA3 on AS security but one solution direction for IOT devices may be that these devices don’t mandatorily support AS security. The other issue is the UE power consumption which is very important for IOT devices. Combined with no AS security and UE power consumption, these devices should be able to transmit and receive data with very low overheads of signalling. It is very important that subsequent sessions should be lean.

Observation 3	Subsequent data transmission sessions in inactive state may have no AS security.

We think that the presence of IOT devices in a network should be taken into account while designing the inactive state and above observations should be captured as design principles.

Proposal	RAN2 to discuss and agree above observations and capture them as design principles for inactive state.

# 3 Conclusion

In this contribution, We propose RAN2 to discuss following observations and agree the proposal:

Observation 1	gNB may have a capacity to store a finite number of UE contexts due to storage limitations and the amount of resources required for maintaining resources for inactive UEs like e.g. a GTP-U tunnel and PDCP state machine for individual IOT device. This storage of number of UE contexts may be less if, inactive is a sub-state of connected mode, because of maintaining more information and resources as part of the UE context.

Observation 2	A unique identity in inactive state is required. This could be either I-RNTI (for NR inactive state) or C-RNTI (if inactive is a sub-state of connected mode). How to ensure C-RNTI uniqueness during mobility should be studied further.

Observation 3	Subsequent data transmission sessions in inactive state may have no AS security.

Proposal	RAN2 to discuss and agree above observations and capture them as design principles for inactive state.

# 4 References

- RP-251881. New SID: Study on 6G Radio.

- Minutes RAN2#131bis, Prague.