3GPP TSG-RAN WG2 Meeting #132	R2-2508657

Dallas, USA, Nov. 17th – 21st 2025

Agenda item:		10.4

Source:		Lenovo

Title:		Discussion on 6G Mobility and measurement

Document for:		Discussion and Decision

- Introduction

This paper focusses on Mobility and measurement for RRC_CONNECTED states.

- Discussion

Mobility evolution in 5G

Mobility is important for the user experience, applications but 5G has extended the sophistication, perhaps much beyond what will ever be deployed. There is too many mobility features standardized in 5G starting since Rel-15 L3 command-based handover. Only a few are deployed on the field and activated in the commercial UE. Roughly one-third of the entire 5G RRC specification is devoted to mobility — covering measurements, handovers, dual connectivity, failures and recovery — making mobility the single largest functional area of the protocol. The following figure lists the mobility features in 5G.

Fig. 1: Mobility evolution in 5G

- Rel-15 L3 command-based HO:

  - When UE receives RRCReconfiguration message including reconfigurationWithSync IE from source cell, UE applies RRCreconfiguration message including reconfigurationWithSync IE immediately[1].

  - Complete candidate configuration and delta configuration based on source configuration are supported.

  - Both early UL/DL sync and subsequent HO are NOT supported.

  - RACHless is supported after Rel-15.

- Rel-16 CHO:

  - When UE receives RRCReconfiguration message including CHO candidate configuration and L3 execution condition from source cell, UE stores it. CHO candidate configuration is included in ConditionalReconfiguration of RRCReconfiguration (i.e., RRCReconfiguration  ConditionalReconfiguration  condReconfigToAddModList  CondReconfigToAddMod  CondReconfigId +CHO candidate configuration+execution condition). UE applies CHO candidate configuration when CHO execution is met.

  - Complete candidate configuration and delta configuration based on source configuration are supported.

  - Both early UL/DL sync and subsequent HO are NOT supported.

  - RACHless is supported.

- Rel-16 DAPS:

  - A handover procedure that maintains the source gNB connection after reception of RRC message for handover and until releasing the source cell after successful random access to the target gNB. That means UE will maintain both source link and target link during cell switching which aims to achieve almost ‘0’ DL interruption time.

- Rel -18/Rel-19 MAC CE based Intra-CU/Inter-CU LTM:

  - When UE receives LTM candidate configuration included in LTM-Config IE of RRCReconfiguration, UE stores it. UE applies LTM candidate configuration once LTM cell switch command MAC CE is received. LTM candidate configuration is included in LTM-Config IE of RRCReconfiguration (i.e., RRCReconfiguration  LTM-Config IE  ltm-CandidateToAddModList  LTM-Candidate  ltm-CandidateId + LTM candidate configuration).

  - Complete reconfiguration message and delta configuration based on reference configuration are supported.

  - Both early UL/DL sync and subsequent HO are supported.

  - RACHless is supported.

- Rel-19 Intra-CU CLTM:

  - When UE receives CLTM candidate configuration included in LTM-Config IE of RRCReconfiguration, UE stores it. UE applies CLTM candidate configuration once LTM (L1 or L3) execution condition is received. LTM candidate configuration is included in LTM-Config IE of RRCReconfiguration (i.e., RRCReconfiguration  LTM-Config IE  ltm-CandidateToAddModList  LTM-Candidate  ltm-CandidateId + LTM candidate configuration).

  - Complete reconfiguration message and delta configuration based on reference configuration are supported.

  - Both early UL/DL sync and subsequent HO are supported.

  - RACHless is supported.

Observation 1: The following mobility features are supported for standalone 5G.

- L3 command-based HO without supporting early UL/DL sync, subsequent HO;

- CHO without supporting early UL/DL sync and subsequent HO;

- DAPS;

- MAC CE based Intra-CU/Inter-CU LTM with supporting early UL/DL sync, subsequent HO, reference configuration and RACHless;

- Intra-CU CLTM with supporting early UL/DL sync, subsequent HO, reference configuration and RACHless.

Observation 2: Roughly one-third of the entire 5G RRC specification is devoted to mobility — covering measurements, handovers, dual connectivity, failures and recovery — making mobility the single largest functional area of the protocol.

Mobility Principle for 6G

The basic aim of the different mobility features and enhancements are for reliability and short interruption time. But since it is complex and costly to implement each of these, and also since deployment of each is definitely an overkill and will not take place, it is better to shortlist mobility features that offer best value for money. To understand which mobility features and enhancement are ‘simple and sufficient’, we should look at the following questions:

Low latency with robustness:

- Conditional HO&network centric mobility: statistics could be needed to show if the probability of ‘too early HO’, ‘too late HO’ or ‘HO to wrong cell’ can be reduced

- Actual UE measurement&AI/ML based measurement prediction: According to the simulation result of 5G, the RRM prediction and L3 event prediction are helpful for improving HO performance.

- How much more latency is added due to L3 Handover when compared with L2 based LTM mobility if there are same assumption e.g., supporting early UL/DL sync.

Short interruption time:

- Is ‘0’ ms interruption time really needed? or applications can withstand limited interruption using their own buffering? According to the progress of RAN plenary, the minimum requirement for mobility interruption time is 0 ms at least for mobility between TRxPs of a single base station. We need to further check the progress of RAN plenary if ‘0’ ms interruption time is really needed for normal case e.g., Inter-BS HO.

Others:

- Security risks of using unprotected MAC CE for mobility command – DoS attacks?

- TN/NTN mobility: both L3 command-based HO and CHO are supported by NTN system. There is specific execution condition designed for NTN.

It is likely that there are more than one mobility solutions needed allowing some flexibility to the operator and also requiring different UE capabilities (device type, mobility levels etc.). Even more than one may be necessary to be configured to the UE to ensure mobility reliability and fulfill QoS. If so, how to unify their measurement configuration, align their execution decision (CU Vs DU) and possibly have a common execution, should also be addressed at an early stage.

Proposal 1: Shortlist mobility solutions considering 5G Mobility features e.g., network centric mobility and conditional HO, can be taken as baseline.

Proposal 2: RAN2 to study how to find common framework (e.g., common procedure for air interface and Xn-like interface, common configuration) among the candidate solutions.

Based on the analysis of Mobility evolution in 5G, LTM supports early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition and reference configuration. Regarding L3 command-based HO, it does not support early DL/UL synchronization, fast processing, pre-configurations, conditional handover and reference configuration. CHO does not support early DL/UL synchronization, early CSI acquisition and reference configuration. Therefore, LTM framework can be taken as baseline.

Proposal 3: RRC framework for LTM supporting early DL/UL synchronization, fast processing, pre-configurations, conditional handover, early CSI acquisition and reference configuration can be taken as baseline.

According to the KPI, mobility interruption time shall be sufficiently minimized to support a seamless application performance. The minimum requirement for mobility interruption time is 0 ms at least for mobility between TRxPs of a single base station. In 5G, RACHless HO using early TA is supported for reducing interruption time though RACHless cannot reach ‘0’ interruption time. Regarding DAPS-like mobility mechanism, it aims to get the target of ‘0’ interruption time. Based on the current requirement of the service in RAN plenary, ‘0’ interruption time in KPI is restricted in mobility between TRxPs of a single base station. Regarding normal HO e.g., Inter-BS HO, there is no requirement of 0 ms. Therefore, RACH-less HO using early TA can be supported for reducing interruption time at an early stage since. We don’t see the strong motivation to have DAPS-like mobility for ‘0’ mobility interruption time in DAY ONE.

Proposal 4: RACH-less HO using early TA can be supported for reducing interruption time at an early stage.

L1/L3 Measurement

Fig. 2: measurement model

The Fig.2 shows high-level measurement model for RRC_CONNECTED UE. The UE in RRC_CONNECTED measures multiple beams of a serving/neighbor cell and the measurements results are averaged to derive the cell quality. UE performs filtering at two different levels i.e., the physical layer for beam quality and RRC level for cell quality from multiple beams. Regarding RRM, the framework with MeasObject (MO), ReportConfig and MeasId is designed for RRM. MeasId is used to identify a measurement configuration, i.e., linking of a measurement object and a reporting configuration.

Regarding L1 measurement report for LTM, the IE LTM-CSI-ResourceConfig defines a group of one or more CSI resources for one or more LTM candidate configurations. The IE LTM-CSI-ReportConfig is used to configure L1 measurement report for LTM. Once the report type is met, UE is triggered to report L1 measurement via MAC CE.

Regarding beam management for serving cell, the CSI resource for beam management provided by CSI-ResourceConfig is included in csi-MeasConfig of ServingCellConfig. The IE CSI-ResourceConfig defines a group of one or more NZP-CSI-RS-ResourceSet, CSI-IM-ResourceSet and/or CSI-SSB-ResourceSet. The IE CSI-ReportConfig for beam management is used to configure a periodic, semi-persistent, aperiodic report sent on PUCCH/PUSCH on the cell.

In 5G, the different signaling structures for L1/L3 measurement and beam management, resulting in heavy signaling overhead. For example, there are three separate IEs specified for RRM, L1 measurement for LTM and beam management, respectively. And the structure with resource configuration and report configuration is similar. Therefore, we suggest finding common framework at least for L3 measurement for mobility, L1 measurement for mobility, and beam management. If the common framework is decided, the LS to RAN1 is needed.

Proposal 5: RAN2 to study how to find common framework at least for L3 measurement for mobility, L1 measurement for mobility, and beam management. LS to RAN1 is needed once framework is decided.

- Conclusion

In summary, we have following observations and proposals:

Observation 1: The following mobility features are supported for standalone 5G.

- L3 command-based HO without supporting early UL/DL sync, subsequent HO;

- CHO without supporting early UL/DL sync and subsequent HO;

- DAPS;

- MAC CE based Intra-CU/Inter-CU LTM with supporting early UL/DL sync, subsequent HO, reference configuration and RACHless;

- Intra-CU CLTM with supporting early UL/DL sync, subsequent HO, reference configuration and RACHless.

Observation 2: Roughly one-third of the entire 5G RRC specification is devoted to mobility — covering measurements, handovers, dual connectivity, failures and recovery — making mobility the single largest functional area of the protocol.

Proposal 1: Shortlist mobility solutions considering 5G Mobility features e.g., network centric mobility and conditional HO, can be taken as baseline.

Proposal 2: RAN2 to study how to find common framework (e.g., common procedure for air interface and Xn-like interface, common configuration) among the candidate solutions.

Proposal 3: RRC framework for LTM supporting early DL/UL synchronization, fast processing, pre-configurations, conditional handover, early CSI acquisition and reference configuration can be taken as baseline.

Proposal 4: RACH-less HO using early TA can be supported for reducing interruption time at an early stage.

Proposal 5: RAN2 to study how to find common framework at least for L3 measurement for mobility, L1 measurement for mobility, and beam management. LS to RAN1 is needed once framework is decided.

- References

3GPP TS 38.331 v19.0.0