3GPP TSG-RAN WG2 Meeting #132					                R2-2508682

Dallas, USA, Nov. 17th - 21st

Source:	AUMOVIO

Title:                       Discussion on Topology 2 for AIoT

Agenda Item:	9.2.2

Document for:	Discussion and Decision

# Introduction

According to the approved Work Item Description for Release 20 Ambient IoT Phase 2 [1], Objective 2 specifies support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions:

- According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769 (No specification support for CW control)

- According to D2T2-C for active Device(s) (Start after March 2026)

- Traffic types DO-DTT and DT, and DO-A (only for active Device(s))

- Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command)

- FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum

The following objectives are defined for RAN2 [1]:

- The objective includes specification of necessary Uu procedures to enable the A-IoT operations.

- RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios.

- A-IoT radio resource of the UE reader is only valid within the serving cell.

- Resource allocation to the UE Reader shall be via Uu RRC signaling.

This contribution discusses enhancements to RRC procedures for Radio Link Failure (RLF) recovery and handover for intermediate UE Readers operating in D2T2 configurations, addressing the above requirements and constraints.

# Discussion

  - Radio Link Failure Handling and Handover Procedure

In D2T2 deployments, intermediate UE Readers play a key role in enabling communication between the gNB and A-IoT devices. These Readers must maintain RRC CONNECTED state to support continuous Ambient IoT operations. However, when RLF occurs due to mobility, interference, or coverage limitations, service interruption and resource reconfiguration are required.

During ongoing A-IoT operations, the UE Reader accumulates operational context including device paging status, inventory progress, trigger message delivery states, and resource usage patterns. When RLF occurs, this context is lost at the network side, resulting in inefficient recovery and redundant signaling to devices that have already responded. To improve recovery efficiency, we propose enhanced RRC re-establishment procedures with two complementary mechanisms:

Context retention at gNB: The UE Reader includes a Reader-ID and a dedicated cause value (e.g., ambientIoTRLF) during RRC re-establishment. The gNB may optionally retain the A-IoT context for a limited duration using a configurable timer. If the UE reconnects within the same cell during this period, the gNB can restore the previous configuration.

Context transfer from UE Reader: Upon RLF detection, the UE Reader locally stores relevant A-IoT procedure context and operational statistics. After successful RRC re-establishment or handover completion, the UE Reader may transfer selected context information to the target gNB. This enables the gNB to resume A-IoT operations based on actual state rather than applying default configurations.

Examples of context information that may be transferred include:

- Percentage of devices that successfully responded to Msg1 (paging)

- Percentage of Msg3 (trigger response) completions

- Number of pending device interactions

- Resource usage patterns prior to RLF

This approach supports optimized resource allocation and avoids redundant operations such as re-paging devices that have already responded. The specific content and format of the transferred information require further study to balance signaling overhead and utility.

Proposal 1: RAN2 should specify enhanced RRC re-establishment procedures for UE Readers, including A-IoT-specific context retention mechanisms at the gNB to support seamless recovery when reconnecting to the same cell after RLF.

Proposal 2: RAN2 should specify mechanisms for UE Readers to transfer A-IoT operational context and statistics to the target gNB during RRC re-establishment or handover completion, enabling efficient resource allocation and procedure resumption.

In current deployments, UE Readers operating in Topology 2 configurations rely on trial-and-error connection attempts to determine cell compatibility, as system information does not provide any indication of A-IoT support. This leads to unnecessary signaling, failed A-IoT operations, and increased latency.Introducing a broadcast indication of Topology 2 support in system information would enable UE Readers to identify suitable target cells prior to RRC connection establishment. Additionally, broadcasting the availability of A-IoT resource configuration (e.g., via a flag) would allow UE Readers to avoid connecting to cells that support Topology 2 but are temporarily unable to allocate A-IoT resources due to load or policy constraints. This supports more efficient cell selection and reduces signaling overhead.These enhancements complement predictive mobility handling and fast context transfer mechanisms proposed for handover, enabling seamless and efficient A-IoT operation across cells.

Proposal 3: RAN2 should consider specifying mechanisms in system information to assist UE Readers during cell selection and reselection, including:

- Indication of A-IoT Topology 2 support

- Indication of availability of A-IoT resource configuration

  - A-IoT Resource Management and Information Exchange

To support efficient A-IoT operations in Topology 2 deployments, it is recognized that coordination between gNB and UE Reader is essential for resource configuration and transmission control. Based on RAN2#131bis agreements, the following aspects are considered [2]:

Agreements

- 1.  	gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN.  FFS what information

2.	gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling.   The resource allocation can be triggered by CN request.   UE reader may provide assistance information related to AIoT transmissions.  FFS what the information may be useful to be provided to gNB.  It is up to gNB how the information is used.  It is understood that assistance information is not mandated for the gNB to determine resource allocation.

3. 	the reader schedules the AIoT interface transmissions within the resources allocated by the gNB.

To improve resource efficiency, it is observed that A-IoT operations are task-oriented (e.g., scanning, command delivery). Upon task completion, the UE Reader may indicate resource release to the gNB. This supports timely resource reconfiguration and power saving.

Proposal 4: RAN2 should consider specifying UE-initiated resource release procedures for A-IoT operations to support efficient resource handling and transmission control.

2.3 A-IoT UE Reader Capability Framework

Different UE Readers have different capabilities. Some UE Readers support only Device 1, while others support Device 2b or both. Some UE Readers can generate carrier wave, while others cannot. The number of devices that a UE Reader can handle also varies between implementations.

Different cells have different deployments. Some cells have only Device 1 deployed, some have only Device 2b, and some have both. In some cells, the network provides carrier wave. In other cells, the UE must generate carrier wave.

To support diverse Ambient IoT deployments, it is essential to define standardized mechanisms for UE Readers to report their capabilities (e.g., supported device types, waveforms, CW generation, traffic types) and for the network to inform UE Readers about cell-specific configurations. This enables efficient resource allocation, avoids operational mismatches, and ensures interoperability across vendors.

Proposal 5: RAN2 should consider specifying capability exchange procedures between UE Readers and the network to support flexible Ambient IoT deployments.

# Conclusions

This contribution presents proposals to enhance RRC re-establishment and handover procedures for intermediate UE Readers in Ambient IoT deployments. These enhancements aim to improve service continuity, reduce signaling overhead, and support reliable operation in mobile scenarios, in alignment with the objectives of the Release 20 WID. Additionally, we request RAN2 to discuss the following proposals:

Proposal 1: RAN2 should specify enhanced RRC re-establishment procedures for UE Readers, including A-IoT-specific context retention mechanisms at the gNB to support seamless recovery when reconnecting to the same cell after RLF.

Proposal 2: RAN2 should specify mechanisms for UE Readers to transfer A-IoT operational context and statistics to the target gNB during RRC re-establishment or handover completion, enabling efficient resource allocation and procedure resumption.

Proposal 3: RAN2 should consider specifying mechanisms in system information to assist UE Readers during cell selection and reselection, including:

- Indication of A-IoT Topology 2 support

- Indication of availability of A-IoT resource configuration

Proposal 4: RAN2 should consider specifying UE-initiated resource release procedures for A-IoT operations to support efficient resource handling and transmission control.

Proposal 5: RAN2 should consider specifying capability exchange procedures between UE Readers and the network to support flexible Ambient IoT deployments.

# References

[1] RP-252894: Work Item Description for Ambient IoT Phase 2

[2] RAN2#131bis Chair note