3GPP TSG-RAN WG2 Meeting #132	R2-2508698

Dallas, USA, 17th - 21st, November 2025

Agenda item:	8.9.2

Source:	Sateliot, Novamint, Thales, Sharp, Nordic

Title:	On the need to pass Satellite ID to upper layers

WID/SID:	IoT_NTN_Ph3 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

On the use of Satellite IDs, RAN2#131 made the following agreements:

We send an LS to CT1 asking whether the Satellite ID information broadcast in SIB should be sent to NAS

We clarify the stage-2 description in 36.300 to state that the satellite identifier values used at the AS-level (e.g., satellite identifiers included in SIB messages) and the NAS-level (e.g. satellite identifiers included in the S&F Monitoring list) are set consistently, i.e. with the same value, when they refer to the same satellite (FFS on the actual wording)

A related LS [1] was sent to CT1, and an LS reply [2] indicates the following:

- CT1 would like to inform that in the context of S&F Satellite Operation, Satellite ID information read from SIB is used by NAS as already specified in TS 24.301 V19.4.0 (2025-09).

Given the positive response from CT1, the objective of this paper is to reflect the necessary change in a related RAN2 specification.

# 2	Discussion

## 2.1      Satellite ID of a Serving Cell

As indicated in [2][3] and [4], satellite ID of a serving cell is used in a number of situations by NAS – brief information of a few instances where it has been used is provided below:

- The estimated S&F uplink delivery time duration is associated only with the satellite ID of the satellite which provides the estimated S&F uplink delivery time duration [3].

- +CSTFOR passes satellite ID of the serving cell on to the application layer [4]

Observation 1: Satellite ID of the satellite that operates the serving cell is used by NAS.

As specified in [5], SIB31/SIB31-NB contains satellite assistance information including the satellite ID of the serving cell. Hence, whenever a UE reads SIB31/SIB31-NB content, the UE needs to pass the satellite ID on to upper layers.

Therefore, it is important that RAN2 specification captures a related UE behaviour.

Observation 2: On reading SIB31/SIB31-NB content, the UE shall forward the satellite ID of the serving cell to upper layers.

Proposal 1: RAN2 is respectfully requested to add a stage-3 description in clause 5.2.2.39 of TS 36.331 to state that the UE shall forward the satellite ID of the serving cell to upper layers (related TP provided in the Annex).

# 3	Conclusion

This paper tries to act upon the LS response [2] received from CT1 by making the following observations and a proposal:

Observation 1: Satellite ID of the satellite that operates the serving cell is used by NAS.

Observation 2: On reading SIB31/SIB31-NB content, the UE shall forward the satellite ID of the serving cell to upper layers.

Proposal 1: RAN2 is respectfully requested to add a stage-3 description in clause 5.2.2.39 of TS 36.331 to state that the UE shall forward the satellite ID of the serving cell to upper layers (related TP provided in the Annex).

# Reference

- R2-2506285, LS on S&F Operation, RAN2#131, Bangalore, India, Aug. 2025.

- R2-2508004, Reply LS on S&F Operation, Dallas, USA, Nov. 2025.

- TS 24.301, Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3, V19.4.0 (2025-09).

- TS 27.007, AT command set for User Equipment (UE), V19.4.0 (2025-09).

- TS 36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification, V19.0.0 (2025-10).

# Annex: TP for TS 36.331

## 5.2.2.39	Actions upon reception of SystemInformationBlockType31

Upon receiving SystemInformationBlockType31 (SystemInformationBlockType31-NB), the UE shall:

- start or restart timer T317 with the duration ul-SyncValidityDuration from the subframe indicated by epochTime;

- forward the t-ModeSwitching to upper layers, if present;

- forward the satelliteId to upper layers, if present.