3GPP TSG-WG2 Meeting #132	R2-2508733

Dallas, USA, November 17 – 21, 2025

Agenda Item:	8.11.2

Source:	Ericsson

Title:	MAC remaining issues

Document for:	Discussion

# 1	Introduction

In this document, we discuss some remaining MAC issues.

# 2	Discussion

## 2.1 MAC-1: Whether/how to address the issue in case RO type switching and Msg1 repetition number fallback are triggered simultaneously

For the new issue MAC-1

MAC-1: Whether/how to address the issue in case RO type switching and Msg1 repetition number fallback are triggered simultaneously.

As other companies indicated during the offline discussion, this issue does not cause any critical issue, so no change is essential. In addition, the proposed TP does not implement the intended behaviour correctly. There are several cases for which the UE can check both conditions of RO type switch and Msg1 repetition number. As a summary, we think the changes are not needed to address the issue.

No changes needed in MAC spec to address the issue in case RO type switching and Msg1 repetition number fallback are triggered simultaneously.

## 2.2 MAC-2: Content to be captured in the LS for requesting RAN1 to update the definition of the first PRACH occasions in TS 38.213, to let it inclusively cover the ROs for cases where tdd-UL-DL-ConfigurationCommon is not applied

For this issue, we agree with the proposal suggested by the rapporteur.

[MAC-2] Proposal: RAN2 to send an LS to RAN1, requesting an update to the definition of the first PRACH occasions in TS 38.213, to let it inclusively cover the ROs for cases where tdd-UL-DL-ConfigurationCommon is not applied (e.g., FDD, SUL, TDD without tdd-UL-DL-ConfigurationCommon configured).

RAN2 to send an LS to RAN1, requesting an update to the definition of the first PRACH occasions in TS 38.213, to let it inclusively cover the ROs for cases where tdd-UL-DL-ConfigurationCommon is not applied (e.g., FDD, SUL, TDD without tdd-UL-DL-ConfigurationCommon configured).

## 2.3 MAC-3: UE transmit power continuity during RO type switching

Issue description (Nokia)

During RAN2#131, companies agreed to compensate for the UE transmit power difference between legacy RO and SBFD RO by introducing a power offset: Proposal: For the RO type fallback between legacy RO and additional RO for RACH configuration option 2, a power offset given by the difference between the two quantities of preamble power ramping steps is added.. One of the arguments for supporting this power offset compensation is that the UE should not change its transmit power by a value higher than 6 dB between mgs1 transmissions, as described in TS 38.331: TS 38.331: powerRampingStep ENUMERATED {dB0, dB2, dB4, dB6},. By using this approach, the UE power continuity is ensured given that the preamble received target power is the same. However, in Nokia's view, the current proposal is not completely solving the UE transmit power continuity issue when switching between SBFD RO to legacy RO or vice versa. The reason is that current proposal only compensates for the potential differences in the power ramping step but doesn’t take into account the potential differences in the preamble received power. In Configuration 2, the UE will receive 2 independent RACH-ConfigCommon configurations, which among others, contain different preambleTargetReceivedPower and/or different powerRampingStep (among others). Therefore, the UE transmit power continuity is not ensured. This can lead to a larger power ramp-down or power ramp-up of 6 dB, which is not acceptable from UE vendors point of view as per last meeting’s discussion. Therefore, it is required to find a new way of compensating for both the differences in power ramping offsets and preamble received target powers.

For this issue, we think the situation is different in different scenarios/use cases of SBFD operation i.e., coverage/range extension, or capacity/latency improvement. In the scenarios where SBFD operation is applied to extend coverage/range, the UE may select legacy ROs for initial PRACH transmissions. Failures of initial PRACH transmissions are typically due to bad coverage e.g., the UE is far away from the gNB or the UE uses unsuitable PRACH preamble formats. Adding additional power offset would not help in this case. Instead, the UE should apply longer PRACH preamble formats for subsequent RACH attempts.

In the scenarios where SBFD operation is applied to extend coverage/range, adding additional power offset would not help when the UE falls back from legacy ROs to additional ROs. Instead, the UE should apply longer PRACH preamble formats for subsequent RACH attempts.

In the scenarios where SBFD operation is applied to improve RACH capability or reduce RACH latency, the UE may select the additional ROs for initial PRACH transmissions. If the UE experiences failures during initial PRACH transmissions, it may be beneficial for the UE to fall back to legacy ROs while maintaining the same power level for subsequent RACH attempts. However, we believe the values of preamble received target power configured between additional ROs and legacy ROs are typically similar. Therefore, additional time offset reflecting the difference of preamble received target power would be unnecessary.

In the scenarios where SBFD operation is applied to improve RACH capability or reduce RACH latency, it may be beneficial for the UE to fall back from additional ROs to legacy ROs while maintaining the same power level for subsequent RACH attempts. However, we believe the values of preamble received target power configured between additional ROs and legacy ROs are typically similar. Thus, addition additional power offset may only give marginal gain.

All in all, we think the additional power offset reflecting the difference in preamble received target power between SBFD RO and legacy RO is unnecessary.

Not introduce the additional power offset for the difference in preamble received target power between SBFD RO and legacy RO.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1	In the scenarios where SBFD operation is applied to extend coverage/range, adding additional power offset would not help when the UE falls back from legacy ROs to additional ROs. Instead, the UE should apply longer PRACH preamble formats for subsequent RACH attempts.

Observation 2	In the scenarios where SBFD operation is applied to improve RACH capability or reduce RACH latency, it may be beneficial for the UE to fall back from additional ROs to legacy ROs while maintaining the same power level for subsequent RACH attempts. However, we believe the values of preamble received target power configured between additional ROs and legacy ROs are typically similar. Thus, addition additional power offset may only give marginal gain.

Based on the discussion in the previous sections we propose the following:

Proposal 1	No changes needed in MAC spec to address the issue in case RO type switching and Msg1 repetition number fallback are triggered simultaneously.

Proposal 2	RAN2 to send an LS to RAN1, requesting an update to the definition of the first PRACH occasions in TS 38.213, to let it inclusively cover the ROs for cases where tdd-UL-DL-ConfigurationCommon is not applied (e.g., FDD, SUL, TDD without tdd-UL-DL-ConfigurationCommon configured).

Proposal 3	Not introduce the additional power offset for the difference in preamble received target power between SBFD RO and legacy RO.

# 4	References

- [1] 	RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei,

- Shanghai, China, June 17-20, 2024, RAN#104.