R2-2508745

3GPP TSG-RAN WG2 Meeting #132

Dallas, USA, Nov. 17th - 21st 2025

Source: 		Lenovo, Aumovio

Agenda Item: 	10.2.3

Title:			Day1 Services and Forward compatible design

# Introduction

In Prague, RAN2#131bis was convened and first and very useful agreements could be realized.  This document build on these and is focused on following highlighted Agenda:

10.2.3 – Design approaches, New services, Others

Including contributions on guidelines and views on scalability, extensibility and future proofness including support of diverse device types (e.g., categories) and how this would affect our design approach for L2 and L3 protocols.

Including contributions on what services to support in the baseline design, what to optimize for day-1 and what forward compatible aspect to consider for futures not included in Day-1.

# Statistics

We are at an interesting point in time where the standardization of 5G is slowly paving way for the next generation, 6G. Comparison between the predecessor and current generations is a norm. Like comparisons have been made between 4G and 5G success, some claiming that the latter has not seen LTE kind of success which arguably may hold some merit since the vision of 5G was mainly centred at the verticals. Fast forward - in years to come 6G will be compared against 5G. From purely subscription point of view 5G has been seeing stronger deployments recently which should keep growing in years to come. Here is some interesting data before we start our technical discussion.

Figure 1: 5G growth expectation until 6G deployment

From, the Figure 1, whether the potential (z-score) for 5G means a regional delay for 6G adoption or rather depicts a bigger Pie for 6G investment can be individually interpreted by the reader!

With this, let’s start our discussion.

# Discussion:

# Services

Industry experience from 5G so far hints, why “simple and short” can be useful for 6G. Operators largely focused on NSA first; SA ramped (very) slowly. Verticals like URLLC/TSN for industry, network slicing end-to-end, NR sidelink (V2X), IAB backhaul, MBS/broadcast, NTN, even RedCap—saw limited, trial-mainly/ only deployment. Designing all of this together diluted focus, led to vendor lock-in, and slowed 5G returns. The lesson for 6G: baseline around broadband, voice/emergency, and massive IoT; day-1 optimized control-plane and energy efficiency; with added focus on forward-compatibility so new features can be plugged-in in later without denying the legacy UEs of even emergency calls.

The below table collects our opinion for 6G on some main features:

Table 1: Potential Services for Day-1 6G

| Area | Baseline | Day-1 optimize for | Forward-compatibility |
| --- | --- | --- | --- |
| Voice + emergency calls | IMS availability (no Fallback) | Quicker setup + reconf. for Voice profile. Non-Zero but low latency HOs. | Design ensures that future features do not devoid ‘legacy’ UEs to make EM Calls. |
| Mobile broadband | QoS via small 6QI set, MPTCP/QUIC friendly | PDCCH efficiency, fast DRX (BWP?) changes, optimizations (for video bursts) to avoid HOL issues | Small but varied 6QI set can allow variability among eMBB traffic, Lossless mobility across RAT/NTN with max 2 mobility options |
| Massive IoT + RedCap | Low-overhead attach/ connection, small data (C-plane & U-plane), extended DRX, coverage enhancement | Wider/ group-wise RRC (re)configuration, long-periodic UL grants, contention-based access | 2-Lite profiles, CE |
| Multicast/Broadcast (MBS) | Not required in the first release of 6G |  |  |
| Sidelink | Not required in the first release of 6G |  |  |
| NTN | Integrated with TN but with provisioning of a subset of services |  |  |
| XR/Interactive | XR R18 and R19 enhancements for finer-grained QoS differentiation as 6G Day 1 features, including PDU sets, data bursts, and MMSID awareness at the RAN. |  |  |
| Public safety | Priority/QoS pre-emption | Coverage-extended PDCCH | Mission-critical groupcast; sidelink fallback for later releases |

Features to be supported on Day1: On integrated protocol supporting TN-NTN communication support Voice call, Emergency call, eMBB, 2 levels of reduced capability devices, XR and PS services. Do not support PC5 interface, MBS and URLLC in the first release.

Proposal 1: RAN2 study aims at supporting Voice call, Emergency call, eMBB, 2 levels of reduced capability devices, XR and PS services. But PC5 interface, MBS and URLLC in the first release is not required to be supported.

The Data Plane (Sensing, AIML, SON/MDT, QOE etc.) is not seen as “service” for this discussion. And we have not mentioned Slicing and some other features in the Table 1, but these may also be discussed to refine the table for RAN2 study purpose:

Proposal 2: RAN2 refines and stabilizes the Table 1 with features to be supported for Day1 and captures the same in TR.

# Forward compatibility

NES is a classic example of how a future feature can potentially jeopardize provisioning of the most basic service – like an emergency call to a legacy UEs. For some features in 5G we relied on the MIB barring to keep the legacy UEs out and designed a workaround barring for the new features (in SIB1). The Cell could potentially serve EM calls and provide some other services to UEs.

Future feature introductions can unintentionally compromise delivery of the most basic services—for example, emergency calls for legacy UEs. In 5G, interactions between legacy access restrictions and new feature introduction illustrated how protective measures can go against ordinary service provisioning at cell level. A cell might still need to serve emergency traffic while differentiating other services, yet faulty designs risk unintentional results across UE types and releases. Looking ahead, the challenge is to define clear, release-robust service availability signaling that preserves legacy access for critical functions and remains comprehensible to devices over time—even as features evolve and are regrouped across generations.

Proposal 3: RAN2 study means to define clear, release-robust service availability signaling that ensures legacy access for critical functions.

# Conclusion

This document discussed what services to support in the baseline design, what to optimize for day-1 and what forward compatible aspect to consider for futures not included in Day-1. The following proposals are made:

Proposal 1: RAN2 study aims at supporting Voice call, Emergency call, eMBB, 2 levels of reduced capability devices, XR and PS services. But PC5 interface, MBS and URLLC in the first release is not required to be supported.

Proposal 2: RAN2 refines and stabilizes the Table 1 with features to be supported for Day1 and captures the same in TR.

Proposal 3: RAN2 study means to define clear, release-robust service availability signaling that ensures legacy access for critical functions.

Based on Ericsson Mobility Report, gsmaintelligence, 5gamericas.org and some Country-level specific data.