3GPP TSG-RAN WG2 Meeting #132   	                              R2-2508834

Dallas, USA, November 17 - November 21, 2025

Agenda item:	9.2.2

Source:	CMCC

Title:	Discussion on Topology 2 for A-IoT

Document for:	Discussion

Introduction

In RAN2#131bis meeting, the following agreements are made [1]:

- Agreements - 1. gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN. FFS what information - 2. gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling. The resource allocation can be triggered by CN request. UE reader may provide assistance information related to AIoT transmissions. FFS what the information may be useful to be provided to gNB. It is up to gNB how the information is used. It is understood that assistance information is not mandated for the gNB to determine resource allocation. - 3. the reader schedules the AIoT interface transmissions within the resources allocated by the gNB. - Continue discussion next meeting on how resource allocation looks like.

In this contribution, we would like to provide our further views on the issues related to A-IoT Topology 2.

Discussion

2.1 Relationship with R19 Topology 1

In Release 19, A-IoT topology 1 has been specified to support A-IoT. When it starts specifying topology 2 in R20, some clarification is necessary on the relationship between topology 1 and topology 2. To be specific, the objective of R19 A-IoT is towards D1T1, where indoor A-IoT devices communicate with an indoor base station. In R20, D2T2-B scenario, where UE-reader assists the indoor A-IoT device to communicate with outdoor base station, is going to be specified. Therefore, it is obvious that the A-IoT-enabled gNBs for R19 and those for R20 are not co-located. In other words, we do not need to consider the scenario where the same gNB supports both Topology 1 and Topology 2 for device 1.

Observation 1: The objective of R19 A-IoT is towards D1T1, where indoor A-IoT devices communicate with an indoor base station. In R20, D2T2-B scenario, where UE-reader assists the indoor A-IoT device to communicate with outdoor base station, is going to be specified.

Proposal 1: The scenario where the same gNB supports both Topology 1 and Topology 2 for device 1 is not considered in R20.

2.2 General analysis of topology 2

### 2.2.1 Protocol stack of topology 2

In the revised WID, it has been clearly stated that [2]:

- Specify RRC-based architecture, in coordination with SA2.

It is clear that only RRC-based solution is specified for R20 topology 2. The protocol stack of Topology 2 is given in Figure 1, where the design of A-IoT radio interface in R19 is fully reused and UE-reader communicates with A-IoT enabled gNB via the control plane.

Figure 1. Protocol stack of Topology 2.

Proposal 2: The protocol stack of Topology 2 as shown in Figure 1 can serve as the baseline.

### 2.2.2 Uu Enhancement for A-IoT

#### 2.2.2.1 Radio bearer for transmitting A-IoT related message(s)

For Topology 2, it is essential to study the necessary Uu enhancements to support A-IoT operations of the UE-reader. This includes transmitting inventory or command requests to the UE-reader, allocating or releasing of A-IoT radio resources, and reporting of inventory or command responses. Only UE-readers in RRC_CONNECTED state are considered for these operations, which ensures that network has an active UE context and is capable of performing inventory or command operations reliably, avoiding unnecessary signaling attempts for idle or inactive UE.

We have noticed that, in NR sidelink, the RRCReconfiguration message transmitted in SRB1 is used to the dedicated configurations for NR sidelink communication/discovery/positioning and the data of remote UE is transmitted via the established DRB between gNB and remote UE. Inspired by this, we think that RAN2 is asked to discuss the SRB for allocating A-IoT radio resources and SRB for transmitting inventory/command request/response separately. That is, when discussing the SRB(s) to transmit A-IoT related messages, RAN2 is asked to discuss the following cases:

- SRB used to allocate A-IoT radio resources for UE-reader;

- SRB used to transmit inventory/command request/response.

From our point of view, for both cases, we are also open to discuss whether legacy SRB is used or a new SRB is designed.

Proposal 3: For the SRB(s) to transmit A-IoT related messages, RAN2 is asked to discuss the following cases:

- SRB used to allocate A-IoT radio resources for UE-reader;

- SRB used to transmit inventory/command request/response.

#### 2.2.2.2 Dealing with inventory/command responses

With the consideration that forwarding inventory/command responses device by device from the UE-reader may cause significant signalling and energy overhead, it is recommended that the UE-reader can buffer some collected inventory/command responses for a period of time. Besides, when scheduled, the UE-reader can report the collected inventory/command responses from multiple A-IoT devices in one RRC message.

Besides, another important issue to be discussed is how the UE-reader handles the collected D2R upper-layer data (i.e., inventory/command responses) during RLF case, i.e., whether to discard it. From our perspective, the collected inventory/command responses are buffered in RRC entity before composing the RRC signaling and deliver it to the lower layers. For the inventory/command responses that still buffered in RRC entity during RLF, UE-reader can maintain them.

In terms of the inventory/command responses which have been delivered to PDCP/RLC/MAC layers, we have the following observation. In legacy NR, for SRBs, when PDCP entity is re-established, all PDCP PDUs and SDUs are discarded. For inventory response and command response, we do not think they are important enough to require modifying the behavior of PDCP entity re-establishment. While, it is also possible that UE-reader selects the suitable cell which is a LTM candidate cell and executes LTM successfully. In such case, RLC entity or PDCP entity may not be re-established, so the inventory response and command response may not be discarded. In summary, we think that there is no need to ensure that inventory responses or command responses are not discarded, that is, no specification change on re-establishment of RLC entity or PDCP entity for A-IoT is needed.

Finally, if UE-reader can not find a suitable cell and enters RRC_IDLE after RLF is declared, just like releasing security keys, measurement reports, etc., UE-reader should release all collected D2R upper-layer data directly. Therefore, we think there is no need to ensure that inventory/command responses are not discarded during RLF.

Proposal 4: UE-reader is allowed to buffer some collected inventory/command responses for a period of time.

Proposal 5: UE-reader is allowed to report the collected inventory/command responses from multiple A-IoT devices in one RRC message.

Proposal 6: During RLF, UE-reader can maintain the collected inventory/command responses that still buffered in RRC entity.

Proposal 7: During RLF, for the inventory/command responses which have been delivered to lower layers (i.e. PDCP/RLC/MAC/PHY layers), no specification change on re-establishment of RLC entity or PDCP entity for A-IoT is needed.

2.3 Resource of A-IoT radio interface

### 2.3.1 A-IoT radio resource

The A-IoT radio resource includes both RA resources and data transmission resources. For topology 1, the paging message schedules the resource for A-IoT device to transmit A-IoT MSG2, the A-IoT MSG2 schedules the resource for A-IoT device to transmit A-IoT MSG3 and the R2D Upper Layer Data Transfer message schedules the resource to transmit D2R upper-layer data. For topology 2, the A-IoT radio resource between the UE-reader and A-IoT device should be controlled by network as well.

In RAN2#131bis meeting, it has been agreed that:

- 2. gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling. The resource allocation can be triggered by CN request. UE reader may provide assistance information related to AIoT transmissions. FFS what the information may be useful to be provided to gNB. It is up to gNB how the information is used. It is understood that assistance information is not mandated for the gNB to determine resource allocation. - 3. the reader schedules the AIoT interface transmissions within the resources allocated by the gNB. - Continue discussion next meeting on how resource allocation looks like.

Based on the agreements, it is clear that the scheme that gNB allocates the A-IoT radio resources to UE-reader directly after receiving the Inventory Request message or Command Request message is considered as the baseline. In addition to the baseline, whether UE is allowed to provide some assistance information for reference when gNB allocates/modifies A-IoT radio resources needs further discussion.

Observation 2: The gNB allocates the A-IoT radio resources to UE-reader directly after receiving the Inventory Request message or Command Request message is considered as the baseline.

#### 2.3.1.1 Resource allocation

In Topology 1, the interference between gNB-readers is not severe, since the gNB generally has good interference coordination capability based on network implementation. However, in Topology 2, as one UE-reader can not obtain any information of other UE-readers, efficient interference coordination among UE-readers becomes really challenging. Therefore, it is important to avoid interference among UE-readers when gNB allocates the A-IoT radio resources to different UE-readers. From our perspective, dedicated and orthogonal resource allocation is recommended so that no resources are shared between UE-readers, preventing the potential interference and ensuring predictable performance.

Triggered by the received inventory/command request from CN, the method by which the gNB dynamically allocates resources to the UE-reader for one A-IoT procedure seems to maximize the utilization efficiency of A-IoT air interface resources, and thus should be adopted as the baseline. As for the parameters for allocating the A-IoT radio interface, we think that the frequency resource, starting time and time domain duration can be indicated to UE-reader. What’s more, an A-IoT radio release message is also supported to to ensure the flexibility of gNB in releasing A-IoT radio resources. To this end, within this serving cell, in the case that UE-reader is not indicated to release the resource, the A-IoT radio resource is valid between starting time and ending time. Note that the validity of A-IoT radio resource under some special cases, e.g. handover, RLF and state transition, is analyzed later.

Proposal 8: A-IoT radio resources for UE-readers should be allocated in an orthogonal manner, such that the resources are not shared with or interfered by other UE-readers.

Proposal 9: Triggered by the received inventory/command request from CN, the method by which the gNB dynamically allocates resources to the UE-reader for one A-IoT procedure should be adopted as the baseline.

Proposal 10: The frequency resource, starting time and time domain duration can be provided to UE-reader to indicate the allocated A-IoT radio resource.

Proposal 11: RAN2 is asked to support gNB release the A-IoT radio resource allocated to UE-reader via RRC message.

#### 2.3.1.2 Assistance information provided by UE-reader

From our perspective, UE-reader should be allowed to provide some assistance information for gNB’s reference to re-allocate/modify A-IoT radio resources to improve the spectrum efficiency of A-IoT radio interface. To be specific, when the UE-reader finds that the allocated AIoT radio resources cannot complete the entire AIoT procedure, or when it completes the entire AIoT procedure before resources are used up, it can provide assistant information to assist the gNB in modifying resource allocation. Different assistance information may be provided by UE-reader for different cases. For the case that UE-reader completes the entire AIoT procedure in advance, it can just notify the gNB that it has completed. However, for the case that the allocated A-IoT radio resource is not sufficient, it may provide request indication or the additional time domain resource needed or the estimated number of uninventoried A-IoT devices or any other information to the gNB.

Proposal 12: UE-reader is allowed to provide some assistance information for gNB’s reference to re-allocate/modify A-IoT radio resources.

Proposal 13: When UE-reader completes the entire AIoT procedure before resources are used up, it can notify the gNB that it has completed through assistance information.

Proposal 14: When the allocated A-IoT radio resource is not sufficient, RAN2 can discuss the following potential assistance information:

- resource request indication;

- the additional time domain resource needed;

- the estimated number of uninventoried A-IoT devices;

- FFS other information.

### 2.3.2 Validity of A-IoT radio resources during mobility

As mentioned in the WID, the UE-reader's A-IoT resource is valid only within the serving cell. From our perspective, the scenario that the UE-reader handovers to another cell, where the A-IoT procedure has been started or while it has already not completed when the UE-reader receives HandoverCommand, should be discussed. From our perspective, some interaction between the UE-reader and the gNB before the handover is necessary and valuable to lower adverse impact of the handover procedure on the A-IoT radio. For example, the inventory responset or command response that has been buffered at UE side may be triggered to transmit to the source gNB before handover. For the inventory case, although the procedure may be still on going, the partial result is still beneficial. Because the motivation of A-IoT inventory service request is to find the target device(s), the reported device IDs can be identified in core network and subsequent command can be handled toward them.

Taking the basic handover procedure into account, the UE-reader reports measurement results so that the network can decide whether to trigger a handover procedure. Once the UE-reader receives the HandoverCommand, which contains the configuration of target gNB, from the source gNB, it begins accessing the target gNB. Before sending this HandoverCommand, the source gNB may request the UE-reader to report any partial inventory or command feedback. Moreover, the gNB can implicitly instruct the UE-reader, within the HandoverCommand, to terminate the A-IoT service and release the A-IoT resource by no A-IoT radio resource configuration of target gNB included in HandoverCommand.

Proposal 15: Support A-IoT service continuity (release/suspend/continue) for UE-reader HO case.

Proposal 16: The gNB can request the UE-reader to report any partial inventory response(s) or command response(s) before HO.

Proposal 17: Support network to terminate the A-IoT service and release the A-IoT resource, e.g., gNB can implicitly instruct the UE-reader to do this within the HandoverCommand.

### 2.3.3 Validity of A-IoT radio resources during RLF

- Since it has been clarified that A-IoT radio resource of the UE-reader is only valid within the serving cell in the WID and it is possible for the UE-reader to change the serving cell when RLF occurs, for example, the UE-reader the UE has successfully completed RRC re-establishment in a new cell that was not its last serving cell. Therefore, one important issue to be discussed is the validity of A-IoT radio interface resource. During the R19 SI, the following scenarios are discussed [3]:

- Scenario 1: In RRC_CONNECTED, the UE-reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface: - Option 1: UE-reader receives the resources configuration in NR Uu dedicated RRC signalling. The radio resources remain valid until the network releases them explicitly. This would be supported by the NR Uu RRC reconfiguration; and/or - Option 2: UE-reader receives the resources configuration in NR Uu dedicated RRC signalling, which configures a time period in which the corresponding resource can be used. The UE-reader considers that the resources remain valid for that time period, unless the resource configuration is explicitly released by the network by the NR Uu RRC reconfiguration. - Scenario 2: In RRC_CONNECTED, the UE-reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface, upon its temporary out of connection scenarios (e.g., RLF and handover cases): - Option 1: The UE-reader considers the resources as temporarily invalid during the temporary out of connection condition. The resource may become valid again after the UE recovers from the condition. - Option 2: The UE-reader considers that the resources remain valid for a time period. It needs to be further discussed on whether this time period belongs to the one in Option 2 for Scenario 1 or is a different time period. It needs to be further discussed on whether the duration of this time period is related to the ongoing A-IoT operations.

- Considering the mobility nature of UE-reader, we think it is more suitable to suspend the A-IoT radio interface resource when RLF occurs (i.e., Scenario 2, Option 1). As for the starting time for UE-reader suspending the A-IoT radio interface resource, there are following two options:

- Option 1: when UE declares RLF;

- Option 2: when UE selects a new suitable cell which is different from the serving cell where it declares RLF or T311 expires.

- For us, we prefer it to be aligned with the time for UE-reader declaring RLF (i.e., Option 1) to avoid the potential interference as much as possible.

- In terms of whether the A-IoT radio interface resource can be resumed, we think that it relates to whether UE-reader can initiate the RRC re-establishment procedure and whether network can find and verify a valid UE context. To be specific, the following cases are discussed.

- Case 1: UE-reader can not initiate the RRC re-establishment procedure.

There are three situations resulted to Case 1: AS security of UE-reader has not been activated or AS security has been activated, but SRB2 and at least one DRB or multicast MRB are not setup, or UE-reader can not find a situable cell within a certain time after RLF was declared. In Case 1, the UE-reader enters RRC_IDLE as specified in TS 38.300, therefore it should release the A-IoT radio interface resource.

- Case 2: UE-reader can find a suitable cell and initiates the RRC re-establishment procedure, and the network can find and verify a valid UE context after RLF declared.

- In this case, if the configuration of A-IoT radio interface is determined to be added to UE context by RAN3, then the UE-reader can be instructed to resume the A-IoT radio interface resource in the latter selected cell.

- Case 3: UE-reader can find a suitable cell and initiates the RRC re-establishment procedure, but the network cannot find or verify a valid UE context after RLF declared.

- In this case, when UE-reader enters RRC_CONNECTED again, we are open to discuss whether it can request to resume/reconfigure the A-IoT radio interface resource from latter selected cell.

Proposal 18: RAN2 is asked to determine that when UE-reader declares RLF, it suspends the A-IoT radio interface resource at the same time.

Proposal 19: RAN2 is asked to discuss the following three cases when determining whether UE-reader can resume the suspended A-IoT radio interface resource after RLF declaration:

- Case 1: UE-reader can not initiate the RRC re-establishment procedure.

- Case 2: UE-reader can find a suitable cell and initiates the RRC re-establishment procedure, and the network can find and verify a valid UE context after RLF declared..

- Case 3: UE-reader can find a suitable cell and initiates the RRC re-establishment procedure, but the network cannot find or verify a valid UE context after RLF declared.

2.4 Behavior of UE-reader when RRC state transits

- In addition to RLF, there are also other events (such as dataInactivityTimer expires, handover failure etc.) which may cause the RRC state transition of UE-reader. Aligned with the behavior described in RLF case, the following principle can be summarized:

- When UE-reader transits from RRC_CONNECTED to RRC_IDLE/RRC_INACTIVE:

- it releases the A-IoT radio interface resource; and,

- it discards the inventory response(s) and command response(s) (if any).

Proposal 20: When UE-reader transits from RRC_CONNECTED state to RRC_IDLE/RRC_INACTIVE state:

- it releases the A-IoT radio interface resource; and,

- it discards the inventory response(s) and command response(s) (if any).

2.5 ID related issue

### 2.5.1 Transaction ID

The transaction ID contained in the A-IoT paging message is a service request related ID. That is, from the devices' perspective, the transaction ID can be used to identify the A-IoT service request. When the device receives the A-IoT paging message with same transaction ID, the device may initiate re-access, if it is failed to access before; if the transaction ID in the paging message is new, the device responds to the new service request. In topology 1, The transaction ID can be generated by reader based on CN corelation ID. While, in topology 2, the transaction ID generation should be further discussed. At least, UE-reader generation or gNB coordination can be considered. For the case that the UE-reader generates the transaction ID based on the correlation ID, it seems to be aligned with the mechanism in topology 1. However, considering the mobility of UE-reader, the transaction ID may be conflicted and interference. For the case that gNB generates, gNB can coordinate the transaction ID used by different UE-readers, so that the UE-readers connected to same gNB can avoid using same transaction ID at same time. In addition, the transaction ID allocation can be along with the A-IoT service request delivery via SRB.

Proposal 21: For topology 2, the following two alternatives can be considered:

- Alt 1: UE-reader generates transaction ID based on the correlation ID;

- Alt 2: gNB generates transaction ID and transmits it to UE-readers.

Alt 2 is preferred.

### 2.5.2 AS ID

The AS ID is allocated in Msg2/R2D Upper Layer Data Transfer message, and can be carried in the subsequent R2D message to address the device. In R19, during A-IoT CBRA, the AS ID can be indicated to reuse the RN16 in Msg1 if no collisions or can be re-allocated by the reader. If such AS ID acquirement way is reused in topology 2, collision and interference issues should be considered for UE-reader. Different from the transaction ID, the number of AS ID may be very large for inventory service and the AS ID allocation is about to real-time. It is not an efficient way for the device to request an AS ID from the gNB during the RA procedure. Thus, the follow 2 alternatives can be considered to avoid the AS ID collision issue: 1) UE-reader implementation based, i.e., with UE-reader identification as part of the AS ID; 2) gNB allocation based, i.e., pre-configured AS ID group,i.e. gNB specifies the range within which the UE-reader can generate the AS ID.

Proposal 22: The following 2 alternatives can be considered to generate the AS ID:

- Alt 1: UE-reader implementation based, i.e., with UE-reader identification as part of the AS ID;

- Alt 2: gNB allocation based, i.e., pre-configured AS ID group.

2.6 Signalling and procedure

Based on above analysis, we design the A-IoT related messages and an overview of the procedure in this section

### 2.6.1 A-IoT related message(s)

In RAN2#131bis meeting, it has been agreed that:

- 1. gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN. FFS what information

From our perspective, there are several types of A-IoT related messages that need to be specified between UE-reader and gNB. The Inventory/Command Trigger message, which is sent from the gNB to the UE-reader to initiate inventory or command procedure. Another one is the Inventory/Command Response message, which is sent by the UE-reader to report the inventory responses or command responses collected from A-IoT device to the gNB. Besides, we think that it is also necessary to request UE-reader to report the responses and/or command responses by gNB. Besides, there may be some messages which is related to the allocation and/or modification and/or release of the A-IoT radio resources. Based on above analysis, we think that the following messages should be considered:

1). Inventory/Command Trigger message

In R19, RAN3 has defined the Inventory Request message and Command Request message, which are transferred in NGAP. After checking the IEs in the Inventory Request message, we think that the Message Type, A-IoT Correlation ID, A-IoT Device Identification Requested, Inventory assistance information and Follow on Command Indication (if provided by CN) may need to be transmitted to UE-reader. Note that whether A-IoT Correlation ID is needed or not depends on whether UE-reader is responsible for generating the transaction ID. To this end, we think that at least Message Type, A-IoT Device Identification Requested, Inventory assistance information and Follow on Command Indication (if provided by CN) can be transmitted to UE-reader to trigger an inventory procedure. As for the command procedure, after checking the Command Request message, we think that at least Message Type, A-IoT NAS PDU and Command Assistance Information (if provided by CN) can be transmitted to UE-reader.

In addition to the information related to inventory/command procedure, the Inventory/Command Trigger message may also include the configuration of A-IoT radio resources, so that the UE-reader can perform the corresponding procedure directly.

The A-IoT Resource Request/Release message, through which the UE-reader can request additional resources if needed, or release excess resources if UE-reader completes the inventory/command procedure in advance.

2). Inventory/Command Response message

This message contains the inventory response(s) and/or the command response(s) of one or more A-IoT device(s).

3). Inventory/Command Response Request message

With the consideration of the timing for UE-reader transmitting the inventory/Command Respond message, both the proactive reporting by UE-reader and the method based on gNB requests should be studied. In terms of gNB-requested transmission of the inventory/Command Response message, we think that gNB can initiate this at least before handover/RRC state transition of the UE-reader with the Inventory/Command Response Request message.

4). A-IoT Radio Resource Allocation/Release message

This message is used to allocate or release resources of A-IoT radio interface to the UE-reader. The details of A-IoT radio resources are analyzed in section 2.3.1.1. With the consideration that the configuration of A-IoT radio resource can also be carried in the Inventory/Command Trigger message, we are also open to discuss whether the A-IoT Radio Resource Allocation/Release message and the Inventory/Command Trigger message can be merged. And we are also open to discuss whether the legacy RRCReconfiguration message can be enhanced to achieve the function of A-IoT Radio Resource Allocation/Release message.

5). A-IoT Radio Resource Assistance message

This message is transmitted from the UE-reader to the gNB to provide assistant information, which assists the gNB in modifying the allocated AIoT radio resources. For example, when the UE-reader finds that the allocated AIoT radio resources cannot complete the entire AIoT procedure, or when it completes the entire AIoT procedure before resources not sufficient, it can provide assistant information to assist the gNB in modifying resource allocation. The details of the assistant information are given in section 2.3.1.2.

Observation 3: Among the information received from CN, at least Message Type, A-IoT Device Identification Requested, Inventory assistance information and Follow on Command Indication (if provided by CN) can be transmitted to UE-reader to trigger an inventory procedure. At least Message Type, A-IoT NAS PDU and Command Assistance Information (if provided by CN) can be transmitted to UE-reader to trigger a command procedure.

Proposal 23: The following A-IoT related messages between UE-reader and gNB should be studied:

- gNB->UE-reader: Inventory/Command Trigger message, to initiate the inventory or command procedure for UE-reader, it may also contain the configuration of A-IoT radio resource.

- UE-reader->gNB: Inventory/Command Response message, to report the inventory response(s) and/or command response(s) to gNB.

- gNB-UE-reader: Inventory/Command Response Request message, to request UE-reader to report the inventory response(s) and/or command response(s) to gNB, before, e.g., HO or RRC state transition.

- gNB->UE-reader: A-IoT Radio Resource Allocation/Release message, to allocate or release A-IoT radio resource of the UE-reader. FFS whether the legacy RRCReconfiguration message can be enhanced to achieve the function of this message.

- UE-reader->gNB: A-IoT Radio Resource Assistance message, to provide assistant information to gNB for the allocation of A-IoT radio resource.

### 2.6.2 Overall signalling procedure

Based on above analysis, an example of the basic procedure of Topology 2 is given in Figure 2. In general, the inventory and command procedures between UE-reader and A-IoT devices should reuse Rel-19 mechanisms as much as possible. This allows implementation to leverage existing procedures, reduces the need for new signaling definitions, and maintains backward compatibility. In terms of the Uu interface, RAN2 is asked to discuss the necessary enhancement as introduced in section 2.2.2.1, 2.2.2.2 and 2.2.2.3. To be specific, the basic procedure shown in Figure 2 includes:

- gNB transmits the Inventory/Command Trigger message, which also includes the configuration of A-IoT radio resource, to initiate the inventory or command procedure for UE-reader;

- UE-reader initiates the inventory/command procedure in accordance with that in R19;

- If the allocated A-IoT radio resource is not sufficient and the inventory/command procedure has not been completed, UE-reader transmits the A-IoT Radio Resource Assistance message to indicate that more resources is needed to the gNB;

- In response to A-IoT Radio Resource Assistance message, the gNB transmits the A-IoT Radio Resource Allocation message to UE-reader to allocate additional A-IoT radio resources;

- With the additional A-IoT radio resources, UE-reader continues the inventory/ command procedure;

- After the inventory/command procedure is completed, UE-reader transmits the Inventory/Command Response message to report the collected inventory/command responses from A-IoT devices to the gNB.

Figure 2: Example of inventory or command A-IoT devices in Topology 2.

Proposal 24: Reuse Rel-19 inventory and command procedures for the A-IoT radio interface between the UE-reader and the A-IoT device(s).

Proposal 25: RAN2 is asked to take the procedure given in Figure 2 as a starting point.

Conclusions

Based on the discussions mentioned above, in this contribution we provide some discussions on A-IoT Topology 2:

Observation 1: The objective of R19 A-IoT is towards D1T1, where indoor A-IoT devices communicate with an indoor base station. In R20, D2T2-B scenario, where UE-reader assists the indoor A-IoT device to communicate with outdoor base station, is going to be specified.

Observation 2: The gNB allocates the A-IoT radio resources to UE-reader directly after receiving the Inventory Request message or Command Request message is considered as the baseline. In addition to the baseline, UE is allowed to provide some assistance information for gNB’s reference to re-allocate/modify A-IoT radio resources.

Observation 3: Among the information received from CN, at least Message Type, A-IoT Device Identification Requested, Inventory assistance information and Follow on Command Indication (if provided by CN) can be transmitted to UE-reader to trigger an inventory procedure. At least Message Type, A-IoT NAS PDU and Command Assistance Information (if provided by CN) can be transmitted to UE-reader to trigger a command procedure.

Relationship with R19 Topology 1

Proposal 1: The scenario where the same gNB supports both Topology 1 and Topology 2 for device 1 is not considered in R20.

Protocol stack of topology 2

Proposal 2: The protocol stack of Topology 2 as shown in Figure 1 can serve as the baseline.

Radio bearer for transmitting A-IoT related message(s)

Proposal 3: For the SRB(s) to transmit A-IoT related messages, RAN2 is asked to discuss the following cases:

- SRB used to allocate A-IoT radio resources for UE-reader;

- SRB used to transmit inventory/command request/response.

Dealing with inventory/command responses

Proposal 4: UE-reader is allowed to buffer some collected inventory/command responses for a period of time.

Proposal 5: UE-reader is allowed to report the collected inventory/command responses from multiple A-IoT devices in one RRC message.

Proposal 6: During RLF, UE-reader can maintain the collected inventory/command responses that still buffered in RRC entity.

Proposal 7: During RLF, for the inventory/command responses which have been delivered to lower layers (i.e. PDCP/RLC/MAC/PHY layers), no specification change on re-establishment of RLC entity or PDCP entity for A-IoT is needed.

- A-IoT radio resource allocation

Proposal 8: A-IoT radio resources for UE-readers should be allocated in an orthogonal manner, such that the resources are not shared with or interfered by other UE-readers.

Proposal 9: Triggered by the received inventory/command request from CN, the method by which the gNB dynamically allocates resources to the UE-reader for one A-IoT procedure should be adopted as the baseline.

Proposal 10: The frequency resource, starting time and time domain duration can be provided to UE-reader to indicate the allocated A-IoT radio resource.

Proposal 11: RAN2 is asked to support gNB release the A-IoT radio resource allocated to UE-reader via RRC message.

Assistance information provided by UE-reader

Proposal 12: UE-reader is allowed to provide some assistance information for gNB’s reference to re-allocate/modify A-IoT radio resources.

Proposal 13: When UE-reader completes the entire AIoT procedure before resources are used up, it can notify the gNB that it has completed through assistance information.

Proposal 14: When the allocated A-IoT radio resource is not sufficient, RAN2 can discuss the following potential assistance information:

- resource request indication;

- the additional time domain resource needed;

- the estimated number of uninventoried A-IoT devices;

- FFS other information.

Validity of A-IoT radio resources during mobility

Proposal 15: Support A-IoT service continuity (release/suspend/continue) for UE-reader HO case.

Proposal 16: The gNB can request the UE-reader to report any partial inventory response(s) or command response(s) before HO.

Proposal 17: Support network to terminate the A-IoT service and release the A-IoT resource, e.g., gNB can implicitly instruct the UE-reader to do this within the HandoverCommand.

Validity of A-IoT radio resources during RLF

Proposal 18: RAN2 is asked to determine that when UE-reader declares RLF, it suspends the A-IoT radio interface resource at the same time.

Proposal 19: RAN2 is asked to discuss the following three cases when determining whether UE-reader can resume the suspended A-IoT radio interface resource after RLF declaration:

- Case 1: UE-reader can not initiate the RRC re-establishment procedure.

- Case 2: UE-reader can find a suitable cell and initiates the RRC re-establishment procedure, and the network can find and verify a valid UE context after RLF declared..

- Case 3: UE-reader can find a suitable cell and initiates the RRC re-establishment procedure, but the network cannot find or verify a valid UE context after RLF declared.

Behavior of UE-reader when RRC state transits

Proposal 20: When UE-reader transits from RRC_CONNECTED state to RRC_IDLE/RRC_INACTIVE state:

- it releases the A-IoT radio interface resource; and,

- it discards the inventory response(s) and command response(s) (if any).

Transaction ID

Proposal 21: For topology 2, the following two alternatives can be considered:

- Alt 1: UE-reader generates transaction ID based on the correlation ID;

- Alt 2: gNB generates transaction ID and transmits it to UE-readers.

Alt 2 is preferred.

AS ID

Proposal 22: The following 2 alternatives can be considered to generate the AS ID:

- Alt 1: UE-reader implementation based, i.e., with UE-reader identification as part of the AS ID;

- Alt 2: gNB allocation based, i.e., pre-configured AS ID group.

A-IoT related message(s)

Proposal 23: The following A-IoT related messages between UE-reader and gNB should be studied:

- gNB->UE-reader: Inventory/Command Trigger message, to initiate the inventory or command procedure for UE-reader, it may also contain the configuration of A-IoT radio resource.

- UE-reader->gNB: Inventory/Command Response message, to report the inventory response(s) and/or command response(s) to gNB.

- gNB-UE-reader: Inventory/Command Response Request message, to request UE-reader to report the inventory response(s) and/or command response(s) to gNB, before, e.g., HO or RRC state transition.

- gNB->UE-reader: A-IoT Radio Resource Allocation/Release message, to allocate or release A-IoT radio resource of the UE-reader. FFS whether the legacy RRCReconfiguration message can be enhanced to achieve the function of this message.

- UE-reader->gNB: A-IoT Radio Resource Assistance message, to provide assistant information to gNB for the allocation of A-IoT radio resource.

Overall signalling procedure

Proposal 24: Reuse Rel-19 inventory and command procedures for the A-IoT radio interface between the UE-reader and the A-IoT device(s).

Proposal 25: RAN2 is asked to take the procedure given in Figure 2 as a starting point.

References

- Chair’s notes, RAN2 Chair(InterDigital), RAN2#131bis.

- RP-252894, Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2, RAN#109.

- TR 38.769, Study on solutions for Ambient IoT (Internet of Things) in NR (Release 19), V19.0.0.