3GPP TSG RAN WG2 #132		R2-2508971

Dallas, Nov 17th – Nov 21st, 2025

Agenda Item:	10.4

Source:	Indian Institute of Technology Madras (IITM)

Title:	Views on 6GR Mobility Framework

Document for:	Discussion

# Introduction

In RAN2 #131-bis [1], it was agreed to further study the following:

Agreements 1 Study mobility with the following requirements in 6G mobility design: - Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions. - Robustness of mobility procedures - Energy efficiency for both UE and NW 2 Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

# Background

5G NR specifications introduced several procedures for mobility management to meet the requirements of eMBB, URLLC and mMTC. Procedures like Conditional Handover (CHO), Dual Active Protocol Stack (DAPS), and Lower Layer Triggered Mobility (LTM) have each addressed specific shortcomings of traditional Layer 3 (L3) handover (HO).

However, these solutions operate independently, each optimized for a particular scenario, leading to fragmented mobility strategies and inconsistent user experiences. At the same time, these fragmented mobility strategies increase the implementation complexity. As a result, these solutions have not seen much real-time deployment by either NW or UE vendors.  The 6GR mobility framework should build on existing solutions but should aim to integrate into a single, AI-driven, adaptive, and cross-layer mobility system.

Observation 1: 5G NR Mobility solutions/procedures are designed to operate independently and are optimized for specific scenarios leading to fragmented mobility strategies and inconsistent user experiences.

Proposal 1: The 6GR mobility framework should build on existing solutions but should aim to integrate them into a single, AI-driven, adaptive, and cross-layer mobility system.

Traditional L3 handovers are reactive, signaling-heavy, and unsuitable for ultra-dense networks and high-mobility users (e.g., drones, high-speed trains). Conversely, CHO, DAPS, and LTM provide speed, reliability, and autonomy — but each in isolation.

Proposal 2: 6GR Mobility frameworks should consider at least the following aspects: (1) near-zero interruption handovers, (2) cross-layer coordination, (3) predictive and autonomous execution and (4) self-learning based mobility optimization.

# Views on framework for 6GR Mobility

The evolution towards 6G demands a shift from reactive to proactive and intelligent mobility management. Traditional L3-based handover mechanisms, though reliable, are limited by latency and signaling overhead. Enhancements such as Conditional Handover (CHO), Dual Active Protocol Stack (DAPS), and Lower Layer Triggered Mobility (LTM) in 5G Releases 16 and 17 improved reliability and responsiveness but still operated as independent features. The introduction of AI/ML mobility in 3GPP Releases 19 and 20 opens the door towards enabling predictions of RRM measurements and HO predictions before the actual values.

Therefore, the use cases of AIML mobility can be leveraged to predict the HO and the candidate list for the HO. Then, the serving gNB can prepare the conditional handover (CHO) configurations based on AI/ML predictions, transfer them to the UE and inform the neighboring gNBs. These CHO commands may include pre-set conditions — such as RSRP thresholds, hysteresis, and expiry timers — allowing the UE to execute a handover autonomously when the condition is met. Before HO conditions are met, the UE can prepare for early UL and DL synchronizations. If HO conditions are met, the UE can leverage the early synchronizations to switch to the target cell faster, avoiding higher interruption times. This also avoids the dependencies on DAPS, which is a complex procedure to implement.

This hybrid execution leverages CHO for configuration, LTM for reaction, and AI/ML for decision intelligence, ensuring a seamless transition without waiting for higher-layer RRC signaling.

Proposal 3: Consider the study of mobility frameworks that transform (or co-exist with) the existing mobility management from a deterministic event to an adaptive and predictive ecosystem. By integrating AI analytics with conditional and lower-layer execution, it achieves lower handover latency and reliability across diverse scenarios — from ultra-dense urban environments to high-speed rail networks.

Proposal 4: Study the unification of at least AI/ML mobility, CHO and LTM procedures, to effectively minimize the latency and interruption time and throughput degradations during handover.

Proposal 5: Further study is needed to determine whether or how DAPS can be integrated into a unified 6GR mobility framework.

Proposal 6: Develop a framework that defines thresholds, and fallback strategies between AI/ML mobility, CHO, and LTM, ensuring reliability and safety.

# 4 References

[1] RAN2#131-bis Chair Notes, Prague, October 2025