3GPP TSG-RAN WG2 #132                                                                                           R2-2508985

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	    10.4

Source: 	    MediaTek Inc.

Title: 	    Mobility for 6GR

Document for:	    Discussion and decision

# 1 Introduction

Study on 6G radio was approved in RP-251881[1] in RAN#108. One of the study objectives is Mobility for 6GR (for all RRC states), including related RRM (objective#4).

In RAN2#131bis, RAN2 has the following agreements related to 6G mobility:

Agreements

1	Study mobility with the following requirements in 6G mobility design:

-	Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions.

-	Robustness of mobility procedures

-	Energy efficiency for both UE and NW

2	Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

In this document, we continue to discuss the measurement aspects, IDLE and CONNECTED mobility procedure in 6GR.

# 2 Discussion

## 2.1 Measurement

One of the fundamental aspects of measurement is that which RS is required to be measured for mobility purpose.  In 5G NR, it is SSB that used for both idle and connected mobility. However, the requirements for in idle mode mobility are different from connected mode mobility. Reduction of network energy consumption is targeted for idle mode, while enhanced mobility performance (e.g., fast link set-up and higher beam-level SNR) is targeted for connected mode. This difference is expected to become even more pronounced in 6GR as we target for even lower energy consumption and higher mobility performance.

A single SSB cannot simultaneously satisfy the wide-beam, low-duty-cycle needs of idle mode and the narrow-beam, high-gain demands of connected mode. Therefore, reuse of the same RS for idle and connected mode for 6GR could be sub-optimal in catering for the different requirements of each mode. To allow the 6GR design to optimally cater for both idle and connected mode mobility needs, separate RS for IDLE and CONNECTED should be considered for 6G.

In RAN1#122bis, RAN1 had some agreements for 6GR sync signal design and agreed to consider the additional sync signal structure.

RAN1#122bis agreements from “11.1 Overview of 6GR air interface” - High-level aspects to consider for the 6GR sync signal structure include, but not limited to - Sync raster design - Spectrum allocation - smallest maximum supported RF and BB UE BW without spectrum aggregation - mobile broadband service requirements as high priority - Energy efficiency for both BS and UE - Detection/tracking performance, latency, and complexity - Including initial cell search - Coverage target - Common design for diverse device types - Consideration of the supported deployment - Consideration on whether the single sync signal structure is sufficient - Note: Aspects impacting on the periodicity is to be discussed under AI11.5

RAN2 should wait RAN1 progress on the RS design. At the same time, it would be good for RAN2 to start thinking about the procedure impact (if any) based on new RS design.

Observation 1: RAN1 agreed to consider more than one sync signal. RAN1 conclusions on RS signal design may impact RAN2 mobility procedure.

While the detail of RS design is up to RAN1, we provide our assumption on 6G RS usage for different purpose in below Table 1.

Table 1 – RS for different purpose

| Function | RS |
| --- | --- |
| Idle mode Mobility (Cell detection, coarse DL sync) | Cell-level RS NR SSB-like with longer periodicity. |
| Connected mode Mobility (Fine DL sync, TRP detection) | TRP-level RS Including NR TRS functionality and part of NR CSI-RS function. Lower bandwidth / complexity compared to NR CSI-RS. |
| Early CSI acquisition and beam management |  |

From R2 perspective, we could observe that there is independent measurement configuration for L3 HO, LTM, and BM. The independent measurement RS and reporting configuration makes the RRC signaling structure complicate and creates some duplication. It also increases the UE power consumption and implementation complexity to measure multiple RS. To reduce the measurement effort, the same RS should be used for both beam management and CONNECTED mode beam-level mobility. This could also help the network to reduce resource overhead. In 6G, RAN2 should aim to have a better measurement configuration framework that could be used for different purposes (i.e. same configuration structure for both L1 and L3 measurement, for both BM and mobility purpose, and for early CSI acquisition).

Proposal 1: RAN2 should aim to have a unified measurement configuration for the purposes of CONNECTED mode mobility, beam management, and early CSI acquisition. RAN2 assumes that same RS could be used for these purposes.

For 5G NR, the measurement model is defined in 38.300 as below.

Figure 1 – 5G Measurement Model from TS 38.300 Figure 9.2.4-1

The measurement model is suitable for legacy L3 handover defined in Rel-15. However, with the introduction of inter-cell beam management (ICBM) and LTM, the boundary between beam-based and cell-based mobility becomes indistinct. Since the UE ultimately chooses the best beam for data transmission, it is unclear whether a cell-level result is still necessary. In 6G, we expect UE just report the measurement result per RS only.

Since the introduction of LTM, the terminology “L1 measurement” and “L3 measurement” are widely used during the discussion. For the same beam, both L1 and L3 measurement refer to the same RS but with different requirement (i.e., L1 RSRP has a higher synchronization requirement). This is quite confusing and complicated. We prefer not to have this kind of definition in 6G. The requirement of measurement results should not be based on where the UE reports these results.

Observation 2: In NR, the measurement requirement is coupled with the report container (i.e. where to report the measurement result).

Measurement reporting is duplicated today in mobility and BM. In NR, SSB-based RSRP measurement result could be reported 1) in L1 via UCI / PUCCH, 2) in MAC-CE for LTM measurement event, and 3) in legacy RRC measurement report message. Unified measurement report in single place (e.g. MAC-CE or RRC) is preferred in 6G. We recommend a single, unified measurement framework that covers both beam-level and cell-level mobility to reduce SPEC complexity and implementation effort.

Proposal 2: RAN2 should aim to have a unified measurement reporting for the purposes of CONNECTED mode mobility and beam management. FFS the report is in RRC or MAC-CE.

We expect to have a simplified measurement model for 6G as in Figure 2. The UE reports measurement results per RS. The measurement process includes a UE-side filter (similar to L1 filter) and NW-configured filter (similar to L3 filter). The network also configures the triggering condition to send measurement report or the execution condition (like CHO) to trigger different procedure. There is no need to report measurement report for the same RS in different ways (e.g., different report containers in BM/Mobility procedures). The difference on how well the UE has synchronized to the RS can be reported explicitly. E.g., sync status indication can be included in the measurement report.

Figure 2 – 6G Measurement Model

## 2.2 Measurement gap

Measurement gap is considered one of the main topics for RAN4, but it also closely related to many RAN2 main topics, for example, measurement framework and mobility. Moreover, in most cases, new measurement gap features still require corresponding designs from RAN2, such as related procedures and signalling design. In 6G, it is expected that many new measurement gap features will be proposed. RAN2 does not necessarily have to wait for RAN4's proposals to fully mature before starting to make progress. RAN2 can start from the design of signalling and procedures, while RAN4 can focus on defining the measurement gap (MG) periodicity, length, pattern, and corresponding requirements.

Proposal 3: For 6G measurement gap, RAN2 can start from the design of procedure and signalling. RAN4 can determine the MG periodicity, length, pattern, and corresponding requirement.

In traditional measurement procedures, the MG is configured by the network based on predefined patterns. However, the need for MG is highly UE-specific and varies among different UEs, depending on factors such as UE capability, DL/UL assignment, and measurement configuration as shown in Fig. 3. In 5G, MG configuration has become more dynamic and tailored to individual UEs, with features such as Pre-configured Gap (Pre-MG) for BWP switching, Concurrent Gap (Con-MG) for multiple MG configurations, and Network Controlled Small Gap (NCSG), which allows the UE to report whether the interruption is needed.

Figure 3 – Illustration of different MG/interruption requirements for different UEs

One of the pain points of 5G is that the aforementioned MG enhancement was not part of the design plan on 5G day 1. For example, the signalling for NeedforGap was not supported at the beginning of 5G. Most features were added continuously with new demands and new UE capabilities, leading to high complexity in supporting various different features. Furthermore, despite above 5G improvements, it remains challenging for the network to provide tailor-made configurations for each UE using predefined patterns. Collecting complete information from the UE to identify its exact MG requirements is difficult, as some details are related to proprietary designs, such as UE’s RF design.

Observation 3: One of the pain points of 5G is that multiple additional MG-related features, e.g., NeedforGap Pre-MG, Con-MG, NCSG, was not included in 5G Day-1, which increase the complexity to support different features. A simple and flexible MG signaling procedure should be supported in 6G Day-1.

In 6G, we propose that the UE directly report its MG/interruption requirements to NW, as illustrated in Fig. 4. Once UE receives the necessary configuration, it can determine the MG/interruption needs based on its current capability, e.g., any redundant RF chain, current DL/UL assignment, and measurement task. The UE can then report its minimal MG/interruption needs to the NW. This report can be aperiodic, periodic or event-triggered according to NW configuration, or it can be autonomously triggered by UE if its status changes. After receiving the report, NW could decide whether to apply the requested MG pattern and send the response to UE. Once the pattern is accepted, UE can apply it and NW will re-schedule UL/DL based on the new MG pattern if need.

Figure 4 – Illustration of UE-request MG procedure.

Since the UE has comprehensive knowledge of its own MG/interruption requirements, this approach allows NW to dynamically configure tailor-made MG for different UEs. Configuring minimal MG/interruption to each UE can reduce unnecessary interruptions and enhance overall system performance, benefiting both NW and UE. This UE-request MG mechanism can also be easily extended to accommodate 6G new UE features and capabilities, such as AI-based measurement gap reduction and various RF capabilities, if the report signalling, e.g., pattern format, is well-design.

Proposal 4: RAN2 aim to design a measurement gap signaling framework that allows UE to request measurement gap dynamically based on current UE configuration and status.

## 2.3 IDLE mode Mobility procedure

5G idle mode mobility ensures robust and simple performance. The S-criteria is used to select valid cell and the ranking orders the valid cell by signaling quality. The reselection priority decides the frequency or RAT the UE should camp on. These mechanisms keep the UE reachable via paging and ready to initiate connection, while supporting UE-autonomous mobility without network awareness and minimizing power consumption through lightweight RRM measurements. 6G should retain this design to maintain a simple, efficient idle mobility foundation.

To ensure a smooth evolution to 6G, it is essential to maintain the principle of minimized UE capability in Idle state, as gNB does not know the UE's capabilities or its precise location. Any additional feature in Idle should be well-designed so that UEs not supporting it can still operate effectively. For example, for low power Radio (LR) design, RAN2 should discuss whether it is mandatory for all UE(s). If not, what’s the fallback UE behavior if not supporting LR.

Proposal 5: For 6GR idle mode mobility design:

- RAN2 should reuse 5G mechanisms/procedures (e.g. S-criteria, ranking, and reselection procedure, reselection priority) as baseline to enable long-DRX, UE-based mobility in idle state.

- Design features based on minimized UE capabilities.

- Analyze the impact of low power radio (LR) design in IDLE mode.

## CONNECTED mode Mobility procedure

The concept of a generic/unified mobility procedure has been widely proposed and discussed in last RAN2 meeting. Most companies believe that 6G should have a generic mobility procedure to cope with various scenarios, also to avoid redundant configurations or additional network/UE procedures.

In additional to “unification”, we think forward compatibility is an important aspect in mobility design. The lack of sufficient forward compatibility in 5G NR has led to duplicate functions in subsequent mobility features and also resulted in incompatibility between different mobility procedures. For example, the co-existence of CHO and LTM needs to be discussed, rather than being naturally integrated.

To ensure forward compatibility, the mobility framework should be modularized with different steps. RAN2 should not create a new procedure every time we want to improve some part of mobility functionality. Instead, enhancement on different “mobility module” could be considered. At the configuration level, extensions should be added as interfaces so new features could plug in without redundant configuration (e.g., avoid splitting LTM-CSI-ResourceConfig vs. CSI-ResourceConfig as NR did).

Observation 4: To support forward compatibility, mobility procedure should be modularized.

Taking NR L3 HO, CHO, LTM, C-LTM into consideration, we believe that 6G mobility could include below steps:

- Mobility Pre-configuration

  - In NR, the candidate pre-configuration was introduced in Rel-16 CHO, Rel-18 LTM and Rel-19 CLTM. The main purposes of introducing candidate pre-configuration are twofold: to support conditional mobility (CHO, CLTM) and fast switching of LTM. For a 6G generic mobility procedure, to support fast switching and conditional mobility, candidate configuration should be supported.

  - For conditional mobility, candidate configuration must be paired with mobility-triggering conditions. These conditions define triggers on candidates to enable UE-initiated mobility without explicit network indication.

- Mobility Pre-processing

  - To improve mobility performance, NR mobility procedure may optionally include the following steps:  early DL/UL synchronization, early UE configuration processing, and early CSI acquisition. These mechanisms primarily aim to reduce the interruption and throughput degradation during switching. However, it's impractical to perform all these preparations for every candidate. Therefore, the implementation of pre-processing also requires further interaction with the network to narrow down the range of target.

  - If conditional mobility is used, the event evaluation for mobility triggering is performed.

- Mobility Triggering

  - In NR, mobility triggering can be initiated by the network or by the UE. Conditional mobility has demonstrated reliability gains and more UE flexibility. These benefits could be further amplified in 6G scenarios (e.g., direction/distance/location info in NTN scenario for mobility triggering). In last meeting, RAN2 already agreed to study conditional mobility and considering mobility robustness as design target. We believe that conditional mobility should also be supported in 6G Day-1 as a complement to NW-triggered mobility.

- Mobility execution

  - In general, this refers to the UE’s detachment from the source and attachment to the target. The baseline should consist of the minimal set of actions (such as L1/L2 reconfiguration), while optional components can be added to meet higher performance requirement (e.g., early CSI) or omitted if they were already completed during preparation (e.g., Early DL/UL sync, early UE configuration processing).

Proposal 6: RAN2 to study a modularized 6G mobility procedure consists of the following steps:

- Pre-configuration: candidate configurations and triggering conditions.

- Pre-processing: early DL/UL synchronization, early UE configuration processing, early CSI acquisition, and triggering event evaluation.

- Triggering: Both network-initiated and UE-initiated triggering are supported.

- Execution: Release source configuration and apply target configuration. (Additional processing is needed if not yet done in pre-processing step)

## 2.5 Intelligent mobility framework

With the increasing complexity of 6G scenarios and the challenges of coverage and throughput mismatches, the legacy rule-based handover procedures may no longer be suitable in all cases. While pre-configured mobility parameters (such as TTT and A3 offset) can optimize the average performance for all UEs within a cell, they may not be optimal for each individual UE or for every handover decision. Instead, it would be beneficial for certain mobility parameters to be dynamically adjusted by the UE based on its real-time link conditions to enhance the user experience.

AI is one of promising approaches for further enhancing 6G mobility. AI-based prediction can help UEs dynamically determine optimal parameters, or even decide whether or not to trigger a handover procedure. In NR AI Mobility, 3GPP proposes using AI to predict RRM measurements and the timing of measurement events defined by fixed configurations. We agree that standardizing AI-based mobility enhancements in 3GPP would be valuable. However, it is also important to allow for AI native mobility solutions or smart UE implementations (for example, to adjust parameters as described above) to provide greater flexibility and enable faster, more practical deployment.

To support this vision, some modifications to the specifications will be necessary. For example, allowing a range of acceptable mobility parameter values instead of fixed values, and introducing signaling for UE recommendations, etc. In 6G, we aim to design an extendable/flexible connected mode mobility procedure that allow to trigger (or not trigger) handover with new 6G features, including AI enhancement, such that the mobility can better reflect the real-time link conditions and improve the user experience.

Proposal 7: 6G mobility procedure should allow to trigger (or not trigger) connected mode mobility when necessary, from link condition (for data transmission) point of view.

# 3 Conclusions

Based on the discussion in section 2, we have the following observations and proposals:

Measurement

Observation 1: RAN1 agreed to consider more than one sync signal. RAN1 conclusions on RS signal design may impact RAN2 mobility procedure.

Proposal 1: RAN2 should aim to have a unified measurement configuration for the purposes of CONNECTED mode mobility, beam management, and early CSI acquisition. RAN2 assumes that same RS could be used for these purposes.

Observation 2: In NR, the measurement requirement is coupled with the report container (i.e. where to report the measurement result).

Proposal 2: RAN2 should aim to have a unified measurement reporting for the purposes of CONNECTED mode mobility and beam management. FFS the report is in RRC or MAC-CE.

Measurement Gap

Proposal 3: For 6G measurement gap, RAN2 can start from the design of procedure and signalling. RAN4 can determine the MG periodicity, length, pattern, and corresponding requirement.

Observation 3: One of the pain points of 5G is that multiple additional MG-related features, e.g., NeedforGap Pre-MG, Con-MG, NCSG, was not included in 5G Day-1, which increase the complexity to support different features. A simple and flexible MG signaling procedure should be supported in 6G Day-1.

Proposal 4: RAN2 aim to design a measurement gap signaling framework that allows UE to request measurement gap dynamically based on current UE configuration and status.

IDLE

Proposal 5: For 6GR idle mode mobility design:

- RAN2 should reuse 5G mechanisms/procedures (e.g. S-criteria, ranking, and reselection procedure, reselection priority) as baseline to enable long-DRX, UE-based mobility in idle state.

- Design features based on minimized UE capabilities.

- Analyze the impact of low power radio (LR) design in IDLE mode.

CONNECTED

Observation 4: To support forward compatibility, mobility procedure should be modularized.

Proposal 6: RAN2 to study a modularized 6G mobility procedure consists of the following steps:

- Pre-configuration: candidate configurations and triggering conditions.

- Pre-processing: early DL/UL synchronization, early UE configuration processing, early CSI acquisition, and triggering event evaluation.

- Triggering: Both network-initiated and UE-initiated triggering are supported.

- Execution: Release source configuration and apply target configuration. (Additional processing is needed if not yet done in pre-processing step)

Intelligent Mobility

Proposal 7: 6G mobility procedure should allow to trigger (or not trigger) connected mode mobility when necessary, from link condition (for data transmission) point of view.

# 4 References

[1] RP-251881:	“3GPP Work Item Description - Study on 6G Radio”, NTT DOCOMO, RAN#108