3GPP TSG-RAN WG2 Meeting #132							R2-2508994

Dallas, United States, Nov. 17th-21st

Agenda item:		8.4.2

Title: 	Consideration on LP-WUS RRC Issues

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

Introduction

In this contribution, we will discuss some remaining issues on LP-WUS RRC Issues and present our views.

Discussion

It has been agreed that the LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message, while activation/deactivation of LP-WUS monitoring via L1/L2 signaling is not supported.

But how gNB decide to activate/deactivate LP-WUS for UE in RRC CONNECTED has not been discussed yet. Generally, it is based on gNB implementation, e.g. based on periodic CSI report, but the periodic CSI report is based on MR measurement, which cannot be used to provide an accurate evaluation of LR quality.

Observation 1: gNB cannot know the LR quality accurately for UE in RRC CONNECTED state to decide whether and when to activate or deactivate the LP-WUS.

Considering that the coverage of LP-WUS may be smaller than or equal to the cell coverage, even the LP-WUS related configuration is provided, UE may not be able to monitor the LP-WUS when the UE is close to the cell edge.

Per RAN2 agreement, if network configure the LP-WUS with RRC message, the LP-WUS will be activated. UE need to monitor LP-WUS only when it is configured by network.

To allow reasonable decision from network side on whether and when to configure the LP-WUS for RRC_CONNECTED state UE, LP-WUS monitoring entry/exit condition (e.g. (LP-)RSRP and optional (LP-)RSRQ threshold) should be configured to UE (e.g. by RRCReconfiguration message), and UE can report LP-WUS activation/deactivation assistance information, e.g. “condition met” to network via UEAssistanceInformation message.

Proposal 1: LP-WUS monitoring entry/exit condition (e.g. (LP-) RSRP and optional (LP-) RSRQ threshold) should be defined and configured to UE in RRC_CONNECTED.

Proposal 2: UE reports LP-WUS entry/exit condition met indication to network via UEAssistanceInformation message to assist network decision on whether and when to activate and deactivate LP-WUS monitoring for UE in RRC_CONNECTED.

- Conclusion

Based on the discussion we propose the following:

Observation 1: gNB cannot know the LR quality accurately for UE in RRC CONNECTED state to decide whether and when to activate or deactivate the LP-WUS.

Proposal 1: LP-WUS monitoring entry/exit condition (e.g. (LP-) RSRP and optional (LP-) RSRQ threshold) should be defined and configured to UE in RRC_CONNECTED.

Proposal 2: UE reports LP-WUS entry/exit condition met indication to network via UEAssistanceInformation message to assist network decision on whether and when to activate and deactivate LP-WUS monitoring for UE in RRC_CONNECTED.

- Reference

- RP-243293, Revised WID on introduction of IoT-NTN TDD mode

- Draft_RAN2_131bis_MeetingReport_v1