3GPP TSG-RAN WG2#132	R2-2509046

Dallas, TX, USA, 17-21 November 2025

Agenda Item:	10.2.3

Source: 	MediaTek Inc.

Title:  	Initial RAN2 steps on scalability, extensibility and future proofness in view of diverse device types

Document for:	Discussion, decision

# 1	Introduction

Device types are under discussion at the plenary level with RAN#109 / Sep 2025 outcome indicated hereafter, in view of [2] (source: [3]):

“conclusion: proposal 1 is endorsed, proposal 3 & 4 are endorsed for RAN only (no WG discussion)”

Where proposal 1 relates to RAN1, i.e., minimum spectrum allocation and smallest maximum UE BW, and proposals 3 and 4 are duplicated hereafter:

Proposal 3: To investigate further: - Motivations/justifications behind the proposed diverse device types, which should be a limited set - Whether/how to have one or more device types for eMBB or 6G IoT - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc. - Whether/how to explicitly standardize device types - Ensuring forward compatibility - Minimizing/avoiding potential market fragmentation Note: the terminology “device type” is subject to further discussion and possible refinement. Proposal 4: In terms of diverse device types study further: - Possible parameters/factors, e.g.: Number of Tx antennas/chains, Number of Rx antennas/chains, Power classes, Maximum UE bandwidth (DL/UL), Peak data rate (DL/UL), Maximum MIMO layers (DL/UL), Duplex mode, Max modulation order (DL/UL), CA/spectrum aggregation (DL/UL), UE processing capabilities, Coverage, Energy efficiency, Mobility/speed, Sensing, AI Note: some of the above parameters/factors may be related with form factor Note: aim to have a focused/limited set of parameters/factors for a device type - The value(s) for the identified parameters for a device type

This contribution, while acknowledging the above conclusions and the consequent constraints on WG discussion, also intends to progress the related discussions in RAN2 in view of RAN2 agenda regarding: “guidelines and views on scalability, extensibility and future proofness including support of diverse device types (e.g., categories) and how this would affect our design approach for L2 and L3 protocols.”, i.e., considerations on scalability, extensibility and future proofness (forward compatibility) in view of diverse types of devices that will arise throughout the 6GR lifetime. It does not dwell on Proposals 3 and 4 above.

# 2	Discussion

## 2.1	Generalities

A first exercise consists in identifying a common minimum functional capability (“nucleus”) for all devices; this is a fundamental aspect of 6GR design in order to avoid the “expanding down” problems that came with defining lower-capability devices in later releases of past systems. This means that no device (or network) will be able to feature (and signal) lower capabilities than the above throughout the 6GR lifetime.

The common minimum functionality should ensure all 6G devices (initial and future) are able to obtain basic connectivity from any 6G network, from a functional compatibility perspective. This should include support for 6G native features for wide area coverage, energy saving and MRSS.

Proposal 1: RAN2 6GR design assumes that all 6GR devices will support a common minimum functionality allowing them to obtain basic connectivity from any 6G network, from a functional capability perspective.

Parameters and factors affecting the device design envelope are under discussion at RAN plenary, including those defining the common minimum. It is therefore expected that scalability, extensibility and future proofness be lower-bounded by this common minimum.

Proposal 2: RAN2 will align with plenary decisions regarding a common minimum functional capability and related parameters and factors affecting the device design envelope.  Scalability, extensibility, and future-proofness will be lower-bounded by this common minimum.

It should, however, also be taken into account that the 6GR Study targets the following:

i)	Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs

A sound interpretation thereof is that “eMBB” is the prime target from which scaling up or down will be possible as illustrated in figure 2.1-1.

Figure 2.1-1: Native scalability

## 2.2	Scope of spec impact

Irrespective of whether or not actual “device types” will be specified, there will be a variety of scenarios requiring different idle/inactive behaviors as well as different connected behaviors; for instance, for some scenarios such as low-end IoT, very long DRX cycles will be beneficial that would be unrealistic for e.g. eMBB scenarios.  Depending on how UE contexts are managed in the network, features like DRX that affect idle/inactive UEs may require CN-RAN coordination in connection with the capability.

In the transition to RRC_CONNECTED, it could be considered necessary for the BS to learn key characteristics to support differential handling in the connection process.  However, this outcome seems undesirable; it complicates the access procedure, and it could be difficult to scale for the introduction of future devices.  Ideally, 6G should have a unified access/connection procedure, with dependencies on different device types introduced only if they become clearly necessary.  Note that some access-related functions might be expected to be correlated with device types (e.g., SDT would be useful for IoT devices with intermittent traffic) but not restricted to them (SDT could also be used by a smartphone for keep-alive traffic).

Proposal 3: Device differentiation at connection time should be introduced only if clearly needed, and if so, with minimum spec impact.

Depending on the need to group devices into some sort of “types”, (pending RAN plenary outcome), it may be possible to do so according to a profile of UE capability parameters.  However, this approach may be too flexible and lead to over-categorization.  We can expect plenary action on this subject; in the meantime, RAN2 should consider the signalling impact of such a grouping (e.g., to UE capabilities) in an agnostic way.  However, in view of the plenary status, it may not be possible for RAN2 to move forward on the signalling aspects yet.

Proposal 4: Alternative ways of capturing a hypothetical “device type” concept in UE capability signalling are not constrained from RAN2 perspective.  Await RAN plenary guidance before progressing this aspect of the device discussion.

The capability framework itself should not depend on device types.

Proposal 5: 6G has a unified capability framework that does not vary according to any potential “device type” concept.

# 3	Conclusion

This document promulgated the following proposals:

Proposal 1: RAN2 6GR design assumes that all 6GR devices will support a common minimum functionality allowing them to obtain basic connectivity from any 6G network, from a functional capability perspective.

Proposal 2: RAN2 will align with plenary decisions regarding a common minimum functional capability and related parameters and factors affecting the device design envelope.  Scalability, extensibility, and future-proofness will be lower-bounded by this common minimum.

Proposal 3: Device differentiation at connection time should be introduced only if clearly needed, and if so, with minimum spec impact.

Proposal 4: Alternative ways of capturing a hypothetical “device type” concept in UE capability signalling are not constrained from RAN2 perspective.  Await RAN plenary guidance before progressing this aspect of the device discussion.

Proposal 5: 6G has a unified capability framework that does not vary according to any potential “device type” concept.

# 4	References

[1]	RP-252518, “Scalability for Diverse device types”, MediaTek Inc., RAN#109, September 2025

[2]	RP-252873, “Summary of 6G Diverse Device Types (including Minimum UE Bandwidth)”, Apple Inc. (Moderator), RAN#109, September 2025

[3]	RP-252971, RAN#109 meeting report