3GPP TSG-RAN WG2 Meeting #132	R2-2509101

# Dallas, USA, November 17-21, 2025

Source:			Skylo Technologies, Lockheed Martin, EchoStar, Sateliot, ViaSat, ESA, Aalyria

Title:			Discussion on Rel-20 IoT NTN Ph4 Scope Update

Agenda item:	9.7.2

Document for:	Discussion

# Introduction

- Satellite Switch with Resynchronization support for IoT NTN

R2-2505933	TEI19] Introduction of Satellite Switch with Resynchronization support for IoT NTN	Skylo Technologies, Lockheed Martin, EchoStar, Sateliot	discussion	Rel-19

R2-2505934	SatSwitchwithReSync support for NB-IoT NTN	Skylo Technologies, Lockheed Martin, EchoStar, Sateliot	CR	Rel-19	36.300	18.5.0	1429	-	B	IoT_NTN_Ph3-Core

R2-2505938	SatSwitchwithReSync support for IoT NTN	Skylo Technologies, Lockheed Martin, EchoStar, Sateliot	CR	Rel-19	36.321	18.4.0	1594	-	B	IoT_NTN_Ph3-Core

R2-2505939	SatSwitchwithReSync support for IoT NTN	Skylo Technologies, Lockheed Martin, EchoStar, Sateliot	CR	Rel-19	36.331	18.6.0	5149	-	B	IoT_NTN_Ph3-Core

All contributions to be discussed first in offline 301

-	Viasat, Echostar, ESA, Aalyria also support this proposal

-	HW also see some benefits to support this

-	Xiaomi thinks that also for hard switch there is RAN4 impact

-	Aalyria thinks hard switch can also be supported without RAN4 work

-	MTK thinks this is the last meeting for Rel-19 and it’s the first time we see this proposal. We could consider it for the next release

-	Ericsson thinks NB-IoT does not support connected mode mobility and they would like this to stay like that. Also this would be needed for connections that last for a long time and this is not the case for typical IoT scenarios

-	Session chair thinks this could also be considered as an update for the R20 IoT NTN WID

RAN2 thinks this could be further considered in Rel-20

During RAN2#131, a TEI19 item addressing Satellite Switch with Resynchronization support for IoT-NTN was proposed in R2-2505933[1]. Following the feedback documented in R2-2506203 [2] (as reproduced below for reading convenience), RAN2 recommended that the TEI19 proposal in R2-2505933 [1] could be further considered in Rel-20. And the Session chair thinks this could be considered as an update for the Rel-20 IoT NTN WID.

Accordingly, we have prepared the contribution for RAN2#132 to discuss the RAN2 Rel-20 IoT NTN WID update in this document.

# Discussion

In an NGSO constellation, the satellites are constantly moving across the sky relative to the user on the ground. For a call to last longer than the serving satellite's visibility (which can be short, depending on the orbit altitude), the device must transition to an upcoming target satellite. Without an efficient switch procedure, the call will drop at every satellite change.

It is thus proposed to update the WID to study a resource-efficient quick switchover to avoid a time-consuming and power-intensive process cell reselection procedure, especially over high-latency satellite links. This seamless switch is important for maintaining the QoS required for voice, even low-bitrate Voice over NB-IoT-NTN.

Specified in TS 38.331, TS 38.321 and TS 38.300, the satellite switch with re-synchronization procedure is currently support for NR-NTN. A UE capable of hard satellite switch with resynchronization in RRC_CONNECTED initiates the procedure when SatSwitchWithReSync and t-Service are included in SIB19. The procedure is executed by the UE in the RRC_CONNECTED state based on network-provided information in SIB19, which contains parameters for a target satellite required (e.g., t-ServiceStart) to perform satellite switch with resynchronization. One of the essential requirements of this satellite switch with resynchronization is that a currently serving cell (i.e., operated by a source satellite) and a target cell (i.e., operated by a target satellite) use the same PCI for their respective cells targeting the same geographical area. Accordingly, with a help of some essential parameters that the source satellite provides in its SIB19, namely ssb-timeoffset and t-ServiceStart pertaining to the target (cell) satellite, UEs know when to start the synchronization process. For (quasi-)earth-fixed deployments in NB-IoT NTN, the rapid movement of satellites across the sky can benefit from a similar procedure in order to facilitate seamless mobility between different satellites. The support of a "satellite switch with re-synchronization" procedure is preferred. This procedure enables a UE in the RRC_CONNECTED state to transition from a serving satellite to a new target satellite seamlessly without having to perform a blind search and synchronization with a new satellite on the horizon. In addition, such a switchover can avoid L3-based handover procedure that is not well supported by NB-IoT.

For NB-IoT, a new or modified SIB would be used to broadcast the satSwitchwithReSync parameters, which contain orbital parameters and configurations for upcoming target satellites. A UE in the RRC_CONNECTED state reads and processes this SIB. Based on its internal clock, position, and the provided satellite orbital parameters, the UE's RRC layer can pre-determine the precise time when it needs to switch to a new satellite.

At the pre-determined time, the UE initiates the switch. It applies the resynchronization parameters from the SIB to adjust its timing and frequency, enabling it to quickly re-synchronize with the new satellite. Since the PCI is the same, the physical layer transition is simplified.

Observation 1: The Satellite Switch with Re-synchronization procedure can and should be supported for NB-IoT NTN to support the voice call over NB-IoT-NTN currently studied in RAN2 WID IoT_NTN_Ph4.

# Proposal

Based on the preceding discussion, we propose to update WID in RP-252473 [3] to include the scope of studying the support of satellite switch with resynchronization for NB-IoT-NTN as outlined in [4].

Proposal 1: It is proposed to update the IoT NTN Ph4 WID to Support of Satellite Switch with Resynchronization for NB-IoT-NTN (hard switch only).

# Reference

[1] R2-2505933, [TEI19] Introduction of Satellite Switch with Resynchronization support for IoT NTN, RAN2#131.

[2] R2-2506203, Report from session on NR NTN and IoT NTN, RAN2#131.

[3] RP-252473, New WI: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4, RP#109.

[4] R2-250xxxx, Rel-20 New WID Scope Update: NTN for IoT Ph4, RAN2#132