3GPP TSG-RAN WG2 Meeting #133	R2-2601133

Goteborg, Sweden, Feb 9th – 13th, 2026

Agenda item:	11.3

Source: 	Session Chair (ZTE Corporation)

Title:	Report from Break-out session on NR-NTN and IoT-NTN

Document for:	Approval

Organizational

- All organization emails and notes will be shared over the following email discussion throughout the meeting:

[AT133][300] Organizational – NR-NTN and IoT-NTN session

Scope:

    - Share plans for the meeting and list of ongoing email discussions for the sessions related to NR-NTN and IoT-NTN

    - Share meetings notes and agreements for review and endorsement

Schedule/Plan

|  | Main room | Brk 1 room | Brk 2 room | Brk 3 room* |
| --- | --- | --- | --- | --- |
| Monday |  |  |  |  |
| 09:00 – 10:40 | [1], [2], [3], [7.0] R18 common (Diana) [7.0.1] UE capabilities [8.0] NR19 common Break out [8.19] TEI19 corrections [10.1] 6GR organizational [10.2.1] 6G UE capabilities | Breakout to start after completion of 7.0.1 and common ASN.1 1 discussion EUTRA&NR15161718 (Mattias) Except NTN/Positioning - start with R16 1T-1T tx switching and other capabilities maintenance until 6GR capability discussion starts in main to avoid collision. - Other maintenance in order | Breakout to start after completion of 7.0.1 and ASN.1 discussion Rel-19 corrections (Erlin) [8.4] NR LPWUS |  |
| 11:00-11:40 | Carolyn remembrance gathering |  |  |  |
| 11:40 – 13:00 | [10.1] 6GR organizational con’t [10.2.1] 6G UE capabilities @12:15 [10.3.2] 6GR Control Plane [10.3.2.2] RRC structure and (re) configuration - start with email discussions | EUTRA&NR15161718 (Mattias) con’t [8.10] NR19 SONMDT [0] (Mattias) | [8.5] NR19 NES (Kyeongin) [8.5.1], [8.5.2] |  |
| 14:30 -16:30 | [10.3.2] 6GR Control Plane [10.3.2.2] RRC structure and (re) configuration - start with email discussions [10.3.2.1] RRC modeling | [7.0.2.16] R18 XR (Dawid) [8.7] NR19 XR [0] (Dawid) | Rel-19 corrections (Erlin) [8.12] NR MIMO @14:30-15:30 [8.11] NR SBFD @15:30-16:30 |  |
| 17:00– 19:00 | [10.2] 6GR General [10.2.3] Others Security and other aspects | [8.9] R19 IoT NTN [0] Sergio [8.9.1] [8.9.2] [8.9.3] | [8.20] NR others (Erlin) [8.20.1] [8.20.2] |  |
| Tuesday |  |  |  |  |
| 08:30 – 10:30 | [10.3.1] User Plane [10.3.1.0] Mobile AI transmission charact. [10.3.1.2] QoS, QoE | [7.0.2.22] NR18 Mob (Kyeongin) [8.6] NR19 Mob (Kyeongin) [8.6.1], [8.6.2] | NR19 SL Relay (Nathan) NR18 Pos (Nathan) |  |
| 11:00 – 13:00 | [10.3.1] User Plane [10.3.1.2] QoS, QoE con’t [10.3.1.1] Functionalities of UP | [9.3] NR20 AI mobility [0.5] (Kyeongin) [9.3.1] Organizational [9.3.2] RRM measurement prediction | EUTRA&NR15161718 (Mattias) con’t [8.10] NR19 SONMDT [0] (Mattias) con’t |  |
| 14:30 -16:30 | [10.3.3] Common CP/UP [10.3.3.2] AI use cases | [8.2] NR19 Ambient IoT [0] (Nathan) [9.2] NR20 AIoT [9.2.1 Organizational [9.2.2] T2 selected topics TBD depending on time | R17/18 NR / IoT NTN (Sergio) [4.1] R17 IoT NTN corrections [6.1.3] R17 NR NTN corrections [8.8] NR19 NR NTN (Sergio) [0] [8.8.1], [8.8.2] |  |
| 17:00– 19:00 | [10.3.2] 6GR Control Plane [10.3.2.3] System information [10.3.2.4] Paging | [8.1] NR19 AI/ML PHY [0] (Erlin) @17:00-18:00 [9.1] NR20 AI/M PHY [1] (Erlin) @18:00-19:00 [9.1.1] [9.1.2] | [9.7] R20 IoT NTN [0.5] [9.7.1], [9.7.2] |  |
| Wednesday |  |  |  |  |
| 08:30 – 10:30 | [10.3.1] 6GR User Plane [10.3.1.1] Functionalities of UP con’t [10.3.1.4] Others |  | [9.2] NR20 AIoT [1] (Nathan) [9.2.2] remaining topics |  |
| 11:00 – 13:00 | [10.3.1] 6GR User Plane [10.3.1.3] Scheduling | [9.1] NR20 AI/M PHY [1] (Erlin) [9.1.2] cont. [9.1.3] | Reserved Offline slot for CP offlines (time TBD) |  |
| 14:30 -16:30 | [10.3.3] Common UP/CP [10.3.3.3] Energy efficiency | [9.4] NR20 Mobility [0.5] (Kyeongin) [9.4.1] Organizational [9.4.2] LTM SCell activation improvement [9.4.3] Dynamic L1 measurement and reporting configuration change | Reserved offline slot for AI/ML use cases if needed (time TBD) |  |
| 17:00 – 19:00 | [10.4] 6GR Mobility | [8.17] R19 IoT NTN TDD mode [0] [8.19] TEI19 (NTN related aspects) [8.20.1] NR Others (RAN4) (NTN related aspects) |  |  |
| Thursday |  |  |  |  |
| Colourful Polo day |  |  |  |  |
| 08:30 – 10:30 | [10.3.2] 6GR Control Plane [10.3.2.4] Paging cont’ [10.3.2.3] Others | CBs (Erlin) Details TBD for the following CBs [8.12] NR MIMO [8.11] NR SBFD [8.4] NR LPWUS | CB Mattias CB EUTRA&NR15161718 (Mattias) [8.10] SON/MDT R19 |  |
| 11:00 – 13:00 | [10.3.2] 6GR Control Plane [CB] - TBD | CB Erlin (~1hr) @11:00-12:00 [8.1] NR19 AI/ML PHY [0] (Erlin) Details TBD for CB | Reserved Offline slot for UP offlines if needed (time TBD) |  |
| 14:30 -16:30 | @11-12:00 [10.4] 6GR Mobility NTN mobility related aspects [10.3.3] Common CP/UP [10.3.3.2] AI use cases cont’ CB | CB Dawid XR CB Erlin @15:30-16:30[8.20] NR others (Erlin) Details TBD for CB | CB Nathan [7.0.2.21] NR18 Positioning [8.13] NR19 SL relay [8.2] NR19 AIoT [9.2] NR20 AIoT (if any offline is allocated) |  |
| 17:00 – 19:00 | [10.3.3] Common CP/UP [10.3.3.1] Data transfer framework | CB Kyeongin Comebacks [7.0.2.22], [8.5], [8.6] | CB Sergio [8.9] R19 IoT NTN [8.9.1] report of 301 [8.9.2] report of 302, 303, 304 |  |
| Friday |  |  |  |  |
| 08:30 – 10:30 | CB Diana TBD [10.3.1] 6GR User Plane CB – TBD | CB Kyeongin @8:30-9:30 AI Mobility Comebacks [9.3.2] [9.3.3] RRM measurement event prediction CB Erilin @9:30 [9.1] NR20 AI/M PHY [1] (Erlin) Details TBD for CB | CB Sergio NTN |  |
| 11:00 – 13:00 | [10.3.1] 6GR Control Plane CB – TBD Reports from breakout sessions EoM |  |  |  |
| 14:30 – 16:00 |  |  |  |  |
| 16:00 – 17:00 |  |  |  |  |

* Offline discussions should be well scoped and only 30mins in duration.

Breaks

Morning coffee: 		10:30 to 11:00

Lunch: 			13:00 to 14:30

Afternoon coffee:	16:30 to 17:00

List and details of [AT133] offline discussions

NOTE:  No offline email discussions will be kicked off before Monday Feb 9th, 09:00 local time

[AT133][301][R19 IoT NTN] PWS Support indication for acceptable cell (Samsung)

Scope: discuss a TP introducing a new indicator in SIB1-NB to indicate whether the NB-IoT cell supports PWS broadcasting and specifying that A PWS-capable UE may camp on an acceptable NB-IoT cell for the purpose of receiving PWS if the NB-IoT cell supports PWS broadcasting

Intended outcome: Agreeable TP

Deadline for companies' feedback:  Thursday 2026-02-12 14:30

Deadline for rapporteur's summary (in R2-2601205):  Thursday 2026-02-12 16:30

[AT133][302][R19 IoT NTN] AS RAI reporting (Google)

Scope: discuss AS RAI reporting for NB-IoT

Intended outcome: summary of the offline discussion

Deadline for companies' feedback:  Thursday 2026-02-12 14:30

Deadline for rapporteur's summary (in R2-2601206):  Thursday 2026-02-12 16:30

[AT133][303][R19 IoT NTN] S&F cell suitability (Sateliot)

Scope: continue the discussion on R2-2600733

Intended outcome: summary of the offline discussion

Deadline for companies' feedback:  Thursday 2026-02-12 14:30

Deadline for rapporteur's summary (in R2-2601207):  Thursday 2026-02-12 16:30

[AT133][304][R19 IoT NTN] Cell measurement and reselection based on AS layer S&F mode info (Google)

Scope: continue the discussion on R2-2600387

Intended outcome: summary of the offline discussion

Deadline for companies' feedback:  Thursday 2026-02-12 14:30

Deadline for rapporteur's summary (in R2-2601208):  Thursday 2026-02-12 16:30

[AT133][305][R19 IoT NTN] MAC CR (Mediatek)

Scope: Draft a MAC CR with meeting agreements so far

Intended outcome: Agreeable MAC CR

Deadline for revised CR (in R2-2600200):  Thursday 2026-02-12 16:30

[AT133][306][R19 IoT NTN] RRC CR (Huawei)

Scope: Draft a RRC CR with meeting agreements so far

Intended outcome: Agreeable RRC CR

Deadline for revised CR (in R2-2601201):  Thursday 2026-02-12 16:30

## 4.1	EUTRA corrections Rel-19 and earlier

(LTE_NBIOT_eMTC_NTN; leading WG: RAN1; REL-17; WID: RP-211601)

R2-2600172	Corrections on EphemerisOrbitalParameters in IoT NTN	THALES, Apple, Iridium, Eutelsat, Airbus, ESA	CR	Rel-17	36.331	17.15.0	5187	-	F	LTE_NBIOT_eMTC_NTN

Revised in R2-2601117

R2-2601117	Corrections on EphemerisOrbitalParameters in IoT NTN	THALES, Apple, Iridium, Eutelsat, Airbus, ESA, Ericsson	CR	Rel-17	36.331	17.15.0	5187	1	F	LTE_NBIOT_eMTC_NTN

R2-2600173	Corrections on EphemerisOrbitalParameters in IoT NTN	THALES, Apple, Iridium, Eutelsat, Airbus, ESA	CR	Rel-18	36.331	18.8.0	5188	-	A	LTE_NBIOT_eMTC_NTN

Revised in R2-2601118

R2-2601118	Corrections on EphemerisOrbitalParameters in IoT NTN	THALES, Apple, Iridium, Eutelsat, Airbus, ESA, Ericsson	CR	Rel-18	36.331	18.8.0	5188	1	A	LTE_NBIOT_eMTC_NTN

R2-2600174	Corrections on EphemerisOrbitalParameters in IoT NTN	THALES, Apple, Iridium, Eutelsat, Airbus, ESA	CR	Rel-19	36.331	19.1.0	5189	-	A	LTE_NBIOT_eMTC_NTN

Revised in R2-2601119

R2-2601119	Corrections on EphemerisOrbitalParameters in IoT NTN	THALES, Apple, Iridium, Eutelsat, Airbus, ESA, Ericsson	CR	Rel-19	36.331	19.1.0	5189	1	A	LTE_NBIOT_eMTC_NTN

## 6.1	Common

(NR_NTN_solutions-Core; leading WG: RAN2; REL-17; WID: RP-211557)

### 6.1.3	Control Plane corrections

#### 6.1.3.1	NR RRC

Corrections to 38331, and related change to other TS if applicable, except UE caps.

R2-2600169	Corrections on EphemerisInfo orbital parameters in NR NTN	THALES, Apple, Iridium, Eutelsat, Airbus, ESA	CR	Rel-17	38.331	17.15.0	5631	-	F	NR_NTN_solutions-Core

Revised in R2-2601114

R2-2601114	Corrections on EphemerisInfo orbital parameters in NR NTN	THALES, Apple, Iridium, Eutelsat, Airbus, ESA, Ericsson	CR	Rel-17	38.331	17.15.0	5631	1	F	NR_NTN_solutions-Core

R2-2600170	Corrections on EphemerisInfo orbital parameters in NR NTN	THALES, Apple, Iridium, Eutelsat, Airbus, ESA	CR	Rel-18	38.331	18.8.0	5632	-	A	NR_NTN_solutions-Core

Revised in R2-2601115

R2-2601115	Corrections on EphemerisInfo orbital parameters in NR NTN	THALES, Apple, Iridium, Eutelsat, Airbus, ESA, Ericsson	CR	Rel-18	38.331	18.8.0	5632	1	A	NR_NTN_solutions-Core

R2-2600171	Corrections on EphemerisInfo orbital parameters in NR NTN	THALES, Apple, Iridium, Eutelsat, Airbus, ESA	CR	Rel-19	38.331	19.1.0	5633	-	A	NR_NTN_solutions-Core

Revised in R2-2601116

R2-2601116	Corrections on EphemerisInfo orbital parameters in NR NTN	THALES, Apple, Iridium, Eutelsat, Airbus, ESA, Ericsson	CR	Rel-19	38.331	19.1.0	5633	1	A	NR_NTN_solutions-Core

R2-2600630	Correction to event D1	Huawei, HiSilicon, CSCN	CR	Rel-17	38.331	17.15.0	5649	-	F	NR_NTN_solutions-Core

R2-2600631	Correction to event D1 and event D2	Huawei, HiSilicon, CSCN	CR	Rel-18	38.331	18.8.0	5650	-	A	NR_NTN_solutions-Core

R2-2600632	Correction to event D1 and event D2	Huawei, HiSilicon, CSCN	CR	Rel-19	38.331	19.1.0	5651	-	A	NR_NTN_solutions-Core

R2-2600633	Discussion on NTN-NTN redirection	Huawei, HiSilicon	discussion	Rel-17	NR_NTN_solutions-Core

#### 6.1.3.2	UE capabilities

UE cap corrections 38306, 38331.

R2-2601087	Clarification on coarse location reporting capability in NTN	Ericsson	CR	Rel-17	38.306	17.14.0	1420	-	F	NR_NTN_solutions-Core

R2-2601088	Clarification on coarse location reporting capability in NTN	Ericsson	CR	Rel-18	38.306	18.8.0	1421	-	A	NR_NTN_enh-Core

R2-2601089	Clarification on coarse location reporting capability in NTN	Ericsson	CR	Rel-19	38.306	19.1.0	1422	-	A	NR_NTN_Ph3-Core

## 7.0	Common

### 7.0.2	Rel-18 corrections

#### 7.0.2.17	NR NTN enhancements

(NR_NTN_enh-Core; leading WG: RAN1; REL-18; WID: RP-232669)

#### 7.0.2.18	IoT NTN enhancements

(IoT_NTN_enh-Core; leading WG: RAN2; REL-18; WID: RP-223519)

## 8.8	NTN for NR Ph3

(NR_NTN_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-251954 )

Time budget: 0 TU

Tdoc Limitation: 1 tdoc

(corrections for LTE_TN_NR_NTN_mob, if any, should be submitted to AI 8.19.1)

### 8.8.1	Organizational

LS, Rapporteur input, including open issues lists, etc.

Rapporteur inputs do not count towards the tdoc limitation.

Incoming LSs

R2-2600007	Reply LS on removal of support of PWS over satellite NG-RAN in Rel-17 and 18 (CP-252246; contact: Qualcomm)	CT	LS in	Rel-17	5GSAT_ARCH-CT	To:RAN	Cc:CT1, SA, SA1, RAN3, RAN2, SA2, CT4, CT6

R2-2600034	Reply to Reply LS on removal of support of PWS over satellite NG-RAN in Rel-17 and 18 (S2-2511155; contact: Samsung)	SA2	LS in	Rel-19	5GSAT_ARCH	To:CT, RAN	Cc:CT1, SA, SA1, RAN3, RAN2, CT4, CT6

R2-2600012	LS reply on OCC for RACH-less HO (R1-2509590; contact: Thales)	RAN1	LS in	Rel-19	NR_NTN_Ph3-Core	To:RAN2

Rapporteur CRs

R2-2601085	Miscellaneous correction for NR NTN Phase 3	Ericsson, Nokia	CR	Rel-19	38.331	19.1.0	5682	-	F	NR_NTN_Ph3-Core

### 8.8.2	Corrections

Corrections to TS 38.300, TS38.304, TS 38.306, TS 38.321 and TS 38.331.

[OCC to RACH-less HO in NTN]

(moved from 8.8.1)

R2-2601086	Applicability of OCC to RACH-less HO in NTN	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

Observation 1	RAN1 has confirmed the configuration of OCC for CG PUSCH and DG PUSCH RACH-less handover is supported without any further enhancement to their specifications.

Observation 2	The target gNB might require information such as received UL power level and carrier frequency offset to find suitable candidates for pairing of UEs for OCC and configure OCC RACH-less handover.

Observation 3	Additional information such as Power Headroom Report, UL received power level and UL Carrier Frequency Offset measurement may be needed in the target gNB to configure OCC RACH-less handover in NTN.

Proposal 1	To fully support RACH-less handover with OCC in NTN, include additional information the handover request RRC message from the source node to the target node which includes power headroom report, UL received power level and/or UL carrier frequency offset measurement.

R2-2600709	Support for OCC RACH-less and other corrections	Samsung	discussion	Rel-19

Proposal 1: Confirm that OCC can be applied for RACH-less handovers based on network implementation, at least for intra-satellite handovers. UE-pairing during OCC RACH-less is up to network implementation.

Proposal 2: RAN2 to document OCC being applicable for RACH-less handovers in Stage 2 as seen in Annex A.

R2-2600391	Discussion on time based measurement and OCC for RACH less HO	Xiaomi	discussion	Rel-19	NR_NTN_Ph3

Proposal 1: The OCC for RACH-less handover in NTN is not supported in Rel-19.

Proposal 2: Clarify that the OCC for RACH-less handover is not supported in stage 2 spec as follows:

[closest reference location report]

R2-2600274	RRC Corrections for Rel-19 NR NTN	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: Change the need code of the field refLocMapList in SIB2/SIB4 to need S.

Proposal 2: Clarify the field description of the refLocOffset in SIB4 that the offset applies to the reference location for which its index is configured via the refLocMapList in SIB4 for the frequency.

Proposal 3: Remove the reference clause 5.7.4 for the closest reference location report within the RRC Resume procedure within clause 5.3.13.4.

R2-2600302	Corrections to smtc5list and closest reference location report	ZTE Corporation, Sanechips	CR	Rel-19	38.331	19.1.0	5637	-	F	NR_NTN_Ph3-Core

R2-2600702	The corrections on the reference location list	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1	To align with the agreements, include the frequency specific offset IE, i.e., refLocOffset into SIB2 and adopt the TP in Annex1.

Proposal 2	To avoid the confusion, clarify the referenceLocation in the field description of refLocList indicate the reference location of serving cell in SIB19. And adopt the TP in Annex2

R2-2600719	Open issues on NR NTN	Nokia	discussion	NR_NTN_Ph3-Core

Observation 2: The location offsets for inter-frequency measurements are not referenced in the UE assistance information report, causing ambiguity for the network as it does not know whether it should create a measurement gap.

Observation 3: The ambiguity is only present in case the refLocList offsets are used.

Proposal 3: RAN2 to discuss how to specify that there may be an ambiguity in the report in case idle mode information are used to report reference location i.e. using the text proposal in Annex B.

[smct5list]

R2-2600394	Corrections on smtc5list in SIB2, SIB4 and MeasObjectNR	CATT	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: Modify “The network only includes SMTC offsets in smtc5list for (quasi-)Earth fixed cells.” as “The network includes SMTC offsets in smtc5list only for (quasi-)Earth fixed cells.”.

Proposal 2: Remove the restriction of “The total number of configurable SMTC offsets across smtc4list and smtc5list is 6.”

Proposal 3: Modify “If an entry in smtc5list is present but the pci-List, and/or offset fields are absent, the UE applies the value of the corresponding field from the entry at the same position in smtc4list and the periodicity value in smtc5list. If an entry in smtc5list is present but its fields are absent, the UE applies the values from the entry at the same position in smtc4list.” as “If an entry in smtc5list is present but the pci-List and offset fields are absent, the UE applies the value of the corresponding field from the entry at the same position in smtc4list. If an entry in smtc5list is present but the periodicity is absent, the UE applies the value in smtc.”.

Proposal 4: For Proposal 1-3, adopt the TP provided in the Annex A.

Proposal 5：Modify “smtc5list is only configured to indicate an additional SMTC periodicity to the one indicated in smtc1” as “smtc5list is only configured to indicate pci-List(s) and offset(s) corresponding to an additional SMTC periodicity to the one indicated in smtc1.”.

Proposal 6: For Proposal 5, adopt the TP provided in the Annex B.

R2-2600639	Remaining issue on smtc5list	Huawei, HiSilicon	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: Update the agreement from RAN2#132 to “RAN2 understands that, if a UE does not support additional SMTC periodicity, it ignores the periodicitySMTC(s) provided in smtc5list, if any, and applies the periodicity provided in smtc.” Otherwise, the UE will make up a wrong SMTC configuration (e.g., configured SMTC offset and duration of SSB-MTC5 in combination with the legacy SMTC periodicity).

Proposal 2: Adopt the TP in the Annex.

R2-2600719	Open issues on NR NTN	Nokia	discussion	NR_NTN_Ph3-Core

Observation 1: A legacy UE being configured to measure with smtc periodicity, will not be able to measure correctly in case smtc5list defines a larger periodicity.

Proposal 1: RAN2 to indicate in the field descriptions of smtc5list that the new period used by delta configuration should be larger than smtc to guide network implementation.

Proposal 2: RAN2 to agree on text proposal in Annex A

[coverage enhancements]

R2-2600709	Support for OCC RACH-less and other corrections	Samsung	discussion	Rel-19

Proposal 3: For searchSpaceLinkingId-CE, capture the configuration restriction in RRC field description that “The two linked SS sets are configured with the same controlResourceSetId.”.

Proposal 4: RAN2 to agree Text proposal in Annex B.

R2-2600944	Correction on downlink coverage enhancement for NR NTN phase 3	CMCC, ZTE Corporation, Sanechips	CR	Rel-19	38.300	19.1.0	1101	-	F	NR_NTN_Ph3-Core

[other]

R2-2600391	Discussion on time based measurement and OCC for RACH less HO	Xiaomi	discussion	Rel-19	NR_NTN_Ph3

Proposal 3: For the regenerative payload, time-based measurement initiation for feeder link switchover is not needed.

Proposal 4: Clarify that time-based measurement initiation for feeder link switchover is applicable only for transparent payload in stage 2 spec as follows:

The time-based measurement initiation may be applicable for the feeder link switchover case of the transparent payload for cell re-selection.

## 8.9	IoT NTN Ph3

(IoT_NTN_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-252504)

Time budget: 0 TU

Tdoc Limitation: 2 tdocs

### 8.9.1	Organizational

LS, Rapporteur input, including open issues lists, etc.

Rapporteur inputs do not count towards the tdoc limitation.

Incoming LSs

R2-2600024	Reply LS on CQI reporting for CB-Msg3 EDT for eMTC UE (R4-2523056; contact: CMCC)	RAN4	LS in	Rel-19	IoT_NTN_Ph3-Core	To:RAN2	Cc:RAN1

Noted

R2-2600028	Reply LS on PWS support in NB-IoT terrestrial networks (S1-254494; contact: Qualcomm)	SA1	LS in	Rel-19	NR_NTN_Ph3-Core	To:RAN2, CT1	Cc:SA2

Noted

R2-2600030	Reply LS on the paging in Store and Forward (S2-2510930; contact: CICT Mobile)	SA2	LS in	Rel-19	IoT_NTN_Ph3-Core	To:RAN2	Cc:RAN3

Noted

Rapporteur CRs

R2-2600200	Corrections for CB-MSG3-EDT	MediaTek Inc.	CR	Rel-19	36.321	19.1.0	1601	-	F	IoT_NTN_Ph3-Core	Late

[AT133][305][R19 IoT NTN] MAC CR (Mediatek)

Scope: Draft a MAC CR with meeting agreements so far

Intended outcome: Agreeable MAC CR

Deadline for revised CR (in R2-2600200):  Thursday 2026-02-12 16:30

R2-2600593	Rapporteur correction on IoT NTN Ph3	Huawei, HiSilicon	CR	Rel-19	36.331	19.1.0	5192	-	F	IoT_NTN_Ph3-Core

Endorsed as a baseline

Revised in R2-2601201 to reflect meeting agreements this week

R2-2601201	Rapporteur correction on IoT NTN Ph3	Huawei, HiSilicon	CR	Rel-19	36.331	19.1.0	5192	1	F	IoT_NTN_Ph3-Core

[AT133][306][R19 IoT NTN] RRC CR (Huawei)

Scope: Draft a RRC CR with meeting agreements so far

Intended outcome: Agreeable RRC CR

Deadline for revised CR (in R2-2601201):  Thursday 2026-02-12 16:30

R2-2600878	Support of power boost in Rel-19 NB-IoT NTN	Qualcomm Inc.	CR	Rel-19	36.306	19.1.0	1939	-	F	IoT_NTN_Ph3-Core

- Mediatek and Samsung think we should use the work item code for this

- Ericsson thinks the affected clauses should be updated

Revised in R2-2601202 to change the work item code and to fix the list of affected clauses

R2-2601202	Support of power boost in Rel-19 NB-IoT NTN	Qualcomm Inc.	CR	Rel-19	36.306	19.1.0	1939	1	F	XYZ (RAN4 WI code)

R2-2600879	Support of power boost in Rel-19 NB-IoT NTN	Qualcomm Incorporated	draftCR	Rel-19	36.331	19.1.0	F	IoT_NTN_Ph3-Core

Revised in R2-2601203 to change the work item code

Draft a separate RRC CR in R2-2601204 (Huawei) for the other needed changes

R2-2601203	Support of power boost in Rel-19 NB-IoT NTN	Qualcomm Incorporated	draftCR	Rel-19	36.331	19.1.0	F	XYZ (RAN4 WI code)

R2-2601204	Support of power boost in Rel-19 NB-IoT NTN	Huawei	CR	Rel-19	36.331	19.1.0	XXXX	-F	XYZ (RAN4 WI code)

Withdrawn

R2-2600253	Rapporteur correction on IoT NTN Ph3	Huawei, HiSilicon	CR	Rel-19	36.331	19.1.0	5160	5	F	IoT_NTN_Ph3-Core	R2-2507787	Withdrawn

### 8.9.2	RRC corrections

Corrections to TS 36.331.

[Paging relaxation indication]

R2-2600257	Paging Monitoring Relaxation for Store and Forward	Huawei, HiSilicon, Nokia, CATT, Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 1: Paging message shall be delivered from the MME to the eNB before the service link becomes available and therefore the eNB can properly decide whether to relax paging by implementation.

Observation 2: The specification impact is small and no new timing information is needed.

Proposal 1: An indication is introduced in the system information to indicate that the UE could skip monitoring paging.

Proposal 2: UE relies on the existing t-ModeSwitching/t-Service to stop relaxing monitoring paging.

R2-2600792	On remaining open issues for IoT-NTN	Nokia	discussion

Observation 1: All the pending CN paging messages are delivered to eNB when feeder link is unavailable. eNB operating in SF mode knows all the pending messages to be delivered towards UE in SF mode.

Observation 2: eNB knows the service area of UE to be paged and current coverage area for the pending paging message. eNB can deduce when to relax and enable paging monitoring based on its service area.

Proposal 1:  New parameter to skip paging monitoring is introduced in SIB31. When the paging monitoring is to be enabled this parameter is modified without system information change notification procedure.

R2-2600703	Discussion on paging relaxation for S&F operation	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 1	The CN-initiated paging remains possible during feeder link unavailability even if there is no pending downlink data since the network-triggered signaling may occur unexpectedly. So the UE should still be required to monitor CN-initiated paging, which leaves very limited space for paging monitoring relaxation.

Observation 2	The power saving gain from paging relaxation in S&F mode is marginal, as UE already achieves significant power saving through long eDRX cycle, and their paging monitoring opportunities during the satellite's brief service window are inherently minimal.

Proposal 1	RAN2 does not support paging monitoring relaxation for cell operating in S&F mode.

- MTK agrees with Oppo. QC also agrees

- Samsung also thinks this is monitoring relaxation is not needed and relying on eDRX is sufficient

- Xiaomi thinks that based on SA2 response we should not support monitoring relaxation in S&F mode

R2-2600054	Discussion on paging relaxation for S&F operation	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: Do not consider paging relaxation for S&F operation in this release.

We do not consider paging relaxation for S&F operation in this release

R2-2600654	Remaining issue of S&F	NEC	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal: RAN2 not to pursue introduction of a new indication for skipping paging monitoring in Rel19.

R2-2600414	RRC remaining issues for R19 IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 2: RAN2 discuss the following options regarding relaxing monitoring paging indication:

Option 1: No new indication will be introduced in SIB for indicating UE to relax monitoring paging when cell is in S&F mode.

Option 2: RAN2 ask RAN3 whether a S1 signaling enhancement, e.g., the CN can explicitly inform the eNB whether there is still buffered data/network-triggered signaling, can be considered.

[PWS Support indication for acceptable cell]

(copied from 8.9.3)

R2-2600818	Remaining issues on PWS support	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1	Confirm NB-IoT UE supports acceptable cell functionality in TN networks.

We confirm NB-IoT UE supports acceptable cell functionality also in TN networks (no spec change is expected to clarify this)

Proposal 2	If proposal 1 is confirmed, introduce network indication of support of PWS to prevent UEs camping on cells that will not support PWS broadcasting.

- HW thinks this is not needed as is not needed in legacy.

- Nokia thinks we can handle this in a unified manner in Rel-20. QC thinks Rel-20 is only for IoT NTN

Continue the discussion based on p6 in R2-2600201

Proposal 3	Clarify in TS 36.306 that support of acceptable cell is conditional on the support of pws-Support-r19.

(copied from 8.9.3)

R2-2600051  Other corrections on IoT NTN	Samsung	discussion	Rel-19

Proposal 7: Introduce a network indication of support of PWS broadcasting. Only if indicated, a UE may camp on the cell as an acceptable cell.

(copied from 8.9.3)

R2-2600201	Remaining Issues for IoT-NTN	MediaTek Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 5: RAN2 confirms that PWS reception is supported in NB-IoT terrestrial network from Rel-19.
Proposal 6: Introduce a new indicator in SIB1-NB to indicate whether the NB-IoT cell supports PWS broadcasting. A PWS-capable UE may camp on an acceptable NB-IoT cell for the purpose of receiving PWS if the NB-IoT cell supports PWS broadcasting.

CB Thursday with a TP introducing a new indicator in SIB1-NB to indicate whether the NB-IoT cell supports PWS broadcasting and specifying that A PWS-capable UE may camp on an acceptable NB-IoT cell for the purpose of receiving PWS if the NB-IoT cell supports PWS broadcasting (the TP should only affect the acceptable cell part and not impact the behaviour for suitable cells). Aim to take a final decision on Thursday

[AT133][301][R19 IoT NTN] PWS Support indication for acceptable cell (Samsung)

Scope: discuss a TP introducing a new indicator in SIB1-NB to indicate whether the NB-IoT cell supports PWS broadcasting and specifying that A PWS-capable UE may camp on an acceptable NB-IoT cell for the purpose of receiving PWS if the NB-IoT cell supports PWS broadcasting

Intended outcome: Agreeable TP

Deadline for companies' feedback:  Thursday 2026-02-12 14:30

Deadline for rapporteur's summary (in R2-2601205):  Thursday 2026-02-12 16:30

(copied from 8.9.3)

R2-2600479	Remaining issues for S&F mode	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 5: Introduce PWS-support indicator in NB-IoT cell(s) for both TN and NTN. Only when PWS-support is present, UE considers the NB-IoT cell is an acceptable cell.

R2-2600792	On remaining open issues for IoT-NTN	Nokia	discussion

Proposal 3:  System Information changes for indication of PWS capability of the cell is not supported in Rel-19.

[PWS support for IoT NTN]

R2-2600275	Clariifcation on PWS Support in IoT NTN	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: Clarify warningAreaCoordinates included in SIB10(-NB) for ETWS primary notification is only indicated in an NTN cell in 36.331 spec.

Proposal 2: Clarify warningAreaCoordinatesSegment included in SIB11(-NB) for ETWS secondary notification is only indicated in an NTN cell in 36.331 spec.

Proposal 3: For NB-IoT NTN, clarify warningAreaCoordinatesSegment included in SIB12-NB for CMAS notification is only indicated in an NTN cell.

- Ericsson thinks that if we agree on this we would change how CMAS works

Proposal 4: Clarify in the 36.300 that the geofencing for CMAS notification for IoT NTN is supported.

- HW thinks we could just have a Stage 2 change, copying the SA1 text, without the RRC changes

- Nokia supports to have changes in RRC

[Extended k-Mac]

R2-2600052	Extended k-Mac for IoT NTN	Samsung, Thales	discussion	Rel-19

Proposal 1: Extended k-Mac (k-Mac-r19)  is introduced for general Rel-19 NB-IoT NTN. Barring is introduced for UEs that do not support this extended k-Mac.

- Xiaomi wonders if we really need to support this scenario. Ericsson thinks we don’t need to support this. CATT agrees

We do not extend k-Mac (k-Mac-r19 for general Rel-19 NB-IoT NTN

Proposal 2: Agree Text proposal in Annex A.

[Other]

R2-2600396	Corrections on the CB-Msg3-ConfigSIB-NB	CATT	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: Adopt the following corrections for the IE CB-Msg3-ConfigSIB-NB.

-	For the filed cb-Msg3-TxWindow-NB-r19 and cb-Msg3-MaxAttemptNum-NB-r19, modify the need code as Need OP.

-	Add suffix “-NB” after the “SIB2” for the conditions and field descriptions of IE CB-Msg3-ConfigSIB-NB.

Agreed (to be captured in the RRC CR)

Proposal 2: If Proposal 1 is accepted, the TP provided in the Annex A can be adopted.

Agreements:

1.	We do not consider paging relaxation for S&F operation in this release

2.	We confirm NB-IoT UE supports acceptable cell functionality also in TN networks (no spec change is expected to clarify this)

3.	We do not extend k-Mac (k-Mac-r19 for general Rel-19 NB-IoT NTN

Withdrawn

R2-2600155	Discussion on paging relaxation for S&F operation	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core	Withdrawn

### 8.9.3	Other corrections

Corrections to TS 36.300, TS 36.304, TS 36.306 and TS 38.321.

- MAC related

[CQI reporting in eMTC]

R2-2600945	Discussion on RAN4 Reply LS on CQI reporting for CB-Msg3 EDT for eMTC UE	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: It is proposed that CQI reporting for CB-Msg3 EDT procedure for eMTC UE is not supported in R19 IoT-NTN.

Proposal 2: If P1 is agreed, kindly suggest RAN2 to agree the draft LS in annex.

R2-2600201	Remaining Issues for IoT-NTN	MediaTek Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 2: CQI reporting for CB-Msg3 EDT is not supported for eMTC UE.

R2-2600704	Discussion on open issues for CB-Msg3 EDT	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1	From RAN2 perspective, it is unfeasible to support CQI reporting for CB-Msg3 for eMTC unless the following spec changes are made:

⁃	A new period for CQI reporting similar to T1 for NB-IoT is defined for eMTC.

⁃	The narrowband to perform DL channel quality measurement is determined based on the narrowband for MPDCCH of CB-Msg4.

R2-2600819	Discussion on LS replies on S&F mode and CB-Msg3 EDT	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 2	Reply to RAN4 that, from RAN2 perspective it is feasible for an eMTC UE to measure CQI before the CB-Msg3 PUSCH transmission.

R2-2600395	Discussion on reply LS from RAN4 on CQI reporting	CATT	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: Reply LS to RAN4 to provide RAN2’s answers for the questions in R4-2523056:

-	CQI reporting in CB-Msg3 is intended to be supported for eMTC UEs, only if it requires no significant spec impact for RAN1 and RAN4.

-	RAN2 observes that it is feasible for an eMTC UE to measure CQI before CB-MSG3 transmission, but this question is ultimately up to RAN1.

R2-2600055	Discussion on AS RAI report and CQI report for CB-Msg3 EDT	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 3: RAN2 to wait for RAN1’s conclusion on whether it is possible for an eMTC UE to measure CQI before CB-MSG3 transmission.

CB Thursday to check if RAN1 has decided on this by then

R2-2600051	Other corrections on IoT NTN	Samsung	discussion	Rel-19

Proposal 6: RAN2 to discuss the following options for CQI reporting in eMTC CB-Msg3-EDT:

-	1) Send an LS to RAN1 whether possible to introduce possiblity to measure CQI as for NB-IoT for CB-Msg3-EDT,

-	2) Revert previous agreement and specification text support on CQI support for eMTC CB-Msg3-EDT.

[AS RAI reporting]

R2-2600388	Issues on transmitting RAI for NB-IoT UEs	Google	discussion	Rel-19	IoT_NTN_Ph3-Core	R2-2508309

Proposal 1	NB-IoT UEs shall trigger the AS RAI in a CB-Msg3 transmission, when rai-ActivationEnh is configured in SIB2-NB.

-	Nordic thinks this should be a “may”. MTK agrees and then sees no need for spec changes.

Continue in offline 302

Proposal 2	If P1 is agreed, adopt the TP in Annex for amending the current 36.321 specification.

Proposal 3	NB-IoT UEs shall set ‘Quality Report’ to the value ‘noMeasurement’ in the “DCQR and AS RAI MAC CE”, when transmitting the “DCQR and AS RAI MAC CE” in a CB-Msg3 and in a non-anchor carrier.

[AT133][302][R19 IoT NTN] AS RAI reporting (Google)

Scope: discuss AS RAI reporting for NB-IoT

Intended outcome: summary of the offline discussion

Deadline for companies' feedback:  Thursday 2026-02-12 14:30

Deadline for rapporteur's summary (in R2-2601206):  Thursday 2026-02-12 16:30

R2-2600051	Other corrections on IoT NTN	Samsung	discussion	Rel-19

Proposal 4: AS RAI for NB-IoT CB-Msg3 is introduced (not for legacy Msg3).

Proposal 5: Introduce new capability without signalling for AS RAI for NB-IoT CB-Msg3.

R2-2600201	Remaining Issues for IoT-NTN	MediaTek Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: RAN2 confirms that, when rai-ActivationEnh is configured in SIB2-NB, current SPEC allows the NB-IoT UE to transmit “DCQR and AS RAI MAC CE” in Msg3 or CB-Msg3 for AS RAI purpose. (No SPEC change is needed).

R2-2600477	Remaining MAC issues in IoT NTN	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 2: RAN2 to implement the previous agreement for UE to indicate during CB-Msg3-EDT procedure whether DL data following the UL data in CB-Msg3 is expected or not, via RAI.

R2-2600419	MAC remaining issues for R19 IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 2a: RAN2 confirms that, for CB-Msg3-EDT in NB-IoT NTN, if rai-ActivationEnh is enabled and applicable as specified in TS 36.331 [8], either BSR=0 MAC CE or DCQR and AS RAI MAC CE can be used to report RAI information to NW. No spec impacts for this confirmation.

Proposal 2b: RAN2 confirms that, for CB-Msg3-EDT in NB-IoT NTN, if only cqi-Reporting-r14 is enabled as specified in TS 36.331 [8], UE send CQI report (if available for anchor carrier) via Msg3 RRC messages. If cqi-Reporting-r14 is not enabled, NW would assume no CQI report in Msg3. That means for UE, neither cqi-NPDCCH in Msg3 RRC messages nor DCQR part in “DCQR and AS RAI MAC CE” need to be set, for example, “noMeasurements” can be used. No spec impacts for this confirmation.

Proposal 3: It’s suggested RAN2 to discuss, if both rai-ActivationEnh and cqi-Reporting-r14 are enabled, whether UE can use “DCQR and AS RAI MAC CE” for CQI and RAI report alone with CB-Msg3 and skip CQI report via RRC Msg3.

R2-2600055	Discussion on AS RAI report and CQI report for CB-Msg3 EDT	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 1: In legacy NB-IoT, “DCQR and AS RAI MAC CE” in Msg3 is intentionally not supported, which makes it even more difficult and less motivated to support “DCQR and AS RAI MAC CE” in CB-Msg3 for NB-IoT.

Proposal 1: NB-IoT UEs are not allowed to transmit the “DCQR and AS RAI MAC CE” in CB-Msg3.

Proposal 2: “DCQR and AS RAI MAC CE” in CB-Msg3 is supported for eMTC, and RAN2 to adopt the above TP for MAC spec.

R2-2600539	On open issue for Uplink Capacity enhancements in IoT-NTN	Nokia	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: For NB-IoT CP solution, RAN2 to confirm the NAS-level Release Assistance Information (NAS RAI) can be included in CB-Msg3.

Proposal 2: For NB-IoT UP solution, any enhancement to support AS RAI in Msg3 should be discussed in legacy NB-IoT session.

Proposal 3: When rai-ActivationEnh is configured in SIB2-NB, the NB-IoT UE is not allowed to transmit the “DCQR and AS RAI MAC CE” in CB-Msg3.

[MPDCCH narrowband selection]

R2-2600051	Other corrections on IoT NTN	Samsung	discussion	Rel-19

Observation 1: The UE contention resolution identity is a field in a MAC PDU, not an integer.

Observation 2: The UE contention resolution identity is a field in a MAC PDU, not an UE-internal variable.

Observation 3: The modulus 2 operation of an integer based on a bit-string, is equivalent to taking either the leftmost or rightmost bit of the bit-string.

Observation 4: If modulus 2 of the UE Contention Resolution Identity where the integer is interpreted as having the rightmost bit being the MSB is used, then all UEs will select the same MPDCCH narrowband.

Observation 5: If modulus 2 of the UE Contention Resolution Identity where the integer is interpreted as having the leftmost bit be the MSB is used, then:

•	For UP solution with full resume ID, the UEs may uniformly select MPDCCH narrowband

•	For UP solution with truncated resume IDs, the UEs will all select the same MPDCCH narrowband

•	For CP solution, it cannot be expected that the UEs will select the MPCCH narrowband uniformly

Observation 6: Modulus 2 of a Contention Resolution ID does in general not result in uniform selection of narrowbands.

-	QC thinks in the Resume ID case there is no issue, but acknowledges there is an issue in the CP case and agrees that the p2 would solve the issue

-	Nokia thinks there is an issue if the short resume ID is used, but not if the full Resume ID is used, but don’t think there is an issue in CP case.

-	Ericsson thinks we should not support 2 MPDCCHs or leave it broken

-	HW thinks we don’t need to change anything

Proposal 1: RAN2 to discuss and resolve the following issues with MPDCCH narrowband selection:

•	A) Referring to the UE Contention Resolution Identity as a UE variable

•	B) Ambiguities how the UE contention resolution identity is interpreted as an integer

•	C) Preventing all UEs from selecting the same narrowbands, or selecting the narrowbands non-uniformly

Proposal 2: To solve proposal 1: UE selects the MPDCCH narrowband according to the 16th bit of the CCCH SDU (or a UE contention resolution field).

UE selects the MPDCCH narrowband according to the 16th bit of the contention resolution ID

Proposal 3: Agree text proposal in Annex B.

[SCS indication]

R2-2600419	MAC remaining issues for R19 IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 1: In legacy system, it’s only allowed to change SCS from Msg3, e.g., to assign a new SCS for Msg3 which is different from the SCS used in Msg1. But it’s not allowed to further change SCS hereafter Msg3.

Proposal 1: It’s no need to introduce new field in the subheader of CMR to indicate uplink SCS.

We don’t introduce a new field in the subheader of CMR to indicate uplink SCS

R2-2600477	Remaining MAC issues in IoT NTN	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: Do not pursue introducing SCS indication in Msg4, in light of RAN1 design to use the SCS configured in SIB for subsequent NPUSCH transmissions.

[Other]

R2-2600276	MAC Correction for CB-Msg3 EDT	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation: When a UE configured with PUR receives a TAC and the Random Access procedure is successfully completed, the UE (re)starts the pur-TimeAlignmentTimer, if configured.

Proposal: When a UE configured with PUR receives a TAC included in a successful CMR, the UE (re)starts the pur-TimeAlignmentTimer, if configured.

-	Nokia thinks this is not needed

-	QC thinks this makes sense

R2-2600704	Discussion on open issues for CB-Msg3 EDT	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 3	UE behaviour on PUSCH transmission assuming NTA=0 in CB-Msg3 EDT has been captured in RAN1 spec.

Proposal 2	Remove “set current NTA to 0” in 5.1a.1 in TS36.321.

-	QC thinks this will be captured in RAN1 specs

R2-2600419	MAC remaining issues for R19 IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 4: It’s suggested to indicate the last CB-Msg4 message within the current CB-Msg3 response window. One possible option is to define a new MAC header for CB-Msg4 by extending “T=11”.

R2-2600646	MAC correction for CB-Msg3 transmission	NEC	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: A specification should capture “RV0 is used to transmit the first repetition of each CB-Msg3 replica in the transmission window” for multiple replica transmissions. Regarding which specification, the following two options may be considered:

Option 1: Capturing RV0 for multiple replicas in TS 36.321.

Option 2: Sending an LS to ask RAN1 to add RV description.

Proposal 2: If option 1 is adopted, RAN2 to agree to add the following text in 5.4.2.2 (TP #1 in Annex):

“ For NB-IoT UEs, BL UEs or UEs in CE mode A, the RV 0 is used to transmit the first repetition of each CB-Msg3 replica within the CB-Msg3 transmission window for CB-Msg3-EDT procedure.”

Proposal 3: For NB-IoT UE, RSRP measurement for selecting coverage enhancement can be measured in the anchor carrier. RAN2 to agree the following text in 5.1a.1(TP #2 in Annex):

“NOTE 1:	An NB-IoT UE measures RSRP on the anchor carrier.”

Proposal 4: When E field is set to “0”, only MAC CMR or padding starts at the next byte, i.e. “a MAC SDU” should be removed. RAN2 to agree the following text in 6.2.5 (TP #3 in Annex):

“The E field is set to "0" to indicate that either a MAC CMR or padding starts at the next byte”

Agreements:

1.	UE selects the MPDCCH narrowband according to the 16th bit of the contention resolution ID

2.	We don’t introduce a new field in the subheader of CMR to indicate uplink SCS

- Idle mode related

[S&F cell suitability]

R2-2600733	Considering a NAS to AS indication in cell (re)Selection	Sateliot, Novamint, Thales	discussion	Rel-19

Observation 1: if NAS cannot initiate any NAS procedure in a cell, an ability for a UE to expect normal service in that cell can be impaired questioning the cell suitability.

Observation 2:  the UE AS can make use of the NAS indication to find a suitable cell in the case of S&F Satellite Operation.

Observation 3: Current NOTE appearing in section 5.2.4.4 does not clarify whether the UE is S&F capable or not, and does not mention whether the intention of deprioritising or not considering S&F cells is dictated by configuration or should be assumed as a default behaviour

Proposal 1: RAN2 is respectfully requested to add corresponding stage-3 descriptions in clauses 4.2 and/or 5.2.4.4 of TS 36.304 (related TP provided in the Annex).

-	QC thinks that based on CT1 discussion a UE would still have to receive paging

Continue in offline 303

[AT133][303][R19 IoT NTN] S&F cell suitability (Sateliot)

Scope: continue the discussion on R2-2600733

Intended outcome: summary of the offline discussion

Deadline for companies' feedback:  Thursday 2026-02-12 14:30

Deadline for rapporteur's summary (in R2-2601207):  Thursday 2026-02-12 16:30

R2-2600479	Remaining issues for S&F mode	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: There is no further AS layer impact (e.g., suitable cell determination, cell reselection) due to MME provisioned S&F Monitoring List and/or Wait Timer running.

Proposal 2: No impact to service link coverage determination is foreseen from MME provisioned S&F Monitoring List and S&F Wait Timer.

[Cell measurement and reselection based on AS layer S&F mode info]

R2-2600387	Impact of the S&F mode transition time on AS	Google, ASUSTeK, Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 1	As the S&F mode indication (i.e., sf-OperationMode) and the legacy barring indications can only change at the boundaries of SI modification periods, the S&F mode transition time (i.e., t-ModeSwitching) signaled in SIB31 allows UEs to know more precisely when the serving cell changes its operational mode.

Observation 2	If a UE not supporting the S&F operation considers a cell as barred because it's currently operating in S&F mode, the UE is mandated by legacy rules to exclude the cell as a candidate for cell selection/reselection for 300 seconds. This exclusion persists even if the cell reverts to normal mode (and thus becomes suitable again) before the 300 seconds expire.

Observation 3	The current measurement rules for cell reselection in TS 36.304 are primarily driven by signal quality thresholds. However, these rules fail to trigger neighbor cell measurements when the serving cell enters the S&F mode. Consequently, the recently introduced NOTE in TS 36.304 is considered incomplete as it does not account for this specific transition.

Proposal 1	If a Rel-19 UE considers a cell as barred due to the S&F operation, the UE excludes the cell as a candidate for cell selection/reselection only until t-ModeSwitching.

-	HW supports this

-	QC thinks it’s obvious but wonders if this needs any 304 changes

-	Samsung wonders if this for UEs supporting S&F or not. Google thinks this is for all R19 UEs

Continue in offline 304

[AT133][304][R19 IoT NTN] Cell measurement and reselection based on AS layer S&F mode info (Google)

Scope: continue the discussion on R2-2600387

Intended outcome: summary of the offline discussion

Deadline for companies' feedback:  Thursday 2026-02-12 14:30

Deadline for rapporteur's summary (in R2-2601208):  Thursday 2026-02-12 16:30

Proposal 2	If P1 is agreeable, RAN2 to adopt the 36.304 TP attached in Annex A.

Proposal 3	A Rel-19 UE can trigger early measurements for cell reselection based on the transition time from the normal mode to the S&F mode.

Proposal 4	Being able to trigger early measurements for cell reselection based on the S&F mode transition time is an optional capability without RRC signaling.

Proposal 5	If P3 and P4 are agreeable, RAN2 to adopt the 36.304 TP and 36.306 TP attached in Annex B and C.

R2-2600201	Remaining Issues for IoT-NTN	MediaTek Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 4: Leave the prioritization or de-prioritization of cell reselection based on S&F mode to UE implementation (i.e. no need to change current 36.304 NOTE to normative text).

[S&F mode indication and discontinuous coverage]

R2-2600438	Impact on discontinuous coverage for Store and Forward	Huawei, HiSilicon	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: If the S&F Monitoring List and the S&F Wait Timer have been received from the upper layers, an S&F UE needs also to consider this NAS information when determining whether it is out of NTN coverage, besides SIB32.

CB Thursday

R2-2601064	Discussion on paging monitoring relaxation and discontinuous coverage	ETRI	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 2	S&F mode of each satellite is broadcast for discontinuous coverage in SIB32.

Proposal 3	Together with S&F mode, time information of transitions from normal to S&F mode and S&F to normal mode is broadcast for discontinuous coverage in SIB32.

CB Thursday

- Other

[PWS support for eMTC]

R2-2600818	Remaining issues on PWS support	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 4	Update the stage 2 specification that PWS is not supported over Rel-17 and Rel-18 NTN. Draft CR is provided in Annex.

Proposal 5	It is up to Rel-17 or Rel-18 NTN capable UE whether it can receive PWS in Rel-19 NTN cell. No stage 3 specification change is needed.

CB Wednesday (as part of the R19 NR NTN session)

(copied here from 8.9.2)

R2-2600414	RRC remaining issues for R19 IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: From RAN2 perspective, PWS for eMTC (WB-E-UTRAN) over TN/NTN can be supported from R17, which is not clear from incoming LS from SA2 (and maybe CT1 specification), further clarification with CT1/SA2 is needed.

CB Wednesday (as part of the R19 NR NTN session)

Withdrawn

R2-2600156	Discussion on AS RAI report for CB-Msg3 EDT	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core	Withdrawn

R2-2600964	Discussion on MAC remaining issues for CB-Msg3 EDT	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core 	Withdrawn

## 8.17	IoT-NTN TDD mode

(IoT_NTN_TDD; leading WG: RAN1; REL-19; WID RP-243293)

Time budget: 0TU

Tdoc Limitation: 1 tdoc

Corrections to all specs.

Incoming LS

R2-2600025	LS Reply on precompensation for NB-IoT NTN TDD mode (R4-2523100; contact: Iridium)	RAN4	LS in	Rel-19	IoT_NTN_TDD-Core	To:RAN1	Cc:RAN2

Rapporteur CRs

R2-2600594	Rapporteur correction on IoT NTN TDD	Huawei, HiSilicon, Iridium, Nokia	CR	Rel-19	36.331	19.1.0	5193	-	F	IoT_NTN_TDD

R2-2600816	Corrections to valid uplink subframes for IoT NTN TDD	Toyota ITC (Rapporteur), Nokia	CR	Rel-19	36.321	19.1.0	1602	-	F	IoT_NTN_TDD-Core

Revised in R2-2601137

R2-2601137	Corrections to valid uplink subframes for IoT NTN TDD	Toyota ITC (Rapporteur), Nokia	CR	Rel-19	36.321	19.1.0	1602	1	F	IoT_NTN_TDD-Core

[neighbour cell measurements]

R2-2600158	Correction on radioFrameOffset for IoT NTN TDD	Xiaomi	discussion	Rel-19	IoT_NTN_TDD

In SIB4-NB (for intra-freq measurement):

radioFrameOffset

Offset, in number of radio frames, from the start of the 90 milliseconds IoT NTN TDD frame of serving cell to the start of the nearest 90 milliseconds IoT NTN TDD frame of the neighbour cell, at the uplink time synchronization reference point defined in clause 16.1.2 of TS 36.213 [23]. The UE may use the radioFrameOffset and the propagation delay difference between serving cell and the neighbour cell to perform intra-frequency neighbour cell measurement.

In SIB5-NB (for inter-freq measurement):

radioFrameOffset

Offset, in number of radio frames, from the start of the 90 milliseconds IoT NTN TDD frame of serving cell to the start of the nearest 90 milliseconds IoT NTN TDD frame of the neighbour cell, at the uplink time synchronization reference point defined in clause 16.1.2 of TS 36.213 [23]. The UE may use the radioFrameOffset and the propagation delay difference between serving cell and the neighbour cell to perform inter-frequency neighbour cell measurement.

Proposal 1: RAN2 to adopt above field description of radioFrameOffset in SIB4/5-NB.

R2-2600255	Remaining issues on neighbour cell measurement	Huawei, HiSilicon	discussion	Rel-19	IoT_NTN_TDD

Proposal: RAN2 to clarify that UE should perform neighbour cell measurement within the 8.28ms duration from the start of the valid downlink duration according to the TDD radio frame offset.

[CB-msg3-EDT]

R2-2600705	Discussion on CB-Msg3 EDT in IoT NTN TDD mode	OPPO	discussion	Rel-19	IoT_NTN_TDD

Observation 1	Multiple of 90 ms NPUSCH periodicity for CB-Msg3 EDT may cause an ambiguity on the time-domain resource of NPUSCH occasions between UE and network.

Proposal 1	Introduce start H-SFN, e.g., npusch-StartHSFN in cb-Msg3-TimeResource-NB-r19, which is defined as conditional optional field in IoT NTN TDD mode.

Proposal 2	RAN2 agrees in IoT NTN TDD mode, it may rely on network to reconfigure the start H-SFN/SFN/subframe for NPUSCH for CB-Msg3 EDT upon H-SFN wraps round to avoid the ambiguity on NPUSCH occasions between UE and network.

Proposal 3	For collision between NPUSCH for CB-Msg3 EDT and NPRACH in IoT NTN TDD mode, follow the current principle, i.e., NPRACH is prioritized and NPUSCH is postponed.

Proposal 4	RAN2 assumes it can be up to NW implementation to avoid the collision between NPUSCH for CB-Msg3 EDT and other PUSCH in IoT NTN TDD mode.

[misc]

R2-2600304	Miscellaneous corrections to IoT NTN TDD	ZTE Corporation,  Sanechips	CR	Rel-19	36.331	19.1.0	5190	-	F	IoT_NTN_TDD

Withdrawn

R2-2600254	Rapporteur correction on IoT NTN TDD	Huawei, HiSilicon	CR	Rel-19	36.331	19.1.0	5161	6	F	IoT_NTN_TDD	R2-2507788	Withdrawn

R2-2600397	Corrections on IoT NTN TDD	CATT	discussion	Rel-19	IoT_NTN_TDD	Withdrawn

## 8.19	TEI19

Time budget: 0 TU

Tdoc Limitation: 1

No new proposals expected for TEI19.

Companies are encouraged to submit co-sourced contributions, which will have priority for discussion in RAN2#133

### 8.19.1	RAN2-led

(Also including corrections, if any, for LTE_TN_NR_NTN_mob, leading WG: RAN2, Rel-19 WID: RP-251974 )

R2-2600634	Correction to TN-NTN redirection [IoT_TN_NTN_redir][EUTRAN-to-NBIoTNTN]	Huawei, HiSilicon	CR	Rel-19	36.306	19.1.0	1938	-	F	TEI19

R2-2600710	RRC corrections to LTE TN to NR NTN	Samsung	discussion	Rel-19

### 8.19.2	Other WG-led

R2-2600175	Introduction of SIB1 PDSCH repetition for FR1 TN [SIB1_rep_TN]	THALES, Huawei, HiSilicon, Vivo, CATT, Ericsson	CR	Rel-19	38.300	19.1.0	1085	-	F	TEI19

R2-2600540	Introduction of PDSCH repetition carrying SIB1 (Rel-19 NTN) for TN	Nokia, Qualcomm	CR	Rel-19	38.300	19.1.0	1094	-	B	TEI19

## 8.20	NR Others

Tdoc limit: 2

Specific items may be allocated to a breakout session for treatment.

Impacts from Other RAN WGs and TSGs that has no separate TU budget in RAN2. LS ins for Rel-19 specific WIs/SIs that has no RAN WI.

Additional tdocs on top of limit can be allowed for co-sourced contribution with 3 or more companies

### 8.20.1	RAN4

R2-2600180	Clarification of FR1-NTN band in TS 38.306 and TS 38.331	Sharp, Huawei, HiSilicon, Ericsson	discussion	Rel-19	NR_NTN_Ku_bands

R2-2600635	Introduction of FR1-NTN terminology	Huawei, HiSilicon, Sharp, Ericsson	CR	Rel-19	38.331	19.1.0	5652	-	B	NR_NTN_Ku_bands

R2-2600636	Introduction of FR1-NTN terminology	Huawei, HiSilicon, Sharp, Ericsson	CR	Rel-19	38.306	19.1.0	1417	-	B	NR_NTN_Ku_bands

## 9.7	IoT NTN Ph4

(IoT_NTN_Ph4; leading WG: RAN2; REL-20; WID: RP-253813

Time budget: 0.5 TU

Tdoc Limitation: 1 tdocs

### 9.7.1	Organizational

Incoming LSs

R2-2600029	Reply LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC (S1-254508; contact: Qualcomm)	SA1	LS in	Rel-20	FS_5GSAT_Ph4_ARC	To:SA2	Cc:RAN2, SA4, CT1, SA3, RAN1

R2-2600037	Reply LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC (S3-253797; contact: vivo)	SA3	LS in	Rel-20	FS_5GSAT_Ph4_SEC	To:SA2	Cc:RAN2, SA4, CT1, SA1, RAN1

### 9.7.2	Other

Contributions should focus on the possible solutions to avoid/mitigate the potential issues when handling voice packets of different sizes.

[SPS support & variable size packets support]

R2-2600107	Discussion on RAN Scheduling for GSO Voice Traffic	vivo	discussion	Rel-20	IoT_NTN_Ph4-Core

Observation 1: The size of the bundled packet from the upper layer varies with its composition (e.g., pure voice packets, mixed voice, SID, No_Data packets).

Observation 2: The composition of bundled packets is unpredictable due to the random switching between active speak periods and silence period, resulting in size differences across different bundling periods.

•	Option 1: Single SPS resource configuration with a single fixed TBS, as shown in Fig.2

•	Option 2: Two simultaneously active SPS resource configurations with two different fixed TBS, as shown in Fig.3

•	Option 3: Two TDMed active SPS resource configurations with two different fixed TBS, as shown in Fig.4-1 and Fig.4-2

Proposal 1: A single SPS configuration is supported for UL and DL, respectively.

Proposal 2: RAN2 further discusses whether/how two SPS configurations are supported for UL and/or DL.

Proposal 3: From RAN2 perspective, RRC is used to provide SPS parameters (e.g., periodicity), while whether/how DCI is used to activate/deactivate SPS depends on RAN1.

Observation 3: According to RAN1 rely LS R1-2508096, the probability of having 16 or 64 packets consecutively lost is negligible under typical operating conditions. It can be assumed that the RoHC state transition is a very rare case in practice.

Proposal 4: For variable-sized packets due to RoHC state transition, RAN2 assumes that the network can handle scheduling via dynamic scheduling.

R2-2600946	Considerations on support of IMS voice call over IoT-NTN	CMCC	discussion	Rel-20	IoT_NTN_Ph4

Proposal 1: To schedule the variable sized packets, the following 3 solutions could be discussed:

Option 1: Single SPS configuration

Option 2: Multiple SPS Configuration with Dynamic Activation

Option 3: Combination of SPS and dynamic scheduling (Deprioritized)

R2-2600050	Discussion on various issues for voice over NB-IoT NTN	Samsung	discussion	Rel-20

Proposal 1: Downlink/Uplink SPS for NB-IoT is activated using DCI as in 4G LTE SPS.

Proposal 2: Introduce multiple SPS configurations that can be switched using an SPS configuration index in the DCI.

Proposal 3: The maximum number of SPS configurations is limited to 2.

Proposal 4: For the issue of varying packet sizes it can be solved either using SPS to re-configure the TBS or by configuring a large TBS in anticipation of varying packet sizes (based on network implementation).

R2-2600256	Further discussion on voice over NB-IoT NTN via GSO	Huawei, HiSilicon	discussion	Rel-20	IoT_NTN_Ph4-Core

- Downlink scheduling

Proposal 3: RAN2 confirms that for downlink NW can handle scheduling of variable-size packets by implementation.

- Uplink scheduling

Proposal 4: For UL, RAN2 strives to design a scheduling mechanism to handle the variable-size packets which minimizes the resource waste and doesn’t introduce additional scheduling delay.

Alternative solutions

Option 1: Multiple SPS without resource overlap

Option 2: SPS reconfiguration or dynamic scheduling

Option 3: Preconfigure two SPS and switch based on the ROHC feedback

Option 4: Single SPS with flexible resource usage (similar to the edt-SmallTBS mechanism), or with two resource sizes with one contained within the other.

Observation 1: Option 4 provides a better way which minimizes the resource waste, saves UE power and ensures UL coverage, while doesn’t introduce additional scheduling delay.

Therefore, it is proposed:

Proposal 5: For UL scheduling of variable-size packets, RAN2 adopts a single SPS configuration with flexible resource usage based on the packet size.

R2-2600820	Support of voice over NB-IoT	Qualcomm Incorporated	discussion	Rel-20	IoT_NTN_Ph4

Proposal 13	To handle variable packet size, consider following solutions for different scenarios #1: Dynamic SPS activation/deactivation, #2: RLC segmentation, #3: Aggressive resource allocation and #4: Multiple SPS for UEs in good coverage.

R2-2600699	Discussion on voice support over NB-IoT NTN	OPPO	discussion	Rel-20	IoT_NTN_Ph4

Proposal 1	RAN2 sends an LS to SA2 to ask new QoS requirement for IMS voice over GEO.

Proposal 2	If the clarified QoS requirement can be satisfied with additional UL scheduling delay of UE-eNB RTT, RAN2 confirms it can be up to network implementation to handle the packet size variation by issuing a dynamic scheduling, and no additional enhancements are needed.

Proposal 3	If the new QoS requirement cannot be met, RAN2 further studies more efficient UL scheduling mechanism to satisfy the requirement for IMS voice over GEO.

Proposal 4	RAN2 discusses the following scheduling scheme to handle packet size variation due to talk-silent transition.

⁃	Option1: Support two SPS configurations for voice packet and SID packet, respectively.

⁃	Option2: Use one SPS configuration with over-provisioning of resources to support both voice packet and SID packet.

Proposal 5	If RAN2 agrees to go to option 2, send a LS to RAN1, asking the feasibility of TB size scaling on SPS grant for voice.

Proposal 6	RAN2 confirms that SPS periodicities not divided by 10204 can be supported by the current SPS occasion calculation formula, which is sequentially determined according to SFNstart time and subframestart time.

R2-2600884	NB-IoT NTN voice over GSO adaptations	Ericsson	discussion	Rel-20	IoT_NTN_Ph4-Core

Proposal 1	RAN2 sends an LS to SA4 informing them that a voice encoder having a talk state with a certain packet size and a silent state with equal or smaller packet size, and the packet periodicity for the silent state is a multiple of the packet periodicity in talks state, will result in efficient scheduling in the RAN. Then ask SA4 whether such a voice encoder can be specified.

Proposal 6	RAN2 to discuss whether support for any SPS periodicity on non-anchor carrier will be beneficial.

R2-2600184	Voice Support in NB-IoT NTN	Sharp	discussion	Rel-20	IoT_NTN_Ph4

Proposal 7: SPS periodicity not divided by 10240ms is supported. NR formula is a baseline.

[RRC connection setup procedure]

R2-2600398	Discussion on supporting IMS voice call over GSO for NB-IoT	CATT	discussion	Rel-19	IoT_NTN_Ph4

Proposal 3: Introduce an indication in SIB2-NB to indicate network supports IMS voice call over GSO for NB-IoT.

Proposal 4: Add an establishment cause for IMS voice call in the EstablishementCause-NB IE.

Proposal 5: Introduce an indication in SIB1-NB to indicate network supports IMS emergency call over GSO for NB-IoT.

Proposal 6: In TS 36.304, update the definition of acceptable cell and the “Figure 5.2.2-2: RRC_IDLE Cell Selection and Reselection for NB-IoT” to support emergency call service for NB-IoT.

R2-2600699	Discussion on voice support over NB-IoT NTN	OPPO	discussion	Rel-20	IoT_NTN_Ph4

Proposal 7	Add values of mo-VoiceCall and emergency in EstablishmentCause-NB.

Proposal 8	Update the definition of acceptable cell for NB-IoT to support emergency calls.

Proposal 9	Introduce two indications in SIB-NB to indicate network support of IMS voice call and emergency call, respectively

[number of DRBs]

R2-2600413	Efficient scheduling and more DRBs support for NB-IoT over GEO	ZTE Corporation, Sanechips	discussion	Rel-20	IoT_NTN_Ph4-Core

Proposal 5a: It’s suggested to introduce at least one more DRB (besides the existing two DRBs) for UP solution.

R2-2600056	Discussion on IMS voice over GSO	Xiaomi	discussion	Rel-20	IoT_NTN_Ph4

Proposal 2: The Maximum DRB number for NB-IoT is increased beyond 2. The exact number can wait for SA2’s further input.

R2-2600820	Support of voice over NB-IoT	Qualcomm Incorporated	discussion	Rel-20	IoT_NTN_Ph4

Proposal 2	For UP solution, wait for SA2 progress on the maximum number of DRBs (e.g., 4 or 8) that need to be supported simultaneously.

R2-2600256	Further discussion on voice over NB-IoT NTN via GSO	Huawei, HiSilicon	discussion	Rel-20	IoT_NTN_Ph4-Core

Proposal 1: Send an LS to SA2 to ask whether concurrent services need to be supported in NB-IoT NTN via GSO and if yes, how many.

Proposal 2: If concurrent services are to be supported, extend the maximum number of DRB accordingly.

[other]

R2-2600884	NB-IoT NTN voice over GSO adaptations	Ericsson	discussion	Rel-20	IoT_NTN_Ph4-Core

Proposal 2	The RoHC profiles for RTP/UDP/IP, 0x0001 and 0x0101, are supported for NB-IoT voice UEs.

Observation 2	The LS S4-251584 assumes use of the optional features HARQ feedback disabling and TA reporting, further it is assumed that the UE specific Koffset is perfectly tuned to the UEs timing advance.

Observation 3	If non-perfect Koffset is used, there must be a part of the SPS period to account for the difference between the UEs actual TA and Koffset. If cell specific Koffset is used then the minimum compensation is equal to Koffset minus minimum TA in the cell.

Proposal 3	NB-IoT NTN UEs without TA reporting and without UE specific Koffset capability are supported in voice over GSO.

R2-2600050	Discussion on various issues for voice over NB-IoT NTN	Samsung	discussion	Rel-20

Observation 1: The minimum configurable discard timer value of 5120 ms is not suitable for voice purposes.

Proposal 6: RAN2 to discuss suitable discard timer values for voice over NB-IoT NTN. This includes whether discard timer values from normal E-UTRAN are suitable or whether new set of discard timer values are needed for voice over NB-IoT NTN.

Observation 2: Normal LTE access barring momentarily bars a UE based on the access barring parameters, which is then alleviated.

Observation 3: For NB-IoT access barring, the UE is barred indefinitely based on the access barring parameters.

Observation 4: Access barring for voice services is important as the resources for serving a UE will be significantly higher compared to typical NB-IoT traffic.

Proposal 7: RAN2 to discuss access barring mechanisms for voice over NB-IoT NTN, such as whether a barring alleviation mechanism is needed.

Observation 5: Counter Check procedures were introduced for E-UTRAN and NR to support a network to detect packet insertions.

Observation 6: Counter check procedures were not introduced for NB-IoT.

Proposal 8: RAN2 to discuss introducing counter check procedures for voice over NB-IoT NTN.

R2-2600413	Efficient scheduling and more DRBs support for NB-IoT over GEO	ZTE Corporation, Sanechips	discussion	Rel-20	IoT_NTN_Ph4-Core

Proposal 5b: The choice between the normal UP solution or the UP optimization solution can be left to implementation decision based on the NW and UE capabilities.

R2-2600261	Discussion on variable voice packets handling for IMS voice over IoT-NTN	Quectel	discussion

R2-2600476	Discussion on voice over GEO	Apple	discussion	Rel-20	IoT_NTN_Ph4

R2-2600490	Consideration of SPS in NB-IoT voice over GEO	China Telecom	discussion	Rel-20	IoT_NTN_Ph4

R2-2600510	Discussion on IoT-NTN to support IMS voice call	HONOR	discussion	Rel-20	IoT_NTN_Ph4

R2-2600541	Discussion on support of voice over NB-IoT NTN via GEO	Nokia	discussion	Rel-20	IoT_NTN_Ph4

R2-2600626	Discussion on SPS for the support of voice over NB-IoT-NTN	ETRI	discussion	Rel-20	IoT_NTN_Ph4

R2-2600653	Voice over GSO	NEC	discussion	Rel-20	IoT_NTN_Ph4

R2-2600657	Discussion on support of NB-IoT-NTN voice	Spreadtrum, UNISOC	discussion	Rel-20

R2-2600753	Considerations on UP solution for voice support over IoT-NTN	Lenovo	discussion	Rel-20

R2-2600991	Semi-persistent scheduling for supporting voice over NB-IoT-NTN	InterDigital Communications	discussion	Rel-20

Withdrawn

R2-2600157	Discussion on IMS voice over GSO	Xiaomi	discussion	Rel-20	IoT_NTN_Ph4	Withdrawn

## 9.8	E-UTRA TN to NR NTN HO

(LTE_TN_NR_NTN_HO; leading WG: RAN2, Rel-20; WID  RP-252890)

Time budget: 0 TU

Tdoc Limitation: 0 tdocs

# Summary

Agreed CRs

R17 IoT NTN (LTE_NBIOT_eMTC_NTN)

R17 NR NTN (NR_NTN_solutions-Core)

R19 IoT NTN (IoT_NTN_Ph3-Core)

R19 NR NTN (NR_NTN_Ph3-Core)

IoT_NTN_TDD

TEI19

Approved LSs out

[Post133] Email discussions

Short

Long