3GPP TSG-RAN WG2 Meeting #133	R2-26xxxxx

Göteborg, Sweden,  9-13 February 2026

Source: 	Session Chair (MediaTek)

Title:	Report from session on positioning, sidelink relay, and ambient IoT

# 4	EUTRA Rel-17 and earlier

Only essential corrections. No documents should be submitted to 4. Please submit to 4.x

## 4.2	Positioning corrections Rel-16 and earlier

(LTE_NavIC-Core, LTE TEI16 Positioning), REL-15 and Earlier WIs related to positioning are in scope but not listed explicitly (long list).

Tdoc Limitation: 1 tdoc

# 5	NR Rel-15 and Rel-16

Essential corrections only.

Tdoc Limitation: 3 Tdocs in total for agenda item 5 (incl. its sub agenda items) and agenda item 6 (incl. its sub agenda items)

In case a correction needs to be reflected in both NR TS and LTE TS, the corrections should be submitted under one single AI (so the NR and LTE correction can be treated together), the sub-Ais below this

## 5.2	NR Positioning Support

(NR_newRAT-Core; leading WG: RAN1; REL-15; started: Mar. 17; closed: Jun. 19: WID: RP-191971)

(NR_pos-Core; leading WG: RAN1; REL-16; started: Mar 19; target; Jun 20; WID: RP-200218).

(NR TEI16 Positioning)

Stage 2 corrections shall be discussed with the specification rapporteur (Sven Fischer sfischer@qti.qualcomm.com) before submission. Stage 2 CRs not discussed with the specification rapporteur will not be treated.

# 6	NR Rel-17

Essential corrections only.  Editorial/clarifications should be sent to be reviewed and approved by spec rapporteurs prior to submission.  Editorials should only be submitted by spec rapporteurs.

Tdoc Limitation: 3 Tdocs in total for agenda item 5 (incl. its sub agenda items) and agenda item 6 (incl. its sub agenda items)

## 6.2	NR positioning enhancements

(NR_pos_enh-Core; leading WG: RAN1; REL-17; WID: RP-210903)

# 7	NR Rel-18

## 7.0	Common

Rel-18 WIs not covered under an explicit AI in 7.x.  Multi-WI Rel-18 items, e.g. cross-WI-issues not handled under another WI. UE capabilities.

### 7.0.2	Rel-18 corrections

Essential corrections only. For smaller corrections please contact CR editor / Rapporteur directly.  Coordinate with rapporteurs and chair if input above limit is required

Tdoc limitation: 3

#### 7.0.2.19	Enhanced NR Sidelink Relay

(NR_SL_relay_enh-Core; leading WG: RAN2; REL-18; WID: RP-223501)

#### 7.0.2.21	Expanded and improved NR positioning

(NR_pos_enh2-Core; leading WG: RAN1; REL-18; WID: RP-232670)

Including outcome of email discussion [Post132][401] NCD-SSB configuration for serving cell in RRC_INACTIVE (China Telecom)

Email discussion summary and related CRs

R2-2600684	Summary of [POST132][401][POS] NCD-SSB configuration for serving cell in RRC_INACTIVE (China Telecom)	China Telecom	discussion	Rel-18

Noted

Discussion:

CATT clarify that the agreement includes not introducing the previously discussed ssb-TimeOffset.

Nokia understand that the field description restricts to RedCap UE, which is not clear from the proposal in the discussion document; they are not sure that the restriction is correct based on the RAN1 agreement that was brought up in the discussion.  CATT think this aligns with the RAN1 parameter list, where the NCD-SSB case is provided only for RedCap UE positioning.  Xiaomi think the restriction is correct.

Nokia can accept the CRs if other companies think it is correct, but they do not quite see where the restriction comes from.  ZTE indicate that it is from RAN1’s agreement, which applies to “at least” RedCap UEs; they understand that this does not give us scope to apply the change to non-RedCap UEs.

CATT indicate that the field description for the nonCellDefiningSSB in RRC is scoped only to RedCap.

Proposal 1: Not introduce any new parameters (including ssb-TimeOffset) in RRCRelease message.

Proposal 2: Update the description of ssb-Ncell to explicitly clarify applicability to both serving-cell and neighbour-cell cases. The change will be introduced starting from Rel-18.

R2-2600321	Correction on the description of ssb-Ncell	CATT, Ericsson, China Telecom, ZTE	CR	Rel-18	38.331	18.8.0	5639	-	F	NR_pos_enh2-Core

Coversheet template to be updated

Agreed as R2-2601222

R2-2600322	Correction on the description of ssb-Ncell	CATT, Ericsson, China Telecom, ZTE	CR	Rel-19	38.331	19.1.0	5640	-	A	NR_pos_enh2-Core

- Coversheet template to be updated

- Agreed as R2-2601223

SL-AoA measurements

R2-2600282	Correction on the field description of sl-AoA-Meas	Xiaomi	CR	Rel-18	38.355	18.6.0	0018	-	F	NR_pos_enh2-Core

Agreed

Discussion:

vivo think it is something of an enhancement, but the CR is all right.

R2-2600283	Correction on the field description of sl-AoA-Meas	Xiaomi	CR	Rel-19	38.355	19.0.0	0019	-	A	NR_pos_enh2-Core

Agreed

Positioning calculation assistance support description

R2-2600320	Correction on description of nr-PosCalcAssistanceSupport in the table of NR-DL-AoD-ProvideCapabilities field descriptions	CATT	CR	Rel-18	37.355	18.7.0	0569	-	F	NR_pos_enh2-Core

Coversheet template to be updated

Agreed as R2-2601224

R2-2601056	Correction on description of nr-PosCalcAssistanceSupport in the table of NR-DL-AoD-ProvideCapabilities field descriptions	CATT	CR	Rel-19	37.355	19.1.0	0571	-	A	NR_pos_enh2-Core

- Coversheet template to be updated

- Agreed as R2-2601225

SRSp frequency hopping bandwidth

R2-2600640	Correction on the bandwidth of positioning SRS frequency hopping-r18	ZTE Corporation	CR	Rel-18	38.331	18.8.0	5655	-	F	NR_pos_enh2-Core

Wording to be changed to “The network always configures this field when srs-PosTx-Hopping is configured”.

Coversheet template to be updated

Agreed with these changes as R2-2601226

Discussion:

Chair wonders if it is a mandatory CR on the network side; ZTE think so.

Ericsson think the network functionally has to provide the BWP if it wants to configure the feature; they note that it is optional Need R, so the network has to keep providing it if there is a reconfiguration, and they do not see the change as essential.

Huawei think it is not a necessary change; there is no problem from the UE perspective and for the network it is up to implementation.

Samsung checked the intention of the RAN1 agreement and agree that the network has to provide the BWP, so they agree with the CR.

ZTE think it is necessary to clarify that FH is always on a separate BWP that needs to be identified.

Ericsson think it is already clear in the field descriptions.

ZTE think there is a functional restriction from the RAN1 agreement, and we need to reflect that; the current field description only says that the field is used when there is a separate BWP.

Nokia think we could make it network implementation guidance rather than using the word “mandatory”.  Ericsson suggest that we could put in the field description of srs-PosConfig that “if network configures srs-PosConfig, then bwp is also configured/provided”.  ZTE find this wording confusing; they could accept something like “network provides this field if srs-PosConfig is configured”.

Toyota note that “The network always configures this field when…” is the phraseology usually used in RRC.

Nokia understand there may be a RAN3 discussion that depends on this.  ZTE do not think the discussions are very tightly linked.

R2-2600641	Correction on the bandwidth of positioning SRS frequency hopping-r19	ZTE Corporation	CR	Rel-19	38.331	19.1.0	5656	-	A	NR_pos_enh2-Core

- Wording to be changed to “The network always configures this field when srs-PosTx-Hopping is configured”.

- Coversheet template to be updated

- Agreed with these changes as R2-2601227

# 8	NR Rel-19

## 8.2	Ambient IoT

(Ambient_IoT_solutions, leading WG: RAN1; REL-19; WID: RP-250796)

Time budget: 0 TU

Tdoc Limitation: 1 tdoc

### 8.2.1	Organizational

LS, Rapporteur input, including workplan, etc.

Incoming LS with RAN2 in Cc:

R2-2600005	LS on the indication to inform the AIoT device is permanently disabled (C1-257575; contact: LGE)	CT1	LS in	Rel-19	AmbientIoT-CT	To:CT1	Cc:RAN2

Incoming LS with “take into account” action only and no related document

R2-2600047	Reply LS on Security parameter in A-IoT paging (SP-251691; contact: CMCC)	SA	LS in	Rel-19	AmbientIoT-SEC	To:RAN2, SA3	Cc:RAN3, CT1

Incoming LS with “take into account” action only and related document

R2-2600039	Reply LS on integrity failure (S3-254709; contact: Xiaomi)	SA3	LS in	Rel-19	AmbientIoT-SEC	To:RAN2	Cc:SA2, RAN3, CT1

R2-2601130	Contact company input on SA3 reply LS in R2-260039	Xiaomi	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 1: No specification change to TS 38.391 is needed for no NAS response due to integrity check failure of A-IoT NAS message (which means to support an indistinguishable AS response (i.e., "MDI = 0 + zero-length SDU") from other NAS failure case).

Proposal 1a: No further LS is needed to SA3 or CT1 (with CT1 Spec already supporting the indication from A-IoT NAS to A-IoT MAC for no NAS response due to integrity check failure).

Other Incoming LS

R2-2600006	Reply LS on Structure updates of AIoT Identifiers (CC4-255349; contact: CICT Mobile)	CT4	LS in	Rel-19	AmbientIoT-ARC, AmbientIoT-CT	To:SA2, RAN2, RAN3	Cc:SA3, CT1

Rapporteur CR to 38.391

R2-2600326	Rapporteur corrections for A-IoT	Huawei, HiSilicon, LG Electronics Inc.	CR	Rel-19	38.391	19.1.0	0002	-	F	Ambient_IoT_Solutions-Core

### 8.2.2	Corrections

Corrections only.  Companies should follow guidance from rapporteurs.

General issues and LS replies

R2-2600327	A-IoT remaining issues	Huawei, HiSilicon	discussion	Rel-19

R2-2600287	Discussion on integrity check failure issue in A-IoT	OPPO	discussion	Rel-19	Ambient_IoT_Solutions

R2-2600334	Remaining Issue on Paging Response for Permanently Disabled AIoT Device	NEC	discussion	Rel-19	Ambient_IoT_Solutions

R2-2600339	Discussion on cross layer interaction for permanent disable command	CATT	discussion	Rel-19	Ambient_IoT_Solutions

R2-2600525	Discussion on LSes from CT1 and SA3	LG Electronics Inc.	discussion	Ambient_IoT_Solutions

R2-2600718	On AS response in case of integrity failure	Nokia	discussion	Rel-19	Ambient_IoT_Solutions

R2-2600938	Discussion on integrity failure issue in R19 A-IoT	CMCC	discussion	Rel-19	Ambient_IoT_Solutions

R2-2600547	Remaining issues in R19 Ambient-IoT	ZTE Corporation, Sanechips	discussion	Rel-19	Ambient_IoT_Solutions

R2-2600785	Remaining issues of Rel-19 Ambient IoT	Qualcomm Incorporated	discussion	Ambient_IoT_Solutions-Core

Corrections

R2-2600228	Remaining issues for Rel-19 A-IoT	Xiaomi	discussion	Ambient_IoT_Solutions-Core

Proposal 1: RAN2 to reply to CT1 that from AS layer perspective, whether the permanently disabled device stops monitoring in A-IoT radio interface can be based on device implementation. Therefore, it is not necessary for the upper layer to provide an indication to the AS informing the device is permanently disabled; however this can be left to device implementation without any spec impact if the upper layer is willing to.

Proposal 2: RAN2 to agree the device shall not monitor Msg2 upon reception of NACK feedback message addressed to the device, and adopt the TP in Table 1 to avoid confusion for device implementation.

R2-2600305	Discussion  on exceptional A-IoT data handling	vivo	discussion	FS_Ambient_IoT_solutions

Proposal 1.	RAN2 to agree the device behavior when it receives a R2D message containing a reserved message type value as follows:

•	dicard the received R2D message;

•	no subsequent D2R message transfer.

R2-2600470	Correction on A-IoT MAC procedures	Apple	CR	Rel-19	38.391	19.1.0	0003	-	F	Ambient_IoT_Solutions

R2-2600571	Corrections on R19 A-IoT	ASUSTeK	discussion	Rel-19	Ambient_IoT_Solutions

R2-2600585	Remaining issues on A-IoT procedures	SHARP Corporation	discussion

Proposal 1: The device considers the CFA procedure is ended when receiving a paging message. And the device, which has no stored AS ID, considers the R2D ULDT message is for it only if the CFA procedure is not considered as ended.

R2-2601029	Corrections to A-IoT MAC	Ericsson	CR	Rel-19	38.391	19.1.0	0005	-	F	Ambient_IoT_Solutions

Withdrawn/Not available

R2-2600595	Open issues for TS 38.391	Ericsson	CR	Rel-19	38.391	19.1.0	0004	-	F	Ambient_IoT_Solutions	Withdrawn

## 8.13	NR sidelink multi-hop relay

(NR_SL_relay_multihop; leading WG: RAN2; REL-19; WID: RP-250188)

Time budget: 0 TU

Tdoc Limitation: 2 tdocs

### 8.13.1	Organizational

LSs and rapporteur input

Rapporteur CR to 38.331

R2-2600435	Corrections for Multihop SLRelay	Huawei, HiSilicon (Rapporteur)	CR	Rel-19	38.331	19.1.0	5642	-	F	NR_SL_relay_multihop-Core

Revised in R2-2601221 (merge with other decisions of this meeting)

Discussion:

CATT note the date is wrong on the coversheet.  ZTE note the coversheet template needs to be updated.

### 8.13.2	Control plane corrections

Impact to 38.331 (except for capability issues), 38.304

R2-2600189	Discussion on correction for Paging request at the Relay UE	OPPO	discussion	Rel-19	NR_SL_relay_multihop

First text proposal to be merged into the revised rapporteur CR

Second text proposal to be updated to scope the behaviour to the last relay UE, then merged into the revised rapporteur CR

Noted

Discussion:

Apple this the first TP is OK, but for the second TP they understand that only the last relay UE will monitor paging.  OPPO intend that the change does not restrict whether it is last or intermediate.  Apple think it should be restricted to the last relay UE.  ZTE think the behaviour mentioned by Apple is for idle/inactive, and the SUI is for connected mode.

ZTE think in the “else” branch of TP2, when the UE receives a release from its child UE, it will ask the network to release the paging information, and this may not be correct if there are multiple child UEs; they suggest “the UE shall update the information”.

Qualcomm agree with Apple’s comment: We do not require the intermediate relay UE to monitor paging, so the intermediate relay UE should always forward the paging information to the last relay UE, and we should not require the intermediate relay UE to add paging monitoring.

OPPO think we need to check the paging request procedure in general to make sure the restriction to the last relay UE is there.

Proposal 1	RAN2 to agree the corrections for Paging request at the Relay UE as above TP1 and TP2.

R2-2600436	Harmonization of Multi-hop Relay Definitions in TS 38.331	Huawei, HiSilicon	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

Discussion:

Qualcomm think this is not just a definitional change but affects the functionality.  Apple have the same view; it is a valid stage 2 statement but not part of the definition, and they would rather remove it from 38.300 if anything.

Toyota think if we keep this sentence, it should not go in the definition.

Proposal 1: The definitions of First U2N Relay UE and Intermediate U2N Relay UE in TS 38.331 should be aligned with the definition in TS 38.300, as indicate in the text proposal in the Annex 1.

R2-2600469	Miscellaneous RRC Corrections for Multi-hop SL Relay	Apple	CR	Rel-19	38.331	19.1.0	5643	-	F	NR_SL_relay_multihop

In change 1, “child UEs beyond next hop” should be “remote UEs beyond next hop”

Changes 3 and 4 to be checked offline

Changes 1, 2, 5, 6, 7 to be merged into rapporteur CR

Merged into R2-2601221

Discussion:

OPPO understand on the first change, the terminology is clear already and aligned across multiple specs, so maybe the procedural text should be checked instead of changing the definition.  Apple think the child UE terminology is used for paging forwarding as well as communication; they understand that relay “communication” refers only to user data.  OPPO think we should avoid using “child” for indirectly connected UEs and instead use terminology like “indirectly connected remote UE”.  OPPO think there are dependencies in SRAP and maybe stage 2 on the current terminology.

Samsung and OPPO think we could change the second sentence in the first change to refer to “remote UEs beyond the next hop”.

OPPO think on the third change, it is up to the network to enforce and no spec change is needed.  Apple understand that we have redundant requirements today.  ZTE have the same view as OPPO and think the single-hop field refers to Rel-17 single-hop operation, as distinct from a multihop service with only one relay UE.  Apple understand that if the single-hop indication is not present, the ASN.1 conditions clarify that the multihop indication cannot be set, so the single-hop condition is redundant.

OPPO think on change 4, the current specification is not wrong; they understand that the two bullets are for model A and model B.  Apple understand that the second bullet is a subset of the first.  OPPO think the difference is in the condition on having a PC5 connection.

OPPO wonder about the intention of change 6.  Apple indicate that the indirectly-connected UEs will be reported under the individual destinations.

[AT133][401][Relay] Checking of changes from R2-2600469 (Apple)

Scope: Check changes 3 and 4 from R2-2600469 and determine if they are acceptable to merge into the rapporteur CR.

Intended outcome: Report to CB session

Deadline: Wednesday 2026-02-11 1900 CET

R2-2600569	Correction to multi-hop L2 U2N relay	CATT	discussion	Rel-19	NR_SL_relay_multihop-Core

TPs to be merged into rapporteur CR

Noted

Proposal 1: Add the procedure to include sl-PagingDelivery in UuMessageTransferSidelink message if the Paging message received by L2 U2N Relay UE from network containing the ue-Identity of the Child UE. Agree the TP1 in Annex 5.1.

Proposal 2: Add the action to L2 Intermediate U2N Relay UE when paging message/SIBs(other than SIB1) of the child UE is received/acquired. Agree the TP2 in Annex 5.2.

R2-2600797	Correction on RemoteUEInformationSidelink for multi-hop relay	ZTE Corporation, Sanechips	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

Discussion:

CATT have some concern on P2 in relation to the first document discussed, on the “child UE” terminology.

Samsung and Huawei think the first change scopes the whole operation to idle/inactive, which is not correct.

Toyota wonder on P3 what happens if the intermediate relay does not monitor paging; will the child UE get the paging from somewhere else?  ZTE clarify it will come from the last relay UE.

Qualcomm cannot agree to any of the proposals; they understand that we agreed not to specify these aspects of intermediate relay UE behaviour.  ZTE think P3 fixes a place where monitoring is made mandatory for the intermediate relay UE.  Qualcomm are not sure anything needs to be captured for this.

OPPO understand that the intermediate relay UE should send the remote UE information to the parent for all states.  ZTE agree with OPPO on this point, but they think some updates are needed in the text.

ZTE think we could still take P3.  Apple think there are pieces of the procedural text that could be modified and we would not need the NOTE.  Qualcomm would prefer to scope the requirement to monitor only to the last relay UE, and say nothing about the intermediate.

Proposal 1.	For intermediate relay UE, modify the description to clarify the transmission of RemoteUEInformation shall be restricted to IDLE/INACTIVE intermediate relay UE. See TP in Annex clause.

Proposal 2.	When entering into RRC connected state, relay UE will release the sl-PagingInfo-RemoteUE-List. See TP in Annex clause.

Proposal 3.	Clarify that once the intermediate relay UE moves in the cell coverage, it is left to intermediate relay UE implementation whether to start monitoring paging directly for its child UEs. See TP in Annex clause.

R2-2600806	Correction on Notification Message for multi-hop relay	ZTE Corporation, Sanechips	discussion	Rel-19	NR_SL_relay_multihop-Core

P2 and P3 to be merged into rapporteur CR

Noted

Discussion:

Chair thinks the third change also requires adding the new fields in level 5 bullets; ZTE agree this should be done.

Kyocera think the first change is not necessary because the child UE will already be triggered to do relay reselection.

OPPO understand the intention of P1 but think the current text in the NOTE is not clear; they are concerned about two notifications separated by a long interval.  ZTE agree with Kyocera that the double-trigger is not a functional problem, but they think we need implementation guidance that it may be suppressed.

Toyota wonder if the NOTE is correctly phrased since it is from the perspective of the child UE rather than the intermediate relay UE.

Proposal 1.	To avoid double trigger, clarify in specification that the notification message may be omitted if there is a notification message which has been sent to child UE.

Proposal 2.	Capture the missing UE behaviour for submitting the notification message in clause 5.8.9.10.3 if the UE is acting as U2N Relay UE in case of single hop or Last U2N Relay UE. See TP in Annex clause.

Proposal 3.	Capture the missing UE behaviour for reception of mh-indicationType/sl-IndicationType in notification message in clause 5.8.9.10.4. See TP in Annex clause.

R2-2600817	Multi-hop Relay and Intermediate/Last Relay operation	TOYOTA ITC, ZTE, FirstNet, Apple	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

Discussion:

Huawei think no change is needed and this would be a network error case.

Chair wonders if something will go wrong in a real deployment.

Apple understand that the threshold has to be guaranteed by the network, and maybe we could capture a NOTE.  ZTE think it is useful implementation guidance.

OPPO understand that the agreement last meeting was to use the Rel-17 threshold value, and if we have to change something it should go back to Rel-17; they see it as the same issue as having the remote and relay thresholds overlap in Rel-17.

Qualcomm think we could capture something in the field descriptions.

Toyota see that if the UE detects two potential relays, one intermediate and one last, with different RSRP values, it is possible that both conditions could be met simultaneously.  They think it is not safe to rely on the meeting minutes for something that could result in misbehaviour in the field; they are concerned about a UE trying to act in both roles at the same time, as well as latency if a UE takes an extra hop to the network.

Kyocera wonder if the discovery process can already resolve this by selecting a path with fewer hops.

Qualcomm think the upper layer specifies that the UE can consider number of hops in selecting a candidate relay.

OPPO think from upper-layer perspective, the intermediate and last relays will have different authorisations and be handled separately, so they think the error case will not happen.

Proposal 1. Reflect in TS 38.331 that if the threshold conditions for acting as intermediate relay and last relay are met, the UE should prioritize acting as a last relay over intermediate relay. Adopt the TP in the Annex in Clause 5 in this document.

Withdrawn/Not available

R2-2600240	Correction to multi-hop L2 U2N relay	CATT	CR	Rel-19	38.331	19.1.0	5634	-	F	NR_SL_relay_multihop-Core	Withdrawn

### 8.13.3	User plane corrections

Impact to 38.351, 38.321, and 38.323.

### 8.13.4	Other corrections

Impact to specs not listed above, including capability aspects of 38.331.

R2-2600188	Discussion on Stage-2 corrections for multi-hop U2N Relay	OPPO	discussion	Rel-19	NR_SL_relay_multihop

Postponed

R2-2600238	TP for TS 38.300 to limit resource allocation mode for U2N intermediate relay	NEC Corporation	discussion	NR_SL_relay_multihop

Postponed

R2-2600437	Corrections for Multi-hop Relay in 38.300	Huawei, HiSilicon	discussion	Rel-19	NR_SL_relay_multihop-Core

Postponed

R2-2600570	Correction on procedure for multi-hop L2 U2N Remote UE connection establishment	CATT	discussion	Rel-19	NR_SL_relay_multihop-Core

Postponed

Withdrawn/Not available

R2-2600241	Correction on procedure for multi-hop L2 U2N Remote UE connection establishment	CATT	CR	Rel-19	38.300	19.1.0	1087	-	F	NR_SL_relay_multihop-Core	Withdrawn

## 8.15	NavIC L1 SPS A-GNSS support

(LCS_NAVIC_L1_SPS_NR_LTE-Core; leading WG: RAN2; REL-19; WID RP-251552

Time budget: 0 TU

Tdoc Limitation: 1 tdoc

Corrections to all specs.

## 8.16	BDS B2b in A-GNSS

LCS_BDS_B2b_LTE_NR; leading WG: RAN2; REL-19; WID RP-250767)

Time budget: 0 TU

Tdoc Limitation: 1 tdoc

Corrections to all specs.

## 8.19	TEI19

Time budget: 0 TU

Tdoc Limitation: 1

No new proposals expected for TEI19.

Companies are encouraged to submit co-sourced contributions, which will have priority for discussion in RAN2#133

### 8.19.2	Other WG-led

R2-2600198	Discussion on S2-2511306 of L3 U2U and support of L3 U2N relay for multi-hop relay	ZTE Corporation, Sanechips	discussion

Postponed (wait for TEI20 to open)

R2-2600300	Discussion on SA2 LS in S2-2511306	OPPO	discussion	Rel-20	TEI20

Postponed (wait for TEI20 to open)

# 9	NR Rel-20

## 9.2	Ambient IoT Ph2

(Ambient_IoT_Solutions_Ph2, leading WG: RAN1; REL-20; WID: RP-252105)

Time budget: 1 TU

Tdoc Limitation: 1 tdoc

### 9.2.1	Organizational

R2-2600040	LS on scope alignment for R20 AIoT (S3-254759; contact: OPPO)	SA3	LS in	Rel-20	FS_AIoT_SEC_Ph2	To:SA2, RAN2

R2-2600700	[Draft] Reply LS on scope alignment for R20 AIoT	OPPO	LS out	Rel-20	Ambient_IoT_Solutions	To:SA3	Cc:SA2

### 9.2.2	Topology 2

Contributions on support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions.  Only for traffic types DO-DTT and DT.

Scenarios and support indications

R2-2600308	Discussion on Topology 2	vivo	discussion	Ambient_IoT_Solutions_Ph2

Proposal 1.	UE reports its UE Reader capability for A-IoT operation. The exact set capability parameters are FFS.

Proposal 2.	Introduce an indicator in SIB1 to indicate a cell supports the UE reader operation.

Proposal 3.	The UE acting as an A-IoT reader should indicate its UE reader role to the gNB during the Uu access procedure.

R2-2600319	Discussion on D2T2 in A-IoT	SHARP Corporation	discussion

Proposal 1:	As a baseline, a UE Reader supports only A-IoT radio interface operations for Device 1 (i.e. as already specified in Rel-19).

	Support for Device 2 should be explicitly indicated by the UE Reader.

R2-2600937	Discussion on Topology 2 for A-IoT	CMCC	discussion	Rel-20	Ambient_IoT_Solutions_Ph2

Proposal 1: The scenario where the same gNB supports both Topology 1 and Topology 2 for device 1 should not be considered in R20.

Proposal 28: RAN2 is asked to take the procedure given in Figure 3 as a starting point for the inventory and command procedure in Topology 2.

Identifiers

R2-2600288	Discussion on topology 2 for A-IOT	OPPO	discussion	Rel-20	Ambient_IoT_Solutions_Ph2

Proposal 1: Transaction ID of the A-IOT service shall be generated by the gNB as the central manager and conveyed to the UE Reader in the RRC inventory request related message.

Proposal 2: RAN2 to discuss whether gNB-based solution or UE-based solution to be adopted to allocate A-IOT device Uu interface IDs to UE Readers.

Proposal 3: A-IOT device Uu interface ID shall be at least 16-bit long to align with the AS ID.

R2-2600432	Topology 2 for AIOT	InterDigital, Inc.	discussion	Rel-20	Ambient_IoT_Solutions_Ph2-Core

Proposal 2:	The gNB provides both the correlation ID and a UE reader ID to be used by the UE reader to generate the transaction ID. FFS on how they are combined (e.g., number of bits from each).

Proposal 4: 	The AS ID is reported by the UE reader to the gNB with inventory results and is used by the gNB to reference a specific device in subsequent command procedures.

Proposal 5: 	The UE reader assigns a new AS ID (i.e., in MSG2) to a device if that device’s random ID in MSG1 conflicts with an already assigned/maintained AS ID at the UE reader.

R2-2600468	Discusion on Topology 2	Apple	discussion	Rel-20	Ambient_IoT_Solutions_Ph2

Proposal 2 	An 8-bit or 12-bit ID is introduced between UE reader and gNB to associate with a specific device for command procedure, when needed.

Resource allocation: basic mechanisms

R2-2600227	Further discussion on the support of A-IoT Topology 2	Xiaomi	discussion	Rel-20	Ambient_IoT_Solutions_Ph2-Core

Proposal 10: The A-IoT resource configured by the gNB is a set resources used in A-IoT radio interface which are continuous in time domain and are comprised of contiguous frequency domain resources in the UL spectrum. The valid duration of the A-IoT resource configuration is determined, either by an explicit release indication from the gNB or by the expiry of a resource validity timer (depending on final conclusion of Proposal 5). Do not pursue periodical A-IoT resource configuration recurring periodically in time domain.

Proposal 11: Uu RRC configuration parameters for A-IoT resource configuration is up to RAN1. Send LS to RAN1, informing RAN2 assumption to RAN1 on how A-IoT resource configuration in Topology 2 looks like (taking Proposal 10 as RAN2 assumption) and requesting RAN1 to work on specific Uu RRC configuration parameter design.

Proposal 12: UE reader may provide assistance information to request addition/modification of A-IoT resources configured by the gNB. RAN2 further discusses the specific information signalled (e.g. number of frequency/time resources requested, number of retransmission needed, etc.), and the trigger for this assistance information report.

Proposal 13: In Topology 2, UE reader in the configured A-IoT radio resources:

•	autonomously selects the resources for the transmission of each R2D message (including A-IoT paging, Access Trigger, Msg2 and R2D Upper Layer data transfer);

•	determines access occasions and signals the AO related information in A-IoT paging message/Access Trigger message to the devices, each of which selects the AO for a Msg1 transmission following Rel-19 CBRA procedure;

•	schedules the resources for the transmission of "D2R Upper layer data transfer", and signals the scheduled D2R resources info to the device via R2D message.

R2-2600526	Discussion on Topology 2 for AIoT	LG Electronics Inc.	discussion	Ambient_IoT_Solutions_Ph2

Proposal 4. The following AIoT resource allocation methods should be supported.

-	The UE reader receives the AIoT radio resources configuration in RRC signalling. The AIoT radio resources remain valid until the network releases them explicitly.

-	The UE reader receives the AIoT radio resources configuration in RRC signalling, which configures a time period in which the corresponding resource can be used. The UE reader considers that the AIoT radio resources remain valid for that time period, unless the resource configuration is explicitly released by the network.

R2-2600548	Discussion on Ambient-IoT topology 2	ZTE Corporation, Sanechips	discussion	Rel-20	Ambient_IoT_Solutions_Ph2

Proposal 15: If CW is transmitted from UE reader, a whole set of AIoT resources without differentiating R2D/D2R can be allocated to UE reader. Scheduling related information, e.g., time duration of continuous AIoT resources and optionally periodicity of the resources is beneficial to be provided as assistance information to gNB.

Proposal 16: If CW is transmitted from outside the topology, gNB can allocate two sets of resources for R2D and D2R respectively to UE reader. Scheduling related information (e.g., time duration of continuous D2R/R2D resources and optionally periodicity) is beneficial to be provided as assistance information to gNB.

Proposal 17: RAN2 to send an LS  to RAN4 to confirm which of the cases studied during SI phase and listed in TR 38.769 applies to topology 2 in R20, i.e. whether CW is transmitted from inside or outside the topology.

Resource allocation: out-of-connection and service continuity

R2-2600280	Resource configuration, utilization and control for topology 2 readers	Qualcomm Incorporated	discussion	Rel-20	Ambient_IoT_Solutions_Ph2-Core

Proposal 2:  	The temporary out-of-connection A-IoT resource validity timer is configured by RRC dedicated signaling as part of A-IoT resource configuration.

Proposal 3:  	The temporary out-of-connection A-IoT resource validity timer is started when UE detects RLF, when HO command is received, or when HO event is triggered.

Proposal 4:  	Continuity of an ongoing A-IoT session (e.g., usage/reporting of the A-IoT results up to that point) temporarily interrupted by a connected mode mobility or RLF event is supported.

Proposal 5:  	The UE reader stores and transfers the A-IoT intermediate results, and other necessary session related information to the target cell following a successful handover or re-establishment.

R2-2600328	A-IoT T2 discussion	Huawei, HiSilicon	discussion	Rel-20

Proposal 3:	RAN2 to confirm that the resource validity timer for HO or RLF scenarios is an optional configuration.

Proposal 4:	The starting condition of OoC A-IoT resource validity timer is upon reception of network configuration (i.e., to configure A-IoT resource and the validity timer).

Proposal 5:	As a baseline, the UE reader releases the A-IoT service related configuration/context after completing HO or RRC reestablishment.

Reader Uu operation (inc. coordination on Uu and A-IoT interfaces)

R2-2600335	Discussion on Topology 2 for Ambient IoT	NEC	discussion	Rel-20	Ambient_IoT_Solutions_Ph2

Proposal 6:	Incorporate A-IoT MAC layer channel activities into data inactivity monitoring.

Proposal 7:	UE reader assembles all data segments belonging to the same upper-layer data transfer for the Ambient IoT device.

R2-2600432 revisited (InterDigital)

Proposal 16:	An intermediate UE performs legacy Uu operations on slots which are not associated with allocated/activated resources for AIOT.

Proposal 17:	Support allocated/activated AIOT resources overlapping with usable Uu resources, and decide which alternative(s) to use to satisfy WID restriction: 1) defining slot-based prioritization rules between an AIOT TX/RX and Uu TX/RX in a given slot and 2) suspend AIOT operations for a period of time when Uu data TX/RX is performed on slots with allocated/activated AIOT resources.

R2-2600596	Aspects for Ambient IoT Topology 2	Ericsson	discussion	Ambient_IoT_Solutions_Ph2

Proposal 10	Use existing SRB for delivery of A-IoT resource management related information in both UL and DL signalling. FFS details (including whether existing RRC messages are reused or not).

Proposal 11	Define a new SRB for delivery of A-IoT upper layer data. FFS details.

R2-2600308 revisited (vivo)

Proposal 6.	Introduce a new SRB, e.g., SRBx, to transfer RRC messages containing A-IoT data or related information between an A-IoT UE reader and its serving gNB.

Proposal 7.	The New SRBx can be configured to an A-IoT UE reader only after AS security activation.

Proposal 8.	For an A-IoT UE reader, a configuration with new SRBx for A-IoT data, without any DRB/MRB, is supported.

Message contents

R2-2600328 revisited (Huawei)

Proposal 6a:	For topology-2, introduce two new Uu RRC messages, one for UL and the other for DL, in total between UE reader and BS, and inside the RRC message, each IE-level signalling is corresponding to one NGAP message.

R2-2600340	Discussion on Topology-2 for Ambient IoT	CATT, CBN, China Broadnet	discussion	Rel-20	Ambient_IoT_Solutions_Ph2

Proposal 11: The content of the RRC Msg for command request should include the following information,

- Transaction ID;

- Multiple device entries with each one including:

- AS ID;

- A-IoT NAS PDU (upper layer command);

- Command assistance information.

Proposal 12:	The content of the RRC Msg for command response should include the following information,

- Transaction ID;

- Multiple device entries with each one including:

- AS ID;

- A-IoT NAS PDU (upper layer command response).

R2-2600319 revisited (Sharp)

Proposal 4:	Time Interval as part of the Inventory Assistance Information is sent (if available) to the UE reader.

Proposal 5:	UE reader performs Inventory report aggregation when request by AIOTF.

Parallel sessions and multi-reader cases

R2-2600340 revisited (CATT)

Proposal 2: Topology-2 follows the same principle as Topology-1 for parallel service requests, i.e., it’s up to UE reader implementation to execute the multiple A-IoT sessions in sequence, which are triggered by one RRC Msg for inventory request.

R2-2600379	Topology 2 aspects	Nokia	discussion	Rel-20

Proposal 8: RAN2 to assume that at least Rel-20 devices other than Type 1 devices may be served by multiple readers. FFS details and possible inclusion of device Type 1.

Proposal 9: Reader ID is supported.

Proposal 10: RAN2 to send an LS to RAN1 whether preambles can be used to indicate paging/reader/transaction ID.

Other contributions

R2-2600212	Discussion on A-IoT deployment scenario for D2T2	Tejas Network Limited	discussion	Rel-20

R2-2600483	Discussion on Topology 2 for A-IoT	China Telecom	discussion	Rel-20	Ambient_IoT_Solutions_Ph2

R2-2600511	Discussion on Topology 2 for AIoT	HONOR	discussion	Rel-20	Ambient_IoT_Solutions_Ph2

R2-2600574	Discussion on Topology 2 for A-IoT	ASUSTeK	discussion	Rel-20	Ambient_IoT_Solutions_Ph2

R2-2600664	Discussion on A-IoT topology 2	Spreadtrum, UNISOC	discussion	Rel-20

R2-2600744	Discussion for Topology 2 for Rel-20 Ambient IoT	Lenovo	discussion	Rel-20

R2-2600758	Discussion on Topology 2 resource handling for Ambient IoT	Sony	discussion	Rel-20	Ambient_IoT_Solutions_Ph2

R2-2600784	Considerations for Deployment Scenario 2 with Topology 2	Panasonic	discussion

R2-2600829	Discussion on Topology 2 for AIoT	AUMOVIO	discussion

R2-2600867	RAN2 impacts to support D2T2 for DT and DO-DTT traffic	Ofinno	discussion	Rel-20	Ambient_IoT_Solutions_Ph2

R2-2600928	Consideration of A-IoT resource management for Topology 2 	Kyocera 	discussion	Rel-20

R2-2601038	Discussion on Topology 2 for Ambient IoT	CEWiT	discussion	Rel-20	Ambient_IoT_Solutions_Ph2

R2-2601090	Discussion on Topology 2 for A-IoT	KT Corp.	discussion	Rel-20	Ambient_IoT_Solutions_Ph2