3GPP TSG-RAN2 Meeting #132	R2-2509348

Dallas, USA, 17 - 21 November 2025

Title:	DRAFT Reply LS on CN assigned subgroup ID for LP-WUS

Response to:	LS (S2-2509834/R2-2508030) Reply LS on CN assigned subgroup ID for LP-WUS

Release:	Rel-19

Work Item:	NR_LPWUS-Core

Source:	Ericsson [TSG RAN WG2]

To:	SA2, RAN3

Cc:	RAN1

Contact Person:

#### Name:	Martin van der Zee

E-mail Address:	martin.van.der.zee@ericsson.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN2 thanks SA2 for the LS on CN assigned subgroup ID for LP-WUS. RAN2 would like to provide the following answers to the questions:

- What is the typical area that a PO-to-LO association configuration will apply to? (e.g., is the configuration uniform in a PLMN, different in every cell, etc, and is it mandatory for an LP-WUS capable gNB to support all 3 PO-to-LO association configurations?) This information will help SA2 to understand what kind of solution to propose.

Answer to a):

- The PO-to-LO association configuration must be homogeneous at least within the same registration area. But the PO-to-LO association configuration may also be homogeneous in the complete PLMN, e.g. when UEs have different registration areas. The same applies to the number of subgroups per PO and the number of UEID-based subgroups.

- The PO-to-LO association configurations the gNB supports is up to gNB implementation.

b) 	Whether the paging PO-to-LO association configuration will be changed often, e.g., to save base station energy when the paging load is less?

Answer to b):

- The PO-to-LO association configuration is not expected to change often, if at all.

- When the PO-to-LO association configuration changes some UEs that have a CN-assigned subgroup ID may fail to receive paging because the RAN no longer supports that CN-assigned subgroup ID.

c) 	Will RAN3 specify the relation between subgroup ID value range and the PO-to-LO association (e.g. based on RAN1 specification)? This could avoid SA2 having to refer to a complex part of TS 38.213.

Answer to c):

- This is for RAN3 to answer.

2. Actions:

To SA2

ACTION: 	RAN2 kindly asks SA2 to take note of the answers provided by RAN2.

3. Date of Next TSG-RAN2 Meetings:

TSG RAN2 Meeting #133		9 – 13 February 2025		Gothenburg

TSG RAN2 Meeting #133-bis	13 – 17 April 2025		Malta