3GPP TSG-RAN WG2 Meeting #132                                    	R2-2509201

Dallas, USA, November 17th-21st, 2025

Agenda item:	6.1.3.1

Source:	Qualcomm Incorporated

Title:	Report of [AT132][302][R17 NR NTN] Epoch Time

Document for:	Discussion and Decision

# Introduction

This document provides the summary of the following discussion.

[AT132][302][R17 NR NTN] Epoch time (QC)

Scope: continue the discussion on Epoch time based on CR 5607

Intended outcome: Summary of the offline discussion

Deadline for rapporteur's summary (in R2-2509201):  Thursday 2025-11-20 08:00

In this document, we discuss remaining issues.

# Discussion

In RAN2#132, following understanding is captured regarding the interpretation of epoch time for neighbor cell.

RAN2 understands that if the epoch time is absent for the neighbor cell NTN-NeighCellConfigList and when it uses the epoch time of the serving cell, the absolute time of the epoch time is same for the serving cell and neighbor cell (continue in offline 302 to discuss whether any spec change is needed)

The reason to confirm this understanding is to avoid any confusion on the use of SFN value for epoch time as the epoch time for serving cell can be only in present or in future. But for the neighbor cell, the epoch time can be in past, present or future as specified in the field description of explicit epoch time, i.e., SFN and subframe.

NTN-Config field descriptions

ephemerisInfo This field provides satellite ephemeris either in format of position and velocity state vector or in format of orbital parameters. This field is excluded when determining changes in system information, i.e. changes to ephemerisInfo should neither result in system information change notifications nor in a modification of valueTag in SIB1.

epochTime Indicate the epoch time for the NTN assistance information. When explicitly provided through SIB, or through dedicated signaling, the EpochTime is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number signaled together with the assistance information. For serving cell, the field sfn indicates the current SFN or the next upcoming SFN after the frame where the message indicating the epochTime is received. For neighbour cell, the sfn indicates the SFN nearest to the frame where the message indicating the epochTime is received. The reference point for epoch time of the serving or neighbour NTN payload ephemeris and Common TA parameters is the uplink time synchronization reference point of the serving cell. In case of handover or conditional handover, the reference point for epoch time of the target NTN payload ephemeris and Common TA parameters is the uplink time synchronization reference point of the target cell. If this field is absent for the serving cell, the epoch time is the end of SI window where this SIB19 is scheduled. This field is mandatory present when ntn-Config is provided in dedicated configuration. If this field is absent in ntn-Config provided via NTN-NeighCellConfig the UE uses epoch time of the serving cell, otherwise the field is based on the timing of the serving cell, i.e. the SFN and sub-frame number indicated in this field refers to the SFN and sub-frame of the serving cell. In case of handover or conditional handover, this field is based on the timing of the target cell, i.e. the SFN and sub-frame number indicated in this field refers to the SFN and sub-frame of the target cell. For the target cell the UE considers epoch time, indicated by the SFN and sub-frame number in this field, to be the frame nearest to the frame in which the message indicating the epoch time is received. This field is excluded when determining changes in system information, i.e. changes to epochTime should neither result in system information change notifications nor in a modification of valueTag in SIB1.

If we look at the explicit epoch time for serving cell and neighbor cell (for example, as given in Rel-17 TS 38.331), the interpretation of use SFN is different.

For neighbour cell, the sfn indicates the SFN nearest to the frame where the message indicating the epochTime is received

To avoid confusion, one clarification that can be added is that following interpretation is only for the case epoch time is explicitly indicated in NTN-NeighCellConfig for neighbor cell. One example of clarification is following:

For neighbour cell when explicit epoch time is present, the sfn indicates the SFN nearest to the frame where the message indicating the epochTime is received

Q: Do you agree to have some clarification in the specification regarding this. If yes, please elaborate you suggestion:

| company | Yes/No | Comments/Suggestion |
| --- | --- | --- |
| Apple | See comments | From our understanding, the following text is saying UE uses the absolute time point of epoch time of the serving cell (the current SFN or the next upcoming SFN after the frame), but not saying UE refers to the epochTime field for serving cell and re-interprets it (as the nearest SFN) for neighbour cell. Perhaps we could first try to achieve if this is the common understanding. If yes, no further clarification is needed. If this field is absent in ntn-Config provided via NTN-NeighCellConfig the UE uses epoch time of the serving cell, |

# Conclusion

Following proposals are made.

# No table of contents entries found.