3GPP TSG-RAN WG2 Meeting #132                                                                    R2-2509207

Dallas, USA, November 17th-21st, 2025

Agenda item:	8.17

Source:	CATT

Title:	Report of [AT132][304][IoT NTN TDD] SPS

Document for:	Discussion and Decision

# Introduction

This document provides the summary of the following discussion.

[AT132][304][IoT NTN TDD ] SPS (CATT)

Scope: discuss whether/which solution we need for SPS in IoT NTN TDD

Intended outcome: summary of the offline discussion

Deadline for rapporteur's summary (in R2-2509207):  Friday 2025-11-21 08:00

In this document, we discuss remaining issues.

# Discussion

In RAN2#129bis meeting [1], RAN2 discussed the issue of UL SPS overlaps with non-U NB-IoT subframe in IoT NTN TDD mode and reached the following agreement.

RAN2#129bis Agreement When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe

In the last meeting, the SPS collision issue due to SPS postponement was discussed but not concluded [2]:

RAN2#131bis Agreement Come back to the SPS collision issue (due to SPS postponement) in the next meeting.

Based on companies’ contribution and the online discussion, candidate solutions are summarized as following:

- Option 1: Not support SPS for IoT NTN TDD.

- Option 2: It relies on NW implementation to avoid SPS collision issue as much as possible, e.g., configure different frequency resources for potentially conflicting UEs. [3-6]

- Option 3: the values of semiPersistSchedIntervalUL-r15 are interpreted as new values that are multiple of 90ms, e.g, sf128 can be interpreted as value sf180. [7]

- Option 4: Distinguish the postponed SPS positions by configuration.

  - Option 4a: UE is configured with a subframe id within the 8 UL NB-IoT subframes [8].

  - Option 4b: UE is configured with a UL-SPS-postponed-offset to offset the SPS transmission to one of the 8 consecutive uplink subframes. [9]

Q1: On the issue of how to handle the SPS collision issue (due to SPS postponement), which option do you prefer?

| Company | Option | Comments |
| --- | --- | --- |
| Nordic | 1 | I really do not see the use case for R15 SPS-based scheduling request config in NTN TDD mode. However, we should not say “Not support SPS for IoT NTN TDD” but rather “Not support SPS-based scheduling request for IoT NTN TDD”, since we are not talking entire SPS concept but R15 SPS-based scheduling request support. |
| Thales | 1 | Same view as Nordic. First all, the SPS seems to not have interest for Iridium’s use cases since it is limited to BSR for NB-IoT. It can be revised if IMS over NB-IoT NTN is applicable to non-GSO scenario and if it is applicable to IoT NTN TDD. |
| Samsung | 1 | In rel-20 we will introduce a more useful type of SPS in Rel-20. If companies want it to be supported for IoT NTN TDD, they can bring contributions to this (we do not see an issue if companies think that “real” SPS is important for IoT NTN TDD and want to discuss it in Rel-20). For rel-19, the network based scheduled “SPS”. In other words the network continuously schedules the UE according to the TDD mode using PDCCH, which is much more reasonable. |

Summary:

[TBD]

For option 2, the intra-UE collision issue may need to be considered additionally, i.e., collision between different SPS occasions for the same SPS configuration (due to SPS postponement). For this issue, two options are on the table:

- Option 2-1: The NW can predict the potential collision and it can configure a larger SPS resource to accommodate multiple packets. In case of SPS collision, multiple packets are allowed to be re-combined and transmitted on the single SPS occasion. [4]

- Option 2-2: It relies on UE implementation to transmit the packets in the MAC queue in order, e.g., transmit the first packet in the collision SPS resource. Further enhancement is not needed.

Q2: If option 2 is agreeable for Q1, which option do you prefer to handle the intra-UE collision issue?

| Company | Option | Comments (if enhancement is needed, please gives specific spec impacts) |
| --- | --- | --- |

Summary:

[TBD]

# Conclusion

Following proposals are made.

# Reference

- RAN2#129bis ChairNotes.

- RAN2#131bis ChairNotes.

- R2-2508160	Discussion on SPS collision issues for IoT NTN TDD	CATT	discussion	Rel-19	IoT_NTN_Ph3-Core

- R2-2508278	Remaining issues for IoT NTN TDD	Huawei, HiSilicon	discussion	Rel-19	IoT_NTN_TDD

- R2-2508912	Discussion on remaining issue for IoT NTN TDD mode	ZTE Corporation,  Sanechips	discussion	Rel-19.

- R2-2508915	Discussion on the remaining issues for the IoT NTN TDD	Xiaomi	discussion	Rel-19	IoT_NTN_TDD

- R2-2508811	SPS for NB-IoT NTN TDD mode	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_TDD

- R2-2508299	Discussion on SPS collision in IoT NTN TDD mode	OPPO	discussion	Rel-19	IoT_NTN_TDD

- R2-2508314	Discussion on remaining open issues for IoT NTN TDD	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_TDD