3GPP TSG-RAN WG2 Meeting #132	DRAFT R2-2509210

# Dallas, USA, 17th – 21st November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR | 1444 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Assistance info for IoT-NTN to NR-NTN Cell Selection [IoT_NR_NTN_CellSelect] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | EchoStar, Boost Mobile Network, Qualcomm, Aalyria, Terrestar, Skylo, Sateliot, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-11-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To enable an enhanced mechanism for inter-RAT cell selection between IoT NTN and NR NTN. This ensures that UEs supporting both NR-NTN (or NR-TN) and IoT NTN can effectively perform NR-NTN cell selection as needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Description of assistance information for cell selection/idle mobility to NR NTN from IoT NTN (BL UEs, UE in CEs and NB-IoT UE connected to E-UTRAN over NTN) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inter-RAT cell selection to NR NTN for UEs with dual NTN RAT capabilities (NR-NTN and (NB-)IoT NTN) may be less efficient without assistance info and may result in service interruption or degraded performance. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4, 6.19 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS36.331 CR 5182, TS36.306 CR 1935 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: adding section 7 as impacted section |  |  |  |  |  |  |  |  |

Start of change

<<Unchange parts are omitted>>

## 7.4	System Information

System information is divided into the MasterInformationBlock (MIB) and a number of SystemInformationBlocks (SIBs):

-	MasterInformationBlock defines the most essential physical layer information of the cell required to receive further system information;

-	SystemInformationBlockPos contains positioning assistance data;

-	SystemInformationBlockType1 and SystemInformationBlockType1-BR (for a BL UE or UE in enhanced coverage) contain information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information blocks;

-	SystemInformationBlockType2 contains common and shared channel information;

-	SystemInformationBlockType3 contains cell re-selection information, mainly related to the serving cell;

-	SystemInformationBlockType4 contains information about the serving frequency and intra-frequency neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SystemInformationBlockType5 contains information about other E-UTRA frequencies and inter-frequency neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters). It can also contain information about E-UTRA and NR idle/inactive measurements;

-	SystemInformationBlockType6 contains information about UTRA frequencies and UTRA neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SystemInformationBlockType7 contains information about GERAN frequencies relevant for cell re-selection (including cell re-selection parameters for each frequency);

-	SystemInformationBlockType8 contains information about CDMA2000 frequencies and CDMA2000 neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SystemInformationBlockType9 contains a home eNB name (HNB name);

-	SystemInformationBlockType10 contains an ETWS primary notification;

-	SystemInformationBlockType11 contains an ETWS secondary notification;

-	SystemInformationBlockType12 contains a CMAS warning notification;

-	SystemInformationBlockType13 contains MBMS-related information;

-	SystemInformationBlockType14 contains information about Extended Access Barring for access control;

-	SystemInformationBlockType15 contains information related to mobility procedures for MBMS reception;

-	SystemInformationBlockType16 contains information related to GPS time and Coordinated Universal Time (UTC);

-	SystemInformationBlockType17 contains information relevant for traffic steering between E-UTRAN and WLAN;

-	SystemInformationBlockType18 contains information related to sidelink communication;

-	SystemInformationBlockType19 contains information related to sidelink discovery;

-	SystemInformationBlockType20 contains information related to SC-PTM;

-	SystemInformationBlockType21 contains information related to V2X sidelink communication;

-	SystemInformationBlockType24 contains information about NR frequencies and NR neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency), which can also be used for NR idle/inactive measurements;

-	SystemInformationBlockType25 contains information about UAC parameters;

-	SystemInformationBlockType26 contains additional information related to V2X sidelink communication;

-	SystemInformationBlockType26a contains information related to NR bands list which can be used for EN-DC operation with the serving cell;

-	SystemInformationBlockType27 contains assistance information for inter-RAT cell selection to NB-IoT and/or to NR NTN;

-	SystemInformationBlockType28 contains information related to NR sidelink communication;

-	SystemInformationBlockType29 contains information related to common resource reservation;

-	SystemInformationBlockType30 contains information related to disaster roaming;

-	SystemInformationBlockType31 contains information required for accessing an NTN cell;

-	SystemInformationBlockType32 contains assistance information for discontinuous coverage in NTN;

-	SystemInformationBlockType33 contains assistance information related to neighbouring NTN cells for measurements.

System information for NB-IoT is divided into the MasterInformationBlock-NB (MIB-NB) and a number of SystemInformationBlocks-NB (SIBs-NB):

-	MasterInformationBlock-NB defines the most essential information of the cell required to receive further system information;

-	SystemInformationBlockType1-NB contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information blocks;

-	SystemInformationBlockType2-NB contains common radio resource configuration information;

-	SystemInformationBlockType3-NB contains cell re-selection information for intra-frequency, inter-frequency;

-	SystemInformationBlockType4-NB contains neighboring cell related information relevant for intra-frequency cell re-selection;

-	SystemInformationBlockType5-NB contains neighboring cell related information relevant for inter-frequency cell re-selection;

-	SystemInformationBlockType10-NB contains an ETWS primary notification;

-	SystemInformationBlockType11-NB contains an ETWS secondary notification;

-	SystemInformationBlockType12-NB contains a CMAS warning notification;

-	SystemInformationBlockType14-NB contains information about access barring;

-	SystemInformationBlockType15-NB contains information related to mobility procedures for MBMS reception;

-	SystemInformationBlockType16-NB contains information related to GPS time and Coordinated Universal Time (UTC);

-	SystemInformationBlockType20-NB contains information related to SC-PTM;

-	SystemInformationBlockType22-NB contains common radio resource configuration information for paging and random access procedure on non-anchor carriers;

-	SystemInformationBlockType23-NB contains common additional radio resource configuration information for random access procedure on anchor and non-anchor carriers;

-	SystemInformationBlockType27-NB contains assistance information for inter-RAT cell selection to E-UTRAN, GERAN and/or NR NTN;

-	SystemInformationBlockType31-NB contains information required for accessing an NTN cell;

-	SystemInformationBlockType32-NB contains assistance information for discontinuous coverage in NTN;

-	SystemInformationBlockType33-NB contains assistance information related to neighbouring NTN cells for measurements.

On MBMS-dedicated cell, only system information relevant for receiving MBMS service is broadcasted. MasterInformationBlock-MBMS (MIB-MBMS) and SystemInformationBlockType1-MBMS (SIB1-MBMS) are used instead of MIB and SIB1 respectively:

-	MasterInformationBlock-MBMS defines the most essential physical layer information of the cell required to receive further system information on MBMS-dedicated cell;

-	SystemInformationBlockType1-MBMS contains information relevant for receiving MBMS service and defines the scheduling of other system information blocks on MBMS-dedicated cell;

The MIB is mapped on the BCCH and carried on BCH while all other SI messages are mapped on the BCCH and BR-BCCH, and carried on DL-SCH. Except for BL UEs, UEs in enhanced coverage and NB-IoT UEs, all other SI messages than the MIB which are dynamically carried on DL-SCH, can be identified through the SI-RNTI (System Information RNTI). Both the MIB and SystemInformationBlockType1 (SystemInformationBlockType1-BR for BL UEs and UEs in enhanced coverage) use a fixed schedule with a periodicity of 40 and 80 ms respectively. The scheduling of other SI messages is flexible and indicated by SystemInformationBlockType1 (SystemInformationBlockType1-BR for BL UEs and UEs in enhanced coverage, and SystemInformationBlockType1-NB for NB-IoT). For NB-IoT, the MIB-NB is mapped on the BCCH and carried on BCH while all other SI messages are mapped on the BCCH and carried on DL-SCH. Both the MIB-NB and SystemInformationBlockType1-NB use a fixed schedule with a periodicity of 640 and 2560 ms respectively. The MIB-NB contains all information required to acquire SIB1-NB and SIB1-NB contains all information required to acquire other SI messages.

On MBMS-dedicated cell, the MIB-MBMS and SIB1-MBMS use a fixed schedule with a periodicity of 160 ms. Additionally, SIB1-MBMS may be scheduled in additional non-MBSFN subframes indicated in MIB-MBMS.

For NB-IoT, in TDD mode, the MIB-TDD-NB is transmitted on the same NB-IoT carrier as NPSS/NSSS, SystemInformationBlockType1-NB can be transmitted on NB-IoT carrier other than the MIB-NB, and the SI messages can be transmitted on a NB-IoT carrier other than the MIB-NB. At most two NB-IoT carriers are used to transmit the MIB-NB, SystemInformationBlockType1-NB and the SI messages.

Except for NB-IoT, the eNB may schedule DL-SCH transmissions concerning logical channels other than BCCH or BR-BCCH in the same subframe as used for BCCH or BR-BCCH. The minimum UE capability restricts the BCCH or BR-BCCH mapped to DL-SCH e.g. regarding the maximum rate.

The Paging message is used to inform UEs in RRC_IDLE and UEs in RRC_CONNECTED about a system information change. For NB-IoT UEs, BL UEs, and UEs in CE, the UE is not required to detect SIB changes when in RRC_CONNECTED, and the network may release the NB-IoT UE, BL UE or UE in CE to RRC_IDLE if it wants the NB-IoT UE, BL UE or UE in CE to acquire changed SIB(s).

Except for NB-IoT, system information may also be provided to the UE by means of dedicated signalling e.g. upon handover.

<<Unchange parts are omitted>>

Next change

<<Unchange parts are omitted>>

## 23.21	Support for BL UEs, UEs in enhanced coverage and NB-IoT UEs over Non-Terrestrial Networks

<<Unchange parts are omitted>>

### 23.21.4	Mobility Management

#### 23.21.4.1	Mobility Management in ECM-IDLE

The principles described in clause 10.1.1 apply in NTN unless specified otherwise hereafter.

The network may broadcast more than one TAC per PLMN in an NTN cell. The AS layer indicates all received TACs for the selected PLMN to the NAS layer. The network may update the UEs upon TAC removal. The UEs may, by UE implementation, check whether a TAC has been removed.

For quasi-Earth-fixed cells, timing information on when the cell is going to stop serving the area may be broadcast by the network. This may be used by the UE to start measurements on neighbour cells before the broadcast stop time of the serving cell, while the exact start of the measurements is up to UE implementation.

Initiation of measurements based on location and time is supported for cell reselection.

The network may provide assistance information for inter-RAT cell selection from IoT-NTN to NR NTN. A UE may use	 the assistance information (e.g., ephemeris information, carrier frequency) provided by the network for cell selection from IoT NTN to NR NTN.

End of change