3GPP TSG-RAN WG2 Meeting #133	R2-2600223

# Gothenburg, Sweden, 9th – 13th Feb, 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 1412 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Per band and Per band combination |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi, Samsung, Ericsson, Qualcomm Incorporated, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_air, NR_MIMO_Ph5 |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Following agreements were agreed in RAN2 #132 meeting on ‘per band and per BC’ capability: Agreements from RAN2 perspective - No matter CA is configured or not, if the capability/component is not counted across CCs, the minimum capability between per BC capability and per band capability should be applied for a band in case of band combination (CA). - For the capability/component is counted across CCs, wait for RAN1 for the conclusion. - When per BC capability is indicated but some subset of the bands in the BC doesn’t indicate per band capability, the capability/component is not supported in the band without per band capability, the capability/component is supported only in the band with per band capability signalled. This applies to both capability/component that is counted/not counted across CCs. - When the UE reports the per band capability but does not include the per BC capability for a certain BC, the UE supports the feature as indicated in the per band capability without further per BC limitations. This applies to both capability/component that is counted/not counted across CCs. - If Feature B is prerequisite of Feature A and both features are ‘per band and per band combination’: a. A UE supporting feature A per band shall also indicate support of feature B at the given band; b. A UE supporting feature A per BC shall also indicate support of feature B at the given BC. 6 For per-band per-BC capabilities for MIMO codebook capabilities: - If a UE supports such capabilities in a set of bands separately (per-band) then the UE must always support those capabilities in any CA combination composed of the respective bands (per-BC). No further enhancement is needed for this case. 8 Inform RAN1 about our agreements 7 Update at least Rel-19 CRs. FFS whether we need Rel-16, 17, 18 CRs Furthermore, RAN1 further agreed: - When UE indicates both per band and per BC capability, if the capability/component is counted across CCs and CA is not configured, then - gNB only considers per Band capability regardless of reported per BC capability (i.e., per BC capability can be ignored in this case) - When UE indicates both per band and per BC capability, if the capability/component is counted across CCs and CA is configured, then - if all CCs over the CA are within the same band (intra-band CA), the CA is restricted by the per band capability/component corresponding to the CA - if the CCs of the CA are associated with a band combination (inter-band CA), the CA is restricted by the per BC capability/component corresponding to the CA. |
| Summary of change: |  | - Introduce the definition of ‘per band and per band combination’ granularity. - Introduce the general principle for final capability and prerequisite for capability with granularity of ‘per band and per band combination’. Impact Analysis Impacted 5G architecture options: NR SA, NR-DC Impacted functionality: MIMO, AI/ML air Inter-operability: If the network is implemented according to this CR and UE is not, or If the UE is implemented according to the CR and network is not, the UE and the NW may have different interpretation on the actual UE capability especially when per band and per BC capability are different or one of capabilies is absent. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Final capability and prerequisite of features with ‘per band and per band combination’ is not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR …CR … |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

# 4	UE radio access capability parameters

## 4.2	UE Capability Parameters

### 4.2.1	Introduction

The following clauses define the UE radio access capability parameters. Only parameters for which there is the possibility for UEs to signal different values are considered as UE radio access capability parameters. Therefore, mandatory features without capability parameters that are the same for all UEs are not listed here.

The network needs to respect the signalled UE radio access capability parameters when configuring the UE and when scheduling the UE.

For capabilities that required to be set consistently for all FDD-FR1 bands (i.e. capabilities that are supposed to be per UE), the UE shall also set capability values for all SUL bands with same values for FDD-FR1 bands if SUL band is supported by the UE.

The UE may support different functionalities between FDD and TDD, and/or between FR1 and FR2. The UE shall indicate the UE capabilities as follows. In the table of UE capability parameter in subsequent clauses, "Yes" in the column by "FDD-TDD DIFF" and "FR1-FR2 DIFF" indicates the UE capability field can have a different value for between FDD and TDD or between FR1 and FR2 and "No" indicates if it cannot. "(Incl FR2-2 DIFF)" in the column by "FR1-FR2 DIFF" indicates the UE capability field can have a different value for between FR2-1 and FR2-2. Regarding to the per UE capabilities that are FDD/TDD differentiated(i.e. capabilities indicated as "Yes" in the column by "FDD-TDD DIFF"), the corresponding capabilities indicated by the FDD capability is applied to SUL/SDL if SUL/SDL band is supported by the UE. "FD" in the column indicates to refer the associated field description. "FR1 only" or "FR2 only" in the column indicates the associated feature is only supported in FR1 or FR2 and "TDD only" indicates the associated feature is only supported in TDD and not applicable to SUL/SDL carriers. "N/A" in the column indicates it is not applicable to the feature (e,g. the signalling supports the UE to have different values between FDD and TDD or between FR1 and FR2).

1>	set all fields of UE-NR/MRDC-Capability except fdd-Add-UE-NR/MRDC/Sidelink-Capabilities, tdd-Add-UE-NR/MRDC/Sidelink-Capabilities, fr1-Add-UE-NR/MRDC-Capabilities and fr2-Add-UE-NR/MRDC-Capabilities, to include the values applicable for all duplex mode(s) and frequency range(s) that the UE supports;

1>	if UE supports both FDD (or SUL/SDL) and TDD and if (some of) the UE capability fields have a different value for FDD (or SUL/SDL) and TDD:

2>	if for FDD (and, if the UE supports SUL/SDL, for SUL/SDL), the UE supports additional functionality compared to what is indicated by the previous fields of UE-NR/MRDC-Capability/SidelinkParameters:

3>	include field fdd-Add-UE-NR/MRDC/Sidelink-Capabilities and set it to include fields reflecting the additional functionality applicable for FDD;

2>	if for TDD, the UE supports additional functionality compared to what is indicated by the previous fields of UE-NR/MRDC-Capability/SidelinkParameters:

3>	include field tdd-Add-UE-NR/MRDC/Sidelink-Capabilities and set it to include fields reflecting the additional functionality applicable for TDD;

1>	if UE supports both FR1 and FR2 and if (some of) the UE capability fields have a different value for FR1 and FR2:

2>	if for FR1, the UE supports additional functionality compared to what is indicated by the previous fields of UE-NR/MRDC-Capability:

3>	include field fr1-Add-UE-NR/MRDC-Capabilities and set it to include fields reflecting the additional functionality applicable for FR1;

2>	if for FR2, the UE supports additional functionality compared to what is indicated by the previous fields of UE-NR/MRDC-Capability:

3>	include field fr2-Add-UE-NR/MRDC-Capabilities and set it to include fields reflecting the additional functionality applicable for FR2;

NOTE 1:	The fields which indicate "shall be set to 1" or "shall be set to supported" in the following tables means these features are purely mandatory and are assumed they are the same as mandatory without capability signalling.

NOTE 2:	For the case where the UE is allowed to support different functionality between FDD and TDD and between FR1 and FR2 according to the specification, the UE capability indication is clarified in Annex B.

NOTE 2a:	In this release of the specification, if the UE is allowed to support different functionalities between FDD and TDD, and/or between FR1 and FR2, these functionalities are signalled per band with the text "UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively".

For optional features, the UE radio access capability parameter indicates whether the feature has been implemented and successfully tested. For mandatory features with the UE radio access capability parameter, the parameter indicates whether the feature has been successfully tested. In the table of UE capability parameter in subsequent clauses, "Yes" in the column by "M" indicates the associated feature is mandatory and "No" indicates the associated feature is optional. "CY" in the column indicates the associated feature is conditional mandatory and the condition is described in the field description and the associated feature is considered mandatory with capability parameter, when the described condition is satisfied. "FD" in the column indicates to refer the associated field description. Some parameters in subsequent clauses are not related to UE features and in the case, "N/A" is indicated in the column.

UE capability parameters have hierarchical structure. In the table of UE capability parameter in subsequent clauses, "Per" indicates the level the associated parameter is included. "UE" in the column indicates the associated parameter is signalled per UE, "Band" indicates it is signalled per band, "BC" indicates it is signalled per band combination, "FS" indicates it is signalled per feature set (per band per band combination), "FSPC" indicates it is signalled per feature set per component carrier (per CC per band per band combination), and "FD" in the column indicates to refer the associated field description. "Per band and per band combination" indicates a UE capability parameter defined in both per band and per band combination with same feature components.

For a capability with same comprised parameter(s) defined "per band and per band combination":

-	When the UE signals "per BC" but doesn’t signal "per band" capability on some subset of the bands in the BC, the UE does not support the capability/comprised parameter(s) in the band without "per band" capability.

-	When the UE signals "per band" but does not include "per BC" for a certain BC, the UE supports the capability/comprised parameter(s) as indicated in the "per band" without further per BC limitations. The UE shall also support the signalled "per band" capabilities in any CA combination composed of the respective band.

-	When the UE signals both "per band" and "per BC" capability, if capability/ comprised parameter(s) is not counted across CCs, the minimum capability between "per BC" capability and "per band" capability is applied to a band for which the UE capability is signalled; if the comprised parameter(s) is counted across CCs and CA is not configured, the "per band" capability is applied regardless of reported per BC capability; if the comprised parameter(s) is counted across CCs of intra-band CA (all CCs over the CA are within the same band), the "per band" capability is applied across CCs within the corresponding intra-band CA; if the comprised parameter(s) is counted across CCs of inter-band CA (all CCs over the CA are associated with more than one band), the "per BC" capability is applied across CCs within the corresponding inter-band CA.

For "per band and per band combination" capabilities with prerequisite capability in "per band and per band combination", the UE shall indicate support of the prerequisite capability in the corresponding band/BC, respectively.

NOTE 3:	Unless otherwise specified, for dependent capabilities with prerequisite capability in a finer granularity, the UE should indicate support of the prerequisite capability in at least one finer granularity. And the dependent capability is supported only in the finer granularity where the prerequisite capability is supported, e.g. a UE indicating support of supportNewDMRS-Port-r16 (dependent capability which is defined per band) should indicate at least one band combination where singleDCI-SDM-scheme-r16 (prerequisite capability which is defined per feature set) is supported in the corresponding band. In this case, supportNewDMRS-Port-r16 is considered supported only in the corresponding band of the band combination where singleDCI-SDM-scheme-r16 is supported.

End of CHANGES