3GPP TSG-RAN WG2 Meeting #133											R2-260xxxx

Gothenburg, Sweden,  Feb. 09 – 13

Source: 		Offline Rapporteur (ZTE)

Title:                Offline discussion summary: [AT133][008][6G] Security (ZTE)

Agenda item:   10.2.3

Document for: Discussion and Decision

# Status of the online discussion

- At least ask is PQC applicable to the UE air interface.

For SA3

Indicate that currently in 5G

a.	MAC layer has no Sequence Number and no in-sequence delivery.

b.	MAC CEs can be grouped together with other MAC CEs and/or MAC SDUs

RAN2 will discuss based on new requirements coming from SA3 how to modify the MAC

Ask questions

-	[CB]What are the expected increase in complexity on the network side relative to 5G

Indicate

-	Certain MAC CEs (e.g., BSR, PHR, TA) need to be sent before security establishment.

-	[CB] identify time critical for MAC CE for UL and DL and can identify the MAC CEs that are determined late in the TB construction, which may affect the timing of security processing in some designs.

-	[check if we have an order of magnitude security processing currently for 5G to understand time criticality]

[AT133][008][6G] Security (ZTE)

Intended outcome:

Discuss the CBs remaining from MAC CE and if they are any other questions.

Check if there are easy/generic questions for SIB security and determine after offline if we send the questions to SA3.

Draft LS will be post meeting

Deadline:  Thursday

So, the offline discussion should focus on the above [CB] topics first and then we can see if we have time for any other issues.

# Time critical MAC CEs

What is meant by time critical?

- Inclusion of a MAC CE in a transport block may be determined late in the TB construction, which may mean that adding security requirement on top may make it problematic and/or

- Processing the MAC CE quickly is important and hence adding security processing latency is not desirable as it may impact the system performance

Rapporteur Notes: check above understanding and try to reach a common view

Discussion:

- Rapporteur notes:

  - Some companies said that additional processing latency is only in the order of few 10s of micro seconds (e.g. see Figures 3 – 6 in R2-2600205 for instance). But others seem to think this is still something that may be problematic (R2-2600777). Seems this aspect needs further discussion unless the above definition of “time criticality” solves this issue.

  - Note that even if we don’t include some MAC CE as “time critical”, it doesn’t mean there are no other RAN2 concerns/considerations. For instance, overhead criticality may still be applicable (likely for all MAC CEs) regardless (hopefully this is clear to SA3?).

Discuss which MAC CEs fall under the “time critical” category per above definition?

|  | MAC CE | Time critical or Y/N? | Comments/Discussion |
| --- | --- | --- | --- |
| UL | BSR/DSR/PHR | ? |  |
|  | C-RNTI MAC CE | ? |  |
|  | Configured Grant Confirmation and Multiple Entry Configured Grant Confirmation | ? |  |
|  | Anything else??? | ? |  |
| DL | Absolute Timing Advance Command MAC CE | ? |  |
|  | UE Contention Resolution Identity MAC CE | ? |  |
|  | Timing Advance and Absolute Timing Advance | ? |  |
|  | LTM Cell Switch Command and Enhanced LTM Cell Switch Command | ? |  |
|  | LTM Candidate Timing Advance Command | ? |  |
|  | PUCCH Spatial Relation Activation/Deactivation | ? |  |
|  | TCI State Indication for UE-specific PDCCH | ? |  |
|  | TCI States Activation/Deactivation for UE-specific PDSCH | ? |  |
|  | Anything else??? | ? |  |

# SIB Security

Check if following questions can be asked (it was suggested online to keep the questions high level – so start with below)

| Question | Discussion |
| --- | --- |
| Is security (integrity protection) applicable to system information? |  |
| What is the overhead for SIB security? |  |
| Anything else to ask/tell SA3?? |  |

# Security for paging

Check if following questions can be asked

| Question | Discussion |
| --- | --- |
| Is security applicable to paging information? |  |
| What is the overhead associated with security for paging? |  |
| Anything else to ask/tell SA3?? |  |

# Algorithm complexity

Rapporteur Notes:

- Quick discussion on network side complexity for MAC CE security to see if we can have any consensus to capture something and ask SA3 any specific questions.

- Some companies point out that there was interest in understanding the algorithmic complexity (affecting both network and UE sides) and whether we will face the “hard” PQC algorithms on the critical path for processing signalling.

Rapporteur Notes:

- The intention seems to be to understand hardware impacts on network side

- Note that we focus on MAC security – i.e. the MAC CEs that are generated not that often (i.e. not as often as UP packets)

| Question | Discussion |
| --- | --- |
| Any questions on impact on hardware with the new MAC security requirements that we should ask SA3? | MTK: Everyone in RAN2 is assuming that we will not have to go through a PK-type process for every MAC CE, but it would be good to understand if SA3 have some insight into what level of processing will be expected, for instance, to generate a MAC-I. |

# AoB

| Question | Discussion |
| --- | --- |
| Any other critical information to convey to SA3 to help our joint session? | Sony: Can the lower layer security/AS security be optional at least for low complexity devices like IoT? LG: - If PQC is applied, what are the radio protocol impacts in air interface? - Should the UE support only PQC algorithm, or both legacy security algorithm and PQC algorithm? |
| Any other critical question to ask SA3 to help our joint session? | vivo: Consult SA3 on the order of magnitude of processing latency for MAC CE security operations, i.e., What is the order of magnitude of processing latency for MAC CE security operations? |

# Conclusion

TBD

- References