3GPP TSG RAN WG2 #133		R2-26xxxxx

Gothenburg, Sweden, 9th – 13th February 2026

Title:	LS on capability reporting for PRS Processing Window

Release:	Release 19

Work Item:	NR_AIML_air_Ph2

Source:	RAN WG2

To:	RAN WG1

Cc:

Contact Person:

Name:	Ritesh Shreevastav

E-mail Address:	ritesh.shreevastav@ericsson.com

Attachments: -

1. Overall Description:

RAN2 would like to get feedback from RAN1 on below capability 58-2-11 [R1-2506627].

| 58. NR_AIML_Air | 58-2-11 | Support of more than one activated PRS processing windows across all active DL BWPs for UE-based positioning Case 1 | 1. Number of supported activated PRS processing windows | 58-2-9 | N/A | N/A | DL PRS measurement outside MG and in a PRS processing window for UE-based positioning Case 1 is not supported | Per UE | No | No | No | Candidate values: {2, 3, 4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

We observed that for legacy (non-AIML) positioning, the capability

27-23: Support of more than one activated PRS processing windows across all active DL BWPs  is sent only to gNB.

Thus, RAN2 would like to get an answer from RAN1 if the capability 58-2-11 should be known only to gNB or both gNB and LMF.

Further, RAN2 would like to get confirmation from RAN1 on whether the capability 58-2-9 and 58-2-4 should also be known to gNB or not. We observe that it has been set to N/A and NO respectively for the field “Need for the gNB to know if the feature is supported”. In legacy PPW config, the details are known to gNB so that gNB can configure the PPW configuration as per UE capability.

| 58. NR_AIML_Air | 58-2-9 | DL PRS measurement outside MG and in a PRS processing window for UE-based positioning Case 1 | 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG and in a PRS processing window 2. Support of priority handing options of PRS: Option1, Option2 or Option3 − Option 1: Support of "st1" and "st3" defined in clause 5.1.6.5 of TS 38.214 [20] − Option 2: Support of "st1", "st2", and "st3" defined in clause 5.1.6.5 of TS 38.214 [20] - Option 3: Support of "st1" only defined in clause 5.1.6.5 of TS 38.214 [20] | 58-2-4 | N/A |
| --- | --- | --- | --- | --- | --- |

| 58. NR_AIML_Air | 58-2-4 | DL PRS Processing Capability for UE-based positioning Case 1 | 1. Maximum DL PRS bandwidth in MHz, which is supported and reported by UE. 2. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 3. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. 4. Max number of DL PRS resources that UE can process in a slot under it Note: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) of no more than 30%. |  | No |
| --- | --- | --- | --- | --- | --- |

2. Actions:

To RAN WG1

ACTION: 	RAN2 kindly asks RAN1 to provide feedback on the above questions.

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN WG2 Meeting RAN2#133-bis	Malta		2026-04-13	2026-04-17

TSG-RAN WG2 Meeting RAN2#134		Dalian		2026‑05‑18	2026‑05‑22