3GPP TSG-RAN2 Meeting #133	Draft R2-2601191

Gothenburg, Sweden, Feb. 09th – 13th, 2026

Title:	[Draft] Reply LS on further signaling for MPR enhancement

Response to:	LS (R2-2600022/R4-2522409) LS on further signaling for MPR enhancement

Release:	Rel-19

Work Item:	NR_ENDC_RF_Ph4-Core

Source:	Apple [To be RAN2]

To:	RAN4

Cc:

Contact Person:

#### Name:	Yuqin Chen

E-mail Address:	yuqin_chen@apple.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN2 thanks RAN4 for the LS on further UE capability signaling for MPR enhancement. RAN2 discussed the LS and achieved the following agreement to introduce a positive capability instead of a negative one.

RAN2 also would like to ask RAN4 whether it is acceptable to change the granularity of the new UE capability to “per band”, instead of “per band per band combination”, considering that the mprReductionExtensionRatio-r19 is limited to non-CA case (for both DL and UL).

mprReductionExtensionRatio Indicates the pair of lower-sided and higher-sided extension ratio as specified in TS 38.101-1 [15]. The ratio1 represents (1/2, 0), the ratio2 represents (0, 1/2), ratio3 represents (1/2, 1/2), ratio4 represents (1/4, 0), ratio5 represents (0, 1/4), ratio6 represents (1/4, 1/4). The first/leftmost value of the pair represents the lower-sided extension ratio, the second value of the pair represents the higher-sided extension ratio. The network can only configure this field in case of non-CA in NR SA.

Below please find the RAN2 agreements:

RAN2 define the capability to indicate UE is capable to apply Rel-19 requirement related to mprReductionExtensionRatio-r19 and powerBoostPi2BPSK-r18/powerBoostQPSK-r18. If this capability bit is absent, the UE would only be able to meet the requirements per Rel-18 power boosting.

Ask RAN4 whether it is acceptable to change the granularity to “per band”, and inform the RAN2 understanding that the configuration of mprReductionExtensionRatio-r19 is limited to non-CA case (for both DL and UL).

Send a response LS to RAN4 with the above agreement/question.

2. Actions:

To RAN4

ACTION: 	RAN2 kindly asks RAN4 to provide answer to the question and inform RAN2 if there is any concern on the agreement.

3. Date of Next TSG-RAN2 Meetings:

TSG RAN2 Meeting #133-bis	13 – 17 April 2026		Malta

TSG RAN2 Meeting #134                       18 – 22 May 2026                       China